Figure 1.142 Replacement of  End Inverters by capacitive L-sections.

iii ) Scale Inverter

This command enables one to adjust the inverter constant or adjust the element values on one or both sides of the inverter (Figure 1.143). Several options exist depending on the position of inverter in the circuit:

a) Scaling internal inverters:

If the inverter is inside the circuit (not at the source or load end), the Scale Inverter command displays the following options:

- Specify New K

- Specify Transformer

- Equate Inductors or SC Stubs (if there are inductors or SC stubs on the two sides of inverter).

- Equate capacitors or OC stubs (if there are capacitors or OC stubs on the two sides of inverter).

In all of these options the inverter constant is changed by extracting a transformer. The transformer is used to adjust the desired element values. Transformer may be placed on the left or right of the inverter. User can specify either the new inverter constant or the transformer turns ratio. When there are identical type elements on the two sides (inductors, capacitors, SC stubs or OC stubs on both sides), then one has also the options of equating these elements. In Figure 1.143, inductors on both sides of an internal inverter are equated by scaling the inverter. The inductors become equal after shifting the resulting transformer righmost.

Figure 1.143. Scale Inverter command on an internal inverter.

b) Scaling End Inverters:

If the scale inverter command is applied on the inverters at source or load ends, then in general the option Specify Inverter option appears. The new inverter constant is specified. Then inverter constant is scale to the new vlaue while extracting a transformer. The transformer may be placed on the right or left of inverter and is shifted to adjust the element values of circuit or to adjust source resistance (Figure 1.144). If the element adjacent to the inverter is an inductor, capacitor or LC resonator in shunt or series, then two more options appear where we can specify the targeted inductane or capacitance for the adjacent element.

Figure 1.144. Scale Inverter command applied on an end inverter.

iv ) Scale RS/RL By Inserting Inverter

This command inserts proper inverters at source or load ends (with cursor being under RS or RL) such that RS or RL will get the specified values (Figure 1.145).

Figure 1.145. Scale RL/RS By Inserting Inverters command.

v ) Scale Element By Inverter

Since inverter constants are free parameters chosen at the outset, one can change the impedances of elements between inverters by choosing different inverter constants (Figure 1.146). Such impedance scaling can also be performed later after the original element is replaced by its inverter  equivalent. This is possible by placing the cursor under the element to be scaled (in between two ideal inverters) and applying Scale Element By Inverters command. For some elements, the new value of element is asked by Filpro while in general, a constant multiplying impedance of the element is asked from the user. When these parameters are typed in, the inverter constants are automatically adjusted to set the new value of the element.

Figure 1.146.  Scaling element values between inverters by assigning new elemnt value and by assigning an impedance multiplying factor.

vi ) Scale Circuit By Inserting Inverter:

This command is used to insert inverters at both ends of  the filter in order to scale up or down all the element values of the circuit while keeping source impedances unchanged (Figure 1.147).

Figure 1.147. Scaling circuit elements by inserting inverters.

In general user specifies the inverter constant. However, if there is an inductor, capacitor, SC Stub, OC Stub or parallel combinations of the elements as the first element adjacent to the source, then user may also specify the targeted value of this element. Then, Filpro calculates the inverter constant necessary  to scale this element to the specified value and inserts the proper inverters at source and load ends.

vii ) Insert Inverter at Interior:

This command is used to insert an inverter at an interior point of the circuit (Figure 1.148). Inverter is inserted at the left of the cursor. After insertion of the inverter, dual of the portion of the circuit to the right of the cursor appears on the screen. One of the main use of this command is conversion of an antimetric filter into physically symmetric form, if the inverter constant is selected as K= (RS*RL)1/2.

Figure 1.148. Inserting inverter at an interior point.

viii ) Absorb Transformer by Inverter

An ideal transformer adjacent to an ideal inverter can be absorbed into the inverter as described in Figure 1.149.

Figure 1.149. Absorption of transformers by inverters.

# IX ) Pi-to- Inverter

This command converts inductive or capacitive Pi-sections into their inverter equivalents. In order this command to work, the shunt elements must be of parallel LC resonators (Figure 1.150).

Figure 1.150. Application of Pi-to-Inverter command.

# X ) Inverters – All Equal Left-right

This command is used to equate all inductors/SC stubs or capacitors/OC stubs of a filter where these elements are separated by inverters, as described in Figure 1.151. The algorithm equates the elements to the leftmost one.

Figure 1.151. Equating shunt inductors by applying the command Inverters / All Equal Left-Right.

1. 8. 9. Constant Reactances

Constant reactances are ideal, frequency independent, lossless elements. They are extremely useful components in many narrowband applications. This menu item has the following options  (Figure 1.152):

Figure 1.152. Constant Reactance command and options.

i ) Replace By Constant Reactance

This command calculates reactance of the element pointed by the cursor at the frequency specified by the user and replaces the element by the calculated constant reactance, as described in Figure 1.153.

Figure 1.153. Replacing an element by its constant reactance equivalent.

ii ) Replace Constant Reactance By

This is the reverse of previous command. User specifies the frequency of approximation and selects the desired element type. Since a constant reactance  has many lumped or distributed equivalents, this is a nonunique operation providing a high degree of flexibility to designers for finding alternative equivalent circuits. Some of the possible replacements are programmed into Filpro, as shown in Figure 1.154.

iii ) All  L, C to Constant Reactance:

This is a short cut to convert all the elements of a circuit into constant reactance form calculated at the specified frequency.

Figure 1.154. Replacing a constant reactance by some lumped and distributed elements.

1. 8. 10. Evanescent Waveguides

Command set under this item is shown in Figure 1.155. These commands are used to develope evanescent waveguide filters (simple or cross-coupled) using either Design / By Synthesis or Design / By Prototype with Inverters approaches. See Chapter-14 and Chapter-16 for detailed descriptions of a large variety of circuit transformations related to evanescent waveguides.

Figure 1.155. Commands related to Evanescent Waveguides.

1. 9. PLOT  MENU

Insertion loss, return loss, insertion phase, group delay and Smith Chart responses of the displayed circuit can be plotted using Plot menu commands.

1. 9. 1. Insertion Loss

Insertion loss response can be plotted as function of frequency by selecting Insertion Loss command or simply pressing the short cut keys Shift+F1 (Figure 1.156).

- Response is calculated at 100 equally spaced frequencies. This number can be increased to 200 by the user.

- Frequency limits, maximum and minimum insertion loss plot limits and number of frequencies can be set by the user.

- Rescaling can be done by pressing the key F10. The same key can also be used to plot other responses (Return Loss, etc.).

- A marker is available moving with Right-Left arrows or fast moving with Ctrl+Right-Left arrows. Frequency and insertion loss at the point of marker is displayed at the left bottom corner.

Figure 1.156.  Insertion loss response.

- Pressing Space Bar places a Delta Marker at the position of marker. From that point on, the delta marker is used as reference for measuring frequency and insertion loss at the position of marker and the differences are displayed at the bottom-left corner of the display, together with the actual position of marker. Default position of Delta Marker is origin (0, 0).

- Home key can be used to increase the maximum insertion loss plot limit. End key decreases this limit.

- Page Up-Page Down keys can be used to shift the baseline (zero insertion loss reference) up or down.

- Frequency span can be increased or decreased using Insert-Delete keys.

- The key F9 can be used to zoom the region between Delta Marker and Marker.

- All plots are default in overplot mode. The key F8 can be used to refresh the screen.

- The key F7 centers the cursor position on the screen.

- Reference lines can be placed to highlight the passbands and stopbands of the filter (see Reference Lines command).

1. 9. 2. Return Loss Response

- Return loss response can be plotted by selecting Return Loss command in Plot menu or by simply using the short cur key, Shft+F2. Default limit is 50 dB and can be set by the user (Figure 1.157).

- All the keys that were described for insertion loss response are also valid for return loss.

Figure 1.157. Return loss plot.

1. 9. 3. Insertion Phase

Phase response of the output signal with reference to input signal (Insertion Phase) can be plotted as function of frequency using the command Insertion Phase in the Plot menu or using the short cut key Shift+F3. Limits are default 0 degree to 360 degrees and can be set by the user (Figure 1.158).

Figure 1.158. Insertion phase plot.

1. 9. 4. Group Delay

Group delay is negative of the slope of insertion phase. It can be plotted by selecting Group Delay command in Plot menu or by using the short cut keys Shift+F4 (Figure 1.159).

Figure 1.159. Group delay plot.

1. 9. 5. Smith Chart

Smith Chart response can be plotted by selecting Smith Chart command in Plot menu (Figure 1.160) or by using the short cut key Shift+F6. Smith Chart has two moving markers. One of them (+) moves on the response curve and is termed as Impedance or S11 indicator while the other one is symmetric with respect to origin and is termed as Admittance marker. Markers move with Left-Right arrows. The frequency, S11, input impedance, input admittance and Voltage Standing Wave Ratio (VSWR) at the position of the marker are all displayed in the inset at top right corner.

Figure 1.160. Smith Chart response.

1. 9. 6. Reference Lines

Reference lines can be placed to show the boundaries (Brick Walls) through which the response curve must pass without touching them. Besides being a visual aid, these lines are also necessary in the Monte Carlo, Yield and  Best Case  analyses that will be described later.

Plot / Reference Lines command displays the menu shown in Figure 1.61. Menu items are (hoped to be) self explanatory. Some examples are shown in Figures 1.162-163. In usual practice it is quicker to use Auto Add Reference lines / Return Loss only command, as it places the reference line in passband with default return loss level of the designed filter.

Figure 1.161. Reference Line commands.

Figure 1.161. Reference Line commands (continued).

Figure 1.162. A return loss response with reference line.

Figure 1.163. Reference lines in insertion loss response.

1. 9. 7. Save Response

This command saves the last plotted response in frequency-magnitude pairs into a text file with extension Filename.RSP. File can be viewed using any ASCII editor or viewer. Figures 1.164 and 165 show how to  save a response and viewing the saved response.

Figure 1.164. Save Response command on return loss response of a filter named as Comb112.fil.

Figure 1.165. Viewing the textfile of Comb112.fil.

1. 9. 8. Save S-Parameters

This command calculates and saves S-parameters of the circuit with reference to the displayed terminations. S-parameters are saved in Touchstone-Libra (TM) format with same filename of the circuit but with extension .S2P. Files can be saved in Magnitude-Angle, dB-Angle or Real+Imaginary  formats (See also External 2-Port command). Figure 1.166 shows saving S-parameters of a filter named as Comb112.fil which is a 780 MHz-820 MHz bandpass filter. S-parameters are saved in Magnitude-Angle format, within the frequency range of f1=750 MHz, f2=850 MHz to cover the passband.

Figure 1.167 shows how to display the textfile Comb112.S2P.

Figure 1.168 shows how to use the saved two port circuit as a black box with specified two port S-parameters. The two port is inserted between RS and RL through the following steps:

- The command set Design / By Inserting Elements are applied to display RS=50 ohms and RL=50 ohms circuit (Figure 1.168.a).

- With cursor under RL, the command External 2-Ports / Comb112.S2p is applied to insert the saved two port circuit to the left of RL (Figure 1.168.b).

- This circuit can now be analyzed by plotting its insertion Loss, Return Loss, Delay, Phase and Smith Chart. One example is shown in Figure 1.168.c where the insertion loss is plotted in the frequency range of 750 MHz-to-850 MHz. Since the S-parameters were measured and saved in this frequency range, the responses are correct only in this frequency range. Outside this range, response is not valid. Such an example is shown in Figure 1.168.d where insertion loss is plotted in the range of 600 MHz-to-1000 MHz. Only the region 750-850 MHz is correct.

- We may also plot response of the same circuit with different terminations, as shown in Figure 1.169. Thus, we can form and save S-parameters of the two port circuit  with respect to different terminations.

Figure 1.166. Save S-Parameters command.

Figure 1.167. Displaying the textfile Comb112.S2P.

Figure 1.168. Plotting response of an External 2-Port element with saved S-parameters.

Figure 1.169. Plotting response of an External 2-Port with different terminations.

1. 10. TOOLS MENU

The following commands exist in this menu (Figure 1.170):

Figure 1.170. Tools menu.

1. 10. 1. List Elements

Displays elements of the circuit together with their parameter values. This command can be used to copy the whole circuit and paste it to somewhere else.

1. 10. 2. Find Dual

Finds dual of the displayed circuit. This command warns the user in cases when dual of a component in the circuit is not available in the library (like, exponential line or coupled line components). Cursor can be anywhere in the circuit.

1. 10. 3. Symmetrize

This command copies the mirror image of circuit on the left of cursor, including the element pointed by cursor, to the right of the cursor (Figure 1.171). It is useful for avoiding duplication of circuit transformations in physically symmetric filters. The cursor is placed under the middle element. Since the element pointed by the cursor is also duplicated, it may be necessary to delete the extra element produced in the middle.

Figure 1.171. Symmetrize command.

1. 10. 4. Reverse Circuit

This command reverses the complete circuit, including RS and RL. That is the source will be applied to the output of the original (unreversed) circuit. The roles of RS and RL will be reversed. Input will become output and vice versa.

1. 10.5. Renormalize

Element values of the complete circuit will be recalculated in accordance with the new (user specified) source end termination, RS.

1. 10. 6. External 2-Port

Filpro has the ability to accept non-library elements for analysis purposes as External 2-Port elements characterized with their measured or calculated two port parameters data (See also Save S-Parameters command). Thus, it is easy to analyze cascaded element networks having amplifiers, isolators, phase equalizers, etc. with Filpro. User may design, for example, matching circuits for amplifiers and optimize them. TouchstoneR  2-port data file format (with  .S2P  file extension) is used to characterize external two port elements of Filpro. An external two port may be formed in the following ways:

- A TouchstoneR  output data in  .S2P format can be directly used in Filpro for response calculation.

- Measured characteristics of a two port may be put into TouchstoneR    .S2P format.

- If available, analytically or numerically calculated two port parameters may be put into TouchstoneR .S2P format.

- S-parameters of any circuit that can be designed through Filpro can be saved using Save S-Parameters command and then this circuit can be inserted into any other circuit as External 2-Port element (See Save S-Parameters command).

- After creating and saving such an External 2-Port file, that element can be inserted into any position in the displayed circuit by using Tools/External 2-Ports command  (Figure 1.172). The external two port will be inserted at the left of the element pointed by the cursor. The parameters dialog will ask then

- File name (.S2P):	EXAMPLE		(no extension is required, default is .S2P)

- 2-port name:  	Quad+neighbors		(optional descriptive name, for view purposes)

Note: File must exist in path specified in Options: Directories: Data File Directory.

Then, Filpro reads the required data file for response calculation. If file contains invalid format either in unit declaration or in data section, the user is warned for the type error. If no error occurs, response is plotted. If frequency of interest is out of the given data range, external 2-port element is assumed to have unity transmission coefficient, thus not effecting response for that frequency range.

Figure 1.172. Insertion of an External Two Port element into the circuit.

Filpro requires at least 8, and accepts at most 101 frequency-data points for external files. Data beyond that will be ignored. Following is an example for an .S2P file. More detailed explanation for .S2P files can be found in TouchstoneR reference books.

- Files should contain one of the following 2-port parameters in unit declaration line (beginning with #):

- S, Z, Y or H parameters

- The two port parameters can be placed into the file in one of the following forms:

- MA (magnitude-angle),   DB (dB-angle) or    RI (real-imaginary) format

- Optional reference terminations may also be placed into the file.

Some example files are given below.

Example: Let's consider a ladder two port formed in Touchstone with the following data files:        (Capacitances in pF, Inductances in nH).

RL1 1 0 50

IND 1 0 10.1801568

CAP 1 0 207.350242

IND 1 2 645.787097

CAP 2 3 3.26865928

IND 1 3 155.234707

CAP 1 3 13.5978481

IND 3 0 10.1801568

CAP 3 0 207.350242

RL2 3 0 50

S-parameters in magnitude-angle (MA) form are obtained as follows:

#MHZ MA S R 50.00

! F   S11mag  S11ang  S12mag  S12ang  S21mag  S21ang  S22mag  S22ang

!---  ------  ------  ------  ------  ------  ------  ------  ------

70  0.9999  163.52  0.0128   73.52  0.0128   73.52  0.9999  163.52

75  0.9999  160.26  0.0128   70.26  0.0128   70.26  0.9999  160.26

80  0.9999  155.88  0.0107   65.88  0.0107   65.88  0.9999  155.88

85  1.0000  149.59  0.0028   59.59  0.0028   59.59  1.0000  149.59

90  0.9997  139.63  0.0236 -130.37  0.0236 -130.37  0.9997  139.63

95  0.9921  120.55  0.1256 -149.45  0.1256 -149.45  0.9921  120.55

100  0.7780   66.07  0.6282  156.07  0.6282  156.07  0.7780   66.07

105  0.0742  148.08  0.9972   58.08  0.9972   58.08  0.0742  148.08

110  0.0222  -95.51  0.9998   -5.51  0.9998   -5.51  0.0222  -95.51

115  0.0273 -157.95  0.9996  -67.95  0.9996  -67.95  0.0273 -157.95

120  0.7780  -66.07  0.6282 -156.07  0.6282 -156.07  0.7780  -66.07

125  0.9850 -114.26  0.1728  155.74  0.1728  155.74  0.9850 -114.26

130  0.9986 -132.62  0.0533  137.38  0.0533  137.38  0.9986 -132.62

135  0.9999 -142.33  0.0146  127.67  0.0146  127.67  0.9999 -142.33

140  1.0000 -148.46  0.0007  -58.46  0.0007  -58.46  1.0000 -148.46

145  1.0000 -152.72  0.0075  -62.72  0.0075  -62.72  1.0000 -152.72

150  0.9999 -155.88  0.0107  -65.88  0.0107  -65.88  0.9999 -155.88

Z-parameters in magnitude-angle (MA) form are obtained as follows:

#MHZ MA Z R 50.00

! F   Z11mag  Z11ang  Z12mag  Z12ang  Z21mag  Z21ang  Z22mag  Z22ang

!---  ------  ------  ------  ------  ------  ------  ------  ------

70    0.00    7.24   -0.00    0.33    0.00    0.33    0.00    7.24

75    0.00    8.70   -0.00    0.33   -0.00    0.33    0.00    8.70

80    0.00   10.68    0.00    0.28    0.00    0.28    0.00   10.68

85    0.00   13.59    0.00    0.08   -0.00    0.08    0.00   13.59

90    0.00   18.38   -0.00   -0.67   -0.00   -0.67    0.00   18.38

95    0.00   28.70   -0.00   -4.19    0.00   -4.19    0.00   28.70

100   -0.00  122.73   -0.00  -84.36    0.00  -84.36    0.00  122.73

105   -0.00   28.64   -0.00   54.02   -0.00   54.02    0.00   28.64

110    0.00 -421.24    0.00 -423.09    0.00 -423.09    0.00 -421.24

115   -0.00  -19.68   -0.00  -52.38   -0.00  -52.38   -0.00  -19.68

120    0.00 -122.73   -0.00   84.36    0.00   84.36    0.00 -122.73

125    0.00  -32.66   -0.00    6.19   -0.00    6.19    0.00  -32.66

130    0.00  -21.96   -0.00    1.59   -0.00    1.59    0.00  -21.96

135    0.00  -17.05   -0.00    0.41    0.00    0.41    0.00  -17.05

140    0.00  -14.12   -0.00   -0.02   -0.00   -0.02    0.00  -14.12

145    0.00  -12.13    0.00   -0.20   -0.00   -0.20    0.00  -12.13

150    0.00  -10.68   -0.00   -0.28   -0.00   -0.28   -0.00  -10.68

Other characterizations may be found in Touchstone reference books. Either of these filetypes can be used to characterize the external two port element. Measured or calculated two parameters must also be put into one of these formats.

1. 10. 7. External 1-Port

A previously saved one-port circuit can be inserted at the point indicated by cursor (Figure 1.173). This command is programmed mainly for forming and analyzing multiplexers by inserting channel filters in parallel or in series with or without a manifold circuit (See Chapter-17). This command can also be used to form and analyze multiport networks formed by branches in shunt arms, like Wilkinson dividers (without isolation resistors).

Figure 1.173. Insertion of an External One Port into a circuit.

1. 10. 8. External Cross-Coupled

Not operative yet. Planned for future addition of ability for conversion of sections of cascaded element filters with finite transmission zeros into cross-coupled form to be handled as External 2-Port elements.

1. 10. 9. Append Filter                                                                                                                                                        A previously saved filter may be inserted into the displayed circuit at a position the pointed by the cursor.

1. 10. 10. Macro

The Toolbox approach of Filpro provides extreme flexibility for tackling with a wide spectrum of filter design problems. The price paid for this advantage seems the tedious nature of passing through many circuit transformation steps. Further, it is usually not possible for the users to keep all the transformation steps in mind. Therefore user needs the manual to track the steps of design through the examples. These disadvantages can be overcome using Macro recording feature of Filpro. After forming a design procedure for a certain filter class, it is not necessary to pass through all of these tedious transformation steps. The circuit transformations that are carried out in a filter design procedure can be learned by the program and can be repeated in subsequent trials (Figure 1.174 ).  This tool is extremely handy for the design of filters which require too many circuit transformation steps and/or for designs requiring many iterative trials. Further, this tool enables the users to create their own One Push Button Softwares.

Macro feature can be enabled after Quit Design command. That is, it works only on circuit transformations stages, not in Design stage. It can be used as follows:

- Right after Quit Design command or during any circuit transformation stage, the command Macro / Start Recording is invoked. Then Filpro starts to record all the transformation steps. After finishing the transformations, Macro / Stop Recording command is invoked.

- Then the macro can be saved using Save Macro command (for example with extension  .Mac).

- As long as a new Macro recording is not attempted, the recorded macro stays operative. Whenever it is needed during a new design trial, the command Macro / Run Macro is applied. Macro performs all the circuit transformations automatically, stopping at the steps where input data is needed from the user.

- If the computer is shut down, or if several macro recordings are done, then the command Macro / Load Macro / ..  is applied to load a desired macro.

- New macro steps can be added to an already existing Macro program using Append Macro command.

- If a mistake is done in some circuit transformations steps, then Macro / Clear Macro command is applied to clear the recorded macro and then Macro / Start Macro command is applied to start a new macro recording stage. {Care is needed in macro recording. If possible, try to avoid too much UNDO!. If macro doesn’t work as expected, it is better to repeat all the transformation steps carefully by cearing the previous recordings}.

- Once a neat Macro is saved for a class of filter (for example Combline filters of degree 6, 8, 10, 16, etc.), then after Quit Design step, just load the proper macro and run.

Figure 1.174. Macro command.

1. 11. ANALYSIS

There are several analysis tools in addition to response plotting features. These can be summarized as shown below.

1. 11. 1. Tune Multiplexer

This command is used to display and tune insertion losses of all the channels of a multiplexer simultaneously. Return loss of any multiplexer can be plotted using the conventional the menu Plot / Return Loss. However, if the multiplexer is a complex one like manifold, cascaded LP-HP or parallel BP, it is either impossible or cumbersome to plot insertion losses, phases and delays of some channels appearing as external one ports. It is also cumbersome to tune element values of such channels. Tune Multiplexer command is developed to plot insertion losses, phases or group delays of all channels simultaneously on the same graph. This command enables the user to tune elements of all channels while observing these responses. An example is shown in Figure 1.175 where a four channel multiplexer is formed. Three channels are shown as External 1-Port circuits connected to the fourth channel. Channels are separated by transmission line pieces. If an element of the displayed channel is to be tuned, then cursor should be placed under this element and command Tune Multiplexer is applied. After tuning the element, the key F2 is pressed to save the new element value. If an element of a channel appearing as External 1-Port is to be tuned, then the cursor should be placed under this External 1-Port and when the command Tune Multiplexer is applied, the circuit of this channel will be displayed automatically. The element to be tuned is pointed by the cursor and tuning increment is typed in. After tuning of an element is completed, the new element value can’t be saved automatically. The key F2 doesn’t work to save the tuned elements of External 1-Ports. After tuning is finished with all elements, the original circuit file of this channel should be loaded and the new values of the tuned elements should be changed manually (using Change command). The execution steps of tuning an element of Channel-1 of the multiplexer are shown in Figure 1.175.

Figure 1.175. Tune Multiplexer command.

Figure 1.175. Tune Multiplexer command (continued).

1. 11. 2. Element Sensitivity

One parameter of a pointed element is varied automatically in equal five steps between two limits set by the user and responses are displayed in overplot mode (Figure 1.176).

Figure 1.176. Sensitivity Analysis command applied on the first inductor displaying the return loss response.

1. 11. 3. Tune Element

One parameter of an element pointed by cursor can be increased (up arrow) or decreased (down arrow) by incremental steps typed in by the user while observing any one of the responses (Insertion Loss, Return Loss, Phase, Delay, Smith Chart). Figure 1.177 shows application of this command on an inductor of a circuit. This command was found to be very useful for the following applications:

- Assessing sensitivity of response(s) to variations in element values.

- Narrowband impedance matching circuit design by systematically varying element values (for circuits with small number of elements).

- Finding the (missing ) value of an element in filters.

- Manual optimization of responses within limited ranges with respect to some selected elements.

- Design and optimization of multiplexers by manual tuning.

Figure 1.177. Tune Element command on an inductor of a circuit.

1. 11. 4. Standard Values:

This command replaces lumped element capacitors and inductors by the standard available values saved in the library of Filpro (Figure 1.178). This is useful to inspect deformations in response when actual practical values are used.

Figure 1.178. Standard Values command.

1. 11. 5. Monte-Carlo

Parameters of selected elements of a circuit are randomized within a user-set percentage (default: 5%) around the original values with either Gaussian or Uniform probability density and responses are plotted in overplot mode. This analysis can be repeated manually or automatically as many times as user likes.

- When Monte Carlo command is chosen, the menu shown in Figure 1.179 appears. The first step in using this command would be to set number of automatic iterations through the command “Preferences /Auto Plot At Every Iteration/ Number of Iterations:..”.

- Then the parameters of elements to be placed into Monte Carlo List are set by placing the cursor under each element and applying the command “Add a Parameter / Gaussian or Uniform Distribution / .. %”. All parameters of all elements may be placed into the list, if the circuit is not too large.

- For quick search purposes (when the command Add A Parameter is tedious to use), the options “Automatic Operations / Add All First Parameters or Add All First Two Parameters” can be used (Figure 1.180).

- The percentage deviations of all elements may also be adjusted by a single command (to the same percentage) using the command “Automatic Operations / Scale All Deviations”.

- Some or all of the Monte Carlo parameters may be deleted through the commands “Delete A parameter” and “Clear All Parameters”. An example run on return loss is shown in Figure 1.181.

- It is natural to place Reference Lines into the plots (See Reference Lines command).

Figure 1.179. Monte Carlo  command and adding a parameter into the  Monte Carlo List.

Figure 1.179. Monte Carlo  command and adding a parameter into the  Monte Carlo List (continued).

Figure 1.180. Inclusion of first parameters of all elements into the  Monte Carlo List.

Figure 1.181.  Monte Carlo Analysis of return loss of a filter with all first parameters included in the list.

1. 11. 6. Worst Case Analysis

Monte Carlo analysis is carried out by a specified number of times and the worst one is selected by comparing each response with the original response. The worst case circuit is displayed. The original circuit can be recovered by using Undo command.

1. 11. 7. Best Case Analysis

This is an adaptive, user guided-directed local optimization program running on Monte Carlo analysis and using Return Loss Reference Lines as criterium.

- Typical application starts by assigning a low return loss reference line.

- A Monte Carlo List is formed by including the elements to be optimized. Inclusion of all elements or all parameters of all elements is not recommended. At initial stages, sensitivity of response to various selected-suspected element groups are inspected by including only these elements into the Monte Carlo List. After getting an insight on the effects of variations of these elements, some may be retained and some are discarded while adding new elements into the Monte Carlo List. This way, the element list may gradually be enlarged.

- Initially a relatively large percentage deviation may be assigned for the Monte Carlo parameters. If the response crosses the boundary formed by the reference line, then this solution is discarded and Monte Carlo analysis continues.

- If a solution is found satisfying the return loss requirements (not crossing the reference line) then iteration stops and Filpro declares that a solution is found (Figure 1.182).

Figure 1.167. Best Case analysis approach for a guide local optimization.

- At this stage, we can use Esc command if we are content with response. The resulting Best Case Circuit is automatically saved, replacing the original circuit.

- If a better response is sought, then reference line is leveled up and Monte Carlo parameter percentage deviations are decreased because a solution is already found and a better solution will possibly be found about these values. Iterations are restarted automatically and continue until the next circuit satisfying the new requirements is found. This way, the return loss response is improved until no more improvement is obtained.

- In usual applications, this approach founds the optimum quickly, if the optimum is near. However, by assigning large percentage deviations in Monte Carlo parameters, this procedure may also work as a Random Optimizer to find the global optimum (in a longer time interval, naturally). This procedure works efficiently in impedance matching and filtering-multiplexing circuits. The present version of Filpro works as best case analyzer  on Return Loss only.

1. 11. 8. Yield Analysis

Yield Analysis is carried out as Monte Carlo analysis in the presence of reference lines to compare response. If the response curve for a set of randomized element values exceeds these limits, then this trial is assumed to be “failed”. Otherwise success is noted. When Yield analysis is started, first the frequency region in which analysis will be carried out has to be typed in together with number of iterations. The results of iterations are presented as a list as described in Figure 1.183. At the end of iterations all elements assume their original values.

Figure 1.183. Yield Analysis command and presentation of results.

1. 12. Options

There are several optional features:

1. 12. 1. Directories:

Here, the Data Directory, Standard Capacitance file, Standard Inductance file and Realizable Element Value Limits file are displayed (Figure 1.184). Capacitance and Inductance files contain the standard available element values. They can be updated whenever necessary. Realizable limits for element parameters are also saved as a file. When the circuit element values come out to be outside the range defined by this file, that element may be colored (See Preferences: Paint Unrealizable Elements).

Figure 1.184. Options / Directories  menu.

1.12. 2.Change Units:

This command is basically used to change units of inductors and capacitors as shown in Figure 1.185.

- Unit of frequency is MHz and it can’t be changed.

- Unit of length is mm and it can’t be changed.

- Unit of resistance is Ohms and it can’t be changed.

- Unit of Attenuation over lossy transmission lines and stubs must be specified as dB/meter.

Figure 1.185. Units of capacitance, inductance, resistance, frequency, length and attenuation.

1. 12. 3. Preferences:

The optional preferences list is shown in Figure 1.186. Preferences are self explanatory. A preference may be checked by pressing Space Bar.

Figure 1.186. Preferences list.

1. 13. HELP menu

This menu contains the items described in Figure 1.187.

Figure 1.187. Help menu.

1. 13. 1. Available Elements:

Displays the schematics of elements available from the library of Filpro.

1. 13. 2. Calculators:

This menu item has a number of useful features. It has two calculators for simple arithmetic operations, and also  it directs the preaasigned key groups for displaying coupling coefficients, External Q, resonance frequencies of resonators, Node voltage-Current and Power variables. It also shows the keys assigned for calculation of loss resistors of inductors and capacitors due to their finite Q.

1. 13. 2. 1. One and Two Operand Calculators:

A handy calculator is frequently needed for simple calculations in most filter design applications. One and two operand calculators of Filpro satisfies most of such needs (Figure 1.188).

Figure 1.188. One and Two Operand Calculators command.

1. 13. 2. 2. Loss resistor calculations for inductors and capacitors:

Series resistance of inductors and parallel resistance of capacitors due to their finite Q may be calculated using these options (Figure 1.189). Element value, frequency and Q of the element should be entered by the user.

Figure 1.189.  Calculation of resistors of lumped inductors and capacitors due to finite Q.

1. 13. 2. 3. Coupling Coefficients:

Filpro can calculate and display Coupling Coefficients of inductively, capacitively and inverter coupled shunt LC or shunt OC+SC Stub or shunt SC Stub+Capacitor type resonators by placing the cursor under the coupling element and pressing the key combination Alt+F6 (Figure 1.190). In Cascaded Triplet (CT),  Cascaded Quadruplet (CQ) and Generalized Cascaded Quadruplet (GCQ) filters the cursor is placed under the CT or CQ elements. Calculation of coupling coefficients require three frequencies: Passband center frequency and two frequencies on the two sides of the passband center. These two frequencies can usually be selected as the passband edge frequencies for narrow-medium bandwidth filters. Resonance frequencies of coupled resonators are also displayed besides the coupling coefficients. In direct coupled resonator filters, all resonator frequencies are identical while in CT or CQ filters resonance frequencies may differ.

Figure 1.190. Coupling Coefficient command.

- Coupling coefficients of Direct Coupled resonators are displayed by by placing the cursor under the coupling (middle) element and pressing the key combination Alt+F6. Figure 1.191.a shows an all-stub resonator inverter coupled bandpass filter with passband edges fp1=780 MHz and fp2=820 MHz.. Sign of one of the inner inverters is changed to negative, just for demonstration purposes. With cursor under the positive coupling inverter, pressing the keys Alt+F6 displays inductive coupling coefficient, simply because the inverter is positive. The same command when applied to the negative coupling inverter displays capacitive (negative) coupling coefficient. That is, it is inherently assumed that the positive inverters will be replaced by their inductive equivalents while the negative ones will be replaced by capacitive equivalents, which are not necessarily correct.

Figure 1.191. Displaying coupling coefficients of some direct coupled resonator filter types.

- Figure 1.191.b show the coupling coefficients when the OC+SC Stub resonators are replaced by their SC Stub+Lumped Capacitor equivalents.

- Figure 1.191.c shows coupling coefficients when the resonators are replaced by their lumped LC equivalents.

- Figure 1.191.d shows coupling coefficients after the inverters are replaced by their inductive and capacitive Pi-section equivalents and simplified by combining the negative shunt elements of the Pi-sections with adjacent positive elements of the resonators.

- Coupling coefficients of Cascaded Triplets (CT) elements are displayed by placing the cursor under these elements and pressing the keys Alt+F6. Figure 1.192.a shows a filter with All Stub CT sections (with SC+OC Stub resonators). Figure 1.192.b shows the same filter with Mixed Element CT sections (with resonators of the form SC Stub+Lumped Capacitor). Figure 1.192.c shows the same filter with CT sections in the form of Lumped Element Bridge Tees. Finally, Figure 1.192.d shows the same filter in the special CT forms of Filpro, namely C/LL, L/LL types). In all these filters coupling coefficients of the CT sections are displayed by placing the cursor under these elements and pressing Alt+F6. The resonance frequencies of the associated three shunt resonators are also displayed together with coupling coefficients. In Figure 1.192.a, the coupling coefficients between direct coupled resonators are also shown. Note the differences between resonance frequencies of various resonators.

Figure 1.192. Displaying coupling coefficients of various Cascaded Triplet (CT) section types.

- Coupling coefficients of Cascaded Quadruplet (CQ) and Generalized Cascaded Quadruplet (GCQ) elements are also displayed by placing the cursor under these elements and pressing Alt+F6, as shown in Figure 1.193 for various forms of CQ and GCQ elements. Figures 1.193.a shows a filter with asymmetrically placed finite transmission zero pair having GCQ type elements with ideal inverters. Figure 1.193.b shows the same filter where now the ideal inverters of GCQ elements are replaced by their capacitive and inductive Pi-equivalents.

Figure 1.193. Displaying coupling coefficients and resonance frequencies of resonators of  GCQ and CQ type quadruplets.

- Figure 1.193.c shows a filter with a symmetrically placed finite transmission zero pair, having CQ type elements with ideal inverters. Figure 1.193.d shows the same filter where the ideal inverters of CQ element are replaced by their capacitive and inductive Pi-equivalents. Resonance frequencies of the four resonators are also displayed.

- Cross-Coupling Coefficients of Symmetric Cross-Coupled filters can be displayed from the odd halves of these circuits. See Chapter-15 for details.

# 1. 13. 3. 4. External Q:

Power losses in resistive terminations RS and RL of the filters can be associated with the resonators closest to source and load, thus decreasing their quality factor Q. This Q is called as External Q (or loaded Q).  For shunt resonators directly attached to RS and RL, these resistances are taken into account as being in parallel with resonators. For series resonators adjacent to RS and RL, these resistors are taken into account as being in series with resonators. If the resonators are coupled to RS and RL through some elements, like a coupling capacitor, coupling inductor, coupling inverter, transmission line, etc. then RS and RL will be transformed by these coupling elements, and  the effective resistance seen by the resonators will differ from the actual RS and RL. In Filpro, External Q of filters can be calculated and displayed by pressing the keys Alt+F7 (Figure 1.194). Filpro asks the passband center and two frequencies about passband center which are usually specified as passband edge frequencies for narrowband filters. These frequencies are used in calculation of suscepteance slope parameters needed in finding External Q. Cursor can be anywhere in the circuit. Some examples are presented in the following Figures:

Figure 1.194. External Q command.

- Figure 1.195 shows circuits where shunt lumped LC, SC+OC Stub and SC Stub+Lumped Capacitor type resonators are adjacent to RS and RL. All three filters are different versions of the same bandpass filter with passband edges fp1=780 MHz, fp2=820 MHz. Therefore their External Q comes out to be identical or very close to each other. Alt+F7 command is also valid for the duals of these circuits where the resonators become series type (series LC in series arm, Series OC+SC Stub in series arm, etc.).

Figure 1.195. Displaying External Q of resonators when resonators are adjacent to RS and RL.

- Figure 1.196 shows the same filters of the previous figure, now impedance scaled by inserting inverters at source and load ends. Here, the resonators are coupled to RS and RL through ideal inverters. Note that External Q is still the same as in the previous figure. Alt+F7 command applies to the Dual circuits where now the resonators are of series type.

Figure 1.196. Displaying External Q when the end resonators are coupled to source and load

through ideal inverters.

- Figure 1.197 shows the same circuits when the ideal inverters are replaced by various L-section type equivalents leading to single series coupling elements.  Note that very small differences exist between different circuits. The same (Alt+F7) command applies to series type resonators coupled to RS and RL through single shunt coupling elements.

Figure 1.197. Displaying  External Q when the end resonators are coupled to RS and RL through single series elements.

- Figure 1.198 shows the filter of Figure 1.196.a with the ideal inverters replaced by transmission line pieces which are quarter wavelength long at passband center. The displayed External Q is slightly higher compared to the ideal inverter case, because the quarter wavelength line is a poor approximation for ideal inverter.

Figure 1.198. Displaying External Q of a filter which is coupled to RS and RL through quarter

wavelength line pieces.

1. 13. 3. 5. List of Coupling Coefficients and External Q

Coupling coefficients and External Q of a coupled resonator filter can also be displayed as a single list by pressing the keys Ctrl+F6. Figure 1.199 shows an example on a filter with passband edges fp1=780 MHz, fp2=820 MHz, having a pair of unsymmetrically placed finite transmission zeros leading to a Generalized Quadruplet  (GCQ) element and another finite transmission zero leading to a Triplet (CT) element. Filpro asks the passband center and two frequencies on the two sides of passband center. Also the type of circuit is asked to differentiate whether the circuit is of Cascaded Element type or Even-Odd halves of a symmetric cross-coupled filter. If the filter is not an even or odd half of a symmetric cross-coupled filter, then the option Cascaded should be selected. Then Filpro displays the source and load end External Q’s, the coupling coefficients between resonators numbered as kij where i and j are node numbers of resonators counted from left end, on the right of RS.

Figure 1.199. List of Coupling Coefficients and External Q’s.

1. 13. 3. 6. Displaying Node Voltages, Currents and Powers

Voltage, current and real power at the nodes at a specified frequency can be displayed as a list by pressing the keys Alt+F8. These quantities can be displayed either in Mangitude-Angle or Real Imaginary formats. Figure 1.200 shows these quantities for the filter of Figure 1.199. Source voltage and frequency should be specified by user.

Figure 1.200. Displaying node voltage, current and real powers.

1. 13. 4. Information:

Program Information: This item presents some information about the software.

Standard Element Values: This item lists standard inductor and capacitor values and realizability limits of some variables used in Filpro.

Figure 1.201. Information on software and standard element values.

Call:

The author Prof. Dr. Nevzat Yildirim of EE Dept. of Middle East Technical University, Ankara, Turkey can be reached at the E-mail adress:

nyil@metu.edu.tr

Figure 1.202. Contacting the author.

Further info can be obtained from the WEB page:

http://www.eee.metu.edu.tr/~nyil

where this manual is also available together with the student version of Filpro (DEMO2.zip).

The manual and student version are also available in the following web sites:

http://rf.rfglobalnet.com/software_modeling/categories/9.htm

and

http://www.wirelessdesignonline.com/content/downloads/Home.asp

REFERENCES AND FURTHER READING

[ 1] E. L. Norton, “Constant Resistance Networks with Applications to Filter Groups”, Bell System Tech. J. vol. 16, pp.178-193,  Apr. 1937.

[ 2 ] J. E. Colin, “De Quelques transformations de quadripoles”, Cables et Transmission, Vol. 44, pp. 314-344, Oct. 1956.

[ 3] R. Saal, and E. Ulbrich, “On the design of filters by Synthesis”, IRE Trans. Circuit Theory,vol.CT-5, no. 4, pp. 284-327, 1958.

[ 4]  P. R. Geffe, “Simplified Modern Filter Design”, Hayden Book Co., NY, 1963.

[ 5] G. L. Matthaei, L. Young, E. M. Jones, “Microwave Filters, Impedance Matching Networks and Coupling Structures”, Mc Graw Hill , NY, 1964.

[ 6]  G. L. Matthaei, “ Tables of impedance transforming networks of lowpass filter form”, Proc. IEEE, vol. 52, pp. 939 - 963, Aug. 1964.

[ 7] G. Szentirmai, "Band-pass matching filter in the form of polynomial low-pass filter", IEEE Trans. On Circuit Theory ,  vol. CT-11,  pp. 177-178, March 1964.

[ 8]  R. Levy, “A General Equivalent Circuit Transformation for Distributed Networks”, IEEE Trans. CT, Vol. CT-12, No 3,  pp. 457-458, Sept. 1965.

[ 9]  A. Zverev, “A Handbook on Filter Synthesis”,  John Wiley and Sons, Inc., NY 1967

[10] V. Belevitch , “Classical Network Theory”, San Francisco, Calif.Holden-Day, 1968.

[11] H. J. Orchard and G. C. Temes, “Filter Design Using Transformed Variables”, IEEE Trans CT, vol. CT -15, No.5, pp. 385-408, Dec. 1968.

[12] M. C. Horton, “Synthesis of Multisection Networks using the W- plane”, 1970 IEEE Int. Microwave Symposium Digest, pp.76 - 84, 1970.

[13] D. S. Humpherys, “The Analysis, Design and Synthesis of Electrical Filters”, Prentice Hall, Inc., 1970.

[14] G. Reiter and G. Hammer, “ Stripline Filters with Coupled / 8 Line Sections”, 1973 European Microwave Conference, B.3.2, Brussels, 1973.

[15] G. Hoffmann, “ The Direct Synthesis of Optimum Commensurate Distributed Filters by Digital Computer”, 1973 European Microwave Conference Proc., B.2.2, Brussels, 1973.

[16]  G. C. Temes and S. K. Mitra, “Modern Filter Theory and Design”, J. Wiley and Sons, NY, 1973.

[17] R. Levy, “A Generalized Design Technique for Practical Distributed Reciprocal Ladder Networks”, IEEE Trans. MTT, vol. MTT-21, No.8 , pp. 519- 526, Aug. 1973.

[18] U. H. Gysel, “New Theory and Design for Hairpin Line Filters”, IEEE Trans MTT, vol. MTT-22, No. 5, pp. 523-531, May 1974.

[19]  R. W.  Daniels, “Approximation Methods for Electronic Filter Design”, Mc Graw Hill, NY, 1974.

[20]    V. K. Tripathy, “Asymmetric Coupled Transmission Lines in an Inhomogeneous medium”, IEEE Trans. MTT, Vol. MTT- 23, No. 9,  pp. 734 -739, Sept. 1975.

[21]  W. K.  Chen , “Theory and Design of Broadband Matching Networks”,  Pergamon Press, 1976.

[22]  J. D.  Rhodes , “Theory of Electrical Filters”, J. Wiley and Sons, NY, 1976.

[23]  G. C. Temes and J. W. La Patra, “Introduction to Circuit Synthesis and Design”, Mc Graw Hill, NY, 1977.

[24] M. C. Horton, “ TEM Bandpass Filters Having Eight Wavelength Coupled Stubs”, 1977 IEEE,   MTT-S Int. Microwave Symposium Digest, San Diego , 1977.

[25] V. K. Tripathy, “Equivalent Circuits and Characteristics of Inhomogeneous Nonsymmetrical Coupled Line Two-Port Circuits”, IEEE Trans. MTT,  pp. 140 -142,  Feb. 1977 .

[26]  R. Saal, “Handbook of Filter Design”,  AEG Telefunken,1979.

[27]  D. R. J. White, “A Handbook on Electrical Filters”, Don White Consultants Inc., 1980.

[28]  J. D. Rhodes and S. A. Alseyab, “The Generalized Chebyshev Low Pass Prototype   Filters”, Int. J. Circuit Theory and Applications,Vol. 8, pp. 113 - 125, 1980.

[29] J. A. G. Malherbe, “Microwave Transmission Line Filters”, Artech House, 1980.

[30]  S. A. Alseyab, “A Novel Class of Generalized Chebyshev Low Pass Prototype for Suspended Substrate Stripline Filters”, IEEE Trans. Microwave  Theory and Tech. MTT-30, No. 9, pp. 1341-1347, Sept. 1982.

[31] J. B. Minnis, “Classes of Sub Miniature Microwave Printed Circuit Filters with Arbitrary Passband and Stopband Widths”, IEEE Trans. MTT, Vol. MTT-30,  pp. 1893 - 1900,  Nov. 1982.

[ 32]  P. La Tourette, “High Pass Filter Design”, 12. European Microwave Conference Proc., pp. 233-238, England, 1982.

[33]  J. Ness  and S. Johnson, “Narrowband Elliptic Filters on Microstrip”, Microwaves and RF,Nov. 1984.

[34]  M. Hasler and J. Neirynck , “Electric Filters”. Artech House Inc. Mass.1986.

[35] Y. Sen, “Direct Design of  Lumped and Distributed Filters Using Transformed Variable Techniques”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara,Turkey, 1989.

[36] C. M. Tsai and K. C. Gupta, “A Generalized Model for Coupled Lines and its applications to Two-Layer Planar Circuits”, IEEE Trans. MTT, Vol. MTT- 40,  pp. 2190 - 2199, Dec. 1992.

[37] U. Koyaz, “Synthesis of Lumped and Distributed Filters and Diplexers”, Ph. D. Thesis, EEE Dept. Middle East Technical University, Ankara,Turkey, 1994.

[38]  J. C. Tippet and M. J. Shian, “Circuit Transformations for realization of a Class of  Miniature  Printed Circuits”, IEEE,  MTT-S  Digest, pp. 621- 624, 1994.

[39]  A. J. Klappenberger, “Computer Design of Equal Shunt Value Tubular Bandpass Filters”, RF Design, pp. 114 -120, March 1994.