1. 6. CIRCUIT TRANSFORMATIONS-1

The menu blocks Transform-1, Transform-2 and Transform-3 contain a large number of equivalent circuit transformations, some exact and some approximate. Most of these transformations are needed in converting filters obtained at the end of Design stage into desired topologies with realizable element values. Most of them are well-known transformations while some of them are original. Many of these transformations have rather long, complicated derivations. Derivations are not given in this manual to save space. However they could be derived by the interested users just by equating the ABCD matrices of the original and transformed versions of the circuit sections.

1. 6. 1. Parasitics

This command mainly applies to lumped LC elements, open and short circuited stubs and transmission lines. There are four options in this command: Lossless-to-Lossy,  Lossy-to-Lossless and All Lossy Resonators (Figure 1.50 ). Lossless-to-Lossy command converts ideal lossless inductor, capacitor, second or fourth order LC sections pointed by the cursor into their lossy (practical) equivalents. Filpro asks Q of elements, frequency of approximation, lead inductances of capacitors (Ld) and winding capacitance of inductors (Cw).

- A lossy inductor has a series resistance due to finite Q of inductor (RQL) and a lumped capacitor in parallel with inductor to account interwinding capacitance.

- A lossy capacitor has a parallel resitance to account for the finite Q of capacitors (RQC) a series lead inductance and lead resistance.

- A shunt parallel LC resonator can be splitted into its L and C components and these elements can then be converted into their lossy models by typing the Q of each element. However in practice there are many resonators which are modeled as LC resonators, but actually they are not simple parallel combinations of an inductor and a capacitor. Some resonators may be in an OC or SC stub form or combinations of  OC and SC stubs. Some may be in the form of stubs in parallel with lumped capacitors while some may be in cavity resonator form. In Filpro such resonators can be converted into their LC equivalent models. Then the command Lossless-to-Lossy can be used to convert a shunt LC resonator into shunt LCR resonator  where R is the loss resistance found from the user specified Q of the resonator. Since Q is specified at a single frequency, the resulting LCR model is valid only for narrow-medium bandwidth filters.

- Lossy-to-Lossless command converts lossy element back to lossless form.

- All Lossy Resonators command is used as a short cut to convert all shunt LC resonators of the circuit into LCR form with a single command as shown in Figure 1.50.e. In this command all resonators will have the same Q.

- Lossless-to-Lossy command converts also a lossless transmission line or stub into lossy line or stub with a user specified attenuation per meter.

- If the command Lossless-to-Lossy is applied onto a CT or CQ element, the resistors of shunt resonators are calculated in accordance with the user specified Q. Generalized CQ elements, All-Stub Bridge Tee and Mixed Bridge Tee elements do not have lossy versions (too many parameters for Filpro!).

Figure 1.50. Application of  Parasitics / Lossy Element command on L, C and LC elements.

Figure 1.50. Application of Parasitics / Lossy Element command on L, C and LC elements (continued).

Figure 1.51. Application of Losless to Lossy command on stubs.

1. 6. 2. Impedance Matching

This command contains many elementary narrowband lumped and distributed element impedance matching techniques (Figure 1.52). A given real source impedance RS can be matched to a real load RL or a complex source impedance RS+jXS can be matched to a complex load RL+jXL at a single frequency (See Chapter 18 for applications).

Figure 1.52. Impedance Matching command.

a) R-Lsr-to-R-Lsh:

This command splits an inductance in series with RS or RL and converts it into an LRight or LLeft  section to produce a transformer, as described in Figure 1.53 at the source end. The mirror image of this transformation can be performed at the source end. The resulting transformer turns ratio is set by the user within the range specified at the bottom of the screen. This transformer is used to scale up or down the element values of the rest of the circuit by shifting it to right. Although this is an approximate transformation which holds exactly at a single frequency, it can be used for impedance level adjustment of filters with narrow and medium bandwidths. If the transformer at the load end is specified as the inverse of the transformer produced at the source end, then the two transformers can be made to cancel.

Figure 1.53. R-Lsr-to-R-Lsh command.

b ) R-Lsh-to R-Lsr:

This is the reverse of the previous command, applied on shunt inductor adjacent to RS or RL to convert it into inductive L section together with a transformer. The transformer is used to scale elements of the filter or the terminations.

c ) R-Csr-to-R-Csh and  RCsh-to-RCsr:

The transformation on inductors increase the number of inductors which is undesirable usually. Therefore in most cases the R-Csr-to-R-Csh or R-Csh-to-R-Csr are used for such purposes. The transformation R-Csr-to-R-Csh is described in Figure 1.54 while R-Csh-to-R-Csr is described in Figure 1.55.

Figure 1.54. R-Csr-to-RCsh transformation at source and  load ends of a bandpass filter.

Figure 1.55. The transformation R-Csh-to-R-Csr at source end.

In narrowband bandpass filters, series arm inductors come out to be too large while series arm capacitors are too small. The reverse is true for shunt arm elements. The transformers created in these transformations can be used for such adjustments. This transformation eases adjustments of the inner element values as well through Pi – Tee-- L transformations that can be applied on the series and shunt arm capacitors of the transformed circuit sections of Figure 1.55. Although the transformations are approximate, degradation in response is almost negligible for for narrow bandwidth filters and may be acceptable for medium and wide bandwidth filters.  Some applications are given in Chapter 6.

d) Match Real RS to RL:

This command can be used to match an RS-RL pair formed for example using the command Design / By Inserting Elements where RS and RL can be set as desired. With the cursor under RL, applying command Impedance Matching / Match Real RS-to-RL displays several matching circuit options shown in Figure 1.56 which are described below:

Figure 1.56. Match Real RS to RL command.

Figure 1.56. Match Real RS to RL command.

User may select L, Pi or TEE type matching sections. Each type offers several options as described in the figure. The resulting circuits are displayed in constant reactance forms. The constant reactances may then be replaced various equivalent L, C or LC sections to produce more alternatives using Constant Reactances / Replace Constant Reactance By / ... command. Resistive matching option produces a resistive L section.

e ) Match Complex ZS to ZL:

This command offers various circuits to match complex source and load impedances at a single frequency. Solvabilty of problems depend on values of RS and RL as well as the reactive parts of ZS and ZL. The circuit types shown in Figure 1.57.a and b can be handled by this command automatically using the command Match Complex ZS to ZL (Figure 1.58). In these circuits reactive parts of ZL and ZS can only be in series or in parallel with RS or RL. The number of reactive elements are not limited. The circuits of Figure 1.51.c and d may be matched using the techniques of Match Real RS-RL command by deleting the reactive parts and fitting an appropriate L type matching section  which contain reactances similar to those of reactive parts of  ZS and ZL. The choices LLeft, LRight, Pi, Tee and Inverter + Constant Reactance have further  options. The user must try several solutions until the best one is found (if exists).

Figure 1.57. Complex source-load pairs that can be handled by Filpro.

Figure 1.58. Application of Match Complex ZS-ZL /LLeft / Lowpass command.

1. 6. 3. Coils

Various transformations between coupled coils, inductive Tee / Lright / LLeft sections, ideal transformers, tapped coils and capacitance-inductance-coil combinations are performed under this command. Command list is shown in Figure 1.59. Transformations are described in Figures 1.60-65.

Figure 1.59. Coupled coil transformations.

In the command list, Lsh means shunt inductance, pLCsr means parallel LC resonator in series arm, sLCsh means series LC resonator in shunt arm, etc. CapCoil means coupled coil with a capacitor. These transformations are useful in lumped element lowpass, highpass and bandpass filters. Some examples are given in Chapters 3, 4 and 6.

Figure 1.60. Conversions between inductive Tee, LRight , LLeft sections and coupled coils.

Figure 1.61. Replacing a coupled coil by a circuit involving ideal transformer .

Figure 1.62.Conversion of circuit sections involving ideal transformer into coupled coils.

Figure 1.63. Conversions between  coupled coils and tapped coils.

Figure 1.64. Conversions between  tapped coils and ideal transformers.

Figure 1.65. Replacing lowpass and highpass sections having finite transmission zero elements by                     coupled coils in series with capacitors (CapCoil).

Figure 1.65. Replacing lowpass and highpass sections having finite transmission zero elements by                     coupled coils in series with capacitors (CapCoil) (continued).

1. 6. 4. Dumbbell Transformation

This transformation is applied on lumped element bandpass filters with a single transmission zero at s=0 in the form of a series capacitor, to get an easy to construct structure called Dumbbell or Tubular filter [35]. Transformation is described in Figure 1.66.a. With cursor under the first shunt capacitor, application of this transformation yields the circuit on the right, having a symmetric Pi-section of capacitors. By repetitive application, the series arm capacitor can be distributed between series inductors of the banpass filter, as components of Pi-sections, as described in Figure 1.66.b and c. For application examples, see Chapter 6.

Figure 1.66. a) Dumbbell transformation. b) A bandpass filter with one transmission zero at  f = 0.

c) Dumbbell (Tubular) realization  after successive application of Dumbbell transformation.

1. 6. 5. Resonator Operations:

This command contain a set of operations related to lumped element parallel LC resonators in shunt arm (pLCsh) or in series arm (pLCsr) and on series resonators in series arm (sLCsr) or in shunt arm (sLCsh). The list of commands are shown in Figure 1.67.

Figure 1. 67. Resonator Operations command list.

a) L or C to LC Resonator:

In some narrowband applications an inductive or capacitive reactance may be simulated by an LC resonator instead of a single inductor or capacitor. This is a nonunique replacement that will effect both passband and stopband of filters. Careful control of some parameters may yield a controllable bandwidth. With this command, a series arm inductor or capacitor may be replaced by a parallel LC in series arm (pLCsr) or a series LC in series arm (sLCsr), and a shunt arm inductor or capacitor can be replaced by a series LC in shunt arm (sLCsh) or by a parallel LC in shunt arm (pLCsh) (Figure 1.68). In these replacements, one may either specify the resulting inductor or capacitor together with frequency of approximation. In cases of replacing an Lsr by sLCsr or a Csh by pLCsh, one may also choose specifying two reactances at two frequencies. They are useful for transforming a lowpass filter into dual bandpass filter (automatically done using Design / Dual Band Matching).

Figure 1.68. Replacing L or C by LC having the same reactance at a desired frequency.

b) pLCsh-to- Quad:

This command replaces a second order parallel LC resonator in shunt arm with a fourth order LC resonator by equating their ABCD parameters at two frequencies (usually passband edge frequencies of bandpass filters) (Figure 1.69.a). The designer can also specify one or two frequencies where the fourth order section will have zero impedance (SC in shunt arm) to produce finite transmission zeros on one or two sides of the passband of filters.

c) pLCsh-to-Half Wave TLine:

This command replaces a parallel LC resonator in shunt arm with transmission line which is half wavelength long at the frequency of resonance (Figure 1.69.b).

d ) pLCsh-to-Inverter+TLine +Inverter:

The parallel LC resonator in shunt arm is replaced by a triplet formed by two inverters surrounding a transmission line segment (Figure 1.69.c).

e) pLCsh-to-4 Step TLine:

This command replaces a parallel LC resonator in shunt arm surrounded by two ideal inverters by a four step transmission line (Figure 1.69.d) in between two ideal inverters. The middle two steps of line have identical impedances and lengths. Similarly, the two outer steps have identical impedances and lengths.

f ) pLCsh-to- SC Stub:

A shunt arm parallel LC resonator is replaced by a SC Stub which is quarter wavelength long at the frequency of LC resonance (Figure 1.69.e).

g) pLCsr-to- SC Stub:

A series arm parallel LC resonator is replaced by a SC Stub which is quarter wavelength long at the frequency of LC resonance (Figure 1.69.f).

h) sLCsh-to- OC Stub:

A shunt arm series LC resonator is replaced by an OC Stub which is quarter wavelength long at the frequency of LC resonance (Figure 1.69.g).

i ) sLCsr-to- OC Stub:

A series arm series LC resonator is replaced by an OC Stub which is quarter wavelength long at the frequency of LC resonance (Figure 1.69.h).

Figure 1.69. Replacement of lumped LC resonators by various equivalent circuits.

k ) Dummy Resonators:

In some cases it may be useful or advantageous in some way to treat a single inductor as a parallel LC resonator with zero capacitance or to treat a single capacitor as a parallel LC resonator with infinite inductance. These resonators are termed as Dummy Resonators. The command “L or C to Dummy LC” converts single L or C into dummy resonator while the command “Dummy LC to L or C” does the reverse (Figure 1.70 ). In the case of the options involving “All to Dummy LC” and “All Dummy LC to L or C” the commands are applied all the elements of a given circuit. This command is especially useful for application of some Pi-Tee-L transformations which are developed for LC elements on the Pi-Tee-L sections made up of single elements or mixed LC and single elements.

Figure 1.70. Dummy resonator commands.

1. 6. 6. CQ-CT Operations

The commands under this menu item are used for conversion of  proper circuit sections into Cascaded Quadruplet (CQ) and Cascaded Triplet (CT) elements and modification of CQ and CT elements. See Chapter-24, 25 and 26 for the details of these commands.

1. 6. 7. Shift Transformer

In some circuit transformations too many transformers result. These transformers can be shifted to any position in the circuit by placing the cursor under the transformer and entering this command. It is also possible to absorb transformers in load or source resistance. Options are as follows (Figure 1.71):

Shift Transformer / Rightmost: Shifts transformer to the rightmost position, just on the left of the load resistance. Short cut key: Ctrl+Q

Shift Transformer / n-Right: Shifts transformer to the right by a user specified number of elements.

Shift Transformer / Leftmost: Shifts transformer to the leftmost position, adjacent to the source resistance. Short cut key: Ctrl+A

Shift Transformer / n-Left: Shifts transformer to the left by user specified number of elements.

Absorb Transformer in RS: Changes RS to n2.RS  and deletes transformer.

Absorb Transformer in RL:  Changes RL to RL / n2   and deletes transformer.

All Rightmost: Shifts all the transformers in the circuit to rightmost position. Short cut key: Ctrl+W. One may also use the command  Simplify Circuit command with short cut key:F6.

Figure 1.71. Shift transformer command and options.

1. 7. CIRCUIT TRANSFORMATIONS-2

The transformations collected under this menu are mostly applicable only to distributed element filters.

1. 7. 1. Kuroda and Kuroda-Levy Transformations

In Kuroda Transformation (Figure 1.72), unit element (UE-transmission line of length commensurate with lengths of stubs of the filter) followed or preceded by a series or shunt arm OC or SC stub can be converted to an equivalent form in which part or the whole of the stub is transferred to the other side of the UE (Figure 1.73). In this transformation, cursor is always placed under the UE. When the command is applied, the choices Right and Left appear. If a stub on right of unit element is to be transferred to left then Right option will be chosen, and vice versa. Then the following options are displayed:

- Full Stub Transfer: Transfers the whole stub on one side of the unit element to the other side.

- Specify UE impedance:  Part of the stub will be transferred to the other side such that after transformation, the UE will have  the impedance  specified by the user.

- Specify Stub Impedance: Part of the stub will be transferred to the other side such that, after transfer, the remaining stub (not the transferred stub!) will have the impedance  specified by the user.

- Equal Stub Impedances: Transfers part of the stub such that in the resultant circuit the stubs on the two sides of element  will have equal impedances. Applicable to series OC and shunt SC stubs only (Figure 1.73.c, d).

- Specify Transformer:

Transfers part of the stub such that the resultant circuit will have the specified transformer. Applicable to series OC and shunt SC stubs only (Figure 1.73.c, d).

Figure 1.72. Kuroda / Kuroda Levy commands.

Figure 1.73. Basic Kuroda identities.

Figure 1.73. Basic Kuroda identities (continued).

A UE followed or preceded by a shunt arm series stub resonator or series arm parallel stub resonator can be converted to the forms shown in Figure 1.74.a and b respectively. This is called Kuroda-Levy Transformation [30] and it involves a negative element which has to be absorbed by the neighboring positive element of the same kind.

Figure 1.74. Basic Kuroda-Levy identities.

1. 7. 2. Multiple Kuroda Transformations-1: Kuroda Pi  command.

These transformations are applicable if a UE is followed or preceded by a series stub resonator in series arm or parallel stub resonator in shunt arm (Figure 1.75). In the case of series resonators, the SC stub is split into two. Then, using Kuroda transformation, the first part of SC stub is transferred to left side of the unit element. Then the OC stub and finally the second part of SC stub are transferred to the left, giving the Pi-section of OC stubs on left. In case of shunt arm parallel resonator, the SC stub is splitted and the three pieces are transferred to left in the proper order to form, now, a Pi-section of SC stubs. In applying this command, we have the following options:

Symmetric Pi-section:	Yields a symmetric Pi-section on the other side of the unit element.

Specify transformer :	Yields the specified value for the transformer at the expense of 
                                                                producing an asymmetric Pi section.

The open and short circuited Pi sections are then converted to open or short circuited comb type parallel coupled lines for realization. These transformations are among the essential tools for realization of distributed element bandpass filters involving unit elements. Some examples are presented in Chapter 9, 12 and 13.

Figure 1.75. Multiple Kuroda transformations to get a Pi-section of stubs (Kuroda-Pi transform).

1. 7. 3. Multiple Kuroda Transformations-2: Kuroda - Minnis C/D  command.

This is a more general version of Kuroda-Pi transformation (Figure 1.76) where the series OC stub is splitted by a specified ratio and the stub resonator (SC + OC stub) is translated to left to form a Pi-section. This transformation is used to distribute the single series OC stub arising in synthesis of Minnis-Class-C/D type bandpass filters between unit elements and to form Pi-sections of OC stubs, as described in the examples of Chapter 12.

Figure 1.76. Kuroda-Minnis-C/D  transformation.

1. 7. 4. Multiple Kuroda Transformations-3: Kuroda - Minnis E/F  command.

The transformation shown in Figure 1.77.a is used to distribute the shunt and series OC stubs of Minnis-Class-E / F type bandpass filters [31] between unit elements to form symmetric Pi sections of OC stubs. The version with SC stubs (Figure 1.77.b) may be used in dual cases. In these transformations, user may specify either impedances of the two unit elements (shown as TL-1 and TL-2 in Figure 1.77.a) or the shunt OC impedances of symmetric Pi section and the impedance of first transmission line (TL-1). Some examples are given in Chapter 12.

Figure 1.77. Kuroda-Minnis-E/F transformations a) with OC stubs, b) with SC stubs.

1. 7. 5. Parallel Coupled Lines (PCL)

This command replaces some commensurate stub and transmission line combinations by their parallel coupled line (PCL) equivalents. PCL sections are four port networks characterized by the parameter set             
 				(Zc1 , Zp1  , Rc  , Rp , ec  , ep , l )

where Zc1 and Zp1 are common and difference (Pi ) mode impedances of one of the two coupled lines, Rc and Rp are termed as voltage transformer coefficients for common and difference (Pi ) mode, ec and ep are effective relative dielectric constants of each mode and  l is the physical length of the lines. If these parameters are known, then the common and difference mode impedances of the second line are found through relations

Zc2 = - Rc Rp Zc1	and    Zp2 = - RcRp Zp1

If ec and ep are not equal, then the medium is said to be inhomogeneous. For such cases common and difference mode electrical lengths are not equal. Therefore two different quarter wavelength frequencies  (fqp and fqp  ) exists, leading to two different transformed (Richards’ ) frequencies, c and p , complicating design problems (See Chapter 7).

For coupled lines in homogeneous medium, we have ec = ep and common and difference mode electrical lengths become equal. Replace By PCL commands are applicable under the assumption of homogeneous medium. That is, the Stub+Unit Element sections are replaced by coupled lines in homogeneous medium. In practice, wave propagation medium usually have some inhomogenity, leading to distortions in response. Such cases can be simulated in Filpro by replacing ec and ep with actual (measured, calculated or specified) ec and ep. This replacement effects all other parameters also requiring recalculation. Therefore this process is specially presented as an option as Change Dielectric Constant.

In general, Rc and Rp are complicated functions of line parameters [25], [26]. However, for symmetric (identical ) coupled lines (the usual cases) they simplify to

Rc = -Rp = 1

For such cases we have the terminology

Zc1 = Zc2 = Z0e  : Even mode impedance

Zp1 = Zp2 = Z0o : Odd mode impedance

Since there are too many types of coupled line sections, each equivalent to different Stub+TLine sections with more than one elements and since the same Stub+TLine section may have more than one PCL equivalents, care should be observed in applying these transformations at following points:

Cursor must be placed under the proper elements indicated in each figure. Convention is as follows:

- If  Stub+ TLine circuit has three elements, (or the circuit has a schematic symmetry) cursor is   placed under the middle element always.

- In all other cases cursor is placed under TLine.

- After placing the cursor and entering PCL command, the program can’t find the proper PCL equivalent automatically because of nonuniqueness. It must be guided by the user by first choosing the proper class of coupled lines from the dialog box listing the classes and then choosing the proper PCL circuit in the selected class.

- Classification is made as shown in Figure 1.78 to 1.83. In these figures, XL and XR represent left and right terminations which may be

open circuit (oc), short circuit (sc), open circuited stub (OS) , short circuited stub (SS), lumped capacitor (Cap)

Depending on the terminations, each class may have several different coupled line equivalent circuits, named in accordance to their class and terminations as follows:

i) Comb Type parallel coupled lines (Figure 1.78):

These are coupled lines with adjacent ports on one side are used as input and output while the other two ports are terminated in OC, SC, capacitors, stubs or shorted to each other to form hairpin line section. They are named as follows:

scscCOMB, scocCOMB, ocscCOMB, ococCOMB, SSSSCOMB, OSOSCOMB, CapCOMB, Hairpin

The names are (hoped to be) self explanatory. For example scocCOMB has short circuit on left termination and open circuit on right termination. OSOSCOMB has open circuited stubs on both terminations. CapCOMB has capacitive terminations. Hairpin line has terminations shorted to each other. {See Filpro-Insert-Parallel Coupled Lines-Comb Lines for schematics of each PCL.}. Some of these coupled lines can be obtained from their stub+TLine equivalents through Replace By PCL command, as described in Figure 1.78.

Figure 1.78. Replacing  some stub circuits by comb type equivalents.

Figure 1.78. Replacing  some stub circuits by comb type equivalents (continued).

ii) Interdigital Type parallel coupled lines (Figure 1.79):

These are coupled lines with diagonal ports used as input and output while the other diagonal ports are terminated  in OC, SC, open circuited stub (OS), short circuited stub (SS) or capacitance. The following types are programmed into Filpro:

scscINTD, ococINTD, SSSSINTD, OSOSINTD, CapINTD

Only the types scscINTD  and  ococINTD can be obtained through the transformation  Replace By PCL, as described in Figure 1.79.

Figure 1.79. Replacing some Stub+ TLine circuits by interdigital equivalents.

iii) Spur Type parallel coupled lines (Figure 1.80):

In spur type lines, the adjacent ports on one side of coupled lines are shorted to each other and used as input or output. The following kinds are programmed into Filpro:

lfSPUR, rgSPUR, lfscSAITO, lfocSAITO, rgscSAITO, rgocSAITO, lfCapSPUR, rgCapSPUR

Among these, only the ones shown in Figure 1.81 are obtainable through the transformation Replace By PCL.

Figure 1.80. Spur type coupled lines.

Figure 1.81. Replacing some stub+ TLine circuits by spur type parallel coupled lines.

iv) Ikeno Type parallel coupled lines (Figure 1.82):

These are parallel coupled lines with both input and output ports on the same line. The ports of second line are terminated in OC, SC, open circuited stub or short circuited stub. The following types exist in Filpro:

scscIKN,   scocIKN,  ocscIKN,  ococIKN, scSSIKN, scOSIKN, ocSSIKN, ocOSIKN,

SSscIKN, SSocIKN, OSscIKN, OSocIKN, SSSSIKN, SSOSIKN, OSSSIKN, OSOSIKN

In Figure 1.83, the equivalent circuits are shown from which some of the Ikeno type lines can be obtained through the command Replace By PCL.

Figure 1.82. Ikeno Type coupled lines.

Figure 1.83. Replacing some stub+ TLine circuits by Ikeno type parallel coupled lines.

v) Double Combs:

Some all-stub circuits can be replaced by their coupled line equivalents made up of parallel connection of comb-type lines as described in Figure 1.84.

Figure 1.84. Double Comb type parallel coupled lines.

In the library of parallel coupled lines there are also special structures which may not be reached by ReplaceBy PCL command. These are Stepped-Comb, Stepped Interdigital type coupled lines. They are placed into Filpro just for some trials for novel structures.

1. 7. 6. Tapped PCL

This command is used to convert an interdigital line or combline (including CT and CQ types) filter designed by inserting redundant unit elements, into tapped input-output form (Figure 1.85). For detailed descriptions, see Chapter 9 for Tapped Interdigital filters, Chapter 11 for tapped combline filters and Chapters 24, 25 and 26 for tapped CT and CQ filters.

Figure 1. 85. Command for tapping interdigital and combline filters.

1. 7. 7. Replace By Stepped Resonator

This command replaces some all-stub resonators (open and short circuit stub cominations) by their stepped stub equivalents.

Series and parallel connected open and short circuited stubs in shunt arms or in series arms can be converted to two-step resonators (two-step open or short circuited stubs) as described in Figure 1.86.

Figure 1.86. Replacing some stub resonators by two-step resonators.

The option Four Step Resonator applies to fourth order sections, as described in Figure 1.87. An all-stub fourth order section can directly be replaced by either a single Four Step Resonator or by a pair of two-step resonators. It may also be first splitted into two second order all-stub resonators, then the second order resonators can be converted into two-step or  four-step resonators. Four step resonators are used in distributed element bandpass filters to create a pair of finite transmission zeros on S= jaxis. In some bandpass filters, some of the four step impedances may come out to be unrealizable. In such cases one may try the two two-step resonator  equivalents.

Figure 1.87. Replacing some stub resonators by two or four step resonators.

1. 7. 7. Half Angle (p-Transform)  (l / 8 Equivalent)

Commensurate line distributed element filters are formed by stubs and transmission line pieces of equal length or multiples of a common length. In Filpro, all such filters are synthesized as having equal lengths. The frequency at which the common length l  becomes quarter wavelength long (l / 4) is termed as quarter wave frequency, fq and is needed in specifications stages of synthesis procedure. After synthesis is completed, one may change the terminology by halving the common length. That is, the new common length is  lh= l/2  with new quarter wavelength frequency  being doubled:  fqh = 2fq. Since circuit is unchanged, response remains the same. However, with new common length, the models of circuit elements will change as follows:

- All single transmission lines become double (cascaded) transmission lines with halved lengths (Figure 1.88.a).

- All single stubs become second order stub resonators which are equivalent to two-step resonators with equal step impedances and halved lengths, as shown in Figure 1.88.b-to-e.

Figure 1.88. Half angle equivalents of  TLine and single stubs (continued).

- With this transformation, a shunt arm all-stub second order section of length l  (Figure 1.83 ) becomes an  all-stub fourth order section with halved lengths which is equivalent to two parallel second order sections. In Filpro application of Half Angle command to a second order section brings two choices:

Double Resonator

Double Stub

Double resonator option is the exact equivalent of the fourth order section resulting from this transformation. Double stub option is an approximation obtained by replacing the stub  resonators by open circuited uniform stubs. This approximation is useful for narrow bandstop filters (see Chapter 8). These interrelations are described in Figure 1.89.

Figure 1.89. Half angle equivalents of  a shunt arm series stub resonator.

- The dual of the previous operation is shown in Figure 1.90. Here, a series arm all-stub second order section is replaced by two second order sections with halved lengths.

Figure 1.90. Half Angle Equivalents of a series arm parallel stub resonator.

- The equivalence shown in Figure 1.85 is derived from the previous ones and is used to convert distributed element highpass filters into bandpass forms. Note that effectively, the degree of filter will be doubled after that transformation.

Figure 1.91. Half angle equivalent of  Stub+ TLine version of an open circuited interdigital

parallel  coupled line section.

Half Angle transformations are mainly used for two purposes in Filpro:

- After application of Half Angle transformation, distributed element high-pass filters become distributed element bandpass filters with halved stub lengths. This property can be used to derive some topologies for bandpass filters from highpass filter topologies.

- Some unrealizable elements or sections may become realizable after replacing them by Half Angle equivalents.

1. 7. 8. Hairpin  Operations

As will be desribed in Chapter 8, a class of distributed element highpass filters (open circuited parallel coupled line filters) can be converted to hairpin type filters. In Filpro this conversion can be carried out using the two transformations described below (Figure 1.92 ):

Figure 1.92. Hairpin Operations command.

- Hairpin Bandwidth Scale

This command is used in deriving hairpin type bandpass filter element values from a parallel coupled line filter where the bandwidth of parallel coupled line filter is to be narrowed for converting the structure into hairpin form (see Chapter 9). In order to apply this transformation, cursor can be placed anywhere in the parallel coupled line filter.

- 2 TLine-to-Hairpin

This command applies to two cascaded transmission lines (with cursor under left TLine) and converts Two TLine section into all-stub equivalent of an uncoupled hairpin type line, as shown in Figure 1.78 (see also PCL - parallel coupled line). This all-stub model is used in design of hairpin-line distributed highpass filters (see Chapter 9).

Figure 1.93. Replacing two cascaded transmission lines by its all-stub model which can then be

converted into an uncoupled  hairpin line.

1. 7. 9. Homogeneous-to-Inhomogeneous (Change Dielectric Constant)

This command is mainly developed for application to parallel coupled lines to simulate inhomogeneities (Figure 1.94 ). However it is also applicable to single lines and stubs.

- This command has to be used to simulate small differences in even and odd mode dielectric constants causing inhomogeneity in parallel coupled lines. Due to a set of complicated interrelations among the parameters characterizing a coupled line, any change in the even (common) and odd (Pi ) mode dielectric constants will also change all the other line parameters (even and odd mode characteristic impedances and voltage transformation ratios, Rc,p). Therefore such modifications in dielectric constants have to be made carefully. If no change is desired in the other parameters of the coupled lines, then it is better to use the command Edit / Change.

- This command can be applied to single transmission lines and stubs just to calculate their new physical lengths to keep their electrical length constant as the dielectric constant is changed.

Figure 1.94. The command Homogeneous-to-Inhomogeneous / Dielectric Constant.

1. 8.  CIRCUIT TRANSFORMATIONS - 3

The circuit transformations collected under this menu are of mixed lumped -distributed nature. Some circuit transformations are applicable to both lumped and distributed ones while some are applicable only to distributed elements.

1. 8. 1. Norton Transformations

Norton transformations are the basis of most of the circuit transformations which enable us to form a variety of equivalent circuits.

The two basic Norton transformations are shown in Figure 1.95.a and b [32]. In these transformations a single series (shunt) element (lumped elements or stubs) is replaced by a Tee (Pi ) network together with a transformer. Depending on whether the transformer ratio k is greater or less than one, either the left or the right element of Tee and Pi sections will be negative. The negative element have to be absorbed by the neighboring positive elements.

Figure 1.95. Norton / Specify Trf   (or Specify Left Elm or Specify Right Elm) transformations                         applied on single elements.

When Norton command is applied to a single element, the following options will be available:

- Specify Transformer

- Specify Left Element

- Specify Right Element

as described in Figure 1.95. The user can specify these element values within the limits displayed at the bottom of the screen.

If the command is applied on a resonator (second order element), then only Specify Transformer option is available as shown in Figure 1.96.

Figure 1.96. Norton / Specify Transformer command applied on second order elements.

The command Norton/Scale Pi-Tee-Resonators can be used in scaling some element values in bandpass filters at the expense of increasing number of elements in the circuit. This command is applicable in the following cases:

- Scaling Shunt Arm Parallel Resonator:

- A shunt arm parallel resonator in between two series arm capacitors can be scaled as shown in Figure 1.97.a. Here, cursor should be under the shunt resonator and the option Parallel Resonator in Shunt Arm will be selected. The desired value of shunt inductor will be specified by the user.

Figure 1.97.  Scaling   a) shunt arm resonator and   b) series arm resonator (by specifying desired inductance)  using   Norton / Scale Pi - Tee - Resonator command.

- Scaling Series Arm Series Resonator:

A series arm series resonator in between two shunt arm capacitors can be scaled as shown in Figure   
1.91.b. With the cursor under the series resonator, the option Series Resonator in Series Arm 
should be selected. The desired value of inductor will be specified by the user.In these commands Norton command is actually applied on the two outside capacitors automatically such the the resulting transformers will scale the middle inductor to the specified value.

- Scaling a Fourth Order Section:

- When the Norton/Scale Pi-Tee-Resonator command is applied to a shunt arm fourth order section in between two series arm capacitors, as shown in Figure 1.98, then Filpro directly asks the new value of the inductor of the parallel resonator section of fourth order section and applies necessary transformations.

- Scaling elements of Pi and Tee sections:

- Pi or Tee sections made up of similar elements may be scaled at the expense of increasing element numbers, as shown in Figure 1.99.

- All these transformations are also applicable to all-stub filters.

Figure 1.98. Scaling  a shunt fourth order section using  Norton / Scale Pi -Tee-Resonator command. Negative capacitors may be absorbed in the positive capacitor of parallel resonator section.

Figure 1.99. Scaling  a) series element of a Pi- section and   b) shunt element of a Tee- section                         using  Norton / Scale Pi -Tee-Resonator command.

1. 8. 2. Pi-Tee-L Transformations

This command is one of the most frequently needed transformations for conversion of circuits from one topology into another. It can be applied on Pi, Tee or L (LRight, LLeft) type circuits formed by the same type elements (all inductors, all capacitors, all OC stubs or all SC stubs, all parallel resonators, all resistors, all constant reactances, etc.). These transformations are nothing but more “refined” versions of Norton transformations. In addition to the basic Pi-Tee-L conversions, there are also some more complex transformations developed for special filter configurations or to get special topologies. Let’s summarize all these commands.

Basic Pi-Tee-L transformations:

Any Pi, Tee or L type section can be converted into equivalent Pi, Tee or L sections having some specified properties using the command sets (Figure 1.100). In these transformations, whenever it is possible, the user can specify the value of one element of the resulting structure within the upper and lower limits displayed at the bottom of the screen. Thus, besides being very useful tools to reconfigure circuits of lumped and distributed element bandpass filters, these commands may also be used to scale some element values.

Figure 1.100. Pi-Tee-L Transformations.

- LLeft or LRight-to-Pi-Tee-L transformations:

An LLeft or LRight section may be transformed into LRight, LLeft, Pi or Tee sections. User has the options of setting one of the parameters of the resulting circuit. Figure 1.101 shows some examples.

Figure 1.101. Some LLeft-to-Pi-Tee-L transformations.

- Pi-to-Pi-Tee-L transformations:

A Pi section may be converted into LLeft,  LRight, Tee or another Pi section. User have also the options of replacing a Pi section by a double Pi section and replacing a symmetric Pi section by two symmetric Pi sections. Some examples are shown in Figure 1.102.

Figure 1.102. Some Pi-to-Pi-Tee-L transformations.

- Tee-to-Pi-Tee-L Transformations:

A Tee section may be replaced by Lleft, Lright, Pi, another Tee or double Tee sections. Some examples are shown in Figure 1.103.

Figure 1.103. Examples for Tee-to-Pi-Tee-L transformations.

- Pi-to-Sym Pi,  LLeft-to-Sym Pi and LRight to- Sym Pi:

In many applications Pi, LLeft and LRight sections have to be converted into symmetric Pi sections because symmetric Pi sections are by far the most frequently needed circuits. Therefore the transformations Pi-to-Sym Pi, LLeft-to-Sym Pi and LRight-to Sym Pi are programmed separately, as short cuts for application.

Pi-to-Sym Pi command, in addition to the circuits described above, may also be applied on Pi sections formed by parallel LC resonators or mixed L, C and LC elements or Pi sections formed by inductively or capacitively coupled shunt parallel LC resonators, as shown in Figure 1.104.

Figure 1.104. Application of Pi-to-Sym Pi command on a Pi sections of LC resonators.

Figure 1.104. Application of Pi-to-Sym Pi command on a Pi sections of LC resonators (continued).

- Pi-Tee-L / All Equal Shunt:

This command is a generalization of Pi-to-Sym Pi command to the whole circuit where all shunt inductors or all shunt SC Stubs of the circuit are to be equated (The option for equating shunt capacitors or shunt OC stubs is INOPERATIVE in this version of Filpro!. If necessary, such an operation can be carried out by repeated application of Pi-to-Sym Pi command on all Pi sections). This is possible only if at least one parallel LC resonator exists in the shunt arms and a parallel LC resonator exists in one of the series arms. Then, by successive and iterative application of Pi-to-Sym Pi command to each Pi section, the shunt arm inductors or capacitors may be equated  to each other. One example is shown in Figure 1.105. Note that all shunt arm elements now became LC resonators, with equal inductors. If no series arm parallel resonator exists in series arms then it may not be possible to equate all shunt inductors or capacitors depending on the element types in the circuit. Also, if the element values are in the extremes (as usual in typical synthesis procedures involving high degree filters), the algorithm may not converge to its target. In such cases simple Norton or other Pi-Tee-L commands may be used to scale element values to reasonable levels and then apply the command All Equal Shunt.

Figure 1.105. Pi-Tee-L / All Equal Shunt / Inductors (Capacitors) command.

- Pi-Tee-L / Comb-All Equal Shunt

This is a transformation developed to equate all shunt SC stubs of  combline and evanescent waveguide filters. These filters have a single transmission zero at f=0. During synthesis stage, this transmission zero appears as a shunt SC stub somewhere in the circuit. In order to form the standard comb or evanescent waveguide filter topologies, this shunt SC stub should be distributed into the circuit such as a SC stub will appear in parallel with all shunt OC stubs. This target may be reached using Pi-Tee-L transformations applied on the SC stub L or Pi sections iteratively. However this is a tedious process. The command Pi-Tee-L / Comb All Equal Shunt carries out this iterative process leading to all equal shunt SC stubs(Figure 1.106). If desired, further adjustments of SC or OC stub impedances may then be carried out using the other Pi-Tee-L commands (See Chapter-11 on combline filters and Chapter-14 on Evanescent Waveguide filters).

Figure 1.106. Pi-Tee-L / Comb-All Equal Shunt command.

- CC-All Equal Shunt (Cross Coupled-All Equal Shunt):

This command is similar to All Equal Shunt command, but it applies only to Even or Odd Halves of Symmetric Cross Coupled filters. See Chapter-15 for details.

- Scale Stub Pi-to-Pi:

This is an approximation applicable to Pi-sections of OC or SC stubs. A Pi-section of stubs is replaced by another Pi-section with shorter or longer stub lengths resulting in increased or decreased impedance levels (Figure 1.107). This transformation is useful in adjusting impedance levels of Pi-sections of stubs in narrowband filters.

Figure 1.107. Scale Stub Pi-to-Pi command for adjusting impedance level of a Pi- section of stubs by changing length of the stubs.

1. 8. 3. Double Norton Transformations

These transformations are more sophisticated versions of Norton transformations. In these commands Norton transformation is applied on two elements simultaneously to reach a targeted topology. This command has the following options shown in Figure 1.108.

Geffe Equivalent, Inverse Geffe Equivalent

Colin Transform-Right, Colin Transform-Left

Humphery’s Transform-Right, Humhery’s Transform-Left

Equal Shunt Inductors, Equal Shunt Capacitors

Figure 1.108. Double Norton /Geffe command and options.

a) Geffe Equivalent:

Geffe transformation is applicable to the circuits shown in Figures 1.109-to-1.112. Such circuits arise in elliptic and Generalized Chebyshev bandpass filters mapped from lowpass prototypes. A finite transmission zero producing section of the lowpass prototype is mapped into the bandpass filter as a fourth order section producing two finite transmission zeros on the two sides of passband. These fourth order sections can be splitted into two adjacent second order sections to give the circuits shown on the left hand sides of Figures 1.109-to-1.112. Usually the two adjacent resonators are undesirable, especially the series arm parallel resonators where the parasitic capacitance to ground appears as a troublesoem factor. Geffe transformation is nothing but application of Norton Transformation to the two adjacent resonators to insert another resonator branch in between them, as shown on the right sides of Figures 1.109-to-1.112. This transformation may also be used to increase redundancy of the circuit for the sake of obtaining better behaved element values.

In these transformations Filpro first asks the transformer ratio of the first Norton transformation that will be applied on the resonator on left. Then three options will be displayed as shown in Figure 1.108:

- Unity Total Transformer option sets the second Norton transformer (that will be applied on the second resonator) as the inverse of the first transformer, resulting in no net transformer.

- The second option is either No Middle Inductor or No Middle Capacitor depending on the element values and topology. These options enable designer to eliminate one element of the resonator that will be inserted in between the two original resonators.

c)   Specify Second Transformer option leaves specification of the second transformer to the user. In this  case, a desired total transformer will be obtained on the right of resulting circuit.

Figure 1.109. a) Geffe transformation on series arm fourth order section (two series arm second order sections in series). b) Option without middle shunt arm inductor.

Figure 1.110. a) Geffe transformation on shunt fourth order section (two parallel second order sections).        b) Option without middle series arm capacitor.

Figure 1.111. a) Geffe transformation on stub version of shunt fourth order section (two parallel second order sections). b) Option without middle series open circuited stub.

Figure 1.112. a) Geffe transformation on stub version of series arm fourth order section (two series arm second order sections in series). b) Option without middle shunt arm short circuited stub.

b) Inverse Geffe Equivalent:

As the name implies, Inverse Geffe transformation does the reverse of the Geffe transformation. This transformation applies on the circuits obtained using Synthesis approach. In Synthesis approach, when two finite transmission zero sections are extracted one after the other, the circuit sections shown in Figures 1.113 - 114 are obtained . In some cases it may be more advantageous to have fourth order sections in the circuit. Inverse Geffe transformation combines the two adjacent finite transmission zero sections so as to get a fourth order section. The fourth order sections appear either as two adjacent shunt arm series resonators or two adjacent series arm parallel resonators. These transformations are derived by applying the basic Norton transformations to the two LC resonators forming the finite transmission zeros (FTZ) in such a way that the resulting negative elements are used to eliminate the undesired elements and the two FTZ resonators will come out to be adjacent.

Figure 1.113. Inverse Geffe transformations on cascaded lumped element finite transmission zero sections extracted in synthesis stages.

Figure 1.113. Inverse Geffe transformations on cascaded lumped element finite transmission zero sections extracted in synthesis stages (continued).

Figure 1.114. Inverse Geffe transformations on stub versions of cascaded finite transmission zero sections extracted in synthesis stages.

- c) Colin Transformations:

- These transformations can be used either to convert a series arm finite transmission zero section        (parallel resonator in  series arm) into shunt arm finite transmission zero section (shunt arm series resonator) and vice versa [2], [8, p.565], or to reverse the sense of L-section formed by  resonators as described in Figure 1.115. These transformations are obtained by using the  Norton equivalents of both resonators. They all produce a negative capacitor at one end. Therefore these transformations are applicable only if the negative capacitor can be absorbed by a positive neighboring capacitor. See Chapter-6 for examples.

- Figure 1.115. Colin transformations.

d) Humpherys Transformation:

Humpherys transformation is shown in Figure 1.116. This transformation is useful in reducing the number of inductors which is quite important in  practice. The resulting circuit contains a negative element which should be absorbed by a positive neighbor, if exists. See Chapter-6 for examples.

Figure 1.116. Humpherys transformations.

e) Equal Shunt Inductor:

These commands are applicable to the circuit types shown in Figure 1.117.a-to-g. The cursor should be under the middle shunt element. There should be at least one shunt parallel resonator. In the resulting circuit all the shunt inductors can be made equal. {Equal shunt capacitor case is not programmed yet, but it can be carried out using Pi-to-Sym Pi / Equal Shunt Cap command iteratively}.

Figure 1.117. Circuits on which Double Norton/Equal Shunt Inductor command can be applied.

1. 8. 4. Replace Transmission Line By

This command replaces transmission line pieces (TLine) by their stub or lumped element equivalents, chosen from the dialog box shown in Figure 1.118. Various models are shown in Figures 1.119-120. In Tee and Pi models only all-stub versions are exact. All the other versions are approximate and the program  asks the frequency of approximation.

a) Tee Models:

The All-Stub Tee model of a transmission line involves SC and OC stubs with lengths equal to half of that of the line (Figure 1.113.a). The SC stub in shunt arm has negatice characteristic impedance. Therefore it is useless in this form. However net reactance of the shunt arm at a frequency less than the quarter wavelength frequency of the line enables us to replace this arm by a lumped capacitance evaluated at band center. If the series arm SC stubs are also replaced by their lumped inductive equivalents then we get the all lumped element Tee model shown on the right of Figure 1.113.a. This all-lumped model is a good approximation at frequencies much less than the quarter wavelength frequency of the line.

b ) Pi Models:

The All Stub Pi equivalent of a line piece involves OC and SC stubs with lengths equal to half of the length of the line (Figure 1.113.b). The middle series arm element is made up of parallel connection of a positive SC stub and a negative OC stub. Therefore this model is useless in this form. However the net reactance of this middle element at frequencies less than the quarter wavelength frequency of the line enables us to replace this element by a series lumped inductor. If  the shunt OC stubs are also replaced by their lumped capacitor equivalents, then we get the all-lumped model shown on the right of Figure 1.119.b. This model is a good approximation at frequencies much less than the quarter wavelength frequency of the line.

Figure 1.118. Command sets for replacement of a transmission line piece by various equivalents.

Figure 1.119. Tee and Pi models of a transmission line piece.

c) Various Approximate models:

Figure 1.120  shows some approximate equivalents of a transmission line piece are shown. Some of these models are exact only at one frequency and  valid approximately below the quarter wavelength frequency. They may be used for development of special topologies. Figure 1.121 shows one application example. The transformations shown in Figures 1.120.e and f are exact. In these transformations a single transmission line piece is replaced by a parallel coupled line with secondary line end being open or short circuited. Figure 1.122 shows an example.

Figure 1.120. Various approximate models of a transmission line.

Figure 1.121. Replacement of a line piece by an approximate equivalent made up of two line pieces and a shunt OC stub.

Figure 1.122. Replacement of a transmission line piece by a symmetric parallel coupled line  with open circuited secondary line ends.

1. 8. 5. Replace By TLine

This command replaces some elements by their transmission line equivalents. The applicable elements are shown in Figure 1.123. These are all approximate transformations, some of which requires frequency of approximation or impedance of  the line.

Figure 1.123. Elements that can be replaced by transmission line equivalents.

1. 8. 6. Replace By Lumped Equivalent

This command replaces open circuited stubs by lumped capacitors and short circuited stubs by lumped inductors, evaluated at a frequency of approximation specified by the user (Figure 1.124). Also a transmission line is replaced by its equivalents studied in the command Replace Tline By/...

Figure 1.124. Replace By Lumped equivalent command on SC and OC stubs.

In combline type filters when the shunt OC stubs are replaced by lumped capacitors, the bandpasss edge frequencies of the filter will shift. In order to avoid this shift, impedance of the shunt SC stub adjacent to the OC stub should also be modified slightly. It may also be required to find the lumped equivalent of the SC stub as well. All these modifications are programmed in Filpro under the menu item Replace By Lumped. In order to apply this modification, the adjacent shunt SC and OC stubs should be combined to form a single element called SC+OC stub resonator (See Figure 1.125).

Figure 1.125. Replace By Lumped Equivalent command as applied on OC+SC stub parallel resonators to get their SC Stub+Capacitor models and then their lumped LC models.

When we apply Replace By Lumped command to this element, Filpro will offer the options: SC Stub+Cap,  All SC Stub+Cap, Lumped LC, All Lumped LC and Half Length OC Stub. The SC Stub+Cap option replaces the SC+OC Stub element by parallel combination of a SC Stub and a lumped capacitor. All SC Stub+Cap option is a short cut where Filpro converts all the SC+OC Stub elements of the filter into SC Stub+Cap with a single command. Lumped LC option converts the SC+OC Stub element into the equivalent lumped LC. In All Lumped LC option all the SC+OC Stub elements of the filter are converted into lumped LC form automatically. If Replace By Lumped command is applied to SC Stub+Cap elements, then this element is converted into its lumped LC equivalent. The option Half Length OC Stub replaces the OC stub by its halved (or user set length) equivalent to improve the stopband performance of combline filter. After selecting one option, Filpro asks the band passband edge frequencies of the filter and the new element values are calculated to keep the total reactance the same at the two specified frequencies.

Replace By Lumped command converts also an All Stub Bridge Tee into a Mixed Element Bridge Tee and converts a Mixed Element Bridge Tee into an All Lumped Bridge Tee. As extension of Bridge Tees, an All-Stub CT section can be converted into a Mixed CT section and a Mixed CT section is converted into an All-Lumped CT section.

1. 8. 7. Replace By Stub(s)

This command replaces some elements by their approximate stub equivalents. Applicable elements are shown in Figure 1.126.

Figure 1.126. Elements that can be converted into equivalents involving  stubs.

Figure 1.126. Elements that can be converted into equivalents involving  stubs (continued).

1. 8. 8. Inverters

The use of inverters in converting synthesized filters into different topologies is an old but still one of the most powerful approach providing great flexibility for finding realizable equivalent circuits [1], [36]. They come into picture whenever an exact circuit transformation can’t be found. Therefore their popularity never decreases. In fact inverters also used as one of the essential circuit elements in some synthesis approaches. Inverters occupy a wide space in Filpro. When “Inverters” command is invoked, the options shown in Figure 1.127 are displayed in the dialog box. Some of these options form wide subjects offering many alternatives, as described below.

Figure 1.127. Inverters command and the options.

- Replace By Ideal Inverter

This command replaces a shunt (series) element pointed by cursor by a series (shunt) element placed between two ideal inverters (Figure 1.128-129) in cascade with a phase changing transformer (1:-1).

Figure 1.128. Replace By Ideal Inverter command on lumped elements.

Figure 1.129. Replace By Ideal Inverter command on stubs.

Ideal inverters are frequency independent two port networks having ABCD matrices of the form

where K is termed as impedance inverter constant. Since a single element is replaced by three elements in this transformation, a redundancy exists, giving us the freedom of choosing inverter constant K. Thus, an element may have infinitely many inverter equivalents corresponding to different K values.

In applying this transformation, user sets the desired inverter constant K and Filpro calculates the values of resulting elements. Ideal inverters replace shunt inductors, capacitors, OC stubs and SC stubs by series capacitors, inductors, SC stubs and OC stubs respectively and vice versa. A shunt arm parallel (series) resonator becomes a series arm series (parallel) resonator and vice versa, as described in Figures 1.128 and  129.

ii ) Replace Ideal Inverter By:

This command is used to replace ideal inverters by some practical circuits which are equivalent to ideal inverters only at one frequency. That is an approximation is involved which will degrade response to some extent. Both passband return loss and stopband insertion loss will change passband edges will shift. Such degradations are usually negligible for narrowband filters, but can be considerable for medium/wideband filters. Therefore some kind of trimming-tuning-optimization is usually necessary after replacement of inverters by their practical equivalents. In order to minimize such tuning-optimization  needs, whenever possible, the ideal inverters are treated together with their adjacent elements and the adjacents elements are also adjusted automatically while replacing the inverters. Adjustments are usually done such that the passband edge shifts (of bandpass filters) will be minimized. This is made possible by equating the ABCD parameters of the original and new circuit sections at two frequencies (usually passband edges) selected by the user.

Most of the practical inverter circuits placed in Filpro are commonly used ones while some are thought to be new, developed in expectation to find new applications. After choosing the desired practical circuit section for replacement of ideal inverter section, Filpro asks either the center frequency of application or passband edges of the filter. Also if the new circuit section involves stubs or transmission line pieces, Filpro asks more information, such as quarter wavelength frequency (fq), or lengh or impedances of stubs and/or transmission lines.

When this command is prompted, a dialog box appears showing the available options. Some of the alternative circuits are grouped under these options. These are summarized below (Figure 1.130).

Figure 1.130. Replace Ideal Inverter By command and the options.

a) Pi-Tee Type inverters:

When Pi-Tee Type option is selected another dialog box will appear as shown in Figure 1.131. Inductive, capacitive, SC Stub and OC Stub Pi and Tee inverters are available. The outside elements (shunt elements of Pi sections and series elements of Tee sections) are always negative. They should be absorbed by the neighboring positive elements of the same kind, if available. Therefore such practical inverters are aimed for use in bandpass filters only where there are always positive shunt or series arm LC or SC+OC stub elements to absorb the negative elements of inverter models. {For LP, HP and BS filters, the option Inverters / Replace Ideal Inverter By Constant Reactance / Pi-section should be used. Then the constant reactances should be replaced by their practical inverters.}

When the desired Pi-Tee type practical inverter option is selected Filpro will ask the quarter wavelength frequency fq (applicable to distributed element filters only) and the passband edge frequencies, as f1 and f2. Filpro will calculate the elements of Pi or Tee sections so that the original and new circuit will be equivalent at f1 and f2. This approach is aimed at keeping the passband edges of the filter unchanged. This approximation will cause some degradation in passband return loss which can be corrected by slight tuning the elements of the filter.

Figure 1.131. Pi-Tee type inverter options.

Figure 1.132.a shows a distributed element bandpass filter with 780 MHz-820 MHz passband. It involves  ideal inverters and SC+OC stubs. The quarter wavelength frequency setting the length of stubs is fq=1500 MHz. The ideal inverters are replaced by their SC Stub Pi equivalents using the command Inverters / Replace Ideal Inverter By / Pi-Tee / SC Stub Pi: fq=1500 MHz, f1=780 MHz, f2=820 MHz (Figure 1.132.b). The negative shunt SC stubs are absorbed into the adjacent positive SC stubs by applying the command Simplify Circuit (Figure 1.132.c). Return Loss response is slightly degraded while pasband edges are kept unchanged.

Figure 1.132. Replacement of ideal inverters by SC Stub Pi sections.

Figure 1.132. Replacement of ideal inverters by SC Stub Pi sections (continued).

b) Levy Inverters:

In these options ideal inverters are approximated by elements placed in between two transmission line pieces (Figure 1.133) [36]. User will specify impedances of the two outer transmission lines together with some further parameters needed in calculation of element values. In some of these circuits negative lengths will result. They can be converted to positive values by adding half wavelength lines with the same characteristic impedance. However, in an actual application they can be left as they are if a positive length neighboring line exists. It should be noted that increasing or decreasing the lengths of outer lines by quarter wavelength narrows the bandwidth. In the case of a Pi or Tee section in between the two lines, user is asked to type in the Pi or Tee element values and the impedances and lengths of outer lines are found by iterating on line impedances. This is necessary because of nonlinear relations among various parameters and not all user specified parameters lead to realizable solutions.

Figure 1.133. Levy inverters.

Four examples are shown in Figure 1.134 where a 50 ohms source impedance is matched to 10 ohms load through an ideal inverter of 22.3 ohms. The first practical inverter a series inductor in between two transmission line pieces, the second has a shunt short circuited stub, the third one being a three step line and finally, the fourth one involving constant reactance Pi section in the middle. The last equivalence needs trial error type adjustments for constant reactances until a solution is found. The constant reactances may then be replaced by capacitprs, inductors, stubs or resonators, using the command “Constant Reactances Replace Constant Reactance By / ..”.

Figure 1.134. Examples for Levy inverters .

Figure 1.134. Examples for Levy inverters (continued).

Figure 1.134. Examples for Levy inverters (continued).

c) Constant Reactance inverters:

In this option the ideal inverter can be replaced by circuits involving constant reactances. Three options are programmed (Figure 1.35): Constant Reactance Pi, Constant reactance Tee and a shunt constant reactance placed in between two transmission line pieces (TL+Xsh+TL). Constant reactances are extremely versatile tools for all types of filters, LP, HP, BP or BS. The constant reactances can be replaced by L, C series or parallel LC resonators, SC Stubs or OC Stubs using Replace Constant Reactance By /..  command. When Pi or Tee type section is needed in LP, HP or BS filters, this option can be used. Thus, many more alternative practical inverter circuits can be formed using that approach. The Pi, Tee circuits and some of the Levy inverters presented above are just some special cases of these constant reactance versions.

Figure 1.135. Constant Reactance equivalents of ideal inverters.

d) Quarter Wavelength Line:

A transmission line works as an inverter at and around the frequency where it is quarter wavelength long. This replacement is described in Figure 1.136. When this command is applied, Filpro asks the center frequency and calculates length of the line such that it will be quarter wavelength long at that frequency. Characteristic impedance of line is equal to the inverter constant K.

Figure 1.136. Replacement of an ideal inverter by a transmission line at f=1000 MHz.

e) Quarter Wavelength line and Shunt SC Stubs:

A quarter wavelength transmision line piece with negative shunt SC stubs on both sides having the same length as the transmission line is equivalent to an ideal inverter at a much wider frequency range compared to a single quarter wavelength line (Figure 1.137). The characteristic impedance of stubs are negatives of the characteristic impedance of the quarter wavelength line. However, contrary to the single quarter wavelength line case, impedance of the line is not equal to the inverter constant K. When this command is applied two frequencies will be asked by Filpro: The quarter wavelength frequency and another frequency which will set the low end of the passband. Clearly, this model can only be used if there are positive neighboring stubs and negative shunt stubs can be absorbed by these positive stubs. That is, this model can be used only in distributed element highpass (bandpass) filters.

Figure 1.137. Replacement of an ideal inverter by SC Stub+TLine+SC Stub section.

f ) Transmission line and series OC Stubs:

This is the dual of previous case. A transmission line with two negative series OC stubs on each side is also equivalent to an inverter over a much wider bandwidth compared to single quarter wavelength case (Figure 1.138).  Response of the circuit of Figure 1.138 will be the same as that in Figure 1.137. This model can only be used if there are adjacent positive stubs to absorb the negative stubs, that is, in highpass or bandpass filters.

Figure 1.138. Replacement of an ideal inverter by a transmission line with series OC stubs (OSsr+TL+OSsr).

g ) Transmission line and constant reactance L-sections:

A transmission line with constant reactance L-sections on both sides can be used to simulate ideal inverter at a single frequency in much the same way as the previous three inverters with quarter wavelength lines. When the line is loaded by constant reactances on both sides, the line length can be made much smaller than the quarter wavelength at the frequency of operation. We can also adjust the impedance of line and constant reactances. The constant reactances may then be replaced by a proper lossless element like capacitor, inductor or OC-SC stubs. Figure 1.139 shows an example. Note that the series and shunt constant reactances are of opposite sign. This necessitates the use of two kinds of lossless elements in the circuit. For example if the series element comes out as capacitive then the shunt element will be inductive.

Figure 1.139. Replacement of an ideal inverter by a transmission line with reactive L-sections on both sides.

h ) Transmission line and capacitive L-sections:

In the previous approach, the series and shunt constant reactances of L-sections were of opposite sign leading to different types of elements. In some cases this may not be desirable. At high frequencies it is usually preferable to have capacitive reactances because of relative ease in realization. The inverter approximation with a transmission line loaded by capacitive L-sections on both sides may be a desirable solution. This is described in Figure 1.140.

Figure 1.140. Replacement of an ideal inverter by a transmission line with capacitive L-sections on both sides.

i ) Capacitively Loaded Evanescent Waveguide:

See Chapter-14 on Evanescent waveguide filters.

k ) End Inverters:

Ideal inverters adjacent to source and load need special treatment because in the equivalent circuit no nagative element is allowed adjacent to RS and RL. The command “Inverters / Replace Ideal Inverters By / End Inverter to / ..” offers Pi and L-sections of capacitors, inductors, OC stubs and SC stubs with positive shunt elements on source or load sides. In Pi-sections, user sets the value of element adjacent to RS or RL. In L-section options the resulting L-section will have negative shunt element which should be absorbed by an adjacent positive element of the same kind (Figure 1.141 and 1.142). User sets the frequency of approximation.

Figure 1.141. Replacement of End Inverters by Pi and L-sections.