# CHAPTER 1

A GUIDED TOUR OF FILPRO

# 1. 1. INTRODUCTION

This chapter introduces basic functions of Filpro some of which will be detailed in the succeeding chapters together with filter design applications.

Filpro is an integrated synthesis, circuit transformation and analysis tool developed mainly for design of amplitude filtering circuits formed by passive, lumped and /or distributed elements in cascade topologies. However it can also be used to design and analyze symmetrical cross-coupled filters, many multiplexer configurations, branchline or parallel coupled line directional couplers and impedance matching circuits. Active or nonreciprocal elements characterized through their measured or calculated scattering parameters may also be included in the circuits.

The functions of Filpro may be collected in three groups:

a) Design.

b) Circuit transformations to reach a targeted structure.

c) Analysis.

a) Design stage consists of the following options:

- Filter design using built-in LC prototypes.

- Filter design using prototypes with inverters.

- Filter design using synthesis techniques.

- Filter design by user defined prototypes.

- Linear phase symmetric cross-coupled filter design using lowpass prototype tables.

- Approximate design of selective linear phase symmetric cross-coupled filters with one or two cross-couplings and one or two poles in each stopband.

- Circuit design by inserting elements from the library of Filpro or by cascading/paralleling readily designed and saved circuits.

- Lowpass/ Highpass diplexer design.

- Multiplexer analysis and design of most kinds.

- Branchline, parallel coupled line and rat race directional coupler design and analysis.

- Two way, four way and eight way Wilkinson divider design and analysis.

- Lumped and distributed element Impedance transforming filter design.

- Dual Band Impedance matching filter design.

- Cascaded Triplet (CT) filter design using cascade synthesis approach.

- Cascaded Quadruplet (CQ) filter design using cascade synthesis approach.

- Generalized Cascaded Quadruplet (GCQ: CQ sections with diagonal cross coupling) filter design using cascade synthesis approach.

- Filters involving GCQ, CQ and CT type elements.

Designed circuit schematics appear on the lower half of the screen together with a circuit cursor pointing elements. Parameters characterizing the pointed element appear in a small box on the right bottom corner of the screen. One can move either the cursor or the whole circuit to right and left to inspect various sections.

b) Circuit transformations section forms the most significant contribution of Filpro in filter realization area. Usually a large number of circuit transformations have to be applied on synthesized filters to put the circuit into a realizable form with realizable element values. A proper transformation can turn a seemingly unrealizable filter into a form which is very easy to realize. These circuit transformations are usually the most cumbersome and tedious stage of any RF and microwave filter realization process. This is because of existence of too many special transformation techniques aiming at diverse special structures. In Filpro, almost all commonly used and some novel circuit transformations are programmed in menu format. The transformations can be chosen from the menu and applied to a section of displayed circuit which is pointed by cursor, resulting in replacement of that section by the new equivalent. Successive application of the proper transformations leads to the desired circuit. In practice, many iterations may be needed by changing some design parameters to reach a realizable circuit in the chosen topology. For such applications, Macro programming facility of Filpro can be used in which case Filpro learns the order of transformations in the first iteration and it can repeat them in the successive trials when called. This feature enables the designers to create their own “One Push Button” softwares. Using these tools, one can design almost all RF and microwave filters, classical or novel, subminiature, superconducting, Cascaded Triplet, Cascaded Quadruplet, etc.

c ) Analysis tools of Filpro are developed to answer almost all practical needs at any stage of design process. Insertion Loss, Return Loss, Phase, Delay, Smith Chart responses, Element Sensitivity, Tuning Element, Tuning Multiplexer, Monte Carlo, Worst Case, Best Case, Yield Analysis techniques, overplotting, replotting, rescaling, zooming, brick walls, reference lines, moving marker and delta marker tools are available. Best Case analysis is also a local random optimizer working in terms of Monte Carlo parameters. By assigning large deviations in Monte Carlo parameters, it may also work as a global optimizer to a limited extent. Coupling Coefficients and resonance frequencies of direct-coupled and cross-coupled filters can be displayed.

In this chapter a guided tour of Filpro is presented just to make a quick survey of the available techniques and tools. The contents of this chapter can best be evaluated while studying the forthcoming chapters which also form a review of the whole RF and microwave filter design theory with a new eye. A large number of design examples are presented as applications of the available tools of Filpro.

- 1. 2. INSTALLING FILPRO AND HARDKEY (For Windows 95/98/NT)

STEP-1: Create a directory (e.g. C:\MYFILPRO) and copy all Filpro and Hardkey files into this directory. Switch to MS-DOS and change directory to C:\MYFILPRO and then pass to Step-2.

(If you do not know how to create a directory and copy files into it, you may wish to see the detailed version of this step in the next page).

Hardkey installation files are: HINSTALL.EXE and HDD.HLP.

STEP-2. Extract Filpro files.

- Being in the directory C:\MYFILPRO> try to extract the file FilproXX.zip using Winzip as follows:

- Pass to Windows by pressing Alt-Tab.

- Run Windows Explorer and find the directory C:\MYFILPRO.

- Double-click on FILPRO25.ZIP. Winzip will probably run.

- Click Extract button in WinZip and tell it to extract Filpro25.zip into C:\MYFILPRO.

- If everything is OK, then pass to Step 3.

STEP-3. Install Hardkey through the command  C:\MYFILPRO>HINSTALL –I

- Connect the hardkey to the parallel port.

- You should still be in DOS and in the directory C:\MYFILPRO> to do this step.

- Install the hardkey through the command

C:\MYFILPRO>HINSTALL  –I

(Hinstall blank –I)

Follow the displayed messages. If it is successful, your computer is ready to run FILPRO. Installation procedure is finished.

STEP-4: Running Filpro

- Still being in C:\MYFILPRO>, type in Filpro and press Enter. Filpro will start. It may take 20-25 seconds to start up depending upon your computer. Enjoy.

- In some computers it may be necessary to restart the computer after installation of the hardkey. For example in Win-95, the commands Shut Down / Restart Computer or Restart Computer in MS-DOS mode will do the job. Then run Filpro through the command C:\MYFILPRO>Filpro or using Windows Explorer to click Filpro.exe.

Uninstalling Filpro:

If it is necessary to UNINSTALL Filpro someday, pass to C:\MYFILPRO and run

HINSTALL  -R

This command will remove the Hardkey driver software. Then Delete all files in C:\MYFILPRO using Windows Explorer or MS-DOS.

After installing Filpro, it can be run as any other DOS software. Filpro will first present an About dialog box (Figure 1.1).

Figure 1.1. About dialog box.

About box will disappear after pressing Esc or Enter, displaying now a list of headings, representing groups of menus and items below them. Terminology associated with menus and items can be summarized as follows:

- Commands are messages ordering a task to be done. They can be invoked by responding to dialog boxes or by activating the items grouped under the menus. Items can be invoked using any one of the following approaches:

- By selecting an item with  up-down arrows and  pressing  Enter.

- By pressing the highlighted letter of the desired item.

- By using the short-cut key (if exists).

- Menus can either be directly selected by pressing Alt+ Highlighted Menu Letter or by switching between menus using  Left-Right arrows. If Esc key is pressed twice (usually unintentionally), menus will disappear. Press Enter key to recover menus. F10 will activate the first menu if no menu is active.

- In the foregoing sections and chapters, items will be referenced using the format  Menu / Item.

1. 3. FILE  MENU

This menu contains basic file handling operations (Figure 1.2):

Figure 1.2. File menu.

New: Opens a new design session. It will then switch to the Design menu in the adjacent column.

Load: Loads a previously designed and saved circuit file. When invoked, a box prompts a filename to be specified. If a full file name is specified, then the file will be loaded if it is found among the saved files. .FIL is the default extension of Filpro files. However file extensions are not needed in the Load command.

Example: Let the filename be Comb111. FIL. The load command is Load/Comb111. The program automatically adds .FIL extension to the filename. If Enter key is pressed without typing any name, then the complete list of saved files will appear. Then user chooses the file to be loaded using Up-Down arrows. If the file can’t be loaded then check if Filpro is looking at the right directory or not by using the command Options / Directories and type in the correct directory name.

If user specifies a file mask, then a list of files is displayed in which the user may pick a file to load. File masks are the usual DOS file masks: * substitutes one or more letters and  ? is used for single letter. For more info, user is referred to a DOS manual. The default filename input is *.FIL corresponding to all files in working directory

Save: Saves the circuit into the file which is already assigned to that circuit previously with a Save As command. If the file has the name Noname.FIL, then it will automatically call for Save As procedure.

Save as: Saves the circuit with a new name assigned by the designer. Example: Save As/ Comb111. The extension .FIL will be added by the program.

Save as Text: Saves the circuit under different text file formats, such as Default ASCII File or TouchstoneTM and LibraTM file. The file will carry the the same main file name but the extension will be .txt for ASCII files and .ctk for Touchstone/Libra files (e.g. Comb111.Txt and Comb111.Ckt). In order to change the file name, the file has to be first saved under a different name using  File / Save As:…FIL command.

View File: Opens a circuit file for viewing purposes only. It displays the circuit schematics with a cursor to point elements for displaying element values. It does not discard the current file. Pressing  Esc after viewing will return to the previous file and program environment.

View Text: Opens a text file for viewing purposes only. Any file with any extension can be viewed if it is written in conventional  ASCII format. For example: View Text/ Comb111.txt. It will display the element parameter values. However this utility is mainly used for viewing S, Z, Y parameter files needed in inserting an external two port into the circuit. (See Save S-Parameters command). Such files are limited to view maximum 1000 lines.

View Dir: Displays list of files in the specified path+file mask. It substitutes the DOS command: Dir.

Exit: Exits the program and returns to DOS prompt.

1. 4.  DESIGN MENU

1. 4. 1. Introduction

Design menu is activated after executing New command in File menu (File / New).

There are several approaches to design diverse kinds of lumped-distributed-mixed filters, multiplexers matching and coupling circuits (Figure 1.3):

Figure 1. 3. Design menu.

Design / By Built-in LC Prototypes: This option is directed to filter design using the classical reactance transformation techniques from the available (tabulated or formulated) lowpass LC prototypes that had been placed into Filpro.

Design / By Prototypes with Inverters: Filters are designed by mapping from built in lowpass prototypes containing ideal inverters. This option yields symmetric circuits for all degrees.

Design / By Synthesis: This item is the most powerful and flexible filter design tool. It activates one of the two synthesis programs that are used to design doubly or singly terminated filters by placement of transmission zeros (pole placement technique). Equiripple or maximally flat passband general stopband filters in lumped or distributed element form can be designed in cascaded element form. For very narrowband and highly selective filters sections of the circuit can be converted into cross-coupled form leading to Cascaded Triplet (CT) and Cascaded Quadruplet (CQ) type filters.

Design /	By Used Defined Prototypes: This option is also directed at filter design by the classical reactance transformations from prototypes. However in this case the prototype is prepared by the user, either by inserting elements of the prototype from sources found elsewhere (e.g. handbooks) or the prototype can be prepared using the synthesis approach described above. Some overlapping and redundancies exists among these three options. This is justifiable because each approach may be advantageous to use in different occasions. Also users with different backgrounds in filter theory may find one approach easier to use.

Design / By Inserting Elements: This option is prepared for configuring and analyzing general cascaded element type circuits, not necessarily filters. For example impedance matching circuits, amplifiers, directional couplers, symmetrical cross-coupled structures, multiplexers, etc.

Design /	LP-HP Diplexer: This option is a short cut way for designing Lowpass / Highpass type diplexers, which can also be designed using other approaches.

Design / Dual Band Pass Filter: This option is used to form a dual passband filter by proper frequency transformations form two different quasi-lowpass impedance matching filters (See Chapter-18.7).

Symmetric Cross Coupled Circuits: This option enables design and analysis of various symmetric coupled circuits through even-odd mode analysis approach. Symmetric cross-coupled filters with one and two cross-coupling elements can be designed having either a linear phase response together with a pole in each stopband or two poles in each stopband. Linear phase filters with any number of cross-couplings can be designed using the available lowpass prototype tables. Symmetric cross-coupled filters with any number of cross-couplings, symmetric branchline, parallel coupled line and rat race directional couplers can be analyzed using this approach.

All these techniques will be intensively used in the foregoing chapters. Therefore let’s just summarize their functions and use them with some introductory examples.

Four of the above items are directed towards filter design: Built-in LC Prototype, Prototype with Inverters, User Defined Prototype and Synthesis. Commonly used simple filters can be designed using any one of these four approaches. Therefore, at first sight it may appear that “too much  redundancy” exists for filter design that may confuse the novice users of Filpro. Among these techniques, weight will be given to Design By Synthesis approach, as it is the most powerful and flexible one in all respects, covering the other techniques as special cases. However other techniques have some advantages and features which may be useful in some “unpredictable” occasions. Some of the techniques have historical-academic importance while some are well-known techniques in filter community, though not the best approach. Therefore all of these techniques are included in Filpro, mostly for the sake of completeness. Whenever their use has some advantages, the use of each of these techniques will be described in detail in the foregoing chapters.

1. 4. 1. FILTER DESIGN USING BUILT-IN LC PROTOTYPES

Since most of the filter designers are familiar with the classical lumped element lowpass prototype approach, in order to familiarize beginners with the use and terminology of Filpro, we will start with Design By Built-in LC Prototype approach.  However this approach is by no means the best one. More design examples will be presented using Design By Synthesis approach in the next subsection.

The command sets for filter design by Built in LC Prototypes approach are shown in Figure 1.4.

Figure 1.4. Command set for starting design of filters with Built in LC prototypes.

- The command set  New / Design / By Built-in LC Prototype are invoked.

- Filter Type is selected using Right-Left and Up-Down arrows.

- Structure: Lumped element (LC) or commensurate length distributed element option will be selected.

- Passband : Low Pass - High Pass - Band Pass - Band Stop. Passband type is selected.

- Response (Approximation Type): Four options are available for response type:

Butterworth

Chebyshev

Generalized Chebyshev with 1 TZ at Infinity    (GCh-1)

Generalized Chebyshev with 3 TZs at Infinity  (GCh-3)

Butterworth and Chebyshev types are the classical maximally flat passband and equiripple passband filters respectively. Their stopbands have maximally flat behaviour.

The Generalized Chebyshev filters are quasi-elliptic filters uncovered and tabulated by Rhodes and Alseyab 28, 30. The lowpass prototypes denoted by GCh-1 have one transmission zero at f= while the ones denoted by GCh-3  have three transmission zeros at f=. The finite transmission zeros of these prototypes are all collected at a single frequency near the passband edges, providing high selectivities close to true elliptic type filters of the same degree but with the advantage of producing narrower dynamic range for element values. Only odd prototype degrees are tabulated, giving physically symmetric filters. Therefore RS=RL throughout the design stage. For Generalized Chebyshev filters, limited choices are available for both passband ripple and minimum stopband insertion loss, displayed in the dialog box. Generalized Chebyshev filters are just special cases of filters that are designable by Synthesis approach. They are included in Filpro just as a quick design tool.

- Terminations: The desired resistive terminations are typed in. Filters are usually designed with equal resistive terminations, RS=RL=50 . If they are not equal then the problem becomes a matching-filtering type problem, requiring special techniques, as will be described in the succeeding chapters. Terminations can be changed to desired levels after the design is completed, using the command Renormalize command of Tools menu. Default terminations are RS=50 and RL=50. This design approach poses the following limitations on terminations for some filter types:

- For Generalized Chebyshev filters only odd degree prototypes are available and terminations must be identical in the design stage. Resulting circuits are always symmetric.

- Odd degree Chebyshev filters with RS=RL come out to be symmetric always. For RLRS odd degree filters will not be symmetric and a flat passband loss will appear. The passband ripple appears as superimposed on the flat loss.

- Even degree Chebyshev filters are not allowed to have RS=RL condition in the design stage. RL must be selected to be different from RS, leading to a flat passband loss on which passband ripple is superimposed. Passband ripple level can be controlled within a restricted range by adjusting RL. Resulting circuit will be antimetric. If desired, the antimetric Chebyshev filters can be converted to symmetric forms by inserting inverter at symmetry plane (See the command Inverters / Insert Inverter at Interior). Symmetric even degree Chebyshev filters can be designed using either Design / By Prototypes with Inverters approach or Design / By Synthesis approach together with some circuit transformations.

- Odd degree Butterworth filters come out to be symmetric if RL=RS. For RLRS, resulting circuit will not be symmetric and a flat passband loss will appear.

- Even degree Butterworth filters will have a flat passband loss if RLRS.

Because of all these limitations, Built-in LC Prototype approach will be rarely used in the usual designs. Synthesis approach should be preferred in most of the applications. Built-in LC Prototype approach and other lowpass prototype approaches are usually needed for designing bandstop filters in an easier and quicker way, because Synthesis approach can’t design bandstop filters directly. In this area, Synthesis approach can be used to prepare a variety of lowpass prototypes. Then User Defined Prototype approach can be used to design bandstop filters. In this respect, Built-in LC Prototype and Prototype with Inverters approaches seems to be redundant, and they will be used rarely. They are included in Filpro mostly for the sake of completeness, their historical and academic value, and their easy to use nature for some cases.

Figure 1.5 and 1.6 show typical dialog boxes for Chebyshev and Butterworth filters, including the comments on passband loss.

Figure 1.5. Typical specifications for Butterworth lowpass filters.

Figure 1.5. Typical specifications for Butterworth lowpass filters (continued).

Figure 1.6. Typical specifications for Chebyshev filters.

- Attenuation Specifications: When invoked, two choices appear in the dialog box for Butterworth and Chebyshev filters:

- Passband + stopband specifications:

PB Ripple  (3 dB default for Butterworth)

fp  or  fp1  and / or  fp2

fs   or  fs1    or   fs2

f q       (for distributed element filters only)

- Passband specifications + degree of prototype:

PB Ripple  (3 dB for Butterworth)

fp  or  fp1  and / or  fp2

fq     (for distributed element filters only)

Prototype Degree N

In the first option, the following parameters are typed in:

PB Ripple : Passband Ripple (3 dB is default for Butterworth and can’t be changed)

fp                : Passband edge frequency for LP and HP filters.

fp1 , fp2   : Passband edge frequencies for BP and BS filters.

SB Edge Loss: Insertion loss at the edge of stopband.

fs              : Stopband edge frequency for LP and HP filters.

fs1 , fs2  : Stopband edge frequencies for BP and BS filters. Only fs2  is typed in because fs1   is  	                set through the relation  fs1 fs2 = fp1 fp2 = fo2  where fo  is the (geometric) center of 		                passband for bandpass filters and center of stopband for bandstop filters.

fq          :    Quarter wavelength frequency corresponding to commensurate stub and transmission           
                            line length (for distributed element filters only).

In the second option, only passband ripple, passband edge frequency(ies) and quarter wavelength frequency (for distributed element filters only) are typed in together with desired prototype degree (N). Then these parameters can be readjusted after inspecting the response of filter.

For Generalized Chebyshev filters, the following parameters can be chosen:

Passband Ripple

fp      :  Passband edge frequency (ies)

SB Edge Loss  : Minimum stopband insertion loss

fq      : Quarter wavelength frequency (for distributed element filters only)

N  : Prototype degree (odd)

- Find Circuit: When this command is invoked, the proper filter circuit is formed automatically by Filpro and displayed at the bottom half of the screen, with a movable cursor used to point elements as shown in Figure 1.7 where the circuit of a Degree N=7, Lumped Element, Lowpass, Chebyshev filter with specifications fp=100 MHz, Passband Ripple=0.05 dB, RS=RL=50 ohms is displayed. The circuit cursor can be moved to right or left using the key combination Ctrl + Right/Left arrows. The cursor is coupled to a box displayed at right bottom corner of the screen. The following information is displayed in this box related to the pointed element:

- Values of parameters of the pointed element.

- A number indicating the relative position of element in the circuit (counted by starting from source resistance RS which is numbered as zero).

- Units of inductors and capacitors used in the design (Units can be set using Change Units command of Options menu.)

Figure 1.7. Circuit schematics and information windows after Find Circuit command.

If the number of elements in the circuit to be displayed is larger than about 25-30 (depending on the sizes of element schematic drawings) then only the source side elements are displayed, with some elements on the load side remain hidden.

In order to view hidden elements, the whole circuit can be moved to left using the key Page Down. The key Page Up moves the circuit right. Exercise with these keys to inspect how the circuit moves.

The key combinations  Ctrl + Page Down and Ctrl + Page Up moves the whole circuit to leftmost and rightmost positions so that either the load side or source side part of the circuit is displayed respectively.

- New Trial:  After inspecting the response of filter (see the Plot menu below), if it is necessary to change some parameters and redesign filter, than New Trial command is used.

- Quit Design : Design stage of filter using prototypes is finished by that command, enabling (activating) all other menus used for various circuit transformations. Warning: Transformations menu will NOT be enabled without quitting the design menu!.

PLOT Menu

Before invoking New Trial or Quit Design commands of Design menu, it is natural to inspect the response. This is possible without quitting the design menu. Select Plot menu (See also Section 1.8  for more of the Plot Menu). The following responses can be displayed (Figure 1.8):

- Insertion Loss      (Short cut key: Shift+F1)

- Return Loss          (Short cut key: Shift+F2)

- Insertion Phase    (Short cut key: Shift+F3)

- Group Delay        (Short cut key: Shift+F4)

- Smith Chart         (Short cut key: Shift+F6)

Figure 1.8. Plot menu and scaling for insertion loss.

Figure 1.9. Short cut keys used inside plotting mode and insertion loss response in normal and zoomed modes.

These five different responses can be viewed by executing the relevant commands.

- After a response plot type is selected the scales are automatically displayed for typing in maximum and minimum values of vertical and horizontal (frequency) axis and number of frequency steps.

- All responses are plotted in overplot mode. The vertical axis scaling is valid only for the last plot. If the screen bocomes too crowded, it can be refreshed by pressing the key F8. This command will refresh and display only the last plot.

- The most recent plot is displayed as Yellow. Older plots change color as newer graphs are overplotted.

- Default number of frequency steps is 100. It can be increased to 200.

- A Plot Marker is available which can be moved over the response curve by Right-Left arrows (slow motion) or by Ctrl+Right, Ctrl+Left (fast motion) keys. The coordinates of marker are displayed in a small box at the left bottom corner of the screen.

- A Delta Marker can be placed at any point on the response curve by pressing Space Bar. Differences between readings of Plot Marker and Delta Marker are also displayed in the small box at the left bottom corner.

The following features are available adjustments of plots:

- F10:	to replot the response with a new scaling.

- F8: 	to refresh the screen and replot the current response.

- F9: 	to zoom the response between the Plot Marker and Delta Marker.

- F7: 	to make marker position as frequency span center and replot.

- Insert: 	Span up.

- Delete: 	Span down.

- Page up: 	Reference up.

- Page Down: 	Reference down.

- Home: 	Vertical Scale up.

- End: 	Vertical Scale down.

Reference Lines:

This command can be used to place brick walls into passband and stopbands of response both for easing visualization of response and to be used as criteria for Monte Carlo analysis and related operations like Worst Case, Best Case and Yield analysis, as will be described later. For the example at hand, a reference line is placed into return loss response only, at the return loss level corresponding to the specified passband ripple of 0.05 dB, using the command set Reference Lines / Auto Add Reference Lines / Return Loss Only / .. (Figure 1.10). The reference line level can be adjusted using the command Reference Lines / Edit Reference Line / .. .

Figure 1.10. Responses with a return loss reference wall.

Application examples using Built-in LC Prototype approach will be given in the succeeding chapters, especially in Chapters 3, 4, 5, 6, 8 and 9.

1. 4. 2. FILTER DESIGN BY SYNTHESIS

Design / By Synthesis menu activates a powerful lumped and distributed filter synthesis program, based on the classical transformed frequency domain techniques [11], [19], [35] further augmented

- to improve accuracy,

- to include contributing unit elements for distributed element filters,

- to design singly or doubly terminated filters.

In this approach, lumped or distributed, lowpass, highpass and bandpass filters can be synthesized with either equiripple or maximally flat passbands, with or without finite transmission zeros and with or without contributing unit elements. (See Chapter 2 for synthesis of lumped element filters and Chapter 7 for synthesis of distributed element filters). This is the most flexible design tool of Filpro. All the other design tools are covered by this approach as special cases. It does not have the limitations of other approaches. However Synthesis approach can’t design bandstop filters directly. Bandstop filters are normally designed using Built in LC prototype approach. Synthesis approach can be used to design special lowpass prototypes which are not available in Built in LC Prototype approach and then these lowpass prototypes can be used in Design By User Defined Prototype approach to design the desired bandstop (and other) filters.

1. 4. 2. 1. Filter Specifications

The command list shown in Figure 1.11 appears when Design / By Synthesis option is invoked:

Figure 1.11. Design By Synthesis commands.

### New / Design / By Synthesis

### Filter Type:

### Lumped-Distributed: Select one of the lumped element (LC) filter or commensurate length distributed element filter options by Left-Right arrows.

### LP- HP- BP: Select passband type.

### Max. Flat - Equiripple: Select passband approximation type.

### Doubly Terminated- Singly Terminated: Select Doubly terminated option in the usual filters. Singly Terminated filters may be needed only in some multiplexer types.

### Type A-Type B: Type A and Type B choices produce dual circuits. Type A circuit starts with a series element while Type B starts with shunt type element on the source side. This selection may not be important, as the dual of the circuit can always be formed by using Find Dual command of Filpro, after the design stage.

- PB and Terminations: After choosing the proper filter type, the following filter specifications are asked.

- PB Ripple / PB Edge Loss: For equiripple passband filters, the desired pasband insertion loss ripple must be specified. For maximally flat passband filters, the desired insertion loss at the passband edge frequencies (termed as passband edge loss) should be specified.

- fp1: The lower passband edge frequency (in bandpass filters)

- fp2: Upper passband edge frequency (in bandpass filters).

- fp : Passband edge frequency in lowpass and highpass filters.

- fq: In distributed element cases filters are formed by short circuited stubs, open circuited stubs and transmission lines (Unit Elements) of all identical lengths termed as commensurate length. This length is set by the frequency fq which is termed as quarter wavelength frequency. The stubs and transmission lines become quarter wavelength long at this frequency (90 degrees of electrical length at this frequency). fq is asked only if the filter is of distributed element type.

Choose passband edge frequencies as fp1=40 MHz, fp2=60 MHz, specify  PB Edge Loss=0.1 dB, and terminations RS=RL=50 ohms.

Next, transmission zeros (poles) will be entered (Figure 1.12):

Specify TZs  (Transmission Zeros):

Transmission zeros are critical frequencies where signal transmission between input and output is stopped. The transmission zero frequencies are used (together with the passband edge frequencies and passband ripple) to form the transfer function between input and output of the filter, for shaping response of the filter.

- Transmission zeros must be placed in stopbands of the filters. Placement of transmission zeros into passbands is prevented by Filpro.

- When Specify TZs command is invoked, usually the first step is to clear the transmission zeros of the previous design. This should be done by invoking the command Delete TZs / Delete All TZs. Then we start to place the new transmission zeros.

- Shaping up of insertion loss and return loss responses can be viewed from the small inset plot at the top right corner. In order to view proper frequency ranges, Plot Response command should be invoked in which case only insertion loss response is plotted on the whole screen with user specified frequency limits. Then pressing Esc key, we can return to the original display. Now we are ready to place the necessary transmission zeros.

- The displayed response is calculated from the transfer function formed by the transmission zeros, not the response of actual filter yet. The response of the actually realized circuit should match the response calculated, if it can be synthesized accurately.

The following transmission zero types are available:

TZ at Zero:

When invoked, this command places a transmission zero at f=0, adding one degree to the transfer function of the filter circuit. These transmission zeros are needed in highpass and bandpass filters. Increasing the number of transmission zeros at f=0 increases slope (selectivity) of the filter in lower stopband more compared to upper stopband slope. In realization stage, for lumped element filters, each transmission zero at f=0 produces a series capacitor or a shunt inductor. In distributed element filters, the resulting elements will be either series open circuited stubs or shunt SC stubs.

Place 3 TZs at f=0 (Nzero=3) by invoking the item TZ at Zero three times.

TZ at Infinity:

When invoked, places a transmission zero at f=, adding one degree to the filter circuit. These transmission zeros are needed in lowpass and bandpass filters. Increasing the number of transmission zeros at f= increases the slope of upper stopband more compared to the lower stopband slope. In realization stage, for lumped element filters, each transmission zero at f= produces a series inductor or shunt capacitor.

For distributed element filters, this command places a transmission zero at the quarter wavelength frequency f=fq  which is infinity in the transformed frequency domain (Richards’ frequency domain-See Chapter-7). That is, the term TZ at infinity doesn’t actually mean f=. In s=+jw frequency domain, it places a Finite Transmission Zero at s=jwq.

For distributed element filters, the resulting elements will be series short circuited stubs or shunt open circuited stubs. Invoke this command seven times to place seven transmission zero at f=.

TZ at Finite Frequency (FTZ):

This command places a finite nonzero transmission zero at the specified frequency, on s=jw axis, as pairs s=  jwz =  2 fz where fz is the specified frequency. Each finite transmission zero adds two degrees to the filter circuit. In lumped element filters these transmission zeros are realized either as series arm parallel LC resonators or shunt arm series LC resonators. The series arm parallel resonators stop signal flow by being open circuit at the frequency of transmission zero while the shunt arm series LC elements become short circuit at the frequency of transmission zero. The corresponding elements in distributed element filters are paralleled OC and SC stubs in series arms or series combination of OC and SC stubs in shunt arms.

- Let’s place two FTZs at fz1=34 MHz and f z2=66MHz.

UE (Unit Element):

This command is used to insert a contributing Unit Element in distributed element filters only (See Chapter-7). Unit Element is a transmission line segment with length equal to the length of stubs, set by the quarter wavelength frequency selected by the designer. This element adds also a single degree to the filter.

Figure 1.12. Placement of transmission zeros and shaping up of response by adding transmission zeros.

While the transmission zeros are entered or deleted, the insertion loss and return loss at the inset plot are refreshed automatically. Hence, effects of multiplicities and positions of transmission zeros on response can be judged instantly, speeding up response shaping stage. The multiplicities and positions  of transmission zeros and resulting degree of the filter are also displayed in small boxes under the response curve. Response can only be drawn if numbers of transmission zeros at zero and /or infinity are proper for the type of filter  (LP, BP, HP) to be designed. For bandpass filters it degree of filter must be even. The numbers and frequencies of transmission zeros are also displayed at the inset under the response plot.

Tune  FTZ : Using this command, it is possible to adjust the frequencies of finite transmission zeros while inspecting the response. When this command is activated, first scaling parameters of insertion loss response are displayed for adjustments and then the finite transmission zero to be tuned and increment for tuning frequency are asked. After these information are typed in, the response is ready for adjustment by increasing or decreasing the frequency of finite transmission zero using Up-Down arrows. The tuned FTZ Frequency is displayed at the right bottom of the screen. When tuning of the FTZ is finished, the key F2 is pressed to save the last frequency.

Delete TZs :

In shaping the response curve, it may be necessary to delete some transmission zeros. Select the transmission zero to be deleted and press Enter. When starting a new design, it may be necessary to clear the transmission zeros remaining from the previous design. In such cases Delete / All TZs command can be used.

Plot Response:

If detailed response is to be observed, then Plot Response command can be used. This is usually needed in adjustment of frequency range of inset plot. Cursor can be moved by Left-Right arrows or Ctrl+Left-Right arrows to display the frequency and insertion loss at the position of cursor on the response plot.

Extract Transmission zeros:

After all the specifications are completed Element Extraction stage begins by invoking Extract TZ command bringing in a new menu. This stage will be described in the next subsection.

Restart Synthesis:

If some of the specifications need be changed, then Restart Synthesis command is invoked.

1. 4. 2. 2. Element Extraction

After forming the desired response, we are now ready to realize the circuit. Realization is accomplished by extracting all transmission zeros placed in the previous stage as actual circuit elements.

Before element extraction, the circuit appears as a black box on display. Element extraction from this black box starts by invoking the command Extract TZs which displays a new menu, as shown in Figure 1.13. In this menu there are several transmission zero extraction modes which may be useful in different occasions.

Figure 1.13. Element (TZ) extraction menu.

Manual Extraction:

In this mode, user just invokes the transmission zeros that are placed in the previous stage, one by one, in any order:

- TZ at Zero: When invoked, one of the transmission zeros at f=0 is extracted either as a series capacitor or a shunt inductor in lumped element filters and either as a series arm OC stub or shunt arm SC stub in distributed element filters. The extracted element is displayed on the left hand side of the remaining (new, modified) black box.

- TZ at Infinity: When invoked, one of the transmission zeros at f= is extracted either as a series inductor or a shunt capacitor in lumped element filters and either as a series arm SC stub or shunt arm OC stub in distributed element filters. The extracted element is displayed on the left hand side of the remaining black box.

- TZ at Finite Frequency: When invoked, one of the finite transmission zeros (selected by the user) is extracted either as a series arm parallel LC section or shunt arm series LC section in lumped element filters, and either series arm paralleled OC and SC stubs or series combination of SC and OC stubs in shunt arm in distributed element filters. These second order sections are accompanied by elements on their left which represent partial extraction of transmission zeros at f=0 (for FTZs in lower stopband) or at f= (for FTZs in upper stopband). These partial extractions are necessary for the creation of FTZs at the desired frequencies.

- UE  (Unit Element): When invoked, a transmission line piece (termed as unit element) is extracted from the black box. The transmission line piece has the same length as the OC and SC stubs of the distributed element filter.

The transmission zeros that are available for extraction are shown in enabled (highlighted) form. If finite transmission zeros exist in lower stopband then the transmission zeros at f=0 can’t be finished before extracting all the finite transmission zeros in lower stopband. If there are finite transmission zeros in upper stopband, then the transmission zeros at f= can’t be finished before extracting all the finite transmission zeros in upper stopband. The transmission zeros which can’t be extracted (either because they are finished or it is forbidden) are shown as white items.

In manual extraction mode, as each transmission zero section is extracted, the circuit schematics in the lower half of the screen is updated showing the extracted elements. Their parameters and values are displayed in the box at the right bottom corner. Hence, the user may terminate the procedure by just inspecting the extracted element values and restart extraction by changing the order of elements or restart synthesis by changing some design parameters. In Figure 1.14, the initial stages of element extraction process for the filter defined so far are shown.

Undo Last Extract:

The transmission zeros can be extracted in any desired order to produce many alternative equivalent designs. In case of a desire to change the order of extraction, it is possible to Undo the previous extractions using Undo Last Extract command.

Exercise by extracting the transmission zeros placed in previous stages in different orders to see the alternative equivalent realizations.

Filpro learns the order of manual extraction carried out for use in the succeeding extraction trials, as described in the following command.

Macro Auto Extract:

This mode is enabled (highlighted) only after a manual extraction stage where Filpro learned the order of extraction. When invoked, the transmission zeros are extracted automatically, in the same order as in the previous manual extraction stage. In this mode the whole circuit is instantly displayed. This command is a very useful short cut in applications where many synthesis trials have to be performed by changing some parameters (like for example ripple, FTZ) while keeping the number, type and order of extraction of transmission zeros the same.

Standard Auto Extract:

For some commonly used transmission zero patterns, one of the commonly preferred element extraction orders is programmed as built-in order of extraction. If the transmission zeros entered by the user fit to a (built-in) standard one, then this item is enabled (highlighted). Invoking this command yields the circuit automatically.

Tune FTZ and Auto Extract:

This mode enables the user to inspect changes in the extracted element values when a finite transmission zero is adjusted. After entering a new value of finite transmission zero, Macro Auto Extract  command is activated automatically, yielding the circuit for inspection.

Restart Extract:

This command enables to restart element extraction procedure, for example, to inspect the circuits resulting from different element extraction orders.

Restart Specify TZs: Transmission zeros can be modified with this command. This command is also used to go back to Restart Synthesis stage.

Figure 1.14. Element extraction stages of a filter.

A large number of application examples will be given in the succeeding chapters. However, just as  demonstrations of some commonly used commands of Filpro that will be described in the following sections, let’s work out some examples which are familiar to most designers.

## Example 1. 1. Lumped Element Direct coupled resonator filters

Direct coupled resonator filters form the most widely used filter class. They are formed by coupling shunt or series resonators by inductors or capacitors. Resonators may be of series or parallel LC type or single short circuit (SC) stub, open circuit (OC) stub, OC+SC Stub in parallel or in series, SC Stub+lumped Capacitor type, cavity type, dielectric resonator type, etc. (see Chapter-6 and Chapter-11 for such filters). Let’s design some lumped element Chebyshev bandpass filters in direct coupled shunt resonator form with following specifications:

Passband edges: fp1=780 MHz, fp2=820 MHz, Passband Ripple=0.05 dB

Let’s assume that we are forced to use only five resonators. Each resonator provides two degrees. Thus filters will be of degree ten. That is ten transmission zeros will be placed during the design stage. Chebyshev filters require transmission zeros only both at f=0 and f=. The transmission zeros can be shared between f=0 and f= in different ways leading to different alternatives having the same passband properties but different stopband performances (different slopes). Some alternatives are as follows:

a) Nzero=1, Ninf=9 (Inductively coupled resonator filter)

Placing one transmission zero at f=0 and seven at f= leads to sharper slope (higher selectivity) in the upper stopband. The command sets in the Design stage are as follows:

## New / Design: By Synthesis / Filter Type: Lumped, Bandpass, Equiripple, Doubly Terminated, Type B

PB and Terminations: PB Ripple=0.05 dB, fp1=780 MHz, fp2=820 MHz, RS=50, RL=50

Specify TZs: Delete All TZs / Zero, Inf, Inf, Inf, Inf, Inf, Inf, Inf: (Nzero=1, Ninf=7)

Plot Response: fmin=750, fmax=850, ILmin=0, ILmax=100, No. of Freq. Steps=200

Extract TZs: Inf-Inf-Inf-Inf-Inf-Zero-Inf-Inf-Inf-Inf  (Extraction order is not important!)

## Quit Synthesis

- Response is shown in Figure 1.15.a.

- The synthesized filter circuit is shown in Figure 1.15.b. Note that element values are abnormally high or low. Note also the appearance of a transformer at the load end. Also topology of the circuit is not in coupled resonator form. In fact no resonator exists in this circuit. These are among the classical problems which form headaches for filter designers. A variety of tedious circuit transformations are needed to reach targeted structures with realizable element values. At this stage one of the main contributions of Filpro comes into picture. Complicated circuit transformations are carried out on Filpro with simple commands. These commands will be described in the rest of this chapter.  In order to just give an idea on the nature of such circuit transformations let’s apply some of them on our example.

- Using Left-Right arrows, move to the menu Transformations-3 and select the item Pi-Tee-L and then select the option All Equal Shunt / Inductors (and SC Stubs). Invoking this command, Filpro starts an iterative procedure to convert the synthesized circuit into coupled shunt LC resonators form, as shown in Figure 1.15.b-c. Note that all element values are now reasonable though still unrealizable yet. All shunt inductors are equal. Note also that in this circuit all the coupling elements are inductive. This is because we have placed nine transmission zeros at f=, and one at f=0. If more transmission zeros were placed at f=0, then we would get some capacitive coupling elements.

- Another point to note is that the transformer at load end is now almost unity. It can be deleted by moving to Edit menu and selecting Delete command (Figure 1.15.c), or simply by pressing the Delete key of the keyboard.

- Thus the targeted structure is reached. However element values are still unrealizable. For example let’s target to make all shunt inductors 10 nH. This is possible by moving to the menu Transformations-3 and invoking Inverters item. Then a list of commands will be displayed as shown in Figure 1.15.c. The command Scale Circuit By Inserting Inverters, as its name tells, scales all element values up or down by inserting ideal inverters at both source and load ends. This command brings some further options, like Specify Inverter Constant to be placed at ports or Specify Inductance of the resonator adjacent to source or Specify Capacitance of the resonator adjacent to source.  Let’s select Specify Inductor option. This will bring a window to type in the desired inductance. After typing in 10 nH, and pressing Enter, Filpro will calculate the necessary inverter constants, scale all the element values accordingly and display the modified circuit as shown in Figure 1.15.d.

- The next step will be replacement of ideal inverters by their practically realizable equivalents. One of the many options may be as follows: Move the cursor to point one of the inverters and apply the command set Inverters / Replace Ideal Inverter By / End Inverter to / Capacitive Pi or L / L-Section: f=800 MHz. The ideal inverter will be replaced by its capacitive L-Section equivalent with negative shunt capacitor. Equivalence is exact only at a single frequency that will be typed in by the designer. This frequency is usually selected as passband center. Equivalence is approximately valid over narrow-medium bandwidths. Repeating this command on both inverters, we get the circuit shown in Figure 1.15.e.

- The negative shunt capacitors can be absorbed into their positive neighbors by applying either Edit / Combine command (with cursor under the left element to be combined) or by applying Edit / Simplify Circuit command giving the targeted circuit shown in Figure 1.15.f.

- Insertion and Return Loss responses are shown in Figure 1.15.g. Distortion in return loss is due to replacement of ideal inverters by L-section equivalents. These distortions can be corrected to a certain extent by tuning element values of the filter.

- Effect of finite Q of shunt resonators can be inspected by applying the command Parasitics/All Lossy Resonators / f0=800 MHz, Q=1000. Here we specify the Q of resonators at passband center (Figure 1.15.h-i). Passband insertion loss will increase while the pasband edges are rounded as seen from the plot.

- If shunt inductors are to be realized as distributed element form, for example as SC stubs, then we can transform the shunt LC resonators into shunt SC Stub+Capacitor forms with user specified length for stubs as follows: Place the cursor under an LC resonator. Move to the menu: Transform-3 and invoke the command sets Replace By Stub /Parallel LC to Capacitor+SC Stub: fq=1500 MHz, f1=780 MHz, f2=820 MHz, Er=1. Here fq is the quarter wavelength frequency of the stubs, f1 and f2 are selected as passband edge frequencies and Er is the dielectric constant of stub. This transformation replaces the LC resonator with a SC Stub in parallel with slightly modified capacitor (Figure 1.15.m-n). By specifying f1 and f2 as passband edges we avoid shifts in these edges because of the approximate nature of transformation.

- Coupling coefficients and External Q’s can be displayed as a list using the keys Ctrl+F6. Coupling coefficients between two resonators and resonance frequencies of  the coupled resoantors can be displayed by placing the cursor under the series coupling element and pressing Alt+F6. Node voltage, current and real power parameters can be displayed by pressing the keys Alt+F8 (See Help: Calculators and Section 1.11), as shown in Figure 1.15.n, for the lumped element filter.

- More circuit transformations will be described in this and forthcoming chapters.

Figure 1.15. Design steps of an inductively coupled resonator filter

Figure 1.15. Design steps of an inductively coupled resonator filter (continued).

Figure 1.15. Design steps of an inductively coupled resonator filter (continued).

Figure 1.15. Design steps of an inductively coupled resonator filter (continued).

b) Nzero=5, Ninf=5

Placing equal number of transmission zeros at f=0 and f= yields a relatively more symmetric response compared to Nzero=1, Ninf=9 case. The command sets for the design are the same as previous case, except the numbers of transmission zeros. The transmission zeros may be extrcated in different orders.

- Let’s try the order Zero,Zero,Inf,Inf,Zero,Zero,Inf,Inf,Zero,Inf  giving the circuit of Figure 1.16.a.

- Application of Pi-Tee-L / All Equal Shunt / Inductors command converts the circuit into coupled resonators form as shown in Figure 1.16.b. Note that the coupling elements are inductive and capacitive alternately.

- Element value adjustment steps are similar to the previous filter.

- As exercise, try extracting transmission zeros in different orders to see other alternative topologies.

Figure 1.16. Coupled resonator filter with Nzero=5, Ninf=5.

c) Nzero=9, Ninf=1

In this case we will get slightly higher slope in upper stopband and the coupling elements will all be capacitive, as shown in Figure 1.17.a-b. Here, transmission zero extraction order is not important, as all orders will yield the same circuit..

Figure 1.17. Coupled resonator filter with Nzero=9, Ninf=1.

## Example 1. 2 Distributed Element Coupled Resonator Filters

The approach used in the previous coupled resonator filters can also be applied to design All-Stub or mixed Lumped+Distributed element filters. Let’s design the filter previous example by starting with distributed element approach with specifications:

PB Ripple=0.05 MHz, fp1=780 MHz, fp2=820 MHz, fq=1500 MHz  to be realized in Combline form (See Chapter-11).

Here, fq is the quarter wavelength frequency which sets the commensurate length of stubs. It should be selected by the user. fq=1500 MHz choice will give 50 mm stub length.

Combline filters require Nzero=1 always, thus we have to set Ninf=9 to form five resonators.

- Design command sets are shown in Figure 1.18.a.

- Standard Auto Extract method is used (because Filpro knows this pattern for element extraction resulting in the circuit shown in Figure 1.18.b.

- The command Pi-Tee-L / All Equal Shunt / SC Stubs is applied to convert the circuit into coupled resonator form with equal shunt SC Stubs (Figure 1.18.c). The shunt resonators are formed as parallel combinations of SC and OC stubs. Couplings between resonators are in the form of series arm SC subs.

- We can continue design with OC stubs, however in practice it is usually preferred to replace the OC stubs by lumped capacitors. In Filpro this is possible by placing the cursor under the OC+SC Stub elements, moving to menu Transformations-3 and selecting the command Replace By Lumped / SC Stub+Cap (or in a shorter way, All SC Stub+Cap) / f1=780 MHz, f2=820 MHz. The OC+SC stub elements will be replaced by new SC stubs in parallel with lumped capacitors (Figure 1.18.c-d). This is an approximation which keeps passband edges unaffected. There will be some slight distortions in passband return loss.

- This transformation destroys equality of shunt SC stubs. This property can be recovered by applying the command Pi-Tee-L / All Equal Shunt SC Stubs again (with cursor under a series SC stub) (Figure 1.18.d-e).

- The shunt SC Stub impedances are too low for realization. They can be adjusted to any desired value, for example 60 ohms, by applying the command Inverters / Scale Circuit By Inserting Inverters / Specify Adjacent SC Stub: Zsc=60 ohms (Figure 1.18.f).

- The end inverters can be replaced by proper realizable equivalents, for example, by capacitive L-sections, as is done in the previous example (Figure 1.18.g).

- If desired we can convert the resulting circuit into all lumped element form (for example to compare with the element values with those of lumped element design of previous example) by applying Replace By Lumped  command (Figure 1.18.h).

- External Q, Coupling Coefficients and nodal parameters (V-I-P) can be displayed using the keys Ctrl+F6 and Alt+F8 respectively, as was done in the previous example.

- Effects of finite Q of shunt resonators can only be inspected from the all-lumped model, using Parasitics / All Lossy Resonators command.

More circuit transformations that can be applied to such circuits will be described in the following subsections of this chapter, and more examples will be given in Chapter 11 and in other related chapters.

Figure 1.18. Design steps of a coupled resonator distributed element filter.

Figure 1.18. Design steps of a coupled resonator distributed element filter (continued).

## Example 1. 3. Lumped Element Cascaded Triplet and Quadruplet filter

Cascaded Triplet (CT) and Cascaded Quadruplet filters are special simpler forms of Canonical Cross-Coupled filters. These classes of filters are described in Chapters-24, 25 and 26. This example is just intended to show the features of Filpro for shaping response of filters by placing finite transmission zeros while also give an idea on some circuit transformations that will be described later in detail.

Let’s design the filter of previous examples with more severe constraints on selectivities in both stopbands.

Passband specifications are the same:

PB Ripple=0.05 MHz, fp1=780 MHz, fp2=820 MHz,

We are allowed to use eight resonators. Thus, filter degree will be N=16.

In lower stopband, we are asked to place two finite transmission zeros (FTZ), one at fz1=770 MHz and one at fz2=775 MHz (for example to reject some interferences at these frequencies).

In upper stopband another finite transmission zero is required at  fz3=825 MHz.

The three finite transmission zeros provide six degrees. Thus, the remaining 10 degree will be provided by transmission zeros at f=0 and f=. Let’s select Nzero=1 and Ninf=9.

- Filter specifications are typed in using the design command sets

- New / Design / By Synthesis

- Filter Type: Lumped-BP-Equiripple-Doubly Terminated-Type B

- PB And Terminatios: PB Ripple=0.05 dB, fp1=780 MHz, fp2=820 MHz, RS=RL=50 ohms.

- Specify TZs: Zero - Inf - Inf-…-Inf - FTZ1= 770 MHz – FTZ= 775 MHz – FTZ= 825 MHz

Response is shown in Figure 1.19.a. The finite transmission zeros can be tuned using Tune FTZ command to see how stopband losses change with position of FTZs.

- Transmission zeros are extracted using the following order (see Chapters-24, 25, 26 to see other possible transmission zero extraction orders:

### Inf – Inf - FTZ1- Inf - Inf - Inf - Inf- FTZ2 - FTZ3 - Zero - Inf- Inf – Inf

The resulting circuit is shown in Figure 1.19.b.

- Quit Synthesis command is applied to enable the circuit transformations.

- The command Pi-Tee-L / All Equal Shunt / SC Stubs is applied to convert the circuit into coupled resonator form with equal shunt SC Stubs (Figure 1.19.c). The shunt resonators are formed as parallel LC sections. Couplings between resonators are in the form of either series inductors or series arm parallel LC sections which form the finite transmission zeros by becoming open circuit at the frequencies of finite transmission zeros.

- The first three resonators on left can be used to form a mutually coupled triplet by bridging Node-1 and Node-3 with a capacitor or inductor.  The resulting element is termed as CT element. This operation replaces the LC type coupling between Node-2 and Node-3 with a single element (L or C) coupling. So, the finite transmission zero will be realized by the feedback element between Node-3 and Node-1. Conversion of such circuit sections into such CT elements is carried out in Filpro by placing the cursor under the series arm LC element and applying the command CQ-CT Oprs / Convert to CT. In this conversion we get the options of capacitive or inductive bridge coupling. We could as well  place the bridge coupling between Node-2 and Node-4. The resulting element as shown in Figure 1.19.d. See Chapter-24 for definitions related to CT elements. Note that Node-2 disappeared after conversion into CT form. Actually Node-2 is now hidden in the CT element. CT elements are named as L/LL, C/LL, L/CC, etc. In this notation, the first letter denotes the type of bridge coupling (Nodes-1 and3), the second and third letters denote coupling types between Nodes 1-2 and 2-3 respectively. In the choice of  C/LL, we have selected capacitive bridge coupling. CT elements realize a single transmission zero. The first finite transmission zero on left was the one at f=770 MHz. So, the CT section realized this finite transmission zero.

- The second and third finite transmission zero sections are on the right side of the circuit. These two sections can be converted into a cross-coupled quadruplet of resonators as follows:

- With cursor under the shunt resonator on the left of FTZ2 section (Node-5), applying the command Double Norton / Inverse Geffe converts the circuit such that the two series arm parallel LC sections become adjacent as shown in Figure 1.19.d.

- Simplify Circuit command clears the internal transformers by collecting them on the load end while also combining the adjacent shunt resonators at Node-7 (Figure 1.19.e).

- Then with cursor under the left series arm parallel resonator, applying Combine command the two series arm parallel LC resonators are combined to form a single fourth order section, as shown in Figure 1.19.f. In this circuit Node-7 disappeared.

- With cursor under the fourth order element, applying the command CQ-CT Oprs / Convert to CQ / Right converts the circuit section between Node-5 and Node-8 into a cross-coupled quadruplet of resonators, termed as CQ sections (Figure 1.19.g). CQ sections have four nodes, numbered as 1-2-3-4, starting from left upper node and moving counter clockwise. Node-5 and Node-8 of Figure 1.19.g  correspond to Nodes 1 and 4 of the CQ section. Node-2 and Node-3 of CQ section correspond to Nodes 6 and 7 of Figure 1.19.g, and they are hidden in the CQ section. These nodes have also parallel LC resonators to ground. See Chapters 25 and 26 for detailed circuits and definitions of CQ sections. Cross-couplings are introduced between Nodes 1-4 and Nodes 1-3 or Nodes 2-4, depending on the circuit topology. All direct and cross-coupling elements are in ideal inverter form.

- The transformer resulting from conversion into CQ is shifted to rightmost position by applying Simplify Circuit command (Figure 1.19.h).

- These transformation destroy equality of shunt inductors. If desired, inductors of all shunt resonators can be equated using the following procedure:

- Applying the command Pi-Tee-L / All Equal Shunt / Inductors, the shunt inductors of resonators nodes 3-4-5 are equated. (Figure 1.19.h-g).

- Inductors of resonators inside CT and on the left of CT section can be equated by applying the command CQ-CT Oprs / Modify CT / Set L2, Set L1, Set L3, Set L2,… iteratively. In a few iteration, inductors of all three resonators of CT section become equal. However, in this case equality of shunt inductors at the inner nodes 3-4-5 is destroyed. Applying Pi-Tee-L / All Equal Shunt / Inductors command again restores the equality.

- Finally, the inductors of resonators of CQ section are equated, now by using the command CQ-CT Oprs / Modify CQ / Set L1-L2-L3-L4 (Figure 1.19.i)..

- The shunt resonator impedances are too low for realization. They can be adjusted to any desired value, for example 10 nH, by applying the command Inverters / Scale Circuit By Inserting Inverters / Specify Adjacent Inductor: L=10 nH (Figure 1.19.k).

- The transformer at load end can be absorbed into the inverter using the command Inverters / Absorb Trf By Inverter (Figure 1.19.m).

- The end inverters can be replaced by proper realizable equivalents, for example, by capacitive L-sections, as is done in the previous examples (Figure 1.19.n).

- Inverters of the CQ section are replaced by their inductive or capacitive Pi-section equivalents by applying the command CQ-CT Oprs / CQ Inv-to-CQLC (Figure 1.19.n).

- External Q, Coupling Coefficients , resonance frequencies and nodal parameters (V-I-P) can be displayed using the keys Ctrl+F6, Alt+F6 and Alt+F8 respectively, as was done in the previous examples.

More detailed examples will be given in Chapter 24, 25 and 26 on CT and CQ type filters.

Figure 1.19. Design steps of a Cascaded Triplet and Quadruplet filter.

Figure 1.19. Design steps of a Cascaded Triplet and Quadruplet filter (continued).

Figure 1.19. Design steps of a Cascaded Triplet and Quadruplet filter (continued).

1. 4. 3. FILTER DESIGN BY PROTOTYPES WITH INVERTERS

In some applications it may be easier to design filters from lowpass prototypes with ideal inverters. This approach yields always symmetric filters independently of degree of the filters.

Two lowpass prototypes are programmed into Filpro. One with series inductors separated by ideal inverters and the other shunt capacitors separated by ideal inverters. Let’s describe the use of this approach on an example.

## Example 1. 4. Direct coupled resonator filter design using Prototype with Inverters approach

Let’s design a five resonator filter bandpass with specifications

Passband Ripple=0.05 dB, fp1=780 MHz, fp2=820 MHz

The prototype will have degree 5 so that the bandpass version will have degree 10 (five resonators).

Filter design using this approach has three stages:

- First, the prototype with shunt capacitors and inverters is selected by specifying desired passband ripple and prototype degree using the command sets shown in Figure 1.20, using the command set:

### New / Design / By Prototype with Inverters / Choose a Standard Circuit: Shunt Caps+Inverters /

Prototype Degree=5, PB Ripple=0.05

The prototype circuit will be displayed on the screen, as shown in Figure 1.20.a. The circuit of Figure 1.20.b would result if the option Series Inductors+Inverters were selected. The prototype circuits will always have unity terminations.

- Then desired filter type, passband edge frequencies and actual terminations are typed in through the commands

### Filter Type: Lumped, BP

Passband and Terminations: fp1=780 MHz, fp2=820 MHz

Then Find Circuit command displays the designed filter consisting of shunt LC resonators separated by ideal inverters, as shown in Figure 1.20.c. The circuit of Figure 1.20.d would be obtained if the prototype with series inductors were used. In this case the series inductors of lowpass prototype will be converted into series LC resonators. In this stage the classical reactance transformations are applied on the prototype to design the desired LP, HP, BP and BS filters in lumped or distributed element form. Reactance transformations keep ideal inverters unchanged.

- Finally the ideal inverters are replaced by proper practical equivalent circuit sections using the command Replace Ideal Inverter By / …. There are many practical inverter circuits. One example is shown in Figure 1.21.a-b where the ideal inverters are replaced by their inductive Pi-section equivalents. In the Pi-sections the shunt inductors are negative, hence to be absorbed by their positive neighbors. This is done by applying Simplify Circuit command giving the circuit of Figure 1.21.c. We could use capacitive Pi-section equivalents to get capacitively coupled resonator circuits. Responses show slight degradations because of the approximate nature of practical inverters. Scaling of element values can be carried out using the approach described in the previous examples.

More examples will be desribed in later chapters.

Figure 1.20. Coupled resonator filter design using Prototype with Inverters approach.

Figure 1.20. Coupled resonator filter design using Prototype with Inverters approach (continued).

Figure 1.21. Replacement of ideal inverters by inductive Pi-sections.

# 1. 4. 4. FILTER DESIGN BY USER DEFINED PROTOTYPES

This option is programmed for those who would like to use lowpass prototypes found elsewhere (handbooks, papers and other publications) or the prototypes developed by the user himself using Synthesis approach. Reactance transformations to get desired LP, HP, BF or BS filters are the same as those used in Design / By Built-in Prototype approach. Two choices exists for forming the prototype circuit (Figure 1.22.a):

Load a prototype:

A previously configured and saved prototype is loaded on which reactance transformations will be applied.

Choose a standard circuit:

When this option is chosen the following choices are listed in dialog box (Figure 1.22.b, c) :

Simple LC Ladder starting with series L

Simple LC Ladder starting with shunt C

Elliptic Type Circuit starting with series L

Elliptic Type Circuit starting with shunt C

One of these topologies can be chosen and entered. Then number of branches are typed in. The circuit is displayed with unity default values for all elements. Then, the following commands will be used to form the prototype:

Change values

The element values of circuits are changed to the values given in the actual prototype filter to be configured by this command.

Insert Element

Simple lumped elements (L, C and combinations) can be inserted into the circuit to modify the prototype       (Figure 1.22.d).

Delete Element

Some elements can be deleted from the circuit for modification purposes using that command.

After Load A Prototype command or after configuring the prototype by choosing a standard circuit and modifying it, the circuit is ready for reactance transformation that will be carried out through the following commands (Figure 1.22.e, f):

Filter Type:

Structure: Lumped- Distributed

Passband Type: LP, HP, BP or BS

Filter Specs

Passband edge frequencies, quarter wave frequency fq (for distributed elements only) and terminations are typed in.

Find Circuit

This command applies the necessary reactance transformations to the prototype and displays the desired filter circuit. Before transformation, program asks to save the prototype for cases where it may be needed again.

Quit Design

This command is necessary to enable other menus.

Example 1. 5: Use Choose a Standard Circuit command and choose Elliptic Type Circuit starting with shunt C, with N=5 elements. Then use Change Values command to type in the following prototype element values:

C1 = 0. 845, C2 = 0. 152, L2 = 1. 19, C3 = 1. 61, C4 = 0. 152, L4 = 1.19, C5 = 0. 845

Choose BP, lumped options and enter default values as frequency specifications. Invoke Find Circuit command to display the resulting bandpass filter. Inspect insertion loss response using various frequency scales.

Figure 1.22. Command sets for Design / By User Defined Prototype  approach.

1. 4. 5. CIRCUIT DESIGN BY INSERTING ELEMENTS                                                                           
            (Filpro AS A NETWORK ANALYZER)

This option of Design menu provides a versatile, easy to use, general purpose tool to configure and analyze circuits (not necessarily filters) with cascaded section topologies. When this command is invoked, a source-load network is displayed on the screen, ready for insertion of desired elements. After typing in values for RS and RL, the program passes automatically to Insert command of Edit menu. Insert command enables insertion of over hundred different lumped and distributed elements from the library of the program. The elements in the library are grouped and displayed in a dialog box as follows (see also Edit / Insert command  in section 1. 5. 3):

Lossless Lumped Elements		Lossy Lumped Elements

Transmission Line, Stubs, Resonators, Inverters

Lossy Lines and Stubs	                             Mixed Lumped+Distributed Elements

Stepped Resonators	                             Exponential Line Elements

Constant Impedances		               Parallel Coupled Lines

Rectangular Evanescent and propagating Waveguides

Attenuators, Dividers	                            CQ-CT-Bridge TEEs

With these elements (most of which are in fact subcircuits) it is believed that most of the frequently used practical filtering matching and coupling circuits can be simulated. When a group is selected, the list of available elements in that group are displayed which can be selected by right-left-up-down arrows. As arrow moves over the elements, the schematic drawings of elements are also displayed in the box. When the desired element is selected, another dialog box appears asking the values of parameters characterizing the element. These values are typed-in and Enter key inserts the element to the left of position indicated by circuit cursor. Use Help / Available Elements command to display the schematic drawings and names of all the available elements.

All the elements in library are passive. Therefore using Insert command, only passive circuits can be formed. Active two-port networks can be inserted into the circuit using the commands External Two-Ports. External Two-Port circuits can be characterized by their scattering matrix parameters measured by e.g. automatic network analyzers or calculated numerically and saved as a file ( .S2p files).

1. 4. 6. DESIGN LP-HP DIPLEXER

This is a short-cut menu for designing a special class of diplexers called singly terminated Lowpass- Highpass diplexers. See Chapter 17 for details and examples. This menu item will be enriched by addition of singly terminated parallel bandpass filter multiplexers.

1. 4. 7. DUAL PASSBAND  FILTER

This command is used to form a dual passband filter by proper frequency transformations form two different quasi-lowpass impedance matching filters (See Chapter 18.7).

1. 4. 8. SYMMETRIC CROSS COUPLED CIRCUITS

This command enables to design and/or analyze various symmetric coupled circuits through even-odd mode analysis approach (Figure 1.23):

- Filter Design with Single Cross-Coupling: Linear phase filters or filters with single finite transmission zeros on both sides of passband can be designed using this menu item (See Chapter 15).

- Filter Design with Double Cross-Coupling: Linear phase filters with one loss pole in each stopband or selective filters with two loss poles on each side of passband can be designed using this menu item (See Chapter 15).

- Analyze Cross-Coupled Filter: This is a routine for plotting responses of cross-coupled filters.  Plot menu doesn’t work for cross-coupled filters. This routine works on even and odd halves of the coupled circuit and superpose their responses to get the complete response (See Chapter 15).

- Analyze Branch Line Directional Couplers: This routine plots the four port signals of branch line directional couplers (See Chapter 20). Couplers are designed from tables available in literature and circuits are formed using even-odd mode approach. Impedance transforming and lumped element versions are available. Miniature lumped element couplers can be designed through equivalent circuit transformations.

- Analyze Parallel Coupled Line Directional Couplers: This is developed to plot port signals of four port parallel coupled line directional couplers (See Chapter 21). Couplers are designed using the tables available in literature. Miniature lumped element couplers can be designed by replacing the coupled transmission lines by their lumped equivalents.

- Analyze Rat Race Hybrids: Plots port signals of rat race hybrids (See Chapter 22). Couplers are designed using  even-odd mode approach. Miniature lumped element couplers can be designed by replacing the transmission lines with lumped element equivalents.

Figure 1.23. Menu items of Symmetric Cross-Coupled Circuits command.

In the following sections, the editing, circuit transformations and other menus will be described.

1. 5. EDIT MENU

The commands in this menu performs simple circuit editing functions, as summarized below (Figure 1.24):

Figure 1.24. Edit commands.

1. 5. 1. Undo

Entering Undo command obviates the circuit transformations performed in the previous steps. It can be invoked eleven times successively to go to eleven steps back. Shortcut Key is Ctrl+End.

1. 5. 2. Redo

The circuit transformations obviated by Undo command are re-applied.

1. 5. 3. Change

This command is used to change the values of parameters describing the element. Shortcut Key is Space Bar. When Space Bar is pressed parameters of element pointed by the cursor are displayed in a dialog box. New values of the parameters are typed in and Enter key is pressed.

1. 5. 4.  Insert

This command is used to insert elements selected from the component library into the circuit, at the left of the element pointed by the cursor. Shortcut key is Insert. All parameters describing the element to be inserted are displayed before insertion for adjustment of values. When Insert command is invoked, the element group list will first be displayed. The elements that are available in the library are grouped as shown Figure 1.25.

Figure 1.25. Inserting an element into the displayed circuit.

Element groups are as follows:

a) Lossless Lumped Elements:

These elements are shown in Figure 1.26. They are the classical inductors, capacitors, second and fourth order LC resonators, Ideal Transformer, Coupled Coils, Capacitively loaded Coupled Coil, Tapped Coils (with left and right tappings) and Bridge Tees formed by LC resonators. Second and fourth order LC resonators are treated as single elements. They may be splitted into simpler forms by Split command, whenever possible.

The terminology for describing series and shunt arm elements and series or parallel combination of inductors and capacitors is as follows:

Figure 1.26. Lossless lumped elements.

- For example a series arm inductor is termed as Lsr, while a shunt arm inductor is denoted as Lsh.

- Parallel combination of an inductor and capacitor has the prefix “p”. If this element is in series arm then it is termed as pLCsr, meaning parallel LC in series arm. If this parallel combination is in shunt arm, then it has the name pLCsh. The element pLCsh can always be split into its L and C components using Split/ 2-Element Resonator command.

- Series connection of an inductor and a capacitor has the prefix “s”. If this element is in series arm then it is termed as sLCsr and if it is in shunt arm, then it is termed as sLCsh. The element sLCsr may always be splitted into its L and C components using Split / 2-Element Resonator command.

- A shunt arm fourth order section having a series LC resonator in series with a parallel LC resonator is named as psLCsh. It can always be splitted into two second order series LC resonators in shunt arm by applying Split/ Fourth Order Section command.

- A series arm fourth order section having a series LC resonator in parallel with a parallel LC resonator is named as psLCsr. It can always be splitted into two second order parallel LC resonators in series arm by applying the command Split / Fourth Order Section.

b) Lossy Lumped Elements and Parasitics command:

Lumped elements can be made to include all parasitics and resistive losses. They are termed as Lossy Lumped Elements and are shown in Figure 1.27. A lossless lumped element can be converted into its lossy counterpart by applying the command Parasitics / Lossless-to-Lossy. In this conversion, the parasitic parameters should be entered by the user. Parasitic elements of inductors are:

- Q of inductor represented by a resistor (in series with inductor).

- Interwinding capacitance of inductor (in parallel with inductor).

Parasitics of capacitors are as follows:

- Q of capacitor represented by resistor in parallel with capacitor.

- Lead inductance and lead resistance of capacitors placed in series with capacitor.

Shunt arm parallel LC resonators are by far the most frequently used elements in filters. Such LC resonators can also simulate distributed element or mixed lumped+distributed element resonators. Therefore, although it is possible to split an LC resonator into its L and C components and then consider the Q of each element separately, a special command is prepared to account their finite Q.  Lossless-to-Lossy command when applied to and LC resonator, converts it into a parallel LCR element where R is the resistance calculated from the user specified Q of the resonator at its resonance frequency. This Q is treated as constant at all frequencies. Therefore this model holds only for narrow-medium bandwidth applications.

The command All Lossy Resonators will convert all the shunt LC resonators of the circuit into LCR resonators.

Lossy-to-Lossless command recovers the lossless model of the element.

Figure 1.27. Lossy lumped elements.

Figure 1.27. Lossy lumped elements (continued).

c) Transmission Line, Stubs, Inverter:

This list includes transmission line, ideal impedance inverter, series and shunt open and short circuited stubs, series and parallel combinations of of stubs (termed as stub resonators of degree two and four) in series and shunt arms (Figure 1.28).

- Transmission line is lossless and is represented through it’s characteristic impedance, relative dielectric constant and physical length in mm.

- Ideal impedance inverter is represented by its inverter constant K.

- Open and short circuited stubs and their combinations are represented by their characteristic impedance, relative dielectric constant and physical length in mm.

Figure 1. 28. Transmission Line, Impedance Inverter and elements formed by open and short circuited stubs.

d) Lossy Tline + Stubs:

In order to get a rough idea on the effects of dielectric and conductor losses of transmission lines and stubs on response of filters, their lossy models are also included in the library (Figure 1.29). A lossless element is converted into lossy one through the command Parasitics / Lossless-to-Lossy. Loss associated with these elements is to be specified as “Attenuation in dB per meter”. Since length of stubs or lines are known, the total attenuation is calculated by Filpro automatically.

Figure 1.29. Lossy transmission lines and stubs.

e) Mixed Lumped + Distributed Elements

The namelist of mixed elements and their schematics are shown in Figure 1.30. These elements are combinations of lumped elements and OC or SC stubs. They can be used for studying hybrid lumped + distributed element filters.

Figure 1. 30. Mixed Lumped+Distributed elements.

f ) Stepped Stubs:

This list contains shunt arm two step and four step open circuited stubs, shunt two step short circuited stub and series two step open and short circuited stubs (Figure 1.31). Shunt two-step stubs are used to realize single finite transmission zeros while the four step stub can realize double finite transmission zeros.

Figure 1.31. Stepped stubs.

g ) Exponential Line Elements:

This list consists of the basic exponential line, shunt open and short circuited exponential stubs and a shunt component made up of series connection of lumped capacitor  and two exponential line pieces (Figure 1.32). These elements may be used as representatives of continuously nonuniform transmission lines.

Figure 1.32. Components with exponential transmission lines.

h ) Constant Impedances:

This list includes series and shunt resistances, series and shunt constant reactances (ideal frequency independent reactances) and their combinations with inductors and capacitors (Figure 1.33).

Figure 1.33. Resistances, frequency independent (constant) reactances and their combinations with inductors and capacitors.

Figure 1.33. Resistances, frequency independent (constant) reactances and their combinations with inductors and capacitors (continued).

i ) Parallel Coupled Lines:

The classical TEM mode parallel coupled lines are collected in the following groups in Filpro (Figure 1.34):

Comb Type Lines

Interdigital Type Lines,

Spur Type Lines

Ikeno Type Lines,

Stepped Parallel Coupled Lines

Stepped Interdigital Lines

Double Comb Lines

Miscellaneous Coupled Lines

Figure 1.34. Parallel coupled line groups.

The parallel coupled lines listed in these groups are all two-port elements formed by terminating the two unused ports by open circuit, short circuit, open stub, short stub, etc. Each group includes several topologies set by the terminations placed in the unused port, as described below.

- Comb Type Lines:

These are parallel coupled line two port elements whose input and output ports are on the two lines and adjacent to each other as described in Figure 1.35. The output ports are terminated in open circuit, short circuit, open circuit stub, short circuit stub or lumped capacitors. If the output ports are connected to each other then it is termed as hairpin line.

Figure 1.35. Comb type parallel coupled lines.

- Interdigital Type Lines:

In these lines the input and output ports are located at the diagonally opposite end of the two lines as shown in Figure 1.36. The other ports are terminated in open or shortcircuits or in lumped capacitors.

Figure 1.36. Interdigital type parallel coupled lines.

- Spur Type Lines:

In these lines input and ouyput ports are located at the two ends of the same line and one end of the adjacent line is shorted to the main line as described in  Figure 1.37. When the free end of the coupled line is is terminated in stubs, it is called as Saito line.

Figure 1.37. Spur type parallel coupled lines.

- Ikeno Type Lines:

These are coupled lines whose input and output are on the same line which is termed as the main line. The ports of  coupled (second) line can be terminated in open circuit, short circuit or in stubs, as described in Figure 1.38 to produce a variety of circuits to be used as filter components. Since such lines may be considered as special cases of general coupled line Ikeno loop, these coupled lines are termed as Ikeno type lines.

Figure 1.38. Ikeno type parallel coupled lines.

- Stepped Comb Lines:

These are comb type lines with two steps. If the step impedances are identical, then they reduce to the classical comblines(Figure 1.39).

Figure 1.39. Two step comb type parallel coupled lines.

- Stepped Interdigital Lines:

These are interdigital lines with two steps. If step impedances are identical, then they reduce to the classical interdigital lines (Figure 1.40).

Figure 1.40. Two step interdigital type parallel coupled lines.

- Double Comb Lines:

These are formed by parallleling two combline sections.  The case with one side OC type comb and the other side hairpin line is also included in this class (Figure 1.41).

Figure 1.41. Comb type lines paralleled in series arm (Double Combs).

- Miscellaneous Parallel Coupled Lines

Some special line sections are collected under this heading (Figure 1.42). One is the well-known Ikeno-Loop which is a square loop made up of (actually uncoupled) four quarter wavelength lines. The second one is parallel combination of an open circuited comb section with two unit elements, termed as UEComb. The tappings used in the classical combline and interdigital line filters are also modeled as separate components in Filpro to ease the analysis of such filters. See Chapter 9 for tapped interdigital  filters and Chapter 11 for tapped combline filters.

Figure 1.42. Miscellaneous line sections.

A large variety of parallel coupled line circuits can be configured by inserting these elements. Most of these elements have Stub+ TLine equivalents which can be replaced by their coupled line counterparts by a simple command (see Parallel Coupled Lines-PCL command below).

k ) Rectangular Evanescent and Propagating Waveguide Elements

Evanescent waveguides elements that can be used to form evanescent waveguide filters and cros-coupled evanescent waveguide filters are included in this list.

- A rectangular waveguide is described by its cross-sectional dimensions, dielectric loading and physical length (in mm). It is named with letters ER, as shown in Figure 1.43.a.

- A short circuited and an open circuited evanescent waveguide is shown as in Figure 1.37.b and c respectively.

- The waveguide components shown in Figure 1.43.d-to-g are used in analysis of cross-coupled evanescent waveguide filters as cross-coupling elements. See Chapter 16 for their definitions.

- Although these components are defined for use in evanescent waveguide filters, since their responses are correct outside the evanescent mode region (propagation region), they may also be used to fom and analyze normal rectangular waveguide filters.

Figure 1.43. Evanescent waveguide components.

m ) Attenuators and dividers

Pi and Tee type attenuators (Figure 1.44.a and b) can be inserted into the circuit with desired attenuation constants. Attenuators are formed by three resistors calculated in accordance with the desired attenuation in decibels and reference terminations.

Two, four and eight way dividers may be formed by cascading the components shown in Figure 1.44.c and d.

Figure 1.44. a) Pi-attenuator, b) Tee-Attenuator. c) One output branch of Wilkinson divider.                d) Two output branches of a Wilkinson divider.

Figure 1.45. a) General schematics of a four way Wilkinson divider (without isolation resistors).

b) Equivalent circuit of the component 1W (one output branch of divider).

c) Equivalent circuit of the component 2W ( two of the output port branches of divider).

d) Formation of a two way divider on Filpro. e) Formation of a four way divider on Filpro.

f ) Formation of an eight way divider on Filpro.

As an example, in Figure 1.45.a, a four way divider is shown without isolation resistors. {Dividers with isolation resistors can’t be hadled by Filpro because of non-cascade and non-tree type structure}. The element named as 1B describes any one of the output branches, as shown in Figure 1.45.b. Two of the output branches can be represented by the element named as 2B as described in Figure 1.45.c. These two new elements may be used to form two way, four way and eight way dividers, as described in Figure 1.45.d, e and f. In Figure 1.45.f, the external one port is a four way divider formed previously and saved. Thus, the circuit of Figure 1.45.f involves parallel connection of two four way dividers.

n) CQ-CT and Bridge Tee’s:

CT (Cascaded Triplet), CQ (Cascaded Quadruplet) and GCQ (Generalized CQ) are classes of filters made up by cascading the element groups termed as triplets and quadruplets. Quadruplets are formed by four coupled shunt LC resonators while triplets are formed by three coupled shunt LC resonators. Generalized quadruplets have coupling between the diagonal resonators also. Couplings among the resonators can be capacitive, inductive, or through ideal inverters. The elements that can be used to form triplets and quadruplets of resonators are shown in Figure 1.46. See Chapter 24, 25 and 26 for detailed definitions of CQ and CT filters where detailed definitions and classifications of  Bridge Tees and other elements are also presented.

Figure 1.46. Bridge Tee, CT, CQ and Generalized CQ elements.

1. 5. 5. Delete

This command deletes the element  pointed by the cursor. Short cut key is Delete.

1. 5. 6. Split

This command is used to split the element pointed by cursor in various ways. The commands are shown in Figure 1.47. Shortcut key is End.

The following options are displayed:

Split / 2 Equal: 		Splits an element equally into two.

Split / 3 Equal: 		Splits an element equally into three.

Split / Specify Left: 	The value of left piece is specified by the user.

Split / Specify L/R ratio: The ratio of left piece to right piece is specified by the user.

Some examples for these commands are shown in Figure 1.47.

Figure 1.41. Split command.

Figure 1.41. Split command (continued).

Split / 2-Element Res: A two-element resonator (series or shunt arm LC resonators or stub resonators formed by OC and SC stubs connected in series or in parallel) is split into its component elements (Figure 1.48).

Split / All 2-Elm resonators: All second order (two-element) resonators are split into their L and C or  OC stub-SC stub components.

Figure 1.48. Split 2-Element resonators command (Combine in reverse direction).

When this command is applied to a fourth order section (QUAD) it is split into two second order sections, as shown in Figure 1.49.

Figure 1.49. Splitting lumped and distributed element fourth order sections (quads)

into second order sections (Combine in reverse direction).

1. 5. 7. Combine

This command is the reverse of Split command and can be used to perform several different transformations. Shortcut key is Home.

- Two neighboring elements of the same type (e.g. two series / parallel L,C, stubs, two transmission lines of identical impedance, etc.) can be combined to form a single element (Figure 1.47-48).

- Some elements can be combined with their neighbors to form a single, higher complexity element. An L and C forming an LC resonator or an OC and SC stub, forming a stub resonator, two series resonators connected in shunt arms in parallel or two parallel resonators cascaded in series arm can be combined to form single Fourth Order Sections (Quads) (Figure 1.49).

1. 5. 8. Exchange

Exchanges the pointed element with its neighbor on its right. Cursor must always be placed under the element on left. Shortcut key is Ctrl+X.

1. 5. 9. Copy

Copies the pointed element to buffer to be pasted into any other location in circuit.

1. 5. 10. Paste

Pastes the copied element to the left of the position indicated by the cursor.

1. 5. 11. Simplify Circuit

Simplifies circuit by reducing redundancies. It combines adjacent same type or combinable elements, combines   transformers and shifts the resulting transformer to the load end.