CHAPTER 2

A SUMMARY OF BASIC CONCEPTS ON FILTERS

2. 1. INTRODUCTION

Filters are circuits used to shape frequency response of networks in which they are placed. There are diverse kinds of filters which can be classified in various respects like passive-active, lumped-distributed, lossless-lossy, analog-digital, doubly terminated-singly terminated, amplitude-phase, etc. etc. Filpro is developed to design filters with following properties:

- Filpro can synthesize only lossless filters. Losses of actual practical elements can be incorporated after synthesis by replacing ideal elements with their practical models.

- Filpro can synthesize either lumped or commensurate length distributed element filters. However the resulting structures can be transformed into mixed (lumped+distributed) form with noncommensurate lengths.

- Filpro can synthesize only passive and reciprocal element filters. However active or nonreciprocal components, characterized through measured scattering parameters, may also be inserted to simulate and design active filters.

- Filpro can synthesize doubly terminated or singly terminated filters.

- Filpro can synthesize only two port filters. However, routines exist to design lowpass-highpass diplexers with adjustable cross-over using singly terminated filters. It is also possible to design and simulate lowpass-highpass, bandpass-bandstop, parallel bandpass and manifold type  multiplexers by interconnecting individually synthesized filters.

- Filpro can synthesize circuits for amplitude filtering applications. Lowpass, highpass, bandpass and bandstop filters with equiripple or maximally flat passband, general stopband filters can be designed. It is also possible to display and analyze insertion phase, delay and return loss, input impedance, input admittance, S11 parameter, and input voltage standing wave ratio functions of the filters.

- In addition to the synthesis of classical cascaded element filtering structures, Filpro can also be used to synthesize the Cascaded.Triplet and Cascaded Quadruplet type filters.

- Filpro can be used to synthesize symmetric cross-coupled, selective linear phase filters with or without finite transmission zeros by pole placement approach. Such filters are analyzed using Even-Odd mode decomposition approach.

- Filpro is also capable of synthesizing unsymmetric cross-coupled filters with vertical and diagonal cross-couplings.

- Filpro can be used to design and analyze impedance transforming symmetric branch line and parallel coupled line directional couplers.

- More than hundred different elements are available in the library of the program, ready to be inserted into the circuits to simulate diverse kinds of structures.

In summary, Filpro can synthesize two-port amplitude filtering structures formed by cascading or cross-coupling passive, lossless, reciprocal and lumped or distributed elements.

In this chapter a short review of basic concepts and definitions related to the classical lumped element filter synthesis techniques will be presented together with a qualitative description of stages from filter specifications to element extraction.

2. 2. VOLTAGE-CURRENT CHARACTERIZATION AND SYNTHESIS OF ONE-PORT CIRCUITS

Response of networks can be studied in terms of voltage and current parameters defined at their ports. A one port network (Figure 2.1) can be characterized, in frequency domain s=+jw, by its input impedance Z(s) or input admittance Y(s) defined in terms of port voltage V(s) and port current I(s) as

Z(s)=1/Y(s)=V(s)/I(s) 	(2.2.1)

Figure 2. 1. A one port network.

Analytical properties of input immittances of passive, lossless one ports are of fundamental importance in synthesis of filters. Therefore let's summarize some basic ones [17].

Theorem 2. 1.

Driving point impedance Z(s) (and Y(s)) of a linear, time invariant, passive and lumped one-port is a positive real function (PRF). That is,

i) Z(s) is real for s real, 	(2.2.2)

ii) Re[Z(s) ]>0 for Re(s)> 0.	(2.2.3)

This theorem establishes both necessary and sufficient conditions for realizability of a given Z(s) (or Y(s)). These properties reflect themselves on the polynomials forming Z(s) as follows:

Let

Z(s)= 1/Y(s) = N(s)/D(s) 	(2.2.4)

Then:

i) N(s) and D(s) has no roots in the right half of s=+jw plane (in other words, N(s) and D(s) are Hurwitz polynomials).

ii) The roots on s=jw axis, including those at s=0 and s=, if they exist, must have unit multiplicity with positive residues.

iii) Condition “ii” implies that degrees of N(s) and D(s) may differ at most by unity.

These properties may further be detailed for the special (but practically the most important) case of lossless networks through the theorem due to Foster:

Theorem 2. 2. Foster's reactance theorem.

For a positive real rational function Z(s) = 1/Y(s) to be realizable as the driving point impedance of a lossless one-port, the necessary and sufficient condition is that it should be expressible in the form

Z(s) or Y(s) = [an (s2 + w12 ) (s2 + w32 )....]/[bm s (s2 + w22) (s2 + w42 )....]	(2.2.5)

where an and bm are constants and

i ) 0 w1 < w2  <w3...	(2.2.6)

(Interlacing poles and zeros, all on jw axis)

ii ) Degrees of numerator (n) and denominator (m) must differ by unity.

From these conditions, the following properties can be deduced:

a) Unity degree difference between numerator and denominator implies that Z(s) must have either a single pole or a single zero at both s= 0 and s= . Therefore the function Z(s) or Y(s) will belong to one of the four types:

pp-type: Pole at  s=0 and pole at  s=.

pz-type: Pole at  s=0 and zero at  s=

zp-type: Zero at  s=0 and pole at  s=

zz-type: Zero at  s=0 and zero at  s=

b) Z(jw) is purely reactive. Therefore it can be written as

Z(jw) = jX(w),

where X(w) is the input reactance with

X(w) or 1/X(w) = [an (w12 - w2) (w32 - w2)....]/[ bm w (w22 - w2) (w42 - w2)...] 	(2.2.7)

Alternation of poles and zeros leads to the property

0 <|X(w)|/w < dX(w) /dw 	(2.2.8)

That is, the reactance X(w) is always an increasing function of frequency.

The rational functions satisfying these requirements are called “reactance” or “Foster” functions.

c) Since all poles of Z(s) and Y(s) are on s=jw axis, they can always be expanded as

Z(s)= ko /s+k  s+[ (2ki s)/(s2+wi2) ] 	(2.2.9)

and

Y(s)= ho /s+h  s+ (2hi s)/(s2 + wi2)	(2.2.10)

where the constants “k” and “h” are termed as residues of the respective poles. Physically, they correspond to simple network elements, as follows:

- If Z(s) has a pole at s=0, then it can be extracted as a series capacitor of value Co = 1/ko calculated as,

1/Co = ko = [sZ(s) ]s=0 	(2.2.11)

- If Z(s) has a pole at s = , then it can be extracted as a series inductor of value L = k, calculated as

L  = k = [Z(s) /s]s=  	(2.2.12)

- If Z(s) has a pole at s = jwi, then it can be extracted as a series arm parallel resonator, with element values

1/ Ci = 2ki =[(s2 + wi2 )Z(s) /s]s=jwi  ,              Li = 1/ (wi2 Ci)	(2.2.13)

- If Y(s) has a pole at s= 0, then it can be extracted as a shunt inductor of value Lo, calculated as

1/ Lo = ho = [sY(s) ]s=0	(2.2.14)

- If Y(s) has a pole at s=, then it can be extracted as a shunt capacitor of value C =h , calculated as

C  = h = [Y(s) /s]s= 	(2.2.15)

- If Y(s) has a pole at s=jwi, then it can be extracted as a shunt arm series resonator, with element values

1/ Li = 2hi = [(s2 + wi2 )Y(s)/s]s=jwi ,            Ci = 1/ (wi2 Li) 	(2.2.16)

Note that if a pole of Z(s) at s=0 or s= is extracted, then a zero appears at that frequency automatically in the remaining impedance function which acts as a pole of the remaining admittance function. Hence, given Z(s), one can synthesize a variety of one-port circuits all having the same input impedance but with different structures, by extracting elements in different orders from impedance or admittance functions.

Figure 2.2. Extracted poles of Z(s) and Y(s). a) Pole of Z(s) at s=0. b) Pole of Z(s) at s=.                   c) Pole of Z(s) at s=jwi. d) Pole of Y(s) at s=0. e) Pole of Y(s) at s=. f) Pole of Y(s) at s= jwi.

2. 3. VOLTAGE-CURRENT CHARACTERIZATION OF TWO-PORT CIRCUITS

A two port network (Figure 2. 3) can be characterized in a variety of ways in terms of the four port variables V1(s), V2(s), I1(s), I2(s) by taking any two as excitations and the other two as responses. In filter practice, the following matrix descriptions are frequently needed:

Figure 2. 3. A two-port network.

i) Impedance matrix parameters (Z-parameters) are defined as

(2.3.1)

where

Z11 = V1/I1      and         Z21 = V2 /I1      with I2 = 0,

Z12 = V1/I2      and         Z22 =V2 /I2      with I1= 0

For reciprocal two ports, Z12 = Z21.

ii) Admittance matrix parameters (Y-parameters) are defined as

(2.3.2)

where

Y11 = I1 /V1       and          Y21 = I2 /V1        with V2 = 0

Y12 = I1 /V2       and          Y22 = I2 /V2        with V1 = 0

For reciprocal two ports, Y12 = Y21.

iii) ABCD matrix parameters relate input port variables to output port variables through

(2.3.3)

where

A = V1 /V2         and             C= I1 /V2           with I2 = 0

B = V1 /(-I2)      and             D= I1 / (-I2)        with V2 = 0

For reciprocal two ports AD - BC = 1

Any one of these matrices can be used for analysis and synthesis of two port networks with various termination conditions. In Filpro, the circuit types shown in Figure 2.4 can be synthesized. The circuit of Figure 2.4.a is one kind of singly terminated two ports and the one in Figure 2.4.b is a doubly terminated two port.

Figure 2.4.a) A singly terminated two port. b) A doubly terminated two port.

Doubly terminated two ports are by far the most frequently used types. Therefore in the rest of this overview, we will be dealing only with this type of two ports. In Filpro, singly terminated two ports are synthesized only to construct some diplexers and multiplexers.

Impedance and admittance matrix parameters are usually used in synthesis stages while ABCD parameters are best suited for analysis of cascaded element type structures (Figure 2.5.a). Most of the practical filter structures are of this type or, even in simpler form, termed as “ladder structure” (Figure 2.5.b).

Figure 2.5.a) Cascaded element structure. b) Ladder structure.

In these circuits the overall ABCD matrix of the two port can be calculated simply as ordered products of individual ABCD matrices of cascaded elements:

(2.3.4)

ABCD matrices of elements of ladder type structures are of the form

(2.3.5)

where ZS and YP are impedance and admittance of series and shunt elements respectively. Hence both characterization of individual elements and programming for calculation of response are simpler in terms of ABCD parameters.

2. 4. SCATTERING DESCRIPTION OF TWO-PORT NETWORKS

The most convenient characterization tool for doubly terminated two ports in the approximation stage of filter synthesis and in analysis or measurements is the scattering matrix parameters (S-parameters) because it is easier to describe real power flow process in terms of these parameters. Desired filter power transfer characteristics can be obtained from scattering parameters or vice versa easily. Also in high frequency practice it is easier to measure real power and phase compared to voltage and current measurements.

Scattering concepts had originated from wave theory of transmission lines. Therefore let's first summarize basic definitions and concepts related to transmission lines.

Consider for simplicity of explanations, a lossless transmission line on which a single Transverse Electro Magnetic wave mode (TEM) is supported (Figure 2.6). Let’s also assume that the line is fed by a source with internal resistance RS equal to the characteristic impedance Z0 of the line. On such lines it is possible to define voltage and current wave solutions as

V(x)=Vi exp (-jx)+Vr exp (jx)=Vi [exp (-jx)+L exp(jx)] 	(2.4.1)

I(x)= [Vi exp(-jx)-Vr exp(jx)]/Z0= (Vi /Z0)[exp(-jx)-L exp(jx)]	(2.4.2)

where

x : Distance from load end.

w(1/2 = w(LC)1/2 : Propagation (phase) constant.	(2.4.3)

Z0 = (L/C)1/2 : Characteristic impedance of  transmission line. 	(2.4.4)

  :Permeability and permittivity of medium of propagation.

L,C:Inductance and capacitance per unit length of the line.

L = (ZL-Z0 ) / (ZL+Z0) : Load reflection coefficient.	(2.4.5)

ZL  (or RL ): Load Impedance.

Vi =VS / 2: Amplitude of incident voltage wave for RS = Z0.	(2.4.6)

Vr= LVi : Amplitude of reflected voltage wave. 	(2.4.7)

Figure 2. 6. A transmission line loaded at both ends.

The components with factors exp(-jx) are called incident waves (waves traveling in +x direction) while those with factors exp(jx) are called reflected waves (traveling in -x direction). Real power flow associated with these waves can be calculated as

P= Re[V(x)I*(x)]=Pi -Pr	(2.4.8)

where

Pi =|Vi|2 /Z0=|Vs|2/4Z0 :     Incident real power for RS =Z0.	(2.4.9)

(Also maximum available power of the source.)

Pr=|L|2 Pi = 2PiReflected real power.	(2.4.10)

where =|L| is magnitude of the load reflection coefficient. It is clear that part of the incident power launched by the source is reflected from the load because of impedance mismatch between line and load (ZLZ0). The assumption RS = Z0 avoids re-reflection of the wave from line-source junction, hence avoids multiple reflection problems. All the power reflected from the load is absorbed by the source, without re-reflection. The concepts of voltage and current which are the starting points of the above analysis are not the best suiting quantities to use at especially high frequencies where it is difficult to measure or even may not be possible to define uniquely. The “power wave” or “scattering signal” concepts have originally developed for such applications and then it was found out that they can also be used in analysis and synthesis of low frequency, lumped element circuits in a more efficient way than voltage and current concepts. Power waves are related to incident and reflected voltage waves (if defined) through

a(x)= [V(x)/(Z0)1/2 + (Z0 )1/2 I(x)]/2 =Vi exp(-jx)/(Z0)1/2 	(2.4.11)

(Incident power wave or incident scattering signal.)

b(x)= [V(x) /(Z0)1/2 -(Z0)1/2 I(x) ]/2 = LVi exp(jx)/(Z0)1/2 	(2.4.12)

(Reflected power wave or reflected scattering signal.)

Some useful properties of these signals may be noted as follows:

i ) The name “power wave” comes from the fact that |a|2 and |b|2 are incident and reflected real powers themselves:

|a|2  =  a(x)a*(x) = Pi = |VS|2/4Z0 : Incident real power or available power.	(2.4.13)

|b|2  =  b(x)b*(x) = Pr (w2 ) = |L|2|VS|2 /4Z0 = 2 Pi : reflected real power           (2.4.14)

Note that incident real power Pi is a constant, independent of frequency and independent of the circuit to which it is connected. Therefore it is the best reference for power measurements or calculations in the circuit it is feeding. Reflected power Pr is a function of frequency w (actually, function of w2). From Equation 2.4.14, it is directly proportional to Pi. Since Pi is independent of frequency, frequency dependence of Pr represents in fact frequency dependence of the circuit terminating the line (ZL). Therefore measurement of Pr can be used to display frequency characteristics of the load circuit.

ii) The power waves at different points along the line have the same magnitude, differing in phase only:

a(x2 )=a(x1)exp[ j(x2 -x1)],             b(x2)= b(x1) exp[ j(x2 -x1)] 	(2. 4.15)

iii) The ratio of b(x) to a(x) gives directly the reflection coefficient at point x:

b(x)/a(x)= (x) = L exp(j2x) 	(2. 4.16)

giving also

|b|2 /|a|2 = Pr /Pi = 2	(2. 4.17)

anywhere on the line.

In high frequency practice, it is easy to distinguish and measure power and phase of waves traveling in opposite directions. Therefore power wave concepts are the best suiting quantities for such applications.

Now let's see the extension of these concepts to lumped element circuits. Consider a two-port network fed by two sources through transmission lines of characteristic impedances equal to the source impedances R1 and R2 respectively (Figure 2.7). Power waves a1 (x), b1 (x), a2 (x), b2 (x) on these lines can be defined in accordance with Equations 2.4.11 and 12. If lengths of lines are assumed to be zero, then these waves become port signals a1 , b1 , a2  and b2 are related to the port voltages and currents through

a1 = [ V1 /(R1 )1/2  + (R1 )1/2 I1 ]/ 2 	(2 4.18a)

b1 = [ V1 /(R1 )1/2  - (R1 )1/2 I1 ]/2 	(2 4.18b)

a2 = [ V2 /(R2 )1/2 +(R2 )1/2 I2 ]/2 	(2 4.18c)

b2 = [ V2 /(R2 )1/2  - (R2 )1/2 I2 ]/2 	(2 4.18d)

Figure 2. 7. Power waves at the ports of a doubly terminated two-port.

Scattering matrix parameters of the two port are defined in terms of these signals as follows:

(2.4.19)

where

S11 = b1 /a1     ,      S12 = b2 /a1        with a2 = 0

S12 = b1 /a2   ,       S22 = b2 /a2        with a1 = 0	(2.4.20)

Here a2 = 0 means

i) No source is connected to port-2

ii) Port-2 is terminated in its (reference) resistance R2 (or RL).

That is, either connection of a source to that port or terminating that port with a resistance different from R2 will cause a wave traveling towards port-2 which is treated as “incident” signal a2 for that port which is actually a “reflection from the load” when termination is different from reference value R2.

Similarly, a1 = 0 means terminating port-1 in its reference resistance R1 (or RS) without any source. It is clear that S-parameters of a given circuit are not unique. They depend on the termination resistances used in the definitions or measurements. Therefore a given set of S-parameters describe the circuit together with its specific set of terminations R1 and R2.

Consider the same two port, now fed from port-1 only (Figure 2. 8). In this circuit a2= 0. Therefore Equation 2.4.2 gives

b1 (s) = S11 (s)a1 (s)

b2 (s) = S21 (s)a1 (s) 	(2.4.21)

In this description, the incident signal a1 (s) is the excitation while the reflected signal b1 (s) at port -1 and transmitted signal b2(s) at port-2 are responses. Significance of these signals come from their relation to reflected and transmitted real power. Since real power is defined only on s=jw axis, let's repeat Equation 2.4.21 for s=jw :

b1 ( jw)= S11 ( jw)a1 ( jw)

b2 ( jw)= S21 ( jw)a1 ( jw) 	(2.4.22)

Using definitions given in Equations 2. 3. 13 and 14, we have

|a1|2 = a1 (s) a1 (-s)|s=jw =Pi = |Vs|2 /4RS : Incident real power 	(2.4.23)

|b1|2 = b1 (s) b1 (-s)|s=jw = |S11 (jw)|2 |a1|2  = Pr (w2 ) : Reflected real power.	(2.4.24)

|b2|2 = b2 (s) b2 (-s)|s=jw = |S21 (jw)|2 |a1|2 = Pt (w2 ) : Transmitted real power. 	(2.4.25)

Figure 2.8. Power flow in a doubly terminated two port fed from one port only.

That is, the source is assumed to pump all of its available real power Pi = |a1|2 to the input port of circuit. Part of that power is reflected back to the source ( Pr = |b1|2 ), part of it is transmitted to output port (Pt = |b2|2) and the rest is dissipated (Pd ) in the resistive parts of circuit if there is any. For lossless networks Pd = 0. Therefore lossless and passive two-ports, conservation of real power principle gives

Pi = Pr (w2 ) + Pt (w2 ) 	(2.4.26)

which can be written in terms of a1, b1 and b2 as

|a1 |2 = |b1 |2 +|b2 |2	(2.4.27)

or in terms of scattering matrix parameters,

|S11 |2 +|S21 |2 =1 	(2.4.28)

This result is termed as unitary property of lossless, passive two-ports. It is clear from this property that specification of either one of S21 or S11 is sufficient to determine the other one also. With these properties, the scattering parameters S21 and S11 have the following interpretations:

| S21 |2 = |b2 |2 /|a1 |2  = Pt /Pin =G : Transducer power gain. 	(2.4.29)

| S11 |2= |b1 |2 /|a1 |2  = Pr /Pin  = 2 :Power reflection coefficient. 	(2.4.30)

That is, |S21|2 is the power transmission coefficient itself (usually termed as transducer power gain G) and |S11|2 is power reflection coefficient. This is the most important advantage of scattering parameter approach over the other matrix descriptions. In this approach, specification of desired input/output power transfer (transducer power gain) characteristics gives directly the parameter S21 from which the circuit can be synthesized, as described in the next section.

In practice, incident, reflected and transmitted real powers and their phases relative to the input signal phase are conveniently measurable quantities. It is usual to express measured real powers in decibels referenced to the incident real power. In this convention, reflected power is characterized through Return Loss concept defined as

Return Loss= 10 log ( Pi /Pr ) = -10 log [(S11 )2]    (dB)	(2.4.31)

Transmitted power is expressed as Insertion Loss concept defined as

Insertion Loss= 10 log ( Pi /Pt ) = -10 log [(S21 )2]  (dB)	(2.4.32)

2. 5. APPROXIMATION

Filters are designed to approximate unity power gain in the frequency band(s) called passband, and to approximate zero gain in frequency band (s) called stopband:

|S21 (w)|2 = 1 for w in passband

|S21 (w)|2 =  0 for w in stopband	(2.5.1)

Circuits made up of ideal, lossless elements like inductors, capacitors, transmission lines, stubs, coils, etc. can be tailored to approximate these properties. For lumped element circuits scattering matrix parameters (like all other two port matrix parameters) come out to be rational functions of complex frequency s = +jw. Belevitch [7] had shown that scattering matrix parameters of two ports made up of lumped, lossless, reciprocal, passive elements can be described in terms of only three real polynomials, fe (s) or fo (s), h(s) and g(s) as :

(2.5.2)

or

(2.5.3)

Here, f(s) = fe(s) or f(s) = fo(s) is either even or odd polynomial, g(s) is a strictly Hurwitz polynomial (all roots in the right half of s=+jw plane), while h(s) is a real polynomial linked to f(s) and g(s) through an equation called Feldtkeller Equation:

g(s)g(-s)= h(s)h(-s)+f(s)f(-s) 	(2.5.4)

which is a generalization of the unitary property given in Equation 2.4.28, to cover the whole s=+jw plane. Thus, circuits formed by only lumped inductors and capacitors have scattering parameters of the form

S21 (s)=f(s)/g(s),       S11 (s)=h(s)/g(s) 	(2.5.5)

from which the transducer gain of the two port can be obtained as

G(w2 )= S21 (s) S21 (-s)|s=jw = [f(s) f(-s) ] /[g(s)g(-s)]|s=jw 	(2.5.6)

This relation can also be used in reverse form to obtain S21 (s) from a properly specified power gain function G(w2 ) through

S21 (s)S21(-s)= G(w2)|w=s/j 	(2.5.7)

It is clear that the poles and zeros of S21 (s) have the primary roles in shaping frequency response of the two-port. Therefore let’s write the product S12 (s) S21 (-s) in more detailed form:

(2.5.8)

where

K(s)K(-s) = [h(s)h(-s)]/[f(s)f(-s)]	(2.5.9)

The frequencies which make

S21 (s) S21 (-s)=0 	(2.5.10)

are called transmission zeros (TZ) because at these frequencies no signal can reach to output port. Similarly the frequencies which make

S21 (s) S21 (-s)=1 	(2.5.11)

are termed as reflection zeros (RZ) of the two-port because at this frequency all the incident power is delivered to the load without any reflection. In order to inspect the nature of transmission and reflection zeros let’s write f(s) and h(s) in expanded forms :

(2.5.12)

(2.5.13)

where fm and hn are constant coefficients, mo and no are multiplicities of the roots of f(s) and h(s) at s=0, sk and sr are the finite nonzero roots of f(s) and h(s).

From Equation 2.5.8-to-13 we can make the following observations:

i) If n>m, then  S21 (s) S21 (-s)=0 at s=.Therefore the two-port has a transmission zero at s=  of multiplicity n = n-m. The degree of g(s) is then set by h(s) (through Feldtkeller Equation) as n, which will be referred to as degree of the filter.

ii) If m>n, then S21 (s) S21 (-s)=1 at s= and we say that there is a reflection zero at s= with multiplicity m  =m-n. Then the degree of g(s) is equal to m.

iii) In Equations 2.5.12 and 13 we will have either mo=0 or no=0 (cancellation in Equation 2.5.9). If mo>0, then there will be a transmission zero at s=0 of multiplicity mo. If no>0, then there will be a reflection zero at s= 0 with multiplicity no.

iv) The “finite - nonzero” roots sk of f(s)=0 are called “finite transmission zeros - FTZ” of the two port, because at these frequencies we have S21 (sk ) S21 (-sk )=0. Similarly, the finite-nonzero roots sr of  h(s)= 0 at which  S21 (sr )S21 (-sr )=1 are called “finite reflection zeros (FRZ)” of the two-port.

v) Total number of transmission zeros is equal to the total number of reflection zeros. Degree of a filter is equal to the larger one of degrees of f(s) and h(s). Therefore degree of a filter is also equal to the total number of transmission zeros or reflection zeros. Each transmission zero at s=jw=0 or at s=jw= adds one degree while finite transmission zeros add double degrees.

Filter transmission and reflection characteristics are shaped by the multiplicities and positions of transmission and reflection zeros on s= s+jw plane. Since almost all signals of interest are on s=jw axis (sinusoidal signals) they can be most efficiently filtered by choosing all transmission and reflection zeros on s= jw axis (optimal amplitude filtering). The zeros at s= and s=0 are considered to be on jw axis. Hence for optimal filtering finite transmission and reflection zeros sk and sr will have the forms

sk =jwk     and     sr = jwr	(2.5.14)

to be on s=jw axis. They must also come in conjugate pairs to form real polynomials :

f(s)= fm smo  (s-jwk ) (s+jwk )= fm smo  (s2 + wk2 )	(2.5.15)

h(s)= hn sno  (s-jwr ) (s+jwr )= hn sno  (s2 + wr2 )	(2.5.16)

It is seen that f(s) is an even polynomial if m0 is even and it is an odd polynomial if m0 is odd. We also notice that when all finite reflection zeros are chosen on s-jw axis, h(s) becomes also an even or odd polynomial. h(s) is an even polynomial if n0 is even and it is an odd polynomial if n0 is odd. That is, the parity of transmission and reflection zeros at f=0 sets the parity of f(s) and h(s). Using the even / odd properties of f(s) and h(s) in Equations 2.5.3 and 2.5.4, we can make the following observations:

- For electrically symmetric two ports (S11 = S22 ), if f(s) is even, then h(s) is odd and vice versa.

- For electrically antimetric two ports (S11 = -S22), both f(s) and h(s) are even polynomials. Both f(s) and h(s) can’t be odd, otherwise g(s) would also have a root at s=0, which is not allowed in Strictly Hurwitz polynomials. Also it is physically meaningless to place both a transmission zero and a reflection zero at the same frequency, s=0.

From 2.5.15 and 16 we get the functions

f(s)f(-s) = fm2 s2mo  (s2+wk2 )2 	(2.5.17)

and

h(s)h(-s) = hn2 s2no (s2+wr2 )2 	(2.5.18)

which will in turn give power gain functions of the form

G(w)= S21(jw) S21(-jw)= 1/ [1+ F2(w)] 	(2.5.19)

where

F(w) = (hn / fm) w (no-mo) [ (w2 -wr2 ) / (w2 -wk2 )] 	(2.5. 20)

with either no or mo being zero. G(w) may also be written in the expanded form

(2.5.21)

Either of the functional forms given in Equations 2.5.19 or 21 can be used in “amplitude filtering” approximations. They can be tailored to various forms for lowpass (LP), highpass (HP), bandpass (BP) or bandstop (BS) filters by placing the zeros in proper positions and by adjusting the degrees n and m of polynomials. Clearly, all reflection zeros must be placed in passband(s) to facilitate transfer of power to output with minimal reflection and all transmission zeros must be placed in stopband(s) to maximize attenuation of signals in these frequency bands. The two classical techniques to approximate flat gain (unity or zero) within a finite range are as follows:

i) Maximally flat approximation: If all zeros of a function are collected at a single point, then the function will be maximally flat around that point.

ii) Equiripple approximation: By properly locating zeros and poles of a function, the function can be made to deviate from the targeted constant value in an equal ripple manner within a certain range.

Both approaches can be used to approximate pass and stop bands of filters.

Some commonly used approximation types for filters which are also available in Filpro are as follows :

- Maximally flat behaviors in both pass and stop bands (Butterworth filters) (Figure 2.9.a for a LPF)

- Maximally flat passband, equal ripple stopband (Inverse Chebychev filters) (Figure 2.9.b)

- Maximally flat passband general stopband filters (Figure 2.9.c)

- Equal ripple passband maximally flat stopband  (Chebychev filters) (Figure 2.9.d)

- Equal ripple passband equal ripple stopband (elliptic filters) (Figure 2.9.e)

- Equal ripple passband general stopband filters (Figure 2.9.f)

Figure 2.9. Some classical approximation techniques of unity gain between f=0 and f=fp   (lowpass filters).

Once the desired maximum passband attenuation and minimum stopband attenuation are set together with the type of approximation, then the positions of reflection and transmission zeros are no longer independent of each other and usually only the positions of transmission zeros are selected by the designer. In Synthesis option of Filpro user can select the type of approximation and specify the transmission zeros. The transfer funcion(s) are then automatically formed to give the specified response. Details of these subjects are beyond the scope of this manual and can be found in many classical textbooks on filters.

The filters that can be synthesized in Filpro have the following transmission zero constraints:

- Lowpass Filters: Lowpass filters must have at least one transmission zero at w= (n >0). Therefore n>m and degree of filter will be equal to n. No transmission zero is allowed in the passband w wp. Therefore no transmission zero exists at w= 0 (m0 = 0). That is f(s) is always even for lowpass filters. So, symmetrical filters will be obtained if h(s) is an odd polynomial (odd degree, n) and antimetric filters will result for even h(s) (even degree, n). Finite transmission zeros, if there are, have to be placed at frequencies above the passband edge frequency (wk > wp). All reflection zeros lie in the range 0 wr  wp. Therefore we may have n0 >0. Since n0 sets the parity of h(s) (odd n0 means odd n), the parity of reflection zeros at s= 0 sets the symmetry or antimetry of the resulting structure.

- Highpass Filters: Highpass filters must have at least one transmission zero at w=0 (m0 >0). No transmission zero is allowed in the passband wwp. So, no transmission zero exists at w= . Therefore m  n and degree of filter will be set by the degree m of f(s). Finite transmission zeros, if there are, must be placed below the passband edge frequency (fp ). Since all reflection zeros must be in the passband, wwp, no reflection zero exists at w=0 (n0 = 0). Hence h(s) is an even polynomial for highpass filters. For this reason, if f(s) is odd (odd degree m), we get symmetric filters and if f(s) is even (even degree, m), we get antimetric filters.

- Bandpass Filters: Usual bandpass filters have transmission zeros both at w=0 and w=. Since there are some transmission zeros at w=, we have n>m and degree of the filter is set by the degree n of h(s). If no transmission zero is placed at w=0, then the filter may also be treated as a quasi-lowpass filter with some loss near w=0. If no transmission zero is placed at w=, then the filter may be treated as a quasi-highpass one, with some loss at high frequencies. Finite transmission zeros, if there are, have to be placed outside the passband. Bandpass filters have no reflection zeros at w=0 and w=. Therefore n0 = 0, giving even parity for h(s). Only finite reflection zeros are allowed, all of them being inside the pass band. Since h(s) is even, and since the degree of bandpass filters is set by h(s), bandpass filters will always have even degree. Symmetric filters will be obtained if f(s) is odd (odd number m0 of transmission zeros at w=0) and antimetric filters will be obtained for even f(s)(even number of transmission zeros at w=0).

- Bandstop Filters: Bandstop filters must not have transmission zeros at w=0 (m0=0, even m, even f(s)) and at w= (m>n). Thus, degree of the filter is equal to the degree m of f(s) and is always even. Only finite transmission zeros are allowed for bandstop filters, all to be placed inside the stop band. Reflection zeros lie in the lower and upper pass bands. No reflection zero exist inside the stopband. The multiplicity n0 of reflection zeros at w=0 sets the symmetry (if n0 is odd) and antimetry (if n0 is even).

2. 6. FILTER SPECIFICATIONS IN TERMS OF INSERTION LOSS

Typical gain of filters lie in the ranges

0.5  G(w) 1          for w in passband

10-10  G(w) 10-2     for w in stopband

Such a wide dynamic range for gain is more practical to express in logarithmic scale decibel (dB). The concepts of insertion loss (IL) and return loss (RL) are used for that purpose. They are defined as follows:

IL= (w)=10log (1/|S21|2)= 10log(1/G) (dB)	(2.6.1)

RL=10log (1/|S11|2)                              (dB) 	(2.6.2)

Insertion loss becomes infinite at transmission zeros and becomes zero at reflection zeros. In this respect, it can be treated as a band tied to zero at reflection zeros and tied to infinity at transmission zeros to approximate a filter characteristics as shown in Figure 2.10 for a lowpass filter.

Figure 2.10. Transmission and reflection zeros in shaping up pass and stop bands in a lowpass filter.

In the usual practice, filter specifications are given in terms of insertion loss limits in passbands and stopbands in “brick wall” form as follows:

- For lowpass filters (Figure 2.11.a),

0  (w)p      for   w wp             (passband)

(w)  s               for   wws                (stopband)

- For highpass filters (Figure 2.11.b),

(w)  s             for    w ws                        (stopband)

0  (w)  p   for    wwp              (passband)

- For bandpass filters (Figure 2.11.c),

(w)  s1       for      w ws1               (lower stopband)

0  (w)  p  for      wp1 w  wp2     ( passband)

(w)  s2       for      wws2                (upper stopband)

- For bandstop filters (Figure 2.11.d),

(w)  p       for   w wp1                  (lower passband)

w)  s       for   ws1 w ws2           (stopband)

(w)  p       for   wwp2                   (upper passband)

In equiripple passband filters p1, 2 are called maximum passband losses or passband ripples and in maximally flat passband cases they are called passband edge loss. s1,2 are called minimum stopband losses or stopband edge losses. wp1,2 and ws1,2 denote passband and stopband edge frequencies. The actual response will lie within these limits. The region between a passband edge and the adjacent stopband edge is called transition band. The slope of response in transition band (skirt slope) shows “selectivity” of the filter. Transition bandwidth should not be made too narrow, as the practical circuits with reasonable number of elements can’t approximate such “too high” selectivities and negative element values may result in synthesis stage. Although some theoretical works exist on this subject, the limits of selectivities are usually found by trial.

Typical values for p and s lie in the ranges

p=0.01 dB -to- 3 dB ,              s=20-30 dB -to- 100-150 dB

depending on the requirements of application.

Figure 2.11. Brick-wall type insertion loss specifications for a) lowpass, b) highpass, c) bandpass   d) bandstop filters.

Figure 2.12. Insertion and return loss of an equal ripple passband bandpass filter.

Return Loss concept is usually used in applications where very good impedance matching is required in passband(s) because it is usually a “clearer” indicator of mismatch than passband ripple. Typical return loss limits lie in the range

10 dB  RL  30 dB

Return loss becomes infinite at reflection zeros (reflected power is infinite dB smaller than the incident power) and it becomes zero at transmission zeros (reflected power is equal to incident power). In Figure 2.12 return loss of an equal ripple passband bandpass filter is shown. Note that at frequencies where insertion loss is zero (reflection zeros) return loss becomes infinite and at frequencies where passband ripple takes its maximum value, return loss assumes its minimum value.

Exercise 2.1: Design some lumped element Chebychev bandpass filter using “Prototype” approach with the default values given in Filpro for all specification parameters except the passband ripple. For passband ripple use p = 0.01, 0.05, 0.1, 0.5 and 1 dB. For each design, overplot insertion loss and return loss and find out the minimum passband return loss values corresponding to the specified maximum passband insertion loss values (passband ripple). Inspect also the link between passband ripple and skirt slope. Read the reflection zero frequencies.

2. 7. ELEMENT EXTRACTION

Once the proper transducer power gain function G(w) is formed to approximate the required specifications then the next stage is extraction of the parameters S21(s) and S11(s) using the relations

S21 (s) S21 (-s)=G(w)|w=-js	(2.7.1)

S21 (s)= f(s) /g(s)	(2.7.2)

S11 (s)= h(s) /g(s)	(2.7.3)

In Equation 2.7.1 the zeros and poles of S21(s) S21(-s) are shared between S21(s) and S21(-s) by observing the properties of polynomials f(s) and g(s) stated in Belevitch Representation Theorem. Then h(s) is solved from Feldtkeller Equation

h(s)h(-s)= g(s)g(-s) - f(s)f(-s)	(2.7.4)

now by taking the analytical properties of h(s) into consideration. Once S21(s) and S11(s) are formed, all the other two-port parameters (Z, Y, ABCD, etc.) can be found using interrelations among them. In general the resulting h(s) (hence the parameters S21 , S11 , etc.) is nonunique because of freedom of choice in sharing the zeros of h(s) h(-s) between h(s) and h(-s). However, for the cases where all the roots of h(s) are on s=jw axis (optimal filters), the roots of h(s)h(-s) are double roots, still on s=jw axis. Therefore h(s) and h(-s) have identical roots, simplifying the root sharing problem. Even in these simplest cases where no choice exist for zeros, the choice of + or - sign for h(s) yields two circuits which are called “dual” circuits. The circuit(s) corresponding to these parameters are found by extracting elements from driving point functions like S11 (s), Zin (s), Yin (s), Z11(s), Y11 (s), etc. In Filpro, elements are extracted from either Z11 (s) or Y11 (s), which from now on will be referred to as Z(s) and Y(s). They are reactance functions, because their output port is either open or short circuited, eliminating the only resistive component of the circuit, RL. Therefore the element extraction techniques described in Section 2. 2 can be used. Element extraction is carried out in such a way as the resulting circuit will possess the transmission and reflection zeros immersed in the gain function G(w). The simplest practical structures are obtained by extracting groups of elements called “transmission zero sections” in cascaded form. Each section is responsible for creation of a transmission zero. A transmission zero is formed either by creating an open circuit in the series arm (Type-A TZ) or by creating a short circuit in the shunt arm (Type-B TZ) to stop signal flow between input and output. The three kinds of transmission zeros on s=jw axis are realized as follows:

- Transmission zeros at w=0 can be realized by either a series capacitor (becoming a series arm open circuit at w=0, hence Type-A) or a shunt inductor (becoming a shunt arm short circuit at w=0, hence Type-B) (Figure 2.13).

- Figure 2.13. Elements producing transmission zeros at w=0

- .

As described in Section 2.2, Z(s)= Z11 (s) has either a pole or a zero at s=0. The + or - sign choice in forming h(s) in Equation 2.7.4 sets whether Z(s) will have a zero or a pole at s=0. If Z(s) has a pole, then this pole can be extracted as a series arm capacitor of value

C=[1/ sZ(s) ]s=0 	(2.7.5)

to create a transmission zero at s=0. If, instead, Z(s) has a zero at s=0, then it means Y(s) = 1/Z(s) has a pole at s=0 which can be extracted as a shunt arm inductance of value

L = [ Z(s)/ s]s=0 	(2.7.6)

If there are more than one transmission zeros at s=0, then they are realized alternately by series capacitors and shunt inductors because, as it was stated in studying lossless one-ports that, if a pole of input impedance (admittance) at s=0 is extracted, then a zero appears at that frequency automatically, acting as the pole of the remaining input admittance (impedance).

Exercise 2.2: Design a Chebychev highpass filter using either “Prototype” or “Synthesis” approach with degree five (five TZs at w= 0). Find (or synthesize) also “dual” filter and inspect element types to verify above statements.

- Transmission zeros at w= can be realized by either a series inductor or a shunt capacitor (Figure 2.14). Series inductor becomes open circuit (Type-A transmission zero) while shunt capacitor becomes short circuit at w= (Type-B transmission zero) to block signal path to output port.

Figure 2.14. Elements for producing transmission zeros at w=.

Z(s) has either a pole or a zero at s=, depending on the choice of + or - sign in Equation 2.7.4. If Z(s) has a pole, then it can be extracted as a series arm inductance of value

L=[Z(s)/s]s= 	(2.7.7)

If Z(s) doesn’t have a pole at s=, then it has a zero there which means Y(s) = 1/Z(s) has a pole at s=. It can be removed as a shunt arm capacitor of value

C=1/[sZ(s)]s= 	(2.7.8)

If more than one transmission zeros exist at s=, then they are realized as series inductors and shunt capacitors alternately.

Exercise 2.3: Design a Chebychev lowpass filter of degree five (five TZs at s=) using both “Synthesis” and “Prototype” approaches. Find (or synthesize) their duals to verify above statements.

- Finite transmission zeros can be produced either by a parallel resonator in series arm which becomes a series arm open circuit at its resonance frequency or by a shunt arm series resonator which becomes short circuit at resonance to stop signal flow to output port (Figure 2.15).

Figure 2.15. Elements for producing finite transmission zeros.

Contrary to the extraction of transmission zeros at s=0 or s=, finite transmission zeros can rarely be extracted in such a direct manner. This is because in order to extract such sections, Z(s) or Y(s)= 1/Z(s) must have a pole at the finite transmission zero frequency. Usually this is not the case because of existence of transmission zeros at s=0 and /or s=. However by extracting a piece of transmission zero from either s=0 or from s= (partial pole extraction) it is possible to form a remainder impedance function with a pole or zero at the desired transmission zero frequency s=jwi as follows:

i) If Z(s) has a pole at s=0, then part of that pole can be extracted (as a series capacitor C1) such that the remaining impedance function Z1(s) will have a zero at s=jwi :

Z1(s)|s=jwi = Z(jwi )-1/(jwiC1 )=0 	(2.7.9)

Hence, the value of capacitor is found as (hopefully positive !)

C1 = [1/ sZ(s)]s=jwi 	(2.7.10)

Now Z1 (s) has a zero at s=jwi, which means Y1(s)=1/Z1(s) has a pole at that frequency. It can be extracted as a shunt arm series resonator with element values

1/L2=[(s2+wi2) Y1 (s)/s]s=jwi     ,            C2 = 1/ (wi2 ) L2 	(2.7.11)

The resulting structure is shown in Figure 2.16.a. Partial extraction of the pole at s=0 shifts the zeros of Z(s) towards s=0, one of which will be coinciding with the desired zero at s=jwi.

ii) If Z(s) has a pole at s=, then zero shifting process may be carried out by partial removal of that pole in the same manner, yielding now a series arm inductor followed by a shunt arm series resonator, as shown in Figure 2.16.b.

iii) If Y(s) has a pole at s=0, then zero shifting process results in a shunt inductor followed by s series arm parallel resonator, as shown in Figure 2.16.c.

iv) If Y(s) has a pole at s= then the same procedure gives a shunt capacitor followed by a series arm parallel resonator (Figure 2.16.d).

In these processes there is no guarantee to get positive element values under all conditions. Especially filters with too tight conditions (e.g. ws too close to wp , or too small passband ripple, etc.) may yield negative values in shifting zeros in some of the approaches described above. Trial-error and readjustment of filter specifications are the usual methods in such cases. In bandpass filters, experience shows that possibility of getting negative element values decrease by extracting finite transmission zeros in lower stopband by partial pole removal from s=0 and finite transmission zeros of upper stopband by partial pole removal from s=. Filpro is programmed that way.

Figure 2.16. Realization of finite transmission zeros by partial removal of a) a pole of Z(s) at s=0, b) a pole of Z(s) at s=, c) a pole of Y(s) at s=0, d) a pole of Y(s) at s=.

Since these filters are synthesized using only input immitance functions (Z11 or Y11), the load resistance RL doesn’t involve anywhere in the formulations for element extractions. Therefore the resulting structures may need an ideal transformer at the load end to get a match with the specified load resistance RL. Since all transmission and reflection zeros are chosen to be on s=jw axis, the resulting filters will have either electrical symmetry (S11 = S22) or electrical antimetry (S22 = -S11). This property enables us to determine the required ideal transformer automatically at the end of synthesis procedure. In Filpro calculation of the required transformer ratio is carried out by evaluating the insertion loss of synthesized filter without transformer at a reflection zero frequency. This loss is then compensated by inserting a transformer of proper turns ratio. Electrical symmetry of filters do not guarantee physical symmetry for the resulting structure. The nature of symmetry/ antimetry and transformer problem is different in lowpass, highpass and bandpass filters, as will be demonstrated in the following examples.

i) Electrically Symmetric Lowpass Filters:

If the number of transmission zeros at f= are odd, then electrically symmetric filter structures will be obtained at the end of element extraction process. That is, the input and output ports of these filters may be interchanged without causing any change in response. No transformer will be needed in synthesis of such filters if RL = RS. However these structures come out to be physically symmetric only for some special cases. If all transmission zeros are at f=, then resulting structures will always be physically symmetric. If finite transmission zeros exists, then physical symmetry can be obtained only for a careful choice of finite transmission zero frequencies and multiplicities.

EXAMPLE 2. 1: Synthesis of an electrically symmetric lowpass filter.

Let the specifications of a lowpass filter be given as p=0.1 dB, fp=100 MHz, with transmission zeros Ninf =3, FTZ1=120 MHz, FTZ2 =130 MHz, FTZ3=150 MHz.

Steps of synthesis procedure are as follows:

New / Design / By Synthesis

Filter Type: Lumped, Lowpass, Equiripple, Doubly Terminated, Type A (or Type B)

Passband Specs and Terminations: PBRipple = 0.1, fp = 100, RS = 50, RL = 50

Transmission Zeros: INF, INF, INF, FTZ 1 (at 120 MHz), FTZ 2 (at 130 MHz), FTZ 3 (at 150 MHz)

As these transmission zeros are entered, insertion loss response is automatically displayed on the screen, as shown in Figure 2.17.

Extract Transmission Zeros:

Transmission zeros can be extracted in different orders by invoking the commands Zero, Infinity or Finite Transmission Zero. Some examples are shown in Figure 2.17 where the orders of extractions are also indicated. Note that no transformer exists in these circuits. Standard Auto Extract command is enabled if some preset numbers of transmission zeros exist at zero and infinity, yielding a preset topology.

Figure 2.17. Command set for synthesis and various realizations of an electrically symmetric lowpass filter.

Figure 2.17. Command set for synthesis and various realizations of an electrically symmetric lowpass filter (continued).

Exercise 2. 4: Discover new topologies by extracting transmission zeros in different orders in Type A and Type B forms.

Exercise 2. 5: Repeat the design with Ninf = 1, with all other parameters being the same and comment on the types of finite transmission zero sections.

Exercise 2. 6: Repeat the design with all finite transmission zeros placed at the same frequency, e.g. f = 130 MHz. Extract transmission zeros in such a way that a physically symmetric filter will result.

Exercise 2. 7: Repeat the design by placing double transmission zeros at each frequency f = 120 MHz and f = 130 MHz. Extract transmission zeros in such a way that a physically symmetric filter will result.

Exercise 2. 8: Repeat the design by placing double transmission zeros at each frequency f = 120 MHz and f = 130 MHz and a single transmission zero at f = 140 MHz. Extract transmission zeros in such a way that a physically symmetric filter will result (Extract the transmission zero at f = 140 MHz in the middle of the structure).

ii) Electrically Antimetric Lowpass Filters:

Lowpass filters with even number of transmission zeros at f= (even degree lowpass filters) are electrically antimetric filters (S22 = -S11). For such filters, the required load resistance RL (to get the specified filter characteristics) is always different from RS. Therefore if the condition RL=RS is enforced, then an ideal transformer will result at the load end of the synthesized structure. This transformer may be absorbed in the load (using “Shift Transformer/Absorb Trf in RL” command) to get the proper load resistance. Physical antimetry shows itself through the property

RS RL= Z1 Zn = Z2 Zn-1 = ..

where Zi are impedances of elements of the filter. If all transmission zeros are at infinity, then we automatically get physically antimetric filters. If finite transmission zeros exist, then frequencies and multiplicities of them requires care to choose to get physical antimetry.

EXAMPLE 2. 2: Synthesis of an electrically antimetric lowpass filter.

Let the specifications of a lowpass filter be given as p = 0.1 dB, fp = 100 MHz, with two transmission zeros at f= 0 and three finite transmission zeros at 120, 130 and 150 MHz. Following the same steps as in the previous example, we get the response shown in Figure 2.18. Some circuits realized by extracting transmission zero sections in different orders are also shown in the same figure. Repeat the designs to find out the element values and transformer turns ratios. Discover some new topologies by extracting transmission zeros in different order.

Figure 2.18. Insertion loss response and some realizations for an electrically antimetric lowpass filter.

Exercise 2.9: Repeat the design with Ninf = 4 by extracting transmission zero sections in different orders. Compare the available types of finite transmission zero sections with those of the previous example. Compare also the dynamic ranges of element values.

Exercise 2.10: Design an equiripple lowpass filter of degree N=20, with all transmission zeros being at s=. Use antimetry relation between the element values to judge the accuracy of element extraction procedure of Filpro for various values of passband ripple. Repeat the procedure for higher degrees. Repeat also for maximally flat filters.

iii) Electrically Symmetric Highpass Filters:

If the number of transmission zeros at f=0 are odd, then electrically symmetric highpass filters will be obtained. That is, the input and output ports of these filters may be interchanged without causing any change in response. No transformer will be needed in synthesis of such filters if RL= RS. However, similar to the electrically symmetric lowpass filters, these structures come out to be physically symmetric only for some special cases. If all transmission zeros are at f=0, then resulting structures will always be physically symmetric. If finite transmission zeros exists, then physical symmetry can be obtained only for a careful choice of finite transmission zero frequencies and multiplicities.

EXAMPLE 2. 3: Synthesis of an electrically symmetric highpass filter.

Let the specifications of a lowpass filter be given as p = 0.1 dB, fp = 100 MHz, with transmission zeros Nzero = 3,  FTZ1= 50 MHz,  FTZ2= 80 MHz,  FTZ3= 90 MHz.

Steps of synthesis procedure are as follows:

“New / Design / By Synthesis”

“Filter Type: Lumped, Highpass, Equiripple, Doubly Terminated, Type A (or Type B)”

“Passband Specs and Terminations: PBRipple = 0.1, fp = 100, RS = 50, RL = 50”

Transmission Zeros: Zero, Zero, Zero, FTZ 1 (at 50 MHz), FTZ 2 (at 80 MHz), FTZ 3 (at 90 MHz)

As these transmission zeros are entered, insertion loss response is automatically displayed on the screen, as shown in Figure 2.19.

Extract Transmission Zeros:

Transmission zeros can be extracted in different orders. Some examples are shown in Figure 2.19 where the orders of extractions are also indicated. Note that no transformer exists in these circuits.

Figure 2.19. Insertion loss response for an electrically symmetric highpass filter.

Figure 2.20. Some realizations for an electrically symmetric highpass filter.

Exercise 2.11: Discover new topologies by extracting transmission zeros in different orders in Type A and Type B forms.

Exercise 2.12: Repeat the design with Ninf = 1 in Type A and Type B forms and compare the types of finite transmission zero sections. Compare also the dynamic range of element values.

Exercise 2.13: Design some odd order equiripple highpass filters with all transmission zeros at s=0 to observe symmetry. Increase the order until numerical accuracy problems show up to find out limitations of Filpro for various passband ripple values.

- iv) Electrically Antimetric Highpass Filters:

- Highpass filters with even number of transmission zeros at f=0 (even degree highpass filters) are electrically antimetric filters (S22= -S11). For such filters, the required load resistance RL (to get the specified filter characteristics) is always different from RS. Therefore if the condition RL=RS is enforced, then an ideal transformer will result at the load end of the synthesized structure. This transformer may be absorbed in the load (using “Shift Transformer / Absorb Trf in RL” command) to get the proper load resistance.

EXAMPLE 2.4: Synthesis of an antimetric highpass filter.

Let the specifications of a highpass filter be given as p = 0.1 dB, fp = 100 MHz, with two transmission zeros at f = 0 and three finite transmission zeros at 50, 80 and 90 MHz. Following the same steps as in the previous example, we get the response and some realizations shown in Figure 2.21 and 22.

Figure 2.21. Insertion loss of an electrically antimetric highpass filter.

Figure 2.22. Some realizations for an electrically antimetric highpass filter.

Exercise 2. 14. Repeat the designs to find out the element values and transformer turns ratios. Discover some new topologies also

v) Electrically Symmetric Bandpass Filters:

Bandpass filters with odd number of transmission zeros at f=0 come out to be electrically symmetric. If there is no finite transmission zero, then physically symmetric structures can be obtained by alternately extracting transmission zeros at f=0 and f=. Random extraction of these transmission zeros will lead to physically unsymmetric structures with transformers at the load end for RL = RS cases. If there are finite transmission zeros than physical symmetry is possible only if finite transmission zero frequencies and multiplicities permit a symmetric transmission zero extraction process (like for example, all finite transmission zeros located at the same frequency or all finite transmission zeros are of even multiplicity, etc.).

EXAMPLE 2. 5: Synthesis of an electrically symmetric bandpass filter.

Let’s consider a bandpass filter with all finite transmission zeros in lower stopband. Let the specifications of filter be given as p= 0.1 dB, fp1 =40 MHz, fp2 = 60 MHz with transmission zeros Nzero=1, Ninf =3, FTZ1=20 MHz, FTZ2=27 MHz, FTZ3=35 MHz.

Steps of synthesis procedure are as follows:

“New / Design / By Synthesis”

“Filter Type: Lumped, Bandpass, Equiripple, Doubly Terminated, Type A (or Type B)”

“Passband Specs and Terminations: PBRipple = 0.1, fp1 = 40, fp2 = 60, RS = 50, RL = 50”

Transmission Zeros: Zero, Inf, Inf, Inf, FTZ 1 (at 20 MHz), FTZ 2 (at 27MHz), FTZ 3 (at 35MHz)

As these transmission zeros are entered, insertion loss response is automatically displayed on the screen, as shown in Figure 2.23.

Figure 2.23. Insertion loss response of an electrically symmetric bandpass filter with Nzero=1, Ninf=3 and all finite transmission zeros in the lower stopband..

Figure 2.24. Some realizations for an electrically symmetric bandpass filter with Nzero=1, Ninf=3 and all finite transmission zeros in the lower stopband..

Extract Transmission Zeros:

Transmission zeros can be extracted in different orders. Some examples are shown in Figure 2.24 where the orders of extractions are also indicated. Note that a transformer exists in all circuits, because physical symmetry is not possible with the chosen finite transmission zeros. Note also that all finite transmission zeros are extracted by partial pole removal from f = 0. This is the usual approach to decrease the possibility of producing negative elements in the extraction process. Lower stopband finite transmission zeros extracted by partial pole removal from f = 0 while upper stopband finite transmission zeros are extracted by partial pole removal from f = . Since there is only one transmission zero at f= 0, all finite transmission zero sections of the circuit are of the same type.

Exercise 2. 15: Discover new topologies by extracting transmission zeros in different orders in Type A and Type B forms

Exercise 2. 16: Response and some realizations of an electrically symmetric bandpass filter with all transmission zeros in lower stopband are shown in Figures 2.25 and 26. Specifications are p = 0.1 dB, fp1 = 40 MHz, fp2 = 60 MHz with transmission zeros Nzero= 3, Ninf= 1, FTZ1= 25 MHz, FTZ2= 30 MHz. Repeat the designs to find element values and transformer ratios. Note that all finite transmission zeros are extracted by partial pole removal from f= 0. Since in this circuit there are more than one transmission zeros at f=0, more variations are possible for realization of transmission zero sections, as is clear from the synthesized circuits.

Figure 2.25. Insertion loss response of an electrically symmetric bandpass filter with Nzero=3, Ninf=1 and all finite transmission zeros in lower stopband.

Figure 2.26. Some realizations for an electrically symmetric bandpass filter with Nzero=3, Ninf=1 and all finite transmission zeros in lower stopband.

Exercise 2. 17: In Figure 2.24 response and some realizations of an electrically symmetric bandpass filter with Nzero=Ninf=1 are shown. Specifications are p = 0.1 dB, fp1 = 40 MHz, fp2 = 60 MHz with all finite transmission zeros in lower stopband, at frequencies FTZ1= 20 MHz, FTZ2= 30 MHz, FTZ3=35 MHz. Repeat the designs to find element values and transformer ratios. Note that all finite transmission zeros are extracted by partial pole removal from f = 0.

Figure 2.27. Insertion loss response and some realizations for an electrically symmetric bandpass filter with Nzero= Ninf=1 and with all finite transmission zeros in lower stopband.

Exercise 2. 18: In Figure 2.28 response and some realizations of an electrically symmetric bandpass filter with all transmission zeros in upper stopband are shown. Specifications are p = 0.1 dB, fp1 = 40 MHz, fp2 = 60 MHz with transmission zeros Nzero= 1, Ninf= 3, FTZ1= 70 MHz, FTZ2= 80 MHz. Repeat the designs to find element values and transformer ratios. Note that all finite transmission zeros are extracted by partial pole extraction from f = .

Figure 2.28. Insertion loss response and some realizations for an electrically symmetric bandpass filter with all finite transmission zeros in upper stopband, Nzero=1 and Ninf=3.

Exercise 2. 19: Response and some realizations of an electrically symmetric bandpass filter with Nzero= Ninf=1 are shown in Figures 2.29 and 30. Specifications are p = 0.1 dB, fp1 = 40 MHz, fp2 = 60 MHz, with all finite transmission zeros in upper stopband at FTZ1= 70 MHz, FTZ2= 80 MHz, FTZ3= 100 MHz. Repeat the designs to find element values and transformer ratios.

Figure 2.29. Insertion loss response of an electrically symmetric bandpass filter with Nzero= Ninf=1 and with all finite transmission zeros in upper stopband.

Figure 2.30. Some realizations for an electrically symmetric bandpass filter with Nzero= Ninf=1 and with all finite transmission zeros in upper stopband.

Exercise 2. 20: In Figure 2.31 response and some realizations of an electrically symmetric bandpass filter with finite transmission zeros in both lower and upper stopband are shown. Specifications are p = 0.1 dB, fp1 = 40 MHz, fp2 = 60 MHz with transmission zeros Nzero=3, Ninf =3, FTZ1=35 MHz, FTZ2=65 MHz, FTZ3=80 MHz. Repeat the designs to find out element values.

Figure 2.31. Insertion loss response and some realizations for an electrically symmetric filter with finite transmission zeros in both stopbands.

vi) Antimetric Bandpass Filters

Bandpass filters with even number of transmission zeros at f=0 will result in antimetric structures, having always a transformer for the condition RL=RS. Design some exercises similar to the exercises of symmetric bandpass filters presented above.

It is clear that synthesis problems have nonunique solutions because transmission zeros can be realized in more than one ways and the order of extraction of transmission zero sections can be changed. This property is useful in producing equivalent alternative solutions in the search of an easy to realize structure. The structures resulting from synthesis procedures are rarely in desired topologies. Further, the element values are usually unrealizable high or low. Therefore it is usual to apply some circuit transformations to get realizable circuits. In most of these transformations ideal transformers are produced. These transformers can be used to eliminate the transformers resulting from synthesis, as will be seen in the applications that will be presented in the succeeding chapters.

2. 8. FILTER DESIGN FROM LC PROTOTYPES

Many practical filter design problems can be adequately solved by applying proper frequency (reactance) transformations to the readily available lowpass prototype filters to get desired lowpass, highpass, bandpass or bandstop filters. Element values of the prototype filters are available in many handbooks, textbooks or in scattered publications, either in tabular form or formulas are given to calculate them. The “Synthesis” approach of Filpro can also be used to produce further lowpass prototype structures not available from literature. The four classical (most commonly used) lowpass prototype topologies are shown in Figure 2.32. The one in Figure 2.32.a (termed as Type A in Filpro) and its dual, in Figure 2.32.b (termed as Type B in Filpro) are for simple filters with all transmission zeros at infinity, like Butterworth and Chebychev filters. The structure of Figure 2.32.c (Type A)and its dual, in Figure 2.32.d (Type B) are for filters with finite transmission zeros, like Elliptic, inverse Chebychev, generalized Chebychev filters, with one transmission zero at f=. Element values (gi ) are usually given in magnitude normalized (RS = 1) and frequency normalized (wp=1) form. The classical reactance transformations are applied as follows:

Impedances of inductors and capacitors of prototypes in the normalized frequency w ' are

ZL= jw'L'  and           ZC = 1/w'C'	(2.8.1)

where L' and C' are normalized inductance and capacitance values, denoted as gi in Figure 2.27. Desired lowpass, highpass, bandpass and bandstop filter structures and element values are obtained from these impedance expressions simply by replacing the normalized frequency w' with a new frequency w which is related to w' in a special way to yield the desired filter type, as described below.

Figure 2.32. Lowpass prototype structures for a) Type-A filters with all transmission zeros at infinity, b) Type-B filters with all transmission zeros at infinity, c) Type-A filters with finite transmission zeros, and d) Type-B filters with finite transmission zeros.

- Lowpass filters: Transformation function is

w'=w/wp 	(2.8.2)

where wp is the passband edge frequency of the desired lowpass filter (Figure 2.33.a). Using 2.8.2 in 2.8.1 we get

ZL=jw'L'=jw(L'/wp )=jwL

ZC =1/jw'C'=1/jw(C'/wp )=1/jwC 	(2.8.3)

where L and C are the values of inductors and capacitors of the actual lowpass filter evaluated from 2.8.3 as

L=L'/wp,

C=C'/wp 	(2.8.4)

Hence, by simply replacing all inductors L' and capacitors C' of the prototype by the new inductors L and capacitors C respectively, the desired lowpass filter will be obtained (Figure 2.33). Once the specifications “fp , p , fs and s” are given, the prototype that will yield the desired filter is found by first finding ws'=ws/wp and then searching for the proper degree that has the specified s at ws', in the tables with the properp.

Exercise 2. 21: Using “Design / By Built-in Prototype” approach design a Chebychev lowpass filter with the default specifications in Filpro. Plot insertion loss response. Apply “Find Dual” command to get the dual filter. Compare number of capacitors and inductors of the two circuits.

Exercise 2. 22: Repeat the previous exercise by designing a Generalized Chebyshev-1 lowpass filter with default specifications.

(Further examples and exercises on actual filters will be given in Chapter 3)

Figure 2.33. Prototype-to-Lowpass mapping.

- Highpass filters: Frequency transformation function is

w'= -(wp )/w 	(2.8.5)

Using this expression in Equation 2.8. 1 yields

ZL= jw'L'= -jwpL/w=1/jwC	(2.8.6)

ZC =1/jw'C'= -w/(jwpC') =jwL	(2.8.7)

where

C=1/(wpL')   and      L=1/(wpC')	(2.8.8)

Figure 2.34. Prototype-to-highpass mapping.

Hence an inductor of the prototype becomes a capacitor and a capacitor of the prototype becomes an inductor in the resulting highpass filter (Figure 2.34). Once the specifications “fp , p, fs and s” are given, the prototype that will yield the desired filter is found by first finding ws'=|wp/ws| and then searching for the proper degree that has the specified s at ws', in the tables with the properp.

Exercise 2.23: Design a Chebychev highpass filter using “Design/ By Built-in Prototype” approach. Plot insertion loss response. Use “Find Dual ”command to get the dual circuit. Compare numbers of capacitors and inductors in the two circuits.

Exercise 2.24: Repeat the previous exercise on a Generalized Chebyshev-1 highpass filter.

(Further examples and exercises will be given in Chapter 4.)

- Bandpass filters: Frequency transformation function is

w'= (wo /B) (w/wo -wo /w)		(2.8.9)

where B = wp2 -wp1 is the pass bandwidth of the bandpass filter with wp1 and wp2 denoting the passband edge frequencies. wo is termed as geometric center of the passband (Figure 2.35), calculated as

wo2 = wp1wp2		(2.8.10)

Substitution of 2.8.9 into 2.8.1 yields an expression which is similar to impedance of a series LC resonator:

ZL = jw'L'=jw(L'/B) - jL'wo2 /(wB) = jwL+1/(jwC)		(2.8.11)

giving

L=L'/B

C=1/(Lwo2)		(2.8.12)

Hence, an inductor of the prototype becomes a series LC resonator with resonance frequency being wo. Similarly, a capacitor of the prototype transforms as a parallel LC resonator with element values

C=C'/B,

L=1/(Cwo2)		(2.8.13)

Once the specifications “fp1 , fp2 , p , fs2  (or fs1 - in Filpro fs2 is used) and s” are given, the prototype that will yield the desired filter is found by first finding ws' through Equation 2.8.9 and then searching for the proper degree that has the specified s at ws', in the tables with the properp.

Exercise 2.25: Design a Generalized Chebyshev-1 bandpass filter using “Design/By Built-in Prototype” approach with the following parameters:

fp1=40 MHz, fp2=60 MHz, N=7 (prototype degree),  p=0.05 dB,  s=60 dB

Inspect the elements of resulting filter. Note that fourth order sections (termed as Quad in Filpro) exist in the circuit. These are the transformed versions of the series LC resonators of the prototype.

a) Plot the insertion loss response. Note that the minimum stopband losses on the two sides of passband are equal. This is a property of frequency transformation used in mapping from prototype. This transformation produces always “geometrically symmetric” response around the center frequency fo. Calculate fo using fp1 and fp2. Use marker to read the finite transmission zero frequencies (fz1, fz2) and verify that

fz1 fz2 = fp1 fp2 = fo2

b) Place the cursor under a fourth order section and apply “Split” command. It will be splitted into two second order (LC) resonators, resonating at the two finite transmission zero frequencies on the two sides of passband. Use the command “Tune Element” to find out which resonator of the splitted fourth order section creates which finite transmission zero.

Figure 2.35. Prototype -to- bandpass mapping.

c) Use marker to read frequencies at which the skirt attenuations are equal to the minimum stopband loss of 60 dB (to be treated as stopband edge frequencies, fs1 and fs2). Show that they are also geometrically symmetric around fo:

fs1 fs2 = fo2

d) Use “Undo” command to get the original form of circuit and apply “Find Dual” command to get the dual circuit. Inspect the series arm versions of fourth order sections. Split them also and find out which second order resonator creates which finite transmission zero. Further examples and exercises will be given in Chapter 6.

- Bandstop Filters:  Frequency transformation to get bandstop filters is

w'= B/[(w02 /w)-w ] 	(2.8.14)

where

B = wp2 -wp1 	(2.8.15)

Figure 2.36. Prototype-to-bandstop mapping.

is the stop bandwidth as defined by the edges wp1 and wp2 of lower and upper pass bands. w0 is the geometric center of stopband, defined as

w02 = wp1 wp2 	(2.8.16)

Using 2.8.14 in 2.8.1 yields the following results:

An inductor L' of the prototype becomes a parallel LC resonator with element values

C = 1/(BL'),        L=1/(Cwo2)	(2.8.18)

A capacitor C' of the prototype becomes a series LC resonator with values

L = 1/(BC'),        C=1/(Lwo2) 	(2.8.19)

Some possible bandstop filter structures are shown in Figure 2.36. Once the specifications “fp1 , fp2 , p ,  fs2  or fs1 (in Filpro fs2 is used ) and s” are given, the prototype that will yield the desired filter is found by first finding ws' through Equation 2.8.14 and then searching for the proper degree that has the specified s at ws', in the tables with the properp. Since bandstop filters have only finite transmission zeros, there is no possibility of forming alternative equivalent structures. Therefore designing filters by synthesis or by prototype give the same circuits. For this reason, in Filpro only “Design/By Built-in LC prototype” approach is programmed for bandstop filters. If the built-in prototypes are not satisfactory for some reason, it is always possible to develop new prototypes using “Synthesis” method and then use “Design /By User Defined Prototype” approach where the same frequency transformations are applied on the prototypes developed or inserted by the user. Naturally, prototypes found elsewhere can also be inserted in that approach. Examples will be presented in Chapter 5.

2. 9. FILTER DESIGN USING PROTOTYPES WITH INVERTERS

Ideal impedance inverters of inverter constant K (impedance K) has the frequency independent ABCD matrix

These are extremely flexible tools used for overcoming realizability problems of many narrow and medium bandwidth filters. Definitions and circuit transformations involving ideal inverters are described in Chapter 1. Lowpass prototype filters can also be designed using ideal inverters. Inverters of such lowpass prototypes will map to bandpass (and other) filters as they are because their characteristics are independent of frequency. The ideal inverters will then be replaced by their practical approximate equivalents. Frequency independent characteristics of inverters can be approximated in narrow and medium bandwidths with a large variety of practical circuit sections most  of which are available in Filpro.

Filter design using prototypes with inverters is programmed under the menu “Design / By Prototypes with Inverters”. A typical bandpass filter design procedure is described in Figure 2.37 and Figure 2.38:

- The command set of Figure 2.37 produces and displays the lowpass prototype selected. In these commands we specify the circuit type (shunt capacitors or series inductors), passband ripple and degree of the prototype. The resulting prototype may be saved for later use if desired.

- The command set described in Figure 2.38 is applied after the prototype is displayed. Here we specify the filter type (LP, BP, etc.), passband edge frequencies and terminations. Find circuit command displays the desired filter circuit.

Figure 2.37. Design steps for Prototype with Inverters approach leading to the prototype with shunt capacitors and inverters.

Figure 2.38. Command set for obtaining the desired bandpass filter from the prototype with inverters.

- One advantage of prototype with inverters approach over the LC prototype is that it produces always symmetric filters for both even and odd degree filters. This is provided through the properties of ideal inverters used in formulation stage for finding the element values. One example will be given below.

- All filters that can be designed with inverter approach are also designable with both LC Prototype and Synthesis approaches. Inverter approach is placed just for quick design purposes to avoid circuit transformations that are necessary in other approaches to reach the same target.

- This approach can only be used for medium or narrow band applications. It will lead to gross distortions in both ripples and passband edges for wide band applications.

If necessary, a typical lowpass prototype filter with inverters can be designed and saved using Synthesis approach  as follows:

- A lowpass filter is synthesized with unity terminations and normalized passband edge frequency fp= 0.159 using the command set described in Figure 2.39. Degree is taken as n=10, even, giving the antimetric filter shown in Figure 2.40.

Figure 2.39. Command set for synthesizing a lowpass prototype with inverters: Formation of response.

Figure 2.40. Command set for synthesizing a lowpass prototype: Element extraction.

This circuit can be put into symmetric form in several ways using Filpro. One way can be as follows:

- In Figure 2.41.a the resulting antimetric LC circuit is repeated. Place the cursor under the element on the right of the antimetry axis and apply Symmetrize command, giving the circuit of Figure 2.41.b.

- Delete the element around which circuit is symmetrized.

- In place of the deleted element, insert an ideal inverter using Insert command (Figure 2.41.c). Inverter constant should be equal to the inverse of the ideal transformer at the load end of the circuit. Theoretically, it is equal to the geometric mean of  RS and RL when the ideal transformer is absorbed in RL (or RS). Inspection of response shows that the new circuit is exactly equivalent to the original circuit. Thus we have obtained a symmetric circuit at the price of introducing an ideal inverter which also increases the total number of elements.

- Placing the cursor under series inductors and applying command “Replace By Ideal Inverter/K=1” all the inductors will be replaced by shunt capacitors in between two ideal inverters whose impedance K is selected by the user (Figure 2.41.d).

- Using Simplify Circuit command all the ideal (phase inverting) transformers may be eliminated (Figure 2.41.e). Save the circuit. The circuit is now ready to be used in User Defined Prototype approach

- With the same approach, we may convert all shunt capacitors into series inductors.

- Inverter constants can be adjusted to rescale the element values using Scale Inverter command. For example all capacitors may have identical values.

- .

- Figure 2.41. Development of a lowpass prototype with inverters.

REFERENCES AND SUGGESTED READING

- E. L. Norton, “Constant Resistance Networks with Applications to Filter Groups”, Bell System Tech. J. vol. 16, pp. 178-193, Apr. 1937.

- T. Fujisawa, “Realizability Theorem for Mid-series and Mid-shunt Lowpass Ladder Networks without Mutual Induction”, IRE Trans. CT, vol. CT-2, pp. 320-325, Dec. 1955.

- R. Saal, and E. Ulbrich, “On the design of filters by Synthesis”, IRE Trans. Circuit Theory,vol.CT-5, no. 4, pp. 284-327, 1958.

- G. Szentirmai, "Band-pass matching filter in the form of polynomial low-pass filter", IEEE Trans. On Circuit Theory, vol. CT-11, pp. 177-178, March 1964.

- A. Zverev, “A Handbook on Filter Synthesis”, John Wiley and Sons, Inc., NY 1967.

- H. J. Orchard and G. C. Temes, “Filter Design Using Transformed Variables”, IEEE Trans. CT, vol. CT -15, No.5, pp. 385-408, Dec. 1968.

- V. Belevitch, “Classical Network Theory”, San Francisco, Calif.Holden-Day, 1968.

- D. S. Humpherys, “The Analysis, Design and Synthesis of Electrical Filters”, Prentice Hall, Inc.,   
 1970.

- G. C. Temes and S. K. Mitra, “Modern Filter Theory and Design”, J.Wiley and Sons, NY, 1973.

- R. W. Daniels, “Approximation Methods for Electronic Filter Design”, Mc Graw Hill, NY, 1974.

- J. D. Rhodes, “Theory of Electrical Filters”, J. Wiley and Sons, NY, 1976.

- W. K. Chen, “Theory and Design of Broadband Matching Networks”, Pergamon Press, 1976.

- G. C. Temes and J. W. La Patra, “Introduction to Circuit Synthesis and Design”, Mc Graw Hill, NY, 1977.

- R. Saal, “Handbook of Filter Design”, AEG Telefunken, 1979.

- J. D. Rhodes and S. A. Alseyab, “The Generalized Chebychev Low Pass Prototype Filters”, Int. J. Circuit Theory and Applications, Vol. 8, pp. 113-125, 1980.

- S. A. Alseyab, “A Novel Class of Generalized Chebychev Low Pass Prototype for Suspended Substrate Stripline Filters”, IEEE Trans. Microwave Theory and Tech. Vol. MTT-30, No. 9, pp. 1341-1347, Sept. 1982.

- H. Baher, “Synthesis of Electrical Networks”, J. Wiley and Sons, NY, 1984.

- M. Hasler and J. Neirynck, “Electric Filters”, Artech House Inc. Mass. 1986.

- U. Koyaz, “Synthesis of Lumped and Distributed Filters and Diplexers”, Ph. D. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1994.

- Y. Sen, “Direct Design of Lumped and Distributed Filters Using Transformed Variable Techniques”, M. Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1989.

- M. Karaaslan, “Network Transformations for Realization of Lumped and Distributed Filters”, M. Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1994.