CHAPTER 3

LUMPED ELEMENT LOWPASS FILTER DESIGN

3. 1. INTRODUCTION

In this chapter some typical lumped element lowpass filter design examples will be presented as demonstration of some capabilities of Filpro. Lowpass filters may be designed in Filpro in several ways, like Design / By Built-in Prototype, and Design / By Synthesis and Design / By User Defined Prototype approaches. Chebyshev and Butterworth filters are quicker (and more accurate for some very high degree cases) to design using  Prototype approach. Equiripple or maximally flat passband, general stopband filters can be designed in the most general forms using Synthesis approach. The special cases like, equal stopband ripple (elliptic or inverse Chebyshev) filter characteristics can be obtained by using Tune a FTZ (Finite Transmission Zero)  tool of  FILPRO which eases shaping of the response of filter a lot. Other special classes, like GeneralizedChebyshev Filters (if passband is of equal ripple type), or Generalized Butterworth Filters (if the passband is of maximally flat type) can be obtained by placing all transmission zeros at the same frequency in the stopband. Generalized Chebyshev type filters with one or three transmission zeros at f = 0 with limited choices for passband ripple and stopband loss and with limited, odd degrees are also available in the Prototype approach. More general prototypes can be developed and saved using Synthesis approach, to be used with Design / By User Defined Prototype approach.

Examples will be mainly be given using Built-in Prototype and Synthesis approaches, as categorized in two groups:

i) Chebyshev and Butterworth filters.

ii) Filters with finite transmission zeros.

These groups may further be subdivided in accordance with various criteria like, equal and unequal terminations. Examples will also involve some circuit transformations. Further examples can be created by the user, for example, to check accuracy limitations of different design  approaches and to discover uses of various circuit transformations.

3. 2.  CHEBYSHEV AND BUTTERWORTH  LOWPASS FILTER DESIGN USING  LOWPASS PROTOTYPES

Chebyshev and Butterworth lowpass filters can be designed either by Built-in Prototype or by Synthesis approaches. Prototype approach is quicker to use while synthesis approach is more instructive and flexible but requires more steps. Accuracy limitations of  different approaches depend on different parameters and have to be checked by the user especially for very high degree filters (greater than about 25-30). In such filters Prototype approach comes out  to be more accurate in most cases.

Filters with equal terminations can only be designed with odd order. Even order filters require  RL  RS. Such cases will be treated in a later section (Section 3.6), separately.

EXAMPLE  3. 1. A Chebyshev lowpass filter

Let’s design a Chebyshev LPF with specifications

PBRipple=0.01 dB,    fp=100 MHz,     SBLoss=40 dB,       fs= 130  MHz,    RL = RS =500 

Design steps using prototype approach are as follows ( Figure 3.1):

“New / Design / By Built-in Prototypes / Lumped, Lowpass, Chebyshev, / RS=RL=500 /

PB+SB Specs: PB Ripple=0.01, fp=100,  SB Loss=40, fs =130 / Find Circuit”

- The circuit will be displayed automatically  (Figure 3.2.a).

- Quit design menu to activate circuit transformation and editing functions.

Figure 3.1. Command sets for the design of a lowpass filter using Built-in Prototype approach.

Figure 3.2. Lowpass filter of Example 3.1  and various responses.

- Use the keys Ctrl + right (or left) arrow to move the circuit cursor pointing the elements. Parameter values characterizing the pointed elements are displayed  in a box at the right bottom corner of the screen. Move the cursor on elements and inspect element values.

- Plot the Insertion Loss of filter using the short-cut key Shft+F1 with scales

- fmin=0  to fm a x= 200  MHz,   ILm a x= 100 dB  (Figure 3.2.b).

- Overplot the Return Loss on Insertion Loss curve (short cut key: Shft + F2)  (Figure 3.2.b).

- Use Right-Left arrows to move the marker over the response curve. Press Space Bar to place a Delta Marker at any desired frequency. Then move marker right and press  F 9  to zoom the region between delta marker and marker.

- Use the key F 10  to Rescale and Plot (Replot) the response. For example plot the passband response with scaling   fmin=0  to fmax=100  MHz,  ILmax=0.02 dB to inspect the equal ripple property (Figure 3.2.c).

- Exercise with Phase ( short cut key: Shft+F3), Delay ( short cut key: Shft+F4), and Smith Chart  
       responses ( short cut key: Shft+F5 ),(Figure 3.2.d, e, f ).

- Exercise with Home and End keys for zooming on amplitude axis.

- Use the command Element Sensitivity to inspect sensitivity of response to variations in element values.

- Place the cursor under all elements one by one  and apply  Parasitics / Lossy Element  or Lossy Circuit command which replaces the ideal lossless elements with their practical realization models including lead inductances and resitances of capacitors, resistances and interwinding capacitances of inductors (Figure 3.3). Use the default values for all quantities asked by the program and inspect variations in response.

- Practical model of an inductor includes a resistor in series with inductor to account for dissipative losses and a capacitor in parallel with inductor to represent interwinding capacitances. The value R of resistor is calculated from the  Q of the inductor (specified by the user), at the frequency of interest  (specified by the user),  using the formula

Q=w0L/R

- For lowpass filters it is usual to define Q at the band edge frequency. The value of  parasitic capacitance is also specified by the user.

- The practical model of capacitors  include a lead inductance in series and a resistance  in parallel with the capacitor. Lead inductance is also specified by the user. The  resistance is calculated from the Q of  capacitor, using the formula  Q=w0RC

Figure 3.3. Lowpass filter with practical inductors and capacitors.

Figure 3.3. Lowpass filter with practical inductors and capacitors (continued ).

- Inspect deformations in insertion and return loss responses (Figure 3.4). Explain the slight improvement in return loss.

Figure 3.4. Effects of finite Q and parasitics of elements on response.

- Use  Reference Lines command to form brick walls to display the limits  not to be crossed by the insertion loss response ( Figure 3.5).

- Figure 3.5. Placing reference lines.

- Figure 3.6. Monte Carlo Analysis with brick wall boundaries.

- Make exercises with Monte Carlo analysis. One example is shown in Figure 3.6 with arbitrarily large tolerances for element values.

- Make some Yield  Analysis  with the same brick walls and tolerances as in Monte Carlo case.

- Use Find Dual command to inspect the dual circuit (Figure 3.7). Compare the element values, number of capacitors and inductors.

Figure 3.7. The dual lowpass filter.

Exercise 3. 1.  Repeat the design using now the option PB Specs+ Prototype Degree command set

“New / Design / By Built-in Prototypes / Lumped, Lowpass, Chebyshev / RS=RL=500/

PB Specs + Prototype Degree: PB Ripple=0.01, fp=100, Degree N=11   / Find Circuit ”

- Compare the element values with those of previous filter.

- Use New Trial command to decrease and increase the order of filter and compare responses by overplotting them.

- Change PB Ripple by keeping the degree unchanged and compare responses.

- Make New Trials by choosing Butterworth option and compare responses with Chebyshev filters of the same degree.

- Note that it is not possible to design even degree Chebyshev filters with RL=RS. Even degree filters require unequal terminations and will be discussed in the next section  3.6.

3. 3. CHEBYSHEV AND  BUTTERWORTH LOWPASS  FILTER  DESIGN  USING  SYNTHESIS  APPROACH

Design / By Synthesis approach can also be used to design Chebyshev and Butterworth lowpass filters though it has not much advantage compared to Prototype approach for RL = RS cases. Let’s demonstrate the procedure through an example.

EXAMPLE  3. 2.  Lowpass  filter design by “Synthesis” approach

Let

p=0.1 dB,  fp=100 MHz ,  s=40 dB,  fs=130 MHz, RL=RS=500 

The steps are as follows (Figure 3.7):

“New / Design / By Synthesis”

“Filter Type:  Lumped, LowPass, Equiripple, Doubly terminated, Type A”

“Passband and Terminations :  PBRipple=0.1, fp=100 , RS=RL=500”

“Specify TZs: INF, INF,.... , INF  (11 transmission zeros)”

As the transmission zeros are placed, the insertion loss builds up in a small box on the top right corner of the screen. In this example 11 transmission zeros at infinity (N=11) yielded a response satisfying the condition s>40 dB at fs=130 MHz. If finer details of response is to be inspected, then one may use Plot Response command, which also scales the small plot. After shaping the response, we pass to element extraction (TZ extraction) stage. Extract TZ command can be carried out using Standard Auto Extract command (if enabled), yielding the circuit instantly. One may as well extract transmission zeros manually one by one, by invoking  TZ at Infinity  button eleven times. After each manual extraction, the extracted element appears in the circuit schematics at the bottom of the screen . Hence one can inspect the type and value of extracted element instantly, before the circuit is completed. After all transmission zeros are extracted the program learns the order of extraction and in the next design trial (if there is any) one may also use Macro Auto Extract  mode to speed up synthesis procedure. Standard auto extract  mode is the usual one if  the transmission zero pattern is recognized by the program. A default element extraction order is programmed for some commonly used filter types. This will be seen from the highlighted (enabled) Standard Auto Extract button. The case when all transmission zeros are at infinity is among the types that the program knows. In fact when all transmission zeros are at infinity, the circuit is unique (no alternative equivalent circuits except the dual circuit). Therefore  Standard Auto Extract  mode is the usual one. The synthesized circuit is shown in Figure 3.7. Compare element values with those of the circuit designed by Prototype approach.

Figure 3.8. Command sets for synthesizing a Chebyshev lowpass filter.

Figure 3.8. Command sets for synthesizing a Chebyshev lowpass filter (continued).

Remark: Degree of lowpass filters with all transmission zeros at infinity is equal to the number of transmission zeros.

Remark: Odd degree lowpass filters with all transmission zeros at infinity come out to be physically symmetric always, while even degree comes out antimetric always.

Remark:  Element extraction is carried out from  the parameters  Z11   or Y11. Therefore in this process the load  resistance RL has no function. It is placed in the specification stage just as an information to be used in schematics and to calculate response. For odd degree filters RL  must be the same as RS . Otherwise an ideal transformer will appear at the load end to present a load impedance equal to the source impedance. The cases with RL RS  require even degree to avoid transformer. Such cases will be studied in section 3.6.

Exercise 3. 2: Repeat the design  to get the dual structure by choosing the option Type-B in  Filter Type item.

Exercise 3. 3: Repeat the design with  p = 0.01 dB and compare the element values with those of the above example.

3. 4. GENERALIZED CHEBYSHEV LOWPASS  FILTERS FROM BUILT  IN PROTOTYPES

GeneralizedChebyshev prototypes are quasi-elliptic filters uncovered and tabulated by Rhodes and Alseyab [5], [6]. Two classes of tables are available: One with single transmission zero at infinity (termed as Generalized Chebyshev-1 in FILPRO) , and the other with three transmission zeros at infinity (termed as Generalized Chebyshev- 3 in FILPRO). All the finite transmission zeros placed at the same frequency  close to passband edge to increase selectivity. They have lower element value spread compared to true elliptic filters. Some tables with limited choices of parameters are placed in Filpro just as a convenience for quick survey purposes, for those who don’t want to bother with steps of  Design / By Synthesis approach. Only odd degree tables are available leading to symmetric filters. Therefore they can’t be used for nonidentical termination cases. The same (and more general) filters can be designed using  Synthesis approach , as will be described in the next section.

EXAMPLE 3. 3.

Let’s design  a GeneralizedChebyshev-1 lowpass filter with specifications

p=0.01 dB,     fp=100 MHz,      s=60 dB,      N=9,   RS=RL=50 

Design steps are as follows (Figure 3.9):

New / Design / By Built-in Prototypes / Lumped, Lowpass, Gen.Cheb-1/ RS =RL=50 / fp=100, N=9 / PB Ripple=0.01,  SB Loss=60 / Find Circuit

Figure 3.9. Command sets for the design of a Generalized Chebyshev lowpass filter.

Figure 3. 10. Coupled coil realization of Generalized Chebyshev filter.

Figure 3.11. The dual GeneralizedChebyshev lowpass filter realized in coupled coil form.

- The resulting circuit is shown in Figure 3.10.a. Place the cursor under the first shunt arm resonator and apply command

“Coils / sLsr+LCsh+Lsr-to-CapCoil / Inductor on right”

- This command replaces the triplet “Series L+Shunt LC+Series L”  section by a coupled coil with a capacitor (CapCoil) followed by a series arm inductor on the right or left of the coil section. Repeating on all shunt arm resonators, we get the circuit of Figure 3.10.b.

- Get the dual of circuit of Figure 3.10.a using  Find Dual command, giving the circuit of Figure 3.11.a.

- Place the cursor under series arm resonators and apply command “Coils / Csh+pLCsr+Csh-to-CapCoil / Capacitance on right ” to get the coupled coil equivalent shown in Figure 3.10.b. Response of filters are identical and is shown in Figure 3.11.

Exercise 3. 4:  Repeat the designs with coupled coils by choosing Inductors on Left and Capacitors on Left options and compare the resulting element values.

Exercise 3. 5: Replace inductors and capacitors of  circuits of Figure 3.10.a and 3.11.a by their lossy equivalents and inspect  deformations in responses.

Figure 3.12. Sensitivity of insertion loss response to variations in the capacitors C1 and C5.

Figure 3.13.a) Monte Carlo Analysis results for 5 %  standard deviation with Gaussian  probability density function. b) Monte Carlo Analysis results for 5 % uniform probability density function.

Exercise 3. 6: Find sensitivity of response to variations in element values using commands Tune Element or  Element Sensitivity. As examples, variation of insertion loss response with variations in  the first capacitor  (C1) and the middle capacitor (C5) are drawn in Figure 3.12.

Exercise 3. 7: In Figure 3.13, Monte Carlo Analysis results  of the above filter are shown. In these trials all the elements are included in the Monte Carlo Parameters List, with  5 % deviation. The results shown in Figure 3.13.a and b are for Gaussian and Uniform distributions respectively. As expected, Gaussian distribution gives better results, while Uniform distribution is more pessimistic. The Brick Wall boundaries are placed at 3 dB in passband, f = 0 to 95 MHz and 55 dB in stopband, f = 125 to 500 MHz. Yield Analysis  applied on the Gaussian case gave 100 % success. Apply Yield Analysis to the Uniform case to find out the success percentage.

Exercise 3. 8: Design a Generalized Chebyshev-3 lowpass filter (3 Transmission Zeros at f = )  with specifications

p=0.01 dB,   fp=100 MHz,  s=60 dB,   N=9

Compare the structure and response with those of the previous example.

Exercise 3. 9: Design a Generalized Chebyshev-3 lowpass filter  having specifications

p=0.01 dB,   fp=1500 MHz,  s=60 dB,   N=11.

Compare element values with the circuit of previous example.

3. 5. SYNHESIS OF LOWPASS FILTERS WITH FINITE TRANSMISSION ZEROS

The most general lowpass filters with equiripple or maximally flat passband and general stopband responses can be designed using By Synthesis approach. Stopband loss can be shaped by, in addition to transmission zeros at infinity, the finite transmission zeros (FTZ) placed at various positions in stopband. Several classical filter types can readily obtained as follows:

- If all finite transmission zeros are placed at a single frequency near the passband edge, then GeneralizedChebyshev filters will be obtained if the passband is of equiripple type, and GeneralizedButterworth filters will result if the passband is of maximally flat type.

- Elliptic filter characteristics will  result if the passband is of equiripple type and FTZs are distributed in stopband properly to give equal stopband ripple.

- Inverse Chebyshev filters will be obtained if the passband is of maximally flat and stopband is of equiripple type.

A variety of responses can be formed by adjusting numbers and positions of finite transmission zeros.  Tune a FTZ command of FILPRO enables the designer to adjust positions of finite transmission zeros while observing the response.

Using more than one transmission zeros at infinity increases the number of equivalent alternative circuits by extracting transmission zeros in different orders.

EXAMPLE  3. 4. An Elliptic lowpass filter.

Let’s design an elliptic filter with specifications

p=0.5 dB,  fp=40 MHz,  Ninf =1 (1 TZ at infinity),  NFTZ=4  (4 FTZs to be found by iterations to get equal stopband ripple).

Steps:	New / Design / By Synthesis

Filter Type: Lumped, LowPass, Equiripple, Doubly Terminated, Type A

Passband and Terminations:

Passband Ripple = 0.5 dB,	fp= 40 MHz,	RS=50,	RL=50

Specify TZs:     INF  (1 TZ at infinity),	FTZ1=45 MHz,	FTZ 2=50  MHz,

FTZ 3=60 MHz,	FTZ 4=100 MHz

These finite transmission zeros are placed by intuition while watching the build up of the response, and then  Tune FTZ  command is used on each FTZ  until equal stopband ripple is obtained as shown in Figure 3.14, for the FTZ frequencies  45 MHz, 48 MHz, 58 MHz  and 100 MHz.

Figure 3.14. Command sets for synthesis of an elliptic lowpass filter.

Figure 3.14. Command sets for synthesis of an elliptic lowpass filter (continued).

After forming the desired insertion loss pattern, the transmission zero sections are extracted in the following order: Extract TZs : FTZ1, FTZ2, FTZ3, FTZ4, Inf

- The circuit shown in Figure 3.15 is obtained. The same structure would be obtained using Standard Auto Extract command. Inspect the response of resulting structure. Note that the minimum stopband loss is  68 dB, with stopband edge frequency fs = 44.7 MHz.

Figure 3.15. An elliptic lowpass filter.

- In Figure 3.16, the dual structure (obtained by choosing Type-B in  Filter Type  specifications) is shown. The element values underneath the circuit are obtained for different orders of finite transmission zero extraction, as indicated.

Figure 3.16. Elliptic filter element values obtained by extracting finite transmission zero sections in different orders.

Exercise 3. 10: Repeat the design for less passband ripple values (for example 0.01 dB). Compare the minimum stopband losses and element values.

Exercise 3. 11: Carry out Sensitivity Analysis, Monte Carlo Analysis and Yield Analysis by forming some Brick Walls using Reference Lines.

Exercise 3.12: Synthesize the same filter by extracting transmission zeros in different orders to complete the table in Figure 3.16.

Exercise 3.13: Synthesize the same filter by placing all FTZs at f=55 MHz giving a GeneralizedChebyshev filter with roughly the same minimum stopband attenuation as in elliptic case. Compare the selectivities (e.g. loss at f =44.7 MHz) and element value spreads with those of the elliptic one. Note that GeneralizedChebyshev filter is symmetric, while the elliptic one is not because all transmission zeros are at different frequencies making physical symmetry impossible though the filter is symmetric electrically (input and output can be interchanged).

Exercise 3. 14: Repeat the design by placing the four transmission zeros in pairs at f1=45 MHz and f2=55 MHz. Extract  FTZ sections in proper order to get a physically symmetric structure.

EXAMPLE 3. 5. An elliptic lowpass filter with three transmission zeros at infinity.

Let’s design a filter with specifications	p=0.5 dB, fp=40 MHz, Ninf=3  (3 TZs at infinity), NFTZ=4  (four FTZs to be found by iterations to get equal stopband ripple).

Steps:	New / Design / By Synthesis

Filter Type: Lumped, LowPass, Equiripple, Doubly Terminated, Type A

Passband and Terminations: Passband Ripple=0.5 dB, fp=40 MHz,  RS=50, RL=50

Specify TZs:  INF,  INF,  INF, FTZ 1=46.3  MHz,	FTZ 2=48.5  MHz,	FTZ3=55 MHz, FTZ 4=75 MHz  (Found by using  Tune FTZ command  giving about 90 dB minimum stopband loss)

Extract TZs :Transmission zeros are extracted in the following different orders:

a) INF, FTZ (46.3 MHz) , FTZ (48.5 MHz) , FTZ (55 MHz) ,  FTZ (75 MHz) ,  INF, INF

b) FTZ (46.3 MHz) , FTZ (48.5 MHz) , FTZ (55 MHz) ,  FTZ (75 MHz) ,  INF, INF, INF

c) FTZ (46.3 MHz) , INF,  FTZ (48.5 MHz) ,  INF,  FTZ (55 MHz) ,  FTZ (75 MHz) ,  INF

d) FTZ (46.3 MHz) , INF,  FTZ (48.5 MHz) , FTZ (55 MHz) , INF,  FTZ (75 MHz) ,  INF

The resulting circuits are shown in Figure 3.17.a to d, with some responses drawn with different scales in pass and stop bands. Further equivalent alternative circuits can be found by applying Find Dual command on these circuits and /or by changing the extraction order of finite transmission zeros.

Figure 3.17.  Some equivalent alternative lowpass filters with three transmission zeros at infinity.

3.  6. LOWPASS  FILTERS WITH UNEQUAL TERMINATIONS

Lowpass filters with unequal terminations  have unavoidable mismatch loss at f = 0, because at this frequency load appears right across the source. This loss is termed as DC loss [1], [ 8], calculated as

o=10 log [4 RL*RS / (RL+RS)2 ]   dB

Since transformers are also useless at DC, some loss will always appear in passband of lowpass filters with RL RS. Lowpass filters designed by Built-in LC Prototype approach will have a flat loss throughout the passband. If a better match is desired in some region of the passband, then a bandpass filter can be designed using Synthesis approach and used as a quasi-lowpass filter having some loss near f=0. In this respect, synthesis approach is much more  flexible compared to prototype approach, as will be described in the following examples.

a) By Built-in LC Prototype Approach

EXAMPLE 3. 6.   Chebyshev lowpass filters with unequal terminations and flat loss .

Let’s design two  Chebyshev lowpass filters with

p=0.5 dB, fp=100 MHz ,  fs=130  MHz ,  s=40 dB ,  RS=50  ,   RL=250 

one with odd order and one with even order. The ratio  RL /RS=5 gives a DC loss of 2.55 dB.

Design steps for even order filter are shown in Figure 3.18. The resulting filter circuits are shown in Figure 3.19. The following observations may be noted:

- Odd order choice yields degree 9, while even order choice yields degree 10 filters.

- For the odd degree filter the choice of 0.5 dB ripple makes the passband response to oscillate between 2.55 and 3.05 dB. That is the specified passband ripple is superimposed on the flat (DC) loss of 2.55 dB leading to maximum passband loss greater than DC loss by the amount equal to the specified ripple.

- For the even degree filter oscillation is between 2.05 dB and 2.55 dB. That is  for these filters DC loss forms the upper bound of passband loss .That is why passband ripple  is to be chosen less than or equal to DC loss.

- For these reasons it is usual to prefer even degree filters for  RL  RS  if the flat loss is acceptable.

Figure 3.18. Command set for design of Chebyshev lowpass filter with unequal terminations using                 Built-in LC Prototype approach.

Figure 3.19. a) An even degree lowpass filter with unequal terminations. b) An odd degree                  lowpass filter with unequal terminations.

Exercise 3. 15: Find RL for which passband ripple upper bound is 0.5 dB.

Exercise 3. 16: Repeat design as Butterworth filter and see that the passband loss is flat at 2.55 dB (DC loss).

- b) Synthesis approach for impedance transforming quasi-lowpass filters

- Bandpass filters without any transmission zero at f=0 have quasi lowpass characteristics. Such filters have unavoidable insertion loss in a region  near f=0. The width of this region depends on RS/RL ratio. If this loss is tolerable, then such filters may be used as quasi-lowpass filters.

- In lossless two port filters, the following four parameters are closely linked:

- Pass bandwidth.

- Passband ripple for Chebyshev filters and passband edge loss in maximally flat filters that define the passband width.

- Termination impedances RS and RL (or, better, the impedance transforming ratio, RS/RL).

If any three of these quantities are specified by the designer, then the third parameter is automatically set through the interrelations among them. On the other hand, in Filpro, all the four parameters, passband edge frequencies, passband ripple, RS and RL are to be specified by the designer. Since they are not independent of each other, and since the designer doesn’t know the proper set of parameters in advance, these parameters will form an INCONSISTENT SET. However, Filpro synthesizes the filter from a knowledge of only input immittance parameters, Z11 or Y11 without any consideration of the load RL. In specification stage, load resistance RL is asked just as a convenience to draw response of the resulting structure with the specified load. The actual load resistance that filter requires to have the desired specifications is calculated by Filpro automatically and it is compared with the specified load resistance. If there is a difference, then an ideal transformer is placed in front of the load for compensation. If that transformer is absorbed in the load resistance (using Shift Transformer / Absorb Trf In RL command), then the actual necessary load resistance is found. Thus Filpro corrects this inconsistency by sticking a transformer at the load end. Existence of this “nonunity” transformer is an indication of inconsistency of the specified set of parameters. This is shown in the designs of Figure 3.20. In this figure, two bandpass filters are designed one with RL=50 ohms and the other with RL=250 ohms. The lower pasband edge is set very close to zero, fp1=0.0001, to simulate a lowpass filter. The upper band edge is fp2=100 Mhz. The passband ripple is 0.01 dB. The response plot shown in Figure 3.20.a is valid for both filters, because as it was mentioned above, Filpro places a transformer to match the specified RL to the value set by the other three parameters (ripple, bandwidth and RS). This transformer has a turns ratio close to unity when RL=RS and it deviates from unity when RS and RL differs, as shown in Figure 3.20.b and c. For RL=250 ohms, turns ratio came out to be n=2.13.

Figure 3.20. Bandpass filters with RL=50 and RL=250 ohms loads for fp1=0.0001 MHz.

Figure 3.21. Bandpass filters (quasi-lowpass filters) with RL=50 and RL=250 ohms loads                     when fp1 =50 MHz.

In order to eliminate the transformer (i.e. to get unity transformer), we should adjust the other three parameters. In most applications RS and passband ripple should remain unchanged. For such cases the only adjustable parameter is bandwith. For example, the previous design is repeated in Figure 3.21 by increasing   the lower band edge from fp1=0.0001 to fp1=50 MHz,  while the upper band edge is kept at fp2=100 MHz. then the  transformer turns ratio for RL=250 ohms decreases from n=2.13 to n= 0.54, as shown in Figure 3.21.c. This means that for a unique value of fp1, the transformer turns ratio becomes unity. Thus, we would get a transformerless design with RS=50 ohms and RL=250 ohms. However, the response plot shown in Figure 3.21.a indicates the price paid for this success: The lower stopband which was absent when fp1=0.0001 MHz, is now no longer negligible.  A loss region which increases gradually towards f=0 appeared. That is why such filters are termed as quasi-lowpass filters. If this loss is acceptable, then the filter can be used.

Thus, if load resistance can’t be changed and also if no transformer is desired, then the designer has to change some of the specified parameters and repeat the design. These iterative trials will continue until a unity transformer is obtained at the load end.

In Filpro, the following procedure should be followed:

- Design / By Synthesis option will be selected.

- Bandpass Filter option will be chosen , but no transmission zeros will be placed at f=0. All transmission zeros will be at infinity or at finite frequencies in upper stopband.

- The number of transmission zeros should be even (band pass filters have even degree always- See also Chapter 6).

- The actual desired RS and RL should be specified.

- If RS <RL, then Type A will be chosen and if RS >RL, then Type B should be chosen.

- Trial fp1 and fp2 should be typed in together with the desired passband ripple or band edge loss.

- Filter is synthesized and the transformer turns ratio is checked. Then either the band edge frequencies or the ripple are changed, and the design is repeated.

- If the resulting bandwidth comes out to be unacceptably narrow, then degree of filter should be increased.

EXAMPLE 3.7: Impedance Transforming Quasi Lowpass Chebyshev Filter

Let’s design a degree  N=8 Chebyshev bandpass filter with RS=50  to RL=250 , with a passband below  f=100 MHz,  and with passband return loss better than 26 dB (passband ripple 0.01 dB). Our aim is to see how much bandwidth we can get with N=8 and without transformer.

- The command set for the first trial design is shown in Figure 3.22.a. Passband ripple is set as 0.01 dB, and is not a trial parameter. So, we are left with only the pass band edge frequencies for trials. As the initial trial, we set fp1=10 MHz and fp2 =110 MHz. Since the response is plotted as if a proper termination exists as the load, it should be disregarded.

- Extracting elements, we get a transformer with ratio greater than unity (Figure 3.20.b). This means we should decrease the bandwidth.

- After a few design trials, we get unity turns ratio transformer for fp1=42.4 MHz and fp2=100 MHz (Figure 3.22.c-d). 26 dB return loss band width came out to be about 58 MHz.

Exercise 3. 15: Repeat the previous designs for  RS=250 ohms and RL=50 ohms.

Figure 3.22. Design steps for an impedance transforming Chebyshev quasi-lowpass filter.

EXAMPLE 3. 8. Synthesis of  maximally flat quasi-lowpass  (bandpass)  impedance transforming filters

Now, let’s design a maximally flat bandpass filter RS=50  to RL=250 , with passband below  f=100 MHz,  with a band as wide as possible. Let’s choose the degree as N=8. We could proceed in exactly the same way as in the previous example, but an interesting approach exists which holds for only maximally flat filters. Let’s describe this approach.

- First, the DC loss of the filter is found. This quantity is easiest to find by using “Design / By Inserting Elements” approach. The circuit involving only RS and RL will be displayed automatically. Type in the desired RS and RL values and plot Insertion Loss. The displayed insertion loss is 2.55273 dB and this is the required DC loss.

- The command set “Design / By Synthesis/Lumped, Band Pass,  Max.Flat, Doubly Terminated, Type A” is selected.

- The measured DC loss is assigned as Passband Ripple (Passband Edge loss).

- In order to get the targeted return loss in the desired band, the upper band edge fp2 should be selected greater than the desired upper band edge (by an unknown amount).

- The lower band edge is selected as much less than fp2, close to zero.

- The desired RS and RL values are typed in, transmission zeros are entered and elements are extracted.

- In the resulting circuit, the transformer adjacent to the load is DELETED REGARDLESS OF ITS TURNS RATIO!!!.

The last step is naturally raising a question mark, but it turns out that the resulting filter has a perfectly matched band in a region below fp2. The edges of this band are unpredictable, and should be seen by trials.

Let’s try  fp1=1 MHz , fp2=100 MHz, RS=50, RL=250, PB Edge Loss (Ripple)=2.55273

The design commands are shown in Figure 3.23.a. The resulting circuit is shown in Figure 3.23.b. After deleting the transformer, we get the circuit of Figure 3.23.c. Response plots before and after deleting the transformer are shown in Figure 3.23.d and e. It is clear that we have to delete the transformer to get the proper response. The filter has 30 dB return loss (perfect matching) in the band from about 55  MHz to 85 MHz. In the rest of the band loss increases gradually to 2.55 dB (DC loss) at the specification band edges      (f=0  and f=100 MHz).

Exercise 3. 16: Try also fp1 = 0.1 and 10 MHz  to see their effects on response.

Exercise 3. 17: Change various parameters like fp1 , band edge loss, RL and degree to inspect the matching bandwidth and deformations in passband response for the maximally flat case.

Exercise 3. 18: Explain why we should delete the resulting transformer.

Exercise 3. 19: Repeat the design by following the iterative approach used in Chebyshev impedance transforming filter.

Figure 3.23. Design steps of a maximally  flat impedance transforming quasi lowpass filter.

EXAMPLE 3. 9. A quasi-lowpass filter with finite transmission zeros and unequal terminations.

Let’s design a matching-filtering structure with finite transmission zeros having a quasi-lowpass  (or quasi-bandpass) characteristics with equal stopband ripple, to match RS=50  to RL=250   within a band below 10 MHz using the approach described in Example 3.8. It will be designed as a bandpass filter without any transmission zeros at f=0.

Steps:

New / Design / By Synthesis

Filter Type:  Lumped, BandPass, Max. Flat, Doubly terminated, Type A

Passband Specs:

PB Edge Loss=2.55273 dB         (DC loss)

fp1=0.01MHz   (or 0.1 or 1 MHz-try!)

fp2=10 MHz

Specify TZs:

INF, INF, FTZ  (at 11 MHz), FTZ  (at 12 MHz), FTZ  (at 16 MHz)

(Finite transmission zeros are found through “Tune a FTZ” command to form roughly equal ripple stopband)

Extract TZs:

INF, FTZ  (at  11 MHz),  FTZ  (at 12 MHz),  FTZ (at 16 MHz) , INF

The circuit shown in Figure 3.24 is obtained.

Figure 3.24. An equiripple stopband quasi-lowpass filter with nonidentical terminations.

Exercise 3. 20: The above filter may also be obtained as follows:

- Use Synthesis approach again. However now specify the actually desired band edge loss to get the targeted minimum passband return loss. For example let the minimum return loss be 20 dB, corresponding to passband edge loss of  0.0434 dB.

- Set the RS and RL as desired.

- In this approach now both lower and upper passband edge frequencies are unknown. We have to make trials to set them properly to get the infinite return loss at band center, defined as f0 = (fp1*fp2)1/2 . Select trial fp1 and fp2 having this property. Synthesize the circuit. Inspect turns ratio of the transformer at load end. If it is different from unity, then try a new set of fp1 and fp2. Iterate until the transformer turns ratio comes out as unity. Then the design will be completed.

3. 7. DISTRIBUTED  ELEMENTS  APPROXIMATING  LUMPED  ELEMENTS

At high enough frequencies (when dimensions of elements are comparable to wavelength) some lumped inductors and capacitors can be replaced by their transmission line or stub equivalents. Let’s see some examples.

EXAMPLE 3. 10. Distributed element approximations for lumped elements

Let’s design a Chebysev filter with specifications

p=0.1dB,   fp=1000 MHz,    N=7,   RS=RL=50 ohms

Circuit is shown in Figure 3.25.a. Place the cursor under a series inductor and apply command Replace By TLine. The program will ask characteristic impedance and effective relative dielectric constant of the transmission line. Since inductors are best simulated by high  impedance (thin) lines, type in the highest impedance realizable with the intended  technology of production (for example 200 . Let effective dielectric constant be unity. The program will calculate the necessary length of the line to give approximately the same inductance. Repeat that transformation on all inductors giving the circuit of Figure 3.25.b. Insertion losses of the filter before and after these replacements are shown in Figure 3.25.c. Inspect deformations in passband by plotting insertion loss with suitable scales.

Figure 3.25. A lowpass filter with inductors replaced by high impedance transmission lines.

- Shunt capacitors may be replaced by shunt open circuited stubs. Place cursor under a capacitor and apply command “Replace By OC stub” command. Here again impedance and dielectric constant of the line will be asked. Exercise by entering low impedance values. For example for 30 ohms impedance the circuit of Figure 3.26.a will be obtained. The new response is shown in Figure 3.26.b. Inspect deformations in passband response by replotting  with suitable scales.

Figure 3.26. The lowpass filter of Figure 3.25.b after the shunt capacitors replaced by shunt open circuited stubs.

Figure 3.27. The lowpass filter after shunt capacitors  replaced by low impedance transmission lines.

Shunt capacitors may also be replaced by transmission line pieces (if length is of no problem). For that purpose, use Undo command to get back the “TLine + capacitor”  circuit of Figure 3.25.b. Place the cursor under a shunt capacitor and apply the command “Replace By Tline”. For capacitors low impedance lines are needed. Therefore type in a  low practical impedance (for example 30 ohms. The circuit of Figure 3.27.a will be obtained, with the response shown in Figure 3.27.b. Inspect responses after each replacement. Note that this is an all-TLine circuit with noncommensurate lengths.

Exercise 3. 21: Consider the elliptic and Generalized Chebyshev filters of the previous examples. Try to replace the series inductors and shunt capacitors by transmission line pieces, and shunt resonators by shunt OC stubs. Observe deformations in responses.

REFERENCES AND SUGGESTED READING:

- A. Zverev, “A Handbook on Filter Synthesis”,  John Wiley and Sons, Inc., NY 1967.

- D. S. Humpherys, “The Analysis, Design and Synthesis of Electrical Filters”, Prentice Hall, 1970.

- R. W.  Daniels, “Approximation Methods for Electronic Filter Design”, Mc Graw Hill, NY, 1974.

- J. D.  Rhodes , “Theory of Electrical Filters”, J. Wiley and Sons, NY, 1976

- W. K.  Chen , “Theory and Design of Broadband Matching Networks”,  Pergamon Press, 1976.

- R. Saal, “Handbook of Filter Design”,  AEG Telefunken, 1979.

- D. R. J. White, “A Handbook on Electrical Filters”, Don White Consultants Inc.,1980.

- J. D. Rhodes and S. A. Alseyab, “The Generalized Chebyshev Low Pass Prototype Filters”, Int. J. Circuit Theory and Applications, Vol. 8, pp. 113-125, 1980.

- S. A. Alseyab, “A Novel Class of Generalized Chebyshev Low Pass Prototype for Suspended Substrate Stripline Filters”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-30, No. 9, pp. 1341-1347, Sept. 1982.

- Hasler and J. Neirynck , “Electric Filters”. Artech House Inc. Mass.1986.