CHAPTER 4

LUMPED ELEMENT HIGHPASS FILTER DESIGN

4. 1. INTRODUCTION

Lumped element highpass filters can be designed using either Built-in Prototype or Synthesis approaches of Filpro. Chebyshev, Butterworth and Generalized Chebyshev filters are quicker to design using Prototype approach. Synthesis approach is required in designing filters with equiripple or maximally flat passband and general stopbands. The special cases like, equal stopband ripple (elliptic or inverse Chebyshev) filter characteristics can be obtained through Synthesis approach by using Tune a FTZ (Finite Transmission Zero) tool of  FILPRO which eases shaping of stopband response a lot. Other special classes, like Generalized Chebyshev filters (equiripple passband) or Generalized Butterworth filters (maximally flat passband) that are not included in the Built-in Prototype approach can be obtained by placing all transmission zeros at the same frequency in the stopband. More general lowpass prototypes can be developed and saved using Synthesis approach and to be used with Design / By User Defined Prototype approach. Bandpass filters with very wide passbands may also be used as highpass filters. Therefore bandpass filter synthesis approach will also be described in this chapter.

4. 2. CHEBYSHEV AND BUTTERWORTH HIGHPASS FILTER DESIGN EXAMPLES

EXAMPLE  4. 1. A Chebyshev highpass filter design using Built-in Prototype approach.

Let’s design a Chebyshev highpass filter with specifications

p=0.1 dB, fp=100 MHz , s=30 dB, fs=75 MHz , RS=RL=50 ohms

Steps of design using Prototype approach are as follows (Figure 4.1):

“New / Design / By Built-in Prototypes / Lumped, Highpass, Chebyshev / RS=RL=50 /

PB+SB specs: PB Ripple= 0.1, fp=100, SB Loss=30, fs=150 / Find Circuit”

Figure 4.1. Command sets for design of a Chebyshev highpass filter.

Figure 4.1. Command sets for design of a Chebyshev highpass filter (continued).

- A circuit satisfying these specifications will be displayed automatically (Figure 4.2.a).

- Quit Design. Use Find Dual command to display the dual circuit (Figure 4.2.b).

- Figure 4.2. The designed highpass filter and its dual.

- Insertion loss and return loss responses are plotted in Figure 4.3.

- Figure 4.3. Insertion and return loss responses.

- Phase, Delay and Smith Chart responses are shown in Figure 4.4. Exercise with Replot (F10) and Zoom (F9) keys.

- Figure 4.4. Phase, delay and Smith Chart responses.

- Use Parasitics / Lossy Element command to replace ideal inductors and capacitors by their practical equivalents containing parasitic effects, like Q of inductors and capacitors, lead inductances of capacitors, interwinding capacitances of inductors (Figure 4.5).

- Figure 4.5. Lossy model of the highpass filter.

- Figure 4.5. Lossy model of the highpass filter (continued).

- In Figures 4.6.a-to-d, degradations in insertion loss response are studied by changing various parasitic parameters:

- In Figure 4.6.a, effects of Q of capacitors and their lead inductances and resistances are displayed by assuming lossless inductors.

- Effects of Q of inductors and their parasitic (interwinding) capacitances, assuming lossless capacitors are studied  in Figure 4.5.b. In Figure 4.6.c, a wideband response is shown to demonstrate the effect of parasitic capacitances of inductors.

- The combined effects of lead inductances of capacitors and parasitic capacitances of inductors are shown in Figure 4.6.d.

Figure 4.6. Effects of parasitics on insertion loss.

Figure 4.6. Effects of parasitics on insertion loss (continued).

- In order to get an idea on mass production success rate, a Monte Carlo Analysis is carried out which includes only lead inductances of series arm capacitors and parasitic capacitances of shunt arm inductors as Monte Carlo Parameters. Lead inductances are assigned 2 nH and parasitic capacitances are assigned 3 pF each, with 90 % uniform distribution. Reference lines are placed as “30 dB, f=0-to-70 MHz” and  “3 dB, f=105-to-300 MHz”. Results are shown in Figure 4.7.

Figure 4.7. Monte Carlo Analysis results by varying only lead inductors of capacitors and parasitic capacitances of inductors.

Exercise 4. 1. Use Tune Element command on the main inductances and capacitances to improve the degraded return loss response of the lossy  filter.

Exercise 4. 2. Design some Butterworth filters with the same degree as above example and compare selectivities, phase linearities and delay flatness.

EXAMPLE  4. 2. A Chebyshev highpass filter with unequal terminations.

Design / By Built-in Prototype approach can be used to design highpass filters with unequal resistive terminations at the expense of an extra passband loss (termed as DC loss in lowpass filters) proportional to degree of mismatch between RS and   RL [1], [8]. Highpass filters with unequal terminations and no flat loss can be designed using Synthesis approach only. Let’s design two Chebyshev filters one with  “RS=50 RL= 250 ” and the other with RS=50 RL=10 . It is usual to use even degree filters, because odd degree ones produce larger ripple. Steps of design are as follows:

“New / Design / By Built-in Prototypes / Lumped, Highpass, Chebyshev /  RS=50 , RL=250 (or 10) / Even Order / PB Ripple=2.55,  fp=100,  SB Loss= 30,  fS=75 / Find Circuit”

The resulting circuits are shown in Figure 4.8.a and b, with identical insertion loss in Figure 4.8.c. Note that  a series arm capacitor appears at the low impedance termination side in both cases.

Figure 4.8. Chebyshev highpass filters with unequal terminations.

Exercise 4. 3. Use Find Dual command to show that actually, the circuits of  Figure 4.8.a and b are duals of each other.

Exercise 4. 4. Design some odd degree Chebyshev highpass filters  with the same terminations as in the above example and compare ripples.

Exercise 4. 5. Design some odd and even degree Butterworth filters with the same terminations as the above example and compare responses.

EXAMPLE 4. 3. Synthesis approach for design of Chebyshev and Butterworth highpass filters

Let’s design some Chebyshev and Butterworth highpass filters using Synthesis approach, to get exercise with the procedures. Filter specifications are given as

fp=100 MHz , RS=RL=50 ohms ,  N=9.

The following passband ripples will be tested for Chebyshev filters:

p=0.01 dB,  0.1 dB,  0.5 dB

For Butterworth filters, 3 dB  and  0.5 dB passband edge losses will be used.

The steps are as follows ( Figure 4.9):

“New / Design / By Synthesis /

Filter Type:  Lumped,  Highpass, Equiripple (or Maximally Flat), Doubly Terminated, Type-A

Passband Specs and Terminations: PBRipple= ... , fp=100, RS=50, RL=50

Transmission Zeros:  ZERO, ZERO, ......., ZERO  (9 Tzs)

Figure 4.9. Command sets for a Chebyshev or Butterworth filters designed through Synthesis approach.

Figure 4.9. Command sets for a Chebyshev or Butterworth filters designed through Synthesis approach (continued).

For PB Ripple, the specified passband ripples of the Chebyshev filters or the passband edge losses of Butterworth filters will be used. As the transmission zeros are inserted, building up of the insertion loss can be observed in the small box on the right of the screen. After inspecting the insertion loss of a filter, use Restart Synthesis command to study the next filter response. In Figure 4.10, responses of  the five filters are shown in overplotted mode. Note that selectivity (slope of the skirt) of Chebyshev filters increases as the passband ripple increases. Slopes of Butterworth filters don’t change by the passband edge loss, because one response is simply the shifted version of the other.

In Figure 4.11, element values of the several filters are shown underneath the circuit.

Figure 4.10. Insertion losses of three Chebyshev filters and two Butterworth filters of the same degree with differing passband ripples (passband edge losses).

Figure 4.11. Chebyshev and Butterworth filters designed through Synthesis approach.

- Remarks on Chebyshev and Butterworth highpass filter design with unequal terminations using synthesis approach:

Theoretically it is possible to synthesize highpass filters with unequal terminations as special cases of bandpass filters with no transmission zeros at infinity  (similar to lowpass filters with unequal terminations). For that purpose,

i) choose Bandpass Filter / Maximally Flat   options in specification stage of synthesis procedure,

ii) enter the band edge frequency of highpass filter as the lower passband edge of bandpass filter,

iii) enter an arbitrary upper passband edge as a trial parameter,

iv) specify the DC loss as band edge loss,

v) assign even number of transmission zeros at f=0.

The response will have an unpredictable pass bandwidth with zero loss. However, this approach may yield useful results only for low orders due to additional numerical inaccuracy problems because of absence of transmission zeros at infinity. Therefore this approach is not advisable. Instead, it would be better to design a normal wideband bandpass filter with some transmission zeros at infinity, resulting in circuits with readily adjustable parameters for impedance matching. Furthermore, this isalso useful for absorbing parasitic capacitors of shunt inductors which causes highpass filters to have bandpass character, as studied in Section 4.5, in this chapter.

4. 3. GENERALIZED CHEBYSHEV HIGHPASS FILTER DESIGN FROM  PROTOTYPES

Generalized Chebyshev filters are quasi-elliptic filters with all finite transmission zeros collected at a single frequency near the passband edges. In the LP prototype tables prepared by Rhodes and Alseyab [5], [6] finite transmission frequencies are adjusted to give some (commonly used) minimum stopband losses of 40, 50 and 60 dB. For passband ripple, only 0.01 and 0.05 dB are available. Tables are prepared for one and three transmission zeros at infinity. The transmission zeros of lowpass prototype at f= maps to f=0 in highpass filters. Therefore these filters may also be referred to as filters with one or three transmission zeros at f=0. Only odd degrees are available giving symmetric filters (Nmax=19  for Ninf =1 and  Nmax=15 for  Ninf=3). These tables are thought to be sufficient for many applications. Therefore no further table preparation is attempted in this book. For more general requirements, one may always resort to Design/By Synthesis approach giving all kinds of flexibilities in passband and stopband ripples, degree, RS/RL  ratio, number of transmission zeros at infinity, number of finite transmission zeros, etc.

EXAMPLE  4. 4. Some Generalized Chebyshev-1 highpass filters.

Let’s design some Generalized Chebyshev-1 highpass filter with following specifications:

fs=60 MHz , Degree N=9,  RS=RL=50 

For passband ripple and minimum stopband loss, we will try the values

p=0.01 dB  and   0.05 dB,  and     S=40 dB, 50 dB  and  60 dB

Design steps are shown in Figure 12:

“New / Design / By  Built-in  Prototypes / Lumped,  Highpass,  Generalized  Chebyshev-1 / RS= RS=50 / fp=60 , Degree N=9 / PBRipple=0.01, SB Loss= 60 / Find Circuit”

Figure 10.12. Command set for the design of a Generalized Chebyshev filter.

- The circuit shown in Figure 4.13.a will be displayed for  p=0.01 dB  and  S= 60 dB.

- Find Dual command gives the dual circuit shown in Figure 4.13.b. The other filters are also designed in the same way.

- Figure 4.13. Dual Generalized Chebyshev-1  highpass filters with p=0.01 dB and S= 60 dB.

- In Figure 4.14.a, effect of choosing p=0.01 dB  and   0.05 dB on  insertion loss is displayed.

- In Figure 4.14.b, effects of choosing  S=40 dB , 50 dB  and  60 dB are displayed. Note that selectivity (slope of skirt) increases as p  is increased. Also the lower minimum stopband loss yields higher selectivity for the given passband ripple.

Exercise 4. 6: Complete the element values of Figures 4.13.a and b for other passband ripple and minimum stopband loss values.

Exercise 4. 7: Use Tune Element command to tune one resonator element to inspect the change in response.

Figure 4.14. Variation of skirt slopes with passband ripple and minimum stopband loss.

EXAMPLE 4. 5: Realization of Generalized Chebyshev filter with coupled coils.

Consider the circuit of Figure 4.13.a repeated in Figure 4.15

Figure 4.15. Conversion of a hughpass filter into coupled coil form.

- Place the cursor under the first shunt resonator of circuit and apply command

“Coils / Csr + sLCsh + Csr-to-CapCoil / Capacitance on right (left)”

The triplet “Series capacitor+Shunt resonator+series capacitor” will be replaced by its coupled   
      coil equivalent having a remanent capacitor on its right or left, depending on the choice (Figure  
      4.15).

- Repeating this transformation on each shunt resonator and shifting transformers rightmost, we get the circuit of Figure 4.16.a.

- Combining the transformers, the circuit of Figure 4.16.b will be obtained.

Figure 4.16. A Generalized Chebyshev highpass filter in coupled coil forms.

- The remaining capacitor may be left at the source end by choosing Capacitor on Left option.

- Similar commands can also be applied on the dual circuit shown in Figure 4.17. With cursor under the series arm parallel resonator (pLCsr), applying command “Coils/ Lsh+pLCsr+Lsh-to-CCoil / Inductance on Right” will give the coupled coil equivalent of  the triplet Lsh+pLCsr+Lsh with a remaining shunt inductance and an ideal transformer (Figure 4.18.a).

Figure 4.17. Coupled coil commands.

- Shifting the transformer rightmost and applying the same command on all the triplets, we get the coupled coil version of the filter with a shunt inductor and ideal transformer as shown in Figure 4.18.b.

Figure 4.18. The dual Generalized Chebyshev filter in coupled coil form.

EXAMPLE  4. 6. A Generalized Chebyshev highpass filter with parasitics

Let’s consider a Generalized Chebyshev-1 filter with specifications

p=0.05 dB, fp=100 MHz,  s=60 dB, degree N=9

- Designed circuit is shown in Figure 4.19.a.

- With cursor under the shunt resonators, apply command Lossy/Lossless. This command converts ideal components into practical ones and vice versa. When applied to the shunt resonators, Filpro asks Q of inductor and capacitor together with the frequency. It then automatically calculates the series resistance Rs of inductor and parallel resistance Rp of capacitor. Then, it also asks the interwinding capacitance of the inductor, to be placed in parallel with  the L-R element. Using Q=50 for inductor, Q=100 for capacitor and C=0.1 pF for interwinding capacitors for all shunt resonators, we get the circuit of Figure 4.19.b.

- Responses are shown in Figure 4.20.a and b, for both ideal and practical cases. Note the rounding of sharp passband edge corner and degradation in return loss.

Figure 4.19. Lossy model of the Generalized Chebyshev highpass filter.

Figure 4.20. Effects of parasitics on response.

- The effects of higher interwinding capacitance can be studied by applying Change command on these elements to change that capacitance. As example, all the 0.1 pF capacitances are changed to 5 pF, to give the response shown in Figure 4.21 which looks like a bandpass filter rather than a highpass one.

- For these reasons it is wiser to design such highpass filters as wideband bandpass filters in which case the parasitic capacitances of inductors may be absorbed into the shunt capacitors that will come from transmission zeros at infinity (See Section 4.5).

Figure 4.21. Effect of interwinding capacitance.

Exercise 4.8. Consider the dual circuit of this example to study effects of winding capacitances of all inductors.

4. 4. SYNTHESIS OF HIGHPASS FILTERS WITH FINITE TRANSMISSION ZEROS

Using  Design-By Synthesis approach, it is possible to design filters with equiripple or maximally flat passband and general stopband characteristics. The stopband is shaped by  the passband ripple, the number of transmission zeros at f=0, and  the numbers and frequencies of finite transmission zeros. Positions of finite transmission zeros can be adjusted while observing the response using Tune a FTZ command. Transmission zero sections can be extracted in different orders, leading to different equivalent circuits. It is clear that with so many adjustable parameters and equivalent alternative circuits, Synthesis approach forms a flexible and powerful tool to satisfy widely differing requirements. Let’s see a typical design procedure on an example.

EXAMPLE  4. 7. An elliptic Highpass filter

Synthesize an equripple passband and equiripple stopband highpass (elliptic) filter with

p=0.5 dB, fp=60 MHz , Nzero=1 (1 TZ at  f=0), NFTZ=4  (four finite transmission zeros to be adjusted  to get an equal stopband ripple). As an initial trial value for the finite transmission zeros, let

FTZ1=30 MHz,  FTZ2=40 MHz, FTZ3=50 MHz  and FTZ4=55 MHz.

Design Steps are shown in Figure 4.22:

New / Design / By Synthesis

Filter Type:  Lumped,  Highpass, Equiripple, Doubly Terminated, Type-A

Passband and Terminations: PBRipple=0.5 , fp=60 , RS=50 ,  RL=50

Specify TZs: “ZERO, FTZ (at 30 MHz) , FTZ (at 40 MHz), FTZ (at 50 MHz),FTZ (at 55 Mhz)”

- As the transmission zeros are placed, the response is automatically updated and displayed in a small box on the top right of the screen.

- After typing in finite transmission zeros, use  Tune a FTZ  command to adjust them by up-down arrows with increments set by the user until equal stopband ripple is obtained. After some trial using 1 MHz  increments, the following FTZ frequencies are found to give equal stopband ripple (Figure 4.22).

FTZ1 = 25 MHz,  FTZ2 = 44 MHz, FTZ3 = 52 MHz  and FTZ4 = 55 Mhz.

- After adjustment of response, transmission zeros are extracted using Extract TZ Standard Auto Extract command. Different circuits can be obtained by extracting finite transmission zero sections in different orders. The circuit shown in Figure 4.23.a is obtained by Auto Extract option which extracts the transmission zeros in the order

“FTZ=25 MHz - FTZ2=44 MHz - FTZ3=52 MHz - FTZ4=55 MHz - TZ at ZERO”

- The circuit of Figure 4.23.b is the dual one, found through Find Dual command.

Exercise 4. 9: Extract TZ sections in different orders and compare turns ratio of transformers in each case. Compare the element values of resulting circuits.

Exercise 4.10 : In the previous filter place all finite transmission zeros at f=45 MHz. Compare responses and element value spreads.

Exercise 4.11 : Design an inverse Chebyshev type highpass filter (maximally flat passband and equiripple stopband) with the same passband edge frequency and same degree as previous ones. Compare Insertion Loss and Phase linearity with previous filters.

Exercise 4.12 : Design a Generalized Butterworth filter (maximally flat passband and all FTZs placed at the same frequency) with the same passband edge and same degree as previous ones.

Exercise 4.13: Replace ideal capacitors and inductors with practical ones (Lossy/Lossless command)  to inspect effects of losses.

Figure 4.21. Command sets for synthesizing an elliptic highpass filter.

Figure 4.22. Elliptic response after application of Tune FTZ command.

Figure 4.23. The dual circuits obtained through Synthesis approach.

Exercise 4.14: Convert the circuits of Figure 4.23 into the forms involving coupled coils.

Exercise 4.15: Use Sensitivity Analysis command on the elements of previous filters to find out the elements effecting the response most.

Exercise 4.16: Design some equiripple passband equiripple stopband filters with three transmission zeros at f=. Then collect all transmission zeros at the same frequency and redesign the filter. Compare the spreads of element values.

4. 5. WIDEBAND  BANDPASS  FILTERS  AS  QUASI-HIGHPASS  FILTERS

Highpass filters with passbands extending to infinity are not feasible in practice, mainly due to the parasitic capacitances of shunt arm inductances, as is already seen in the previous examples. These capacitances form a transmission zero at infinity, converting the highpass filter into a bandpass structure. The effects of these capacitances can be taken under control by placing an intentional transmission zero at infinity, leading to a shunt capacitor. This capacitor can be used to absorb the parasitic capacitances of shunt inductors. Let’s see some examples.

EXAMPLE 4. 8. Wideband Chebyshev bandpass filter as a quasi-highpass filter.

Assuming that we are interested in a passband from 20 MHz to 100 MHz. Instead of designing a highpass filter with passband edge at 20 MHz, it would be more convenient to design a bandpass filter with passband edges at fp1=20 MHz and fp2=100 MHz. Let’s design a Chebyshev bandpass filter with 0.01 dB passband ripple, one transmission zero at infinity and 9 transmission zeros at zero.

- Command set is given in Figure 4.24, with response shown in Figure 4.25. In this command set, the single transmission zero at infinity is extracted first and Type B is chosen to get the first element as shunt capacitor. Part of this capacitor will be shared to inductors to simulate their interwinding capacitances.

- The extracted circuit is shown in Figure 4.26.a. Let’s assume that each of the shunt inductors has about 3 pF of interwinding capacitance. The single shunt capacitor will be distributed in the circuit so that 3 pF of capacitors will appear in parallel with each inductor.

- The shunt capacitor is exchanged with its neighbor to form an LRight section (Figure 4.26.b).

- Using “Split / Specify Left Value: 3 pF” command, the shunt capacitor is splitted and the 3 pF is left adjacent to the first shunt inductor. The remaining shunt capacitor forms an LRight section with the adjacent capacitor in series arm (Figure 4.26.c).

- Pi-Tee-L / LRight-to-LLeft command is applied on the LRight section to move the shunt capacitor adjacent to the next shunt inductor (Figure 4.26.d). The resulting capacitor is shifted to righmost position. The shunt capacitor is moved to the right of the second shunt inductor using Exchange command to form another LRight section.

- The shunt capacitor is splitted again, leaving 3 pF in parallel with the second inductor, and the remaining LRight section is converted into LLeft section again. The transformer is shifted to the load end (Figure 4.26.e, f).

- This procedure is repeated until the load end is reached  as shown in Figure 4.26.g-to-n. In the circuit all inductors have 3 pF capacitance in parallel. A shunt capacitor of 2.68 pF remains as unabsorbed by the inductors. It can be left as it is. However, problem is not solved completely yet. There is a transformer at the load end. This transformer may be absorbed in the load if RL were different from RS, or it should be eliminated if RS=RL. In any case, a transformer elimination procedure is likely needed. This procedure can be carried out using the Pi-Tee-L transformations applied on the Pi sections formed by the capacitors, in an iterative way. One example is shown in Figure 4.26.n trying to create a transformer to reduce the transformer at the load end. Iterations are needed because any Pi-to-Pi transformation corrects one condition while some other conditions are upset. Naturally, this procedure requires knowledge of capacitors associated with inductors.

Figure 4.24. Command set for synthesis of bandpass filter.

Figure 4.25.Response of the bandpass filter.

Figure 4.26. Development of a wideband bandpass filter to simulate a highpass filter with inductors having parasitic interwinding capacitances.

Figure 4.26. Development of a wideband bandpass filter to simulate a highpass filter with inductors having parasitic interwinding capacitances (continued).

Exercise 4. 17: Extract the single transmission zero in the middle of the circuit. Split it into two equal pieces and distribute the two pieces into the two halves of the circuit in a symmetric manner to get a physically symmetric structure.

Exercise 4. 18: Use the transformer at load end for impedance matching to a load different from 50 ohms. That is the Norton transformers will not have to cancel each other.

EXAMPLE 4. 9. A wideband Generalized Chebyshev bandpass filter as a quasi highpass filter

Let’s design a wideband bandpass filter to pass the frequency range fp1=20 MHz and fp2=100 MHz, to be used as a quasi-highpass filter. Passband ripple will be 0.01 dB. There will be one transmission zero at zero and one transmission zero at infinity. Four finite transmission zeros will be placed, all in the lower stopband, at 10 MHz.

- Command set is shown in Figure 4.27.

- Transmission zeros are extracted in the order

Inf - FTZ - FTZ - FTZ - FTZ - Zero

giving the circuit shown in Figure 4.28.a.

Figure 4.27. Design steps of a Generalized Chebyshev bandpass filter.

Figure 4.27. Design steps of a Generalized Chebyshev bandpass filter (continued).

- The single shunt capacitor adjacent to the source (transmission zero at infinity) can be distributed into the circuit by applying Norton / Specify Transformer command on the series arm parallel resonators (FTZ sections) adjacent to the shunt capacitor. Norton command replaces the series arm resonator with Pi-section of parallel resonators, as shown in Figure 4.28.b together with the specified transformer. For transformer values greater than unity, the resonator elements on the left come out to be negative. These negative elements should be absorbed by the existing positive shunt inductor and capacitor. Therefore the turns ratio of transformer is bounded by the requirement of having net positive elements on the left. By adjsuting the turns ratio of transformer, it is possible to adjust the net (total) capacitance on left to the desired value, which will simulate the parasitic capacitance of the adjacent shunt  inductor.

- Let’s assume that the parasitic capacitances should be in the range of 2-to-3 pF.

- Trials with the turns ratio of the Norton transformer gave that k=1.075 is suitable for that purpose giving  2.79 pF net capacitance for the first shunt inductor (Figure 4.28.b, c). The net element values are found using Combine command. The transformer is shifted to rightmost position. Circuit is simplified by combining adjacent (combinable) elements. This way, the desired shunt capacitor is placed across the first shunt inductor while a second shunt capacitor is created adjacent to the second series arm resonator.

- The same transformation is repeated on the next series arm resonator, again by trials on transformer ratio, placing the parasitic capacitor of the second inductor and creating the third shunt capacitor.

- The procedure is repeated this way, until the load end is reached.

- Depending on required actual parasitic capacitors, Norton transformations may be repeated iteratively. At the end of procedure, we might have one or more unabsorbed shunt capacitors left in the circuit. They may be realized as they are.

Exercise 4. 19: Extract the transmission zero of infinty in the middle of the circuit and repeat the procedure while observing the symmetry of the structure.

Figure 4.28. Norton transformations for distributing the single shunt capacitor into the circuit.

Exercise 4. 20: Use the transformer at load end for matching to a load different from 50 ohms.

Exercise 4. 21: In the previous exercise, after matching the load, add a reactive part to the load in series or in parallel to simulate a complex load and use Tune Element command on elements of the filter to match the load to the real load (Single Matching problem).

Exercise 4. 22: Continue the previous exercise by adding a reactive part to the source resistance (Double Matching problem)

REFERENCES AND SUGGESTED READING:

- A. Zverev, “A Handbook on Filter Synthesis”,  John Wiley and Sons, Inc., NY 1967.

- D. S. Humpherys, “The Analysis, Design and Synthesis of Electrical Filters”, Prentice Hall, 1970.

- R. W.  Daniels, “Approximation Methods for Electronic Filter Design”, Mc Graw Hill, NY, 1974.

- J. D.  Rhodes , “Theory of Electrical Filters”, J. Wiley and Sons, NY, 1976

- W. K.  Chen , “Theory and Design of Broadband Matching Networks”,  Pergamon Press, 1976.

- R. Saal, “Handbook of Filter Design”,  AEG Telefunken, 1979.

- D. R. J. White, “A Handbook on Electrical Filters”, Don White Consultants Inc.,1980.

- J. D. Rhodes and S. A. Alseyab, “The Generalized Chebyshev Low Pass Prototype Filters”, Int. J. Circuit Theory and Applications, Vol. 8, pp. 113-125, 1980.

- S. A. Alseyab, “A Novel Class of Generalized Chebyshev Low Pass Prototype for Suspended Substrate Stripline Filters”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-30, No. 9, pp. 1341-1347, Sept. 1982.

- M. Hasler and J. Neirynck , “Electric Filters”. Artech House Inc. Mass.1986.