CHAPTER 5

LUMPED ELEMENT BANDSTOP FILTERS

5. 1. INTRODUCTION

In Filpro, lumped element bandstop filters can be designed using either  Built-in LC Prototype, Prototypes with inverters or User Defined Prototype approaches. Synthesis approach is not programmed for bandstop filters, as it would give the same circuits as those mapped from prototypes. This is because bandstop filters have only finite transmission zeros (no transmission zeros at zero and infinity), therefore  no flexibility exists for synthesizing alternative equivalent circuits. Thus, the readily available bandstop filter types are limited to Chebyshev, Butterworth, Generalized Chebyshev-1 and Generalized Chebyshev-3  prototypes of Filpro. However, for applications involving specifications that can’t be satisfied by these prototypes (like distinct or special finite transmission zeros, special passband ripples, etc.)  it  is always possible to prepare lowpass prototype tables with desired specifications by using Synthesis approach of Filpro in lowpass filter mode with  normalized terminations and unity passband edge frequency. Then Design/ By User Defined Prototype approach can be applied on this prototype. Naturally, User Defined Prototype approach is also applicable to prototypes found elsewhere (various books, handbooks, articles, etc.) and inserted element by element.

Prototypes with Inverters approach may be used to design bandstop filters with narrow passbands. That is, the designed filter will have a lowpass, highpass or bandpass nature with a narrow bandstop region inside the passband.

As will be seen in the forthcoming examples, narrow band BS filters have extreme element values. Too small and too large inductors and capacitors simultaneously in the same circuit leading to realizability problems. Fortunately in most applications narrow bandstop region is located within the passband of a cover LPF, HPF or BPF. As will be described in later parts of this chapter, the elements of the cover filter can be used to scale the unrealizable elements of the notch filter. In such cases the notch (SB region) separates the passband of the cover filter  into two passbands. Such filters are named as Dual Band Filters and treated in Chapter-16 using different exact approaches. This technique can also be used to scale the element values of the notch filter.

In bandstop filters transmission zeros of the lowpass prototype at infinity are mapped to the geometric center of stopband while the finite transmission zeros are mapped on the two sides of that geometric center. Lower and upper passband ripples are identical. Insertion loss response is geometrically symmetric around the center of passband f0  defined as

f02 = fp1  fp2  = fs1  fs2 = ...

where fp1 ,  fp2  and fs1  , fs2  are passband and stopband edges respectively.

5. 2. CHEBYSHEV AND BUTTERWORTH BANDSTOP FILTERS

Let's see the use of Filpro to design Chebyshev or Butterworth bandstop filters on an example.

EXAMPLE 5. 1. Some Chebyshev bandstop filters.

Let the lower and upper passband edges be

fp1=90 MHz ,  fp2=110 MHz

with upper stopband edge frequency specified as fs2=105 MHz  (setting also the lower stopband edge frequency as fs1 =fp1 fp2 /fs2=94.3  MHz). The minimum stopband loss at these frequencies is desired to be  40 dB. Terminations are RS=RL=50 Ohms. As passband ripple, we will try the values

PBRipple = 0.01 dB, 0.1 dB and  0.5 dB

Design step are (Figure 5.1):

"New / Design / By Built-in Prototypes / Lumped, Bandstop, Chebyshev / RS=50,  RL=50 / PB+SB Specs: PB Ripple=0.01 (or 0.1 or 0.5), fp1=90, fp2 =110, SB Loss=40, fs2=105 / Find Circuit"

The resulting circuit is shown in Figure 5.1.a. All three filters came out to be of order 14. Insertion loss responses for these three filters are shown in Figure 5.1.b.

Note that the spread of inductors and capacitors are too wide. Lmax / Lmin=22 and Cmax/ Cmin =20.  This is because the filter is a narrowband one. If these values are problematic for realization then the techniques described in section 5.5  of this chapter can be applied for scaling element values to realizable levels.

Figure 5.1. Some Chebyshev bandstop filters and insertion losses.

Exercise 5. 1: Consider the circuit for p =0.1 dB repeated in Figure 5.2.a. Place cursor under each element and apply  Replace By Lossy  command, getting the circuit of Figure 5.2.b. Choose Q=100 for both capacitors and inductors. The parasitic (winding) capacitances (Cw ) of shunt arm inductors were observed to distort the response most. In Figure 5.2.c insertion loss responses for Cw=0.01 pF, 0.5 pF, 1 pF and 2 pF are shown. Redraw the response for various lower Q values. Insert some inductors in series with series arm resonators to simulate lead inductances and replot responses.

Figure 5.2. Effect parasitic (winding) capacitance of shunt arm inductors on insertion loss of  a bandstop filter.

Exercise: Design narrower stopband filters (e.g  fp1=95 MHz, fp2=105 MHz, fs2=102.5 MHz) and inspect the effect of winding capacitance of shunt arm inductance Cw=0.01, 0.1, 0.5 and 1 pF. Explain heavier distortions in response compared to the above example. Repeat the design for wider stopband cases and inspect the same effect.

Exercise: In Figure 5.3.a, insertion loss responses of lossless and lossy cases with QL=50, QC=100 Cw=1 pF (winding capacitance of shunt inductor) are overplotted with reference lines placed at 3 dB,  f=0-to-85 MHz, “40 dB, f=95-to-103 MHz”, “3 dB, f=115-to-200 MHz ”. In Figure 5.3.b, results of a Monte Carlo Analysis is displayed where all parameters of the lossy circuit are included in the Monte Carlo Parameters List, with 5 % Gaussian Distribution. Repeat the analysis with  5 % Uniform Distribution. Why is the Uniform Distribution yields always worse results?

Figure 5.3. a) Effects of losses and parasitics on insertion loss response and , b) results of a Monte Carlo Analysis.

Exercise: Design some Butterworth bandstop filters with the same edge frequencies and stopband loss. Compare the orders, slopes and element values with those of Chebyshev ones. Repeat the same exercises with lossy elements.

5. 3. GENERALIZED CHEBYSHEV BANDSTOP FILTERS

Generalized Chebyshev lowpass prototypes have finite transmission zeros, all placed at a single frequency close to passband edge to produce the standard 40 dB, 50 dB and 60 dB minimum stopband loss [7], [8]. These zeros map to two frequencies on the two sides of center of stopband of bandstop filter, providing sharp skirts. Prototypes with one transmission zero at infinity (Generalized Chebyshev-1) yield bandstop filters with single transmission zero at stopband center and those with three transmission zeros at infinity (Generalized Chebyshev-3) produce three transmission zeros at stopband center.

EXAMPLE 5. 2. A Generalized Chebyshev-1 BS Filter

Let's design a Generalized Chebyshev-1 filter with specifications

PBRipple = 0.05 dB, fp1=90 MHz, fp2=110 MHz, SB Loss=60 dB, N= 7,   RS=RL=50 .

Design steps are as follows:

“New / Design / By Built-in Prototypes / Lumped, Bandstop, Generalized Chebyshev-1/ RS= RL=50 / fp1=90, fp2=110, N=7/ PBRipple=0.05, SBLoss=60 / Find Circuit”

The circuit of Figure 5.4.a will be displayed.

- The fourth order sections of the circuit create a pair of finite transmission zeros on the two sides of stopband center. They can be broken down into two second order sections by placing the cursor under these components and applying Split  command (Figure 5.4.b) (after Quit design command).

- By placing the cursor under each element, use command Replace By Lossy  to replace elements with their practical equivalents, giving the circuit of Figure 5.4.c. In this transformation, Q of capacitors and inductors are to be typed in by the user. In the shunt arm resonators, parasitic (interwinding) capacitances of  inductors (Cp) should also be entered by the user. This parameter seems to be the most effective one in distorting the response, especially for narrow stopband filters where capacitors in series with inductors are small enough to be comparable to the parasitic capacitances. The effects of finite Q of elements and parasitic capacitances are displayed in Figures 5.4.e-to-g, where insertion losses for lossless and lossy cases are shown for various Q and Cw  (winding capacitance) values.

- In Figures 5.5.a, b and c, sensitivity of response to series and shunt arm resonator element values of the lossless filter are displayed. These results are obtained by placing the cursor under the resonator to be inspected and command Sensitivity Analysis  is applied. In this analysis, one parameter of the element (e.g. inductance or capacitance) is varied between two limits specified by the user. Response is calculated and overplotted at ten equally spaced values of the parameter chosen. This analysis gives an idea of how effective an element will be in distorting the response. In Figure 5.5.d, a Monte Carlo Analysis is displayed where only one parameter of each element is inluded in the Monte Carlo Parameter List , with  only 2 % Gaussian standard deviation from the specified value is assumed.

- We note that element value spread is much worse than that of the Chebyshev case: More than 50 for both inductors and capacitors. Therefore element value adjustment techniques of section 5.5 of this chapter need be applied.

Exercise: Include all parameters of all elements in the Monte Carlo Parameter List and repeat the analysis. Repeat the analysis for Uniform  distribution, with 5 % deviation

Exercise: Apply Find Dual command to the original circuit shown in Figure 5.4.a. The circuit of Figure 5.6.a will result. The fourth order sections in series arms can also be broken down into two second order sections using Split  command (Figure 5.6.d). Repeat Sensitivity and Monte Carlo Analysis.

Figure 5.4. a) A Generalized Chebyshev -1 bandstop filter.  b) Splitted fourth order sections.

c) Filter with lossy practical elements. d, e, f, g) Insertion loss responses for various Q and Cw  values.

Figure 5.5. a) Sensitivity Analysis with respect to inductance of  the first series arm resonator on left

b) Sensitivity Analysis with respect to inductance of  the first shunt arm resonator on left, forming one of the lower frequeny finite transmission zeros (FTZ-1).

c) Sensitivity Analysis with respect to the second shunt arm resonator on left (one of the higher frequency finite transmission zeros- FTZ-2).

d) Monte Carlo Analysis.

Figure 5.6. Dual circuit of the filter of Figure 5.4.

Exercise: Design a Generalized Chebyshev-3 bandstop filter with the same specifications as the previous example. Compare responses and element values.

5. 4. BANDSTOP  FILTER  DESIGN  BY  SYNTHESIS AND USER  DEFINED  	PROTOTYPE  APPROACH

Although User Defined Prototype approach is intended basically for use with lowpass prototype tables found elsewhere (textbooks, handbooks, periodicals, etc.), lowpass prototype tables can always be prepared by the designer using Synthesis approach and these prototypes may then be used to design bandstop filters through User Defined Prototype approach. As a demonstration, let’s design an elliptic bandstop  filter by first designing an elliptic lowpass prototype.

EXAMPLE 5. 3. Elliptic Bandstop Filter design from an elliptic LP prototype

Preparation of the elliptic LP Prototype:

By definition, prototype filters are lumped element lowpass filters with unity passband edge radian frequency, wp =1 (giving  fp = 1/(2=0.15923 ) and unity source resistance. If degree of  the prototype is odd, then load resistance will also be unity. For even degrees, load resistance will be different from source resistance, and it is unknown. Its value is found by the program using the reflection zeros of the specified filter. Then the difference between the specified load resistance and calculated one is compensated by an ideal transformer in front of the load. Absorption of the transformer in the load gives us the actual value of the load resistance.

Steps are as follows:

New / Design / By Synthesis

Filter Type:      Lumped, Lowpass, Equiripple, Doubly Terminated, Type A

Passband Specs and Terminations:  PBRipple= 0.1 , fp = 1/ (2= 0.15923  ,  RS = RL = 1

Transmission Zeros:   Infinity

FTZ 1: 0.17

FTZ 2: 0.21

FTZ 3: 0.3

Finite transmission zeros are tuned using Tune FTZ  command until an approximately equal stopband ripple is obtained as described in Figure 5.7.a.

Extract TZs: Transmission zeros are extracted in the order

FTZ 1,  FTZ 2,  FTZ 3,  INF

giving the circuit of Figure 5.7.b. Inspection of response of the resulting circuit shows that RL =1 is the proper load (giving the specified passband ripple of  0.1 dB with passband edge at fp = 0.15923).

Save As: Proto-1. The resulting circuit is saved with the name  Proto-1.

Now this elliptic prototype is ready to be used in designing any type (lumped or distributed, LP, HP, BP or BS) of filter.

Elliptic bandstop filter design from the LP Prototype:

In order to design a bandstop filter with passband edge frequencies fp1 =90 MHz, fp2 =110 MHz, the following procedure has to be used:

New / Design / By User Defined Prototype

Load a Prototype: Proto-1

Element Type: Lumped

Passband type: Bandstop

Passband Specs: fp1 = 90 ,  fp2 = 110

Find Circuit

The bandstop filter circuit shown in Figure 5.7.c will be obtained with the response shown in Figure 5.7.d. It seems that element value spread problem is worse than in the Generalized Chebyshev case. That is, the element value adjustment techniques of the next section are needed.

Figure 5.7. a) Insertion loss response obtained in the synthesis specification stage through Tune FTZ command b) The synthesized lowpass prototype filter. c) The bandstop filter obtained using Design / By User Defined Prototype approach. d) Insertion loss response of the elliptic bandstop filter.

Exercise: Repeat the synthesis of the prototype by extracting finite transmission zero sections in different orders and compare the element values of the resulting elliptic filters with those of the Generalized Chebyshev one.

Exercise: Design an even degree lowpass prototype and then design a bandstop filter based on this prototype.