5. 5. 	NOTCH  IN  A PASSBAND

As it was seen in the examples of the previous section, for narrow reject bands, the approaches described above yield unrealizable (extreme) element values. Since BSF’s have no transmission zeros at f=0 and f=, we don’t have much flexibility for changing the topology or adjusting the element values. Only one design is possible. If a notch is required within a specified frequency band, that is, if the broad band characteristics of the overall filter can be lowpass, highpass or bandpass in nature, with a notch inside the passband, then we will have transmission zeros at f=0 and / or f= and we will have some flexibility for element value adjustments. Several approximate or exact solutions with realizable element values can be formed using the techniques described in this section.

The element value adjustment problem of narrowband BSF’s may be solved if we enforce some transmission zeros at f=0 and/or at f= by brute force without affecting the stopband response appreciably. In other words, our notch will be located  within the passband of a LPF, HPF or a BPF. Actually this is what we have in real life where we desire a stopband within a certain limited passband. That is we will combine a BSF circuit with a LPF, HPF or BPF in such a way that in the resulting topology circuit transformations would be possible to adjust the extreme element values. This process may need iterative trials or optimization. Actually in Chapter-16 Dual Passband design techniques are described. A BSF within the passband of a BPF splits the passband of the BPF into two passbands. Therefore the exact dual passband BPF techniques of Chapter-16 may be used to solve the element value adjustment problems that will be studied in this section. However the brute force and approximate approaches that are presented in this section may prove to be easier or quicker.

Let’s see some examples.

EXAMPLE 5. 4.  Notch within the passband of a LPF, a HPF or a BPF

Let’s consider the design of a BSF with following specifications:

PBRipple = 0.01 dB, fs1 = 390 MHz, fs2 = 410 MHz,

Equal ripple (Elliptic) Stop Band response is desired with a minimum of 40 dB loss.

We can first design a LP Prototype with a finite transmission zero and then use Design / By User Defined Prototype approach of the previous section to design a BFS with FTZ’s. However just for the sake of demonstration of the techniques let’s use Generalized Chebyshev-1 option of the Built in LC Prototype approach because it is quicker.

Design command set is shown in Fig. 5.8.

- Design / By Built in LC Prototype options are selected.

- Lumped - BSF- Generalized Chebyshev-1 options are selected.

- Terminations : RS=RL=50 ohms is typed in.

- Attenuation Specs: fp1=382 MHz,  fp2 = 418.5 MHz, Degree N=5  are typed in. At this stage, the values fp1=382 and fp2=418 MHz are found by trials to get fs1=390 MHz and fs2=410 MHz.

- PBRipple=0.01 dB and SB Loss=40 dB option is selected.

- Find Circuit command gives the circuit of Fig. 5.8.a.

- Quit Design command is invoked to enable the circuit transformations menu.

- Find Dual command gives the dual circuit shown in Figure 5.8.b which will be used to design the filter.

- With cursor under the series arm fourth order sections Split command is applied to split them into two second order sections. The circuit is saved as BS1a.fil. Note the extreme inductances. The shunt arm inductors are about 350 nH and 157 nH while the series arm inductors are below 1 nH. The ratio of maximum inductance to minimum inductance is greater than 350. An unacceptable figure. Such a spread of inductance values would give very poor practical response (low Q).  There is no way to reduce the spread of this circuit. Our aim is to reduce the spread by brute force insertion of proper elements into the circuit as will be shown in the succeeding stages. The element values to be inserted into the circuit can best be found by designing a LPF or HPF of the same degree as the BSF (degree 5).

Figure 5.8. A Generalized Chebyshev BSF.

Design of a LPF:

- Let’s first consider a BSF within the passband of a LPF.

- Just as a trial, let’s design a LPF with PBRipple=0.01 dB, fp=2000 MHz and degree N=5 using Built-in LC Prototype approach. We get the circuit shown in Figure 5.9.a.

- Find Dual command is applied to get the dual form which is more suitable for the use with the BSF we have designed (Fig. 5.9.b). It is saved as  LP1.fil.

- Response is shown in Fig. 5.9.c.

- The transformations that we want to apply are shown in Figs. 5.9.d and e. That is, in the BSF we have designed we will place shunt capacitors in parallel with shunt arm series resonators and series inductors in series with series arm parallel resonators. When we combine the shunt capacitor with the shunt arm series resonator, we get a shunt arm third order section whose inductor is less than the original inductor of the shunt arm series resonator. On the other hand when the series arm inductor is combined with series arm parallel resonator we will get a series arm third order section with inductances larger than the inductances of the original inductors. These properties will help to adjust the extreme inductances.

Figure 5.9. Design of a LPF.

Notch in the passband of the LPF:

Now we will combine the BSF with LPF:

- BS1a.fil is loaded (Fig.5.10.a).

- Then with cursor under the load, apply command Append Filter: LP1.fil to cascade the filter LP1 with BS1a (Fig. 5.10.b).

- Then with cursor under the rightmost element of BS1a, the command Exchange (Short Cut key: Ctrl+X) is applied to move the shunt capacitor of LP1 to the left of the pointed element (Fig. 5.10.c). This command is repeated until the shunt capacitor is moved to the leftmost position as shown in Figure 5.10.d.

- Then the same command is used to move the other elements of LP1 into BS1a as shown in Fig. 5.10.e. Note that the elements of LP1 are interleaving the elements of BS1a.

- In order to assign one C or L to each resonator of BS1a, we Split the series inductors to get the circuit of Fig. 5.10.f. Thus, we have formed the standard pairs of Csh+LC resonator and Lsr+LC resonator to apply the transformations for conversion of them into third order sections which will reduce the spread of inductors (capacitors).

- Finally, we apply Combine command to the adjacent pairs of C+LC elements to form third order sections (named as Triads on Filpro) to reduce the spread of inductor values (or capacitors) as shown in Fig. 5.10.h. Note that the ratio of maximum inductance to minimum inductance is reduced from 350 to about 10.

Exercise: Repeat the design with different values for fp of the LPF to get the least spread for inductors.

Exercise: Repeat the design of the BSF using User Defined Prototype approach with lower and higher degrees.

Figure 5.10. Notch in the passband of a LPF.

Figure 5.10. Notch in the passband of a LPF.

Design of a HPF:

Let’s now consider a BSF within the passband of a HPF.

- Just as a trial, let’s design a HPF with PBRipple=0.01 dB, fp=50 MHz and degree N=5 using Built-in LC Prototype approach. We get the circuit shown in Figure 5.11.a.

- Find Dual command is applied to get the dual form which is more suitable for the use with the BSF we have designed (Fig. 5.11.b). It is saved as  LP1.fil.

- Response is shown in Fig. 5.11.c.

- The transformations that we want to apply are shown in Figs. 5.11.d and e. That is, in the BSF we have designed we will place shunt inductors in parallel with shunt arm series resonators and series capacitors in series with series arm parallel resonators. When we combine the shunt inductor with the shunt arm series resonator, we get a shunt arm third order section whose inductances are less than the original inductor of the shunt arm series resonator. On the other hand when the series arm capacitor is combined with series arm parallel resonator we will get a series arm third order section with inductance larger than the inductance of the original inductor. These properties will help to adjust the extreme inductances.

Figure 5.11. Design of a HPF.

Notch in the passband of the HPF:

Now we will combine the BSF with HPF:

- BS1a.fil is loaded (Fig.5.12.a).

- Then with cursor under the load, apply command Append Filter: HP1.fil to cascade the filter HP1 with BS1a (Fig. 5.12.b).

- Then with cursor under the rightmost element of BS1a, the command Exchange (Short Cut key: Ctrl+X) is applied to move the shunt capacitor of HP1 to the left of the pointed element (Fig. 5.12.c). This command is repeated until the shunt capacitor is moved to the leftmost position as shown in Figure 5.12.d.

- Then the same command is used to move the other elements of HP1 into BS1a as shown in Fig. 5.12.e. Note that the elements of HP1 are interleaving the elements of BS1a.

- In order to assign one C or L to each resonator of BS1a, we Split the series capacitors to get the circuit of Fig. 5.12.f. Thus, we have formed the standard pairs of Csh+LC resonator and Lsr+LC resonator to apply the transformations of Fig. 5.11.d and e.

- Such brute force insertion of elements into the BSF will definitely distort the response. Since the LPF passband edge is selected far from the SB region the effect is not heavy as shown in wide and narrow span responses of Fig. 5.12.f. The degradation seen in the narrow span insertion loss response can be corrected by a slight tuning applied on resonators (Fig. 5.12.g).

- Finally, we apply Combine command to the adjacent pairs of C+LC elements to form third order sections (named as Triads on Filpro) to reduce the spread of inductor values  (or capacitors) as shown in Fig. 5.12.h. Note that the ratio of maximum inductance to minimum inductance is reduced from 350 to about 10.

Exercise: Repeat the design with different values for fp of the HPF to find the least inductance spread.

Exercise: Repeat the design of the BSF using User Defined Prototype approach with lower and higher degrees.

Figure 5.12. Notch within the passband of a HPF.

Figure 5.12. Notch within the passband of a HPF.

# Notch within the passband of a BPF: Dual band filters

Since a notch within the passband of a BPF separates the passband into two passbands, the resulting filter is named as Dual Band Filter for which exact and systematic approaches are described in Chapter-16. However the brute force technique used in the previous examples are also instructive, giving more insight into the problems and more flexible in setting the pass bandwidths of the two passbands. It should be noted that the approaches of the previous examples with LPF and HPF become just special cases of the dual band filters with fp1 going to zero (in LPF’s) or fp2 going to infinity (in HPF’s).

# Design of a wideband BPF:

The notch filter BS1 that we have designed will be located within the passband of a BPF with passband edges fp1BP < fp1BS  and fp2BP > fp2BS. For the BPF, we will use the terminology cover filter. In Chapter-16 on dual passband filters it was observed that if the cover filter band edges have the same geometric center as the stopband, then there will be less degradation in the responses of both BS and BP filters. That is it is advisable to have the following property:

fp1BP*fp2BP = fp1BS*fp2BS= fo2

Therefore let’s try geometrically symmetric band edges, for example

fp2BP = 2000 MHz,   fp1BP = fo2 / fp1BP = 79.8 MHz

The design command set is given in Fig. 5.13 for a Chebyshev BPF with BPRipple=0.01 dB, Degree N=5. The option pLCsh is selected. The resulting filter is saved as BP1.fil.

Figure 5.13. Design of a wideband cover BP filter.

Notch in the passband of a wideband BPF:

Now we will combine the BSF  BS1a  with BP1:

- BS1a.fil is loaded (Fig.5.14.a).

- Then with cursor under the load, apply command Append Filter: BP1.fil to cascade the filter BP1 with BS1a (Fig. 5.14.b).

- Then with cursor under the rightmost element of BS1a, the command Exchange (Short Cut key: Ctrl+X) is applied to move the shunt capacitor of BP1 to the left of the pointed element (Fig. 5.14.c). This command is repeated until the shunt capacitor is moved to the leftmost position as shown in Figure 5.14.d.

- Then the same command is used to move the other elements of BP1 into BS1a as shown in Fig. 5.14.e. Note that the elements of BP1 are interleaving the elements of BS1a. We also note that there is almost negligible distortion in both SB region and in PB regions.  This is due to the geometric symmetry of band edges we have selected. All other choices would lead to slightly or appreciably higher degradations (Exercise?).

- In order to assign one C or L to each resonator of BS1a, with cursor under any shunt arm parallel resonator (pLCsh) or under series arm series resonator (sLCsr) we apply Split / All Two Element Resonators command to split all the shunt parallel LC resonators and series arm LC resonators into their inductor and capacitor components to get the circuit of Fig. 5.14.f.

- Contrary to the cases with LPF or HPF cover filters, in BPF cover case we have more options for the formation of different third order sections. We can combine the resonators with adjacent C or L to form third order sections (Triads) to get better behaved element values. In order to see which option leads to better inductor (capacitor) values we have to try both options.

- For example the highest inductance of the circuit is the leftmost one which is 126 nH. We should reduce it. For this purpose we use Exchange command to place this inductor to the left of the shunt arm series resonator (Figs. 5.14.f-g). Then Combine command is applied giving the third order section shown in Fig. 5.14.h which has 93 nH and 33 nH. If 93 nH is considered to be still too high then we can reduce the original 126 nH by brute force (e.g. using Tune Element command) while observing the degradations in responses. We can stop reducing inductance when acceptable degradation limit is reached.

- Next, we consider the series arm elements. The series capacitor of 29 nH is Split into two equal (or unequal) parts and using Exchange command one piece is moved to be adjacent to the left parallel resonator as shown in Fig. 5.14.h.

- Using Combine commands we combine the series capacitors with the adjacent parallel resonators to form triads as shown in Fig. 5.14.i. Note that the inductors of about 0.9 nH are increased to about 13-14 nH. (Exercise: Repeat this procedure by splitting the series inductor for comparison).

- Next, the shunt elements in the middle of the circuit will be considered. Combining the shunt LC resonator with the adjacent shunt inductor will yield a two inductor triad while combining the resonator with shunt capacitor will yield one inductor triad with lower inductance. Therefore we apply Exchange command to move the capacitor to be adjacent to the resonator (Fig. 5.14.k) and then Combine them (Fig. 5.14.m).  Because of symmetry, the right half of the circuit can be formed using Symmetrize and Delete commands (Fig. 5.14.m).

Exercise: Repeat the design with different values for fp1 and fp2 of the BPF to settle inductor values at a higher level.

Exercise: Repeat the design with values fp1 and fp2 of the BPF not satisfying the geometric symmetry property.

Exercise: Repeat the design of the BSF using User Defined Prototype approach with lower and higher degrees.

Figure 5.14. Notch within the passband of a wideband BPF.

Figure 5.14. Notch within the passband of a wideband BPF.

# Design of a medium bandwidth BPF:

The notch filter BS1 that we have designed will be located within the passband of a BPF with passband edges fp1BP < fp1BS  and fp2BP > fp2BS. For the BPF, we will use the terminology cover filter. In Chapter-16 on dual passband filters it was observed that if the cover filter band edges have the same geometric center as the stopband, then there will be less degradation in the responses of both BS and BP filters. That is it is advisable to have the following property:

fp1BP*fp2BP = fp1BS*fp2BS= fo2

Just to see what will happen if we don’t consider this constraint, let’s try the following band edges for the cover filter:

fp1BP = 250 MHz,   fp2BP = 500 MHz

The design command set is given in Fig. 5.15 for a Chebyshev BPF with BPRipple=0.01 dB, Degree N=5. The option pLCsh is selected. The resulting filter is saved as BP2.fil.

Figure 5.15. Design of a medium bandwidth BP cover filter.

Figure 5.16. Notch within a medium bandwidth BP filter.

Figure 5.16. Notch within a medium bandwidth BP filter.

Figure 5.16. Notch within a medium bandwidth BP filter.

REFERENCES AND SUGGESTED READINGS:

- D. S. Humpherys, “The Analysis, Design and Synthesis of Electrical Filters”, Prentice Hall, 1970.

- K.  Chen , “Theory and Design of Broadband Matching Networks”,  Pergamon Press, 1976.

- Zverev, “A Handbook on Filter Synthesis”,  John Wiley and Sons, Inc., NY 1967.

- W.  Daniels, “Approximation Methods for Electronic Filter Design”, Mc Graw Hill, NY, 1974.

- Saal, “Handbook of Filter Design”,  AEG Telefunken, 1979.

- R. J. White, “A Handbook on Electrical Filters”, Don White Consultants Inc.,1980.

- D. Rhodes and S. A. Alseyab, “The Generalized Chebyshev Low Pass Prototype Filters”, Int. J. Circuit Theory and Applications,Vol. 8, pp. 113-125, 1980.

- A. Alseyab, “A Novel Class of Generalized Chebyshev Low Pass Prototype for Suspended Substrate Stripline Filters”, IEEETrans. Microwave  Theory and Tech. Vol. MTT-30, No. 9, pp. 1341-1347, Sept. 1982.

- D.  Rhodes , “Theory of Electrical Filters”, J. Wiley and Sons, NY, 1976.

- Hasler and J. Neirynck , “Electric Filters”. Artech House Inc. Mass., 1986.

- P. R. Geffe, “Novel designs for elliptic bandstop filters”, RF Design, pp. 75-76, Feb. 1999.