CHAPTER 6

LUMPED  ELEMENT  BANDPASS  FILTERS

6. 1. INTRODUCTION

Bandpass filters are by far the most frequently needed filters compared to other types. This is because of crowded nature of frequency spectrum forcing all communication systems to work only in specified frequency bands with strict constraints on bandwidth, selectivity, stopband attenuation, etc. Also frequency up or down conversion techniques inside the communication systems require bandpass filters with diverse specifications. Each system, depending on specific properties of application and the technology of production used, requires bandpass filters of widely differing topologies and properties. Fortunately, because of existence of transmission zeros both at f=0 and f=, bandpass filters can be synthesized in a large number of exactly or approximately equivalent circuits.

Coexistence of transmission zeros at f=0 and f= enables one also to develop circuit transformations for

- adjusting element values,

- impedance matching,

- converting filter circuits into proper topologies for realization.

For these reasons the largest portion of Filpro is assigned to design and circuit transformation tools for bandpass filters.

Lumped versus Distributed:

This chapter will focus on only lumped element filters. However as frequency gets higher, lumped and distributed element filters become difficult to differentiate, especially when physical size of elements become comparable with the wavelength in the frequency range of operation. This is because some elements which are treated as lumped at low frequencies change nature as frequency gets high and it may be better to treat them as distributed elements while some distributed elements may be treated as lumped elements at low enough frequencies. Thus, most of the high frequency filters can be treated as mixed lumped and distributed element filters. In such cases both lumped and distributed element filter design approaches can be used to design the filters. For this reason some of the filters that will be described in this chapter may also be designed using the techniques that will be described in the forthcoming chapters on distributed element filters. Filpro has the tools for conversion of lumped filters into distributed and vice versa.

Bandpass filter Classifications:

A systematic presentation and treatment of bandpass filters is a really complex problem because of several factors:

- Diverse kinds of manifacturing technologies enforce the use of certain preferred topologies and certain element types. We will call these topologies Filter Types.

- Different design techniques exist some of which suits better for the design of some of the filter types.

- Different designs techniques require different circuit transformation tools to convert the designed circuits into desired topologies with realizable element values.

- A systematic presentation for educational purposes and for professional use have different priorities.

This chapter, as a continuation of the previous chapters, will give weight on educational aspects of the problems to make the user familiar with the links between theoretical and practical aspects of filters while also introducing the basic design and circuit transformation tools. In depth treatments of certain commonly used filter types will be studied after the introduction of distributed element filters in Chapter-7.

This chapter will be limited to only some typical lumped element examples as demonstrations for the uses of some of the design techniques and circuit transformation tools of Filpro.

Design Tools:

The following tools are available in Filpro capable of designing lumped element bandpass filters:

- Design / By Built-in LC Prototype

- Design / By Prototype with Inverters

- Design / By Synthesis

- Design / By User Defined Prototypes

Among these, Design / By Synthesis is the most general and most flexible approach, covering all the other approaches as special cases. Built in LC Prototype and Prototype with Inverters approaches have limited capabilities and may contain gross approximations in wideband filters. However they are also useful in many cases, providing short cut tools for surveying or other purposes. User Defined Prototype approach is intended for use with design of special filter types whose lowpass prototype tables are not available in Filpro, but available in other resources (handbooks, textbooks, periodicals, etc.).

Why Synthesis?

Design / By Synthesis approach is the most general and versatile tool of Filpro for designing filters. Placement of finite transmission zeros and tuning them while observing response of the filter and variety of circuit transformations for converting the synthesized filter into various topologies with realizable element values are the most distinguishing features of this approach.

Compared to the classical elliptic type filters designed from lowpass prototype tables, the following important advantages worth to mention:

- In filters designed by mapping from LP prototypes, equal number of transmission zeros exist at f=0 and f= while in Synthesis, one can place different number of transmission zeros at f=0 and f=, with the only constraint being Nzero+Ninf = Even Integer.

- In filters designed from LP prototypes with finite transmission zeros, a finite transmission zero of the prototype is mapped onto two sides of the passband, leading to geometrically symmetric response. That is fz1fz2=fp1fp2 =f02 where fz1 and fz2 are the finite transmission zero frequencies, fp1 and fp2 are passband edge frequencies and f0 is the geometric center of passband. The minimum stopband insertion loss is the same in both stopbands. In Synthesis approach there are no such constraints. Any number of finite transmission zeros can be placed in any stopband. Thus asymmetric upper and lower stopband losses can be formed with increased rejection in selected frequency ranges.

- Different numbers of transmission zeros at f=0 and f= may provide great flexibility for transforming the synthesized circuit into different topologies.

Classifications: Symmetry-Antimetry

One classification of bandpass filters from Network Theory point of view is symmetry and antimetry. Electrically symmetric filters will result if

Nzero= Odd integer  and   Ninf = Odd integer

and antimetric filters will result if

Nzero= Even integer  and  Ninf = Even integer

As will be seen in elliptic filters electrical symmetry does not guarantee physical symmetry. Physically symmetric structures may be formed by proper circuit transformations. Bandpass filters with Nzero+Ninf=Odd integer can not be synthesized using Filpro. Such filters are termed as parametric filters, requiring special approximation theory approach. However, for narrowband cases, such filters can be obtained through some approximations, as will be demonstrated later in this section.

Classifications: Type A, B (Duals) -Minimum Inductor–Direct Coupled Resonator–Cross Coupled Resonator Configurations

Just to ease the presentation of examples of this chapter, we may classify circuit types as follows:

- In the Design stage we have the option of selecting one of the two dual circuit solutions termed as  Type A filters  where the circuits start with series elements and Type B filters where the circuits start with shunt type elements. In any stage of circuit transformations we can pass to the dual circuit by applying Find Dual command. One form may prove to be more convenient for reaching the targeted structure.

- One of the commonly used class may be termed as Minimum Inductor configurations where among the multitude of solutions, the ones with least number of inductors is selected and in the circuit transformation stages care is exercised not to increase the number of inductors.

- Another commonly used class is termed as Direct Coupled Resonator configurations where shunt or series resonators are coupled to each other through L, C or LC elements. The LC elements create the finite transmission zeros.

- Cross Coupled Resonator configurations involve bridge type coupling between nonadjacent resonators. In these structures the bridge couplings are responsible for creating the finite transmission zeros.

Still further classifications are possible depending on various other factors but these are beyond the targets of this manual.

Transmission zero extraction order?

Filpro allows extraction of the last transmission zero at f=0 after all the finite transmission zeros in lower stopband are extracted. Similarly, extraction of the last transmission zero at f= is allowed after all the finite transmission zeros in upper stopband are finished. Beyond these constraints, transmission zeros can be extracted in any order leading to different solutions. When degree of the filter is high, too many solutions may exist and the “best” solution may not be clearly seen. For some commonly used transmission zero patterns, built in extraction orders are programmed as the command Standard Auto Extract to provide the user a guideline. In general it is usual to work out all possible solutions. However the following  rules of thumbs may be used as guide:

- There are two rough indicators of improper transmission zero extraction orders: Element values and the transformer at the load end. If transmission zeros are extracted in random order, element values may have extremely wide dynamic ranges. That is, too large and too small inductor and capacitor values may result. Also the load end transformer will have extremely large or small turns ratio. The circuit transformations that will be carried out in the later stages can overcome both element value dynamic range and the transformer turns ratio problems. However a judicious choice of element extraction order may directly lead to a reasonable transformer turns ratio and less element value dynamic range, easing the jobs of circuit transformation stages. Experience shows that a roughly symmetrical appearance of the resulting circuit may lead to better behaved element values and a load end transformer close to unity.

- If minimum number of inductors is desired, then we have to check all the possible transmission zero extraction orders. Numbers of inductors are set by the following factors:

- Numbers of transmission zeros at f=0 and f=.

- Numbers of finite transmission zeros in lower and upper stopband.

- Extraction orders of finite transmission zeros and the transmission zeros at f=0 and f=.

- In Filpro a transmission zero in lower stopband (LSB) can be extracted only if there is a transmission zero at f=0. Therefore the transmission zeros at f=0 should not all be extracted before extracting all the finite transmission zeros in LSB. The finite transmission zeros are realized in one of the two forms shown in Figure 6.1.a and b. The one in Figure 6.1.a has less number of inductors. Therefore if minimum number of inductors is targeted, then a transmission zero in LSB should be extracted in this form. The conditions under which this form of extraction is possible for ALL transmission zeros in LSB depends on the number of transmission zeros at f=0, their extraction orders and on selection of Type A or Type B in Design stage.

- In Filpro a transmission zero in upper stopband (USB) can be extracted only if there is a transmission zero at f=. Therefore the transmission zeros at f= should not be finished (extracted) before extracting all the finite transmission zeros in USB. Such finite transmission zeros are realized in one of the two forms shown in Figure 6.1.c and d. The one in Figure 6.1.c has less number of inductors. Therefore if minimum number of inductors is targeted, then a transmission zero in LSB should be extracted in this form, if possible. The conditions under which this form of extraction is possible for ALL transmission zeros in USB depends on the number of transmission zeros at f=, their extraction orders and on selection of Type A or Type B in Design stage. In summary, minimum number of inductor configurations need searching of all the possible solutions. Some typical examples will be presented below as demonstrations for the use of some circuit transformations and some systematic approaches to get realizable standard topologies.

- Coupled resonator type filters are more amenable to systematic treatment. Well developed algorithms exist in Filpro which are applicable to all coupled resonator type filters.

Figure 6.1. Circuit sections realizing FTZ’s in lower stopband (LSB) and upper stopband (USB).

{It should be noted that by the term minimum inductor configurations, we are talking about the possible solutions that can be obtained from Filpro, by placing all transmission zeros only on s=jw axis and extracting them in different orders in Type A and Type B forms. In network theory terminology, minimum inductor circuits corresponds to circuits involving an s= axis  transmission zero, a subject  which is not included into the present version of  Filpro.}

Applicable circuit transformations:

As will be seen, synthesized circuits will have the following types of problematic points needing special treatments to overcome:

- Depending on the transmission zero extraction order, element value dynamic ranges are usually too wide. That is too large and too small inductances and capacitances can result in the same circuit.

- An ideal transformer results at the load end of the circuit. It must be eliminated.

Both of these problems can be overcome by using the Pi-Tee-L, Norton and Double Norton transformations of Filpro which produce transformers that can be used to scale elements and eliminate the load end transformer. Since all of these transformations increase the number of elements, in applications where minimum number of inductors is important, if possible, Pi-Tee-L transformations are applied on Pi-Tee-L sections made up of capacitors, for example as described in Figure 6.2.a and b. If no such circuit sections can be formed or if the existing sections can’t produce sufficient transformer turns ratio, or if more proper for any other reason, we may also mostly apply Norton transformation on the finite transmission zero producing elements as shown in Figure 6.2.c and d. In Figure 6.2.c the shunt LC element pointed by the cursor is multiplied by constants and added to both sides of the pointed element as series LC. The added elements on one side are negative. The negative elements must be absorbed by the neighboring positive LC elements. Therefore the element on which Norton command is applied must have a positive series LC on one side. On the other side there must be an inductor to avoid increasing the number of inductors of the circuit. Note that in this circuit the total number of inductors remained the same.

Similar reasoning apply to the transformation shown in Figure 6.2.d, in dual form. In these transformations the resulting transformers help both elimination of the load end transformer and scaling of the other elements of the circuit. The negative elements also aid in scaling the adjacent elements.

Clearly, Norton transformation and all other Pi-Tee-L transformations can be used if number of inductors is of no concern.

- Double Norton commands are just more sophisticated forms of Norton transformations. They are applied on double finite transmission zero sections that are extracted back to back and used for both element values adjustment and elimination of load end transformers. Some applications examples will be presented in the succeeding examples.

- Even if the two problems stated above are overcome, the overall impedance level of the circuit may be too low or high. This problem is most severe in narrowband filter circuits. Impedance level of such circuits  can be raised to desired levels by inserting ideal inverters at source and load ends. These ideal inverters can also be used to absorb the load end transformer, if it was not possible to eliminate it in the previous transformations. Then in the final stage, these ideal transformers can be replaced by their proper practical equivalents. In Filpro, the inverter approach and many of the other circuit transformations are elaborated mostly for Type B filters. Type A filters can be treated by converting them into Type B form using Find Dual command.

- Both dynamic range problem and impedance level problems of some Type A filters may also be treated using the series-to-shunt conversion commands listed under Impedance Matching item, together with Pi-Tee-L, Norton and Double Norton transformations, as will be demonstrated in some of the following examples.

Figure 6.2. Some Pi-Tee-L commands as applied on capacitive circuit sections and Norton commands applied on finite transmission zero producing resonators without increasing the number of inductors.

6. 2.  A CLASSIFICATION  FOR CHEBYSHEV-BUTTERWORTH  TYPE BANDPASS FILTERS

Bandpass filters without finite transmission zeros (having transmission zeros only at f=0 and f=) are usually simpler to construct. Therefore they are among the most commonly used filters. Chebyshev and Butterworth filters are among such filters. Since they have no finite transmission zeros, their selectivity is controlled only by the degree of the filter. In Filpro, such filters can be designed in the quickest way by using  Design / By Built in LC Prototype approach. This approach offers the most commonly used 15 exactly or approximately equivalent alternative circuits instantly. However formulations for some types involve gross approximations leading to erroneous results for some cases. Therefore we will use this approach just to describe a classification for such filters. Our main design tool will be Synthesis approach because of its generality and exact nature. Design / By Prototypes with Inverters approach will also be described as an alternative tool that may be useful a short cut way to get symmetric filters for both odd and even degree filters.

EXAMPLE 6.1 Bandpass filter classification using Built in LC Prototype approach

Built in LC Prototype approach can be used to display the most frequently bandpass filter types instantly. As an example, let’s list the filter types that are readily available for a Chebyshev filter with specifications:

PBRipple = 0.01 dB, fp1 =95 MHz,  fp2 =105 MHz RS=RL=50 Ohms, Degree =10  (prototype degree N=5)

Command sets using  Design / By Built in LC Prototype  approach are as follows (Figure 6.3):

“New / Design / By Built in Prototype / Lumped , BP, Chebyshev /  RS =RL=50 / PB Specs + Filter Degree: PB Ripple=0.01, fp1=95,  fp2 =105, Prototype Degree N=5 / Find circuit”

After entering the last command, the name list of 15 alternative bandpass filter structures are displayed in the dialog box to be selected by the user. The list, at first sight, seems as an “alphabet soup”, requiring the user to now the terminology:

Type-1:    sLCsr + pLCsh, 			Type-2:     pLCsh + sLCsr,

Type-3:    Csr + CCoil + Csh   		Type-4:     Csh + CCoil + Csr,

Type-5:    Csr+ CCoil + Csr,	 	Type-6:     sLCsr + Lsh

Type-7:    pLCsr + Csr,	            		Type-8:     sLCsr + Csh,

Type-9:    pLCsh + Lsr    			Type-10:   sLCsr + Lsh / Csh,

Type-11:  sLCsr + Csh / Lsh,  		Type-12:   pLCsh + Csr /Lsr,

Type-13:  pLCsh + Lsr / Csr		Cohn: Inv+pLCsh

Cohn: Inv+sLCsr

The corresponding circuits are shown in Figure 6.4. The terminology used for the names of circuits is hoped to be self explanatory, as the letters used describe the first two or three elements of the circuits. For example

sLCsr + pLCsh       : Series LC in series arm + parallel LC in shunt arm

Csh + CCoil + Csr  : Shunt arm C + coupled coil + series arm C

pLCsh + Lsr / Csr  : Parallel LC in shunt arm followed by series arm L and  series arm C, 				                                  in alternating order.

Besides these types, the two prototypes of Cohn [8] (Inverter+pLCsh and Inverter+sLCsr) involving ideal inverters and LC resonators are also included in the options list. These two prototypes can also be used to derive the other 13 filter types. However their main use will be clear in Chapter 10, on Distributed Element filters where the lumped LC resonators will be replaced by equivalent distributed element resonators.

Figure 6.3. Command set for Design / By Built in LC Prototype approach and alternative solutions.

Figure 6.4. Alternative solutions available through Design / By Built in LC Prototype approach.

All of these alternative circuits are derivable from Type-1 filter using some exact or approximate transformations as will be shown in the forthcoming sections. Also all of these and many more alternative bandpass filter circuits can be designed in an exact manner using Synthesis approach. Realizability of the resulting element values in a preferred topology is usually the main concern in most of the design problems. Depending passband center frequency, pass bandwidth, passband ripple, filter degree and terminations, some of these circuits may yield better behaved element values. This is the main reason for producing so many alternative circuits [15].

Inspection of the circuits shows that most of them involve series and/or parallel LC resonators coupled through L, C, LC, coupled coils and ideal inverters. It is common to classify such filters as Direct Coupled Resonator Filters. Some of these circuits can be grouped in accordance with the commonality of circuit elements, topologies and circuit transformations that they involve on the way towards realization, as follows.

a) Direct coupled resonator filters with inductive or capacitive couplings:

Most of the circuits listed in Figure 6.4 are formed by shunt parallel LC resonators (pLCsh) coupled to each other through  series inductors or capacitors, or series arm series LC resonators (sLCsr) coupled to each other through shunt inductors or capacitors. Type-6 to Type 13 fall into this group. These filters are by far the most frequently used ones because of simplicity of couplings between resonators.

b) Direct coupled resonator filters with LC type couplings:

Type 1 and Type 2 involves LC type elements in both series and shunt arms. Therefore they may be treated as coupled shunt resonator (pLCsh) filters with series LC type couplings or coupled series resonator (sLCsr) filters with shunt LC type couplings. In general they deserve a separate treatment from the other coupled resonator filters types because they form a reference circuits in many respects. All the other filters types and many more alternative circuits can be derived from these two types.

c) Inverter coupled resonator filters:

These filters are formed by either shunt LC resonators (pLCsh) coupled by ideal inverters or series LC resonators (sLCsr) coupled through ideal inverters. Ideal inverters provide great flexibility in many respects like element value adjustments, impedance matching, conversion of the circuits into desired forms, etc., as will be seen later in this chapter and in most of the succeeding chapters. After the inverters are used for such purposes, towards the end of the design, they are replaced by their practical equivalents. In the convention used in Filpro, a positive coupling inverter means an inductive coupling and negative inverter means capacitive coupling. I

d) Coupled Coil Filters

These filters are formed by series and /or shunt capacitors coupled through magnetically Coupled Coils. Type 3, Type 4 and Type 5 fall into this category.

In the following sections some of these categories will be studied in detail through typical design examples which will also serve to introduce some commonly used circuit transformation tools.

6. 3. DIRECT COUPLED RESONATOR FILTERS

6. 3. 1. BUILT-IN LC PROTOTYPE APPROACH

On Filpro the filters with shunt or series resonators with single element type coupling (inductive or capacitive) will be termed as direct coupled resonator filters. In Built in LC Prototype approach, the solutions Type 6 to Type 13 fall into this class. In order to survey different design and circuit transformation techniques that are available in Filpro and also to give some insight on the effects of various approximations, examples will be presented using different approaches. However the main design tool will be Design / By Synthesis approach. As will be seen, both synthesis and circuit transformation tools are common to all such filters. Filters with shunt resonators and series resonator are just duals of each others. Most of the circuit transformations are developed for shunt resonator type filters to avoid unnecessary duplication. Therefore design of series resonators filters will initially be designed as shunt resonator filters and they will be converted into series resonator forms by taking the dual of the circuit after the necessary circuit transformations.

- Example 6. 2: Built in LC Prototype approach for direct coupled resonator filters

As examples to compare the results of exact designs that will de presented in the succeeding sections, the following Chebyshev filters are designed using Built in LC Prototype approach:

PBRipple =0.01 dB, RS=RL=50 Ohms, Degree=10  (prototype degree N=5)

with band edge frequencies

fp1=95 MHz,   fp2=105 MHz , for Filter-1   (10% bandwidth narrowband filter)

fp1=80 MHz,   fp2 =120 MHz , for Filter-2   (40% bandwidth medium band filter)

Wideband filters should not be designed using this approach, because the formulations involve gross approximations leading to erroneous results in most of the direct coupled resonator filters.

Design command sets are:

“New / Design / By Built in LC Prototype / Lumped , BP, Chebyshev /  RS=RL=50 / PB Specs + Filter Degree: PB Ripple=0.01, fp1 =95,  fp2=105, Prototype Degree N=5 / Find circuit: pLCsh+Lsr , …”

In the list of solutions, the options highlighted in Figure 6.5 belong to the class of direct coupled resonator filters. Results of designs are presented in the following subsections.

Figure 6.5. List of direct coupled resonator type filters as options of Built in LC Prototype approach.

a) Shunt inductor coupled series resonator and series capacitor coupled shunt resonator filters

Inductively coupled series resonator filters (sLCsr+Lsh, Type 6) and capacitively coupled shunt resonator filters (pLCsh+Csr, Type-7) are duals of each other. The circuits and element values are shown in Figure 6.6.a. A typical response is shown in Figure 6.6.c. It is seen that low side skirt slope is much higher than upper side. This is because, as will be shown in Synthesis approach, in these filters there are only one transmission zero at f= while there are N-1 transmission zeros at f=0.

Figure 6.6. Element values and responses for shunt inductor coupled series resonator filters (Type-6), series capacitor coupled shunt resonator filters (Type-7), shunt capacitor coupled series resonator filters (Type-8) and series inductor coupled shunt resonator filters (Type-9)..

b) Capacitively coupled series resonator and inductively coupled shunt resonator filters

Capacitively coupled series resonator filters (sLCsr+Csh, Type 8) and inductively coupled parallel resonator (pLCsh+Lsr, Type 9) filters are duals of each other. Element values of these two filters are shown in Figure 6.5. A typical insertion loss plot (Figure 6.6) shows that these filters have high upper stopband skirt slope compared to lower stopband slope. This is because there are only one transmission zero at f=0 while there are N-1 transmission zeros at f=.

c) Coupled resonator filters with mixed couplings

In order to improve skirt symmetry, mixed types of couplings can be used between resonators of the filters. In terms of transmission zero terminology, this corresponds to having equal or nearly equal number of transmission zeros at f=0 and f=. The filters Type 10-to-Type-13 belong to this class. Element values and a typical response are shown in Figure 6.7.

Return loss responses are distorted heavily in all these cases because of gross approximations in the formulations. For wideband filters this approach may even give negative element values, leading to erroneous results. Therefore for wideband cases Synthesis approach should be preferred which gives exact results always.

Figure 6.7. Element values of direct coupled resonator filters with mixed types of couplings                      and a typical response.

6. 3. 2. SYNTHESIS APPROACH FOR DIRECT COUPLED RESONATOR FILTERS

Inspection of the direct coupled resonator filter circuits discloses the following facts on the transmission zeros of these filters:

- The transmission zeros can be shared between f=0 and f= in different ways to produce different type of couplings, leading to different skirt slopes in lower and upper stopband.

- The number of transmission zeros at f=0 affect the lower stopband skirt slope more while the transmission zeros at f= affect the upper stopband skirt slope more.

- If there are NR resonators, then degree of the filter will be N=2*NR. That is each resonator contributes two degrees.

- It will be seen that direct coupled resonator topologies that described above can only be obtained if Nzero=Odd Integer. This means such filters have electrical symmetry always. Physical symmetry is possible if it is possible to extract transmission zeros in a symmetric manner. For the cases where Nzero=Even Integer, filter comes out to be antimetric and it is not possible to convert all elements into resonators.

Figure 6.8. Qualitative description of effects of finite transmission zeros.

In order to clarify the relation between behaviour of elements and the number of transmission zeros, consider the inductively coupled resonator filter with five resonators studied in the previous section, shown in Figure 6.8.a. It has a single transmission zero at f=0 and N-1=9 transmission zeros at f=. The single transmission zero of f=0 is recognized by setting f=0 which makes the capacitors open circuit (OC) and inductors short circuit (SC), as shown in Figure 6.8.b. The capacitors in shunt arms become ineffective when they are open circuited. The series arm short circuits cause the shunt arm short circuits to be in parallel in effect. Thus, the circuit simplifies to a single effective short circuit in shunt arm, as shown in Figure 6.8.b, on right. This shunt SC provides the single transmission zero of the filter at f=0, helping to stop low frequency signals (attenuate signals by reflecting them back to the source). This means that all the shunt inductors are parts of the same single transmission zero. That is, we have a redundancy of shunt inductors in the circuit. Instead of so many shunt inductors, we could as well place a single equivalent shunt inductor. The reason why we use so many redundant shunt inductors will be clear when we inspect element values of the circuits resulting from synthesis approach. At f=, the capacitors become short circuit while inductors become open circuit, as shown in Figure 6.8.c. The shunt SC and series OC branches help stopping high frequency signal flow by placing two different kinds of barriers: Alternating SC and OC type reactances in cascade. Alternation of SC and OC branches increases effectiveness of high frequency signal attenuation a lot compared to the same type of cascaded barriers. Effectively, we say that there are N-1 transmission zeros at f= (Figure 6.8.c).

Design / By Synthesis approach will be our main design tool for all coupled resonator filters, because it always produces exact solutions. The design command sets and circuit transformations are also the same for all types of direct coupled resonator filters. Let’s study design of different types of coupled resonator filters and related circuit transformations on some examples.

6. 3. 3. SERIES INDUCTOR COUPLED SHUNT RESONATOR FILTERS

Series inductively coupled shunt resonator filters and shunt capacitor coupled series resonator filters have one transmission zero at f=0 (Nzero=1) and N-1 transmission zeros at f= (Ninf=N-1). Because of the larger number of transmission zeros at f=, upper skirt slope is higher. Since Nzero is odd, these filters are electrically symmetric and hence, they can always be synthesized or converted into physically symmetric forms. Let’s see further properties and details on some wideband and narrowband examples.

EXAMPLE 6. 3: A wideband inductively coupled shunt resonator filter

Let’s consider a degree 10 wideband filter with following specifications:

PBRipple=0.01 dB, fp1=50 MHz, fp2=150 MHz, Nzero=1, Ninf=N-1=9.

- Design command set is shown in Figure 6.9. In Filter Type menu, Lumped / BP / Doubly Terminated / Type B (Type A) options are selected. Type B option is needed to get the circuit starting with shunt elements that will lead to a shunt resonator type filter. If Type A option was selected, then we would get the dual form which will lead to series resonator type filter. Since most of the circuit transformations are developed for shunt resonator type filters we usually prefer to start the design of coupled resonator filters as Type B. Then, if series resonator versions are needed we find the dual form of the circuit.

- PB and Terminations menu is filled in by typing in the passband ripple and passband edge frequencies while leaving the terminations as 50 ohms.

- At Specify Transmission Zeros stage, first we Delete any transmission zeros left from a previous design. Then the new transmission zeros are entered by pressing Zero key once and Infinity key nine times, leading to Nzero=1, Ninf =9. Build up of insertion and return loss responses while transmission zeros are entered can be seen from the inset plot. Inset plot scales can be adjusted by using Plot Response command which will display the response on the whole screen for closer inspection and scaling of frequency and amplitude axes.

- Next, Extract Transmission Zeros command is invoked and the transmission zeros are extracted by pressing Infinity and Zero keys until all transmission zeros are finished. Transmission zeros can be extracted in any order. However whenever possible it is advisable to extract a symmetric topology. If exact symmetry is not possible then we try a topology close to symmetry. Symmetry is important because for high order filters extraction of transmission zeros in an arbitrary order may lead to numerical accuracy problems that may yield erroneous element values.  For example in this filter we can form a symmetric circuit by extracting the transmission zeros in the order, Inf-Inf-Inf-Inf-Zero- Inf-Inf-Inf-Inf-Inf  to get the circuit shown in Figure 6.9.a. We could as well select Standard Auto Extract in which case Filpro will extract the transmission zeros in a built in order. {This mode of transmission zero extraction is possible if Standard Auto Extract command is enabled.}

- When extracting the transmission zeros in manual mode by pressing Zero and Infinity keys, the extracted transmission zeros appear as circuit elements on the screen, one by one. When the transmission zeros are finished the relevant key is disabled automatically. Finally Quit Synthesis key is pressed to finish with Synthesis stage and enable the circuit transformations menu.

- When transmission zero extraction stage is completed, usually an ideal transformer appears between the last extracted element and load (Figure 6.9.a). This transformer accounts for the difference between the specified load resistance RL and the actual load resistance that the synthesized circuit wants to have the specified response. This transformer comes out unity (or very close to unity) when the filter is electrically symmetric (odd number of transmission zeros at f=0) and if a physically symmetric realization is carried out by extracting the elements in a symmetric manner. In all other cases a nonunity transformer will result and it should be eliminated during the circuit transformation stages. Since Nzero=1 implies that the filter has electrical symmetry, if we extract the transmission zeros in a symmetric fashion, then the transformer at load end will be close to unity. Any deviation from unity will be due to numerical accuracy problems during element extraction stages. The numerical accuracy problems become severe when the order of filter gets larger. In such cases, it may worth to try different orders for the extraction of transmission zeros.

- The circuit is saved as shntrs1.fil. The single transmission zero at f=0 appears as a single shunt inductor in the middle part of the circuit. That is, no redundancy is allowed in elements in synthesis stage leading to a canonical structure (minimum number of elements).

- Element values seem to be realizable. However just as exercise to see the other possibilities and problems let’s convert this structure into coupled shunt LC resonator forms with different numbers of resonators.

- If we apply Simplify Circuit command we get the circuit of Figure 6.9.b where the shunt L and C elements are combined to form a single LC element. We can treat this filter as a Single Shunt Resonator version.

- By applying Find Dual command we get the version with Single Series Resonator (Figure 6.9.c).

- The number of shunt LC resonators can be increased by sharing the single shunt inductor in the middle to the other shunt elements. For this purpose we load the circuit of shntrs1.fil (Figure 6.9.d).

- In this circuit the shunt inductor in the middle of the circuit and the series inductor on its left form an LLeft-section of inductors. With cursor under the shunt inductor of the LLeft-section the command Pi-Tee-L / LLeft-to-Symmetric Pi is applied. This command converts the LLeft-section into a Pi-section of inductors, thus placing a shunt inductor in parallel with the shunt capacitor on left (Figure 6.9.e). Thus, we now have two shunt LC resonators. The resulting transformer is shifted rightmost.

- In order to have a symmetric circuit we can form an LRight-section of inductors by applying Exchange command on the shunt inductor in the middle (Figure 6.9.f). Then the command Pi-Tee-L / LRight-to-Sym Pi is applied to convert the LRight-section into symmetric Pi-section, thus placing a shunt inductor in parallel with the shunt capacitor on right (Figure 6.9.g).

- So we have now three shunt LC resonators. The inductors of these resonators can be equated by placing the cursor under the series elements of the Pi-sections and applying the command Pi-Tee-L / Pi-to-Sym Pi/Equal Shunt Inductors iteratively. After a few rounds of iterations the shunt inductors will be almost equal. Thus, we get three shunt resonators version with equal shunt inductors (Figure 6.9.h). We note that by equating the shunt inductances they are increased from 69 nH to about 274 nH.

- Applying the command Find Dual we get the three series resonator version with equal capacitors (Figure 6.9.i).

- The version with five shunt resonators can be obtained by loading shntres01.fil and applying the command Pi-Tee-L / Equal Shunt Inductors.  This command applies the LRight (LLeft)-to-Sym Pi command iteratively in the background until all shunt inductors are equated (Figure 6.9.k-m).

- We may attempt to get equal shunt capacitor version by applying the command Pi-Tee-L / Equal Shunt Capacitors. The algorithm works but we get negative elements (Figure 6.9.n). This is because the filter is a wideband one and we have only one transmission zero at f=0. If we had more transmission zeros at f=0, we would possibly be successful in forming an equal shunt capacitors version (Exercise?).

- Figure 6.9.p shows the five series resonator version with equal series capacitors found by taking the dual of the five shunt resonator version with equal shunt inductors shown in Figure 6.9.m.

The commands Pi-Tee-L / LRight-to-Sym Pi, LLeft-to-Sym Pi and Pi-to-Sym Pi can also be applied on Capacitive-L and Pi sections. Therefore they are extremely useful for obtaining equivalent alternative filter circuits and for adjusting element values. It should be noted that these commands are increasing the number of elements of the circuit. This is the price we pay for the advantages we get.

In the next example the techniques for narrowband filter design will be described.

Figure 6.9. A wideband filter: Versions with different numbers of resonators.

Figure 6.9. A wideband filter: Versions with different numbers of resonators (cont’d).

EXAMPLE 6. 4: A narrowband inductively coupled shunt resonator filter

Filter to be designed has the specifications PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Number of resonators is NR=5. Hence, N=2*NR=10 and hence Nzero=1, Ninf=N-1=9.

- Design command set is shown in Figure 6.10. In Filter Type menu, Lumped / BP / Doubly Terminated / Type B options are selected. Type B option is needed to get the circuit starting with shunt elements that will lead to a shunt resonator type filter.

- PB and Terminations menu is filled in by typing in the passband ripple and passband edge frequencies while leaving the terminations as 50 ohms.

- At Specify Transmission Zeros stage, the transmission zeros are entered by pressing Zero key once and Infinity key nine times, leading to Nzero=1, Ninf =9.

- Next, Extract Transmission Zeros / Standard Auto Extract command is invoked giving the circuit of Figure 6.10.a.

- The single transmission zero at f=0 appears as a single shunt inductor in the middle part of the circuit with an extremely small value. Also capacitor values change in an extremely wide range. These problems can readily be solved by distributing the single shunt inductor into the circuit by placing a piece of inductor in parallel with each shunt capacitor. This can be done by applying the command  Pi-Tee-L / Equal Shunt Inductors (or Capacitors). Applying this command with Equal Shunt Inductors option, after some automatic iterations, we get the circuit of Figure 6.10.b. Iterations may take longer time for high degree filters, depending also on bandwidth of the filter. For very high degree cases or when the transmission zero extraction order yields extreme element values, automatic iterations may have difficulty in converging to the solution. In such cases it may be helpful to change the transmission zero extraction order. If this approach doesn’t work, then a few steps of Pi-Tee-L / LLeft-to-Pi or LRight-to-Pi transformations to can be carried out manually. In these transformations the aim should be to bring the abnormal transformer value at load end to a manageable level. Then the command Pi-Tee-L / Equal Shunt Inductors (Capacitors) command should be applied. In the circuit of Figure 6.10.b, all shunt inductors are equated at 11.6 nH while capacitors vary in the range of 240-258 pF. So we have got the targeted structure:  Series inductor coupled shunt resonator filter. We save the circuit as, for example, shntrs11.fil to use at later stages also.

- By applying the command Pi-Tee-L / Equal Shunt Capacitors we get the equal shunt capacitor version of the inductor coupled shunt resonator filter (Figure 6.10.c). It is saved as shntrs12.fil.

- Now we are also ready to find the series resonator version. Applying Find Dual command to shntrs12.fil, we get the shunt capacitor coupled series resonator filter with equal series inductors (Figure 6.10.d). If we had applied the same command to the circuit of shntrs11.fil then we would get the equal series capacitor version. This class of filters will be studied later in this chapter. It is included here just to show the link between the two types of filters.

- Obtaining a circuit with equal shunt inductors or equal shunt capacitors or any other solution with less dynamic range for element values does not mean necessarily that the elements are realizable. At this stage we have only solved the problem of dynamic range of the element values. Especially in narrowband filters the resulting element values may still be unrealizable with the technology used. Further scaling is needed for the element values. This can be done for shunt and series resonator filters as follows.

Impedance Scaling for Series Inductor Coupled Shunt Resonator Filters:

- Let’s assume that we want all shunt inductors of the shunt resonator filter to be 50 nH. We Load the circuit of shntrs11.fil (Figure 6.10.e).

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Adjacent Inductor (or capacitor) / L=50 nH is applied. This command scales the circuit by inserting ideal inverters at source and load ends. Impedances of inverters is such that the inductance adjacent to the source will be scaled to 50 nH (Figures 6.10.f). We could as well specify the capacitor adjacent to the source or we could specify the inverter constant itself.

- Next, the ideal inverters will be replaced by their proper practical equivalents. In Filpro, it is possible to replace such End Inverters by either capacitive or inductive Pi or L-sections. Figure 6.10.f-g shows replacement of end inverters by Capacitive-L-sections. The shunt legs of the L-sections are negative. They must be absorbed into the adjacent positive capacitor. This can be done by applying Simplify Circuit command (or Combine command). That is the positive capacitor must be greater in magnitude than the negative capacitor. This fact forms a limitation of the approach. Fortunately this approach works for almost all such applications. This replacement is an approximation which is correct at one frequency selected by the user (usually band center). Therefore there will be some degradation in return loss depending on bandwidth of the filter. Such degradations can be corrected by tuning some of the elements. The resulting circuit is saved as shntrs15.fil (Figure 6.10.h).

Displaying External Q, Coupling Coefficients and Resonance Frequencies:

- In some applications Coupling coefficients and Resonance Frequencies of the coupled resonators and External Q of the circuit may be useful for implementation and tuning (See Ch-12 for the definitions and use of Coupling Coefficients and ExternalQ). External Q and Coupling Coefficients can be displayed by pressing the keys Ctrl+F6 and selecting the option Direct Coupled, as shown in Figure 6.10.h. The individual Coupling Coefficients may also be displayed together with the Resonance Frequencies by placing the cursor under the series (coupling) element and pressing the keys Alt+F6.

Parasitics:

- Effects of parasitics and losses of elements can be studied in general by splitting the shunt LC resonators into L and C components and applying the command Parasitics / Lossless-to-Lossy on each element one by one (by placing the cursor under each element). If the LC resonators are handled as single elements with the same specified Q values, then the command Parasitics / All Lossy Resonators / f0=100 MHz, Qres=100 can be applied (Figure 6.10.i-k). When the series elements are also converted into lossy models we get the circuit of Figure 6.10.m. Insertion loss responses of lossless, only lossy shunt resonators and all lossy elements cases are shown in Figure 6.10.m for Qres=100 and Q=100 for single inductors and capacitors.

Displaying Voltage-Current-Power:

- Voltage-Current-Real Power variables at junctions of elements at a single frequency can be displayed in Magnitude-Angle or Real-Imaginary format, by pressing the keys Alt+F8. In Figure 6.10.n, these quantities are displayed for a voltage source of 100 volts, at f=105 MHz and f=106 MHz for the lossy filter. These tables can be used to search for the maximum voltage points in the circuit to avoid any breakdown phenomena or to inspect the power loss in the circuit or to search the maximum current to avoid too much heating at a branch. Voltage-Current-Power quantities are defined at the junctions of two cascaded elements by using ABCD Matrix notation. That is the currents are always the series arm current leaving one element and entering the next element on its right. For example at 105 MHz the real power at the right of RS and at the right of the last capacitor on right (left of RL). Real power at source end is about 50 watts (available power of the source) while the real power at the load is reduced to about 18 watts because of the losses in elements.

Exercise: Display Voltage-Current-Power list for the lossless case at one frequency and compare all quantities with the lossy case node-by-node.

Exercise: Display Voltage-Current-Power list for several frequencies in lower stopband near the lower passband edge and at several frequencies in upper stopband near the upper passband edge. Inspect the maximum voltages. Find the frequency at which maximum voltage occurs. You will notice that voltage maximum occurs always at a frequency close to passband edge where the slope is maximum (sharper edge).

Monte Carlo Optimization:

- Replacement of ideal inverters by practical circuit sections degrade response to some extent. If the filter degree is small then the degradation can be corrected by manual tuning of a few elements. For higher degree filters manual tuning may be tedious. In such cases the Monte Carlo optimization routine using Best Case command can be applied as described in Figures 6.9.p-q-r:

- The circuit of shntrs15.fil is loaded (Figure 6.10.p).

- In Monte Carlo routine, all or some of the parameters can be varied within ranges specified by the user. First Monte Carlo List is prepared by placing the cursor under the elements to be included in the list and applying the command Monte Carlo / Add a Parameter. In applying this command we point the parameter of the element to be included in the list and press Enter. Filpro will ask the Percentage Deviation to be applied on this parameter and also Uniform or Gaussian Distribution option is selected for random variations. If all parameters of all elements are to be included in the list then we can use the short cut command Monte Carlo / Automatic Operations / Add of All of All Parameters.

- Then we place a Reference Line on Return Loss response only by applying the command Reference Line / Auto Add Reference Line / Return Loss Only: fp1=95 MHz, fp2=105 MHz, Ref, Value=22 dB. Here instead of 26 dB return loss (corresponding to 0.01 dB passband ripple) we assign a lower level to ease the job of optimization routine.

- Next the command Best Case: f1=90 MHz, f2=110 MHz is applied. We assign lower and upper frequency limits close to passband edges for easier monitoring of the variations in response during optimization. Since the circuit is symmetric we select Preserve Symmetry option. That is the symmetric elements will be changed simultaneously during the optimization. With this command the parameters in the Monte Carlo List are changed in a random manner within the assigned limits and the resulting Return Loss responses are overplot. From these responses we can get an idea on the effectiveness of the percentage variations in element values. If any one response which is not touching the shaded (forbidden) region is encountered then Filpro displays the comment that it had found a qualifying circuit and asks if to continue by increasing the reference level and/or percentage deviation of element values. We may stop optimization process or increase the reference level and decreasing the tolerance by a small amount and continue the search. The number of iterations is set by the user. If a solution can’t be found after one round of iteration process then Filpro stops and asks if to reduce the reference level to ease the job of optimization routine. We may reduce the reference level and change percentage tolerance and me may even change the min-max frequencies of the shaded region just to catch a solution to use as a starting platform to continue. This simple Return Loss based routine works very well for all filters and multiplexers designed on Filpro. The approach is successful because we ae usually not very far from the optimum solution.

- Figure 6.10.q shows the optimized circuit while Figure 6.10.r shows return loss response before and after optimization.

Exercise: Display Voltage-Current-Power list for the lossless case at one frequency and compare all quantities with the lossy case node-by-node.

Exercise: Display Voltage-Current-Power list for several frequencies in lower stopband near the lower passband edge and at several frequencies in upper stopband near the upper passband edge. Inspect the maximum voltages. Find the frequency at which maximum voltage occurs. You will notice that voltage maximum occurs always at a frequency close to passband edge where the slope is maximum (sharper edge).

Exercise: Repeat the design by replacing end inverters with Capacitive Pi, Inductive Pi and L  sections.

Exercise: Repeat the design for wider bandwidth cases.

Figure 6.10. Synthesis of inductively coupled shunt resonator filters.

Figure 6.10. Synthesis of inductively coupled shunt resonator filters (cont’d).

Figure 6.10. Synthesis of inductively coupled shunt resonator filters (cont’d).

Figure 6.10. Synthesis of inductively coupled shunt resonator filters (cont’d).

EXAMPLE 6. 5.  Distributed element realization of shunt resonators

At high enough frequencies where wavelength becomes comparable to the physical size of elements it may be advantageous to treat some elements as distributed elements. Although distributed element filters will be studied in the succeeding chapters (see Chapters-9, 10, 11, 12, 13), just as a demonstration of some links between lumped and distributed elements, let’s see realization of shunt resonators in distributed element form. Filter specifications are:

PBripple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, Nzero=1, Ninf=9

- Filter is designed using the same steps as in the previous examples. Specifications and response are shown in Figure 6.11.a.

- Standart Auto Extract mode is used for element extraction and the command Pi-Tee-L / Equal Shunt Inductors is applied to yield the coupled resonator form shown in Figure 6.11.b.

- Shunt inductors are scaled to 10 nH using the command Inverters / Scale Circuit By Inserting Inverters / Specify Adjacent Inductor: L=10 nH (Figure 6.11.c).

- Then inverters are replaced by their capacitive L-section equivalents giving the all lumped element form of the filter (Figure 6.11.d).

- At such high frequencies it is easier to realize shunt resonators by short circuited stubs paralleled by lumped capacitors, like for example coaxial line type Scstubs or . The characteristic impedance of the equivalent shunt SC stubs can be found by placing the cursor under the shunt LC resonators and applying the command Replace By Stub/ Parallel LC-to-SC Stub+Cap: Lng=… mm, f1=795 MHz, f2=805 MHz. In this command the quantity Lng is the length of SC stubs. It sets the frequency fq where the stub length will be a quarter wavelength. The default value is twice the passband center. That is, fq=2f0=1600 MHz. Upon application of the command the LC resonator is replaced by parallel combination of a SC Stub and a lumped capacitor. That is in this conversion the lumped capacitor is also modified. This is necessary to keep the passband edges unaffected by the transformations. Impedance of the SC stub is calculated by Filpro automatically to give the same effective inductive reactance as the original inductor. Since the quarter wavelength  frequency (length) is set by the user, it is clear that infinitely many solutions exist for stub lengths, hence for stub impedances. After setting fq, if the resulting SC stub impedance is not proper for realization, then we have to go back and adjust the shunt inductors again by inserting new ideal inverters at the ports. This procedure should be iterated until the desired length and impedance is obtained. A shorter way can be seen in Chapter-11 using distributed element approach. In Figure 6.11.e we got 60.7 ohms impedances for the stubs. It should be noted that the associated lumped capacitors are changed appreciably.

- The coupling between these shunt resonators are provided by lumped inductors. In microstrip, stripline, slabline and evanescent waveguide type structures, it is possible to obtain magnetic couplings between adjacent resonators by placing them in closely controlled proximity of each other. In such cases it may also be possible to simulate the couplings by series arm SC stubs. On Filpro this conversion of series lumped inductors into series SC stubs can be carried out by placing the cursor under the series inductors and applying the command Replace By Stub/ SC Stub: f0=800 MHz, fq=1600 MHz (Figure 6.11.f). It is advisable to set fq to be the same as the fq that would result from the length (Lng) specified for the shunt SC stubs in the previous stage. This is easier by setting the length of series SC stub to be equal to that of shunt SC stub. In this equivalence, only passband center of the filter and the quarter wavelength frequency fq are set by the user. Therefore in medium / wideband applications shifts may be observed in passband edges together with some distortions in return loss. These shifts can be corrected by tuning the element values slightly. Figure 6.11.g shows insertion loss and return loss. Since the filter is a narrowband one, no appreciable distortions are observed in responses.

- The quarter wavelength frequency of series arm SC stubs may be less than the shunt SC stubs by introduction of grounded separations between the shunt SC stubs (See Chapter-11).

- Magnetic coupling between resonators may also be provided by opening holes (irises) in the walls separating adjacent resonators (See Chapter-11).

Exercise 6. 5: We could as well replace the shunt LC resonators by single SC stubs using the command Replace By Stub / Parallel LC-to- SC stub. This would avoid using lumped capacitors in parallel with SC stubs. Exercise with this command and see the problematic points (different lengths and/or different impedances for SC stubs!).

Figure 6.11. Development of mixed lumped and distributed element shunt resonator filter.

Figure 6.11. Development of mixed lumped and distributed element shunt resonator filter.

EXAMPLE 6. 6.  Prototype with Inverters approach

For narrow-medium bandwidth filters Prototype with Inverters approach is a quick and simple design tool. It has a slight advantage that it always yields symmetric filters, for both odd and even degree prototypes.  Let’s consider the narrowband filter with specifications

PBRipple =0.01 dB, RS=RL=50 Ohms, prototype degree N=5 with fp1=95 MHz, fp2=105 MHz

Design steps are shown in Figure 6.12.

- Design / By Prototype with Inverters commands are invoked. Choose a Standard Cct command offers two options. The option Series Inductors+Inverters leads to a series resonator filter. We select the option Shunt Capacitors+Inverters to get a shunt resonator filter.

- Prototype Degree and Passband Ripple are typed in. The lowpass prototype filter will be displayed (Figure 6.12.a). If necessary we can edit and modify this circuit. However in the usual approach we directly pass to the command Filter Type and select the options Lumped / BPF.

- PB and Terminations command is invoked and filter passband edge frequencies are typed in.

- Find Circuit command displays the targeted inverter coupled shunt resonator bandpass filter (Figure 6.12.b). If necessary for later use, we may also save the LP prototype circuit.

- The element values can be modified by using several commands in the Inverters menu, like Scale Inverter, Scale Element By Inverter, etc. If identical resonators will be used then with cursor under any inverter the command Inverters / All Equal Left-Right / Equal Inductors is applied. This command equates all inductors and all capacitors (Figure 6.12.c).

- Next, the coupling inverters will be replaced by suitable practical approximate equivalents. If inductive couplings are desired between resonators then with cursor under the inverters, the command Inverters / Replace Inverter By / Pi-Tee Type / Inductive Pi: f1=95 MHz, f2=105 MHz is applied. This command calculates the inductors of the equivalent Pi-section such that the passband edges will be preserved after the approximations. The Pi-sections have negative shunt inductors (Figure 6.12.d). The negative inductors are absorbed by the adjacent LC resonators by applying Simplify Circuit command (Figure 6.12.e). Thus, the targeted series inductor coupled shunt resonator filter is obtained.

- Next the elements will be scaled to desired levels. For example let’s target to make all shunt inductors 50 nH. This is done by applying the command Inverters / Scale Circuit By Inserting Inverters / Specify Adjacent Inductor: L=50 nH. This command asks the desired value of either the inverter or value of the element adjacent to the source and inserts inverters at source and load ends. The resulting circuit is shown in Figure 6.12.f. The inductors adjacent to source and load are 50 nH, but the inner inductors are different because of the replacement of the inverters by Pi-sections in the previous step.

- Next the end inverters will be replaced by practical equivalents. For example let’s make input-output couplings series capacitors. This is done by applying the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L: f0=100 MHz. The end inverters are replaced by Capacitive-L sections with negative shunt inductors which are absorbed into the adjacent positive shunt inductors after applying Simplify Circuit command (Figure 6.12.g-h).

- If necessary all the shunt inductors can be equated by applying the command Pi-Tee-L / Equal Shunt Inductors (Figure 6.12.i). They are equated at a level slightly higher than 50 nH.

- Figure 6.12.k shows the response. Return loss is degraded slightly because of replacement of ideal inverters by practical equivalents. It can be recovered to a great extent by tuning all elements slightly (Exercise).

Exercise: Repeat the design by replacing the coupling inverters with their Capacitive-Pi equivalents to get a Capacitively Coupled Shunt Resonator filter.

Exercise: Repeat the design by replacing some of the coupling inverters with their Capacitive-Pi equivalents and others by Inductive-Pi equivalents to get Shunt Resonator Filters with Mixed Couplings.

Exercise: Repeat the design for wider bandwidth filters and inspect the limits of validity of the approach.

Exercise: Repeat the design by selecting the option Series Inductors and Inverters to form an inverter coupled series resonator filter. Then replace inverters by their Capacitive-Tee and/or Inductive-Tee equivalents to get series resonator filters with shunt capacitive or shunt inductive or mixed couplings.

Figure 6.12. Prototype with Inverters approach for designing coupled resonator filters.

Figure 6.12. Prototype with Inverters approach for designing coupled resonator filters (cont’d).

6. 3. 4. SHUNT CAPACITOR COUPLED SERIES RESONATOR FILTERS

These types of filters are formed by series LC resonators in series arms (sLCsr) coupled by shunt capacitors. These filters are duals of inductively coupled shunt resonator filters of the previous section. That is they have one transmission zero at f=0 (Nzero=1) and  N-1 transmission zeros at f= (Ninf=N-1). Leading to lower skirt slope in lower stopband, higher skirt slope in upper stopband. Since Nzero=1 is odd, they can always be realized in physically symmetric form. In order to make use of the circuit transformations developed for the shunt resonator filters, all the design and circuit transformation stages can be carried out in inductively coupled shunt resonator forms and then the circuit can be converted into capacitively coupled series resonator forms using Find Dual command as described in Example 6.4 and repeated in the following example. A special but important version of this filter is named as Dumbbell filter. It will be studied in a special section later in this chapter.

EXAMPLE 6. 7: Synthesis of shunt capacitor coupled series resonator filters

Filter to be designed has the specifications:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=N-1=9.

- Synthesis  approach is applied in exactly the same way as in Example 6.4, leading to the circuit shown in Figure 6.13.a.From this stage on, if we apply the command Pi-Tee-L / All Equal Shunt Inductors, we will get the equal shunt inductors in the shunt resonator version of the filter (Figure 6.13.b). However this circuit will correspond to Equal Series Capacitor version of the dual (series resonator) circuit. Usually equal series inductor version is preferred. It can be obtained by first forming the equal shunt capacitor version of shunt resonator form as shown in Figure 6.13.b-c, and then finding the dual circuit (Figure 6.13.d). This circuit is saved as serrs12.fil. From now on we will use only this circuit.

- Element value adjustments is carried out by inserting inverters in the same way as in the previous examples. For example all the series inductors can be made 50 nH by applying the command  Inverters / Scale Cct By Inserting Inverters / Specify Adjacent Inductor: L=50 nH  (Figure 6.13.e-f).  We could as well specify the capacitor adjacent to the source.

- Next the inverters are replaced by proper practical equivalents.  For example they can be replaced by Capacitive L-sections by applying the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L (Figure 6.13.g). It should be noted that contrary to the shunt resonator filters of the previous examples the series elements of L-sections come out to be negative. The negative series capacitor is absorbed into the adjacent positive capacitor by applying Simplify Circuit command (Figure 6.13.h). The resulting circuit is saved as serrs13.fil.

- Further element value adjustments are possible by using Pi-Tee-L transformations that can be applied on the L-sections of capacitors formed by adjacent series and shunt capacitors. For example let’s adjust all the internal inductors to 100 nH. This is possible as follows:

- First, with cursor under any series resonator the command Split / All Two Element Resonators is applied to split the LC resonators into L and C components (Figure 6.13.h-i). The resulting circuit is saved as serrs14.fil to be used in other applications as well.

- Then with cursor under the shunt leg of the capacitive-LLeft section at Node-2 the command Pi-Tee-L / LLeft-to-Pi-Tee-L /LLeft-to-Tee/Specify TRF: n=0.707 is applied. Here the transformer turns ratio is calculated as the square root of the ratio of the old inductance (50 nH) to the new inductance (100 nH). The LLeft section becomes a TEE section together with a transformer. Shifting the transformer rightmost, all the series inductors on the right of Node-2 becomes 100 nH (Figure 6.13.k-m).

- Next we will produce an inverse transformer of turns ratio 1/0.707=1.4142 to eliminate the first transformer and also to make the inductor at load end 50 nH. For this purpose we form a Capacitive-LRight section at Node-5 (symmetric of Node-2) by using Exchange command (Figure 6.13.k-m). Then with cursor under the shunt leg of the LRight-section the command     Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-Tee / Specify TRF: n=1.4142 is applied (Figure 6.13.n). Simplify Circuit command eliminates the transformers giving the targeted circuit in which the outermost series inductors are 50 nH and inner ones are 100 nH It is saved as serrs15.fil.Figures 6.12.p-to-w describe various operations on the filter circuits, like displaying nodal V-I-P, effects of losses and displaying External Q and Coupling Coefficients (See Ch-12 for the definitions and use of Coupling Coefficients and ExternalQ). Note that External Q and Coupling Coefficients of serrs13.fil and serrs15.fil are identical. That is the internal element value scaling doesn’t effect these quantities.

Figure 6.13. Design of shunt capacitor coupled series resonator filters.

Figure 6.13. Design of shunt capacitor coupled series resonator filters (cont’d).

Figure 6.13. Design of shunt capacitor coupled series resonator filters (cont’d).

Figure 6.13. Design of shunt capacitor coupled series resonator filters (cont’d).

EXAMPLE 6. 8. Shunt capacitor coupled distributed element series resonators

In high frequency applications the series LC resonators of the filter described in the previous example may also be realized as transmission lines acting as Half Wavelength resonators. Let’s consider a filter with specifications

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, Nzero=1, Ninf=9.

- Filter is synthesized in shunt resonator form, as shown in Figure 6.14.a.

- The command Pi-Tee-L / Equal Shunt Capacitors is applied to get coupled shunt resonator form with equal shunt capacitors (Figure 6.14.b).

- Find Dual command is applied to get the series resonator form with equal series inductors (Figure 6.14.c).

- Circuit is scaled up to have equal shunt inductors of 20 nH by inserting inverters (Figure 6.14.d). Here, 20 nH is a trial value. It can be iterated until better element values are obtained.

- End inverters are replaced by their Capacitive L-section equivalents (Figure 6.14.e).

- With cursor under each resonator, the command Replace By TLine is applied to convert the resonators into half wavelength transmission line pieces. The resulting structure is a capacitively loaded transmission line filter (Figure 6.14.f-g).

- Return loss response degrades because of approximation of series resonators by half wavelength lines (Figure 6.14.h). Degradation decreases as the series inductors are decreased.

- Return loss response can be made equal ripple by using Monte Carlo optimization routine (Figure 6.14.i-k).

- The shunt capacitors may be replace by OC stubs as shown in Figures 6.13.m-n-p.

- Return loss shows that this transformation causes appreciable band shrinkage which should be taken into account during synthesis stage (Figure 6.14.q).

- Figure 6.14.r shows wide span insertion loss which displays many spurious passbands which limits the use of this structure.

Exercise: Repeat the design by starting from Prototype with Inverters approach.

Figure 6.14. Development of capacitively loaded half wavelength filter.

Figure 6.14. Development of capacitively loaded half wavelength filter.

Figure 6.14. Development of capacitively loaded half wavelength filter (cont’d).

6.  3. 5. SERIES CAPACITOR COUPLED SHUNT RESONATOR FILTERS AND SHUNT INDUCTOR COUPLED SERIES RESONATOR FILTERS

Capacitively coupled shunt resonator filters and shunt inductor coupled series resonator filters have a single transmission zero at f= and N-1 transmission zeros at f=0 (Nzero=N-1, Ninf=1). Because of larger numbers of transmission zeros at f=0, skirt slope is higher in lower stopband. Since physical symmetry requires odd numbers of transmission zeros at f=0, and since degree of bandpass filters is even, Nzero=N-1 is always odd. Therefore we can always get physically symmetric circuits. Design and circuit transformations are similar to the inductively coupled resonator case, as will be seen in the following example.

EXAMPLE 6. 9: Capacitively coupled shunt resonator filters and inductively coupled series resonator filter

Filter to be designed has the specifications

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Number of resonators, NR=5. Hence, N=2*NR=10,

Nzero=N-1=9, Ninf=1.

Synthesis  approach is applied as follows:

- Design command sets are shown in Figure 6.15. In Filter Type menu, Lumped / BP / Doubly Terminated / Type B options are selected. Type B option is needed to get the circuit starting with shunt elements.

- PB and Terminations menu is filled in by typing in the passband ripple and passband edge frequencies with the terminations being 50 ohms.

- At Specify Transmission zeros stage, first we delete any transmission zeros left from a previous design. Then the transmission zeros are specified by pressing Zero key nine times and Infinity key once leading to Nzero=9, Ninf =1. Build up of insertion and return loss responses while transmission zeros are entered can be seen from the inset plot. Inset plot scales can be adjusted by using Plot Response command which will display the response on the whole screen for closer inspection and scaling of frequency and amplitude axes.

- Extract Transmission Zeros command is invoked and the transmission zeros are extracted by pressing Infinity and Zero keys until all transmission zeros are extracted. Transmission zeros can be extracted in  any order, for example, Zero-Zero-Zero-Zero-Zero-Inf-Zero-Zero-Zero-Zero to get the circuit shown in Figure 6.15.a. {Standard Auto Extract mode is not operative for this transmission zero pattern.}

- Finally Quit Synthesis key is pressed to finish with Synthesis stage and enable the circuit transformations menu.

- Application of Pi-Tee-L / Equal Shunt Inductors will convert the shunt elements into coupled resonator form with equal inductors (Figure 6.15.b). It is saved as shntrs21.fil.

- By applying the command Pi-Tee-L / Equal Shunt Capacitors we can get the equal shunt capacitor version (Figure 6.15.c). It is saved as shntrs22.fil.

- Applying Find Dual command to shntrs22.fil we get the shunt inductively coupled series resonator version with equal series inductors (Figure 6.15.d).

- Scaling of element values can be done by inserting inverters at source and load ends as shown in Figures 6.14.e-to-k for shunt and series resonator filters.

Exercise: Display External Q’s and Coupling Coefficients by pressing the keys CTRL+F6 and Alt+F6.

Exercise: Inspect effects of losses and parasitics associated with each elements by applying Parasitics command.

Exercise: Display ABCD matrix node voltages, currents and powers at different frequencies in passband using Alt+F8 command.

Exercise: Repeat the design by starting from Prototype with Inverters approach.

Figure 6.15. Design of capacitively coupled shunt resonator and inductively coupled series             resonator filters.

Figure 6.15. Design of capacitively coupled shunt resonator and inductively coupled series             resonator filters.

EXAMPLE 6. 10. Series capacitor coupled distributed element shunt resonators

In high frequency applications the shunt LC resonators of the filter described in the previous example may also be realized as quarter wavelength SC stubs or shorter SC stubs loaded by capacitors. Let’s consider a filter with specifications

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, Nzero=9, Ninf=1.

- Filter is synthesized in shunt resonator form, as shown in Figure 6.16.a.

- The command Pi-Tee-L / Equal Shunt Inductors is applied to get coupled shunt resonator form (Figure 6.16.b).

- Circuit is scaled up to have shunt inductors of 10 nH by inserting inverters (Figure 6.16.d). Here, 10 nH is a trial value. It can be iterated until better element values are obtained.

- End inverters are replaced by their Capacitive L-section equivalents (Figure 6.16.e). The circuit is saved as distrs71.fil.

- With cursor under each resonator, the command Replace By Stub / SC Stub is applied to convert the resonators into (approximately) quarter wavelength SC stubs. The SC stubs are about quarter wavelength long at passband center and hence appear OC at their inputs, thus simulating shunt LC resonators. The SC stubs may be for example coaxial lines shorted at one ends. The coupling capacitors connect the center conductors of the adjacent coaxial resonators. (Figure 6.16.f-g).

- Instead of the quarter wavelength line SC stub resonators we may as well replace the shunt LC resonators by capacitively loaded SC stub resonators which can be made much shorter than the quarter wavelength stubs. This is carried out by placing the cursor under the LC resonators and applying the command Replace By Stub / Cap+SC Stub: Lng=.., f1=975 MHz, f2=1025 MHz (Figure 6.16.g-h). In this command we can set the length of stubs usually less than a quarter wavelength (at pasband center). The default value is half of the quarter wavelength (wavelength/8). The capacitors in parallel with the SC stubs make them shorter in length.

- Figure 6.16.i shows a pictorial description and Figure 6.16.k shows the return loss which can be tuned to be equal ripple.

Exercise: Repeat the design by starting from Prototype with Inverters approach.

Figure 6.16. Series capacitor coupled distributed element shunt resonator filters.

Figure 6.16. Series capacitor coupled distributed element shunt resonator filters.

EXAMPLE 6. 11.  A wideband series capacitor coupled shunt resonator filter as HIGHPASS filter

As the bandwidth gets wider care is needed in applying the transformations Pi-Tee-L / Equal Shunt Inductor because it may not be possible to equate the shunt inductors without allowing some capacitors to assume negative values. The negative capacitor values may lead the algorithm to converge at a wrong circuit. This point must be checked by plotting the response after applying such transformations. This is especially the case when we have only series capacitors as coupling elements. Actually the equal shunt inductor case may not be the optimum topology because, contrary to the narrowband filters, element values of wideband filters may not have extreme values. Therefore it may not be necessary to equate the shunt inductors. In such circuits it is easier to equate the shunt capacitors using either the command Pi-Tee-L / Equal Shunt Capacitors or try to equate or adjust the shunt inductors by using LLeft-to-Sym Pi, LRight-to-Sym Pi or Pi-to-Sym Pi transformations.

Filter to be designed has the specifications

PBRipple=0.01 dB, fp1=50 MHz, fp2=250 MHz, Number of resonators, NR=7. Hence, N=2*NR=14,

Nzero=1, Ninf=N-Nzero=13.

Such wideband bandpass filters with a single transmission zero at f= are actually used in place of  highpass filters. This is because the usual highpass filters with no transmission zero at f= are formed by ladder circuits with series capacitors and shunt inductors and the parasitic interwinding capacitors of the shunt resonators actually creates an uncontrolled transmission zero at f=. For this reason it may be wiser to place a controllable shunt capacitor in parallel with each shunt inductor as described below.

Design steps are the same as previous shunt filters:

- In Filter Type menu, Lumped / BP / Doubly Terminated / Type B options are selected. Type B option is needed to get the circuit starting with shunt elements.

- PB and Terminations menu is filled in by typing in the passband ripple and passband edge frequencies with the terminations being 50 ohms.

- The transmission zero at f= is extracted in the middle of the circuit to get a symmetric topology giving the circuit of Figure 6.17.a.

- The command Pi-Tee-L / Equal Shunt Capacitors is applied to convert the structure into coupled resonator forms with roughly equal shunt capacitors (Figures 6.16.b). If we had applied the command Pi-Tee-L / Equal Shunt Inductors we may get erroneous results depending on the bandwidth of the filter (Exercise?).

- Wide bandwidth slows down the convergence of the iterative algorithm. Therefore if exactly equal shunt capacitors are needed then we can apply the command Pi-Tee-L / Equal Shunt Capacitors a couple of times (Figure 6.17.c).

- Further adjustments of element values is possible to a limited extent. For example if we want to equate the first two shunt inductors on left by using the command Pi-Tee-L / Pi-to-Sym Pi / Equal Shunt Inductors  we get a negative capacitor as shown in Figure 6.17.d. It is possible to equate the inner shunt inductors using this command (Exercise). Further trimming of the element values is possible by using Pi-Tee-L / LLeft-to-Pi and LRight-to-Pi commands (Exercise).

- Find Dual command can be used to form the version with shunt inductor coupled series resonators.

Figure 6.17. A wideband series capacitor coupled shunt resonator filter.

6. 3. 6.  DIRECT COUPLED RESONATOR FILTERS WITH MIXED COUPLINGS

In between the two extreme types defined by the transmission zero patterns Nzero=1, Ninf=N-1 and Nzero=N-1, Ninf=1, we can share the N transmission zeros between f=0 and f= in any way to form a variety filters with mixed inductive and capacitive couplings. The number of transmission zeros to be placed at f=0 and f= can be set by the selectivity requirements on the upper and lower stopband skirts. If an electrically symmetric filter is desired, then Nzero should be selected as an odd number. If Nzero is an even number, then antimetric structure will result with a series arm element at one end. This element may be used for impedance matching requirements.

As in the previous examples the designs will be done for shunt resonator forms which may then be converted into series resonator forms.

EXAMPLE 6. 12.  A five resonator filter with Nzero=3 and Ninf=7

Filter to be designed has the specifications

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Number of resonators, NR=5. Hence, N=2*NR=10,

Nzero=3, Ninf=N-Nzero=7.

Design steps are shown in Figure 6.18:

- In Filter Type menu, Lumped / BP / Doubly Terminated / Type B options are selected. Type B option is needed to get the circuit starting with shunt elements.

- PB and Terminations menu is filled in by typing in the passband ripple and passband edge frequencies with the terminations being 50 ohms.

- The transmission zeros are extracted in such a way as no two series elements will be adjacent to avoid double element type coupling between resonators. One possible extraction order is shown in Figure 6.18.a.

- The command Pi-Tee-L / Equal Shunt Inductors is applied to convert the structure into coupled resonator forms (Figures 6.17.b). We get one capacitive and three inductive couplings. In order to have physical symmetry the number of inductive couplings should be even. For this reason the resulting structures can’t have physical symmetry in any extraction order. The transformers comes out to be different from unity. However in this case the series capacitor is near the center part. Therefore the transformer comes out to be very close to unity. In the cases where the series capacitive coupling is nearer to source or load the transformers would not be ignorable (Exercise?).

- Find Dual command gives the series resonator version with equal series capacitors.

- Equal shunt capacitor version can be obtained by applying the command Pi-Tee-L / Equal Shunt Capacitors command on the equal shunt inductor version (Figure 6.18.c). Its dual will be series resonator version with equal inductors (Figure 6.18.d).

- Element values may be scaled to desired levels by inserting inverters as was done in the previous examples (Exercise).

- Note that the series capacitor can be used to form Capacitive-LLeft or LRight sections. These L-sections can be used to adjust some of the element values or to adjust load impedance for impedance matching purposes by applying Pi-Tee-L commands (Exercise).

- For higher frequency applications the resonators may be replaced by their distributed version equivalents as was done in previous examples (Exercise).

Figure 6.18. Direct coupled resonator filters with mixed couplings.

EXAMPLE 6. 13.  A wideband  seven resonator filter with Nzero=5 and Ninf=9

For wideband filters care is needed in applying the transformations Pi-Tee-L / Equal Shunt Inductor or Equal Shunt Capacitor because it may not be possible to equate them without allowing some elements to assume negative values. The negative element values may lead the algorithm to converge at a wrong circuit. This point must be checked by plotting the response after applying such transformations. Actually the equal shunt inductors or equal capacitors cases may not be the optimum topology because, contrary to the narrowband filters, element values of wideband filters may not have extreme values. Therefore it may not be necessary to equate them. In such circuits it is easier to adjust element values using the commands Pi-Tee-L / LLeft-to-Sym Pi, LRight-to-Sym Pi or Pi-to-Sym Pi transformations.

Filter to be designed has the specifications

PBRipple=0.01 dB, fp1=50 MHz, fp2=250 MHz, NR=7 and Nzero=5, Ninf=2NR-Nzero=9.

Design steps are as follows (Figure 6.19):

- In Filter Type menu, Lumped / BP / Doubly Terminated / Type B options are selected. Type B option is needed to get the circuit starting with shunt elements.

- PB and Terminations menu is filled in by typing in the passband ripple and passband edge frequencies with the terminations being 50 ohms.

- The transmission zeros are extracted in an order to get a topology close to symmetric. One extraction order gives the circuit shown in Figure 6.19.a.

- The commands Pi-Tee-L / Equal Shunt Capacitors or Equal Shunt Inductors converge at wrong circuits (exercise!). In such cases it is wise first to try to form a symmetric circuit which will balance the element values while also eliminate the transformer. Inspection of the circuit shows that topological symmetry can be obtained if the third shunt capacitor on left can be distributed to the three shunt inductors in the middle of the circuit. This is possible by forming Capacitive-L sections using Exchange command and then converting the L-sections into symmetric Pi-sections by applying the command Pi-Tee-L / LRight-to-Sym Pi /Equal Shunt Capacitors, as described in Figure 6.19.a-to-e.

- In the circuit of Figure 6.19.e one capacitor is placed in parallel with the shunt inductors. The circuit  now has topological symmetry, but not symmetrical in element values yet. It can be made fully symmetric if the three shunt capacitors are made equal. This is possible by applying the command Pi-Tee-L / Pi-to-Sym Pi / Equal Shunt Capacitors to the Pi-sections with series capacitors and shunt LC resonators. This command have to be repeated until all the three shunt capacitors are equal.

- Since the shunt elements are not all resonators, this filter may not be treated as coupled resonator filter or may be treated as quasi-coupled resonator filter. If the number of shunt inductors is increased by applying Pi-Tee-L / LLeft (LRight)-to-Pi command to the L-sections formed by the inductors to place a shunt inductor in parallel with the remaining shunt capacitors then we would get a true coupled resonator filter (Exercise!).

- Find Dual command can be used to form the version with shunt inductor coupled series resonators.

Exercise: Use different extraction orders and try to form symmetric circuits.

Exercise: Repeat the design by starting from Prototype with Inverters approach.

Figure 6.19. A wideband degree 14 filter with mixed couplings.

EXAMPLE 6. 14. An antimetric filter

If even number of transmission zeros are placed at f=0, then an antimetric filter will result. Such filters are best suited for applications where RLRS. Just as exercise with Pi-Tee-L transformations, let’s consider a filter with specifications

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=4, Ninf=4.

- Filter is synthesized as shown in Figure 6.20.a. Because of the series element left at the load end, the command Pi-Tee-L / All Equal Shunt command can’t work properly for conversion into coupled resonator form. Therefore we will try the manual mode of this command which consists of a series of Norton and Pi-to- Pi commands.

- With cursor under the first three series elements on left, the command Norton / Specify Transformer: n=0.1 is applied. This command converts the series elements into Pi-sections together with a transformer specified by the user (Figure 6.20.b). One of the shunt legs of Pi-sections is always negative. The transformer turns ratio is arbitrarily assigned, after a few random trials, with two aims in mind:

- Elimination of the transformer at load end.

- Scaling of elements to reasonable values.

- Simplify Circuit command combines all transformers at load end. In the trial with n=0.1 turns ratio for transformers, we get a remaining transformer of turns ratio about 3.5 (Figure 6.20.c).

- The transformer at load end can now be eliminated by producing another transformer with inverse turns ratio. Placing the cursor under one of the series elements of the first two Pi sections on left and applying the command Pi-Tee-L / Pi-to-Pi-Tee-L / Pi-to-Pi / Specify Trf: n=1/3.5=0.2857, the necessary transformer is produced (Figure 6.20.d).

- Applying Simplify Circuit command we get the circuit shown in Figure 6.20.e. From this point on, using again Pi-Tee-L commands, the element values may further be trimmed.

- It should be noted that Norton transformation can’t be applied on the last two elements at load ends because there are no elements to absorb the resulting negative elements. Therefore not all shunt elements can be converted into resonators. We may try other transmission zero extraction orders (Exercise).

Figure 6.20. An antimetric filter to match 10 ohms to 50 ohms.

6. 4. FILTERS  WITH Nzero=Ninf  (Type-1 and Type-2 Filters)

The bandpass filters named as Type 1 and Type 2 in the tabulation of Built-in LC Prototype approach have equal number of transmission zeros at f=0 and f=. That is, the same circuits can also be obtained by assigning Nzero=Ninf in the Synthesis approach. One property of such filters is the geometric symmetry of response about passband which can be described as fp1*fp2=f02.

One of the advantages of such filters is the existence of ready capacitive and inductive L-sections providing great flexibility for circuit transformations through Pi-Tee-L and other Norton transformations for adjusting the element values.

EXAMPLE 6. 15: Type 1 and Type 2 filter design using Built in LC Prototype approach.

In Design / By Built in LC Prototype approach, selection of the option sLCsr+pLCsh displays the circuit shown in Figure 6.21.a, (termed as Type 1 filter) and selection of the option pLCsh+sLCsr yields the circuit of Figure 6.21.b (Type 2). They are duals of each other. Type 1 starts with series elements while Type 2 starts with shunt element. All series and shunt arms are made up of series resonators and parallel resonators respectively.

Element values for the three filters, 95-105 MHz (Filter-1), 80-120 MHz (Filter-2) and 50-150 MHz (Filter-3) with PBRipple=0.01 dB are shown under each element. It should be noted that for narrowband filters, series arm inductors and capacitors have too large and too small values respectively compared to the inductances and capacitances in shunt arms. That is the ratios of maximum and minimum inductances and capacitances gets larger as the bandwidth gets narrower. This is one of the main problems of narrowband filters to be treated by circuit transformations.

The structures has equal number of transmission zeros at f=0 and f=  (Nzero=5, Ninf =5 in this example). Insertion and return loss responses are shown in Figure 6.21.c. Responses has geometric symmetry around the center frequency of passband, defined as fp1fp2=f02. This symmetry is due to the equal numbers of transmission zeros at f=0 and f=. Note that upper skirt selectivity decreases a lot for wideband filters (Filter-3).

In Built in LC Prototype approach, for odd prototype degrees, the circuit comes out symmetric always because the number of transmission zeros at f=0 is odd while for even degrees it is antimetric always because the number of transmission zeros at f=0 is even. In Chebyshev filters, even degree filters require specification of RL to be different from RS. If RS is desired to be equal to RL, then circuit transformations can be used to make them equal.

The same circuits can also be designed using Synthesis approach as will be described in the next example where application of Pi-Tee-L transformations will also be described to get an all equal inductor filter.

Figure 6.21. Type-1 and Type 2 bandpass filters.

EXAMPLE 6. 16: Synthesis of Type 2 filter and an equal inductor filter

The specifications are:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=Ninf=5

- Design command set is shown in Figure 6.22. Transmission zeros are extracted in the order zero-inf-zero-inf-zero-inf-..  to give the circuit shown in Figure 6.22.a. This circuit is in the form of Type-2. Its dual will be of Type-1.

- Let’s target to form a filter with all inductors are 50 nH. First the command Inverters / Scale Circuit By Inserting Inverters / Specify Adjacent Inductor: L=50 nH is applied to make the inductors next to the source and load 50 nH (Figure 6.22.b).

- Now the internal inductors will be scaled to 50 nH by applying Pi-Tee-L transformations on L-sections of capacitors or inductors. Since these transformations increase the number of elements. And since we usually don’t want to increase the number of inductors they are applied on capacitive L-sections. In order to form L-sections we split the first series capacitor on left into two equal parts and then use Exchange command to move one piece to the other side of the adjacent series inductor (Figure 6.22.b-c).

- Similarly, the shunt capacitor in the middle of the circuit is split into two equal parts and one piece is moved to the left of the adjacent shunt inductor. Thus, in the left half of the circuit we have formed one LRight-section and one LLeft-section of capacitors as shown in Figure 6.22.c. The LRight-section will be used to scale down the 4992.4 nH series inductor to 50 nH while the LLeft-section will be used to scale the shunt inductor on its right to 50 nH.

- With cursor under the shunt leg of the LRight-section, the command Pi-Tee-L / LRight-to-Pi-Tee-L/ LRight-to-Pi / Specify TRF: n=9.92212 is applied. Here n=9.92212 is found as the square root of the ratio 4992.4/50. The LRight-section will become a Pi-section while also producing the necessary specified transformer (Figure 6.22.d).

- Shifting the transformer rightmost using the keys CTRL+W, we get the circuit of Figure 6.21.e where the series inductor on left is now 50 nH.

- Next the 0.2435 nH shunt inductor in the middle will be scaled to 50 nH. This is done by placing the cursor under the shunt leg of the LLeft-section, and applying the command Pi-Tee-L / LLeft-to-Pi-Tee-L/ LLeft-to-Pi / Specify TRF: n=0.069789 is applied. Here n=0.069789 is found as the square root of the ratio 0.2435/50. We get the circuit of Figure 6.22.f. The transformer is shifted rightmost scaling the inductor to 50 nH. In order to avoid repetition of these transformations in the right half of the circuit the cursor is placed  under the symmetry axis (the 50 nH shunt inductor) and Symmetrize command is applied (Figure 6.22.g-h).

- Finally the inverters are replaced by capacitive-L sections to get the final form of the circuit (Figure 6.22.i).

- The same technique can be used for adjustment of capacitance values also.

Exercise: Note that in the final circuit we have Pi-sections of capacitors. With cursor under the series elements of these Pi-sections, apply command Pi-Tee-L / Pi-to-Tee to convert them into Tee-sections and see if the capacitor values get better behaved or not.

Exercise: Try the commands LRight-to-Tee and LLeft-to-Tee to get a different version of filter with all inductors set to 50 nH.

Exercise: Try equating the capacitors using the same approach.

Exercise: Try to minimize the ratios (Lmax/Lmin) and  (Cmax/Cmin) using Pi-Tee-L transformations.

Figure 6.22. Synthesis of Type-2 filter and formation of equal inductor version.

Figure 6.22. Synthesis of Type-2 filter and formation of equal inductor version (cont’d).

EXAMPLE  6. 17: Synthesis of Type-1 filter and equal inductor version

Let’s synthesize a Type-1 filter with parameters,

PBRipple = 0.01 dB, fp1 = 95 MHz , fp2 =105 MHz,  Nzero=Ninf=5  (degree N=10), RL=RS=50.

Design command set is shown in Figure 6.23.

- Type-A is selected during Synthesis stage to get series elements at the ports.

- The transmission zeros are extracted in the same order giving the circuit of Figure 6.23.a. It is Type-1 form, with alternating series-shunt resonators.

- Since this is a narrowband filter, element values need be scaled. The series inductors are too large and shunt inductors are too small and vice versa for capacitors. Element value scaling is done by inserting inverters at source and load ends by applying the command Inverters / Scale Circuit By Inserting Inverters / Specify Adjacent Inductor: L=50 nH. This command makes the series inductors at source and load ends 50 nH and the internal series inductors and capacitors are scaled to reasonable values However the shunt inductors and capacitors became worse (Figure 6.23.b).

- Next the inverters are replaced by proper practical equivalents, for example capacitive L-sections by applying the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L (Figure 6.21.c). The L-sections have negative series capacitors. They are absorbed into the adjacent positive capacitor using Exchange and Combine command  (Figure 6.23.c-d).

- Next, we will adjust the internal element values. Let’s target to make all the internal inductors 50 nH. This is possible by using Pi-Tee-L transformations which increase the number of elements and change the topology of the circuit. Since it is usually not desirable to increase the number of inductors Pi-Tee-L transformations are applied only on capacitive Pi, Tee or L-sections. In order to form L-sections we split the first shunt capacitor on left into two equal parts and then use Exchange command to move one piece to the other side of the adjacent shunt inductor (Figure 6.23.c-d).

- Similarly, the series capacitor in the middle of the circuit is split into two equal parts and one piece is moved to the left of the adjacent shunt inductor. Thus, in the left half of the circuit we have formed one LLeft-section and one LRight-section of capacitors as shown in Figure 6.23.d. The LLeft-section will be used to scale up the 0.5078 nH shunt inductor to 50 nH while the LRight-section will be used to scale the series inductor on its right to 50 nH. Symmetrize command can be used to map these operations to the right half of the circuit.

- With cursor under the shunt leg of the LLeft-section, the command Pi-Tee-L / LLeft-to-Pi-Tee-L/ LLeft-to-Pi / Specify TRF: n=0.100785 is applied. Here n=0.100785 is found as the square root of the ratio 0.5078/50. The LLeft-section becomes a Pi-section while also producing the necessary specified transformer (Figure 6.23.e).

- Shifting the transformer rightmost using the keys CTRL+W, we get the circuit of Figure 6.23.f where the shunt inductor on left is now 50 nH.

- Next the 10265 nH series inductor in the middle will be scaled to 50 nH. This is done by placing the cursor under the shunt leg of the LRigth-section and applying the command Pi-Tee-L / LRight-to-Pi-Tee-L/ LRight-to-Pi / Specify TRF: n=14.32866 is applied. Here n=14.32866 is found as the square root of the ratio 10265/50. We get the circuit of Figure 6.23.g. The transformer is shifted rightmost scaling the inductor to 50 nH. In order to avoid repetition of these transformations in the right half of the circuit the cursor is placed  under the symmetry axis (the 50 nH shunt inductor) and Symmetrize command is applied to give the circuit of Figure 6.23.h where now all inductors are 50 nH.

Exercise: Note that in the final circuit we have Pi-sections of capacitors. With cursor under the series elements of these Pi-sections, apply command Pi-Tee-L / Pi-to-Tee to convert them into Tee-sections and see if the capacitor values get better behaved or not.

Exercise: Try the commands LRight-to-Tee and LLeft-to-Tee to get a different version of filter with all inductors set to 50 nH.

Exercise: Try equating the capacitors using the same approach.

Exercise: Try to minimize the ratios (Lmax/Lmin) and  (Cmax/Cmin) using Pi-Tee-L transformations.

Figure 6.23. Type-1 filter and equal inductor version.

Figure 6.23. Type-1 filter and equal inductor version (cont’d).

6.  5. COUPLED COIL FILTERS

Inductors of LC coupled resonator filters (Type 1 and Type 2) and some inductively coupled resonator filters can be manipulated so as to form coupled coils. Some examples are given below.

6.  5. 1. TYPE-3 and TYPE-4 FILTERS  (Csr+CCoil+Csh, Csh+Ccoil+Csr)

# EXAMPLE 6. 18: Csr+CCoil+Csh type filter

Filters of the previous Built in LC Prototype examples can be realized in coupled coil forms with alternating series and shunt capacitors by selecting these options. Type 3 starts with series capacitor and Type 4 starts with shunt capacitor. Element values are shown in Figure 6.24.a and b.

Figure 6.24. Element values of Type-3 and Type 4  filters.

Type 3 filter can be obtained from Type-1 filter using Coils transformation of Filpro, as follows:

- Consider the Type-1 circuit repeated in Figure 6.25.a. All resonators are splitted into L and C components using  Split / All 2 Element Resonators  command (Figure 6.25.b).

- Place the cursor under the internal series and shunt inductors and apply command Split / 2 Equal Pieces to split these inductors into two (Figure 6.25.c). Then use Exchange command to shift one piece to the left / right of the adjacent capacitors. Exchange also the first series arm inductor and capacitor. Thus, an LLeft and an LRight sections of inductors are formed as shown in Figure 6.25.d.

- Then, by placing the cursor under shunt legs of these L-sections apply command Coils / LLeft (LRight)- to-CoupCoil to convert them into coupled coils (Figure 6.25.e). Since the transformations are exact, this filter has the same response as the original one.

Exercise 6. 22: Show that Type-4 circuit can also be obtained from Type-2 circuit by splitting the internal series inductors and applying LLeft (LRight)-to-CCoil commands as described in Figure 6.26.

Figure 6.25. Obtaining Type-3 filter from Type-1.

Figure 6.26. Obtaining Type 4 bandpass filter from Type-2.

6. 5. 2. TYPE-5  FILTER (Csr + CCoil + Csr)

This structure involves only series capacitors and coils. It may be preferred over the previous two structures because of simplicity in construction. Approximate circuit transformations are used to get this structure from the exact LC prototypes. Because of nonuniqueness of circuit transformations to produce this circuit, many  equivalent versions with differing element values are possible. The version that is placed into Filpro is an equal series capacitor one. It can also be obtained from Synthesis or from Prototypes with inverter approaches.

EXAMPLE 6. 19: Design of Coupled Coil filter with series capacitors (Type-5) using Built in LC Prototype approach

Selection of the option Csr+Ccoil+Csr yields the element values shown in Figure 6.27 for the three filters studied in previous examples: PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz for Filter-1, fp1=80 MHz, fp2=120 MHz for Filter-2 and fp1=50 MHz, fp2=150 MHz for filter-3. All with prototype degree 5. Due to approximations involved in formulation of element values, response is degraded heavily especially for wideband cases. Therefore it is usual to prefer Design / By Synthesis approach that will be described later in this section.

Figure 6.27. Coupled coil filter with series capacitors (Type-5).

In order to get exercise with some tools of Filpro, let’s derive this filter from Type 1 filter.

EXAMPLE 6. 20: Inverters and derivation of Type-5 filter from Type-1 filter

Consider the Type-1 filter shown in Figure 6.28.a.

- At the first stage we will convert the shunt parallel resonators into series resonators. This is done by placing the cursor under shunt resonator and the command Inverters / Replace By Ideal Inverters / K=.. is applied giving circuit of Figure 6.28.b. Here K is the inverter constant which has to be specified by the user. Its value can be adjusted at this stage or later to scale the resulting series arm inductors and capacitors. Let’s select it as K=50. The phase changing transformer (1:-1) can be deleted.

- Next, cursor is placed under ideal inverters and command Inverters / Replace Ideal Inverter By / Pi-Tee Type / Inductive Tee / f1 =95 MHz, f2=105 MHz is applied to give the circuit of Figure 6.28.c. The inverters are now approximated by inductive Tee sections having negative series arms. These negative elements will be absorbed by the neighboring positive elements after application of Simplify Circuit command (Figure 6.28.d). Some deformations in return loss should be expected. These deformations can be corrected by tuning some elements.

- Splitting the series arm inductors and using Exchange command we form inductive Tee sections (Figure 6.28.e-f). Then with cursor under shunt inductor of Tee sections, apply command Coils / Tee-to-CCoil  we get the desired circuit shown in Figure 6.28.g.

- It is clear that the resultant circuit is nonunique. Several alternatives may be formed by choosing different inverter constants and/or by splitting the inductors in different proportions in forming the Tee sections.

- Replacing an element by ideal inverter doesn’t change the response. However replacing an ideal inverter by a practical equivalent circuit is an approximation which holds in narrow bands. Therefore response will change to some extent depending on the bandwidth of the filter. The distortions in response can be corrected by using Tune Element and/or Best Case commands. The main source of deformations is the fact that after replacement of ideal inverters by practical equivalents the number of transmission zeros at f= is reduced to unity while the number of transmission zeros at f=0 is increased to N-1=9 where N=10 is the degree of the bandpass filter. Hence, the slope of lower stopband skirt is increased while that of upper stopband skirt is decreased.

- Deviation from equal passband ripple property and band edge frequency shifts may be unacceptable for wideband filters. In such cases it is better to use  Design-By Synthesis  approach.

Figure 6.28. Development of Type-5 circuit from Type-1 circuit.

EXAMPLE 6. 21: Synthesis approach for Type-5 filters

Let’s design the filter with fp1=80 MHz, and fp2=120 MHz using synthesis approach.

- The targeted Type-5 filter has one transmission zero at f= and N-1=9 transmission zeros at f=0. Therefore the command set for Synthesis approach will be “New / Design / By Synthesis / Lumped, BP, Equiripple, Doubly Terminated, Type B / PB Ripple=0.1, fp1 =80, fp2 =120 / RS =RL= 50 / Nzero =9, Ninf =1”

- Here Type B option is selected to produce filter with shunt resonators in order to make use of some circuit transformations which are available for shunt resonators only.

- The transmission zero sections can be extracted in any order, for example, Zero / Zero / Zero /Zero / Zero / Inf /Zero / Zero / Zero / Zero giving the circuit shown in Figure 6.29.a.

- Applying the command Pi-Tee-L / All Equal Shunt / Inductors (or Capacitors) we get the capacitively coupled shunt resonator filter with equal shunt inductors (Figure 6.29.b).

- Find Dual command yields the targeted series resonator filter with equal series capacitors (Figure 6.29.c).

- Split / All Two Element Resonators command splits LC resonators into L and C components (Figure 6.29.d).

- The internal series inductors are split into two equal parts using Split / Two Equal command and Exchange command is used to move one piece to the other side of the adjacent capacitor. This way Tee sections of inductors are formed about each shunt inductor (Figure 6.29.e).

- Finally, with cursor under each shunt inductor, the command Coils / Tee-to-Coupled Coil is applied to convert the filter into a coupled coil filter with series capacitors (Figure 6.29.f). Response remains as exact equiripple, as no approximation is involved in the design (Figure 6.29.g).

Figure 6.29. Synthesis approach for designing coupled coil filter with series capacitors.

6. 6. COHN’S APPROACH FOR COUPLED RESONATOR FILTER DESIGN USING INVERTERS

Besides the technique Design / By Prototype with Inverters, it is also possible to design the inverter coupled resonator filters by using Design / By Built-in LC Prototype approach in which Cohn’s original formulation [3] is placed.  Since both approaches yield identical results there is a redundancy in this respect. Cohn’s formulation is placed into Filpro just for the sake of completeness. It may be slightly easier to use for some applications.

Ideal inverters are extremely useful elements which have lots of applications in filters. Some frequently needed circuit transformations involving inverters are programmed into Filpro for quick application. Cohn used inverters to design bandpass filters having only shunt or only series LC resonators interleaved by ideal inverters. Ideal inverters exist at source and load ends also. This is the main difference between Cohn’s approach and Prototype with Inverters approach. Therefore such filters can be designed using the Built in LC Prototype approach slightly faster, by selecting the option  Inv+pLCsh or Inv+sLCsr.  These filters may also be obtained from Type-1 or Type-2 filters by applying Inverters / Replace By Ideal Inverter command to series or shunt resonators. Ideal inverters can be used to scale elements and to change the topology of circuits. In this section, some circuit transformations involving ideal inverters will also be described together with Cohn type filters.

EXAMPLE 6. 22: Inverter Coupled Shunt Resonator Filter design using Built-in LC Prototype approach

Let’s use Cohn’s approach to design the three bandpass filter with specifications

PB Ripple= 0.01 dB, Prototype Degree N=5, RS=RL=50 Ohms, fp1 = 95 MHz, fp2 = 105 MHz

- Using Built in LC Prototype approach and selecting the option Cohn: pLCsh, a bandpass filter will be obtained with only shunt parallel resonators interleaved by ideal inverters. In developing the filter, Filpro asks the capacitance of each LC resonator in the order of appearance from the left end.  Thus, we can set the capacitances of resonators right at the outset. Initially all capacitors are selected as 10 pF and after inspecting the associated inductors, New Trial command is used and they are set as 50 pF to get all inductors about 50 nH (Figure 6.30.a). Let’s see some examples on what can be done using inverters.

- Parameters of any element in between two inverters can be adjusted to desired level. For example all shunt inductors can be set to exact 50 nH by placing cursor under the resonators and applying the command Inverters / Scale Element By Inverters / Specify Inductor: L=50  nH  (Figure 6.30.b).

- The inverters at source and load ends can be absorbed into RS and RL to scale the terminations by placing the curso  under RS or RL and applying the command Inverters / Scale RS-RL By Inverters / Absorb Inverter in RS-RL (Figure 6.30.c). Using Undo command twice we can recover the original circuit (Figure 6.30.d).

- It is possible to rescale the circuit by changing source and load end inverters. This is done by placing the cursor under the end inverters and applying the command Inverters / Scale Inverter / Specify Inverter: K=150 (Figure 6.30.d-e). Applying Simplify Circuit command eliminates the transformers giving the new scaled circuit of Figure 6.30.f.

- Inverters can be inserted at interior points of the circuit by placing the cursor under any element at  which inverter is to be inserted and applying the command Inverters / Insert Inverter at Interior / K=.. . This command places the inverter on the left of the pointed element and the dual of the circuit  the right of the inserted inverter is formed. This operation converts all shunt parallel LC resonators on the right of the inverter into series LC resonators as shown in Figure 6.30.g. Thus, we can form mixed series and shunt resonator filters.

- If two adjacent inverters are combined we get a transformer as shown in Figure 6.30.g-h. Negative sign of the transformer changes the phase of signal by 180 degrees.

- The inverters can be replaced by inductive or capacitive Pi- or Tee-sections as shown in Figure 6.30.h-i. The shunt legs of Pi-sections and series arms of Tee-sections are negative. They have to be absorbed in the adjacent positive elements. In Figure 6.30.i-k this is done by applying Simplify Circuit command.

- Simplify Circuit command moves the transformer to the load end. This transformer can be absorbed by the inverter at the load end by applying the command Inverters / Absorb TRF by Inverter (Figure 6.30.k-m).

- Let’s apply Undo command five times to get back to the circuit of Figure 6.30.f, repeated in Figure 6.30.n. The internal inverters are replaced by their capacitive and inductive Pi-section equivalents and the negative parts are absorbed into the adjacent positive elements (Figure 6.30.n-p-q).

- Finally the end inverters are replaced by their Capacitive-L equivalents with negative shunt legs which are then absorbed into the adjacent positive elements giving a shunt resonator filter with mixed couplings (Figure 6.30.q-r-s).

- Response is shown in Figure 6.30.t. Return loss degradation is mainly due to replacement of end inverters. It can be recovered to a great extent by tuning all elements slightly.

Figure 6.30. Cohn’s approach for inverter coupled shunt resonator filters.

Figure 6.30. Cohn’s approach for inverter coupled shunt resonator filters.

Figure 6.30. Cohn’s approach for inverter coupled shunt resonator filters.

EXAMPLE 6. 23: Obtaining Inv+pLCsh type filter from Type-1 filter

Inv+pLCsh type filters may also be obtained from Type-1 filters by applying proper circuit transformations. In Figure 6.31.a, the solution for narrowband case of Type-1 filter is shown.

Figure 6.31. Obtaining Cohn:Inv+pLCsh type filter from sLCsr+pLCsh type filter.

With the cursor under the series resonators, applying the command “Inverters / Replace By Ideal Inverter / K= ..” the series LC resonators will be replaced by shunt arm parallel LC resonators in between two ideal inverters with specified inverter constants. Here, we arbitrarily set all inverter constants as K=50. There are also phase changing (1:-1) transformers associated with the pairs of inverters. For amplitude filtering applications, they can be deleted without effecting the amplitude response (Figure 6.31.b-c). Next, the elements values of LC resonators should be adjusted to desired levels. This is possible by placing the cursor under the resonators and applying the command “Inverters / Scale Element By Inverter / Inductor / L=50 nH”. Here, we set all inductances to 50 nH, arbitrarily. Figure 6.33.d is the desired topology.

Exercise: Obtain the Cohn:Inv+sLCsr type filter from Type-2 filter by applying the same set of commands as in the previous example. Here, the shunt resonators will be converted to series resonators.

Ideal inverters can be used for many circuit transformations to either adjust element values or to put the circuit into a desirable topology. After the targeted topology is reached, ideal inverters should be replaced by various practical equivalents to yield a variety of structures. This is possible by using the command Inverters / Replace Ideal Inverter By/... command. Some examples will be shown below, just as exercise with the some of the circuit transformations involving inverters, without aiming at any specific target. Further applications of these commands will be seen occasionally in the forthcoming sections.

EXAMPLE 6.24: Inserting inverter at interior points - Antimetric to symmetric transformation

Ideal inverters can be inserted at interior points of a circuit using the command Inverters / Insert Inverter at Interior. This command takes the dual of the portion of the circuit to the right of the cursor, including the element pointed by the cursor and scales these elements in accordance with the specified inverter constant. In Figure 6.32, an application is described. In this example, first a Chebyshev bandpass filter with even degree lowpass prototype is designed. Even degree prototypes require different source and load resistances. Therefore RL should be specified different from RS. The difference between RS and RL sets also an upper bound on the permissible passband ripple. The resulting filter is antimetric. We can play with the circuit topology by inserting inverters at interior points. We can even make the circuit symmetric. Let’s consider two cases. In the first case, the passband ripple will be selected as the maximum permissible ripple and in the second case, a lower ripple will be typed in.

- The design command set for Case-1 is shown in Figure 6.32.a. The resulting circuit is shown in Figure 6.32.b.Let’s place the cursor under the series resonator in the middle and apply the command Inverters / Insert Inverter at Interior / K=50. The resulting circuit is shown in Figure 6.32.c. Here, K=50 is arbitrarily selected.

- This circuit can be made symmetric as follows: With the cursor under the inverter, the command Inverters / Scale Inverter / Equal inductors (or Equal Caps) is applied. The resulting transformer is absorbed in RL using the command Shift Trf / Absorb Trf in RL. Note that the circuit is symmetric.

- In Figure 6.33, the command set and circuit transformations for Case-2 are shown. Here, the passband ripple is chosen as 0.01 dB, less than the maximum permissible.

- Inserting an inverter at midpoint and applying the same transformation didn’t yield a symmetric circuit.

- Conclusion: Antimetric to symmetric transformation requires the use of maximum permissible passband ripple.

Figure 6.38. Antimetric to symmetric transformation by inserting inverter at midpoint.

Figure 6.32. Antimetric to symmetric transformation by inserting inverter at midpoint (continued).

Figure 6.33. Inserting inverter at midpoint of a Chebyshev filter with arbitrary passband ripple.

6. 7. TUBULAR (DUMBBELL) FILTERS

A class of bandpass filters termed as Tubular or Dumbbell  has the topology shown in Figure 6.34. Such filters are suitable for realization in either coaxial line form or in microstrip/suspended strip forms. That is, actually they are distributed element filters. However, for short commensurate lengths, their design may also be carried out as if they are lumped element filters. The distributed element versions are described in CH-11 (named as Minnis Class-C, D and E filters). The techniques described in this section clarify the links between lumped and distributed element Dumbbell filters.

These filters have one transmission zero at f=0 and N-1 transmission zeros at f=.

Figure 6.34. Dumbbell  (Tubular) filter structure.

EXAMPLE 6. 25: Synthesis of a wideband dumbbell filter

Let   PBRipple=0.01 dB, fp1 =750 MHz, fp2 =1250 MHz ,   RS=RL=50 ,  N=10  (Nzero=1,  Ninf =9).

Design steps are shown in Figure 6.35.

- New/Design / By Synthesis / Lumped, Bandpass, Equiripple, Doubly Terminated, Type B options are selected and PB Ripple, fp1and fp2  parameters are typed in.

- Transmission zeros Nzero=1, Ninf=9 are typed in. Standard Auto Extract command gives the circuit of Figure 6.35.a. It is saved as DB01.fil to be used later in another design approach.

- The command Pi-Tee-L / Equal Shunt Capacitor converts the circuit into coupled resonator form shown in Figure 6.35.b with shunt resonators. Find Dual command gives the targeted series LC resonator filter coupled with shunt capacitors as shown in Figure 6.35.c. We could as well select the option Type A at the beginning to get the same circuit, but Equal Inductor and Equal Capacitor options are programmed only for filters with shunt resonators. The Equal Shunt Capacitor version of the shunt resonator form yields Equal Series Inductor version in the dual form.

- With cursor under any series LC resonator, the command Split / All Two Element Resonators  is applied to split them into their L and C components (Figure 6.35.d).

- Then splitting the series capacitors into two equally and exchanging one with the neighboring inductor we form the symmetric structure shown in Figure 6.35.e. This circuit has Tee sections of capacitors separated by series inductors.  It may also be used for realization if the element values are realizable with this topology. It is saved as DB02.fil. However the usual approach is to convert the Tee sections into Pi sections. This is done by placing the cursor under the shunt legs of the Tee sections and applying the command Pi-Tee-L / Tee-to-Pi-Tee-L / Tee-to-Pi giving the circuit of Figure 6.35.f. It is saved as DB03.fil. This circuit is the standard lumped element prototype for obtaining Dumbbell filters. Depending on the frequency range involved this circuit can be realized in lumped LC form. However at high frequencies like in this example the series inductors and shunt capacitors may also be realized in transmission line form leading to the distributed element version of Dumbbell structure. This conversion is carried out as follows.

- First the series inductors are converted into transmission lines. With cursor under the series inductors the command Replace By TLine / f0=1000 MHz, Z0=200 ohms / Csh+TLine+Csh is applied. In this command the center frequency is selected for approximation. The inductors can best be simulated by high impedance lines. Therefore we select Z0=200 ohms (for example, if realizable!).  We have the options TLine Only and Csh+TLine+Csh. The second option is a better approximation for replacement of an inductor. However it involves negative shunt capacitors. Therefore this option can be used only if the negative shunt capacitors can be absorbed by the neighboring positive shunt capacitors. If there is no such a neighboring capacitor we can simply Delete the negative capacitors, leading to the same result as the TLine Only option. In this example we have positive shunt capacitors to cancel the inner negative capacitors. Simplify Circuit command eliminates the inner negative capacitors by combining them with their neighbors. However the capacitors at the ends are left unabsorbed. Therefore we delete them. Since the circuit is symmetric we can use Symmetrize command to reduce the labor (Figure 6.35.g-h). These conversions may degrade response appreciably depending on the impedance selected for the transmission lies. Fortunately the original response can be recovered simply by tuning a few elements (Figure 6.35.i).

- Next the shunt capacitors will be replaced by transmission lines by applying the command Replace By TLine / f0=1000 MHz, Z=25 ohms / Lsr+TLine+Lsr (Figure 6.35.k-m). In this case the lowest realizable impedance have to be selected for the transmission line to simulate the shunt capacitors better. Here also we have the options TLine Only and Lsr+TLine+Lsr where the series inductors are negative. The second option is a better replacement for the shunt capacitors. It can be selected if they can be absorbed into the neighboring element. In the circuit of Figure 6.35.m, most of the negative inductors have to be deleted because there are no positive neighbors to absorb. We only keep the negative inductors adjacent to the series capacitors. Simplify Circuit command combines the negative inductors with the adjacent positive capacitors to form LC resonators as shown in Figure 6.35.n.

- Next with cursor under the LC resonators, the command Constant Reactances / Replace By Constant Reactance: f0=1000 MHz is applied to convert them into constant reactances (Figure 6.35.p). Then the constant reactances are converted into capacitors by applying the command Constant Reactances / Replace Constant Reactance By / Capacitor: f0=1000 MHz (Figure 6.35.q). This way, instead of deleting the negative inductors (that would lead to higher degradation in response), we have absorbed them into the adjacent positive capacitors (less degradation in response!). Even for this case we get a heavy degradation in response (see Figure 6.35.u) because the 25 ohms characteristic impedance is not low enough. Response can be recovered to a great extent by first tuning elements coarsely and then applying Monte Carlo optimization routine (Figure 6.35.q-s). Figure 6.35.u shows the degraded and recovered responses.

- Figure 6.35.t shows a pictorial description for possible realization. The series capacitors can be realized in different forms depending on the technology used. For coaxial (tubular) realizations, the parallel plate capacitance action between the ends of the dumbbell sections can be used while for stipline-suspended stripline-microstrip applications conductor coated dielectric patches can be used overlapping with the two neighboring dumbbell sections. A more general procedure may be to convert the Pi-section of capacitors into parallel coupled line form by first converting all the capacitors into OC stubs. However this would give asymmetric parallel coupled lines which may be difficult to handle. This technique will be described after converting the Pi-sections into equal shunt capacitor form in the following lines.

Figure 6.35. Design of a wideband dumbbell filter.

Figure 6.35. Design of a wideband dumbbell filter (continued).

Figure 6.35 Design of a wideband dumbbell filter (continued).

Figure 6.35. Design of a wideband dumbbell filter (continued).

Equal Shunt Capacitor Versions:

In some cases it may be desirable to have equal shunt capacitors. The procedure is as follows.

- The circuit of DB03.fil is loaded (Figure 6.36.a). The shunt legs of the each capacitive Pi-sections are equated by applying the command Pi-Tee-L / Pi-to-Sym Pi command. The resulting transformers are moved away by applying Simplify Circuit command. Since the circuit is symmetric no transformer is left after all Pi-sections are made symmetric (Figure 6.36.b).

- The first Pi-section has shunt capacitors of 1.14 pF while the second one has 1.47 pF. Next these two values will be equated. This can be carried out on the left half of the circuit by transferring part of the series capacitor of the first Pi section to the second one as follows.

- With cursor under the right leg of the first Pi section, the command Pi-Tee-L / LLeft-to-Tee / Specify Transformer: n=0.88 is applied. Here the transformer turns ratio n is calculated from the ratio of the capacitances 1.14 pF and 1.47 pF. This command converts the LLeft section into Tee section producing also the specified transformer (Figure 6.36.c).

- The transformer is moved away and the series inductor and capacitor are swapped using Exchange command to form an LLeft section of capacitors (Figure 6.36.d).

- Then the LLeft section is converted into LRight section using the command Pi-Tee-L / LLeft-to-Pi-Tee-L / LLeft-to-LRight. Then Simplify Circuit command is applied to move away the transformer and also combine the adjacent series capacitors of the second Pi section (Figure 6.36.e-f).

- Next the shunt capacitors of the Pi-sectons are made equal by applying the command Pi-Tee-L / Pi-to-Sym Pi (Figure 6.36.g). Now the shunt capacitors of the first and second Pi sections become closer to each other. This procedure may be iterated a few times if exact equality of the shunt capacitors is needed. The circuit is saved as DB04.fil.

- Conversion of the series inductors and shunt capacitors into transmission lines is carried out in the same way as in the previous case leading to the circuit of Figure 6.36.h. The response is shown in Figure 6.36.i.

Figure 6.36. Alternative designs of the wideband dumbbell filter.

Figure 6.36. Alternative designs of the wideband dumbbell filter (continued).

Figure 6.36. Alternative designs of the wideband dumbbell filter (continued).

Comb Type Coupled Line Version:

If desired, instead of the lumped capacitors, the capacitive Pi sections may be replaced by Comb type parallel coupled lines as follows.

- The circuit of DB04.fil is loaded. The series inductors are replaced by their Csh+TLine+Csh equivalents (Figure 6.36.k-m).

- The negative inductors at source and load ends are deleted while the inner negative shunt capacitors are combined with their neighbors giving the circuit of Figure 6.36.n. Response have to be optimized coarsely after each replacement. Then the final circuit can be optimized by Monte Carlo optimization routine giving the circuit of Figure 6.36.p.

- The Pi sections are made symmetric by applying the command Pi-Tee-L / Pi-to-Sym Pi (Figure 6.36.p-q).

- The lumped capacitors are converted into OC stubs by applying the command Replace By Stub / Specify Length: Lng=20 mm, f0=1000 MHz (Figure 6.36.r). Here we arbitrarily select the length of stubs (all identical!). We may try different lengths. Response will be effected slightly which can be corrected by a slight tuning.

- With cursor under the series elements of the Pi sections of OC stubs the command Replace By PCL / Combline / ococCOMB / Symmetric is applied to convert the Pi sections into comb type OC parallel coupled lines (Figure 6.36.s).

- Figure 6.36.t shows a pictorial description. It is seen that line lengths are widely differing. This is because instead of equal inductors we have selected the option equal shunt capacitors. This lead to widely different inductors, hence different lengths for the corresponding transmission lines.

Dumbbell command for Equal Shunt Capacitor cases:

Filpro has a special command to reduce the labor for conversion of the synthesized filter into equal shunt capacitor form described in Figure 6.37:

- The circuit of DB01.fil is loaded (Figure 6.37.a). Its dual is formed by applying Find Dual command (Figure 6.37.b).

- The series capacitor in the middle is split into two equal parts and Exchange command used to form a symmetric circuit to work only on the left half of the circuit (Figure 6.37.c).

- With cursor under the shunt capacitor of the LRight section of capacitors the command Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-LLeft is applied to move the series capacitor to the left of the shunt capacitor (Figure 6.37.d). The resulting transformer is shifted rightmost.

- Now in the circuit of Figure 6.37.d, we have a shunt capacitor followed by a series inductor and capacitor. This circuit section can be transformed as shown in Figure 6.37.e into a symmetric Pi section of capacitors followed by a series inductor and an LRight section of capacitors. With cursor under the shunt capacitor the command Dumbbell/ C=1.114 is applied. Here we can specify the shunt capacitor of the resulting symmetric Pi section within the limits displayed at the bottom. The proper value for the capacitance is found by iterative trials as follows.

- After applying the Dumbbell command with a trial capacitance we get the circuit of Figure 6.37.f. The LRight section on the right of the symmetric Pi section is then converted into symmetric Pi section by applying the command Pi-Tee-L / LRight-to-Sym Pi (Figure 6.37.g). After this conversion we compare the shunt capacitors of the  two Pi sections. If they are not close enough in value then we use Undo command and repeat Dumbbell command with a new capacitance. A few iterations are sufficient to get equal shunt capacitor version.

- Finally, Symmetrize command is applied together with Simplify Circuit command to eliminate the transformers to give the circuit of Figure 6.37.h which has all equal shunt capacitors.

- The rest of the procedure is identical to those in the previous figures.

Figure 6.37. Dumbbell command for obtaining equal shunt capacitor version.

EXAMPLE 6.26. Using Cohn’s technique for designing Dumbbell filter

On Filpro it is possible to design Dumbbell filters using a variety of approaches. As another example let’s start the design from Cohn’s approach now for a narrowband filter with following parameters:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, LP Prototype Degree N=5

Design steps are shown in Figure 6.38.

- Design / By Built-in LC Prototype approach is selected.

- Lumped-Bandpass-Chebyshev options are selected.

- Terminations are left at 50 ohms.

- Attenuation Specs / PB Specs and Prototype Degree options are selected and PBRipple, fp1 and fp2 values are typed in.

- Find Circuit command displays the list of alternative solutions. We select the solution Cohn: Inv+sLCsr (series LC resonators coupled by inverters). Then we type in the inductor values of LC resonators. Just as a trial we assign L=10 nH for all inductors getting the circuit shown in Figure 6.38.a with all series resonators having 10 nH inductance.

- With cursor under any LC resonator the command Split / All Two Element Resonators is applied to split them into L and C components (Figure 6.38.b).

- Next the inverters at source and load ends are replaced by their capacitive L-section equivalents by applying the command Inverters / Replace Inverter By / End Inverter to / Capacitive L / f0=1000 MHz. The series arm of the L-section has negative value which will be absorbed into the adjacent series capacitor of the LC resonator (Figure 6.38.c).

- Next the inner inverters are replaced by their capacitive Tee equivalents by applying the command Inverter / Replace Inverter By / Pi-Tee Type / Capacitive Tee / f1=975 MHz, f2=1025 MHz (Figure 6.38.d).

- Simplify Circuit command combines adjacent elements to form series LC resonators again (Figure 6.38.e). Response will be degraded slightly by replacement of inverters with their practical equivalents. Response can be recovered to a great extent by tuning all capacitors slightly (Figure 6.38.f).

- The series LC resonators are split into their L and C components again by applying Split / All Two Element Resonators (Figure 6.38.g).

- Next we will form capacitive Pi-sections between inductors. This is done by first splitting the series capacitors into two equal parts (unequal split may also be tried!) by applying the command Split / Two Equal (Figure 6.38.h).

- Then Exchange command is applied to move halves of the capacitors to the other sides of the adjacent inductors as shown in Figure 6.38.i.

- With cursor under the shunt legs of the capacitive L-sections at ports, the command Pi-Tee-L / LRight (LLeft)-to-Symmetric Pi is applied to form Pi-sections of capacitors at ports. Then the inner Tee-sections of capacitors are converted into Pi-sections by applying the command Pi-Tee-L / Tee-to-Pi-Tee-L / Tee-to-Pi (Figure 6.38.k).

- Simplify Circuit command eliminates the resulting transformers (Figure 6.38.m). This is the targeted structure known as Dumbbell filter. It has equal series inductors while the inner Pi-sections of capacitors have unequal shunt legs. This circuit is saved as dmb01.fil.

- The shunt capacitors of inner Pi-sections can be equated by placing the cursor under the series element of the Pi-sections and applying the command Pi-Tee-L / Pi-to-Sym Pi (Figure 6.38.n). This circuit is saved as dmb02.fil.

- The series inductors can be converted into transmission line pieces with high characteristic impedances and shunt capacitors can be converted into low impedance transmission lines. However these conversions will be described in the next example with wider bandwidth.

Figure 6.38. Design of Dumbbell filter by starting from Cohn’s approach.

Figure 6.38. Design of Dumbbell filter by starting from Cohn’s approach (continued).

Figure 6.38. Design of Dumbbell filter by starting from Cohn’s approach (continued).

Figure 6.38. Design of Dumbbell filter by starting from Cohn’s approach (continued).

6. 8. WIDEBAND BANDPASS FILTERS CASCADES OF  LP AND HP FILTERS

In some cases it may be preferable to form a wideband filter by cascading a LP and a HP filter. Let’s see and example.

Example 6.27. Wideband BPF by cascading a LP and a HP filter

Desired filter specifications are as follows:

PBRipple=0.01 dB, fp1=100 MHz, fp2=500 MHz, Nzero=5, Ninf=5

This filter can be directly synthesized as a bandpass filter as described in the previous sections. Now let’s design it by cascading a LPF with specifications

PBRipple=0.01 dB,  fp=500 MHz, Ninf=5

and a HPF with specifications

PBRipple=0.01 dB, fp=100 MHz, Nzero=5

- Design of the LPF is described in Figure 6.39.a, using Built-in LC Prototype approach (simplest). It is saved as LP1.fil.

- Design of the HPF is described in Figure 6.39.b using again Built-in LC Prototype approach. It is saved as HP1.fil.

- With the HP1.fil being on the display, placing the cursor under the load and applying the command Append Filter: LP1.fil, the LP filter is placed on the right of the HPF. We could as well place the LPF on the left of the HPF.

- Inspection of the return loss shows that this brute force cascading leads to slight shifts in band edges and about 3 dB lowering in return loss level. These effects become even less as the component filter band edges get far from each.

Exercise: Find dual of HP1.fil and append it to LP1.fil.

Exercise: Find dual of LP1.fil and append it to LP1.fil.

Exercise: Find duals of HP1.fil and LP1.fil and then cascade them.

Exercise: Apply Pi-Tee-L commands to the L-sections of capacitors and inductors to form a variety of topologies to adjust levels of element values.

Exercise: Design narrower bandwidth filters and append them to study their interaction that degrades the response.

Exercise: Use Synthesis approach to design LP and HP filters of different types and different band edges  and append them to form BP filters with medium and wide bandwidths.

Figure 6.39. Design of wadeband BP filters by cascading LP and HP filters.

Figure 6.39. Design of wadeband BP filters by cascading LP and HP filters.