6. 8.  ELLIPTIC BANDPASS FILTERS

6. 8. 1. INTRODUCTION

The filters having finite transmission zeros (FTZ’s) are conventionally termed as elliptic filters or quasi-elliptic filters. The name elliptic remains from the historical development stages of the theory of filter synthesis where elliptic functions were used in the approximation functions to get both equal ripple passband and equal ripple stopbands. We will continue to use this name though our approximating functions do not involve elliptic functions anymore. Design / By Synthesis approach enables us to place FTZ’s anywhere in the stopbands to shape responses as needed in the specifications, like to increase the skirt slopes or eliminating an interference.

Compared to the classical elliptic type filters designed from the lowpass prototype tables that are available in many textbooks or handbooks, Synthesis by placement of FTZ’s offer the following important advantages:

- In filters designed by mapping from elliptic LP prototypes, a transmission zero at f=0 of the prototype is mapped as two transmission zeros in the bandpass filter, one being at f=0 and the other being at f=. Therefore we always get equal numbers of transmission zeros at f=0 and f=. In Synthesis by placement of transmission zeros we can place any number of transmission zeros at zero  and infinity leading to great flexibility in shaping the response.

- A finite transmission zero of the elliptic LP prototype is mapped onto two sides of the passband in a geometrically symmetric manner. That is fz1fz2=fp1fp2 =f02 where fz1 and fz2 are the finite transmission zero frequencies, fp1 and fp2 are passband edge frequencies and f0 is the geometric center of passband. In Synthesis approach any number of finite transmission zeros can be placed in any stopband. Thus asymmetric upper and lower stopband losses can be formed with increased rejection in selected frequency ranges.

- In filters designed by mapping from elliptic prototypes the minimum stopband insertion loss is the same in both stopbands while in Synthesis approach different insertion loss levels can be formed in the two stopbands.

- In Synthesis approach by placement of transmission zeros, one may adjust the numbers and locations of FTZ’s and the numbers of transmission zeros at f=0 and f= to get alternative topologies which may be more convenient for realization, like for example minimizing the number of inductors.

The following constraints exist on the number of transmission zeros:

- The degrees of bandpass filters must be even. Since each FTZ provides double degree, this constraint reduces to the condition

Nzero+Ninf= Even Integer.

- This condition can be satisfied by selecting either

Nzero= Odd integer  and   Ninf = Odd integer

leading to an electrically symmetric filter or

Nzero= Even integer  and  Ninf = Even integer

leading to an antimetric filter.

Bandpass filters with Nzero+Ninf=Odd integer can not be synthesized using Filpro. Such filters are termed as parametric filters, requiring special approximation theory approach. However, for narrowband cases, such filters may also be obtained using Filpro through brute force approximations.

Let’s see further details through examples.

6. 8. 2. DEGREE  SIX  FILTERS  WITH FINITE TRANSMISSION ZEROS

EXAMPLE 6. 29. A degree six with filter Nzero=3, Ninf=1 and a single FTZ in lower stopband:

Minimum inductor form

Let’s consider a filter with following specifications:

PBRipple =0.01 dB, fp1 =95 MHz, fp2=105 MHz, Nzero=3, Ninf=1, NFTZ=1 at fz=85 MHz,  RS=RL=50

Different realization approaches are possible for this filter, like for example, minimum inductor configuration or coupled resonator configurations or cross-coupled configurations. In this example we will study the minimum inductor form.

- Design steps are shown in Figure 6.40. The option Type A is selected because number of inductors come out to be less than in Type B. This point can be checked by using Find Dual command on Type B circuit. Experience shows that element values are better behaved if the circuit has a symmetric or close to symmetric topology. Such topologies also yield load end transformers with reasonable values. In this example we have a single finite transmission zero and a single transmission zero at f=. In order to get a  symmetric structure such single transmission zeros should be extracted in the middle part of the circuit.  In this example, it is found that the following extraction order produces unity load end transformer and symmetric circuit: Zero-FTZ-Inf-Zero-Zero. The resulting circuit is shown in Figure 6.40.a. Symmetry is around the two elements in the middle.

- Element values need be adjusted in this circuit. The main tool that can be used for this purpose is Pi-Tee-L transformations. These transformations can be applied to L, Tee or Pi sections of capacitors or inductors. Since these transformations increase the number of redundant elements, it is usually preferred to apply them on only capacitive Pi, Tee, L sections. In this example, we search for capacitive L-sections and see that two such sections can be formed by splitting the shunt capacitor in the middle and using Exchange command to move one piece to the left of the FTZ element as described in Figures 6.78.b. In the left half of the circuit an LLeft-section is formed and in the right half, an LRight-section is formed. Now the symmetry axis of the circuit is the shunt FTZ element.

- Pi-Tee-L transformations are used to adjust internal elements. Before adjusting the internal element values it is wise to adjust the end elements because they effect the whole circuit. The end elements should be scaled by inserting inverters. We have shunt inductors as end elements. They can be scaled to, for example, 50 nH  by applying the command Inverters / Scale Circuit by Inserting Inverters / Specify Adjacent Inductors: L=50 nH (Figure 6.40.c).

- Next, the 11.861 nH inductor of the FTZ section at the center will be scaled up to 50 nH. With cursor under the shunt leg of LLeft-section the command Pi-Tee-L / LLeft-to-Pi-Tee-L/  LLeft-to Pi / Specify TRF: n=0.48705  is applied converting the LRight-section into a Pi-section while also producing the specified transformer. n=0.48705 is calculated as the square root of the ratio 11.861/50. The transformer is shifted right most by pressing the keys Ctrl+W. Then with cursor under the symmetry element, Symmetrize command is applied to avoid the repetition of this transformation on right. Simplify  Circuit command combines the shunt L and C elements to convert them into shunt LC resonators (Figure 6.40.e). Now all inductors are 50 nH.

- Finally, the inverters are replaced by their practical equivalents. We usually prefer to replace them by their Capacitive-L equivalents by applying the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L. The shunt legs of the L-sections are negative. They need be absorbed into adjacent positive capacitors. The adjacent capacitors were created during LLeft-to-Pi operation. That is why we have left the conversion of inverters into L-sections to the end of the procedure  (Figure 6.40.f).

- Simplify Circuit command combines the negative shunt capacitors with adjacent positive capacitors and also combines the two capacitors on the two sides of the FTZ element (Figure 6.40.g).

- Figure 6.40.h shows the lossy version of the filter after converting all elements into lossy form by specifying Q=100 in the command Parasitics / Lossless-to-Lossy. The nodal Voltage-Current-Power variables are displayed by pressing the keys ALT+F8 and specifying Vs=100 volts and a frequency. A search is made to find the frequency at which maximum nodal voltage occurs. It is located at about f=91 MHz (always on the skirt with higher slope, near the edge of the passband).

- Figure 6.40.i shows the insertion loss before and after conversion into lossy form.

Exercise: Repeat the design by selecting Type-B and apply similar transformations to see the problems (too many inductors!).

Figure 6.40. Degree six filter with a single FTZ in lower stopband.

Figure 6.40. Degree six filter with a single FTZ in lower stopband.

EXAMPLE 6. 30. A degree six filter with Nzero=1, Ninf=3 and a single FTZ in lower stopband:

Minimum inductor form

Let’s see the realization by placing mode transmission zeros at f=:

PBRipple =0.01 dB, fp1 =95 MHz, fp2=105 MHz, Nzero=1, Ninf=3, NFTZ=1 at fz=85 MHz,  RS=RL=50

- Design steps are shown in Figure 6.41. The option Type A is selected.

- It is found that the following extraction order can lead to a symmetric realization: Inf-FTZ-Zero-Inf-Inf. The resulting circuit is shown in Figure 6.41.a.

- The LLeft-section of capacitors in the right half is converted into LRight-section. Shifting the resulting transformer away we get a symmetric circuit. The capacitor in the middle is split into two equal parts and one piece is moved to the left of the adjacent FTZ element, thus forming an LLeft-section in left half and an LRight section in the right half (Figure 6.41.c).

- The end inductors are scaled up to 50 nH  by applying the command Inverters / Scale Circuit by Inserting Inverters / Specify Adjacent Inductors: L=50 nH (Figure 6.41.d).

- Then the inverters are replaced by their capacitive L-section equivalents by applying the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L. The series arms of the L-sections are negative. They are absorbed into adjacent positive capacitors by using Exchange and Combine commands (Figure 6.41.e-f). Symmetrize command is used to map the operations done in left half to right.

- Next, the 12.748 nH inductor of the FTZ section at the center will be scaled up to 50 nH. With cursor under the shunt leg of LLeft-section the command Pi-Tee-L / LLeft-to-Pi-Tee-L/  LLeft-to Pi / Specify TRF: n=0.5049  is applied converting the LRight-section into a Pi-section while also producing the specified transformer. n=0.5049 is calculated as the square root of the ratio 12.749/50. The transformer is shifted right most by pressing the keys Ctrl+W. Then with cursor under the symmetry element, Symmetrize command is applied to avoid the repetition of this transformation on right. Simplify  Circuit command moves away the transformers and combines the shunt capacitors on the two sides of the FTZ element giving the circuit of Figure 6.41.i. Now all inductors are 50 nH.

- Figure 6.41.k shows the lossy version of the filter after converting all elements into lossy form by specifying Q=100 in the command Parasitics / Lossless-to-Lossy. Figure 6.78.i shows the insertion loss before and after conversion into lossy form.

Figure 6.41. A degree six filter with Nzero=1, Ninf=3 and a FTZ in lower stopband.

Figure 6.41. A degree six filter with Nzero=1, Ninf=3 and a FTZ in lower stopband (cont’d).

EXAMPLE 6. 31. A degree six filter with Nzero=1, Ninf=3 and a single FTZ in lower stopband:

Direct Coupled and Cross-Coupled Resonators versions

Let’s realize the filter the previous  example in Direct Coupled and Cross-Coupled Resonator forms:

PBRipple =0.01 dB, fp1 =95 MHz, fp2=105 MHz, Nzero=1, Ninf=3, NFTZ=1 at fz=85 MHz,  RS=RL=50

- Design steps are shown in Figure 6.42. Option Type B is selected.

- The transmission zeros are extracted in the same order as the previous example: Inf-FTZ-Zero-Inf-Inf. The resulting circuit is shown in Figure 6.42.a.

- The command Pi-Tee-L / Equal Shunt Inductors is applied to convert the circuit into direct coupled resonators form (Figure 6.42.b). It is saved as cplres01.fil to be used later for obtaining cross-coupled resonators version.

- The circuit is scaled so that all shunt inductors are 50 nH by applying the command Inverters / Scale Circuit By Inserting Inverters / Specify Inductor: L=50 nH (Figure 6.42.c).

- The transformer at load end is absorbed into inverter by applying the command Inverters / Absorb TRF in Inverter (Figure 6.42.d).

- Then the inverters are replaced by their capacitive L-section equivalents by applying the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L. The shunt arms of the L-sections are negative. They are absorbed into adjacent positive capacitors by using Simplify Circuit command giving the targeted direct coupled resonator version of the filter (Figure 6.42.e-f).

- The coupling between nodes 1 and 2 are of parallel LC type which is easy to construct at low frequencies in lumped element form but may be problematic in high frequency applications. The LC type coupling can be replaced by a single element type, like a simple inductor or capacitor by introducing a cross-coupling between node-1 and node-3. The theory, details and alternative structures are explained in Chapter-14. Here just a demonstration will be made to give an idea on cross-coupled filters.

- The circuit of cplres01.fil is loaded (Figure 6.42.g). With cursor under the series arm FTZ element the command CQ-CT Oprs / Convert to CT / CLL is applied. Here, the term CLL means that the cross-coupling element between node-1 and node-3 is a capacitor (C13) while the couplings 1-2 and 2-3 are of inductors ( L12, L23). The circuit section between node-1 and node-3, excluding the resonators of node-1 and node-3 is shown as a box as shown on the right of Figure 6.42.h

- Next the circuit is scaled by inserting inverters, in the usual manner (Figure 6.42.h-i-k).

- The lossy version can be formed by applying the command Parasitics / All Lossy Resonators (Figure 6.42.m).

- The cross-coupled resonator forms are especially useful when the FTZ is too close to the passband edge and the bandwidth is very narrow (See Chapter-14).

Exercise: Realize the degree six filter with Nzero=3 and Ninf=1 with one FTZ in lower stopband in direct coupled and cross-coupled forms and compare the resulting circuits with those of this example.

Figure 6.42. Direct coupled and cross-coupled versions of the six resonator filter.

Figure 6.42. Direct coupled and cross-coupled versions of the six resonator filter.

EXAMPLE 6. 32. A degree six filter with Nzero=1, Ninf=3 and a single FTZ in upper stopband:

Minimum inductor version

Let’s now place a FTZ in upper stopband and realize a minimum inductor configuration:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=3, one FTZ at fz=115 MHz,  RS=RL=50

- Design steps are shown in Figure 6.43. Option Type A is selected.

- The transmission zeros are extracted in the same order as the previous example: Inf-FTZ-Zero-Inf-Inf. The resulting circuit is shown in Figure 6.43.a. It can be put into symmetric form about the FTZ element by splitting the series capacitor into two equal parts and shifting one piece to the left of the FTZ element using Exchange command (Figure 6.43.b).

- The circuit is scaled so that the series inductors at source and load ends are 50 nH by applying the command Inverters / Scale Circuit By Inserting Inverters / Specify Inductor: L=50 nH (Figure 6.43.c).

- Then the inverters are replaced by their capacitive L-section equivalents by applying the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L. The series arms of the L-sections are negative. They must be absorbed into adjacent positive capacitors. However there are no capacitors at these positions.  They will be provided by applying Pi-Tee-L / LRight-to-LLeft (or LRight-to-Tee) to the LRight-section of capacitors in left half and the command Pi-Tee-L / LLeft-to-LRight to the LLeft-section of capacitors in the right half (Figure 6.43.c-d).

- The negative capacitors are combined with adjacent positive capacitors using Exchange and Combine commands. The transformers are moved rightmost by pressing the keys Ctrl+W and Symmetrize command is applied to map the left half to right (Figure 6.43.d-e).

- Next we want to scale the inductor of the FTZ element in the middle to 50 nH also. However this value can’t be achieved because the capacitor values of the LLeft and LRight sections can’t produce the necessary transformer turns ratio. The maximum transformer that the L-sections  can give is obtained by converting LLeft-section to LRight and LRight-section to LLeft.  When we do these conversions we get about 47 nH for this inductor whch is close to 50 nH (Figure 6.43.f).

- Figure 6.43.g shows insertion and return loss responses. We notice a shrink in return loss bandwidth. This is because of the approximation of the inverters by capacitive L-sections and can be corrected to a certain extent by tuning elements (Exercise).

Exercise: Repeat the design to get direct coupled and cross-coupled resonator forms.

Exercise: Repeat the design for Nzero=3 and Ninf=1 with fz=115 MHz to get minimum inductor form.

Exercise: Repeat the design for Nzero=3 and Ninf=1 with fz=115 MHz to get direct coupled and cross-coupled resonator forms.

Figure 6.43. A degree six filter with Nzero=1, Ninf=3 and a FTZ in upper stopband.

Figure 6.43. A degree six filter with Nzero=1, Ninf=3 and a FTZ in upper stopband (cont’d).

EXAMPLE 6. 33. A degree six filter with Nzero=1, Ninf=1 and two FTZ’s in lower stopband

Let’s now place two FTZ’s in lower stopband:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=3, NFTZ=2, at fz1=74 MHz and fz2=85 MHz to get equiripple lower stopband,  RS=RL=50

- Design steps are shown in Figure 6.44. Option Type B is selected. The transmission zeros are extracted in the order FTZ1-FTZ2-Inf-Zero. The resulting circuit is shown in Figure 6.44.a. It can be put into shunt resonator form with parallel LC coupling elements by applying the command Pi-Tee-L / Equal Shunt Capacitors (or Equal shunt inductors) (Figure 6.44.b).

- Find Dual command gives the version with series LC resonators in both series and shunt arms. (Figure 6.44.c). Both versions are realizable. No reduction is possible in the number of inductors. Element values can be scaled by inverters, as was done in the previous examples.

Figure 6.44. Degree six filters with two FTZ’s in lower stopband.

EXAMPLE 6. 34. A degree six filter with Nzero=1, Ninf=1 and two FTZ’s in upper stopband

Let’s now place two FTZ’s in upper stopband:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=3, NFTZ=2, at fz1=115 MHz and fz2=124 MHz to get equiripple lower stopband,  RS=RL=50

- Design steps are shown in Figure 6.45. Option Type B is selected. The transmission zeros are extracted in the order FTZ1-FTZ2-Inf-Zero. The resulting circuit is shown in Figure 6.45.a. It can be put into shunt resonator form with parallel LC coupling elements by applying the command Pi-Tee-L / Equal Shunt Inductors (or Equal shunt capacitors) (Figure 6.45.b).

- Find Dual command gives the version with series LC resonators in both series and shunt arms. (Figure 6.45.c). Both versions are realizable. No reduction is possible in the number of inductors. Element values can be scaled by inverters, as was done in the previous examples.

Figure 6.45. Degree six filters with two FTZ’s in upper stopband.

EXAMPLE 6. 35. A degree six filter with Nzero=1, Ninf=1 and one FTZ’s in each stopband

Let’s now place one FTZ in each stopband:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=3, NFTZ=2, at fz1=85 MHz and fz2=115 MHz,  RS=RL=50

- Design steps are shown in Figure 6.46. Option Type B is selected. The transmission zeros are extracted in the order FTZ1-FTZ2-Inf-Zero. The resulting circuit is shown in Figure 6.46.a.

- It can be put into shunt resonator form with parallel LC coupling elements by applying the command Pi-Tee-L / Equal Shunt Inductors (or Equal shunt capacitors) (Figure 6.46.b). This circuit is one of the many possible solutions. It is saved as shres11.fil.

- With cursor under the first shunt resonator on left the command Double Norton / Inverse Geffe is applied. This command eliminates the shunt resonator in between the two FTZ sections (Figure  6.84.c). Simplify Circuit command gives the circuit of Figure 6.46.d which another possible solution. It is saved as shres12.fil.

- With cursor under the left FTZ section, Combine command is applied to combine the two FTZ sections to form a fourth order section as shown in Figure 6.46.e. This solution is saved as shres13.fil.

- With cursor under the shunt resonator on the left of the fourth order section the command Double Norton / Geffe Equivalent is applied. This command asks a transformer turns ratio k1 and converts the circuit back to the three shunt resonator form of Figure 6.46.b (shres11.fil), but with several options:

- The option Unity Total Transformer yields a circuit with no transformer.

- The option No Middle Lsh eliminates the inductor of the second shunt resonator in the middle, thus reducing the number of inductors by one.

- In the third option turns ratio of a second transformer (k2) is asked. This produces a circuit with a transformer in its right which can be used to scale the circuit sections on the right.

- Figure 6.46.f shows the option No Middle Lsh. This solution is saved as shres14.fil.

- The other possible solutions are just the duals of these circuits, as shown in Figures 6.84.g-to-k.

- Element values can be adjusted by using inverters as was described in the early examples.

Figure 6.46. Degree six filters with Nzero=1, Ninf=1 and one FTZ in each stopband.

Figure 6.46. Degree six filters with Nzero=1, Ninf=1 and one FTZ in each stopband (cont’d).

6. 8. 3. DEGREE EIGHT ELLIPTIC FILTERS

EXAMPLE 6. 36. A degree eight filter with Nzero=1, Ninf=5 and one FTZ’s in lower stopband

As the number of transmission zeros at f=0 and f= increase we get more flexibility for adjusting impedance levels and form different topologies. This example forms a good exercise for such adjustments. Filter specifications are as follows:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=5, one FTZ at fz=85 MHz,  RS=RL=50

Minimum Inductor Configuration:

- Design steps are shown in Figure 6.47. Option Type A is selected. The transmission zeros are extracted in such a way that a close to symmetric structure will result. For this reason we extract the single FTZ and the single transmission zero at f=0 near the central part of the circuit: Inf-Inf-Inf-FTZ-zero-Inf-Inf. The resulting circuit is shown in Figure 6.47.a. Unfortunately it is still far from being symmetric. This is because of the existence of the series inductor on the left of the FTZ element. It has no symmetric counterpart. Since we don’t want to increase the number of inductors it will not be possible to make the circuit fully symmetric. However we should try to make the circuit as much symmetric as possible. This will also help to eliminate the transformer at load end while adjusting the element values. Let’s assume that we are asked to make all the inductors 50 nH and also eliminate the transformer.

- Let’s start the adjustment of inductors from left end by inserting inverters through the command Inverters / Scale Circuit By Inserting Inverters / Specify Inductor: L=50 nH (Figure 6.47.b). This command makes the leftmost series inductor 50 nH.

- Then the inverters are replaced by capacitive-L sections using the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L.  This command will bring negative series capacitors. In order to absorb them we have to bring positive capacitors in series with the inductor and negative capacitor. This is possible by converting the LRight and LLeft sections shown in the figure into Tee sections by applying the commands Pi-Tee-L / LRight (LLeft)-to-Tee / Symmetric Tee as shown in Figure 6.47.c.

- The resulting transformers are moved away by pressing the keys CTRL+W.

- Then using Exchange and Combine commands the negative capacitors are absorbed into the positive ones (Figure 6.47.d-e).

- Next, the 51 nH series inductor is scaled to 50 nH by applying Pi-Tee-L / Tee-to-Tee / Specify TRF command to the Tee-section of capacitors on left (Figure 6.47.e-f). The resulting transformer is moved away by CTRL+W command.

- Now we will make preparations to adjust the 4.7 nH of the FTZ element to 50 nH. We have to form a capacitive L-section on the left of the FTZ element. For this purpose we Exchange the positions of the series capacitor and series inductor on the left of FTZ element to make the capacitor adjacent to FTZ element. Also with cursor under the FTZ element the Exchange command is applied to bring the shunt capacitor on the right of FTZ element to its left. Thus, we have formed an LLeft-section on the left of the FTZ element (Figure 6.47.h).

- Now the 4.7 nH inductance of the FTZ element can be scaled to 50 nH by applying the command Pi-Tee-L / LLeft-to-Tee / Specify TRF to the LLeft-section (Figure 6.47.h-i).

- Finally, the rightmost series inductor is scaled to 50 nH by applying the command Pi-Tee-L / LLeft-to-Tee / Specify TRF to the LLeft-section on the left of this inductor (Figure 6.47.i-k).

- Combining all transformers we see that they ended up in unity, thus deleted (Luck?). So, we have reached the target. All inductors are 50 nH and we don’t have any transformer.

- Further trimming can be done for example to adjust capacitance values. This is possible by applying Pi-Tee-L transformations to the L-sections and Tee-sections.  As an example the Tee-section near the load can be converted to Pi-section by applying Pi-Tee-L / Tee-to-Pi command giving more reasonable values for capacitors.  Further adjustments of capacitors may necessitate changes in the inductor values (Exercise?).

Exercise: Start the Synthesis by Type-B option and search if it possible to form a symmetric structure without increasing the number of inductors. Then apply Pi-Tee-L / Equal Shunt Inductor command to form the direct coupled resonator version of the filter.

Figure 6.47. A degree eight filter with Nzero=1, Ninf=5 and a single FTZ in lower stopband.

Figure 6.47. A degree eight filter with Nzero=1, Ninf=5 and a single FTZ in lower stopband (cont’d).

EXAMPLE 6. 37. A degree eight filter with Nzero=5, Ninf=1 and one FTZ’s in lower stopband

Let’s now see the effects of selecting five transmission zeros at f=0 and keeping the FTZ in lower stopband. Filter specifications are as follows:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=5, Ninf=1, one FTZ at fz=85 MHz,  RS=RL=50

Minimum Inductor Configuration:

- Design steps are shown in Figure 6.48. Option Type A is selected. The transmission zeros are extracted in such a way that a close to symmetric structure will result. For this reason we extract the single FTZ and the single transmission zero at f=0 near the central part of the circuit: zero-zero-Inf-FTZ-zero-zero-zero. The resulting circuit is shown in Figure 6.48.a. Unfortunately it is still far from being symmetric. This is because of the existence of the series inductor on the left of the FTZ element. It has no symmetric counterpart. Since we don’t want to increase the number of inductors it will not be possible to make the circuit fully symmetric. However we should try to make the circuit as much symmetric as possible while also eliminating the transformer at load end. Let’s assume that initially we are allowed to make all the inductors between 10 nH-50 nH. After the design is completed we can further adjust the element values.

- Let’s start the adjustment of inductors from left end by inserting inverters through the command Inverters / Scale Circuit By Inserting Inverters / Specify Capacitor: C=50 pF (Figure 6.48.b). In this command we can only specify the capacitor adjacent to source. This makes the leftmost shunt inductor 51 nH. In the circuit we have many series capacitors but no shunt capacitor to form L-sections. We create shunt capacitors by replacing the inverters with Capacitive-L sections using the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L. The negative series capacitors are absorbed in the adjacent positive series capacitors (Figure 6.48.c-d).

- We will move the leftmost shunt capacitor into the circuit. This is possible by applying the command Pi-Tee-L / LRight-to-Pi / Specify TRF: n= 1.3. Here n=1.3 is a trial value of the first round of iterative design, adjusted not to decrease the shunt inductor too much (Figure 6.48.d-e). The resulting transformer is moved away by pressing the keys CTRL+W.

- Then using Exchange commands the shunt capacitor is moved to the right of the leftmost shunt inductor (Figure 6.48.e-f).

- Thus, we have formed an LRight-section on the left of the 2557 nH series inductor. It can be scaled down, for example to 50 nH by applying the command Pi-Tee-L / LRight-to Tee / Specify TRF: 7.15 (Figure 6.48.g). The right series arm capacitor of the Tee-section is moved to the right of the 50 nH series inductor using Exchange command as a preparation for scaling the shunt inductor of the FTZ element (Figure 6.48.g-h).

- Also as a preparation for formation of new L-sections, the rightmost shunt capacitor is moved into the circuit by applying LLeft-to-LRight command (Figure 6.48.f-g). Then using Exchange command the shunt capacitor is moved to the left of the adjacent shunt inductor (Figure 6.48.g-h).

- The shunt capacitor is then moved further left by applying Pi-Tee-L / LLeft-to-LRight command once more (Figure 6.48.i). It is then split into two and one piece is moved to the left of the FTZ element to form an LLeft-section there (Figure 6.48.k).

- Next, the 4.65 nH inductor of the FTZ element is scaled to 50 nH by applying Pi-Tee-L / LLeft-to-Pi / Specify TRF command to the LLeft-section of capacitors on the left of the FTZ element (Figure 6.48.k-m). The resulting transformer is moved away by CTRL+W command.

- The final operation must both eliminate the transformer at load end and also adjust the shunt inductor. Here, the priority is elimination of the transformer. This is done by applying the command Pi-Tee-L / LRight-to-Pi / Specify TRF to the LRight-section on the left of the shunt inductor. Here we  specify the inverse of turns ratio of the load end transformer. The inductor is scaled down to 11 nH. If this is too low, then we can trim all element values by using Pi-Tee-L transformations on all Pi, Tee and L-sections (Exercise).

Exercise: Extract the transmission zeros in the order zero-zero-FTZ-inf-zero-zero-zero and manipulate the circuit again to see if any advantage is gained.

Exercise: Start the Synthesis by Type-B option and search if it possible to form a symmetric structure without increasing the number of inductors. Then apply Pi-Tee-L / Equal Shunt Inductor command to form the direct coupled resonator version of the filter.

Figure 6.48. A degree eight filter with Nzero=5, Ninf=1 and a single FTZ in lower stopband.

Figure 6.48. A degree eight filter with Nzero=5, Ninf=1 and a single FTZ in lower stopband (cont’d).

EXAMPLE 6. 38. A degree eight filter with Nzero=3, Ninf=3 and one FTZ’s in lower stopband

Let’s now see the effects of selecting three transmission zeros at both f=0 and f=. Filter specifications are:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=3, Ninf=3, one FTZ at fz=85 MHz,  RS=RL=50

Minimum Inductor Configuration:

- Design steps are shown in Figure 6.49. Option Type A is selected. The transmission zeros are extracted in the order  inf-zero-Inf-FTZ-zero-zero-inf. The resulting circuit is shown in Figure 6.49.a.

- Let’s start the adjustment of inductors from left end by inserting inverters through the command Inverters / Scale Circuit By Inserting Inverters / Specify Inductor: L=50 nH (Figure 6.49.b).

- Then the inverters are replaced by their capacitive-L section equivalents using the command  Inverters / Replace Inverter By / End Inverter to / Capacitive-L. The negative series capacitors are absorbed in the adjacent positive series capacitors using Exchange and Combine commands (Figure 6.49.c-d).

- Next, the 0.58 nH shunt inductor is scaled by applying the command Pi-Tee-L / LLeft-to-Pi / Specify TRF: n=0.108. Here n=0.108 is found as the square root of 0.58/50 (Figure 6.49.d-e). The resulting transformer is moved away by pressing the keys CTRL+W.

- Then using Exchange commands the shunt capacitor is moved to the right of the new 50 nH shunt inductor to form an LRight-section to scale the inductance of the FTZ element  (Figure 6.49.e-f).

- The 8832 nH inductor of the FTZ element is scaled by applying the command Pi-Tee-L / LRight-to-Pi-Tee-L / Specify TRF: n=13.09 (Figure 6.49.g).

- The final operation must both eliminate the transformer at load end and also adjust the rightmost series inductor. Here, the the elimination of the transformer has the priority. This is done by first forming an LRight section using Exchange command as shown in Figure 6.49.g-h and then applying the command Pi-Tee-L / LRight-to-Pi / Specify TRF: n=1/0.41 where 0.41 is the transformer turns ratio at load end. Figure 6.49.i is the final circuit where the rightmost series inductor has also got about 50 nH inductance (luck?).

Exercise: Extract the transmission zeros in the order zero-zero-FTZ-inf-zero-zero-zero and manipulate the circuit again to see if any advantage is gained.

Exercise: Start the Synthesis by Type-B option and search if it possible to form a symmetric stucture without increasing the number of inductors. Then apply Pi-Tee-L / Equal Shunt Inductor command to form the direct coupled resonator version of the filter.

Figure 6.49. A degree eight filter with Nzero=3, Ninf=3 and a single FTZ in lower stopband.

Figure 6.49. A degree eight filter with Nzero=3, Ninf=3 and a single FTZ in lower stopband (cont’d).

EXAMPLE 6. 39. A degree eight filter with Nzero=5, Ninf=1 and one FTZ’s in upper stopband

Let’s now consider a degree eight filter a single FTZ in upper stopband. Filter specifications are:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=5, Ninf=1, one FTZ at fz=115 MHz,  RS=RL=50

Minimum Inductor Configuration:

- Design steps are shown in Figure 6.50. Option Type A is selected. The transmission zeros are extracted in the order zero-zero-Inf-FTZ-zero-zero-zero giving the circuit of Figure 6.50.a. The first thing we notice in this circuit is that the FTZ section is a shunt capacitor followed by a series arm parallel LC circuit. We prefer this type because it has a single inductance. The other possibility would be a series inductor followed by shunt arm series LC which has two inductors, hence not preferred in minimum inductor configurations. Only these two configurations are obtained when the FTZ is in upper stopband.

- Let’s assume that initially we are allowed to make all the inductors between 10 nH-50 nH. After the design is completed we can further adjust the element values. Let’s start the adjustment of inductors from left end by inserting inverters through the command Inverters / Scale Circuit By Inserting Inverters / Specify Capacitor: C=50 pF (Figure 6.50.b). In this command we can only specify the capacitor adjacent to source. This makes the leftmost shunt inductor 51 nH. In the circuit we have many series capacitors but not enough shunt capacitors to form L-sections. We create extra shunt capacitors by replacing the inverters with Capacitive-L sections using the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L. The negative series capacitors are absorbed in the adjacent positive series capacitors (Figure 6.50.c-d). Also using Exchange command the series capacitor on the right of the FTZ section is moved to one step left as a preparation to form L-section.

- We will move the leftmost shunt capacitor into the circuit. This is possible by applying the command Pi-Tee-L / LRight-to-Pi / Symmetric Pi (or Specify TRF). This makes the shunt inductor about 24 nH (Figure 6.50.d-e). Also the LRight-section on the left of FTZ section is converted into symmetric Tee-section. The resulting transformer is moved away by pressing the keys CTRL+W.

- Then using Exchange commands the shunt capacitor adjacent to the leftmost shunt inductor is moved to its right. Also the series capacitor of the Tee-section is moved one step left to form an LRight-section for adjustment of the series inductor (Figure 6.50.e-f).

- The 2095 nH series inductor is then scaled by converting the LRight-section on its left into symmetric Tee-section. It came out to be about 47 nH. (Figure 6.86.f-g).

- The right series arm capacitor of the Tee-section is moved to the right of the 50 nH series inductor using Exchange command as a preparation for scaling inductor of the FTZ element. A Tee-section is formed on the left of the FTZ element (Figure 6.50.g-h).

- The inductor of the FTZ element is scaled up to 51 nH by applying the command Pi-Tee-L / Tee-to-Tee / Specify TRf (Figure 6.50.h-i).

- Finally, the LLeft-section at load end is converted into Pi-section to create a transformer to eliminate the load end transformer giving the circuit of Figure 6.50.k. The rightmost shunt inductor came out to be about 12 ohms. If this is too low, then we can trim all element values by using Pi-Tee-L transformations on all Pi, Tee and L-sections (Exercise).

Exercise: Extract the transmission zeros in the order zero-zero-FTZ-inf-zero-zero-zero and manipulate the circuit again to see if any advantage is gained.

Exercise: Try different extraction orders to form shunt type FTZ section. Compare the number of inductors with that of the above example.

Exercise: Start the Synthesis by Type-B option and search if it possible to form a symmetric stucture without increasing the number of inductors. Then apply Pi-Tee-L / Equal Shunt Inductor command to form the direct coupled resonator version of the filter.

Figure 6.50. A degree eight filter with Nzero=5, Ninf=1 and a single FTZ in upper stopband.

Figure 6.50. A degree eight filter with Nzero=5, Ninf=1 and a single FTZ in upper stopband (cont’d).

EXAMPLE 6. 40. A degree eight filter with Nzero=3, Ninf=3 and one FTZ’s in upper stopband

Let’s now consider the effects of selecting Nzero=3 and Ninf=3 with a single FTZ in upper stopband. Filter specifications are:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=3, Ninf=3, one FTZ at fz=115 MHz,  RS=RL=50

Minimum Inductor Configuration:

- Design steps are shown in Figure 6.51. Option Type A is selected. The transmission zeros are extracted in the order inf-zero-FTZ-inf-zero-zero-inf giving the circuit of Figure 6.51.a. The extraction order is selected such that the FTZ section comes out as a shunt capacitor followed by a series arm parallel LC circuit. We prefer this type because it has a single inductance. The other possibility would be a series inductor followed by shunt arm series LC which has two inductors, hence not preferred in minimum inductor configurations. These are the prototypes when the FTZ is in upper stopband.

- Let’s start the adjustment of inductors from left end by inserting inverters through the command Inverters / Scale Circuit By Inserting Inverters / Specify Inductor: L=50 nH (Figure 6.51.b).

- Then inverters are replaced by Capacitive-L sections using the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L (Figure 6.51.c).

- The negative series capacitors are absorbed in the adjacent positive series capacitors by using Exchange and Combine commands (Figure 6.51.c-d).

- Next, the inductor of the FTZ element is adjusted to 50 nH by applying the command Pi-Tee-L / LLeft-to-Tee / Specify TRF to the LLeft-section on the left of the FTZ element (Figure 6.51.d-e).

- Then the 6.08 nH shunt inductor is scaled to  50 nH by first forming an LLeft-section on its left and then applying the command Pi-Tee-L / LLeft-to-Pi / Specify TRF (Figure 6.51.e-f-g).

- Finally, in order to eliminate the load end transformer and also to scale down the series inductor there an LRight-section is formed using Exchange command and the command Pi-Tee-L / LRight-to-Pi-Tee-L / Specify TRF: n=1/0.1082 is applied. Here 0.1082 is the turns ratio of the transformer to be eliminated (Figure 6.51.h). We notice that while eliminating the transformer the series inductor is also scaled down to 50 nH. Interesting?.

Exercise: Try different extraction orders to form shunt type FTZ section. Compare the number of inductors with that of the above example.

Exercise: Start the Synthesis by Type-B option and search if it possible to form a symmetric stucture without increasing the number of inductors. Then apply Pi-Tee-L / Equal Shunt Inductor command to form the direct coupled resonator version of the filter.

Exercise: Repeat the design for Nzero=1, Ninf=5.

Figure 6.51. A degree eight filter with Nzero=3, Ninf=3 and a single FTZ in upper stopband.

Figure 6.51. A degree eight filter with Nzero=3, Ninf=3 and a single FTZ in upper stopband (cont’d).

EXAMPLE 6. 41. A degree eight filter with Nzero=1, Ninf=3 and two FTZ’s in lower stopband

Let’s make a design having two FTZ’s in lower stopband with Nzero=1 and Ninf=3. Filter specifications are:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=3, one FTZ at fz1=77 MHz, fz2=85 MHz, RS=RL=50

Minimum Inductor Configuration:

- Design steps are shown in Figure 6.52. Option Type A is selected. The transmission zeros are extracted in the order inf-FTZ1-inf-FTZ2-zero-inf giving the circuit of Figure 6.52.a. The extraction order is selected such that the FTZ sections come out in shunt form. We prefer this type because it has a single inductance. The other possibility would be a shunt inductor followed by series arm parallel LC section which has two inductors, hence not preferred in minimum inductor configurations.

- Let’s start the adjustment of inductors from left end by inserting inverters through the command Inverters / Scale Circuit By Inserting Inverters / Specify Inductor: L=50 nH (Figure 6.52.b).

- Then inverters are replaced by Capacitive-L sections using the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L (Figure 6.52.c).

- The negative series capacitors are absorbed in the adjacent positive series capacitors by using Exchange and Combine commands (Figure 6.52.c-d).

- Next, the 3.6877 nH inductor of the first FTZ element is adjusted to 50 nH by first moving the shunt capacitor on its right to left and then applying the command Pi-Tee-L / Lleft-to-Pi / Specify TRF to the LLeft-section on the left of the FTZ element (Figure 6.52.d-e-f).

- Then, the 1417.3 nH inductor of the second FTZ element is adjusted to 50 nH by first moving the shunt capacitor on the left of the first FTZ element to right (thus forming an LRight-section) and then applying the command Pi-Tee-L / Lleft-to-Pi / Specify TRF to the LRight-section (Figure 6.52.f-g-h).

- Finally, in order to eliminate the load end transformer and also to scale down the series inductor there an LRight-section is formed using Exchange command and the command Pi-Tee-L / LRight-to-Pi-Tee-L / Specify TRF: n=1/0.4169 is applied on the LRight-section. Here 0.4169 is the turns ratio of the transformer to be eliminated (Figure 6.52.h-i-k). We notice that while eliminating the transformer the series inductor is also scaled down to 50 nH.

Exercise: Try different extraction orders to form shunt type FTZ section. Compare the number of inductors with that of the above example.

Exercise: Start the Synthesis by Type-B option and search if it possible to form a symmetric stucture without increasing the number of inductors. Then apply Pi-Tee-L / Equal Shunt Inductor command to form the direct coupled resonator version of the filter.

Exercise: Repeat the design for Nzero=3, Ninf=1.

Figure 6.52. A degree eight filter with Nzero=1, Ninf=3 and two FTZ’s in lower stopband.

Figure 6.52. A degree eight filter with Nzero=1, Ninf=3 and two FTZ’s in lower stopband (cont’d).

EXAMPLE 6. 42. A degree eight filter with Nzero=3, Ninf=1 and two FTZ’s, one in each stopband

Let’s now make a design having two FTZ’s, one in each stopband, with Nzero=1 and Ninf=3. Filter specifications are:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=3, Ninf=1, one FTZ at fz1=85 MHz, fz2=115 MHz, RS=RL=50

Minimum Inductor Configuration:

- Design steps are shown in Figure 6.53. Option Type B is selected because Type A leads to more inductors (exercise?). The transmission zeros are extracted in the order zero-FTZ1-zero-FTZ2-inf-zero giving the circuit of Figure 6.53.a. The extraction order is selected such that the FTZ sections come out in minimum inductor forms. That is the FTZ in lower stopband in the form of a series capacitor with shunt LC and the FTZ in upper stopband in shunt capacitor with series arm LC.

- Let’s start the adjustment of inductors from left end by inserting inverters through the command Inverters / Scale Circuit By Inserting Inverters / Specify Inductor: L=50 nH (Figure 6.53.b).

- Then inverters are replaced by Capacitive-L sections using the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L (Figure 6.53.c).

- The negative shunt capacitors must be absorbed by positive neighbors. On load side we have a positive shunt capacitor and it is used to absorb the negative one by using Exchange and Combine command as described in Figures 6.91.c-d. However on the source side there is no positive shunt capacitor. Therefore we have to create it. This is possible as described in Figures 6.91.c-d-e-f-g by applying Pi-Tee-L transformations.

- Next, the 146.04 nH inductor of the first FTZ element is adjusted to 50 nH by applying the command Pi-Tee-L / Pi-to-Pi / Specify TRF to the Pi-section on the left of the FTZ element (Figure 6.53.g-h).

- Next, we would like to adjust the 2.66 nH inductor of the second FTZ element to 50 nH by using the LLeft-section on its left. This action necessitates a transformer turns ratio of SQRT(2.66/50)=0.231 However this LLeft-section can’t create this turns ratio. The minimum it can produce is about 0.3 by applying the transformation Pi-Tee-L / LLeft-to-LRight. Therefore, for the time being we accept this result which scales the inductance to about 28 nH (Figure 6.53.i). If a higher inductance is a must then we have to go back and readjust the inductance of the first FTZ element so as to create different L-sections and repeat the trials.

- Finally, in order to eliminate the load end transformer and also to scale down the series inductor there an LRight-section is formed using Exchange command and the command Pi-Tee-L / LRight-to-Pi-Tee-L / Specify TRF: n=1/2.73 is applied on the LRight-section. Here 2.73 is the turns ratio of the transformer to be eliminated (Figure 6.53.k). We notice that while eliminating the transformer the series inductor is also scaled down to 50 nH.

Exercise: Try different extraction orders to form shunt type FTZ section. Compare the number of inductors with that of the above example.

Exercise: Start the Synthesis by Type-B option and search if it possible to form a symmetric stucture without increasing the number of inductors. Then apply Pi-Tee-L / Equal Shunt Inductor command to form the direct coupled resonator version of the filter.

Exercise: Repeat the design for Nzero=1, Ninf=3.

Figure 6.53. A degree eight filter with Nzero=3, Ninf=1 and two FTZ’s, one in each stopband.

Figure 6.53. A degree eight filter with Nzero=3, Ninf=1 and two FTZ’s, one in each stopband (cont’d).

EXAMPLE 6. 43. A degree eight filter with Nzero=1, Ninf=1 and three FTZ’s in lower stopband

Let’s now make a design having three FTZ’s in lower stopband, with Nzero=1 and Ninf=1. Filter specifications are:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=1, FTZ’s at at fz1=62.2 MHz, fz2=80 MHz, fz3=85 MHz, RS=RL=50

Coupled Resonator Configurations:

- Maximum selectivity requirement necessitates the use of maximum number of FTZ’s. With a degree N filter, the maximum number of FTZ’s is (N-2)/2. That is, only one transmission zero is left at both f=0 and f=. This leaves not much flexibility for adjusting the element values individually. That is, no capacitive or inductive L-sections can be formed for application of Pi-Tee-L transformations without increasing the number of inductors.  If the Minimum Inductor Configuration comes out to be unrealizable then the only way to get realizable values is to increase the number of inductors also.

- For N=8 filter, the maximum number of FTZ’s is NFTZ=3. Let’s see what type of solutions we will get. Design steps are shown in Figure 6.54. Option Type A is selected to get minimum inductor configuration. The transmission zeros are extracted in the order FTZ1-FTZ2-FTZ3-zero-inf giving the circuit of Figure 6.54.a. The circuit is in the form of shunt FTZ elements coupled by series capacitors. It is indeed in minimum inductor form, but element value spread is extreme, hence the circuit is unrealizable in this form.

- One way would be to place the cursor under each shunt FTZ element and apply the command Norton / Specify Transformer.  This would convert all series elements into series LC resonators, some with negative elements. It is possible to make all elements positive by further adjustment of the transformer turns ratio of Norton transformations. However this is extremely tedious and a simpler version is programmed into Filpro as follows.

- Find Dual command is applied to convert the structure into series FTZ elements coupled by shunt inductors (Figure 6.54.b).

- Then the command Pi-Tee-L / Equal Shunt Inductors (or Equal Shunt Capacitors) is applied to convert the circuit into direct coupled resonator form with coupling elements being parallel LC elements (Figure 6.54.c). This is one of the possible realizable forms. It can be scaled by inserting inverters as described in the previous examples.

- Applying Find Dual command again we get the series resonator form coupled by shunt arm series LC elements. Note that all series capacitors are equal. If we had selected the command Pi-Tee-L / Equal Shunt Capacitors then we would get equal series inductor version. This circuit may also be scaled by inverters as was done in the previous examples.

- Due to inflexibilities of such circuits we usually prefer to use either Nzero=1, Ninf=3 or Nzero=1 , Ninf=1, as studied in the previous examples.

Exercise: Repeat the design by placing all three FTZ’s in at the same frequency and compare the element value spread.

Exercise: Repeat the design for all three FTZ’s in upper stopband.

Exercise: Repeat the design for all two FTZ’s in upper stopband and one in lower stopband.

Figure 6.54. A degree eight filter with Nzero=1, Ninf=1 and three FTZ’s in lower stopband.

6. 8. 4. DEGREE TEN AND HIGHER ORDER ELLIPTIC FILTERS

Degree ten or higher order filters are also treated in the same ways as the degree eight or degree six filter examples of the previous sections. Therefore only two examples will be presented.

EXAMPLE 6. 44. A degree ten filter with Nzero=1, Ninf=1 and four FTZ’s in lower stopband

Let’s design a filter having four FTZ’s, two in lower stopband and two in upper stopband with Nzero=1 and Ninf=1. Filter specifications are:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=1, FTZ’s at at fz1=fz2=85 MHz, fz2=fz3=115 MHz, RS=RL=50

Coupled Resonator Configurations:

- For N=10 filter, the maximum number of FTZ’s is NFTZ=4. Since only one transmission zero exists at both f=0 and f=, not much flexibility exists for individual adjustments of element values. Let’s see what type of solutions we will get. Design steps are shown in Figure 6.55. Option Type A is selected to get minimum inductor configuration. The transmission zeros are extracted in the order FTZ1-FTZ3-FTZ2-FTZ4-zero-inf giving the circuit of Figure 6.55.a. The circuit is in the form of shunt FTZ elements coupled by series capacitors. It is in minimum inductor form, but element value spread is extreme, hence the circuit is unrealizable in this form.

- One way would be to place the cursor under each shunt FTZ element and apply the command Norton / Specify Transformer.  This would convert all series elements into series LC resonators, some with negative elements. It is possible to make all elements positive by further adjustment of the transformer turns ratio of Norton transformations. However this is extremely tedious and a simpler version is programmed into Filpro as follows.

- Find Dual command is applied to convert the structure into series FTZ elements coupled by shunt inductors (Figure 6.55.b).

- Then the command Pi-Tee-L / Equal Shunt Inductors (or Equal Shunt Capacitors) is applied to convert the circuit into direct coupled resonator form with coupling elements being parallel LC elements (Figure 6.55.c). This is one of the possible realizable forms. It is saved as shtres81.fil to be used later.

- Applying Find Dual command again we get the series resonator form coupled by shunt arm series LC elements. Note that all series capacitors are equal. If we had selected the command Pi-Tee-L / Equal Shunt Capacitors then we would get equal series inductor version. This circuit is saved as serres91.fil.

- Let’s load shtres81.fil and see what transformations we can apply on this filter to get simpler versions (Figure 6.55.e). First, the shunt inductors are scaled to 50 nH by applying the command  Inverters / Scale Cct By Inserting Invs / Specify Ind: L=50 nH (Figure 6.55.f).

- Then with cursor under the first shunt resonator on left (node-1), the command Double Norton / Inverse Geffe is applied to eliminate the second shunt resonator. This command is repeated by placing the cursor under the third shunt resonator (node-3) to eliminate the fourth shunt resonator (Figure 6.55.g). Simplify Circuit command gives the circuit of Figure 6.55.h where only three shunt resonators are left. Thus, we have reduced the number of inductors by two. This command applies only if the transmission zeros of the adjacent FTZ elements (series arm LC elements) are located in lower and upper stopbands.  It doesn’t work when they are located in the same stopband.

- Further modifications can be done on this circuit by placing the cursor under the first shunt resonator on left and applying the command Double Norton / Geffe Equivalent / k1=2 / No Middle L or C. This command is the reverse of Double Norton / Inverse Geffe  because it is now attempting to reinsert the second shunt resonator back into its original position, but now we have several options. The option No Middle L or C places only a shunt capacitor at node-2 (if k1 is properly selected). This capacitor may be used to account for the stray capacitors of the series arm inductors to ground. This transformation doesn’t increase the number of inductors (Figure 6.55.i). In this command k1 is a user selected parameter which can be iterated to adjust element values. It is one of the two Norton Transformers that are applied in the Double Norton command. The second transformer (k2) is selected by Filpro in accordance with the selected option.   In this case we have arbitrarily selected k1=2.

- Simplify Circuit command moves the resulting transformer away (Figure 6.55.i-k).

- Then the same command is repeated by placing the cursor under the third shunt resonator, now by  selecting k1 more carefully in order to avoid creation of extreme transformers at load end. After a few trials k1=0.76 is found to give a transformer close to unity at load end (Figure 6.55.m). After inspecting the element values we may repeat the procedure by using different k1 values.

- As the final step the load end transformer is absorbed into the adjacent inverter and then the inverters are replaced by their capacitive-L section equivalents giving the circuit of Figure 6.55.n.

- The same type of transformations may also be applied on the dual circuit as follows.

- Load serres91.fil (Figure 6.55.p). First, the series inductor at source end is scaled to 50 nH by applying the command Inverters / Scale Cct By Inserting Invs / Specify Ind: L=50 nH (Figure 6.55.q).

- Then with cursor under the first series resonator on left, the command Double Norton / Inverse Geffe is applied to eliminate the second series resonator. This command is repeated by placing the cursor under the third series resonator to eliminate the fourth shunt resonator (Figure 6.55.r). Simplify Circuit command gives the circuit of Figure 6.55.s where only three series resonators are left. Thus, we have reduced the number of series inductors by two. This command applies only if the transmission zeros of the adjacent FTZ elements (series arm LC elements) are located in lower and upper stopbands.  It doesn’t work when they are located in the same stopband.

- Further modifications can be done on this circuit by placing the cursor under the first series resonator on left and applying the command Double Norton / Geffe Equivalent / k1=0.55 / No Middle L or C. This command is the reverse of Double Norton / Inverse Geffe because it is now attempting to reinsert the second series resonator back into its original position, but now we have several options. The option No Middle L or C places only a series inductor (or series capacitor for some values of k1) (Figure 6.55.t). In this command k1 is a user selected parameter which can be iterated to adjust element values. It is one of the two Norton Transformers that are applied in the Double Norton command. The second transformer (k2) is selected by Filpro in accordance with the selected option. In this case we have arbitrarily selected k1=0.55.

- Simplify Circuit command moves the resulting transformer away (Figure 6.55.t-u).

- Then the same command is repeated by placing the cursor under the third shunt resonator, now by  selecting k1 more carefully in order to avoid creation of extreme transformers at load end. After a randomly selected value of  k1=1.4 we obtain the circuit of Figure 6.55.v. After inspecting the element values we may repeat the procedure by using different k1 values.

- As the final step the load end transformer is absorbed into the adjacent inverter and then the inverters are replaced by their capacitive-L section equivalents giving the circuit of Figure 6.55.x. Note that we have got a negative series capacitor at load end. It is because the series capacitor of the filter couldn’t absorb the negative capacitor of the inverter. Therefore we have to repeat the design for different values of k1 (Exercise?).

- Other alternative structures can be obtained by combining the adjacent FTZ elements of the circuits of shtres82.fil and serres92.fil, as described in Figures 6.93.x and y. When two FTZ elements are combined we get fourth order sections. For narrowband filters element values of fourth order sections may take extreme values, hence not recommended. For some medium bandwidth filters this approach may be preferable (Exercise with different bandwidths).

Exercise: Repeat the design for wider bandwidth filters.

Figure 6.55. A degree ten filter with Nzero=1, Ninf=1 and four FTZ’s, two in each stopband.

Figure 6.55. A degree ten filter with Nzero=1, Ninf=1 and four FTZ’s, two in each stopband (cont’d).

Figure 6.55. A degree ten filter with Nzero=1, Ninf=1 and four FTZ’s, two in each stopband (cont’d).

Figure 6.55. A degree ten filter with Nzero=1, Ninf=1 and four FTZ’s, two in each stopband (cont’d).

EXAMPLE 6. 45. A degree twelve filter with Nzero=1, Ninf=3 and four FTZ’s in lower stopband

Let’s consider the previous filter, but by increasing the number of transmission zeros at f= to three:

PBRipple=0.01 dB, fp1=95 MHz, fp2=105 MHz, Nzero=1, Ninf=3, FTZ’s at at fz1=fz2=85 MHz, fz2=fz3=115 MHz, RS=RL=50

Minimum Inductor Configuration

Existence of more than one transmission zeros at f= enables us to design minimum inductor versions with flexibility of adjusting the individual inductor values.

- Design steps are shown in Figure 6.56. Option Type A is selected to get minimum inductor configuration. The transmission zeros are extracted in the order   inf-FTZ1-FTZ3-FTZ2-FTZ4-inf-zero-inf giving the circuit of Figure 6.56.a.

- Let’s target the inductor values less than about 50 nH. First the leftmost series inductor is scaled by inserting inverters using the command Inverters / Scale Cct By Ins Inverters / Specify Ind: L=50 nH (Figure 6.56.b).

- Then the inverters are replaced by Capacitive-L equivalents using the command Inverters / Replace Inv By / End Inv to / Capacitive-L (Figure 6.56.c).

- The negative capacitors are absorbed into the adjacent positive capacitors by using Exchange and Combine commands (Figure 6.56.c-d). The transformer is shifted rightmost by applying CTRL+W command.

- As a preparation for adjusting the next inductor (the inductor of the first FTZ element in shunt arm), an LLeft-section of capacitors is formed by moving the shunt capacitor on the right of the FTZ element to left using Exchange command (Figure 6.56.c-d).

- With cursor under the shunt leg of the LLeft-section the command Pi-Tee-L / LLeft-to-Pi / Specify TRF: n=SQRT (6.909/50) is applied to scale the 6.909 nH inductor of the FTZ element to 50 nH (Figure 6.56.d-e). Shifting the resulting transformers rightmost by applying CTRL+W command we see that all the other inner inductors are about 40 nH. Therefore they are not scaled. Only the rightmost inductor needs be scaled. It will be scaled while also eliminating the transformer at load end. This is done by applying the command Pi-Tee-L / LRight-to-Pi / Specify TRF: n=1/0.1098 where 0.1098 is the transformer turns ratio at load end.  We get the realizable circuit of Figure 6.56.e.

Exercise: Convert the Pi-section of capacitors into Tee-sections to see if better capacitor values can be obtained or not.

Exercise: Repeat the design for wider bandwidth filters.

Exercise: Repeat the design for Nzero=3 and Ninf=1 case to see if any advantage arises or not.

Exercise: Form the Direct Coupled Resonator versions of the filter.

Exercise: Repeat the design for less number of FTZ’s placed at different locations.

Exercise: Repeat the design for higher degree filters.

Figure 6.56. A degree twelve filter with Nzero=1, Ninf=3 and four FTZ’s, two in each stopband.

6. 8. 5. SYNTHESIS OF ANTIMETRIC ELLIPTIC FILTERS

Bandpass filters with even number of transmission zeros at zero (and at infinity) are termed as antimetric filters. Such filters start with series arm elements and end with shunt arm elements or vice versa. Let’s see an example together with some circuit transformations that can be applied to such filters.

EXAMPLE 6. 46. An antimetric elliptic filter

Let	PBRipple= 0.01 dB, fp1 = 95 MHz, fp2 =105 MHz, Nzero=2,  Ninf =2, two finite transmission zeros in lower stopband and two finite transmission zeros in upper stopband. Finite transmission zero positions will be adjusted so as to get equal stopband ripples in both stopbands (true elliptic response).

- Design command set is shown in Figure 6.57. Finite transmission zeros are adjusted using Tune FTZ command giving the response shown in the figure:

fz1= 82.5 MHz, fz2=87 MHz, fz3=115 MHz, fz4=121 MHz.

- The transmission zeros are extracted in the order inf-FTZ1-FTZ4-FTZ2-FTZ3-inf-Zero-zero.      This order of extraction gives the minimum inductor version. It is saved as antim1.fil. Antimetry shows itself in the type of elements at source and load ends where we have series elements and shunt elements respectively. That is we shouldn’t expect topological symmetry. This fact renders the usual inverter approach of scaling elements at source and load ends useless (Exercise?). However we have another approach which can be used to scale the elements at source and load ends independently of each other, as follows.

- Let’s target to make all inductors 50 nH or less with about 10 nH minimum. The 625.62 nH series inductor at source end can be scaled down to 50 nH by placing the cursor under the source resistance and applying the command Impedance Matching / R-Csr-to-R-Csh / Trf n= SQRT (625.62/50). This command creates a shunt capacitor adjacent to the source while also producing the specified transformer (Figure 6.57.b). Moving the transformer away we get the circuit of Figure 6.57.c where the series inductor is now 50 nH.

- Next, the 5 nH inductance of the first FTZ element will be scaled to 50 nH. This is done by first forming an LLeft-section of capacitors using Exchange command as shown in Figures 6.95.c-d. Then applying the command Pi-Tee-L/ LLeft-to-Pi/ Specify TRF gives the circuit of Figure 6.57.e.

- Next, the 107 nH inductor of the second FTZ element will be scaled down to 50 nH. This necessitates formation of an LRight-section as described in Figures 6.95.e-f. Then applying the command Pi-Tee-L / LRight-to-Tee / Specify TRF we get the circuit of Figure 6.57.g. In this command we have preferred to convert the LRight-section into Tee-section instead of Pi-section just to use the right series arm capacitor of the Tee-section in the next stage.

- Next, the 37.4 nH inductor of the third FTZ element and 24.84 nH of the fourth FTZ element will be scaled simultaneously because we don’t have enough elements to scale them independently of each other. Actually we need not scale them because they are within the specified inductance limits. Just as exercise let’s scale up the 37.4 nH inductance of the third FTZ section to 50 nH. This is done again by first forming an LLeft-section and then applying the command Pi-Tee-L / LLeft-to-Pi / Specify TRF (Figure 6.57.g-h-i). Inductance of the fourth FTZ element is scaled to about 32 nH.

- Finally, the transformer at load end is eliminated by applying the command Pi-Tee-L / LRight-to-Pi / Specify TRF: n=1/0.0542 where 0.0542 is the turns ratio of the transformer. This transformation scales the 3449 nH shunt inductor at load end to 10.15 nH.

Exercise: Try to decrease the element value range of the inductors by assigning different inductances.

Figure 6.57. An antimetric filter with Nzero=2, Ninf=2 and four FTZ’s, two in each stopband.

Figure 6.57. An antimetric filter with Nzero=2, Ninf=2 and four FTZ’s, two in each stopband (cont’d).

EXAMPLE 6. 47: On parametric filters and brute force reduction of number of inductors:

In filters with finite transmission zeros, the true minimum number of inductors can only be obtained by  special design approaches termed as parametric filters, having transmission zeros on real axis of s=+jw plane also 9, 18. These approaches usually lead to unequal ripple responses. One true equal ripple response technique is presented in [25]. Such special techniques are left outside the scope of the present version of Filpro. However, if desired, we may use brute force approach followed by a slight optimization to reduce the number of inductors, especially in narrowband filters. Consider for example the antimetric filter (antim1.fil) of the previous example repeated in Figure 6.58.a.

Since the filter is narrowband, we may try the following approximation:

- Using Pi-Tee-L transformations let’s eliminate the transformer at load end (Figure 6.58.a-b-c-d).

- The series LC section at input and shunt LC section at output ports resonate at frequencies close to the passband center, 100 MHz, presenting short and open circuit respectively. Thus, they are ineffective at band center. Let’s delete them to reduce the number of inductors. The resulting circuit and response are shown in Figure 6.58.e and f.  Indeed return loss response didn’t changed at band center, but degraded towards band edges.

- Optimization of return loss response using Best Case command gave the circuit of Figure 6.58.g with response shown in Figure 6.58.h. Return loss level became equiripple at about 12 db which is a big decrease from the original level of 26 dB. This sems the price paid for reduction in the number of inductors.

- The similarity of resulting structure to parametric (or zigzag) filters should be noted 18, p.89.

Figure 6.58. Brute force reduction of number of inductors.

Figure 6.58. Brute force reduction of number of inductors.

6. 8. 6. COLIN AND HUMPHERYS TRANSFORMATIONS

- Colin transformations ([2], [8, p.565]) can be used either to convert a series arm FTZ element (parallel resonator in series arm) into shunt arm FTZ element (shunt arm series resonator) and vice versa or to reverse the sense of L-section formed by resonators as described in Figure 6.59.a. These transformations are obtained by using the Norton equivalents of both resonators (hence the name Double Norton). They all produce a negative capacitor at one end. Therefore these transformations are applicable only if these negative capacitors can be absorbed by a positive neighboring capacitor.

- A handier form of Colin transformation applied twice to the adjacent sections is described by    
Humpherys [8, p.566]. It is termed as Humpherys transformation in Filpro placed under the menu item Double Norton and is shown in Figure 6.59.b. This transformation is useful in reducing the number of inductors of bandpass filters obtained by the classical mapping from LP prototypes. If the bandpass filter at hand can be designed by Synthesis approach of Filpro, then these transformations are not necessary because using Synthesis approach we can readily design the minimum inductor versions while the bandpass filters obtained from LP prototypes necessitate reduction of the number of inductors.

- Let’s see their applications in the examples designed by User Defined Prototype approach discussed in the next section.

- Figure 6.59. Colin and Humpherys transformations.

6.  8. 7. USER  DEFINED LOWPASS  PROTOTYPES  USING  SYNTHESIS APPROACH

Design / By User Defined Prototype approach is prepared basically for filters whose lowpass prototypes are not available in built-in library of Filpro and which can’t be designed directly as bandpass filter using Filpro. A lowpass prototype that can be found elsewhere (textbooks, handbooks, and other literature) can be placed into Filpro using Insert command and saved. Then, the Design By User Defied Prototype approach is used to design the targeted filter using this prototype. Some lowpass prototypes which are not available in built-in library of Filpro may also be designed using Synthesis approach and saved.

The following points should be taken into account in developing a lowpass prototype filter:

- Prototype filter must have passband edge frequency at wp=1 which correspond to fp=0.159.

- Although frequency is stated as MHz, it should be treated as normalized frequency.

- Source and load impedances must be set as unity (normalized terminations).

- Any transformer that may result from the synthesis should be absorbed in the load impedance RL using Shift Trf / Absorb Trf in RL command.

- After synthesizing the prototype filter, it will be saved in the same way as any other design.

- After designing a filter using this prototype, units may be reset properly.

An example will be following to illustrate both preparation of a lowpass prototype using Synthesis approach and an application of User Defined Prototype approach.

EXAMPLE 6. 48. Preparation of an antimetric elliptic lowpass prototype

Let’s prepare an antimetric elliptic lowpass prototype with even order which is not in the built in library of Filpro. The lowpass prototype filter has the following specifications:

PBRipple=0.01 dB  and SBLoss=40 dB and degree N=6, with Ninf=2 and two finite transmission zeros tuned to a proper frequency to give 40 dB stopband loss.

- The command set is shown in Figure 6.60.

New / Design / By Synthesis / Filter Type: Lumped, Lowpass, Equiripple, Doubly Terminated,       Type  A

- At this step, Type B may as well be chosen to get the dual structure starting with a shunt element.

Passband and Terminations: PBRipple=0.01 dB, fp =0.159, RS=1, RL=1

Specify Transmission Zeros: Inf, Inf, fz1=0.18, fz2=0.18

- Here, initially we have placed the two FTZ’s at an arbitrary frequency, and then tuned to get the required 40 dB stopband loss using Tune FTZ command. 40 dB stopband loss is obtained for finite transmission zeros at fz1=fz2=0.26.

- Find Circuit command gives the circuit as shown in Figure 6.60.a.

- The transformer at load end is absorbed into load resistance RL using command Shift Transformer / Absorb Transformer in RL. One LP prototype is ready (Figure 6.60.b). Let’s save it as  prot11.fil.

- Applying Find Dual command the dual prototype can be formed (Figure 6.60.c).

- Both prototypes can be used for designing any LP, HP, BS or BP filters. Let’s see a bandpass filter design example.

Warning: The element values of LP prototype are dimensionless. In Synthesis procedures with RS=1 the units of inductors and capacitors are automatically set as micro Henry and microfarads.

Figure 6.60. Preparation of lowpass prototype filters.

EXAMPLE 6. 49. Antimetric bandpass filter design through User Defined Prototype approach

Let’s design a bandpass filter with passband edges at fp1=90 MHz and fp2=110 MHz, using the antimetric elliptic prototype prepared above.

- Command set is given in Figure 6.61. The resulting bandpass filter circuit is shown in Figure 6.61.a. This is one of the many possible solutions. Let’s save it as antim11.fil.

- Applying Find Dual command we get the dual circuit which may also form another solution (Figure 6.61.b. Let’s save it as antim21.fil.

- Figure 6.61.c shows the insertion loss response. It is seen that the single FTZ of the LP prototype is mapped as two geometrically symmetric FTZ’s  (fz1*fz2=f02 ).

- These two solutions can be used as starting points for developing a variety of different solutions. One of them is described below.

Minimum Inductor Version (Humpherys transformations) :

If the number of inductors is to be minimized, then the circuit transformation due to Humherys [9] can be used. On Filpro they are named as Humpherys Transformations and described in Figure 6.60.b. Each application of this transformation reduces the number of inductors by one. This transformation is applied on a pair of adjacent series FTZ elements in between two shunt resonators. In the resulting circuit one of the series FTZ elements is converted into shunt FTZ element accompanied a reduction of the number of inductors by one. Depending on the locations of the adjacent FTZ elements, one or more of the capacitors of the resulting circuit will come out to be negative. If the adjacent FTZ elements are correctly located in the circuit then only the series capacitor at one end of the circuit section will come out to be negative and it can be absorbed into a positive capacitor of the neighboring circuit section. If internal capacitors come out to be negative then we should Exchange the adjacent FTZ elements and apply the transformation again. The procedure is  applied as follows.

- The circuit of antim11.fil is loaded (Figure 6.62.a).

- The fourth order sections are split into two second order sections (series FTZ elements) by placing the cursor under the fourth order sections and applying Split command (Figure 6.62.b).

- With cursor under the second series arm FTZ element the command Double Norton/ Humpherys-Left is applied. This transformation converts the first series arm FTZ element into a shunt arm FTZ element with a negative series capacitor on left thus reducing the number of inductors by one as shown in Figure 6.62.c.

- The negative series capacitor is absorbed into the adjacent positive capacitor by application of Simplify Circuit command, giving the circuit of Figure 6.62.d. Note that the number of inductors is reduced by one.

- This transformation is repeated on the next section, but resulting in negative internal capacitors. Therefore we repeat the command by exchanging the positions of the FTZ elements giving the circuit of Figure 6.62.e. Simplify Circuit command combines the negative series capacitor with the adjacent positive series capacitor giving the circuit of Figure 6.62.f.

- The resulting circuit can be scaled and element values can be adjusted as described in the previous example. Let’s adjust the inductors as usual.

- The 318.85 nH series inductor at source end can be scaled down to 50 nH by placing the cursor under RS and applying the command Impedance Matching / R-Csr-to-R-Csh / TRF: n=SQRT (318.85/50). The transformer is moved away by applying CTRL+W command (Figure 6.62.f-g).

Figure 6.61. Command set for designing bandpass filters from a user defined lowpass prototype.

- Next, the shunt 40.843 nH of the shunt FTZ element and the 6.91 nH inductor of the series FTZ element will be scaled up simultaneously, because we don’t have enough number of capacitors to form different L-sections for scaling them individually. Let’s scale up the smaller one (6.91 nH) to 20 nH. This is done by forming an LLeft-section of capacitors using Exchange command as shown in Figure 6.63.g-h and then applying the command Pi-Tee-L / LLeft-to-Pi / Specify TRF: n=SQRT (6.91/20).  This will also scale up the 40.843 nH to about 108 nh  (Figure 6.62.i).

- The remaining inductors are about 100 nH, 17 nH and 13 nH. Let’s assume that they are realizable and concentrate on load resistance. From the lowpass prototype it was mapped as 55 ohms. Let’s convert it to 50 ohms. First by placing the cursor under the transformer the command Shift TRF / Absorb TRF in RL is applied converting the 55 ohms load resistor to 34.7 ohms (Figure 6.62.k).

- Next an LLeft-section of capacitors is formed by splitting the LC resonator at load end into its L and C components. Then the command Pi-Tee-L / LLeft-to-Pi / Specify TRF: n= SQRT (34.7/50) is applied to produce a transformer for scaling 34.7 ohms into 50 ohms. This transformer is then shifted to the load end by pressing the keys CTRL+W (Figure 6.62.m-n).

- With cursor under this transformer the command Shift TRF / Absorb TRF in RL is applied scaling RL to 50 ohms (Figure 6.62.p).

Exercise: Repeat the design for fp1=95 MHz, fp2=105 MHz.

Exercise: Repeat the design on the dual form (antim21.fil).

Figure 6.62. Minimum inductor version of the filter obtained by User Defined Prototype approach.

Figure 6.62. Minimum inductor version of the filter obtained by User Defined Prototype approach (cont’d).

EXAMPLE 6. 50. Colin Transformations on a symmetric bandpass filter

Just as an example for application of Colin transformation let’s design a symmetric bandpass filter with passband edges at fp1=90 MHz and fp2=110 MHz, using a symmetric lowpass protoype with PBRipple=0.01 dB, two FTZ’s both at fz=0.25 and Ninf=3.

- Preparation of the LP prototype is shown in Figure 6.63.a. The prototype is shown in Figure 6.63.b. It is saved as protsym.fil.

- The command set for the design of the bandpass filter is shown in Figure 6.63.c giving the circuit of Figure 6.63.d. It is saved as sym50.fil. Its dual may also be used as another solution.

- The fourth order sections are split into two second order sections by applying Split command (Figure 6.63.d-e). The four FTZ elements are all in series arm parallel LC elements. The first FTZ element on left is the one creating the FTZ in lower stopband and the second one creates the FTZ in upper stopband. These points can be checked by applying Tune Element command to these elements. This circuit is saved as sym51.fil to be used in different approaches to get different topologies.

Solution-1:

- If it is advantageous for some reason, some of the series type lower stopband FTZ elements can be converted into shunt type FTZ elements using Colin Transformations as follows.

- With cursor under the leftmost shunt resonator, the command Double Norton / Colin–Right is applied. This operation produces the circuit section shown in Figure 6.63.f where the first FTZ element is now in shunt form. The first series capacitor at left end is negative. It is absorbed into the adjacent positive capacitor by applying Combine command (Figure 6.63.g).

- It should be noted that if we had interchanged the positions of the first two FTZ elements it would not be possible to apply this transformation (Exercise?). This is because realization of an upper stopband shunt FTZ element need to have a series arm inductor (Exercise: Inspect this point by using Synthesis approach and extracting an upper stoband shunt type FTZ element )

- Since the circuit is symmetric we could use Symmetrize command to map the left half to right. However just as exercise we repeat Colin Transformation in the right half as follows.

- Using Exchange command, positions of the two series type FTZ elements are interchanged to move the lower stopband FTZ element nearer to the load end (Figure 6.63.e-f).

- Then with cursor under the rightmost shunt resonator the command Double Norton / Colin-Left is applied to convert the series type FTZ element into shunt type with a negative series capacitor on right (Figure 6.63.g).

- The negative series capacitor is combined with the adjacent positive capacitor giving the circuit of Figure 6.63.h. Now the circuit is in the form with two lower stopband FTZ elements in shunt form and two upper FTZ elements in series form.

- The rest of Figure 6.631 is related to the scaling of inductances in the usual way, as in the previous examples. Exercise for different targets.

Exercise: Continue trials if it is possible to get realizable circuits with any reduction in the number of resonators.

Exercise: Design a degree 14 filter with Nzero=3, Ninf=3, FTZ1=FTZ2=85 MHz and FTZ3=FTZ4=116.5 MHz to get the SAME circuit as the above example. Then extract transmission zeros in different orders to get different alternatives, especially the minimum inductor version.

Figure 6.63. Application of Colin transformations on a symmetric filter.

Figure 6.63. Application of Colin transformations on a symmetric filter (cont’d).

Solution-2:

One may also follow another path, as described in Figure 6.64. In this approach first the transformation  Double Norton / Geffe Eqv is applied to insert shunt capacitors between the adjacent series arm resonators and then Colin transformation is applied.

Exercise: Scale inductors of this circuit.

Figure 6.64. Geffe and Colin transformations.

Solution-3:  The dual form

The dual of the solution sym50.fil may also be used after modifications as described in Figure 6.65.

- The circuit of sym50.fil is loaded and its dual is formed by applying Find Dual command (Figure 6.65.a-b).

- The circuit is scaled by inserting inverters to make the source and load end shunt inductors 50 nH and the inverters are replaced by their capacitive-L equivalents (Figure 6.73.b-c-d-e).

- Next, the 1158.3 nH series inductors are scaled down to 50 nH by first forming an LRight-section using Split / Two Element Resonator command and then applying the command Pi-Tee-L / LRight-to-Pi / Specify TRF: n=SQRT(1158.3/50). The resulting transformer is shifted rightmost (CTRL+W).

- Next the inductors of the fourth order sections will be scaled. An easy way of scaling them is just to Split them into two second order sections. For this filter the inductor values of the second order sections came out to be realizable (Figure 6.65.e-f-g).

- Finally, in order to avoid repetition of these steps in the right half the cursor is placed under the middle element and Symmetrize command is applied giving the circuit of Figure 6.65.g.

Figure 6.65. The dual circuit.

6. 8. 8. WIDEBAND ELLIPTIC FILTERS AS CASCADES OF LP AND HP FILTERS

In some cases it may be preferable to form a wideband filter by cascading a LP and a HP filter. Let’s see an example.

Example 6.51. Wideband elliptic BPF by cascading a LP and a HP filter

Desired filter specifications are as follows:

fp1=100 MHz, fp2=500 MHz,

Return loss should be greater than 20 dB in passband. Insertion loss should be greater than 62 db for f<91 in lower stopband and should be greater than 70 dB for f>565 MHz in upper stopband.

The upper stopband specifications can be satisfied by a LPF with following specifications:

PBRipple=0.01 dB, fp=500 MHz, Ninf=3, FTZ1=567 MHz, FTZ2=572 MHz, FTZ3=665 MHz, FTZ4=888 MHz.

The lower stopband specifications can be satisfied by a HPF with following specifications:

PBRipple=0.01 dB, fp=100 MHz, Nzero=3, FTZ1=60 MHz, FTZ2=78 MHz, FTZ3=87.5 MHz, FTZ4=91 MHz. The FTZ locations are found by trials using Tune a FTZ command. The PBRipple is selected to be smaller than necessary for the specified return loss because we expect degradation in return loss after cascading the two filters.

- Synthesis of the LPF is described in Figure 6.66.a and b. It is saved as LP11.fil.

- Synthesis of the HPF is described in Figure 6.66.c-d. It is saved as HP11.fil.

- With the HP11.fil being on the display, placing the cursor under the load and applying the command Append Filter: LP11.fil, the LP filter is placed on the right of the HPF (Figure 6.66e). We could as well place the LPF on the left of the HPF.

- Inspection of the return loss shows that this brute force cascading leads to about 6 dB lowering in return loss level (Figure 6.66.f).

Exercise: Find dual of HP11.fil and append it to LP11.fil.

Exercise: Find dual of LP11.fil and append it to LP11.fil.

Exercise: Find duals of HP11.fil and LP11.fil and then cascade them.

Exercise: Apply Pi-Tee-L commands to the L-sections of capacitors and inductors to form a variety of topologies to adjust levels of element values. Use also inverters to scale the element values.

Figure 6.66. Design of a wideband elliptic bandpass filter by cascading a LPF and a HPF.

Figure 6.66. Design of a wideband elliptic bandpass filter by cascading a LPF and a HPF.

REFERENCES AND FURTHER READING:

- E. L. Norton, “Constant Resistance Networks with Applications to Filter Groups”, Bell System Tech. J. vol. 16, pp.178-193,  Apr. 1937.

- J. E. Colin, “De Quelques transformations de quadripoles”, Cables et Transmission, Vol. 44, pp. 314-344., Oct. 1956.

- S. B. Cohn, “Direct coupled resonator filters”, Proc. IRE 45, pp. 187-16, Feb. 1957.

- R. Saal, and E. Ulbrich, “On the design of filters by Synthesis”, IRE Trans. Circuit Theory, vol. CT-5, no. 4, pp. 284 - 327, 1958.

- P. R. Geffe, “Simplified Modern Filter Design”, Hayden Book Co., NY, 1963.

- G. Szentirmai, “Band-pass matching filter in the form of polynomial low-pass filter”, IEEE Trans. On Circuit Theory ,  vol. CT-11,  pp. 177-178, March 1964.

- A. Zverev, “A Handbook on Filter Synthesis”,  John Wiley and Sons, Inc., NY 1967.

- H. J. Orchard and G. C. Temes, “Filter Design Using Transformed Variables”, IEEE Trans CT, vol. CT -15, No. 5, pp. 385 - 408, Dec. 1968.

- D. S. Humpherys, “The Analysis, Design and Synthesis of Electrical Filters” Prentice Hall Inc., 1970.

- G. C. Temes and S. K. Mitra, “Modern Filter Theory and Design”, J. Wiley and Sons, NY, 1973.

- R. W.  Daniels, “Approximation Methods for Electronic Filter Design”, Mc Graw Hill, NY, 1974.

- J. D.  Rhodes , “Theory of Electrical Filters”, J. Wiley and Sons, NY, 1976.

- W. K.  Chen , “Theory and Design of Broadband Matching Networks”,  Pergamon Press, 1976.

- G. C. Temes and J. W. La Patra, “Introduction to Circuit Synthesis and Design”, Mc Graw Hill ,NY, 1977.

- R. Saal, “Handbook of Filter Design”,  AEG Telefunken,1979.

- D. R. J. White, “A Handbook on Electrical Filters”, Don White Consultants Inc., 1980.

- J. D. Rhodes and S. A. Alseyab, “The Generalized Chebyshev Low Pass Prototype Filters”, Int. J. Circuit Theory and Applications, Vol. 8, pp. 113 - 125, 1980.

- S. A. Alseyab, “A Novel Class of Generalized Chebyshev Low Pass Prototype for Suspended Substrate Stripline Filters”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-30, No. 9, pp. 1341-1347, Sept. 1982.

- M. Hasler and J. Neirynck , “Electric Filters”, Artech House Inc., Mass. 1986.

- Y. Sen, “Direct Design of Lumped and Distributed Filters Using Transformed Variable Techniques”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara,Turkey, 1989.

- M. L. Jewell, “On High Frequency Narrowband Elliptic Filters”, IEEE Trans. CT, Feb. 1990.

- A. J. Klappenberger, “Narrow Bandwidth Elliptic Bandpass Filters with low Parts Value Spreads”, RF Design, pp. 25 - 29, Dec. 1992.

- A. J. Klappenberger, “Computer Design of Equal Shunt Value Tubular Bandpass Filters”, RF Design, pp. 114 -120, March 1994.

- U. Koyaz, “Synthesis of Lumped and Distributed Filters and Diplexers”, Ph. D. Thesis, EEE Dept. Middle East Technical University, Ankara,Turkey, 1994.

- M. Karaaslan, “Network Transformations for Realization of Lumped and Distributed Filters”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara,Turkey, 1994.

- S. Y. Swang, “Equiripple approximation for the synthesis of minimum inductor bandpass filters”, IEEE Trans. CT., pp. 64-66, March 1968.