CHAPTER 8

DISTRIBUTED ELEMENT LOWPASS AND BANDSTOP FILTER DESIGN

8. 1. INTRODUCTION

A commensurate line distributed element lowpass filter in S=j domain has a quasi lowpass characteristics on s=+jw domain, mapped through Richards transformation

Ω=Tanπ2ffq

where fq is the quarter wavelength frequency corresponding to the common length of the distributed elements (stubs and transmission lines) as seen from the insertion loss responses in Figure 8.1.a. This transformation maps =0 to f=2nfq while is mapped to f=(2n+1)fq  where n=0, 1, 2, … The passband edge frequency p maps to fp and 2fq-fp, 2fq+fp, 4fq-fp, 4fq+fp, .. . Thus we get periodic passbands and stopbands with a period of 2fq. The lowpass response along axis between =0 and =maps to the range f=0 and f=fq on f-axis, with the passband between f=0 and f=fp and the first stopband centered at fq, extending  from f=fp to f=2fq-fp. The second passband (usually termed as upper passband) is centered at f=2fq and extends from f=2fq-fp to f=2fq+fp. The second stopband is centered at 3fq, extending from 2fq+fp to 4fq-fp. Hence such filters can be used either as a quasi-lowpass filter or as a quasi-bandstop filter with the following considerations:

- A quasi LPF can be formed by treating the second stopband as a spurious passband. The spurious passband may be made ineffective either by pushing it away from the fist passband by selecting a high enough fq (small commensurate length) or by cascading another LPF.

- A quasi BSF can be formed by selecting the quarter wavelength frequency fq as the stopband center, fo=fq. In this case the second stopband around f=3fo=3fq appears as a spurious stopband which can’t be eliminated.

Figure 8.1.b shows the insertion loss responses of an S-domain bandstop filter along both axis and f-axis. The bandstop characteristics in the range 0<  < is mapped to the range 0< f < fq while its mirror image around fq is mapped to the range fq<f<2fq. The response in the range 0 < f < 2fq repeats itself along f-axis with a period of 2fq. The stopband center  is mapped to fo and the passband edge frequencies  p1 and  p2 are mapped to fp1 and fp2 respectively. The nearest stopbands are centered at 2fq-fo and 2fq+fo. The second passband (upper PB) is centered at 2fq and extends from fp2 to 2fq-fp2. We note the important point that the stopband center fo and quarter wavelength frequency fq are independent of each other. This property leads to the following important results:

- The nearest stopbands can be moved away by selecting fq>2fo leading to wider second (upper) passband.

- A dual bandstop filter can be formed by selecting fq<2fo as shown in Figure 8.1.c.

- The case with fq=2fo yields the same f-axis response as in the distributed element LPF. Thus, distributed element LPF’s may also be treated as a special case of distributed element BSF’s with fq=2fo.

In the first half of this chapter we will concentrate on distributed element LPF’s and BSF’s obtained from S-domain LPF’s. The subject of distributed element (S-domain) BSF’s will be studied in the second half of this chapter, aiming to design bandstop filters with wide upper passbands.

Figure 8.1. Insertion loss characteristics of  an  a) S-domain lowpass filters and

b) S-domain bandstop filter.

8. 2. DISTRIBUTED ELEMENT LOWPASS FILTERS

Main design tools for distributed element lowpass filters are Design / By Synthesis, Design / By Built-in LC Prototype and  Design / By Prototype with Inverters approaches. Synthesis approach is the most general one, employing both stubs and contributing unit elements. The other approaches may also be used by taking their limitations into consideration. For example LC Prototype approach can yield only all-stub filters while in the Prototype with Inverter approach the inverters has limited practical equivalents. Design / By User Defined Prototype approach can always be used to make use of lowpass prototype tables that are not available in Filpro.

In any approach some circuit transformations are usually needed to reach realizable or desired topologies. The need to have physical separation between stubs makes the use of transmission line pieces between stubs (unavoidable in most cases). If the filter is designed in all-stub form, then transmission line pieces (termed as unit elements-UE ) can be inserted from the ports in a redundant manner (no contribution to response). Since transmission lines make filter longer and cause unaccounted losses (conductor and dielectric  losses), whenever possible, contributing unit elements should be preferred to make use of extra selectivity they can provide.

It will be seen that element values of lowpass-banstop filters are strongly dependent on fp/fq  ratio where fp is the passband edge and fq is the quarter wavelength frequency. Elements may take extreme values for fp<<fq or fp too close to fq. On the other hand in stripline type structures realizability constraints limit normalized stub and line impedances roughly to 10-30<Zc<100-300 ohms depending on the production technology used. Some of such problems may be solved using coupled lines or using redundant unit elements or using other equivalent circuit transformations.

Commensurate length distributed element lowpass filters (S=j domain lowpass filters) are synthesized by cascading the elements shown in Figure 8.2. The series arm short circuited stub (Figure 8.2.a) and shunt open circuited stub (Figure 8.2.b) are used to create transmission zeros at S=jwhich correspondsto w=wq on s=jw axis where wq=2fq is the quarter wavelength frequency of the stubs of length d related through the definition

fq=c4dεr

Shunt arm and series arm open and short circuited stub combinations of  Figures 8.2.c and  d create finite transmission zeros on S=j axis which correspond to f=fq, 3fq, 5fq,.. on real frequency axis in the s=+jw plane. The series combination of equal length OC and SC stubs in shunt arm and parallel combinations of OC and SC stubs in series arm are used to create finite transmission zeros on S=j  axis (Figure 8.2.c-d). The unit element (transmission line piece of length d in Figure 8.2.e) produces a   transmission zero on real axis, at S= leading to a real transmission zero on s= axis. Unit elements and shunt open circuited stubs are favored components in stripline-like structures because of ease of construction. The finite transmission zero section of Figure 8.2.c is also easy to realize in its equivalent Two Step Resonator form shown in the figure. The series arm short circuited stub and the finite transmission zero section of Figure 8.2.d are difficult (if not impossible) to implement. Either because of implementation difficulties or because of severe limitations on realizable impedance values, considerable amount of equivalent circuit transformations had been developed to convert some unrealizable circuit sections into realizable or desirable forms. Some of these transformations will be presented in the examples of the succeeding sections.

Figure 8.2. Transmission zero producing elements used in lowpass filters.

8. 2. 1. CHEBYSHEV LOWPASS FILTER EXAMPLES

In this section some typical design examples will be presented for Chebyshev type distributed element filters in commonly used topologies. In order to inspect effects of both contributing unit elements and choosing high quarter wavelength frequency on response and other element values, various filters will be designed, first in all redundant UE  forms, than in  all-contributing UE  and mixed (redundant + contributing UE) forms.

EXAMPLE  8. 1: Chebyshev lowpass filters with all redundant unit elements

Let’s synthesize some all-stub Chebyshev lowpass filters with specifications

PBRipple= 0.01 dB,   fp=1000 MHz ,   Ninf=5,    RS=RL=50  ohms

Let’s consider the following quarter wavelength frequencies:

i)   fq= 2000 MHz

ii)  fq= 4000 MHz

iii) fq= 8000 MHz.

Although Design / By Synthesis approach is quicker, just as a demonstration let’s use Design / By Built-in LC Prototype approach. Design steps are shown in Figure 8.3:

- The command set  New / Design / By Built-in Prototype / Distributed, Lowpass, Chebyshev /  RS = RL = 50 / PB Specs + Filter Degree: PB Ripple=0.01, fp=1000, fq =...  , N=5 / Find Circuit  gives the circuit of Figure 8.3.a.

- We can continue with this circuit, but the dual circuit is easier to deal with. Find Dual command gives the circuit of Figure 8.3.b.

- In order to convert the series SC stubs into shunt OC stubs we need transmission lines (Unit Elements). These transmission lines will also provide physical separation between the shunt OC stubs to prevent coupling. With cursor under the first shunt OC stub on left, the command Insert / TLine: Zc=50 ohms is applied twice to insert two transmission lines at source end. Similarly, with cursor under load resistor, two transmission lines are inserted at load end also (Figure 8.3.b-c).

- Then with cursor under the transmission lines adjacent to the stubs on left side the command Kuroda / Right / Full Stub is applied to move the transmission line pieces into the circuit while also converting the series SC stubs into shunt OC stubs. Then with cursor under the transmission lines on the right side the command Kuroda / Left / Full Stub is applied to move them into the circuit yielding a circuit with all shunt OC stubs separated with transmission line pieces (Figure 8.3.c-d-e). Element values seem to be realizable. The resulting circuit is saved as LP2000.FIL.

- Figure 8.3.f shows a pictorial description of the circuit. The total length is about 150 mm.

- Figure 8.3.g shows the wideband insertion loss response. Clearly, the spurious passbands will be problematic to deal with. The nearest spurious passband start after 2000 MHz.

- In order to both decrease the total length and also move the nearest spurious passband away from the main passband the design is repeated for fq=4000 MHz. The resulting circuit and wideband response are shown in Figures 8.3.h-i-k.  Total length is halved while the nearest spurious passband is pushed away to above 6000 MHz. However the element values start to be problematic. The highest TLine impedance is about 175 ohms while the lowest stub impedance is  13.1 ohms. The 13.1 ohms stub can be split into two 26.2 ohms stubs which can be realized. The 175 ohms TLines may be realized in suspended stripline form.

- In order to see further problems and possible solutions let’s increase fq to 8000 MHz. Using the same procedure we get the circuit of Figure 8.3.m. We have now extreme values of 338 ohms for inner TLines and 6.31 ohms for the shunt stub in the middle. They can’t be realized in distributed element form.

- First the shunt OC stubs of 10.7 ohms are split into two 21.4 ohms stubs. Also the stub in the middle is replaced by its lumped capacitor equivalent by applying the command Replace By Lumped / Single C: f0=1000 MHz (Figure 8.3.n).

- Then the high impedance TLines are converted into series inductors by applying the command Replace By Lumped / Pi Model / Csh+Lsr+Csh: f0=1000 MHz. We get two small capacitors on the two sides of the series inductor (Figure 8.3.p). The one adjacent to the OC stub is deleted while the one adjacent to the capacitor in the middle is combined with that capacitor. The same procedure is repeated in the right half giving the circuit of Figure 8.3.q.

- Thus, we have a mixed Lumped+Distributed element filter. Figure 8.3.r shows a pictorial description while Figure 8.3.s shows wideband insertion loss responses of both original distributed element filter and the final mixed element filter. It is seen that the spurious passband is moved away and suppressed a lot.

- Clearly, in the limit of very high fq, we have to convert all TLines into series inductors and all shunt OC stubs into shunt capacitors giving us the lumped element lowpass filter circuit (exercise?).

Figure 8.3. Design of all redundant Unit Element lowpass filter.

Figure 8.3. Design of all redundant Unit Element lowpass filter (cont’d).

Figure 8.3. Design of all redundant Unit Element lowpass filter (cont’d).

Figure 8.3. Design of all redundant Unit Element lowpass filter (cont’d).

EXAMPLE  8. 2: Chebyshev lowpass filters with contributing unit elements

In the previous example the Unit Elements were all redundant, having no contribution to the response. Let’s repeat the lowpass filter of the previous example, now by using all contributing Unit Elements and see the effects of Unit Elements on response and on element values.

In this approach we have to use Synthesis approach because Built in LC Prototype approach doesn’t involve Unit Elements. Design command set is shown in Figure 8.4. Distributed / LP / Equiripple / Doubly Terminated / Type B options are selected. In Type B option the first extracted element will give a shunt stub. In order to get the same structure as the previous filters, that is, to get five shunt OC stubs and four Unit Elements, we have to type in Ninf=5 and NUE=4, leading to a degree of N=11. This fact tells us that we will get a higher skirt slope compared to the previous example where we had total degree of N=5 only.

- Transmission zeros are extracted in the order Inf-ue-Inf-ue-Inf-ue-Inf-ue-Inf giving the circuit of Figure 8.4.a. It is saved as LP2000UE.FIL. In order to compare the two designs the circuit of LP2000.FIL is also shown in the same figure. It is seen that element values of both circuits are in realizable range. As expected, the insertion loss response of the filter with contributing Unit Elements is far steeper compared to that with redundant Unit Elements. That is, we could design a filter with lower degree to get the same response as the redundant Unit Element filter. Figure 8.4.b shows the same comparison for the filters with fq=4000 MHz. We note that in the contributing Unit Element filter the transmission line impedances became higher that may cause problems.

Figure 8.4. Lowpass filters with contributing Unit Elements.

Figure 8.4. Lowpass filters with contributing Unit Elements (cont’d).

Exercise: From the two examples it is seen that introduction of contributing unit elements improves skirt slopes of filters appreciably for moderate to wide stop bandwidths. However, for wide stopband filters unit elements get too high values. This problem may be eased to some extent by using less number of contributing unit elements than necessary and compensate the rest by inserting redundant unit elements. Design LP filters with both contributing and redundant Unit Elements to see if any improvement can be obtained in problematic element values of filters with high fq.

EXAMPLE  8. 3: Stepped transmission line(all UE) lowpass filters

Now let’s see an example for a lowpass filter formed with Unit Elements only, without transmission zeros at infinity. Specifications are

PBRipple= 0.01 dB,   fp=1000 MHz ,   NUE=9,    RS=RL=50  ohms

Let’s consider the following quarter wavelength frequencies:

i)   fq=2000 MHz

ii) fq=4000 MHz

Design command sets for both fq’s are shown in Figure 8.5. The following points can be noted:

- Since there is no any s=jw axis transmission zero, insertion loss can’t become infinite. It passes through a maximum at fq. Therefore the stop bandwidth widens as fq is increased while the maximum attenuation in the stopband increases also. Therefore higher fq seems to lead to  better response accompanied by shorter overall length.

- Impedances of Unit Elements change in stepwise manner, as low and high impedances. However as fq is increased impedances start to get extreme values. Therefore as fq is increased realization difficulties start. An optimum fq can be found by trials or the approximate approach of the following example can be used.

Figure 8.5. Synthesis of stepped transmission line lowpass filter.

EXAMPLE  8. 4. Approximation of stubs by transmission lines

For lowpass applications deformation or elimination of the nearest spurious passband is allowable, even desirable. For such cases an approximate and brute force techniques can be used to adjust impedance levels of transmission lines and/or stubs leading to different lengths for transmission lines and stubs. Some examples follow.

Let’s design a lowpass filter with specifications

PBRipple=0.01 dB,   fp=1000 MHz ,   fq=4000 MHz ,   N=9

using Synthesis approach.

- The resulting all-stub circuit is shown in Figure 8.6.a.

- Placing the cursor under the series SC stubs and applying the command Replace By TLine / f0=1000 MHz / Z=125 ohms all the series SC stubs can be converted into transmission lines with different lengths and equal characteristic impedance of 125 ohms (Figure 8.6.b). Here Zc must be selected as the highest realizable value for a good approximation. Thus, compared to the all contributing unit element design of the previous example where the Unit Elements had very high impedances, in this filter we have all equal and realizable transmission lines.  The circuit is saved as LP10.FIL.

- The shunt OC stubs have too low impedances. We can split them into two equal parts to get higher impedances (Figure 8.6.c).

- Figure 8.6.d shows a pictorial description of the filter. Note that the line lengths are different.

- Figure 8.6.e shows wideband insertion loss response where distortions in the first spurious passband are clearly seen.

- Figure 8.6.f shows the insertion loss and return loss in and around passband. There is a band edge shift which must be compensated by using a slightly higher fp. There is also a slight degradation in passband return loss.

- The low impedance OC stubs may also be replaced by low impedance transmission line pieces.  The circuit of LP10.FIL is loaded and with cursor under the stubs the command Replace By TLine / f0=1000 MHz / Z=25 ohms (or any suitable low impedance) is applied. We get different length lines (Figure 8.6.g-h).

- Figure 8.6.i shows a pictorial description of the filter.

- Figure 8.6.k shows insertion and return loss responses. Appreciable band edge shift is observed. The stopband response is effected heavily because no transmission zero is left on s=jw axis. (The transmission lines create transmission zeros on real axis). Such filters may found applications in cases where stopband response is not of much importance.

Exercise: Find dual of the circuit of Figure 8.6.a and apply the same transformations.

Figure 8.6. Approximations in distributed element lowpass filters.

Figure 8.6. Approximations in distributed element lowpass filters.

Figure 8.6. Approximations in distributed element lowpass filters.

8. 2. 2. ELLIPTIC LOWPASS FILTERS

The filters studied in the two examples of previous section involved either redundant unit elements or transmission lines obtained by approximating series SC stubs. Much sharper skirt slopes and controlled stop band responses can be obtained with comparable sizes by synthesizing filters with contributing unit elements and with carefully placed finite transmission zeros. Although all unit elements could be chosen as contributing, if it eases some realization problems, one may also use some redundant unit elements and / or use the technique of replacing series SC stubs by transmission lines. Conventionally filters with equal ripples in both pass and stop bands are referred to as elliptic filters. We will use this convention for filters with arbitrarily placed finite transmission zeros also.

A topology readily lending itself for realization is shown in Figure 8.7. Such structures can be formed by using one or more transmission zero at infinity, some finite transmission zeros and sufficient  number of contributing Unit elements to convert any series SC stub into shunt OC stub. The finite transmission zero sections come out to be as series combinations of SC and OC stubs in shunt arm. They can be converted into two step shunt OC stubs (Stepped Resonators).  One may also introduce redundant Unit Elements at input and output ports whenever it eases the element value problems. Let’s continue discussions through examples.

Figure 8.7. Easy to realize topology for elliptic lowpass filters.

EXAMPLE 8. 5. A degree 5 elliptic lowpass filter

Let’s design a lopwpass filter with following specs:

PBRipple=0.01 dB, fp=1000 MHz, fq=3000 MHz, Ninf=1, NUE=2, NFTZ=1

The finite transmission zero will be tuned to shape the stopband response so that the insertion loss will be greater than 40 dB for f>1900 MHz.

- Design command set is shown in Figure 8.8. Using Tune FTZ command the finite transmission zero is moved to fz=2000 MHz to get 40 dB loss for f>1900 MHz.

- The transmission zeros are extracted in the order ue-FTZ-ue-Inf giving the circuit of Figure 8.8.a.

- With cursor under the first unit element on left, the command Kuroda / Right / Full Stub is applied to convert the series SC stub into shunt OC stub.

- With cursor under the OC+SC Stub element the command Step Resonator is applied to convert it to a two step OC stub (Figure 8.8.b).

- The element values seem to be realizable in stipline-like structures. Figure 8.8.c shows a pictorial description.

Exercise: Repeat the design for different PBRipple and with different positions of the FTZ. Inspect the impedances of two-step resonator as the position of the FTZ is changed. Note that if FTZ frequency fz=fq/2=1500 MHz, the two-step stub becomes a uniform stub. That is, the step impedances become equal.

Exercise: Repeat the design with different quarter wavelength frequencies to see the variation of element values.

Figure 8.8. A degree 5 elliptic lowpass filter.

EXAMPLE 8. 6. Degree 7 elliptic lowpass filters

Let’s design degree 7 lowpass filters with specs

PBRipple=0.01 dB, fp=1000 MHz, fq=3000 MHz

Degree 7 filters can be formed by selecting different numbers of transmission zeros. Let’s consider two cases.

Case-1: Ninf=3, NUE=2, NFTZ=1 with FTZ adjusted to give  insertion loss of greater than 45 db for f>1450 MHz.

Case-2: Ninf=1, NUE=2, NFTZ=2, with FTZ’s adjusted to give equal ripple stopband with loss greater than 60 dB for f>1600 MHz.

- Case-1 is described in Figures 8.9. Tune FTZ command is used to place the FTZ at fz=1500 MHz to satisfy the requirement of greater than 45 dB loss for f>1450 MHz. Type A is selected to get the FTZ section as OC+SC stub in shunt arm. Extraction order is shown in the figure. The resulting circuit is in Figure 8.9.a.

- Selection of Type A led to series SC stubs at input and output. There is not enough number of unit elements to convert them into shunt OC stub form. We should have place four unit elements instead of two. But this would lead to degree 9 filter (See next example). Let’s continue with this case. In order to convert the series SC stubs into shunt OCV stubs we Insert transmission lies (Redundant Unit Elements) at input and output (Figure 8.9.b).

- Using Kuroda / Right-Left / Full Stub transformation the series SC stubs are converted into shunt OC stubs. Also the OC+SC stub combination is converted into two-step resonator using Step Resonator command. Thus we get the circuit of Figure 8.9.c.

- Figure 8.9.d shows pictorial description. Note that since we have selected fz=fq/2=1500 MHz, the two-step stub came out to be uniform.

- Case-2 is described in Figures 8.10. Tune FTZ command is used to set the two FTZ’s at fz1=1670 MHz, fz2=2000 MHz to get equal ripple stopband loss. These FTZ’s may also be set to adjust the selectivity to higher levels by moving them closer to the passband, at the expense of less stopband loss.

- Figure 8.10.a shows the synthesized circuit. We need two Redundant Unit Elements in this circuit also. Thus, we Insert two transmission lines at input and output (Figure 8.10.b).

- Using Kuroda / Right-Left / Full Stub transformation the series SC stubs are converted into shunt OC stubs. Also the two OC+SC stub elements are converted into two-step resonators using Step Resonator command (Figure 9.10.c).

- The middle shunt OC stub has too low impedance. Therefore it is split into two to get two stubs with doubled impedances. Thus we get the circuit of Figure 8.10.d. Figure 9.10.e shows a pictorial description.

It is seen that in both cases we needed redundant unit elements which have no contribution on response while they will increase the loss and length of the filter. Therefore it may worth to specify two contributing unit elements during Synthesis stage, increasing the degree to 9, as studied in the next example.

Exercise: Repeat the designs by selecting Type B option to see the problems.

Figure 8.9. Design of a degree 7 elliptic lowpass filter with one FTZ.

Figure 8.10. Design of a degree 7 elliptic lowpass filter with two FTZs.

EXAMPLE 8. 7. Degree 9 elliptic lowpass filters

Let’s design a degree 9 lowpass filter with specs

PBRipple=0.01 dB, fp=1000 MHz, fq=3000 MHz

The stopband loss should be greater than 60 db for f>1300 MHz.

Degree 9 filters can be formed by selecting different numbers of transmission zeros. However most needs redundant unit elements. The following transmission zero pattern yields a circuit which is easy to implement:

Ninf=1, NFTZ=2, NUE=2FTZ=4.

The FTZ’s can be adjusted to give the desired selectivity and stopband loss level.

- The design command set is shown in Figure 8.11.  The two FTZ’s are set to 1670 MHz and 2000 MHz to get the required stopband specifications.

- The elements are extracted in the order shown giving the circuit of Figure 8.11.a.

- The series SC stubs are converted into shunt OC stubs by applying Kuroda / Right / Full Stub command and the OC+SC stub elements are converted into stepped resonators (Figure 8.11.b).

- The middle shunt OC stub is split into two to get higher impedance stubs (Figure 8.11.c). Figure 8.11.d shows the pictorial description. It seems realizable.

Exercise: Repeat the designs with different PBRipple levels to see its effect on element values.

Exercise: Repeat the design for different fq values.

Exercise: Try to form different transmission zero patterns of total degree 9 to get realizable circuits.

EXAMPLE  8. 8. Degree 13 elliptic lowpass filter

Let’s design a degree 13 elliptic lowpass filter with specifications

PBRipple=0.01 dB ,   fp=1000 MHz ,    fq=3000 MHz , Ninf=1, NFTZ=3, NUE=2NFTZ=6

The FTZ’s will be tuned to get at least 60 dB loss for f>1090 MHz.

- The design command set is shown in Figure 8.12.  The three FTZ’s are set to 1090 MHz, 1130 MHz and 1252 MHz to get the required stopband specifications.

- The elements are extracted in the order shown giving the circuit of Figure 8.12.a.

- The series SC stubs are converted into shunt OC stubs by applying Kuroda / Right / Full Stub command and the OC+SC stub elements are converted into stepped resonators (Figure 8.12.b).

- The two interior shunt OC stubs are split into two to get higher impedance stubs (Figure 8.12.c).

- Figure 8.12.d shows the pictorial description. Some element values of two-step resonators seem to be too high. This is because we have place a FTZ too close to the passband.

Exercise: Repeat the designs with different PBRipple levels to see its effect on element values.

Exercise: Repeat the design for different fq values.

Exercise: Repeat the design by tuning FTZ’s to different positions to decrease the maximum impedance of the stepped stub.

Exercise: Note that the circuit is not symmetric. This is because the FTZ’s are placed arbitrarily. In order to get a symmetric circuit place two of the FTZ’s at the same frequency. The third one can be placed at any suitable location. Then extract the transmission zeros so that a symmetric circuit will be obtained. The third FTZ must be extracted in the middle.

Exercise: Try to form different transmission zero patterns of total degree 9 to get realizable circuits.

Exercise: Design degree 15 and 17 degree filters using similar approaches.

Figure 8.11. Design of a degree 9 elliptic lowpass filter.

Figure 8.12. Design of degree 13 elliptic filter.

EXAMPLE  8. 9. Symmetric version of degree 13 elliptic lowpass filter

Note that the circuit of the previous example is not symmetric. This is because the FTZ’s are placed arbitrarily. In order to get a symmetric circuit we should place two of the FTZ’s at the same frequency. The third one can be placed at any suitable location. Then the transmission zeros can be extracted in a proper order so that a symmetric circuit will be obtained. Let’s design a degree 13 elliptic lowpass filter with the same specifications

PBRipple=0.01 dB ,   fp=1000 MHz ,    fq=3000 MHz , Ninf=1, NFTZ=3, NUE=2NFTZ=6

But with two of the FTZ’s being at the same frequency. The FTZ’s will be tuned to get at least 60 dB loss for f>1090 MHz.

- The design command set is shown in Figure 8.13.  The two FTZ’s are set to 1090 MHz and the third one is set at 1220 MHz to get the required stopband specifications.

- The elements are extracted in the order shown giving the circuit of Figure 8.13.a.

- The series SC stubs are converted into shunt OC stubs by applying Kuroda / Right / Full Stub command and the OC+SC stub elements are converted into stepped resonators. Note that we now have a symmetric circuit. The impedances of the stepped resonator in the middle and on the right are changed appreciably compared to the previous circuit  (Figure 8.13.b). One of the step impedances of the outermost stepped resonators is too high. This is because these two FTZ’s are too close to the passband edge.

- The two interior shunt OC stubs are split into two to get higher impedance stubs (Figure 8.13.c).

- Figure 8.13.d shows the pictorial description.

Figure 8.13. Design of a symmetric degree 13 elliptic lowpass filter.

EXAMPLE  8. 10. Elliptic Lowpass Filter With Hairpin-Comb lines and Unit Elements.

The symmetric filter synthesized in the previous example may also be realized in coupled line form as follows:

Specification and FTZ locations are the same as the previous example:

PBRipple= 0.01 dB ,   fp= 1000 MHz ,    fq= 3000 MHz , Ninf= 1, NFTZ= 3,  NUE=2 NFTZ = 6

The two FTZ’s are set to 1090 MHz and the third one is set at 1220 MHz.

- The command set is shown in Figure 8.14. Here we select the option Type A to get the dual circuit of the previous example (Figure 8.14.a).

- Place the cursor under the third unit element from left and apply Kuroda-Right / Full Stub  command to move the adjacent shunt OC stub on the right to left of the unit element as series SC stub (Figure 8.14.b).

- Then place the cursor under the second unit element and repeat Kuroda-Right command to translate the series SC stub to left as shunt OC stub, giving the circuit of Figure 8.14.c. Then use command  Split / Specify Left Element  on this stub to split it such that equal shunt stubs will result on the two sides of series arm parallel stub resonator (a symmetric Pi-section). Move the remaining stub to right, back to its original position using now Kuroda-Left / Full Stub  command getting the circuit of Figure 8.14.d.

- With cursor under the middle element of the circuit (the series arm OC+SC stub) apply Symmetrize command giving the symmetric circuit of Figure 8.14.e. This command avoids repetition of the transformations carried out on the left half.

- Place the cursor under the first series arm OC+SC stub elemnt (middle element of Pi-section) and apply command PCL / Double Comb Lines / Hairpin-ocComb. This command replaces the Pi-section by an Hairpin-Comb type parallel coupled line section. Repeating this transformation on all Pi sections, a structure involving Hairpin-Comb lines separated by double unit elements is obtained as shown in Figure 8.14.f. In applying this command the constants k1 and k2 must be found by trails.

- Figure 8.14.g shows a pictorial description.

This example is just a demonstration of an alternative topology. Apparently it has no advantage over the topology of the previous example. On the contrary, it is more difficult to implement because of the coupled lines.

Figure 8.14. Development of a Hairpin-Comb elliptic lowpass filter with contributing unit elements.

Figure 8.14. Development of a Hairpin-Comb elliptic lowpass filter with contributing unit elements.

EXAMPLE  8. 11. Digital elliptic filters.

Digital elliptic filter structures are among the classical topologies [9], [16]. They are realized in the form of n-coupled lines derived directly from all-stub filter structures (no need for redundant or nonredundant transmission line sections, hence a compact structure using the space allocated to filter efficiently). As an example let’s design a Generalized Chebyshev-1 filter with

PBRipple=0.01 dB,    fp=1000 MHz,     fq=3000 MHz,  Ninf=1, NFTZ=4,

All FTZ’s will be at the same frequency fz=1250 MHz

- Design command set is shown in Figure 8.15 giving the circuit of Figure 8.15.a.

- Figure 8.15.b shows the idea of how to realize the series and shunt OC stubs. Four parallel conducting bars with open ends are placed above a ground plane. The separations between bars and ground plane form shunt OC stubs while the two wire lines formed by adjacent bars form the series OC stubs. In this figure the idea of how to realize a series SC stub is also explained on a coaxial SC line.

- In Figure 8.15.d, the coaxial SC stubs are placed inside the coupled bars leading to a compact structure.  Further details and variations of such structures can be found in [9].

Figure 8.15. Design and realization of a Digital Elliptic LP filter.

BANDSTOP FILTERS

DESIGNED AS

DISTRIBUTED ELEMENT LOWPASS FILTERS

8. 3. DISTRIBUTED ELEMENT LOWPASS FILTERS AS BANDSTOP FILTERS

Inspection of the insertion loss responses of the lowpass filters of the previous examples shows that they can also be treated as bandstop filters with fundamental stopband centered at fo=fq. Therefore the nearest spurious stopband appears at 3fo=3fq. The design approach for such filters is the same as distributed element lowpass filter described in the previous examples. That is, any one of Design / By Synthesis or By Built-in LC Prototype or By Prototype with Inverters can be selected. Medium-narrow stopband cases lead to extreme element values which necessitates the use of parallel coupled lines as will be described in the following examples.

If the location of the spurious stopband cause problems it can be moved away from 3fo to get wider second passband by designing a Distributed Element BSF and selecting fq>2fo. This subject will be treated in the next section.

EXAMPLE  8. 12. Realization of a moderate bandwidth bandstop filter with spurlines

Let’s design a bandstop filter with following specifications:

PBRipple=0.01 dB, fp1=900 MHz, fp2=1100 MHz, Ninf=5, NUE=6.

We will design the filter as a lowpass filter with fp=900 MHz and fq=1000 MHz.

- Design command set is shown in Figure 8.16 giving the circuit of Figure 8.16.a. Element values seem to be realizable. Therefore it is possible to realize it as shown in Figure 8.16.d.

- An alternative realization is also possible using Spur Type Coupled Lines. For this purposee the shunt OC stub in the middle of the circuit is split into two equal parts (Figure 8.16.b) to form pairs of TLine+Shunt OC stubs.

- With cursor under transmission lines on left half, applying command Replace By PCL / SpurLines / LeftSpur / n=… (default value), we get spur type coupled line. Similarly, with cursor under the transmission lines in right half, the command Replace By PCL / SpurLines / RightSpur / n=… (default value) is applied giving the coupled line circuit of Figure 8.16.c. The even and odd mode impedances are shown beneath the coupled line elements. The parameter n displayed in the command can be used to form symmetric or asymmetric coupled lines. The displayed default value gives symmetric coupled line which is the usual choice.

- Figure 8.16.d and e show pictorial descriptions of SpurLine and Stub+TLine forms of the filter.  It is seen that spurline saves space in transversal direction.

For narrower stop bandwidths the Stub+TLine form becomes unrealizable because of the extreme shunt OC stub impedances. Similarly the SpurLine version is also unrealizable because of the very tight couplings of spur lines. For such filters we prefer to use the approach described in the next example where coupled lines with L-Type Resonators are used.

Exercise: We notice that effects of contributing Unit Elements decreases as the stop bandwidth decreases. This may also be seen from the fact that the transmission line impedances are all about 50 ohms. Exercise by changing the numbers of contributing and redundant Unit Elements in the design.

Figure 8.16. Spur Line and Stub+TLine realizations of moderate stopband BS filter.

EXAMPLE  8. 13: A narrowband bandstop filter with OC L-Resonator Type coupled lines

Let’s now consider a narrow stopband band filter with following specifications:

PBRipple=0.01 dB, fp1=975 MHz,  fp2=1025 MHz, Ninf=5, NUE=6

The filter will be designed as a lowpass filter with fp=975 MHz, fq=1000 MHz. The procedure is as follows [15]:

- Design command set is shown in Figure 8.17, giving the circuit of Figure 8.17.a.

- With cursor under the transmission lines, the command Replace By PCL / I-L-U Resonator Lines / ocOS-L / Zev=.. , Zstub=..,  n=.. (for symmetric case) is applied. Here ocOS prefix means that the left end of the coupled (auxiliary) line is open circuited while the right end is terminated in an Open Stub (OS). At this step, the even mode impedance of the main line is asked. Entering the default value as a trial, another parameter n will be asked. The default value of n will give a symmetric coupled line. Entering that value, the resulting stub impedance (terminating the auxiliary line) will be displayed. If the stub impedance come out to be unrealizable, than iteration can be restarted by changing the value of Zev. The result will be replacement of unit element+stub pair  (with stub being on the right of unit element) by an ocOS-L type coupled line as shown in Figure 8.17.b. Note that the rightmost line is left as it is because it is not paired with a shunt OC stub. It may be deleted because its impedance is very close to 50 ohms.

- Figure 8.17.c shows a pictorial description.

Exercise: Split the middle shunt OC stub into two equal parts and then realize the coupled lines in the right half as OSoc-L type coupled lines.

Exercise: Repeat the example by using Ninf=5 and NUE=5 to see the problem (Transformer!).

Figure 8.17. Design of a narrowband bandstop filter with L-Resonator type coupled lines.

EXAMPLE  8. 14.  A moderate bandwidth elliptic BSF using SC L-Resonators.

Let’s design a lowpass-bandstop filter to stop the frequency band  850 MHz<f< 1150 MHz  by at least  60 dB with a return loss of about 26 dB.

Trials gave the following specifications:

PBRipple=0.01 dB ,   fp=800 MHz,   fq=1000 MHz

with the following transmission zeros:

Ninf =1    (1 TZ at infinity (i.e. at f = fq  ))

NFTZ =3  at  FTZ1 = 860 MHz, FTZ2 = 860 MHz, FTZ3 = 920 MHz

(FTZ’s are found by using Tune FTZ  command to get approximately equal ripple stopband)

NUE=2*NFTZ=6

- Design command set is shown in Figure 8.18, giving the circuit of Figure 8.18.a.

- The series SC stubs are converted into shunt OC stubs by using the command Kuroda / Right / Full Stub (Figure 8.18.b). The circuit is saved as BS51.fil for use at later stages.

- In order to see realization problems, the FTZ sections (OC+SC Stub elements) are converted into two-step resonators by applying Step Resonator command (Figure 8.18.c). It is seen that the second steps of resonators have extreme impedances. Therefore we will try another approach.

- The circuit of BS51.fil is loaded (Figure 8.18.d). With cursor under the FTZ sections Half Angle / Stpepped Resonators command is applied to convert this element into two two-step resonators with halved lengths (Figure 8.18.e). It is seen that the impedances are still unrealizable. With cursor under these stepped resonators the command Step Resonator is applied to convert them into FTZ sections (OC+SC stub elements) with halved lengths (Figure 8.18.f).

- Then the command Half Angle is applied to the elements shown in Figure 8.18.f to convert them into halved length equivalents (Figure 8.18.g). It is seen that a shunt OC stub becomes a series combination of OC and SC stubs with halved lengths. A transmission line becomes two transmission lines with halved lengths. The 90 ohms shunt OC stubs are left as they are because they are readily realizable.

- With cursor under the transmission lines adjacent to the FTZ sections, the command Replace By PCL / I-L-U Resonator Lines / ocSS-L (or SSoc-L) / n=.. is applied. Here in the term ocSS,  oc prefix means that the left end of the coupled (auxiliary) line is left open circuited while the right end is terminated by a Short Circuited Stub. At this step, the parameter n will be asked. The default value of n will give a symmetric coupled line. Entering that value, the result will be replacement of unit element+FTZ element pair by L-resonator type coupled lines (ocSS or SSoc) as shown in Figure 8.18.h. It is seen that all element values are realizable.

- Figure 8.18.i shows a pictorial description while Figures 8.18.k and m show insertion and return loss responses.

- One disadvantage of this approach is the fact that the number of bandstop sections are too many compared to the number of return loss peaks. This is because of the redundancy generated by splitting the FTZ sections into their half length equivalents. Another disadvantage is that SC stubs are usually problematic to realize. Simpler alternative solutions will be presented in the forthcoming examples.

Figure 8.18. Design of a moderate stopband elliptic bandstop filter with coupled lines.

Figure 8.18. Design of a moderate stopband elliptic bandstop filter with coupled lines.

Figure 8.18. Design of a moderate stopband elliptic BSF with coupled lines.

EXAMPLE  8. 15. An elliptic BSF with non-commensurate lines and  OC L-Resonators

In order to overcome realizability restrictions of narrowband elliptic bandstop filters, an approximate equivalence is introduced by Malherbe [13], [16] as described in Figure 8.19. Here a stub resonator with stub length d is first converted to its Half Angle equivalent consisting of two parallel second order sections in the form of series stub resonators with commensurate length d/2. Then each second order section is approximated by  open circuited uniform stubs of different (noncommensurate) lengths giving resonance at the same frequencies as the stub-resonators. This approximation works very well for narrowband bandstop filters. This transformation is carried out in Filpro by placing the cursor under the stub-resonator and applying command Half Angle / Uniform Stubs. Let’s see its application on a narrowband filter with specifications

Figure 8.19. A noncommensurate line approximation for Half Angle  equivalence

Consider a narrowband bandstop filter with PBRipple=0.01 dB,  fp=950 MHz ,   fq=1000 MHz , Ninf= 1,  NFTZ = 3   with FTZ1= 965 MHz, FTZ2 = 970 MHz,  FTZ3 = 982 MHz obtained by Tune a FTZ command to stop the band between 965 MHz and 1035 MHz by 60 dB. The procedure is as follows:

- Design steps are shown in Figure 8.20. Synthesized circuit is shown in Figure 8.20.a. Kuroda transformations are applied to convert the series SC stubs into shunt OC stubs (Figure 8.20.b). Since the FTZ’s are all at different locations, although we have topological symmetry, the element values are not equal. That is there is no physical symmetry.

- Three redundant unit elements are inserted from source and load sides (Figure 8.20.c).

- Two of the redundant unit elements are moved into the circuit using Kuroda and Kuroda-Levy identities (Figure 8.20.c-d-e-f-g-h). The negative and positive stubs resulting from Kuroda-Levy transformations are moved using Kuroda transformations to proper positions to be combined with the stubs of the same kind.

- The same operations are performed on the load side in a mirror image symmetry manner.

- With cursor under FTZ elements (OC+SC Stubs), the command Half Angle/Uniform Stubs is applied. This command replaces the FTZ element by two uniform OC stubs with different lengths (hence the name non-commensurate) (Figure 8.20.i). Although this transformation is an approximation, it holds almost perfectly for such narrow band applications.

- With cursor under the first unit element on left, apply command Replace By PCL / I-L-U Resonators / ocOS-L / Zev = 60 / n = ... / Zstb =..”. The UE+ OC stub pair will be replaced by a coupled line section named as ocOS-L. In this coupled line section, the auxiliary line is left open at its left port (hence the prefix oc) while the right port is terminated in an open circuited stub (hence the prefix OS-open stub). In this command Filpro asks the even mode characteristic impedance Zev of the main line and an implicit parameter n. The displayed default value of the parameter n yields symmetric parallel coupled line. We type in a trial value for Zev and accept the default value for n. Then impedance of the stub terminating the auxiliary line, Zstub will be displayed. If Zstub comes out to be unrealizable then other trials can be made by typing in a new value for Zev until a satisfactory value is obtained for Zstub (and also for Zod).

- Next, place the cursor under the second unit element and the same command is applied, yielding another ocOS-L type line. However in this case an extra transmission line piece is produced on the left of the coupled line with “negative” length. This is because the length of OC stub is longer than that of the unit element (hence the name “noncommensurate”). In order to carry out the transformation  (which holds for equal length unit element and stub only) the difference of two lengths is added and subtracted, leaving a negative length line piece on left.

- Then, the cursor is placed under the third unit element and the command  PCL / I-L-U Resonators / Osoc-L / Zev = ..  / n = ..  is applied replacing the  OC stub+unit element pair by now  Osoc-L   type line which is the reversed  form of ocOS-L type coupled line. Since this stub is shorter than the unit element, the resulting extra line has a positive length which is equal to the difference between lengths of stub and unit element. The same commands are repeated on all other pairs, giving the circuit of Figure 8.20.m. Since the L-Type resonators have OC stubs they are easier to construct than the SC L-Type resonators of the previous example.

- Figure 8.20.n indicates the negative length lines. They can be absorbed by overlapping the adjacent coupled lines, as described in Figure 8.19.p.

- Figure 8.20.q shows a pictorial description of the resultant structure.

- Figure 8.20.r shows the response which displays no degradation in response confirming that the approximation of FTZ section by different length stubs is a good one for narrowband filters.

- The disadvantage of this approach is the increase in the number of elements caused by the redundancy introduced by splitting the FTZ sections into two non-commensurate stubs. We have 10 bandstop resonators but only seven return loss peaks. In the next example a more efficient and simpler approach will be described.

Figure 8.20. Elliptic bandstop filter using non-commensurate approximations.

Figure 8.20. Elliptic bandstop filter using non-commensurate approximations.

Figure 8.20. Elliptic bandstop filter using non-commensurate approximations.

EXAMPLE 8.16. A simpler approach for noncommensurate elliptic bandstop filters

The approach of Malherbe described in the previous examples lead to more than necessary number of bandstop sections. In that example we got only seven FTZs in the stopband region by using 10 bandstop resonators. The approach used in this example leads to simpler solutions.

Let’s assume that we need to stop the band between fs1=975 MHz and fs2=1025 MHz, with more than 50 dB attenuation.  The return loss in the passbands will be greater than 20 dB. Five bandstop sections (five FTZ’s) will be used.

The procedure is as follows:

- We will start the design as a Chebyshev filter and later we will convert the filter into an elliptic one by brute force tuning.

- On Filpro we can specify only passband edges, not the stopband edges. Therefore we have to make some trials to set the correct passband edges. As a first trial let’s take fp1=950 MHz and fp2=1050 MHz, PBRipple=0.01 dB.

- Let’s target to get a symmetric filter. In order to get a symmetric structure we have to use either Ninf=5, NUE=6 or Ninf=5, NUE=4 that would give an odd degree. However just as a variation let’s take Ninf=5 and NUE=5 to see the problems and solutions.

- Figure 8.21 shows the design steps. The transmission zeros are extracted in the order shown giving the circuit of Figure 8.21.a. we have a transformer at the load end. This is because we have an even degree filter. There is no way to eliminate this transformer but Delete it by brute force. Fortunately response is not effected much by this action.

- The transmission line impedances are about 50 ohms, hence realizable. However the shunt OC stubs are unrealizable. Therefore we will use the coupled line approach and convert the Transmission line-OC Stub pairs into coupled lines with L-Type resonators. This is done by placing the cursor under the transmission line and applying the command Replace By PCL / I-L-U Resonator Lines / ocOS-L:  Zev=.. , Zstub=.. . In applying this command Filpro  first displays a default trial value for the even mode impedance Zev of the coupled line section. If it is accepted then the resulting transformer turns ratio is displayed for symmetric parallel coupled line. If a different turns ratio is typed in then we will get asymmetric coupled lines. The usual approach is to form symmetric coupled lines. Therefore we accept the displayed value. Then the resulting L-section stub impedance Zs is calculated and displayed. If we find this impedance realizable then press No option to finish the conversion into coupled line form, otherwise press Yes option to continue trials by changing the even mode impedance. When a suitable Zev, Zs pair is obtained we stop trials and get the L-resonator type coupled line shown in Figure 8.21.b. At this step we check the odd mode impedance for realizability. If it comes out difficult to realize then use Undo command and repeat the conversion with a new Zev value.

- This command is repeated on all transmission line-OC Stub pairs giving the circuit of Figure 8.21.f where the Insertion Loss response is also shown. The shaded region is the forbidden region defining the specs of stopband. It is placed by using the command Reference Line / Add a Reference Line / Return Loss Only and/or applying the command Reference Line / Edit Reference Line to adjust frequency and Loss level.  Clearly, the response is not satisfactory yet, because all five transmission zeros are at S=Infinity which corresponds to five FTZ’s at  f=fq=1000 MHz on s=jw axis.

- We can move some of the five FTZ’s at f=1000 MHz  to different locations in the stopband to form equiripple stopband response. This can simply be done by tuning the lengths of stubs of the L-sections. For example, lengths of the two L-sections on left are increased by tuning with increments of 0.1 mm. The new FTZs will show  up during tuning process. Then we tune lengths of the right two L-sections to move two other FTZs to proper locations. We keep the length of center L-section unchanged. So it will remain at f=1000 MHz. Then continue tuning lengths of stubs by finer increments until an equiripple stopband response is obtained not passing through the shaded region, as shown in Figure 8.21.h.

- This is a noncommensurate filter because the lengths of stubs are different. The five FTZs are created by five L-Resonator sections. Therefore it is more efficient than the approach of the previous example.

Exercise: Repeat the design for the transmission zero patterns “Ninf=5, NUE=4”, “Ninf=5, NUE=6”, “Ninf=6, NUE=5”, “Ninf=6, NUE=7”, “Ninf=7, NUE=8”, “Ninf=8, NUE=9”, etc. Tune all FTZs to different frequencies.

Figure 8.21. Design of an elliptic bandstop filter using L-Resonator sections.

Figure 8.21. Design of an elliptic bandstop filter using L-Resonator sections (cont’d).

Figure 8.21. Design of an elliptic bandstop filter using L-Resonator sections (cont’d).