DISTRIBUTED ELEMENT BANDSTOP FILTERS

WITH

WIDE UPPER PASSBANDS

8. 4. BANDSTOP FILTERS WITH WIDE UPPER PASSBANDS

8. 4. 1. INTRODUCTION

In the classical approaches the BS filters are designed as a distributed element LPF in which case the stopband center fo coincides with the quarter wavelength frequency fq. In such filters the nearest spurious stopband is centered at 3fo. Thus, the spurious free second passband will be in the range between fo and 2fq-fo. This is a severe limitation for the width of the upper passband. In practice much wider upper passbands may be desirable. This barrier was first overcome through the practical works of R. Snyder-S. Shin [28] and then its theoretical basis was presented  in the paper of R. Levy, R. Snyder and S. Shin [29]. In these works the upper passband limit was extended up to six times the main stopband center.

In this section the tools of Filpro will be used to show that the upper passband limit can be further extended to much higher frequencies.

The basis of the new approach is to design the filter as distributed element BSF with fq>2fo. This approach will push the nearest spurious stopband to 2fq-fo and the second passband will be in the range from fo to 2fq-fo, hence wider than the previous cases. As will be shown the upper limit will be constrained only by the realizability limits of the parallel coupled lines and associated elements.

Theoretically the transfer functions of commensurate length BSF’s can be formed by placing Finite Transmission Zeros (FTZ) and Unit Elements (UE’s). Then synthesis can be accomplished by extraction of the transmission zeros using the classical cascade synthesis techniques. However as will be seen, very high degree filters are necessary for many applications. This fact forms a barrier on the exact synthesis technique due to severe numerical accuracy problems. For this reason instead of exact synthesis a simpler alternative route is taken which involves a LP Prototype-to-Distributed Element BS mapping resulting in circuits made up of OC and SC stubs and ideal inverters followed by replacement of inverters by one or more quarter wavelength transmission line pieces (Unit Elements). This replacement leads to great degradations in both passband and stopband responses. Then optimization is used to recover the targeted response. LP-Prototype to Distributed BSF mapping is described in Figure 8.22.

Figure 8.22. LP-Prototype-to-Distributed BS mapping.

In this figure a LP prototype in the form of shunt capacitors coupled by inverters is used to design the BSF. Any other LP prototype may also be used. fp1 and fp2 are the passband edge frequencies of the lower and upper passbands, fo is the stopband center while fq is the quarter wavelength frequency which sets the commensurate length of the filter elements. The classical LP Prototype-to-Lumped BSF mapping can be adopted to define a LP Prototype-to Distributed BSF mapping by replacing the radian frequency  with Richards transformed frequency  as shown in the figure. This transformation converts the shunt capacitors into shunt arm series OC and SC stubs, with impedances shown in the figure. The series combinations of SC and OC stubs forms distributed resonators forming Finite Transmission Zeros (FTZ’s) which shapes the stopband of the filter. The dual circuit is also shown in the same figure. In this circuit the resonators (FTZ’s) are formed by parallel combinations of SC and OC stubs in series arm. The nearest spurious stopband of such filters are located at 2fq-fo. By selecting fq properly the nearest spurious stopband can be placed anywhere above fo. If fq is selected close to fo then the second SB will be close to the main SB, forming a dual band BSF. For fq=2fo, the classical distributed element LPF will be obtained. For fq>2fo a wide upper passband is obtained which is the subject of this section.

On Filpro, design of such BSF’s is possible using either Built-in LC Prototype approach or Prototype with Inverters approach or User Defined Prototype approach. The BSF’s designed through any of the above mentioned approaches will involve ideal inverters. When the inverters are replaced by their practical equivalents responses degrade considerably. Therefore optimization is needed to recover the proper response in any case. Monte Carlo-Best Case routine of Filpro is a powerful and flexible tool for optimizing return loss responses, as will be described in the following examples.

Inverters of the resulting filter circuit can be approximated by transmission lines which are quarter wavelength long at any selected frequency fox. The response would then be preserved at and around fox. If fox is selected within the stopband then the response around the fundamental stopband is preserved, leading to degradation in the passbands. If it is selected within the upper passband then response in the fundamental stopband would be degraded. That is, in any case an optimization process is needed to recover the original response. Fortunately the optimization process converges smoothly always. The selection of fox is important as it also sets the topology of the circuit. For fox=fq the inverters would be converted into a single transmission line whose length is the same as the commensurate length of the filter, hence it can be named as Unit Element (UE). If fox=fq/2, then the resulting TL length would be twice the length of a single UE, hence inverters would be replaced by two UE’s, if fox=fq/3 then the resulting TL length would be three times the length of a single UE, hence three UE’s and so on. That is, the filter will become a commensurate length filter containing UE’s by selecting fox=fq/k where k is an integer (Figure 8.23).

Figure 8.23. Replacement of inverters with several Unit Elements.

It is found out that after optimization the contribution of UE’s to the response becomes comparable to the contributions of the resonators. Therefore the number of UE’s, hence the selection of approximation frequency fox is an the important design parameter. Since each UE increases degree of the transfer function of the filter by one, by replacing each inverter with several UE’s the total degree of typical filters can get to very large leading to considerable improvements in response compared to filters without UE’s.

For wide stopband filters the shunt arm series OC+SC stub resonators can be converted into two-step resonators as shown in Figure 8.24.a. For narrow stopband filters the adjacent UE+Stub resonator pairs can be converted into L-type or capacitively or inductively loaded parallel coupled line (PCL) resonators, as shown in Figure 8.24.b-to-e.

In the examples that we will be studying we will use Design / By Built-in LC Prototype approach to design Chebyshev type filters in which all FTZ’s are collected at stopband center. We will then tune the FTZ’s to relocate them at proper frequencies to get equal ripple stopbands. It is also possible to start the design as Generalized Chebyshev or elliptic filters. However these approaches give not much advantage because  approximation of the inverters by Unit Elements will degrade responses and hence adjustment of FTZ’s is necessary for such approaches also.

Figure 8.24. Transformations for realization of some circuit sections into two step, L-type, inductively loaded and capacitively loaded parallel coupled line resonators.

BSF’s will be treated in three groups as narrow-medium stopband and medium-wide stopband filters simply because the circuit transformations necessary for conversion of the topologies into realizable forms are different. Let’s start with narrow-medium stopband BSF’s.

8. 4. 2. NARROW-MEDIUM BANDWIDTH  BSF’S WITH WIDE UPPER PASSBANDS

We will consider the following BSF problem:

The stop band edge frequencies are specified as fs1=962 MHz and fs2=1206 MHz with higher than 40-50 dB  insertion loss.

The upper edge of the lower passband (fp1) should be above 900 MHz.

The lower edge of the upper passband (fp2) should be below 1275 MHz.

The return loss in the passbands should be above 15-18 dB.

In order to get a clear insight on the theory and problematic stages of the design and possible solution techniques, we will start with lower degree filters. Therefore the initial examples are just for searching purposes.

The upper limit of the upper passband is set by the quarter wavelength frequency fq. In the initial examples we will target the upper edge as 9000 MHz and then increase  it to 18000 MHz.

EXAMPLE 8. 17. Seven resonator medium bandwidth bandstop filter design with wide upper passband

In order to see the effects of medium stop bandwidth and wide upper passband, let’s consider a degree N=7 BSF with following specifications:

The stop band edge frequencies are fs1=962 MHz and fs2=1205 MHz with higher than 50 dB insertion loss.

The passband edge frequencies are fp1=900 MHz or above and fp2=1275 MHz or below.

The upper edge of the upper passband should be above 9000 MHz.

Return loss should be better than 18 dB in the passbands.

Stopband finite transmission zeros should be adjusted to give equal stopband ripple to get an elliptic response.

Selection of quarter wavelength frequency:

The passband center is fo=(fp1+fp2)/2 = 1087.5 MHz

The quarter wavelength frequency fq sets the upper passband center. In order to get upper edge of the upper passband above 9000 MHz, we should select fq somewhere about 5000 MHz. Although not a necessity, let’s select fq to be a multiple of fo. So, we set fq as

fq= 5fo = 5437.5 MHz

Design will be carried out by using Built in LC Prototype approach.

Design steps are shown in Figure 8.25.

- New / Design / By Built-in LC Prototype commands are invoked.

- Distributed- BSF / Chebyshev options are selected.

- Terminations are set as 50 ohms.

- Attenuation Specs / PB Specs+Prototype Degree option is selected and PBRipple= 0.05 dB fp1 = 900 MHz, fp2=1275 MHz, fq=5437.5 MHz and Degree n=7 are typed in.

- Find Circuit command gives the circuit shown in Figure 8.25.a. The circuit is made up of series and shunt arm stub resonators formed by OC and SC stubs.

- The series arm resonators can be converted to shunt arm resonators by applying the command Inverters / Replace by Inv. Eqv: K=50 ohms (Figure 8.25.b).

- Find Dual command gives the circuit shown in Figure 8.25.c. We will be mostly working on this structure.

- The wide span response shows that the nearest spurious SB is about 9400 MHz. The narrowband insertion loss response shows with the given specifications the main stopband loss between 962 MHz and 1205 MHz is about 35 dB. The loss is unnecessarily high at passband center. This is because all FTZ’s are collected at that frequency. Better stopband loss shapes can be formed by tuning FTZ’s to different locations. For example In Figure 8.25.d a circuit is formed with equiripple stopband loss giving a loss of about 80 dB within the targeted stopband. This is done by placing the cursor under the series arm SC+OC stub resonators and applying the command Tune Element / Tune Symmetric too? NO / Select Zsc / Keep FTZ constant? NO. Using this command for example the leftmost resonator can be tuned to create the leftmost FTZ, the next resonator the next FTZ, and so on. After these adjustments the circuit will no longer be symmetric. The stopband loss is increased to about 80 dB within the targeted band with minor degradations in skirts. The circuit is saved as BS100.

- In the next stage the inverters will be converted into TL pieces. A TL piece works as inverter at and around its quarter wavelength frequency. Since an inverter is ideal (frequency independent) element its conversion into a TL piece needs specification of the frequency at which the TL will work as inverter. If this frequency is selected within the stopband then the stopnband response will be preserved but the passband responses will degrade heavily. If it is selected within the upper passband then the stopband and lower passband responses will degrade heavily. Since we have a very wide frequency band in which we would like to preserve the response it seems that great degradations are unavoidable and optimization is necessary in any case. In order to carry out a systematic investigation, let’s consider the following cases.

Figure 8.25. A degree 7 bandstop filter with inverters.

a)  All inverters simulated by five Unit Elements

For this case the inverters are replaced by five TL pieces (5 UE’s) which are quarter wavelength long at fq=5437.5 MHz. Their total length of the five TL’s is a quarter wavelength at stopband center fo=1087.5 MHz. That is, effectively the inverters are approximated by TL pieces which are quarter wavelength long at fo=1087.5 MHz. Therefore this approximation preserves the response of the filter at and around the passband while great degradations arise in the passbands. This operation is described in Figures 8.26.a-b.

- With cursor under the end inverters the command Inverters / Replace Inverter By / Unit Elements / NUE=1: foInv=5437.5 MHz, fq=5437.5 MHz is applied to convert the inverters into TL pieces with length Lng=13.783 mm as shown in Figure 8.26.b. Experiments indicate that the end inverters are effective only for matching. Therefore they are replaced by a single UE.

- With cursor under the inner inverters the command Inverters / Replace Inverter By / Unit Elements / NUE=5 / foInv=1087.5 MHz, fq=5437.5 MHz is applied to convert the inverters into five TL pieces each with length Lng=13.783 mm (Figure 8.26.c).

- Since the filter elements have commensurate lengths the response will be periodic. Note the mirror image of the return loss response around fq=5437.5 MHz.

- The narrow span insertion loss response shown in the same figure shows that symmetry of the skirt slopes of the main stopband is preserved while the rejection level between the SB edge frequencies of 962 MHz-1205 MHz is about 80 dB.

- The return loss responses in both passbands are heavily degraded. The worst return loss is less than 3 dB. The passband return loss will be recovered by optimization using Monte Carlo-Best Case routine as described in Figure 8.26.d.

  - We place impedances of all TL’s and SC stub impedances (Zsc) of all resonators into the Monte Carlo List by invoking the command Monte Carlo / Automatic Operations / Add All First Parameters / Uniform Distribution / 5 % Parameter Tolerance. The parameter tolerance can be adjusted later depending on the progress of the optimization procedure

  - Then reference lines in both lower and upper passbands are placed by using the command Reference Lines / Add a Reference Line / Lower (Upper) Passband: fmin=0, fmax=900 MHz for lower PB and fmin=1275 MHz and fmax=6000 MHz for the upper PB. The levels of reference lines should be set low enough so that the optimization routine can easily catch it. In this case 3 dB level is good to ease the initial job of the optimizer (Figure 8.26.d).

  - Then Best Case command is applied to start optimization. Then Filpro asks Preserve Symmetry? because the circuit has a topological symmetry. We select NO option because the physical symmetry of the circuit is lost after tuning the FTZ’s. Then Filpro asks the question Keep FTZ’s constant? because during optimization the Zsc parameters of the resonators will also change leading to shifts in the FTZ’s. We select YES option because we would like to preserve the locations of the FTZ’s.

  - If the optimizer finds a solution which doesn’t touch to shaded regions then Filpro stops the search and if to continue optimization by changing reference level. We increase the reference level with very small steps (0.01 dB, 0.02 dB, 0.05 dB, 0.1 dB, ..) and increase or decrease the percentage tolerance slightly to continue optimization.

  - If no solution is found then we decrease the reference level slightly to help optimizer and/or change the percentage tolerance slightly. A slow and tedious procedure, but controllable and works well.

  - During the optimization the equiripple property of the stopband loss may be lost and need to be recovered occasionally by tuning the Zsc parameters of the resonators.

- Figure 8.26.e shows responses before and after the optimization. Iterations stopped when the return loss level reached at about 20 dB. The stopband loss is decreased from 80 dB to about 70 dB. We may continue to increase the return loss level further. However this would lead to further decrease the stopband loss. The resulting circuit is also shown in the same figure.

- Now we will transform the circuit into realization topologies involving parallel coupled line (PCL) resonators. Experiments show that if the Zsc parameter of a resonator is above 50-60 ohms then the even-odd mode impedances come out to be problematic to realize. Therefore we split such resonators into two as shown in Figure 8.26.e-f.

- Next, the Resonator +TL pairs will be converted into PCL resonators. If the resonator is on the left of a TL then with cursor under the TL the command Replace by PCL / I-L-U Resonators / OSsc-L / n=.. / Zstb=.. / Try again? is applied to convert the Resonator+TL pair into PCL resonator with OC stub (OS) on the left and SC (sc) on the right port of the coupled line. In this command the parameter n is specified by the user. The displayed default value of n yields symmetric PCL. When the default value of n is accepted then Filpro displays the stub impedance Zstb. If a different Zstb is desired then we can try a different n resulting in an asymmetric PCL. In this (and in most of the following) examples only symmetric PCL’s are considered.

- If the resonator is on the right of a TL then we apply the command Replace by PCL / I-L-U Resonators / scOS-L.  The resulting PCL resonator will have sc on its left and OS on its right.

- Applying these commands on all the resonators we get the circuit shown in Figure 8.26.g. We note that the coupled lines have realizable even and odd mode impedances. However the OC stubs have too low impedances. They can either be split into two to increase their impedances (leading to a more complex structure) or replace them by their lumped capacitor equivalents. The latter is done by placing the cursor under the PCL resonators and applying the command Replace by Lumped / fo=1087.5 MHz. The resulting circuit is shown in Figure 8.26.h. The upper passband return loss response is degraded heavily by some spurious stopbands. Response can be improved by another optimization stage which involves tuning of all parameters (too tedious on Filpro!). Just as a trial, only the TL impedances and lengths are optimized giving the responses shown in 8.26.i. So, each TL has a different length, varying between 9 mm and 16 mm. The passband return loss response is improved to about 12 dB while the stopband loss is still about 70 dB.

- The even mode impedances of the fourth and fifth PCL’s from left are about 194 ohms. These may create realization problems. They can be reduced to realizable levels at the expense of some loss in stopband rejection and passband return loss through another optimization stage (Exercise?).

- There are 32 elements. Assuming each having the same length of about 14 mm, the total filter length is about 32*1.4=44.8 cm with air dielectric. Seems too long. The shorter versions will be treated in the following cases.

Figure 8.26. Bandstop filter with all inverters simulated by five Unit Elements.

Figure 8.26. Bandstop filter with all inverters simulated by five Unit Elements.

Figure 8.26. Bandstop filter with all inverters simulated by five Unit Elements.

b) All inverters simulated by four Unit Elements

For this case the inverters are replaced by four TL pieces which are quarter wavelength long at fq=5437.5 MHz. The total length of the four TL’s is quarter wavelength at foInv=fq/4=1359.375 MHz. That is, effectively the inverters are approximated by TL pieces which are quarter wavelength long at foInv=1359.375 MHz. Since this frequency is far from the stopband center we expect great degradations in stopband region also. This operation is described in Figures 8.27.a-b.

- With cursor under the end inverters the command Inverters / Replace Inverter By / Unit Elements/ NUE=1: foInv=5437.5 MHz, fq=5437.5 MHz is applied to convert the inverters into TL pieces with length Lng=13.783 mm as shown in Figure 8.27.b.

- With cursor under the inner inverters the command Inverters / Replace Inverter By / Unit Elements / NUE=4 / foInv=1359.375 MHz, fq=5437.5 MHz is applied to convert the inverters into four TL pieces each with length Lng=13.783 mm (Figure 8.27.c).

- Since the filter elements have commensurate lengths the response will be periodic. Note the mirror image of the return loss response around fq=5437.5 MHz.

- The narrow span insertion loss response shown in the same figure shows the asymmetry of the skirt slopes of the main stopband due to approximation of inverters at a frequency above and away from the stopband center. The SB loss has still seven transmission zeros, but equiripple property is lost. The rejection between the stopband edge frequencies of 962 MHz-1205 MHz is about 70 dB. Equiripple property can be recovered by tuning the FTZ’s (resonators).

- The return loss responses in both passbands are heavily degraded. The worst return loss is less than 3 dB. The passband return loss will be recovered by optimization using Monte Carlo-Best Case routine as described in Figure 8.23.d.

  - We place impedances of all TL’s and SC stub impedances (Zsc) of all resonators into the Monte Carlo List by invoking the command Monte Carlo / Automatic Operations / Add All First Parameters / Uniform Distribution / 5 % Parameter Tolerance. The parameter tolerance can be adjusted later depending on the progress of the optimization procedure

  - Then reference lines in both lower and upper passbands are placed by using the command Reference Lines / Add a Reference Line / Lower (Upper) Passband: fmin=0, fmax=900 MHz for lower PB and fmin=1275 MHz and fmax=6000 MHz for the upper PB. The levels of reference lines should be set low enough so that the optimization routine can easily catch it. In this case 1 dB or less is good to ease the initial job of the optimizer (Figure 8.27.d).

  - Then Best Case command is applied to start optimization. Then Filpro asks Preserve Symmetry? because the circuit has a topological symmetry. We select NO option because the physical symmetry of the circuit is lost after tuning the FTZ’s. Then Filpro asks the question Keep FTZ’s constant? because during optimization the Zsc parameters of the resonators will also change leading to shifts in the FTZ’s. We select YES option because we would like to preserve the locations of the FTZ’s.

  - If the optimizer finds a solution which doesn’t touch to shaded regions then Filpro stops the search and if to continue optimization by changing reference level. We increase the reference level with very small steps (0.01 dB, 0.02 dB, 0.05 dB, 0.1 dB, ..) and increase or decrease the percentage tolerance slightly to continue optimization.

  - If no solution is found then we decrease the reference level slightly to help optimizer and/or change the percentage tolerance slightly. A slow and tedious procedure, but controllable and works well.

  - During the optimization the equiripple property of the stopband loss may be lost and need to be recovered occasionally by tuning the Zsc parameters of the resonators.

- Figure 8.27.e shows responses before and after the optimization. Iterations stopped when the return loss level reached at about 20 dB. The stopband loss is settled at about 60 dB. We may continue to increase the return loss level further. However this would lead to further decrease the stopband loss. The resulting circuit is also shown in the same figure.

- Now we will transform the circuit into realization topologies involving parallel coupled line (PCL) resonators. Experiments show that if the Zsc parameter of a resonator is above 50-60 ohms then the even-odd mode impedances come out to be problematic to realize. Therefore we split such resonators into two as shown in Figure 8.23.e-f.

- Next, the Resonator +TL pairs will be converted into PCL resonators. If the resonator is on the left of a TL then with cursor under the TL the command Replace by PCL / I-L-U Resonators / OSsc-L is applied to convert the Resonator+TL pair into PCL resonator with OC stub (OS) on the left and SC (sc) on the right port of the coupled line.

- If the resonator is on the right of a TL then we apply the command Replace by PCL / I-L-U Resonators / scOS-L.  The resulting PCL resonator will have sc on its left and OS on its right.

- Applying these commands on all the resonators we get the circuit shown in Figure 8.27.g. We note that the coupled lines have realizable even and odd mode impedances. However the OC stubs have too low impedances. They can either be split into two to increase their impedances (leading to a complex structure) or replace them by their lumped capacitor equivalents. The latter is done by placing the cursor under the PCL resonators and applying the command Replace by Lumped / fo=1087.5 MHz. The resulting circuit is shown in Figure 8.27.h. The upper passband return loss response is degraded heavily by some spurious stopbands. Response can be improved by another optimization stage which involves tuning of all parameters (too tedious on Filpro!). Just as a trial, only the TL impedances and lengths are optimized giving the responses shown in Figure 8.27.i. So, each TL has a different length, varying between 11 mm and 16 mm. The passband return loss response is improved to about 12 dB while the stopband loss is still about 60 dB.

- The even-odd mode impedances seem to be at realizable levels.

- There are 26 elements. Assuming each having the commensurate length of 14 mm (average), the total filter length is about 26*1.4=36.4 cm with air dielectric. Shorter than the previous solution, but it seems still too long (?).

Figure 8.27. Bandstop filter with all inverters simulated by four Unit Elements.

Figure 8.27. Bandstop filter with all inverters simulated by four Unit Elements.

Figure 8.27. Bandstop filter with all inverters simulated by four Unit Elements.

c)  All inverters simulated by three Unit Elements

For this case the inverters are replaced by three TL pieces which are quarter wavelength long at fq=5437.5 MHz (three UE’s). The total length of the three TL’s is a quarter wavelength at foInv=fq/3=1812.5 MHz. That is, effectively the inverters are approximated by TL pieces which are quarter wavelength long at foInv=1812.5 MHz. Since this frequency is far from the stopband center we expect great degradations in stopband region also. This operation is described in Figures 8.28.a-b.

- With cursor under the end inverters the command Inverters / Replace Inverter By / Unit Elements / NUE=1: foInv=5437.5 MHz, fq=5437.5 MHz is applied to convert the inverters into TL pieces with length Lng=13.783 mm as shown in Figure 8.28b.

- With cursor under the inner inverters the command Inverters / Replace Inverter By / Unit Elements / NUE=3 / foInv=1812.5 MHz, fq=5437.5 MHz is applied to convert the inverters into three TL pieces each with length Lng=13.783 mm (Figure 8.28.c).

- Since the filter elements have commensurate lengths the response will be periodic. Note the mirror image of the return loss response around fq=5437.5 MHz.

- The narrow span insertion loss response shown in the same figure shows the great asymmetry of the skirt slopes of the main stopband due to approximation of inverters at a frequency far from the stopband center. The SB loss has still seven transmission zeros, but equiripple property is lost. The rejection between the stopband edge frequencies of 962 MHz-1205 MHz is about 60 dB. Equiripple property can be recovered by tuning the FTZ’s (resonators) at the expense of further decrease in stopband loss level.

- The return loss responses in both passbands are heavily degraded. The worst return loss is less than 3 dB. The passband return loss will be recovered by optimization using Monte Carlo-Best Case routine as described in Figure 8.28.d.

  - We place impedances of all TL’s and SC stub impedances (Zsc) of all resonators into the Monte Carlo List by invoking the command Monte Carlo / Automatic Operations / Add All First Parameters / Uniform Distribution / 5 % Parameter Tolerance. The parameter tolerance can be adjusted later depending on the progress of the optimization procedure

  - Then reference lines in both lower and upper passbands are placed by using the command Reference Lines / Add a Reference Line / Lower (Upper) Passband: fmin=0, fmax=900 MHz for lower PB and fmin=1275 MHz and fmax=6000 MHz for the upper PB. The levels of reference lines should be set low enough so that the optimization routine can easily catch it. In this case 1 dB or less is good enough to ease the initial job of the optimizer.

  - Then Best Case command is applied to start optimization. Then Filpro asks Preserve Symmetry? because the circuit has a topological symmetry. We select NO option because the physical symmetry of the circuit is lost after tuning the FTZ’s. Then Filpro asks the question Keep FTZ’s constant? because during optimization the Zsc parameters of the resonators will also change leading to shifts in the FTZ’s. We select YES option because we would like to preserve the locations of the FTZ’s.

  - If the optimizer finds a solution which doesn’t touch to shaded regions then Filpro stops the search and if to continue optimization by changing reference level. We increase the reference level with very small steps (0.01 dB, 0.02 dB, 0.05 dB, 0.1 dB, ..) and increase or decrease the percentage tolerance slightly to continue optimization.

  - If no solution is found then we decrease the reference level slightly to help optimizer and/or change the percentage tolerance slightly. A slow and tedious procedure, but controllable and works well.

  - During the optimization the equiripple property of the stopband loss may be lost and need to be recovered occasionally by tuning the Zsc parameters of the resonators.

- Figure 8.28.e shows responses before and after the optimization. Iterations stopped when the return loss level reached at about 20 dB. The stopband loss is settled at about 44 dB. The resulting circuit is also shown in the same figure after splitting the resonators with high Zsc parameters.

- Next, the Resonator +TL pairs will be converted into PCL resonators. If the resonator is on the left of a TL then with cursor under the TL the command Replace by PCL / I-L-U Resonators / OSsc-L is applied to convert the Resonator+TL pair into PCL resonator with OC stub (OS) on the left and SC (sc) on the right port of the coupled line.

- If the resonator is on the right of a TL then we apply the command Replace by PCL / I-L-U Resonators / scOS-L.  The resulting PCL resonator will have sc on its left and OS on its right.

- Applying these commands on all the resonators we get the circuit shown in Figure 8.28.f. We note that the coupled lines have realizable even and odd mode impedances. However the OC stubs have too low impedances. They can either be split into two to increase their impedances (leading to a complex structure) or replace them by their lumped capacitor equivalents. The latter is done by placing the cursor under the PCL resonators and applying the command Replace by Lumped / fo=1087.5 MHz. The resulting circuit is shown in Figure 8.28.g.

- The upper passband return loss response is degraded heavily by some spurious stopbands. Response can be improved by another optimization stage which involves tuning of all parameters (too tedious on Filpro!). Just as a trial, only the TL impedances and lengths are optimized giving the responses shown in Figure 8.28.h. So, each TL has a different length, varying between 11 mm and 16 mm. The passband return loss response is improved to about 15 dB while the stopband loss is reduced to about 44 dB. Compared to the 60 dB rejection of the previous filter where each inverter was approximated by four UE’s, the decrease to 44 dB is completely due to decrease in the number of Unit Elements. So, at this point we note the contributions of the Unit Elements into the stopband loss in such filters.

- The even-odd mode impedances seem to be at realizable levels.

- There are 20 elements. Assuming each having the commensurate length of 14 mm (average), the total filter length is about 20*1.4=28 cm with air dielectric. Shorter than the previous solution, at the expense of appreciable reduction in stopband loss (from 60 dB to 44 dB).

Figure 8.28. Bandstop filter with all inverters simulated by three Unit Elements.

Figure 8.28. Bandstop filter with all inverters simulated by three Unit Elements.

Figure 8.28. Bandstop filter with all inverters simulated by three Unit Elements.

d)  All inverters simulated by two Unit Elements

For this case each inverter is replaced by two TL pieces which are quarter wavelength long at fq=5437.5 MHz. The total length of the two TL’s is a quarter wavelength at foInv=fq/2=2718.75 MHz. That is, effectively the inverters are approximated by TL pieces which are quarter wavelength long at foInv=2718.75 MHz. Since this frequency is very far from the stopband center we expect great degradations in stopband region while not much degradation would occur in the upper passband. This operation is described in Figures 8.29.a-b-c.

- With cursor under the end inverters the command Inverters / Replace Inverter By / Unit Elements / NUE=1: foInv=5437.5 MHz, fq=5437.5 MHz is applied to convert the inverters into TL pieces with length Lng=13.783 mm as shown in Figure 8.29b.

- With cursor under the inner inverters the command Inverters / Replace Inverter By / Unit Elements / NUE=2 / foInv=2718.75 MHz, fq=5437.5 MHz is applied to convert the inverters into two TL pieces each with length Lng=13.783 mm (Figure 8.29.c).

- Since the filter elements have commensurate lengths the response will be periodic. Note the mirror image of the return loss response around fq=5437.5 MHz.

- The narrow span insertion loss response shown in the same figure shows the great asymmetry of the skirt slopes of the main stopband due to approximation of inverters at a frequency very far from the stopband center. The SB loss has still seven transmission zeros, but equiripple property is lost. Equiripple property can be recovered by tuning the FTZ’s (resonators) at the expense of great reduction in stopband rejection level.

- The responses indicate great widening in the stopband. The stopband return loss must be narrowed down to fit into the window between fp1=900 MHz and fp2=1275 MHz while also improving the return loss of the passbands. This can be done by optimization using Monte Carlo-Best Case routine as described in Figure 8.29.d.

  - We place impedances of all TL’s and SC stub impedances (Zsc) of all resonators into the Monte Carlo List by invoking the command Monte Carlo / Automatic Operations / Add All First Parameters / Uniform Distribution / 30 % Parameter Tolerance. The parameter tolerance can be adjusted later depending on the progress of the optimization procedure

  - Then reference lines in both lower and upper passbands are placed by using the command Reference Lines / Add a Reference Line / Lower (Upper) Passband: fmin=0, fmax=900 MHz for lower PB and fmin=1275 MHz and fmax=6000 MHz for the upper PB. The levels of reference lines should be set low enough so that the optimization routine can easily catch it. In this case 0.001 dB is good enough to ease the initial job of the optimizer. During initial iterations the user can observe if the stopband return loss can be fit into the window or not and adjust the percentage tolerance and reference level. Once the return loss curve fits into the window then we can increase the reference level to speed up convergence.

  - Then Best Case command is applied to start optimization. Then Filpro asks Preserve Symmetry? because the circuit has a topological symmetry. We select NO option because the physical symmetry of the circuit is lost after tuning the FTZ’s. Then Filpro asks the question Keep FTZ’s constant? because during optimization the Zsc parameters of the resonators will also change leading to shifts in the FTZ’s. We select YES option because we would like to preserve the locations of the FTZ’s.

  - If the optimizer finds a solution which doesn’t touch to shaded regions then Filpro stops the search and if to continue optimization by changing reference level. We increase the reference level with very small steps (0.001 dB, 0.01 dB, 0.02 dB, 0.05 dB, 0.1 dB, ..) and increase or decrease the percentage tolerance slightly to continue optimization.

  - If no solution is found then we decrease the reference level slightly to help optimizer and/or change the percentage tolerance slightly. A slow and tedious procedure, but controllable and works well.

  - During the optimization the equiripple property of the stopband loss may be lost and need to be recovered occasionally by tuning the Zsc parameters of the resonators.

- Figure 8.29.e shows responses before and after the optimization. Iterations stopped when the return loss level reached at about 20 dB. The stopband loss is settled at about 25 dB. A great reduction compared to the 44 dB of the previous case. The resulting circuit is also shown in the same figure.

- Next, the Resonator +TL pairs will be converted into PCL resonators. If the resonator is on the left of a TL then with cursor under the TL the command Replace by PCL / I-L-U Resonators / OSsc-L is applied to convert the Resonator+TL pair into PCL resonator with OC stub (OS) on the left and SC (sc) on the right port of the coupled line.

- If the resonator is on the right of a TL then we apply the command Replace by PCL / I-L-U Resonators / scOS-L.  The resulting PCL resonator will have sc on its left and OS on its right.

- Applying these commands on all the resonators we get the circuit shown in Figure 8.29.f. We note that the coupled lines have realizable even and odd mode impedances. However the OC stubs have too low impedances. They can either be split into two to increase their impedances (leading to a complex structure) or replace them by their lumped capacitor equivalents. The latter is done by placing the cursor under the PCL resonators and applying the command Replace by Lumped / fo=1087.5 MHz. The resulting circuit is shown in Figure 8.29.g.

- The upper passband return loss response is degraded slightly. Response can be improved by another optimization stage which involves tuning of all parameters (too tedious on Filpro!). Just as a trial, only the TL impedances and lengths are optimized giving the responses shown in Figure 8.29.h. So, each TL has a different length, varying between 12 mm and 15 mm. The passband return loss response is improved to about 18 dB while the stopband loss is still about 25 dB. Compared to the 44 dB rejection of the previous filter where each inverter was approximated by three UE’s, the decrease to 25 dB is completely due to decrease in the number of Unit Elements.

- The even-odd mode impedances seem to be at realizable levels.

- There are 14 elements. Assuming each having the commensurate length of 14 mm (average), the total filter length is about 14*1.4=19.6 cm with air dielectric. Much shorter than the previous solutions, but it has too low stopband loss (only 25 dB).

Figure 8.29. Bandstop filter with all inverters simulated by two Unit Elements.

Figure 8.29. Bandstop filter with all inverters simulated by two Unit Elements.

Figure 8.29. Bandstop filter with all inverters simulated by two Unit Elements.

OBSERVATIONS:

- When the inverters are replaced with quarter wavelength TL pieces they act as contributing Unit Elements after optimization.

- A seven resonator filter can be designed to give a stopband loss of about 70 dB, with a physical length of about 45 cm by approximating each inverter by 5 Unit Elements.

- A seven resonator filter can be designed to give a stopband loss of about 60 dB, with a physical length of about 36 cm by approximating each inverter by 4 Unit Elements.

- A seven resonator filter can be designed to give a stopband loss of about 44 dB, with a physical length of about 28 cm by approximating each inverter by 3 Unit Elements.

- A seven resonator filter can be designed to give a stopband loss of about 25 dB, with a physical length of about 20 cm by approximating each inverter by 2 Unit Elements.

- The case where each inverter is approximated by one UE is not investigated as it will give a stopband loss less than 20 dB, hence useless.

- Clearly, a strong dependence exist between the equiripple stopband loss level and the number of Unit Elements, hence the physical length of the filter. Higher stopband loss requires longer filter (Higher number of Unit Elements).

- Naturally each inverter of a filter may be replaced by different number of Unit Elements to reach at the targeted  stopband loss level (Exercise).

- One may increase the number of resonators instead of increasing the number of Unit Elements to get similar stopband loss. However tests have to be made to get an idea on the comparative contributions of the extra resonators and extra Unit Elements. An optimum solution may be found.

- The resulting circuits show that the even and odd mode impedances get problematic (higher even mode and lower odd mode impedances) when the inverters are approximated by higher number of Unit Elements. When the even-odd mode impedances get impractical to realize then one may try splitting the resonators into two, as it was done in the above examples.

EXAMPLE 8. 18. Eleven resonator bandstop filter with upper passband extended to 18 GHz

Under the light of the experiments of the previous example, let’s now attempt to design a BSF with ultra wide second passband.

The stop band edge frequencies are fs1=962 MHz and fs2=1205 MHz with higher than 50 dB insertion loss.

The passband edge frequencies are fp1=950 MHz or above and fp2=1275 MHz or below.

The upper edge of the upper passband should be above 18000 MHz.

Return loss should be better than 16 dB in the passbands.

Stopband finite transmission zeros should be adjusted to give equal stopband ripple to get an elliptic response.

Selection of quarter wavelength frequency:

The quarter wavelength frequency fq sets the upper passband center. In order to get upper edge of the upper passband above 18000 MHz, we select fq as 9000 MHz.

fq= 9000 MHz

Since the upper passband width is challenging let’s select a high degree filter, for example an eleven resonator filter.

Using Built in LC Prototype approach we get the following results.

Design steps are shown in Figure 8.30.

- New / Design / By Built-in LC Prototype commands are invoked.

- Distributed- BSF / Chebyshev options are selected.

- Terminations are set as 50 ohms.

- From the previous experience we know that the stopband will shift upward after replacing the inverters with TL equivalents, making optimization a bit troublesome. In order to compensate that shift to some extent let’s design a narrower bandwidth filter with passband edges shifted downwards on frequency axis. Let’s try fp1=920 MHz, fp2=1070 MHz. Optimization will settle the passband edges to the desired locations.

- Attenuation Specs / PB Specs+Prototype Degree option is selected and PBRipple= 0.05 dB fp1 = 920 MHz, fp2=1070 MHz, fq=9000 MHz and Degree n=11 are typed in.

- Find Circuit command gives the circuit shown in Figure 8.30.a. The circuit is made up of series and shunt arm stub resonators formed by OC and SC stubs.

- The series arm resonators can be converted to shunt arm resonators by applying the command Inverters / Replace by Inv. Eqv: K=50 ohms (Figure 8.30.b).

- Find Dual command gives the circuit shown in Figure 8.30.c.

- As expected the stopband is shifted downward.

- Next, we will replace the inverters with their multiple TL equivalents. In the previous examples we have replaced all inner inverters with 5, 4 , 3 or 2 QW TL pieces. Just as exercise let’s replace inverters with different numbers of TL’s.

- The end inverters are replaced by a single QW TL by applying the command Inverters / Replace Inverter By / Unit Elements / NUE=1: foInv=9000 MHz, fq=9000 MHz. Length of the UE’s are Lng=8.3275 mm (Figure 8.30.d).

- With cursor under the second inverter from left end and from right end, the command Inverters / Replace Inverter By / Unit Elements / NUE=4 / foInv=2250 MHz, fq=9000 MHz is applied to convert the inverters into four TL pieces each with length Lng=8.3275 mm (Figure 8.30.d).

- With cursor under all other inverters, the command Inverters / Replace Inverter By / Unit Elements / NUE=3 / foInv=3000 MHz, fq=9000 MHz is applied to convert the inverters into three TL pieces each with length Lng=8.3275 mm (Figure 8.30.d).

- We could as well replace all inner inverters by four QW TL pieces to get higher SB loss, but the filter would be too long. Therefore just as a trial we have replaced only two of them by four QW TL pieces. If the SB loss comes out to be insufficient we may resort to replacing some or all inverters by four or five QW TL’s.

- The stop band insertion loss response can be made to fit the desired stopband range of fs1=962 MHz, fs2=1205 MHz in an equiripple manner by tuning the FTZ’s (resonators) as shown in Figure 8.30.d-e. The circuit becomes unsymmetric.

- The response indicate upward widening of the stopband. The stopband return loss must be narrowed down to fit into the window between fp1=950 MHz and fp2=1275 MHz while also improving the return loss of the passbands. This can be done by optimization using Monte Carlo-Best Case routine as described in the previous examples

- Figure 8.30.f shows responses after the optimization. Iterations stopped when the passband return loss level reached at about 16 dB. The stopband loss is settled at about 50 dB. The passband edge requirements at fp1=950 MHz and fp2=1275 MHz are satisfied approximately.

- Next, the Resonator +TL pairs will be converted into PCL resonators. Through the experience with the previous examples, we split the resonators with high Zsc parameters into two to avoid problematic even-odd mode impedances (Figure 8.30.f-g).

- Then with cursor under the TL’s on the left of the resonators, the command Replace by PCL / I-L-U Resonators / scOS-L is applied to convert the Resonator+TL pair into PCL resonator with SC (sc) on the left and Open Stub (OS) on the right port of the coupled line.

- Then with cursor under the TL’s on the right of the resonators, the command Replace by PCL / I-L-U Resonators / OSsc-L is applied to convert the Resonator+TL pair into PCL resonator with Open Stub  (OS) on the left and SC (sc) on the right port of the coupled line.

- Applying these commands on all the resonators we get the circuit shown in Figure 8.30.h. We note that the coupled lines have realizable even and odd mode impedances. However the OC stubs have too low impedances. They can be replaced by their lumped capacitor equivalents by placing the cursor under the PCL resonators and applying the command Replace by Lumped. The resulting circuit is shown in Figure 8.30.i.

- The upper passband return loss response is degraded slightly after replacement of stubs with capacitors. Response can be improved by another optimization stage which involves tuning of all parameters (too tedious on Filpro!). Just as a trial, only the TL impedances and lengths are optimized giving the responses shown in Figure 8.30.k. So, each TL has a different length, varying between 7.69 mm and 9.2 mm. The passband return loss response is improved to about 17 dB while the stopband loss is still about 50 dB.

- There are 34 elements. Assuming each having the commensurate length of 0.85 cm (average), the total filter length is about 34*0.85=29 cm with air dielectric.

Exercise: Convert the Resonator+TL pairs into asymmetric PCL’s by applying the command Replace by PCL / I-L-U Resonators / scOS-L (or OSsc-L) / n= .. / Zstb= .. / Try again?. Then convert the OC stubs into lumped capacitors. Inspect changes in even and odd mode impedances together with capacitors to see if any advantages can be obtained or not.

Figure 8.30. A BSF with upper passband extended to 18 GHz.

Figure 8.30. A BSF with upper passband extended to 18 GHz.

Figure 8.30. A BSF with upper passband extended to 18 GHz.

Figure 8.30. A BSF with upper passband extended to 18 GHz.

A trial to shift fp2 from 1275 MHz to 1250 MHz:

In order to see how the passband fp2 edge effects the stop band loss, an optimization is carried out by decreasing  fp2 from 1275 MHz to 1250 MHz. As seen in Figure 8.31, the stopband loss decreases from 50 dB to about 33 dB. This was expected because the FTZ’s becomes closer to the passband edge fp2.

Figure 8.31. Effect of decreasing fp2 from 1275 MHz to 1250 MHz on stopband loss.

EXAMPLE 8. 29. Thirteen resonator bandstop filter with upper passband extended to 18 GHz

Let’s now try to make fp2 closer to fs2 to make upper skirt slope closer to the lower skirt slope, for example with the following specs:

The stop band edge frequencies are fs1=962 MHz and fs2=1207 MHz with higher than 40 dB insertion loss.

The passband edge frequencies are fp1=950 MHz or above and fp2=1230 MHz or below.

The upper edge of the upper passband should be above 18000 MHz.

Return loss should be better than 15 dB in the passbands.

Stopband finite transmission zeros should be adjusted to give equal stopband ripple to get an elliptic response.

Selection of quarter wavelength frequency:

The quarter wavelength frequency fq sets the upper passband center. In order to get upper edge of the upper passband above 18000 MHz, we select fq as 9000 MHz.

fq= 9000 MHz

Let’s select degree n=13.

Using Built in LC Prototype approach we get the following results.

Design steps are shown in Figure 8.32.

- New / Design / By Built-in LC Prototype commands are invoked.

- Distributed- BSF / Chebyshev options are selected.

- Terminations are set as 50 ohms.

- From the previous experience we know that the stopband will shift upward after replacing the inverters with TL equivalents, making optimization a bit troublesome. In order to compensate that shift to some extent let’s design a narrower bandwidth filter with passband edges shifted downwards on frequency axis. Let’s try fp1=945 MHz, fp2=1130 MHz. Optimization will settle the passband edges to the desired locations. Better fp1 and fp2 values could be found by trials to ease the optimization process.

- Attenuation Specs / PB Specs+Prototype Degree option is selected and PBRipple= 0.05 dB fp1 = 945 MHz, fp2=1130 MHz, fq=9000 MHz and Degree n=13 are typed in.

- Find Circuit command gives the circuit shown in Figure 8.32.a. The circuit is made up of series and shunt arm stub resonators formed by OC and SC stubs.

- The series arm resonators can be converted to shunt arm resonators by applying the command Inverters / Replace by Inv. Eqv: K=50 ohms (Figure 8.32.b).

- Find Dual command gives the circuit shown in Figure 8.32.c.

- Next, we will replace the inverters with their QW TL equivalents.

- The end inverters are replaced by a single QW TL by applying the command Inverters / Replace Inverter By / Quarter Wavelength TL / Single QW TL: foInv=9000 MHz, fq=9000 MHz or by applying the command  Inverters / Replace Inverter By / Unit Elements / NUE=1 / foInv=9000 MHz, fq=9000 MHz to convert the inverters into TL pieces with length Lng=8.3275 mm as shown in Figure 8.32.d.

- Let’s replace all inner inverters with three QW  TL’s (3 UE’s). With cursor under all inner inverters, the command Inverters / Replace Inverter By / Unit Elements / NUE=3 / foInv=3000 MHz, fq=9000 MHz is applied to convert the inverters into three TL pieces each with length Lng=8.3275 mm (Figure 8.32.d).

- The stop band insertion loss response is made to fit the desired stopband range of fs1=962 MHz, fs2=1207 MHz in an equiripple manner by tuning the FTZ’s (resonators). The response after tuning is shown in Figure 8.32.e. The circuit becomes unsymmetric. The response indicate upward widening of the stopband. The stopband return loss must be narrowed down to fit into the window between fp1=950 MHz and fp2=1230 MHz while also improving the return loss of the passbands. This can be done by optimization using Monte Carlo-Best Case routine as described in the previous examples

- Figure 8.32.f shows responses when optimization is paused at about 10 dB return loss. For this return loss level the stopband loss is about 55 dB.

- Iterations stopped when the passband return loss level reached at about 15 dB (Figure 8.32.g). The 5 dB improvement in return loss caused a large decrease in stopband loss which is settled at about 40 dB. The passband edge requirements at fp1=950 MHz and fp2=1230 MHz are satisfied.

- Next, the Resonator +TL pairs will be converted into PCL resonators. With cursor under the TL’s on the left of the resonators, the command Replace by PCL / I-L-U Resonators / scOS-L is applied to convert the Resonator+TL pair into PCL resonator with SC (sc) on the left and Open Stub (OS) on the right port of the coupled line.

- Then with cursor under the TL’s on the right of the resonators, the command Replace by PCL / I-L-U Resonators / OSsc-L is applied to convert the Resonator+TL pair into PCL resonator with Open Stub  (OS) on the left and SC (sc) on the right port of the coupled line.

- Applying these commands on all the resonators we get the circuit shown in Figure 8.32.h. We note that the coupled lines have realizable even and odd mode impedances. However the OC stubs have too low impedances. They can be replaced by their lumped capacitor equivalents by placing the cursor under the PCL resonators and applying the command Replace by Lumped. The resulting circuit is shown in Figure 8.32.i.

- The upper passband return loss response is degraded very slightly after replacement of stubs with capacitors. Response can be improved by another optimization stage which involves tuning of all parameters (too tedious on Filpro!). Just as a trial, only the TL impedances and lengths are optimized giving the responses shown in Figure 8.32.k. So, each TL has a different length, varying between 7.66 mm and 9.2 mm. The passband return loss response is improved to about 16 dB while the stopband loss is still about 40 dB.

- We note that the 12 internal inverters are converted into 12*3=36 UE’s. Counting also the two UE’s at ports, thirteen resonators are accompanied by 38 UE’s. Counting each resonator as 2 degrees, the total degree of the filter is 38+13*2=64.

- The 13 stub resonators are converted into 13 PCL resonators with realizable even-odd mode impedances.

- There are 38 elements. Assuming each having the commensurate length of 0.85 cm (average), the total filter length is about 38*0.85=32.3 cm with air dielectric.

Exercise: Convert the Resonator+TL pairs into asymmetric PCL’s by applying the command Replace by PCL / I-L-U Resonators / scOS-L (or OSsc-L) / n= .. / Zstb= .. / Try again?. Then convert the OC stubs into lumped capacitors. Inspect changes in even and odd mode impedances together with capacitors to see if any advantages can be obtained or not.

Figure 8.32. A 13 resonator BSF with upper passband extended to 18 GHz.

Figure 8.32. A 13 resonator BSF with upper passband extended to 18 GHz.

Figure 8.32. A 13 resonator BSF with upper passband extended to 18 GHz.

Figure 8.32. A 13 resonator BSF with upper passband extended to 18 GHz.

EXAMPLE 8. 20. An eleven resonator filter with upper passband extended to 18 GHz

In order to get an idea on the effects of higher number of unit elements, let’s now try to satisfy the specs of the previous example with a BSF with lower number of resonators, but higher number of unit elements. For example let’s take the number of resonators as 11 and replace all inverters with four Unit Elements.

The specs are repeated:

The stop band edge frequencies are fs1=962 MHz and fs2=1207 MHz with higher than 40 dB insertion loss.

The passband edge frequencies are fp1=950 MHz or above and fp2=1230 MHz or below.

The upper edge of the upper passband should be above 18000 MHz.

Return loss should be better than 15 dB in the passbands.

Stopband finite transmission zeros should be adjusted to give equal stopband ripple.

Selection of quarter wavelength frequency:

The quarter wavelength frequency fq is selected as fq= 9000 MHz and degreee of the prototype is set as n=11.

Design steps are shown in Figure 8.33.

- New / Design / By Built-in LC Prototype commands are invoked and Distributed- BSF / Chebyshev options are selected. Terminations are set as 50 ohms.

- From the previous experience we know that the stopband will shift upward after replacing the inverters with TL equivalents, making optimization a bit troublesome. In order to compensate that shift to some extent let’s design a narrower bandwidth filter with passband edges shifted downwards on frequency axis. Let’s try fp1=920 MHz, fp2=1070 MHz. Optimization will settle the passband edges to the desired locations. {Better fp1 and fp2 values could be found by further trials to ease the optimization process.}

- Attenuation Specs / PB Specs+Prototype Degree option is selected and PBRipple= 0.05 dB fp1 = 920 MHz, fp2=1070 MHz, fq=9000 MHz and Degree n=11 are typed in.

- Find Circuit command gives the circuit shown in Figure 8.33.a. The circuit is made up of series and shunt arm stub resonators formed by OC and SC stubs.

- The series arm resonators are converted to shunt arm resonators by applying the command Inverters / Replace by Inv. Eqv: K=50 ohms (Figure 8.33.b).

- Find Dual command gives the circuit shown in Figure 8.33.c.

- Next, we will replace the inverters with their QW  TL equivalents (UE’s).

  - The end inverters are replaced by a single QW TL by applying the command Inverters / Replace Inverter By / Quarter Wavelength TL / Single QW TL: foInv=9000 MHz, fq=9000 MHz or by applying the command  Inverters / Replace Inverter By / Quarter Wavelength TL / Multiple QW TL / N=1 / foInv=9000 MHz, fq=9000 MHz to convert the inverters into TL pieces with length Lng=8.3275 mm as shown in Figure 8.33.d.

  - Let’s replace the inner inverters with 4 UE’s. With cursor under all inner inverters, the command Inverters / Replace Inverter By / Quarter Wavelength TL / Multiple QW TL / N=4 / foInv=2250 MHz, fq=9000 MHz is applied to convert the inverters into three TL pieces each with length Lng=8.3275 mm (Figure 8.33.d).

- The stopband loss response is made to fit the desired stopband range of fs1=962 MHz, fs2=1207 MHz in an equiripple manner by tuning the FTZ’s (resonators). The response after tuning is shown in Figure 8.33.e. The stop band insertion loss level is roughly 60 dB. The circuit becomes unsymmetric. The stopband return loss curve must be narrowed down to fit into the window between fp1=950 MHz and fp2=1230 MHz while also improving the return loss of the passbands. This is done by optimization using Monte Carlo-Best Case routine as described in the previous examples.

- Iterations stopped when the passband return loss level reached at about 16 dB for which case the stopband loss is settled at about 40 dB. The passband edge requirements at fp1=950 MHz and fp2=1230 MHz are satisfied (Figure 8.33.e-f).

- Next, the Resonator +TL pairs will be converted into PCL resonators. However we note that some of the stub resonator impedances are too high leading to unrealizable PCL even-odd mode impedances. Therefore we split them into two (Figure 8.33.f-g).

- Then with cursor under the TL’s on the left of the resonators, the command Replace by PCL / I-L-U Resonators / scOS-L is applied to convert the Resonator+TL pair into PCL resonator with SC (sc) on the left and Open Stub (OS) on the right port of the coupled line.

- Then with cursor under the TL’s on the right of the resonators, the command Replace by PCL / I-L-U Resonators / OSsc-L is applied to convert the Resonator+TL pair into PCL resonator with Open Stub  (OS) on the left and SC (sc) on the right port of the coupled line. We get the circuit shown in Figure 8.33.h.

- We note that the coupled lines have realizable even and odd mode impedances. However the OC stubs have too low impedances. They can be replaced by their lumped capacitor equivalents by placing the cursor under the PCL resonators and applying the command Replace by Lumped. The resulting circuit is shown in Figure 8.33.i.

- The upper passband return loss response is degraded slightly after replacement of stubs with capacitors. Response can be improved by another optimization stage which involves tuning of all parameters (too tedious on Filpro!). Just as a trial, only the TL impedances and lengths are optimized giving the responses shown in Figure 8.33.k. So, each TL has a different length, varying between 7.84 mm and 8.8 mm. The passband return loss response is still about 16 dB and the stopband loss is about 42 dB.

- We note that the eleven resonators are accompanied by 42 UE’s. Counting each resonator as 2 degrees, the total degree of the filter is 42+11*2=64.

- Since some of the stub resonator impedances led to unrealizable PCL even-odd mode impedances we had to split such resonators into two, thus increasing the total number of PCL resonators into 16. In effect instead of 11, we have now 16 PCL resonators.

- There are 42 elements. Assuming each having the commensurate length of 0.85 cm (average), the total filter length is about 42*0.85=35.7 cm with air dielectric.

Observations and conclusions:

Let’s denote the circuit of the previous example as Filter-1 and the circuit of this example as Filter-2. Comparing them we can note down the following conclusions:

- In Filter-1 each inverter is simulated by three UE’s. Thus, the 12 internal inverters produced 3*12=36 UE’s. Counting also the UE’s at ports, we have 38 UE’s. So the total degree of the filter is 38+2*13= 64, leading to a total length of about 33 cm.

- In Filter-2 each inverter is simulated by four UE’s. So the 11 resonators are accompanied by 42 UE’s, resulting in a degree 64 filter with total length of about 36 cm.

- Both filters satisfy the targeted passband and stopband specs.

- In Filter-1 all stub resonators led to realizable PCL resonator even-odd mode impedances. Hence each resonator is converted into a single PCL resonator.

- In Filter-2 some stub resonator impedances came out to be problematic and we have split them into two parts to get adjacent PCL resonator pairs with realizable even-odd mode impedances, increasing the total number of PCL resonators.  Such adjacent pairs of resonators may cause field interaction problems during implementation.

- Experiments show that if the inverters are simulated by higher number of UE’s then the resulting PCL even mode impedances get higher which may lead to realization problems.

- It seems that Filter-1 is advantageous for realization because,

  - It has less number of PCL resonators.

  - It has no adjacent PCL resonators.

  - It is 3 cm shorter in length.

- In Filter-2 the even and odd mode impedances of PCL resonators are closer to each other which means looser couplings, making their realization easier.

- The experiments with 18 GHz upper stopband filters indicate that contributions of UE’s to the stopband loss level are comparable to those of the resonators. Therefore once the total length (passband Insertion Loss) is roughly set, we have to find a balanced set of  “Number of Resonators and Number of UE’s” to satisfy the required specifications.

- The same experiments have also shown one or two UE representations of inverters have unsatisfactory contributions to the response. Although five or more UE representations improve the stopband loss considerably, they lead to unacceptable increase in the total filter length. Therefore for such filters it came out that the optimum solution may be to use a mixture of three and four UE representations for the inverters.

Figure 8.33. An eleven resonator BSF with upper passband extended to 18 GHz.

Figure 8.33. An eleven resonator BSF with upper passband extended to 18 GHz.

Figure 8.33. An eleven resonator BSF with upper passband extended to 18 GHz.

8. 4. 3. MEDIUM-WIDE  BANDWIDTH  BSF’S WITH EXTENDED UPPER PASSBANDS

Bandstop filters with wide stopbands and wide upper passbands are relatively easier to design because conversion of inverters into QW TL pieces introduce relatively less degradation into the upper passbands. Also impedances of the resonators come out to be realizable as stepped resonators.

Let’s see details on an example.

EXAMPLE 8. 21. Five resonator wide stopband BS filters with wide upper passband

Desired specifications are as follows:

Five resonator filter will be used.

PBRipple=0.05 dB (20 dB return loss)

Stopband edge frequencies are fs1=4600 MHz, fs2=5600 MHz with insertion loss will be set by the passband edge frequencies.

For the lower passband edge fp1, we will two different sets as: fp1=3750 MHz and fp1=4350 MHz.

For the upper passband egde fp2, we will try also two different frequencies: fp2=6350 MHz and fp2=6900 MHz. These passband edges will be forced through optimization.

However we need some trial values to start the design. We set them as fp1=3700 MHz, fp2=6500 MHz.

The upper edge of the upper passband should be above 18000 MHz. So, the quarter wavelength frequency fq is selected as fq= 12500 MHz.

Design steps are shown in Figure 8.34.

- New / Design / By Built-in LC Prototype commands are invoked and Distributed- BSF / Chebyshev options are selected. Terminations are set as 50 ohms.

- Attenuation Specs / PB Specs+Prototype Degree option is selected and PBRipple= 0.05 dB fp1 = 3700 MHz, fp2=6500 MHz, fq=12500 MHz and Degree n=5 are typed in. The targteded passband edges will be obtained by optimization.

- Find Circuit command gives the circuit shown in Figure 8.34.a. The circuit is made up of series and shunt arm stub resonators formed by OC and SC stubs.

- The series arm resonators are converted to shunt arm resonators by applying the command Inverters / Replace by Inv. Eqv: K=50 ohms (Figure 8.34.b).

- This is a degree 10 filter where the degree is set by the five FTZ’s, all at the stopband center fo. The total number of transmission zeros is thus 2*5=10. In S=jTan (w/2wq) domain the locations of the FRZ’s are geometrically symmetric about the stopband center because the filter is mapped from a LP prototype. This property is also seen in the frequency response along f-axis in the range from f=0 to f=fq (S=). There are six reflection zeros (RZ’s) spread to the range f=0 and f=fq, one at f=0, one at f=fq (S=) and the rest four at finite frequencies (FRZ’s), two in lower passband and two in upper passband. Counting the FRZ’s as degree two, the total number of RZ’s is  2*4+1+1=10, the same as the total TZ’s.  {In calculating the degree, the RZ at f=fq should be counted as a unity degree because actually it is the single RZ at S=}

- With cursor under the resonators the command Step Resonators is applied to convert them into two step resonators (Figure 8.34.c). The circuit is saved as  BS55.fil.

- Since all transmission zeros (TZ’s) are collected at a single frequency at stopband center fo, insertion loss is greatest around fo and decreasing fast towards the stopband edges. Better insertion loss responses can be obtained by moving the TZ’s to proper locations within the desired stopband between fs1=4600 MHz and fs2=5600 MHz leading to different circuits. For example it is possible to move the five TZ’s to all different locations to get an equiripple stopband but asymmetric filter or pairs of TZ’s are  located at the same frequencies giving a symmetric filter.

A)   Asymmetric solutions:

- The stopband insertion loss response is made to fit the desired stopband range of fs1=4600 MHz, fs2=5600 MHz in an equiripple manner by placing the cursor under the stepped resonators and tuning them by applying the command Tune Element / Tune Symmetric Too / Select impedance Z1 of step-1 for tuning / Keep FTZ Constant? NO. In order to see the stopband range clearly, a reference line is placed by applying the command Reference Lines / Add A Reference Line / Stop Band: fmin=4600 MHz, fmax=5600 MHz, Ref. Level=70 dB / Pass Region: 1. The response after tuning is shown in Figure 8.34.d. The stop band insertion loss level is above 70 dB. The circuit becomes unsymmetric. We get an equiripple stopband response with five FTZ’s (hence degree 10 filter!). However the passband edges are shifted considerably. The circuit is saved as  BS56.fil.

Next, we will replace the inverters with their Unit Element (UE) equivalents which are nothing but transmission line pieces which are quarter wavelength long at f=fq=12500 MHz. At this point we have several options. Each inverter can be replaced by one, two or more UE’s leading to different alternative solutions.  Let’s see some typical ones.

Inverters simulated by two Unit Elements:

- All inverters are replaced by a their Two UE equivalents by applying the command Inverters / Replace Inverter By / Unit Elements/ NUE = 2 / foInv=6250 MHz, fq=12500 MHz. Each inverter is converted into two TL pieces as shown in Figures 8.34.e-f. Here foInv is the frequency of approximation of the inverters by two UE sections. The response is preserved at and around that frequency. This is seen from the return loss plot which is degraded in stopbands and both passbands except at and around foInv=6250 MHz. We also note that this conversion decreases the upper skirt slope of the stopband insertion loss curve. This is because foInv is in the upper side of the stopband. We will try to recover both responses by optimization.

- Since the inner TL’s act as contributing UE’s, we expect the degree of the filter to increase. Degree increase can only be seen from the increase in the number of Reflection Zeros because the Transmission Zeros of UE’s are NOT on s=jw axis. However since the response is not optimized yet, it may not be possible to clearly see all the Reflection Zeros. Even in this nonoptimum circuit we see that the number of  Reflection Zeros in the upper passband (up to fq=12500 MHz) is increased from 3 to 4 (including the RZ at f=fq). The number of RZ’s in the lower passband is increased from three to 4. It seems that some RZ’s need be recovered through optimization.

- The TL pieces at ports are redundant UE’s for the time being because they are 50 ohms. After optimization they will also become contributing UE’s.

- The stopband return loss curve must be narrowed down to fit into the targeted window between fp1 and fp2 while also improving the return loss of the passbands. This is done by optimization using Monte Carlo-Best Case routine as described in the previous examples. Let’s try two different passband edge sets: fp1=3750 MHz, fp2=6900 MHz, to allow low skirt slopes and fp1=4250 MHz, fp2=6300 MHz for shaper skirt slopes.

Optimization with fp1=3750 MHz and fp2=6900 MHz:

In order to settle the passband edges to these frequencies we place two Reference Lines using the following commands:

- Reference Lines / Add a Reference Line / Lower Passband: fmin=0, fmax=3750 MHz, Ref. Level: 0.1 dB, Pass Region: 1.

- Reference Lines / Add a Reference Line / Upper Passband: fmin=6900, fmax=18500 MHz, Ref. Level: 0.1 dB, Pass Region: 1

So, we place some walls (shaded regions or forbidden regions) on return loss responses of the passbands. The initial return loss level is kept very small. It is set as 0.1 dB in the above commands. If necessary we may also set them as 0.01 dB or 0.005 dB so that the optimization routine can easily catch a solution which is not touching the shaded region.

- Return Loss plot scales are adjusted by using the command Plot / Return Loss / fmin=0, fmax=12500 MHz, RLmin=0 dB, RLmax=1 dB. The RLmax is also set small so that the shaded region can be seen clearly. Since response is mirror imaged about fq=12500 MHz, we plot only the left half, between f=0 and f=fq.

- Delete Reference Line of the stopband if there is any because optimization will be performed on Return Loss plots by comparing responses with reference lines of the passbands only.

- Then the command Monte Carlo / Automatic Oprs / Add All First Parameters / Uniform Distribution / Tolerance= ?? % is applied to form a Monte Carlo List in which only the first parameters of all elements will be included. That is, only impedances of TL’s and the first impedance Z1 of the stepped resonators will be included in the list. These parameters will be randomly varied within the  specified percentage tolerance. The second step impedance Z2 of the step resonators will automatically be included because Z1 and Z2 set the resonance frequency of the resonators (Finite Transmission Zeros) and during optimization, the question “Keep FTZ’s Constant?” will be asked. Then Z2 will be changed accordingly. The tolerance is set by trials.

- Next, Best Case command is applied to start optimization procedure which changes the Monte Carlo Parameters randomly, calculates and plots return loss responses and compares with the forbidden range levels. If a solution is encountered which is not touching the forbidden range then the search stops declaring “A Solution is found. Continue Iteration?”.  If a solution can’t be found then iterations stops after 100 iterations and Filpro declares “No Solution. Continue Iteration?”. In either case the Reference level and Parameter percentage Tolerance is adjusted very slightly, increased if a solution is found and decreased if a solution can’t be found so that optimizer can catch the solution in the next iteration cycle.

- As the optimization proceeds successfully the reference level increases and the return loss vertical scale needs adjustment. Also the equal ripple property of the stopband insertion loss degrades. Therefore it may be necessary to retune the FTZ’s occasionally to recover the equiripple property.

- Figure 8.34.g shows the resulting circuit and responses when optimization is stopped after return loss level reached at about 27 dB with the stopband return loss being at about 75 dB level.

- Since all UE’s are now contributing to the response, counting each UE  as degree 1, and FTZ’s as degree 2 sections we now have a degree 22 filter. Therefore theoretically it is possible to continue optimization until all reflection zeros are recovered. However some of the return loss peaks may be hidden well near the stopband edges which is difficult to recover by brute force optimization. Therefore iterations are stopped after getting sufficient return loss level.

- Slight asymmetry is skirt slopes of the stopband insertion loss is clear. This is because of the selection of foInv within the upper passband. The lower skirt slope is sharper. In the next case we will try to sharpen the upper skirt slope also, by setting the upper passband edge fp2 closer to the stopband edge fs2.

- The impedances of stpped resonators are mostly in realizable ranges. The leftmost and rightmost stepped resonators are too high and may be difficult to realize. Such stepped resonators, together with one of the adjacent TL piece, can be converted into parallel coupled line (PCL) resonators as described in Figure 8.34.g-h. With the cursor under a TL adjacent to the high impedance stepped resonator, the command Replace by PCL / I-L-U Resonators / SSoc-L (or ocSS-L) / n=nsym / Zstub=..  is applied. The stepped resonator+TL pair is converted into L-type PCL resonator. In this resonator the second line (coupled line) is terminated in oc at one port and in SC stub (SS) in the other port. Hence the terminology ocSS-L or SSoc-L. In applying this command a parameter n is asked by Filpro. If the displayed default value is selected for n then a symmetric PCL will result. In symmetric PCL’s Zev1=Zev2 and Zod1=Zod2. That is the even and odd mode impedances of the two lines will be equal. For any other value of n  asymmetric PCL’s will result which are described through the parameters Zev1, Zev2 and Zod1, Zod2. Once the parameter n is specified then Filpro calculates the impedance of the resulting SC stub and displays it. If it comes out to be realizable then conversion into PCL is completed. If the SC stub impedance comes out to be unrealizable then another value is typed in for n. Figure 8.34.h shows the circuit after converting the high impedance stepped stubs into symmetric PCL resonators.

- Figure 8.34.i shows a realization schematics.

Optimization with fp1=4250 MHz and fp2=6350 MHz:

- In order to get sharper skirt slopes the reference lines are placed closer to the stopband edges, at fp1=4250 MHz and fp2=6350 MHz and optimization is repeated.

- The circuit and responses shown in Figure 8.34.k are obtained. Iterations are stopped after about 25 dB passband return loss. The price paid for sharpening the skirt slopes appears to be great reduction in the stopband insertion loss level which is dropped from 75 dB to about 46 dB. This should be expected because in effect we are moving the FTZ’s of the stopband closer to the passband edges.

- Another disadvantage of this filter shows itself in the high impedances of the stepped stubs. They are converted into L-type PCL resonators as shown in Figure 8.81.m.

- Figure 8.34.n shows a realization schematics.

Figure 8.34. Wide stopband wide upper passband BSF’s with inverters simulated by two Unit Elements.

Figure 8.34. Wide stopband wide upper passband BSF’s with inverters simulated by two Unit Elements.

Figure 8.34. Wide stopband wide upper passband BSF’s with inverters simulated by two Unit Elements.

Inverters simulated by three Unit Elements:

Figure 8.35 shows design steps when the inverters are converted into three Unit Elements.

- The circuit of  BS56 is loaded.

- With cursor under the inverters the command Inverters / Replace Inverter By / Unit Elements/ NUE=3 / foInv=4166 MHz, fq=12500 MHz. Each inverter is converted into three TL pieces as shown in Figures 8.35.a-b. Here foInv=4166 MHz is the frequency of approximation of inverters by three UE sections. This frequency is close to but lower than the stopband center. Therefore degradations in stopband insertion loss and lower passband return loss are not great while we have great degradation in upper passband return loss. We also note that this conversion decreases the lower skirt slope of the stopband insertion loss curve. This is because foInv is in the lower side of the stopband. We will try to recover responses by optimization.

- We place the lower passband reference line edge at fp1=3750 MHz. Since upper skirt slope is higher we can place the reference line for upper passband closer to the upper stopband edge, for example at fp2=6300 MHz.

- After optimization we get the circuit and responses shown in Figure 8.35.c. We get about 70 dB stopband loss. Upper and lower skirt slopes are close to each other. The most important aspect of this solution is that the edge of the upper passband, fp2=6300 MHz. In the previous approach where each inverter is converted into two UE’s, when fp2 is made 6350 Hz the stopband loss decreased to 46 MHz. Now we have 70 dB loss with fp2=6300 MHz. That is, if sharper upper skirt slopes are needed then it is better to use three UE version.

- Some of the stepped resonators have high impedances. They can be converted into parallel coupled line equivalents with SC stubs as shown in Figure 8.35.d.

- We note that the first UE on source side has impedance of 50 ohms. This tempts us to reduce the number of unit elements at source and load ends. For example we may place only one UE at both ends of the filter and reoptimize  the response (Exercise). This will lead to some reduction in the length of the filter.

- It is also possible to convert some inverters into two UE sections and the others into three UE sections to reduce the length further at the expense of some sacrifice in response (Exercise).

Figure 8.35. Wide stopband wide upper passband BSF’s with inverters simulated by three Unit Elements.

Inverters simulated by one Unit Element:

If length of the filter is more important then we may replace each inverter by one Unit Element as shown in Figure 8.36.

- The circuit of  BS56 is loaded.

- With cursor under the inverters the command Inverters / Replace Inverter By / Unit Elements/ NUE=1 / foInv=12500 MHz, fq=12500 MHz. Each inverter is converted into one TL piece as shown in Figures 8.36.a-b. Here foInv=12500 MHz is the frequency of approximation of inverters by one UE. This frequency is very far from the stopband center, leading to great degradations everywhere except the upper passband center. We also note that this conversion decreases the upper skirt slope of the stopband insertion loss curve and the lower edge of the upper passband (fp2) is shifted upward appreciably. This is because foInv is very high, too far from the stopband region. We will try to recover responses by optimization.

- We place the lower passband reference line edge at fp1=3750 MHz.

- Since upper skirt slope is low and fp2 is shifted upward the reference line for upper passband is also selected a bit far from the stopband edge, for example at fp2=6900 MHz.

- After optimization we get the circuit and responses shown in Figure 8.36.c. We get about 30 dB stopband loss with 22 dB passband return loss. The great reduction in stopband loss is due to reduction in the number of UE’s indicating that in the previous filters the UE’s contributed to the stopband loss appreciably.

- Upper skirt slope is still too low compared to lower skirt.

- WE note that all stepped resonators have high impedances. Therefore they are converted into PCL resonators with SC stubs as shown in Figure 8.36.d.

- Figure 8.36.e shows a realization schematics.

- Any attempt to sharpen skirts lead to further reduction in stopband loss. For example in Figure 8.36.f the results of an optimization with fp1=4250 MHz and fp2=6350 MHz. The stopband insertion loss decreased to about 22 dB with 15 dB passband return loss.

Figure 8.36. Wide stopband wide upper passband BSF’s with inverters simulated by one Unit Element.

Figure 8.36. Wide stopband wide upper passband BSF’s with inverters simulated by one Unit Element.

B) Symmetric solution:

A physically and electrically symmetric solution can be formed by placing pairs of FTZ’s at the same frequencies as described below.

- The stopband insertion loss response can be shaped by moving the TZ’s to any location within the stopband set by the frequencies fs1 and  fs2. There are five TZ’s. If pairs of TZ’s are moved to the same location, we can still form an equiripple stopband, but the TZ’s collected at three frequencies only.

- The circuit of BS55 is loaded (Figure 8.37.a).

- In order to see the stopband range clearly, a reference line is placed by applying the command Reference Lines / Add A Reference Line / Stop Band: fmin=fs1=4600 MHz, fmax=fs2=5600 MHz, Ref. Level=70 dB / Pass Region: 1.

- With cursor under the leftmost stepped resonators the command Tune Element / Tune Symmetric Too / Select impedance Z1 of step-1 for tuning / Keep FTZ Constant? NO is applied to move the pair of FTZ’s created by the leftmost and the symmetric (rightmost) stepped resonators towards fs1.

- Then with cursor under the second resonator from left the same command is applied to move the pair of FTZ’s formed by that resonator and its symmetric counterpart towards fs2 until roughly equal ripple stopband is formed.

- The middle stepped resonator may be left as it is or tuned slightly. It creates the FTZ at the center of the stopband.  Thus, the FTZ’s are collected at three distinct frequencies creating an equiripple stopband with 70 dB loss as shown in Figure 8.37.b. The passband edges are shifted considerably.

- Next, the inverters will be converted into UE equivalents. Let’s consider only the case where all inverters are replaced by their two UE equivalents. (Exercise with other cases ). This is done by placing the cursor under the inverters and applying the command Inverters / Replace Inverter by / Unit Elements / NUE=2 / foInv=6250 MHz, fq=12500 MHz.

- We delete one UE from each end of the filter to reduce size of the filter.

- After convrsion of inverters into UE’s the stopband response becomes asymmetric. The upper skirt is tilted down because the inverters are approximated by UE’s at a frequency higher than the passband center. Return loss response is also degraded together with large shifts in passband edges as shown in Figure 8.37.c.

- The passbands can be adjusted through optimization. Many different solutions can be formed by placing reference lines in both lower and upper passbands for setting fp1 and fp2. In this example we set fp1=3950 MHz and fp2=6500 MHz, aiming at a roughly symmetric stopband insertion loss response.

- Just for curiosity, optimization is continued to improve the return loss as far as it goes until all reflection zeros are recovered. Results are shown in Figure 8.37.d. After about 28 dB return loss, all possible reflection zeros are recovered. There are 5 RZ’s in the lower passband, one at f=0, contributing a single degree and 4 FRZ’s contributing 8 degrees. In the upper passband there are 6 RZ’s, one at f=fq=12500 MHz contributing single degree and 5 FRZ’s contributing 10 degrees. Hence we get a total degree of 1+4*2+5*2+1=20.  In another terminology, we can say that 10 degrees come from the 5 resonators while the rest 10 degrees come from the 10 UE’s. So all 10 UE’s contribute to the response.

- The stopband insertion loss decreased from 75 dB to 60 dB because fp1 and fp2 are made closer to fs1 and fs2 respectively.

Exercise: Don’t delete the UE’s at source and load ends. Apply optimization to see if it is possible to reach at 22 degrees.  This would mean those UE’s also would be contributing to the response.

- The passband and stopband edge constraints are all satisfied. However we note that the leftmost and rightmost stepped resonators have too high impedances (242 ohms). They can be converted into PCL resonators with SC stubs as shown in Figures 8.37.e-f-g. In another approach they can be tuned to realizable levels by applying the command Tune Element / Tune Symmetric Too / Keep FTZ Constant and then re-optimize the circuit. For example in Figures 8.37.g-h, impedance Z1 of the first (and last) resonator is tuned to 150 ohms. Tuning degrades the responses. Therefore we have to repeat optimization, but now excluding the tuned resonator (to prevent increasing its impedances again). This constraint limits the return loss to about 20 dB as shown in Figure 8.37.i. The SB loss increases back to about 70 dB. Naturally we loose some of the RZ’s.

Figure 8.37. Symmetric wide stopband and wide upper passband BSF.

Figure 8.37. Symmetric wide stopband and wide upper passband BSF.

8. 4. 4. OTHER BSF DESIGN APPROACHES ON FILPRO

Although the approach described in the previous subsections is capable of forming all possible transfer functions by adjusting the locations of FTZ’s, in some cases it may be advantageous to use some of the other techniques of Filpro. Just as exercise, let’s see some examples without considering realizability issues.

EXAMPLE 8. 22. BSF design using an elliptic LP Prototype

Specifications of the desired bandstop filter are set as follows:

The stop band edge frequencies are fs1=962 MHz and fs2=1206 MHz with higher than 60 dB insertion loss.

The passband edge frequencies are fp1=900 MHz or above and fp2=1275 MHz. The upper edge of the upper passband should be above 7000 MHz.

Return loss should be better than 18 dB in the passbands.

Seven resonators will be used.

Stopband finite transmission zeros should be adjusted to give equal stopband ripple, hence elliptic response.

Selection of quarter wavelength frequency:

The passband center is fo=(fp1+fp2)/2 = 1087.5 MHz

The quarter wavelength frequency fq sets the upper passband center. In order to get upper edge of the upper passband above 7000 MHz, we should select fq somewhere about 4000 MHz. Let’s select fq to be a multiple of fo. So, we set fq as

fq= 4fo = 4350 MHz

Design steps are shown in Figure 8.38. We will first design an elliptic LP prototype and then use this prototype to design the targeted BSF.

Synthesizing the LP prototype:

- New / Design / By Synthesis commands are invoked.

- Filter Type: Lumped-LP-Equiripple-Doubly Terminated-Type A options are selected.

- Passband and Terminations: PBRipple=0.05 dB, fp=0.159, RS=RL=1 are typed in. Here, fp=0.159 corresponds to wp=1 which is the passband edge frequency of the normalized LP prototype.

- Specify Transmission Zeros: Inf-FTZ1=0.22, FTZ2=0.26, FTZ3=0.44  are typed in. The FTZ’s are set using Tune a FTZ command to get equal stopband ripple at about 60 dB level.

- Extract TZ’s: Standard Auto Extract command gives the circuit shown in Figure 8.38.a. This is the desired LP prototype filter. It is saved as PR80.fil.

Design of the bandstop filter:

- New / Design / By User Defined Prototype commands are invoked.

- Load a Prototype option is selected and the prototype PR80 is loaded.

- Filter Type: Distributed –BSF options are selected.

- PB and Terminations: fp1=900 MHz, fp2=1275 MHz and fq=4350 MHz are typed in.

- Find Circuit command gives the circuit shown in Figure 8.38.b. The circuit is made up of series arm parallel OC+SC stub resonators and shunt arm fourth order sections. Note that the circuit is unsymmetric which is the case when all FTZ’s are located at different frequencies.

- Inverters / Replace by Inverter Eqv: K=50 command is used to convert the series arm parallel resonators into shunt arm series resonators as shown in Figure 8.38.c.

- The fourth order sections are split into two resonators using Split command as shown in Figures 8.38.c-d.

- The wide span response shows that the nearest spurious SB is above 7000 MHz. The narrow span insertion loss response shows the equiripple stopband loss of 60 dB with seven notches.

- Next, with cursor under the inverters the command Inverters / Replace Inverter By / Unit Elements / NUE=4: fo=1087.5 MHz, fq=4350 MHz is applied to convert each inverter into four UE’s which are TL pieces quarter wavelength long at fq=4350 MHz. We get the circuit shown in Figure 8.38.e. The four UE’s together approximates an inverter at stopband center fo=1087.5 MHz. Therefore negligible degradation will arise in and around the stopband while return loss in the passbands are heavily degraded. The passband return loss will be recovered by optimization. Note the mirror image of the return loss response around fq=4350 MHz.

- Return loss response can be improved by applying Monte Carlo-Best Case routine as was done in the previous examples.

- Iterations stopped after about 24 dB passband return loss level is reached. That is the original 20 dB return loss level is exceeded. We get the circuit shown in Figure 8.38.f. The circuit is saved as BS80A.fil. Responses are shown in Figure 8.38.g.

Now we will transform the circuit into possible realization topologies.

PCL resonators with Short Circuit Stubs:

- The circuit of BS80A is loaded (Figure 8.38.h).

- With cursor under the TL on the right (left) of an OC+SC stub resonator, the command Replace by PCL / I-L-U Resonators / SSoc-L (or ocSS-L) / n=.. / Zstb=.. / Try again? is applied to convert the Resonator+TL pair into PCL L-resonator with SC stub (SS) on the left port and OC (oc) on the right port of the coupled line ( reverse for ocSS-L type PCL). When the TL is on the left of the resonator then the option ocSS-L is selected. In applying this command Filpro asks a parameter n. If the displayed default value is accepted for n then a symmetric PCL will result. Filpro will display the resulting SC stub impedance. If it is a realizable value then we accept the displayed n. Otherwise a new value can be tried for n to get a realizable stub impedance.

- This command is repeated on all TL’s which are adjacent to resonators. The stub impedances are adjusted to be less than 150 ohms. We get the circuit shown in Figure 8.38.i. It is saved as BS80B.

- Figure 8.38.k shows a realization schematics.

Exercise: Place cursor under the PCL resonators and apply command Replace by Lumped. This command will convert the SC stubs of the PCL resonators into lumped inductors. Inspect the return loss response. Apply  optimization to improve return loss response.

Exercise: Split the single SC+OC stub resonators into two and repeat the conversions into PCL forms. Compare the even-odd mode impedance levels with those of the above example.

PCL resonators with Open Circuit Stubs:

- An alternative may be formed from the dual circuit. We apply Find Dual command on BS80A and save the resulting circuit as BS80C (Figure 8.38.m-n).

- With cursor under the TL’s on the right of resonators, the command Replace by PCL / I-L-U Resonators / OSsc-L / n=.. / Zstb=.. / Try again? is applied to convert the TL+Resonator combination into OC stub loaded parallel coupled line. In this command the notation OSsc-L implies right port of the coupled (second) TL is Short Circuited (sc) while its left port is loaded by a OC stub (OS). If the TL is on the left of a resonator then the option scOS-L should be selected. In applying this command Filpro asks a parameter n. If the displayed default value is accepted for n then a symmetric PCL will result. Filpro will display the resulting SC stub impedance. If it is a realizable value then we accept the displayed n. Otherwise a new value can be tried for n to get a realizable stub impedance.

- Targeting stubs impedances to be greater than 15 ohms, we get the circuit of Figure 8.38.p after applying this command to all Resonator+TL pairs.

- Figure 8.38.q shows a schematics for implementation.

- Inspecting the resulting even and odd mode impedances we see that most of the PCL’s are unrealizable in slab line form because the even mode impedances should be below 150 ohms, preferably below 130 ohms and odd mode impedances greater than 15 ohms. Hence the previous structure seems easier to realize (?). We may as well try the approaches used in the previous sections.

Exercise: Repeat conversion of the Resonator+UE combinations into PCL form by accepting default n leading to symmetric PCL’s. Then use Replace by Lumped command to convert the OC stub impedances into lumped capacitors. Inspect responses and use optimization to improve return loss.

Figure 8.38. BSF design using elliptic LP Prototype.

Figure 8.38. BSF design using elliptic LP Prototype.

Figure 8.38. BSF design using elliptic LP Prototype.

Figure 8.38. Elliptic BSF design with medium stopband and wide upper passband.

EXAMPLE 8. 25. Generalized Chebyshev BSF

Let’s treat the problem of the previous example by designing a quasi-elliptic solution by collecting the FTZ’s at the same frequency close to stopband edge. This version can be designed either by the User Defined Prototype approach of the previous example or through Generalized Chebyshev option of Built-in LC Prototype / approach. Let’s use the latter approach just as an exercise.

Specifications of the desired bandstop filter are repeated below:

The stop band edge frequencies are fs1=962 MHz and fs2=1206 MHz with higher than 60 dB insertion loss.

The passband edge frequencies are fp1=900 MHz or above and fp2=1275 MHz or below. The upper edge of the upper passband should be above 7000 MHz.

Return loss should be better than 18 dB in the passbands.

Seven resonators will be used.

Selection of quarter wavelength frequency:

The passband center is fo=(fp1+fp2)/2 = 1087.5 MHz

The quarter wavelength frequency fq sets the upper passband center. In order to get upper edge of the upper passband above 7000 MHz, we should select fq somewhere about 4000 MHz. Let’s select fq to be a multiple of fo. So, we set fq as

fq= 4fo = 4350 MHz

Design steps are shown in Figure 8.39.

- New / Design / By Built-in LC Prototype commands are invoked.

- Filter Type: Distributed- BSF-Generalized Chebyshev-1options are selected. In Filpro, Generalized Chebyshev LP prototype tables with one and three TZ’s at f= are available. These tables are taken from Rhodes and Alseyab [18]-[19]. In these prototypes all FTZ’s are placed at the same frequency leading to symmetric filters with narrow dynamic range for element values. Let’s select GenCheb-1 option which has one TZ at infinity and the rest at a finite frequency close to band edge to give the required stopband loss.

- Terminations are equal to 50 ohms.

- Attenuation Specs: fp1=900 MHz, fp2=1275 MHz, fq=4350 MHz and Prototype Degree N=7 are typed in and the options PBRipple=0.05 dB, SBIL=60 dB are selected.

- Find Circuit command gives the circuit shown in Figure 8.39.a. The circuit is made up of series arm parallel OC+SC stub resonators and shunt arm fourth order sections. Note that the circuit is symmetric which is the case when all FTZ’s are located at the same frequency.

- The wide span response shows that the nearest spurious SB is about 7400 MHz. The narrow span insertion loss response shows the equiripple stopband loss with three notches.

- Inverters / Replace by Inverter Eqv: K=50 command is used to convert the series arm parallel resonators into shunt arm series resonators as shown in Figure 8.39.b.

- The fourth order sections can be split into two resonators using Split command (Figures 8.39.c). The circuit is saved as BS82A. We can continue the design using this circuit following the same steps as in the previous example. However since we now have some experience with the previous example, let’s target the solution with PCL resonators with OC stubs or lumped capacitors. This solution is found from the dual version of BS82A found by applying Find Dual command (Figure 8.39.d). It is saved as BS82B.

The rest of the design steps are similar to those of the previous example (Exercise ).

Figure 8.39. Generalized Chebyshev BSF.

8. 5.  BANDSTOP  FILTERS USING BRANCHLINE  COUPLERS AND BANDPASS FILTERS

Branch line directional couplers can be used as two port bandpass or bandstop filters with unused ports left open circuited. In Figure 8.40.a schematics of a 3 dB branchline coupler is shown with element values (ideal inverters) adjusted for operation into 50 ohms terminations. This structure can be analyzed in Filpro in terms of its even and odd halves by treating port-1 as input and port-2 as output. Even-odd decomposition requires splitting of coupling inverters, as described in Figure 8.40.b, leading to the even and odd half circuits shown in Figure 8.40.c. The even and odd half circuits are saved as e.g. EVCPLR and ODCPLR.

Figure 8.40. Even and odd half decomposition of a 3 dB branchline coupler.

We will consider the structure obtained when a bandpass filter is bridged between ports 3 and 4, as described in Figure 8.41. It will be shown that this structure converts the bandpass nature of the bridging filter into a bandstop filter.

Figure 8.41. A bandstop filter structure obtained by bridging ports 3 and 4 by a bandpass filter.

- Even-odd mode analysis of this structure requires the bandpass filter to be symmetric. As an example, in Figure 8.42.a a lumped element bandpass filter is shown which is designed using Design / By Prototype with Inverters approach with specifications

fp1 = 95 MHz, fp2 =105 MHz, PBRipple= 0.05 dB, Prototype Degree= 4

In analysis of the composite structure we need the even and odd halves of this filter. They are also  
      shown in the same figure.

Figure 8.42. a) A symmetric bandpass filter. b)Bandpass filter with middle inverter replaced by its constant reactance Pi model. c)Even mode half of the filter. d) Odd mode half of the filter.

- Open and short circuits at the ends of even and odd halves of the bandpass filter are taken into consideration in the shunt constant reactance adjacent to the load resistance. The resistance appearing as load is assigned a very large value to be ineffective. {Actually RL should not exist there but programming difficulties impose existence of a resistance there}.

- Even and odd halves of bandpass filter are also saved with names e.g. EVBP and ODBP.

- The insertion and return loss responses of the bandpass filter are shown in Figure 8.43.

Figure 8.43. Insertion and return loss of the bandpass filter.

- Next, the even half of the coupler (EVCPLR) is loaded. The 50 ohms Port-3 (or port-4) resistance is deleted. In place of that resistance even half of bandpass filter is inserted by placing cursor at that position and applying command External 1-Port / Insert External 1-Port / EVBP.  Thus even half of the bandstop filter is formed. The resulting structure is saved as EVBS (Figure 8.44.a).

- The same procedure is repeated by loading the odd half of the coupler (ODCPLR )and the odd half of bandpass filter is inserted in place of the Port-3 resistance. Resulting circuit is saved as ODBS.fil (Figure 8.44.b).

Figure 8. 44. Even and odd halves of  the bandstop filter.

- Now the bandstop filter is ready. It’s response can be analyzed using the command set shown in Figure 8.45.  It is seen that response of bandstop filter is completely the same as bandpass filter, but with insertion and return loss curves interchanged.

- Thus, it is seen that the branchline coupler converts a bandpass filter connected in between its output ports into a bandstop filter.

Exercise: Replace ideal inverters by proper practical lumped element circuits to obtain a lumped element bandstop filter.

Exercise: Repeat the procedure for a distributed element bandpass filter and replace the ideal inverters of branchline coupler by proper stub elements to complete  the design of distributed element bandstop filter. Then repeat the design by replacing ideal inverters with quarter wavelength lines and compare responses. (See Chapter 20 for branchline couplers).

Exercise:  Repeat the design by using an impedance transforming branchline coupler to match 50 ohms source to a higher or lower impedance cross-coupled filter.

Exercise: Repeat the procedure by connecting a lowpass filter at the output ports.

Exercise: Repeat the design by placing a cross-coupled filter at the output ports. Can the second branch of coupler be integrated with elements of cross-coupled filter?

Exercise: Repeat the procedure by connecting a bandstop filter at the output port.

Figure 8.45. Insertion and return loss response of the bandstop filter.

REFERENCES AND FURTHER READING:

- Matthaei, G. L., Young L., E. M. Jones, “Microwave Filters, Impedance Matching Networks  and Coupling Structures”, Mc Graw Hill , NY, 1964.

- G. L. Matthaei, “ Tables of impedance transforming networks of lowpass filter form”, Proc. IEEE, vol. 52, pp. 939 - 963, Aug. 1964.

- R. J. Wenzel, “Exact Design of TEM Microwave Networks Using Quarter-Wave Lines”, IEEE Trans. MTT,  Vol. MTT-12, pp. 94 -111, Jan. 1964.

- E. G. Crystal, “Tables of Maximally Flat Impedance Transforming  Networks of lowpass filter form”, IEEE Trans. On  MTT ,  vol. MTT-13, No. 5,  pp. 693- 695,  Sept. 1965.

- M. C. Horton and R. J. Wenzel, “General Theory and Design of Optimum Quarter Wave TEM filters”, IEEE Trans. MTT,  vol. MTT-13, No. 3, pp. 316-327, May 1965.

- R. Levy, “A General Equivalent Circuit Transformation for Distributed Networks”, IEEE Trans. CT, Vol. CT-12, No 3,  pp. 457-458,Sept. 1965.

- B. M. Schiffmann and G. L. Matthaei, “Exact Design of Bandstop Microwave Filters”, IEEE Trans. MTT-12, pp. 6 -15,  Jan. 1965.

- R. Levy and I. Whiteley, “Synthesis of Distributed Elliptic Function Filters from Lumped Constant Prototypes”,  IEEE Trans. MTT, Vol. MTT-14, No. 11,  pp. 506-517, Nov. 1966.

- M. C. Horton and R. J. Wenzel, “The Digital Elliptic Filter- A Compact Sharp Cutoff Design for Wide Bandstop or Bandpass Requirements”, IEEE Trans. MTT, Vol. MTT-15, No 5,  pp. 307-314, May 1967.

- O. P. Gupta and R. J. Wenzel, “Design tables for a class of optimum microwave bandstop filters,” IEEE Trans. MTT-18, No. 7,  pp. 402-404, Jul. 1970.

- R. Levy, “A Generalized Design Technique for Practical Distributed Reciprocal Ladder Networks”, IEEE Trans. MTT, vol. MTT-21, No. 8, pp. 519 - 526, Aug. 1973.

- J. D.  Rhodes, “Theory of Electrical Filters”, J. Wiley and Sons, NY, 1976.

- J. A. G. Malherbe, “TEM Pseudo Elliptic Bandstop Filters Using Noncommensurate Lines”, IEEE Trans. MTT, Vol. MTT-24, No 5,  pp. 242-248, May 1976.

- J. A. G. Malherbe, “Realization of Elliptic Function Bandstop Filters by means of Resonated prototypes”,  IEEE Trans. MTT, Vol. MTT-25, No. 8, p. 717,  Aug.1977.

- R. N. Bates, “Design of Microstrip Bandstop Filters”, IEE Microwaves, Optics and Acoustics, pp. 209-214,  Nov. 1977.

- J. A. G. Malherbe, “Microwave Transmission Line Filters”, Artech House, 1980.

- L. Young, “Microwave Filters Using Parallel Coupled Lines”, Artech House,1980.

- J. D. Rhodes and S. A. Alseyab, “The Generalized Chebyshev Low Pass Prototype  Filters”, Int. J. Circuit Theory and Applications, Vol. 8, pp. 113-125, 1980.

- S. A. Alseyab, “A Novel Class of Generalized Chebyshev Low Pass Prototype for Suspended Substrate Stripline Filters”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-30, No. 9, pp. 1341-1347, Sept. 1982.

- C. Nguyen, C. Hsieh and D. W. Ball, “ Millimeter  Wave Printed Circuit Spurline Filters”, 1983 IEEE, MTT-S Proc., pp. 98 - 100, 1983.

- C. Nguyen and K. Chang, “Analysis and Design of Spurline Bandstop Filters”, 1985 IEEE, MTT-S  Conference Proc., pp. 445 - 448, 1985.

- H. C. Bell, “Narrow Bandstop Filters”, 1991 IEEE, MTT- S Conference Proc., pp.539 - 542, 1991.

- V. K. Tripathy, “Asymmetric Coupled Transmission Lines in an Inhomogeneous medium”, IEEE Trans. MTT, Vol. MTT- 23, No. 9,  pp. 734-739, Sept. 1975.

- V. K. Tripathy,  “Equivalent Circuits and Characteristics of Inhomogeneous Nonsymmetrical Coupled Line Two-Port Circuits”, IEEE Trans. MTT,  pp. 140-142,  Feb. 1977 .

- C. M. Tsai and K. C. Gupta, “A Generalized Model for Coupled Lines and its applications to Two-Layer Planar Circuits”, IEEE Trans. MTT, Vol. MTT- 40,  pp. 2190 - 2199, Dec. 1992.

- T. M. Weller and L. P. B. Katehi, “A millimeter wave micromachined lowpass filter using lumped elements”, IEEE, MTT-S, pp. 631-634, 1996.

- H. C. Bell, “L-Resonator Bandstop filters,” IEEE Trans. MTT-44, No. 12, pp. 2669-2672, Dec. 1996.

- R. V. Snyder and S. Shin, “Parallel coupled notch filters with wide spurious free passbands,” in IEEE MTT-S Int. Mic. Symp. Dig., 2005, Paper TU4A, CD ROM.

- R. Levy, R.S. Snyder and Sanghoon Shin, “Bandstop filters with extended upper passbands,” IEEE Trans. MTT-54, No. 6,  pp. 2503-2515, June 2006.