9. 8. HALF WAVELENGTH END COUPLED LINE FILTERS

End coupled line filters are the simplest distributed element bandpass filter structures made up of half wavelength line pieces coupled end to end as shown in Figure 9.43.a and b for microstrip and stripline version. Coupling is provided through electric field lines as described in Figure 9.43.c. The field lines starting from the end of one line and terminating on the other line are modelled as series capacitors between the two lines. The fringing fields from the ends of lines to ground plane are modelled as shunt capacitors. The half wavelength transmission lines are modelled as shunt LC resonators. Thus, the circuit model for the filter is shunt LC resonator filter with capacitive Pi-section couplings (Figure 9.43.d). Such filters may be treated as the generic simplest version of Class-C, D, E and F type Capacitively Loaded End Coupled Filters that will be studied in Chapter-12.  That is, if the Tlines of this filter are loaded by lumped capacitors then the other classes can be generated. This class of filters can be designed in several ways. One approach using Cohn’s method [1] is described in the following example. Then in the succeeding example Synthesis method will be used.

Figure 9.43. Microstrip and stripline end coupled line filters.

EXAMPLE 9. 29: An end coupled line filter design using Cohn’s method

Let specifications of the filter be

PBRipple=0.05 dB,  fp1=1900 MHz,   fp2=2000 MHz, Prototype Degree N=5.

The initial topology of the filter will be of the form parallel LC resonators coupled by ideal inverters. Later both inverters and LC resonators will be replaced by the proper equivalents to reach the targeted topology. Let’s use Design / By Built in LC Prototype approach. We may as well use Design / By Prototype with Inverters approach to get the same result.

- Figure 9.44 shows the command set. At stage Find Circuit, select the option Cohn: Inverter+pLCsh. This option asks the shunt capacitor values of each LC resonator in between inverters. It is usually better to select all of them equal. This will lead to identical shunt resonators. Let’s set all shunt capacitors arbitrarily at for example 1 pF, giving the circuit of Figure 9.44.a.

- Next, the shunt resonators are replaced by their half wavelength transmission line resonators by placing the cursor under the resonators and applying the command Replace By TLine: Z=…. Filpro asks characteristic impedance of the TLine. The displayed default value is the best in the sense that it keeps passband edges of return loss response unaffected from this approximation. We get 126 ohms for TLine impedances (Figure 9.44.b). This impedance can be set to any desired value by placing the cursor under these TLines and applying the command Inverter / Scale Element By Inverters / Z=…. Let’s set all TLine impedances to 70 ohms, as described in Figure 9.44.b-c.

- In the circuits of Figure 9.44.b-c the source and load are represented by lumped resistors RS and RL while actually the filter is fed by transmission lines at both ends. Therefore TLine pieces of 50 ohms impedance and arbitrary length (for example 20 mm) can be inserted at source and load ends without affecting the response. This operation is also described in Figure 9.44.a-b.

- Next the ideal inverters will be replaced by proper practical inverter circuits having a series arm lumped capacitor. In Filpro, there are several approximate equivalents of ideal inverters which involve series capacitors. One approach is described in Figure 9.44.c-e where the inverters are replaced by their capacitive Pi-section equivalents by applying the command  Inverters / Replace Ideal Inverter By / Pi-Tee/ Capacitive Pi /f1=1900, f2=2000.

- This approximation distorts return loss response but keeps passband edges unaffected, as seen from the plots shown in Figure 9.44.d.

- The shunt capacitors of the capacitive Pi-equivalents are negative (Figure 9.44.e) while we need positive shunt capacitors to account the fringing fields. We can follow a brute force approximation approach by setting the negative shunt capacitors to the desired positive values and then tune the lengths of TLines and series capacitors until the original return loss is recovered. This approach may work but with a big IF. These operations need the knowledge of shunt capacitor values. The series and shunt capacitors of a gap between two lines are dependent on the physical parameters like widths and thickness of lines, distances of lines to ground planes, gap length between lines, dielectric constants, frequency, etc. Worse, the series and shunt capacitors are interdependent through these parameters. Therefore usually they can’t be adjusted independently. For these reasons the design should be treated as an optimization problem in which as many of the physical parameters of TLines as possible are used as optimization parameters.

- Despite this fact, let’s continue our design just to get more insight on the problems by taking for example all shunt capacitors  as 0.01 pF.

- Consider the two negative shunt capacitors on the two sides of the second TLine in Figure 9.44.e. Using Change command their capacitance is set to 0.01 pF. Using Symmetrize command the symmetric counterparts are also set to 0.01 pF (Figure 9.44.f). Return loss response is degraded heavily, as seen from the figure. It can be recovered to a great extent by applying Tune Element command to the Length of the TLine between the two capacitors. This operation tunes the symmetric TLine also (Figure 9.44.g). That is, the changes in the reactances caused by brute force setting of capacitances are compensated by the change in the length of TLine.

- The same operation is applied on the next negative capacitor+TLine+negative capacitor triplet as shown in Figure 9.44.g-h, and then on the triplet in the middle (Figure 9.44.i). This way all the inner negative capacitors are set to 0.01 pF. In these operations the series capacitors may also need slight tuning.

- Finally, we set the negative shunt capacitors nearest to source and load to 0.01 pF. Since there is no tunable length TLine adjacent to these capacitors (the adjacent lines are 50 ohms, hence their lengths doesn’t effect response), return loss degradation should be corrected by tuning the adjacent series capacitors together with all the other series capacitors and TLine lengths, as described in Figures 9.47.k-m-n. Return loss response of the circuit of Figure 9.44.n is shown below the circuit schematics. It is equiripple but pass bandwidth is slightly shifted. This can be avoided by assigning slightly lower edge frequencies. Wideband response shows a spurious passband about twice the passband.

- We note another problematic point in this circuit: The first and last series capacitors are too large compared to inner capacitors. They may be difficult to realize by coplanar end coupled lines. This may be a limitation for bandwidth of filters that can be realized using coplanar end coupled lines. Narrower bandwidths will lead to smaller series capacitors (wider gaps).

Figure 9.44. Design steps of end coupled line filter.

Figure 9.44. Design steps of end coupled line filter (continued).

Figure 9.44. Design steps of end coupled line filter (continued).

EXAMPLE 9. 30. Capacitor or stub loaded end coupled line filters

At low frequencies end coupled line filters become too long, because the line pieces are half wavelength long. These lines may be shortened by loading them with stubs or lumped capacitors.  Let’s design the filter of the previous example in different forms to get exercise with different tools of Filpro.

- As a variation, Prototype with Inverters approach will be used. The command set is shown in Figure 9.45. The option Shunt Capacitors with Inverters is selected (Figure 9.45.a) leading to the bandpass filter shown in Figure 9.45.b. The LC resonators are converted into half wave line forms by applying the command Resonator Operations / pLCsh-to-Half Wave TLine (Figure 9.45.c). Characteristic impedances of half wave lines came out to be different. They can be equated by applying the command Inverters / All Equal Left-Right (Figure 9.45.d). Impedances are equated to the leftmost line impedance (4.04 ohms).

- TLine impedances are too low for realization. We can scale them up by applying the command Inverters / Scale Cct By Inserting Inverters / K=…. Here, inverter constant can be adjusted to scale up TLine impedances to any level. For example the value K=209 would give the same circuit as the previous example. However let’s follow a different approach to see some alternatives. We can replace the ideal inverters at source and load ends by their quarter wavelength TLine equivalents having the same impedance as K. Therefore let’s select a realizable impedance level, for example K=100 ohms which will become a 100 ohms TLine later. This inverter constant scales up the inner TLine impedances to 16.1 ohms which is still low for realization (Figure 9.45.e). However we will level them up soon, with another tool, as will be seen below.

- Place the cursor under the leftmost TLine of impedance   16.1 ohms and length 76.9 mm. Apply Split / Two Equal command to split it into two lines with halved lengths. Then apply the same command to halved pieces again. Thus this TLine will be split into four equal length line pieces each 19.2 mm long (rounded number!) (Figure 9.45.f).

- Use Combine command to combine the middle two pieces to form a TLine of length 38.4 mm (Figure 9.45.g). Thus, actually, the original TLine is split into three pieces, with the middle one having double the length of the outer ones.

- With cursor under the longer (middle) TLine apply command Replace TLine By / Tee Model / SSsr+Csh+SSsr to replace it with a series SC stub+shunt Capacitor+series SC stub triplet (Figure 9.45.h). With cursor under the TLine on the left of the Tee-section, apply command Kuroda / Right / Full Stub to convert the left series SC stub into a shunt OC stub on the left of the TLine. Similarly, place the cursor under the TLine on the right and apply command Kuroda / Left / Full stub to convert the right series SC stub into shunt OC stub (Figure 9.45.i).

- Now convert the shunt OC stubs into lumped capacitors by applying the command Replace By Lumped / Single C / f=1950 MHz (Figure 9.45.k).

- Thus, we have converted a TLine of impedance 16.1 ohms and length 76.9 mm into a TLine of 32.2 ohms impedance and halved length (38.4 mm), having lumped capacitive loadings.  This is a nice method for both increasing impedance and decreasing length of lines. Decreasing length may be an important issue for end coupled line filters, as they tend to be too long.

- The same approach is applied to the other TLines of the circuit (Figure 9.45.m). Next, we convert the inner inverters into their capacitive Pi-section equivalents with negative capacitors by applying the command Inverters / Replace Ideal Inverter By / Pi-Tee/ Capacitive Pi (Figure 9.45.n).

- The negative shunt capacitors are absorbed into the capacitors at the ends of the TLines by applying Simplify Circuit command (Figure 9.45.p).

- We could as well convert the inverters at terminations into capacitive Pi-sections, as was done in the previous example. However just as a variation, let’s convert them into quarter wavelength lines by applying the command Inverters / Replace Ideal Inverter By / Quarterwave Line (Figure 9.45.n-p). Thus, the design is finished on Filpro. Response is also shown in Figure 9.45.p. Passband is widened appreciably. This is because of replacement of shunt OC stubs by lumped capacitors which is not a good approximation for medium bandwidth filters (1900-2000 MHz filter is not narrow enough for such approximations!). The wideband response shows a spurious passband about 3000 MHz.

- Figure 9.45.q shows a pictorial schematics. In this structure the shunt capacitors of the gap can now be absorbed into the lumped capacitors at the ends of TLines.

- If lumped capacitors are not suitable for incorporation into the structure, then we can use the approach described in Figures 9.48.q-to-t.

- The circuit of Figure 9.45.i is repeated in Figure 9.45.q. With cursor under the lumped capacitors we apply the command Replace By Stub / Specify Impedance (or Specify Length) to replace lumped capacitors with OC stubs of realizable (and proper) impedances. The stub type circuit and its return loss response are shown in Figure 9.45.r. Contrary to the previous approach, no band edge shifts are observed in this case. That is lumped capacitors are well approximated by the OC stubs.

- The end inverters are replaced by quarter wavelength TLines while the inner inverters are replaced by their OC stub Pi-section equivalents with negative shunt OC stubs (using the command Inverters / Replace Ideal Inverter By / Pi-Tee/ OC Stub Pi). These replacements will degrade response slightly which can be tuned out. Figure 9.45.s shows the new circuit together with its narrow and wideband responses. The negative shunt OC stubs are absorbed by their positive neighbors by applying Combine command (Figure 9.45.t). The wideband response shows a closer spurious passband.

- The OC stub Pi-sections can be converted into OC combline type parallel coupled line sections by applying the command Replace By PCL / Combline / ococCOMB (Figure 9.45.u). Thus, in this approach the end couplings of the two adjacent lines are provided by two edge type coupled lines. This type of coupling can be easier to control compared to the simple end couplings where response is extremely sensitive to gaps.

- Figure 9.45.v shows pictorial description of two possible realizations.

- Figure 9.45.x summarizes the four approximately equivalent realizations of shunt parallel LC resonators. Further variations will be studied in the succeeding examples.

Figure 9.45. Design of capacitively loaded end coupled line filter.

Figure 9.45. Design of capacitively loaded end coupled line filter (continued).

Figure 9.45. Design of capacitively loaded end coupled line filter (continued).

Figure 9.45. Design of capacitively loaded end coupled line filter (continued).

EXAMPLE 9. 31: Synthesis of End Coupled Line filters as a direct coupled resonator  filter

Just as exercise with other design tools of Filpro, let’s redesign the filter of previous example (with 0.01 pF shunt capacitors) using Synthesis approach. In bandpass form, the previous filter was of degree 10. Topologically, it is a capacitively coupled shunt LC resonator filter. Therefore we can synthesize it with the following specifications:

PBRipple=0.05 dB,  fp1=1900 MHz,   fp2=2000 MHz, Nzero=9, Ninf=1, Total degree, N=10

- The command set for synthesis is shown in Figure 9.46. The transmission zeros are extracted to produce a symmetric circuit (single transmission zero of infinity is extracted in the middle), as a shunt capacitor, as shown in Figure 9.46.a.

- The command Pi-Tee-L / All Equal Shunt / Inductors is applied to get a coupled resonator form (Figure 9.46.b). The resulting transformer which is very close to unity is deleted.

- The capacitive Pi-sections with 0.01 pF shunt capacitors will be formed by splitting the shunt LC resonators properly. Then the remaining shunt LC resonators will be replaced by half wavelength transmission lines. However in order to do these we need to scale up the circuit such that the resulting transmission line impedances will have at a realizable level. Let’s say that this impedance level is about 70 ohms. We will scale up the circuit by inserting inverters at source and load ends. In order to get an idea on the scaling inverter constant we make the tests shown in Figure 9.46.b-to-e, as follows:

- With cursor under the LC resonator at load end the command Resonator Operations / pLCsh-to-Half Wave TLine is applied  to replace the resonator with its Half Wavelength line equivalent (Figure 9.46.c).

- Then we apply the command Inverters / Scale Circuit By Inserting Inverters / Specify Tline: Z=70. It is seen that inverter constant is K=205 (Figure 9.46.d). Now we know the impedance scaling inverter constant.

- The circuit of Figure 9.46.b is re-loaded (Figure 9.46.e) and the command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=205 is applied giving the circuit of Figure 9.46.f. This circuit will be modified to form the topology needed to simulate the end coupled line filter.

- The command Split / All Two Element Resonators is applied to split all LC resonators into their L and C components (Figure 9.46.g).

- With cursor under the shunt capacitors, the command Split / Specify Left: C=0.01 pF is applied to produce 0.01 pF shunt capacitors needed for formation of capacitive Pi-sections. This command is applied twice to the inner shunt capacitors where two Pi-sections will be formed on the two sides of each shunt inductor (Figure 9.46.h)

- Exchange command is used to position the 0.01 pF capacitors properly to form the capacitive Pi-section (Figure 9.46.i).

- Combine command is used to combine the remaining shunt capacitors (from which 0.01 pF capacitors are extracted) to form the LC resonators again, as shown in Figure 9.46.k. Thus, we have got the topology where shunt LC resonators are coupled to each other through capacitive Pi-sections. The return loss response is not degraded till now, as is seen from the figure.

- Next, the ideal inverters at source and load ends will be replaced by their capacitive Pi equivalents with negative shunt capacitors by applying the command Inverters / Replace Ideal Inverter By / Pi-Tee Type / Capacitive Pi (Figure 9.46.m). The response is degraded a lot by this operation. It will be corrected later.

- The source and load end terminations are seen as lumped resistors RS and RL. We provide transmission line terminations by inserting transmission lines of 50 ohm characteristic impedance at both ends (Figure 9.46.k-m). Lengths of these lines are immaterial, as they don’t affect response.

- Now we have to adjust the negative shunt capacitors of the capacitive Pi-sections produced by the inverters. The negative shunt capacitors which are adjacent to the LC resonators are absorbed within the LC resonator by applying Combine command and then the capacitors of these resonators are Split to produce 0.01 pF capacitors (Figure 9.46.m-n). This action doesn’t affect response.

- The negative shunt capacitors at source and load sides have no neighbors to absorb them. Therefore we use brute force and use Change command to change their capacitance to 0.01 pF (Figure 9.41.m-n). This action degrades the response appreciably. Response will be recovered partially by tuning the first two series capacitors (Figure 9.46.p).

- Now we start converting the shunt LC resonators into half wavelength transmission lines by applying the command Resonator Oprs / pLCsh-to-Half Wave TLine. This operation introduces some degradation in response which can be corrected by tuning the lengths of resulting TLines. First the inner LC resonators are converted into TLines because their degradations will be easier to correct. Then the LC resonators at source and load ends are converted into TLine form. This conversion introduces heavy degradation in return loss which can be recovered by tuning all series capacitors, lengths and impedances of TLines (Figure 9.46.q). The resulting circuit and response are shown in Figure 9.46.r. It has better return loss than the original level, with no shifts in band edges.

Figure 9.46. Design of end coupled line filter using Synthesis approach.

Figure 9.46. Design of end coupled line filter using Synthesis approach (continued).

Figure 9.46. Design of end coupled line filter using Synthesis approach (continued).

9.  9.  SHORT CIRCUITED EDGE COUPLED LINE FILTERS

Short circuited edge coupled line filters are just duals of the open circuited edge coupled line filters studied in the previous sections. Therefore similar design approaches can be applied. However because of practical difficulties of realization of a good SC at the ends of a coupled line this structure is rarely used. Therefore only a few design examples will be presented.

EXAMPLE 9. 32. A wideband SC edge coupled line filter design using Synthesis approach

Consider the octave band distributed element highpass filter with specifications:

PBRipple=0.01 dB, fp=4000 MHz, fq=6000 MHz,

with six unit elements (NUE=6) and one transmission zero at f=0 (Nzero=1), to work as a bandpass filter with fp1=4000 MHz and fp2=8000 MHz.

- Synthesis command set is shown in Figure 9.47. In this set we select the option Type-B to get the transmission zero at f=0 as a shunt SC stub.

- The transmission zeros are extracted in symmetric manner, yielding the circuit shown in Figure 9.47.a.

- With cursor under the unit elements (Tlines) adjacent to the SC stub, we apply the command Kuroda / Distribute Stub to transfer part of the shunt SC stub to the other side of the Tline. This command is repeated until a stub is placed on both sides of each Tline (Figure 9.47.b).

- The inner stubs are split into two equal parts by applying the command Split / Two Equal to form SC Stub+Tline+SC Stub triplets for conversion into coupled line form (Figure 9.47.c).

- In order to equate the SC stub impedances, we apply the command Kuroda / All Equal Stubs / Include End Stubs. The stub impedances will be equated after an iterative procedure applied by Filpro (Figure 9.47.d).

- Finally, with cursor under the Tlines, we apply the command Replace By PCL / Interdigital / scscINTD / Symmetric to convert the Stub+Tline+Stub triplets into parallel coupled line form (Figure 9.47.e). The even and odd mode impedances are shown underneath each coupled line.

Figure 9.47. Design of a short circuited edge coupled line filter by synthesis approach.

EXAMPLE 9. 33. A narrowband SC edge coupled line filter design using Built-in LC Prototype  approach

Consider the narrowband distributed element highpass filter of the previous sections with specifications:

PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz,

- Built in LC Prototype approach is the shortest route to the design. Actually the design command sets are the same as the narrowband OC edge coupled line filters with redundant unit elements at ports, as shown in Figure 9.48.

- The resulting filter comes out in the form of Series OC stub+Tline+Series OC Stub  triplets, ready for conversion into OC edge coupled line form.

- Applying Find Dual command we get the Shunt SC Stub+Tline+Shunt SC Stub triplets (Figure 9.48.b). These triplets are ready for conversion into SC edge coupled line form. However we may like identical SC stubs, leading to equal even mode impedances (roughly equal line widths). Such a circuit can be obtained by placing the cursor under any Tline and applying the command Kuroda / All Equal Stubs / Include End Stubs (Figure 9.48.c).

- Finally, with cursor under the Tlines, the command Replace By PCL / Interdigital / scscINTD /  Symmetric is applied leading to SC edge coupled line sections shown in Figure 9.48.d.

Figure 9.48. Design of a narrowband SC edge coupled line filter using Built in LC Prototype approach.

9. 10. WIDEBAND SHORT CIRCUIT STUB AND UNIT ELEMENT FILTERS

Shunt SC stubs interleaved by Tlines are among the simplest structures for realization. However such structures can be used to design only wideband filters. Narrowband designs will produce extreme impedances.

Traditionally such filters are designed as Distributed Element Highpass Filters. One disadvantage of this approach is that the longitudinal size comes out to be large, especially for low frequency applications. This is because the transmission line lengths are set by the quarter wavelength frequency fq which coincides with the passband center f0=fq. When this size is problematic, we can design the filter as Distributed Element Bandpass Filter with quarter wave frequency fq much larger than the passband center f0, leading smaller line and stub lengths. Another disadvantage of the highpass filter approach is that the nearest spurious passband is too close to the main passband, thus necessitating the use of a lowpass filter for its elimination. Bandpass filter approach has the advantage of moving this spurious passband away or eliminate it completely by using lumped capacitors in places of OC stubs. Let’s see some examples.

EXAMPLE 9. 34. An octave band SC Stub+UE filter designed as a highpass filter

Consider the octave band distributed element highpass filter with specifications:

PBRipple=0.01 dB, fp=4000 MHz, fq=6000 MHz,

with six unit elements (NUE=6) and one transmission zero at f=0 (Nzero=1), to work as a bandpass filter with fp1=4000 MHz and fp2=8000 MHz.

- Synthesis command set is shown in Figure 9.49. In this set we select the option Type-B to get the transmission zero at f=0 as a shunt SC stub.

- The transmission zeros are extracted in symmetric manner, yielding the circuit shown in Figure 9.49.a.

- With cursor under the unit elements (Tlines) adjacent to the SC stub, we apply the command Kuroda / Distribute Stub to transfer part of the shunt SC stub to the other side of the Tline. This command is repeated until a stub is placed on both sides of each Tline (Figure 9.49.b).

- In order to equate the SC stub impedances, we apply the command Kuroda / All Equal Stubs / Include End Stubs. The stub impedances will be equated after an iterative procedure applied by Filpro (Figure 9.49.c). Further Kuroda transformations can be applied to trim impedances of stubs and Tlines. However the stub and Tline impedances seem to be realizable. Therefore design is stopped at that stage.

- Figure 9.49.d shows a pictorial description of the structure. Its length is close to 80 mm.

- The spurious passband starts after about 12000 MHz. Therefore a lowpass filter need be cascaded to eliminate the spurious passband.

Figure 9.49. Design of a wideband SC Stub+UE filter as a distributed element highpass filter.

EXAMPLE 9. 35. An octave band SC Stub+UE filter designed as a bandpass filter

Let’s design the same octave band filter as a distributed element bandpass filter with specifications:

PBRipple=0.01 dB, fp1=4000 MHz, fp2=8000 MHz and fq=12000 MHz , with seven resonators.

- In order to get seven resonators, we need to place six unit elements (NUE=6).

- One transmission zero will be placed at f=0 (Nzero=1) and it will be distributed into the circuit using Kuroda transformations to generate a shunt SC on each side of the unit elements.

- Since we are designing a bandpass filter, we have to place some transmission zeros at infinity. Our aim is to place a shunt OC stub in parallel with each shunt SC stub to form OC+SC Stub resonators. In the highpass filter version the resonators were the shunt SC stubs which resonate at the passband center of f0=fq=6000 MHz.  Now with the new SC stubs resonate at fq=12000 MHz. In order to have them resonate at f0=6000 Mz, we need to place a shunt OC stub in parallel with each shunt SC stub. That is we have to place seven transmission zeros at infinity. Thus, Ninf=7.

- Synthesis command set is shown in Figure 9.50. In this set we select the option Type-B to get the transmission zero at f=0 as a shunt SC stub.

- The transmission zeros are extracted in the order shown in the figure so as to get the transmission zeros at infinity as shunt OC stubs. The transmission zero at f=0 is extracted in the middle to get a symmetric circuit yielding the circuit shown in Figure 9.50.a where a shunt OC stub exists on each side of the Tlines.

- Simplify Circuit command combines the adjacent SC and OC stubs at the center of the circuit to form SC+OC Stub resonator (Figure 9.50.b). Then, with cursor under the Tlines adjacent to the SC+OC Stub resonator, the command Kuroda / Distribute Stub is applied to transfer part of the SC stub to the other side of the Tlines. Simplify Circuit command is applied again to form the new SC+OC Stub resonators. Then the command Kuroda / Distribute Stub is repeated by placing the cursor under the Tlines adjacent to the newly formed SC+OC Stub resonators. This way, a SC+OC Stub resonator is generated on each side of each Tline, as described in Figures 9.53.b-to-e.

- In order to equate the SC stub impedances, we place the cursor under any Tline and apply the command Kuroda / All Equal Stubs / Include End Stubs. The stub impedances will be equated at 152 ohms level Filpro (Figure 9.50.e-f).

- Figure 9.50.g shows a pictorial description of the structure. Impedances of all OC stubs, SC stubs and Tlines seems realizable. However the OC stubs generate a spurious passband above 12000 MHz, similar to the highpass design of the previous example. It could be pushed away from the main passband by selecting even higher fq. However an easier way is to convert the shunt OC stubs into lumped capacitors. This is done by placing the cursor under the SC+OC stub resonators and applying the command Replace by Lumped / All SC Stub+Cap (Figure 9.50.h-i). This command converts the OC stubs into lumped capacitors in such a way that the passband edges are kept unaffected. Since the passband is wide, this approximation distorts the passband return loss. Also equality of the shunt SC stub impedances is lost. Equiripple property of return loss can be recovered at a few dB lower level by tuning some elements.

- Equality of shunt SC stubs can be recovered by applying the command Kuroda / All Equal Stubs / Include End Stubs.again. They are equated at 101 ohms level (Figure 9.50.k).

- It is seen that  the spurious passband is eliminated.

- Figure 9.50.m shows a pictorial description. The lumped capacitors may also be realized as patches like short radial OC stubs. Note that the total length is halved compared to the highpass approach.

Figure 9.50. Design of a wideband SC Stub+UE filter as a distributed element bandpass filter.

Figure 9.50. Design of a wideband SC Stub+UE filter as a distributed element bandpass filter.

EXAMPLE 9. 36. An ultra wideband SC Stub+UE filter designed as a bandpass filter

Let’s test the limitations of the approach for an ultra wideband filter with specifications:

PBRipple=0.1 dB, fp1=2000 MHz, fp2=6000 MHz and fq=7000 MHz with seven resonators.

It was observed that as the passband ripple is decreased and fq is increased, resonator impedances get too high. The above values for PBRipple and quarter wave frequency are selected after a few design trials.

- In order to get seven resonators, we need to place six unit elements (NUE=6).

- One transmission zero will be placed at f=0 (Nzero=1) and it will be distributed into the circuit using Kuroda transformations to generate a shunt SC on each side of the unit elements.

- We have to place seven transmission zeros at infinity to form shunt OC stubs in parallel with each shunt SC stub to form OC+SC Stub resonators. Thus, Ninf=7.

- Synthesis command set is shown in Figure 9.51. In this set we select the option Type-B to get the transmission zero at f=0 as a shunt SC stub.

- The transmission zeros are extracted in the order shown in the figure so as to get the transmission zeros at infinity as shunt OC stubs. The transmission zero at f=0 is extracted in the middle to get a symmetric circuit yielding the circuit shown in Figure 9.51.a where a shunt OC stub exists on each side of the Tlines.

- Simplify Circuit command combines the adjacent SC and OC stubs at the center of the circuit to form SC+OC Stub resonator (Figure 9.51.b). Then, with cursor under the Tlines adjacent to the SC+OC Stub resonator, the command Kuroda / Distribute Stub is applied to transfer part of the SC stub to the other side of the Tlines. Simplify Circuit command is applied again to form the new SC+OC Stub resonators. Then the command Kuroda / Distribute Stub is repeated by placing the cursor under the Tlines adjacent to the newly formed SC+OC Stub resonators. This way, a SC+OC Stub resonator is generated on each side of each Tline (Figures 9.54.b-c).

- In order to equate the SC stub impedances, we place the cursor under any Tline and apply the command Kuroda / All Equal Stubs / Include End Stubs. The stub impedances will be equated at 229 ohms level Filpro (Figure 9.51.d).

- The shunt OC stubs are converted into lumped capacitors by placing the cursor under the SC+OC stub resonators and applying the command Replace by Lumped / All SC Stub+Cap (Figure 9.51.e.). This command converts the OC stubs into lumped capacitors in such a way that the passband edges are kept unaffected. Since the passband is wide, this approximation distorts the passband return loss. Also equality of the shunt SC stub impedances is lost.

- Equiripple property of return loss can be recovered at about 14 dB level by tuning some elements (Figure 9.51.e-f). The nearest spurious passband is at about 14000 MHz. If it is desired to be eliminated, then a LP filter need be cascaded.

- Equality of shunt SC stubs can be recovered by applying the command Kuroda / All Equal Stubs / Include End Stubs equating them at 127 ohms level (Figure 9.51.g-h).

- Figure 9.51.i shows a pictorial description. The total length is about 64.24 mm.

- Figure 9.51.k shows the version desinged as a Distributed Element Highpasss Filter with fp=2000 MHz, fq=4000 MHz. It is seen that this filter is much longer than the bandpass version (112.42 mm). Also the spurious passband is nearer, starting form 8000 mHz, necessitating a lowpass filter with passband edge at this frequency.

Figure 9.51. An ultra wideband SC Stub+UE filter.

Figure 9.51. An ultra wideband SC Stub+UE filter (cont’d).

9. 11. WIDEBAND HP FILTERS WITH MULTIPLE TRANSMISSION ZEROS AT f=0

As it was seen in the previous section, when the bandwidth gets wider element values get too high values. This problem can be overcome by placing more transmission zeros at f=0. This leads to both series OC stubs and shunt SC stubs in the filter. The series OC stubs can be realized as OC edge coupled line sections while the shunt SC stubs can be realized as they are. Lets see some examples.

EXAMPLE 9. 37. Ultra wideband edge coupled line filters with Nzero=3

Lets consider a distributed element highpass filter with specifications

PBRipple=0.01 dB, fp=2000 MHz, fq=4000 MHz

to work as a bandpass filter with fp1=2000 MHz and fp2=6000 MHz.

- We will use Design / By Synthesis approach. The three transmission zeros at f=0 will lead to either two series OC stubs and one shunt SC stub if Type A is selected in the Synthesis stage or two shunt SC stubs and one series OC stub would be produced if Type B is selected.

- The required selectivity will be satisfied by placing more Unit Elements. Each series OC stub necessitates placement of a Unit Element to form an OC edge coupled line section. Thus, the minimum number of Unit Elements is equal to the number of series OC stubs. If symmetry is desired than the number of Unit Elements must be even so that the total degree of the filter will be odd. Let’s exercise by placing different numbers of Unit Elements.

a) Nzero=3, NUE=1

- For this case we target to get a single edge coupled line section. Therefore only one of the transmission zeros of f=0 will be extracted as series OC stub, leaving two of them as shunt SC stubs. This will be possible by selecting the option Type B during synthesis stage. The command set for synthesis and element extraction order are shown in Figure 9.52. After extracting the second transmission zero as a series OC stub, we extract a unit element (Figure 9.52.a).

- With cursor under the unit element, the command Kuroda / Distribute Stub is applied to share the series OC stub on the two sides of the unit element to form a triplet of OSsr+Tline+OSsr (Figure 9.52.b) which is then converted into an OC edge coupled line section by applying the command Replace By PCL / Interdigital / ococINTD / Symmetric (Figure 9.52.c).

- Figure 9.52.d shows a pictorial description for realization. The shunt SC stubs have realizable impedances (72 ohms) while the even-odd mode impedances are to far from each other which means a broadside coupling is necessary.

b) Nzero=3, NUE=2

- For this case we will test two OC edge coupled line sections and a single shunt SC stub. Therefore we place two unit elements. The command set and element extraction order are shown in Figure 9.53. Type A is selected to get the first element as series OC stub. A unit element is extracted after each series OC stub (Figure 9.53.a).

- With cursor under the unit elements, Kuroda / Left / Equal Stub is applied to form triplets of OSsr+Tline+Ossr (Figure 9.53.b).

- Simplify Circuit command is applied to collect all transformers at rightmost to combine and then Delete it (Figure 9.53.c).

- Then with cursor under the unit elements again, the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to form the OC edge coupled line sections (Figure 9.53.d).

- Figure 9.53.e shows a pictorial description. The shaded and white lines show opposite sides of a stripline or suspended stripline realization. It is seen that spurious couplings may exist between the shunt SC stub and the adjacent edge coupled line sections. Therefore more unit elements may be used to separate these sections.

c) Nzero=3, NUE=3

- No realizable solution was found for this case. Coupled section impedances come out to be too high (Exercise?).

Figure 9.52. Design of a filter with Nzero=3 and NUE=1.

Figure 9.53. Design of a filter with Nzero=3 and NUE=2.

d) Nzero=3, NUE=4

Development of one possible solution is described in Figure 9.54.

- Type A is selected. Transmission zeros are extracted in the order zero-UE-UE-zero-UE-UE-zero (Figure 9.54.a).

- The series OC stubs are converted into OSsr+TLine+OSsr triplets by applying the command Kuroda / Right (Left) / Equal Stub (Figure 9.54.b). Simplify Circuit command eliminates the transformers.

- These triplets are then converted into OC edge coupled line sections by applying the command Replace By PCL / Interdigital / ococINTD / Symmetric (Figure 9.54.c).

- With cursor under the unit elements, the command Kuroda / Distribute Stub is applied to place a shunt SC stub on each side of each unit element (Figure 9.54.d). Then the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the shunt SC stubs (Figure 9.54.e).

- Figure 9.54.f shows the pictorial description where the white and black strips shows conductors on two opposite sides of a central dielectric strip. Element values seems realizable, though we have very tight coupling in the coupled line sections.

e) Nzero=3,  NUE=6

This filter can be treated in the same way as the previous one. That is, two OC edge coupled lines will be placed on the two ends while the inner elements will be in Shunt SC stub+Unit Element form. Design steps are shown in Figure 9.55. Element values come out to be close to the previous one.

Observation:

Inspection of the previous two structures shows that the as the number of Unit Elements (NUE) is increased we will get a standard structure made up of Shunt SC Stub+Unit Elements in between the two OC edge coupled line sections with the number of shunt SC stubs increased to NUE-2. Actually, the section formed by the Shunt SC Stub+Unit Elements is equivalent to an N-coupled line interdigital filter that will be studied in the next chapter. Thus, this structure can be treated as the ultra wideband version of N-coupled line interdigital filters.

Figure 9.54. Design of a filter with Nzero=3 and NUE=4.

Figure 9.55. Design of a filter with Nzero=3 and NUE=6.

EXAMPLE 9. 38. Ultra wideband edge coupled line filters with Nzero=5

Lets consider the same distributed element highpass filter with specifications

PBRipple=0.01 dB, fp=2000 MHz, fq=4000 MHz

to work as a bandpass filter with fp1=2000 MHz and fp2=6000 MHz.

The five transmission zeros at f=0 will produce either two series OC stub and three shunt SC stub or three series OC stub and two shunt SC stubs. Selectivity will be adjusted by setting the number of Unit Elements, with at least one unit element will be assigned to each series OC stub for conversion into OC edge coupled line form. Let’s see various cases by changing the number of Unit Elements, NUE.

- Nzero=5,  NUE=2

- This case is studied in Figure 9.56. Since we have only two unit elements, we must get two series OC stubs which can be obtained by selecting the option Type B. This option leads to a shunt SC stub as the first element. The order of element extraction is shown in the figure, leading to the circuit of Figure 9.56.a.

- With cursor being under the unit elements, the command Kuroda / Left / Equal Stub is applied to form triplets of OSsr+Tline+OSsr. Then Simplify Circuit command is applied to eliminate the transformers resulting form these operations (Figure 9.56.b).

- Then with cursor being under the unit elements, the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to give the circuit shown in Figure 9.56.c.

- Figure 9.56.d shows the pictorial view with black and white lines showing conductors at two different layers, for example etched on the two sides of a middle dielectric layer. Compared to the previous cases the even and odd mode impedances are higher in level, but have less difference, hence lighter coupling.

- Nzero=5, NUE=3

- The three unit elements can be used to form three OC edge coupled line sections. In order to get three series OC stubs,  Type A is selected and a unit element is extracted after each series OC stub giving the circuit of Figure 9.57.a.

- With cursor under the unit elements, the command Kuroda / Left / Equal Stub is applied to form the triplets of OSsr+TLine+OSsr  and Simpify Circuit command is used to eliminate transformers (Figure 9.57.b). Then these triplets are converted into coupled line form by applying the command Replace By PCL / Interdigital / ococINTD / Symmetric (Figure 9.57.c).

- Figure 9.57.d shows a pictorial description. Tight coupling necessitates the use of broadside coupled lines.

- Nzero=5,  NUE=4

- One possible solution for this case is described in Figure 9.58. Two coupled line sections are used. Therefore Type B is selected to get only two series OC stubs.  This leads to three shunt SC stubs, as shown in Figure 9.58.a.

- The series OC stub+Unit Element pairs are converted into coupled line form by using the commands Kuroda / Right (Left) / Equal Stub and then Replace By PCL / Interdigital / ococINTD / Symmetric, as shown in Figures 9.58.b-c.

- With cursor being under the unit elements on the two sides of the middle shunt SC stub, the command Kuroda / Distribute Stub (or Kuroda / Right (Left) / Equal Stub) command is applied to move parts of that SC stub to the other sides of the unit elements, as shown in Figure 9.58.d. Then the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate impedances of the three interior shunt SC stubs (Figure 9.58.e).

- Figure 9.58.f shows the pictorial description.

- Nzero=5,  NUE=5

- This case can be treated in the same way as the previous one, as shown in Figure 9.59. That is, we have a shunt SC stub followed by a coupled line at each port and in the interior region we have shunt SC stubs separated by unit elements. Increasing the number of unit elements will increase the number of interior shunt SC stubs.

Figure 9.56. Design of a filter with Nzero=5 and NUE=2.

Figure 9.57. Design of a filter with Nzero=5 and NUE=3.

Figure 9.58. Design of a filter with Nzero=5 and NUE=4.

Figure 9.59. Design of a filter with Nzero=5 and NUE=5.

9. 12. WIDEBAND BP FILTERS AS CASCADES OF LP AND HP FILTERS USING MIXED LUMPED AND DISTRIBUTED ELEMENTS

Near the low end of microwave frequencies some elements are easier to realize in lumped element form. Also Element values may be relaxed by designing LP and HP sections separately and then cascading them to get wideband HP filters.

Example 9. 39. A 2-6 GHz LP-HP filter

Let’s consider a BP filter with following specifications:

Return loss> 10 dB, fp1=2000 MHz, fp2=6000 MHz.

Minimum stopband loss must be at least 60 dB below  f=1650 MHz and above f=6700 MHz. The filter will be constructed by cascading a HPF with fp=2000 MHz and a LPF with fp=6000 MHz. Selectivity requirement in lower and upper stopbands will be satisfied by finite transmission zeros that will be placed in the HPF and LPF respectively. In order to allow for return loss degradation after cascading the two filters the PBRipple of the individual HP and LP filters will be taken as 0.1 dB (Return Loss=16 dB).

Design of HPF:

After some trials by changing the number of transmission zeros at f=0, the number of Unit Elements and positions and numbers of finite transmission zeros and the quarter wavelength frequency fq, the following filter is found suitable:

PBRipple=0.1 dB, Nzero=5, NUE=4, NFTZ=2 both at f=1500 MHz, fq=7500 MHz.

The Unit Elements are needed mainly for physical separation between the shunt resonators to avoid spurious couplings. The quarter wavelengh frequency is selected by inspecting the total length and transversal size of the filter.

- The design command set and response are shown in Figure 9.60. Transmission zeros are extracted in the following order: zero-UE-FTZ1-zero-ue-zero-UE-FTZ2-zero-ue-zero giving the circuit of Figure 9.60.a.

- The shunt FTZ sections are in the form of series connected OC and SC stubs. These are unrealizable in this form. They can be converted into two-step stubs by applying Step Resonator command (Figure 9.60.b).

- The stepped resonators have too high and too low step impedances. First the high impedance parts are brought to realizable level by creating transformers by moving the series OC stub on right of the first transmission line to its left using Kuroda / Right / Full Stub command. Similarly the command Kuroda / Left / Full Stub is applied on the transmission line at load end to create the inverse transformer to cancel the first one after Simplify Circuit command (Figure 9.60.c). This brings down the first step of the stepped resonators to 49 ohms level. Impedance of the second step falls down to about 5 ohms. It will be replace by lumped capacitor later.

- The previous operation lowered impedance of the shunt SC stub in the middle down to about 12 ohms which is difficult to realize. It can be increased to realizable levels by moving the series OC stubs on one sides of the interior transmission lines to the other sides by applying Kuroda transformations (Figure 9.60.d).

- Next, the series OC stubs are approximated by lumped capacitors at f=2000 MHz by applying Replace By Lumped / Single C: f0=2000 MHz (Figure 9.60.e).

- Finally the low step impedances of stepped resonators are converted into lumped capacitors by applying the command Replace By Lumped / Stub+Cap: f0=2000 MHz (Figure 9.60.f). Design is finished. The circuit is saved as HP1.fil.

- Insertion and return loss responses are shown in Figure 9.60.g. Little degradation erased in the 2000-6000 MHz band. This circuit will be cascaded with the lowpass filter that will be designed next.

Figure 9.60. Design of HPF for the wideband LP-HP type bandpass filter.

Figure 9.60. Design of HPF for the wideband LP-HP type bandpass filter.

Design of LPF:

After some trials the following distributed element lowpass filter is designed:

PBRipple=0.1 dB, fp=6000 MHz, fq=14000 MHz, Ninf=1, NUE=6, NFTZ=3, all FTZ’s at fz=7000 MHz.

Here, fq=14000 MHz is selected to be twice the FTZ frequencies in order to get the two step stubs as uniform stubs.

- Design command set is shown in Figure 9.37. Extracted circuit is shown in Figure 9.61.a.

- First the series SC stubs are converted into shunt OC stubs by applying Kuroda / Right / Full Stub (Figure 9.61.b).

- Thn the shunt arm series OC+SC stubs are converted into two step stubs by applying Step Resonator command (Figure 9.61.c). LP filter design is finished. All impedances are realizable. It is saved as LP1.fil.

- The highpass filter HP1.fil can be cascaded with LP1.fil by placing it to the right or left of LP1.fil. In Figure 9.61.d it is placed to the left of LP1.fil by placing the cursor under the first element adjacent to RS and applying the command Append Filter / HP1.fil. Response is shown in Figure 9.61.e. The requirements are satisfied.

Exercise: Repeat the design with different fq’s and with different PBRipple for the HPF and inspect their effects on impedances and degradations in return loss.

Figure 9.61. Design of the lowpass section and formation of the LP-HP filter.

Figure 9.61. Design of the lowpass section and formation of the LP-HP filter (cont’d).

EXAMPLE 9. 40. A 400 MHz-2000 MHz LP-HP filter

Let’s now consider a wider bandwidth filter with following specifications:

Return Loss>10 dB, fp1=400 MHz, fp2=2000 MHz

Minimum Stopband Loss>60 dB for f<350 MHz and for f>2500 MHz

Design of HPF:

The specifications for the HPF are found after a few trial designs as follows:

PBRipple=0.1 dB, fp=400 MHz, Nzero=3, NFTZ=3, all at fz=330 MHz and NUE=4

- Design commands are shown in Figure 9.62 yielding the circuit of Figure 9.62.a.

- The shunt OC+SC stub elements are converted into stepped stubs by applying Step Resonators command (Figure 9.62.b).

- In order to scale down the impedances of interior elements and to get a physically symmetric circuit Kuroda transformations are used as shown in Figures 9.62.b-c. The circuit is saved as HP10.fil.

- The second steps of the stepped resonators have too low impedances. Therefore they are converted into lumped capacitors by applying the command Replace By Lumped / Stub+Cap: f0=400 MHz (Figure 9.62.d). The circuit is saved as HP11.fil.

- Finally the series OC stubs are replaced by lumped capacitors by applying the command Replace By Lumped / Single C: f0=400 MHz.

- Figure 9.62.e and f show the resulting circuit and responses.

Other alternatives can be formed as follows:

- The capacitors of shunt resonators of HP11.fil are rather large (28.6 pF for outer and 21.3 pF for the middle resonators). They can be scaled down to a certain extent by Kuroda transformations as shown in Figures 9.62.g-k.

- First with cursor under the leftmost transmission line the command Kuroda / Left / Equal Stub (or Specify Trf) is applied to move part of the OC stub on left to its right producing also a transformer. Simplify Circuit command moves the transformer to load end, thus scaling down the capacitors while scaling up the inductors. The symmetric version of the same transformation must also be applied on the transmission line at load end giving the circuit of Figure 9.62.h.

- Similar transformations can be applied to the other transmission lines to adjust the parameters of the middle resonator (Figure 9.62.i).

- Finally the series OC stubs are converted to lumped capacitors giving the circuit of Figure 9.62.k.

- Another approach may be replacement of the shunt two-step resonators into lumped LC form (if the resulting L and C are realizable at the frequencies of interest) as described in Figures 9.62.m-q.

- The circuit of HP10.fil is loaded  (Figure 9.62.m) and with cursor under the shunt two-step resonators the command Replace By Lumped / LC Resonator: f0=400 MHz is applied (Figure 9.62.n).

- If the resulting L and C values need be adjusted the series OC stubs can be translated to the other sides of the adjacent transmission lines by Kuroda transformations as was done n the previous approach. Then they are converted into lumped capacitors (Figure 9.62.p).

- If easier to realize, the shunt SC stubs may also be replaced by shunt lumped inductors (Figure 9.62.q).

Exercise: Design a LPF and append it to the above HPF to form the desired LP-HP type bandpass filter.

Exercise: Repeat the designs by changing fq and PBRipple parameters.

Figure 9.62. Design of 400 MHz HPF.

Figure 9.62. Design of 400 MHz HPF (cont’d).

Figure 9.62. Design of 400 MHz HPF (cont’d).

REFERENCES AND FURTHER READING:

- S. B. Cohn, “Direct coupled resonator filters”, Proc. IRE, pp. 187-196, Feb. 1957.

- H. Ozaki, and J. Ishii, “Synthesis of a Class of Stripline Filters”, IRE. Trans. Circuit Theory, Vol.CT-5, pp. 104 -109, 1958.

- S. B. Cohn, “Parallel Coupled Transmission Line Resonator Filters”, IEEE Trans MTT, vol. MTT-6, No 2, pp. 223 - 231, Apr. 1958.

- G. L. Matthaei, L. Young, E. M. Jones, “Microwave Filters, Impedance Matching Networks and  Coupling Structures”, Mc Graw Hill , NY, 1964.

- R. J. Wenzel, “Exact Design of TEM Microwave Networks Using Quarter-Wave Lines”,IEEE Trans. MTT,  Vol. MTT-12, pp. 94 - 111, Jan. 1964.

- M. C. Horton and R. J. Wenzel, “General Theory and Design of Optimum Quarter Wave TEM filters”, IEEE Trans. MTT,  vol. MTT-13, No. 3, pp. 316 - 327, May 1965.

- R. J. Wenzel, “ Exact Theory of Interdigital Bandpass Filters and related Coupled Structures”, IEEE Trans MTT, vol. MTT- 13 ,  pp. 559 - 575, Sept. 1965.

- M. Dishal, “A Simple Design Procedure for Small Percentage bandwidth round rod interdigital filters”, IEEE Trans MTT, vol. MTT-13, pp. 696 - 698, Sept. 1965.

- R. J. Wenzel, “Theoretical and practical applications of  Capacitance matrix transformations to TEM Network Design”, IEEE Trans MTT, vol. MTT-14, No. 12, pp. 635 - 647, Dec. 1966.

- J. R. Pyle, H. E. Green,“Exact Design of capacitive gap filters” Microwaves, pp.28-33, May, 1966.

- R. Levy, “ A New Class of Distributed Prototype Filters with applications to Mixed Lumped / Distributed Components”, IEEE Trans. MTT, Vol. MTT-18, pp. 1064 - 1071, Dec. 1970.

- E. G. Crystal, “New Design Equations for a Class of Microwave Filters”, IEEE Trans MTT, vol. MTT-19, No. 5, pp. 486 - 490, May 1971.

- E. G. Crystal and S. Frankel, “Hairpin Line and Hybrid Hairpin Line Half Wave Parallel Coupled Line Filters”, IEEE Trans MTT, vol. MTT-20, No. 11, pp. 719-728,  Nov. 1972.

- G. Hoffmann, “ The Direct Synthesis of Optimum Commensurate Distributed Filters by Digital Computer”, 1973 European Microwave Conference Proc., B.2.2, Brussels, 1973.

- K. K. Pang, “Design of Microwave Filters by Sine-plane approach”, IEEE Trans. MTT, vol. MTT-21, pp. 607- 611, Oct. 1973.

- R. Levy, “A Generalized Design Technique for Practical Distributed Reciprocal Ladder Networks”, IEEE Trans. MTT, vol. MTT- 21, No. 8, pp. 519 - 526, Aug. 1973.

- G. Reiter and G. Hammer, “Stripline Filters with Coupled  / 8 Line Sections”, 1973 European Microwave Conference, B.3.2, Brussels, 1973

- G. C. Temes and S. K. Mitra, “Modern Filter Theory and Design”, J. Wiley and Sons, NY, 1973.

- U. H. Gysel, “New Theory and Design for Hairpin Line Filters”, IEEE Trans MTT, vol. MTT-22, No. 5, pp. 523 - 531, May 1974.

- G. Reiter, “ Stripline Filters with Lumped Capacitors”, 5. European Microwave Conference Proc., pp. 426 - 430. 1975.

- E. G. Crystal, “Tapped-Line Coupled Transmission Lines with Applications to Interdigital and Combline Filters”, IEEE Trans. On  MTT ,  vol. MTT-23, No. 12,  pp. 1007-1012, Dec.1975.

- J. D. Rhodes, “Theory of Electrical Filters”, London, England, Wiley, 1976.

- M. C. Horton, “TEM Bandpass Filters Having Eight Wavelength Coupled Stubs”, 1977 IEEE, MTT- S Int. Microwave Symposium Digest, San Diego , 1977.

- H. Howe, “Stripline Circuit Design”, Artech House, 1979.

- J. A. G. Malherbe, “Microwave Transmission Line Filters”, Artech House, 1980.

- M. Makimoto, S. Yamashita, “Bandpass filters using parallel coupled stripline stepped impedance resonators”, IEEE Trans. MTT, vol. MTT-28, No. 12, pp. 1423-1417, Dec. 1980.

- J. B. Minnis, “Printed Circuit Coupled Line Filters for Bandwidths up to and Greater than an Octave”, IEEE Trans. MTT, vol. MTT-29, No. 3, pp. March 1981.

- B. Easter, K. Merza, “Parallel Coupled Line Filters for Inverted Microstrip and Suspended Substrate MICs”, 11. European Microwave Conference Proc., pp.164 - 168, A.2.4, 1981.

- P. La Tourette, “High Pass Filter Design”, Proc. EuMC’82, pp. 233 - 238, 1982.

- J. B. Minnis, “Classes of Sub Miniature Microwave Printed Circuit Filters with Arbitrary Passband and Stopband Widths”, IEEE Trans. MTT, Vol. MTT-30,  pp. 1893 - 1900,  Nov. 1982.

- C. I. Mobbs and J. D. Rhodes, “A Generalized Chebychev Suspended Stripline Bandpass Filter”, IEEE Trans. MTT, Vol. MTT- 31,  pp. 397- 402,  May 1983.

- B. Rawat, R. Miller, and B. Pontius, “Bandpass filter for Mobile Communications”, Microwave Journal, Vol. 27, No. 10, pp. 146-150, 1984.

- A. Riddle, “High Performance Parallel Coupled Line Microstrip Filters”, 1988 IEEE, MTT-S Conference Proc., pp. 427 - 430, 1988.

- M. Gat, “Commensurate Line Microstrip Bandpass Filters”, 1988 IEEE, MTT-S Conference Proc., pp. 423- 426, 1988.

- K. Takahashi, M. Sagawa , “Miniaturized Hairpin Resonator Filters and Their Application to Receiver Front End MICs”, IEEE MTT-S Digest, pp. 667- 670, 1989.

- Y. Sen, “Direct Design of Lumped and Distributed Filters Using Transformed Variable Techniques”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1989.

- J. H. Kotze, “Classes of Bandpass Filters Having an Arbitrary wide Stopband  and Design Tables for One Such Class”, IEEE MTT-S, pp. 123 - 126, 1990.

- S. Herbert, “Procedure Produces Miniature MIC bandpass filters”, Microwaves and RF, pp. 203-209, Apr. 1990.

- M. M. Akkul, “Design and Realization of Microwave Bandpass Filters in Stripline and Suspended Stripline Structures”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1991.

- I. Tekin, “Design and Realization of Narrowband Microwave Bandpass Filters Using  Stripline-like Structures”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1992.

- C. M. Tsai and K. C. Gupta, “A Generalized Model for Coupled Lines and its applications to Two-Layer Planar Circuits”, IEEE Trans. MTT, Vol. MTT- 40,  pp. 2190 - 2199, Dec. 1992.

- J. C. Tippet and M. J. Shian, “Circuit Transformations for realization of a Class of  Miniature  Printed Circuits”, IEEE,  MTT-S  Digest, pp. 621- 624, 1994.

- U. Koyaz, “Synthesis of Lumped and Distributed Filters and Diplexers”, Ph. D. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1994.

- D. Zhang, G. C. Liang, C. F. Shih, and R. S. Withers, “Narrowband Lumped Element Microstrip Filters Using Capacitively Loaded Inductors”, 1995 IEEE  MTT-S  Digest, pp. 379- 382, 1995.

- A. F. Sheta, K. Hettak, J. Coupez, C. Person, and S.Toutain, J.P. Blot, “A New Semi-Lumped microwave Filter Structure”,  1995  IEEE MTT-S Digest, pp. 383 - 386, 1995.

- M. Karaaslan, “Network Transformations for Realization of Lumped and Distributed Filters”, M. Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1994.

REFERENCES FOR STEPPED IMPEDANCE RESONATORS (single and dual band)

- M. Makimoto and S. Yamashita, “Bandpass filters using parallel coupled stripline stepped impedance resonators”, IEEE Trans. Microwave Theory Tech, Vol. MTT-28, pp. 1413-1417, Dec. 1980.

- A. F. Sheta, J. P. Coupez, G. Tanne,  S. Toutain, and J. P. Blot, “Miniature microstrip stepped impedance filters and diplexers for mobile communications,” 1996 IEEE MTT-S Digest, pp. 607-610.

- M. Sagawa, M. Makimoto and S. Yamashita, “Geometrical structures and fundamental characteristics of microwave stepped impedance resonators,” IEEE Trans. Microwave Theory Tech. , Vol. 45, No. 7, pp. 1078-1085, July 1997.

- J. T. Kuo and E. Shih, “Stepped impedance resonator bandpass filters with tunable transmission zeros and its application to wide stopband design,”  2002 IEEE MTT-S Digest, pp. 1613-1616.

- M. G. Banciu, A. Joachim and R. Ramer, “Miniature microstrip resonators and filters for GSM / GPRS,” 2002 IEEE MTT-S, pp. 41-44.

- J. T. Kuo and E. Shih, “Microstrip Stepped impedance resonator bandpass filters with an extended optimal rejection bandwidth,”  IEEE Trans. Microwave Theory Tech. , Vol. 51, No. 5, pp. 1554-1559, May 2003.

- M. G. Banciu, R. Ramer and A. Joachim, “Compact Microstrip Resonators for 900 MHz frequency band,”  IEEE Microwave and Wireless Components Letters, Vol. 13, No. 5, pp. 175-177, May 2003.

- S. Avrillon, I. Pele, A. Chousseaud and S. Toutain, “Dual band power divider based on semi loop stepped impedance resonators,” IEEE Trans. Microwave Theory Tech. , Vol. 51, No. 4, pp. 1269-1273, May 2003.

- J. T. Kuo, W. H. Hsieh and M. Jiang, “Design of two stage UIR and SIR bandpass filters with and elliptic function like response,” 2004 IEEE MTT-S Digest, pp. 1609-1612.

- H. Wang and L. Zhu, “Aperture backed microstrip stepped impedance resonator with ultra broad band rejection bandwidth,” Electronics Letters, Vol.40, No. 19, Sept. 2004.

- H. M. Lee, C. R. Chen, C. C. Tsai and C. M. Tsai, “Dual band coupling and feed structure for microstrip filter design,” 2004 IEEE MTT-S Digest, pp. 1971-1974.

- S. F. Chang, Y. H. Jeng and J. L. Chen, “dual band step impedance bandpass filter for multimode wireless LANs,” Electronics Letters, Vol. 40, No. 1, 8th Jan 2004.

- Q. X. Chu and W. Ma, “A new dual band step impedance bandpass filter,” 2005 IEEE MTT-S Digest, pp. 610-613.

- J. T. Kuo, T. H. Yeh and C. C. Yeh, “Design of microstrip bandpass filters with a dual passband response,” IEEE Trans. Microwave Theory Tech, Vol. 53, No.4,  pp. 1331-1337, April 2005.

- H. Zhang and K. J. Chen, “A trisection stepped impedance resonator for cross-coupled bandpass filters,” IEEE Microwave and Wireless Components Letters, Vol. 15, No. 6, pp. 401-403, June 2005.

- S. Sun and L. Zhu, “Compact dual band microstrip bandpass filter without external feeds,” IEEE Microwave and Wireless Components Letters, Vol. 15, No. 10, pp. 644-646, Oct. 2005.

- S. Sun and L. Zhu, “Coupling dispersion of parallel coupled microstrip lines for dual band filters with controllable fractional pass bandwidths,”  2005 IEEE MTT-S Digest, pp. 2195-2198.

- H. M. Lee, C. M. Tsai and C. C. Tsai, “Transmission line filter design with fully controllable second passband,” ,”  2005 IEEE MTT-S Digest, pp. 2191-2194.

REFERENCES FOR CALCULATION OF COUPLED LINE PARAMETERS

- Wheeler, H. A., “The Transmission Line Properties of Round Wires Between Parallel Planes”, IRE, Trans. Antenna and Propagation, AP-3, No. 10, pp. 203-207, Oct. 1955.

- S. B. Cohn, “Shielded Coupled Microstrip Lines” IRE Trans. MTT-3, No.10, pp.29-38, Oct. 1955.

- Wheeler, H. A., “Transmission Line Properties of Parallel Strips Separated by Dielectric Sheet,” IEEE MTT-13, pp. 172-185, 1965.

- Cristal, E.G., “Coupled Circular Cylindrical Rods Between Parallel Ground Planes”, IEEE Trans. MTT-12,  No. 7, pp. 428-438, July 1964.

- Getsinger, W. J., “Microstrip Dispersion Model,” IEEE MTT-21, pp. 34-39, 1973.

- Wheeler, H. A., “Transmission Line Properties of a Stripline Between Parallel Plates”, IEEE MTT-26, No.11, pp. 866-876, Nov. 1978.

- T. Itoh, “Generalized Spectral Domain Method for Multiconductor Printed Lines and its Application to Tunable Suspended Microstrips,” IEEE MTT-26, No. 12, pp. 983-987, Dec. 1978.

- Gupta, K. C., R. Garg, I. J. Bahl, “Microstrip Lines and Slotlines”, Norwood, MA, Artech House, 1979.

- Hammerstad, E. and O. Jensen, “Accurate Models for Microstrip Computer Aided Design”, IEEE MTTS-1980, Washington D.C., pp. 407-409, May, 1980.

- Gupta, K. C., R. Garg, and R. Chadha, “Computer-Aided Design of Microwave Circuits,” Norwood, MA, Artech House, 1981.

- Kisching, M. and R. H. Jansen, “Accurate Model for Effective Dielectric Constant of Microstrip with Validity up to Millimeter Wave Frequencies,” Elect. Letters, Vol.18, No. 6, pp. 272-273, March 1982.

- Kirshing, M., and R. Jansen, “Accurate Wide-Range Design Equations for the Frequency Dependent Characteristics of Parallel Coupled Microstrip Lines,” IEEE MTT-32, No. 1, pp. 83-90, Jan. 1984. Also in IEEE MTT-33, No. 3, p. 288, March 1985.

- Pramanick, P. and P. Bathia, “Computer Aided Design Models for Millimeter Wave Finlines and Suspended Substrate Lines”’ IEEE MTT-33, No. 12, pp. 1429-1435, Dec. 1985.

- Bedair, S. S., “Characteristics of Asymmetrical Coupled Shielded Microstrip Lines,” IEE Proc. Vol. 132, pp. 342-343, 1985.

- Abdallah, E. A., El-Deeb, N. A., “Fast Accurate Computational Method for Asymmetric Coupled Microstrip Parameters,” IEE Proc., Vol. 132, Pt. H, No. 1, pp. 69-70, 1985.

- Stracca, G. B., G. Macchiarella, and P. Politi, “Numerical Analysis of Various Configurations of Slab Lines,” IEEE MTT-34, No. 3, pp. 359-363, March 1986.

- E. Yamashita, M. Nakajima, K. Atsuki, “Analysis Method for Generalized Suspended Striplines,” IEEE MTT-34, No. 12, pp. 1457-1463, Dec. 1986.

- Hoffmann, R. K., “Handbook of Microwave Integrated Circuit”, Norwood, MA, Artech House, 1987.

- Tomar K. S., P. Bhartia, “New Quasi-Static Models for Computer Aided Design of Suspended and Inverted Microstrip Lines”, IEEE MTT-35, No. 4, pp. 453-457, April 1987.

- Rosloniec, S., “Design of Coupled Microstrip Lines by Optimization Methods,” IEEE MTT-35, No. 11, pp. 1072-1074, Nov. 1987.

- Shu, Y. Xiao-xia, Q. Wang Y., “Analysis Equations for Shielded Suspended Substrate Microstrip Line and Broadside Coupled Stripline,” IEEE MTTS Digest, pp.693-696, 1987.

- Wang Y., Gu, K., Shu, Y., “Synthesis Equations for Shielded Suspended Substrate Microstrip Line and Broadside Coupled Stripline,” IEEE MTTS Digest, pp.331-334, 1988.

- Rosloniec, S., “An Improved Algorithm for the Computer Aided Design of Coupled Slab Lines,” IEEE Trans. MTT-37, No. 1, pp. 258-261, Jan. 1989.