9. 7. HAIRPIN FILTERS

9. 7. 1. INTRODUCTION

The half wavelength long internal lines of an edge coupled line filter can be folded into various forms as described in the previous section. If the arms of the U-folded version are made closer so as to get a controlled electromagnetic coupling then the filter is named as hairpin filter [13], [19], [35]. Figure 9.32.a shows the classical edge coupled line filter while Figure 9.32.b shows the hairpin structure. Figure 9.32.c shows a variation of the hairpin structure.

Figure 9.32. Hairpin line filter structures.

Such filters are handled in Filpro using the similarity between all-stub circuit models of two cascaded unit elements and a hairpin section, as follows:

Figure 9.33.a shows the all-stub model of a pair of UE’s. Circuit model of a cascaded UE pair has a parallel OC+SC stub element in series arm with impedances Zoc and Zsc, with two shunt OC stubs on each side with impedances ZA and  ZB. These impedances satisfy the following relations:

Zsc= ZA+ZB	, 	Zoc= - (ZA+ZB)

and hence

Zoc= - Zsc

Figure 9.33. a) All-stub circuit model of a cascaded unit element pair and Filpro command to convert two UE’s into an uncoupled hairpin. b) All-stub circuit model of a hairpin coupled line. c) Filpro command for conversion of uncoupled hairpin into coupled hairpin.

On Filpro, two adjacent equal length TL’s can be converted into their all-stub equivalents by applying the command Hairpin Operations / Two Tline-to-Hairpin. The resulting all-stub section is exactly equivalent to the pair of TL’s, with the property of Zoc=-(ZA+ZB)=-Zsc. The two adjacent TL’s may also be shown in folded form, as shown in the rightmost of Figure 9.33.a, provided there is no coupling between the folded arms. We will call this folded circuit section as Uncoupled Hairpin, as it looks like a hairpin.

If coupling is allowed between the folded arms, then we get the actual hairpin coupled line (Figure 9.33.b). The slot between the two arms acts as a two-wire transmission line short circuited at one end, that is a SC stub between the open ends of the two lines. In the circuit model coupling between the two arms shows itself as appearance of a short circuited stub of impedance Zs as in Figure 9.33.b. Characteristic impedance Zs of the SC stub formed by the slot is set by the separation between the arms of the folded line. Since this SC stub appears to be in parallel with the series arm OC+SC stub element, the total SC stub impedance will decrease leading to Zsc< Zocwhile the negative OC stub impedance remains the same, Zoc=(ZA+ZB). Thus, the actual hairpin structures are characterized by the properties

Zoc= -(ZA+ZB),

and

Zsc < Zoc

Thus, the difference between an uncoupled hairpin and coupled hairpin is that in coupled hairpin we have Zsc < Zocwhile in uncoupled case we have Zoc=-Zsc.

The circuit model of uncoupled hairpin structure can be converted into coupled hairpin form by simply modifying impedances ZA, ZB, Zsc and Zoc so as to satisfy the conditions Zoc=-(ZA+ZB) and Zsc<Zoc. However an arbitrary scaling of impedances in a filter is not possible because this will destroy characteristics of the filter. Fortunately, Gysel [19] noticed that a simple scaling obtained by multiplying all OC stub impedances by a constant () and dividing all SC stub impedances by the same constant would lead to a change only in the bandwidth of the filter while all the other specifications remain unaffected:

Z’A=  ZA ,  	Z’B= ZB  ,	 Z’oc = Zoc ,  	Z’sc = Zsc /

This leads to the conditions of a coupled hairpin structure:

Z’oc= - (Z’A+Z’B),	Z’sc < Z’oc

This type of scaling of element values is typical of reactance transformations from LP to HP, from HP to HP or from BP to BP in LC circuits or all-stub circuits which shifts the passband. In BP filters this scaling contracts or expands the bandwidth. For  >1 bandwidth will contract. This is not applicable to filters having transmission lines (UE’s). This is why we have to convert the stub-UE model of edge coupled line filter into all-stub form.

In Filpro, impedance scaling is carried out by applying the command Hairpin Oprs / Hairpin BW Scale: Alpha=.. , as shown in Figure 9.33.c.

These two commands of Filpro is used for conversion of an edge coupled line filter into a hairpin filter as described in Figure 9.34:

- Remembering that the circuit models of edge coupled line filters involve pairs of adjacent TL’s, we can use the command Hairpin Oprs / Two TLine-to-Hairpin to convert edge coupled line filters into exactly equivalent uncoupled hairpin form (Figure 9.34.a and b).

- The uncoupled hairpin sections of the filter can be converted into coupled hairpin form by applying the command Hairpin Oprs / Hairpin BW Scale: Alpha=.. (Figure 9.34.b-c). In this command, the scale constant Alpha will be selected by the designer and the contraction of bandwidth will be inspected for determining the proper band edge frequency specifications.

The all-stub circuit model of a coupled hairpin section can be converted into coupled line form by applying the command Replace By PCL / Comblines / Hairpin, as described in Figure 9.34.c. Coupling coefficient can then be determined from the resulting even-odd mode impedances of the resulting hairpin coupled section. Figure 9.34.d-e describes conversion of a whole edge coupled line filter into hairpin form.

Let’s continue discussions on examples.

Figure 9.34. Cohn’s model for edge coupled line section and coupled resonator form of the hairpin filter.

9. 7. 2. NARROWBAND HAIRPIN FILTERS

EXAMPLE 9. 22.  Hairpin filter design using Built in LC Prototype approach

Let’s consider the same narrowband edge coupled line filter example of the previous section. The specifications are

PBRipple=0.01, fp=975 MHz, fq=1000 MHz, N=7

- The Built in LC Prototype approach with the options With Redundant UE / TL+Inverter+TL and 50 ohms Resonator Impedance will be used giving the circuit of Figure 9.35.a.

- All the inner TL’s are in adjacent pairs, ready for conversion into hairpin form. However the TL’s at ports are single. In order to make them also pairs, we insert redundant TL’s at both ports (Figure 9.35.b). These pairs will increase the size, hence loss of the filter, as we will have two more hairpin sections. An alternative approach would be to delete the single TL’s at ports and use tapping. This approach will be discussed in the next example.

- With cursor under the left one of the pairs of TL’s, the command Hairpin Oprs / Two Tline-to-Hairpin is applied to convert the pairs of TL’s into their all-stub equivalents (Figure 9.35.c). These all-stub circuit sections are actually uncoupled hairpin sections. That is they are just another representation of the adjacent pairs of TL’s. This is seen from the impedances of the series arm OC+SC stub elements. The OC stub impedance is –100 ohms and the SC stub impedance is 100 ohms. That is, we have the property of Zsc=-Zoc of uncoupled hairpin sections.

- In order to convert the uncoupled hairpin sections into actual coupled hairpins we apply the command Hairpin Oprs / Hairpin Bandwidth Scale: Alpha=1.1. Here the constant Alpha=1.1 is a trial value. All OC stub impedances are multiplied by this constant and all SC stub impedances are divided by this constant, as shown in Figure 9.35.d. Thus, we now have Zsc<Zoc, a condition which implies that the arms of the hairpin sections are now coupled.

- This operation shrinks the bandwidth of the filter, as shown in Figure 9.35.e. The lower edge of the passband is shifted from 975 MHz to 976.75 MHz. A 1.75 MHz shift arised. Return loss level is not effected from this operation. In order to compensate this frequency shift, we restart the design by specifying a passband edge of  973.25 MHz instead of 975 MHz. The new circuit is shown in Figure 9.35.f and saved as hairp.fil for use at later stages.

- Next the command Hairpin Oprs / Two Tline-to-Hairpin is applied to convert the TL pairs into their all-stub equivalent. Note that the series arm OC+SC stub element has impedances  Zoc=-100 ohms and Zsc=100 ohms, indicative of the uncoupled nature of the hairpin. Note also that the shunt OC stubs has 50 ohms impednaces which are the impedance of the TL’s.

- Next the command Hairpin Oprs / Hairpin BW Scale: Alpha=1.1 on this circuit to get the all-stub form of the hairpin  filter (Figure 9.35.g-h). Here we have Zsc<Zoc which indicates that the arms of hairpin are coupled to each other. Note also that the shunt OC stub impedances are scaled up from 50 ohms to 55 ohms.

- Finally, in order to see the even-odd mode impedances of the hairpin coupled sections, we place the cursor under the series arm OC+SC stub elements and apply command Replace By PCL / Combline / Hairpin (Figure 9.35.h-i). Thus, we get the standard direct coupled resonator filter topology with hairpin resonators and with inverter couplings. Note that after band shrinkage the passband edge is shifted from 973.25 MHz to the desired 975 MHz (Figure 9.35.k).

- The degree of coupling between arms of a hairpin section can be inspected by the difference between even and odd mode impednaces shown in Figure 9.35.i. As the difference increases we get tighter coupling which means smaller separation between arms of the hairpin section. For looser coupllings even and odd mode impedances gets closer to each other. One common measure used to denote the degree of coupling is

Hairpin Coupling = 20 Log [(Zeven-Zodd)/Zeven+Zodd)] = - 20.4 dB

- Finding even-odd mode impedances of the edge coupled sections are problematic, as follows.

- Figure 9.35.m-n shows the steps for displaying the even-odd mode impedances of the original edge coupled line filter (before Hairpin Bandwidth Scaling). We just note down these even and odd mode impedances for comparison with the ones that will be obtained after the bandwidth scaling.

- Since the all-stub version of the circuit is scaled, the even and odd mode impedance parameters of the edge coupled sections are also changed. This is expected, because we noted that impednaces of the shunt OC stubs of the all-stub models are increased from 50 ohms to 55 ohms. Actually, we have a more basic problem here: The concepts of even-odd mode impedances are defined only for a pair of coupled lines while in this application we have many lines coupled to each other. To be more specific, in the previous applications we were converting the TL+Inverter+TL triplets into edge coupled line form. That is, the an inverter must have TL’s on both sides. However now inverters have coupled hairpin sections on both sides. Therefore at first sight it seems that the usual even-odd mode impedance descriptions are not valid for such multi-coupled lines. However, if the degree of hairpin coupling is low, we may assume that we still have TL’s on the two sides of inverters. That is we will unfold the hairpins as described in Figure 9.35.p and then convert the resulting TL+Inverter+TL triplets into edge coupled form. This is a gross approximation. However it can be used as a starting point for a more sophisticated electromagnetic analysis and optimization procedure or a simpler cut-and-try approach.

- Now the inverters have 55 ohms TL’s on both sides instead of 50 ohms of the unscaled version. In order to find even-odd description, we load the circuit hairp.fil and use Change command to change the 50 ohms TL impedances to 55 ohms (Figure 9.35.q-r).

- Then apply command Replace By PCL / Interdigital / ococINTD: f0=1000 MHz to get the edge coupled sections (Figure 9.35.s). Inspection of the even-odd mode impednaces of edge coupled sections shows that the new even-odd mode impedances are approximately equal to the original even-odd mode impedances multiplied by the factor Alpha=1.1. Therefore we may use this result as a rough starting point for an iterative search for a better solution.

- Figure 9.35.t  shows a pictorial description of the hairpin filter.

Figure 9.35. Hairpin filter design using Cohn’s model for edge coupled lines.

Figure 9.35. Hairpin filter design using Cohn’s model for edge coupled lines (cont’d).

Figure 9.35. Hairpin filter design using Cohn’s model for edge coupled lines (cont’d).

EXAMPLE 9. 23. Tapped hairpin filters

In the previous example the hairpin and edge coupled line sections at the ports are acting just as impedance transformers for scaling down internal impedance level. However this is is too much price paid for that purpose because we have introduced four coupled line sections (two edge coupled and two hairpin) which increase the size and loss of the filter. Further the edge coupled line sections at the ports are much more tightly coupled than the internal edge coupled sections making realization problematic. We can simulate the action simply by tapping the first hairpin resonator line, as was described in the tapped edge coupled and tapped folded  examples of the previous sections. This is described below.

- The initial design steps are the same as the previous example, giving the circuit which was saved as hairp.fil.. Therefore we start by loading the circuit hairp.fil (Figure 9.36.a).

- The two TL’s at source (and load) end are deleted so that inverters will appear at these ports (Figure 9.36.b). This circuit is saved as hairp2.fil for use at later stages. The inverters at the ports will be simulated by tapping the TL adjacent to the inverter, as was done in the previous tapped filter examples.

- In replacement of the inverters at ports by tapped resonator sections, Filpro requires a half wave TL adjacent to these inverters. Therefore we combine the two quarter wave TL’s adjacent to the  the inverters (Figure 9.36.c).

- With cursor under the end inverters the command Tapped PCL: Z0=50, f0=1000, fq=1000 MHz is applied to find the tapping point on the half wave TL. The piece of TL on the left of the tapping point appears as a shunt OC stub in Figure 9.36.d. As it was seen in the previous tapping examples, this tapping approach degrades response appreciably and needs tuning of all the other elements. This circuit is saved as hairp3.fil. It contains tapping information to be used later.

- Now we will convert the adjacent TL pairs into hairpin forms. However conversion of the end inverters into tapped form used up some parts of the half wave TL as seen in the circuit of hairp3.fil. That is, the tapped resonators are no more in Two-TL form. Therefore in order to continue the design we re-load the original circuit hairp2.fil which has inverters at the ports (Figure 9.36.e) and apply command Hairpin Oprs / Two Tline-to-Hairpin on the pairs of TL’s to form uncoupled hairpins (Figure 9.36.f).

- Then the command Hairpin Oprs / Hairpin Bandwidth Scale / Alpha=1.1 is applied to form the actual all-stub equivalents of the coupled hairpin sections (Figure 9.36.g). Here we note down the change in the impedance of the shunt OC stubs after bandwidth scaling.  Originally they were 50 ohms which is the impedance of the resonator lines of the unscaled circuit. After bandwidth scaling this impedance is increased to 55 ohms.

- In order to display the even and odd mode impedances of hairpin coupled sections we use command Replace By PCL / Comblines / Hairpin is applied (Figure 9.36.h). Note that we have two less hairpin sections and two less edge coupled sections compared to the previous design. Therefore it is natural to use this approach. We save this circuit as hairp4.fil.

- Next, we will determine the parameters of edge coupled line sections. We again start by loading hairp2.fil (Figure 9.36.i). Since the TL impedances were increased to 55 ohms after bandwidth scaling we change the TL impedances of hairp2.fil to 55 ohms (Figure 9.36.k) and apply command Replace By PCL / Interdigital / ococINTD: f0=1000 MHz to convert the TL+Inverter+TL triplets into edge coupled sections. The resulting circuit gives the even and odd mode impedances of edge coupled line sections. It is saved as hairp5.fil (Figure 9.36.m).

- Thus, the parameters needed for realization of the actual hairpin filter are dispersed into the three circuits: hairp3, hairp4 and hairp5. They are repeated in Figure 9.36.k, leading to the pictorial description shown in Figure 9.36.m.

- We should note that we have two less edge coupled sections and two less hairpin sections compared to the circuit of the previous example. Therefore it is natural to prefer this approach whenever possible.

Figure 9.36. Design of a tapped hairpin filter.

Figure 9.36. Design of a tapped hairpin filter.

EXAMPLE 9. 24. Filters with stepped edge coupled and hairpin coupled sections

More variations can be obtained for hairpin type filters by using stepped resonators. Stepped resonators can shift the spurious passbands away from the harmonics of passband center. Let’s design the same filter of the previous examples:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, N=7

- Using Design / By Prototypes with Inverters approach we get the filter of Figure 9.37.a, as in the previous examples and the shunt LC resonators are equated by applying the command Inverters / All Equal Left-Right (Figure 9.37.b).

- The circuit is impedance scaled by applying the command Inverters / Scale Cct By Inserting Inverters / Specify Tline: Z=50 ohms (Figure 9.37.c).

- Let’s design a tapped input-output filter. For this purpose, convert the LC resonators adjacent to the inverters at the source and load ends into half wave TL’s by applying the command Resonator Operations / pCLsh-to-Half Wave Tline: Z=50 ohms, Er=1 where Z is the impedance and Er is the dielectric constant of the TL (Figure 9.37.d).

- Then with cursor under the inverters at source and load ends the command Tapped PCL: Z=50, f0=fq=1000 MHz is applied  to find the tapping point. The piece of the half wave TL between the tap point and the open end of the line appears as a shunt OC stub (Figure 9.37.e).

- Next, the inner LC resonators are replaced by their four step TL equivalents by applying the command Resonator Operations / pCLsh-to-Four Step Tline: Z1=100, Z2=50, L2=30 mm. Here Z1, L1 and Z2, L2 are the impedances and lengths of the inner and outer pairs of TL sections as shown in the figure. We specify Z1=100 ohms, Z2=50 ohms and L2=30 mm (as a trial)  and Filpro calculates L1=28.757 mm (Figure 9.37.f). Thus, the TL pieces on each side of each inverter will have 50 ohms impedance and 30 mm lengths, except the outermost TL lengths. We split these TL’s also to form 30 mm length TL’s by applying the command Split / Specify Left: Lng=30 mm and use Exchange command to move the 30 mm TL adjacent to the inverter (Figure 9.37.g). This circuit is saved as STEP20.fil (for example) for use at a leter stage of the design procedure. The wideband insertion loss and passband return loss are shown in the same figure. Wideand response shows the shift in the first spurious passband away from 2f0. Degradation in return loss is due to tapping. It can be corrected by tuning all elements, however we leve this process to the last stage.

- The circuit sections corresponding to edge coupled lines are indicated in the figure. These circuit sections are interleaved by pairs of identical TL’s of length 28.757 mm. Now we will convert these pairs of TL’s into hairpin forms by brute force. The procedure is described in Figures 9.37.h-to-m:

- First we delete all the elements except a circuit section in between two successive inverters (Figure 9.37.h-i). The circuit of Figure 9.37.k is equivalent to a shunt LC resonator in between two inverters where in place of LC resonator we have its four step TL equivalent. Insertion loss response of this circuit section is shown on the right. As expected it shows a resonance behavour at the passband center of 1000 MHz.

- Next, with cursor under the left one of the two adjacent 28.757 mm TL’s, we apply the command Hairpin Oprs / Two TL-to-Hairpin to convert the TL pair into its all-stub equivalent (Figure 9.37.k). Since this is an exact equivalence, response will not change. It should be noted that the series arm OC stub has negative impedance which is equal in magnitude to the series arm SC stub impedance. Also the magnitude of OC and SC stubs is equal to the sum of the two shunt OC stub impedances.

- Next, we apply command Hairpin Oprs / Hairpin Bandwidth Scale: Alpha=1.1 to convert the all stub circuit section into a coupled hairpin equivalent by multiplying the OC stub impedances and dividing the SC stub impedance by Alpha (Figure 9.37.m). The series OC stub impedance magnitude is still equal to the sum of the wto shunt OC stub impedances. However the SC stub impedance is smaller than the magnitude of series OC stub which means that this circuit is now equivalent to a hairpin section with electromagnetic coupling between the arms. Since this is a brute force change, response will change as shown in the figure. The introduction of coupling between the arms of hairpin section shifted the resonance to a higher frequency.

- With cursor under the series arm OC+SC stub element we apply the command Replace By PCL / Comblines / Hairpin to convert the all-stub circuit section into hairpin form, as shown in Figure 9.37.n.

- Next, by applying Tune Element command we tune the length of the hairpin section until the original resonance frequency is recovered (Figure 9.37.p). The new length of the hairpin section comes out as 30.907 mm.

- Using  Edit / Copy command of Filpro we copy the hairpin section to be placed into the original filter circuit which was saved as STEP20.fil.

- The circuit  STEP20.fil  is loaded. The pairs of TL pieces of length 28.757 mm are deleted and the hairpin section is inserted into their place by using Edit / Paste command (Figure 9.37.q-r). Since the resonance frequencies of the original four step resonator and its hairpin version are the same, response of the filter will remain the same (See Figure 9.37.t). Save the circuit as STEP21.fil.

- Now we replace the TL+Inverter+TL triplets by their edge coupled line equivalents by applying the command Replace By PCL / Interdigital / ococINTD: f0=1000 MHz (Figure 9.37.s).

- Final stage is recovery of return loss response by tuning elements or by applying Best Case command. Result of a quick trial is shown in Figure 9.37.t. Further improvement is possible at the expense of a slight shrinkage in passband.

- Figure 9.37.u shows a possible pictorial description of the circuit.

Figure 9.37. Design of a filter having both edge coupled and hairpin coupled sections.

Figure 9.37. Design of a filter having both edge coupled and hairpin coupled sections. (cont’d).

Figure 9.37. Design of a filter having both edge coupled and hairpin coupled sections (cont’d).

EXAMPLE 9. 25. Medium bandwidth hairpin filter design using Prototype with Inverters approach

Built in LC Prototype and Prototype with inverters approaches are easiest to use for narrowband filters. They can also be used for medium and wideband filters. However in medium-wide band filters the approximations involved in circuit transformations used in these approaches cause large shifts in band edges leading to narrowing of the passband accompanied by gross distortions in return loss response. These distortions can be corrected by a careful tuning process and by specifying wider initial trial bandwidths which is somewhat tedious. Let’s see details on an example with the following specifications:

PBRipple=0.01 dB, fp1=850 MHz, fp2=1150 MHz (30 % bandwidth),

Seven resonators will be used (NR=7), with resonator impedances of 50 ohms.

- Design command set is shown in Figure 9.38. Design / By Prototype with Inverters approach is used. Shunt Capacitors+Inverters option is selected. After typing in the prototype degree and PBRipple the lowpass prototype circuit is displayed automatically. Then Filter Type is invoked and Lumped-Bandpass options are selected. After typing in the passband edge frequencies we get the circuit of the bandpass filter in lumped LC resonator form coupled with ideal inverters (Figure 9.38.a).

- Applying Inverters / All Equal Left-Right command we get the identical LC resonator form (Figure 9.38.b). We save this circuit for use in later stages of the design, as for example hrp21.fil. We can also display and note down the External Q and Coupling Coefficients of the circuit as the targeted parameters (by pressing the key combination Ctrl+F6 and selecting the option Direct Coupled). These parameters will not change much in the forthcoming circuit transformations as long as the circuit response remained the same.

- Next, we start the procedure for transforming this lumped element filter first into and edge coupled line form and then into Hairpin form. The first step is to convert the LC resonators into Half Wave TL resonators. This is possible either by using the command Replace By TL or Resonator Oprs / pLCsh-to-Half Wave TL. Both gives the same result (Figure 9.38.c-d). We accept the displayed default value (26.7 ohms) as the impedance of the Half Wave TL. This conversion will cause gross degradations in response. Degradation in return loss ripple can be corrected by tuning. However shifts in passband edges can’t be done by simple tuning and  necessitates new design trials by specifying different band edge frequencies. Since more band edge shifts will come from conversion into hairpin form, we continue the design to see how much total shifts will result from the overall design procedure.

- In the next step impedances of TL’s will be raised from 26.7 ohms to 50 ohms. This is done by applying the command Inverters / Scale Circuit By Inserting Inverters / Specify TL: Z=50 ohms. Here the inverter impedance that will gave 50 ohms TL impedance is found after a few trials (Figure 9.38.e).

- Next, in order to see the level of even and odd mode impedances the circuit will be converted into edge coupled line form. This is done by splitting the Half Wave TL’s into two Quarter Wave TL’s by placing the cursor under the TL’s and applying the command Split / Two Equal, thus forming TL+Inverter+TL triplets (Figure 9.38.f).  This circuit is saved as hrp22.fil to be used in later stages.

- Then these triplets are converted into OC Edge Coupled Line form by placing the cursor under the inverters and applying the command Replace By PCL / Interdigital / ococINTD / f0=998.865 MHz (Figure 9.38.f-g). Here, we type in the maximum frequency displayed at the bottom of the screen. This maximum frequency will be the actual quarter wave frequency of the splitted TL pieces which resulted from the LC resonators. It is close to the arithmetic band center of 1000 MHz. Return loss response before and after conversion into edge coupled line form are shown in the figures. It is seen that this conversion also introduces some more degradation in response.

- We inspect the even-odd mode impedance levels for realizability. If it is OK, then continue the design. If not we have to re-scale the circuit by introducing a different inverter constant.

- Next, we load the circuit hrp22.fil and with cursor under the left one of the splitted TL pairs, the command Hairpin Oprs / Two Tline-to-Hairpin is applied to replace the TL pairs with their exact all-stub equivalents with negative OC Stub in series arm (Figure 9.38.h-k). These all-stub equivalents can be interpreted either as two adjacent TL’s or a hairpin section with no coupling between the folded arms.

- In order to introduce couplings between the hairpin arms, we apply the command Hairpin Oprs / Hairpin Bandwidth Scale: Alpha=1.1. Here the scaling constant Alpha=1.1 introduces of about 20.4 dB coupling between the arms while also shrinking the bandwidth (see previous examples for the definition of this coupling). This operation multiplies all OC stubs and divides all SC stubs by the same constant Alpha=1.1. In the resulting circuit we should note that the shunt OC stub impedances are increased from 50 ohms to 55 ohms. This means that in the triplets of TL+Inverter+TL for forming OC edge coupled line forms, the new TL impedances will be 55 ohms. That is conversion into hairpin form will increase impedance level of the edge coupled line sections.

- Finally, with cursor under the series arm OC+SC stub elements, the command Replace By PCL / Comblines / Hairpin is applied to convert the shunt OC Stub+Series OC+SC Stub+Shunt OC stub triplets into hairpin forms (Figure 9.38.k). We plot the return loss to note down the resulting lower and upper band edges. The return loss ripples could be equated by tuning elements, however at this stage we don’t need to do this. We note that the lower edge is shifted up from 850 MHz to roughly 872 MHz. A shift or 22 MHz is observed. The upper edge is shifted from 1150 MHz to 1105 MHz. A shift of 45 MHz is observed. We have to repeat the design by further decreasing the lower edge and increasing the upper edge.

- The new edge frequencies need be found by trials. As a first trial, we specify fp1=825 MHz and fp2=1225 MHz to see how much closer we will get to the targeted frequencies of  850-1150 MHz (Figure 9.38.m).

- The same steps are repeated as the first trial. The resulting filter with equal LC resonators is saved as hrp31.fil. Then the LC resonators are replaced by their Half Wave TL equivalents (Figure 9.38.n) and the circuit is scaled by inserting inverters to get 50 ohms TL impedances again. This time an inverter constant came out to be K=61.1 ohms (Figure 9.38.p-q).

- The Half Wave TL’s  are splitted into two Quarter Wave TL’s (Figure 9.38.q-r).

- The resulting circuit is then converted into hairpin form by first applying the commands Hairpin Oprs / Two Tline-to-Hairpin (Figure 9.38.r-s) and then Hairpin Oprs / Hairpin Bandwidth Scale: Alpha=1.1 (Figure 9.38.s-t) and then applying the command Replace By PCL / Combline / Hairpin (Figure 9.38.u). We also convert the inverters at source and load ends to Quarter Wave TL’s by applying the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline (Figure 9.38.u-v). The resulting response is shown below the circuit. The upper band edge coincided with the targeted 1150 MHz, but the lower edge needs a slightly more trimming by further lowering the low edge. However this small shift is acceptable as more tuning-trimming need be done during fabrication stages.

- The return loss response can be made equal ripple by tuning only the inverters and the TL impedances at source and load ends, as shown (Figure 9.38.w). In this circuit we just note down the resulting even-odd mode impedances of the hairpin coupled sections.

- The parameters of the OC edge coupled sections can be found by first loading the circuit of hrp31.fil and then use Change command to change impedances of TL’s into 55 ohms (shunt OC stub impedances of the all-stub forms of the hairpin sections) and then converting the triplets of TL+Inverter+TL into edge coupled line form, as shown in Figures 9.38.x-y-z1-z2. We also delete the inverters at source and load ends and insert the quarter wave TL’s with tuned impedances. Response of this circuit will have wider passband because the hairpin sections are now in unfolded form. When they are folded response will have the targeted passband.

- Figure 9.38.z3 shows a pictorial description showing the even and odd mode impedances of hairpin and edge coupled sections.

- Note that we have used only quarter wave TL’s for scaling impedance level of the circuit. That is, the impedance scaling inverters at source and load ends are replaced by simple quarter wave transformers. This seems a simpler solution compared to insertion of redundant unit elements which increase the number of edge coupled sections by two.

Tapping Approach:

- We may also try Tapping approach. However this approach will lead to extra degradation in return loss because the filter is not a narrowband one. Let’s see this approach also.

- We load the circuit with inverters at source load ends. That is, hrp31.fil (Figure 9.39.a).

- With cursor under the inverters at source and load ends, the cmmand Tapped PCL: f0=1000, fq=1000 MHz is applied. This command calculates the tapping length LT measured from the left end of the first half wave resonator and a shunt OC stub (OSsh) of length LT is inserted. The remaining piece of the first resonator of length L-LT with L being the full (commensurate) length of the original resonator  is left as TL (Figure 9.39.b). The mirror image of this structure appears at the load end.

- This transformation degrades response appreciably. Tuning the lengths LT and L-LT, together with the inner inverter constants enables us to get a roughly equal ripple return loss. However two of the return loss peaks disappeared as shown in the figure. This is because, contrary to the case with inverters, now the 50 ohms end resonators appear as just extensions of the source and load resistors RS=RL=50 ohms. That is they are no more resonators. This is basically because the tapping length is very small, and thus, effectively the input signal appears to be fed through a 50 ohms TL which can’t contribute the response.

- Inspection of the resulting band edges shows that we get additional band shrinkage which is due to tapping. Thus, we have to go back and make further adjustment in band edges. But before going back for re-design, we have to finish this design to see how much total band contraction we will get after conversion into hairpin form.

- In order to convert the circuit into hairpin form using the standard procedure, we need to have Half Wave TL pieces while the TL’s at left and right ends  are shorter and have lengths of      L-LT where L is the Half Wave Length. We can make these lengths also half wave length by using Change command while also introducing TL’s with negative length of –LT as shown in Figure 9.39.d-e.

- Then we split the half wave TL’s into two (Figure 9.39.e-f) and apply commands Hairpin Oprs / Two TL-to-Hairpin (Figure 9.39.f-g) and then Hairpin Oprs / Hairpin Bandwidth Scale: Alpha=1.1 to get the bandwidth scaled all-stub hairpin form (Figure 9.39.h). Then the all-stub hairpin sections are converted into Hairpin coupled line form by applying the command Replace By PCL / Combline / Hairpin (Figure 9.39.i). Inspection of response of this circuit shows how much correction must be done on passband edges.

- Figure 9.39.k shows a pictorial description of the tapped circuit.

Figure 9.38. A medium bandwidth hairpin filter with quarter wave line impedance scaling.

Figure 9.38. A medium bandwidth hairpin filter with quarter wave line impedance scaling (cont’d).

Figure 9.38. A medium bandwidth hairpin filter with quarter wave line impedance scaling (cont’d).

Figure 9.38. A medium bandwidth hairpin filter with quarter wave line impedance scaling (cont’d).

Figure 9.38. A medium bandwidth hairpin filter with quarter wave line impedance scaling (cont’d).

Figure 9.39. Tapped version of the medium bandwidth hairpin filter.

Figure 9.39. Tapped version of the medium bandwidth hairpin filter (cont’d).

9. 7. 3. HAIRPIN FILTER DESIGN USING GYSEL’S APPROACH

As it was seen in the previous example, medium-wideband filters are tedious to design using the Built in LC Prototype or Prototype with Inverters approaches. In this section an approach originated by Gysel [19] will be elaborated and combined with the Synthesis approach which will enable us to handle medium and wideband filters. In order to clarify the theory, before going directly into the design of a hairpin filter, step by step description of development of alternative equivalent circuits for edge coupled line filters will be given in the following example. Then the actual designs with realizable impedances will be described.

EXAMPLE 9. 26. Hairpin-Comb type circuit model for edge coupled line filters

Let’s consider the synthesis of a medium bandwidth edge coupled line filter, designed as a distributed element highpass filter with specifications,

PBRipple=0.01 dB, fp1=850 MHz, fq=1000 MHz

to get a bandpass filter with fp1=850 MHz and fp2=1150 MHz.

- Design command set is shown in Figure 9.40. Distributed / Highpass / Type A options are selected. Transmission zeros with Nzero=1 and NUE=NR-1=6 are placed. They are extracted in the order ue-ue-ue-zero-ue-ue-ue giving the symmetric circuit of Figure 9.40.a.

- Kuroda / Distribute Stub command is applied by placing the cursor under the TL’s adjacent to the OC stub to move part of the OC stub to the other side of the TL’s. This action is repeated until all TL’s have an OC stub on both sides (Figure 9.40.b).

- With cursor under the OC stubs in between the TL’s the command Split / Two Equal command is applied to split them into two parts so that separate triplets of OC Stub+TL+OC Stub will be formed which will be converted into edge coupled line sections (Figure 9.40.c).

- Then with cursor under any TL, the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate all the stubs (Figure 9.40.d). We save the circuit as filt51.fil .

- With cursor under the TL’s the command Replace By Tline / Interdigital / ococINTD / Symmetric is applied to convert the triplets into edge coupled line sections (Figure 9.40.e). We note down the even-odd mode impedances of the coupled line sections and save the circuit, for example, as filt52.fil to compare with the results of another approach that will be described in the following lines. Impedance level is too high for realization, but for the time being we will disregard this aspect. It will be taken into account in the next example.

- Load the circuit of filt51.fil (Figure 9.40.f). The triplets which are equivalent to the edge coupled line sections are denoted by PCL1, PCL2, etc. In this example we will develop new circuit sections which are also equivalent to the edge coupled line sections.

- With cursor under the TL’s-1, 3, 5, apply the command Kuroda / Right / Full Stub to move the OC stubs on their rights to their left side. Similarly, with cursor under the TL’s-2, 4 and 6, apply command Kuroda / Left / Full Stub to move the OC stub on left to their right sides (Figure 9.40.g). Each application of this command produce a transformer. With cursor under any transformer we apply command Shift Trf / All Rightmost to shift the transformers to rightmost position and combine them there to end up in unity, hence deleted.

- We also Combine the translated stubs with the stubs which were already there, getting the circuit of Figure 9.40.h. Note that in this circuit we have adjacent pairs of TL’s which will be converted into hairpin forms by applying the command Hairpin Oprs / Two Tline-to-Hairpin. However before doing this, we may want to equate the impedances of adjacent TL’s. This is possible by translating a small piece of OC from the left to right or from right to left of the TL pair In this example impedance of the leftmost TL is 16.4 ohms while the adjacent is 16.1 ohms (numbers are rounded in the figures). The middle TL’s have already equal impedances.  With cursor under the left one of the left TL pair, the command Kuroda / Left / Specify TRF: n=0.99215 is applied to move part of the stub on the left to the right of the leftmost TL while also producing a transformer (Figure 9.40.i). The transformer turns ration n=0.99215 is found by trials until equal impedances are obtained for this pair of TL’s. Then with the cursor under the next TL, Kuroda / Left / Full Stub is applied to move the stub in between the two TL’s to the right of the second TL while producing another transformer (Figure 9.40.j). Then, with cursor under any transformer, the command Shift TRF / All Rightmost is applied to move the transformers to the load end.  The stub translated to the right of the second TL is combined with the adjacent stub there. In order to avoid repetition of thiese commands at load end we place the cursor under the middle element and apply Symmetrize command and Delete the extra element produced in this operation.  The circuit of Figure 9.40.k shows that the adjacent TL’s have equal impedances (16.5 ohms). Thus, the target is reached. This result is obtained for the choice on turns ration n=0.99215 used in the first Kuroda transformation described above. This value of n is found after a few trials. We save this circuit as filt53.fil.

- In order to find the edge coupled line filter corresponding to this equal TL version (filt53.fil) the cursor is placed under the TL’s and the command Kuroda / Left (right) / Equal Stub is applied to form OC Stub+TL+OC Stub triplets (Figure 9.40.m-n). The circuit of Figure 9.40.n is saved as filt54.fil. Then the triplets are converted into edge coupled line forms as shown in Figure 9.40.n-p. This circuit is also saved as filt55.fil, to be used as reference for derivation of hairpin filter. At this stage we can compare the even and odd mode impedances of filt54.fil and filt55.fil with those of filt51.fil and filt52.fil to inspect the effects of equating adjacent TL’s.

Folding into hairpin form without hairpin-coupling:

- Now we are ready for formation of hairpin version of this filter. We load the circuit with equal impedance adjacent TL’s (filt53.fil) and by placing the cursor under the left one of the TL pairs the command Hairpin Oprs / Two TLine-to-Hairpin is applied to replace the TL pairs by their exact all-stub equivalents made up of shunt two OC stubs interleaved by a series OC+SC stub element with negative OC stub (Figure 9.40.q-r). Remember that we have termed this circuit section as uncoupled hairpin, because, being exact equivalent of a pair of adjacent TL’s, it is also equivalent to a hairpin section with no coupling between the hairpin-folded arms. This circuit is saved as filt56.fil.

- In order to verify that these circuit sections lead indeed to uncoupled hairpin sections, we place the cursor under the series arm OC+SC stub elements and apply command Replace By PCL / Combline / Hairpin. This command gives the hairpin coupled line sections, as shown in Figure 9.40.s. The circuit is saved as filt57.fil. The even and odd mode impedances of these coupled line sections are equal. Equality of even-odd mode impedances means that these lines are actually uncoupled.  So, we have folded the half wave resonator TL’s into hairpin forms, but without coupling between the hairpin arms yet. Later we will scale this circuit to get actual hairpin couplings.

- Note that in the circuit of filt56.fil we have lost the familiar OC Stub+TL+OC Stub triplets that correspond to the edge coupled sections. Thus, at first sight it seems that we have lost the means to display the even-odd mode impedances of the edge coupled sections. Fortunately, this information will appear by converting this circuit into another equivalent form, as described next.

- Load the circuit of filt56.fil (Figure 9.40.t).. In the circuit of filt56.fil we see inverted L-sections of OC stubs, some looking left, named as LLeft sections and some looking right named as LRight sections. With cursor under the shunt legs of the Lleft sections we apply command Pi-Tee-L / Lleft-to-Sym Pi to form symmetric Pi-sections of OC stubs. Similarly, with cursor under the shunt legs of the LRight sections we apply command Pi-Tee-L / LRight-to-Sym Pi to form symmetric Pi-sections of OC stubs (Figure 9.40.t-u).

- Each application of these commands produce a transformer. The transformers are shifted rightmost and combined to form unity transformer, hence deleted by applying the command Simplify Circuit (F6) giving the circuit of Figure 9.40.v. This circuit is saved as filt58.fil.  This circuit contains even-odd mode impedances of edge coupled sections which can be displayed as described below.

- The symmetric Pi-sections of OC stubs are equivalent to comb-coupled line sections. With cursor under the middle element of the Pi-sections (series OC stub), applying command Replace By PCL / Combline / ococCOMB / Symmetric, we get the OC comb type coupled line sections as shown in Figure 9.40.w. The circuit us saved as filt59.fil. This circuit has an interesting property. In Figure 9.40.w, the circuit of the edge coupled line filter, filt55.fil is also repeated. Comparison of the even-odd mode impedances of the coupled line sections of filt59.fil and filt55.fil shows that the coupled line sections have identical impedances. Thus, filt59.fil is another representation of edge coupled line filter. In other words, the edge coupled line sections of filt55.fil appear as comb sections in filt59.fil.

- This representation is more informative compared to the circuit of filt55.fil, because this circuit displays the hairpin-folded nature of the half wave resonators while also giving the even-odd mode impedances of the edge coupled sections. The arms of hairpin foldings of this circuit are actually uncoupled. This is clear from the equality of magnitudes of the parallelled OC and SC stubs in series arms or from the equality of even and odd mode impedances of hairpin sections of filt57.fil.

- Figure 9.40.w shows also a pictorial description of the folded edge coupled line filter. The actual hairpin filter will be formed by introducing couplings between the arms of the hairpin folded sections, as will be described next.

Bandwidth Scaling and obtaining hairpin filter:

- The all-stub equivalent of the hairpin folded edge coupled line filter circuit, filt58.fil is loaded (Figure 9.40.x). Its return loss response is shown below the circuit.

- The command Hairpin Oprs / Hairpin Bandwidth Scale: Alpha=1.1 is applied. This command multiplies all OC stub impedances and divides all SC stub impedances by the scaling constant Alpha=1.1 (Figure 9.40.y). This is the targeted hairpin filter, in all-stub form. It is saved as filt60.fil. Here, Alpha is a constant that will be set by the user to adjust the amount of coupling between the arms of hairpin sections by destroying the equality of magnitudes of OC and SC stub impedances of the parallelled OC+SC stub elements in the series arms. Alpha=1.1 corresponds roughly about –20.5 dB of coupling.

- This scaling shrinks the bandwidth of the filter, as shown in the return loss plot. The Lower passband edge is shifted from 850 MHz to 863 MHz while the upper edge is shifted from 1150 MHz to 1137 MHz. A 13 MHz of inward shift is observed in both edges. For this reason we should re-start the design by specifying a wider bandwidth to account for this shrinkage.

- Figure 9.40.z1 shows the coupled line version obtained by placing the cursor under the series OC stubs of the Pi-sections and applying the command Replace By PCL / Combline / ococCOMB / Symmetric. This is the targeted hairpin filter structure. It is saved as filt61.fil. In this circuit the even-odd mode impedances Z’even and Z’odd of the comb sections are the even and odd mode impedances of the edge coupled sections of the hairpin filter. Comparison of  Z’even and Z’odd of the hairpin filter with the even odd mode impedances Zeven and Zodd of  filt55.fil (the edge coupled line filter without coupling between hairpin arms) shows that  the new even and odd mode impedances are just the old ones multiplied by the scaling constant Alpha=1.1:

Z’even =(Alpha) (Zeven)

and

Z’odd= (Alpha) (Zodd)

Thus, at first sight it seems that it is not necessary to go through so many steps to get the even and odd mode impedances of the hairpin folded edge coupled sections. We could as well get the same results by simply multiplying the even and odd mode impedances of the unfolded edge coupled sections by the bandwidth scaling constant Alpha. However unfortunately, we still have to repeat these steps in order to see how much band shrinkage will result.

- Thus, we have determined the even-odd mode impedances of edge coupled sections.

- Although not necessary, we can also determine the even and odd mode impedances of hairpin folded arms. This way we can verify that these coupled lines have about -20.5 dB coupling coefficients corresponding to Alpha=1.1. For this purpose we load the circuit of filt56.fil (Figure 9.40.z2).

- The hairpin sections will be formed from the Pi-sections having OC+SC stubs in series arm with negative OC stub, surrounded by two shunt OC stubs.  However in order that such a Pi-section to represent a hairpin coupled section, we need to satisfy the condition ZA+ZB=-Zoc where Zoc is the negative impedance of the OC stub component of the series arm OC+SC stub element and ZA and ZB are the impedances of the shunt OC stubs of the Pi-section on the two sides of series Zoc. Since we have equal shunt OC stubs, this condition reduces to ZA=-Zoc/2. Such a Pi-section can be formed by splitting the shunt OC stubs on the two sides of the series OC+SC stub elements by applying the command Split / Specify Left: Z=-Zoc/2. This splitting will produce some negative shunt OC stubs, together with the necessary shunt OC stubs, as shown in Figure 9.40.z3. The circuit sections corresponding to hairpin coupled lines are indicated in the figure.

- Now, with cursor under the series OC+SC stub elements, the command Replace By PCL / Combline / Hairpin is applied. Thus, we get the circuit of Figure 9.40.z4.  The even and odd mode impedances can be used to calculate the coefficient of coupling as shown in the figure. They came out to be about –20.4, -20.5  dB, as expected.

- Figure 35.z5 summarizes the results of our efforts in a pictorial description where the even and odd mode impedances of edge coupled line sections and coupling coefficients of the hairpin coupled sections are shown.

Since the resulting impedances are unrealizably high, unfortunately our design task is not ended yet. Impedance level can be scaled down either by inserting redundant unit elements, thus increasing the number of edge couplings by two, or, if possible, by tapping the first and last resonators, as we had done in the previous sections. Lets see these approaches on examples with modified passband edges to account shrinkage due to hairpin bandwidth scaling.

Figure 9.40. Development of hairpin-comb type circuit model for edge coupled line filters.

Figure 9.40. Development of hairpin-comb type circuit model for edge coupled line filters (cont’d).

Figure 9.40. Development of hairpin-comb type circuit model for edge coupled line filters (cont’d).

Figure 9.40. Development of hairpin-comb type circuit model for edge coupled line filters (cont’d).

Figure 9.40. Development of hairpin-comb type circuit model for edge coupled line filters (cont’d).

EXAMPLE 9. 27. Synthesis of a hairpin filter with redundant unit elements

Let’s consider the same distributed element highpass filter of the previous example:

PBRipple=0.01 dB, fp=850 MHz, fq=1000 MHz

with Nzero=1 and NUE=6, to get a bandpass filter with fp1=850 MHz and fp2=1150 MHz. In order to take the inward band edge shifts of 13 MHz at each passband edge frequency, the lower edge of the highpass filter will be specified as fp=837 MHz. Thus the initial band edges will be fp1=837 MHz and fp2=1163 MHz.

- Design steps will be similar to those of the previous example up to step-d of Figure 9.40, for fp=837 MHz. That is, the transmission zeros are extracted in the order UE-UE-UE-zero-UE-UE-UE (Figure 9.41.a) and Kuroda / Distribute Stub command is used to place OC stubs on each side of each TL (Figure 9.41.b). Then the inner stubs is split into two pieces by applying Split / Two Equal command thus forming the OC Stub+TL+OC Stub triplets (Figure 9.41.c). Then the OC stubs are equated by applying the command Kuroda / All Equal Stubs/ Include End Stubs (Figure 9.41.d). After this step we deviate from the steps of the previous example temporarily just to consider the impedance level of the circuit.

- Knowing that if impedance scaling is not done, the approach will lead to too high impedances, we will scale the circuit by inserting redundant unit elements (quarter wave TL’s) at each end of the circuit and re-distribute the stubs to form two more coupled line sections. The redundant unit elements can be inserted as 50 ohms TL’s at the source and load ends without disturbing the response of the circuit. However this may lead to a too low impedance level. In order to have a better control on the impedance level of the resulting circuit, we prefer to scale the circuit by inserting inverters and then replace the inverters by quarter wave TL’s.  By adjusting the inverter constant, we can get any targeted impedance level. The price of this extra flexibility is some degradation in response which can be corrected by tuning a few elements, as described in the following lines.

- With cursor anywhere in the circuit the command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=75 ohms is applied getting the impedance scaled circuit of Figure 9.41.e. Here the inverter constant K=75 ohms is a trial value which need be iterated until the resulting impedance level is close to the desired level. The case with K=50 corresponds to the classical redundant unit element insertion approach (exercise with this value to see what impedance level you would get at the end  ).

- With cursor under the inverters at source and load ends the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline / f0=1000 MHz is applied to convert the inverters into unit elements (Figure 9.41.f). {The name redundant unit element may not be proper for this unit element because the classical redundant unit elements don’t contribute response while this unit element has some marginal effects on response. However we will continue to use the name redundant unit element to differentiate these extra TL’s from the inner TL’s}.

- Below Figure 9.41.f, the return loss responses before and after replacement of inverters with quarter wave TL’s are shown. The shaded region corresponds to the passband between fp1=837 MHZ and fp2=1163 MHz. It is seen that replacement of ideal inverters by TL’s degraded response appreciably. This degradation increases at the inverter constant K gets further away from K=50 ohms case. K=50 will not introduce any degradation, and would be equivalent to insertion of 50 ohms TL’s at source and load ends..

- Degradation in return loss can be corrected by tuning a few elements close to source and load ends, as shown in Figure 9.41.g. Note that return loss is improved from 26.3 dB to about 29 dB.

- After recovering return loss, the stubs adjacent to the TL’s at source and load ends are splitted into two by applying the command Split / Two Equal and then with cursor under these TL’s the command Kuroda / Distribute Stub is applied to translate some stubs to the other sides of these TL’s (Figure 9.41.g-h).

- Next, the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate all the stubs (Figure 9.41.i). We save this circuit as for example, filt71.fil.

- In order to get an idea on the resulting even-odd mode impedance level the triplets of OC Stub+TL+OC Stub are converted into edge coupled line form by placing the cursor under the TL’s and applying the command Replace By PCL / Interdigital / ococINTD / Symmetric (Figure 9.41.j). Although the displayed even-odd mode impedances will be increased slightly after conversion to hairpin form, these impedances can give us an idea for selecting a different impedance scaling inverter constant K or not. Let’s continue the design by assuming that the impedances shown in Figure 9.41.j are feasible.

- Load filt71.fil (Figure 9.41.k). We will try to form adjacent pairs of TL’s. For this purpose we willl transfer the stubs in between the TL’s to the other sides of the TL’s. This is done by placing the cursor under the TL’s-1,3 and 5, apply the command Kuroda / Right / Full Stub and then with cursor under the TL’s-2,4 and 6 apply the command Kuroda / Left / Full Stub (Figure 9.41.m).

- Then with cursor under any transformer, the command Shift TRF / All Rightmost is applied to move all transformers to rightmost position  where they are combined and deleted because they add up to unity. We also Combine the translated OC stubs with the stubs which were already there (Figure 9.41.n).

- Thus, we now have adjacent pairs of TL’s which can be converted into hairpin form by folding. However we may want to have equal impedances for the adjacent TL’s for simplicity in realization. In this circuit the left pair of TL’s have impedances 19.4 and 15.9 ohms while the next pair have impedances of 9.64 and 9.32 ohms. In order equate the impedances of the left pair, first we place the cursor under the left TL and apply command Kuroda / Left / Specify TRF: n=0.91. Here the transformer turns ratio is a trial value and found after a few iterations. This command translate part of the stub on left of the left TL to a position in between the two TL’s (Figure 9.41.n-p).

- Then with cursor under the right TL of the pair, the command Kuroda / Left / Full stub is applied to move the stub in between the two TL’s to the right of the right TL (Figure 9.41.p-q). The translated stub is then combined with the adjacent stub there. Also the resulting transformers are moved away to load end by applying the command Shift TRF/ All Rightmost. These operations equated the impedances of the left TL pair at 21.3 ohms.

- Next, he same opeartions are carried out on the next pair of TL’s. They are equated at 14.5 ohms level, for the transformer turns ratio of n=0.984, as described in Figures 9.41.q-r-s-t. The resulting circuit is saved as filt72.fil.

- Now we are ready for folding the adjacent pair of TL’s which are actually Half Wave Resonators into hairpin form. This is done in two steps. At first step with cursor under the left TL of the pairs, the command Hairpin Oprs / Two Tline-to-Hairpin is applied (Figure 9.41.t-u). This command replaces the TL pairs by their exact all-stub equivalents with two shunt OC stubs separated by a series OC+SC stub element with negative OC stub impedance. This all-stub circuit section is also equivalent to a hairpin folded TL, but with no coupling between the folded arms. This property is seen from the equality of the magnitudes of OC and SC stubs of the series OC+SC stub element.

- In the second step the command Hairpin Oprs / Hairpin Bandwidth Scale: Alpha=1.1 is applied to introduce coupling between the hairpin folded arms. Introduction of coupling shows itself as the difference between magnitudes of OC and SC stubs of the series OC+SC stub elements (Figure 9.41.v). The coupling coefficient is about 20.5 dB for Alpha=1.1, as was shown in the previous example.

- This brute force introduction of coupling will change the circuit response. In all-stub circuits this change appears as a simple shifts in band edge frequencies which were taken into account by specifying a wider bandwidth at the beginning of the design. Indeed the passband edge frequencies are shifted to the desired fp1=850 MHz and fp2=1150 MHz, as seen from the return loss plot in Figure 9.41.v.

- The next step is determination of the even-odd mode impedances of the edge coupled line sections. This is done by converting the LLeft and LRight sections of OC stubs into Symmetric Pi sections, as shown in Figure 9.41.v-w. After shifting all transformers rightmost we get the circuit of Figure 9.41.w.

- With cursor under the middle elements of the symmetric Pi sections, the command Replace By PCL / Combline / ococCOMB / Symmetric is applied to convert the symmetric Pi-sections into comb type coupled line sections (Figure 9.41.x).

- As was shown in the previous example, the even-odd mode impedances of these comb type coupled lines are equal to the even-odd mode impedances of the edge coupled sections of the hairpin filter. The edge coupled line sections corresponding to these even-odd mode impedances are shown in the pictorial description of Figure 9.41.x.

Figure 9.41. Synthesis of a medium bandwidth hairpin filter with redundant unit elements.

Figure 9.41. Synthesis of a medium bandwidth hairpin filter with redundant unit elements (cont’d).

Figure 9.41. Synthesis of a medium bandwidth hairpin filter with redundant unit elements (cont’d).

Figure 9.41. Synthesis of a medium bandwidth hairpin filter with redundant unit elements (cont’d).

EXAMPLE 9. 28. Synthesis of a medium bandwidth tapped hairpin filter

Let’s consider the same distributed element highpass filter of the previous example:

PBRipple=0.01 dB, fp=850 MHz, fq=1000 MHz

with Nzero=1 and NUE=6, to get a bandpass filter with fp1=850 MHz and fp2=1150 MHz. In order to take the inward band edge shifts of 13 MHz at each passband edge frequency, the lower edge of the highpass filter will be specified as fp=837 MHz. Thus the initial design band edges will be fp1=837 MHz and fp2=1163 MHz.

- Design steps will be similar to those of the previous example up to step-d of Figure 9.40. That is, the transmission zeros are extracted in the order UE-UE-UE-zero-UE-UE-UE (Figure 9.42.a) and Kuroda / Distribute Stub command is used to place OC stubs on each side of each TL (Figure 9.42.b). Then the inner stubs is split into two pieces by applying Split / Two Equal command thus forming the OC Stub+TL+OC Stub triplets (Figure 9.42.c). Then the OC stubs are equated by applying the command Kuroda / All Equal Stubs/ Include End Stubs (Figure 9.42.d). The circuit is saved as filt81.fil.

- After this step we deviate from the steps of the previous example temporarily to adjust the impedance level of the circuit by using tapping instead of inserting redundant unit elements.

- The triplets of OC Stub+TL+OC Stub are replaced by their coupled line equivalents by placing the cursor under the TL’s and applying the command Replace By PCL / Interdigital / ococINTD / Symmetric (Figure 9.42.e).  Thus, we have formed the edge coupled line filter to see the even-odd mode impedances.

- Then the command Inverters / Scale Cct By Inserting Inverters / Specify Inverter: K=33 is applied to scale down the even-odd mode impedances to realizable level (Figure 9.42.f).  Here, K is selected by inspecting the resulting impedances. At this step we have to keep in mind that these impedances will change after conversion to hairpin form and tapped form.

- After determining a proper inverter constant K, we load the circuit of filt81.fil. We will scale down its impedances by the same inverter constant by applying the same command. However for tapping applications we first Insert  quarter wavelength long TL’s at both source and load ends (Figure 9.42.g-h). Then scale down the circuit impedance level (Figure 9.40.h-i).

- With cursor under the inverters, the command Tapped PCL: Z=50, f0=1000, fq=1000 is applied. This command replaces the inverter and the adjacent TL by a tapped Half Wave TL, as shown in Figure 9.42.j. One portion of the tapped TL appears as shunt OC stub of length L1 and the rest appears as a TL of length  L2=L-L1 where L is the commensurate (quarter wave) length of the filter. In application of this command, we can specify the impedance of the Half Wave TL. This specification can effect response at later stages of the design. Therefore different values may be tried to get the best possible result. The frequencies f0 and fq that asked by Filpro must be typed in as the center frequency. This coarse approximation degrades the response appreciably. Response can be recovered to a great extent by applying both Tune Element and Best Case commands to lengths L1, L2 and impedances of all series OC stubs and inner TL’s. More than one solutions can be obtained. The one which possess all seven original return loss peaks have rather long L2 (more than half wave length). Another one which has L2 less than quarter wavelength is displayed in Figure 9.42.k, but this one has too small L1, leading to disappearance of two of the seven return loss peaks, as displayed.  Let’s continue description on this option.

- The OC stub impedances are changed after optimization. They can be re-equated by applying the command Kuroda / All Equal Stubs / Exclude End Stubs (Figure 9.42.k-m).

- Then adjacent TL pairs are formed by applying Kuroda / Right-Left / Full Stub commands as described in Figures 9.40.m-n. The resulting transformers are shifted rightmost by applying the command Shift TRF/ All Rightmost. Also the translated stubs are combined with the adjacent stubs (Figure 9.42.p).

- Thus, the adjacent TL pairs are formed. Impedances of these TL pairs can be equated by shifting a small piece of stub from left to the right of the pair, as described in Figure 9.42.p-q-r-s. The turns ratio of the first Kuroda transformation is set by trials until the impedances of the pair are equal. After clearing all transformers we get the circuit of Figure 9.42.s where TL pairs have equal impedances.

- Now we are ready for folding. First, the TL pairs are converted into their all-stub equivalents by placing the cursor under the left TL and applying the command Hairpin Oprs / Two Tline-to-Hairpin (Figure 9.42.t). The resulting all-stub equivalents of pairs of TL’s are also equivalent to hairpin folded  version but without coupling between the lines. This property is seen from the equality of magnitudes of OC and SC stubs of the series arm OC+SC stub elements.

- Coupling will be introduced between the folded arms of hairpin structure by applying the command Hairpin Oprs / Hairpin Bandwidth Scale/ Alpha=1.1 (Figure 9.42.u). This command is applicable only to all-stub circuits. It multiplies all OC stubs by the constant Alpha and divides all SC stubs by the same constant, thus making magnitudes of OC and SC stub impedances of the OC+SC stub elements different. Alpha=1.1 introduces about 20.4-20.5 dB coupling between the folded arms of the hairpin. This modification changes only the bandwidth of the filter, shrinking it by an amount depending on the constant Alpha. However in this circuit we have TL’s which are not transformed into all-stub forms. Filpro doesn’t change the impedance of these TL’s. This is an approximation. Therefore we should expect further degradation in response. Note that impedance of the piece of tapped half wave TL appearing as shunt OC stub is also changed from 50 ohms to 55 ohms. We can reset it to 50 ohms.

- Next. The LLeft and LRight sections of OC stubs are converted into symmetric Pi sections by applying the command Pi-Tee-L / LLeft (LRight)-to-Sym Pi (Figure 9.42.v).

- After clearing the transformers we get the circuit of Figure 9.42.w. With cursor under the middle elements of the symmetric Pi sections, the command Replace By PCL / Combline / ococCOMB / Symmetric is applied to convert the symmetric Pi sections into comb type coupled line sections (Figure 9.42.x). The even and odd mode impedances of the comb sections are the same as the even and odd mode impedances of the edge coupled sections of the hairpin filter, as described pictorially in the same figure.

- This approach adds a tapped TL at input and output ports which can be tuned during realization or EM simulation stages. Impedance of the tapped TL is set by the user. Therefore it can be selected to be the same width as the width of the first edge coupled line section to avoid discontinuity.

Figure 9.42. Synthesis of a medium bandwidth tapped hairpin filter.

Figure 9.40. Synthesis of a medium bandwidth tapped hairpin filter (cont’d).

Figure 9.42. Synthesis of a medium bandwidth tapped hairpin filter (cont’d).

Figure 9.42. Synthesis of a medium bandwidth tapped hairpin filter (cont’d).