CHAPTER 9

SIMPLE DISTRIBUTED  ELEMENT

HIGHPASS-BANDPASS  FILTERS

CONTENTS															Page

9. 1. INTRODUCTION													9.2

9. 2. WIDEBAND OPEN CIRCUITED EDGE OR BROADSIDE COUPLED LINE FILTERS

9.2.1. INTRODUCTION													9.8

9.2.2. MEDIUM-WIDE BANDWIDTH EDGE COUPLED LINE FILTERS			9.14

9.2.3. ULTRA-WIDEBAND EDGE COUPLED LINE FILTERS					9.25

9.3. NARROWBAND OPEN CIRCUITED EDGE OR BROADSIDE COUPLED LINE FILTERS

9.3.1. INTRODUCTION													9.31

9.3.2.  SYNTHESIS APPROACH FOR NARROWBAND FILTERS					9.32

9.3.3. PROTOTYPE WITH INVERTERS APPROACH  FOR NARROWBAND FILTERS 9.36

9.3.4. BUILT-IN LC PROTOTYPE APPROACH FOR NARROWBAND FILTERS		9.50

9.3.5. STUB OR CAPACITIVELY LOADED RESONATOR FILTERS				9.55

9. 4. STEPPED IMPEDANCE RESONATOR FILTERS

9.4.1. INTRODUCTION													9.64

9.4.2. COUPLED LINE FILTERS WITH 3-STEP RESONATORS					9.64

9.4.3. COUPLED LINE FILTERS WITH 5-STEP RESONATORS					9.74

9.4.4. COUPLED LINE FILTERS WITH STUB LOADED STEPPED RESONATORS	9.80

9. 5. DUAL BAND STEPPED RESONATOR FILTERS							9.77

9. 6. EDGE COUPLED LINE FILTERS WITH BRUTE FORCE FINITE TRANSMISSION

ZEROS																9.90

9. 7. HAIRPIN FILTERS

9.7.1. INTRODUCTION													9.93

9.7.2. NARROWBAND HAIRPIN FILTERS									9.96

9.7.3. HAIRPIN FILTER DESIGN USING GYSEL’S APPROACH					9.119

9. 8. HALF WAVELENGTH END COUPLED LINE FILTERS					9.139

9. 9. SHORT CIRCUITED EDGE COUPLED LINE FILTERS					9.155

9. 10. WIDEBAND SHORT CIRCUITED STUB AND UNIT ELEMENT FILTERS	9.159

9. 11.WIDEBAND HP FILTERS WITH MULTIPLE TZ’s AT f=0				9.167

9. 12. WIDEBAND BP FILTERS AS CASCADES OF LP AND HP FILTERS USING MIXED LUMPED AND DISTRIBUTED ELEMENTS							9.178

REFERENCES AND FURTHER READING									9.188

CHAPTER 9

SIMPLE DISTRIBUTED  ELEMENT

HIGHPASS – BANDPASS  FILTERS

9. 1. INTRODUCTION

Using equal (commensurate) length stubs and Unit Elements as transmission zero producing elements, it is possible to synthesize distributed element highpass filters and distributed element bandpass filters in S=+j domain (see Chapter-7). The highpass and bandpass characteristics obtained on S=j axis are mapped to the s=jw axis  through the mapping function (Richards transformation)

f=tan(f/2fq)

where fq is the quarter wavelength frequency corresponding to the commensurate length selected by the designer:

fq=c4Lεr

with L being the physical length, c being the velocity of light and r being the relative effective dielectric constant of the medium.

- The function f maps the entire range 0  into finite periodical ranges on the real frequency  f-axis, with a period of 2fq, as follows:

  - =0 is mapped to f= 0, 2fq, 4fq, 6fq, .. .

  - = is mapped to f=fq, 3fq, 5fq, .. .

This mapping results in production of infinitely many and periodic passbands and stopbands, We are usually interested in the response within the frequency range 0f2fq. Let’s inspect responses obtained on f-axis by mapping the S-domain highpass and bandpass filters separately.

Distributed Element Highpass Filters:

Distributed element highpass filters are synthesized in S=+j domain by using the following  transmission zeros:

    - Transmission zeros at =0 which are realized either as series OC stubs or shunt SC stubs.

    - Transmission zeros at S==1 which are realized as Unit Elements (transmission line pieces with same length as the stubs).

    - Some finite transmission zeros can be used to increase selectivity which are realized as stepped stubs.

Figure 9.1.a shows a typical distributed element highpass filter response on -axis and f-axis.

- The top figure is a typical highpass response with passband edge at p and having a finite transmission zero at z and some transmission zeros at =0. The response on f-axis is calculated from the highpass characteristics through the mapping function) f=tan(f/2fq). Close inspection of this mapping reveals the following properties within the frequency range 0f2fq:

  - The transmission zeros at at =0 are mapped to both f=0 and f=2fq.

  - The finite transmission zeros at =z are mapped to both  f=fz and f=2fq-fz.

  - The passband edge at =p is mapped to both f=fp and f=2fq-fp.

  - The frequency = is mapped to f=fq.

  - Thus, we notice that the highpass characteristics seen on the entire 0 range is squeezed into the range 0ffq. So, the response in the frequency range 0ffq is of highpass filter type. However the mirror image of this response is also mapped into the range fqf2fq. That is, within the frequency range 0 f 2fq, instead of a highpass response we get a symmetric bandpass response with passband edges being fp1=fp, fp2=2fq-fp and passband center being f0=fq. Since fq=(fp1+fp2)/2, the passband center f0=fq is usually termed as arithmetic center of the passband.

  - The stopband 0p of the domain highpass filter characteristics is mapped to both the range 0ffp and 2fq-fpf2fq. Thus, the frequency ranges 0ffp and        2fq-fpf2fq can be treated as lower and upper stopbands of the bandpass response within the range 0f2fq.

- If we consider the whole response on f-axis, it is seen that the upper stopband extends from 2fq-fp to 2fq+fp where the next passband starts which is centered at 3f0=3fq.

- Thus, a distributed element highpass filter need be treated as a quasi-bandpass filter with main passband centered at f0=fq and the others (named as spurious passbands) centered at 3fq, 5fq, . . Usually only the nearest spurious passband at f=3f0 is of concern.

- Such filters are among the simplest and most commonly used bandpass filters in applications where the existence of the spurious passband at 3f0 is not a problem.

- The basic limitations of distributed element highpass filters are now clear:

- Since in highpass filter approach the passband center f0 is the same as the quarter wave frequency fq, length of stubs and Unit Elements (the commensurate length) is fixed by the desired passband center.

- There is no chance of moving the nearest spurious passband away. It must be eliminated by some other means like cascading lowpass filters. If this is a problem, then we have to resort to using distributed element bandpass filter approach instead of distributed element highpass filter approach.

- Since fq sets the length of stubs and Unit Elements, physical size of the filter is fixed by the center frequency f0=fq. If miniaturization is needed then one should use distributed element bandpass filter approach instead of distributed element highpass filter approach.

- Response is symmetric about the passband center f0=fq. This is because the transmission zeros at =0 are mapped to f=0 and f=2fq, and any finite transmission zero at =z  is mapped symmetrically to fz and 2fq-fz.  This means we don’t have any flexibility to form an asymmetric response by adjusting the positions of finite transmission zeros in lower and upper stopbands or by placing different number of transmission zeros at f=0 and f=2fq. If asymmetric response is needed than one should use distributed element bandpass filter approach, as will be described below.

Distributed Element Bandpass Filters:

Distributed element bandpass filters are synthesized in S=+j domain by using the following  transmission zeros:

    - Transmission zeros at =0 which are realized either as series OC stubs or shunt SC stubs.

    - Transmission zeros at = which are realized either as series SC stubs or shunt OC stubs.

    - Transmission zeros at S==1 which are realized as Unit Elements (transmission line pieces with same length as the stubs).

    - Some finite transmission zeros in lower and upper stopbands to increase selectivity which are usually realized as stepped stubs (or some other circuit sections like cross-couplings).

The upper picture of Figure 9.1.b shows a typical distributed element bandpass filter response on -axis, with passband edges at p1 and p2a finite transmission zero in lower stopband, at z1, a finite transmission zero in upper stopband, at  z2, and some transmission zeros at =.

The lower plot shows insertion loss response on f-axis obtained through the mapping function

f=tan(f/2fq)

- The function =tan(f/2fq)  maps the range 0  to the range 0ffq. So, we have the desired bandpass response within the range 0ffq.

- We also have mirror image of the response of the range 0ffq about fq, in the range fqf2fq, leading to another passband. So, within one period we have two passbands, located in the ranges fp1<f<fp2 and 2fq-fp2<f<2fq-fp1, with the second one being the nearest spurious passband.

- In the distributed element highpass filters the passband center f0 was defined as the arithmetic man of the passband edges f0=(fp1+fp2)/2 while in bandpass filters obtained by mapping from lowpass prototypes the passband center f0 is defined as the geometric mean of the passband edges (f02=fp1fp2). Contrary to those cases, passband center concept is not defined in the filters synthesized by placement of transmission zeros. Therefore whenever the passband center concept need be referred, we can use either arithmetic mean or geometric mean of the passband edges fp1 and fp2.

- The finite transmission zeros in lower and upper stopbands are placed independently of each other. Also the numbers of transmission zeros at =0 and = are independent of each other. These features enable the designer to form asymmetric responses, hence form an advantage of distributed element bandpass filters over distributed element highpass filters.

- Since the quarter wave frequency fq is not related to fp1 and fp2, infinitely many bandpass filters can be synthesized having the same passband characteristics but having different fq (hence different commensurate length. This is a big advantage which enables the designer to have control on physical size of stubs and hence the overall size of the filter. For example selection of a high fq leads to size miniaturization.

- Selection of high fq leads also widening of the upper stopband which is located within the range fp2<f<fq+fp1, thus moving the first spurious passband way from the main passband. For the cases fq>2f0 where f0 is defined as arithmetic mean of the passband edges, the spurious passband moves away from the main passband while for fq<2f0 the spurious passband gets closer to the main passband.

- The responses for case of fq=2f0 coincide with that of the highpass filter, if the finite transmission zeros are symmetrically located and if the numbers of transmission zeros at =0 and = are equal. Therefore the highpass filters can be treated as special cases of bandpass filters.

In this chapter, we will be dealing with only distributed element highpass filters without finite transmission zeros. That is, the highass filters having Chebyshev or Butterworth passband response will be studied. Such filters form the broadest range of bandpass filter types. Other bandpass filter types will be studied in the succeeding chapters (Chapters-10-11-12-13).

Figure 9.1. Insertion loss of commensurate length distributed element highpass and bandpass filters on axis and  f-axis.

Transmission zero producing sections in Distributed Element HP filters:

S-domain highpass filters are synthesized by cascading three types of transmission zero producing elements:

- Series OC stubs (Ossr) and shunt SC stubs (SSsr) creating transmission zeros at S=+j=0 (Figure 9.2.a,b)

- Series connected OC and SC stub resonators in shunt arms (sRSsh) and parallel connected OC and SC stub resonators in series arms (pRSsr) producing finite transmission zeros on S=j axis (Figure 9.2.c,d)

- Contributing Unit Elements (UE: Transmission line piece having the same length as stubs) producing transmission zeros at  S==1 (Figure 9.2.e).

- The series arm open circuited stub (Ossr) of Figure 9.2.a is not directly realizable as it is. Such stubs are usually used together with UE’s. Kuroda identities can be used to transfer part of the Ossr to the other side of the UE to form a circuit section which is equivalent to an interdigital type open circuited parallel coupled line (ococINTD) and realized in this form as shown in Figure 9.3.a. The interdigital type coupled line has also an equivalent circuit in terms of transmission lines and inverter as shown on the right of Fig. 9.3.a.

- The shunt SC stub (SSsh) of Figure 9.2.b is usually realizable as it is. However in many applications it is preferable to use SSsh element together with UE’s, as shown in Figure 9.3.b. Part of the SSsh can be transferred to the other side of the UE to form a circuit section which is equivalent to an interdigital type short circuited parallel coupled line (scscINTD) and realized in this form.

- Thus unit elements are needed in the network models of both types of coupled lines together with stubs. Therefore unit elements are used not only because of their contribution to response and/or because of the need for physical separation between stubs. Their third function in highpass filters appears in forming the circuit sections equivalent to coupled lines.

- The finite transmission zero (FTZ) section formed by series connection of OC and SC stub pair in shunt arm of Figure 9.2.c is also difficult to realize as it is. It is usually replaced by its two-step open circuited stub equivalent termed as Stepped Resonator (ocSRsh) in Filpro) shown in the figure which is easy to realize.

Figure 9.2. Transmission zero producing elements of highpass filters.

- The finite transmission zero section formed by parallel connection of OC and SC stub pair in series arm of Figure 9.2.d is difficult to realize as it is. It can be replaced by its SC two-step stub as shown in Figure 9.3.d, but this is also difficult to realize. This problem can be solved by using the combination of this type of FTZ with shunt SC stubs (transmission zero at f=0) shown in the right side of Figure 9.3.d. Here the OC stub is replaced by its lumped capacitor approximation while the triplet of shunt and series SC stubs is converted into a comb-like parallel coupled line.

- In realization of some S-domain highpass filters parasitics may introduce transmission zeros at S=, converting the filter into a bandpass structure. In some such cases it may be preferable to introduce the transmission zeros at infinity intentionally in which case we will design the filter as distributed element bandpass filter. In such cases we may also use the elements shown in Figure 9.4 which realize pairs of transmission zeros at S=0 and S=.  The structures shown in Figure 9.4.a are both 6ptimisat to realize while the structures shown in Figure 9.4.b are all realizable. The structures shown in Figures 9.4.c and d employ parallel coupled lines for realization of the pairs of transmission zeros at zero and infinity.

- More combinations will be introduced for realization of groups of transmission zeros as we proceed in this and following chapters.

Figure 9.3. Some equivalent circuits used in realization of highpass filters.

Figure 9.4. Bandpass filter elements for realization of pairs of transmission zeros at S=0 and S=.

In Filpro distributed element highpass filters can be designed using the Synthesis, Built in LC Prototype and Prototype with Inverters approaches. User Defined Prototype approaches may also be used to make use of ready prototype tables found elsewhere and not available in Filpro.

There are many commonly used distributed element HPF types. In this chapter design methods will be described for the most commonly used simple filters, like OC and SC edge coupled line filters, stepped resonator filters, hairpin filters and shunt SC stub and unit element filters having Chebyshev or Butterworth passband response. More complex filters like those involving multiple coupled lines, filters with finite transmission zeros, miniaturized filters and filters involving cross-couplings will be filters studied in the succeeding chapters.

WIDEBAND

OPEN CIRCUITED EDGE AND BROADSIDE COUPLED LINE FILTERS

9. 2. WIDEBAND OPEN CIRCUITED EDGE AND BROADSIDE COUPLED LINE FILTERS

9. 2. 1. INTRODUCTION

Open circuited parallel coupled line filters (termed also as Edge Coupled Line filters or broadside coupled line filters) are among the most commonly used structures because of ease of construction in stripline-like forms. These filters belong to a class of distributed element highpass filters with a single transmission zero at f=0 and as many unit elements as needed to satisfy the selectivity requirements. Generic structures are shown in Figure 9.5. The single transmission zero at f=0 need be extracted as a series OC stub (Ossr). It is then distributed between the unit elements by using Kuroda identities to form circuit sections made up of triplets series OC stub+UE+series OC stub (Ossr+TL+Ossr) for conversion into open circuited interdigital type edge coupled line form as shown in Figure 9.6.a. Then these triplets are converted into symmetric coupled line equivalents through the transformation shown in Figure 9.6.b. The coupled line sections are described in terms of their even mode and odd mode impedances named as Zev and Zod respectively and its even and odd mode effective dielectric constants Eev and Eod, together with their physical lengths. Actually the even and odd mode terms are defined only for symmetric parallel coupled lines. However for simplicity in terminology, in Filpro, these terms are also used for asymmetric coupled line pairs. In general the terms Common Mode and Pi Mode are used in the description of asymmetric coupled lines. Pictorial view of a typical open circuited edge coupled line filter is shown in Figure 9.6.c.

Figure 9.5. Highpass filters with contributing unit elements having a single transmission zero at f=0 in the form of a single series OC stub.

Figure 9.6. a) Generic network model for conversion into open circuited edge coupled line form,         b) Transformation into edge coupled line form. C) Pictorial view of a stripline edge coupled line filter.

An alternative exact equivalent of these structures can be obtained in the form of comb-type coupled lines separated by double unit elements with halved commensurate length, as described in Figure 9.7. The triplets of  Ossr+TL+Ossr is exactly equivalent to a OC comb-type coupled line surrounded by two transmission lines (TL) with halved lengths. This structure is equivalent to a Distributed Element Bandpass filter with Nzero=1, Ninf=1 and NUE=2NUEHP and fq=2fqHP= 2f0. Here, f0 is the passband center which is also equal to the quarter wave frequency fqHP of the original Distributed Element Highpass Filter version. The resulting structure can also be designed as a Distributed Element Bandpass filter with an even higher fq leading to smaller commensurate length, hence longitudinal size reduction. This subject, hence those kinds of miniaturized Distributed Element Bandpass Filters will be studied separately, in Chapter-11.

Figure 9.7. Conversion of interdigital type edge coupled line filter into comb-type coupled line filter.

Filpro has several different approaches for designing interdigital type edge coupled line filters each having different features and advantages. They can be listed as follows:

- Design / By Synthesis.

- Design / By Built in LC Prototypes.

- Design / By Prototype with inverters.

Also wide, medium and narrow band filters can be treated by different approaches. Let’s see some characteristics of edge coupled line filters with different bandwidths on the following example and then pass to treatment of filters with medium, wide and narrowband filters separately.

EXAMPLE 9.1.  Synthesis of wide, medium and narrow band edge coupled line filters

Let’s consider a set of distributed element highpass filters  with PBRipple=0.01 dB, Nzero=1, NUE=6

and bandwidths set as follows:

- fp1=500 MHz, fq=1000 MHz leading to a 3:1 bandwidth with fp2=2fq-fp1=1500 MHz.

- fp1=700 MHz, fq=1000 MHz, leading to fp2=2fq-fp1=1300 MHz, close to an octave bandwidth.

- fp1=750 MHz, fq=1000 MHz, leading to fp2=1250 MHz, a wideband filter.

- fp1=800 MHz, fq=1000 MHz, leading to fp2=1200 MHz, a medium-wide band filter.

- fp1=900 MHz, fq=1000 MHz, leading to fp2=1100 MHz, a medium-wide band filter.

- fp1=975 MHz, fq=1000 MHz, leading to fp2=1025 MHz, a narrowband filter.

Let’s use Synthesis approach for the designs.

- Design command set is shown in Figure 9.8. Note that the options Distributed, Highpass, Doubly Terminated, Type A options are selected. Type A option leads to extraction of the single transmission zero at f=0 as a series OC stub.

- A single transmission zero is placed at f=0 (Nzero=1). Any number of Unit Elements can be placed to satisfy the desired selectivity. The number of Unit Elements will be equal to the number of coupled line sections. Let’s place six Unit Element to form a filter with six coupled line sections (NUE=6).

- The transmission zeros are extracted in a symmetric manner, as UE-UE-UE-Zero-UE-UE-UE to get a physically symmetric structure (Figure 9.8.a).

- With cursor under the Unit Elements  (TL’s) adjacent to the series OC stub, the command Kuroda / Distribute Stub is applied to move part of the series stub to the other side of the TL. This is repeated by moving the cursor under the TL’s adjacent to the translated OC stubs until an OC stub is place on each side of each TL (Figure 9.8.b-c).

- The series OC stubs in between the TL’s are split into two by applying the command Split / Two Equal (Figure 9.8.d). Thus, we have formed the triplets of series OC Stub+TL+series OC stub (Ossr+TL+Ossr) for conversion into coupled lines. However impedances of all TL’s and stubs are different, because we didn’t cared on these impedances in the previous Kuroda transformations. We can equate the series OC stub impedances by placing the cursor under any TL and applying the command Kuroda / All Equal Stubs. This command equates impedances of the stubs after an iterative application of Kuroda transformations (Figure 9.8.e).

- Next, with cursor under the TL’s of Ossr+TL+Ossr triplets, the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to convert these triplets into interdigital type coupled lines, as shown in Figure 9.8.f. A pictorial description is also shown in the same figure.

- The even and odd mode impedances are shown underneath each coupled line section. The parameters nev and nod are transformer turns ratios used in the circuit models of parallel coupled lines. For symmetric coupled lines they are nev=1 and nod=-1. Even and odd mode effective dielectric constants are denoted as Eev and Eod respectively.

- All the other filters are designed by using the same steps and the resulting filters are shown in Figures 9.8.g-to-m.

- The following observations are made on even and odd mode impedances:

- As the bandwidth of filter gets wider the difference between even and odd mode impedances (ratio of even-to-odd impedance) increase. This means the coupling between lines is tighter for wideband filters compared to narrower bandwidth filters.

- As the bandwidth gets narrower, the impedance level of both even and odd mode impedances get higher. Impedance level of the narrowband filters can be decreased by using techniques like tapping or by inserting quarter wave transformers or by inserting redundant unit elements and hence increasing the number of coupled line sections, as will be described later in this section.

- For wideband filters production problems may arise from the extremely tight couplings. Tight coupling problem of such filters may be solved to a certain extent by re-distributing the series OC stubs and using broadside coupled lines.

- It is seen that the medium bandwidth filters don’t have impedance level and even-odd mode impedance ratio problems. They seem to be realizable as they are.

- Another problem seems in the nearest spurious passband which is centered at 3fq=3f0=3000  MHz. As the bandwidth gets wide, the lower edge of this passband gets closer to the actual passband, as seen in the case of fp1=500 MHz. This needs placement of another lowpass filter to eliminate that passband.

- In these designs it is assumed that the even and odd mode effective dielectric constants are equal which may be difficult to achieve in practice. When they are not equal there will be spurious passbands at around 2fo, 4fo, .. as well which may cause harmonic problems in some applications. Such problems can be resolved by shifting the spurious passband away from 2fo by using stepped resonator lines as will be described later in this chapter.

Let’s now see more detailed examples on wide, medium and narrowband filters separately.

Figure 9.8. Design of interdigital type edge or broadside coupled line filters with different pass bandwidths.

Figure 9.8. Design of interdigital type edge or broadside coupled line filters with different pass bandwidths (cont’d).

9. 2. 2. MEDIUM-WIDE BANDWIDTH EDGE COUPLED LINE FILTERS

As exercise with various tools of Filpro, let’s design an octave band filter by using different approaches without giving much consideration to the realizability of element values.

A) SYNTHESIS APPROACH FOR MEDIUM-WIDE BANDWIDTH FILTERS

This is a repetition of the previous example, with some more details.

EXAMPLE 9. 2. An octave band OC edge coupled line filter

Let 	PBRipple= 0.01 dB,   fp=4000 MHz,   fq=6000 MHz. This filter will have a passband covering the frequency range

fp1=fp=4000 MHz  f   fp2=2fq –fp1=8000 MHz

with center frequency f0=fq=6000 MHz. The allowable transmission zero types are a single transmission zero at f=0 and any number of unit elements. The number of unit elements sets selectivity of the filter and also the number of coupled line sections. Let’s design a filter with Nzero=1 and NUE=6 to end up in a filter with 6 coupled line sections.

Steps of  Synthesis approach are as follows (Figure 9.9):

“New / Design / By Synthesis / Distributed, High Pass, Equiripple, Doubly terminated, Type A /           
  PB Ripple=0.01, fp=4000,  fq=6000, RS=RL=50”

Then transmission zeros are placed:	ZERO, UE , UE , UE, UE, UE, UE

As the transmission zeros are placed, shaping up of insertion loss response can be observed in the small box on the upper right corner of the screen  (Figure 9.9).

- Elements are extracted in the order UE-UE-UE-ZERO-UE-UE-UE giving the symmetric circuit of Figure 9.9.a.

- With cursor under the UE’s adjacent to the series stub, apply command Kuroda / Distribute Stub. This command will translate part of the stub to the other side of the UE.

- Apply this command to the UE’s that are adjacent to the translated stubs. Repeating this command, we place a stub on each side of all UE’s (Figure 9.9.a-d).

- Next, split all the internal stubs equally by applying the command Split / Two Equal (Figure 9.9.e). Thus, we have formed Ossr+TL+Ossr triplets each of which is equivalent to an open circuited parallel coupled line (edge coupled line). However the stub and UE impedances of each triplet are different and uncontrolled. This makes realization problematic. The stub impedances can be equated by placing the cursor under a UE and applying command Kuroda / All Equal Stubs/ Include End Stubs. This command applies an iterative procedure which equates the series OC stubs at (32.8 ohms) (Figure 9.9.e-f). This circuit is saved as edge01.fil for use at later stages.

- Finally, placing the cursor under each UE and applying the command Replace By PCL / Interdigital Lines / ococINTD / Symmetric we get the parallel coupled line equivalents of each triplet, as shown in Figure 9.9.g. Even and Odd mode impedances are shown underneath each coupled line section. On Filpro, the terminology Zc (common mode) and Zp  (pi-mode or difference mode) is used to indicate the even and odd mode impedances respectively. It is seen that the odd mode impedances of each section are identical and equal to the series OC stub impedances of 32.8 ohms. Equal odd mode impedance means that the coupled lines will have approximately equal separations, because separation is mostly set by the odd mode impedance.

- It should be noted that the even mode impedance is too high compared to the odd mode impedances. This means too tight coupling which may be problematic for realization. This is a problem of wideband edge coupled line filters. Therefore such edge coupled line structures are useful for medium bandwidth filters.

- Length of each element comes out to be 12.49 mm which is set by the quarter wave frequency fq=6000 MHz.

- Close in and wideband responses are shown in Figure 9.9.h. The wideband response shows the next spurious passband which may need elimination in some way (e.g. by cascading a lowpass filter).

Comb-like coupled line version- The link between distributed element HP and BP filters:

The interdigital type coupled lines can always be replaced by their comb-type coupled line equivalents. This can be done on Filpro by a simple circuit transformation, as follows.

- Load the circuit of edge01.fil (Figure 9.9.i).

- With cursor under each transmission line one by one, apply command Half Angle / Ossr+TL+Oosr. The triplets of  series OC Stub+TL+Series OC Stub will be replaced by a Pi-section of half length OC stubs surrounded by two transmission lines with halved lengths again, as shown in Figure 9.9.k

- With cursor under the series OC stubs of the Pi-sections of OC stubs, the command Replace By PCL / Combline / ococCOMB / Symmetric is applied. This command replaces the Pi-sections with equivalent open circuited comb-type coupled lines (Figure 9.9.m). Thus, interdigital type coupled lines are converted into comb type coupled lines.

- We should note one important theoretical point in this conversion: We had started the design as a Distributed Element HP filter with commensurate length set by fq=6000 MHz and with Nzero=1 and NUE=6. After conversion all the elements are halved in length which means the commensurate length is halved and hence fq is doubled. We also note that In the new circuit all the series OC stubs form a single TZ at f=0, hence Nzero=1 and all the shunt OC stubs form a single TZ at infinity (f=fq), hence Ninf=1. Existence of the TZ at infinity shows that the original HPF is now converted into a BPF. The number of UE’s are doubled, leading to NU=12.  So, now we have a BPF with Nzero=Ninf=1 and NUE=12, with fqBP=2fqHP=12000 MHz. That is, instead of distributed element HPF, we could as well start the design as a distributed element BPF and get the same circuit (Exercise?).  This link between BP and HP filters shows that Distributed Element HP filters are special cases of the Distributed Element Bandpass Filters with fq selected as 2fo where fo is the passband center. In general, in Distributed Element Bandpass Filter approach we can specify fq independently of the passband center and by selecting fq>2f0 we can miniaturize the filter as desired. This is a broad subject and it will be treated separately, in Chapter-11. The conversion described in Figures 9.9.i-to-n is just for pointing out the link between Distributed Element Highpass Filters and Distributed Element Bandpass Filters.

- A pictorial description of this structure is shown in Figure 9.9.n.

- Inspection of impedances shows that even-odd mode impedance ratios of the comb-type coupled line sections are much higher compared to the interdigital type couplings of the original edge coupled version.  Also the even mode impedances are more than double of the interdigital version while the odd mode impedances are smaller. For these reasons such comb-type couplings can be used only when realization with tighter couplings is possible and when the impedances are at realizable level.  That is, for this example the comb type version is unrealisable.

Figure 9.9. Synthesis of an open circuited edge coupled line filter.

Figure 9.9. Synthesis of an open circuited edge coupled line filter.

Equal Even Mode Impedance Solution:

- In this design the odd mode impedances of all coupled line sections came out to be equal. If other solutions are desirable for some reason, then Kuroda transformations can be used for this purpose. This is a tedious procedure but an exact one. Therefore let’s do an example. For example let’s try to equate all even mode impedances.

- Reload the Stub+TL version of the circuit (Figure 9.10.a).

- With cursor under the left one of the adjacent internal stubs, first apply Combine command to combine them and then apply Split / Specify Left-Right Ratio: n=0.9 (for example). Apply this command only to the first two pairs of adjacent stubs on the left half of the circuit. Then with cursor under the middle stub, apply command Symmetrize to map the left half to the right (to avoid 18ptimisat of Split-Combine commands in the right half of the circuit). Thus, The stubs on the two sides of each TL have different impedances (Figure 9.10.b).

- With cursor under each TL, apply command Kuroda / Equate Left-Right. This command will equate impedances of the stubs on each side of the TL’s while also producing a transformer (Figure 9.10.c).

- Shift all transformers to rightmost position by applying command Shift Transformer / All Rightmost. This command will yield a unity transformer at the load end which can be deleted (Figure 9.10.d).

Finally, with cursor under each TL apply command Replace By PCL / Interdigital / ococINTD / Symmetric to convert the circuit sections into coupled line form (Figure 9.10.e). Note that the even mode impedances got closer to each other. With some trials, one can find better split ratios to make all the even mode impedances equal or to set the levels of various impedances to different desired levels.

Figure 9.10. Kuroda transformations for obtaining equal even mode impedances.

B)  BUILT IN LC PROTOTYPE APPROACH FOR MEDIUM-WIDE BANDWIDTH FILTERS

The quickest way to design narrow-medium bandwidth edge coupled line filters in Filpro is the Built in LC Prototype approach. In wideband filters the price paid for quickness is slight degradations in return loss at band edges due to the approximations involved in the formulations ([4], [14], [29]). Further, this approach can be applied only to odd degree LP prototype filters. Even degree filters require specification of RLRS. If an even degree filter is needed with RL=RS, then one may initially assign RLRS and after the design is completed the condition RL=RS is enforced and the elements can be tuned to correct degradations in response. In general for even degree filters it may be easier to use the Synthesis or Prototype with Inverters approach.

EXAMPLE 9. 3. Edge coupled line filter design using Built in LC Prototype approach

Let’s consider the same filter as the previous example:

PBRipple= 0.01 dB, fp=4000 MHz, fq=6000 MHz

As a distributed HP filter, in the actual frequency domain these frequency specifications will lead to a bandpass filter with edges fp1=4000 MHz, fp2=8000 MHz.

- The command set for the design is shown in Figure 9.11:

New / Design / By Built-in LC Prototype / Distributed, High Pass, Chebyshev / PB Ripple=0.01

fp=4000,  fq=6000 / N =11 / Find circuit

- After these commands, the following options will be displayed:

All-Stub Form,      Shunt-SC stub+UE,       Series OC stub+UE,    With Redundant UE

The option Series OC stub+UE gives us the targeted standard OC stub+UE+OC stub triplets which are ready to be converted to edge coupled line form (Figure 9.11.a). The circuit is saved as edge01a.

- Conversion to parallel coupled line form is carried out by placing the cursor under UE and applying commands PCL / IntDigital Lines / ococINTD / Symmetric, as shown in Figure 9.11.b. The even and odd mode impedances are shown underneath each element.

- Element values of this circuit are determined through approximate formulations. Therefore some slight degradations exist in return loss together with a slight shrinkage in bandwidth. These degradations are more noticeable as bandwidth gets wider. They can be corrected by tuning or 20ptimisation.

- One may get equal odd mode impedance case (which means approximately equal separations between the coupled line pairs) as described in Figure 9.11.c-d-e. Here we load the original Stub+TL+Stub circuit edge01a and apply command Kuroda / All Equal Stubs / Include End Stubs. This command equates all the series OC stubs at 34.1 ohms. Conversion of each Stub+TL+Stub triplets into coupled line form gives the circuit shown in Figure 9.11.e. In this circuit all the coupled line sections have equal odd mode impedance which are also equal to 34.1 ohms, the impedance of the series OC stubs of the previous circuit.

- Response of this circuit is shown in Figure 9.11.g which shows unequal return loss ripple and band shrinkage. Rough tuning of return loss using Tune Element command gave the circuit of Figure 9.11.f, with the slightly better response shown in Figure 9.11.g.

- Further adjustment of even-odd mode impedances of various coupled line sections is possible by combining the adjacent OC stubs and re-splitting them by trial-error approach, as was described in the previous example.

Figure 9.11. Design of a wideband OC edge coupled line filter using Built in LC Prototype approach.

Figure 9.11. Design of a wideband OC edge coupled line filter using Built in LC Prototype approach (cont’d).

C) PROTOTYPE WITH INVERTERS APPROACH FOR MEDIUM-WIDE BANDWIDTH FILTERS

Design / By Prototype with Inverters approach has some advantageous compared to the Built-in LC Prototype approach because of the flexibilities provided by ideal inverters for adjusting element values. Also this approach can be used to design even degree filters. However for wideband filters some degradations arise in both passband return loss and stopband insertion loss which can be corrected by tuning and 23ptimisation.

EXAMPLE 9. 4. Edge coupled line filter design using Prototype with Inverters approach through series OC stub circuit model

Let’s consider the same filter as the previous examples:

PBRipple=0.01 dB, fp=4000 MHz, fq=6000 MHz, N=7 to work as a bandpass filter between fp1=4000 MHz and fp2=8000 MHz.

- The command set for designing the lowpass prototype is shown in Figure 9.12. In this example the option of Series Inductors and Inverters is selected because it will give the desired form after transformation into highpass filter. The lowpass prototype that will be used to design the actual filter is displayed in Figure 9.12.a.

- The command set continues by specifying type of the filter as Distributed, Highpass. Passband edge frequency and quarter wavelength frequency (which is also the passband center of the resulting response) are specified. Find Circuit command gives the required highpass filter with series OC stubs separated by ideal inverters as shown in Figure 9.12.b.

- The ideal inverters will later be replaced by circuit sections involving quarter wavelength lines. However before conversion into quarter wavelength line form, let’s take advantage of ideal inverters for adjustment of element values.

- In order to assign one series OC stub to each inverter on each side, the series OC stubs in between inverters need be split. We can split them in any ratio leading to a large number of equivalent alternative solutions. However not all these solutions will have realizable element values. As a trial let’s split them equally using Split / Two Equal command (Figure 9.12.b-c).

- In order to have equal series OC stubs on each side of inverters, the cursor is placed under each inverter and the command Inverters / Scale Inverter / All Equal Left-Right is applied. This command equates impedances of all stubs to the leftmost stub (Figure 9.12.b-c).

- The ideal inverters can be replaced by many practical equivalents available in Filpro. In order to get edge coupled filter structure, we need to use a practical model for inverters which involve quarter wavelength lines. The best model which keeps passband edges unaffected is obtained by placing the cursor under the inverters and applying the command Inverters / Replace Ideal Inv By / Ossr+TL+Ossr / Specify fq: fp=4000 MHz, fq=6000 MHz.  The inverters will be replaced by a UE with negative series OC stubs on each side (Figure 9.12.d).

- The negative stubs are absorbed into the adjacent positive stubs by applying Combine command (Figure 9.12.e-f). Thus, the triplets of Ossr+TL+Ossr are formed for conversion into coupled line form. We can replace them with coupled line equivalents at this stage. However if desired we can make further impedance adjustments. For example we can equate all the series OC stubs by applying the command Kuroda / All Equal Stubs / Include End Stubs (Figure 9.12.g).

- The procedure continues by replacing Stub+TL+Stub triplets with their parallel coupled line equivalents by placing the cursor under each transmission line and applying the command Replace By PCL / Interdigital Line / ococINTD / Symmetric (Figure 9.12.h). Note that all odd mode impedances are equal.

- Insertion and Return loss responses before and after conversion of ideal inverters are shown in Figure 9.12.k. Although this conversion kept the band edges unaffected, heavy distortions arised in both passband return loss and stopband insertion loss. This is the main problem of using Inverter approaches in wideband filters. Distortions may not be that much in narrower band filters (Exercise?). In fact such heavy distortions should be expected because the series OC stub impedances was 32.8 ohms in the design by the exact Synthesis approach of Example 9.2 while in this approach these impedances are reduced to 23.4 ohms. Too much difference. Therefore in wideband filters it is advisable to use the Synthesis  approach.

Figure 9.12. Development of edge coupled line filter using Design By Inverters approach.

Figure 9.12. Development of edge coupled filter using Design By Inverters approach (continued).

9. 2. 3. ULTRA-WIDEBAND EDGE COUPLED LINE FILTERS

Minnis [29 had shown that very broadband filters can be constructed in edge-coupled line form using stripline techniques. This is described in the following example.

EXAMPLE  9. 5. Ultra wideband edge coupled line filters

Specifications are PBRipple=0.01 dB, fp1=3000 MHz, fp2=9000 MHz, fq=6000 MHz, NUE=12, Nzero=1. Design / By Synthesis approach fits best for this example because the band is very wide.

- Design steps are shown in Figure 9.13:

New / Design / By Synthesis / Distributed, High Pass, Equiripple, Doubly Terminated, Type A /  
RS=RL=50  / PB Ripple=0.01, fp=3000, fq=6000 / Nzero=1, NUE=13

- Elements are extracted in symmetric manner (Figure 9.13.a):

UE – UE – UE – UE – UE – UE – ZERO – UE – UE – UE – UE – UE – UE

- The command Kuroda / Distribute Stub is used to place one OC stub on each side of each TL (Figure 9.13.a-c). The resulting circuit is saved as edge101.fil to be used at later stages.

Trial-1:

- Since the bandwidth is very wide we expect problematic element values. In order to see these problems, let’s design the filter in the usual edge coupled line form.

- This is done by first splitting each internal stub into two parts by applying the command Split / Two Equal to form Stub+TL+Stub triplets (Figure 9.13.c-d).

- The command Kuroda / All Equal Stubs / Include End Stubs is applied to equate stub impedances (Figure 9.13.d-e).

- Then the triplets are converted into PCL form using the command Replace By PCL / Interdigital / ococINTD / Symmetric (Figure 9.13.f). Inspection of the even and odd mode impedances shows that the odd mode impedances are too low for practical realization in the usual stripline form (about 14 ohms).

Trial-2:

- As another approach, (), load edge-101 (Figure 9.13.g) and apply command Kuroda / All Equal Stubs / Include End Stubs to equate all the stub impedances (Figure 9.12.h). Note that in this circuit we have only single stub between each TL. Since we need to form stub+TL+stub triplets for conversion into PCL form, it is clear that some TL’s will be left as they are.  Such TL’s will be used to separate the coupled lines.

- Split the stub in the middle of the circuit into two equal parts to work only in the left half of the circuit to make use of the symmetry (Figure 9.13.h-I). Save the circuit as edge102.fil.

- With cursor under the leftmost TL apply command Kuroda / Left / Full Stub to move the leftmost stub to the right of the leftmost TL (Figure 9.13.i-k).

- Shift the resulting transformer rightmost and Combine the adjacent stubs on the right of the leftmost TL (Figure 9.13.m).

- We form stub+TL+stub triplets as described in the circuit of Figure 9.13.m, and convert these triplets into PCL form by using the command Replace By PCL / Interdigital / ococINTD / Symmetric. After applying Symmetrize and Simplify Circuit commands we get the coupled line structure shown in Figure 9.13.n. It is seen that now we have less number of coupled lines. Some TL’s are left as they are. It is also seen that the even mode impedance level of the whole circuit is raised too much. This is caused by the transfer of the leftmost stub to the right of the leftmost TL.

Trial-3:

- Let’s now try a different approach. Load the circuit edge102.fil and transfer the halved stub in the middle to the left of the adjacent TL by using the command Kuroda / Right / Full Stub (Figure 9.13.p). After Symmetrize and Simplify Circuit commands we get the circuit of Figure 9.13.q. In this circuit we now form the stub+TL+stub triplets shown in the figure and convert them into coupled line form. Triplets. Even-odd mode impedances of this circuit seem more reasonable for realization.

Figure 9.13. Development of a wideband Minnis-Type edge coupled line filter.

Figure 9.13. Development of a wideband Minnis-Type edge coupled line filter (continued).

Figure 9.13. Development of a wideband Minnis-Type edge coupled line filter (continued).

NARROWBAND

OPEN CIRCUITED

EDGE and BROADSIDE

COUPLED LINE FILTERS

9. 3. NARROWBAND EDGE OR  BROADSIDE COUPLED LINE FILTERS

9. 3. 1. INTRODUCTION

The main problem associated with narrowband edge coupled line filters is that the synthesized filters have too high impedances. The high impedances are lowered to desired realizable levels by introducing impedance transforming circuit sections at both source and load ends. There are several approaches:

- One classical way to get a transformer is to insert redundant unit elements into both ports and then apply Kuroda transformations to redistribute series OC stubs both to create a proper transformer ratio for impedance scaling and to place OC stubs on each side of each UE to form the standard OC Stub+UE+OC Stub triplets for conversion into edge coupled line form with realizable element values.

- A more general approach which gives more control on the impedance level is to scale the circuit by inserting inverters and then replacing the inverters by either quarter wave transmission lines. Kuroda transformations will then enable us to adjust impedances. This approach is more general because it includes the redundant UE insertion approach as a special case.

- Both insertion of redundant UE’s and inserting inverters create two extra interdigital type parallel coupled line (PCL) sections at input and output. These PCL sections increase both size and loss. Further, they are usually much tightly coupled compared to the inner PCLs. Therefore in most cases it is preferred to use Tapping at input and output transmission lines.

Narrowband edge coupled line filters can be designed by using the same approaches described in the previous sections. However Filpro has also ready to use special formulations for quicker and more versatile designs. Let’s continue discussions through examples.

9. 3. 2.  SYNTHESIS APPROACH FOR NARROWBAND FILTERS

Although Synthesis approach is an exact design tool, it is not the best approach for the design of narrowband edge coupled line filters. As will be seen, the other approaches are more versatile for input-output coupling structures and for development of different layout architectures. This approach is treated first just to give more insight into the problems of narrowband filters for academic completeness.

EXAMPLE  9. 6. Synthesis of a narrowband edge coupled line filter

Let  PBRipple=0.01, fp=975 MHz, fq=1000 MHz, N=7

This is a narrowband filter with passband in the range 975 MHz  f  1025 MHz.

- The command set for Design / By Synthesis approach is shown in Figure 9.14, giving the circuit of Figure 9.14.a.

- The command Kuroda / Distribute Stub is applied to place a stub on each side of each TL (Figure 9.14.b-c).

- Then the internal stubs are splitted into two by using the command Split/Two Equal (Figure 9.14.d).

- The command Kuroda / All Equal Stubs is used to equate the stub impedances (Figure 9.14.e). The circuit is saved as edge201, to be used in later stages.

- If the usual approach is used for conversion of stub+TL+stub triplets into coupled lines we would get the element values shown in circuit of Figure 9.14.f which are too high for realization.

- The classical approach for scaling impedances to realizable levels is to introduce redundant unit elements at both ports of the filter and use Kuroda transformations, as described in Figures 9.14.g-m.

- In Figure 9.14.g, the original circuit edge201 is loaded.

- Using Insert command a transmission line with 50 ohms impedance and having the same length as the stubs (Unit Elements) are inserted at each end of the filter (Figure 9.14.h). Since these TL’s are 50 ohms, they don’t effect the response of the circuit, hence named as redundant UE’s.

- The stubs adjacent to the inserted TL’s are split into two and then with cursor under the inserted TL’s the command Kuroda / Distribute stub is applied to form stub+TL+stub triplets around the inserted TL’s also (Figure 9.14.i).

- The command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the stub impedances (Figure 9.14.k).

- Finally the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to convert the triplets into coupled line form as shown in Figure 9.14.m. It is seen that the even and odd mode impedances are realizable.

- The two redundant UE’s increased the number of coupled line sections by two. It should also be noted that the coupled line sections at the ports are much tightly coupled compared to the inner coupled line sections (Zeven>>Zodd). In this approach we don’t have much control on the impedance level of the whole circuit. A more general approach is described below.

- In Figure 9.14.n the original circuit edge201 is loaded.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=60 ohms is applied to place inverters at both ports to scale up the impedance level of the circuit (Figure 9.14.p). Here K=60 ohms is a trial value which need be iterated after the design is completed. The case with K=50 ohms coincides with the insertion of redundant UE’s, just described above.

- The inverters are then replaced by their quarter Wave Line equivalents by applying the command Inverters / Replace Ideal Inverter By / QuarterWaveLine: f0=1000 MHz (Figure 9.14.q).

- The stubs adjacent to these quarter wave lines are split into two and the command Kuroda / Distribute Stub is applied to these quarter wave lines so as to place stubs on each side of these lines also (Figure 9.14.r).

- Then the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the stub impedances (Figure 9.14.s).

- Finally the stub+TL+stub triplets are converted into coupled line form using the command Replace By PCL / Interdigital / ococINTD / Symmetric (Figure 9.14.t). It is seen that in this circuit the even-odd mode impedance level is higher compared to the previous circuit obtained with 50 ohms redundant line. Other impedance levels can be obtained by selecting different inverters constant K.

- Replacement of ideal inverters by quarter wave lines introduces a slight degradation in response which can be corrected by tuning elements of the filter. In Figure 9.14.t, return loss before and after tuning are shown.

Figure 9.14. Design of narrowband edge coupled line filter using Synthesis approach.

Figure 9.14. Design of narrowband edge coupled line filter using Synthesis approach (cont’d).

Figure 9.14. Design of narrowband edge coupled line filter using Synthesis approach (cont’d).

9. 3.  3. PROTOTYPE WITH INVERTERS APPROACH  FOR NARROWBAND FILTERS

Prototype With Inverter approach is a more versatile approach compared to Synthesis approach because many alternative structures can be formed in an easier way. Further, this approach clarifies the link between direct coupled lumped LC resonator filters and edge coupled line filters. That is the edge coupled line filters are treated as direct coupled resonator filter enabling one to use the concepts of  Coupling Coeficient, External Q and Resonance Frequency parameters of the circuit which may be useful during realization and testing stages. Let’s see the details on an example.

EXAMPLE 9. 7. Prototype with inverters approach for edge coupled line filters

The filter has the same specifications as that of previous example, but with reduced degree, N=5. Now it will be designed as a lumped element bandpass filter which requires specification of both passband edges:

PBRipple=0.01, fp1=975 MHz, fp2= 1025 MHz, N=5

- The command set is shown in Figure 9.15. Note that we have selected the prototype with Shunt Caps and Inverters shown the figure. The resulting bandpass filter is shown in Figure 9.15.a.

- Pressing the key combination CTRL+F6, selecting the option Coupled Res-CT-CQ-CN and typing in the center and passband edge frequencies we can display the Coupling Coefficients and External Q of the circuit as a list (Fig.9.15.b).

- One may also display the individual Coupling Coefficients together with the Resonance Frequencies of the resonators coupled by the ideal inverters by placing the cursor under the inverters and pressing the key combination ALT+F6 as shown in Figure 9.15.c.

- As the first step toward formation of edge coupled line structure we equate all shunt resonator element values by applying the command Inverters / Equate All Elements /Inductors (Figure 9.15.d). The resulting circuit is saved as Filt101.fil. We can display the same Coupling Coefficients, External Q and Resonance Frequencies from this circuit as well. That is, equating the resonators doesn’t effect these parameters. Narrow span and wide span responses are also shown in the same figure.

- In this circuit we replace the shunt LC resonators with their half wavelength transmission line equivalents by placing the cursor under the LC resonators and applying command Replace By TL  / Half WaveLength TL: Z=4.86259 (Figure 9.15.e). Here, the length of TL is set as Half Wavelength at the resonance frequency of the resonator. The default impedance value displayed by Filpro is proper for narrow-medium band filters. This transformation is approximate in nature and may lead to gross distortions in response for wideband filters. The conversion of LC resonators into half wavelength TL’s introduces spurious passbands at all harmonics nfo (n=1, 2, 3,..) of the fundamental passband as shown in Figure 9.15.e.

- Next we scale impedance level of the circuit by applying the command Inverters / Scale Circuit By Inserting Inverters / Specify TL: Zo=50 ohms. The transmission lines will have 50 ohms impedances (Fig. 9.15.f). This is a trial value which may need be revised to have better impedances for the resulting edge coupled lines. The circuit is saved as Filt102.

- In order to form the parallel coupled line sections we first form the TL+Inverter+TL triplets with TLs being quarter wave long at band center. For this purpose we split all the half wave TL’s into two by applying the command Split / Two Equal (Figure 9.15.g).

- Next, the TL+Inverter+TL triplets are converted into OC parallel coupled line sections using the identity shown in Figure 9.15.h, by placing the cursor under the inner inverters and applying the command Replace By PCL / Interdigital / ococINTD: f0=999.687  MHz (Figure 9.15.i) where the frequency fo is the passband center calculated as the resonance frequency of the LC resonators.

- The even and odd mode impedances of coupled lines seem to be realizable. Therefore we don’t need to revise the TL impedances. The circuit is saved as Filt103. We note that the spurious passbands at even harmonics of fo disappeared after conversion into  coupled line form. Only the odd harmonics nfo, n=1,3,5,.. remain.

- The inverter+TL pairs at source and load ends will need special treatment as they can be converted into different forms depending on the type of input/output couplings, as will be discussed next.

Redundant Line approach for input/output Couplings:

- In this approach we create redundant coupled lines at input and output by inserting quarter wavelength long 50 ohms TL’s at input and output (Fig. 9.15.k-m).

- Then with cursor under the inverters the triplets of TL+Inverter+TL are converted into coupled lines as shown in Fig. 9.15.n. The circuit is saved as Filt104. There is no degradation in return loss response and only odd harmonic spurious passbands exist.

- Fig. 9.15.p shows a pictorial description. This old classical approach has several problematic points. One is the too tight coupled lines at input and output, showing itself as the high difference between even and odd mode impedances. Tight couplings necessitate broad side couplings which is possible by using both sides of a suspended stripline. The second disadvantage of this structure is the unnecessary loss caused by insertion of the redundant TL’s. The third disadvantage is the increase in the total length of the filter caused by the two extra coupled lines. These disadvantages make the tapping approach discussed below more attractive.

Tapped Input and Output Couplings:

- The circuit of Filt103 is loaded (Fig. 9.15.q).

- Then with cursor under the inverters at source and load ends the command Tapped PCL / Zo=50 ohms / f0=1000 MHz, fq=1000 MHz is applied.  The inverter+TL pairs will be replaced by a shunt OC stub+TL pairs. The input (output) connection will be made to the first TL on source (load) side at a point whose distance to the OC end is denoted by LTap (Fig. 9.15.r). Length of the TL is also shortened (denoted as LTL in the figure).

- This approximation degrades the return loss response somewhat. However degradation can be overcompensated by a slight optimisation applied on impedances and lengths of the stubs, TL’s and parallel coupled lines using Monte Carlo-Best Case optimisation routine.  Figure 9.15.s shows the optimised circuit together with the degraded and improved return loss responses. We note that return loss response is improved to greater than the original 26 dB!. Wide span response remains the same.

- Figure 9. 15.t shows the pictorial description showing the shunt OC stub and reduced length TL of the tapped lines.

- In the above design the even and odd mode effective dielectric constants of the coupled lines are assumed to be equal which may not be the case in practice. The effects of different even-odd mode dielectric constants are investigated in Figure 9.15.u by setting all Eod=1.1. The most important effect of this change is the re-appearance of spurious passbands at even harmonics f=2fo, 4fo, 6fo, .. . Especially the passband at 2fo may be troublesome in some applications. One solution for this problem is to use it stepped TL resonators with noncommensurate step lengths, as will be studied next.

Graphical Display of Coupling Coefficients:

Just as a demonstration of the use of Filpro for displaying Coupling Coefficients, let’s find the coupling coefficient k12 between the first two Half Wavelength TL resonators.

- The circuit of Filt102 is loaded. In this circuit the resonators are in the form of Half Wavelength TL’s, numbered as 1,2,..,5 (Figure 9.15.w).

- In order to display the coupling coefficient between the first two resonators, we delete all the elements  except the first two TL’s and the inverter coupling them (Figure 9.15.x).

- Then small series capacitors are inserted to couple the resonators to source and load (Figure 9.15.y).

- Insertion loss plot shows two peaks, at frequencies f1=975 MHz and f2=1025 MHz. The coupling coefficient k12 is calculated using f1, f2 and fo=1000 MHz through the formula

k12= (f2-f1)/fo = 0.5

which is close to what we have displayed from the initial filter circuit with LC resonators.

The same approach can be used to display all the other coupling coefficients, from any circuit designed above (Exercise!).

Figure 9.15. Prototype with inverters approach for the design of an edge coupled line filter.

Figure 9.15. Prototype with inverters approach for the design of an edge coupled line filter (cont’d).

Figure 9.15. Prototype with inverters approach for the design of an edge coupled line filter (cont’d).

Figure 9.15. Prototype with inverters approach for the design of an edge coupled line filter (cont’d).

Figure 9.15. Prototype with inverters approach for the design of an edge coupled line filter (cont’d).

Figure 9.15. Prototype with inverters approach for the design of an edge coupled line filter (cont’d).

EXAMPLE 9. 8.  Coupled line filters with reduced length coupled sections

In the previous approaches the coupled line sections were all quarter wavelength at band center. This leads to thin and long structures making folding difficult for length reduction. It is possible to make the coupled line sections shorter than quarter wavelength and leave some uncoupled line pieces in between the coupled line sections. The uncoupled line sections can then be used to fold the filter into hairpin-like forms to make its total length shorter with increased  transverse dimension.

- For this purpose we load Filt102 and split the inner half wavelength TL’s into three pieces. In Figure 9.16.a-b these TL’s are split into three equal length pieces. We may as well split them into unequal length pieces with the condition that the TL pieces on the two sides of the inner inverters be equal in length (Exercise?).

- Next, with cursor under the inner inverters the command Replace By PCL / Interdigital / ococINTD: f0=1000  MHz is applied. {The value displayed for f0 should be disregarded!.}  So we get coupled lines with shorter lengths compared to the previous cases (Figure 9.16.c).

- Inspection of return loss shows negligible degradation. However the wide span insertion loss plot shows some major changes. The spurious passband at 3f0 disappeared and a strong spurious passband appeared at 2f0.

- The source and load end inverters can be converted into either redundant coupled line or into Tapped Line forms. In Figure 9.16.d the version with tapped input-output is shown.

Figure 9.16.e shows pictorial descriptions of different layouts. The uncoupled line pieces provide flexibility for folding the structure into hairpin-like form.

Figure 9.16. Filter with reduced length coupled line sections.

EXAMPLE 9. 9.  Coupled line filters with hairpin folded sections

One of many ways to fold the filter to reduce its length and increase its transversal dimension is to fold the (uncoupled) line pieces which interconnect the coupled line sections. Folding the interconnecting lines may lead to coupling between the folded sections. It is possible to control such couplings as will be described in this example.

Let’s consider the same example as the previous one:

- The filter Filt102 is loaded (Figure 9.17.a).

- In order to expose one of the inner resonators, we Delete all the elements except one inner TL piece and the inverters on its two sides (Figure 9.17.b). We plot return loss (and/or insertion loss) of this resonator to use as reference.

- The TL resonator is split into three parts, with the two outer pieces having length of 20 mm (for example) and the inner piece having the remainder length (about 110 mm). This operation is done by using Split / Specify Left and Exchange commands (Figure 9.17.b-c).

- Then the middle piece is further split into two equal pieces (Figure 9.17.d). Now these two equal pieces will be folded to reduce the length of the filter. With cursor under the left of the TL pair, the command Hairpin Oprs / Two Tline-to-Hairpin is applied. This command replaces the TL pair by its all-stub equivalent as shown in Figure 9.17.e. The equivalent circuit consists of a series arm parallel OC+SC stub element with negative OC stub impedance surrounded by two shunt OC stubs. The series arm SC stub and OC stub have equal magnitude impedances of 100 ohms and –100 ohms respectively. Equal magnitudes indicate that the hairpin line arms are uncoupled. That is we have managed to fold the two adjacent lines without coupling between them which is physically not possible. We will introduce coupling between the arms by brute force as will be described below.

- With cursor under the series OC+SC stub element the command Replace by PCL / Combline / Hairpin is applied to replace the all-stub section with a hairpin coupled line of equal even and odd mode impedances (Figure 9.17.e). Equal even and odd mode impedances means there is no coupling between the arms of hairpin section.

- We can introduce a controlled coupling between the arms of hairpin section by increasing the even mode impedance and decreasing the odd mode impedance. Let’s make Zev=55 ohms and Zod= 45 ohms (Figure 9.17.f). Up to that stage all the transformations were exact. Therefore no change arise in response of the circuit section. However as we change the even and odd mode impedances the response of the resonator section will change appreciably. We can recover the return loss response to its original form by simply tuning the length of the hairpin section as shown in Figure 9.17.g. There is little difference between the original and tuned responses. Now we will use this resonator in place of the original half wavelength resonator.

- We Copy the hairpin section to use it at later stages.

- The original circuit Filt102 is loaded (Figure 9.17.h).

- The inner half wavelength TL pieces are split into two pieces (Figure 9.17.i) and their lengths are set as 20 mm. Then we Paste the copied hairpin sections in between the 20 mm TL pieces (Figure 9.19.k).

- Next the TL+Inverter+TL triplets are converted into coupled lines using the command Replace by PCL / Interdigital Line / ococINTD: fo=1000 MHz (Figure 9.17.m). So, we got the modified (hairpin folded) filter. We save it as Filt102F.

- Figure 9.17.n shows the return loss of the filter. We note a slight shrinkage in bandwidth. It can be recovered by either tuning e few elements or, if shrinkage is appreciable, redesign the filter with slightly wider bandwidth.

- Figure 9.17.p shows the layout of the filter after the end inverters are replaced by tapped lines (Exercise?). This layout is much shrter than the original edge coupled line structure.

- It is possible to introduce some physical distance between the edge coupled line sections as described in Figures 9.17.q-to-t.

- The filter Filt102F is loaded (Figure 9.17.q). With cursor under the edge coupled line sections the command Half Angle is applied to convert them into OC stub Pi-sections surrounded by two TL pieces, all halved in length (Figure 9.17.r).

- Then with cursor under the series OC stubs of the Pi-sections the command Replace by PCL / Combline / ococCOMB is applied to convert the Pi-sections into comb type coupled lines (Figure 9.17.s).

- This is the targeted circuit whose layout is shown in Figure 9.17.t where there are TL pieces separating the comb type coupled lines.

Figure 9.17. Coupled line filter structures with Hairpin folded sections.

Figure 9.17. Coupled line filter structures with Hairpin folded sections.

Figure 9.17. Coupled line filter structures with Hairpin folded sections.

9. 3. 4. BUILT-IN LC PROTOTYPE APPROACH FOR NARROWBAND FILTERS

Built in LC Prototype approach is quicker compared to the Prototype with Inverters approach of the previous subsection. In this approach the filter is designed as a distributed element HPF with quarter wavelength frequency selected as the passband center. One disadvantage of this approach is the Coupling Coefficients and External Q can’t be displayed directly. In order to display them the half wavelength TL pieces must be converted into shunt LC resonators. Therefore in cases where External Q and Coupling Coefficients are needed the Prototype with Inverters approach is better.

Let’s see an example using Built in LC Prototype approach.

EXAMPLE 9. 10. Built in LC Prototype approach for edge coupled line filters

Design steps are shown in Figure 9.18 for the same filter specs.

PBRipple=0.01, fp=975 MHz, fq= 1000 MHz, N=5 to get a BPF with fp1=975 MHz and fp2=1025 MHz.

- Design / By Built in LC Prototype approach is selected with Distributed Element, HPF, Chebyshev options.

- Terminations: RS=50, RL=50 ohms are typed in.

- Attenuation Specs / PB Specs+Prototype Degree option is selected and the filter specifications PBRipple=0.01 dB, fp=975 MHz, N=5 and fq=1000 MHz are typed in.

- Find Circuit command displays four options:

- All Stub option gives the circuit as alternating cascades of series OC stubs and shunt SC stubs.

- Shunt SC Stub+UE option gives the circuit in the form of alternating cascades of shunt SC stubs and Unit Elements.

- Series OC Stubs +UE option gives the circuit in the form of alternating cascades of series OC stubs and Unit Elements. This option gives the same circuit as Synthesis approach {Exercise with this approach by adding redundant UE’s at source and load ends}.

- With Redundant UE option gives two solutions both of which incorporates redundant UE’s for impedance scaling:

- One solution is formed as cascades of TL+Inverter+TL triplets. This solution is the proper one if tapped input-output is desired or if redundant coupled lines will be used at input and output ports.

- The second solution is formed as cascades of Series OC Stub+TL+Series OC Stub triplets. This is the same as we have got using the Synthesis approach, now incorporating the redundant UE’s also. This approach is a quicker version of the synthesis approach (Exercise!).

- Let’s select the option With Redundant UE / TL+Inverter+TL.

- Then Filpro asks the resonator impedance. Typing in 50 ohms for Resonator Impedance we get the circuit of Figure 9.18.a. This circuit is already scaled in impedance by the TL+Inv+TL triplets at the source and load ends of the filter which contains redundant TL’s. The TL’s are of quarter wavelength long at band center. Therefore the adjacent TL pairs actually form uniform Half Wavelength TL’s. For this reason such filters are also named as Half Wavelength Coupled Resonator Filters.

- If redundant coupled lines will be used at input-output ports then we can convert all the TL+Inverter+TL triplets into coupled lines (Exercise!). If tapping approach will be used then we simply delete the redundant 50 ohms lines at the source and load ends.

- Let’s use tapping approach. For that purpose we Delete the redundant TL’s and convert the inner TL+Inv+TL triplets into coupled line form by using the command Replace By PCL / Interdigital / ococINTD: f0=1000 MHz (Figure 9.18.b).

- We are now ready to convert the ideal inverters at load ends adjacent to half wave TL’s into tapped form. This is done by placing the cursor under these inverters and applying the command Tapped PCL / Z0=50, f0=1000 MHz, fq=1000 MHz. The end inverters will be replaced by a shunt OC stub (OSsh) while length of the TL’s adjacent to the inverters will be decreased (Figure 9.18.c). This is a coarse approximation for the end inverters leading to some degradation in response.

- Response is corrected by tuning the even-odd mode impedances and lengths of coupled line sections and the lengths of shunt OC stubs and the TL pieces using Monte Carlo-Best Case routine. One result after a few tuning trials is shown in Figure 9.18.d with responses before and after optimisation shown in Fig.9.18.e. The circuit is saved as Filt105.

- Figure 9.18.f shows a pictorial view where the tapped resonators are shown in 90 degree folded form.

- In the above designs we have assumed that the even and odd mode dielectric constants of coupled line sections are equal. However this is usually not the case in practice. In Figs. 9.18.g-i, the effects of different even-odd mode dielectric constants are demonstrated by setting all Eod=1.1 instead of Eod=1. For Eev=Eod=1 the wide span insertion loss responses show the spurious passbands at about 3fo, 5fo, 7fo, .. . For Eev=1 and Eod=1.1 we see the appearance of additional spurious passbands now at about 2fo, 4fo, .. . Especially the passband at 2fo is problematic in many applications involving nonlinear devices like amplifiers. The remedy for this undesired passband is to shift it away from 2fo. This is possible by using Stepped Impedance Resonators (SIR) instead of uniform half wave resonators, as described in the previous examples. As Exercise repeat the design by using the 3-step and 5-step resonators.

Exercise: Repeat the examples of the previous section now by using Built in LC Prototype approach.

Figure 9.18. Tapped edge coupled line filter design using Built in LC Prototype approach.

Figure 9.18. Tapped edge coupled line filter design using Built in LC Prototype approach (cont’d).

Figure 9.18. Tapped edge coupled line filter design using Built in LC Prototype approach (cont’d).

9. 3. 5. STUB OR CAPACITIVELY LOADED RESONATOR FILTERS

Edge coupled line filters may get too long especially at low frequencies. The half wavelength straight resonators of the edge coupled line filters may be folded in various ways to reduce size in longitudinal direction while the transversal size is increased, as shown in Figure 9.19. Figure 9.19.b shows an L-type folding. This structure may also be named as Stub Loaded Edge Coupled Line Filter, because the folded pieces of lines load the coupled lines as OC stubs. Since the resonator lines are half wavelength long at passband center, the OC stubs are less than quarter wavelength. Therefore they may also be replaced by lumped capacitors in which case they are named as Capacitively Loaded Edge Coupled Line Filters (Figure 9.19.c). Figure 9.19.d shows U-type folding of the adjacent resonators. In these structures coupling between adjacent resonators (inter-resonator coupling) is provided through the interdigital type coupling between the folded arms of adjacent resonators. That is, directions of the adjacent U-sections are opposite. In these structures it is assumed that there is no coupling between the folded arms of a resonator. Therefore they are named them as Interdigital Coupled U-folded Resonators. Figure 9.19.e shows the capacitively loaded version of Interdigital Coupled U-folded Resonators.  Capacitive loading reduces length of resonator lines. If coupling is allowed between the folded arms of the same resonator, then we get the well-known Hairpin Filters. These versions are shown in Figures 9.19.f and g. If all U-sections look in the same direction, then couplings between adjacent resonators will be of OC comb type. Examples are shown in Figures 9.19.h-to-m. These filters are named as Comb Coupled U-folded Filters and Comb Coupled Hairpin Filters. All of these resonators can be used for the design of direct coupled or cross-coupled resonator filters. Some of these structures can be designed by Filpro while for some of them the initial design stages can be started by Filpro and then continued through more general circuit simulator and Electromagnetic simulation softwares for element value adjustments, topology variations and optimization. In this chapter typical structures will be detailed through design examples. Those structures involving interdigital type couplings can be designed by Filpro as Distributed Element Highpass Filter approach in an easier way while handling of filters with comb type couplings is somewhat easier through Distributed Element Bandpass Filter approach.

In the following example an edge coupled line filter will be designed using L-shaped resonators. Further examples will be studied in the succeeding sections where stepped resonators and stub loaded stepped resonators will also be involved.

Figure 9.19. Some folded and capacitively loaded resonators.

EXAMPLE 9. 11. Edge coupled line filters with L-shaped resonators

On Filpro a brute force approach can be used to replace the edge coupled line sections by their folded versions as described in Figure 9.20.a. This procedure is done on Filpro by brute force where we delete the edge coupled line sections and insert their folded version named as OSOSINTD, as described in Figure 9.20.b.

Figure 9.20. Brute force replacement of an edge coupled line section by its L-shaped (stub loaded) equivalent.

In this replacement the unterminated (OC) ends of the coupled line sections will be folded such that the total length of resonator lines will remain the same, thus decreasing the coupling region length. Since coupling length is decreased the net coupling coefficient between two resonators will decrease distorting response. In order to compensate this decrease the separation between the coupled lines will be decreased. This is done by tuning the even and odd mode impedances of the coupled line sections. The OC stubs may also be replaced by lumped capacitors, leading to Capacitively Loaded Edge Coupled Line Filters.

Let’s see the details on the filter with PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz, N=7

- Let’s use the shortest way, that is, Built in LC Prototype approach, by selecting the options With Redundant UE’s and with 50 ohms Resonator Impedance, as shown in Figure 9.21. The resulting circuit is shown in Figure 9.21.a.

- Next, using the command Replace By PCL / Interdigital / ococINTD: f0=1000 MHz, the TL+Inverter+TL triplets are converted into edge coupled line forms (Figure 9.21.b). Each edge coupled section has a length of 74.948 mm and we have 8 such sections, leading to a total length of  599.584 mm which is too long. Let’s try to reduce this length by folding the coupled lines.

- With cursor under the first coupled line, we apply command Insert / Parallel Coupled Line / Interdigital / OSOSINTD. During insertion we assign the same even and odd mode impedances (Zc1 and Zp1 respectively) as the edge coupled line section to be replaced, as initial trial values to start tuning. We set the length of new edge coupled section (Lng) as, for example 50 mm instead of 74.948 mm and thus the stub lengths (LngLeft and LngRight) will be 24.948 mm. We may also set the stub impedances. In this example we have left them at 50 ohms. Then we Delete the old edge coupled line section. Using Symmetrize command we can map the conversion done at the source end to the load end, getting the circuit of Figure 9.21.c.

- Response will be degraded appreciably as shown in the left side of Figure 9.21.e. In order to recover the response we apply Tune Element command first on the odd mode impedance Zp1 (which affects coupling most) and then on Zc1 iteratively. In a few iteration we can recover response roughly as shown in Figure 9.21.e. We may also tune the stub lengths, or leave them to a later stage.

- Then we repeat the same procedure on the second coupled line section, getting the circuit of Figure 9.21.f. In this case we tune the parameters of both the first and the second coupled lines, as they set the response together.  Response is recovered to a great extent.

- Repeating the procedure on the third and fourth coupled lines, we end up with the circuit of Figure 9.21.h which has a return loss response even slightly better than the original one. The total length of this structure is now about 400 mm. Further reduction is also possible by selecting smaller coupling lengths, but that would lead to even tighter couplings. The open circuited stubs may also be replaced by lumped capacitors, if feasible.

Figure 9.21. Design of edge coupled line filter with L-shaped  resonators.

Figure 9.21. Design of edge coupled line filter with L-shaped  resonators.

EXAMPLE 9. 12. A tapped edge coupled line filter with L-shaped resonators

The approach described above can also be applied on the tapped versions of the edge coupled line filters. Let’s consider the tapped narrowband filter with parameters,

PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz, N=7

- Design steps are repeated in Figure 9.22. Design / By Built in LC Prototype approach is used with the option With Redundant UE’s and with 50 ohms resonator impedance, giving the circuit of Figure 9.22.a.

- The Quarter wave Tlines at source and load ends are deleted so that the end elements of the filter will be ideal inverters. The two quarterwave Tlines on the other sides of the inverters are combined to form half wave Tlines. The inner four Tline+Inverter+Tline triplets are converted into coupled line form by applying the command Replace By PCL / Interdigital / ococINTD: f0=1000 MHz (Figure 9.22.b).

- With cursor under inverters the command Tapped PCL: Z0=50, f0=1000, fq=1000 MHz is applied to convert the inverter and the adjacent half wave Tline into tapped forms (Figure 9.22.c). In this procedure the half wave Tline is folded such that one piece becomes shunt OC stub and the remaining piece is still a Tline but its length is less than half wavelength. With cursor under this Tline we split it into two pieces by applying the command Split / Specify Left: Lng=74.948 mm which is the quarter wave length. Then Exchange command is used to move this quarter wave line piece to be adjacent to the inverter to form Tline+Inverter+Tline triplets (Figure 9.22.d) which is then converted into edge coupled line form (Figure 9.22.e).

- Thus, the tapped edge coupled line filter is formed. This conversion degrades response appreciably. Recovery of response needs tuning of all elements and we will leave this procedure to later stages. Now we will replace the edge coupled line sections with their folded equivalents.

- With cursor under the first coupled line, we apply command Insert / Parallel Coupled Line / Interdigital / OSOSINTD. During insertion we assign the same even and odd mode impedances (Zc1 and Zp1 respectively) as the edge coupled line section to be replaced, as initial trial values to start tuning. We set the length of new edge coupled section (Lng) as, for example 50 mm instead of 74.948 mm and thus the stub lengths (LngLeft and LngRight) will be 24.948 mm. We may also set the stub impedances. In this example we have left them at 50 ohms. Then we Delete the old edge coupled line section. Using Symmetrize command we can map the conversion done at the source end to the load end. In order to recover the response we may apply Tune Element command first on the odd mode impedance Zod (which affects coupling most) and then on Zev iteratively. In a few iteration we can recover response roughly. Then we repeat the same procedure on the other coupled line sections, getting the circuit of Figure 9.22.g. The fine tuning of return loss response seems easier to work out by using Best Case command which can be used to optimize Zev, Zod and stub lengths of all sections simultaneously.

Figure 9.22. Design of tapped L-shaped  resonator filter.

Figure 9.22. Design of tapped L-shaped  resonator filter (cont’d).

9. 4.  STEPPED IMPEDANCE RESONATOR FILTERS and MINIATURIZATION

9. 4. 1. INTRODUCTION

In the previous examples we have used uniform Half Wavelength TL resonators. Instead of uniform TL resonators, we can use Stepped Impedance Resonators (SIR) [47]-[64]. This approach has several advantages:

- Spurious passbands and harmonics: In some applications involving nonlinear active elements the harmonics nfo of the fundamental frequency fo pose important problems because they would pass through the spurious passbands of the coupled line filters around the frequencies nfo. These spurious passbands can be shifted away from nfo by using stepped impedance  resonators thus preventing the harmonics.

- Length reduction, folding, miniaturization and cross-coupling: Besides shifting the spurious passbands away from the harmonics nfo, other important applications of stepped impedance resonators appear in miniaturization though length reduction and folding for obtaining compact structures for more efficient use of the space allocated to the filter. The lengths of coupled sections can be reduced leaving some uncoupled TL sections free for folding. We will see that OC stubs can also be incorporated into the stepped resonators for further length reduction, hence miniaturization. The OC stubs can also be used for cross-coupling between different resonators for placement of finite or complex transmission zeros.

On Filpro, there are two commands for splitting a Half Wavelength TL piece into 3 or 5 TL pieces with different impedances to get 3-Step Resonators and 5-Step Resonators. Also a brute force approximate technique will be described for inserting OC stubs into the stepped resonators.

We will be mainly using Prototype with Inverters approach for designing the filters because of it is more advantages in forming different topologies and also ease of displaying the Coupling Coefficients and External Q of the filter.

Let’s study these issues on examples.

9. 4. 2. COUPLED LINE FILTERS WITH 3-STEP RESONATORS

EXAMPLE 9. 13. Coupled line filter with 3-Step resonators

Figure 9.23.a shows a 3-step resonator which is equivalent to a half wavelength TL resonator. Let’s now see what happens if we replace the half wavelength TL resonators with stepped impedance resonator of 3-steps.

- The filter is designed using Prototype with Inverters approach as described in Figure 9.23 giving the circuit of Figure 9.23.a in the form of inverter coupled shunt LC resonators.

- The LC resonators are equated using the command Inverters / Equate All Elements (Figure 9.23.b-c).

- From these LC resonator filters we can display and note down the External Q, Coupling Coefficients and Resonance Frequency parameters of the circuit to be used in realization and test stages.

- The LC resonators are then converted into Half Wavelength TL resonators by applying the command Replace by TL / Half Wavelength TL (Figure 9.23.d).

- Then the TL resonators are scaled to desired impedance level, for example 50 ohms,  by applying the command Inverters / Scale Cct by Inverters / Specify TL: Zo=50 ohms (Figure 9.23.e). This circuit is saved as Filt102.

- With cursor under the inner half wavelength TL pieces the command Resonator Oprs /   Half Wave TL to Stepped TL / 3-Step TL: Z2=50 ohms, Z1=60 ohms / Lng2=35.313mm is applied. This command splits the TL into three pieces. The middle piece has impedance Z1 and length Lng1 while the outer two pieces have equal impedances Z2 and equal lengths, Lng2. User specifies Z2, Z1 and Lng2 while Filpro calculates Lng1 of the inner TL. The displayed default value of Lng2  is set to give the minimum overall length of the   3-step resonator for the specified Z1 and Z2. This value of Lng2 gives Lng1= 2 Lng2.

- Z1 and Lng2 are the main trial parameters. We first try Z1=60 ohms, greater than 50 ohms to see its effect. For Lng2, we accept the default value.

- Figure 9.23.f shows the circuit after splitting the inner TL’s. We don’t split the outermost half wave TL’s because we will use them for tapping. If tapping is not desired then we would also split them.

- Then with cursor under the inverters the command Replace By PCL / Interdigital / ococINTD: f0=1000  MHz is applied. {The value displayed for f0 should be disregarded!.}  So we get coupled lines with shorter lengths compared to the previous cases (Figure 9.23.g). The filter is saved as Filt102a to be used later.

- Responses of this circuit are also shown below the circuit in Figure 9.23.g. We note that in wide span insertion loss response the spurious passbands are moved upward, away from the harmonics nf0.

- Next we try Z1=80 ohms and Z1=100 ohms cases. As Z1 increases the location of the first spurious PB shifts upward, up to about 2.5 fo as seen in Figure 9.23.h.

- Figures 9.23.i and k shows the circuits for these cases. We note that the total length of the resonators (Lng1+2Lng2) gets smaller as Z1 increases. So, besides the shifting of the spurious passbands away from nfo, another advantage of the stepped impedance resonators appears in the total length of the resonators. Length of the original Half Wave Resonators was about 149 mm. The total length of the 3-step resonators is about 141 mm for Z1=60 ohms, 127.7 mm for Z1=80 ohms and 117.5 mm for Z1=100 ohms. The minimum length is obtained under the condition Lng1= 2 Lng2 and Z1= Z2/Tan2 where  is the electrical length of the outer TL pieces.

- In Figures 9.23.m-n the input and output inverters are replaced by Tapped lines. This conversion degrades the return loss which can be recovered by tuning all elements or by applying Monte Carlo-Best Case optimisation routine. Figure 9.23.q shows the return loss before and after optimisation.

- Figure 9.23.p shows the layout of the filter with tapped input-output couplings. Clearly various other layouts are possible by folding the inner TL sections.

- Compared to the original coupled line filter (without steps), the couplings of the coupled line sections gets tighter. That is the even and odd mode impedances get far from each other. That is, as the coupling length Lng2 gets smaller coupling gets tighter. This should be expected because the same amount of power will be coupled over a shorter coupled line length.

Exercise: Change Lng2 by starting form a quarter wave length down to zero. It will be seen that  initially the first spurious passband gets away from 2f0 upward in frequency and after a certain Lng2, it starts to get closer to 2f0 again.  The maximum deviation is a function of Z1 and Z2 and should be found by trials. However it occurs somewhere about  Lng2=quarter wavelength/2.

Exercise:  Try Z1 less than 50 ohms to see that the spurious passbands move downward. Try other values for Z1 and Lng2.

Figure 9.23. 3-Step Resonator version of the coupled line filter.

Figure 9.23. 3-Step Resonator version of the coupled line filter.

Figure 9.23. 3-Step Resonator version of the coupled line filter.

EXAMPLE 9. 14. Comb-like 3-Step Resonator filter

It is possible to convert the interdigital type OC edge coupled line sections into comb type coupled line sections where the OC ends of the coupled lines are on the same side as follows:

- Load the circuit Filt102a of the previous 3-step resonator example (Figure 9.24.a).

- With cursor under the coupled line sections apply command Half Angle. This command replaces the interdigital type coupled line sections with Pi-sections of OC stubs in between two TL pieces (Figure 9.24.b). The lengths of OC stubs and surrounding TL’s are half of the original interdigital type coupled lines.

- Next, with cursor under the middle element of the Pi-section of OC stubs, the command Replace by PCL / CombLine / ococComb / Symmetric is applied to convert the Pi-sections into Comb Type coupled lines (Figure 9.24.c).

- So, in effect what we have done is, besides halving the length of the coupled line sections, we have inverted the sense of coupling also. Now the unterminated (OC) ends of the coupled lines are on the same side.

- The end inverters can be replaced by tapped lines as described in Figure 9.24.c-d or we may insert redundant lines at ports and then form two more coupled line sections.

- Figure 9.24.e shows a possible layout. The longer line pieces may be folded to reduce the length of the filter.

Figure 9.24. Comb-like version of the 3-step resonator filter.

EXAMPLE 9. 15. A  3-Step Resonator filter with Lumped Capacitors:

It is possible to replace the Comb type coupled line sections of the previous example with lumped capacitors to get a mixed element filter as follows:

- Load the circuit Filt102 of the previous 3-step resonator example (Figure 9.25.a).

- With cursor under the half wave TL’s the command Resonator Oprs / Half Wave TL to Stepped TL / 3-Step TL: Z2=30, Z1=100, Lng2=5 mm is applied. Here we select a low impedance for the outer TL sections which will form the coupled lines. We also set their lengths (Lng2) as short as we like (Figure 9.25.b).

- Then coupled lines are formed by placing the cursor under the inverters and applying the command Replace by PCL / Interdigital Line / ococINTD: fo=1000 MHz (Figure 9.25.c). A sketch of the structıre is shown in Figure 9.25.d. Coupling between resonators is provided by the small length low impedance end sections. Small dimensions may enable us to simulate the coupling elements with lumped elements, as follows.

- With cursor under the coupled line sections apply command Half Angle. This command replaces the interdigital type coupled line sections with Pi-sections of OC stubs in between two TL pieces (Figure 9.25.e). The lengths of OC stubs and surrounding TL’s are half of the original interdigital type coupled lines.

- Next, with cursor under the OC stubs of the Pi-sections, the command Replace by Lumped/ Single C: fo=1000 MHz is applied to convert them into lumped capacitors (Figure 9.25.f).

- The end inverters can be replaced by tapped lines as described in Figure 9.25.g-h.

- The capacitive Pi-sections may be realized using the data that can be obtained for the series (coupling) capacitors between two closely spaced TL pieces. Parts of the shunt capacitors may be absorbed into the fringing end capacitors of the low impedance TL pieces.

- As another approach we may Delete the shunt lumped capacitors and compensate their deficiency by adjusting the lengths of the longer TL pieces and the series capacitor through optimization. This is done in Figure 9.25.h-i-k.

- Figure 9.25.m shows the return loss response after optimisation. Note that return loss level reached to about 30 dB which is better than the original level.

- Figure 9.25.n shows pieces of a possible layout. The longer TL’s may be folded to reduce the length of the filter.

Figure 9.25. 3-step resonator filter with lumped capacitor coupling elements.

Figure 9.25. 3-step resonator filter with lumped capacitor coupling elements.

9. 5. 3. COUPLED LINE FILTERS WITH 5-STEP RESONATORS

Let’s now study a coupled line filter using stepped impedance resonators with 5 steps [61]. This approach leads to further reducation in the lengths of resonators. We will consider the same filter as in the previous example.

EXAMPLE 9. 16.  A 5-Step Resonator version of the coupled line filter

- The filter Filt102 is loaded (Figure 9.26.a).

- With cursor under the half wavelength TL pieces the command Resonator Oprs /   Half Wave TL to Stepped TL / 5-Step TL: Z3=50 ohms, Z2= 60 ohms, Z1=80 ohms is applied. This command splits the TL into five pieces. The five step resonator is symmetric about the middle piece with step impedances being denoted as Z1, Z2, and Z3, numbered by starting from the middle piece. Lengths of the four pieces with impedances Z2 and Z3 are equal while the length of the inner step is twice the others. User specifies Z3, Z2, Z1 and Filpro calculates lengths of the TL pieces.

- In this example we target not to use Tapping at input and output. Instead we will use redundant TL pieces to form two extra (redundant) coupled lines at input and output. That is why we split ALL the TL’s into 5-step pieces. If tapping is desired then we would not split the TL’s adjacent to the input and output inverters and follow the steps used in the previous example (Exercise).

- Figure 9.26.b shows the circuit after splitting the TL’s.

- Then with cursor under the inner inverters the command Replace By PCL / Interdigital / ococINTD: f0=1000  MHz is applied to convert the TL+Inverter+TL triplets into coupled lines (Figure 9.26.c). {The value displayed for f0 should be disregarded!.}

- In order to form TL+Inverter+TL pieces at input and output we insert two redundant (50 ohms) TL pieces at both ports (Figure 9.26.d-e). Lengths of the redundant TL’s should be equal to or greater than the TL pieces on the other sides of the adjacent inverters.

- Then with cursor under the outermost inverters the command Replace By PCL / Interdigital / ococINTD: f0=1000  MHz is applied to form the redundant coupled lines  at input and output (Figure 9.26.e-f). Note that the even and odd mode impedances are too far from each other which means extremely tightly coupled lines. Realization of such tightly coupled lines are problematic. Therefore it seems wise to prefer the tapping option (Exercise).

- Responses are shown in Figure 9.26.g. We note that in wide span insertion loss response the spurious passbands are moved upward, away from the harmonics nf0, in a similar way as in the 3-step case. In this respect we see not much difference between 3-step and 5-step resonators, except the extra flexibility provide by the three different impedances.

- The main advantage of the five step resonators may be further reduction in the lengths of the resonators for certain ranges of Z1, Z2 and Z3. In this example the total length of the 5-step resonators is 4x22.25 + 44.5 = 133.5 mm. A considerable decrease compared to 141.1 mm length of the 3-step resonators.

- Figure 9.26.h shows a possible layout of the filter. Clearly various other layouts are possible by using the flexibility provided by the inner line sections. For example in Figure 9.26.i a cross-coupled filter layout is shown where a cross-coupled triplet section is formed by introducing cross-coupling between resonators 2 and 4.

Figure 9.26. 5-Step Resonator version of the coupled line filter.

Figure 9.26. 5-Step Resonator version of the coupled line filter.

EXAMPLE 9. 17. Comb-like 5-Step Resonator filter

It is possible to replace the interdigital type OC edge coupled line sections into comb type coupled line sections where the OC ends of the coupled lines are on the same side as follows:

- Load the circuit Filt102b of the previous 5-step resonator example (Figure 9.27.a).

- With cursor under the coupled line sections apply command Half Angle. This command replaces the interdigital type coupled line sections with a Pi-section of OC stubs in between two TL pieces (Figure 9.27.b). The lengths of OC stubs and surrounding TL’s are half of the original interdigital type coupled lines.

- Next, with cursor under the middle element of the Pi-section of OC stubs, the command Replace by PCL / CombLine / ococComb / Symmetric is applied to convert the Pi-sections into Comb Type coupled lines (Figure 9.27.c).

- So, in effect what we have done is, besides halving the length of the coupled line sections, we have inverted the sense of coupling also. Now the unterminated (OC) ends of the coupled lines are on the same side.

- The end inverters can be replaced by tapped lines as described in Figure 9.27.c-d or we may insert redundant lines at ports and then form two more coupled line sections.

- Figure 9.27.e shows a possible layout. The longer line pieces may be folded to reduce the length of the filter.

- Figure 9.27.g shows a more compact layout which is possible by noting down the External Q and Coupling Coefficients of the filter and then forming coupled lines by introducing couplings between the inner pieces of impedance Z2 of the 5-step resonators instead of using the outer pieces (Z3).

Figure 9.27. Comb type 5-Step resonator filter.

Figure 9.27. Comb type 5-Step resonator filter.

9. 4. 4. COUPLED LINE FILTERS WITH STUB LOADED STEPPED RESONATORS

For either further reduction in the overall length of resonators or for more efficient use of the area occupied by the resonators, shunt OC stubs may be incorporated into the stepped or uniform TL resonators [51], [53], [55], [61]. On Filpro it is possible to design 3 and 5 step resonators loaded with stubs.

Let’s first consider the case with 3-step resonators.

EXAMPLE 9. 18. Solutions involving 3-Step Resonators with Stubs:

In Figure 9.28.a, the filter Filt102 of the previous examples is loaded.

- With cursor under the inner three half wave TL resonators, the command Resonator Oprs / Half Wave TL to Stepped TL / 3-Step TL: Z2=50, Z1= 100, Lng2=29.375 mm is applied to split the TL’s into 3-step resonators (Figure 9.28.b). The middle section has impedance Z1=100 ohms  and length Lng1= 2Lng2=58.75 mm (the numbers in the figure are rounded!). We leave the outer half wave TL’s as they are because we will apply tapping on them. The circuit is saved as Filt102H.

Formation of 3-step resonator with stubs:

- Next, we will insert shunt OC stubs into the 3-step resonators by brute force to covert them into 3-step resonators with stubs. For this purpose we Delete all elements except a 3-step resonator section together with its inverters on its two sides (Figure 9.28.c). We save this circuit as P102A. Then we plot the Insertion Loss and/or Return Loss response of this circuit to use as reference.

- Then with cursor under the middle and right TL pieces (of impedances Z1 and Z2, lengths Lng1 and Lng2), the command Insert / Tline-Stubs / OSsh: Zo= 50 ohms, Lng = 10 mm is applied. This command inserts shunt OC stubs on the two sides of the middle TL piece (Figure 9.28.d). In applying this command user can set impedance and length of the stub.

- Next, we will adjust the response of this circuit to match to the response of P102A. This can be done by tuning the lengths Lng1, Lng2 and impedances Z1 and Z2 of the TL pieces. For this example we notice that tuning only Lng2 of the two outer TL pieces is sufficient to get the same response as P102A. The tuned circuit is saved as P102b (Figure 9.28.e). Lng2 is reduced from 29.375 mm to 21.908 mm. So, length of each stepped resonator is reduced by about 15 mm. We note down the tuned length Lng2=21.908 mm. So, the new 3-step resonator with stubs is ready.

- Next, we load the filter Filt102H for modifications. We insert shunt OC stubs of impedance Zo=50 ohms and Lngstb=10 mm on both sides of the middle resonator of each 3-step resonator (Figure 9.28.f-g).

- We also use Change command to set the lengths of outer TL pieces as Lng2=21.908 mm. Thus we get the filter with 3-step resonators with stubs (Figure 9.28.g-h).

- Finally, with cursor under the inverters, the command Replace by PCL / Interdigital Line / ococINTD: fo=1000 MHz is applied to convert the TL+Inverter+TL triplets into coupled lines (Figure 9.28.i). We note that the bandwidth is slightly contracted. This is due to the brute force approximation. We will have more degradation in return loss after conversion of the input and output inverter+TL elements into Tapped form. Such degradations can be corrected by tuning a few elements slightly after completing the design.

- Figure 9.28.k shows a possible layout of the filter (after replacing end inverter by Tapped lines- Exercise?). Since each resonator length is reduced by about 15 mm, the overall length of the filter is reduced by 3x15=45 mm. Length reduction is made possible by a 10 mm increase in transversal dimension due to the stubs. Different architectures can be formed by folding some portions of the structure. More flexibility exists for formation of different layout architectures if we use 5-Step resonators as will be seen ahead.

Exercise: Try different lengths and impedances for the stubs.

Exercise: Tune Lng1 of the inner TL pieces to inspect alternative solutions.

Figure 9.28. Coupled line filter with 3-Step Resonators with Stubs.

Figure 9.28. Coupled line filter with 3-Step Resonators with Stubs.

EXAMPLE 9. 19. Solutions involving 5-Step Resonators with Stubs

Let’s now consider the case with 5-step resonators. We can get more flexibility in the formation of alternative layout architectures for either further reduction in the overall length of resonators or for more efficient use of the area occupied by the resonators.

- In Figure 9.29.a, the filter Filt102 of the previous examples is loaded.

- With cursor under the inner half wave TL resonators, the command Resonator Oprs / Half Wave TL to Stepped TL / 5-Step TL: Z3=50, Z2= 60, Z1=75 ohms is applied to split the TL’s into 5-step resonators (Figure 9.29.b). All outer sections have equal lengths (Lng2=Lng3=22.2 mm), while the middle section length is twice the others (Lng1= 2 Lng2= 44.2 mm) (the numbers are rounded!). We leave the outer half wave TL’s as they are because we will apply tapping on them. The circuit is saved as Filt102S.

Formation of 5-step resonator with stubs:

- Next, we will insert shunt OC stubs into the 5-step resonators by brute force to convert them into 5-step resonators with stubs. For this purpose we Delete all elements except a 5-step resonator section together with its inverters on its two sides (Figure 9.29.c). We save this circuit as P103A. Then we plot the Insertion Loss and/or Return Loss response of this circuit to use as reference.

- Then with cursor under the middle and right TL pieces the command Insert / Tline-Stubs / OSsh: Zo= 50 ohms, Lng = 10 mm is applied. This command inserts shunt OC stubs on the two sides of the middle TL piece (Figure 9.29.d). In applying this command user can set impedance and length of the stub.

- Next, we will adjust the response of this circuit to match to the response of P103A. This can be done by tuning the lengths Lng1, Lng2, Lng3 and impedances Z1, Z2 and Z3 of the TL pieces. Let’s try tuning only Lng1 of the middle TL piece to get almost the same response as P103A. The tuned circuit is saved as P103B (Figure 9.29.e). Lng1 is reduced from 44.4 mm to 39.06 mm. So, length of each stepped resonator is reduced by about 5 mm. This reduction is less than what we have got by tuning the lengths of outer TL pieces of the previous example. It would possibly be wiser to tune the lengths of outer TL pieces (Exercise?). We note down the tuned length Lng1=39.06 mm. So, the new 5-step resonator with stubs is ready.

- Next, we load the filter Filt102S for modifications. We insert shunt OC stubs of impedance Zo=50 ohms and Lngstb=10 mm on both sides of the middle resonator of each 5-step resonator. We also use Change command to set the lengths of middle TL pieces as Lng1=39.06 mm. Thus we get the filter with 5-step resonators with stubs (Figure 9.29.f-g).

- Finally, with cursor under the inverters, the command Replace by PCL / Interdigital Line / ococINTD: fo=1000 MHz is applied to convert the TL+Inverter+TL triplets into coupled lines (Figure 9.29.h). We note that the bandwidth is slightly contracted. This is due to the brute force approximation. We will have more degradation in return loss after conversion of the input and output inverter+TL elements into Tapped form. Such degradations can be corrected by tuning a few elements slightly after completing the design.

- Figures 9.29.i-to-n show possible layouts of the filter (after replacing end inverter by Tapped lines- Exercise?). Different architectures can be formed by folding some portions of the structure in different ways.

- By selecting stub lengths properly it may also be possible to implement cross-coupling between different resonators. For example Figure 9.29.n shows the formation of cross-coupled triplet section where the cross-coupling is provided through the fringing fields at the OC ends of the stubs of resonators 2 and 4. This cross-coupling can be used to place a finite transmission zero in lower stopband.  Cross-coupled quadruplets and higher order n-tuplets can be formed in the same way [?].

Exercise: Try different lengths and impedances for the stubs.

Exercise: Tune Lng2 and Lng3 of the outer TL pieces to inspect alternative solutions.

Figure 9.29. Coupled line filter with 5-Step Resonators with Stubs.

Figure 9.29. Coupled line filter with 5-Step Resonators with Stubs.

9.  5. DUAL BAND STEPPED RESONATOR FILTERS

Recent mobile communication applications necessitate the use of dual passband filters. Commensurate line Distributed Element filters have periodic passbands about the frequencies fo, 2fo, 3fo, 4fo, etc. with fo being the fundamental passband and the others termed as spurious passbands. The first two passbands of such filters may be used as the dual passbands.  In the usual edge coupled line filters the main passband is at fo=fq and the first spurious passband is around 2fo. In the previous example on stepped impedance resonators we aimed at shifting the first spurious passband away from 2fo, or, if possible, eliminate it completely. In dual band applications we just want it to shift the passband around 2fo to the desired location “without” degrading its return loss response. In this example we will show that it is possible to relocate the first spurious passband to desired location on the two sides of 2fo, within limited ranges using stepped resonators and redundant Unit Elements (TL’s) as described below.

EXAMPLE 9. 20. Dual Band version of the Stepped Impedance Resonator filter

The filter specs are:    PBRipple=0.01, fp1=975 MHz, fp2=1025 MHz, N=5

- The design steps of the filter are repeated in Figure 9.30 giving the coupled resonator version of the filter (Figure 9.30.a). Using the commands CTRL+F6 and ALT+F6, we note down the External Q, Coupling Coefficients and Resonance Frequencies of the resonators to use later during realization and tuning stages.

- The resonators are equated by using the command Inverters / Equate All Elements (Figure 9.30.b-c).

- Using the command Replace by TLine / Half Wavelength TL the LC resonators are replaced by Half Wavelength TL pieces (Figure 9.30.c-d).

- The TL impedances are scaled to 50 ohms by applying the command Inverters / Scale Cct by Inverters / Specify Adjacent TL: Zo=50 ohms (Figure 9.30.d-e).

- The half wave TL pieces are replaced by 3-step resonators by applying the command Half Wave TL to Stepped TL / 3-Step TL / Z2=50 ohms, Z1 = 60 ohms, Lng2=37.474 mm. In applying this command we can select the parameters Z2, Z1 and Lng2 where Z2 is the impedance of the outer steps, Z1 is the impedance of the inner step and Lng2 is the length of outer steps. Lng1 of the inner step will be calculated by Filpro. We leave Z2 as 50 ohms and as an initial trial set Z1=60 ohms. Lng2 is a quarter of the length of the original half wavelength TL. With this choice Lng1 of the inner step will be twice Lng2. We get the circuit of Figure 9.30.f.

- The inner TL+Inverter+TL triplets are equivalent to ococINTD type parallel coupled lines. Conversion into coupled line form is carried out by applying the command Replace by PCL / Interdigital Line / ococINTD: fo=1000 MHz. Here we have to type in the passband center frequency fo=1000 MHz. So, we get the coupled line filter shown in Figure 9.30.g.

- In the previous examples the inverters at source and load ends were used to create Tapped input/output Lines. This action keeps the passband edges and return loss of the main passband least effected while destroying the responses of all other passbands. Since now we want to preserve the response of the second passband also, we should not use tapping approach or we should develop a new tapping technique which keeps the second passband also unaffected. From the previous example which uses redundant Unit Element approach we know that it is possible to keep all spurious responses unaffected at the expense of using redundant coupled lines at input and output. Let’s try that approach. For this purpose we insert 50 ohms (redundant) TL pieces at input and output ports (Figure 9.30.h). Length of these TL pieces should be larger than or equal to the length (Lng2) of the TL piece on the other side of the relevant inverter. We set their length to be the same as Lng2=37.485 mm.

- Thus, we form TL+Inverter+TL triplets at source and load ends also. They are also converted into coupled line form, thus completing the design (Figure 9.30.i).

- Figure 9.30.k shows the layout and wide span insertion loss responses for different Z1 values greater than Z2. We see that the first spurious passband (second passband of the dual band filter) is moved to the right of 2fo as Z1 is increased. For Z1=120 ohms, the second passband is moved to 3fo.

- Figure 9.30.m shows the layout and wide span insertion loss responses for different Z1 values less than Z2. We see that the first spurious passband (second passband of the dual band filter) is moved to the left of 2fo as Z1 is decreased. For Z1=20 ohms, the second and third passbands get too close, hence start to become useless as dual band filter.

Figure 9.30. Design of dual band stepped impedance resonator filters.

Figure 9.30. Design of dual band stepped impedance resonator filters.

Figure 9.30. Design of dual band stepped impedance resonator filters.

9. 6. COUPLED LINE FILTERS WITH BRUTE FORCE FINITE TRANSMISSION ZEROS

Highpass filters with finite transmission zeros can be designed by using exact synthesis approaches that will be described Chapter 13. However in some cases it may be sufficient to impose finite transmission zeros into an edge coupled line filter by brute force and optimization. This imposition will degrade the return loss to some extent. If this approach can’t solve the problem then one may resort to the exact synthesis techniques of Chapter-11. Let’s continue discussions on the narrowband tapped edge coupled line filter of the previous examples. The same technique may also be applied on the wideband filters.

EXAMPLE 9. 21. An edge coupled line filter with brute force finite transmission zeros

- Figure 9.31 shows the design steps of an edge coupled line filter using Design / By Built in LC Prototype approach (quickest!). The resulting circuit is shown in Figure 9.31.a.

- Let’s use Tapping at ports. For this reason we can Delete the redundant TL’s at ports (Figure 9.31.b).

- With cursor under the interior inverters the command Replace by PCL / Interdigital Line / ococINTD: fo=1000 MHz is applied to convert the TL+Inverter+TL triplets into ococINTD type coupled lines.

- With cursor under the inverters at ports, the command Tapped PCL / Zo=50, fres=1000, fq=1000 MHz is applied giving the tapped circuit of Figure 9.31.c.

- With cursor under the element just on the right of the source resistance, apply command Insert / Parallel Coupled Line / I-L-U Res. Lines / ocOS-L is applied to insert a coupled line L-Resonator  (a coupled line with one end being Open Circuit (oc) and the other end loaded by an Open Stub (OS) ), as shown in Figure 9.31.d. Parameters of this element needs be adjusted until the desired finite transmission zero is obtained. This element is used to locate a finite transmission zero in lower stopband. The location of finite transmission zero will be is mainly set by tuning the length of the stub.

- Then an upper stopband transmission zero is placed by placing the cursor under the load resistance and applying the command Insert / Parallel Coupled Line / I-L-U Res. Lines / OSoc-L (Figure 9.31.d).

- Both tapping and brute force insertion of the L-resonators degrade the return loss heavily. We first tune the lengths of the stubs of the L-resonators to locate the FTZ’s at desired frequencies. Then we form a Monte Carlo List for optimisation through Best Case routine.

- Insertion loss and return loss responses before and after placement of the finite transmission zero sections are shown in Figure 9.31.e.

- Figure 9.31.f shows a layout of the circuit.

Figure 9.31. An edge coupled line filter with brute force finite transmission zeros.

Figure 9.31. An edge coupled line filter with brute force finite transmission zeros.