CHAPTER-10

MINIATURE

CHEBYSHEV-BUTTERWORTH

DISTRIBUTED ELEMENT BANDPASS FILTERS

FOR

STRIPLINE-LIKE STRUCTURES

CONTENT									Page

10.1. INTRODUCTION								10.2

10.2. WIDEBAND CLASS-E  FILTERS						10.4

10.3. WIDEBAND CLASS-C FILTERS						10.27

10.4. NARROW-MEDIUM BANDWIDTH CLASS-D FILTERS			10.52

10.5. NARROW-MEDIUM BANDWIDTH CLASS-F  FILTERS			10.63

10.6. FILTERS WITH MULTIPLE TRANSMISSION ZEROS  AT ZERO 		10.70

REFERENCES  AND FURTHER READING					10.83

CHAPTER-10

MINIATURE CHEBYSHEV-BUTTERWORTH

DISTRIBUTED ELEMENT BANDPASS FILTERS

10.1. INTRODUCTION

In Distributed Element Highpass Filters that we have studied in Chapter 09, the quarter wave frequency fqHP coincides with the passband center frequency f0, leading to fp2=2fq-fp1 with fp1 and fp2 being the lower and upper passband edge frequencies of the resulting bandpass characteristics. These filters have two disadvantages:

- They have a spurious passband centered about 3fq=3f0.

- The commensurate length L of stubs and transmission lines, hence the longitudinal size of  the filters is fixed by  the quarter wave frequency fq as L=c4fqεr. Therefore there is not much chance for size reduction.

Both of these disadvantages are eliminated in Distributed Element Bandpass Filters where the quarter wave frequency fqBP of the filter can be set independently of the passband center f0. This extra flexibility enables the designers to control both size of the filter and the location of the spurious passbands while also providing means for obtaining circuit topologies that are not available in Distributed Element Highpass Filters.

Figure 10.1 shows the periodic passbands of Distributed Element Highpass and Bandpass Filters.

Figure 10.1. Responses of Distributed Element Highpass and Bandpass filters.

By defining the passband center f0 as the arithmetic mean of the passband edges fp1 and fp2, as f0=(fp1+fp2)/2, if the quarter wave frequency of the Distributed Element Bandpass Filter is selected to be fq=2f0=fp1+fp2, then the response would be identical to the Distributed Element Highpass Filter case. In this respect the Distributed Element Highpass Filters are special cases of Distributed Element Bandpass Filters. Size reduction would be obtained if fq is selected to be higher than 2f0. Clearly, longer size would be obtained if fq is selected to be less than 2f0.

In the previous two chapters the filters made up of N-parallel coupled lines were studied, as Combline and Capacitively loaded Interdigital filters In this chapter, miniature Distributed Element Bandpass Filters involving simple coupled lines separated by transmission line pieces will be studied. These filters are suitable for implementation in stripline-like structures.

The first systematic survey on realizability conditions of such miniature filters involving contributing unit elements with  fqf0 is due to Minnis 30, [37]. He had defined four classes for bandpass filters termed as Class-A, B, C and D. Later Class-E and Class-F filters are added to this family [42]. Classes A and B are of elliptic type while C, D, E and F are of Chebyshev or Butterworth type. In this chapter only Classes-C,D,E and F will be studied. Classes A and B will be studied in the next chapter.

Since Class-E filters are natural extension of the simplest Distributed Element Highpass Filter, the edge coupled line filter, we will start with this class to display the link between Highpass and Bandpass filters.

  - WIDEBAND CLASS-E  FILTERS

This class of filters are characterized by the transmission zero pattern

Nzero=Ninf =1,  NUE=even

and they are formed by comb-type coupled line sections separated by double unit elements (Figure 10.2). This topology is named as Class-E by Shian and Tippet 42. It is suitable for wideband filters. This class of filters is a generalization of the topology that can be obtained from the well-known Distributed Element Higpass Filter, the edge coupled line filter, as described in the following example.

Figure 10.2. Prototype for Class-E filters.

For narrowband cases, one redundant unit element must be inserted from each end for adjustment of impedance level and the resulting structure is named as Class-F, as will be described later in this chapter.

EXAMPLE 10. 1.  An octave band Class-E filter obtained from a highpass filter

Lets consider an octave band distributed element highpass filter with

PBRipple=0.01 dB, fp=4000 MHz, fq=6000 MHz, leading to a passband with fp1=4000 MHz and fp2=8000 MHz. Let Nzero=1 and NUE=6.

This filter will be designed as an edge coupled line filter, and then it will be converted into a distributed element bandpass filter. Later we will re-design the same filter directly as a distributed element bandpass filter and compare the two approaches.

- Design command set is shown in Figure 10. 3. The transmission zero is extracted in the middle as  a series OC stub, giving the symmetric circuit of Figure 10.3.a.

- By placing the cursor under the Tlines adjacent to the series OC stub, the command Kuroda / Distribute Stub is applied to distribute the series OC stub between the Tlines (Figure 10.3.b).

- Then the stubs in between the Tlines are split into two equal parts to assign one stub on each side of each Tline to form Stub+TLine+Stub triplets (Figure 10.3.c).

- Then with cursor under any Tline, the command Kuroda / All Equal Stubs is applied to equate the series stubs (Figure 10.3.d). This circuit is saved as E01.fil.

- Thus, the circuit is ready for conversion into the classical edge coupled line form, by applying the command Replace By PCL / Interdigital / ococINTD / Symmetric as shown in Figure 10.3.e.

- A pictorial description is shown in Figure 10.3.f, together with the even and odd mode impedances of the coupled line sections. Clearly, the even and odd mode impedances seem difficult to realize, as they are too tightly coupled (Zeven>>Zodd or too high Zeven/Zodd ratio).

- Next, we will find the equivalent distributed element bandpass filter form. This is done on the filter E01.fil as shown in Figure 10.3.g. With cursor under the Tlines, the command Half Angle / OSsr+TLine+OSsr is applied. This command replaces a Stub+Tline+Stub triplet  by a Pi-section of OC stubs surrounded by two Tlines, all with halved lengths (Figure 10.3.h). The resulting circuit is a distributed element bandpass filter because it contains shunt OC stubs which behave as a single transmission zero at f= while the series OC stubs form a single transmission zero at f=0. Note also that the number of unit elements is doubled. This circuit is saved as E02.fil. Since the commensurate length is halved, the quarter wave frequency fqBP of the new bandpass filter is twice the original quarter wave frequency fqHP=6000 MHz of the highpass filter.  That is, fqBP=12000 MHz. In the next example, we will show that the same filter can also be designed as a distributed element bandpass filter with fp1=4000 MHz, fp2=8000 MHz and fq=12000 MHz, with transmission zeros of Nzero=1, Ninf=1 and NUE=12.

- The Pi-sections of OC stubs can be converted into comb-type coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By Lumped / Combline / ococCOMB / Symmetric.  The resulting structure is shown in Figure 10.3.i, together with the even and odd mode impedances of the coupled lines. Figure 10.3.k shows a pictorial description.

- It is seen that the even and odd mode impedance ratio Zeven/Zodd of the coupled line sections are much higher than the corresponding interdigital type coupled line sections displayed in Figure 10.3.f. This is always the case. That is, coupling in comb type coupled lines is much tighter than the coupling in the interdigital counterpart. Hence, realization of the comb-type coupled line sections seems more difficult than the corresponding edge coupled filter. However this problem can be overcome in comb-type coupled lines by using lumped capacitors to realize the series and shunt OC stubs, as described in Figures 10.3.m-n. The distortion in response can be corrected by tuning the element values.

In the next example we will obtain the same topology by designing the filter directly as a distributed element bandpass filter, with quarter wave frequency fq=12000 mHz. Then we will generalize this class of filter by specifying higher quarter wave frequencies to get size reduction. Just for comparison, we note down the total length of the filter as 75 mm, for both interdigital coupled and comb coupled type  filters.

Figure 10.3. Design of an octave band Class-E filter by starting from a distributed element highpass filter.

Figure 10.3. Design of an octave band Class-E filter by starting from a distributed element highpass filter (cont’d).

EXAMPLE 10. 2. Octave band Class-E filters designed as Distributed Element Bandpass filters

Let’s design the octave band filter of the previous example, now directly as a distributed element bandpass filter with specifications

PBRipple=0.01 dB, fp1=4000 MHz, fp2=8000 MHz, Nzero=Ninf =1, NUE=12 (N=14)

The quarter wave frequency will first be selected as fq=fp1+fp2=12000 MHz to get the same filter obtained in the previous example to see the link between Highpass and Bandpass versions, and then we will try fq=18000 and 24000 MHz to get miniaturized versions which will also give an idea on variation of element values with differing fq.

- Synthesis command set is shown in Figure 10.4. Elements are extracted by applying Standard Auto Extract command giving the symmetric circuit of Figure 10.4.a.

- The series OC and SC stubs in the symmetry plane of the circuit are split into two equal pieces and Exchange command is used to form the symmetric halves (Figure 10.4.b).

- Using the command Kuroda / Right / Full Stub, first the series SC stub is moved towards the source end and placed on the right of the first Tline at source end as shunt OC stub (Figure 10.4.c).

- Then the series OC stub is translated to the right of the shunt OC stub by applying the command Kuroda / Right / Full Stub. The transformers resulting form these operations are shifted rightmost by applying Simplify Circuit command. Thus, an LRight section of OC stubs is formed at the source end (Figure 10.4.d). This circuit is saved as mine01.fil.

- An LRight section of OC stubs followed by two Tlines is named as Kuroda-Minnis EF section. This circuit section will be used to form the targeted topology. The cursor is placed under the shunt OC stub of LRight section and the command Kuroda-Minnis E/F-Right is applied. This command splits the OC and SC stubs properly to form a symmetric Pi-section of OC stubs while the remaining portions of these stubs are transferred to the right of the two TLines to produce another LRight section there, producing also a transformer, as shown in Figure 10.4.e. In this complex operation user can set some of the parameters of the resulting circuit by selecting one of the two options displayed upon application of this command:

- Specify TLine-1 and TLine-2: In this option user specifies impedances for the two adjacent unit elements within the limits displayed at the bottom of screen.

- Specify Stub and TLine-1: In this choice, impedance of the shunt stub of the symmetric Pi section and impedance of first unit element will be specified by the user.

- In both cases, the limits of specifiable quantities displayed at the bottom of screen are “outermost bounds”. That is, they are not actual limits. The real limits that will give positive values for all elements involved in the circuit section are difficult to find because of non-linear relations. Therefore the user must make some trials in order to find allowable values for unit elements and stubs. Program automatically rejects the specifications which result in negative element values. Experiments indicated that it is simpler to use Specify TLine-1 and TLine-2 command at the beginning to search for possible impedance levels, because element values are too sensitive to variations in shunt stub values making search difficult using Specify stub+TLine-1 command. In fact, the impedance of first unit element on the left can be used as a first trial. This approach is used in designing the filters of this example. That is impedances of all unit elements are equated to the impedance of first unit element.

- The result of application of the first Kuroda-Minnis-EF command with the option Specify TLine-1 and TLine-2 / Z1=86 ohms, Z2=86 ohms is shown in Figure 10.4.e where the Tline impedances are set as 86 ohms, close to the impedance of the first Tline at source end.

- Resulting transformer is shifted rightmost by applying Simplify Circuit command to form a new Kuroda-Minnis-EF section, as shown in Figure 10.4.f.

- The command Kuroda-Minnis-EF / Right / Specify TLine-1 and TLine-2: Z1=86 ohms, Z2=86 ohms is applied again to form a new symmetric Pi-section of OC stubs while setting the impedances of the next two Tlines as 86 ohms (Figure 10.4.g).

- In the middle of the circuit we have a new LRight section of OC stubs, but only one Tline exists between LRight section and symmetry plane. This LRight section is converted into symmetric Pi section of OC stubs by  placing the cursor under the shunt leg of the LRight section and applying the command Pi-Tee-L / LRight-to-Sym Pi. The resulting transformer is shifted rightmost by applying Simplify Circuit command.

- Since we have physical symmetry, there is no need to repeat this procedure in the right half of the circuit. Symmetrize command will complete the design. The resulting circuit is saved as mine02.fil (Figure 10.4.h).

- Next, the Pi sections of OC stubs will be converted into coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By PCL / Combline / ococCOMB / Symmetric  The even and odd mode impedances of the coupled lines are shown in Figure 10.4.i. igure 10.4.j shows a pictorial description.

- It is seen that the topology of the circuit is the same as the circuit we have found in the previous example by using highpass filter approach. The element values are also very close to those found in the previous example. Thus, at first sight it may seem that  Class-E filter can be designed in both ways. However we haven’t yet utilized all the advantages of Bandpass Filter approach. The main advantage of the bandpass filter approach is that we can specify the quarter wave frequency independently of the passband. In this example we have used fq=fp1+fp2=12000 MHz to get the same filter as the one in the highpass filter case. So, no size (length) reduction is obtained yet. Both approaches gave the length about 75 mm for air dielectric. Also the even and odd mode impedances are unrealizable as they are. By increasing the quarter wave frequency we can shorten the length of filter besides adjusting the even-odd mode impedances of the coupled lines to desired / realizable levels.

- In Figures 10.4.k-m-n results of a trial with fq=18000 MHz are shown while Figures 10.4.p-q-r show the results of a trial with fq=24000 MHz. In Figure 10.4.s impedances for all three fq cases are summarized for comparison. Figure 10.4.t shows wideband responses for fq=12000 and 1800 MHz cases. The wide stopbands are clear. Inspection of impedances shows that as fq is increased to 18000 and 24000 MHz, impedances of transmission lines are increased to about 121 and 157 ohms respectively for fq=18000 and fq=24000 MHz while impedances of the shunt OC stubs decreased to realizable 100-110 ohms and 60-66 ohms levels respectively. The series OC stubs have too low impedances. This leads to high even-odd mode impedance ratio. Therefore the series OC stubs may be replaced by lumped capacitors or the coupling coefficients of coupled lines can be increased by adding lumped capacitors, for example by using overlay capacitors as shown in Figure 10.4.u.

- Length is reduced from 75 mm to 50 mm and 37.5 mm respectively, thus providing considerable miniaturization.

- When NUE/2 is an even number, we get two Tlines in the middle of the circuit about the symmetry plane. When NUE/2 is an odd number, we get a Pi-section of OC stubs (coupled line) in  the symmetry plane, as will be described in the next example.

Figure 10.4. Degree 14 octave band Class-E filters.

Figure 10.4. Degree 14 octave band Class-E filters (cont’d).

Figure 10.4. Degree 14 octave band Class-E filters (cont’d).

EXAMPLE 10. 3. A degree 12 octave band Class-E filter

Let’s design the previous octave band filter with NUE=10 leading to degree 12, to see the topology that will result for NUE/2 is an odd number.

PBRipple=0.01 dB, fp1=4000 MHz, fp2=8000 MHz, fq=18000 MHz, Nzero=Ninf=1, NUE=10

- Synthesis command set is shown in Figure 10.5. Standard Auto Extract command gives the circuit shown in Figure 10.5.a. The LLeft section of OC stubs is converted into a symmetric Pi-section by placing the cursor under the shunt leg and applying command Pi-Tee-L / LLeft-to-Sym Pi. The resulting transformer is shifted rightmost by applying Simplify Circuit command giving the symmetric circuit of Figure 10.5.b.

- Using the command Kuroda / Right / Full Stub repeatedly, first the series SC stub is translated to the source end as shunt OC stub (Figure 10.5.d). Then the series OC stub is translated to the right of the shunt OC stub by applying the command Kuroda / Right / Full Stub repeatedly again. The transformers resulting form these operations are shifted rightmost by applying Simplify Circuit command. Thus, an LRight section of OC stubs is formed at the source end (Figure 10.5.e).

- The LRight section of OC stubs followed by two Tlines is termed as Kuroda-Minnis EF section. With cursor under the shunt OC stub of the LRight section and the command Kuroda-Minnis E/F / Right / Specify TLine-1 and TLine-2: Z1=100 ohms, Z2=100 ohms is applied. This command forms a symmetric Pi-section of OC stubs while the remaining portions are transferred to the right of the two TLines which now have the specified 100 ohms impedances (Figure 10.5.f). The 100 ohms for Tline impedances are found proper after a few trials. After clearing the transformer by applying Simplify Circuit command a new Kuroda-Minnis-EF section is formed (Figure 10.5.g).

- The command Kuroda-Minnis-EF / Right / Specify TLine-1 and TLine-2: Z1=100 ohms, Z2=100 ohms is applied again to form a new symmetric Pi-section of OC stubs while setting the impedances of the next two Tlines to 100 ohms also. Simplify Circuit eliminates the transformer and a new Kuroda-Minnis EF section is formed (Figure 10.5.h).

- This procedure is repeated on the new Kuroda-Minnis EF section giving the circuit of Figure 10.5.i). In the middle section, we still have an LRight section of OC stubs. With cursor under the Tline on the right of this LRight section, Symmetrize command is applied and then this Tline is deleted to form a symmetric circuit with two series  OC stubs in the symmetry plane. Simplify Circuit command combines the two series stubs giving a Pi-section of OC stubs in the middle of the circuit as shown in Figure 10.5.j. Thus, we have obtained the targeted topology formed by Pi-sections of OC stubs separated by pairs of Tlines.

- Next, the Pi sections of OC stubs will be converted into coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By PCL / Combline / ococCOMB / Symmetric  The even and odd mode impedances of the coupled lines are shown in Figure 10.5.k.

- Figure 10.5.m shows a pictorial description.

Exercise: Note down the variation of impedances of the shunt OC stub impedances of the Pi-sections of OC stubs are Figure 10.5.j. Repeat the design by selecting Tline impedances higher and lower than 100 ohms and inspect variation of shunt OC stub impedances. Repeat also the whole design by selecting higher fq to They are too high.

Figure 10.5. A degree 12 octave band Class-E filter.

Figure 10.5. A degree 12 octave band Class-E filter (cont’d).

EXAMPLE 10. 4. A high degree octave band Class-E filter

As exercise with the tools of Filpro, let’s design a high degree octave band Class-E filter with following specifications:

PBRipple=0.1 dB, fp1=4000 MHz, fp2=8000 MHz, fq=12000 MHz, Nzero=Ninf =1, NUE=20 (N=22)

- Synthesis command set is shown in Figure 10.6. Elements are extracted by applying Standard Auto Extract command giving the symmetric circuit of Figure 10.6.a.

- The series OC and SC stubs in the symmetry plane of the circuit are split into two equal pieces and Exchange command is used to form the symmetric halves (Figure 10.6.b).

- Using the command Kuroda / Right / Full Stub, first the series SC stub is translated to the right of the first Tline at source end as shunt OC stub (Figure 10.6.c).

- Then the series OC stub is translated to the right of the shunt OC stub by applying the command Kuroda / Right / Full Stub. The transformers resulting form these operations are shifted rightmost by applying either Shift TRF / Rightmost or (easier) by applying Simplify Circuit command. Thus, an LRight section of OC stubs is formed at the source end (Figure 10.6.d). This circuit is saved as mine01.fil to be used in different design trials.

- An LRight section of OC stubs followed by two Tlines forms a Kuroda-Minnis EF section. The cursor is placed under the shunt OC stub of LRight section and the command Kuroda-Minnis E/F-Right / Specify TLine-1 and TLine-2: Z1=122 ohms, Z2=122 ohms. Here we have assigned the impedance of the leftmost Tline (Figure 10.6.e).

- Resulting transformer is shifted rightmost to form a new Kuroda-Minnis-EF section. The command Kuroda-Minnis-EF / Right / Specify TLine-1 and TLine-2: Z1=122 ohms, Z2=122 ohms is applied again to form a new symmetric Pi-section of OC stubs while setting the impedances of the next two Tlines as 122 ohms (Figure 10.6.f-g).

- This procedure is repeated until the middle of the filter is reached (Figures 10.6.k). In the middle section, we still have an LRight section of OC stubs, but only one Tline exists on the right of the LRight section. This LRight section is converted into symmetric Pi section of OC stubs by  placing the cursor under the shunt leg of the LRight section and applying the command Pi-Tee-L / LRight-to-Sym Pi (Figure 10.6.m). The resulting transformer is shifted rightmost by applying Simplify Circuit command.

- Since we have physical symmetry, there is no need to repeat this procedure in the right half of the circuit. Symmetrize command will complete the design. The resulting circuit is saved as mine02.fil.

- Next, the Pi sections of OC stubs will be converted into coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By PCL / Combline / ococCOMB / Symmetric  The even and odd mode impedances of the coupled lines are shown in Figure 10.6.n.

- Figure 10.6.p shows a pictorial description.

- It is seen that the shunt OC stub impedances are too high. This leads to extreme values for even and odd mode impedances which are impossible to realize. We have to repeat the design by changing some of the design parameters, like fq and PBRipple, degree, or select different impedances for Tlines until realizable impedances are obtained. An easier way to overcome this problem may be replacement of the OC stubs by their lumped capacitor equivalents, as shown in Figures 10.6.q-r. Some degradation will arise in response as shown in the figure which can be corrected by tuning some of the elements.

- A survey is made to get an idea on variation of impedances with different quarter wave frequencies. The results are listed in Figure 10.6.s. It is seen that as fq is increased unit element impedances increase while all stub impedances decrease.

Figure 10.6. A high degree octave band Class-E filter.

Figure 10.6. A high degree octave band Class-E filter (cont’d).

Figure 10.6. A high degree octave band Class-E filter (cont’d).

EXAMPLE 10. 5 . A  3:1 bandwidth Class-E filter

Let’s now consider a filter with specifications

PBRipple=0.01 dB, fp1=2000 MHz, fp2=6000 MHz, fq=10000 MHz, Nzero=1, Ninf=1, NUE=12

This is a 3:1 bandwidth filter.

- Design command set is the same as the previous example, as shown in Figure 10.7. Elements are extracted by applying Standard Auto Extract command giving the symmetric circuit of Figure 10.7.a. The series OC and SC stubs in the symmetry plane of the circuit are split into two equal pieces and Exchange command is used to form the symmetric halves (Figure 10.7.b).

- Using the command Kuroda / Right / Full Stub, first the series SC stub is moved towards the source end and placed on the right of the first Tline at source end as shunt OC stub (Figure 10.7.c).

- Then the series OC stub is translated to the right of the shunt OC stub by applying the command Kuroda / Right / Full Stub. The transformers resulting form these operations are shifted rightmost by applying Simplify Circuit command. Thus, an LRight section of OC stubs is formed at the source end (Figure 10.7.d).

- The LRight section of OC stubs followed by two Tlines forms a  Kuroda-Minnis EF section. The cursor is placed under the shunt OC stub of LRight section and the command Kuroda-Minnis E-F/Right / Specify TLine-1 and TLine-2: Z1=86, Z2=86 ohms is applied. Here we have selected Tline impedances close to the impedance of the Tline at source end which is 86.2 ohms (Figure 10.7.e). Resulting transformer is shifted rightmost by applying Simplify Circuit command to form a new Kuroda-Minnis-EF section, as shown in Figure 10.7.f.

- Next, the command Kuroda-Minnis-EF / Right / Specify TLine-1 and TLine-2 / Z1=86, Z2=86 ohms is repeated. (Figure 10.7.g).

- In the middle of the circuit we have a new LRight section of OC stubs, but only one Tline exists between LRight section and symmetry plane. This LRight section is converted into symmetric Pi section of OC stubs by  placing the cursor under the shunt leg of the LRight section and applying the command Pi-Tee-L / LRight-to-Sym Pi. The resulting transformer is shifted rightmost by applying Simplify Circuit command. Since we have physical symmetry, there is no need to repeat this procedure in the right half of the circuit. Symmetrize command will complete the design. The resulting circuit is saved as E05.fil (Figure 10.7.h).

- The series OC stubs have very small impedances. This means that if the Pi-sections of OC stubs are converted into ococCOMB type coupled line sections then these sections would have extremely tight couplings. Therefore we prefer to convert the series OC stubs directly to lumped capacitors by applying the command Replace By Lumped / Single C: f0=2000 MHz (Figure 10.7.i). Here we select the approximation frequency as the lower passband edge to prevent shifting of this edge, as the lower passband edge is more sensitive to the series OC stubs or series capacitors. The return loss plot shows negligible degradation while the wideband insertion loss plot shows that the spurious passbands are not effected much by this conversion. Figure 10.4.j shows a pictorial description.

Exercise: Repeat the design with fq=8000, fq=10000 MHz and fq=16000 MHz for different PBRipples like 0.05 dB and 0.1 dB and inspect the resulting element values.

Figure 10.7. A 3:1 bandwidth Class-E filter.

Figure 10.7. A 3:1 bandwidth Class-E filter.

EXAMPLE 10. 6. A  5:1 bandwidth Class-E filter

Let’s now consider a filter with specifications

PBRipple=0.01 dB, fp1=400 MHz, fp2=2000 MHz, fq=4000 MHz, Nzero=1, Ninf=1, NUE=12

This is a 5:1 bandwidth filter. Let’s see what problems we will have.

- Design command set is the same as the previous example, as shown in Figure 10.8. Elements are extracted by applying Standard Auto Extract command giving the symmetric circuit of Figure 10.8.a. The series OC and SC stubs in the symmetry plane of the circuit are split into two equal pieces and Exchange command is used to form the symmetric halves (Figure 10.8.b).

- Using the command Kuroda / Right / Full Stub, first the series SC stub is moved towards the source end and placed on the right of the first Tline at source end as shunt OC stub (Figure 10.8.c).

- Then the series OC stub is translated to the right of the shunt OC stub by applying the command Kuroda / Right / Full Stub. The transformers resulting form these operations are shifted rightmost by applying Simplify Circuit command. Thus, an LRight section of OC stubs is formed at the source end (Figure 10.4.d).

- The LRight section of OC stubs followed by two Tlines forms a  Kuroda-Minnis EF section. The cursor is placed under the shunt OC stub of LRight section and the command Kuroda-Minnis E-F/Right / Specify TLine-1 and TLine-2: Z1=75, Z2=75 ohms is applied. Here we have selected Tline impedances close to the impedance of the Tline at source end which is 75.4 ohms (Figure 10.8.e). Resulting transformer is shifted rightmost by applying Simplify Circuit command to form a new Kuroda-Minnis-EF section, as shown in Figure 10.8.f.

- Next, the command Kuroda-Minnis-EF / Right / Specify TLine-1 and TLine-2 is repeated but now by selecting impedances of Tlines as Z1=80 ohms, Z2=80 ohms instead of 75 ohms. This impedance is found after a few trials by inspecting the series capacitors of the resulting circuit at the end of the design, as will be described below (Figure 10.8.g).

- In the middle of the circuit we have a new LRight section of OC stubs, but only one Tline exists between LRight section and symmetry plane. This LRight section is converted into symmetric Pi section of OC stubs by  placing the cursor under the shunt leg of the LRight section and applying the command Pi-Tee-L / LRight-to-Sym Pi. The resulting transformer is shifted rightmost by applying Simplify Circuit command. Since we have physical symmetry, there is no need to repeat this procedure in the right half of the circuit. Symmetrize command will complete the design. The resulting circuit is saved as E11.fil (Figure 10.8.h).

- The series OC stubs have very small impedances. This means that if the Pi-sections of OC stubs are converted into ococCOMB type coupled line sections then these sections would have extremely tight couplings. Therefore we prefer to convert the series OC stubs directly to lumped capacitors by applying the command Replace By Lumped / Single C: f0=400 MHz (Figure 10.8.i). Here we select the approximation frequency as the lower passband edge to prevent shifting of this edge, as the lower passband edge is more sensitive to the series OC stubs or series capacitors. The return loss plot shows negligible degradation while the wideband insertion loss plot shows that the spurious passbands are not effected much by this conversion. Figure 10.8.j shows a pictorial description.

- Inspection of the series capacitors shows that they have large values. This necessitates realization of large capacitors which must be lossless over a wide band. Smaller capacitances can be obtained by collecting the series OC stubs at source and load ends as described in Figures 10.8.k-to-u.

- First, all the Pi-sections of OC stubs in the left half of the circuit are converted into LLeft sections by placing the cursor under the series element and applying the command  Pi-Tee-L / Pi-to-Pi-Tee-L / Pi-to-LLeft (Figure 10.8.k-m). Simplify Circuit command is used to get rid of the transformers. Then the command Kuroda / Right / Full Stub command is used to move the series stubs to the left of the adjacent Tlines (Figure 10.8.m-n). Then the command Pi-Tee-L / LRight-to-LLeft is used to convert the LRight sections into LLeft sections (Figure 10.8.p). Continuing this way, we collect the series OC stubs at the load end and convert it to lumped capacitor there (Figure 10.8.t-u). The series capacitors are now about 16 pF and all stubs and Tlines have realizable impedances.

Exercise: Repeat the design with fq=6000 and fq=8000 MHz and form different alternative circuits by using Kuroda transformations.

Figure 10.8. A 5:1 bandwidth Class-E filter.

Figure 10.8. A 5:1 bandwidth Class-E filter (cont’d).

Figure 10.8. A 5:1 bandwidth Class-E filter (cont’d).

  - WIDEBAND CLASS-C FILTERS

In Class-E filters of the previous examples we have placed a single transmission zero at infinity (Ninf=1). This led to shunt OC stubs or Pi-sections of OC stubs separated by pairs of Tlines. A filter of the same degree can also be realized in even smaller length by using more transmission zeros at infinity, leading to great reduction in the number of Unit Elements, at the expense of asymmetry in insertion loss response. Class-C filter of Minnis is of such a type and characterized by the following transmission zero pattern [30]:

Nzero=1, Ninf=q, NUE=Ninf+1=q+1

This transmission zero pattern leads to structures involving open circuited comb type coupled line sections separated by single Tlines, as shown in Figure 10.9. This class is suitable for wideband filters. In some cases the comb-type couplings can be replaced by lumped capacitors leading to different and easier topologies. Let’s continue the subject on examples.

Figure 10.9. Prototype of  Class-C filter.

EXAMPLE  10. 7. An octave  band Class-C filter

Let’s design the octave bandwidth filter of the previous example in Class-C form:

PBRipple=0.01 dB, fp1=4000 MHz, fp2=8000 Mhz, Nzero=1, Ninf =6, NUE=7 (N=14), fq=12000 MHz, fq=18000 MHz and fq=24000 MHz.

- The synthesis command sets are shown in Figure 10.10. Note that we have selected Distributed and Bandpass options. Transmission zeros can be extracted in any order. However, it is more convenient to extract them in the following order where the single transmission zero section appearing in the middle leading to a symmetric structure: ue-inf-ue-inf-ue-inf-Zero-ue-inf-ue-inf-ue-inf-ue. This order of extraction is also programmed into Filpro in the menu item Standard Auto Extract. Invoking this command, the elements are automatically extracted in that order (Figure 10.10.a).

- In order to get physical symmetry, we place the cursor under the Unit Element (TLine) in the middle and apply command Kuroda / Left / Equal Stub. This command transfers part of the series OC stub on the left of the Tline to its right producing also a transformer. The stub impedances on the left and right of the Tline are equal. Simplify Circuit (F6) command shifts the transformer rightmost and combines it with the transformer at the load end, yielding unity transformer and deletes it automatically (Figure 10.10.b). Thus, we get a symmetric circuit. From now on, we will work only on the left half of the circuit.

- Note that we have an L-section of OC stubs on the left of the middle Tline, looking to right, hence named as LRight section. With cursor under the shunt leg of this LRight section we apply the command Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-LLeft.. This command reverses the direction of L-section while producing a transformer. Simplify Circuit command moves the transformer rightmost position (Figure 10.10.b-c).

- With cursor under the Tline on the left of the series OC stub, we apply the command Kuroda / Right / Full Stub. This command transfers the stub on the right of the Tline to its left while also producing a transformer. Simplify Circuit command shifts the transformer to rightmost (Figure 10.10.d). Thus, we have formed a new LRight section of OC stubs.

- This LRight section is reversed to form an LLeft section by placing the cursor under the shunt leg and applying the command Pi-Tee-L / LRight-to-Pii-Tee-L / LRight-to-LLeft. Also we transfer the shunt OC stub on the left of the second Tline from the source end to its right by applying the command Kuroda / Left / Full Stub. This shunt OC stub is transferred to the right as a series SC stub (Figure 10.10.e).

- Now we have a series combination of a SC and an OC stub on the right of the second Tline. The triplet of Tline+Series SC Stub+Series OC Stub is termed as Kuroda-Minnis C/D section. This prototype circuit section will be used in the following transformations to form Pi-sections of OC stubs in between each Tline.

- With cursor under the Tline on the left of the series OC+SC Stub, the command Kuroda-Minnis CD/ Right / k=0.9 is applied. This command splits the series SC and OC stubs and transfers them to the left of the Tline in such a way that we get a symmetric Pi-section of OC stubs on the left of the Tline. In applying this command, Filpro asks a constant k. This is the only parameter which should be specified by the user to produce different alternative circuits. After specifying a trial value of k=0.9 (for example), Filpro displays impedance of the shunt OC stubs and the new impedance of the Tline and asks if a new k value is tried or not. If these impedances are OK, then we select the option NO and we get the circuit shown in Figure 10.10.f. We also get a transformer and a piece of series OC stub on the right of the Tline The transformer is moved to the load end by  applying Simplify Circuit command.

- Then we pass to formation of a new Kuroda-Minnis CD section. This is done by placing the cursor under the third Tline and applying the command Kuroda / Left / Full Stub. This command transfers the shunt OC stub on the left of the Tline to its right as series SC stub. Then this command is repeated to move the series OC on left to the right of the Tline as shown in Figure 10.10.g. Simplify Circuit command transfers the resulting transformer to the rightmost position.

- Thus, a new Kuroda-Minnis CD section is formed. With cursor under the Tline of this section, the command Kuroda-Minnis CD / Right / k=0.9 is applied again forming the second Pi-section of OC stubs (Figure 10.10.h).

- In the circuit of Figure 10.10.h, we have an LLeft section of OC stubs on the left of the middle Tline. It is converted into a symmetric Pi-section of OC stubs by applying the command Pi-Tee-L / LLeft-to-Sym Pi (Figure 10.10.i). Symmetrize command maps all these circuit transformations done in the left half of the circuit to the right half, completing the circuit transformations. Thus, the circuit is put into a form with Pi-sections of OC stubs separated by Tlines. The circuit is saved as C01.fil.

- The Pi-sections of OC stubs can be transformed into OC comb-type coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By PCL / Combline / ococCOMB / Symmetric, as described in Figures 10.10.j.

- So, the topology of Class-C type is obtained. However we have some problems. It is seen that the odd mode impedances are too low compared to the even mode impedances. This means too tight coupling which is difficult to realize. This problem may be solved by realizing the series OC stubs of the filter C01.fil as lumped capacitors. This is done by loading C01.fil, placing the cursor under the series OC stubs and applying command Replace By Lumped / Single C: f0=4000 MHz (Figure 10.10.k). This approximation is carried out at the low end of the passband because the series OC stubs are more effective in setting the lower passband edge. The resulting circuit is shown in Figure 10.10.m. Figure 10.10.m shows a pictorial description while Figure 10.10.p shows a slight degradation in return loss. It is seen that the series coupling capacitances are about 10 pF which are rather high.

- As another trial the design is repeated for fq=18000 MHz, resulting in the circuit of Figure 10.10.q, saved as C11.fil. In this circuit we realize the Pi-sections of OC stubs at source and load ends as comb type parallel coupled lines while the series OC stubs of the other Pi-sections are replaced by their lumped capacitor equivalents (Figure 10.10.r).

- Figure 10.10.s shows a pictorial description. Some of the shunt OC stubs have small impedances. They can be realized by splitting them. The series capacitors are slightly less than the previous design (about 8 pF), but they are still high. If smaller capacitances are desired we may reduce the number of series OC stubs as follows.

- The circuit of C11.fil is loaded (Figure 10.10.t). The second and third Pi-sections of OC stubs from left are first converted into LRight sections by placing the cursor under the series OC stubs and applying the command Pi-Tee-L / Pi-to-Pi-Tee-L / Pi-to-LRight. Simplify Circuit command moves the resulting transformers rightmost (Figure 10.10.u).

- The series OC stubs of the LRight sections are moved to the right of the adjacent Tlines by using the command Kuroda / Left / Full Stub. Thus, a Tee-section of OC stubs is formed on the left of the middle Tline (Figure 10.10.v).

- The Tee-section is converted into LRight section by applying the command Pi-Tee-L / Tee-to-Pi-Tee-L / Tee-to-LRight (Figure 10.10.w). Symmetrize and Simplify Circuit commands are used to map the left half into the right half and eliminate the transformers resulting from these operations.

- Finally, the Pi-section of OC stubs near the source and load ends are converted into comb type coupled lines while the series OC stubs on the two sides of the middle Tline are converted into lumped capacitors, as described in Figure 10.10.x.

- Figure 10.10.y shows a pictorial description. It is seen that the series capacitors are now reduced to 2.65 pF.  Impedances of some of the shunt OC stubs are too low. They can be split into two to form two OC stubs with doubled impedances or we may also replace them by lumped capacitors.

- Figure 10.10.z shows a slight degradation in return loss which can be corrected by tuning some of the elements.

Figure 10.10. An octave band Class-C filter.

Figure 10.10. An octave band Class-C filter (cont’d).

Figure 10.10. An octave band Class-C filter (cont’d).

Figure 10.10. An octave band Class-C filter (cont’d).

EXAMPLE 10. 8. A degree 10 octave band Class-C filter

Let’s now consider an octave band filter with less degree and even number of unit elements, for example with the following specifications:

PBRipple=0.01 dB, fp1=4000 MHz, fp2=8000 MHz, fq=12000 MHz, Nzero=1, Ninf=5, NUE=6

leading to a degree 12 bandpass filter.

- The design command set is shown in Figure 10.11. The synthesized circuit has an LLeft section of OC stubs in the middle (Figure 10.11.a).

- The LLeft section is converted into symmetric Pi-section of OC stubs by placing the cursor under the shunt leg of the LLeft section and applying the command Pi-Tee-L / LLeft-to-Sym Pi (Figure 10.11.b).

- After applying Simplify Circuit command we get the symmetric circuit shown in Figure 10.11.c.

- In order to work only in one half of the circuit, the series OC stub in the middle is split into two equal parts (Figure 10.11.d).

- Then the LRight section of the left half is converted into LLeft section by placing the cursor under the shunt leg and applying the command Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-LLeft. Thus, the series OC stub is now adjacent to the unit element and we have a transformer in the middle of the circuit (Figure 10.11.e).

- Next, with cursor under the Tline (Unit Element) adjacent to the series OC stub, we apply the command Kuroda / Right / Full Stub to translate the series OC stub to the left of the Tline. The resulting transformer is shifted rightmost by applying the command Shift TRF / Rightmost. With cursor under the middle transformer we apply Symmetrize command to map the left half of the circuit to the right half and then Delete the transformer (Figure 10.11.f).

- Now we have LRight section on the right of the second TLine. It is converted into LLeft section by applying the command Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-LLeft again (Figure 10.11.g). We shift the resulting transformer rightmost and Symmetrize the circuit (Figure 10.11.h).

- The series OC stub on the right of the second Tline is translated to the left of the Tline by applying the command Kuroda / Right / Full Stub again. The resulting transformer is shifted to rightmost and the circuit is symmetrized (Figure 10.11.i).

- The transformers produced in the previous circuit transformations reduced impedances of inner elements to reasonable levels. Further reduction can be obtained by converting the LRight section of OC stubs on the right of the first Tline into different forms. For example it can be converted into a symmetric Pi-section of OC stubs by applying the command Pi-Tee-L / LRight-to-Sym Pi (Figure 10.11.k). The resulting Pi-section of OC stubs is equivalent to ococCOMB type coupled line. However the shunt stub impedances are somewhat high for realization. We can solve this problem by converting the Pi-section into a Tee-section by placing the cursor under the series element of the Pi-section and applying the command Pi-Tee-L / Pi-to-Pi-Tee-L / Pi-to-Tee (Figure 10.11.m). Now the shunt arm of the Tee-section has a reasonable impedance. This circuit is saved as minc11.fil to be used in other stages.

- The series arm OC stubs of the Tee-sections are too small for realization as stubs. Therefore they are converted into lumped capacitors by applying the command Replace By Lumped / Single C: f0=4000 MHz (Figure 10.11.n). This approximation is carried out at the lower edge of the passband to keep this edge unaffected by the approximation. The return loss will be degraded as shown in the figure. It can be recovered by tuning all elements slightly (Figure 10.11.p).

- A pictorial description of possible realization in stripline-like form is shown in Figure 10.11.q. The series capacitors can be realized as dielectric patches with conductor backing bridging the Tline and stubs.

- An alternative stucture is described in Figures 9.11.r-to-u. We load the circuit minc11.fil (Figure 10.11.r). The series arm OC stubs of the Tee-section are translated to the other sides of the adjacent Tlines by applying Kuroda / Righ-Left / Full stub command. After shifting the transformers rightmost and symmetrizing the circuit we get the circuit of Figure 10.11.s.

- Now we convert the translated series OC stubs into lumped capacitors and tune the circuit again (Figure 10.11.t-u). In this structure  the shunt OC stub impedance of the Tee-section has a lower impedance.  Figure 10.11.v shows a pictorial description.

Figure 10.11. Design of a degree 12 octave band Class-C filter.

Figure 10.11. Design of a degree 12 octave band Class-C filter (cont’d).

Figure 10.11. Design of a degree 12 octave band Class-C filter (cont’d).

EXAMPLE  10. 9. A degree 22 octave  band Class-C filter

Let’s design the same octave bandwidth filter with a higher degree:

PBRipple=0.01 dB, fp1=4000 MHz, fp2=8000 Mhz, Nzero=1, Ninf =10, NUE=11 (N=22), fq=12000 MHz, fq=18000 MHz and fq=24000 MHz.

- The synthesis command sets are shown in Figure 10.12. Note that we have selected Distributed, Bandpass, Type A options. Transmission zeros are extracted by using Standard Auto Extract  command giving the circuit of Figure 10.12.a.

- In order to get physical symmetry, we place the cursor under the Unit Element (TLine) in the middle and apply command Kuroda / Left / Equal Stub. This command transfers part of the series OC stub on the left of the Tline to its right producing also a transformer. The stub impedances on the left and right of the Tline are equal. Simplify Circuit (F6) command shifts the transformer rightmost and combines it with the transformer at the load end, yielding unity transformer and deletes it automatically (Figure 10.12.b). Thus, we get a symmetric circuit. From now on, we will work only on the left half of the circuit.

- Our aim in the following transformations is to move the series OC stub on the left of the middle Tline towards the source end, as follows.

- Note that we have an L-section of OC stubs on the left of the middle Tline, looking to right, hence named as LRight section. With cursor under the shunt leg of this LRight section we apply the command Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-LLeft.. This command reverses the direction of L-section while producing a transformer. Simplify Circuit command moves the transformer rightmost position (Figure 10.12.b-c).

- With cursor under the Tline on the left of the series OC stub, we apply the command Kuroda / Right / Full Stub. This command transfers the stub on the right of the Tline to its left while also producing a transformer. Simplify Circuit command shifts the transformer to rightmost (Figure 10.12.d). Thus, we have moved the LRight section of OC stubs leftward, to other side of the adjacent Tline.

- We will repeat the procedure described in the previous two steps to move the LRight section further left. These steps are described in Figures 10.12.d-to-i.

- At step-i of the figure, we transfer the shunt OC stub on the left of the second Tline from the source end to its right by applying the command Kuroda / Left / Full Stub. This shunt OC stub is transferred to the right as a series SC stub (Figure 10.12.j).

- Now we have a series combination of a SC and an OC stub on the right of the second Tline. The triplet of Tline+Series SC Stub+Series OC Stub is termed as Kuroda-Minnis C/D section. This prototype circuit section will be used in the following transformations to form Pi-sections of OC stubs in between each Tline. We save this circuit as minc01.fil to use it as the starting circuit in the iterative trials for formation of alternative equivalent structures, as follows.

- With cursor under the Tline on the left of the series OC+SC Stub, the command Kuroda-Minnis CD/ Right / k=0.5 is applied. This command splits the series SC and OC stubs and transfers them to the left of the Tline in such a way that we get a symmetric Pi-section of OC stubs on the left of the Tline, as shown in Figure 10.12.k. We also get a transformer and a piece of series OC stub on the right of the Tline. In applying this command, Filpro asks a constant k. This is the only parameter which should be specified by the user to produce different alternatives. After specifying a trial value of k=0.5 (for example), Filpro displays impedance of the shunt OC stubs and the new impedance of the Tline and asks if a new k value is tried or not. If these impedances are OK, then we select the option NO and we get the circuit shown in Figure 10.12.k.

- Then we pass to formation of a new Kuroda-Minnis CD section. This is done by placing the cursor under the third Tline and applying the command Kuroda / Left / Full Stub. This command transfers the shunt OC stub on the left of the Tline to its right as series SC stub (Figure 10.12.m).

- Then with the cursor being still under the same Tline, the command Kuroda / Left Full Stub is applied again, now to transfer the series OC stub on left to its right (Figure 10.12.n). Simplify Cct command transfers the resulting transformer to the rightmost position.

- Thus, a new Kuroda-Minnis CD section is formed. With cursor under the Tline of this section, the command Kuroda-Minnis CD / Right / k=0.5 is applied again, forming the second Pi-section of OC stubs (Figure 10.12.p).

- Then the third Pi-section of OC stubs is formed by repeating the previous two steps, as shown in Figures 10.12.q-r-s.

- The same steps are repeated to form the fourth Pi-section of OC stubs (Figure 10.12.t-u-v).

- At the step of Figure 10.12.v, we have an LLeft section of OC stubs on the left of the middle Tline. It is converted into a symmetric Pi-section of OC stubs by applying the command Pi-Tee-L / LLeft-to-Sym Pi (Figure 10.12.w). Symmetrize command maps all these circuit transformations done in the left half of the circuit to the right half, completing the circuit transformations. The circuit is saved as minc02.fil

- The Pi-sections of OC stubs can be transformed into OC comb-type coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By PCL / Combline / ococCOMB / Symmetric, as described in Figures 10.12.x-y. A pictorial description and even-odd mode impedances are also given in the same figure.

- It is seen that the odd mode impedances are too low compared to the even mode impedances. This means too tight coupling which is difficult to realize. This problem can be solved by realizing the series OC stubs as lumped capacitors. Also some of the shunt OC stub impedances are too high. They may also be replaced by their lumped capacitor equivalents. These transformations are shown in Figure 10.12.z1-z2. Such filters may also be called Class-C Capacitively Loaded End Coupled Line Filters. Some deformations will arise because of these approximations. However they are correctable by tuning elements of the circuit.

- Clearly, this design needs iterative trials where the main control parameter is the split ratio k. By trying different values for k, one can adjust impedances of Pi sections and unit elements within some inherent limits set by complicated interrelations among the element values of structure. Further adjustment and refinement in element values is possible using Kuroda transformations and/or by converting the symmetric Pi-sections into asymmetric Pi-sections or L-sections using Pi-Tee-L transformations.

As an example in Figure 10.13, impedances of all transmission lines are set equal to the impedance of the first Tline at left (122 ohms) by applying the command Pi-Tee-L / Pi-to-Pi-Tee-L / Pi-to-Pi / Specify TRF: n=.. . In this application the turns ratio of transformers that will result from Pi-to-Pi conversions is calculated from the ratio of Tline impedance to 122 ohms. These conversions will make the Pi-sections asymmetric. The shunt OC stubs with high impedances can be converted to lumped capacitors while some stubs can be left as they are. Figures 10.13.e-g show such an example.

Exercise: Repeat the design by specifying different k values to see how the element values are effected.

Exercise: Repeat the design with different quarter wave frequencies fq to see variation of the element values with fq.

Figure 10.12. A degree 22 octave band Class-C filter.

Figure 10.12. A degree 22 octave band Class-C filter (cont’d).

Figure 10.12. A degree 22 octave band Class-C filter (cont’d).

Figure 10.12. A degree 22 octave band Class-C filter (cont’d).

Figure 10.13. Adjustment of Tline impedances using Pi-Tee-L transformations.

EAMPLE 10. 9. A 3:1 bandwidth Class-C filter

Let’s now consider a 3:1 bandwidth filter, with the following specifications:

PBRipple=0.01 dB, fp1=2000 MHz, fp2=6000 MHz, fq=12000 MHz, Nzero=1, Ninf=5, NUE=6

- The design command set is shown in Figure 10.14. The synthesized circuit has an LLeft section of OC stubs in the middle (Figure 10.14.a).

- The LLeft section is converted into symmetric Pi-section of OC stubs by placing the cursor under the shunt leg of the LLeft section and applying the command Pi-Tee-L / LLeft-to-Sym Pi. After applying Simplify Circuit command we get the symmetric circuit shown in Figure 9.14.b.

- In order to work only in one half of the circuit, the series OC stub in the middle is split into two equal parts (Figure 10.14.c).

- Then the LRight section of the left half is converted into LLeft section by placing the cursor under the shunt leg and applying the command Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-LLeft. Thus, the series OC stub is now adjacent to the unit element and we have a transformer in the middle of the circuit (Figure 10.14.d).

- Next, with cursor under the Tline (Unit Element) adjacent to the series OC stub, we apply the command Kuroda / Right / Full Stub to translate the series OC stub to the left of the Tline. The resulting transformer is shifted rightmost by applying the command Shift TRF / Rightmost. With cursor under the middle transformer we apply Symmetrize command to map the left half of the circuit to the right half and then Delete the transformer (Figure 10.14.e).

- Now we have LRight section on the right of the second TLine. It is converted into LLeft section by applying the command Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-LLeft again. Simplify Circuit command shifts the transformers rightmost and combines the two adjacent shunt OC stubs in the middle (Figure 10.14.f).

- The series OC stub on the right of the second Tline is translated to the left of the Tline by applying the command Kuroda / Right / Full Stub again. The resulting transformer is shifted to rightmost and the circuit is symmetrized (Figure 10.14.g).

- The transformers produced in the previous circuit transformations reduced impedances of inner elements to reasonable levels. Further reduction can be obtained by converting the LRight section of OC stubs on the right of the first Tline into different forms. For example it can be converted into an LLeft section by applying the command Pi-Tee-L / Pi-to-Pi-Tee-L / LRight-to-LLeft (Figure 10.14.h).

- After simplification we get the circuit of Figure 10.14.i. All impedances are realizable, except the series OC stub. The low impedance shunt OC stubs can be splitted to double their impedances.

- The series arm OC stubs are converted into lumped capacitors by applying the command Replace By Lumped / Single C: f0=3000 MHz (Figure 9.14.k). This approximation is carried out near the lower edge of the passband to avoid excessive shift in the lower edge. keep this edge unaffected by the approximation. The return loss ripples will be degraded as shown in the figure. Return loss can be recovered by tuning all elements slightly (Figure 10.14.m).

- A pictorial description of possible realization in stripline-like form is shown in Figure 10.14.n after splitting the low impedance shunt OC stubs into two. The series capacitors can be realized as dielectric patches with conductor backing bridging the Tline and stubs.

Figure 10.14. Design of a 3:1 bandwidth Class-C filter.

Figure 10.14. Design of a 3:1 bandwidth Class-C filter (cont’d).

EXAMPLE 10. 10. A 5:1 bandwidth Class-C filter

Lets now design an ultra wideband Class-C filter with specifications

PBRipple=0.01 dB, fp1=400 MHz, fp2=2000 MHz, fq=4000 MHz, Nzero=1, Ninf=6, NUE=7

- Design command sets are shown in Figure 10.15. The synthesized circuit is shown in Figure 10.15.a. The circuit is made symmetric by placing the cursor under the Tline in the middle and applying the command Kuroda / Left / Equal Stub. Simplify Circuit command transfers the resulting transformer to the load end and combines it with the transformer there leading to unity turns ratio, hence deleted (Figure 10.15.b).

- In this example we see that the element values are not too far from realizable levels. Therefore instead of transforming the circuit into the standard Class-C form, we inspect the impedances while the series OC stub in the left half of the circuit is translated towards the source.

- First, the LRight section on the left of the middle Tline is converted into LLeft section by applying the command Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-LLeft. Then the circuit is made symmetric by placing the cursor under the middle Tline and applying Symmetrize command. The resulting transformers are eliminated by applying Simplify Circuit command (Figure 10.15.c).

- The series OC stub is moved further left by applying the command Kuroda / Right / Full Stub (Figure 10.15.d). The circuit is made symmetric and the resulting transformers  are eliminated by Simplify Circuit command (Figure 10.15.d).

- This procedure is repeated until the series OC stub is moved to the source and as shown in Figures 10.15.e-to-h.

- Inspection of the impedances of shunt stubs and Tlines shows that all are realizable. The series OC stubs have too low impedances. They are converted into lumped capacitors by applying the command Replace By Lumped / Single C: f0=400 MHz. We get a high capacitor of about 20 pF (Figure 10.15.i). If such a capacitor can be realized then the circuit is realizable.

- Figure 10.15.j shows a pictorial description of the final structure.

- Response is shown in Figure 10.15.k. Degradation in return loss can be recovered by tuning a few elements. If desired, the spurious passband can be moved further away by designing the filter with a higher fq, like fq=6000 or 8000 MHz (Exercise?).

Figure 10.15. Design of a 5:1 bandwidth Class-C filter.

Figure 10.15. Design of a 5:1 bandwidth Class-C filter (cont’d).

Exercise: Figure 10.16 shows the results of some ultra wideband Class-C designs. Verify these results. Also develop different alternatives.  Repeat the designs for different numbers of transmission zeros.

Figure 10.16. Some ultra wideband Class-C filters.

10.4.  NARROW-MEDIUM BANDWIDTH CLASS-D FILTERS

This class is characterized by the transmission zero pattern [30]

Nzero=1, Ninf =q ,   NUE =Ninf-1=q-1

These filters yield the same combline-unit element topology of Class-C types after inserting one redundant unit element at each port (Figure 10.17). For odd NUE, the middle element can be realized as an interdigital coupled line section, as will be shown. This class is suitable for narrow to moderate bandwidth applications, including octave band filters.

Figure 10.17. Prototype topology for Class-D filters.

EXAMPLE 10. 11. A narrowband comb-coupled U-folded Class-D filter with even NUE

Let’s synthesize a filter with following specifications:

PBRipple=0.01 dB, fp1= 975 MHz, fp2=1025 MHz, fq=2000 MHz, Nzero=1, Ninf=11, NUE=10

- Design command set is shown in Figure 10.18. Note that we have selected Distributed and Bandpass options. Transmission zeros can be extracted in any order. However, it is more convenient to extract them in the order programmed under the menu item Standard Auto Extract. Invoking this command, the elements are automatically extracted in the order shown in Figure 10.18.a.

- In order to get physical symmetry, we place the cursor under the SC and OC stubs in the middle of the circuit and apply Split / Two Equal command. Then use Exchange command forming the symmetric halves. From now on, we will work only on the left half of the circuit of Figure 10.18.b.

- Our aim in the following transformations is to move the series OC stub on the right of the left half circuit towards the source end by Kuroda transformations as follows.

- With cursor under the Tline on the left of the OC stub, the command Kuroda / Right / Full Stub is applied to move the stub to the left of the Tline (Figure 10.18.c). The resulting transformer is shifted rightmost by applying the command Shift TRF / Rightmost (Ctrl+Q).

- Then Exchange command is used to exchange the positions of the SC and the translated OC stub so that the OC stub will be adjacent to the Tline on its left (Figure 10.18.d).

- The command Kuroda / Right / Full Stub is repeated by placing the cursor under this Tline to move the OC stub one more step left. Then Shift TRF and Exchange commands are repeated.

- This way, we arrive at the topology shown in Figure 10.18.h where we have a series combination of a SC and an OC stub near the source. Symmetrize command maps the left half of the circuit to right. This circuit is saved as mind01.fil.

- Next, a redundant unit (50 ohms) element will be inserted at the source and load ends for adjusting element values. However, in order to get more flexibility for adjustments, instead of inserting redundant unit elements directly, we first apply command  Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=52 ohms (Figure 10.18.i) and then replace the ideal inverters by unit elements (quarter wave Tlines). Here, the inverter constant K=52 ohms is just a trial value. It will be iterated after inspecting the resultant element values. The case for K=50 ohms would correspond to insertion of redundant unit elements.

- The ideal inverters at source and load ends can be replaced by quarter wave Tlines by applying the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f=2000 MHz (Figure 10.18.i-j). Here, although the passband center is at f0=1000 MHz, we must type in f=fq=2000 MHz to get a Tline which has the same length as the commensurate length of the filter. This approximation will lead to some distortion in response which can be corrected by tuning a few elements as shown in Figure 10.18.k. Thus, we have formed as Kuroda-Minnis CD section at source end (the triplet of Tline+Series SC Stub+Series OC Stub). This circuit is saved as mind02.fil, to be used in iterative trials. Now, we are ready for formation of Pi-sections of OC stubs between adjacent Tlines, as follows.

- Wit cursor under the Tline at source end, the command Kuroda-Minnis CD/ Right / k=0.5 is applied. This command splits the series SC and OC stubs and transfers them to the left of the Tline in such a way that we get a symmetric Pi-section of OC stubs on the left of the Tline, as shown in Figure 10.18.m. We also get a transformer and a piece of series OC stub on the right of the Tline. In applying this command, Filpro asks a constant k. This is the only parameter which should be specified by the user to produce different alternatives. After specifying a trial value of k=0.5 (for example), Filpro displays impedance of the shunt OC stubs and the new impedance of the Tline and asks if a new k value is tried or not. If these impedances are OK, then we select the option NO and we get the circuit shown in Figure 10.18.m. The resulting transformer is shifted rightmost by applying Shift TRF / Rightmost command.

- Then we pass to formation of a new Kuroda-Minnis CD section. This is done by placing the cursor under the second Tline and applying the command Kuroda / Left / Full Stub. This command transfers the series OC stub on the left of the Tline to its right. After moving the transformer to the load end, we get the new Kuroda-Minnis CD section shown in Figure 10.18.n.

- With cursor under the Tline of this section, the command Kuroda-Minnis CD / Right / k=0.5 is applied again, forming the second Pi-section of OC stubs (Figure 10.18.p).

- Then the third, fourth and fifth Pi-sections of OC stubs are formed by repeating the previous two steps, as shown in Figures 10.18.q-r-s-t-u-v.

- At the step of Figure 10.18.v, we have a series OC stub on the left of the sixth Tline while a series SC exists on its rigth. After moving away the transformer, we place the cursor under the sixth Tline and apply command Kuroda / Right / Full Stub. This command transfers the series SC stub on the right of Tline as shunt OC stub on the left, thus forming an LLeft section of OC stubs (Figure 10.18.w). It is then converted into a symmetric Pi-section of OC stubs by applying the command Pi-Tee-L / LLeft-to-Sym Pi (Figure 10.18.x).

- Symmetrize command maps all these circuit transformations done in the left half of the circuit to the right half, completing the circuit transformations (Figure 10.18.y). Inspection of the impedances of Tlines of this circuit shows that the selection of k=0.5 in all transformations leads to a decrease of Tline impedances towards the middle of the circuit, from 98 ohms to 6 ohms. We have to experiment with different k values to get roughly equal Tline impedances or reasonable shunt SC stub impedances.

- As the next trial, we have repeated the design by selecting k=0.45 for all stages. That trial gave Tline impedances decreasing from 107 ohms to 19 ohms (Figure 10.18.z1).

- Then we have tested for k=0.4 which now gave Tline impedances increasing from 118 ohms to 136 ohms, with the middle Tline having an impedance of 62.5 ohms (Figure 10.18.z2).

- As the final trial, k=0.42 is tested, giving Tline impedances varying in the range of 113-to-95 ohms, with the middle Tlines having 39 ohms impedance (Figure 10.18.z3). These values seem reasonable. By further trials it is possible to make all Tline impedances closer to each other except the middle Tline which is roughly half of the other Tline impedances.

- Next, the Pi-sections of OC stubs are transformed into OC comb-type coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By PCL / Combline / ococCOMB / Symmetric, as described in Figures 9.42.z-4. The even and odd mode impedances seem to be realizable.

- Further adjustments can be made by using the following tools:

- Changing the inverter constant K for setting the overall impedance level of the circuit.

- Replacing some or all OC stubs by lumped capacitors.

- Using Pi-to-Pi transformations on the Pi-sections of OC stubs.

- Figure 10.18.z5 shows a pictorial description.

- By replacing all the OC stubs with their lumped capacitor equivalents, we can form the Class-D Capacitively Loaded End Coupled Line Filter, as described in Figures 10.18.z6-z8.

Exercise: Repeat the design with different quarter wave frequencies fq to see variation of the element values with fq. Repeat the design for a wider bandwidth, for example an octave band filter.

Figure 10.18. Design of a degree 22 narrowband Class-D filter.

Figure 10.18. Design of a degree 22 narrowband Class-D filter (cont’d).

Figure 10.18. Design of a degree 22 narrowband Class-D filter (cont’d).

Figure 10.18. Design of a degree 22 narrowband Class-D filter (cont’d).

Figure 10.18. Design of a degree 22 narrowband Class-D filter (cont’d).

EXAMPLE 10. 12. A narrowband Class-D filter with odd NUE

Let’s synthesize a filter with odd number of unit elements, having the following specifications:

PBRipple=0.01 dB, fp1= 975 MHz, fp2=1025 MHz, fq=2000 MHz, Nzero=1, Ninf=6, NUE=5

- Design command set is shown in Figure 10.19. The extracted circuit is shown in Figure 10.19.a.

- In order to get physical symmetry, we place the cursor under the Tline (UE) in the middle of the circuit and apply Kuroda / Left / Equal Stub command. Then Shift TRF / Rightmost command is used to move the transformer to the load end where it is combined with the transformer there to form unity turns ratio, hence deleted (Figure 10.19.b-c).

- Our aim in the following transformations is to move the series OC stub in the left half circuit towards the source end by Kuroda transformations as follows.

- Exchange command is used to exchange the positions of the SC and the translated OC stub so that the OC stub will be adjacent to the Tline on its left (Figure 10.19.d).

- The command Kuroda / Right / Full Stub is repeated by placing the cursor under this Tline to move the OC stub one more step left. Then Shift TRF and Exchange commands are repeated.

- This way, we arrive at the topology shown in Figure 10.19.g where we have a series combination of a SC and an OC stub near the source. Symmetrize command maps the left half of the circuit to right. This circuit is saved as mind11.fil.

- Next, a redundant unit (50 ohms) element will be inserted at the source and load ends for adjusting element values. However, in order to get more flexibility for adjustments, instead of inserting redundant unit elements directly, we first apply command  Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=52 ohms (Figure 10.19.h) and then replace the ideal inverters by unit elements (quarter wave Tlines). Here, the inverter constant K=52 ohms is just a trial value. It can be iterated after inspecting the resultant element values. The case for K=50 ohms would correspond to insertion of redundant unit elements.

- The ideal inverters at source and load ends can be replaced by quarter wave Tlines by applying the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f=2000 MHz (Figure 10.19.h-i). Here, although the passband center is at f0=1000 MHz, we must type in f=fq=2000 MHz to get a Tline which has the same length as the commensurate length of the filter. This approximation will lead to some distortion in response which can be corrected by tuning a few elements as shown in Figure 10.19.k.

- Thus, we have formed as Kuroda-Minnis CD section at source end (the triplet of Tline+Series SC Stub+Series OC Stub). This circuit is saved as mind12.fil, to be used in iterative trials. Now, we are ready for formation of Pi-sections of OC stubs between adjacent Tlines, as follows.

- Wit cursor under the Tline at source end, the command Kuroda-Minnis CD/ Right / k=0.415 is applied. This command splits the series SC and OC stubs and transfers them to the left of the Tline in such a way that we get a symmetric Pi-section of OC stubs on the left of the Tline, as shown in Figure 10.19.m. We also get a transformer and a piece of series OC stub on the right of the Tline. In applying this command, Filpro asks a constant k. After specifying a trial value for k, Filpro displays impedance of the shunt OC stubs and the new impedance of the Tline and asks if a new k value is tried or not. If these impedances are OK, then we select the option NO and we get the circuit shown in Figure 10.19.m. The value k=0.415 is found after a few trials giving Tline impedances roughly in the same order. The resulting transformer is shifted rightmost by applying Shift TRF / Rightmost command.

- Then we pass to formation of a new Kuroda-Minnis CD section. This is done by placing the cursor under the second Tline and applying the command Kuroda / Left / Full Stub. This command transfers the series OC stub on the left of the Tline to its right. After moving the transformer to the load end, we get the new Kuroda-Minnis CD section shown in Figure 10.19.n.

- With cursor under the Tline of this section, the command Kuroda-Minnis CD / Right / k=0.415 is applied again, forming the second Pi-section of OC stubs (Figure 10.19.p).

- Then the third Pi-section of OC stubs is formed by repeating the previous two steps, as shown in Figures 10.19.q-r.

- At the step of Figure 10.19.r, we have a series OC stub on the left of the middle Tline. After moving away the transformer, we place the cursor under the middle Tline and apply Symmetrize command mapping the left half to the right half (Figure 10.19.s).

- The circuit is ready for conversion into coupled line form. First, the Pi-sections of OC stubs are transformed into OC comb-type coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By PCL / Combline / ococCOMB / Symmetric. Then with cursor under the middle Tline, the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to convert the series OC Stub+TLine+Series OC Stub triplet into interdigital type coupled line (Figures 10.19.s-t). The even and odd mode impedances seem to be realizable.

- Figure 10.19.u shows a pictorial description. The interdigital type coupled line in the middle leads to U-sections of the symmetric halves into opposite directions.

- Further adjustments can be made by using the following tools, as was done in the previous example.:

- Changing the inverter constant K for setting the overall impedance level of the circuit.

- Replacing some or all OC stubs by lumped capacitors.

- Using Pi-to-Pi transformations on the Pi-sections of OC stubs.

Figure 10.19. Class-D filter with odd number of unit elements.

Figure 10.19. Class-D filter with odd number of unit elements (cont’d).

Figure 10.19. Class-D filter with odd number of unit elements (cont’d).

  - NARROW-MEDIUM BANDWIDTH CLASS-F  FILTERS

Class-F filters are Class-E filters augmented by inserting redundant unit elements for impedance scaling purposes. They are suitable for medium narrow bandwidth filters. They are characterized by the same transmission zero pattern as Class-E filters. That is,

Nzer=1, Ninf=1, NUE=even

Let’s see some examples.

EXAMPLE 10. 13. A degree 22 narrowband Class-F Filter

Let PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, fq=2000 MHz, Nzero=Ninf=1, NUE=20  (N=22)

- Synthesis command set is shown in Figure 10.20. Standard Auto Extract command gives the circuit shown in Figure 10.20.a.

- The series OC and SC stubs in the symmetry plane of the circuit are split into two equal pieces and Exchange command is used to form the symmetric halves (Figure 10.20.b).

- A 50 ohms Redundant Unit Element is inserted at source end for impedance scaling purposes.

- Using the command Kuroda / Right / Full Stub repeatedly, first the series SC stub is translated to the source end as shunt OC stub (Figure 10.20.c).

- Then the series OC stub is translated to the right of the shunt OC stub by applying the command Kuroda / Right / Full Stub again repeatedly. The transformers resulting form these operations are shifted rightmost by applying Simplify Circuit command. Thus, an LRight section of OC stubs is formed at the source end (Figure 10.20.d). This circuit is saved as minf01.fil to be used in different design trials.

- The LRight section of OC stubs followed by two Tlines is termed as Kuroda-Minnis EF section. With cursor under the shunt OC stub of the LRight section and the command Kuroda-Minnis E/F / Right / Specify TLine-1 and TLine-2: Z1=80 ohms, Z2=80 ohms is applied. This command forms a symmetric Pi-section of OC stubs while the remaining portions are transferred to the right of the two TLines which now have the specified 80 ohms impedances (Figure 10.20.e). The 80 ohms for Tline impedances is found proper after a few trials. After clearing the transformer by applying Simplify Circuit command a new Kuroda-Minnis-EF section is formed (Figure 10.20.f-g).

- The command Kuroda-Minnis-EF / Right / Specify TLine-1 and TLine-2: Z1=80 ohms, Z2=80 ohms is applied again to form a new symmetric Pi-section of OC stubs while setting the impedances of the next two Tlines as 80 ohms (Figure 10.20.g-h).

- This procedure is repeated until the middle of the filter is reached (Figures 10.20.k). In the middle section, we still have an LRight section of OC stubs, but only one Tline exists on the right of the LRight section. This LRight section is converted into symmetric Pi section of OC stubs by placing the cursor under the shunt leg of the LRight section and applying the command Pi-Tee-L / LRight-to-Sym Pi (Figure 10.20.m). The transformer is shifted rightmost by applying Simplify Circuit command. Symmetrize command will complete the design. The circuit is saved as minf02.fil.

- Next, the Pi sections of OC stubs will be converted into coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By PCL / Combline / ococCOMB / Symmetric  The even and odd mode impedances of the coupled lines are shown in Figure 10.20.n.

- Figure 10.20.p shows a pictorial description. It is seen that the shunt OC stub impedances are too high. This leads to high values for even and odd mode impedances of some coupled line sections. We can repeat the design by changing some of the design parameters, like fq and PBRipple, degree, or select different impedances for Tlines until realizable impedances are obtained. However an easier way to overcome this problem may be replacement of such OC stubs by their lumped capacitor equivalents.

- In Figures 10.20.q-r all the stubs are replaced by their lumped equivalents. It is seen that bandwidth is widened which may be taken into account by repeating the design with a narrower bandwidth. Such filters may be called Class-F Capacitively Loaded End Coupled Line filter. Further adjustments can be carried out (for example to equate all shunt stubs) using Kuroda and Pi / Tee / L transformations.

Exercise: Redesign the filter with a higher fq to get a shorter version.

Exercise: Compare element values and lengths of Class-D and Class-F filters of the same degree, bandwidth, Ripple and the same fq.

Figure 10.20. Design of a degree 22 Class-F filter.

Figure 10.20. Design of a degree 22 Class-F filter (cont’d).

Figure 10.20. Design of a degree 22 Class-F filter (cont’d).

EXAMPLE 10. 14. A degree 12 narrowband Class-F Filter

Let’s design the previous filter with a lower degree but a higher fq:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, fq=3000 MHz, Nzero=Ninf=1, NUE=10  (N=12)

- Synthesis command set is shown in Figure 10.21. Standard Auto Extract command gives the circuit shown in Figure 10.21.a. The LLeft section of OC stubs is converted into a symmetric Pi-section by placing the cursor under the shunt leg and applying command Pi-Tee-L / LLeft-to-Sym Pi. The resulting transformer is shifted rightmost by applying Simplify Circuit command giving the symmetric circuit of Figure 10.21.b.

- A 50 ohms Redundant Unit Element (Tline) is inserted at source end for impedance scaling purposes. Symmetrize command is used to get the same redundant Unit Element at load end. Then the series OC stub in the symmetry plane of the circuit is split into two equal pieces to form two symmetric halves (Figure 10.21.c).

- Using the command Kuroda / Right / Full Stub repeatedly, first the series SC stub is translated to the source end as shunt OC stub (Figure 10.21.d). Then the series OC stub is translated to the right of the shunt OC stub by applying the command Kuroda / Right / Full Stub repeatedly again. The transformers resulting form these operations are shifted rightmost by applying Simplify Circuit command. Thus, an LRight section of OC stubs is formed at the source end (Figure 10.21.e).

- The LRight section of OC stubs followed by two Tlines is termed as Kuroda-Minnis EF section. With cursor under the shunt OC stub of the LRight section and the command Kuroda-Minnis E/F / Right / Specify TLine-1 and TLine-2: Z1=100 ohms, Z2=100 ohms is applied. This command forms a symmetric Pi-section of OC stubs while the remaining portions are transferred to the right of the two TLines which now have the specified 100 ohms impedances (Figure 10.21.f). The 100 ohms for Tline impedances are found proper after a few trials. After clearing the transformer by applying Simplify Circuit command a new Kuroda-Minnis-EF section is formed (Figure 10.21.g).

- The command Kuroda-Minnis-EF / Right / Specify TLine-1 and TLine-2: Z1=100 ohms, Z2=100 ohms is applied again to form a new symmetric Pi-section of OC stubs while setting the impedances of the next two Tlines to 100 ohms also. Simplify Circuit eliminates the transformer and a new Kuroda-Minnis EF section is formed (Figure 10.21.h).

- This procedure is repeated on the new Kuroda-Minnis EF section giving the circuit of Figure 10.21.i). In the middle section, we still have an LRight section of OC stubs. With cursor under the Tline on the right of this LRight section, Symmetrize command is applied and then this Tline is deleted to form a symmetric circuit with two series  OC stubs in the symmetry plane. Simplify Circuit command combines the two series stubs giving a Pi-section of OC stubs in the middle of the circuit as shown in Figure 10.21.j. Thus, we have obtained the targeted topology formed by Pi-sections of OC stubs separated by pairs of Tlines.

- Next, the Pi sections of OC stubs will be converted into coupled line sections by placing the cursor under the series OC stubs and applying the command Replace By PCL / Combline / ococCOMB / Symmetric  The even and odd mode impedances of the coupled lines are shown in Figure 10.21.k.

- Figure 10.21.m shows a pictorial description.

Exercise: Note down the variation of impedances of the shunt OC stub impedances of the Pi-sections of OC stubs are Figure 10.21.j. Repeat the design by selecting Tline impedances higher and lower than 100 ohms and inspect variation of shunt OC stub impedances. Repeat also the whole design by selecting higher fq.

Figure 10.21. Design of a degree 12 Class-F filter.

Figure 10.21. Design of a degree 12 Class-F filter.

10. 6. MINIATURE BANDPASS FILTERS WITH MULTIPLE  TRANSMISSION ZEROS  AT ZERO

In the previous applications we have used only a single transmission zero at f=0 to avoid appearance of SC stubs and SC coupled lines. This approach leads to serious constraints on element values and forms a limitation for possible alternative solutions.  If short circuited stubs and/or short circuited coupled lines are allowed, then the number of possible alternative solutions can be increased drastically by placing more than one transmission zeros at f=0. Some examples on such filters are given by S. Herbert 23 using only redundant unit elements. In the following examples filters will be designed using mostly contributing unit elements, disproving the common belief that filters with contributing unit elements are usually unrealizable. On the contrary, in some cases filters with contributing unit elements are easier to realize than those with redundant unit elements.

EXAMPLE 10. 15. Wideband stripline bandpass filters with contributing and redundant unit elements having multiple transmission zeros at f=0

Let 	PBRipple=0.1 dB,  fp1=2000 MHz,   fp2=6000 MHz,  fq=12000 MHz,  Nzero=Ninf=5

and consider three filters with

a)  NUE = 0   (all redundant UEs)

b)  NUE = 4   (4 nonredundant UEs)

c)  NUE = 6   (6 nonredundant Ues)

a) Solution for  NUE = 0:

- In Figure 10.22.a the circuit synthesized for NUE = 0 case is shown.

- Two redundant UE’s are inserted on source side (Figure 10.22.b).

- The middle elements are split equally to produce two symmetric halves (Figure 10.22.c).

- The two redundant UEs are moved into the circuit using Kuroda-Right-Full Stub command as shown in Figure 10.22.c-d. Transformers resulting from Kuroda identities are shifted rightmost.

- With cursor under the UE on left, the command Kuroda Pi / Left / Symmetric Pi is applied. This command translates the series arm OC and SC stub pair (stub resonator) on the left of the UE as a symmetric Pi-section of OC stubs on the right of UE. Resulting transformer is shifted rightmost (Figure 10.22.e).

- Then cursor is moved to point the second UE and command  Kuroda Pi /Right / Symmetric Pi is applied translating the shunt SC and OC stub pair on the right of the Unit Element to its left as a symmetric Pi-section of SC stubs as shown in Figure  10.22.f. The circuit is made symmetric to avoid repetition of the operations done on the left half (Figure 10.22.g).

- Then with cursor under the middle element of OC stub Pi-section, the command Replace By PCL / CombLines / ococComb / Symmetric  is applied converting Pi-section into a COMB section with OC terminations. Similarly, the SC stub Pi section is converted to scscCOMB type parallel coupled line  (Comb section with SC terminations) using command the command Replace By PCL / CombLines / scscComb / Symmetric (Figure 10.22.h).

- Finally, in order to adjust impedance levels of comb sections and the UE in the middle, the shunt SC stub on source side is moved to be adjacent to the first Unit Element on left by using Exchange command (Figure 10.22.h-i).  Then the cursor is placed under this UE and the SC stub on its left is moved to its right by applying the command Kuroda / Left / Full Stub producing also a transformer. Shifting the transformer rightmost gives the element values shown in Figure 10.22.j which seems realizable in microstrip form. The final circuit can be made symmetric using Symmetrize command.

- Figure 10.22.k shows a pictorial description of the filter.

Exercise: Repeat the design for different PBRipples to get an idea on variation of element values.

Figure 10.22. Design of a filter with Nzero=5, Ninf=5 and 4 redundant Unit Elements.

Figure 10.22. Design of a filter with Nzero=5, Ninf=5 and 4 redundant Unit Elements (cont’d).

b) Solution for  NUE=4:

- We can get the same topology as the previous example by using contributing Unit Elements instead of redundant Unit Elements. This will help to improve the response by increasing the slopes of insertion loss.

- Figure 10.23 shows design steps with Nzero=5, Ninf=5 and NUE=4. One extraction order is shown in Figure 10.23.a.

- In this circuit, first the two LLeft sections of OC stubs are converted to symmetric Pi-sections by placing the cursor under the shunt OC stubs and applying the command Pi-Tee-L / Lleft-to- Sym Pi. The resulting transformers are shifted to rightmost position by applying Simplify Circuit command giving the circuit of Figure 10.23.b. This command combines also the adjacent OC and SC stubs to form OC+SC stub elements.

- In the symmetry plane we have a shunt OC+SC Stub element. We split it into two parts by applying the command Split / Two Equal to form two symmetric halves of the circuit (Figure 10.23.c).

- Then, with cursor under the Unit Element on the left of the symmetry plane, the command Kuroda Pi / Right / Symmetric Pi is applied to get a symmetric Pi-section of SC stubs on the left of this Unit Element. Similarly, with cursor under the Unit Element on the right of the symmetry plane we apply the command Kuroda Pi / Left / Symmetric Pi  to form a Pi-section of SC stubs on the right of this Unit Element (Figure 10.23.d).

- The resulting two transformers are combined. Because of the symmetry, they form a unity transformer, hence deleted (Figure 10.23.e).

- Next, the Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric. Similarly, the Pi-sections of SC stubs are converted into SC comb type coupled lines by applying the command Replace By PCL / Combline / scscCOMB / Symmetric. Symmetrize command is used to map the left half of the circuit into right (Figure 10.23.e-f).

- It is seen that the scscCOMB type coupled lines have too high even and odd mode impedances. In order to scale down them cursor is placed under the first UE and Kuroda / Left / Equal Stub command is applied to transfer part of the shunt SC stub from left of the UE to right to produce a transformer. Shifting transformer rightmost we get the circuit of Figure 10.23.g.

- Figure 10.23.h shows a pictorial description of the new filter with non-redundant Unit Elements while in Figure 10.23.i the filter of the previous design with redundant Unit Elements is repeated for comparison. It is seen that we have almost the same topology and element values are close to each other. Considering the improvement of skirt slope, we conclude that it is wise to use non-redundant Unit Elements instead of redundant Unit Elements.

Exercise: Repeat the design for different PBRipples to get an idea on variation of element values.

Figure 10.23. Filter with Nzero=5, Ninf=5 and with 4 contributing UE (NUE=4).

Figure 10.23. Filter with Nzero=5, Ninf=5 and NUE=4 (cont’d).

c) Solution for  NUE=6:

Let’s now see if we can get further advantages by using even more non-redundant Unit Elements.

- In Figure 10.24.a, the circuit synthesized with NUE=6 is shown. Element extraction order shown in the figure is found proper after a few trials.

- With cursor under the second Unit Element from left, the command Kuroda-Pi / Right / Sym Pi is applied to transfer the series OC and SC stubs on the right of the Unit Element to left as a Pi-section of OC stubs. Also the LRight section of SC stubs in the middle is converted into symmetric Pi section by applying the command Pi-Tee-L / LRight-to-Sym Pi. After shifting the  transformers and applying Symmetrize command we get the circuit of Figure 10.24.b.

- It is seen that the third Unit Element from left has too low impedance. It can be scaled up by first converting the Pi-section of SC stubs in the middle into a Tee-section by applying the command Pi-Tee-L / Pi-to-Pi-Tee-L / Pi-to-Tee (Figure 10.24.c). Then with cursor under the Unit Element adjacent to the series arms of the Tee-section, the command Kuroda / Right (Left) / Full Stub is applied to convert the series SC stubs into shunt OC stubs on the other sides of the respective Unit Elements (Figure 10.24.d-e). Now we have more reasonable impedances.

- Next, the Pi- sections of open circuited stubs are replaced by their coupled line equivalents using the command Replace By PCL / Combline / ococComb / Symmetric (Figure 10.24.e-f).

- If desired, the low impedance shunt OC and SC stubs can be split into two to double their impedances (Figure 10.24.g).

- Figure 10.24.h shows a pictorial description of the final structure.

- In Figure 10.24.i insertion losses of all three filters are overplotted for comparison. The advantage of filter with NUE=6 is clear.

Exercise: Repeat the design for different PBRipples to get an idea on variation of element values.

Exercise: Design the filter by using  two contributing unit elements (NUE=2).

Figure 10.24. Filter with Nzero=5, Ninf=5 and NUE=6.

Figure 10.24. Filter with Nzero=5, Ninf=5 and NUE=6 (cont’d).

EXAMPLE 10. 16: Narrowband Filters with contributing and redundant unit elements.

Narrowband filters usually require insertion of redundant unit elements for impedance adjustment purposes. Let’s design some narrowband filter with following specifications:

PBRipple = 0.01 dB,  fp1=975  MHz,   fp2=1025 MHz,  fq=2000 MHz,   Nzero=Ninf =5 and	for the cases of  NUE=0, NUE=4 and  NUE=6.

Solution for NUE= 0 case is described below.

- In Figure 10.25, the command set for synthesis is shown. One possible extraction order gives the circuit shown in Figure 10.25.a.

- Two redundant UE’s are inserted from source side (Figure 10.25.b).

- The first shunt OC stub is moved to source end by applying command Kuroda/Right / Full Stub (Figure 10.25.c).

- The first shunt SC stub is moved to the right of the first UE by Kuroda / Right / Full Stub command. The resulting transformer is shifted rightmost (Figure 10.25.d-e).

- The first series SC stub is moved to the left of the second UE by Kuroda / Right / Full Stub command. Thus we have shunt OC and SC stubs on the right of the first UE (Figure 10.25.f).

- With cursor under the first UE, the command Kuroda-Pi / Right / Symmetric Pi is applied. This command transfers the shunt OC and SC stubs on the right of the UE as a symmetric Pi-section of SC stubs on left, producing also a transformer which is shifted rightmost (Figure 10.25.g).

- The Pi-section of SC stubs is converted into SC comb type coupled lines by applying the command Replace By PCL / Combline / scscCOMB / Symmetric. Also the series OC stub on the right of the second UE is transferred to the left of the UE by applying Kuroda / Right / Full Stub command. Simplify Circuit command moves away the transformer and also combines the adjacent shunt SC and OC stubs in the middle of the circuit to form a shunt parallel SC+OC stub element on the symmetry axis (Figure 10.25.h).

- With cursor under the first UE, the command Kuroda / Right / Equal Stub is applied to place equal series OC stubs on the two sides of this UE (Figure 10.25.i).

- Then with cursor under this UE, the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to form an interdigital OC coupled line. Symmetrize command is used to map the operations done on left half of the circuit to right half. Then the OC+SC stub is split into two equal parts (Figure 10.25.k).

- With cursor under the second UE, the command Kuroda-Pi / Right / Symmetric Pi is applied to transfer the OC+SC stub element on right to left as a Pi-section of SC stubs. The symmetric counterpart is formed by applying the command Kuroda-Pi / Left / Symmetric Pi. Simplify Circuit command eliminates the transformers (Figure 10.25.m).

- Finally the Pi-sections of SC stubs are converted into SC comb type coupled lines by applying the command Replace By PCL / Combline / scscCOMB / Symmetric  (Figure 10.25.n).

- Figure 10.25.p shows a pictorial description. Element values seem to be realizable. However undesired couplings may be problematic.

Figure 10.25. Narrowband filter with Nzero=Ninf=5 and with four redundant Unit Elements.

Figure 10.25. Narrowband filter with Nzero=Ninf=5 and with four redundant Unit Elements.

Exercise : Show that the same filter (with NUE = 0) can also be realized as shown in Figure 10.26.

Figure 10.26. Another realization for NUE = 0

Exercise : Show that the circuit for NUE = 4 can be synthesized as shown in Figure 10.27.a and it can be transformed into the form of Figure 10.27.b (with two redundant UEs).

Figure 10.27. A realization for BPF with NUE = 4

Exercise: Show that the circuit for NUE=6 can be synthesized as shown in Figure 10.28.a and realized as in Figure 10.28.b by inserting one redundant UE from source side.

Figure 10.28.a) Synthesized circuit for NUE=6. b) Realization for NUE = 6.

Exercise: Synthesize the same filters with NUE=8-to-14. See that usually for narrowband filters a redundant UE is needed at each port.

Exercise:  (Ultra wideband filters)

Let  PBRipple=0.1 dB,  fp1=2000 MHz, fp2=18000 MHz,  fq=25000 MHz, Nzero=Ninf=5 and  NUE = 2, 4, 6, ..... ,12. Show that the circuits of Figure 10.29 can be obtained as possible solutions (Hint: Some UEs may be redundant.)

Figure 10.29. Some ultra wideband filters.

REFERENCES  AND FURTHER READING:

- H. Ozaki, and  J. Ishii, “Synthesis of a Class of Stripline Filters”, IRE. Trans. Circuit Theory, Vol. CT-5, pp.104 - 109, 1958.

- R. J. Wenzel, “Exact Design of TEM Microwave Networks Using Quarter-Wave Lines”, IEEE Trans. MTT,  Vol. MTT-12, pp. 94 - 111, Jan. 1964.

- M. C. Horton and R. J. Wenzel, “General Theory and Design of Optimum Quarter Wave TEM filters”, IEEE Trans. MTT,  vol. MTT-13, No. 3, pp. 316 - 327, May 1965.

- R. Levy, “A General Equivalent Circuit Transformation for Distributed Networks”, IEEE Trans. CT, Vol. CT-12, No 3,  pp. 457-458, Sept. 1965.

- R. J. Wenzel, “Theoretical and practical applications of Capacitance matrix transformations to TEM Network Design”, IEEE Trans. MTT, vol. MTT-14, No. 12, pp. 635 - 647, Dec. 1966.

- R. Levy, “ A New Class of Distributed Prototype Filters with applications to Mixed Lumped / Distributed Components”, IEEE Trans. MTT, Vol. MTT-18, pp. 1064 - 1071, Dec. 1970.

- R. J. Wenzel, “Synthesis of Comb-line and Capacitively Loaded Interdigital Bandpass Filters of Arbitrary Bandwidth”, IEEE Trans. MTT, vol. MTT-19, No. 8, pp. 678 - 686, Aug. 1971.

- G. Hoffmann, “ The Direct Synthesis of Optimum Commensurate Distributed Filters by Digital Computer”, 1973 European Microwave Conference Proc., B.2.2, Brussels, 1973.

- G. Reiter and G. Hammer, “ Stripline Filters with Coupled / 8 Line Sections”, 1973 European Microwave Conference, B.3.2, Brussels, 1973

- R. Levy, “A Generalized Design Technique for Practical Distributed Reciprocal Ladder Networks”, IEEE Trans. MTT, vol. MTT- 21, No. 8, pp. 519 - 526, Aug. 1973.

- G. Reiter, “ Stripline Filters with Lumped Capacitors”, 5. European Microwave Conference Proc., pp. 426 - 430. 1975.

- J. D. Rhodes, “Theory of Electrical Filters”, London, England, Wiley, 1976.

- M. C. Horton, “ TEM Bandpass Filters Having Eight Wavelength Coupled Stubs”, 1977 IEEE, MTT- S Int. Microwave Symposium Digest, San Diego, 1977.

- J. A. G. Malherbe, “Microwave Transmission Line Filters”, Artech House, 1980.

- M. Makimoto, S. Yamashita, “Bandpass Filters Using Parallel Coupled Stripline Stepped Impedance Resonators”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-28, No. 12, pp. 1413-1417, Dec. 1980.

- J. B. Minnis, “Classes of Sub Miniature Microwave Printed Circuit Filters with Arbitrary Passband and Stopband Widths”, IEEE Trans. MTT, Vol. MTT-30,  pp. 1893 - 1900,  Nov. 1982.

- C. I. Mobbs and J. D. Rhodes, “A Generalized Chebychev Suspended Stripline Bandpass Filter”, IEEE Trans. MTT, Vol. MTT- 31,  pp. 397- 402,  May 1983.

- B. Rawat, R. Miller, and B. Pontius, “Bandpass filter for Mobile Communications”, Microwave Journal, Vol. 27, No. 10, pp. 146-150, 1984.

- A. Riddle, “ High Performance Parallel Coupled Line Microstrip Filters”, 1988 IEEE, MTT-S Conference Proc., pp. 427 - 430, 1988.

- M. Gat, “ Commensurate Line Microstrip Bandpass Filters”, 1988 IEEE, MTT-S Conference Proc., pp. 423- 426, 1988.

- Y. Sen, “Direct Design of  Lumped and Distributed Filters Using Transformed Variable Techniques”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1989.

- J. H. Kotze, “Classes of Bandpass Filters Having an Arbitrary wide Stopband  and Design Tables for One Such Class”, IEEE MTT-S, pp. 123 - 126, 1990.

- S. Herbert, “Procedure Produces Miniature MIC bandpass filters”, Microwaves and RF, pp. 203-209, Apr. 1990.

- M. M. Akkul, “Design and Realization of Microwave Bandpass Filters in Stripline and Suspended Stripline Structures”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1991.

- I. Tekin, “Design and Realization of Narrowband Microwave Bandpass Filters Using Stripline-like Structures”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1992.

- C. M. Tsai and K. C. Gupta, “A Generalized Model for Coupled Lines and its applications to Two-Layer Planar Circuits”, IEEE Trans. MTT, Vol. MTT- 40,  pp. 2190 - 2199, Dec. 1992.

- J. C. Tippet and M. J. Shian, “Circuit Transformations for realization of a Class of  Miniature  Printed Circuits”, IEEE,  MTT-S  Digest, pp. 621- 624, 1994.

- U. Koyaz, “Synthesis of Lumped and Distributed Filters and Diplexers”, Ph. D. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1994.

- D. Zhang, G. C. Liang, C. F. Shih, and R. S. Withers, “Narrowband Lumped Element Microstrip Filters Using Capacitively Loaded Inductors”, 1995 IEEE  MTT-S  Digest, pp. 379- 382, 1995.

- A. F. Sheta, K. Hettak, J. Coupez, C. Person, and S.Toutain, J.P. Blot, “A New Semi-Lumped microwave Filter Structure”,  1995  IEEE MTT-S Digest, pp. 383 - 386, 1995.