CHAPTER-11

MINIATURE

ELLIPTIC

DISTRIBUTED ELEMENT HIGHPASS AND BANDPASS FILTERS FOR

STRIPLINE-LIKE STRUCTURES

CONTENT                                                                                                             	 Page

11. 1. INTRODUCTION								11. 2

11. 2.  MEDIUM-WIDE BAND DISTRIBUTED  ELEMENT  HIGHPASS

FILTERS  WITH  FINITE TRANSMISSION  ZEROS			11. 7

11. 3.  MEDIUM-WIDE BAND DISTRIBUTED ELEMENT BANDPASS FILTERS

WITH FINITE TRANSMISSION ZEROS					11.29

11. 4.  NARROWBAND HIGHPASS AND BANDPASS FILTERS WITH FINITE TRANSMISSION ZEROS							11. 65

11. 5.  DISTRIBUTED ELEMENT BANDPASS FILTER DESIGN USING                APPROXIMATE    TECHNIQUES					11.113

REFERENCES AND FURTHER READING:					11.119

CHAPTER 11

ELLIPTIC

DISTRIBUTED ELEMENT HIGHPASS AND BANDPASS FILTERS:

ELLIPTIC  BANDPASS  FILTERS  FOR STRIPLINE-LIKE STRUCTURES

11. 1. INTRODUCTION

Distributed element bandpass filters with finite transmission zeros (Elliptic-like filters) involving contributing unit elements are optimal structures which combines miniaturization and selectivity. The first systematic approach towards realization of such filters is presented by Minnis [12]. A complete presentation of the subject will be given in this chapter, extending the works of Minnis.

As it was described in the previous chapters, commensurate length distributed element highpass filters can be treated as special cases of commensurate length distributed element bandpass filters. Therefore, a summary of the basic properties of highpass and bandpass filters are reviewed below.

Using equal (commensurate) length stubs and Unit Elements as transmission zero producing elements, it is possible to synthesize distributed element highpass filters and distributed element bandpass filters in S=+j domain (see Chapter-7). The highpass and bandpass characteristics obtained on S=j axis are mapped to the s=jw axis  through the mapping function (Richards transformation)

f=tan(f/2fq)

where fq is the quarter wavelength frequency corresponding to the commensurate length selected by the designer:

fq=c4Lεr

with L being the physical length, c being the velocity of light and r being the relative effective dielectric constant of the medium.

- The function f maps the entire range 0  into finite periodical ranges on the real frequency  f-axis, with a period of 2fq, as follows:

  - =0 is mapped to f=0, 2fq, 4fq, 6fq, .. .

  - = is mapped to f=fq, 3fq, 5fq, .. .

This mapping results in production of infinitely many and periodic passbands and stopbands, We are usually interested in the response within the frequency range 0f2fq. Let’s inspect responses obtained on f-axis by mapping the S-domain highpass and bandpass filters separately.

Distributed Element Highpass Filters:

Distributed element highpass filters are synthesized in S=+j domain by using the following  transmission zeros:

    - Transmission zeros at =0 which are realized either as series OC stubs or shunt SC stubs.

    - Transmission zeros at S==1 which are realized as Unit Elements (transmission line pieces with same length as the stubs).

    - Some finite transmission zeros can be used to increase selectivity which are realized as stepped stubs.

Figure 11.1.a shows a typical distributed element highpass filter response on -axis and f-axis.

- The top figure is a typical highpass response with passband edge at p and having a finite transmission zero at z and some transmission zeros at =0. The response on f-axis is calculated from the highpass characteristics through the mapping function) f=tan(f/2fq). Close inspection of this mapping reveals the following properties within the frequency range 0f2fq:

  - The transmission zeros at at =0 are mapped to both f=0 and f=2fq.

  - The finite transmission zeros at =z are mapped to both  f=fz and f=2fq-fz.

  - The passband edge at =p is mapped to both f=fp and f=2fq-fp.

  - The frequency = is mapped to f=fq.

  - Thus, we notice that the highpass characteristics seen on the entire 0 range is squeezed into the range 0ffq. So, the response in the frequency range 0ffq is of highpass filter type. However the mirror image of this response is also mapped into the range fqf2fq. That is, within the frequency range 0 f 2fq, instead of a highpass response we get a symmetric bandpass response with passband edges being fp1=fp, fp2=2fq-fp and passband center being f0=fq. Since fq=(fp1+fp2)/2, the passband center f0=fq is usually termed as arithmetic center of the passband.

  - The stopband 0p of the domain highpass filter characteristics is mapped to both the range 0ffp and 2fq-fpf2fq. Thus, the frequency ranges 0ffp and        2fq-fpf2fq can be treated as lower and upper stopbands of the bandpass response within the range 0f2fq.

- If we consider the whole response on f-axis, it is seen that the upper stopband extends from 2fq-fp to 2fq+fp where the next passband starts which is centered at 3f0=3fq.

- Thus, a distributed element highpass filter need be treated as a quasi-bandpass filter with main passband centered at f0=fq and the others (named as spurious passbands) centered at 3fq, 5fq, . . Usually only the nearest spurious passband at f=3f0 is of concern.

- Such filters are among the simplest and most commonly used bandpass filters in applications where the existence of the spurious passband at 3f0 is not a problem.

- The basic limitations of distributed element highpass filters are now clear:

- Since in highpass filter approach the passband center f0 is the same as the quarter wave frequency fq, length of stubs and Unit Elements (the commensurate length) is fixed by the desired passband center.

- There is no chance of moving the nearest spurious passband away. It must be eliminated by some other means like cascading lowpass filters. If this is a problem, then we have to resort to using distributed element bandpass filter approach instead of distributed element highpass filter approach.

- Since fq sets the length of stubs and Unit Elements, physical size of the filter is fixed by the center frequency f0=fq. If miniaturization is needed then one should use distributed element bandpass filter approach instead of distributed element highpass filter approach.

- Response is symmetric about the passband center f0=fq. This is because the transmission zeros at =0 are mapped to f=0 and f=2fq, and any finite transmission zero at =z  is mapped symmetrically to fz and 2fq-fz.  This means we don’t have any flexibility to form an asymmetric response by adjusting the positions of finite transmission zeros in lower and upper stopbands or by placing different number of transmission zeros at f=0 and f=2fq. If asymmetric response is needed than one should use distributed element bandpass filter approach, as will be described below.

Distributed Element Bandpass Filters:

Distributed element bandpass filters are synthesized in S=+j domain by using the following  transmission zeros:

    - Transmission zeros at =0 which are realized either as series OC stubs or shunt SC stubs.

    - Transmission zeros at = which are realized either as series SC stubs or shunt OC stubs.

    - Transmission zeros at S==1 which are realized as Unit Elements (transmission line pieces with same length as the stubs).

    - Some finite transmission zeros in lower and upper stopbands to increase selectivity which are usually realized as stepped stubs (or some other circuit sections like cross-couplings).

The upper picture of Figure 11.1.b shows a typical distributed element bandpass filter response on -axis, with passband edges at p1 and p2a finite transmission zero in lower stopband, at z1, a finite transmission zero in upper stopband, at  z2, and some transmission zeros at =.

The lower plot shows insertion loss response on f-axis obtained through the mapping function

f=tan(f/2fq)

- The function =tan(f/2fq)  maps the range 0  to the range 0ffq. So, we have the desired bandpass response within the range 0ffq.

- We also have mirror image of the response of the range 0ffq about fq, in the range fqf2fq, leading to another passband. So, within one period we have two passbands, located in the ranges fp1<f<fp2 and 2fq-fp2<f<2fq-fp1, with the second one being the nearest spurious passband.

- In the distributed element highpass filters the passband center f0 was defined as the arithmetic man of the passband edges f0=(fp1+fp2)/2 while in bandpass filters obtained by mapping from lowpass prototypes the passband center f0 is defined as the geometric mean of the passband edges (f02=fp1fp2). Contrary to those cases, passband center concept is not defined in the filters synthesized by placement of transmission zeros. Therefore whenever the passband center concept need be referred, we can use either arithmetic mean or geometric mean of the passband edges fp1 and fp2.

- The finite transmission zeros in lower and upper stopbands are placed independently of each other. Also the numbers of transmission zeros at =0 and = are independent of each other. These features enable the designer to form asymmetric responses, hence form an advantage of distributed element bandpass filters over distributed element highpass filters.

- Since the quarter wave frequency fq is not related to fp1 and fp2, infinitely many bandpass filters can be synthesized having the same passband characteristics but having different fq (hence different commensurate length. This is a big advantage which enables the designer to have control on physical size of stubs and hence the overall size of the filter. For example selection of a high fq leads to size miniaturization.

- Selection of high fq leads also widening of the upper stopband which is located within the range fp2<f<fq+fp1, thus moving the first spurious passband way from the main passband. For the cases fq>2f0 where f0 is defined as arithmetic mean of the passband edges, the spurious passband moves away from the main passband while for fq<2f0 the spurious passband gets closer to the main passband.

The responses for case of fq=2f0 coincide with that of the highpass filter, if the finite transmission zeros are symmetrically located and if the numbers of transmission zeros at =0 and = are equal. Therefore the highpass filters can be treated as special cases of bandpass filters.

by making use of the link between S=+j domain highpass and bandpass filters, we can get some of the feasible topologies of bandpass filters from highpass filters by simple modifications and generalizations. Then further variations of bandpass filters can be obtained by placing different numbers of transmission zeros at The common properties of the bandpass filters designed as distributed element highpass filters can be summarized as follows:

- Figure 11.1.a shows insertion loss response of a distributed element highpass filter on axis and f-axis. The top figure is a typical highpass response with passband edge at p and having a finite transmission zero at z and some transmission zeros at =0. The response on f-axis is calculated from the highpass characteristics through the mapping function (Richards transformation)  =tan(f/2fq). This function maps a transmission zero at =0 to f=0, 2fq, 4fq, 6fq, .. .

- The stopband 0<p of the domain highpass filter characteristics is mapped to the range 0<f<fp and 2fq-fp<2fq within 0<f<2fq. The passband p< of the domain highpass filter is mapped into to the range fp<f<fq and also into the range fq<f<2fq-fp. Thus a passband is obtained in the range f=fp to f=2fq-fp with arithmetic symmetry about the passband center f0=fq.

- A finite transmission zero at z is mapped to both f=fz and f=2fq-fz and a transmission zero at =0 will be mapped to both f=0 and f=2fq. Thus, we have full arithmetic (mirror image) symmetry about the passband center f0=fq.

- The response between 0<f<2fq is repeated on f-axis with a period of 2fq. This periodicity leads to  infinitely many spurious passbands and stopbands. Usually the nearest spurious passband is important in practice. It is located as f=3f0=3fq. Since in highpass filter approach the passband center f0 is the same as the quarter wave frequency fq, the spurious passband can’t be moved away. It must be eliminated by some other means like cascading other filters, or distributed element bandpass filter approach can be used.

- Figure 11.1.b shows insertion loss responses of a distributed element bandpass filter on -axis and f-axis.  Note that in this approach the passband center f0 and quarter wave frequency fq are unrelated.

- The function =tan(f/2fq)  maps the range 0< < to the range 0<f<fq. We also have mirror image about fq. Response is periodic with a period of 2fq.  So, within one period we have two passbands, located in the ranges fp1<f<fp2 and 2fq-fp2<f<2fq-fp1, with the second one being the nearest spurious passband. One advantage of the bandpass filter approach is that since fq can be selected independently of the passband center, we can select fq high to push the nearest spurious passband away from the main passband. For the cases fq>2f0, the spurious passband moves away from the main passband while for fq<2f0 the spurious passband gets closer to the main passband. The responses for case of fq=2f0 coincide with that of the highpass filter. Therefore the highpass filters can be treated as special cases of bandpass filters.  Selection of high fq (fq>2f0) leads also size reduction because the quarter wave length will get shorter.

- Since the number of transmission zeros at  =0 and = can be placed independently of each other response need not be symmetric. Further asymmetry can be obtained by placing the finite transmission zeros in lower and upper stopbands at asymmetric positions and/or with different multiplicities.

In this chapter distributed element highpass and bandpass filters having finite transmission zeros will be described. Some of these filters are classical while some of them new.

Figure 11.1. Insertion loss responses of distributed element highpass and bandpass filters.

MEDIUM-WIDE BAND DISTRIBUTED ELEMENT HIGHPASS  FILTERS  WITH  FINITE TRANSMISSION  ZEROS

11. 2.  MEDIUM-WIDE BAND DISTRIBUTED ELEMENT HIGHPASS  FILTERS  WITH  FINITE TRANSMISSION  ZEROS

In Filpro, the usual approach for design of distributed highpass filters with finite transmission zeros is Design / By Synthesis technique. If physically symmetric structures are targeted then we place an odd number transmission zeros at f=0 (Nzero=Odd). If SC stubs are not desirable then we have to place only a single transmission zero at f=0. Since a single transmission zero at f=0 may not provide enough selectivity on low side of the passband, selectivity will be mainly set by the number of Unit Elements (NUE) and the position and number finite transmission zeros (NFTZ). We will concentrate on filters with a single transmission zero at f=0.

Figure 11.2 and 11.3 show typical highpass filter structures with one and three transmission zeros at f=0 obtained by Synthesis approach of Filpro. The shunt arm series OC+SC stub pairs create finite transmission zeros. They are always converted to two-step resonators (two-step shunt OC stubs) because they can’t be realized as they are. The series OC stubs form a single transmission zero at f=0. They have to be realized in coupled line form together with the adjacent Unit Elements. Therefore the minimum number of Unit Elements is NUE=NFTZ+1. The requirement for physical separation of resonators may also increase the number of contributing unit elements. Variations of these prototype topologies can be produced by increasing the number of unit elements (NUE) and number of transmission zeros at f=0 (Nzero). However for most applications these two prototypes yield realizable structures with satisfactory responses.

In distributed highpass filters the passband center f0 coincides with quarter wave frequency fq of the filter. The nearest spurious passbands is about 3f0. If this spurious passband is not desired, then, instead of highpass filter approach, we can use distributed element bandpass filters, enabling us to set fq2f0 to get wider stopbands together with smaller physical sizes, as it will be described in succeeding sections of this chapter. Let’s continue discussions on some examples.

EXAMPLE 11. 1: Some highpass filters with NUE=2 and NFTZ=1

Let’s design some medium-wide band highpass filters with a single finite transmission zero and two Unit Elements (NUE=2). We will consider pass bandwidths of 750 MHz  f 1250 MHz and 500 MHz < f <1500 MHz. Thus, for the highpass filters we have fq=1000 MHz and fp=750 MHz in one case and fp=500 MHz in the other case. SC stubs are not desired. Therefore we will place only one transmission zero at f=0 (Nzero=1). We will test the effects of passband ripple on element values by specifying it as 0.01 dB and 0.1 dB. Position of the finite transmission zero will be changed to see its effects on element values.

- Design steps are shown in Figure 11.2 for the case of PBRipple=0.01 dB, fp=750 MHz and FTZ=600 MHz. Filter is designed using  Synthesis approach. Note that the single FTZ at 600 MHz is also mapped to the symmetric position, at 2fq-FTZ=2000-600=1400 MHz. The first spurious passband is centered at 3fq=3f0=3000 MHz and extends from 2750 MHz to 3250 MHz.

- Element extraction order is ue-FTZ-ue-zero giving the structure shown in Figure 11.2.a.

- With cursor under the Tlines (Unit Elements), the command Kuroda / Distribute Stub is applied to place equal stubs on the two sides of the Tlines as a preparation for conversion into parallel coupled line form. {We could as well use the command Kuroda / Right / Equal Stub command). The shunt arm OC+SC Stub element is converted to two-step resonator using command StepResonator (Figure 11.2.b). The circuit is saved as A21A.fil for use at later stages.

- Then with cursor under the Tlines, the stub-TLine-stub triplets are converted to interdigital type OC coupled line form by applying the command Replace By PCL / Interdigital Lines / ococINTD / Symmetric giving the circuit of Figure 11.2.c. The circuit is saved as A21B.fil.

- Figure 11.2.d shows a pictorial description of a possible realization. The black and white lines may denote the two faces of a stripline structure (if broadside coupled line realization is used, because coupling seems too tight for side coupled realization). In this circuit we note down the even and odd mode impedance ratio, Zeven/Zodd=150/64. This ratio is an indication of the degree of coupling. If it increases coupling gets tighter.

- It is seen that on the symmetry plane we have a complex geometry which may lead undesired interactions between the shunt resonators and adjacent coupled lines and response degradation due to discontinuities.

- If some physical separation is required between coupled lines and shunt resonator, the interdigital coupled lines may be replaced by comb type coupled lines and Tlines as follows.

- The circuit A21B.fil is loaded (Figure 11.2.e). The cursor is placed under the coupled lines and the command Half Angle is applied. This command replaces the interdigital coupled lines with a Pi-section of OC stubs in between two Tlines, all having Half of the original commensurate length (Figure 11.2.f).

- Then with cursor under the series OC stub of the Pi-sections of OC stubs, the command Replace By PCL / Combline / ococCOMB / Symmetric is applied converting the Pi-sections into OC comb type coupled lines (Figure 11.2.g).

- Figure 11.2.h shows a pictorial description which is neater for construction. However comb type coupled lines are more tightly coupled compared to the original interdigital type coupled lines (Zeven/Zodd=251/45.8). This is because the same amount of energy coupling will be carried out along a shorter length (halved length). It is seen that the shunt two-step resonator has still the original length.

- Figure 11.2.i shows the design for PBRipple=0.1 dB. It is seen that the impedance level of the coupled lines is almost doubled while the impedances of the two-step resonator didn’t changed much. Compared to the original even-odd mode impedance ratio of Zeven/Zodd=150/64, we have a better ratio of Zeven/Zodd=201/104. That is, the degree of coupling is reduced.

- Figure 11.2.j shows the design for PBRipple=0.1 dB and the finite transmission zero FTZ=fq/2=500 MHz. For this special position of the FTZ (half of fq), impedances of the shunt two-step resonator become equal, thus the stepped resonator becomes a uniform OC stub.

- Figure 11.2.k shows the design for a wideband filter with PBRippl=0.01 dB, fp=500 MHz, fq=1000 MHz (thus, fp2=1500 MHz, leading to 3:1 bandwidth filter). The finite transmission zero is placed at FTZ=350 MHz. It is seen that as bandwidth gets wider the couplings get tighter (Zeven/Zodd=113/19).

Figure 11.2. Realization of medium-wideband filters with NUE=2 and NFTZ=1.

Figure 11.2. Realization of medium-wideband filters with NUE=2 and NFTZ=1 (cont’d).

EXAMPLE 11. 2. Highpass filters with NUE=4 and NFTZ=1

Let’s increase the number of Unit Elements to increase selectivity. Specifications are:

PBRipple=0.01 dB, fp=750 MHz, fq=1000 MHz, Nzero=1, NUE=4, FTZ=600 MHz

- Design steps are shown in Figure 11.3.  Element extraction order is ue-ue-FTZ-ue-ue-zero giving the structure shown in Figure 11.3.a.

- With cursor under the Tlines adjacent to the series OC stubs, the command Kuroda / Distribute Stub is applied to transfer parts of the stubs to the other sides of the Tlines. This command is repeated until an OC stub is placed on both sides of each Tline (Figure 11.3.b).

- The stubs in between two Tlines are split into two parts by applying Split / Two Equal command. Thus, we form OC Stub+TLine+OC Stub triplets as a preparation for conversion into interdigital type coupled lines. Also the shunt OC+SC Stub element is converted to two-step resonator using command StepResonator (Figure 11.3.c).

- With cursor under any Tline, the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the OC stubs leading also to a symmetric circuit (Figure 11.3.d).

- Finally, with cursor under the Tlines, the stub-TLine-stub triplets are converted to interdigital type OC coupled line form by applying the command Replace By PCL / Interdigital Lines / ococINTD / Symmetric giving the circuit of Figure 11.3.e. The circuit is saved as A41.fil.

- Figure 11.3.f shows a pictorial description of a possible realization. The black and white lines may denote the two faces of a stripline structure (if broadside coupled line realization is used, because coupling seems too tight for side coupled realization). Note that in the middle of the filter the second and third coupled line sections become too close to each other which will lead to unaccounted coupling and a complex discontinuity which may lead to some degradation in response. The degradation may be corrected by tuning efforts. However this coupling may also be treated as a cross-coupling between the half wave resonator lines 1 and 3. Thus, half wave lines 1-2-3 may be treated as a cross-coupled triplet section (CT section- see Chapter-24) which may help to produce a finite transmission zero and may be used as an advantage ( an interesting project?).

- Figure 11.3.g shows another possible realization by folding the coupled lines to form hairpin type sections. However this structure needs trial-error or optimization approach.

- A physical separation can be realized between coupled lines by replacing the second and third coupled lines by their comb type equivalents. For this purpose the circuit A41.fil is loaded (Figure 11.3.h). The cursor is placed under the second and third coupled lines and the command Half Angle is applied. This command replaces the interdigital coupled lines with a Pi-section of OC stubs in between two Tlines, all having Half of the original commensurate length (Figure 11.3.i).

- Then with cursor under the series OC stub of the Pi-sections of OC stubs, the command Replace By PCL / Combline / ococCOMB / Symmetric is applied converting the Pi-sections into OC comb type coupled lines (Figure 11.3.j).

- Figure 11.3.k shows a pictorial description which is neater for construction compared to that of Figure 11.3.f.

- Figure 11.3.m shows another practical realization topology which makes the structure more compact.

Figure 11.3. Design of a medium-wideband filter with NUE=4 and NFTZ=1.

Figure 11.3. Design of a medium-wideband filter with NUE=4 and NFTZ=1 (cont’d).

EXAMPLE 11. 3. A highpass filter with NUE=6 and NFTZ=1

Let’s consider a filter with six Unit Elements and the FTZ moved closer to the passband edge:

PBRipple=0.01 dB, fp=750 MHz, fq=1000 MHz, Nzero=1, NUE=6, FTZ=650 MHz

- Design steps are shown in Figure 11.3. Element extraction order is ue-ue-ue-FTZ-ue-ue-ue-zero giving the structure shown in Figure 11.3.a.

- With cursor under the Tlines adjacent to the series OC stubs, the command Kuroda / Distribute Stub is applied to transfer parts of the stubs to the other sides of the Tlines. This command is repeated until an OC stub is placed on both sides of each Tline (Figure 11.3.b).

- The stubs in between two Tlines are split into two parts by applying Split / Two Equal command. Thus, we form OC Stub+TLine+OC Stub triplets as a preparation for conversion into interdigital type coupled lines (Figure 11.3.c).

- With cursor under any Tline, the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the OC stubs (Figure 11.3.d).

- Next, with cursor under the Tlines, the stub-TLine-stub triplets are converted to interdigital type OC coupled line form by applying the command Replace By PCL / Interdigital Lines / ococINTD / Symmetric giving the circuit of Figure 11.3.e.

- Finally the shunt OC+SC Stub element is converted to two-step resonator using command StepResonator (Figure 11.3.f). The circuit is saved as A61.fil.

- Figure 11.3.g shows a pictorial description of a possible realization. Note that in the middle of the filter the second and third coupled line sections become too close to each other which will lead to unaccounted coupling which may lead to some degradation in response.

- Note that the stepped stub has rather low impedances. If realization of low impedance stub is difficult, then we can split it into two and realize the circuit as shown in Figure 11.3.h.

- A physical separation can be realized between coupled lines by replacing the interdigital coupled lines on the two sides of the stepped stub by their comb-coupled type equivalents by applying Hlaf Angle command as described in Figures 11.3.i-to-k.

- Realization can be done as shown in Figure 11.4.m.

Figure 11.4. Design of a highpass filter with NUE=6 and NFTZ=1.

Figure 11.4. Design of a highpass filter with NUE=6 and NFTZ=1 (cont’d).

EXAMPLE 11. 4. Highpass filters with NUE=3 and Two FTZs

Let’s now increase the number of FTZs. Consider a filter three Unit Elements and two FTZ’s placed at the same frequency of 650 MHz:

PBRipple=0.01 dB, fp=750 MHz, fq=1000 MHz, Nzero=1, NUE=3, FTZ1=650 MHz, FTZ2=650 MHz

- Design steps are shown in Figure 11.5. Element extraction order is ue-FTZ1-ue-FTZ2-ue-zero giving the structure shown in Figure 11.5.a.

- With cursor under the Tlines the command Kuroda / Distribute Stub is applied to transfer parts of the stubs to the other sides of the Tlines. This command places equal stubs on each side of the respective Tlines (Figure 11.5.b). Therefore the triplets of OC Stub+Tline+OC Stub are ready for conversion into coupled line form.

- The shunt OC+SC Stub elements are converted to two-step resonators using command StepResonator (Figure 11.5.c).

- With cursor under the Tlines, the stub-TLine-stub triplets are converted to interdigital type OC coupled line form by applying the command Replace By PCL / Interdigital Lines / ococINTD / Symmetric giving the circuit of Figure 11.5.d. The circuit is saved as A32.fil.

- Figure 11.5.e shows a pictorial description of a possible realization. Note that at the junctions of stepped stubs we have too complex discontinuities. We can ease these problems by converting the interdigital coupled line in between the two stepped stubs into comb-type coupled lines as described in Figures 11.7.f-to-i.

Figure 11.5. Design of a highpass filter with NUE=3 and NFTZ=2.

Figure 11.5. Design of a highpass filter with NUE=3 and NFTZ=2.

EXAMPLE 11. 5. A highpass filter with NUE=4 and NFTZ=2

Let’s now increase the number of Unit Elements to four:

PBRipple=0.01 dB, fp=750 MHz, fq=1000 MHz, Nzero=1, NUE=4, FTZ1=650 MHz, FTZ2=650 MHz

- Design steps are shown in Figure 11.6. Element extraction order is ue-FTZ1-ue-ue-FTZ2-ue-zero giving the structure shown in Figure 11.6.a.

- With cursor under the Tlines the command Kuroda / Distribute Stub is applied to transfer parts of the stubs to the other sides of the Tlines. This command is repeated on the second Tline so that an OC stub is placed in between each Tline pair (Figure 11.6.b).

- The single stub in the middle is split into two so that triplets of  OC Stub+Tline+OC Stub will be formed about each Tline (Figure 11.6.c)

- Then the command Kuroda / All Equal Stubs is applied to equate the OC stubs in the middle sections of the filter (Figure 11.6.d).

- With cursor under the Tlines, the stub-TLine-stub triplets are converted to interdigital type OC coupled line form by applying the command Replace By PCL / Interdigital Lines / ococINTD / Symmetric giving the circuit of Figure 11.6.e.

- The shunt OC+SC Stub elements are converted to two-step resonators using command StepResonator (Figure 11.6.f). The circuit is saved as A42.fil.

- Figure 11.6.g shows a pictorial description of a possible realization. Note that at the junctions of stepped stubs we have too complex discontinuities. We can ease these problems by converting the two interdigital coupled lines in between the two stepped stubs into comb-type coupled lines as described in Figures 11.7.h-to-k.

Figure 11.6. A highpass filter with NUE=4 and NFTZ=2.

Figure 11.6. A highpass filter with NUE=4 and NFTZ=2 (cont’d).

EXAMPLE 11. 6. A highpass filter with NUE=5 and NFTZ=2

Let’s now increase the number of Unit Elements to five and move the FTZ’s closer to the passband edge:

PBRipple=0.01 dB, fp=750 MHz, fq=1000 MHz, Nzero=1, NUE=4, FTZ1=675 MHz, FTZ2=675 MHz

- Design steps are shown in Figure 11.7. Elements can be extracted in different orders. Let’s first try  the order ue-FTZ1-ue-ue-ue-FTZ2-ue-zero giving the structure shown in Figure 11.7.a.

- With cursor under the Tlines the command Kuroda / Distribute Stub is applied to transfer parts of the stubs to the other sides of the Tlines. This command is repeated on all the Tlines having an OC stub on one side so that an OC stub is placed in between each Tline pair (Figure 11.7.b).

- The single stubs in between the Tlines are split into two so that triplets of  OC Stub+Tline+OC Stub will be formed about each Tline (Figure 11.7.c)

- Then the command Kuroda / All Equal Stubs is applied to equate the OC stubs in the middle sections of the filter (Figure 11.7.d).

- With cursor under the Tlines, the stub-TLine-stub triplets are converted to interdigital type OC coupled line form by applying the command Replace By PCL / Interdigital Lines / ococINTD / Symmetric giving the circuit of Figure 11.7.e.

- The shunt OC+SC Stub elements are converted to two-step resonators using command StepResonator (Figure 11.7.f). It is seen that the even and odd mode impedances of the coupled lines in the middle part of the circuit are too high. Therefore we repeat the design by extracting the transmission zeros in the order ue-ue-FTZ1-ue-FTZ2-ue-ue-zero and repeat the previous steps getting the circuit of Figure 11.7.g which has better behaved element values than the previous circuit. This circuit is saved as A52.fil.

- Figure 11.7.h shows a pictorial description of a possible realization. Note that at the junctions of stepped stubs we have too complex discontinuities. We can ease these problems by converting the two interdigital coupled lines in between the two stepped stubs into comb-type coupled lines as described in Figures 11.7.i-to-m.

Figure 11.7. Design of a highpass filter with NUE=5 and NFTZ=2.

Figure 11.7. Design of a highpass filter with NUE=5 and NFTZ=2 (cont’d).

EXAMPLE 11. 7. A highpass filter with NUE=6 and NFTZ=3

Let’s now see an example with three FTZ’s and NUE=6:

PBRipple=0.01 dB, fp=750 MHz, fq=1000 MHz, Nzero=1, NUE=6, FTZ1=675 MHz, FTZ2=675 MHz

- Design steps are shown in Figure 11.8. Elements are extracted in the order ue-ue-FTZ1-ue-ue-FTZ2-ue-ue-FTZ3-zero giving the structure shown in Figure 11.8.a.

- With cursor under the Tlines the command Kuroda / Distribute Stub is applied to transfer parts of the stubs to the other sides of the Tlines. This command is repeated on all the Tlines having an OC stub on one side so that an OC stub is placed in between each Tline pair (Figure 11.8.b).

- The single stubs in between the Tlines are split into two so that triplets of  OC Stub+Tline+OC Stub will be formed about each Tline (Figure 11.8.c)

- Then the command Kuroda / All Equal Stubs is applied to equate the OC stubs in the middle sections of the filter (Figure 11.8.d).

- With cursor under the Tlines, the stub-TLine-stub triplets are converted to interdigital type OC coupled line form by applying the command Replace By PCL / Interdigital Lines / ococINTD / Symmetric giving the circuit of Figure 11.8.e.

- The shunt OC+SC Stub elements are converted to two-step resonators using command StepResonator (Figure 11.8.f. This circuit is saved as A63.fil.

- Figure 11.8.g shows a pictorial description of a possible realization. Note that at the junctions of stepped stubs we have too complex discontinuities. We can ease these problems by converting the two interdigital coupled lines in between the two stepped stubs into comb-type coupled lines as described in Figures 11.7.h-to-k.

Figure 11.8. Design of a highpass filter with NUE=6 and NTFZ=3.

Figure 11.8. Design of a highpass filter with NUE=6 and NTFZ=3 (cont’d).

MEDIUM-WIDE BAND DISTRIBUTED ELEMENT BANDPASS FILTERS

WITH

FINITE TRANSMISSION ZEROS

11. 3. MEDIUM-WIDE BAND DISTRIBUTED ELEMENT BANDPASS FILTERS WITH FINITE TRANSMISSION ZEROS

Distributed element highpass filters are just special cases of distributed element bandpass filters. If the quarter wave frequency fq of a distributed element bandpass filter is taken as fq=2f0 where f0=(fp1+fp2)/2  is the arithmetic center of passband then the resulting bandpass filter will have exactly the same response as the corresponding highpass filter. We can also get the same structure as the highpass filter by designing it as a bandpass filter. As will be seen in the following examples, bandpass  filters  are more advantages in several respects:

- In highpass filters transmission zeros can exist only at zero and any FTZ, fz must be below the passband edge fp. That is fz<fp. Such a filter will have a passband with fp1=fp and fp2=2fq-fp1.  It will act as a bandpass filter having Ninf=Nzero and any FTZ, fz of the highpass filter will lead to an upper stopband FTZ at 2fq-fz. That is we will have complete symmetry about the passband center f0=fq=(fp1-fp2)/2. In bandpass filters we can place different number of TZ’s at zero and infinity. Also the locations of FTZ’s can be asymmetric. These features enable us to design many more alternative structures for the same passband.

- Another important feature of the bandpass filters is that we can set the quarter wave frequency fq independently of the passband center f0. By selecting fq>2f0, we can reduce the commensurate length, hence size of filter compared to the corresponding highpass filter.  This property leads also widening of the upper stopband because the nearest spurious passband at 2fqBP-f0 will be moved away from the passband. If longer size and nearer spurious passband is needed then we can set fq in the range fp2<fq<2f0.

A large variety of bandpass filters can be designed by using different numbers of TZ’s at zero, infinity, finite transmission zeros and Unit Elements. However not all of these designs are realizable because of extreme element values and/or unfeasible topologies. The first systematic survey of feasible solutions was carried out by Minnis  12. He classified the filters having finite transmission zeros as Class-A which is suitable for medium-wideband filters and Class-B which is suitable for narrow-wide bandwidth filters. In this section more variations will be introduced by departing from these classes by changing the numbers of transmisson zeros. In order to use as reference, the basic properties of Class-A and Class-B filters are summarized below.

Minnis-Class-A Filters

The topology of this class is shown in Figure 11.9. It consists of alternating cascades of bandpass sections and fourth order sections defined in the figure. This class is characterized by the following transmission zero pattern:

Nzero=Ninf=1

NFTZ: set by the user, as even integer

NUE=NFTZ+2

Half of the finite transmission zeros (FTZ) are placed  (preferably) at a single frequency in the lower stopband, and the other half is placed (preferably) at a single frequency in the upper stopband. Collection of lower and upper stopband transmission zeros at  single frequencies results in lower spread in element values. Denoting the lower and upper stopband FTZ frequencies by fz1 and fz2 respectively, the following observations are noted by Minnis:

- fz1 and fz2 have to be approximately equally displaced from passband center to assist realization of fourth order sections.

- Realization of fourth order sections becomes more difficulties as fz1 and fz2 move away from fq/2 and are completely unrealizable if fz1fq/8. Thus once fz1 is set, an upper bound for fq appears automatically as  fq,max8fz1.

- Generally these filters seem to be realizable for fractional bandwidths of the range 50-100 percent with stop bandwidths up to seven times the center frequency.

- Realization difficulties decrease as stop bandwidth decreases.

- Filters with narrower or wider pass bandwidths may also be realizable if narrower stop bandwidths  (lower fq ) are acceptable.

- High internal impedances may be scaled down by using symmetric Pi-sections at the ports (by applying Pi-to-Pi transformations).

The number of contributing unit elements in Class-A filters is set as NUE = NFTZ+2, giving alternating cascades of Pi-sections and fourth order sections separated by single unit elements. If NUE is chosen to be greater than  NFTZ+2, then new classes of filters can be produced which may be mixtures of A and E types. Furthermore if  Ninf  1, then mixtures of A and C types could be obtained. These new types may be used to improve selectivity of Class-E or Class-C type filters by introducing finite transmission zero sections. It is possible to synthesize all such filters using Filpro.

Figure 11.9. Class-A filters for NFTZ=2, 4 and 6.

Minnis-Class-B Filters:

This class involves insertion of redundant Unit Element to scale element values. Transmission zero pattern is as follows:

Nzero = Ninf = 1

NFTZ = Number of finite transmission zeros chosen from the set  4, 6, 8, ... half in lower 			          stopband (preferably at a single frequency, fz1) and half in upper stopband  			          (preferably at a single frequency, fz2), close to band edges.

NUE=2*NFTZ-4          (4, 8, 12, 16, ...)

Typical structures are shown in Figure 11.10  for NFTZ=4  and  NFTZ=6. Topology consists of alternating cascades of fourth order sections and a pair of bandpass sections. The end sections are made up of Pi-sections of OC stubs and redundant UE’s.

Due to redundant UEs, these filters are realizable over a broader range of bandwidths (10% to over 100%). Constraints on positions of finite transmission zeros are similar to these in Class-A. Many more variations of this class can be obtained by using different number of Unit Elements, as will be seen in the following examples.

Figure 11.10. Circuits for Class-B filters of  degree N=14 and N=22.

Topologies of both of these classes are derivable from the highpass filter counterpart by applying Half Angle Transformation to all elements. It is seen that the filters designed with the transmission zero patterns of these classes will cover only certain special degrees. In this section we will enlarge the list of realizable degrees by starting form low degrees and gradually increasing the number of different transmission zeros.

EXAMPLE 11. 8. Link between distributed element bandpass filter and highpass filter approaches: A bandpass filter with Nzero=Ninf=1, NUE=4 and NFTZ=2

In order to demonstrate that the highpass filters  are special cases of bandpass filters with properties Nzero=Ninf, fq=2f0 and fz2=fq-fz1 let’s design the highpass filter of Example 11.1 with Nzero=1, NUE=2 and NFTZ=600 MHz. Then we will search for the extra flexibilities and advantages offered by the bandpass filter approach. Let

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz

In order to get the same highpass filter as Example 11.1, we have to set the quarter wave frequency fq of the bandpass filter to be twice the arithmetic center of passband which was also the quarter wave frequency of the highpass filter. That is we will take	fqBP=2f0=2fqHP=2000 MHz.

This property locates the position of the nearest spurious passband center as 3f0. Later we will set fqBP independently of the passband center both to move the spurious passband away from the main passband and to get smaller stub/Tline length leading to miniaturization of the structure. This is one of the most important advantages of the distributed element bandpass filter approach over the highpass filter approach.

The single TZ of the highpass filter at zero (Nzero=1) will appear as a pair of TZ’s in bandpass filter, one being at zero and the other infinity. Therefore for the bandpass filter we will take Nzero=1 and Ninf=1. Similarly, the single FTZ of the highpass filter at f=600 MHz will appear as two FTZ’s, one at fz1=600 MHz and the other at fz2=2fqHP-fz1=1400 MHz. That is, in highpass filters the positions of the FTZ’s in lower and upper stopbands depend on each other. Therefore in order to get the same highpass filter as the previous example we have to take

FTZ1=600 MHz,  FTZ2=1400 MHz

Later we will adjust the positions of fz1 and fz2 independently of each other to get asymmetric insertion loss responses. This is another advantage of the distributed element bandpass filter approach over the highpass filter approach.

The two Unit Elements of the highpass filter will appear as four Unit Elements with halved lengths. Therefore we will set  NUE=4.

With these specifications, the Synthesis steps of the bandpass filter are shown in Figure 11.11.

- Note that we have selected Distributed Element / Bandpass  / Type A options. After typing in the specifications and entering transmission zeros we get exactly the same response as the highpass filter of Example 11.1.

- Transmission zeros are extracted in the order ue-ue-FTZ1-FTZ2-zero-inf-ue-ue giving the circuit shown in Figure 11.11.a.

- With cursor under the series OC stub on left the command Double Norton / Inverse Geffe is applied. This command transforms the two FTZ sections so as the shunt OC+SC stub elements become adjacent as shown in Figure 11.11.b.

- It should be noted that the series arm OC+SC Stub element on the right side has negative impedances. Simplify Circuit command combines the negative impedance elements with positive ones and moves the transformer to load end. We have now all positive elements (Figure 11.11.c).

- With cursor under the Tlines adjacent to the series arm OC+SC stub elements the command Kuroda Pi / Right (Left) / Symmetric Pi is applied to form Pi-sections of OC stubs in between the Tlines as shown in Figure 11.11.d. Simplify Circuit command eliminates the transformer (Figure 11.11.e).

- Next, the Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric (Figure 11.11.e-f).

- This circuit is saved as M42A.fil (e.g.). In the next stage the shunt OC+SC stub elements will be converted into stepped stubs. As the first trial, we place the cursor under the left shunt OC+SC stub element and apply command Step Resonator / Four Step Resonator. This command combines the two adjacent shunt OC+SC stub elements and forms a single four step OC stub with step impedances denoted as Z1, Z2, Z3 and Z4, with lengths of 37.474 mm which is set by the quarter wavelength frequency of fq=2000 MHz, as shown in Figure 11.11.g. Note that we have Z1=Z2 and Z3=Z4. Therefore this four step stub is actually a two-step stub with lengths of  74.948 mm which was the quarter wavelength corresponding to the fqHP=1000 MHz of Example 11.1. This property is due to the fact that fqBP=2f0. Later we will see that for fq2f0 it will be a four step stub with all steps having different impedances.

- Note that this circuit is exactly the same as the highpass filter of Example 11.1 shown in Figure 11.1.g. Figure 11.11.h shows a pictorial description which is the same as Figure 11.1.h of the Example 11.1. Thus, we have proved that the distributed element bandpass filters with fq=2f0 is identical to the highpass filters with fqHP=fqBP/2=f0.

- Instead of a single four-step-stub we can use two two-step stubs. The circuit M42A.fil is repeated in Figure 11.11.i. In this circuit we place the cursor under the shunt OC+SC Stub elements one by one and apply command Step Resonator /Two Step Resonator. This command converts the two OC+SC Stub elements into two different two-step OC stubs as shown in Figure 11.11.k. Figure 11.11.m shows a pictorial description. Note that the first step impedance Z1 of both two-step stubs are equal. This is also due to the property of fq=2f0. If fq2f0 then they would be different as will be seen in the next example. Thus, the distributed element bandpass filter approach offers an alternative realization involving two-step stubs.

- Decision on whether to use a four step stub realization or two-step stub realization depends on realizability of impedances of the steps. Note that one of the two-step stubs has too high impedance. Therefore in this example the four step option seems better. However this may not be the case for other bandwidths, PBRipple, degrees and most important, the position of the FTZ’s with respect to passband center.

- For example in Figure 11.11.n the four-step and two-step resonator versions of the circuits for PBRipple=0.1 dB and FTZ1=600 MHz, FTZ2=1400 MHz are shown. It is seen that increasing the passband ripple increases the Tline and coupled line impedance levels while the four and two step stub impedances are not changed much.

- Figure 11.11.p shows the case for PBRipple=0.01 dB, but the FTZ’s moved away from passband edges, to FTZ1=f0/2=500 MHz and FTZ2=1500 MHz. It is seen that the four-step stub now became a uniform stub at a low impedance level while the two-step stub impedances have extreme Z1/Z2 ratios.

- Figure 11.11.q shows the circuits for the case where the FTZ2s are moved closer to the passband edges, to  FTZ1=675 MHz and FTZ2=1325 MHz. In this case both two-step and four-step stubs have problematic impedances. So, in a typical design, all these factors have to be taken into account to make all impedances realizable.

- In the next example we will inspect the effects of fq>2f0.

Figure 11.11. Design of a highpass filter by using distributed element bandpass filter approach.

Figure 11.11. Design of a highpass filter by using distributed element bandpass filter approach (cont’d).

Figure 11.11. Design of a highpass filter by using distributed element bandpass filter approach (cont’d).

EXAMPLE 11. 9. Distributed element bandpass filter with fq>2f0 and asymmetric FTZ’s

By removing the constraints of the distributed element highpass filter approach we get some important advantages leading to new solutions. Basically these advantages are as follows:

- In highpass filters we have the constraint of fqHP=f0=fqBP/2. By selecting fqBP>2f0, we can decrease the commensurate length leading to size reduction compared to the highpass version. This property leads also widening of the upper stopband because the nearest spurious passband at 2fqBP-f0 will be moved away from the passband. If longer size and nearer spurious passband is needed then we can set fq in the range fp2<fq<2f0.

- In highpass filters we have arithmetic symmetry about passband center f0=fqHP. Effectively this means we have equal number of transmission zeros at zero and infinity. Also any finite transmission zero (FTZ) fz is automatically accompanied by the arithmetically symmetric finite transmission zero at 2fqHP-fz, preserving the arithmetic symmetry. In distributed element approach we can place different number of transmission zeros at zero and infinity and also place the finite transmission zeros independently of each other. These features enable one to get  asymmetric responses.

- More variations can be obtained for circuit topologies, as we have already seen one in Part-a of this example where two-step and four-step stub alternatives were introduced.

In order to proceed systematically, let’s start from the previous example and introduce these features one by one. Let

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz

In order to see the effects of fq>2f0, let’s select

fq=3000 MHz

In order to get the same topology as the previous example, thus to see the differences clearly, let’s set

Nzero=1, Ninf=1

Let’s also keep the number of Unit Elements the same as the previous example:

NUE=4

a) Symmetric FTZ’s:

In the previous example, the two FTZ’s were at fz1=600 MHz and fz2=2fqHP-fz1=1400 MHz. That is, they have arithmetic symmetry. In this example, in order to avoid shadowing the effects of selecting fq>2f0, let’s keep them the same:

fz1=600 MHz, fz2=1400 MHz

We will inspect the effects of asymmetric FTZs in the second part of this example.

With these specifications, the Synthesis steps of the bandpass filter with FTZ1=600 MHz and FTZ2=1400 MHz are shown in Figure 11.12. These steps are the same as the previous example:

- Distributed Element / Bandpass  / Type A options are selected.

- After typing in the specifications and entering transmission zeros we note that the upper stopband is widened appreciably because fq=3000 MHz is now higher than 2f0=2000 MHz as seen from the response plot.

- Transmission zeros are extracted in the order ue-ue-FTZ1-FTZ2-zero-inf-ue-ue giving the circuit shown in Figure 11.12.a.

- With cursor under the series OC stub on left the command Double Norton / Inverse Geffe is applied. This command transforms the two FTZ sections so as the shunt OC+SC stub elements become adjacent as shown in Figure 11.12.b. It should be noted that the series arm OC+SC Stub element on the right side has negative impedances. Simplify Circuit command absorbs the  negative elements into positive ones and moves the transformer to load end (Figure 11.12.c).

- With cursor under the Tlines adjacent to the series arm OC+SC stub elements the command Kuroda Pi / Right (Left) / Symmetric Pi is applied to form Pi-sections of OC stubs in between the Tlines as shown in Figure 11.12.d. Simplify Circuit command moves the transformer to load end (Figure 11.12.e).

- Next, the Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric (Figure 11.12.f).

- This circuit is saved as M42B.fil (e.g.). In the next stage the shunt OC+SC stub elements will be converted into stepped stubs.

- As the first trial, we place the cursor under the left shunt OC+SC stub element and apply command Step Resonator / Four Step Resonator. This command combines the two adjacent shunt OC+SC stub elements and forms a single four step OC stub with step impedances denoted as Z1, Z2, Z3 and Z4, with lengths of 24.9 mm which is set by the quarter wavelength frequency of fq=3000 MHz, as shown in Figure 11.12.g. Note that all steps have different impedances (because fq2f0).

- Figure 11.12.h shows a pictorial description. The circuit for fq=2000 MHz is also shown on the right for comparison. The differences are obvious:

  - The total length is reduced from 149.6 mm to 99.6 mm.

  - In the case of fq=2000 MHz, all Tline impedances were equal (107 ohms) while for fq=3000 MHz Tlines have different impedances at a higher level (154 and 159 ohms).

  - The coupled lines of the circuit for fq=2000 MHz have even-odd mode impedances of 251 ohms and 45.8 ohms. For fq=3000 MHz they reduce to more reasonable levels of 99.4 ohms and 26.8 ohms.

  - The four step stub for fq=2000 MHz case has impedances of 41 ohms and 77.8 ohms. For the case of fq=3000 MHz, they we have 58 ohms, 24 ohms, 143 ohms and 29 ohms. So, as fq gets higher fluctuations of step impedances get higher.

- In order to get the two-step stubs option the circuit of MA421.fil is reloaded (Figure 11.12.h). In this circuit we place the cursor under the shunt OC+SC Stub elements one by one and apply elements into two different two step OC stubs as shown in Figure 11.12.i. Figure 11.12.k shows a pictorial description of both fq=3000 MHz and fq=2000 MHz cases for comparison.

  - For fq=2000 MHz case the step impedance pairs are Z1=82.1, Z2=21.3 ohms and  Z1=82.1, Z2=316 ohms. For fq=3000 MHz, these impedances become Z1=103, Z2=83.6 ohms and Z1=135, Z2=14.3 ohms. Impedances for fq=3000 MHz seem to be better behaved.

  - Note that in fq=2000 MHz case we have equal Z1’s for both two-step stubs while they are different for fq=3000 MHz case.

b) Asymmetric Finite Transmission Zeros

In order to get an idea on the effects of asymmetrically placed FTZ’s, the above design is repeated for the following cases:

- The lower stopband FTZ is moved away from the passband edge, from 600 MHz down to FTZ1=500 MHz while the upper stopband FTZ is kept the same at FTZ2=1400 MHz. The response and resulting four-step-stub and two-step-stub versions of the circuit are shown in Figure 11.12.m.

- The lower stopband FTZ is moved closer to the passband edge, from 600 MHz to FTZ1=650 MHz while the upper stopband FTZ is kept the same at FTZ2=1400 MHz. The response and resulting four-step-stub and two-step-stub versions of the circuit are shown in Figure 11.12.n.

- The lower stopband FTZ is kept the same at FTZ1=600 MHz and the upper stopband  FTZ is moved away from the passband to FTZ2=1500 MHz. The response and resulting four-step-stub and two-step-stub versions of the circuit are shown in Figure 11.12.p.  Note that one of the two-set stubs came out to be uniform (Z1=Z2). This is because of the selection of FTZ2=1500 MHz=fq/2.

- The lower stopband FTZ is kept the same at FTZ1=600 MHz and the upper stopband  FTZ is moved closer to the passband to FTZ2=1350 MHz. The response and resulting four-step-stub and two-step-stub versions of the circuit are shown in Figure 11.12.q.

Exercise: Comment on variation of all impedances in the above designs.

Exercise: Repeat the above designs for PBRipple=0.1 dB and comment on the results.

Figure 11.12. Design of a bandpass filter with fq=3000 MHz.

Figure 11.12. Design of a bandpass filter with fq=3000 MHz (cont’d).

Figure 11.12. Design of a bandpass filter with fq=3000 MHz (cont’d).

Figure 11.12. Design of a bandpass filter with fq=3000 MHz (cont’d).

EXAMPLE 11. 10. Bandpass filters with two FTZ’s in lower stopband

If the FTZ’s are placed only in one stopband, then the topology of the circuit will be different. Four-step resonators of the previous designs will not exist. Let’s consider the same filter as the previous examples, but with both FTZ’s placed in lower or in upper stopbands:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=3000 MHz

FTZ1=FTZ2=650 MHz.

- The design steps for this case are shown in Figure 11.13. When two FTZ sections are in the same stopband it is not possible to combine them to form a single four-step stub. Therefore the transmission zero extractions should be carried out by consideration of realizability in the simplest possible forms. The extraction order must lead to a physically symmetric structure to avoid/ eliminate transformers. The extraction order shown in Figure 11.13.a yields such a structure. Note that FTZ sections are always extracted after extracting a Unit Element for realization of the series OC stubs in coupled line forms.

- The shunt OC+SC stub elements are converted into two-step stubs by using Step Resonator command. The first and last series OC stubs are shared to the two sides of the adjacent Tlines by applying the command Kuroda / Distribute Stub. The series OC stub on the right of the third Tline from left is transferred to the left of this Tline by applying the command Kuroda / Right / Full Stub to form a symmetric circuit after applying Simplify Circuit command (Figure 11.13.b).

- The series OC and SC stubs in the middle of the circuit are split into two equal parts by using Split / Two Equal command and Exchange command is used to form two symmetric halves of the circuit (Figure 11.13.c).

- With cursor under the second Tline from left, the command Kuroda Pi / Right /  Symmetric Pi is applied to transfer the series OC and SC stubs on the right as a Pi-section of OC stubs on left. Similarly, with cursor under the third Tline, the command Kuroda Pi / Left / Symmetric Pi is applied to form a Pi section of OC stubs on the right of that Tline. Simplify Circuit command eliminates transformers resulting from these operations (Figure 11.13.d).

- We save this circuit as M42C.fil for use in different trials towards different realization.

- As the first trial, the triplets of series OC Stub+Tline+series OC stub are converted into equivalent coupled lines by applying the command Replace By PCL / Interdigital / ococINTD / Symmetric.  The Pi-sections of OC stubs are also converted into comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric (Figure 11.13.e).

- Inspection of the even and odd mode impedances of both interdigitaş and comb type coupled lines shows that they are unrealizable because of extreme even-odd mode impedance ratios. Therefore we divert to an alternative structure as follows.

- The circuit of M42C.fil (Figure 11.13.f). With cursor under the series OC stubs of Pi-sections, the command Replace By Lumped / Single C: f=750 MHz is applied to convert the low impedance OC stubs into series lumped capacitors. The approximation is carried out at the low end of the passband to minimize the shift in lower edge because the series OC stubs are most effective on these edges. The shunt OC stubs have realizable impedances. Therefore they are not converted into lumped capacitors.

- Also with cursor under the Tlines of the OC Stub+Tline+OC Stub triplets, the command Half Angle / OSsr+TL+OSsr is applied to convert these triplets into Pi-sections of OC stubs surrounded by two Tlines, all halved in length (Figure 11.13.g).Next the series OC stubs of these new Pi-sections are converted into lumped capacitors by applying the command Replace By Lumped / Single C: f=750 MHz (Figure 11.13.h).

- The shunt OC stubs of the new Pi-sections of OC stubs have very high impedances. They can be deleted or converted into lumped capacitors by applying the command Replace By Lumped / Single C: f=1250 MHz (Figure 11.13.ı). This approximation is carried out at the high end of the passband because the shunt OC stubs are most effective at the upper edge of the passband. Since the resulting capacitors are too small thy may be absorbed into the fringing capacitors of the Tlines. Figure 11.13.j shows a pictorial description of the realization where the shunt OC stubs are not shown. Conversion of stubs into lumped capacitors cause slight degradation in return loss and stopband insertion loss will change, as shown in Figure 11.13.k.

Exercise: Repeat the design for fq=2000 and fq=2500 MHz, for different PBRipples.

Figure 11.13. A bandpass filter with two finite TZ’s in lower stopband.

Figure 11.13. A bandpass filter with two finite TZ’s in lower stopband.

EXAMPLE 11. 11. Bandpass filters with two FTZ’s in upper stopband

Let’s consider a filter with the same passband and degree:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=3000 MHz

with two finite transmission zeros placed in upper stopband at FTZ1=FTZ2=1400 MHz.

With NUE=4, creation of a circuit without shunt SC stubs is not possible unless redundant Unit Elements are inserted from source and load ends. Let’s first inspect a design involving SC stubs.

Case-a) A design involving SC stubs

- The design steps for this case are shown in Figure 11.14. In this approach we select the option Type B. TZ’s are extracted in the order ue-FTZ1-ue-zero-ue-FTZ2-inf-ue giving the circuit shown in Figure 11.14.a. Note that FTZ sections are always extracted after extracting a Unit Element.

- The series SC stubs are translated to the other sides of the adjacent Tlines by applying command Kuroda / Right (Left) / Full Stub and the shunt arm series OC+SC stub elements are converted into two-step stubs by applying Step Resonator command. We get a symmetric circuit (Figure 11.14.b).

- The shunt OC and SC stubs in the middle of the circuit have too small impedances. This problem can be resolved by first combining them to form a single parallel OC+SC stub element (Figure 11.14.c).

- Then with cursor under the parallel OC+SC Stub element the command Replace By Lumped / SC Stub+Cap: f1=750 MHz, f2=1250 MHz is applied. This command converts the OC stub component of this element into a lumped capacitor while modifying the SC stub impedance also to keep the passband edges unaffected (Figure 11.14.d). Since bandwidth is wide this approximation will lead to a degradation in passband return loss and stop band insertion loss will change to some extent. Return loss degradation can be corrected by tuning all elements slightly at the end of the design.

- Note that the resulting lumped capacitance is too large while the SC stub impedance is still too low for realization. These values can be scaled as follows.

- With cursor under the Tlines on the two sides of the SC Stub+Cap element the command Kuroda / Distributed Stub is applied. This command transfers part of the SC stub in the middle to the other sides of the adjacent Tlines (Figure 11.14.e).

- Now we have three shunt SC stubs with realizable impedances. However their impedances are not equal. They can be equated by placing the cursor under any Tline and applyimng the command Kuroda / All Equal Stubs / Exclude End Stubs. Thus, the three shunt SC stubs are equated at about 50 ohms level (Figure 11.14.f).

- Figure 11.14.g shows a pictorial description of a possible realization.

- In Figure 11.14.h-i, another possible realization is shown if the element values are suitable. In this realization the three shunt SC stubs in the middle are realized as an interdigital filter section with the middle resonator having lumped capacitive loading.

- Figure 11.14.k shows wideband insertion loss and passband return loss. Return loss response shows that passband edges are not affected by conversion of the OC stub into lumped capacitor. Degradation is slight and can be corrected by tuning elements.

Exercise: Repeat the design for fq=2000 and fq=2500 MHz, for different PBRipples.

Figure 11.14. Two FTZ’s in upper stopband. A solution involving SC stubs.

Figure 11.14. Two FTZ’s in upper stopband. A solution involving SC stubs (cont’d).

Case-b) A design involving redundant Unit Elements

Let’s design the filter with FTZ1=FTZ2=1400 MHz in a form which doesn’t involve SC stubs. This approach necessitates introduction of redundant Unit Elements as follows.

- The design steps for this case are shown in Figure 11.15. In this approach we select the option Type A. TZ’s are extracted in the order FTZ1-ue-ue-zero-ue-ue-FTZ2-inf giving the circuit shown in Figure 11.15.a.

- Note that the FTZ sections have series SC stubs (because they are in upper stopband). Also at load end we have a series SC stub. These series  SC stubs are unrealizable as they are. They can be converted into shunt OC stubs if there were an adjacent Tline. We provide these Tlnes by inserting redundant (50 ohms) lines from source and load ends using Insert command.

- The shunt arm series OC+SC stub elements are converted into two-step stubs by applying Step Resonator command.

- The series SC stub on the right of the fourth Tline is transferred to the left of that Tline by applying Kuroda / Right / Full Stub command. After deleting the unity transformer at load end we get the circuit of Figure 11.15.b.

- With cursor under the redundant Tlines at source and load ends the command Kuroda / Right (Left) / Full Stub is applied to convert the adjacent series SC stubs into shunt OC stubs on the other sides of the lines.

- The shunt OC stub on the right of the third Tline is transferred to the left of this Tline as a series SC stub by applying Kuroda / Right / Full Stub command. Thus, we now have a symmetric circuit (Figure 11.15.c).

- The series SC and OC stubs in the middle of the circuit are split into two equal parts and Exchange command is used to form two symmetric halves of the circuit as shown in Figure 11.15.d.

- The series SC and OC stub pair on the right of the second Tline are transferred to left as a Pi-section of OC stubs by applying the command Kuroda Pi / Right / Symmetric Pi. Similarly the series SC and OC stub pair on the left of the third Tline are transferred to its right by applying the command Kuroda Pi / Left / Symmetric Pi (Figure 11.15.e).

- The resulting transformers are combined to form unity turns ratio, hence deleted (Figure 11.15.f).

- The Pi-sections of OC stubs are converted into OC comb type parallel coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric, giving the final form of the circuit shown in Figure 11.15.g.

- Figure 11.15.h shows a pictorial description.

Exercise: Repeat the design for fq=2000 and fq=2500 MHz, for different PBRipples.

Figure 11.15. Two FTZ’s in upper stopband. A solution involving Redundant Unit Elements.

Figure 11.15. Two FTZ’s in upper stopband. A solution involving Redundant Unit Elements.

EXAMPLE 11. 12. Degree 14 filters with NFTZ=2 and NUE=8

Let’s now increase the number of unit elements from NUE=4 to NUE=8 while we keep Nzero=Ninf=1 and NFTZ=2.

Let’s consider a filter with the same passband specs:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=3000 MHz

- Entering Nzero=Ninf=1, NUE=8 and two finite transmission zeros placed symmetrically at FTZ1=600 MHz and FTZ2=1400 MHz we get the response shown in Figure 11.16. We may also shift these FTZ’s to form asymmetric response.

- Transmission zeros are extracted in the order ue-ue-ue-ue-FTZ1-FTZ2-zero-inf-ue-ue-ue-ue getting the circuit shown in Figure 11.16.a.

- With cursor under the series OC stub on left the command Double Norton / Inverse Geffe is applied to get the two shunt OC+SC Stub elements adjacent. Simplify Circuit command combines the negative series stubs with adjacent positive stubs giving the symmetric circuit of Figure 11.16.b.

- Using the command Kuroda / Right Full Stub, the series OC and SC stubs on left half are moved towards the source end to form an LRight-section of OC stubs as shown in Figure 11.16.c. Simplify Circuit and Symmetrize commands are used to eliminate the transformers resulting from these transformations.

- With cursor under the shunt leg of the LRight-section of OC stubs, the command Kuroda-Minnis EF / Right / Specify Tline-1 and 2 / Z1=150, Z2=150 ohms is applied to form a Pi-section of OC stubs on the left of the Tline pair whose impedances are set as 150 ohms (Figure 11.16.d). In this command Tline impedances are selected by trials until reasonable impedances are obtained at the end of the design procedure.

- On the right of the Tline pair of 150 ohms impedance a new LRight-section is formed. This LRight-section is converted into a symmetric Pi-section by applying the command Pi-Tee-L / LRight-to-Sym Pi. Simplify Circuit and Symmetrize commands eliminates transformers giving the circuit of Figure 11.16.e.

- The Pi-sections are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric (Figure 11.16.f).

- Finally the shunt SC+OC stub elements will be converted into stepped OC stubs. They can be converted either into two different two-step stubs or a single four step stub. Figures 11.16.g and h show the circuit and pictorial description for the two-step stubs case while Figures 11.16.ı and k show the circuit and pictorial description for the four-step stub case.

Exercise: Inspect element values by shifting the positions of FTZ’s and by changing the passband ripple.

Exercise: Try to find solutions when the FTZ’s are in the same stopband.

Figure 11.16. Design of a degree 14 degree filter with NUE=8 and NFTZ=2.

Figure 11.16. Design of a degree 14 degree filter with NUE=8 and NFTZ=2 (cont’d).

EXAMPLE 11. 13. Degree 14 filters with NFTZ=2, NUE=4 and Ninf=3

In the previous example we have increased the number of Unit Elements to form a degree 14 filter. This approach increases length of the filter. An alternative approach may be to keep NUE=4 and increase the number of TZs at infinity which will be realized as shunt OC stubs, hence will not increase length of the filter. Filter specifications are:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=3000 MHz, Nzero=1, Ninf=5, NUE=4. The two finite transmission zeros placed symmetrically at FTZ1=600 MHz and FTZ2=1400 MHz giving the response shown in Figure 11.17. Because of large number of TZ’s at infinity (actually at fq) we get high suppression in upper stopband. We may also shift the FTZ’s to adjust response.

- Transmission zeros are extracted in the order inf-ue-inf-ue-FTZ1-FTZ2-inf-zero-ue-inf-ue-inf-ue-inf giving the circuit shown in Figure 11.17.a.

- In order to be able to convert all the series SC stubs into shunt OC stubs, we introduce redundant (50 ohms) Tlines at input and output.

- With cursor under the series OC stub on left the command Double Norton / Inverse Geffe is applied to get the two shunt OC+SC Stub elements adjacent. Simplify Circuit command combines the negative series stubs with adjacent positive stubs giving the symmetric circuit of Figure 11.17.b.

- Using Kuroda / Right / Full Stub and Pi-Tee-L /  LRight-to-Sym Pi  command the series OC stub on the right of the third Tline is moved to the position shown in Figure 11.17.g while the series SC stubs on the right of second and third Tlines are converted into shunt OC stubs.

- A Tline followed by series OC and SC stubs as shown at the source end of the circuit of Figure 11.17.g is named as Kuroda-Minnis-CD section.  With cursor under the Tline of this section the command Kuroda-Minnis CD / Right / k=0.2 is applied. This command transfers parts of the series OC and SC stubs to the left of the Tline as a symmetric Pi-section of OC stubs (Figure 11.17.h). Here the parameter k is set by the user by inspecting the resulting Tline and shunt OC stub impedances. The value k=0.2 is found after a few design trials.

- The series and shunt OC stubs left on the right of the first Tline are transferred to the right of the second Tline by using Kuroda / Left / Full Stub command to form a new Kuroda-Minnis CD section as shown in Figure 11.17.i.

- Applying the command Kuroda-Minnis CD / Right : k=0.5 we form the second Pi-section of OC stubs (Figure 11.17.k).

- The LLeft-section on the right of the second Tline is converted into Pi-section of OC stubs by using the command Pi-Tee-L / LLeft-to-Sym Pi. The shunt OC+SC stubs are converted into two-step stubs by applying the command Step Resonators / Two Step Resonators (Figure 11.17.m). Four step resonator version involves more extreme impedances.

- The series OC stub nearest to the source and load are too small. Therefore they are replaced by their lumped capacitor equivalents. The other Pi-sections of OC stubs are converted into OC comb type coupled lines (Figure 11.17.n). However as it is seen their odd mode impedances are also too small for realization and we may as well convert them into lumped capacitors.

- One of the two-step stubs have Z2=4 ohms which is also unrealizable. It can also be converted into a lumped capacitor.

- Figure 11.17.p shows a pictorial description. It is seen tha the total length of the filter is shorter  than that of the previous example, by two Unit Elements.

Figure 11.17. Design of a degree 14 filter with Ninf=5, NUE=4 and NFTZ=2.

Figure 11.17. Design of a degree 14 filter with Ninf=5, NUE=4 and NFTZ=2 (cont’d).

EXAMPLE 11. 14. Degree 16 filters with NFTZ=4, NUE=6 and Ninf=1

In the previous approach we have used four contributing unit elements and inserted two redundant Unit Elements leading in total six Unit Elements. The high selectivity in upper stopband was provided by the five TZ’s at infinity. The unit elements can be used in a more efficient way by designing the filter directly with six contributing Unit Elements while instead of the five TZ’s at infinity, we make Ninf=1 and double the number of FTZ’s.

Filter specifications are:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=3000 MHz, Nzero=1, Ninf=1, NUE=6. The four finite transmission zeros are placed symmetrically at FTZ1=FTZ2=600 MHz and FTZ3=FTZ4=1400 MHz giving the response shown in Figure 11.18. We may also shift the FTZ’s to adjust response.

- Transmission zeros are extracted in the order ue-ue-FTZ1-FTZ3-ue-ue-FTZ2-FTZ4-zero-inf-ue-ue giving the circuit shown in Figure 11.18.a.

- With cursor under the series OC stubs the command Double Norton / Inverse Geffe is applied to make the two shunt OC+SC Stub elements of the FTZ pairs adjacent. Simplify Circuit command moves the transformers rightmost (Figure 11.18.b).

- Using Kuroda / Left / Full Stub command the negative series elements are translated to the right of the fourth Tline and Simplify Circuit command is used to absorb them into the adjacent positive stubs (Figure 11.18.c).

- With cursor under the Tlines adjacent to the series OC+SC stub elements the command Kuroda Pi / Right (Left) / Symmetric Pi is applied to form Pi-sections of OC stubs in between Tlines (Figure 11.18.d).

- Then the Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace BY PCL / Combline / ococCOMB / Symmetric (Figure 11.18.e).

- Finally, the shunt OC+SC stub elements are converted either into two-step stubs as shown in Figures 11.18.f-g or into four-step stubs as shown in Figures 11.18.h-i by using Step Resonator command.

Figure 11.18. Design of a degree 16 filter with NFTZ=4, NUE=6 and Ninf=1.

Figure 11.18. Design of a degree 16 filter with NFTZ=4, NUE=6 and Ninf=1 (cont’d).

EXAMPLE 11. 15. Degree 18 filters with NFTZ=4, NUE=8 and Ninf=1

Let’s now increase the number of Unit Elements by two to get higher suppression in stopbands.

Filter specifications are:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=3000 MHz, Nzero=1, Ninf=1, NUE=. The four finite transmission zeros are placed symmetrically at FTZ1=FTZ2=600 MHz and FTZ3=FTZ4=1400 MHz giving the response shown in Figure 11.19. We may also shift the FTZ’s to adjust response.

- Transmission zeros are extracted in the order ue-ue-FTZ1-FTZ3-ue-ue-ue-ue-FTZ2-FTZ4-zero-inf-ue-ue giving the circuit shown in Figure 11.19.a.

- With cursor under the series OC stubs the command Double Norton / Inverse Geffe is applied to make the two shunt OC+SC Stub elements of the FTZ pairs adjacent. Simplify Circuit command moves the transformers rightmost (Figure 11.19.b).

- Using Kuroda / Left / Full Stub command repeatedly the negative series elements on the left of the third Unit Element are translated to the middle of the circuit. Similarly using Kuroda / Right / Full Stub command repeatedly the series elements on the right of the sixth Unit Element are translated to the middle of the circuit. Simplify Circuit command is used to combine the translated negative and positive stubs in the middle of the circuit (Figure 11.19.c).

- The series OC+SC stub element on the symmetry axis is split into two to form the two symmetric halves (Figure 11.19.d).

- With cursor under the Tlines adjacent to the series OC+SC stub elements the command Kuroda Pi / Right (Left) / Symmetric Pi is applied to form Pi-sections of OC stubs in between Tlines (Figure 11.19.e).

- Then the Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace BY PCL / Combline / ococCOMB / Symmetric (Figure 11.19.f).

- Finally, the shunt OC+SC stub elements are converted either into two-step stubs as shown in Figures 11.19.g-h or into four-step stubs as shown in Figures 11.19.i-k by using Step Resonator command.

Figure 11.19. Design of a degree 18 filter with NFTZ=4, NUE=8 and Ninf=1.

Figure 11.19. Design of a degree 18 filter with NFTZ=4, NUE=8 and Ninf=1 (cont’d).

NARROWBAND HIGHPASS AND BANDPASS FILTERS WITH FINITE TRANSMISSION ZEROS

11. 4. NARROWBAND HIGHPASS AND BANDPASS FILTERS WITH FINITE TRANSMISSION ZEROS

Synthesis of narrowband highpass and bandpass filters result in extreme impedances which necessitate the use of impedance scaling techniques like insertion of redundant transmission lines (impedance transforming lines) or tapping of the first and last resonators.

Let’s first see application of the insertion of transformer line approach on higpass filters with different degrees. Practical realization constraints necessitates the number of Unit Elements NUE>NFTZ. Also if physical symmetry is desired then for Nzero=1, one of the NUE or NFTZ must be odd and the other must be even.

EXAMPLE 11. 16. Degree 5 highpass filters with NFTZ=1, NUE=2, Nzero=1

Let  PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz, NUE=2, NFTZ=1 at fz=960 MHz, Nzero=1

to work as a bandpass filter with fp1=975 MHz and fp2=1025 MHz.

- Design steps are shown in Figure 11.20. Note that Distributed Element / Highpass / Type A options are selected.

- Transmission zeros are extracted in the order ue-FTZ-zero-ue to give a symmetric circuit (Figure 11.20.a). Element values are too high for realization. In order to scale them down the following procedure is used.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.20.b). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=1000 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.20.c).

- With cursor under the Tlines adjacent to the series OC stubs, the command Kuroda / Distribute Stub is applied. This command is repeated until an OC stub is placed on each side of the Tlines (Figure 11.20.d). Then the OC stubs in between two Tlines are split into two to form OC Stub+Tline+OC Stub triplets for conversion into coupled line form (Figure 11.20.e).

- The command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the series OC stubs (Figure 11.20.f).

- With cursor under the Tlines, the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to form interdigital coupled lines (Figure 11.20.g). The circuit is saved as HP111.fil for use in later stages.

- Finally, the OC+SC Stub element in the middle is converted into two-step resonator by applying command Step Resonator (Figure 11.20.h).  Although the coupled line sections have realizable impedances, the stepped stub has unrealizable impedances. Also there is no physical separations for placement of the stub. In order to get around these problems the following procedure is used.

- The circuit of HP111.fil is loaded and with cursor under the coupled lines on the two sides of the middle element (OC+SC Stub) the command Half Angle is applied. This command replaces the coupled line by a Pi-section of OC stubs in between two Tlines with halved lengths (Figure 11.20.k). Then the Pi-section of OC stubs are converted into comb type coupled lines by applying the command Replace By PCL / Comblines / ococCOMB / Symmetric (Figure 11.20.m).

- Then with cursor under the shunt OC+SC stub element the command Half Angle / Double Stub is applied. This command replaces the OC+SC Stub by two uniform stubs with different lengths and realizable impedances as shown in Figures 11.20.n-p.

- We could as well select the option Half Angle / Double Resonator and get two two-step stubs as shown in Figures 11.20.q-r-s-t.

- Figures 11.20.u-v shows another design trials to get higher impedances by specifying higher inverter constant (K=75 ohms).

Figure 11.20.  Narrowband highpass filter with NUE=2, NFTZ=1 and Nzero=1.

Figure 11.20.  Narrowband, degree 5 highpass filter with NUE=2, NFTZ=1 and Nzero=1 (cont’d).

Figure 11.20.  Narrowband, degree 5 highpass filter with NUE=2, NFTZ=1 and Nzero=1 (cont’d).

EXAMPLE 11. 17. Degree 10 bandpass filters with NFTZ=2, NUE=4, Nzero=1, Ninf=1

The highpass filter structure of the previous example can also be designed as a distributed element bandpass filter with more flexibilities like size reduction by selecting higher fq>2f0 and placement of FTZ’s in an asymmetric manner. Let

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz,  fq=2500 MHz, NUE=4, NFTZ=2, one at fz1=960 MHz and the other at fz2=1060 MHz, Nzero=1, Ninf=1

- Design steps are shown in Figure 11.21. Note that Distributed Element / Bandpass / Type A options are selected.

- Transmission zeros are extracted in the order ue-ue-FTZ1-FTZ2-zero-inf-ue-ue to give the circuit of Figure 11.21.a.

- With cursor under the series OC on left half, the command Double Norton / Inverse Geffe is applied to make the OC+SC Stub elements adjacent. Simplify Circuit command combines the negative and positive elements giving the symmetric circuit of Figure 11.21.b

- The adjacent SC+OC Stub elements are converted into Two-step stubs by applying the command Stepped Resonators / Two Step Resonators (Figure 11.21.c). Four step resonator option gives unrealizable impedances.

- Element values are too high for realization. In order to scale them down the following procedure is used.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.21.d). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=25000 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.21.e). Here f0 is actually the quarter wave frequency of the filter.

- With cursor under the Tlines adjacent to the series OC+SC stub element, the command Kuroda / Right / Full Stub is applied repeatedly to move these stubs to the leftmost position to form an LRight-section of OC stubs as shown in Figure 11.21.f.

- With cursor under the shunt leg of the LRight-section, the command Kuroda-Minnis EF / Right / Specify Tline-1 and Tline-2 / Z1=100, Z2=100 ohms is applied. This command forms a Pi-section of OC stubs on the left of the two Tlines and forms a new LRight-section on the right of the two Tlines while also setting impedances of Tlines as specified by the user. In this application 100 ohms impedance is found proper after a few trials (Figure 11.21.g).

- Next, the LRight-section of OC stubs is converted into a Pi-section by using the command Pi-Tee-L / LRight-to-Sym Pi. Simplify Circuit command moves the resulting transformers rightmost (Figure 11.21.h).

- The Pi-sections of OC stubs are converted into comb type OC coupled lines by using Replace By PCL / Combline / ococCOMB / Symmetric.  Symmetrize command maps the operations done on left half to right half. The extra element produced by the Symmetrize command must be deleted. Figure 11.21.ı shows the final circuit and its pictorial description. It is seen that all impedances are realizable.

- If desired, the stepped shunt OC stubs can be converted into uniform OC stubs by applying Replace By Stub command. This command adjust length of the OC stubs to keep the position of the FTZs unaffected. However return loss will be degraded. This degradation can be corrected by tuning impedances of both OC stubs.  Figure 11.21.m shows the resulting circuit and its pictorial description.

- Figures 11.21.n and p shows an alternative realization for the finite transmission zeros using Tee-Resonators. This topology may be useful if the other alternatives come out to be unrealizable.

- The overall impedance level can be adjusted by selecting a different inverter constant. As exercise repeat the design with K=100 Ohms.

Figure 11.21.  Narrowband, degree 10 bandpass filter with NUE=4, NFTZ=2, Ninf=1 and Nzero=1.

Figure 11.21.  Narrowband, degree 10 bandpass filter with NUE=4, NFTZ=2, Ninf=1 and Nzero=1 (cont’d).

Figure 11.21.  Narrowband, degree 10 bandpass filter with NUE=4, NFTZ=2, Ninf=1 and Nzero=1 (cont’d).

EXAMPLE 11. 18. Degree 7  highpass filters with NFTZ=1, NUE=4, Nzero=1

Let’s now consider a highpass filter of degree higher than 5. Degree 6 filter would lead to an antimetric structure while usually symmetric filters are preferred in practice. Therefore let’s consider a degree 7 filter by increasing the number of unit elements to NUE=4. So the specifications are:

PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz, NUE=4, NFTZ=1 at fz=960 MHz, Nzero=1

to work as a bandpass filter with fp1=975 MHz and fp2=1025 MHz.

- Design steps are shown in Figure 11.22. Note that Distributed Element / Highpass / Type A options are selected.

- Transmission zeros are extracted in the order ue-ue-FTZ-zero-ue-ue to give a symmetric circuit (Figure 11.22.a). Element values are too high for realization. In order to scale them down the following procedure is used.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.22.b). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=1000 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.22.c).

- With cursor under the Tlines adjacent to the series OC stubs, the command Kuroda / Distribute Stub is applied. This command is repeated until an OC stub is placed on each side of the Tlines (Figure 11.22.d).

- The OC stubs in between two Tlines are split into two to form OC Stub+Tline+OC Stub triplets for conversion into coupled line form (Figure 11.22.e).

- The command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the series OC stubs (Figure 11.22.f).

- With cursor under the Tlines, the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to form interdigital coupled lines (Figure 11.22.g). The circuit is saved as HP115.fil for use in later stages.

- Finally, with cursor under the OC+SC Stub element in the middle, the command Half Angle / Uniform Resonators is applied to replace this element by two uniform stubs having different lengths which set the two FTZ’s which are located on the two sides of the passband (Figure 11.22.h). The equivalence is approximate but holds almost perfectly for narrowband filters. We could as well use the command Half Angle / Stepped Resonators  to get exact stepped resonator equivalent. However not much would be gained.

- It is seen that the shunt OC stubs have too low impedances in the order of 16-17 ohms. If desired we can scale them up by using a higher inverter constant K. Figure 11.22.i shows the circuit obtained by selecting inverter constant K=100 where the stub impedances are raised to about 35 ohms level while the even mode impedances of the edge coupled sections are increased to above 100 ohms level.

- Figure 11.22.k shows a pictorial description. It should be noted that at the junction in the middle of the circuit we have a complex geometry which may lead to unaccounted cross-couplings and complex junction discontinuities. If these are problematic then we can use the following alternative realization.

- The circuit of HP115.fil is loaded and with cursor under the coupled lines on the two sides of the middle element (OC+SC Stub) the command Half Angle is applied. This command replaces the coupled line by a Pi-section of OC stubs in between two Tlines with halved lengths (Figure 11.22.m-n). Then the Pi-section of OC stubs are converted into comb type coupled lines by applying the command Replace By PCL / Comblines / ococCOMB / Symmetric (Figure 11.22.p). Figure 11.22.q shows a pictorial description of the structure which has physical separations between stubs and coupled lines.

Figure 11.22.  Narrowband, degree 7 highpass filter with NUE=4, NFTZ=1 and Nzero=1.

Figure 11.22.  Narrowband, degree 7 highpass filter with NUE=4, NFTZ=1 and Nzero=1 (cont’d).

Figure 11.22.  Narrowband, degree 7 highpass filter with NUE=4, NFTZ=1 and Nzero=1 (cont’d).

EXAMPLE 11. 19. Degree 14 bandpass filters with NFTZ=2, NUE=8, Nzero=Ninf=1

The bandpass filter counterpart of the degree 7 highpass filter of the previous example with Nzero=1, NUE=4 and NFTZ=1 is a degree 14 bandpass filter with Nzero=Ninf=1, NUE=8, NFTZ=2. The bandpass filter version has extra flexibilities like shorter length, placement of FTZ’s asymmetrically, increasing the number of FTZ’s while decreasing the number of Unit Elements or increasing the number of Ninf while decreasing either NUE of NFTZ. Let’s first consider the simplest case of doubling NUE and NFTZ. That is, let

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz,  fq=2500 MHz, NUE=8, NFTZ=2, one at fz1=960 MHz and the other at fz2=1060 MHz, Nzero=1, Ninf=1

- Design steps are shown in Figure 11.23. Note that Distributed Element / Bandpass / Type A options are selected.

- Transmission zeros are extracted in the order ue-ue-ue-ue-FTZ1-FTZ2-zero-inf-ue-ue-ue-ue to give the circuit of Figure 11.23.a.

- With cursor under the series OC on left half, the command Double Norton / Inverse Geffe is applied to make the OC+SC Stub elements adjacent. Simplify Circuit command combines the negative and positive elements giving the symmetric circuit of Figure 11.23.b

- The adjacent SC+OC Stub elements are converted into Two-step stubs by applying the command Stepped Resonators / Two Step Resonators (Figure 11.23.c). Four step resonator option gives unrealizable impedances.

- Element values are too high for realization. In order to scale them down the following procedure is used.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.23.d). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=2500 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.23.e). Here f0 is actually the quarter wave frequency of the filter.

- With cursor under the Tlines adjacent to the series OC+SC stub element, the command Kuroda / Right / Full Stub is applied repeatedly to move these stubs to the leftmost position to form an LRight-section of OC stubs as shown in Figure 11.23.f.

- With cursor under the shunt leg of the LRight-section, the command Kuroda-Minnis EF / Right / Specify Tline-1 and Tline-2 / Z1=70, Z2=70 ohms is applied. This command forms a Pi-section of OC stubs on the left of the two Tlines and forms a new LRight-section on the right of the two Tlines while also setting impedances of Tlines as specified by the user. In this application 70 ohms impedance is found proper after a few trials. Simplify Circuit command moves the transformer away  (Figure 11.23.g).

- Next, with the cursor under the shunt leg of the new LRight-section of OC stubs, the command Kuroda-Minnis EF / Right / Specify Tline-1 and Tline-2 / Z1=80, Z2=80 ohms is applied again to form a new Pi-section of OC stubs and a new LRight-section. Here Tline impedance is selected again after a few trials while inspecting all the other impedances (Figure 11.23.h).

- Next, the new LRight-section is converted into a Pi-section by applying the command Pi-Tee-L / LRight-to-Sym Pi. Simplify Circuit command moves the resulting transformers rightmost (Figure 11.23.i).

- The Pi-sections of OC stubs are converted into comb type OC coupled lines by using Replace By PCL / Combline / ococCOMB / Symmetric.  Symmetrize command maps the operations done on left half to right half. The extra element produced by the Symmetrize command must be deleted. Figure 11.23.k shows the final circuit and its pictorial description. It is seen that all impedances are realizable. However the low stub impedances may cause realization problems. The low impedance steps may be replaced by lumped capacitors.

- We may try other values for Tline impedances and impedance scaling inverter constant K to find better behaved impedances.

Figure 11.23. Degree 14 bandpass filter with NUE=8, NFTZ=2, Ninf=1 and Nzero=1.

Figure 11.23. Degree 14 bandpass filter with NUE=8, NFTZ=2, Ninf=1 and Nzero=1(cont’d).

EXAMPLE 11. 20. Degree 14 bandpass filters with NFTZ=2, NUE=4, Nzero=1, Ninf=5

In the previous example degree 14 was obtained by setting Nzero=Ninf=1, NUE=8, NFTZ=2. We can as well form a degree 14 filter by decreasing NUE to 4 while increasing Ninf to 5. Let

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz,  fq=2500 MHz, NUE=4, NFTZ=2, one at fz1=960 MHz and the other at fz2=1060 MHz, Nzero=1, Ninf=5

- Design steps are shown in Figure 11.24. Transmission zeros are extracted in the order inf-ue-inf-ue-FTZ1-FTZ2-zero-inf-ue-inf-ue-inf to give the circuit of Figure 11.24.a.

- With cursor under the series OC stub on left half, the command Double Norton / Inverse Geffe is applied to make the OC+SC Stub elements adjacent. Simplify Circuit command combines the negative and positive elements giving the symmetric circuit of Figure 11.24.b

- The adjacent SC+OC Stub elements are converted into Two-step stubs by applying the command Stepped Resonators / Two Step Resonators (Figure 11.24.c). Four step resonator option gives unrealizable impedances. Element values are too high for realization. In order to scale them down the following procedure is used.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.24.d). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=2500 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.24.e). Here f0 is actually the quarter wave frequency of the filter.

- With cursor under the series OC+SC stub element, the command Split / Two Element Resonator is applied to split it into OC and SC stub components. Then Exchange command is used to move the OC stub to the right of the third Tline (Figure 11.24.f).

- Then the command Kuroda / Right / Full Stub is applied to move the OC stub to the left of the third Tline. Shift TRF / Rightmost command is used to move the transformer away (Figure 11.24.g).

- Then Exchange and Kuroda / Right / Full Stub commands are repeated to move the OC stub to the position shown in Figure 11.24.i.

- With cursor under the first Tline, the command Kuroda-Minnis CD/ Right / k=0.5 is applied. This command splits the series OC stub on the right of the Tline, forms a Pi-section of OC stubs on the left of the Tline leaving a piece of the OC stub on right, producing also a transformer. Simplify Circuit  command moves away the transformer. Here k is a parameter which will be used to adjust impedances of the resulting shunt OC stub and Tline impedances. As an initial trial we set it as k=0.5.

- The series OC stub remaining on the right of the first Tline is transferred to the right of the second Tline by applying Kuroda / Left / Full Stub and Simplify Circuit command. Thus a series OC+SC Stub element is formed on the right of the second Tline (Figure 11.24.k-m).

- With cursor under the second Tline, the command Kuroda-Minnis CD / Right / k=0.4 is applied to form a new Pi-section of OC stubs on the left of the second Tline. Simplify Circuit command moves the transformer away  (Figure 11.24.n).

- The series SC stub on the right of the third Tline is moved to its left as a shunt OC stub by using Kuroda / Right / Full Stub command to form an LLeft-section on the left of the third Tline which is then converted into a Pi-section of OC stubs by applying the command Pi-Tee-L / Lleft-to-Sym Pi (Figure 11.24.p-q). The circuit is saved as BP120.fil to be used at later stages.

- The Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric and then Symmetrize command is applied to map the operations done on the left half to the right.

- Figure 11.24.r shows the final impedances. It is seen that the stepped shunt OC stubs have too low impedances. In order to see if we can raise them a new design trial is made by selecting different values for the parameter k in Kuroda-Minnis-CD transformations. One result is shown in Figure 11.24.s. Stepped stub impedances are increased slightly at the expense of appreciable increase in Tline impedances. Several trials indicated that these impedances are strongly dependent on the bandwidth. Wider bandwidths result in higher impedances.

- If realization of the low impedance stepped stubs is problematic, we can adjust them by using asymmetric coupled lines. One example is shown in Figure 11.24.t-to-y.

- The circuit of BP120.fil is loaded (Figure 11.24.t).

- With cursor under the series OC stub of the third Pi-section, the command Pi-Tee-L / Pi-to-Pi-Tee-L / Pi-to-Pi / Specify TRF: n=0.7 is applied. This command converts the symmetric Pi-section into an asymmetric Pi-section while also producing a transformer which is set by the user. This transformer is used to scale up impedances of the stepped stubs (Figure 11.24.u).

- Simplify Circuit command moves away the transformer giving the circuit of Figure 11.24.v.

- The first two I-sections of OC stubs are converted into symmetric coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric. The third Pi-section could  be converted into an asymmetric coupled lines. However just as a variation, we have preferred to replace the series OC stub by its lumped capacitor equivalent and leave the two shunt OC stubs in uncoupled form (asymmetric coupled lines are usually difficult to handle for realization).

- Figure 11.24.x shows the final element values while Figure 11.24.y shows a pictorial description.

- Compared to the case with NUE=8 of the previous example, this circuit has smaller longitudinal size.

Exercise: Repeat the design by selecting different values for the parameter k.

Exercise: Repeat the design by moving the FTZ’s closer and away from the passband edges.

Exercise: Repeat the design with a different fq.

Exercise: Repeat the design with different PBRipples and different bandwidths.

Exercise: Convert the FTZ elements into Tee Resonators.

Figure 11.24. Degree 14 bandpass filter with NUE=4, NFTZ=2, Ninf=5 and Nzero=1.

Figure 11.24. Degree 14 bandpass filter with NUE=4, NFTZ=2, Ninf=5 and Nzero=1 (cont’d).

Figure 11.24. Degree 14 bandpass filter with NUE=4, NFTZ=2, Ninf=5 and Nzero=1 (cont’d).

EXAMPLE 11. 21. Degree 7  highpass filters with NFTZ=2, NUE=2, Nzero=1

In Example 11.18 we have designed a degree 7 highpass filter with a single FTZ and NUE=4. We can also design a degree 7 highpass filter with two FTZ’s by reducing NUE to two.

Let

PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz, Nzero=1, NUE=2, NFTZ=2 with both FTZ’s at fz=960 MHz

- Design steps are shown in Figure 11.25. Note that Distributed Element / Highpass / Type A options are selected. Transmission zeros are extracted in the order FTZ1-ue-ue-FTZ2-zero  (Figure 11.25.a).

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.25.b). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=1000 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.25.c).

- With cursor under the Tlines adjacent to the series OC stubs, the command Kuroda / Distribute Stub is applied. This command is repeated until an OC stub is placed on each side of the Tlines (Figure 11.25.d).

- The OC stub in between two middle Tlines are split into two to form OC Stub+Tline+OC Stub triplets for conversion into coupled line form (Figure 11.25.e).

- The command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the series OC stubs (Figure 11.25.f).

- With cursor under the Tlines, the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to form interdigital coupled lines (Figure 11.25.g).

- Finally, with cursor under the OC+SC Stub element in the middle, the command Half Angle / Uniform Resonators is applied to replace this element by two uniform stubs having different lengths which set the two FTZ’s which are located on the two sides of the passband. We could as well use the command Half Angle / Stepped Resonators to get exact stepped resonator equivalent. The circuit is saved as HP125.fil for use in later stages.

- Figure 11.25.i shows a pictorial description. It should be noted that at the junctions of the shunt stubs and coupled lines we have rather complex geometries which may prove to be problematic discontinuities. In problematic cases we may use the following approaches.

- The circuit of HP125.fil is loaded and with cursor under the coupled lines in the interior of the circuit the command Half Angle is applied. This command replaces the coupled lines by Pi-section of OC stubs in between two Tlines with halved lengths (Figure 11.25.k-m).

- Then the Pi-section of OC stubs are converted into comb type coupled lines by applying the command Replace By PCL / Comblines / ococCOMB / Symmetric (Figure 11.25.n). The circuit is saved as HP126.fil. Figure 11.25.p shows a pictorial description of the structure.

- If further physical separation is required then we can convert the interdigital type coupled lines at ports also into comb type coupled lines, as described in Figures 11.25.q-r-s-t.

- Figure 11.25.u shows a design for the impedance scaling inverter constant K=40 ohms used to decrease the impedance level of the original design with K=50 ohms.

Figure 11.25. Degree 7 highpass filters with Nzero=1, NFTZ=2 and NUE=2.

Figure 11.25. Degree 7 highpass filters with Nzero=1, NFTZ=2 and NUE=2.

Figure 11.25. Degree 7 highpass filters with Nzero=1, NFTZ=2 and NUE=2.

EXAMPLE 11. 22. Degree 14 bandpass filters with NFTZ=4, NUE=4, Nzero=1, Ninf=1

In the previous highpass filter example the structure obtained by Half Angle transformations can also be obtained by designing the filter as a distributed element bandpass filter with Nzero=Ninf=1, NUE=4 and NFTZ=4, with extra features like smaller size and asymmetric FTZ’s. Let’s consider the following specifications:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz,  fq=2500 MHz, NUE=4, NFTZ=4, two at fz1,2=960 MHz and the other two at fz3,4=1060 MHz, Nzero=1, Ninf=1.

- Design steps are shown in Figure 11.26. Transmission zeros are extracted in the order FTZ1-FTZ3-ue-ue-ue-ue-FTZ2-FTZ4-zero-inf to give the circuit of Figure 11.26.a.

- With cursor under the series OC stubs, the command Double Norton / Inverse Geffe is applied to make the shunt OC+SC Stub elements adjacent. Simplify Circuit command moves the transformers rightmost (Figure 11.26.b).

- Using Kuroda / Right-Left transformations the series OC and SC stubs in the interior part of the circuit are collected in the middle of the circuit as shown in Figure 11.26.c.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.26.d). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=2500 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.26.e). Here f0 is actually the quarter wave frequency of the filter.

- With cursor under the series OC+SC stub element in the middle, the command Split / Two Equal is applied to split it into two pieces (Figure 11.26.f).

- Then the commands Kuroda Pi / Right / Symmetric Pi and  Kuroda Pi / Left / Symmetric Pi are applied to transfer the OC+SC stub elements to the other sides of the adjacent Tlines as Pi-sections of OC stubs (Figure 11.26.g).

- The Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric (Figure 11.26.h).

- The shunt SC+OC stub elements are converted into two-step stubs by applying the command Step Resonators / Two-Step Resonators.

- Figure 11.26.i shows the final circuit element values. It is seen that some of the Tlines have too high impedances. In order to scale them down, the design is repeated by using K=40 for the impedance scaling inverter, giving the element values shown in Figure 11.26.k.

- Figure 11.26.m shows a pictorial description. Topology is similar to the highpass filter structure we have obtained in the previous example, but with shorter total length because of high fq.

Figure 11.26. Degree 14 bandpass filter with NFTZ=4, NUE=4, Nzero=Ninf=1.

Figure 11.26. Degree 14 bandpass filter with NFTZ=4, NUE=4, Nzero=Ninf=1 (cont’d).

EXAMPLE 11. 23. Degree 16 bandpass filters with NFTZ=4, NUE=6, Nzero=1, Ninf=1

Let’s now increase degree of the bandpass filter of the previous example by two. Such a topology can’t be obtained by Half Angle transformations from highpass filters. Degree can be increased either by increasing the number of Unit Elements to NUE=6 or by increasing the number of TZ’s at infinity. Let’s try the first approach:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz,  fq=2500 MHz, NUE=6, NFTZ=4, two at fz1,2=960 MHz and the other two at fz3,4=1060 MHz, Nzero=1, Ninf=1.

- Design steps are shown in Figure 11.27. Transmission zeros are extracted in the order ue-ue-FTZ1-FTZ3-ue-ue-FTZ2-FTZ4-zero-inf-ue-ue to give the circuit of Figure 11.27.a.

- With cursor under the series OC stubs, the command Double Norton / Inverse Geffe is applied to make the OC+SC Stub elements adjacent. Simplify Circuit command moves the transformers rightmost (Figure 11.27.b).

- Using Kuroda / Left / Full Stub transformations the negative series OC and SC stubs on the left half of the circuit are moved to the right of the fourth Unit Element as shown in Figure 11.27.c. Simplify Circuit command combines the positive and negative elements to produce net positive elements.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.27.d). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=2500 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.27.e). Here f0 is actually the quarter wave frequency of the filter.

- With cursor under the fifth Tline the command Kuroda-Pi / Right / Symmetric Pi is applied to convert the  series OC+SC stub element on the right into a Pi-section of OC stubs in the middle of the circuit. Simplify Circuit command moves away the resulting transformers (Figure 11.27.f).

- Then the series OC stubs on the left half of the circuit are moved to the source end by using Kuroda / Right / Full Stub command to form an LRight-section of OC stubs as shown in Figure 11.27.g.

- With cursor under the shunt leg of the LRight-section of OC stubs, the command Kuroda-Minnis-EF / Right / Specify Tlines-1,2: Z1=100, Z2=100 ohms is applied. This command converts the LRight-section into a Pi-section while setting the impedances of the adjacent Tlines as specified by the user. In this trial we selected 100 ohms, to see its effect on the other element values. Also a new LRight-section is formed on the right of the Tlines, together with a transformer. The transformer is moved away by Simplify Circuit command (Figure 11.27.h).

- The new LRight-section is converted into Pi-section by using the command Pi-Tee-L / LRight-to-Sym Pi. The transformer is moved away by Simplify Circuit command (Figure 11.27.i).

- The Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric (Figure 11.27.k). Symmetrize command maps the operations done on left half to the right half of the circuit.

- The shunt SC+OC stub elements are converted into two-step stubs by applying the command Step Resonators / Two-Step Resonators (Figure 11.27.m). Some of the impedances of stepped stubs seem to be too low. They can be adjusted by selecting different values for inverters and/or by selecting different impedances for the Tlines in Kuroda-Minnis-EF transformation.

- Figure 11.27.n shows a pictorial description.

Figure 11.27. Degree 16 bandpass filter with Nzero=Ninf=1, NFTZ=4 and NUE=6.

Figure 11.27. Degree 16 bandpass filter with Nzero=Ninf=1, NFTZ=4 and NUE=6 (cont’d).

EXAMPLE 11. 24. Degree 16 bandpass filters with NFTZ=4, NUE=4, Nzero=1, Ninf=3

Let’s now redesign the degree 16 filter of the previous example by increasing the number of transmission zeros at infinity to Ninf=3 while NUE=4:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, fq=2500 MHz, NUE=4, NFTZ=4, two at fz1,2=960 MHz and the other two at fz3,4=1060 MHz, Nzero=1, Ninf=3.

- Design steps are shown in Figure 11.28. Transmission zeros are extracted in the order inf-ue-FTZ1-FTZ3-ue-ue-FTZ2-FTZ4-inf-zero-ue-inf to give the circuit of Figure 11.28.a.

- With cursor under the series OC stubs, the command Double Norton / Inverse Geffe is applied to make the OC+SC Stub elements adjacent. Simplify Circuit command moves the transformers rightmost (Figure 11.28.b).

- Using Kuroda / Left / Full Stub transformations the negative series OC and SC stubs on the left half of the circuit are moved to the right of the third Unit Element as shown in Figure 11.28.c. Simplify Circuit command combines the positive and negative elements to produce net positive elements.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.28.d). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=2500 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.28.e). Here f0 is actually the quarter wave frequency of the filter.

- With cursor under the fourth Tline the command Kuroda-Pi / Right / Symmetric Pi is applied to convert the  series OC+SC stub element on the right into a Pi-section of OC stubs in the middle of the circuit. Simplify Circuit command moves away the resulting transformers (Figure 11.28.f).

- Then the series OC stubs on the right of the second Tline are moved to its left by using Kuroda / Right / Full Stub command to form an LRight-section of OC stubs as shown in Figure 11.28.g.

- With cursor under the shunt leg of the LRight-section of OC stubs, the command Pi-Tee-L / LRight-to-Pi-Tee-L / LRight-to-LLeft is applied to convert it to an LLeft-section as shown in Figure 11.28.h. In this circuit we now have a series OC+SC stub element on the right of the first Tline which lends itself to application of the Kuroda-Minnis-CD transformation.

- With cursor under the first Tline the command Kuroda-Minnis-CD / Right / k=0.45 is applied. This command forms a Pi-section of OC stubs on the left of the Tline while changing impedance of the Tline also. A pieces of series OC stub is left on the right of the Tline. The parameter k can be adjusted to set the resulting impedances. Simplify Circuit command moves away the transformer giving the circuit of Figure 11.28.i.

- An LLeft-section of OC stubs is formed on the right of the first Tline. It is converted into a Pi-section of OC stubs by applying the command Pi-Tee-L / LLeft-to-Sym Pi. Also the shunt OC+SC stub elements are converted into two-step stubs by applying the command Step Resonators / Two-Step Resonators (Figure 11.28.k).

- The Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric (Figure 11.28.m). Symmetrize command maps the operations done on left half to the right half of the circuit.

- Some of the impedances of stepped stubs seem to be too low. They can be adjusted by selecting different values for inverters and/or by selecting different impedances for the Tlines in Kuroda-Minnis transformation.

- Figure 11.28.n shows a pictorial description. It is seen that this structure is shorter than the one obtained in the previous example. This is because the number of unit elements is two less in this example. However element value spreads may be questionable.

Figure 11.28. Degree 16 bandpass filter with Nzero=1, Ninf=3, NFTZ=4 and NUE=4.

Figure 11.28. Degree 16 bandpass filter with Nzero=1, Ninf=3, NFTZ=4 and NUE=4 (cont’d).

EXAMPLE 11. 25. Degree 9  highpass filters with NFTZ=1, NUE=6, Nzero=1

Degree 9 highpass filters with Nzero=1 and a single FTZ can be designed by setting NUE=6, as follows: Let

PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz, Nzero=1, NUE=6, NFTZ=1 at fz=960 MHz

- Design steps are shown in Figure 11.29. Note that Distributed Element / Highpass / Type A options are selected. Transmission zeros are extracted in the order ue-ue-ueFTZ-zero-ue-ue-ue  (Figure 11.29.a).

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=100 ohms is applied. This command places inverters of impedance K=100 ohms at source and load ends (Figure 11.29.b). Here K=100 ohms is a trial value which can be iterated later, after inspecting the resulting element values. We could start iterations by first selecting K=50 ohms as we had done in the previous examples, but after the experience with the previous highpass filters with single FTZ we now know that the shunt stepped stub impedances come out to be too low for realization. Therefore we have started with a higher impedance for K.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=1000 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.29.c). This conversion will cause some distortion in return loss which can be corrected by tuning all elements slightly (right at this stage or after the design is finished).

- With cursor under the Tlines adjacent to the series OC stubs, the command Kuroda / Distribute Stub is applied. This command is repeated until an OC stub is placed on each side of the Tlines (Figure 11.29.d).

- The OC stub in between two Tlines are split into two to form OC Stub+Tline+OC Stub triplets for conversion into coupled line form (Figure 11.29.e).

- The command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the series OC stubs (Figure 11.29.f).

- With cursor under the Tlines, the command Replace By PCL / Interdigital / ococINTD / Symmetric is applied to form interdigital coupled lines. Also with cursor under the OC+SC Stub element in the middle, the command Half Angle / Stepped Resonators (or Uniform Resonators) is applied to replace this element by two shunt stepped OC stubs which set the two FTZ’s which are located on the two sides of the passband (Figure 11.29.g). The circuit is saved as HP135.fil for use in later stages.

- Figure 11.29.h shows a pictorial description. It should be noted that at the junctions of the shunt stubs and coupled lines we have rather complex geometries which may be problematic for tuning. In problematic cases we may use the following approaches.

- The circuit of HP135.fil is loaded and with cursor under the coupled lines on the two sides of the stepped stubs, Half Angle command is applied. This command replaces the coupled lines by Pi-section of OC stubs in between two Tlines with halved lengths (Figure 11.29.i-k).

- Then the Pi-section of OC stubs are converted into comb type coupled lines by applying the command Replace By PCL / Comblines / ococCOMB / Symmetric (Figure 11.29.m). The circuit is saved as HP126.fil. Figure 11.29.n shows a pictorial description of the structure.

- We can as well convert the other interdigital type coupled lines comb type coupled lines. However such a structure is advantageous to design directly a bandpass filter approach as we have seen in the previous examples. Let’s see this approach in the next example.

Figure 11.29. Degree 9 highpass filters with Nzero=1, NFTZ=1 and NUE=6.

Figure 11.29. Degree 9 highpass filters with Nzero=1, NFTZ=1 and NUE=6 (cont’d).

EXAMPLE 11. 26. Degree 18 bandpass filters with NFTZ=2, NUE=12, Nzero=1, Ninf=1

The bandpass filter coonterpart of degree 9 highpass of the previous example is a degree 18 filter with Nzero=Ninf=1, NFTZ=2 and NUE=12. Advantages of bandpass filter version are shorter length and ability to place FTZ’s asymmetrically. Let’s consider the following specifications:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, fq=2500 MHz, NUE=12, NFTZ=2, one at fz1=960 MHz and the other at fz2=1050 MHz, Nzero=1, Ninf=1.

- Design steps are shown in Figure 11.30. Transmission zeros are extracted in the order ue-ue-ue-ue-ue-ue-FTZ1-FTZ2-zero-inf-ue-ue-ue-ue-ue-ue to give the circuit of Figure 11.30.a.

- With cursor under the series OC stubs, the command Double Norton / Inverse Geffe is applied to make the OC+SC Stub elements adjacent. Simplify Circuit command moves the transformers rightmost (Figure 11.30.b).

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.30.c). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=2500 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.30.d). Here f0 is actually the quarter wave frequency of the filter.

- Using Kuroda / Left / Full Stub transformations the series OC and SC stubs on the left half of the circuit are moved to the leftmost position to form an LRight-section of OC stubs as shown in Figure 11.30.e. Simplify Circuit command moves the transformers rightmost. Also the shunt OC+SC stub elements are converted into two-step stubs by applying the command Step Resonators / Two-Step Resonators.

- With cursor under the shunt leg of the LRight-section of OC stubs the command Kuroda-Minnis-EF/ Right / Specify Tline-1,2: Z1=100, Z2=100 ohms is applied. This command forms a Pi-section of OC stubs on the left while setting the impedances of the next two Tlines as specified by the designer. 100 ohms is used as a trial. This command also forms a new LRight-section on the right of the next two Tlines together with a transformer. Simplify Circuit command moves away the transformer giving the circuit of Figure 11.30.f.

- With cursor under the shunt leg of the new LRight-section the command Kuroda-Minnis-EF/ Right / Specify Tline-1,2: Z1=100, Z2=100 ohms is applied again, forming a new Pi-section and an LRight-section as shown in Figure 11.30.g.

- This command is repeated on the new LRight-section (Figure 11.30.h).

- Finally, the last LRight-section is converted into Pi-section by applying the command Pi-Tee-L / LRight-to-Sym Pi (Figure 11.30.i).

- The Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric (Figure 11.30.k). Symmetrize command maps the operations done on left half to the right half of the circuit.

- Some of the impedances of stepped stubs seem to be too low. They can be adjusted by selecting different values for inverters and/or by selecting different impedances for the Tlines in Kuroda-Minnis transformation.

- Figure 11.30.m shows a pictorial description. It is seen that this structure is shorter than the highpass version of the previous example because of higher fq.

Figure 11.30. Degree 18 bandpass filter with NFTZ=2, Nzero=Ninf=1, and NUE=12.

Figure 11.30. Degree 18 bandpass filter with NFTZ=2, Nzero=Ninf=1, and NUE=12 (cont’d).

EXAMPLE 11. 27. Degree 18 bandpass filters with NFTZ=2, NUE=6, Nzero=1, Ninf=7

The filter of the previous example may seem too long. This is because of large number of Unit Elements. We can move some of the Unit Elements to infinity to get a shorter structure. Let’s consider the following specifications:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, fq=2500 MHz, NUE=6, NFTZ=2, one at fz1=960 MHz and the other at fz2=1050 MHz, Nzero=1, Ninf=7.

- Design steps are shown in Figure 11.31. Transmission zeros are extracted in the order ue-ue-ue-ue-ue-ue-FTZ1-FTZ2-zero-inf-ue-ue-ue-ue-ue-ue to give the circuit of Figure 11.31.a.

- With cursor under the series OC stubs, the command Double Norton / Inverse Geffe is applied to make the OC+SC Stub elements adjacent. Simplify Circuit command moves the transformers rightmost (Figure 11.31.b).

- The series SC+OC stub in the left half of the circuit is split into its OC and SC stub components and Exchange command is used to move the OC stub to be adjacent to the Tline on its left (Figure 11.31.c).

- Using Kuroda / Right / Full Stub and Exchange command the series OC stub is moved to the leftmost position as described in Figures 11.31.d-to-h. Shift TRF / Rightmost command is used to move the transformers away.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.31.ı). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=2500 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.31.k). Here f0 is actually the quarter wave frequency of the filter.

- Now we have a series OC+SC Stub element on the right of the Tline at source end. With cursor under the Tline at source end the command Kuroda-Minnis-CD/ Right / k=0.45 is applied to form a Pi-section of OC stubs on the left of the Tline as shown in Figure 11.31.m.

- A transformer and a piece of series OC stub is left on the right of the Tline. The transformer is cleared by Simplify Circuit command and the series OC stub is moved to the right of the adjacent Tline by applying Kuroda / Left / Full Stub command to form a new Tline and OC+SC stub pair (Figure 11.31.n).

- With cursor under the Tline the command Kuroda-Minnis-CD/ Right / k=0.45 is applied again to form a new Pi-section of OC stubs as shown in Figure 11.31.p.

- The remaining series OC stub is  transferred to the right of the adjacent Tline and the command MultiKuroda-CD/ Right / k=0.45 is repeated to form the third Pi-section of OC stubs (Figure 11.31.q-r).

- The series SC stub on the right of the fourth Tline is transferred to left as shunt OC stub, thus forming an LLeft-section of OC stubs as shown in Figure 11.31.q-r. This LLeft-section is converted into Pi-section by applying the command Pi-Tee-L / LLeft-to-Sym Pi (Figure 11.31.s).

- The Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric. Symmetrize command maps the operations done on left half to the right half of the circuit. Also the shunt SC+OC stub elements are converted into stepped resonators by using Step Resonators / Two Step Resonators command (Figure 11.31.t).

- Figure 11.31.u shows a pictorial description. It is seen that this structure is halved in length compared to the previous approach. This is because the number of Unit Elements is halved.

- Some of the impedances of stepped stubs seem to be too low. They can be adjusted by selecting different values for inverters and/or by selecting different constants k in Kuroda-Minnis-CD transformation.

Figure 11.31. Degree 18 bandpass filter with NFTZ=2, Nzero=1, Ninf=7, and NUE=6.

Figure 11.31. Degree 18 bandpass filter with NFTZ=2, Nzero=1, Ninf=7, and NUE=6 (cont’d).

Figure 11.31. Degree 18 bandpass filter with NFTZ=2, Nzero=1, Ninf=7, and NUE=6 (cont’d).

EXAMPLE 11. 28. Degree 18 bandpass filters with NFTZ=4, NUE=8, Nzero=1, Ninf=1

Let’s now increase the number of FTZ’s to four and consider the following specifications:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, fq=2500 MHz, NUE=8, NFTZ=4, one at fz1,2=960 MHz and the other at fz3,4=1040 MHz, Nzero=1, Ninf=1.

- Design steps are shown in Figure 11.32. Transmission zeros are extracted in the order ue-ue-FTZ1-FTZ3-ue-ue-ue-ue-ue-FTZ2-FTZ4-zero-inf-ue-ue to give the circuit of Figure 11.32.a.

- With cursor under the series OC stubs, the command Double Norton / Inverse Geffe is applied to make the OC+SC Stub elements adjacent. Simplify Circuit command moves the transformers rightmost (Figure 11.32.b).

- Using Kuroda / Left (Right) / Full Stub command the negative and positive series OC and SC stubs  in the interior part of the circuit are moved to the middle of the circuit as shown in Figure 11.32.c. Simplify Circuit command is used to move the transformers away.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.32.d). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=2500 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.32.e). Here f0 is actually the quarter wave frequency of the filter. A slight distortion will appear in return loss which can be corrected by tuning all elements slightly, at this stage or after finishing the design.

- The series OC and SC stubs on the source side are moved to the source end by applying the command Kuroda / Right / Full Stub to form an LRight-section of OC stubs as shown in Figure 11.32.f.

- With cursor under the shunt leg of the LRight-section the command Kuroda-Minnis-EF/ Right / Specify Tlines-1,2: Z1=100, Z2=100 ohms is applied to form a Pi-section of OC stubs on the left of the Tline while setting the impedances of the next two Tlines. Also a new LRight-section of OC stubs is formed on the right of the Tlines together with a transformer. The resulting transformer is shifted away by Simplify Circuit command (Figure 11.32.g).

- The new LRight-section is converted into Pi-section by applying the command Pi-Tee-L / LRight-to-Sym Pi. Also, the series OC+SC stub element on the symmetry axis is split into two equal parts to get two symmetric halves of the circuit (Figure 11.32.h).

- With cursor under the Tline on the left of the symmetry axis the command Kuroda-Pi/ Right / Symmetric Pi is applied to transfer the series OC+SC stub element on the right of the Tline to its left as a Pi-section of OC stubs (Figure 11.32.i).

- The Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric. Symmetrize command maps the operations done on left half to the right half of the circuit (Figure 11.32.k).

- Finally the shunt SC+OC stub elements are converted into stepped resonators by using Step Resonators / Two Step Resonators command (Figure 11.32.m).

- Figure 11.32.n shows a pictorial description. It is seen that impedances of stepped stubs are higher compared to the previous example where we had only two FTZ’s, indicating that increasing the number of FTZ’s help these impedances to increase. They can be adjusted by selecting different values for inverters and/or by selecting different Tline impedances in Kuroda-Minnis transformation.

Figure 11.32. Degree 18 bandpass filter with NFTZ=4, Nzero=1, Ninf=1, and NUE=8.

Figure 11.32. Degree 18 bandpass filter with NFTZ=4, Nzero=1, Ninf=1, and NUE=8 (cont’d).

EXAMPLE 11. 29. Degree 18 bandpass filters with NFTZ=4, NUE=6, Nzero=1, Ninf=3

Let’s now try to decrease the length of the degree 18 filter of the previous case by decreasing NUE from 8 to 6 and increasing Ninf from 1 to 3:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, fq=2500 MHz, NUE=6, NFTZ=4, one at fz1,2=960 MHz and the other at fz3,4=1040 MHz, Nzero=1, Ninf=3.

- Design steps are shown in Figure 11.33. Transmission zeros are extracted in the order inf-ue-FTZ1-FTZ3-ue-ue-ue-ue-ue-FTZ2-FTZ4-zero-inf-ue-inf to give the circuit of Figure 11.33.a.

- With cursor under the series OC stubs, the command Double Norton / Inverse Geffe is applied to make the OC+SC Stub elements adjacent. Simplify Circuit command moves the transformers rightmost (Figure 11.33.b).

- Using Kuroda / Left (Right) / Full Stub command the negative and positive series OC and SC stubs in the interior part of the circuit are moved to the middle of the circuit as shown in Figure 11.33.c. Simplify Circuit command is used to move the transformers away.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=50 ohms is applied. This command places inverters at source and load ends (Figure 11.33.d). Here K=50 ohms is a trial value which can be iterated later, after inspecting the resulting element values. The value K=50 doesn’t change element values. Any other value of K would change the element values.

- With cursor under the inverters the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=2500 MHz is applied to convert the inverters into Tlines which have the same length as the other elements of the filter (Figure 11.33.e). Here f0 is actually the quarter wave frequency of the filter. A slight distortion will appear in return loss which can be corrected by tuning all elements slightly, at this stage or after finishing the design.

- The series OC+SC stub on the right of the second Tline is split into its OC and SC stub components by using the command Split / Two Element Resonator and Exchange command is used to place the OC stub adjacent to the second Tline (Figure 11.33.f).

- This OC stub is moved to the left of the adjacent Tline by applying the command Kuroda / Right / Full Stub to form a series OC+SC stub element on the right of the first Tline (Figure 11.33.g).

- With cursor under the first Tline the command Kuroda-Minnis CD/ Right / k=0.45 is applied to form a Pi-section of OC stubs on the left of the Tline. A piece of OC stub is left on the other side of the Tline together with a transformer. The transformer is shifted away by Simplify Circuit command (Figure 11.33.h).

- With cursor under the second Tline the command Kuroda / Right / Full Stub is applied to transfer the series SC stub on right as a shunt OC stub on left to form an LLeft-section of OC stubs as shown in Figure 11.33.i. Also, the series OC+SC stub element on the symmetry axis is split into two equal parts to get two symmetric halves of the circuit

- The LLeft-section is converted into Pi-section by applying the command Pi-Tee-L / Lleft-to-Sym Pi. With cursor under the Tline on the left of the symmetry axis the command Kuroda Pi / Right / Symmetric Pi is applied to transfer the series OC+SC stub element on the right of the Tline to its left as a Pi-section of OC stubs. Finally the shunt SC+OC stub elements are converted into stepped resonators by using Step Resonators / Two Step Resonators command (Figure 11.33.k).

- The Pi-sections of OC stubs are converted into OC comb type coupled lines by applying the command Replace By PCL / Combline / ococCOMB / Symmetric. Symmetrize command maps the operations done on left half to the right half of the circuit (Figure 11.33.m).

- Figure 11.33.n shows a pictorial description. It is seen that this structure is shorter  compared to the previous design because of less number of Unit Elements. The price paid for this advantage seems to be an increase in the first Tline impedance and lowering in impedances of stepped stubs.  They can be adjusted to a certain extent by selecting different values for inverters and/or by assigning a different value to the parameter k in Kuroda-Minnis CD transformation.

Figure 11.33. Degree 18 bandpass filter with NFTZ=4, Nzero=1, Ninf=3, and NUE=6.

Figure 11.33. Degree 18 bandpass filter with NFTZ=4, Nzero=1, Ninf=3, and NUE=6 (cont’d).

11.5. Distributed ELEMENT BANDPASS Filter DESIGN USING APPROXIMATE    TECHNIQUES

Some approximate techniques may also be used to design narrow band elliptic-type filters by starting from lumped element or distributed element LP prototypes [6], [14], [15], [20]. Below is a typical example using the Generalized Chebyshev LP prototype. Design / By Synthesis technique can also be used.

EXAMPLE 11. 30: Narrowband Generalized Chebyshev filters without coupled lines [6], [15]

Let’s design a lumped element Generalized Chebyshev bandpass filter and then convert it into distributed element filter using some approximate transformations (Synthesis with Lumped or Distributed Elements may also be used). Let

PBRipple=0.05 dB,  SBLoss=60 dB, fp1=975 MHz,   fp2=1025 MHz,  N=5

- Design command set is given in Figure 11.34. The resulting circuit has shunt fourth order sections  as shown in Figure 11.34.a. Each fourth order section provide a pair of FTZ’s located on the two sides of the passband.

- Find Dual command is used to get the dual circuit containing series fourth order sections (Figure 11.34.b).

- The fourth order sections are split into two second order sections (parallel LC in series arm-pLCsr) using Split command (Figure 11.34.c).

- The two second order resonators are problematic to realize in adjacent form. A shunt LC section  can be provided through Norton transformations as follows. With cursor under the first pLCsr element on left apply command Norton / Specify Transformer : k=2. Then with cursor under the next pLCsr element apply command Norton / Specify Transformer : k=0.5. These commands replace the pLCsr elements by their Norton Equivalents having the specified transformers. The values of transformers are not important in this example as long as they are inverse of each other for cancellation. Some of the shunt elements will come out to be negative. This is also not important as they will be absorbed into the positive adjacent elements. The same commands are applied to the next pair of pLCsr elements giving the circuit of Figure 11.34.d.

- Simplify Circuit command eliminates transformers and combines the adjacent shunt LC elements (Figure 11.34.e).

- Since the transformer turns ratios were selected arbitrarily in Norton transformations the resulting element values need re-adjustment. The easiest way to do this on Filpro is first to equate the shunt inductors and then use Pi-to-Pi transformations if further adjustments are necessary. The shunt inductors can be equated by placing the cursor under any pLCsr element and applying the command Pi-Tee-L / All Equal Shunt / Inductors. We get the circuit shown in Figure 11.34.f, where all shunt inductors are 0.56 nH. The price paid for this is the appearance of a transformer at load end. This transformer can be eliminated by applying the command Pi-Tee-L / Pi-to-Pi-Tee-L / Pi-to-Pi / Specify TRF to any Pi-section to produce another transformer with inverse turns ratio. However we may also leave the transformer and eliminate it later by another approach.

- Next, with cursor under the series arm LC resonators (pLCsr), the command Inverters / Replace By Ideal Inverter / K=50 is applied. This command replaces the pLCsr elements by shunt arm series resonators (sLCsh) in between two ideal inverters with inverter constant of K=50 ohms The selected inverter constant sets the values of sLCsh element. Sine these element values will be adjusted later, at this stage we let K=50, arbitrarily. Thus, now all the LC elements are in shunt forms separated by ideal inverters (Figure 11.34.g).

- Next, the shunt parallel LC resonators will be replaced by their OC stub equivalents. This is done by placing the cursor under the shunt parallel LC resonators and applying the command Replace By Stub / Par. LC-to-OC Stub (Figure 11.34.h). This command replaces the parallel LC resonator by an OC stub which is half wavelength long at the resonance frequency of 1000 MHz.

- Next, the series LC resonators in shunt arm are replaced by their OC stub equivalents by applying the command Replace By Stub / Single OC Stub: f0=1000 MHz. The resulting OC stub is quarter wave long at the passband center of 1000 MHz (Figure 11.34.i).

- The shunt OC stubs have arbitrary impedances. In order to scale up the first shunt OC stub adjacent to the source we apply command Inverters / Scale Circuit By Inserting Inverters / Specify Adjacent Stub: Z=50 ohms. This command places ideal inverters at both source and load ends scaling up all impedances besides setting the first OC stub to 50 ohms (Figure 11.34.k).

- The transformer at load end is absorbed into the inverter there by applying the command Inverters / Absorb TRF (Figure 11.34.m).

- Since all the stubs are surrounded by ideal inverters on both sides their impedances can be adjusted to any value by changing inverter constants. We can set all stub impedances to 50 ohms by placing the cursor under the element to be scaled and applying command Inverters / Scale Element By Inverters: Z=50 ohms (Figure 11.34.n).

- Finally, all the inverters are replaced by transmission lines which are quarter wave long at band center by applying the command Inverters / Replace Ideal Inverter By / Quarter Wave Tline: f0=1000 MHz (Figure 11.34.p).

- Figure 11.34.q shows a pictorial description of the circuit.

- Effects of all these approximations can be seen from return loss and insertion loss plots shown in Figures 11.34.r and s. Especially the conversion of ideal inverters into quarter wave transmission lines degrades passband return loss and causes band edge shifts. Return loss can be recovered by tuning impedances of the stubs on the two sides of the inverter right after conversion of each inverter. However band edge shifts can only be corrected by re-designing the filter with a wider bandwidth.

- Wideband insertion loss response shows the appearance of many spurious passbands. Therefore this approach is suitable only if these passbands are not important or if they can be eliminated by other filters.

Exercise: Repeat the design using Design / By Synthesis approach which enables placement of transmission zeros arbitrarily.

Figure 11.34. A narrowband elliptic bandpass filter without coupled lines.

Figure 11.34. A narrowband elliptic bandpass filter without coupled lines.

Figure 11.34. A narrowband elliptic bandpass filter without coupled lines.

11.6. ULTRAWIDEBAND BANDPASS FILTER DESIGN AS CASCADES OF LP-HP FILTERS

Bandpass filters can always be designed by cascading LP and HP filters. However if the bandwidth is not wide enough the LP and HP filters interact degrading the response appreciably.  Therefore in such filters some tuning or optimization is needed. The techniques described above may fail to give reasonable element values for very wideband filters. In such cases it may be preferable to use the LP-HP cascade approach, as will be described in the following example.

EXAMPLE 11.31. An Ultra wideband LP-HP filter

Let the specifications of the desired bandpass filter be

fp1=400 MHz, fp2=2000 MHz, PB Return Loss>10 dB (PBRipple<0.5 dB), SB Loss at 350 MHz>60 dB, SB Loss at 2500 MHz>60 dB.

In order to make PBRipple of the bandpass filter less than 0.5 dB, PBRipples of the individual LP and HP filters will be selected to be considerably less than 0.5 dB, by trials because when they are cascaded the return loss will degrade and the degradation level is usually unpredictable. In these trials realizability of the resulting element values need also be taken into consideration. The HP and LP filters will be designed independently of each other. If there is also a size (length-width) constraint then the quarter wavelength frequency fq of the LP and HP sections must be selected as high as possible. However since fq effects the element values there are limitations on these parameters also. Therefore we may have to do several trials until all the specs are set properly. Since fp1=400 MHz is a low frequency for microwave applications some distributed elements may have to be converted into lumped equivalents.

Design of the HP section:

After a few design trials the following specifications are selected which may be subjected to further revisions after the LP-HP cascade:

PBRipple=0.2 dB, fp=400 MHz, fq=2500 MHz, Nzero=3, NUE=4, FTZ1=FTZ2=FTZ3=300 MHz

The Unit Elements are placed just to create physical separation between the shunt stub resonators. They have only a few dB of contribution to the stopband loss.

- Design command set is shown in Figure 11.35. In order to get the first element as shunt SC stub, Type-B option is selected. Type-A option would lead to series capacitors at input and output (Exercise with that approach?). The numbers and positions of transmission zeros are set after some trials while observing the response.

- Transmission zeros are extracted in the order shown in Figure 11.35.a.

- After applying the indicated Kuroda transformations a symmetric structure is obtained as shown in Figure 11.35.b.

- The finite transmission zeros are extracted in the form of shunt arm series connected OC and SC stubs which are unrealizable. They can be converted into two-step stub by applying the command Step Resonator (Figure 11.35.c).

- The first step Impedances of the two-step resonators are too high while those of the seond steps are too low. The high impedances can be scaled down by moving the series OC stubs on one side of the transmission lines near the ends of the circuit to the other side by using Kuroda transformations as shown in Figure 11.35.c-d. Applying Simplify Circuit command moves away the transformers to the load end scaling down all internal impedances. This circuit is saved as HP1.fil to be used in later stages for finding other possible solutions.

- Next, the low second step impedances are converted into lumped capacitors by placing the cursor under the stepped resonators and applying command Replace By Lumped / Stub+Cap: f=400 MHz. This transformation converts the two-step stubs into single step stubs with lumped capacitive loadings (Figure 11.35.e). This approximation is carried out at the passband edge frequency to prevent any shift at this frequency.

- Finally, the series OC stubs are converted into lumped capacitors by applying the command Replace By Lumped / Single C: f=400 MHz, as shown in Figure 11.35.e-f.

- Figure 11.35.g shows a pictorial description of the filter. The capacitors are shown as black patches. Length of the filter is set by the four Unit Elements only. Therefore if a shorter filter is desired the quarter wavelength frequency can be increased (Exercise?). The number of unit elements can’t be deceased because they provide physical separation between the shunt stub resonators. The transversal size is set by the length of the shunt stubs. It is also possible to place the succeeding shunt stubs on the two sides of the mainline if larger transversal size is of no problem.

- Figure 11.35.h shows response of the filter. Both Insertion and Return Loss responses are degraded to some extent above 2000 MHz which will be the upper passband edge of the combined LP-HP filter. There is not much degradation within the band 400-2000 MHz.

- Figures 11.35.i-to-q describe alternative possibilities. The circuit of HP1.fil is loaded and with cursor under the Unit Elements the command Kuroda / Distribute Stub is applied (Figure 11.35.i-k). This command translates part of the stub on one side of the Unit Elements to the other side such that the stubs on the two sides will be equal. We could as well use Kuroda / Right-Left / … commands (Exercise?). These transformations increase all impedances in the interior part of the circuit.

- Then all the series OC stubs are converted into series lumped capacitors by applying the command Replace By Lumped / Single C: f=400 MHz (Figure 11.35.m). It should be noted that the resulting capacitor values are larger than those in the previous approach.

- Next, the two-step stubs can be converted into capacitor loaded stubs as was done in the previous approach. However in Figures 11.35.m-n, an alternative approach is shown where the two step stubs are replaced by lumped series LC resonators by applying the command Replace By Lumped / LC Resonator: f=400 MHz. This approach is possible if the lumped inductors work as an inductor over broad bands.

- Finally the shunt SC stubs at the ports may also be converted into lumped inductors by applying the command Replace By Lumped / Single L: f=400 MHz (Figure 11.35.p).

- Figure 11.35.q shows the responses. It is seen that little degradation exists in both return loss and insertion loss over a broad band.

Figure 11.35. Design of the HPF section of the LP-HP bandpass filter.

Figure 11.35. Design of the HPF section of the LP-HP bandpass filter.

Figure 11.35. Design of the HPF section of the LP-HP bandpass filter.

Design of the LP section:

There are many possible alternatives for the LP section (See CH-08). As a trial example the following LP filter is designed:

PBRipple=0.1 dB, fp=2000 MHz, fq=5000 MHz, Ninf=1, NUE=6, FTZ1=FTZ2=FTZ3=2275 MHz

The quarter wavelength frequency fq must be selected by the stop bandwidth requirements. It also effects impedances of the elements together with the PBRipple and degree of the filter.

- The design command set is given in figure 11.36. Elements are extracted in the order shown in Figure 11.36.a.

- The series SC stubs are converted into shunt OC stubs by applying Kuroda transformations (Figure 11.36.b).

- The shunt arm series combination of SC and OC stubs are converted into two-step resonators by applying Step Resonator command. The resulting impedances seem to be realizable. Thus, the design is finished. It is saved as LP1.fil.

Formation of the LP-HP filter:

The targeted LP-HP type bandpass filter can be formed by cascading the filters LP1.fil and HP2.fil. This is described in Figures 11.36.d-e.

- LP1.fil is loaded. We may as well load HP2.fil.

- With cursor under the load end (or source end), the command Append Filter / HP2.fil is applied. This command sticks HP2.fil to the load end of the LP1.fil.  Thus the bandpass filter is formed.

- Insertion loss and return loss responses are shown in Figure 11.36.f. It is seen that the passband return loss and selectivity requirements are met. The spurious passband is about 8000 MHz. If necessary it can further be moved away by selecting higher quarter wavelength frequeny fq for the LP filter (Exercise?).

Figure 11.36. Design of the LP filter and formation of LP-HP filter.

Figure 11.36. Design of the LP filter and formation of LP-HP filter.

REFERENCES  AND FURTHER READING

- P. R. Geffe, “Simplified Modern Filter Design”, Hayden Book Co., NY, 1963.

- M. C. Horton and R. J. Wenzel, “General Theory and Design of Optimum Quarter Wave TEM filters”, IEEE Trans. MTT,  vol. MTT-13, No. 3, pp. 316 - 327, May 1965.

- R. Levy, “A General Equivalent Circuit Transformation for Distributed Networks”, IEEE Trans. CT, Vol. CT-12, No 3,  pp. 457-458, Sept. 1965.

- R. Levy and I. Whiteley, “Synthesis of Distributed Elliptic Function Filters from Lumped Constant Prototypes”, IEEE Trans. MTT, Vol. MTT-14, No. 11,  pp. 506-517, Nov. 1966.

- M. C. Horton and R. J. Wenzel, “The Digital Elliptic Filter- A Compact Sharp Cut-off Design for Wide Bandstop or Bandpass Requirements”, IEEE Trans. MTT, Vol. MTT-15, No 5,  pp. 307-314, May 1967.

- I. Rubinstein, R. L. Sleven, A. F. Hinte, “Narrow bandwidth Elliptic Function Filters”, IEEE Trans. MTT, Vol. MTT-17, pp. 1108- 11115, Dec. 1969

- R. Levy, and J. D. Rhodes, “A Combline Elliptic Filter”, IEEE Trans. MTT, vol. MTT-19, pp. 26-29, Jan. 1971.

- G. Hoffmann, “ The Direct Synthesis of Optimum Commensurate Distributed Filters by Digital Computer”, 1973 European Microwave Conference Proc., B.2.2, Brussels, 1973.

- J. D. Rhodes, “Theory of Electrical Filters”, London, England, Wiley, 1976.

- M. C. Horton, “ TEM Bandpass Filters Having Eight Wavelength Coupled Stubs”, 1977 IEEE, MTT- S Int. Microwave Symposium Digest, San Diego, 1977.

- J. A. G. Malherbe, “Microwave Transmission Line Filters”, Artech House, 1980.

- J. B. Minnis, “Classes of Sub Miniature Microwave Printed Circuit Filters with Arbitrary Passband and Stopband Widths”, IEEE Trans. MTT, Vol. MTT-30,  pp. 1893 - 1900,  Nov. 1982.

- P. La Tourette, “High Pass Filter Design”, Proc. EuMC’82, pp. 233 - 238, 1982.

- C. I. Mobbs and J. D. Rhodes, “A Generalized Chebychev Suspended Stripline Bandpass Filter”, IEEE Trans. MTT, Vol. MTT- 31,  pp. 397- 402,  May 1983.

- J. Ness  and S. Johnson, “Narrowband Elliptic Filters on Microstrip”, Microwaves and RF, Nov. 1984.

- J. Ness and  J. Dougall, “Microstrip Filters for Lower Frequencies”, RF Design, pp. 52- 61, Jan. 1987.

- Y. Sen, “Direct Design of Lumped and Distributed Filters Using Transformed Variable Techniques”, M. Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1989.

- J. H. Kotze, “Classes of Bandpass Filters Having an Arbitrary wide Stopband  and Design Tables for One Such Class”, IEEE MTT-S, pp. 123 - 126, 1990.

- M. M. Akkul, “Design and Realization of Microwave Bandpass Filters in Stripline and Suspended Stripline Structures”, M. Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1991.

- I. Tekin, “Design and Realization of Narrowband Microwave Bandpass Filters Using  Stripline-like Structures”, M. Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1992.

- J. C. Tippet and M. J. Shian, “Circuit Transformations for realization of a Class of  Miniature  Printed Circuits”, IEEE,  MTT-S  Digest, pp. 621- 624, 1994.

- U. Koyaz, “Synthesis of Lumped and Distributed Filters and Diplexers”, Ph. D. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1994.

- M. Karaaslan, “Network Transformations for Realization of Lumped and Distributed Filters”, M. Sc. Thesis, EEE Dept. Middle East Technical University, Ankara, Turkey, 1994.