CHAPTER-12

INTERDIGITAL, COMBLINE

AND

MIXED DIGITAL FILTERS

CONTENT                                                                                                             	Page

PART-I: BASIC CONCEPTS AND MEDIUM-WIDE BANDWIDTH

INTERDIGITAL, COMBLINE AND MIXED DIGITAL FILTERS

12. 1. INTRODUCTION												12.4

12. 2.  KEY CONCEPTS ON INTERDIGITAL AND COMBLINE STRUCTURES	12.5

12. 3. MEDIUM BANDWIDTH INTERDIGITAL AND CAPACITIVELY LOADED INTERDIGITAL FILTERS											12.24

12.  4. MEDIUM BANDWIDTH COMBLINE FILTERS						12.31

12. 5. MEDIUM BANDWIDTH MIXED DIGITAL FILTERS AND COMPARISON OF RESPONSES														12.35

12. 6. THE USE OF EVEN-ODD MODE IMPEDANCE CONCEPTS			12.41

12. 7. COUPLING COEFFICIENT, EXTERNAL Q AND NODAL RESONANCE    FREQUENCY CONCEPTS											12.45

12. 8. USING REFLECTION ZEROS, PHASE AND DELAY OF S11 FOR       MEASUREMENT AND TUNING										12.67

12. 9. DISPLAYING VOLTAGE, CURRENT AND POWER AT NODES			12.71

12. 10. FINITE TRANSMISSION ZEROS IN COMBLINE AND MIXED DIGITAL              FILTERS															12.80

12.  11. WIDEBAND INTERDIGITAL AND CAPACITIVELY LOADED                  INTERDIGITAL FILTERS											12.91

12. 12. WIDEBAND INTERDIGITAL FILTERS WITH OPEN CIRCUITED                TERMINATING LINES												12.101

PART-II: NARROWBAND INTERDIGITAL, COMBLINE AND MIXED DIGITAL FILTERS

12. 13. NARROWBAND INTERDIGITAL FILTERS						12.106

12. 14.  BRUTE FORCE FINITE TRANSMISSION ZEROS IN INTERDIGITAL

FILTERS															12.125

12. 15. NARROWBAND CAPACITIVELY LOADED INTERDIGITAL FILTERS	12.130

12. 16. NARROWBAND COMBLINE FILTERS							12.159

12. 17. NARROWBAND MIXED DIGITAL FILTERS						12.180

12. 18. STEPPED DIGITAL FILTERS AND SPURIOUS PASSBANDS			12.185

12. 19. NARROWBAND COMBLINE AND MIXED DIGITAL FILTERS

WITH FINITE TRANSMISSION ZEROS									12.191

12. 20.  BRUTE FORCE FINITE TRANSMISSION ZEROS IN COMBLINE

FILTERS															12.198

12. 21.  BLOCK DIELECTRIC FILTERS									12.205

12. 22. INHOMOGENEOUS  BLOCK DIELECTRIC FILTERS				12.219

REFERENCES AND FURTHER READING								12.223

PART-1

BASIC CONCEPTS

AND

MEDIUM-WIDE BANDWIDTH

INTERDIGITAL, COMBLINE AND MIXED DIGITAL

FILTERS

CHAPTER-12

INTERDIGITAL, COMBLINE AND MIXED DIGITAL FILTERS

12. 1. INTRODUCTION

Interdigital, capacitively loaded interdigital, combline and mixed digital filters are formed by N-parallel coupled lines with each line (bar) being short circuited to ground at one end while the other end is either left open or terminated in a capacitive reactance. These filters belong to the family of direct coupled resonator filters. That is, in such N-coupled line filters each line, together with the capacitive loading (if exists), acts as a shunt resonator. The difference between combline and interdigital filters is that in combline filters the SC ends of bars are on the same side (Figure 12.1.a) while in interdigital filters the SC ends alternate (Figure 12.1.b). Conventionally the name interdigital is used for filters without capacitive loadings. In order to differentiate them the name Capacitively Loaded Interdigital Filter will be used if capacitive reactances exist. That is the classical interdigital filters are special cases of capacitively loaded interdigital filters obtained by setting capacitors to zero (Figure 12.1.c). However considering the fringing capacitors at the open ends, these filters can also be handled as capacitively loaded interdigital filters. Figure 12.1.d shows  mixed digital structure where both comb and interdigital type couplings are present.

Figure 12.1. Combline and interdigital filter structures.

Historically combline and interdigital filters have developed along different paths, as completely different filters, having their own special design tools and techniques. On Filpro both filter types can be synthesized by using the same tools. The old special design tools are also included for the sake of completeness and for their historical values. In some occasions these techniques may also be prove to be useful.

The N-parallel coupled line part of these structure needs special treatment to function properly as the core sections of filters. In general analysis of an N-parallel coupled conductor system need be handled through Electro-Magnetic simulation tools. The synthesis stage, that is, developing a proper circuit model to be linked to the filter synthesis tools and then finding the physical dimensions from the element values of the circuit requires a number of simplifying assumptions as follows:

- It will be assumed that only pure TEM or quasi-TEM modes of electromagnetic waves exist in the medium of operation. This assumption enables us to use the commensurate length distributed element filter concepts to develop all-stub circuit models which can be linked easily to the filter synthesis tools.

- Another usual assumption is that the resonator bars are coupled to only the adjacent resonators. That is, all non-adjacent couplings are neglected. This assumption leads to a simple ladder type circuit model which can be converted into a direct coupled resonator form by simply adding the resonating capacitors.

In general these assumptions may hold to varying degrees in different realization structures, like slabline, stripline, microstrip, waveguide, etc. Fortunately, degradation in response because of such non-adjacent couplings and/or because of existence of non-TEM and evanescent wave modes can be corrected to a great extent by simple tuning or optimization either at circuit theory level or during EM analysis and realization stages.

TEM wave assumption enables description of single and multiple coupled transmission lines in terms of static capacitance parameters of the system. The basic definitions related to these concepts are summarized in the following sections together with the derivation of circuit models of interdigital and comb type filters.

12. 2.  KEY CONCEPTS ON INTERDIGITAL AND COMBLINE STRUCTURES

In this section the key concepts related to interdigital, combline and mixed filters will be summarized on medium-wideband filter examples. Further details specific to interdigital, capacitively loaded interdigital, combline and mixed digital filters will be studied in later sections.

A)  Characterization of single TEM Mode Transmission Lines and SC stubs

A single lossless transmission line operating in TEM mode (a two wire line) can be described by its characteristic impedance Z (or admittance Y=1/Z) and its phase constant which are defined as

Z0=1Y0=LC, 		β=ϖLC

where L and C are per unit length series inductance and shunt capacitance of the line, as shown in Figure 12.2.a.

Figure 12.2. Static capacitance model of a TEM-mode transmission line.

In this figure the transmission line is depicted as a two wire line with currents flowing in opposite directions. An infinitesimal length x of the transmission line can be represented by its series inductance Lx and shunt capacitance Cx. The electric and magnetic field lines of a slab type transmission line are also shown in the same figure. For transmission lines operating in TEM mode, L and C are related through the electromagnetic parameters  randr of the medium as

LC=με=μ0μrε0εr=μrεrc2=1v2

where c is the velocity of light, r is relative permeability of the medium (unity for non-magnetic dielectrics), r is the relative permittivity of the medium (dielectric constant) and

v=1με=cμrεr

is the phase velocity of the wave. In a non-magnetic dielectric medium, taking r=1, we have

v=cεr

Thus, a TEM mode transmission line can be characterized by a knowledge of only capacitance C per unit length or only inductance L per unit length, together with the dielectric constant r. Since calculation of capacitance is usually easier to calculate for most structures, it is usual to use this parameter to simplify the definition of characteristic impedance and propagation constant:

Z0=1Y0=LC=1vC=εrcC

β=ϖLC=ωv=ωεrc=2πfεrc

By using the definition of guided wavelength ,

λ=vf=cfεr=λ0εr

with 0 =c/f being the free space wavelength, the expression for the phase constant may also be written in terms of wavelength as

β=2πfεrc=2πλ=2πεrλ0

The capacitance parameter can be calculated through the electrostatic field definition

Q=CV

where Q is the total charge per unit length and V is the amplitude of the voltage wave travelling on an infinitely long conductor. This capacitance is commonly termed as the static capacitance of the transmission line, because the electric field lines of the waves are the same as those in static electric field systems.

A transmission line piece of physical length d can be characterized as follows (Figure 12.3.a):

- The electrical length  of a transmission line with physical length d is then defined as

θ=βd=2πdλ=ωdv=ωdεrc=2πfdεrc

where is the wavelength at frequency f, in the medium with dielectric constant r.

- The frequency at which the physical length d becomes equal to quarter wavelength is termed as the quarter wave frequency fq. Using d=/4 in the above equation it is seen that at this frequency the electrical length is equal to /2:

θ(fq)=2πdλ=π2

- Using this result we can get an expression for fq in terms of the line length d as follows:

θ(fq)=2πfqdεrc=π2

fq=c4dεr

for the usual non-magnetic dielectric media.

- The electrical length at any frequency f  may also be written as

θ(f)=(π2)(ffq)

- When such a transmission line is used as a two-port element, it can be characterized in terms of its ABCD parameters:

A=D=cos,  B=jZ0sin ,  C=jY0sin

- An all-stub Pi and Tee models of such a transmission line are shown in Figure 12.3.b. The Pi-model consists of a parallel combination of a positive SC stub and a negative OC stub in series arm surrounded by two shunt SC stubs, all having half the length of transmission line. The Tee-model has a series combination of a negative SC stub and a positive OC stub in shunt arm surrounded by series SC stubs, all having halved lengths. The parallel and series combinations of SC and OC stubs, one having negative impedance indicates that these elements create a finite transmission zero on real ()axis of S=+j domain. Figure 12.3.c shows all-lumped approximations of a transmission line that are available in Filpro.

- When a transmission line piece of length d is short circuited at one end it becomes a SC stub (Figure 12.4.a). It is characterized by its input impedance:

Zsc(f)=jZ0tanθ(f)=jZ0tan(πf2fq)

- At the quarter wave frequency fq, =/2, hence tan=. So, at about f=fq, a SC stub behaves like a parallel LC resonator. Therefore if a SC stub appears in series arm, it creates a finite transmission zero at fz=fq and if it is connected in shunt arm then it acts as a shunt connected parallel resonator that can be used as resonator element of coupled resonator filters with passband center at f0=fq (Figure 12.4.b).

- For frequencies less than fq, a SC stub presents an inductive reactance. It can be resonated by connecting a capacitive reactance in parallel. Therefore a shunt SC stub in parallel with a capacitive reactance can be used as resonator element of coupled resonator filters with passband center f0<fq and a series arm SC stub in parallel with a capacitive reactance can be used to create a finite transmission zero at fz<fq (Figure 12.4.c).

- Figure 12.4.d shows another circuit model for SC stubs. It is formed by paralleling a SC and an OC stub with doubled impedances and halved lengths. This exact identity can be used in manipulation of filters for finding different equivalent circuits.

Figure 12.3. Characterization of a transmission line of length d.

Figure 12.4. Characterization of a SC stub.

- Figure 12.5 shows the standing wave patterns for voltage (electric field) and current (magnetic field) on a SC stub at different frequencies when it is excited at the other end. In all cases electric field is zero (zero voltage) and magnetic field is maximum (maximum current) at the SC end.

- Figure 12.5.a shows the case for f=2fq at which the length d is equal to half wavelength. At this frequency one full standing wave pattern appears along the line. Electric field magnitude (or voltage) starts from zero at the SC end, becomes maximum at a quarter wavelength distance to the SC end, that is, in the middle of the line and becomes zero again at the source end. Current (magnetic field) shows the opposite behaviour. That is it is maximum at the SC end, minimum in the middle point and maximum (with changed direction) again at the source end. It should be noted that the direction of magnetic field reverses in the middle of the stub. The positions of maximum electric field is important because of the possibility of dielectric breakdown and the positions of maximum current is important because of excessive heating-melting in high power applications.

- Figure 12.5.b shows the case for f=fq in which case the stub length becomes a quarter wavelength. For this case the position of maximum electric field and minimum magnetic field shifts to the source end. This is a critical pattern because many filters use quarter wavelength stubs. The occurrence of maximum electric field (maximum voltage) at the source end may necessitate care for connector types used at this end of the stub in high power applications. At this frequency the input impedance seen at the source end is infinite and the stub can be treated as a parallel resonator. Current is maximum at the SC end. Note that direction of the magnetic field remains the same over the line.

- Figure 12.5.c and d show the cases where f<fq. Since the electrical length is less than a quarter wavelength, the first maximum of the electric field (minimum of magnetic field) can’t be reached over the line. Since the electrical length of the stub is short, little variation is observed in electric field and magnetic field. That is, electric field remains weak while magnetic field remains strong everywhere over the line. This property is used to get magnetic (inductive) coupling between adjacent parallel coupled lines which are shorter than a quarter wavelength.

Figure 12.5. Standing wave patterns over a SC stub at different frequencies.

In synthesis stage of a filter usually only the impedances and lengths of stubs and transmission lines are determined. However in realization stage we need physical dimensions (width and thickness of a line and spacings to ground planes) are needed. If the medium is homogeneous then the parameters like dielectric constant r, wavelength, phase velocity and quarterwave frequency are also known. Therefore the problem is determination of width-thickness-ground plane spacings.  For this purpose first the capacitance C per unit length is found from the impedance Z0 using the above expressions and then the physical dimensions are calculated by using the available tools like analytical formulations, graphs, charts or through electromagnetic simulation tools to realize this capacitance. In an inhomogeneous medium where permittivity r is different in different parts of the medium, an effective permittivity r,eff  need be used in the formulations of phase velocity, wavelength and quarter wavelength frequency fq and capacitance. Usually effective permittivity is also a function of frequency (dispersive medium). Therefore In such applications the effective permittivity appears as an unknown also. Further, practical lines involve discontinuities of diverse kinds. Therefore in general iterative trials through electro-magnetic simulation tools may be needed until the desired capacitance and hence the desired characteristic impedance is obtained 26-48.

B) Characterization of a coupled line pair by three-transmission-line model and capacitance matrix

Figure 12.6.a shows a pair of parallel coupled conductors above a ground plane which forms the third conductor. Such lines may also be called as three conductor lines. The left nodes of the lines are denoted by 1a and 2a while the right nodes are denoted by 1b and 2b respectively. Different one-port, two port and three port circuit elements can be obtained by terminating the ports in various ways, like open circuits, short circuits, stubs or lumped elements. Such coupled lines can be characterized in several different ways, as follows.

An infinitesimal length x of the transmission line system can be approximated by the lumped per unit length capacitors C10x-L10x, C20x-L20x and C12x-L12x as shown in Figure 12.6.a. Here C10x, C20x are capacitances from line-1 to ground and line-2 to ground respectively while C12x is the capacitance between the two lines. Similarly, L10x and L20x are the self inductances of the two lines and L12x is the mutual inductance between the lines. TEM mode assumption enables us to treat this system as consisting of three two-wire transmission line systems as shown in the figure. These two-wire-lines are formed by Line-1 and ground (Line-1-0), having the parameters L10-C10, Line-2 and ground (Line-2-0) having the parameters L20-C20 and Line-1 and Line-2 (Line-1-2) having the parameters L12-C12.  The relation

L10C10=L20C20=L12C12=με=μ0μrε0εr=μrεrc2=1v2				(12.2.1)

holds for all three two-wire lines also. Therefore these lines can be characterized by their own characteristic impedances (admittances):

Z10=1Y10=L10C10=1vC20=εrcC10

Z20=1Y20=L20C20=1vC20=εrcC20

Z12=1Y12=L12C12=1vC12=εrcC12							(12.2.2)

Hence the four port formed by the three transmission lines can be characterized by the static per unit length capacitances C10, C20 and C12 which can be defined as

Q1=C10 V1+ C12 (V1-V2) = C11 V1 – C12V2

Q2=C12 (V2-V1) + C20 V2 = -C12 V1 +C22 V2								(12.2.3)

C11=C10+C12

C22=C20+C12

where Q1 and Q2 are charges per unit length over the two lines, V1 and V2 are the voltages of the lines with respect to ground plane. In this description, the parameter C11 is the total capacitance per unit length of line-1 when the other line is grounded, that is V2=0 while C22 is the total capacitance per unit length of line-2 when line-1 is grounded (V1=0). These capacitances can be related to the physical dimensions of the structure through various means, like EM simulations, empirical or exact analytical formulations, graphs, charts, etc [38]-[60].  Thus, a coupled line pair can be characterized as a four port circuit formed by these three two-wire line systems.  The ports 1a, 1b, 2a and 2b can be terminated in different loads to form a variety of one-port, two-port and three-port circuit sections that can be used as elements of filter elements. In the following subsections the use of these circuit models in developing interdigital and combline filters will be described.

Figure 12.6. a) Three-transmission-line model of a coupled line pair and capacitance matrix description. b)  Coupling of a signal on line-1 to line-2.

C) Wave description of the coupling mechanism in a coupled line pair

In the capacitance matrix (three transmission line) description, coupling between the two lines is simulated by the transmission line Y12=vC12. Thinking in terms of per unit length parameters C12 and L12, it is clear that coupling between the two wires is provided by both of these elements. In order to get more insight on the coupling mechanism let’s inspect the structure using basic differential equations governing voltage (electric field) and current (magnetic field) waves.

Denoting the voltages and currents on the two coupled lines by V1, V2 and I1, I2 (Figure 12.6.a), the equations relating these quantities are as follows:

∂V1∂x=−jω(L11I1+L12I2)

∂I1∂x=−jω(C11V1+C12V2)

∂V2∂x=−jω(L12I1+L22I2)

∂I2∂x=−jω(C12V1+C22V2)												(12.2.4)

These equations tells us that voltage and current on Line-1 has contributions  from Line-2 through the terms L12 and C12. Similar for the voltage and current on Line-2. Let’s define nominal characteristic impedances of the two lines as

Z11=L11C11 and Z22=L22C22   											(12.2.5)

Note that these characteristic impedances are different from the impedances defined for the three transmission line model. In this description the impedance Z11 is defined when line-2 is grounded and Z22 is defined when Line-1 is grounded.

Let’s also define forward and backward scattering waves as a1, a2 and b1, b2 respectively, as follows:

a1=V1Z11+I1Z11

b1=V1Z11−I1Z11

a2=V2Z22+I2Z22

b2=V2Z22−I2Z22												(12.2.6)

Then Equations in 12.2.4 can also be written in the following forms:

da1dx=jβ1a1+jβ1β22(ke−km)a2+(ke+km)b2

db1dx=jβ1b1+jβ1β22(ke−km)a2−(ke+km)b2

da2dx=jβ1a2+jβ1β22(ke−km)a1+(ke+km)b1

db2dx=jβ1b2+jβ1β22(ke−km)a1−(ke+km)b1							(12.2.6)

where 1and 2  are the phase constants defined as:

β1=ωL11C11 	and 	β2=ωL22C22								(12.2.7)

The constants ke and km are referred to as electric (capacitive) and magnetic (inductive) coupling coefficients defined through the per unit length parameters as follows:

ke=C12C11C22 	and 		km=L12L11L22								(12.2.8)

Thus, for example the forward travelling wave a1 on line-1 has contributions from both forward and backward travelling waves a2 and b2 on line-2. Similarly, the forward travelling wave a2 on line-2 has contributions from both forward and backward travelling waves a1 and b1 on line-1. These contributions are made through both electric and magnetic coupling elements C12 and L12. The following observations may be noted:

- It is seen that the total coupling coefficient between waves travelling in the same direction (co-directional waves) is ke-km. That is, the electric and magnetic couplings between co-directional waves tend to cancel each other. This means a weak coupling between co-directional waves.

- The total coupling coefficient between oppositely directed waves (contra-directional waves) is ke+km. That is, the electric and magnetic couplings act in additive manner to enhance the oppositely directed waves. Hence we have strong coupling between contra-directional waves.

- By adjusting the electric and magnetic coupling coefficients (by adjusting separations between the lines and line dimensions) it is possible to make ke-km=0 and hence decouple the co-directional waves completely (as is done in directional couplers!). In such cases coupling between the two lines will be provided though coupling between contra-directional waves, having a total coupling coefficient of 2ke=2km.

- In filtering applications line dimensions and separations are usually set by some other constraints. Therefore in general we have couplings between both co-directional waves and contra-directional waves, making analysis a bit more complicated. That is in parallel coupled lines when a wave is excited on one line in one direction, it creates waves in the second line which travel in both directions. However it is clear that the contra-directional wave on the second line which is created by the net coupling coefficient ke+km will be stronger than the co-directional component which is created through the net coupling coefficient ke-km.

- Length of coupling region effects the amount of coupled energy. Directions of magnetic field or polarity of electric field may change along the coupling length. Thus, the energy coupled in one region may be cancelled by the energy coupled in the other region.

- When both forward and backward travelling waves exist on a line then there will be standing waves over the line where strong and weak fields follow each other periodically. If the coupling region coincides with strong magnetic field region than coupling will be mostly through the magnetic fields (inductive coupling) while if the coupling takes place along a strong electric field region then electric (capacitive) coupling will dominate. The question of whether the magnetic and electric couplings will be additive or subtractive depends also on the relative phases of forward and backward travelling waves.

- Since the relative strength of forward and backward waves are set by the terminations at ports, energy interchange between the lines is also a function of terminations. The positions of peaks and minima of the standing waves are set by the relative magnitudes and phase of the terminations. Therefore the inductive or capacitive or resistive parts of the terminations will also be effective on the amount of coupled energy. For example if the lines have SC or OC terminations then there will be standing waves formed by equal amplitude forward and backward travelling waves. Therefore a coupled line formed by SC or OC stubs will have net couplings through both contra-directional and co-directional components, with contra-directional coupling dominating. The nature of coupling will depend on whether coupling region coincides with the maxima of magnetic fields (inductive coupling) or maxima of electric fields (capacitive coupling) or both.

D) Generalized Norton Transformations

Norton transformations are extremely useful tools for producing equivalent circuits by increasing redundancy. Although originally developed for lumped elements, they are equally valid for distributed elements. They are shown in Figure 12.7.a, on lumped elements. When applied on a series element the element is replaced by a Pi-section formed by the same kind of elements together with a transformer. Depending on the turns ratio n of the transformer one of the shunt elements comes out to be negative. When Norton transformation is applied on a shunt element it is replaced by a Tee-section formed by the same kind of elements together with a transformer. Again one of the series elements comes out to be negative. When Norton transformation is applied on an element of a circuit the resulting negative element can be used to eliminate a neighboring positive element while the transformer can be used to scale values of some elements of the circuit.

Figure 12.7.b shows application of Norton transformations to L-sections (named as LLight and LLeft on Filpro), Pi-sections and Tee-sections formed by elements of identical kind, all inductors, all capacitors or all resistors.  These sections can be converted into different forms by applying Norton transformations on one or more elements and selecting the turns ratios properly. On Filpro these multiple Norton transformations are named as Pi-Tee-L transformations.

Norton transformation can be generalized to distributed elements. This is possible by considering the lumped element circuit model of an infinitesimal length x of a coupled line pair. The elements L10x -C10x and L20x -C20x form the shunt legs of a Pi-section while the elements L12x –C12x form the series element of the Pi-section. That is, it is also applicable to a transmission line piece or a series or shunt OC or SC stub.  Therefore the Pi-Tee-L transformations are also applicable to such a Pi-section to form an LRight, LLeft, Pi or Tee sections. Since this is valid for all points along the lines, one can form different equivalent coupled line circuits. The transformer resulting in each section is ineffective except at the ports.  In Figure 12.8 this is described on a four port circuit formed by two parallel coupled lines. In Figure 12.8.a the Three-Line model of the parallel coupled line pair is shown. In Figure 12.8.b the cross-sectional view is shown. The cross-sectional picture is a Pi-section formed by three transmission lines. In this figure Pi-to-Pi transformation is shown. This transformation can be used to adjust the admittances Y10 and Y20 of the lines. The resulting transformer appears at both port-2a and port-2b.

In Figure 12.8.c, Pi-to-LRight transformation is shown. This transformation reduces the number of transmission lines in the model into two. These transformations can be applied to get various equivalent forms that can be used in derivations of equivalent one-port, two-port and three-port circuit models when some of the ports are terminated in different manners, like OC, SC or stubs or other kinds of elements.  Two examples follow: Interdigital type coupled line pair and comb type coupled line pair which will be used in the filters that will be described in this chapter.

Figure 12.7. Norton and Pi-Tee-L transformations.

Figure 12.8. Norton and Pi-Tee-L transformations generalized to coupled lines.

E) Interdigital Type Coupled Line Pair:

Figure 12.9.a shows development of stub and transmission line model of a two-port interdigital type coupled line. It is obtained by grounding port 1b and port 2a of the three-two wire line model. Looking from the input port 1a, Line-1-0 appears as a SC stub between node-1a and ground. Similarly, at output port (node 2b), Line 2-0 appears as a shunt SC stub. Line-1-2 appears as a transmission line connecting the input node and output node. It should be noted that at the input side one of the two conductors of Line-1-2 is grounded while at the output port the other conductor is grounded. That is, the wave travelling on Line-1-2 changes phase by 180 degrees with reference to ground. This property is taken into account by placing a 1:-1 phase changing transformer in the stub-transmission line model of the two-port shown Figure 12.9.b. So, the two-port formed between port-1a and port-2b is equivalent to a transmission line of characteristic admittance Y12 surrounded by two shunt SC stubs of characteristic admittances Y10 and Y20, followed by a phase changing transformer. This transformer is ineffective for amplitude filtering applications. Therefore it can be neglected.

Figure 12.9. Stub+Transmission line model of an interdigital coupled line and Kuroda transformation for equating shunt SC stubs.

Using the relations Yij=vCij, we can use the static capacitance matrix parameters for calculating the physical dimensions for realization of these admittances.

In practice it is desirable to have identical lines in a coupled line system. We can always convert an asymmetric coupled line system (with Y10Y20) into a symmetric coupled line pair (Y10=Y20). Conventionally this is done by capacitance matrix transformations, as described by Wenzel [8]. Capacitance matrix transformation is a version of generalized Norton transformations described in the previous section, applied on capacitance matrix instead of the characteristic impedances.  On Filpro, Y10 and Y20 of an interdigital coupled line pair is equated by using Kuroda Transformations, as described in Figure 12.9.c. Thus it is seen that Kuroda transformations are also special cases of Generalized Norton transformations.

Figure 12.10 gives a qualitative descriptions of coupling mechanism in an interdigital type coupled line. In Figure 12.10.a the element coupling the shunt resonators (shunt SC Stubs) is shown as a transmission line of impedance Z12, physical length d, and electrical length . In the figure the transmission line is replaced by its all-stub equivalent which has a paralleled OC and SC stubs of impedances –2Z12 and 2Z12 respectively surrounded by two shunt SC stubs of impedance Z12, all having halved length d/2 (electrical length /2).  In this circuit model the coupling element appears as a series arm paralleled OC+SC Stub element with negative OC stub. The impedances of SC and OC stubs are:

Zsc(ω)=j2Z12tanθ(ω)=j2Z12tan(πω2ωq)

Zoc(ω)=−2Z12jtanθ(ω)=j2Z12/tan(πω2ωq)

The total coupling impedance is obtained by paralleling Zsc() and Zoc() is:

Zcp(ω)=Zsc(ω)Zoc(ω)Zsc(ω)+Zoc(ω)=jZ12sinθ(ω)=jZ12sin(πω2ωq)=jX(ω)

X(ω)=Z12sin(πω2ωq)>0  for 0<<2q

where X(w) is the total coupling reactance and it remains positive (inductive) and finite on the whole jaxis.

Since coupling impedance between the two shunt resonators can’t become infinite no transmission zero exists between the two resonators on s=j axis. That is coupling exists at all frequencies. The amount of coupled energy depends on the relative impedances of the shunt elements. For example the shunt SC stubs resonate at =q, becoming open circuit. Therefore at this frequency all the input signal passes to the output port through the transmission line while at the other frequencies below q they present a finite impedance, thus drawing some of the input power. So we get a passband about the quarter wave frequency q.

Figure 12.10.a shows also the electric and magnetic field standing wave patterns over the lines. The current applied at input port 1a travels along the line and reflected back from the SC at port 1b, forming a standing wave with maximum H-field (maximum current) and zero E-field (zero voltage) at SC end. The same pattern appears over the second line, in reversed form, with maximum H and minimum E fields at the opposite end, at port 2a. The TEM mode assumption enables us to use the three two-wire-line representation. In this model the coupling between the two rods (resonators) is provided by the electromagnetic fields travelling in the medium between Line-1 and Line-2, that is, on  Line-1-2 of impedance Z12. The TEM waves in this two-wire line are created by oppositely travelling currents. The magnetic and electric fields of these currents add up in the region in between the two lines. Both magnetic and electric fields transfer power to the second line in aiding manner. Therefore there is no cancellation of power transfer, hence no decoupling of the resonators at any frequency on s=jw axis which means no transmission zero, verifying the observation we have made above. The standing wave patterns gets maximum amplitude variation along the line at frequency f=fq which is the resonance frequency of the rods. At this frequency the electric fields (voltages) at port 1a and 2b get their maximum possible values and magnetic fields (currents) get their maximum values at the SC end. Therefore in high power applications dielectric breakdown may occur near the source end while heating/melting/excessive conductor loss may occur near the SC ends, if such possibilities exist.

Figure 12.10.b shows the effect of loading the rods by capacitive reactances which is equivalent to connection of capacitors in parallel with the shunt SC stubs. This loading brings down the resonance frequency of shunt elements to f0<fq. The series (coupling) reactance remains the same. So, this approach lowers the passband to below fq. Further, the electric fields at ports will be less than the case without capacitors, because the frequency of signals of interest will not reach the quarter wave frequency fq. Since the wavelength at frequencies about the passband f0 is longer than that at fq, a smaller portion of the standing wave patterns will appear over the lines. Since now the electrical length to the SC ends is shorter, the level of electric field remains low while the level of magnetic field remains high. Therefore magnetic coupling dominates for f<fq.

These coupled lines form the basic building blocks of interdigital filters which are formed as n-parallel coupled lines with adjacent lines being shorted to ground at alternate ends.

Figure 12.10. Coupling mechanisms in interdigital coupled line pair.

F) Comb Type Coupled Line Pair:

Figure 12.11.a shows comb type coupling obtained by shorting node-1b and node-2b to ground. Line-1-0 of becomes a shunt SC stub between node-1a and ground, the Line-2-0 becomes a shunt SC stub between node-2a and ground while Line-1-2 becomes a SC stub between node-1a and node-2a, as shown in the figure.

Again the relations Yij=vCij and the static capacitance matrix parameters can be used for calculating the physical dimensions for realization of these admittances. It is always desirable to have identical resonator bars. That is impedances of the shunt SC stubs need be equated. Conventionally this is done by Capacitance matrix transformations of Wenzel 8. On Filpro, this is done by Pi-Tee-L transformations described above. With cursor under the series SC stub of the Pi-section of SC stubs, the command Pi-Tee-L / Pi-to-Sym Pi is applied. This command equates impedances of the shunt SC stubs while also producing a transformer, as described in Figure 12.11.b.

Figure 12.11. All-stub model of a comb type coupled line pair and Pi-Tee-L transformation for equating shunt SC stubs.

In Figure 12.12, the coupling mechanism is described from a different points of view. Consider the two port formed between Port-1a and Port-2a of the short circuited line pair repeated in Figure 12.12.a. The shunt and series SC stubs can be converted into their half length equivalents which are paralleled OC and SC stubs of halved lengths and doubled impedance (Half Angle command in Filpro). Note that the element coupling the two shunt resonators is a paralleled OC+SC stub, similar to that we have seen in interdigital coupled case. However in interdigital coupled case the series arm OC stub was negative while it is positive in comb coupled lines. This is the mechanism differentiating comb and interdigital couplings. The effect of paralleled positive SC and OC stubs shows itself as a finite transmission zero on s=j axis, at =q (f=fq). At this frequency the positive and negative susceptances of OC and SC stubs cancel leading to zero coupling susceptance (infinite series reactance-parallel resonance), hence decoupling the two resonators. In interdigital case there was no such a decoupling action. The shunt elements also become resonant at q. So, contrary to interdigital filters, for comb type couplings, although the shunt resonators are open circuit, hence allow transmission of signal from port-1a to port-2a, signal flow is stopped by the parallel resonator in series arm. Therefore in order to form a passband we have to use other mechanism. A passband can be formed by lowering the resonance frequency of the shunt arm elements by placing capacitive reactances in parallel with the shunt SC stubs. This is described in Figure 12.12.b.

From field theory perspective the coupling mechanism can be explained as follows:

At frequencies about fq, the lines are about quarter wave long. Therefore the electric and magnetic standing waves have maximum amplitude variation along the lines. At SC ends the E-field is zero and H-field is maximum while at the other ends E-field is maximum and H-field is minimum. The magnetic field sets the inductive part of the coupling reactance while electric coupling provides the capacitive part of the coupling reactance. In comb type structures electrically and magnetically coupled signals tend to cancel each other, as we have noticed in the stub model of the circuit. At frequency fq, cancellation of the coupling is complete, thus stopping power transfer between the lines. At frequencies f<fq, wavelength is longer compared to the wavelength at f=fq and hence amplitude of the electric field standing wave remains small while the amplitude of magnetic field standing wave remains high due to the small electrical distance to the SC ends. Therefore the inductive (magnetic field) coupling becomes higher than the capacitive (electric field) coupling. So, the coupling reactance is inductive for f<fq. For these frequencies the shunt elements are also inductive reactances. Therefore the circuit is made up of a ladder of series and shunt inductors as shown in the figure for f<fq. The circuit can be made to posses a passband if the shunt inductors can be converted into parallel LC resonators by adding capacitive reactances in parallel with the shunt inductors, as shown in Figure 12.12.b in which case the circuit section can be treated as a series inductively coupled shunt resonator filter section.

Figure 12.12. Coupling mechanisms in a comb coupled line pair.

Effects of Enclosure: Evanescent waveguide modes

In the above description of comb type coupling it is assumed that only TEM mode waves exist. However when a filter made up of N comb type coupled lines is placed in an enclosure the cross-sectional dimensions of the enclosure can support evanescent waveguide modes. These modes can form unaccounted couplings between resonators. In such cases the section of evanescent waveguide between adjacent resonator operating in TE10 mode can be approximated by a Pi-section of inductors. The shunt inductor of the Pi-section modifies the resonator impedances while the series inductor of the Pi-section modifies the coupling element. That is, the series SC stub representing the TEM mode coupling will be modified by adding a lumped inductive reactance in parallel. Especially in narrowband filters response is very sensitive to coupling element variations. The effect of series inductor in parallel with the series SC stub shows itself as band widening. Since both the series SC stubs and the accompanying series inductors are both simulating coupling through magnetic fields, the degradation in response can be corrected by small adjustments in coupling distances or by inserting tuning screws in between  the resonators to disturb the coupling fields.

G) Even-Odd mode decomposition of coupled lines

Another commonly used description of a coupled line pair involves decomposition of the excitations and hence the line pair into even and odd modes, as described in Figure 12.13 on a symmetric two-port circuit formed by a symmetric comb coupled line pair with SC ends. The same reasoning applies to other symmetric coupled lines. Let’s assume that the two lines are fed by the voltages V1 and V2 at its two ports (Figure 12.13.a). The physical symmetry can be exploited by decomposing the excitations into even and odd modes:

V1=Vev+Vod	V2=Vev-Vod

Solving for Vev and Vod, we get

Vev = (V1+V2)/2	and	Vod = (V1-V2)/2

In the even mode, the two conductors are fed by identical even mode voltages Vev This excitation creates an OC wall on symmetry plane. In the odd mode the ports are excited by equal magnitude but opposite sign voltages, Vod and –Vod in which case the symmetry plane is a zero voltage (SC) plane.  The electric and magnetic field lines for these modes are shown in the figure. The complete electric and magnetic field line pattern can be obtained by superposing the even and odd half field patterns. Clearly, this decomposition simplifies the problem a lot. Instead of analyzing a complex two conductor system, we will analyze only one halves of the even and odd mode systems. That is, we will analyze two single conductor transmission lines under the modified boundary conditions formed by OC plane and SC plane on symmetry plane. The characteristic impedances (admittances) of the single transmission lines obtained under even and odd mode excitations are denoted by Zev (Yev) and Zod (Yod) respectively.  Thus, the complete parallel coupled line system can be characterized by the impedance pair Zev, Zod or admittance pair Yev, Yod. If the medium is inhomogeneous, then the effective permittivity and permeability of the media in which even and odd mode waves travel will be different, leading to different phase velocities, vev and vod, hence to different electrical lengths ev and od and different quarter wave frequencies fq,ev, fq,od.

The circuit theory description of this decomposition process can be made in terms of per unit length capacitances corresponding to these modes, as shown in Figure 12.13.a. The even mode excitation makes the mutual capacitor C12 ineffective, thus discarding it. Each half can then be treated as an isolated transmission line with per unit length capacitance of Cev=C10 and even mode characteristic admittance of Yev=vevCev=vevC10. In the odd mode excitations the SC symmetry plane bisects the mutual capacitance C12 in half, leading to two series capacitors of value 2C12. Thus, the total capacitance from the lines to ground becomes Cod=C10+2C12. The corresponding odd mode admittance is Yod=vodCod.

A similar decomposition can also be made on the circuit in terms of per unit length inductances as shown in Figure 12.13.b. However it is easier to talk (and visualize things) in terms of capacitance parameters. Figure 12.13.c shows the electric and magnetic field standing wave patterns when the right ends of the conductors are shorted to ground for a signal of frequency about fq. As in the single SC stub case, magnetic field becomes maximum at the SC ends while electric field becomes zero. Thus, magnetic coupling becomes more effective towards the SC ends while electric field coupling is more effective towards the other ends. In both capacitance and inductance descriptions of the two wire system shows that coupling between two wires is provided by the mutual capacitance and mutual inductance parameters. Since the elements C12 and L12 are effective in the odd mode only, it is seen that energy exchange between the two conductors are made basically through the odd mode fields. A commonly used measure of degree of coupling between two lines is the voltage coupling coefficient which is defined in terms of even and odd mode impedances as follows:

kv=20Log[(Zev-Zod)/(Zev+Zod)] = 20Log[(Yod-Yev)/(Yod+Yev)] = 20Log[(C12/(C1+C12)]   dB

When there is no coupling, the even and odd mode impedances become equal. As coupling become tighter, the difference between Zev and Zod gets larger with Zev>Zod always. Given the physical dimensions of coupled lines and medium parameters, the corresponding even and odd mode impedances and capacitances together with effective medium parameters (if inhomogeneous) can be found by using some ampirical formulations, charts or through electromagnetic simulation tools.  The reverse procedure, that is, finding the physical dimensions to realize a given set of even and odd mode impedances with targeted effective medium parameters is more tedious, necessitating some iterative search until satisfactory results are obtained. For N-coupled lines the concepts of even and odd mode impedances fails. However assuming that there is no cross-coupling beyond the neighboring lines, the even-odd mode impedance concept can still be used successfully, at least as an initiator for a trial-error type search or optimisation approach.

Figure 12.13. Even and Odd Mode decomposition of a symmetric parallel comb-coupled line pair.

H) Controlled couplings through slots

As it was seen in the previous sections, the natural coupling between the lines of interdigital and comb type coupled lines is of inductive for f<fq. In some applications the TEM resonators are separated by conducting walls and couplings are provided through slots on the common walls. This helps a more controlled coupling.  Further, it is also possible to make coupling through electric field dominate to get capacitive couplings.

- Figure 12.14.a shows a comb-like coupling where the length of coupling region is decreased by inserting a conducting grounded wall between resonators. This wall extends in the region where magnetic field is strongest, so as to decrease the amount of magnetically coupled energy.

- In Figure 12.14.b, the wall is extended to block the magnetic coupling further, until the region where electric field is stronger and magnetic field is weaker.  This way it is possible to make electric (capacitive) coupling dominate.

- Figure 12.14.c shows a controlled magnetic coupling in interdigital coupled structure. The capacitors may be the fringing capacitors at open ends or the capacitive loadings to decrease resonance frequency. Since electric and magnetic couplings are in additive nature and since there is no region where electric field is strong enough to dominate magnetic fields, this approach can control only magnetic coupling.

- There are diverse structures that can be used to couple resonators, like circular or rectangular widows, irises, capacitive probes, evanescent waveguides, etc. These are beyond the scope of this chapter.

Figure 12.14. Controlled couplings in comb and interdigital type coupled lines.

12. 3. MEDIUM BANDWIDTH INTERDIGITAL AND CAPACITIVELY LOADED INTERDIGITAL FILTERS

Interdigital filters are among the most frequently used structures because of their compactness. They use the allocated space efficiently. The classical interdigital parallel coupled line filter structures are shown in Figure 12.15.a while Figure 12.15.b shows capacitively loaded interdigital filters. For even number of resonators the input and output ports are on opposite sides while for odd number of resonators they are on the same side of the structure. These structures are suitable for medium/wide bandwidth filters. For narrowband filters either redundant impedance transforming bars are added at input and output or the input and output resonator bars are tapped, as will be described in later sections.

Figure 12.15. a) Interdigital filters with odd and even number of resonators. b) Capacitively loaded interdigital filters with odd and even number of resonators.

The circuit model developed in Figures 12.9 for interdigital type coupled line pairs can be generalized to N-coupled lines as described in Figure 12.16, for N=5 case. We could as well use the graph transformations of Sato and Cristal [12]. In Figure 12.16.a, the circuit model of N-coupled line is shown as 2N-1 port circuit with left nodes denoted with subscripts a and right nodes with subscript b. The circuit is made up of 2N-1 two-wire-transmission lines, all having the same (commensurate) length. The line-to-ground two-wire lines are characterized by their admittances Yk0, k=1,2,.. while the two-wire lines formed by adjacent rods are characterized by the line-to-line admittances Yk,k+1, k=1,2,.. . These characteristic admittances are related to the per unit length capacitances as

Yk0=vCk0  and    Yk,k+1=vCk,k+1

Figure 12.16.b shows the two-port circuit obtained by short circuiting the alternate ends of the bars to form an interdigital filter. The circuit model consists of cascades of alternating SC stubs of admittances Yk0 and transmission lines of admittances Yk,k+1. It should be noted that the same capacitance matrix parameters are valid as the original 2N-1 port circuit. The following points can be noted for reference:

- The shunt SC stub impedances can be equated by repeated application of Kuroda identities. This is automatized on Filpro through the command Kuroda / All Equal Stubs.

- It is also possible to collect all the shunt SC stubs at a single node by using Kuroda identities, as shown in the same figure. This structure shows that an interdigital filter with N-resonators has N-1 unit elements (NUE=N-1) and a single transmission zero at f=0 (Nzero=1) formed by the single shunt SC stub. The elements can be forced to assume proper values so that the circuit will have a highpass transfer function on S=j axis, passing a frequency range between p<<, with p being the passband edge frequency on j axis. Hence the circuit can be treated as a distributed element highpass filter of degree N with Nzero=1 and NUE=N-1. Clearly, the filtering feature of interdigital filters comes mostly from the N-1 unit elements.

- The highpass nature of the transfer function of the circuit on S=j axis leads to periodic passbands on s=j axis (Figure 12.16.c). The frequency = maps to =q on j-axis while the frequency p is mapped to =p and =2q-p with q=2fq being the quarter wavelength frequency. Thus, a passband is formed in the range pq-p with passband center being 0=q. The nearest spurious passband is centered at 3q=30 while the other spurious passbands are centered at the odd multiples of q.

- Since the passband center 0=2f0 coincides with the quarter wavelength frequency q, electrical length of stubs and lines are 90 degrees at passband center:

θ0=θ(f0)=(π2)(f0fq)=π2

- It is seen that the passband center of the filter sets also the lengths of coupled lines, hence sets the transversal dimension of the filter.

Capacitively loaded Interdigital Filter:

- Figure 12.16.d shows the circuit obtained by loading the ungrounded ends of the interdigital lines with capacitive reactances.

- Kuroda identities can be used to equate the shunt SC stub impedances or to collect all of them at a single node which implies that this circuit has a single transmission zero at f=0 (Nzero=1).

- The number of unit elements is NUE=N-1. The capacitive loadings provide N transmission zeros at = converting the structure into a distributed element bandpass filter with  Nzero=1, Ninf=N and NUE=N-1, having a total degree of 2N. This structure is named as capacitively loaded interdigital filter.

- The elements can be forced to have proper values so that the circuit will have a bandpass characteristics on S=j axis, with passband edges p1 and p2 and passband center 0 in the range p1<0<p2 (Figure 12.16.e). The bandpass nature of the transfer function of the circuit on S=j axis leads to periodic passbands on s=j axis with the first passband lying in the range p1<<p2<q where q=2fq is the quarter wavelength frequency mapped from =. The nearest spurious passband is located in the range 2q-p2<<2q-p1.

- The main difference between interdigital and capacitively loaded interdigital filters shows itself in the specification of the passband and quarter wavelength frequency. In interdigital filters the quarter wavelength frequency fq must be selected to be equal to the passband center while in the capacitively loaded interdigital filters the quarter wavelength frequency fq can be selected independently of passband center, anywhere in the range f>fp2. This is an important flexibility because by selecting fq properly we can adjust the quarter wavelength, hence adjust the transversal size of the filter while also adjusting the position of the nearest spurious passband, as follows:

- If we select fq=2f0 then we obtain the same response as the interdigital filter of the previous example but with halved lengths. The electrical length will also be halved to 45 degrees:

θ0=θ(f0)=(π2)(f0fq)=π4

Thus, capacitive loading halves the transversal dimension of the filter compared to interdigital filter case.

- If fq>2f0, then we get even shorter quarter wave length decreasing transversal dimension further and the spurious passband is moved away from the main passband, providing a wider stopband.

- In applications where unloaded Q is important, length of the lines are desired to be maximum possible. In these cases we should not use capacitive loadings. That is we should design an interdigital filter. However the open ends of interdigital bars are actually loaded by their fringing capacitances. Therefore they can also be treated as capacitively loaded interdigital filters, and hence it should be possible to design them as capacitively loaded interdigital filters.

Figure 12.16. N-parallel coupled line structure and interdigital filter circuit models.

Figure 12.16. N-parallel coupled line structure and interdigital filter circuit models (cont’d).

EXAMPLE 12. 1. A medium bandwidth interdigital filter synthesized as a Highpass filter

Let’s consider a seven resonator (NR=7) interdigital filter with passband edges fp1=750 MHz, fp2=1250 MHz with PBRipple=0.01 dB. If no capacitive loadings are assumed at the OC ends of the bars, then the filter can be designed as a distributed element highpass filter with fp=750 MHz and fq=f0=1000 MHz. Hence the electrical length of resonators at passband center  will be 90 degrees:

θ(f0)=(π2)(f0fq)=π2

The transmission zeros are as follows:

Nzero=1, NUE=NR-1=7-1=6

- Design command sets are shown in Figure 12.17. Design / By Synthesis command is invoked. Distributed / Highpass/ Doubly Terminated / Type B options are selected. Type B is selected to get the single transmission zero at f=0 in shunt SC stub form. After typing in the specifications, PBRipple, fp and fq, the transmission zeros are entered. The response is seen in the inset plot. The spurious passband starts from 2fq+fp=2750 MHz and extends to 3250 MHz, centered at 3fq=3f0=3000 MHz. The skirt slopes on the two sides of the main passband are symmetric.

- Transmission zeros are extracted in the order shown in Figure 12.17.a. The shunt SC stub is extracted in the middle to get a symmetric circuit.

- The SC stub will be distributed between unit elements using Kuroda/ Distribute Stub command. This command is applied by placing the cursor under a TL which has a SC stub only on one side and  part of the stub is transferred to the other side so that the TL will have equal stubs on each side. By moving the cursor sequentially, the SC stub will be distributed between all TL’s (Figure 12.17.b).

- After placing stubs on the two sides of each TL the command Kuroda / All Equal Stubs / Include End Stubs is applied. This command equates impedances of the shunt SC stubs at 37.8 ohms level. These impedances can be scaled up by the techniques that will be studied later. Length of stubs and transmission lines is 74.948 mm, as set by the quarter wave frequency fq=1000 MHz. We save this circuit as INTD1.fil to use at later stages (Figure 12.17.c).

Figure 12.17. Synthesis of an interdigital filter.

EXAMPLE 12. 2. A medium bandwidth capacitively loaded interdigital filter

Let’s now use the seven resonators to design a capacitively loaded interdigital filter to get a small transversal size. This must be designed as a distributed element bandpass filter because the shunt capacitive reactances loading the seven resonators (NR=7) will form seven transmission zero at infinity. Together with the single TZ at 0 we get a domain BPF. Bandpass filter approach gives us the flexibility of selecting the quarter wavelength frequency fq independently of the passband center. We can use this flexibility for size reduction by selecting a high fq. Specifications are:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=2500 MHz

The electrical length of resonators is 36 degrees:

θ0=(π2)(f0fq)=π5

As a distributed element highpass filter the seven resonators of the previous example made the total degree of the filter as N=NR=7. In distributed element bandpass filter approach the same seven resonators will make the total degree of the filter N=2NR=14 because each resonator will be accompanied by shunt capacitive reactances leading to seven TZ’s at . Hence the transmission zeros are:

Nzero=1, NUE=NR-1=6,  Ninf=7

- Design command sets are shown in Figure 12.18. Design / By Synthesis command is invoked. Distributed / Bandpass/ Doubly Terminated / Type B options are selected. Type B is selected to get the single transmission zero at f=0 in shunt SC stub form. After typing in the specifications PBRipple, fp and fq, transmission zeros are entered. It is seen that the spurious passband is moved away from the main passband. It starts from 2fq-f2p=3750 MHz and extends to 4250 MHz, centered at 2fq-f0=4000 MHz. The skirt slopes on the two sides of the main passband are no more symmetric. This is because of the seven transmission zeros at infinity which increase the upper skirt slope.

- Transmission zeros are extracted using the command Standard Auto Extract (Figure 12.18.a). The single TZ at is extracted as a shunt SC stub in the middle of the circuit while the TZ’s at are extracted as shunt OC stubs simulating shunt capacitive reactances for f<fq.

- Simplify Circuit command combines the SC stub with the adjacent OC stub to form a single OC+SC Stub element (Figure 12.18.b).

- The SC stub will be distributed between unit elements using Kuroda/ Distribute Stub command. This command is applied by placing the cursor under a TL which has OC+SC stub element only on one side. This command distributes the SC stub equally on the two sides of the TL. Simplify Circuit command is applied again to combine the translated SC stub with the adjacent OC stub. By moving the cursor sequentially to the other TL’s and applying the same command the SC stub will be distributed between all TL’s, forming OC+SC stub elements at each node (Figure 12.18.c).

- The shunt OC stubs represent the capacitive reactances which are needed to form shunt resonators which resonate at the passband center f0=1000 MHz<fq. Although it is possible to realize the OC stubs as they are it is usually easier to approximate them as lumped capacitors. This is possible by placing the cursor under any OC+SC stub element and applying the command Replace By Lumped / All SC Stub+Cap / fp1=750 MHz, fp2=1250 MHz (Figure 12.18.d).  This transformation is an approximation which keeps the passband edges unaffected at the expense of some degradation in passband return loss about band center. Degradation is negligible in narrow band filters. This conversion leads to another useful result: The first spurious passband is almost eliminated.

- Finally, with cursor under any TL, the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the shunt SC stub impedances (Figure 12.18.e). They are equated at 81.5 ohms level, much higher than the SC stub impedances of the interdigital filter of the previous example. Length of stubs and transmission lines is 29.979 mm, as set by the quarter wave frequency fq=2500 MHz. Much smaller than the interdigital filter case. We save this circuit as CAPINT1.fil to use at later stages.

Figure 12.18. Synthesis of a miniature capacitively loaded interdigital filter.

Figure 12.18. Synthesis of a miniature capacitively loaded interdigital filter (cont’d).

12.  4. MEDIUM BANDWIDTH COMBLINE FILTERS

Figure 12.19 shows some typical combline filter structures using different types of capacitive loadings. It is possible to use lumped capacitors, OC stubs with lengths less than or equal to the coupled line length, or any mixed circuit section having capacitive input reactance.

Figure 12.19. Typical combline filter structures using different capacitive loadings.

The circuit model developed in Figures 12.11 and 12 for two comb coupled lines can be generalized to N-coupled lines as described in Figure 12.20, for N=5 case. In Figure 12.20.a, the circuit model of N-coupled line is shown as 2N-1 port circuit with left nodes denoted with subscripts a and right nodes with subscript b. The circuit is made up of 2N-1 two-wire-transmission lines, all having the same (commensurate) length. The line-to-ground two-wire lines are characterized by their admittances Yk0, k=1,2,.. while the two-wire lines formed by adjacent rods are characterized by the line-to-line admittances Yk,k+1, k=1,2,.. These characteristic admittances are related to the per unit length capacitances as

Yk0=vCk0  and    Yk,k+1=vCk,k+1

Figure 12.20.b shows the two-port circuit obtained by short circuiting the right ends of the bars to form an combline structure. The circuit model consists of cascades of alternating shunt SC stubs of admittances Yk0 and series SC stubs of admittances Yk,k+1. It should be noted that the same capacitance matrix parameters are valid as the original 2N-1 port circuit. The following points can be noted for reference:

- The shunt SC stub impedances can be equated by repeated application of Pi-Tee-L transformations. This is automatized on Filpro through the command Pi-Tee-L / All Equal Shunt / SC Stubs.

- Figure 12.20c shows the circuit model when the left unused ports are loaded by capacitive reactances. Again Pi-Tee-L transformations can be used to equate the shunt SC stub impedances. In this circuit all the shunt SC stubs provide a single transmission zero at f=0 and the series SC stubs provide N-1 transmission zeros at infinity. The shunt capacitive loadings provide another N transmission zeros at infinity. Thus, N-resonator combline filters have Nzero=1, Ninf=2N-1, summing up to a total degree of 2N as a distributed element bandpass filter. Clearly, such a filter will have great asymmetry in amplitude response because of the large difference between Nzero and Ninf.

Figure 12.20. N-parallel coupled line structure and combline filter circuit model.

EXAMPLE 12. 3. Synthesis of a medium bandwidth combline filter

Let’s design a seven resonator combline filter (NR=7) with the same passband specifications as the interdigital and capacitively loaded interdigital filters of the previous two examples:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz

The quarterwave frequency is selected to be the same as the capacitively loaded interdigital  filter:

fq=2500 MHz

So, electrical length is 36 degrees:

θ(f0)=(π2)(f0fq)=π5

This is also a degree 14 bandpass filter with the following transmission zeros:

Nzero=1, Ninf=2NR-1=13

- Design command sets are shown in Figure 12.21. Design / By Synthesis command is invoked. Distributed / Bandpass/ Doubly Terminated / Type B options are selected. Type B is selected to get the single transmission zero at f=0 in shunt SC stub form.

- After typing in the specifications PBRipple, fp and fq, transmission zeros are entered. It is seen that the spurious passband starts from 2fq-f2p=3750 MHz and extends to 4250 MHz, centered at 2fq-f0=4000 MHz, the same as in the capacitively loaded interdigital filter case. However the slope of the lower skirt is much lower while the slope of the upper skirt is much higher than those of the capacitively loaded interdigital case. This is because the Unit Elements of the interdigital case were contributing to the slopes of both upper and lower skirts while in combline case all these transmission zeros are moved to infinity, increasing the upper skirt slope and decreasing the lower skirt slope.

- Transmission zeros are extracted using the command Standard Auto Extract (Figure 12.21.a).

- With cursor under any series SC stub, the command  Pi-Tee-L / All Equal Shunt / SC Stubs is applied. This command distributed the single shunt SC stub into the circuit so as a SC+OC stub element will be formed at each node with equal SC stub impedances (Figure 12.21.b).

- The shunt OC stubs represent the capacitive reactances which are needed to form shunt resonators which resonate at the passband center f0=1000 MHz<fq. Although it is possible to realize the OC stubs as they are, it is usually easier to realize them as lumped capacitors. This is possible by placing the cursor under any OC+SC stub element and applying the command Replace By Lumped / All SC Stub+Cap / fp1=750 MHz, fp2=1250 MHz (Figure 12.21.c). Another useful aspect of this conversion is the heavy suppression of the nearest spurious passband and narrowing of the next spurious passband. This transformation is an approximation which keeps the passband edges unaffected at the expense of some degradation in passband return loss about band center. Degradation is negligible in narrowband filters.

- Conversion of the OC stubs into lumped capacitors destroys also the equality of the shunt SC stub impedances. This property can be recovered by placing the cursor under any series SC stub and  applying the command  Pi-Tee-L / All Equal Shunt / SC Stubs again (Figure 12.21.d). The shunt SC stub impedances are equated at the level of 89.7 ohms, close to but slightly higher than those of the capacitively loaded interdigital filter. Length of stubs and transmission lines is 29.979 mm, as set by the quarter wave frequency fq=2500 MHz, the same as the capacitively loaded interdigital case.

- We save this circuit as COMB1.fil to use at later stages.

Figure 12.21. Synthesis of a seven resonator combline filter.

12. 5. MEDIUM BANDWIDTH MIXED DIGITAL FILTERS AND COMPARISON OF RESPONSES

It is possible to use both interdigital and comb type coupled line sections in the same filter. Such mixed structures may be useful for trimming the asymmetry of the response of combline filters or for placing input and output of the filter on different sides. Figure 12.22 shows some examples and the corresponding circuit models. Different combinations of comb and interdigital coupled sections can be formed by using different NUE and Ninf while keeping the total degree the same (N=2xNumber of resonators) and extracting transmission zeros in different orders.

Figure 12.22. Some mixed digital (Comb+Interdigital) filters and circuit models.

EXAMPLE 12. 4. Synthesis of a medium bandwidth mixed digital (Comb+Interdigital) filter

Let’s design a mixed digital (Comb+Interdigital) filter having the same passband specifications as the previous examples:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=2500 MHz.

The seven resonators will give a total degree of N=2NR=14. There must be a single transmission zero at f=0. Hence Nzero=1. The rest will be shared between NUE and Ninf satisfying the constraint

NUE+Ninf=13

Let’s try the case with

NUE=4, Ninf=9

- Design command sets are shown in Figure 12.23. Design / By Synthesis command is invoked. Distributed / Bandpass/ Doubly Terminated / Type B options are selected. Type B is selected to get the single transmission zero at f=0 in shunt SC stub form.

- After typing in the specifications PBRipple, fp and fq, transmission zeros are entered. It is seen that the spurious passband starts from 2fq-f2p=3750 MHz and extends to 4250 MHz, centered at 2fq-f0=4000 MHz, the same as in the combline and capacitively loaded interdigital filter case. The skirt slopes are somewhere between those of the capacitively loaded interdigital and combline filters, as expected because the number of unit elements and Ninf are somewhat balanced.

- Transmission zeros are extracted in the order inf-ue-inf-ue-inf-inf-inf-zero-inf-ue-inf-ue-inf  giving the circuit of Figure 12.23.a.

- With cursor under any series SC stub, the command  Pi-Tee-L / All Equal Shunt / SC Stubs is applied. This command distributed the single shunt SC stub into the circuit so as a SC+OC stub element will be formed at each node in between the two inner TL’s, with equal impedances (Figure 12.23.b).

- Then with cursor under the TL adjacent to the shunt OC+SC stubs, the command Kuroda / Distribute Stub is applied followed by Simplify Circuit command. Kuroda command transfers part of the shunt SC stub on one side to the other side while Simplify Circuit command combines the transferred stub with the adjacent OC stub to form a new OC+SC stub there. These commands are repeated until an OC+SC stub resonator is formed at each node (Figure 12.23.c).

- The shunt OC stubs are converted into lumped capacitors by placing the cursor under any OC+SC stub element and applying the command Replace By Lumped / All SC Stub+Cap / fp1=750 MHz, fp2=1250 MHz (Figure 12.23.d). This transformation is an approximation which keeps the passband edges unaffected at the expense of some degradation in passband return loss about band center. This conversion also eliminates the first spurious passband.

- Thus, a SC Stub+Cap type resonator is formed at each node. The shunt SC stub impedances in between the two inner TL’s can be equated by placing the cursor under any series SC stub and  applying the command  Pi-Tee-L / All Equal Shunt / SC Stubs again. Then with cursor under any TL, the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the shunt stub impedances on the two sides of the TL’s. However this command destroys equality of inner shunt SC stubs. Therefore these two commands are repeated until all the shunt SC stubs are equated (a few iterations are sufficient) (Figure 12.23.e). The shunt SC stub impedances are equated at the level of 82.1 ohms, closer to but slightly higher than those of the capacitively loaded interdigital filter. Length of stubs and transmission lines is 29.979 mm, as set by the quarter wave frequency fq=2500 MHz.

- We save this circuit as MIXED.fil to use at later stages. Figure 12.24.f shows a pictorial description of the filter.

- Figure 12.24 shows a comparison of wideband insertion loss responses of the four filters. Note that the spurious passband of the interdigital filter about 3f0 disappeared in all cases.

- Figure 12.25 shows the return loss responses of the four filters. The slight degradations about the passband center because of approximating the shunt OC stubs with lumped capacitors in the capacitively loaded filters are clear.

- Figure 12.26 shows delay responses of the four filters. The symmetry of the interdigital and asymmetry of the combline filter are noticeable. Delay is always higher near the passband edge with steeper skirt slope [38].

Figure 12.23. Design of a mixed comb+interdigital filter.

Figure 12.24. Comparison of wide span insertion loss responses of interdigital, capacitively loaded interdigital, combline and mixed digital filters.

Figure 12.25. Comparison of return loss responses of of interdigital, capacitively loaded interdigital, combline and mixed digital filters.

Figure 12.26. Comparison of delay responses.

12. 6. THE USE OF EVEN-ODD MODE IMPEDANCE CONCEPTS

Although the even and odd mode impedance concepts are developed for the characterization of two identical coupled lines we will use these concepts for nonidentical coupled lines also. These concepts are valid when nonadjacent couplings are negligible. In such cases we can treat adjacent pairs of lines as two-conductor line system and use the even and odd mode impedance concepts for finding the physical dimensions through the available charts and formulations. This approach may also be useful to start an iterative search in electromagnetic simulators, to get a rough initial estimate.

Figure 12.27 describes determination of the even and odd mode impedances of adjacent lines in the four types of filters described in the previous examples.

- In the case of interdigital filter of Example 12.1, the filter INTD1.fil is loaded (Figure 12.27.a). With cursor under the first TL, the command Replace By PCL / Interdigital / scscINTD / Symmetric is applied to convert the triplet of SC Stub+TL+SC stub into interdigital coupled line between nodes 1 and 2. Filpro displays even and odd mode impedances of the two lines as Zev1, Zod1 and Zev2, Zod2. When the coupled lines (the shunt SC stubs on the two sides of TL’s) are identical we have Zev1=Zev2 and Zod1=Zod2. Since the two lines are identical the terminology Zev and Zod is used in the figure.

- After noting down the even and odd mode impedances, Undo command is used to get the original circuit back and the cursor is moved to the next (or any other) TL to create new coupled lines. Figure 12.27.b shows examples where coupled lines are formed between nodes-1-2 and 3-4 while Figure 12.27.c shows the coupled lines between nodes 2-3 and 4-5. Because of symmetry only half of the circuit need be converted.

- Figure 12.27.d shows the pictorial description of the interdigital filter together with the even and odd mode impedances formed by adjacent coupled lines.

- In Figure 12.27.e, the circuit of capacitively loaded interdigital filter (Capint1.fil) is shown. The command Split / All Two Element Resonators is used to expose the shunt SC stubs (Figure 12.27.f).

- Then Exchange command is used to form the triplet of SC Stub+TL+SC stub between nodes 1 and 2 (Figure 12.27g).

- With cursor under the TL, the command Replace By PCL / Interdigital / scscINTD / Symmetric is applied to convert the triplet of SC Stub+TL+SC stub into interdigital coupled line between nodes 1 and 2 (Figure 12.27.h).

- After noting down the even and odd mode impedances, Undo command is used to get the original circuit back and the cursor is moved to the next (or any other) TL to create new coupled lines.

- Figure 12.27.i shows the pictorial description together with the even and odd mode impedances of adjacent coupled line pairs.

- In Figure 12.27.k, the circuit of combline filter (Comb1.fil) is shown. The command Split / All Two Element Resonators is used to expose the shunt SC stubs (Figure 12.27.m). Then Exchange command is used to form a Pi-section of SC stubs between nodes 1 and 2 (Figure 12.27.n).

- Then with cursor under the series SC stub of the Pi-section, the command Replace By PCL / Combline / scscCOMB / Symmetric is applied to convert the Pi-section into comb coupled line section between nodes 1 and 2 (Figure 12.27.p). After noting down the even and odd mode impedances, Undo command is used to get the original circuit back and the cursor is moved to the next (or any other) TL to create new coupled lines. Figure 12.27.q shows the pictorial description of the combline filter together with the even and odd mode impedances of adjacent coupled line pairs.

- In Figure 12.27.r, the circuit of mixed digital filter (Mixed1.fil) is shown. The command Split / All Two Element Resonators is used to expose the shunt SC stubs (Figure 12.27.s).

- Then Exchange command is used to form a triplet of SC stub+TL+SC Stub between nodes 1 and 2 to be converted into an interdigital coupled line and also a Pi-section of SC stubs is formed between nodes 3 and 4  to be converted into comb type coupled line (Figure 12.27.t).

- With cursor under the first TL, the command Replace By PCL / Interdigital / scscINTD / Symmetric is applied to convert the triplet of SC Stub+TL+SC stub into interdigital coupled line between nodes 1 and 2. Then with cursor under the series SC stub of the Pi-section, the command Replace By PCL / Combline / scscCOMB / Symmetric is applied to convert the Pi-section into comb coupled line section between nodes 3 and 4 (Figure 12.27.u). After noting down the even and odd mode impedances, Undo command is used to get the original circuit back and the cursor is moved to the next (or any other) TL to create new coupled lines. Figure 12.24.v shows the pictorial description of the combline filter together with the even and odd mode impedances of adjacent coupled line pairs.

- Degree of coupling is measured by the difference between even and odd mode impedances. As the difference increases coupling increases. In this respect we note that comb type coupled lines are more tightly coupled compared to the corresponding interdigital types.

Figure 12.27. Even and odd mode impedances of adjacent coupled lines in the four filters.

Figure 12.27. Even and odd mode impedances of adjacent coupled lines in the four filters (cont’d).

Figure 12.27. Even and odd mode impedances of adjacent coupled lines in the four filters (cont’d).

12. 7. COUPLING COEFFICIENT, EXTERNAL Q AND NODAL RESONANCE    FREQUENCY CONCEPTS

In the classical approaches in which direct coupled resonator filters were designed by mapping from ready lowpass prototype filters, the parameters Coupling Coefficient and External Q were sufficient to characterize the LP Prototype and these parameters can be transformed to bandpass form through the same LP-to-BP frequency transformation function 1, [3], 6, 7, [19], 36, 37. Thus, these types of direct coupled resonator filters can be characterized through the BP transformed k-Qext parameters together with the Nodal Resonance Frequencies. Their practical value comes from both the ease of design and realization by using the LP prototype tables and also the ease of measurements on the practically realized filter which are especially useful in the testing and tuning process in trying to achieve the targeted insertion loss and return loss response. The parameters k-Qext are still useful for realization and testing/tuning stages of filters designed in Filpro where such filters are synthesized directly as bandpass filters. Therefore the parameters k-Qext are not defined from the prototypes, but, since all element values are known, they are obtained by using their definitions in terms of element values of the filters.

Definitions of these parameters can be summarized as follows.

Nodal Resonance Frequency

Consider the direct coupled resonator lumped element filter shown in Figure 12.28.a. The resonator elements are denoted by L10-C10, L20-C20, L30-C30, etc. while the coupling elements are shown as L12, L23, L34, etc. In filter practice resonance frequencies of resonators are important because they are among the easiest parameters to measure. The resonance frequencies of the isolated resonators are set by the elements Lk0-Ck0. However in filter measurements the coupling elements need also be taken into account because the resonance frequency at a node is usually measured by grounding the other nodes which are coupled to this node. The resonance frequency measured by grounding the adjacent nodes is termed as the Nodal Resonance Frequency of that node. When the adjacent nodes are shorted to ground the series coupling elements will become shunt and will be added in parallel with the resonator elements of that node. Therefore Nodal Resonance Frequency is the resonance frequency obtained by paralleling all the reactive elements connected at a node. In Figure 12.28.a the resonator elements of the nodes 1, 2 and 3 are shown. Their nodal resonance frequencies are

f0k=12πLkCk												(12.7.1)

where Lk and Ck are now parallel combination of the node-to-ground elements Lk0 and Ck0 with the coupling elements Lk,k-1 and Lk,k+1 or Ck,k1-1 and Ck,k+1.

In combline filter circuits the resonators can be formed by a shunt SC stub in parallel with lumped capacitors or distributed capacitive reactances, like OC stubs. Figure 12.28.b shows the circuit of a combline filter with lumped capacitors. In the usual combline filters the coupling element is a series SC stub. If the coupling between resonators is of slot type then the coupling element need be modelled as a lumped inductor or other equivalent inductive reactances. When the adjacent nodes are grounded the total reactance at a node is obtained by paralleling all the elements connected to that node. Therefore in the circuit of Figure 12.28.b, the total shunt SC stub impedance is set by parallel combination of the node-to-ground SC stub and the SC stub connecting that node to the adjacent nodes. The nodal resonance frequency f0k=0k/2 at node-k need be found by equating the total susceptance of the node to zero:

ω0kCk−1ZkTan(ω0kdεrc)=0								(12.7.2)

where Zk is the total shunt SC stub impedance and d is the length of stub.

In Figure 12.28.c the circuit of a capacitively loaded interdigital filter is shown. Here, the coupling elements are transmission line pieces having the same length as the SC stubs. Therefore when the nodes adjacent a node-k are grounded the transmission lines become shunt SC stubs which will be paralleled with the SC stubs of the node as described in the figure. Hence, the nodal resonance frequencies will be found by using the same formulation as the combline case.

Figure 12.28.d shows an interdigital filter. The coupling elements are transmission lines and they become shunt SC stubs after grounding. Since there is no capacitor at the nodes the nodal resonance frequency will be just the quarter wave frequency fq corresponding to the commensurate length of the filter. Therefore all resonance frequencies will be identical. However in practice the fringe capacitances at the open ends of the resonator bars need also be taken into account in finding the resonance frequencies.

Figure 12.28. Determination of nodal resonance frequencies.

Figure 12.28. Determination of nodal resonance frequencies (cont’d).

EXAMPLE 12. 5. Graphical display of Nodal Resonance Frequencies on Filpro

The approach described in Figure 12.28 for determination of nodal resonance frequencies can readily be applied on Filpro, as shown in Figure 12.29.

- In Figure 12.29.a, the circuit of INTD1.fil is shown. In order to measure the resonance frequency of Node-1, all the other nodes are grounded. On Filpro this is simulated by deleting all the elements except the shunt SC stub at node-1. Then the transmission line connecting Node-1 to Node-2 is re-inserted as a shunt SC stub, because Node-2 is grounded and this line appears as a SC stub. Plotting the return loss the resonance frequency will appear as a reflection zero. Actually, in interdigital filters there is no need for this simulation because the resonance frequency is set by the commensurate length of the filter. Therefore we don’t proceed for finding the resonance frequencies of the other nodes. They are all the same.

- In Figure 12.29.b, the circuit of COMB1.fil is shown. In order to measure the resonance frequency of Node-1, all the elements except the shunt resonator at node-1 are deleted. Then the series SC stub connecting Node-1 to Node-2 is re-inserted in shunt form because Node-2 is grounded. Plotting Return loss gives the resonance frequency as f01=1102 MHz.

- In Figure 12.29.c, the circuit of COMB1.fil is re-loaded to measure the resonance frequency of Node-2. All the elements except the shunt resonator at node-2 are deleted. Then the series SC stubs connecting Node-2 to Node-1 and Node-3 are re-inserted in shunt form. Plotting Return loss gives the resonance frequency as f02=1088 MHz.

- In Figures 12.29.d and e the same procedure is repeated for Node-3 and Node-4, giving the resonance frequencies as f03=1058 MHz and f04=1056 MHz.

- Figure 12.29.f shows the return loss responses for each of the four cases.

Nodal resonance frequencies of the other filters can be displayed in the same way as the combline filter case.

Figure 12.29. Measuring nodal resonance frequencies on interdigital and combline filters.

Figure 12.29. Measuring nodal resonance frequencies on interdigital and combline filters (cont’d).

Coupling Coefficients:

Two circuits are said to be coupled when they are related in such a way that energy interchange can take place. More specifically current exists in a circuit if and when the other circuit is energized. Since coupled circuits interact on one another, characteristics of one of the circuit are affected by the parameters of the other circuit to some extent depending on the degree of coupling. Therefore a measure of “degree of coupling” is a useful parameter. One measure of coupling between two circuits is the Coupling Coefficient defined in Figure 12.30, on a lumped element direct coupled LC resonator filter shown in Figure 12.30.a.

Consider the nodes 1 and 2, coupled inductively. Removing the shunt capacitors at the nodes we get a Pi-section of inductors (Figure 12.30.b). Coupling coefficient will be defined on this Pi-section.  Feeding this circuit at node-1 and grounding node-2, we get the circuit of Figure 12.30.c. In this circuit the total current I1 entering node-1 is split into two components, I10 and I12. The fractional part I12 of I1, that is, I12/I1 is an index for the degree of coupling:

I12I1=Y12Y10+Y12=1L121L10+1L12									(12.7.3)

Here, Y10=1/jwL10 and Y12=1/jwL12 are susceptances of L10 and L12 respectively. Similarly, feeding node-2 and grounding node-1, we get the circuit of Figure 12.30.d. In this circuit the fraction I21/I2 is also an index for the degree of coupling:

I21I2=Y21Y20+Y21=1L121L20+1L12									(12.7.4)

In filter theory, the geometric mean of these two indices are defined as Coupling Coeficient k12 between the resonators of nodes 1 and 2:

k12=I12I1I21I2=Y12(Y10+Y12)(Y20+Y12)=1L12(1L10+1L12)(1L20+1L12)L1L2L12	 (12.7.5)

where

1L1=1L10+1L12,  1L2=1L20+1L12							(12.7.6)

are the total inductors connected at nodes 1 and 2 respectively. From field theory point of view, coupling between two inductors is made through their magnetic fields. Therefore inductive couplings are also referred to as magnetic coupling coefficients, km. Note that the magnetic coupling coefficient defined in Eq.12.2.8 is just the reverse of that defined in Eq. 12.7.5

Similarly a capacitive coupling coefficient is defined on a Pi-section of capacitors, as shown in Figure 12.30.e:

k23=Y23(Y20+Y23)(Y23+Y23)=C23(C20+C23)(C30+C23)=C23C2C3			(12.7.7)

where C2=C20+C23 and C3=C30+C23 are total capacitors connected to nodes 2 and 3 respectively. Since the actual coupling is made through electric fields, capacitive couplings may also be called as electrical coupling, ke.

In summary, coupling coefficient of any type, L or C, between two nodes is the ratio of the susceptance of the coupling element to the geometric mean of the total susceptance of the two nodes.

If two shunt LC resonators are coupled through a series arm parallel LC resonator, than we can define both electric and magnetic couplings and the net coupling coefficient will be the difference of electric and magnetic coupling coefficients, ke-km.

When the resonators and coupling elements are of distributed or mixed elements the concept of   coupling coefficient can be generalized as described in Figure 12.30.f and g for interdigital and comb type couplings.

Figure 12.30. Coupling coefficients between lumped and distributed element resonators.

Figure 12.30. Coupling coefficients between lumped and distributed element resonators (cont’d).

Figure 12.30. Coupling coefficients between lumped and distributed element resonators.

The expression for coupling coefficient is now written in terms of coupling and shunt susceptances (admittances) as follows:

kij=Bijβiβj														(12.7.8)

where Bij is the coupling susceptance while i and j are susceptance slope parameters of the resonators defined as

βi=ωi02∂Bi∂ω        												(12.7.9)

with derivative evaluated at the nodal resonance frequency i0 of node-i and with Bi being the total susceptance of that node. Formulae for lumped elements are just special cases of this general definition. Figures 12.30.f and g displays the difference between interdigital and comb type couplings:

- In interdigital type coupling the coupling line can be converted into its all-stub Pi-equivalent circuit with halved lengths. In this model the coupling element appears to be the parallel combination of SC and OC stubs in the series arm. Since the OC stub has negative impedance the coupling reactance is in the form:

X12(ω)=Z12Sin(πω2ωq)										(12.7.10)

Since this is positive for 0<<2q, coupling is inductive in this frequency range, becoming maximum at frequency q. Also it is finite for all frequencies. Therefore this parallel combination can’t form a real frequency transmission zero.

- In comb type coupling the coupling element is a series SC stub. This stub may also be converted into its half length equivalent. It is also a parallel combination of SC and OC stubs with halved lengths. However in this case both OC and SC stubs have positive impedances. So, this is the source of main difference between comb and interdigital type couplings. The OC and SC stubs of comb type couplings resonate at frequency q. This is seen from the expression of the coupling reactance:

X12(ω)=Z12Tan(πω2ωq)										(12.7.11)

which becomes infinite at q thus decoupling the resonators forming a finite transmission zero   at that frequency.

In Filpro, coupling coefficients can be displayed by pressing the keys ALT+F6 or CTRL+F6 keys, as will be shown in the examples.

Unloaded Q and External Q:

The quality factor Q of an isolated RLC resonator is termed as Unloaded Q. For a series RLC resonator it is defined as follows (Figure 12.31.a):

Qu=1Rsrω0Csr=ω0LsrRsr										(12.7.12)

while for a shunt RLC resonator,

Qu=Rshω0Csh												(12.7.13)

In these equations Rsr and Rsh are equivalent resistors due to conductor and dielectric loss and w0 is the resonance frequency of resonator. Equivalent loss resistors are usually frequency dependent. However for narrowband filters they may be treated as constant. In practice Rsr is required to be as small as possible while Rsh is required to be very large to simulate ideal lossless LC resonators.

For distributed element shunt resonators unloaded Q is found from the admittance Y() as follows:

Y(ω)=G(ω)+jB(ω)										(12.7.14)

Qu=βRsh													(12.7.15)

where

Rsh=1G(ω0),   β=ω02∂B(ω)∂ω								(12.7.16)

with derivative evaluated at nodal resonance frequency 0. is termed as susceptance slope parameter. For distributed element series resonators the unloaded Q is found from its impedance Z() as follows:

Z(ω)=R(ω)+jX(ω)										(12.7.17)

Qu=R(ω0)χ													(12.7.18)

where is the reactance slope parameter,

χ=ω02∂X(ω)∂ω												(12.7.19)

with the derivative evaluated at the nodal resonance frequency0 of the resonator.

The quality factor of the resonators at source and load terminations are set by the source and load resistors, RS and RL, because these are much more effective than the loss resistors of the resonators. This is termed as loaded Q or External Q (Qext) of the filter (Figure 12.29.b). If a series LC resonator is adjacent to the source (load) resistor RS, then its Qext is:

Qext=1Rsω0Csr=ω0LsrRs										(12.7.20)

while if a shunt LC resonator is adjacent to RS,

Qext=Rsω0Csh											(12.7.21)

Usually some extra elements exist between the first (last) resonator and Rs (RL). For such cases the equivalent resistance and reactance seen when looking from the resonator to the source (load) end is determined and then these resistive and reactive components are treated as integral parts of that resonator, and the above formulations are used to find Qext.

When the resonators and coupling elements are distributed or mixed lumped and distributed element type then the following formula is used to find Qext:

Qext=βRsh, Qu=R(ω0)χ										(12.7.22)

for shunt and series resonators respectively where and are the susceptance and reactance slope parameters of the resonators including the source (load) resistor and reactances of interconnecting elements (Figure 12.31.b), evaluated at 0 as defined above.

Figure 12.31. Unloaded Q and External Q of shunt and series resonators.

EXAMPLE 12. 6. Displaying effects of Unloaded Q of resonators on Filpro

On Filpro, effects of finite unloaded Q of resonators are displayed by first converting all the resonators into Lumped LC form and then converting the LC resonators into RLC resonators by specifying the unloaded Q of the resonators at the passband center. Since the unloaded Q of the real life resonators are  functions of frequency depending on many factors while we specify a constant Q, the simulation we get is valid only about the passband.

- In Figure 12.32.a, the circuit of interdigital filter INTD1.fil is shown. Filpro doesn’t treat the shunt SC stubs as resonator. Therefore we first convert them into lumped LC resonators by applying the command Replace By Lumped / LC Resonator (Figure 12.32.b). Then with cursor under any LC resonator the command Parasitics / All Lossy Resonators / f0=1000 MHz, Q=100 is applied to assign an unloaded Q=100, for example (Figure 12.32.c). Figure 12.32.d shows insertion loss responses before and after conversion into lossy form. The extra insertion loss due to the loss of resonators is clear. The  conversion of SC stubs into lumped LC resonators leads to a widening in bandwidth and degrades response in stopband. However these changes don’t effect the extra insertion loss  introduced by the Q of resonators.

- In Figure 12.32.e, the circuit of COMB1.fil is shown. In this circuit we again convert the SC Stub+Cap type resonators into LC resonators by placing the cursor under a resonator and applying command Replace By Lumped / All Lumped LC: f1=750 MHz, f2=1250 MHz (Figure 12.32.f). This conversion is a more controlled approximation compared to the previous one, as the passband edges are preserved. Then the command Parasitics / All Lossy Resonators / f0=1000 MHz, Q=100 is applied to get the RLC form of the resonators (Figure 12.32.g). Figure 12.32.h shows the insertion loss response before and after conversion into lossy form.

- The conversions of CAPINT1.fil and MIXED1.fil into lossy forms can be done in the exactly the same manner as in COMB1.fil case.

Figure 12.32. Displaying effects of unloaded Q of resonators.

Figure 12.32. Displaying effects of unloaded Q of resonators (cont’d).

EXAMPLE 12. 7. Listing Coupling Coefficients, External Q and Resonance Frequencies on Filpro

On Filpro, Coupling Coefficients of direct coupled shunt resonator filters are calculated by the formulations in terms of susceptance slope parameters:

kij=Bijβiβj

where Bij is the susceptance of the coupling element and i and j are susceptance slope parameters of the coupled resonators defined as

βi=ω0i2∂Bi∂ω

with derivative evaluated the nodal resonance frequency 0i of node-i and Bi being the total susceptance of that node. External Q is also calculated through the formulation in terms of susceptance slope parameters:

Qext=β(ω0)Rsh(ω0)

with being the suseptance slope parameter of the resonator adjacent to the source (load), evaluated at the resonance frequency of that node and Rsh is the resistive part of the impedance  seen when looking towards the source, including the resonator itself.

In both formulations first the nodal resonance frequencies need be calculated and then the slopes of susceptances about the nodal resonance frequency will be evaluated.

- Coupling Coefficient between two resonators can be displayed together with the Nodal Resonance frequencies  by placing the cursor under the coupling element, pressing the keys ALT+F6 and then typing in the center frequency and two frequencies on the two sides of center frequeny. The resonance frequencies are named as f0L and f0R, meaning the Left and Right resonators. In direct coupled filters they should come out to be the same.

- All Coupling Coefficients and the External Q can be displayed as a list by pressing the keys CTRL+F6, selecting the option Direct Coupled and then typing in the center frequency and two frequencies on the two sides of the center frequency.

- If only External Q need be displayed then the keys ALT+F7 will be pressed and then the center frequency and two frequencies on the two sides of the center frequency will be typed in.

In Figure 12.33 the Coupling Coefficients, External Q and Nodal Resonance Frequencies of the four filters designed in the previous examples are shown:

- In Figure 12.33.a of the interdigital filter, since Filpro doesn’t treat a shunt SC stub as a resonator, we first convert all the shunt SC stubs into SC Stub+Cap type resonators with zero capacitance and then display the Coupling Coefficients and External Q by pressing CTRL+F6 (Figure 12.33.b). As a demonstration of displaying individual Coupling Coefficients and Nodal Resonance Frequencies the cursor is placed under the element coupling the nodes 5 and 6 and the keys ALT+F6 are pressed.  Note that the resonance frequencies come out to be identical  for interdigital filters.

- In all the other filters the commands CTRL+F6 and ALT+F6 can be applied directly, as shown in Figures 12.33.c-to-f. Filpro treats the SC Stub+Cap elements as resonators. It should be noted that the nodal resonance frequencies of these filters come out to be different.

- In Figure 12.33.e, a version of the combline filter which is optimised to get equiripple return loss is shown to see the variation in the coupling coefficients. Optimization is carried out by using the Best Case command of Filpro. Optimization is converged at about 25 db return loss level. Some slight changes are observed in coupling coefficients.

- Nodal Resonance Frequency at any node may also be checked by deleting all the other elements except the node-to-ground elements of that node and then re-inserting the elements coupling that node to the neighboring nodes in shunt form. Plotting return loss will show the resonance frequency.

Figure 12.33. Displaying External Q, Coupling Coefficients and Nodal Resonance Frequencies of interdigital, combline and mixed filters.

Figure 12.33. Displaying External Q, Coupling Coefficients and Nodal Resonance Frequencies of  interdigital, combline and mixed filters.

EXAMPLE 12. 8. Graphical display and calculation of coupling coefficients and coupling bandwidth

On Filpro, coupling coefficients between two coupled resonators can also be displayed graphically as described in Figure 12.34 and 35, on the interdigital filter example. In practice coupling between two nodes is measured by grounding all the other nodes coupled to these nodes and then inserting lightly coupled probes to inject signal at one node and get a sample of the coupled signal from the other node.  In order not to effect the fields in the resonators appreciably the probes must be lightly coupled to resonators. As the frequency is swept two peaks are observed in the signal at output. The frequencies at which peaks are detected are noted down as f1 and f2.  The coupling coefficient between nodes is then defined as

ki,i+1=f2−f1f0												(12.7.23)

where f0 is the passband center frequency usually defined as the geometric or arithmetic mean of pasband edges depending on filter types. In bandpass filters obtained by the classical mapping from lowpass prototypes it is taken as the geometric mean of passband edges while in filters directly synthesized by placement of transmission zeros it is dependent on the relative numbers of transmission zeros below and above the passband. For example in bandpass filters designed as Distributed Element Highpass Filters, like for example interdigital filters, insertion loss has arithmetic symmetry and hence the passband center is the arithmetic mean of passband edges while in combline filters there is only one transmission zero at zero and the rest are all at infinity, the definition of passband center is a bit vague. In the formal definition of coupling coefficients between two resonators

kij=Bijβiβj, with βi=ωi02∂Bi∂ω

the susceptance slope parameters i   are evaluated at the nodal resonance frequencies 0i. Therefore it may be reasonable to select the center frequency as the geometric mean of the two nodal resonance frequencies:

ω0ij=ω0iω0j											(12.7.24)

This selection would also be consistent with the case of arithmetic symmetry of the interdigital filters. Since measurement practice is limited in accuracy of reading, any slight error in definition of center frequency would have little effect. For narrowband filters these differences are not effective.

Let’s study the procedure on the interdigital filter example:

- The circuit of filter INTD1.fil is loaded (Figure 12.34.a).

- In this circuit we will measure the coupling coefficient k12 between node-1 and node-2. Therefore Node-3 will be grounded. This means the transmission line coupling node-2 to node-3 will appear as a shunt SC stub. On Filpro this is simulated by first deleting all the elements outside the nodes 1 and 2 (Figure 12.34.b).

- Then the transmission line between nodes 2 and 3 is re-inserted as a shunt SC stub (Figure 12.34.c).

- In order to inject signal to node-1 and to get a sample  from node-2, small series capacitors are connected at these nodes (Figure 12.34.d). Figure 12.34.e shows the insertion loss response. The frequencies of the two peaks are noted as f1=775 MHz and f2=1225 MHz. Taking the center frequency as f0=1000 MHz, we get

k12= (f2-f1)/f0=0.45

which is close to 0.4428 that displayed by the command CTRL+F6 or ALT+F6.

- Figure 12.35 describes measurement of the coupling coefficient k23. In this case node-1 and node-4 are grounded and the transmission lines connecting nodes 1 and 2 and also nodes 3 and 4 are re-inserted as shunt SC stubs. After connecting the probes the frequencies of peaks are measured and coupling coefficient is found as k23=0.306 which is very close to the value k23=0.305 displayed by CTRL+F6.

- All the other filters can be treated in the same way.

Figure 12.34. Graphical display of coupling coefficient k12 on the interdigital filter.

Figure 12.35. Graphical display of coupling coefficient k23 on the interdigital filter.

12. 8. USING REFLECTION ZEROS, PHASE AND DELAY OF S11 FOR MEASUREMENT AND TUNING

Tuning of the parameters of direct coupled resonator filters, like width, thickness and spacings of resonators and capacitors is an extremely tedious process by inspection of only insertion loss and return loss parameters. Adjustment of coupling coefficients of adjacent pairs of resonator is usually sufficient for tuning of the filter. However inspection of the frequencies of Reflection Zeros of the resulting filter and inspection of Delay of S11 when a resonator is shorted to ground may also be helpful for fine tuning 36. Figure 12.36 shows the relations between Reflection Zero frequencies and the phase of S11 of the filters designed in the previous example. It is seen that Phase of S11 jumps by 220 degrees in crossing Reflection Zero frequencies. This property can be used in fine tuning of the filter 19.

Since the reflected signal is very low at these frequencies it may be difficult to measure the reflection zero frequencies accurately. Figure 12.37 shows the Delay of S11 as the resonators are shorted to ground one by one. These delays can be noted down and during adjustment and tuning, these delays should be obtained. Since the short circuited resonator causes full reflection of the incident signal we have no problem of weak signal. Therefore adjustments may be done easily if the Delay of S11 can be measured and adjusted to have the replica of the delays seen in the figures 36.

Figure 12.36. Reflection zeros and 180 degree jumps in the phase of S11.

Figure 12.36. Reflection zeros and 180 degree jumps in the phase of S11 (cont’d).

Figure 12.37. Delay of S11 when resonators are shorted to ground one by one.

12. 9. DISPLAYING VOLTAGE, CURRENT AND POWER AT NODES

If desired, the voltages, currents and real power (V2-I2-P2) at the nodes can be listed at any specified source voltage and frequency either in Magnitude-Angle form or in Real-Imaginary form by loading the circuit and pressing the keys Alt+F8 as shown in Figures 12.38-to-41 for the interdigital, combline, capacitively loaded interdigital and mixed digital filters. In this list the conventions of ABCD matrix parameters are used. In this convention each element is described by its ABCD parameters which are defined in terms of voltages and currents at the input and output of each element. The listed quantities should be interpreted as the voltages, currents and hence power at the output of each element. The reference made as at right of means V2-I2-P2 denote the quantities on the right of each element which are numbered by starting from the left end node. The quantities can be displayed in either Magnitude-Angle or in Real-Imaginary format.

Node voltages are important in high power applications where dielectric breakdown possibility exist. Especially the nodes having maximum voltage are susceptible to breakdown. Voltages at nodes change with frequency. Since stored energy in the elements of the filters is directly proportional to the group delay the node voltages are also related to the group delay [38]. Therefore maximum voltage occurs at the frequency where the group delay is maximum. Delay increases near the band edges, especially on the side where the rejection is steeper, hence larger nodal voltages there. As example, a survey is made at the maximum delay frequencies to inspect the highest voltages listed in the figures. As expected the maximum voltage of the combline filter is higher than the others, because it has steeper skirt slope on the high side. Note that the real power at the load is 50 watts (maximum available power for Vs=100 volts and RS=50 ohms) within the passband, as expected, and it falls outside the passband.

Figure 12.38. Voltage-Current-Power at the nodes of interdigital filter at maximum delay frequency.

Figure 12.39. Voltage-Current-Power at the nodes of capacitively loaded interdigital filter at maximum delay frequency.

Figure 12.40. Voltage-Current-Power at the nodes of mixed digital filter at maximum delay frequency.

Figure 12.41. Voltage-Current-Power at the nodes of combline filter at maximum delay frequency.

12. 10. FINITE TRANSMISSION ZEROS IN COMBLINE AND MIXED DIGITAL FILTERS

Circuit models of the comb-type coupled line pairs involve series arm SC stubs which displays inductive reactance for f<fq. This series inductive reactance can be resonated by paralleling it with a capacitive reactance to create a finite transmission zero. This is possible by introducing a series lumped capacitor between open ends (capacitively loaded ends) of the adjacent lines [Pregla, 10], [Rhodes and Levy 11] [Malherbe 20]. Although the coupling between interdigital lines is also inductive in nature, creation of a finite transmission zero by connecting capacitors between the open ends of adjacent lines in interdigital filters is impractical because the open ends of the resonators are on opposite sides. However if a comb-type section is introduced into a basically interdigital filter (hence a mixed digital filter), then it is possible to create a finite transmission zero.

Let’s see some examples.

EXAMPLE 12. 9. A combline filter with a single finite transmission zero in lower stopband

Let’s design a seven resonator combline filter (NR=7) with the same passband specifications as the previous combline filter example:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=2500 MHz

Let’s place a finite transmission zero in lower stopband, for example at fz=600 MHz. The finite transmission zero is basically used to increase the selectivity in lower stopband or to notch out an undesired signal. However it may also be used to correct asymmetry in both amplitude and delay responses.

This is a degree 14 bandpass filter. Since it is a combline filter, Nzero=1. The finite transmission zero will provide two degrees. Thus, the number of transmission zeros at infinity will be Ninf=14-3=11.

- Design command sets are shown in Figure 12.42. Design / By Synthesis command is invoked. Distributed / Bandpass/ Doubly Terminated / Type B options are selected. Type B is selected to get the single transmission zero at f=0 in shunt SC stub form.

- After typing in the specifications PBRipple, fp and fq, transmission zeros are entered. It is seen that due to the finite transmission zero, the slope of the lower skirt is now closer to that of the upper skirt.

- Transmission zeros are extracted in the order inf-inf-inf-inf-inf-FTZ-zero-inf-inf-inf-inf-inf-inf. Other extraction orders may also be tried. However in order to get a close to symmetry we extract the single finite transmission zero near the symmetry axis (Figure 12.42.a).

- With cursor under any series SC stub, the command  Pi-Tee-L / All Equal Shunt / SC Stubs is applied. This command distributed the single shunt SC stub into the circuit so as a SC+OC stub element will be formed at each node with equal SC stub impedances (Figure 12.42.b).

- The shunt OC stubs are converted into lumped capacitors by placing the cursor under any shunt OC+SC stub element and applying the command Replace By Lumped / All SC Stub+Cap / f1=750 MHz, fp=1250 MHz. Similarly, the OC stub component of the series arm OC+SC stub element  is also converted into lumped capacitor by placing the cursor under this element and applying the command Replace By Lumped / SC Stub+Cap / f1=750 MHz, f2=1250 MHz. (Figure 12.42.c). This transformation is an approximation which keeps the passband edges unaffected at the expense of some degradation in passband return loss about band center. Return loss can be made equiripple by tuning all elements at a lower return loss level. This conversion also eliminates the first spurious passband.

- Conversion of the OC stubs into lumped capacitors destroys also the equality of the shunt SC stub impedances. This property can be recovered by placing the cursor under any series SC stub and  applying the command  Pi-Tee-L / All Equal Shunt / SC Stubs again (Figure 12.42.d). The shunt SC stub impedances are equated at the level of 86.5 ohms. The circuit is saved as FCOMB1.fil.

- Figure 12.42.e shows a pictorial description. The series capacitor can be realized by creating a capacitive effect at the tips of resonators 3 and 4, for example as shown in the figure.

- Figure 12.42.f shows insertion loss and return loss of the filter while figure 12.42.g shows delay response of the filter FCOMB1.fil overplotted with that of COMB1.fil of the previous seven resonator filter. The improvement of symmetry in delay response by the addition of the finite transmission zero is clear.

Figure 12.42. A combline filter with a single finite transmission zero in lower stopband.

Figure 12.42. A combline filter with a single finite transmission zero in lower stopband.

EXAMPLE 12. 10. Synthesis of an elliptic combline filter

Let PBRipple=0.05 dB,  fp1=800 MHz, fp2=1200 MHz, fq=2000 MHz, Nzero=1, Ninf=1, three FTZs at f=700 MHz, three FTZs at f=1300 MHz. Total degree is N=14 leading to seven resonators, NR=7.

Command set in Filpro for synthesis is shown in Figure 12.43.

- Standard Auto Extract command gives the circuit shown in Figure 12.43.a.

- Applying the command Pi-Tee-L / All Equal Shunt / SC Stubs, impedances of all shunt SC stubs are equated as shown in Figure 12.43.b.

- The shunt OC stubs can be converted into lumped capacitors by applying the command Replace By Lumped / All SC Stub+Cap (Figure 12.43.c).

- The series arm OC stubs are replaced by lumped capacitors by placing the cursor under each series arm OC+SC Stub elements and applying the command Replace By Lumped / SC Stub+Cap: fp1=800, fp2=1200 MHz (Figure 12.43.d).

- Equality of the shunt SC stubs will be lost after these commands. However it can be recovered by applying the command Pi-Tee-L / All Equal Shunt / SC Stubs again (Figure 12.43.e). The circuit is saved as ELP1.FIL.

- Since filter is a wideband one, these approximations degrade return loss near the band center (Figure 12.43.f). Response can be corrected by tuning elements slightly.

- This topology may be realized as shown in Figure 12.43.g. The Pi-sections of SC stubs form the fingers of combs with short circuits on the same side. The shunt lumped capacitors can be formed between the OC ends of the resonator rods and ground plane by placing tuning screws through the ground plane. The difficult part is the realization of series coupling capacitors which will form the finite transmission zeros. This may need placement conductor pieces between the tips of adjacent fingers to form lumped capacitors [10], [20].

Figure 12.43. Realization of an elliptic combline filter in various forms.

- Figure 12.43. Realization of an elliptic combline filter in various forms (continued).

EXAMPLE 12. 11. A Half-Wavelength stepped resonator elliptic filter

The OC+SC Stub elements of elliptic filters can be converted into two stepped SC stubs, leading to combline filters with stepped resonators having Half Wave total lengths. Though realizability of such structures seems may involve severe constraints, let’s make an example as a demonstration of the tools of Filpro. Filter specifications are the same as the previous example:

PBRipple=0.05 dB,  fp1=800 MHz, fp2=1200 MHz, fq=2000 MHz, Nzero=1, Ninf=1, three FTZs at f=700 MHz, three FTZs at f=1300 MHz. Total degree is N=14 leading to seven resonators, NR=7.

Command set in Filpro for synthesis is shown in Figure 12.44.

- Standard Auto Extract command gives the circuit shown in Figure 12.44.a.

- Applying the command Pi-Tee-L / All Equal Shunt / SC Stubs, impedances of all shunt SC stubs are equated as shown in Figure 12.44.b.

- The parallel OC+SC Stub elements in shunt and series arms can be converted into two step SC stubs using command Step Resonators (Figure 12.44.c). Now the stubs are stepped and the total  lengths are doubled. That is the total length of stubs are of half wavelength instead of quarter wavelength at fq.  leading to doubling of the transversal dimensions of the filter.

- The shunt arm two-step SC stub resonators may be realized as stepped resonator rods with different widths to get proper impedances shown as Z1 and Z2.

- Impedances of the series arm two-step SC stub resonators may be realized by adjusting the separations between adjacent stepped stubs.

- Since the separations between adjacent stubs and widths of the stubs are interdependent, adjustment of impedances may be problematic and tedious.

- It should be noted that in this realization we don’t use lumped capacitors.

- One disadvantage of this structure is the existence of the periodic spurious passbands. In the previous approach most of these passbands were eliminated by conversion of the OC stubs into lumped capacitors.

Figure 12.44. Realization of an elliptic combline filter using Half Wavelength Stepped resonators.

EXAMPLE 12. 12. Synthesis of a medium bandwidth mixed digital filter with a finite transmission zero

As it was stated previously, interdigital filters don’t allow easy realization of finite transmission zeros. If however it is necessary to include one or more finite transmission zeros into a basically interdigital structure, this is possible by converting some of the couplings into comb type. As an example let’s try inserting a single finite transmission zero into a capacitively loaded interdigital filter with a long   electrical length to get high unloaded Q, close to an interdigital filter without capacitive loadings.

The specifications are as follows:

PBRipple=0.01 dB, fp1=750 MHz, fp2=1250 MHz, fq=1300 MHz

In order to get longer resonator bars the quarter wave frequency fq must be selected to be as low as possible. Since by the definition of bandpass filters, fq must be greater than fp2. So, fp2=1250 MHz  is the lower bound for fq. Let’s select fq=1300 MHz which will lead to a length of 57.6 mm. As fq gets lower the capacitors gets smaller. If it was possible to make fq=1000 MHz, we would get zero capacitive loadings which would correspond to simple interdigital filters. Thus, by selecting lower fq, we are approaching to the simple interdigital case. Considering the fringing capacitors at the open ends of the interdigital filter bars, for some resonators there may not be need to place the external lumped capacitors.

The seven resonators will give a total degree of N=2NR=14. There must be a single transmission zero at f=0. Hence Nzero=1. Two degrees will be coming from the finite transmission zero. The rest will be shared between NUE and Ninf satisfying the constraint

NUE+Ninf=11

Seven resonators will lead to six coupling elements. One of the couplings will be of comb type to realize the finite transmission zero. Therefore five of the couplings will be of interdigital type. Thus we select NUE=5 and Ninf=6.

- Design command sets are shown in Figure 12.44. Design / By Synthesis command is invoked. Distributed / Bandpass/ Doubly Terminated / Type B options are selected. Type B is selected to get the single transmission zero at f=0 in shunt SC stub form.

- After typing in the specifications PBRipple, fp and fq, transmission zeros are entered.

- Transmission zeros are extracted in the order inf-ue-inf-ue-inf-FTZ-zero-ue-inf-ue-inf-ue-inf  giving the circuit of Figure 12.44.a.

- With cursor under the series OC+SC stub, the command  Pi-Tee-L / All Equal Shunt / SC Stubs is applied. This command equates the shunt SC stubs on the two sides of the series OC+SC stub element (Figure 12.44.b).

- Then with cursor under the TL adjacent to the shunt OC+SC stubs, the command Kuroda / Distribute Stub is applied followed by Simplify Circuit command. Kuroda command transfers part of the shunt SC stub on one side to the other side while Simplify Circuit command combines the transferred stub with the adjacent OC stub to form a new OC+SC stub there. These commands are repeated until an OC+SC stub element is placed at each node (Figure 12.44.c).

- Then with cursor under any unit element, the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate the shunt SC stubs. However the shunt SC stubs on the left and right side of the series OC+SC stub element will be equated at different levels (Figure 12.44.d). They will be equated at a later stage..

- The shunt OC stubs are converted into lumped capacitors by placing the cursor under any OC+SC stub element and applying the command Replace By Lumped / All SC Stub+Cap: f1=750 MHz, f2=1250 MHz. This transformation is an approximation which keeps the passband edges unaffected at the expense of some degradation in passband return loss about band center. Then with cursor under the series OC+SC stub element the command Replace By Lumped / SC Stub+Cap: f1=750 MHz, f2=1250 MHz is applied to convert the series OC stub component into lumped capacitor (Figure 12.44.e). This transformation is also intended to keep the passband edges unaffected. However since this element sets the position of the finite transmission zero, a large shift may occur in the finite transmission zero. Therefore the position of finite transmission zero need be iterated until it is located at the right frequency after these transformations.

- The shunt SC stub impedances in the two halves of the circuit will be equated by first placing the cursor under the series SC Stub+Cap element and applying the command Pi-Tee-L / Pi-to-Sym Pi / Equate Shunt SC stubs. Then with cursor under any unit element the command  Kuroda / All Equal Stubs / Include End Stubs is applied to equate the shunt stub impedances. These two command will be repeated until all shunt SC stubs are equated (a few rounds of iteration is sufficient). The resulting circuit is saved as FINT1.fil (Figure 12.44.f). It should be noted that the resulting transformer at load end is close to unity, but not unity. This is because the structure is not symmetric. We would get symmetric structure if the number of resonators was 6 or 8. Nevertheless the transformer can be deleted without much effect on response.

- Figure 12.44.g shows a pictorial description of the structure. Note that the interdigital coupled sections on the two sides of the comb coupled section have reverse sense. The positions of input and output ports are reversed.

- Figure 12.44.h shows the return loss and insertion loss responses. Note that the finite transmission zero is shifted from 600 MHz to 730 MHz. This gross shift degraded the return loss response also heavily. Therefore the design need be repeated by moving the finite transmission zero to lower frequencies.

Exercise: Repeat the design by placing the finite transmission zero at a lower frequency so that it will be shifted to the targeted fz=600 MHz.

Exercise: Repeat the design for realizing the comb type coupling nearer to the source end.

Exercise: Repeat the design with a lower fq’s to get even smaller loading capacitors.

Exercise: Repeat the design for 6 and 8 resonators.

Exercise: Repeat the design for two finite transmission zeros.

Figure 12.44. Design of a mixed digital filter with a single finite transmission zero.

Figure 12.44. Design of a mixed digital filter with a single finite transmission zero (cont’d).

12.  11. WIDEBAND INTERDIGITAL AND CAPACITIVELY LOADED INTERDIGITAL FILTERS

On Filpro, wideband interdigital filters with or without capacitive loadings can be designed by using any one of the Synthesis, Built in LC Prototype or Prototype with Inverters approaches. However, due to wide bandwidth, the approximations involved in circuit transformations cause heavy degradations in response which may lead to tedious tuning procedures. Synthesis approach is the best one. Prototype with Inverters approach may also be used when small capacitances are needed, like just for taking the fringe capacitances into account.

Interdigital filters are usually most suitable for medium bandwidth filters. For wider bandwidths it may be more suitable to use the interdigital filters with open circuited terminating lines.

Let’s continue discussions on examples.

EXAMPLE  12. 13. A wideband interdigital filter design by Synthesis approach

Let’s consider an octave band, seven resonator distributed element highpass filter with  specifications,

PBRipple=0.01 dB, fp=4000 MHz, fq=6000 MHz, Nzero=1, NUE=6

to work as a bandpass filter with fp1=4000 MHz and fp2=8000 MHz.

- Design command steps are shown in Figure 12.45. Designed filter is shown in Figure 12.45.a.

- The single shunt SC stub is distributed between TL’s by placing the cursor under the TL’s adjacent to SC stub and applying the command Kuroda / Distribute Stub. This command is used to place a SC stub on each side of the TL’s (Figure 12.45.b).

- Then the SC stubs are equated by applying the command Kuroda / All Equal Stubs / Include End Stubs (Figure 12.45.c). Thus, the network equivalent of the interdigital filter is ready.

- The next stage is the extraction of physical realization parameters of the 7 coupled lines from this network model, as was described in the previous examples. That is, the per unit length capacitances Ci0, i=1-to-7 from resonators lines to ground will be calculated from the shunt SC stub impedances Zi through the relation Ci0=1/vZi where v is the phase velocity in the medium of wave propagation. Similarly, the per unit length capacitances between adjacent resonator lines (denoted by Ci,i+1, i=1-to-7) are actually the capacitance per unit length of the two-conductor transmission line formed by two adjacent lines and are related to the TL impedances Zi,i+1 of the network model through Ci,i+1=1/vZi,i+1. These relations are shown in Figure 12.45.d. It is seen that impedances of transmission lines between the shunt SC stubs are too low compared to those we have seen in narrowband interdigital filters. This means that the capacitances between adjacent resonator lines will be too high, leading to very narrow gaps between resonator lines. Such low impedances (high capacitances) can be realized only by thick slab lines or by broadside coupled lines. These impedances can be raised to higher levels by placing three (or more) transmission zeros at f=0, leading to OC input-output resonators, as will be described in the next section.

- It should be noted that the SC stub and TL impedances of the network model are in directly realizable range as they are, as shown in Figure 12.45.e. That is, we may not convert the structure into coupled line form. This structure will then be the in the same form as shunt SC Stub+TL filters. However this would then be a physically long filter at low frequencies. This disadvantage can be corrected to a certain extent by using capacitive loadings, as will be seen in the capacitively loaded interdigital filters, in the next example.

- The fringing capacitors at the open ends of resonators may be troublesome to compensate. This is described in Figures 12.45.e-to-k. If the fringing capacitances at the open ends are known, then we can insert them in parallel with the shunt SC stubs. For example in Figure 12.45.e, a  0.1 pF capacitor is placed in parallel with the first and last SC stubs to simulate OC end effect. This will degrade the response. Degradation is then corrected by decreasing the length of the adjacent SC stub.  This action is then repeated on all SC stubs, leading to the capacitively loaded interdigital filter structure shown in Figure 12.45.k. Instead of this tedious procedure, it may be easier to design the filter directly as capacitively loaded interdigital filter, as will be described next.

Figure 12.45. Design of a wideband interdigital filter by Synthesis approach.

Figure 12.45. Design of a wideband interdigital filter by Synthesis approach (cont’d).

EXAMPLE 12. 14. A wideband capacitively loaded interdigital filter design by synthesis

Let’s consider the same octave band filter of the previous example as a distributed element bandpass filter, with specifications,

PBRipple=0.01 dB, fp1=4000 MHz, fp2=8000 MHz

with differing quarter wave frequencies, fq>fp2, like for example fq=8250 MHz to have the long resonator length (we could as well try fq=8001 MHz !) and fq=12000 MHz to get a shorter resonator length. Seven resonators will be used (NR=7). Thus, Nzero=1, NUE=NR-1=6. The lumped capacitors associated with the seven shunt SC stubs will be realized by placing seven transmission zeros at f=. That is, Ninf=7.

- The command set for synthesis of the filter with fq=8250 MHz is shown in Figure 12.46. Note that we have selected Distributed, Bandpass options. The electrical length of resonators is

(f0)= (/2)(f0/fq)= (/2)(6000/8250)=65.45 degrees

Since fq can’t be within the passband, we can’t make the resonators longer than about 65 degrees. Longer resonator lengths can be obtained by using Design / By Prototype with Inverters approach, as will be described in the next example.

- The transmission zeros are extracted in the order inf-ue-inf-ue-inf-ue-inf-zero-ue-inf-ue-inf-ue-inf so that we get alternating unit elements and shunt OC stubs. The single transmission zero at f=0 is extracted in the middle (could be extracted anywhere) as a shunt SC stub (Figure 12.46.a).

- The shunt SC stub will be distributed between the unit elements. For this purpose we first apply the command Simplify Circuit (short cut key: F6) to combine the SC and OC stubs in the middle to form a single SC+OC stub element. Then with cursor under the TL’s adjacent to the SC+OC stub element we apply command Kuroda / Distribute Stub to move part of the SC stub to the other side of the TL. Then Simplify Circuit command is applied again to form new SC+OC stub elements and continue to distribute the shunt SC stubs until a SC+OC Stub element is placed on each side of all TL’s (Figure 12.46.c).

- Insertion loss and return loss plots are shown below the circuit. Since fq is very close to passband the nearest spurious passband is also very close to the main passband. This spurious passband will be eliminated after conversion of the shunt OC stubs to lumped capacitors. This is done by placing the  cursor under any SC+OC stub element and applying the command Replace By Lumped / All SC Stub+Cap (Figure 12.46.d).

- Then the shunt SC stubs are equated by applying the command Kuroda / All Equal Stubs / Include End Stubs (Figure 12.46.e). This conversion eliminates the nearest spurious passband at the expense of some degradation in return loss. Degradation can be corrected to some extent by tuning elements at the expense of some lowering in the overall return loss level by a few dB’s.

- The circuit is ready for realization. The even and odd mode impedances of the adjacent coupled line pairs can be displayed by first applying the command Split / All Two Element Resonators (Figure 12.46.f) and then using Exchange command to form Shunt SC Stub+TL+Shunt SC Stub triplets as shown in Figure 12.46.g. These triplets are then converted into coupled lines by placing the cursor under the TL’s and applying the command Replace By Lumped / Interdigital / scscINTD / Symmetric. Since each SC stub is shared by the two adjacent TL’s, these conversions must be done one by one. In Figure 12.46.g-h some of these conversions are shown. Because of the symmetry, these are sufficient.

- Figure 12.46.i shows pictorial descriptions of the structure together with the even and odd mode impedances of adjacent pairs.

- It should be noted that impedances of stubs and TL’s are realizable as they are, without conversion into interdigital coupled N-line form. This is shown in Figure 12.46.k. The disadvantage of this structure is its length. However since fq is high at high frequency filters,  length  may not be a problem. It is about 54.5 mm. Lumped capacitors may also be realized as patches at the junctions.

- Further eduction in length can be obtained by selecting a higher fq. Figures 12.46.m-to-r show the filter obtained for fq=12000 MHz through the same procedure. For this case the first spurious passband is moved away from the passband leading to wider stopband. High fq leads also smaller stub lengths, thus leading to further miniaturization. It is also seen that impedances of both shunt SC stubs and TL’s are higher compared to the low fq filter.

- Figure 12.46.s shows the realization without conversion into interdigital coupled line form. Overall length is reduced to 37.5 mm.

Figure 12.46. Design of a wideband capacitively loaded interdigital filter by Synthesis.

Figure 12.46. Design of a wideband capacitively loaded interdigital filter by Synthesis (cont’d).

Figure 12.46. Design of a wideband capacitively loaded interdigital filter by Synthesis (cont’d).

EXAMPLE 12. 15. A wideband capacitively loaded interdigital filter design by Prototype with Inverters approach

Let’s treat the same wideband filter with Prototype with Inverters approach.

PBRiiple=0.01 dB, fp1=4000 MHz, fp2=8000 MHz

Seven resonators will be used.

- Command set is shown in Figure 12.47. Design / By Prototype with Inverters commands are invoked and Shunt Capacitors and Inverters option is selected. Degree and PBRipple of the prototype are typed in. The lowpass prototype is displayed automatically. Then Filter Type command is invoked and Distributed / Bandpass options are selected. Passband and Terminations item is invoked and the passband edge frequencies are typed in. In order to get capacitive loadings the quarter wave frequency will be chosen slightly greater than 2f0=12000 MHz, for example  fq=12200 MHz. Find Circuit command displays the filter with shunt OC+SC Stub resonators coupled by inverters (Figure 12.47.a).

- Inverters / All Equal Left-Right command equates the shunt element impedances (Figure 12.47.b). Note that length of all stubs are 6.14 mm (rounded!), set by the quarter wave frequency fq=12200 MHz.

- Now we will convert the OC stubs of the SC+OC stub resonators into lumped capacitors while also setting the new lengths of the SC stubs. This is possible by placing the cursor under the SC+OC stub resonators and applying the command Half Angle / fp1=4000, fp2=8000, Lng=12.28 mm. The stub length limits are shown at the bottom of the screen and we have selected the maximum possible length for the stubs, just as a trial to see the resulting capacitances. The maximum length corresponds to a quarter wave frequency close to the passband center of 6000 MHz, so that electrical length of resonators will be close to 90 degrees.  If shorter electrical length is preferred then we can type in the desired length within the limits.

- Next the inverters will be replaced by their practical equivalents. This is done by placing the cursor under inverters and applying the command Inverters / Replace Ideal Inverter By / SSsh+TL+SSsh / Specify Length: f0=8000 MHz, Lng=12.28 mm. Care should be exercised in applying this command. Here f0 is the frequency of approximation at which the SSsh+TL+SSsh triplet is equivalent to the ideal inverter. The best frequency need be selected by trials. In narrow-medium bandwidth filters it is usually selected about the passband center. However in wideband filters (at least in this example  ) it came out to be better to select it as the upper edge of the passband. So, we have typed in the upper passband edge frequency fp2=8000 MHz as the frequency of approximation. The length must be exactly the same as the SC stubs used in the previous step which was 12.28 mm. This is because we will combine the negative SC stubs of the SSsh+TL+SSsh triplets with the adjacent positive SC stubs. This is done by using Simplify Circuit command (Figure 12.47.d-e).

- Equality of the shunt SC stubs will be lost after combining them with negative SC stubs. We can recover this property by applying the command Kuroda / All Equal Stubs (Figure 12.47.f). We may also leave this step to the end of the design procedure.

- Since bandwidth is very wide the above approximation will degrade response, as seen in Figure 12.47.f. Response can be recovered by tuning elements one by one or by using the Best Case optimizer. The result of an attempt using Tune Element command is shown in Figure 12.47.g.

- The SC stubs of the optimized circuit can be re-equated by applying the command Kuroda / All Equal Stubs (Figure 12.47.h). The loading capacitor values came out to be between 0.025-0.04 pF. These values may also be adjusted to the practical or needed levels and the degradation that would arise from this adjustment process can be remedied by tuning impedances of TL’s and resonators again.

Figure 12.47. Wideband capacitively loaded interdigital filter design by Prototype with Inverters.

Figure 12.47. Wideband capacitively loaded interdigital filter design by Prototype with Inverters.

12. 12. WIDEBAND INTERDIGITAL FILTERS WITH OPEN CIRCUITED TERMINATING LINES

The procedures described in the previous subsections yield rather low impedances for the transmission lines of the network model of the wideband filters. For wideband applications, more suitable element values can be obtained by increasing the number of transmission zeros at zero. Interdigital filters with open circuited terminating lines are of that class, having three transmission zeros at f=0. In Filpro such filters can be designed using only Synthesis approach. Let’s study the procedure on an example.

EXAMPLE 12. 16. An octave band interdigital filter with open circuited terminating lines

Let the specifications of the distributed element highpass filter be given as

fp=4000 MHz, fq = 6000 MHz, Passband Ripple=0.01 dB, Nzero=3,  NUE=6

to yield a bandpass filter with passband edges fp1=4000 MHz and fp2=8000 MHz.

- The command set for synthesis is shown in Figure 12.48.

- Transmission zeros are extracted in the order shown in Figure 12.48.a. Note that the three transmission zeros at f=0 appear as two series OC stubs at input and output and a shunt SC stub in the middle. The circuit is saved as OC-INTD.fil.

- The shunt SC stub in the middle of the circuit is distributed between the transmission lines by placing the cursor under the transmission line adjacent to the SC stub and applying command Kuroda / Distribute stub. This command is repeated until a SC stub is placed on each side of each transmission line. However at the source end we have to use the command Kuroda / Right / Equal Stub command because here there are two stubs on which the command Kuroda / Distribute Stub is applicable. One is the series OC stub and the other is shunt SC stub and Filpro applies this command to the series OC stub which is not the desired one (Figure 12.48.a-b).

- Next, with cursor under any transmission line, the command Kuroda / All Equal Stubs is applied to equate the shunt SC stubs. Figure 12.48.c shows the resulting circuit which has a unity transformer at load end. We keep this transformer because it will be used in the formation of the coupled line at load end. We also Insert a unity transformer at the source end to be used in the formation of the coupled line at this end of the filter (Figure 12.48.d). We save this circuit as      OC-INTD1.FIL to be used at later stages.

- The triplet of  transformer+Series OC Stub+Shunt SC stub at the source end is equivalent to a transformer in cascade with a comb type parallel coupled line with left leg being open circuited and right leg short circuited. This coupled line is termed as ocscCOMB in Filpro. Conversion of this triplet into coupled line form is carried out by placing the cursor under the middle element of triplet (series OC stub) and applying command Replace By PCL / Comblines / ocscCOMB / n=1.8505 (Figure 12.48.d-e). The turns ratio n is set by the user. If a symmetric coupled line is desired then the default value of n is to be used. All other choices yield asymmetric coupled lines. The resulting “remanent” transformer is n1=0.5403 and it will be used to scale the impedance level of the circuit.

- At the load end we have the mirror image of the source end triplet. That is, Shunt SC Stub+Series OC Stub+Transformer. This triplet is equivalent to a coupled line with short circuited left leg and open circuited right leg in cascade with a transformer. This triplet can be converted to coupled line using the command Replace By PCL / Comblines / scocCOMB / n=1.8505, yielding a remaining transformer of tuns ratio n2=1.8505 (Figure 12.48.d-e).

- At this point we just note down the remanent transformer turns ratios n1=0.5403 and n2=1.8505 and then proceed as follows.

- We load OC-INTD1.FIL with unity transformers (Figure 12.48.f).

- The unity transformer at source end is splitted by applying the command Split / Specify Left: n=0.5403 to produce the remanent transformer of n1=0.5403 at the left end (Figure 12.48.g). Similarly the right end transformer is splitted by applying the command Split / Specify Left: n=0.5403 to produce the remanent transformer of the load and with turns ration of n2=1.8505.

- The remanent transformer of the source end is shifted rightmost position by applying the command Shift Trf / Rightmost and it is combined with the right end remanent transformer to produce a unity transformer and deleted (Figure 12.48.h-i). The resulting circuit is the final form of the network equivalent of the interdigital filter with OC terminating lines. It is saved as OC-INTD2.FIL.

- The even and odd mode impedance descriptions of coupled lines can be displayed by converting the triplets of elements into their coupled line equivalents. Two examples are shown in Figure 12.48.k. The end lines are converted into scocCOMB and ocscCOMB forms, as described in the previous stages while the inner SC Stub+TL+SC Stub triplets are converted into scscINTD type coupled lines by placing the cursor under the TL and applying the command Replace By PCL / Interdigital / scscINTD / Symmetric. These conversions must be done one by one and after noting down the even and odd mode impedances, Undo command is applied to recover the original circuit. This is because the adjacent coupled line pairs share one of their SC stubs. The noted even and odd mode impedances can be used as a staring set for an iterative search for finding physical dimension with the tools at hand (preferably an EM simulator).

- Figure 12.48.m shows a pictorial description.

- Capacitively loaded version of this filter is also possible by placing enough number of transmission zeros at infinity, as was done in the previous examples (Exercise?).

Figure 12.48. Design of a wideband interdigital filter with OC terminating lines.

Figure 12.48. Design of a wideband interdigital filter with OC terminating lines (cont’d).