PART-II

NARROWBAND

INTERDIGITAL, COMBLINE AND MIXED DIGITAL FILTERS

12. 13. NARROWBAND INTERDIGITAL FILTERS

Narrowband interdigital filters need be treated separately from wideband ones because, as will be seen, narrow bandwidths lead to unrealizably small resonator (shunt SC Stub) impedances. Therefore impedance scaling is needed. That is, impedance transformer effects are needed at input and output ports. Impedance transformations are usually achieved either by inserting extra coupled lines at input and output (termed as redundant Unit Elements or transformer rods) or input and output are fed to the circuit by tapping the first and last resonators. The first approach increases the size hence loss of the filter. Therefore whenever possible, tapping approach is preferred [13], [21], [27], [28], [29], [30].

On Filpro interdigital filters can be designed using any one of Synthesis or Prototype with Inverters or Built in LC Prototype approaches. Built in LC Prototype is the quickest approach, but it works only for odd degree filters because even degree lowpass prototype filters require RL different from RS. If this approach is used for even degree filters, then some tuning is necessary.

The classical interdigital filters without loading capacitors are designed as distributed element highpass filters. If the effects of loading capacitors are to be accounted or shorter  electrical length is desired then distributed element bandpass filter approach is to be used. Synthesis approach is suitable for both narrowband and wideband filters while Prototype with Inverters approach has some advantages for narrow-medium bandwidth filters.

Several examples using different approaches follows.

EXAMPLE 12. 17: Narrowband interdigital filter design using Synthesis approach

A bandpass filter with specifications

PBRipple=0.01 dB, fp1=975 MHz and fp2=1025 MHz.

will be designed with seven resonators (NR=7) as an interdigital filter using distributed element highpass filter approach. The specifications as a highpass filter are

PDRipple=0.01 dB, fp=975 MHz, fq=1000 MHz

That is, the passband center is taken as the quarterwave frequency of the highpass filter. The resonator (SC Stub) impedances for realization is desired to be about 80 ohms.

- Design command sets are shown in Figure 12.49. Design / By Synthesis commands are invoked. Distributed / Highpass/ Doubly Terminated / Type B options are selected.

- After typing in the specifications, PBRipple, fp and fq, transmission zeros are entered. Note that only one transmission zero is placed at f=0. Type B is selected to get this transmission zero in shunt SC stub form. The number of unit elements are set as NUE=NR-1=6.

- Transmission zeros are extracted in the order shown in Figure 12.49.a. The shunt SC stub is extracted in the middle to get a symmetric circuit. It will be distributed between unit elements using Kuroda/ Distribute Stub command. This command is applied by placing the cursor under a TL which has a  SC stub only on one side. The stub is then distributed equally on the two sides of the TL. By moving the cursor sequentially, the SC stub will be distributed between all TL’s (Figure 12.49.c). In this procedure we don’t care on the impedances of the resulting SC stubs.

- After placing stubs on the two sides of each TL (Figure 12.49.c), we apply the command Kuroda / All Equal Stubs / Include End Stubs. This command will equate the impedances of the shunt SC stubs. We save this circuit as INT01.fil to use at later stages (Figure 12.49.d).

- As it is expected for narrowband filters, SC stub impedances (resonator impedances) are too low for realization (2.56 ohms). These impedances can be scaled up by either inserting redundant UE’s or or by tapping the resonators at source and load ends, as will be described in the following subsections.

- Before continuing with impedance scaling, in order to use as a reference, we display the External Q and Coupling Coefficients of the circuit. This is done by converting the filter into direct coupled resonator form. This is done by placing the cursor under the shunt SC stubs and applying the command Replace By Lumped / LC Resonator (Figure 12.49.e). Then pressing the key combination CTRL+F6, selecting the option Direct Coupled and typing in the numbers f0=1000 MHz, f1=995 MHz, f2=1005 MHz, we get the list of Coupling Coeficients and External Q, as shown in Figure 12.49.e.

A) Impedance scaling with inverters and/or redundant unit elements:

- Impedance scaling can be done by applying the command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=60.6 ohms (Figure 12.49.f-g). Here, the inverter constant K=60.6 ohms is found after a few trials to reach the targeted 80 ohms resonator impedance. That is some trials are necessary to set the inverter constants. The default case with K=RS=50 ohms corresponds to the conventional way of inserting redundant 50 ohms Unit Elements.

- Next, with cursor under the inverters, the command Inverters / Replace Ideal Inverter By / Quarter Wave TL / f0=1000 MHz is applied to replace inverters by a quarter wave transmission line (Unit Element) of impedance Z=K (Figure 12.49.h). This transmission line is conventionally named as Redundant Unit Element, as it has no contribution to response shaping for the case when Z=RS. However in the case Z different from RS, it may not be treated as redundant because it is now functioning as a quarter wavelength transformer. For this reason it is also called as transformer rod. However we may still use the name redundant unit element, just to keep link to the conventional name.  For K different from RS, this replacement is an approximation which leads to slight distortions in return loss response, as shown in the figure. Degradation increases as K gets further away from RS. However response can be recovered by tuning only the shunt SC stub adjacent to the redundant Unit Elements (Figure 12.49.h-i).

- Next, with cursor under the redundant Unit Elements, the command Kuroda / Distribute Stub is applied to move part of the adjacent stub to the other sides of the redundant Unit Elements.  Now we have SC stubs on both sides of all the Unit Elements (Figure 12.49.j).

- Impedances of these UE’s are equated by applying the command Kuroda / All Equal Stubs/ Include End Stubs again (Figure 12.49.k). Thus, the design is finished. It is seen that SC stub impedances are 80.1 ohms, as desired. This circuit is saved as INT02.fil.

- In Figure 12.49.m a pictorial description of the filter is shown which can be used in stripline, suspended stripline or microstrip form. Usually height of the box, dielectric parameters are given and we need to determine the widths of lines (wi) and separations (si,i+1) between adjacent lines. In the final stage, we need to extract these physical parameters for realization. This needs Electromagnetic characterization of the N-coupled line system which is a difficult problem, needing many iterative trials and optimization. In this pictorial description widths of lines are set mainly by the SC stub impedances (resonator impedances) while separations between the lines are set by the Unit Element impedances. On Filpro, we can get a set of impedances which can be used as starting point in an iterative process by several assumptions, as follows:

- We note that if we isolate adjacent pairs of coupled lines from their neighbors, then the pairs can be treated as if they are two symmetric coupled lines for which one may find abundant data for the extraction of width and separation data by using the even and odd mode impedance descriptions. That is, we will convert each SC Stub+TL+SC Stub triplet into its coupled line equivalents one by one, by placing the cursor under each TL and applying the command Replace By PCL / Interdigital Lines / scscINTD / Symmetric. Some examples are shown in Figure 12.49.n-to-r where the even and odd mode impedances of adjacent pairs are displayed. They should be displayed one by one because each SC stub is shared by the adjacent coupled line pairs. The complete list of even-odd mode impedances are shown in Figure 12.49.s. Since these pairs of lines can’t be isolated from their neighbors, this approach will not yield correct results. However such results can be used to start an iterative search procedure.

- A further inherent assumption in this approach is that there is no coupling between non-adjacent lines. Naturally this is not correct.

- We also neglect the fringing capacitors at the OC ends of resonator bars which are usually not negligible.

- For all these reasons the whole structure need be treated as a single element to be handled by an EM simulator (or use cut-and-try methods).

- It should be noted that the input and output lines are much tightly coupled to their neighbors compared to the inner lines. This is seen from the larger differences between even and odd mode impedances. As this difference gets larger the coupling between the lines get tighter. In the limit when the even and odd mode impedances are identical this means there is no coupling between the lines.

- Another commonly used approach in both setting the physical dimensions and during the final realization tests is to employ External Q and Coupling Coefficients of the filter. This is described in Figures 12.49.t-u. The shunt SC stubs are converted into their lumped LC equivalents by applying the command Replace By Lumped / LC Resonator. The circuit is saved as INT03.fil. We can then display the External Q together with Coupling Coefficient list by pressing the keys CTRL+F6, selecting the option Direct Coupled and typing in the center frequency f0=1000 MHz, and two more frequencies, one is named as f1, slightly lower than f0 and the other is f2, slightly higher than f0. For narrowband filters we can use the lower and upper band edge frequencies as f1 and f2 (f1=fp1 and f2=fp2). Note that the External Q is different from the External Q of the original circuit. This is because the shunt resonators adjacent to the source and load are split into two parts and one part is transferred to the other side of the adjacent transmission line. If the External Q of the original circuit is desired to be displayed from this circuit, this can be done by first converting the shunt SC stub on the inner side of the redundant unit element into an LC resonator form (Figure 12.49.v) and then pressing the keys Alt+F7. Typing in the frequencies f0=1000 MHz, f1=995 MHz and f2=1005 MHz (or f1=fp1 and f2=fp2) we get the same External Q as the one obtained for the original circuit (Figure 12.49.w).

- We note that the coupling coefficients k12 and k89 are much higher than the inner coupling coefficients, as was already noted by inspection of the even-odd mode impedances.

- We can also display each coupling coefficient one by one, together with the resonance frequencies of the coupled resonators by placing the cursor under any coupling element (TL) and pressing the keys ALT+F6. Here again we should type in the center frequency and two frequencies on the two sides of the center frequency.

- The coupling coefficients may also be calculated from the circuit as follows:

- Let’s calculate the coefficient k34. For the measurement of coupling coefficient k34, the neighboring resonators (at nodes 2 and 5) need be grounded (or detuned completely). In order to simulate this condition on Filpro, delete all the elements except the ones connected to nodes 3 and 4, as shown in Figure 12.49. x.

- In order to simulate the grounding at node-2 and 5, replace the transmission lines connected to these nodes by shunt SC stubs with the same impedance and length as the TL’s (Figure 12.49.y).

- Now we need to send a small amount of signal from the OC end of the left resonator and get the output from the OC end of the right resonator. In order to do that without effecting the circuit characteristics, we use very lightly coupled probes. On Filpro we can simulate such probes by a series connected very small input and output coupling capacitors, for example, C=0.00005 pF (Figure 12.49.z1).

- Plotting insertion loss of this circuit with vertical scale increased to 200 dB, we get the plot shown in Figure 12.49.z1. Here we see two peaks. The frequencies of these peaks are denoted by f1 and f2. The difference f2-f1 is termed as Coupling Bandwidth CBW. We measure this difference as  f2-f1=1016-884=32 MHz. The coupling coefficient k34 is given by the ratio k34=CBW/f1*f2 = 32/1000 = 0.032 which is almost the same as k34  displayed by the keys CTRL+F6.

- This approach is used in testing and tuning of the realized filters.

Effects of finite Q of resonators can be observed as follows:

- Load the circuit INT03.fil (Figure 12.49.z2).

- With cursor under any LC resonator apply command Parasitics / All Lossy Resonators: f0=1000 MHz, Q=... . Here we type in the Q of resonators. This command will convert all LC resonators into RLC resonators where R represents the resistive losses of resonators (Figure 12.49.z3). Figure 12.49.z4 shows insertion loss responses for several Q values.

Figure 12.49. Interdigital filter design with redundant unit elements using Synthesis approach.

Figure 12.49. Interdigital filter design with redundant unit elements using Synthesis approach (cont’d).

Figure 12.49. Interdigital filter design with redundant unit elements using Synthesis approach (cont’d).

Figure 12.49. Interdigital filter design with redundant unit elements using Synthesis approach (cont’d).

Figure 12.49. Interdigital filter design with redundant unit elements using Synthesis approach (cont’d).

B) Impedance scaling by tapping:

In this approach the input and output of the filter are taken from the taps placed on the first and last resonators of the filter [18]-[26]. This approach avoids introduction of redundant unit elements, hence saves both size and loss while also avoiding the problems associated with tight couplings of input and output redundant lines. The steps for such a design are shown in Figure 12.50:

- Load the designed (unscaled) circuit INT01.fil (Figure 12.50.a). Apply the command Inverters / Scale Circuit By Inserting Inverters / Specify SC stub: Z=80 ohms (Figure 12.50.b). Note that in this scaling we directly assign the resonator (SC stub) impedances. This circuit is saved as INT04.fil.

- With cursor under the inverters at source and load ends we apply the command Tapped PCL:  Z=80 ohms, f0=1000 MHz, fq=1000 MHz. Here Z is the impedance of the resonator to be tapped. That is, we may set it as we like. However it is usual to leave it at the same value as the other resonators. In this approximation we set f0=fq. Response seems to be the least distorted by these values. This command calculates the position of tap point on the first and last resonators. The portion of the resonators between the tapping point and SC end are shown in Figure 12.50.c as shunt SC stubs with length L1 (tapping distance to SC end). The remaining piece of the resonators are shown as transmission lines with length L2=L-L1 where L is the original (commensurate) length of the resonators. This approximation necessitates placement of a capacitor at the OC end of the tapped resonators, shown as CT. Return loss is degraded towards the upper end of the passband.

- In Figure 12.50.d a pictorial description of the tapped structure is shown.

- Return loss can be recovered by tuning the impedances of coupling transmission lines, lengths of shunt SC stubs and tapping lengths, by using both Tune Element and Best Case commands. Results of an optimization trial is shown in Figure 12.50.e. A slight shift of the whole passband towards lower frequencies is observed. This shift comes about 0.5-1 MHz for this filter and can be taken into consideration during the initial design specifications stage.

- Coupling coefficients, External Q and Resonance Frequencies can be displayed by loading the filter INT04.fil and then converting the shunt SC stubs into lumped LC resonators, as described in Figure 12.50.f-g.

- Effects of losses can be seen by converting the LC resonators into RLC form, by using the command Parasitics/ All Lossy Resonators: Q=... (Figure 12.50.h-k).

- Even and odd mode impedances of adjacent pairs can be displayed by converting each SC Stub+TL+SC Stub triplets into SC edge coupled line form, as described in Figure 12.50.m-to-q.  Figure 12.50.r shows the complete list of the even-odd mode impedances of the adjacent pairs of coupled lines.

In the above description we have neglected the effects of the fringing capacitors at the open ends of the resonators which may be important if thick resonators are used. If their orders are known we can Insert them as lumped shunt capacitors in parallel with each shunt SC stub one by one and then Tune (decrease) the length of the adjacent SC stub until return loss is recovered. This brute force approach necessitates adjustment of the lengths of adjacent TL’s. This is because these TL’s are actually simulating the two-wire line effect of the separations between the adjacent resonators. Therefore their lengths are set by the lengths of resonators.  For all these reasons tuning-optimization of the response is more complex, necessitating a more general optimizer than Filpro. Fortunately this problem is resolvable by designing the filter as distributed element bandpass filter which will place capacitors in parallel with each shunt SC stub automatically. These capacitors can then be used to absorb the frinding capacitors of the open ends of resonators, as will be described in the next section.

Figure 12.50. Design of a tapped interdigital filter using Synthesis approach.

Figure 12.50. Design of a tapped interdigital filter using Synthesis approach (cont’d).

Figure 12.50. Design of a tapped interdigital filter using Synthesis approach (cont’d).

EXAMPLE 12. 18: Interdigital filter design with redundant unit elements using Built in LC prototype approach

Built in LC Prototype approach is a short cut approach for designing narrowband interdigital filters with redundant unit elements. Let’s see this approach on the same example as the previous one. Specifications are:

PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz, NR=7, RS=RL=50 to be used as a bandpass filter to pass the frequency range fp1=975 MHz, fp2=1025 MHz. Resonator impedances are desired to be exactly 80 ohms.

- The command set is shown in Figure 12.51. Design / By Built in LC Prototype commands are invoked. Distributed / Highpass Filter options are selected. PBSpecs and Prototype Degree option is selected and the specifications are typed in. Find Circuit command brings several options. We select the option With Redundant Unit Elements. Then the option OSsr+TL+OSsr is selected. Then Resonator Impedance window appears. For interdigital filters, this is not the actual resonator impedance, but just a design parameter which should be iterated by trials until the targeted SC stub impedances are obtained at the end of the design.  In this example we have found it as Z0=36.22 after a few trials. After typing in this number, we get the circuit shown in Figure 12.51.a which is actually a circuit prepared for edge coupled line filters, but useful for interdigital filters as well.

- We apply Simplify Circuit command to combine the adjacent stubs (Figure 12.51.b).

- Then Find Dual command is applied to get the version which is equivalent to interdigital filters (Figure 12.51.c). Then with cursor under any TL, the command Kuroda/ All Equal Stubs / Include End Stubs is applied giving us the circuit with 80 ohms resonator (SC stub) impedances (Figure 12.51.d). Response has some very slight degradations which have no practical significance.

- External Q, Coupling Coefficients, effects of losses, even-odd mode impedances, etc. can be displayed by using the same procedures used in the previous example.

- The effects of fringing capacitors at the open ends of resonators may degrade response appreciably depending on their magnitudes. If these capacitances are known their effects can be taken into account by reducing the lengths of resonators. However as the length of resonators (SC stubs) are reduced the length of TL’s separating the SC stubs need also be reduced. Therefore this tuning-trimming process may be tedious. If preferred, one may solve this problem by designing the filter as a capacitively loaded interdigital filter as will be described in later sections of this chapter.

Figure 12.51. Interdigital filter design with redundant unit elements using Built in LC Prototype approach.

EXAMPLE 12. 19. An interdigital filter design using Prototype with Inverters approach

Prototype with inverters approach is also a quick and flexible tool for designing interdigital filters. Let’s see design and circuit transformation steps on the same filter as the previous examples. Specifications are:

PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz, leading to a bandpass filter with fp1=975 MHz and fp2=1025 MHz. Seven resonators will be used, with 80 ohms resonator impedances.

The command steps in this design approach has two stages: 1) Preparation of the lowpass prototype.         2) Forming the highpass filter. These two stages are shown in Figure 12.52.

- Design / By Prototype with Inverters / Choose a Standard Circuit commands are invoked. The option Shunt Capacitors+Inverters is selected. Then the prototype degree and passband ripple are specified. The resulting prototype circuit is displayed automatically after these commands.

- In the second stage the desired highpass filter is formed by using the lowpass prototype prepared in the first stage. At this stage the command Filter Type is invoked and Distributed / Highpass options are selected. Then Passband and Terminations command is invoked and the passband edge frequency fp and quarter wave frequency fq of the highpass filter are typed in. Here fq is also the passband center. Find Circuit command displays the designed filter (Figure 12.52.a).

- The ideal inverters of this circuit enables easy manipulations for impedance scaling, element value modifications and element types. For example the shunt SC stubs can be equated by applying the command Inverters / All Equal Left-Right (Figure 12.52.a-b). This command scales all the inverters such that elements on its two sides will have equal impedances. In the resulting circuit all shunt SC stubs will have the impedance of the leftmost stub. Since the circuit is symmetric, the transformers arising from this process comes out to be unity.

- Then with cursor under the inverters, one by one, we apply the command Inverters / Replace Ideal Inverters By / SSsh+TL+SSsh / Specify fq / fp=975 MHz, fq=1000 MHz. This command replaces the ideal inverters by quarter wavelength lines with negative shunt SC stubs on the two sides (Figure 12.52.c). This approximation is a good one for narrow-medium bandwidth filters, as almost no degradation arises in response.

- The negative shunt SC stubs are absorbed by their positive neighbors after Simplify Circuit command (Figure 12.52.d). After this command equality of shunt SC stubs is slightly degraded. They are still close to each other, but no more equal. They are equated again by placing the cursor under any TL and applying the command Kuroda / All Equal Stubs / Include End Stubs (Figure 12.52.d-e). Stubs are equated at 2.56 ohms level which is too low for realization. The circuit is saved as, for example Intd21.fil. Impedances of resonators (SC stubs) can be scaled up either by inserting redundant unit elements or by using tapping the input and output resonators.

Impedance Scaling with Redundant Unit Elements:

The procedure for creating redundant unit elements is as follows:

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=60.5 is applied (Figure 12.52.e-f). Here the value K=60.5 is found after a few full design trials to get a circuit with all shunt SC stubs having the targeted 80 ohms. This command inserts inverters at source and load ends while also changing impedances of the circuit.

- Next, with cursor under the inverters, the command Inverters / Replace Ideal Inverter By / SSsh+TL+SSsh / Specify fq: f0=975 MHz, fq=1000 MHz is applied to replace them by a unit element (TL) surrounded by two negative shunt SC stubs (Figure 12.52.g). This is the required redundant unit element which will be used for scaling impedances.

- The negative SC stubs are combined with their neighboring positive stubs by applying Combine command. However the negative SC stubs at the source and load ends have no neighbors to absorb them. Therefore we resort to brute force and simply Delete them (Figure 12.52.h). This will cause a slight degradation in return loss response as shown in the figure. This degradation can be corrected by tuning the SC stub adjacent to the redundant unit elements at ends (Figure 12.52.i).

- With cursor under the redundant unit elements, we apply the command Kuroda / Distribute stub. This command will transfer part of the SC stub adjacent to the unit element to the other of the unit element (Figure 12.52.k). This transfer will create transformers which scales up the impedance level of the circuit. Note that the SC stubs at the source and load ends are 80 ohms.

- Finally with the cursor under any TL, the command Kuroda / All Equal Stubs / Include End Stubs is applied. This command equates all the inner SC stub impedances to the leftmost stub which is 80 ohms (Figure 12.52.m). Thus the design is finished. Coupling Coefficients, External Q, even and odd mode impedances, nodal voltage, current, power, etc. can be displayed through the same steps used in the previous examples.

Impedance Scaling by Tapping:

- In this approach we first load the circuit Int21.fil and apply command Inverters / Scale Circuit By Inserting Inverters / Specify SC Stub: Z=80 ohms. This command scales up the SC stub impedances directly to the desired 80 ohms level by inserting inverters of K=279 ohms at source and load ends (Figure 12.52.n-p).

- Next the inverters at source and load ends will be simulated by taps on the first and last resonators (SC stubs). This is done by placing the cursor under the inverters and applying the command Tapped PCL: Z=80 ohms, f0=1000 MHz, fq=1000 MHz. The inverter will be replaced by a shunt SC stub of length L1 followed by a TL of length L2=L-L1 where L is the commensurate length of the filter. L1 is the distance of the tapping point to the SC end of the resonator while L2 is the distance of the tapping point to the open end of the resonator. This conversion introduces also a lumped capacitor at the open end of the tapped resonator (Figure 12.52.q).

- This approximation degrades the response as shown in the figure. Degradation can be corrected to a certain extent by tuning all elements slightly, as shown in Figure 12.52.r.

- Figure 12.52.s shows a pictorial description of the filter.  Coupling coefficients, effects of losses, even and odd mode impedances can be displayed as described in the previous example.

- The effects of fringing capacitors at the open ends of resonators may degrade response appreciably depending on their magnitudes. If these capacitances are known their effects can be taken into account by reducing the lengths of resonators. However as the length of resonators (SC stubs) are reduced the length of TL’s separating the SC stubs need also be reduced. Therefore this tuning-trimming process may be tedious. If preferred, one may solve this problem by designing the filter as a capacitively loaded interdigital filter as will be described in later sections of this chapter.

Figure 12.52. Interdigital filter with redundant using  Prototype with Inverters approach.

Figure 12.52. Narrowband interdigital filter design using  Prototype with Inverters approach (cont’d).

Figure 12.52. Narrowband interdigital filter design using  Prototype with Inverters approach (cont’d).

12. 14.  BRUTE FORCE FINITE TRANSMISSION ZEROS IN INTERDIGITAL FILTERS

Finite transmission zeros can’t be placed into the transmission zero pattern during the synthesis stage of interdigital filters. This is because its implementation is not practical, as it was described in the medium bandwidth digital filters with finite transmission zeros (see section 12.10). However if mandatory, finite transmission zeros can be added to the resulting interdigital filter by brute force, at the expense of some degradation in return loss, as described in the following example.

EXAMPLE 12. 20. An interdigital filter with brute force finite transmission zeros

Let’s consider the narrowband distributed element highpass filter of the previous examples:

PBRipple=0.01 dB, fp=975 MHz, fq=1000 MHz to work as a bandpass filter with fp1=975 MHz and fp2=1025 MHz. Seven resonators will be used. We will place finite transmission zeros (FTZ) at f1=960 MHz and f2=1040 MHz. Let’s use Design By Built in LC Prototype approach for a quick design.

- The procedure is shown in Figure 12.53 and resulting circuit is shown in Figure 12.53.a. In order to convert this circuit into the standard interdigital topology, first Simplify Circuit command is applied to combine the adjacent OC stubs and then Find Dual command is applied giving the circuit of Figure 12.53.b. Kuroda / All Equal Stubs / Include End Stubs command is used to equate all the shunt SC stubs (Figure 12.53.c). Thus, the main interdigital filter is ready. This circuit is saved for example as Int51.fil.

- Now the first finite transmission zero (FTZ1) section will be prepared to notch out f1=960 MHz and inserted into the main filter by brute force.  One way to form such a circuit is to load Int51.fil and Delete all elements except the first TL and a SC stub (Figure 12.53.d-e). The load resistance RL=50 ohms is changed to RL=99999999 ohms to simulate an open circuit. Then Tune Element command is applied on the SC stub to tune its length while observing the Smith Chart response displayed within the frequency band of 950 MHz and 1050 MHz. In this procedure we first select Length of the SC stub for tuning and when Smith Chart is displayed we set the frequency of interest (960 MHz) by using left-right arrows so that this frequency will appear in the box on the top-right. Then using up-down arrows we move the marker  “+” (NOT the marker “X”!) to the leftmost position on the chart. At this position the input impedance of this circuit is zero, simulating the series resonance in shunt arm. Thus, the notch section is ready. It is saved as FTZ1.fil (Figure 12.53.f-g). As should be expected length of the resonator (SC Stub) ( 78.928 mm) is longer than the commensurate length of 74.948 mm.

- Now load the filter Int51.fil and with cursor under the first TL on left, apply command External 1-Port/ FTZ1.fil / Shunt 1-Port. This command will connect the circuit FTZ1.fil to the left of this TL in shunt Figure 12.53.h-i).  Save the circuit as Int52.fil. Inspection of the insertion loss response shows indeed a transmission zero at 960 MHz (Figure 12.53.k). However this brute force insertion of the FTZ section degraded both insertion loss and return loss heavily. This degradation can be corrected to a limited extent by tuning all elements of the main filter and the FTZ section.

- First, elements of the FTZ section is tuned while observing return loss and occasionally checking variation in the frequency of FTZ on insertion loss.  Elements of the FTZ section can’t be tuned by Tune Element command, because it is not a single element but a circuit represented by a symbol. On Filpro such elements are named as External 1-Port elements and used basically for formation of multiplexers by connecting channel filters to the main manifold. In this respect this FTZ section is similar to a multiplexer branch and its elements can be tuned by using Tune Multiplexer command. With cursor under the External 1-Port element, when this command is applied, the circuit of FTZ1.fil is displayed for selection of the element and parameter to be tuned. Selection is made by moving the cursor with left-right arrows and pressing Enter key (Figure 12.53.m).

- If Insertion loss is to be observed then two output signals will be displayed, one being the signal on the actual load resistance RL at right and the other being the signal on the load resistor inside the FTZ1 circuit which is infinite (999999..). Therefore in insertion loss plots, we will disregard the second plot. Up-down arrows are used to tune the selected parameter. When tuning of a parameter is finished F2 key is pressed to save the last value of that parameter and Esc key is pressed to pass to tuning of the next parameter. We must also note down this last value, as it will be lost after tuning of the External 1-Port section is finished.

- Figure 12.53.n shows the improvement in return loss and insertion loss responses after tuning the elements of the FTZ1 circuit. When tuning of all elements of the External 1-Port circuit is finished we load the circuit FTZ1.fil and use Change command to set the tuned values and save the circuit again (Figure 12.53.p). Next, we use Tune Element command to tune all the other elements of the filter (Figure 12.53.q). Then we tune the FTZ1 circuit again (Figure 12.53.r-s).

- Figures 12.53.t-to-x shows the same steps for the placement of the second finite transmission zero section while Figure 12.53.y shows a pictorial description of the final structure. Note that the extra (FTZ) lines at input and output side have different lengths, setting the finite transmission zeros.

Figure 12.53. Interdigital filter with brute force finite transmission zeros.

Figure 12.53. Interdigital filter with brute force finite transmission zeros (cont’d).

Figure 12.53. Interdigital filter with brute force finite transmission zeros (cont’d).

Figure 12.53. Interdigital filter with brute force finite transmission zeros (cont’d).

12. 15.  NARROWBAND CAPACITIVELY LOADED INTERDIGITAL FILTERS

At low frequencies length of bars of interdigital filters comes out long, increasing the transversal size of the filters. Size reduction, hence miniaturization is possible by loading the open ends of the resonator bars by lumped or distributed capacitive reactances. As it was shown in the case of medium bandwidth capacitively loaded interdigital filters, the capacitive loading also helps widening of the upper stopband by moving the first spurious passband away from the main passband. Another use of capacitive loadings may be to absorb the fringing capacitances at the open ends of the resonator bars of interdigital filters. The fringing capacitances at the open ends resonators of interdigital filters can degrade the response appreciably, as they were not taken into account during the design stages. Although these degradations can be corrected by tuning the length of resonator rods, this may come out to be a tedious process. We can absorb these fringing capacitors into intentionally placed capacitors.

Capacitively loaded interdigital filters are designed as distributed element bandpass filters. The quarter wave frequency fq of the bandpass filter must be selected as fq>fp2 as a constraint of Synthesis theory. As fq gets larger the commensurate length (stub and TL length) gets shorter while the capacitances to be connected at the open ends of resonators get larger. Thus, if size reduction is the main aim, then higher fq should be selected. However size reduction increases loss, hence decreases the unloaded Q of the resonators. Therefore an optimum length may be found depending on the geometry and materials used. In miniaturization respects, capacitively loaded interdigital filters can compete with combline filters. If the aim is just to absorb the fringing capacitances without using the capacitors actually while maintaining the longest length (for high Q purposes) we should select fq very close to fp2. However the condition fq>fp2 places a minimum level for the capacitors. If still longer resonators are desired violating the restriction fq>fp2, then Design By Prototype with Inverters approach can be used as will be described.

Let’s see examples for both Synthesis and Prototype with Inverters approaches.

A) NARROWBAND CAPACITIVELY LOADED INTERDIGITAL FILTER DESIGN BY SYNTHESIS

The usual design approach on Filpro is the Synthesis approach. It is exact design approach, involving no approximations. Let’s see two examples, one involving long resonator bars aiming high unloaded Q and the other having short resonator bars aiming at size miniaturization and wide stopband.

EXAMPLE 12. 21.  Capacitively loaded interdigital filter with =85 degrees electrical length

Let’s repeat the previous example, now by designing it as a distributed element bandpass filter with following specifications:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz,

Seven resonators will be used (NR=7). Resonator impedances will be about 80 ohms. The quarter wave frequency can be selected as anywhere in the range fq>fp2. If fq selected to be close to fp2, then the resulting resonator length will be close to the length obtained in the highpass filter approach of the interdigital filter of the previous examples. That is, the electrical length will be close to 90 degrees. This approach is used to keep the high Q of long resonators and the capacitive loadings will be just about the fringing capacitors at the open ends.

In this example electrical length will be taken as =85 degrees to get long resonators. The quarter wavelength frequency corresponding to this electrical length is calculated from the formula

(f0)=(/2)(f0/fq)

Using f0=1000 MHz and =85 degrees we get fq=1058.8 MHz.

- Design command set is shown in Figure 12.54. We select Distributed/ Bandpass/ Type B options and type in fp1=975 MHz, fp2=1025 MHz and fq=1058.8 MHz.

- Transmission zeros are set as Nzero=1 (to get shunt SC stubs), NUE=NR-1=6 and Ninf=NUE+1=7. Here, NUE is selected to be one less than NR. The single shunt SC stub obtained from the single transmission zero at f=0 will be distributed between the six unit elements to produce the seven resonators.  For each of the seven resonator, we assign one transmission zero at infinity to produce the shunt capacitors in parallel with each SC stub. Therefore we place seven transmission zeros at infinity.

- As the transmission zeros are placed we observe build-up of the insertion loss response. Since fq=1058.8 MHz is very close to the passband center f0=1000 MHz, the first spurious passband of the distributed element bandpass filter at about 2fq-f0 comes out to be too close to the desired passband. This is because the shunt OC stubs (transmission zeros at infinite frequency in Richards frequency domain) appear as finite transmission zeros at f=fq=1058.8 MHz. However this is no problem as this spurious passband will disappear after converting the OC stubs into lumped capacitors.

- The transmission zeros are extracted in the order inf-ue-inf-ue-inf-ue-inf-zero-ue-inf-ue-inf-ue-inf  so that the unit elements and shunt OC stubs will alternate in the extracted circuit. The transmission zero of f=0 can be extracted at any position, as it will be distributed between the unit elements later. However it is always good to get a symmetric circuit for numerical accuracy problems. Therefore we prefer to extract it in the middle of the circuit. The extracted circuit is shown in Figure 12.54.a.

- Simplify Circuit command is applied to combine the shunt SC and OC stubs in the middle of the circuit to form a single OC+SC stub element (Figure 12.54.b).

- With cursor under the TL’s adjacent to the OC+SC stub element we apply the command Kuroda / Distribute Stub. This command moves part of the SC stub of the OC+SC stub element to the other side of the TL. Applying Simplify Circuit command again the translated SC stub will be combined with the adjacent OC stub to form a new OC+SC stub element there. This command is repeated until a OC+SC stub element is placed on each side of each TL (Figure 12.546.c).

- Next we convert the OC stubs of the OC+SC stub elements into lumped capacitors by applying the command Replace By Lumped / All SC Stub+Cap. This command converts the OC+SC Stub elements into SC Stubs in parallel with lumped capacitors (Figure 12.54.d).

- Then we apply command Kuroda / All Equal Stubs / Include End Stubs to equate the SC stub impedances of the SC Stub+Cap elements (Figure 12.54.e). We save the resulting circuit as Capint1.fil to use at later stages.

- Figure 12.54.f shows the insertion loss responses before and after conversion of the shunt OC stubs into lumped capacitors. It is seen that the nearest spurious passband disappeared.

- The next stage is scaling impedances to realizable levels. As in the simple interdigital filters of the previous examples, this can be done either by inserting redundant unit elements or by tapping the first and last resonators.

Impedance scaling by redundant unit elements (transformer rods):

In this approach we insert two redundant unit elements at input and output ports and use Kuroda transformations to create two more shunt SC stubs (resonators) at input and output ports. The function of these two resonators is just impedance transformation. They have no contribution to the response. Therefore they are termed as redundant unit elements, though they are actually not redundant when the inserted line impedance is different from RS.  The procedure is as follows:

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=60.6 is applied to insert the impedance scaling inverters (Figure 12.54.g). Here, the inverter constant of 60.6 ohms was found by trials to get 80 ohms resonator impedances.

- Next with cursor under these inverters we apply the command Inverters / Replace Ideal Inverter By / SSsh+TL+SSsh/ Specify fq: f0=1000 MHz, fq=1058.8 MHz. This command replace the inverters with transmission lines (redundant UE) of the same commensurate length as the other lines, surrounded by negative shunt SC stubs (SSsh) on each side (Figure 12.54.h). The negative inverters on the inner sides of the redundant TL’s are combined with the adjacent positive SC stubs. The negative SC stubs at source and load ends have no neighbors to absorb them. Therefore we simply Delete them. The brute force elimination of these elements will degrade response to some extent. However response can be recovered by tuning the first and last TL’s together with the adjacent SC stubs (Figure 12.54.i).

- With cursor under the redundant TL at source end we apply the command Kuroda / Distribute Stub. This command moves part of the SC stub on the right of the redundant TL to its left, creating a redundant resonator. The same command is repeated at the load end also (Figure 12.54.k).

- Finally we apply the command Kuroda / All Equal Stubs / Include End Stubs to equate all SC stub impedances (Figure 12.54.m). It is seen that the SC stub impedances are equated at 80.2 ohms. The design is finished. The circuit is saved as Capint2.fil. If exact 80 ohms is desired we should repeat the design with a lower value of K, for example K=60.5 ohms (exercise?). Note that the resonator lengths are 70.7 mm. The loading capacitors are about 0.2 pF. If different lengths and capacitors are desired then we should repeat the design with a different fq.

- External Q and  Coupling Coefficients can be displayed by pressing the keys CTRL+F6 and selecting the option Direct Coupled (Figure 12.54.n). Individual Coupling Coefficients and the associated Resonance Frequencies can be displayed by placing the cursor under a coupling element (TL) and pressing the keys ALT+F6. Note that the coupling coefficients of the redundant resonators are much tighter than inner resonators.

- Since this is a multiconductor coupled line system, in general the physical parameters like widths, separations, etc. should be found by using EM (electromagnetic) simulations through iterative search until the desired response is obtained. However this would be an impossibly difficult and tedious job unless a good initial (starting) set of parameters is available. An approximate set for widths and separations of adjacent resonators can be found if we can treat the conductors as adjacent pairs isolated from the other conductors. In this case we can find even and odd mode impedance descriptions of these pairs of coupled lines from the static capacitance matrix description obtained from the circuit Capint2.fil and then use the available charts, tables or formulations developed for even and odd mode impedances of two identical parallel coupled transmission lines. Since this approach neglects the electromagnetic effects of the other lines, we can’t get exact results, but we would be close to the solution. The even and odd mode impedances of adjacent pairs of resonators can be found as follows:

- Load Capint2.fil (Figure 12.54.p). With cursor under any SC Stub+Cap resonator, apply the command Split / All Two Element Resonators. This command will split all SC Stub+Cap elements into their SC stub and capacitor components (Figure 12.54.q).

- Delete the capacitors (Figure 12.54.r). Save the resulting circuit as Capint3.fil. Its static capacitance model is shown in Figure 12.54.s. In this circuit we can form the SC Stub+TL+SC Stub triplets for conversion into coupled lines with SC ends, as described in Figure 12.54.t-u. This is done by placing the cursor under the TL’s and applying the command Replace By PCL / Interdigital / scscINTD / Symmetric. Since all SC stubs are shared by their two neighboring TL’s, these conversions must be done one by one. Figure 12.54.v shows a pictorial view of the capacitively loaded interdigital filter together with the even and odd mode impedances of the adjacent pairs of lines.

- Effects of losses can be inspected by first converting the SC stubs and SC Stub+Cap resonators of Capint2.fil into lumped LC resonators by applying the command Replace By Lumped / All Lumped LC and then converting the LC resonators into RLC form by applying the command Parasitics / All Lossy Resonators:  f0=1000 MHz, Q=... (Figure 12.54.w-x-y). Figure 12.54.z shows responses for various Q values.

- Node voltages, currents and powers can be displayed by loading Capint1.fil, pressing the keys ALT+F8 and typing in the frequency (Exercise?).

Figure 12.54. Synthesis of capacitively loaded interdigital filter with redundant unit elements.

Figure 12.54. Synthesis of capacitively loaded interdigital filter with redundant unit elements (cont’d).

Figure 12.54. Synthesis of capacitively loaded interdigital filter with redundant unit elements (cont’d).

Figure 12.54. Synthesis of capacitively loaded interdigital filter with redundant unit elements (cont’d).

Impedance scaling by Tapping:

The redundant unit element approach increases both size and hence loss of the filter. Further, the redundant resonators are more tightly coupled to the adjacent resonators compared to the inner resonator coupling coefficients. This may also be problematic in some realizations. Tapping approach avoids these problems. In this approach the input and output couplings are provided by taps on the first and last resonators of the filter. The procedure is as follows:

- Load the circuit Capint1.fil (Figure 12.55.a). Apply command Inverters / Scale Circuit By Inserting Inverters / Specify SC Stub: Z=80 ohms, we get the scaled up version as shown in Figure 12.55.b. In this approach we can directly specify the impedance of SC stubs (resonators). This circuit is saved as Capint5.fil.

- With cursor under the source end inverter, apply command Tapped PCL: Z0=80 ohms, f0=997 MHz, fq=1058.8 MHz. This command deletes the inverter and inserts a shunt SC stub of 80 ohms followed by a transmission line of 80 ohms, representing tapped input resonator. In this command user can select the impedance of resonator to be tapped. The frequency f0=997 MHz is calculated by Filpro and it is the nodal resonance frequency of the first resonator (node-1) at which the SC Stub and TL pair is equivalent to the inverter. It may also be changed by the user. However for most cases the displayed default value is the best choice. Length of the shunt SC stub is denoted by L1 and it represents the portion of the first resonator between the tapping point and the SC end of the resonator. Length of the transmission line piece is L2=L-L1 where L is the commensurate length of the filter. This line piece represents the portion of resonator between the tapping point and its open end (capacitively loaded end). The same command is repeated on the load side inverter giving the circuit of Figure 12.55.c. The capacitors of the tapped resonators are shown explicitly in Figure 12.55.c. These capacitors are also modified slightly in this transformation.

- This conversion will introduce some degradation in response. However this degradation can be corrected by tuning all elements slightly, as shown in Figure 12.55.d.

- Figure 12.55.e shows a pictorial description of the tapped interdigital filter.

- Coupling Coefficients, External Q and resonance frequencies can be displayed from the original circuit, Capint5.fil because these parameters don’t change much after tapping. Therefore we load Capint5.fil, press the keys CTRL+F6 and select the option Direct Coupled to display the External Q and Coupling Coefficients (Figure 12.55.f). The individual Coupling Coefficients can also be displayed by placing the cursor under the coupling elements (transmission lines) and pressing the keys ALT+F6. This command displays the resonance frequencies of the coupled resonators also.

- In electromagnetic simulations it is good to have a rough initial set of physical parameters like widths and separations of the resonators to start an iterative search. By neglecting the effects of other lines, any adjacent pairs of coupled lines can be characterized  through their even and odd mode impedances that can be obtained from the static capacitance model. These impedances can then be used to find approximate widths and separations of coupled lines from charts, graphs or formulations developed for two coupled line systems. Figure 12.55.g-to-p describe this procedure:

- The circuit Capint5.fil is loaded (Figure 12.55.g). With cursor under any SC Stub+Cap resonator, the command Split/ All Two Element Resonators is applied. This command splits the SC Stub+Cap resonators into their SC stub and capacitor components (Figure 12.55.h).

- Then the capacitors are deleted leaving only transmission lines separated by SC stubs, saved as Capint6.fil (Figure 12.55.i). Its static capacitance model is shown in Figure 12.55.k. In this circuit we can now form SC Stub+TL+SC Stub triplets for conversion into coupled line form. This is done by placing the cursor under transmission lines and applying the command Replace By PCL / Interdigital / scscINTD / Symmetric (Figure 12.55.m-n). After converting a triplet into coupled line form, we note down the even and odd mode impedances of that coupled line and then apply Undo command to recover the original circuit, for the formation of the next coupled line section. This is necessary because the adjacent coupled line pairs share one SC stub.

- Figure 12.55.p shows a pictorial description together with the even and odd mode impedances of adjacent resonators. Effects of losses can be inspected by first forming the lumped LC resonator form as described in Figure 12.55.q-r by applying the command Replace By Lumped / All Lumped LC on the SC Stub+Cap resonators of Capint5.fil. Then the lumped LC resonators are converted into their lossy counterparts by placing the cursor under any LC resonator and applying the command Parasitics / All Lossy Resonators: f0=1000 MHz, Q=... ( Figure 12.55.s). Here, Q of the resonator is assigned by the designer. Figure 12.55.t shows some responses for Q=200 and Q=500. Since we have two less resonators compared to the filter with redundant unit elements, we have slightly less passband insertion loss.

Figure 12.55. Synthesis of capacitively loaded tapped interdigital filter.

Figure 12.55. Synthesis of capacitively loaded tapped interdigital filter (cont’d).

Figure 12.55. Synthesis of capacitively loaded tapped interdigital filter (cont’d).

EXAMPLE 12. 22. A miniature capacitively loaded interdigital filter

In this example we will design the same capacitively loaded filter of the previous example but the target is to decrease resonator lengths by selecting a higher quarter wave frequency fq. It as a distributed element bandpass filter with specifications,

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz,

Seven resonators will be used (NR=7). Resonator impedances will be about 80 ohms. Let’s select the electrical length as =45 degrees. The quarter wavelength frequency corresponding to this electrical length is calculated from the formula

(f0)=(/2)(f0/fq)

Using f0=1000 MHz and =45 degrees we get fq=2000 MHz

- Design command set is shown in Figure 12.56. We select Distributed/ Bandpass/ Type B options and type in fp1=975 MHz, fp2=1025 MHz and fq=2000 MHz.

- Transmission zeros are set as Nzero=1 (to get shunt SC stubs), NUE=NR-1=6 and Ninf=7. Here, the single shunt SC stub obtained from the single transmission zero at f=0 will be distributed between the six unit elements to produce seven resonators.  For each of the seven resonator, we assign one transmission zero at infinity to produce the shunt capacitors in parallel with each SC stub. Therefore we place seven transmission zeros at infinity.

- As the transmission zeros are placed we observe build-up of the insertion loss response. Since fq=2000 MHz is far from the passband center, we now have a much wider stopband compared to the =85 degrees case. The first spurious passband of the distributed element bandpass filter appears at around 2fq-f0=3000 MHz. This passband is created by the shunt OC stubs. It will be eliminated after converting the shunt OC stubs into lumped capacitors, as will be seen in the succeeding stages.

- The transmission zeros are extracted in the order inf-ue-inf-ue-inf-ue-inf-zero-ue-inf-ue-inf-ue-inf so that the unit elements and shunt OC stubs will alternate in the extracted circuit. The transmission zero of f=0 can be extracted at any position, as it will be distributed between the unit elements later. However it is always good to get a symmetric circuit for numerical accuracy problems. Therefore we prefer to extract it in the middle of the circuit. The extracted circuit is shown in Figure 12.56.a.

- Simplify Circuit command is applied to combine the shunt SC and OC stubs in the middle of the circuit to form a single OC+SC stub element (Figure 12.56.b).

- With cursor under the TL’s adjacent to the OC+SC stub element we apply the command Kuroda / Distribute Stub. This command moves part of the SC stub of the OC+SC stub element to the other side of the TL. Applying Simplify Circuit command again the translated SC stub will be combined with the adjacent OC stub to form a new OC+SC stub element. This command is repeated until a OC+SC stub element is placed on each side of each TL (Figure 12.56.c).

- Next we convert the OC stubs of the OC+SC stub elements into lumped capacitors by applying the command Replace By Lumped / All SC Stub+Cap. This command converts the OC+SC Stub elements into SC Stubs in parallel with lumped capacitors (Figure 12.56.d).

- Then we apply command Kuroda / All Equal Stubs / Include End Stubs to equate the SC stub impedances of the SC Stub+Cap elements (Figure 12.56.e). We save the resulting circuit as Capint11.fil to use at later stages.

- Figure 12.56.f shows wideband insertion loss responses before and after conversion of the shunt OC stubs into lumped capacitors. It is seen that the spurious passbands at f=3000, 5000, 7000, 9000, .. MHz disappeared and new spurious passbands appeared with wider separations, the nearest of which is about 4300 MHz.

- The next stage is scaling impedances to realizable levels. As in the the previous example, this can be done either by inserting redundant unit elements or by tapping the first and last resonators.

Impedance scaling by redundant unit elements:

In this approach we insert two redundant unit elements at input and output ports and use Kuroda transformations to create two more shunt SC stubs (resonators) at input and output ports. The function of these two resonators is just impedance transformation. They have no contribution to the response. Therefore they are termed as redundant unit elements.  The procedure is as follows:

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=54 is applied to insert the impedance scaling inverters (Figure 12.56.g). Here, the inverter constant of 54 ohms was found after a few rounds of design trials to get roughly 80 ohms resonator impedances.

- Next with cursor under these inverters we apply the command Inverters / Replace Ideal Inverter By / SSsh+TL+SSsh/ Specify fq: f0=1000 MHz, fq=2000 MHz. This command replace the inverters with transmission lines (redundant UE) of the same commensurate length as the other lines, surrounded by negative shunt SC stubs (SSsh) on each side (Figure 12.56.h). The negative inverters on the inner sides of the redundant TL’s are combined with the adjacent positive SC stubs. The negative SC stubs at source and load ends have no neighbors to absorb them. Therefore we simply Delete them. The brute force elimination of these elements will degrade response a lot compared to the previous example. Fortunately, response can be recovered by tuning the first and last TL’s together with the adjacent SC stubs (Figure 12.56.i).

- With cursor under the redundant TL at source end we apply the command Kuroda / Distribute Stub. This command moves part of the SC stub on the right of the redundant TL to its left, creating a redundant resonator. The same command is repeated at the load end also (Figure 12.56.k).

- Finally we apply the command Kuroda / All Equal Stubs / Include End Stubs to equate all SC stub impedances (Figure 12.56.m). It is seen that the SC stub impedances are equated at 80.3 ohms. The design is finished. The circuit is saved as Capint12.fil. If exact 80 ohms is desired we should repeat the design with a lower value of K. Note that the resonator lengths are 37.4 mm, almost half of the previous example. The loading capacitors are about 2 pF, much larger than the 0.2 pF of the previous example. If different lengths and capacitors are desired then we should repeat the design with a different fq.

- External Q and  Coupling Coefficients can be displayed by pressing the keys CTRL+F6 and selecting the option Direct Coupled (Figure 12.56.n). Individual Coupling Coefficients and the associated Resonance Frequencies can be displayed by placing the cursor under a coupling element (TL) and pressing the keys ALT+F6. Note that the coupling coefficients of the redundant resonators are much tighter than those of inner resonators.

- Since this is a multiconductor coupled line system, in general the physical parameters like widths, separations, etc. should be found by using EM (electromagnetic) simulations through iterative search until the desired response is obtained. However this would be a difficult and tedious job unless a good initial (starting) set of parameters is available. An approximate set for widths and separations of adjacent resonators can be found if we can treat the conductors as adjacent pairs isolated from the other conductors. In this case we can find even and odd mode impedance descriptions of these pairs of coupled lines from the circuit Capint12.fil and then use the available charts, tables or formulations developed for even and odd mode impedances of two identical parallel coupled transmission lines. Since this approach neglects the electromagnetic effects of the other lines, we can’t get exact results, but we would be close to the solution. The even and odd mode impedances of adjacent pairs of resonators can be found as follows:

- Load Capint12.fil (Figure 12.56.p). With cursor under any SC Stub+Cap resonator, apply the command Split / All Two Element Resonators. This command will split all SC Stub+Cap elements into their SC stub and capacitor components (Figure 12.56.q).

- Delete the capacitors (Figure 12.56.r). Save the resulting circuit as Capint13.fil. Its static capacitance model is shown in Figure 12.56.s. In this circuit we can form the SC Stub+TL+SC Stub triplets for conversion into coupled lines with SC ends, as described in Figure 12.56.t-u. This is done by placing the cursor under the TL’s and applying the command Replace By PCL / Interdigital / scscINTD / Symmetric. Since all SC stubs are shared by their two neighboring TL’s, these conversions must be done one by one. Figure 12.56.v shows a pictorial view of the capacitively loaded interdigital filter together with the even and odd mode impedances of the adjacent pairs of lines.

- Effects of losses can be inspected by first converting the SC stubs and SC Stub+Cap resonators of Capint12.fil into lumped LC resonators by applying the command Replace By Lumped / All Lumped LC and then converting the LC resonators into RLC form by applying the command Parasitics / All Lossy Resonators:  f0=1000 MHz, Q=... (Figure 12.56.w-x-y). Figure 12.56.z shows responses for various Q values.

Figure 12.56. Miniature capacitively loaded interdigital filter with redundant unit elements.

Figure 12.56. Miniature capacitively loaded interdigital filter with redundant unit elements (cont’d).

Figure 12.56. Miniature capacitively loaded interdigital filter with redundant unit elements (cont’d).

Figure 12.56. Miniature capacitively loaded interdigital filter with redundant unit elements (cont’d).

Impedance scaling by Tapping:

Tapping approach helps further miniaturization. The redundant unit element approach increases both size and hence loss of the filter. Further, the redundant resonators are more tightly coupled to the adjacent resonators compared to the inner resonator coupling coefficients and this may be problematic for realization in some cases. In this approach the input and output couplings are provided by taps on the first and last resonators of the filter. The procedure is as follows:

- Load the circuit Capint11.fil (Figure 12.57.a). Apply command Inverters / Scale Circuit By Inserting Inverters / Specify SC Stub: Z=80 ohms, we get the scaled up version as shown in Figure 12.57.b. In this approach we can directly specify the impedance of SC stubs (resonators). This circuit is saved as Capint15.fil.

- With cursor under the inverters, apply command Tapped PCL: Z0=80 ohms, f0=983.2 MHz, fq=2000 MHz. This command deletes the inverter and instead, it will insert a shunt SC stub followed by a transmission line, both having impedance of 80 ohms. This shunt SC Stub and TL pair simulates tapped resonators (Figure 12.57.c). Length of the shunt SC stub is denoted by L1 and it represents the portion of the first resonator between the tapping point and the SC end of the resonator. Length of the transmission line piece is L2=L-L1 where L is the commensurate length of the filter. This line piece represents the portion of resonator between the tapping point and its open end (capacitively loaded end). The capacitors of the tapped resonators are shown explicitly in Figure 12.57.c. These capacitors are also modified slightly in this transformation. The frequency 983.2 MHz is the nodal resonance frequency at node-1 at which the SC Stub and TL pair is equivalent to the inverter.

- This conversion will introduce some degradation in response. However this degradation can be corrected to a great extent by tuning all elements slightly, as shown in Figure 12.57.d.

- Figure 12.57.e shows a pictorial description of the tapped interdigital filter.

- Coupling Coefficients, External Q and resonance frequencies can be displayed from the original circuit, Capint15.fil because these parameters don’t change much after tapping. Therefore we load Capint5.fil, press the keys CTRL+F6 and select the option Direct Coupled to display the External Q and Coupling Coefficients (Figure 12.57.f). The individual Coupling Coefficients can also be displayed by placing the cursor under the coupling elements (transmission lines) and pressing the keys ALT+F6. This command displays the resonance frequencies of the coupled resonators also.

- In electromagnetic simulations it is good to have a rough initial set of physical parameters like widths and separations of the resonators to start an iterative search. By neglecting the effects of other lines, any adjacent pairs of coupled lines can be characterized  through their even and odd mode impedances. These impedances can then be used to find approximate widths and separations of coupled lines from charts, graphs or formulations developed for two coupled line systems. Figure 12.57.g-to-p describe this procedure:

- The circuit Capint5.fil is loaded (Figure 12.57.g). With cursor under any SC Stub+Cap resonator, the command Split/ All Two Element Resonators is applied. This command splits the SC Stub+Cap resonators into their SC stub and capacitor components (Figure 12.57.h).

- Then the capacitors are deleted leaving only transmission lines separated by SC stubs, saved as Capint16.fil (Figure 12.47.i). Its static capacitance model is shown in Figure 12.57.k. In this circuit we can now form SC Stub+TL+SC Stub triplets for conversion into coupled line form. This is done by placing the cursor under transmission lines and applying the command Replace By PCL / Interdigital / scscINTD / Symmetric (Figure 12.57.m-n). After converting a triplet into coupled line form, we note down the even and odd mode impedances of that coupled line and then apply Undo command to recover the original circuit, for the formation of the next coupled line section. This is necessary because the adjacent coupled line pairs share one SC stub.

- Effects of losses can be inspected by first forming the lumped LC resonator form as described in Figure 12.57.q-r by applying the command Replace By Lumped / All Lumped LC on the SC Stub+Cap resonators of Capint5.fil. Then the lumped LC resonators are converted into their lossy counterparts by placing the cursor under any LC resonator and applying the command Parasitics / All Lossy Resonators: f0=1000 MHz, Q=... ( Figure 12.57.s). Here, Q of the resonator is assigned by the designer. Figure 12.47.t shows some responses for Q=200 and Q=500. Since we have two less resonators compared to the filter with redundant unit elements, we have slightly less passband insertion loss.

Figure 12.57. Miniature, capacitively loaded, tapped interdigital filter.

Figure 12.57. Miniature, capacitively loaded, tapped interdigital filter (cont’d).

Figure 12.57. Miniature, capacitively loaded, tapped interdigital filter (cont’d).

Impedance scaling by Capacitive L-sections:

Impedance level of the miniature capacitively loaded interdigital filters may also be scaled by using capacitive L-sections at input and output, as described in Figure 12.58. This approach is not possible for the long electrical length cases, for reasons that will be explained below.

- The impedance scaled circuit with inverters  (Capint15.fil) is loaded (Figure 12.58.a).

- With cursor under the inverters, the command Inverters / Replace Ideal Inverter By / End Inverter to / Capacitive Pi-or-L / L-Section: f0=1000 MHz is applied (Figure 12.58.b). The ideal inverters will be replaced by capacitive L-sections with negative shunt capacitors. In order that this approach work, the negative capacitors must be smaller than the adjacent positive capacitors so that when they are combined we get net positive capacitors, as shown in Figure 12.58.c. The shunt capacitors of the filter are set by the length of stubs. If a long length (low fq) is selected, then these capacitors will be small and may not be sufficient to absorb the negative capacitors of the inverters. Therefore this approach has a limitation.

- This approximation degrades the return loss to some extent, as seen in the figure. This degradation can be remedied by tuning a few elements (Figure 12.58.d).

- If tuning destroys equality of the shunt SC stubs, then we can apply the command Kuroda / All Equal Stubs / Include End Stubs command to recover the equality (Figure 12.58.e).

- Figure 12.58.f shows a pictorial description of the structure.

Exercise: The inverters at source and load ends of the above example may also be replace by other approximate equivalent circuit sections, like Inductive-L section, SC Stub L-section, OC Stub L-section, Quarter wave transmission line, etc. Inspect their feasibilities.

Figure 12.58. Impedance scaling by capacitive L-sections.

B) NARROWBAND CAPACITIVELY LOADED INTERDIGITAL FILTER DESIGN BY PROTOTYPE WITH INVERTERS APPROACH

Prototype with Inverters approach offers some more flexibility for designing capacitively loaded interdigital filters. Let’s see them on an example.

EXAMPLE 12. 23. Capacitively loaded interdigital filter design by Prototype with Inverters approach

Filter specifications are:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz

In order to clarify the link between distributed element highpass filters and distributed element bandpass filters, we will design the filter by using two different quarter wave frequencies. One will be just twice the center frequency, fq1=2f0=2000 MHz, and the second is slightly greater than this frequency, for example fq2=2010 MHz. The first one will coincide with the distributed element highpass filter without loading capacitors and the second one will produce small capacitors which can approximate the fringing capacitors.

a) Design with fq1=2000 MHz:

- The command set is shown in Figure 12.59. The commands Design / By Prototype with Inverters are invoked. The topology with Shunt Capacitors and Inverters is selected. Prototype Degree is typed in as 7 and passband ripple is typed in as 0.01 dB. The lowpass prototype circuit will appear on the screen automatically. This prototype will be used to design the desired bandpass filter.

- We invoke the item Filter Type and select the options Distributed / Bandpass.

- Then Passband and Terminations item is invoked and fp1=975 MHz, fp2=1025 MHz and fq=2000 MHz is typed in. Note that fq=2000 MHz is just twice the passband center f0=1000 MHz. Due to the periodicity of response with frequency, such a specification can lead to the same distributed element highpass filter with fp=975 MHz and fq=1000 MHz, as will be seen.

- Find Circuit command gives the circuit shown in Figure 12.59.a. The shunt elements are parallel combinations of SC and OC stubs of the same length.

- The command Inverters / All Equal Left-Right equates impedances of the shunt elements (Figure 12.59.b). Note that impedances of SC and OC stubs are equal.. This is a property arising under the specification fq=2f0. Any difference between fq and 2f0 will create differing impedances for OC and SC stub impedances.

- With cursor under the SC+OC stub elements, application of the command Half Angle converts these elements into SC stubs of doubled length (Figure 12.59.c) {Note: Parameters shown in the figure are rounded}. That is, if the impedances and lengths of OC and SC stubs of a SC+OC stub element are equal then this element is equivalent to a single SC stub with doubled length. Note that this structure is exactly the same as the circuit obtained when the filter is designed as a distributed element highpass filter without capacitive loadings.

- Next, with cursor under the inverters we apply the command Inverters / Replace Ideal Inverter By / SSsh+TL+SSsh / Specify fq: f0=1000 MHz, fq=1000 MHz. To get the equivalent circuit sections formed by a TL surrounded by two negative shunt SC stubs (SSsh). Note that in this command we have typed in fq=1000 MHz instead of the actual fq=2000 MHz. This is done to make the length of TL and negative SC stubs to be the same as the doubled length of the positive SC stubs (Figure 12.59.d). Simplify Circuit command combines the adjacent positive and negative SC stubs giving the circuit of Figure 12.59.e.

- Kuroda / All Equal Stubs / Include End Stubs command equates the shunt SC stubs (Figure 12.59.f). Now the interdigital filter circuit without capacitive loadings is ready for impedance scaling using the usual methods, like redundant unit element insertion or tapping. For example let’s scale the circuit for tapping purposes. This is done by applying the command Inverters / Scale Cct By Inserting Inverters / Specify SC stub: Z=80 ohms. We get the scaled up circuit of Figure 12.59.g. The rest is the same as the tapped interdigital filters of the previous example.

- It should be noted that by this approach we get the exactly the same circuit obtained by the Distributed element highpass filter approach.  We got this result because we had selected fq of the bandpass filter as twice the passband center 2f0 (or twice the fq of the corresponding highpass filter). If fq of the bandpass filter deviates from 2f0, then we will get capacitive loadings, as will be seen in the following new design.

b) Design with fq=2010 MHz:

- In order to see the effects of selecting fq different from 2f0, we re-start the design as shown in Figure 12.59.h. Here we type in fq=2010 MHz, slightly larger than 2f0=2000 MHz. We get the circuit shown in Figure 12.59.i.

- Applying Inverters / All Equal Left-Right command we get the circuit of Figure 12.59.j. We save this circuit as Int80.fil to use it in later stages also. Note that now, contrary to the case with fq=2f0=2000 MHz, the SC stub impedances are different from OC stub impedances. The OC stub impedances are slightly lower than the SC stub impedances. This difference reflects itself as a lumped capacitor after a transformation which is placed under the command Half Angle as a special case. When this command is applied the SC+OC stub element is replaced by its SC Stub in parallel with a lumped capacitor (Figure 12.59.k). User can set the length of the SC stub within the limits shown below the screen. This ability is extremely useful for both setting the lengths of stubs and setting the associated capacitance. Two examples are prepared. In the first one the maximum possible length is selected to see how much capacitance results and the second one is to see the effects of selecting shorter length for stubs.

- Figure 12.59.k shows lengths Lng=74.578967 mm (maximum allowed) and capacitors of about 0.5 pF. Next, we replace the inverters by their proper equivalents by applying the command Inverters / Replace Ideal Inverter By / SSsh+TL+SSsh / Specify Length: Lng=74.578967 mm (Figure 12.59.m). That is we assign the same length as the SC stubs obtained in the previous step. This is necessary for combining the negative and positive SC stubs and we also need to have the same length for both TL’s and SC stubs.

- Simplify Circuit command combines the adjacent SC stubs giving the circuit of Figure 12.59.n.

- Kuroda / All Equal Stubs / Include End Stubs command equates impedances of all SC stubs (Figure 12.59.p).

- Scaling up the circuit to get 80 ohms SC stub impedances we get the circuit of Figure 12.59.q. Note that we have obtained actual capacitors of about 0.02 pF.

- Clearly we can repeat the design by specifying either new lengths in Half Angle command or new quarter wave frequency fq until we get the capacitor values close to the actual fringing capacitances. If this aim is achieved then there would be no need to place actual lumped capacitors. The fringe capacitors would act as contributing elements of the filter. Figure 12.59.r shows that there is no degradation in return loss.

- Just as an exercise, we make a new design by selecting a shorter length in Half Angle command. We load the filter Int80.fil and with cursor under the SC+OC stub element we apply the command Half Angle : Lng=70 mm. This selection increases the capacitances compared to the previous case (Figure 12.59.t).

- Then the inverters are replaced by their SSsh+TL+SSsh equivalents by applying the command Inverters / Replace Ideal Inverter By / SSsh+TL+SSsh / Specify Length: Lng=70 mm (Figure 12.59.u).

- Simplify Circuit command combines the adjacent SC stubs (Figure 12.59.v).

- Kuroda / All Equal Stubs / Include End Stubs command equates all the SC stub impedances (Figure 12.59.x).

- Finally, the command Inverters / Scale Circuit By Inserting Inverters / Specify SC stub: Z=80 ohms scales the SC stub impedances to the desired 80 ohms level (Figure 12.59.y). Note that the capacitances are now about 0.22 pF, much larger than the previous case.

- Figure 12.59.z shows the return loss. A very slight degradation started to show up indicating that this approach will have some limitations as the bandwidth of the filter gets wider. The main limitation comes from the approximation of inverters by SSsh+TL+SSsh equivalents. Nevertheless, the Design By Prototype with Inverters approach is suitable for narrow-medium bandwidth filters, and it is superior to the synthesis approach in cases when only very small capacitors are needed, like trying to account for fringing capacitors. Synthesis approach has an inherent lower limit for these capacitors due to the constarint of having fq>fp2 in distributed element bandpass filters.

- Impedance scaling by tapping or by inserting redundant unit elements can be done as in the previous examples. Similarly, External Q, Coupling Coefficients, even-odd mode impedances, etc. can be displayed as in the previous examples.

Figure 12.59. Capacitively loaded interdigital filter design by Prototype with Inverters approach.

Figure 12.59. Capacitively loaded interdigital filter by Prototype with Inverters approach (cont’d).

Figure 12.59. Capacitively loaded interdigital filter by Prototype with Inverters approach (cont’d).

Figure 12.59. Capacitively loaded interdigital filter by Prototype with Inverters approach (cont’d).

12. 16.  NARROWBAND COMBLINE FILTERS

As in all narrowband filters, narrowband combline filter element values come out to be too low for realization. Impedance scaling techniques are the same as in capacitively loaded interdigital filters, as will be seen in the following example.

EXAMPLE 12. 24. A Q=450  narrowband combline filter design by synthesis

Let’s consider a five resonator filter with following specifications:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz

Resonator impedances will be about 70 ohms. Electrical length is desired to be =45 degrees. The quarter wavelength frequency corresponding to this electrical length is calculated from the formula

(f0)=(/2)(f0/fq)

Using f0=1000 MHz and =45 degrees we get fq=2000 MHz.

Five resonators of the combline filter lead to a degree 10 and produce one transmission zero at zero (Nzero=1) and nine transmission zeros at infinity (Ninf=9).

- Design command set using Synthesis approach is shown in Figure 12.60. The resulting circuit is shown in Figure 12.60.a.

- The single transmission zero at zero appears as a shunt SC stub in the middle of the circuit. It can be distributed into the circuit to place a shunt SC stub in parallel with each shunt OC stub by applying the command Pi-Tee-L / All Equal Shunt / SC Stubs. Thus, we get all shunt resonators in the form of paralleled OC+SC Stub elements with equal SC stub impedances at 5.62 ohms level (Figure 12.60.b). The resulting unity transformer at load end is deleted.

- With cursor under any one of the OC+SC Stub resonators, the command Replace By Lumped / All SC Stub+Cap: f1=975, f2=1025 MHz is applied to convert the OC stubs into lumped capacitors. Thus, all the shunt resonators are in the form of SC stubs in parallel with lumped capacitors (Figure 12.60.c). Specifications of f1=fp1 and f2=fp2 helps keep the passband edges unchanged after thse approximations. Since the bandwidth is narrow, the distortion in return loss level is negligible.

- This transformation destroys equality of the shunt SC stubs. This property can be recovered by applying the command Pi-Tee-L / All Equal Shunt / SC Stubs command again. The shunt SC stubs are now equated at 4.53 ohms level (Figure 12.60.d). The circuit is saved as Comb11.fil.

- The synthesis of combline filter is finished. However it is clear that realization of resonator impedances of 4.53 ohms is impossible. Therefore the next stage will be scaling up of that impedance level which can be done using several different approaches, as will be described next. However before passing to these techniques, it is useful to record the External Q, Coupling Coefficients and Nodal Resonance Frequencies from this circuit. This is because Filpro can’t display these parameters from the scaled up circuits. These parameters will not change much by the transformations that will be applied in impedance scaling procedures. Pressing the key combination Ctrl+F6 and selecting the option Cascaded and typing in the frequencies f0=1000 MHz, f1=975 MHz and f2=1025 MHz, we get the list of External Q and Coupling Coefficients. With cursor under the series SC stubs, pressing the keys Alt+F6 we get the coupling coefficient between the two resonators coupled by the series SC stub pointed by the cursor, together with the nodal resonance frequencies of the resonators. A nodal resonance frequency at a node is the resonance frequency that is calculated from the parallel combination of all reactances connected to that node. It should be noted that these frequencies at all nodes are almost equal, contrary to the wideband combline filters.

Figure 12.60. Design of a narrowband combline filter.

Impedance Scaling by Capacitive L-sections

This technique is described in Figure 12.61, targeting to scale up the resonator impedances (Shunt SC stub impedances) to 70 ohms.

- The circuit of Comb11.fil is loaded (Figure 9.24.a). The command Inverters / Scale Circuit By Inserting Inverters / Specify SC Stub / Zsc=70 ohms is applied. This command inserts ideal inverters of 196.51 ohms at both source and load ends to scale up the shunt SC stub impedances to 70 ohms (Figure 12.61.b). Next, with cursor under these inverters the command Inverters / Replace Ideal Inverter By / End Inverter to / Capacitive Pi-or-L / L-Section: f0=1000 MHz is applied. This command replaces the ideal inverters with capacitive L-sections having negative shunt legs (Figure 12.61.c). Simplify Circuit command combines the negative shunt capacitors with the adjacent positive capacitors of the resonators (Figure 12.61.d). Degradation in return loss by this approximation can be corrected by tuning the shunt capacitors of the circuit (Figure 12.61.e). Thus, the design is completed. It is saved as Comb12.fil.

- External Q and Coupling Coefficients are displayed by pressing the keys Ctrl+F6 and selecting the option Direct Coupled. External Q is decreased slightly while the Coupling Coefficients are the same as the original filter, Comb11.fil. Individual Coupling Coefficients and Nodal Resonance Frequencies are displayed by placing the cursor under the coupling elements (series SC stubs) and pressing the keys Alt+F6. It is seen that the resonance frequencies of nodes-2 and 6 are changed appreciably because of the series input-output coupling capacitors.

- Resonance frequencies and Coupling Coefficients can also be determined graphically from the circuit using Dishal’s approach [1]. In Figures 12.61.f-to-i, the resonance frequency at node-2 is determined. The series capacitor on the left and the series SC stub on the right of the resonator at node-2 are inserted in shunt form. The series capacitors connecting to source and load are changed to very small values, like 0.0005 pF and all the other elements are deleted leaving only the resonator and source-load coupling capacitors (Figure 12.61.i). Insertion loss plot displays the resonance frequency at this node as 993 MHz which is the same as the one displayed by Alt+F6. The other resonance frequencies can be found in the same way.

- In Figures 12.61.j-k, the determination of the coupling coefficient between node-2 and node-3 is described. The series capacitor on the left of node-2 is inserted in shunt, in parallel with the resonator of node-2 and the series SC stub on the right of node-3 is inserted in shunt, in parallel with the resonator of node-3. The source and load coupling capacitors are changed to 0.0005 pF and all the other elements are deleted (Figure 12.61.k). Insertion loss plot displays two peaks, with frequencies f1=971.5 MHz and f2=1022 MHz. The difference is f=f2-f1=50.5 MHz. The coupling coefficient k23 is calculated as k23=f/f0=0.0505 where f0=1000 MHz is the passband center. This value is close to the one displayed by Ctrl+F6 or Alt+F6. The other coupling coefficients can be found in the same way. If the effects of finite Q of the resonators need be checked, then first, the shunt SC Stub+Cap resonators are converted into lumped LC resonator form by applying the command Replace By Lumped / All Lumped LC (Figure 12.61.m-n). Then with cursor under any shunt resonator the command Parasitics / All Lossy Resonator: f0=1000 MHz, Q=... .is applied to convert the LC resonators into RLC resonators where R is the resistance representing the loss due to finite Q. Figure 12.61.p shows the circuit and the change in insertion loss for Q=100.

- If the even-odd mode impedances of the coupled lines are needed then we first split the SC Stub+Cap resonators into SC Stub and capacitor components by placing the cursor under any SC stub+Cap element and applying the command Split / All Two Element Resonators (Figure 12.61.q-r). Then Exchange command is used to form a Pi-section of SC stubs which are equivalent to comb type parallel coupled line. Then, with cursor under the series SC stub element of these Pi-section the command Replace By PCL / Combline/  scscCOMB / Symmetric is applied to get the coupled line forms. After noting down the even and odd mode impedances, Undo command is used to get the original form and the next Pi-section of SC stubs is formed using Exchange command again for conversion into coupled line form. Figures 12.61.r-s describe this procedure on the two Pi-sections which are sufficient for description of the combline filter due to symmetry.

- Figure 12.61.t shows a pictorial description of the filter together with the even-odd mode impedances of the adjacent pairs of coupled lines. In the actual implementation the input and output coupling capacitors can be realized by probes which are coupled the first and last resonators capacitively, in various forms (nail head, loop, L-shaped probes, etc).

Figure 12.61. Impedance scaling by capacitive L-sections.

Figure 12.61. Impedance scaling by capacitive L-sections (cont’d).

Figure 12.61. Impedance scaling by capacitive L-sections (cont’d).

Impedance Scaling by Tapping:

Impedance scaling can also be implemented by tapping the first and last resonators, as described in Figure 12.62.

- The circuit of Comb11.fil is loaded and the command Inverters / Scale Circuit By Inserting Inverters / Specify SC Stub: Z=70 ohms is applied to get the circuit with 70 ohms shunt SC stubs (Figure 12.62.a-b).

- With cursor under the inverters, the command Tapped PCL: Z0=70 ohms, f0=975.1, fq=2000 MHz is applied. This command deletes the inverters and the shunt SC stubs adjacent to the inverters and inserts a shunt SC stub of length LT less than the commensurate length L of the stubs, followed by a transmission line of length L-LT (Figure 12.62.c).

- The circuit section formed by the shunt SC stub of length LT and the transmission line of length L-LT simulates a resonator of length L tapped at a distance LT to the SC end. In this command the frequency f0=975.1 is actually the nodal resonance frequency of the first resonator. It is calculated by Filpro. The new and old circuit sections are equated at this frequency. User may change this frequency to see its effect on response. The impedance Z0 is the impedance of the new (tapped) resonator. Although it can be specified by the user usually it is taken to be the same as the original resonator impedance (the displayed value).

- It should be noted that this transformation changes the capacitance of the tapped resonator also.

- This transformation degrades return loss response as shown in the figure. Degradation can be corrected by tuning a few elements, giving the circuit of Figure 12.62.d. It is saved as Comb13.fil.

- Figure 12.62.e shows a pictorial description of the tapped structure. The even and odd mode impedances of the coupled lines can be found as described in the previous approach. Also in the final schematics of the circuit we have neglected the effects of coupling between the first and second resonators on the tapping position. Further, the Tee-junction discontinuity will also effect the position of tapping point and also the capacitances of the first and last resonators.

- External Q, Coupling Coefficients and Nodal Resonance Frequencies of the final form of the filter can’t be displayed by using the Ctrl+F6 and Alt+F6 commands of Filpro. These parameters should be displayed before conversion into the tapped form, or we can use the approach of Dishal [1], as described in Figures 12.62.f-to-i. Figures 12.62.f-g describe determination of the resonance frequency of the first resonator (node-2). We insert a very small series capacitors at source and load ends. The series SC stub on the right of node-2 is inserted in shunt form, to the right of the capacitor and all the other elements are deleted giving the circuit of the first resonator(of node-2) coupled to source and load by small probe capacitors (Figure 12.62.g). Insertion loss plot displays the resonance frequency as 994.5 MHz, a little higher than that of the original (untapped) circuit. The other resonance frequencies can be found in the same way.

- Figures 12.62.h-i describe determination of the coupling coefficient k23. Here, we again insert the probe capacitors at the source and load ends. We insert the series SC stub on the right of node-3 in shunt, in parallel with the resonator at node-3 and delete all the other elements on the right of node-3, leaving only the two resonators and the  coupling capacitors (Figure 12.62.i). Insertion loss plot displays two peaks, at f1=972 MHz and f2=1023 MHz. Thus, coupling coefficient is found as k23=(F2-f1)/f0=51/1000=0.051. This value is close to that found in the original circuit. The other coupling coefficients can be displayed in the same way.

Figure 12.62. Impedance scaling by tapping.

Figure 12.62. Impedance scaling by tapping.

Impedance scaling by interdigital type SC transformer rods (Redundant Unit Element approach:

Impedance scaling can also be done by inserting two redundant unit elements leading to increasing the number of resonators by two, as described in Figure 12.63:

- The circuit of Comb11.fil is loaded (Figure 12.63.a).

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=55 ohms is applied to scale up impedances (Figure 12.63.b). Here, K=55 is a trial value which will be iterated until the desired resonator impedance level is obtained with all resonators having the same impedance.

- Next, the ideal inverters are replaced by transmission lines which are quarter wavelength long at fq=2000 MHz, by placing the cursor under these inverters and applying the command Replace Ideal Inverter By / Quarter Wave TL: f0=2000 MHz (Figure 12.63.c). These transmission lines have the same length as the other stubs of the circuit, and named as Redundant Unit Elements.

- This replacement degrades return loss response, as shown in the figure. This degradation can be corrected by tuning a few elements of the circuit (Figure 12.63.d). If we had selected the inverter impedance as K=50 ohms, then we would not have any degradation. The case with K=50 ohms is equivalent to the insertion of redundant unit element which corresponds to the classical approach for impedance scaling in combline or interdigital line filters. However in that case we wouldn’t be able to get the desired resonator impedance level, as will be seen in the following stages.

- The transmission lines at source and load ends can now be used to generate transformers for scaling impedance level by using Kuroda transformations. With cursor under the transmission line at source end, apply command Kuroda / Distribute Stub. This command transfers part of the SC stub on the right of the transmission line to left side while also producing a transformer. The SC stubs on the two sides of the transmission line are equal at 83.8 ohms. The resulting transformer is shifted to the load end, scaling up the impedance level.

- The same command is repeated on the transmission line at load end producing a transformer which cancels the previous transformer (Figure 12.63.e).

- These command raised impedance level of the circuit. However impedances of the SC stubs are not equal. The outer two SC stubs are at about 83.8 ohms while the inner ones are at about 64 ohms. Impedances of the all SC stubs in between the two transmission lines can be equated by placing the cursor under s series SC stub and applying the command Pi-Tee-L / All Equal Shunt / SC Stubs These stubs are equated at 82.9 ohms level while the outer stubs are still 83.8 ohms (Figure 12.63.f).

- Next, with cursor under the transmission lines, the command Kuroda / Equate Left-Right is applied. This command equates the two stubs on the left and right of the transmission lines at 83.6 ohms while also producing a transformer. Simplify Circuit command collects the transformers at load end and combines them to yield unity transformer (Figure 12.63.g).

- These commands destroys equality of the inner stubs again. In order to recover equality of the SC stub impedances we repeat the command Pi-Tee-L / All Equal Shunt / SC Stub again. Now the inner stubs are also equated at 83.6 ohms (Figure 12.63.h). Usually all the SC stubs can be equated after a few such iterative cycles.

- The design is completed with all shunt SC stubs having 83.6 ohms impedances. This impedance level is set by the inverter constant K=55. The desired impedance level can be obtained by repeating the design with a new inverter constant K. A lower (higher) impedance level will be obtained selecting a lower (higher) K.

- The classical Redundant Unit Element approach gives only a single solution corresponding to K=50 ohms. For such cases the designer has no control on the impedance of resonators. Therefore this new approach is more general. We can equate the resonator impedances at any desired level by selecting a proper inverter constant K.

- The SC ends of the impedance transforming rods are on the opposite side of the SC ends of the inner resonators. Therefore these rods form interdigital type coupled lines with the adjacent resonator rods.

Figure 12.63. Impedance scaling with interdigital type SC rods.

Figure 12.63. Impedance scaling with interdigital type SC rods (cont’d).

Impedance scaling by comb type SC transformer rods:

In the previous approach the SC ends of the redundant resonators are on the same side as the capacitors. It is also possible to reverse the transformer rods so as to get all the SC ends on the same side as described in Figure 12.64:

- The circuit of Comb11.fil is loaded (Figure 12.64.a).

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=70 ohms is applied to scale up impedances (Figure 12.64.b). Here, K=70 is a trial value which will be iterated until the desired resonator impedance level is obtained with all resonators having the same impedance.

- Next, the ideal inverters are replaced by an L-section of SC stubs by placing the cursor under these inverters and applying the command Replace Ideal Inverter By / End Inverter to / SC Stub Pi-or-L / L-Section: f0=1010 MHz, fq=2000 MHz (Figure 12.64.c). Here the frequency f0=1010 MHz is found by trials until least degradation is observed on return loss. We could as well select it as the passband center, f0=1000 MHz.

- The shunt legs of L-sections have negative impedance. They are absorbed into the adjacent positive shunt SC stubs by applying Simplify Circuit command (Figure 12.64.d).

- This replacement degrades return loss response, as shown in the figure. This degradation can be corrected by tuning a few elements of the circuit (Figure 12.64.e).

- With cursor under any SC Stub+Capacitor element the command Split / All Two Element Resonators is applied to split these resonators into SC stub and capacitor elements (Figure 12.64.f).

- Exchange command is used to form an L-section of SC stubs at the two ends of the circuit (Figure 12.64.g). Then, with cursor under the shunt legs of these L-sections, the commands Pi-Tee-L / LLeft-to-Sym Pi and Pi-Tee-L / LRight-to-Sym Pi is applied to convert them into symmetric Pi-sections of SC stubs as shown in Figure 12.64.h. The resulting transformers have turns ratios which are inverse of each other and are eliminated by applying Simplify Circuit command (Figure 12.64.i).

- Thus, we now have two extra shunt SC stubs, one at each end of the circuit. So, the number of rods is increased by two. In the next stage we will equate the impedances of these resonator rods by applying the command Pi-Tee-L / All Equal Shunt / SC stubs. This command equates the SC stubs of the SC Stub+Cap type resonators of the circuit, thus excludes the SC stubs at the source and load ends. In order to include these stubs also, we convert them into SC Stub+Cap type resonators with zero capacitance. This is possible by deleting the SC stubs and inserting a SC stub+Cap element (named as pCSsh in Filpro) having the same impedance as the deleted SC stubs and zero capacitance (Figure 12.64.j). Then with cursor under any series SC stub, the command Pi-Tee-L / All Equal Shunt / SC stubs is applied leading to the circuit of Figure 12.64.k, with all shunt SC stubs having 82 ohms impedance.

- The design is completed. If a different impedance is desired for shunt SC stubs, then the design should be repeated with a new impedance scaling inverter constant K.

- If the even and odd mode impedances of adjacent coupled line pairs are desired, then they can be displayed as follows:

- All the SC stub+Cap elements are split into their SC stub and capacitor components by applying the command Split / All Two Element Resonators (Figure 12.64.k-m).

- Using Exchange command Pi-sections of SC stubs are formed. Then with cursor under the series SC stub of these Pi-sections, the command Replace By PCL / Combline / scscCOMB / Symmetric is applied to convert these Pi-sections into SC comb type parallel coupled lines, one by one, as described in Figures 12.64.n-v. We note down the even and odd mode impedances of each coupled line section.

- Figure 12.64.x shows a pictorial description of the structure, together with the even-odd mode impedances. Note that all the SC ends are on the same side. The even-odd mode impedances of the previous approach (with interdigital type SC transformer rods) are also shown in the same figure for comparison. It is seen that the transformer rods of comb type are more tightly coupled to the adjacent resonators compared to the interdigital case. For this reason the interdigital approach may be easier to construct.

Figure 12.64. Impedance scaling with comb type SC transformer rods.

Figure 12.64. Impedance scaling with comb type SC transformer rods (cont’d).

Figure 12.64. Impedance scaling with comb type SC transformer rods (cont’d).

Impedance scaling by comb type OC transformer rods:

It is also possible to use OC impedance transforming rods, as described in Figure 12.65:

- The circuit of Comb10.fil is loaded (Figure 12.65.a). Note that the shunt resonators of this circuit are formed by paralleled OC and SC stubs.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=70 ohms is applied to scale up impedances (Figure 12.65.b). Here, K=70 is a trial value which will be iterated until the desired resonator impedance level is obtained with all resonators having the same impedance.

- Next, the ideal inverters are replaced by an L-section of OC stubs by placing the cursor under these inverters and applying the command Replace Ideal Inverter By / End Inverter to / OC Stub Pi-or-L / L-Section: f0=990 MHz, fq=2000 MHz (Figure 12.65.c). The frequency f0=990 MHz is selected to get minimal degradation on return loss. We could as well leave it at the default value of f0=1000 MHz. The shunt legs of L-sections have negative impedance. This replacement degrades return loss response, as shown in the figure. This degradation can be corrected by tuning a few elements of the circuit (Figure 12.65.d).

- The negative shunt OC stubs are absorbed into the adjacent parallel positive elements by applying Combine command (Figure 12.65.e).

- With cursor under the shunt OC+SC stub resonators, the command Split / All Two Element Resonators is applied to split them into their OC and SC stub components (Figure 12.65.f).

- Exchange command is used to form L-sections of OC stubs, as shown in Figure 12.65.g.

- With cursor under the shunt legs of L-sections, the commands Pi-Tee-L / LLeft-to-Sym Pi and Pi-Tee-L / LRight-to-Sym Pi are applied to convert the L-sections into symmetric Pi-sections of OC stubs (Figure 12.65.h).

- The resulting transformers are shifted to rightmost position, combined to form unity turns ratio and deleted there, giving the circuit of Figure 12.65.i. This circuit is saved as Comb20.fil.

- Thus, we now have two extra shunt OC stubs, one at each end of the circuit, named as transformer rods. So, the number of rods is increased by two. In the next stage we will equate impedances of the OC stubs (transformer rods) to the impedances of the inner shunt SC stubs by an iterative algorithm. Iterations are eased a lot by using the Macro Recording feature of Filpro, as follows:

- Invoke the command Macro / Start Recording.

- With cursor under the middle element of the Pi-section of OC stubs at source end (series OC stub), the command Pi-Tee-L / Pi-to-Pi-Tee-L / Pi-to-Pi / Specify Left: Z=76.228 ohms. In this command we can specify impedance of the left shunt leg of the Pi-section of OC stubs, as desired. We have to type in a trial value for this impedance and iterate it by inspecting the resulting impedances of rods (OC stubs at source and load ends and the inner shunt SC stub circuit) that will come out at the end of this procedure. The value Z=76.228 ohms is reached after about 10 iterative trials through the Macro command.

- This command changes impedances of the OC stubs of the Pi-section, with the left leg having the specified impedance, while also producing a transformer (Figure 12.65.i-j).

- Simplify Circuit command shifts the transformer rightmost while also combining the adjacent SC and OC stubs to form SC+OC Stub resonators (Figure 12.65.k). Symmetrize command is used to avoid repetition of this process on the load side (Figure 12.65.m).

- With cursor under any one of the SC+OC Stub resonators, the command Replace By Lumped / All SC Stub+Cap is applied to convert the OC stub components of these resonators into lumped capacitors (Figure 12.65.n).

- Then, with cursor under any series SC stub, the command Pi-Tee-L / All Equal Shunt / SC stubs is applied to equate all the shunt SC stub impedances (Figure 12.65.p).

- The command Macro / Stop Recording is applied. Then Macro / Save Macro is applied if this procedure will be used later.

- The shunt SC stub impedances which are equated at the last stage are compared with the impedance of the OC stubs at source end (the specified OC stub impedance). If they are different, then we repeat the procedure by first loading the circuit of Comb20.fil and invoking the command Macro / Run Macro. This command runs the procedure described in the steps of Figure 12.65.i-to-p, stopping whenever a user specified input need be typed in. We run the Macro by specifying a different impedance for the shunt OC stub at the source and at each iteration and stop iterations when the inner SC stub impedances become sufficiently close to the shunt OC stub impedance at source (and load) end. In this example, specification of Z=76.228 ohms  gave equal impedances for all shunt OC and SC stubs, as shown in Figure 12.65.p, at 76.228 ohms level. This circuit is saved as Comb21.fil. If a different impedance level is desired then the whole procedure need be repeated with a different impedance scaling inverter constant K.

- If the even and odd mode impedances of adjacent coupled line pairs are desired, then they can be displayed as follows:

- All the SC stub+Cap elements are split into their SC stub and capacitor components by applying the command Split / All Two Element Resonators (Figure 12.65.p-q). The resulting circuit is saved as Comb22.fil.

- In order to display first the even-odd mode impedances of the inner coupled lines, Pi-sections of SC stubs are formed by using Exchange command as shown in Figure 12.65.r. Then with cursor under the series SC stub of these Pi-sections, the command Replace By PCL / Combline / scscCOMB / Symmetric is applied to convert these Pi-sections into SC comb type parallel coupled lines (Figure 12.65.s). We note down the even and odd mode impedances of each coupled line section.

- Filpro can’t display the even-odd mode impedances of the end sections directly, because one leg of these Pi-sections is an OC stub while the other leg is a SC stub. That is, the even-odd mode impedances of an OC stub and a SC stub coupled by a series OC stub is not programmed into Filpro. However, fortunately, these even-odd mode impedances are identical to the even-odd mode impedances of two shunt OC stubs coupled by the same series OC stub. Therefore this problem can be solved through brute force by forming a Pi-section of OC stubs as described in Figure 12.65.t-u-v. We Copy the shunt SC stub at source end and Paste it to the right of the adjacent series OC stub, as shown in Figure 12.65.v. Thus, we have formed a Pi-section of OC stubs by brute force. Then with cursor under the series OC stub, the command Replace By PCL / Combline / ococCOMB / Symmetric is applied to convert it to a comb type parallel coupled line with OC ends (Figure 12.65.x). We note down the resulting even-odd mode impedances. Since this action destroys the circuit response, it is used just for displaying the even-odd mode impedances.

- Figure 12.65.y shows a pictorial description of the structure, together with the even-odd mode impedances. The even-odd mode impedances of the previous approach (with comb type SC transformer rods) are also shown in the same figure for comparison. It is seen that the OC transformer rods have slightly lower impedance level.

Figure 12.65. Impedance scaling with OC transformer rods.

Figure 12.65. Impedance scaling with OC transformer rods (cont’d).

Figure 12.65. Impedance scaling with OC transformer rods (cont’d).

12. 17. NARROWBAND MIXED DIGITAL FILTERS

It is possible to synthesize filters having mixed comb and interdigital type couplings. Such structures may be used to correct the asymmetry of combline filter responses to a certain extent. Such filters also allows easy implementation of finite transmission zeros. Let’s see an example.

Example 12. 25. A five resonator mixed digital filter

Let PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, fq=2000 MHz.

As a trial, let’s take Nzero=1, NUE=2, Ninf=7.

- Synthesis command set is shown in Figure 12.66. In order to get a symmetric structure the transmission zeros are extracted in the order inf-ue-inf-inf-inf-zero-inf-inf-ue-inf, giving the circuit shown in Figure 12.66.a.

- With cursor under a series SC stub, the command Pi-Tee-L / All Equal Shunt / SC Stubs is applied to distribute the single shunt SC stub into the circuit so that each shunt OC stub in between the transmission lines will have a SC stub in parallel, with equal impedances (Figure 12.66.b).

- With cursor under the transmission lines the command Kuroda / Distribute Stub is applied to transfer part of the shunt SC stub on one side to the other side of the transmission lines (Figure 12.66.c). This action destroys equality of shunt SC stubs. Therefore we apply the command Pi-Tee-L / All Equal Shunt / SC Stubs again (Figure 12.66.d).

- With cursor under any one of the shunt OC+SC Stub elements, the command Replace By Lumped / All SC Stub+Cap: f1=975, f2=1025 MHz is applied to convert the OC stub components of these elements into lumped capacitors (Figure 12.66.e).

- This operation will again destroy equality of shunt SC stubs. Their equality can be recovered by first placing the cursor under one of the transmission lines and applying command Kuroda / All Equal Stubs and then placing the cursor under a series SC stub and applying the command Pi-Tee-L / All Equal Shunt / SC stubs. The shunt SC stubs are equated at 4.45 ohms level (Figure 12.66.f).

- Impedance level of the shunt SC stubs (resonator bars) can be scaled up to any realizable level by using different approaches. One common approach is Tapping of the input and output resonator bars and it is described in Figures 12.66.f-to-i. In this approach, the command Inverters / Scale Circuit By Inserting Inverters / Specify SC stub: Z=80 ohms is applied to insert inverters at source and load ends to scale up impedances of SC stubs to 80 ohms (Figure 12.66.g).

- Before converting the inverter to tapped form, it would be wise to record the Coupling Coefficients and External Q of the circuit, as these quantities are easier to display in this form of the circuit. They can be displayed by pressing the key combination Ctrl+F6 and selecting the option Direct Coupled. Then the center frequency and two frequencies below and above the center frequency are typed in for calculation of coupling coefficients (Figure 12.66.g). The individual Coupling Coefficients can also be displayed together with the resonance frequencies of the coupled resonators by placing the cursor under the coupling elements and pressing the key combination Alt+F6.

- Then with cursor under the inverter at source end the command Tapped PCL: Z=80 ohms, f0=982 MHz, fq=2000 MHz is applied (Figure 12.66.h). This command calculates the position of the tap point on the first resonator rod. It can scale up or down the first resonator impedance. However it is usual to leave it the same at 80 ohms. The resonance frequency of the original resonator is also calculated an displayed as 982 MHz. The user may adjust this frequency and impedance of the first resonator to see if a better approximation can be obtained or not. The quarter wave frequency should be left at fq=2000 MHz. After applying this command  the piece of the tapped rod between the tapping point and the SC end is displayed as a shunt SC stub of length LT, referred to as Tap Length. The remaining piece of the firs rod is treated as a transmission line of length L-LT where L is the commensurate length of the filter. This command also modifies the shunt capacitor at the tip of the first rod. The same command is repeated at the load end inverter giving the circuit shown in Figure 12.66.i. Slight degradation in return loss response can be corrected by tuning a few elements.

- Figure 12.66.j shows a pictorial description of the filter.

- Figure 12.66.k shows Delay responses of this comb+interdigital filter together with the delay response of a pue comb filter of the same degree for comparison. Response of the comb+interdigital filter is slightly more symmetric.

- Another approach for impedance scaling is to use impedance transforming rods (redundant Unit Element insertion). This is described in Figures 12.66.m-to-s.

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=52 ohms is applied to the unscaled circuit (Figure 12.66.m-n). Here, the inverter constant K=52 ohms is a trial value which need be iterated in later design cycles until the targeted 80 ohms resonator impedance is achieved.

- With cursor under the inverters, the command Inverters / Replace Ideal Inverter By / Quarter Wave TL: f0=fq=2000 MHz is applied. This command replaces the inverters by transmission line pieces which are of the same length as the other lines and stubs of the filters (Figure 12.66.p).

- The degradation arising from this approximation can be corrected by tuning impedances of a few elements (Figure 12.66.q).

- Then with cursor under the transmission line at source and load ends, the command Kuroda / Distribute Stub is applied to transfer part of the shunt SC stubs on one side to the other side (Figure 12.66.r).

- Then with cursor under any transmission line, the command Kuroda / All Equal Stubs is applied to equate the stubs adjacent to the transmission lines. Then with cursor under the series SC stubs, the command Pi-Tee-L / All Equal Shunt / SC Stubs is applied to equate the inner shunt SC stubs (in between the inner transmission lines).

- The last two command may be iterated a few times until all shunt SC stubs are equated (Figure 12.66.s).

- Figure 12.66.t show a pictorial description of the structure. It is seen that the number of resonator lines is increased by two. The newly added resonator lines are much more tightly coupled to the adjacent lines.  This may be problematic for some applications.

- Clearly the Tapping approach is simpler, if implementation of tapping is not problematic.

Figure 12.66. Design of a five resonator mixed digital filter.

Figure 12.66. Design of a five resonator mixed Comb+İnterdigital filter (cont’d).

Figure 12.66. Design of a five resonator mixed Comb+İnterdigital filter (cont’d).

12. 18. STEPPED DIGITAL FILTERS AND SPURIOUS PASSBANDS

All-stub combline filters, Capacitively loaded and mixed digital filters with passsband center f0 and quarter wave frequency fq have the nearest three spurious passbands centered around 2fq-f0, 2fq+f0 and 4fq-f0. When the shunt OC stubs are replaced by lumped capacitors, the nearest spurious passband at 2fq-f0 disappears while those at 2fq+f0 and 4fq-f0 combine to form a single spurious passband at a position somewhere between 3f0 and 2fq+f0. As fq gets closer to f0, that is when the electrical length of stubs gets closer to 90 degrees, this spurious passband gets closer to 3f0  which may overlap with the third harmonic of the signals inside the passband. This spurious passband can be moved away from 3f0 to an ineffective position by replacing the uniform resonator rods (shunt SC stubs) with stepped resonator rods (stepped shunt SC stubs). In comb type coupled lines this replacement will automatically change the coupling element between the adjacent resonator rods which appears as a uniform series SC stub into a stepped series SC stub as shown in Figure 12.67. In interdigital type coupled lines the uniform transmission line which represents the coupling will automatically become a stepped line as shown in Figure 12.67.b. Therefore in transforming a uniform shunt SC stub into a stepped shunt SC stub, we must also convert the coupling element into its stepped version. Since this procedure is also dependent on physical spacings between the adjacent resonators, it is not possible to treat them on Filpro in an exact quantitative manner at circuit theory level. Therefore just to give an idea on the effects of such transformations some circuit transformations are programmed on Filpro which are applicable only on combline type filters. Similar ideas can be extended to interdigital type couplings as well.

Figure 12.67. Stepped stub equivalents of shunt SC stubs in parallel with capacitors and series SC stubs.

In Filpro the shunt SC stub+Capacitor type resonators can be converted into stepped stubs by applying the command Step Resonators. When this command is applied if the adjacent element is a series SC stub then Filpro offers three options for the modification of the adjacent series SC stub:

- We can leave the series SC stub as it is by selecting the option No Change (in the series SC stub on the right of the resonator pointed by the cursor).

- We can leave it as a series SC stub but with a new length specified by the user by selecting the option New Length. This length can be adjusted properly to simulate the actual coupling coefficient produced by magnetic coupling through various coupling mechanisms.

- We can convert the series SC stub on the right into a stepped resonator by selecting the option Step Resonator.

The stepped shunt SC stub is characterized by the step impedances and lengths Z1-L1 and Z2-L2. In the conversion of the shunt SC stub into a stepped shunt SC stub, Filpro asks the parameters New fq and length L1. The parameter New fq is the resonance frequency of the resulting stepped shunt SC stub alone. Before conversion the shunt SC stub resonance frequency was the same as the quarter wave frequency fq of the filter. If the resonance frequency of the stepped shunt SC stub, (New fq) is set lower than the original fq, then the resulting spurious passband will shift to higher than its original frequency. If it is selected higher than the original fq then the spurious passband will shift to a lower frequency. After specifying New fq and Length L1, the parameters Z1, Z2 and L2 are calculated by Filpro automatically. Experimentation shows that the best value for Length L1 is L/2 where L is the original commensurate length of the stubs. Other choices lead to new spurious passbands, though mostly ineffective.

Let’s see details on an example.

EXAMPLE 12. 26. Spurious Passbands and Combline Filters with stepped shunt SC stubs

Let’s design a combline filter with the following specifications:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, Nzero=1, Ninf=9 (NR=5, N=10)

As quarter wave frequency, we will assign fq =1100 MHz, 1250 MHz and 1500 MHz to get an idea on the position of the spurious passband, and then use fq=1100 MHz as design parameter.

Variation of spurious passbands:

- Design command set for the case fq=1100 MHz is shown in Figure 12.68. Figure 12.68.a shows the synthesized circuit.

- The command Pi-Tee-L / All Equal Shunt/ SC Stubs is applied to get equal shunt SC stub combline structure (Figure 12.68.b).

- The shunt OC+SC stub resonators are replaced by SC Stub+Cap equivalents using the command Replace By Lumped / All SC Stub+Cap command (Figure 12.68.c).

- Equality of shunt SC stub impedances is lost after conversion into leumped capacitor form. It is recovered by applying the command Pi-Tee-L / All Equal Shunt / SC Stubs again (Figure 12.68.d). This circuit is saved as Comb21A.fil.

- Insertion loss response of this filter shows that the first spurious passband is centered at just 3f0=3000 MHz. Thus, this filter will pass the third harmonics of the signals inside the actual passband.

- This spurious passband can be pushed away from 3f0 by selecting a higher fq. For example in Figure 12.68.e a filter with fq=1250 MHz is shown. The spurious passband of this filter has still a portion overlapping with the band about 3f0. Therefore we should select a higher fq. Figure 12.68.f shows a filter with fq=1500 MHz. Clearly, the spurious passband of this filter is sufficiently far away from 3f0.

- Thus, selection of higher fq (shorter commensurate length) solves the third harmonic problem. However reduced length is usually accompanied by reduced Q, leading to higher dissipation losses in the filter. If this is not desirable, then we can use Stepped Resonator approach described below.

Figure 12.68. Spurious passbands near 3f0 for various fq.

Figure 12.68. Spurious passbands near 3f0 for various fq.

Conversion into Stepped Resonator Forms:

In Figure 12.69.a, the filter with fq=1100 MHz is repeated. We will convert the stubs of this filter into stepped form by using the command Step Resonator. In order to get an idea on the effects we will use all options of the Step Resonator command one by one.

With cursor under the shunt SC Stub+Cap resonators, the command Step Resonator/ No Change / New fq=1040 MHz, f1=975, f2=1025 MHz / L1=34.067 mm is applied. Here, the parameter New fq=1040 MHz is found after a few trials by inspecting the shift in the spurious passband. It should be above the upper passband f2=1025 MHz. We leave Length L1 at its default value L/2=34.067 mm which is half the original commensurate length. The option No Change means we will leave the series SC stub on the right of the resonator pointed by the cursor unchanged. This command replaces the original Shunt SC Stub+Cap resonator with the equivalent Stepped Shunt SC Stub+Cap resonator. Repeating this command on all shunt resonators, we get the circuit shown in Figure 12.69.b. It is seen that the spurious passband is shifted upward on frequency axis, away from 3f0=3000 MHz.

In order to see the effect of selecting Length L1 different from L/2=34.067 mm, the conversion is repeated in Figures 12.69.c and d, with the selection of L1=40 mm. Insertion loss response shows that some new spurious responses started to show up.

Figures 12.69.e-f shows the option of conversion of the series SC stubs also into stepped stubs by applying the command Step Resonators / Step Resonator / New fq=1040 MHz, / L1=34.067 mm. This option may be more realistic, as the space between the adjacent resonators may be approximated as a stepped transmission line with a SC at its end. Figure 12.69.h shows a pictorial description.

The option Step Resonator / New Length / New FTZ=... is prepared just to see the effects of changing lengths of the series SC stubs to simulate effects of partial walls between adjacent resonator rods, for example as shown in Figure 12.69.h.

Actually these techniques need be applied on the impedance scaled versions of the filter. That is, the filter will be first impedance scaled by a proper technique, like for example Tapping, Impedance Scaling with redundant transformers, impedance scaling with capacitive L-sections, etc.

Figure 12.69. Conversion of uniform resonator stubs of a combline filter into stepped stubs.

Figure 12.69. Conversion of uniform resonator stubs of a combline filter into stepped stubs.

12. 19. NARROWBAND COMBLINE AND MIXED DIGITAL FILTERS WITH FINITE TRANSMISSION ZEROS

Circuit models of the combline filters involve inductive couplings between adjacent resonators, appearing as series arm SC stubs. The interdigital type filters have also basically inductive couplings represented by transmission lines. If these inductive coupling elements are resonated by paralleling with capacitive reactances it is possible to form finite transmission zero sections. The parallel capacitive reactance can be implemented easily in comb type coupled line sections by narrowing the gap between the ungrounded tips of adjacent lines. However implementation of capacitive reactances in interdigital coupled rods is not easy because the ungrounded tips of adjacent rods are on opposite sides. Therefore if finite transmission zeros are to be included in a capacitively loaded interdigital filter then a comb type coupled sections must also be included. That it, the filter will be of mixed digital type. When the finite transmission zero is too close to passband edges this approach fails and cross-coupling between nonadjacent resonators need be used. These techniques will be discussed in the forthcoming chapters.

Let’s see some examples.

EXAMPLE 12. 27. A six resonator mixed digital filter with a single finite transmission zero

Let PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, fq=2000 MHz with a finite transmission zero at fz=960 MHz.

Degree of a six resonator bandpass filter is N=12. Let’s take NUE=2 to use only two interdigital sections. Since two degrees will be coming from the finite transmission zero, we will have Nzero=1 and  Ninf=7.

- Synthesis command set is shown in Figure 12.70. In order to get a symmetric structure the transmission zeros are extracted in the order inf-ue-inf-inf-inf-FTZ-zero-inf-inf-ue-inf, giving the circuit shown in Figure 12.70.a.

- With cursor under a series SC stub, the command Pi-Tee-L / All Equal Shunt / SC Stubs is applied to distribute the single shunt SC stub into the circuit so that each shunt OC stub in between the transmission lines will have a SC stub in parallel, with equal impedances (Figure 12.70.b).

- With cursor under the transmission lines the command Kuroda / Distribute Stub is applied to transfer part of the shunt SC stub on one side to the other side of the transmission lines (Figure 12.70.c). This action destroys equality of shunt SC stubs. Therefore we apply the command Pi-Tee-L / All Equal Shunt / SC Stubs again (Figure 12.70.d).

- With cursor under any one of the shunt OC+SC Stub elements, the command Replace By Lumped / All SC Stub+Cap: f1=975, f2=1025 MHz is applied to convert the OC stub components of these elements into lumped capacitors (Figure 12.70.e).

- With cursor under the series OC+SC Stub element, the command Replace By Lumped / SC Stub+Cap: f1=975, f2=1025 MHz is applied to convert the OC stub component into lumped capacitors (Figure 12.70.f).

- These operations will again destroy equality of shunt SC stubs. Their equality can be recovered by first placing the cursor under one of the transmission lines and applying command Kuroda / All Equal Stubs and then placing the cursor under a series SC stub and applying the command Pi-Tee-L / All Equal Shunt / SC stubs. These two commands can be repeated until all shunt SC stubs are equated. In this example the shunt SC stubs are equated at 4.27 ohms level in two iterations (Figure 12.70.g).

- Impedance level of the shunt SC stubs (resonator bars) can be scaled up to any realizable level by using different approaches. Let’s use Tapping approach. In this approach, the command Inverters / Scale Circuit By Inserting Inverters / Specify SC stub: Z=80 ohms is applied to insert inverters at source and load ends to scale up impedances of SC stubs to 80 ohms (Figure 12.70.h).

- Before conversion into tapped form we note down the Coupling Coefficients and External Q by pressing the keys Ctrl+F6 and select the option Direct Coupled.

- Then with cursor under the inverter at source end the command Tapped PCL: Z=80 ohms, f0=983.3 MHz, fq=2000 MHz is applied. This command calculates the position of the tap point on the first resonator rod. It can scale up or down the first resonator impedance. However it is usual to leave it the same at 80 ohms. The resonance frequency of the original resonator is also calculated an displayed as 983.3 MHz. The user may adjust this frequency and impedance of the first resonator to see if a better approximation can be obtained or not. The quarter wave frequency should be left at fq=2000 MHz. After applying this command  the piece of the tapped rod between the tapping point and the SC end is displayed as a shunt SC stub of length LT, referred to as Tap Length. The remaining piece of the firs rod is treated as a transmission line of length L-LT where L is the commensurate length of the filter. This command also modifies the shunt capacitor at the tip of the first rod. The same command is repeated at the load end inverter giving the circuit shown in Figure 12.70.i. Slight degradation in return loss response (Figure 12.70.k) can be corrected by tuning a few elements.

- Figure 12.70.m shows a pictorial description of the filter. Note that the series capacitor can be implemented by placing conductor patches at the tips of the adjacent bars or by placing a dielectric piece to concentrate the odd mode electric field between the tips of the adjacent bars.

- Another approach for impedance scaling is to use impedance transforming rods (redundant Unit Element insertion), as was done in the previous example. (Exercise?).

Exercise: Repeat the design by placing the finite transmission zero at 1050 MHz, in upper stopband.

Exercise: Repeat the design by using higher numbers of resonators with different numbers of Unit Elements.

Figure 12.70. A six resonator mixed Comb+İnterdigital filter with a finite transmission zero.

Figure 12.70. A six resonator mixed Comb+İnterdigital filter with a finite transmission zero (cont’d).

EXAMPLE 12. 28. Synthesis of a direct coupled elliptic combline filter

Let PBRipple=0.05 dB,  fp1=975 MHz, fp2=1025 MHz, fq=2000 MHz, Nzero=1, Ninf=1, three FTZs at f=950 MHz, three FTZs at f=1050 MHz. Total degree is N=14 leading to seven resonators, NR=7.

Command set in Filpro for synthesis is shown in Figure 12.71.

- Standard Auto Extract command gives the circuit shown in Figure 12.71.a.

- Applying the command Pi-Tee-L / All Equal Shunt / SC Stubs, impedances of all shunt SC stubs are equated as shown in Figure 12.71.b.

- The shunt OC stubs can be converted into lumped capacitors by applying the command Replace By Lumped / All SC Stub+Cap (Figure 12.71.c).

- The series arm OC stubs are replaced by lumped capacitors by placing the cursor under each series arm OC+SC Stub elements and applying the command Replace By Lumped / SC Stub+Cap: fp1=975, fp2=1025 MHz (Figure 12.71.d).

- Equality of the shunt SC stubs will be lost after these commands. However it can be recovered by applying the command Pi-Tee-L / All Equal Shunt / SC Stubs again (Figure 12.71.e).

- As expected the shunt SC stub impedances are too low for realization. They can be scaled up to desired levels by using the techniques described in the previous narrowband examples.  For example let’s use tapping approach to scale up resonator impedances to 80 ohms.

- First the command Inverters / Scale Cct By Inserting Inverters / Specify SC Stub: Z=80 ohms is applied to insert the inverters at source and load ends (Figure 12.71.f).

- Then with cursor under the source end inverter the command Tapped PCL / Z0=80 ohms, f0=1013 MHz, fq=2000 MHz is applied. This command calculates the resonance frequency of the first resonator as f0=1013 MHz and calculates the distance LT between the tapping point and the SC end of the resonator. The piece of resonator between the tapping point and SC end is then placed into the circuit as a shunt SC stub. The remaining piece of the rod is inserted as a transmission line of length L-LT where L is the commensurate length of the filter set by fq=2000 MHz. This command modifies the adjacent capacitor of the node also. The same command is applied on the inverter at the load end giving the circuit of Figure 12.71.g.

- This topology may be realized as shown in Figure 12.71.h. The Pi-sections of SC stubs form the fingers of combs with short circuits on the same side. The shunt lumped capacitors can be formed between the OC ends of the resonator rods and ground plane by placing tuning screws through the ground plane. The difficult part is the realization of series coupling capacitors which will form the finite transmission zeros. This may need placement conductor pieces between the tips of adjacent fingers to form lumped capacitors [10], [20].

- Figure 12.71.i shows the insertion loss and return loss responses of the tapped filter. It is seen that tapping led to almost no degradation.

Figure 12.71. Design of an elliptic combline filter.

- Figure 12.71. Design of an elliptic combline filter (cont’d).

12. 20.  BRUTE FORCE FINITE TRANSMISSION ZEROS IN COMBLINE FILTERS

Finite transmission zeros (FTZ) may be incorporated into any two port filter by adding an external FTZ creating circuit section and optimizing  element values. In combline filters this can be done by introducing extra resonator rods at input and output. This approach is not an optimum one in the sense that the elements are used inefficiently for obtaining the desired response. The best approach is to use cross-coupling, as will be described in Chapters 24-to-26. However for some applications this approach may be good satisfactory. An example is given below.

EXAMPLE 12. 29. Brute force addition of FTZ sections to a combline filter

Let  PBRipple=0.01 dB,  fp1=975 MHz,   fp2=1025 MHz, fq=2000, NR=5 (Nzero=1, Ninf =9, N=10)

All resonator impedances will be equal and about 80 ohms. A finite transmission zero will be created at fz=950 MHz by inserting external finite transmission zero creating resonator bars.

- Design command set is shown in Figure 12.72, resulting in the circuit of Figure 12.72.a.

- After applying the command Pi-Tee-L / All Equal Shunt / SC stubs we get the circuit of Figure 12.72.b.

- With cursor under the shunt SC+OC Stub elements the command Replace By Lumped / All SC Stub+Cap is applied to convert them into SC stubs in parallel with lumped capacitors (Figure 12.72.c).

- The command Pi-Tee-L / All Equal Shunt / SC stubs is applied again to recover equality of shunt SC stubs (Figure 12.72.d).

- Impedance level of the shunt SC stubs need be scaled up to about 80 ohms. This can be done as follows:

- The command Inverters / Scale Circuit By Inserting Inverters / Specify Inverter: K=55 ohms is applied (Figure 12.72.e). In this command the specified inverter constant K=55 ohms is a trial value. It will be iterated unitl the shunt SC stubs get the targeted impedance, after the following steps.

- With cursor under the inverters, the command Inverters / Replace Ideal Inverter By / Quarter Wave TL: f0=2000 MHz is applied. This command replaces the ideal inverters by transmission line pieces which are quarter wavelength long at frequency f0=fq=2000 MHz (Figure 12.72.f). Actually this frequency have to be specified as f=1000 MHz, so that the lines would work as inverters at the passband center of the filter and this would lead to small degradation in response. However in that case the line length would be different from the commensurate length of the filter. In order to have the length of lines to be the same as the commensurate length of the filter this frequency must be selected as the quarter wave frequency fq of the filter.

- Both approximation of an ideal inverter by a quarter wave line and selection of the approximation frequency different from f0=1000 MHz leads to relatively large degradation in return loss. This degradation can be corrected by tuning impedances of both the transmission lines at source and load ends and the shunt SC stubs adjacent to these lines, as shown in Figure 12.72.g.

- The transmission lines at source and load ends are actually redundant because they have not an appreciable contribution to the response. They will be used to create transformers to scale up impedance level of the circuit. This is done by placing the cursor under these lines and applying the command Kuroda / Distribute Stub (Figure 12.72.h). This command transfers part of the shunt SC stub on one side of the lines to the other side of the lines while producing a transformer. The transformers created at source and load ends cancel each other giving the circuit of Figure 12.72.h where now the shunt SC stub impedances are raised a lot compared to their original values. However they are not equal. Impedances of the SC stubs in between the transmission lines can be equated by applying the command Pi-Tee-L / All Equal Shunt / SC stubs (Figure 12.72.i). This command equates these stubs at 82.9 ohms level while the outer stubs remain at 83.8 ohms.

- Next, with cursor being under the transmission lines, the command Kuroda / Equate Left-Right is applied. Then Simplify Circuit command is applied to collect the transformers at load end where they cancel each other (Figure 12.72.k). This command equates the shunt SC stubs on the two sides of transmission lines, but destroys equality of inner shunt SC stubs.

- Equality of inner stubs is recovered by applying the command Pi-Tee-L / All Equal Shunt / SC stubs again.

- The last two steps can be applied until all shunt SC stubs are equal. In this example they are equated just in one iteration, at 83.7 ohms level (Figure 12.72.m).

- Thus, design of the combline filter without finite transmission zeros is finished. It is saved as ELP10.FIL. If the shunt SC stub impedances need be made closer to 80 ohms, then we should re-start the design by specifying a new inverter constant K, for example K=54 ohms.

- A pictorial description of the filter is shown in Figure 11.n. It should be noted that the first and last resonator bars are coupled to the adjacent bars in an interdigital manner. That is, the SC ends are on opposite sides. The first and last resonator bars are usually termed as transformer lines. Impedance of the transmission line which couples  the transformer line to the adjacent line sets the degree of coupling. The lower transmission line impedance means tighter coupling. That is narrower spacing between the lines.

- The next stage is preparation of the finite transmission zero (FTZ) creating circuit section. This will be done by placing extra bars on the outer sides of the transformer lines. The FTZ sections can be prepared as follows:

- The filter ELP10.FIL is loaded (Figure 12.72.p).

- All the elements except the transmission line and the adjacent SC Stub+Cap resonator are deleted (Figure 12.72.q).

- Load resistance RL=50 is increased to a very high value (RL=9999999) to simulate an OC. Thus, the circuit becomes a lossless circuit section which can be tuned to form a parallel or series resonator.

- Impedance of the transmission line will set the degree of coupling of the FTZ creating rod to the transformer rod. Too tight coupling will load the filter too much, leading to large degradations in response. Therefore a light coupling is usually desirable. As a trial, we set this impedance as 400 ohms.

- The shunt SC Stub+Cap element forms a shunt arm parallel resonator and at its resonance frequency it will show OC. The transmission line on the left of the SC Stub+Cap resonator can convert the shunt arm parallel resonator into a shunt arm series resonator presenting a zero shunt impedance at its resonance frequency. This resonance frequency will act as the FTZ frequency. It can be adjusted to the desired FTZ frequency by tuning one or more of the parameters of the elements of the FTZ section, like capacitance or impedance or length of the SC Stub+Cap resonator or impedance and length of the transmission line. the desired FTZ frequency. In this example we will tune both impedance of the transmission line and the capacitor to set the FTZ frequencies.

- This circuit is saved as FTZ1.FIL and FTZ2.FIL. Initially these two circuits will be identical, but after setting the FTZ frequencies they will be different. So, the FTZ sections are ready for sticking to the main filter, as will be described below.

- Load ELP10.FIL (Figure 12.72.s).

- With cursor under the transmission line element on the load side, the command External 1-Port / FTZ1.fil / Shunt 1-Port is applied. This command appends the circuit of FTZ1.fil (without RS) to the left of the transmission line, as if it is a channel filter of a multiplexer. The circuit is saved as ELP11.fil (Figure 12.72.t).Response of the filter will be degraded a lot by this modification. It can be recovered to a some extent by tuning element parameters of both the FTZ1 section and the main filter.

- First we have to set the resonance frequency of FTZ1 section to 950 MHz. This is done by placing the cursor under the External 1-Port element  and applying the command Tune Multiplexer. This command is exclusively used to tune elements of External 1-Port circuits used in multiplexer circuits. Since our circuit is similar to multiplexers, we can use this command in this application also. When this command is applied the circuit of FTZ1.fil will be displayed for the selection of the element to be tuned (Figure 12.72.t). We move the cursor to point the capacitor and press enter. Then type in the increment for tuning capacitance. In order to set the FTZ1 frequency, we select the option Insertion Loss for the response to be observed. Then use up-down arrows  to tune the capacitance while observing the position of the dip (shunt arm series resonance frequency) on insertion loss response. We tune the capacitance until this frequency is set as 950 MHz.

- After setting this frequency, we press Esc key. Filpro will ask whether any other element will be tuned or not. We select the option Yes and the circuit of FTZ1.fil will be displayed again. Now we move the cursor to point the transmission line and select the impedance parameter Z of the line. This parameter is tuned while observing the return loss. Return loss will improve as the impedance of the line is increased. This is because as this impedance is increased the FTZ1 circuit will be loading the main filter less. During this tuning we occasionally inspect the Insertion Loss response also because tuning impedance will also effect the resonance frequency of the FTZ1 circuit.

- In order to recover the resonance frequency back to 950 MHz, we repeat the tuning of the capacitor.

- After tuning of the FTZ1 circuit is finished we tune the elements of the main filter while observing the return loss by using Tune Element command.

- After a few rounds, we set impedance of the transmission line as 700 ohms and re-tune the capacitor. The resonance frequency is settled at 950 MHz for C=2.429 pF (Figure 12.72.v). The last values of the tuned parameters can’t be saved by Filpro automatically. For this purpose, we note down the tuned values and load the circuit of FTZ1.fil and use Change command to type in the optimized values and then Save the circuit.

- This procedure is repeated for appending and tuning of the FTZ2 circuit, as shown in Figures 12.72.w-to-y.

- All these tuning efforts can’t recover the return loss completely. Some degradation have to be accepted because this is not a regular synthesis, but a brute force tuning. Response can be improved by further optimizing the whole circuit.

- Figure 12.72.z shows a pictorial description of the final circuit.

Figure 12.72. Brute force addition of finite transmission zero sections to a combline filter.

Figure 12.72. Brute force addition of finite transmission zero sections to a combline filter (cont’d).

Figure 12.72. Brute force addition of finite transmission zero sections to a combline filter (cont’d).

Figure 12.72. Brute force addition of finite transmission zero sections to a combline filter (cont’d).

12. 21.  BLOCK DIELECTRIC FILTERS

Block dielectric combline filters with high dielectric constant are among the miniature sized filters used in mobile communication. In dielectric block filters, the legs (resonators) of comb filter are formed as metalized circular cylindrical holes in a low loss dielectric block. Legs are coupled to their neighbors through the dielectric medium which may be homogeneous or inhomogeneous [25], [26], [31], [32], [33]. Mobile communication systems require rather narrow bandwidths, resulting in element values which are too high in series arms and too low in shunt arms, making realization difficult using the classical approaches. In block dielectric filters this problem is solved as follows:

i ) Contrary to the classical approach, quarter wavelength frequency is chosen close to the passband edge (the electrical length of stubs are close to 90 degrees), resulting in relatively well behaved element values and higher Q-values.

ii) Electrical lengths close to 90 degrees result in physically long stubs (long commensurate lengths) especially at low frequencies. This problem is solved by using high dielectric constant materials.

Let’s continue the subject through some examples.

EXAMPLE 12. 30.  A block dielectric combline filter.

Consider the specifications for a four resonator filter to be realized as a block dielectric filter with dielectric constant of Er=50:

PBRipple=0.01 dB,  fp1=975 MHz,   fp2=1025 MHz,  fq=1050 MHz, Nzero=3,  Ninf=5 (NR=4).

Contrary to the classical combline filters which involve only a single transmission zero at f=0, as a demonstration of a flexibility of comb structures, three transmission zeros are placed at f=0. This will lead to two shunt SC stubs left without capacitors.

- The command set is shown in Figure 12.73. Note that Type A option is selected to get the topology starting with a series OC stub after extracting the transmission zeros in the order “Zero, Inf, Inf, Zero, Inf, Inf, Inf, Zero”. The resulting circuit is shown in Figure 12.73.a. Since fq is very close to fp2, the nearest spurious passband is very close to the passband.

The following circuit transformations will be carried out towards obtaining a combline structure:

- As the first step, the LRight section of  SC stubs in the middle is converted to symmetric Pi  section using the command Pi-Tee-L / LRight-to-Sym Pi and the resulting transformer is shifted to rightmost position (Figure 12.73.b):

- Using Exchange command, new LLeft and LRight sections of SC stubs are formed (Figure 12.73.c).

- The LLeft and LRight sections are then converted into symmetric Pi sections by using the commands Pi-Tee-L / LLeft-to-Sym Pi and Pi-Tee-L / LRight-to-Sym Pi placing a shunt SC stub at each node (Figure 12.73.d). Simplify Circuit command combines all transformers to unity and combines the adjacent SC and OC stubs to form OC+SC Stub type resonators (Figure 12.73.f). Note that all the shunt SC stubs have equal impedance of 14.3 ohms.

- At the next step, the shunt OC stubs will be replaced by lumped capacitors. This is done by placing the cursor under one of the SC+OC Stub resonators and applying the command Replace By Lumped / All SC Stub+Cap/ fp1=975, fp2=1025 MHz. The OC+SC Stub elements will be replaced by SC Stub+Cap elements (Figure 12.73.g). This conversion keeps the passband edges unaffected but return loss response is degraded considerably. This is because fq is very close to fp2 and the filter passband is not narrow enough. The degradation in return loss can be corrected by tuning all elements of the filter slightly (Figure 12.73.h). Note that the nearest spurious passband is lowered by about 20 dB. It should also be noted that the equality of the shunt SC stubs is lost. This property will be recovered later.

- The SC Stub+Cap elements can be decomposed into SC Stub and capacitor components by applying Split command (Figure 12.73.h).

- Next, the series OC stubs at the ports are replaced by lumped capacitors by applying the command Replace By Lumped / Single C / f0=1000 MHz (Figure 12.73.i). This replacement also introduces a considerable distortion in return loss which can be recovered to a great extent by tuning all elements (Figure 12.73.k).

- The equality of shunt SC stubs will be recovered as follows:

- With cursor under the single shunt SC stubs (without capacitors), we apply the command Insert / Lossless Lumped Element / Csh=0.00000001 pF. This command inserts a dummy shunt capacitor of zero capacitance in parallel with the SC stubs (Figure 12.73.m).

- Simplify Circuit command combines these capacitors with the adjacent SC stubs to form SC Stub+Cap type resonators. The resonators in the middle of the circuit have nonzero capacitors while the outer ones  have zero capacitors (Figure 12.73.n).

- Finally, with cursor under a series SC stub element, the command Pi-Tee-L / All Equal Shunt / SC Stubs is applied. This command equates all the shunt SC stubs (Figure 12.73.p). It can work only when all the shunt resonator elements are of the same type, like all SC+OC Stub or all SC Stub+Cap. That is why we have to convert the single shunt SC stubs into dummy SC Stub+Cap resonators with zero capacitors.

- The dummy capacitors can be deleted by first splitting the SC Stub+Cap elements as shown in Figure 12.73.p-q.

- Up to this stage, the dielectric constant of the stubs was inherently taken as unity, setting the length of stubs as 71.3 mm. The final operation on the circuit is adjustment of the dielectric constant to the specified value of Er=50. This is done by placing the cursor under all SC stubs (shunt and series) one by one and applying the command Set Dielectric Constant / Er=50 / Adjust Phys. Lng. This command changes the commensurate length to about 10 mm. The high dielectric constant will help also to realize the relatively low characteristic impedances.

- Figure 12.73.s shows pictorial description of a possible realization. The dielectric block is coated with conductor on all faces except the top surface. This coating acts as ground plane. Cylindrical holes are drilled and their surfaces are coated also with a conducting material. They will work as conducting resonator bars (shunt SC stubs). The lower ends of the cylinders are in contact with the lower surface ground plane, thus they act as shunt SC stubs. The lumped capacitors are connected between the top of the cylinders and conducting side surface.

Figure 12.73. Design steps of a block dielectric combline filter.

Figure 12.73. Design steps of a block dielectric combline filter (cont’d).

Figure 12.73. Design steps of a block dielectric combline filter (cont’d).

Figure 12.73. Design steps of a block dielectric combline filter (cont’d).

EXAMPLE 12. 31: A very narrow band block dielectric filter.

Let’s design a high dielectric constant block-dielectric filter with the following specifications:

PBRipple=0.01 dB,  fp1=1499 MHz,   fp2=1501 MHz, Nzero=3,  Ninf=3, Er=50

This is a very narrowband filter. Therefore in order to get reasonable element values we have to choose fq close to the passband edge. Let’s choose it as fq=1520 MHz.

- The command set is shown in Figure 12.74. The transmission zeros are extracted in the order Zero-Inf-Zero-Inf-Inf-Zero. The synthesized circuit is shown in Figure 12.74.a.

- Using the command Pi-Tee-L / Comb-All Equal Shunt, the shunt SC stubs can be equated (Figure 12.74.b).

- The single shunt OC stub is combined with the adjacent SC stub to form an OC+SC Stub element (Figure 12.74.c).

- Then the OC stub component of the OC+SC Stub element is converted into lumped capacitor by applying the command Replace By Lumped / SC Stub+Cap (Figure 12.74.d). This conversion decreases the level of the spurious passband which is just above the main passband.

- However the equality of the shunt SC stubs is lost. It can be recovered by first inserting redundant lumped capacitors of zero value in parallel with each shunt SC stub, combining them with the SC stubs to form SC Stub+Cap elements with zero capacitance (Figure 12.74.d-e).

- Then the command Pi-Tee-L / All Equal Shunt / SC Stubs is applied to equate the shunt SC stubs (Figure 12.74.f-g).

- The redundant zero capacitors are then deleted by first splitting the SC Stub+Cap elements into SC stub and capacitor components (Figure 12.74.h-i).

- Next, the series arm OC stubs are replaced by lumped capacitors (Figure 12.74.h-i). This transformation lowers the spurious passband more. Note that the series arm SC stubs of the circuit are now acting as finite transmission zeros of the filter at f=fq=1520 MHz.

- Physical length of stubs will be shortened by changing the dielectric constant by applying the command Set Dielectric Constant / Er=50 / Adjust physical Lng (Figure 12.74.k). The high dielectric constant will help realization of small impedances.

- Figure 12.74.m shows a pictorial description.

Exercise: Design a block-dielectric filter with the following specifications:

PBRipple=0.01 dB,  fp1=1499 MHz,   fp2=1501 MHz,  fq=1520 MHz, Nzero=3,  Ninf=1.

using the same approach. Note that in this circuit there is no shunt arm capacitor, hence simpler structure.

Exercise: Repeat the previous two examples using higher and lower quarter wavelength frequencies.

Figure 12.74. Development of a very narrow band block dielectric filter.

- Figure 12.74. Development of a very narrow band block dielectric filter (continued).

EXAMPLE 12. 32. A very narrowband block dielectric comb with OC coupled line input and output.

In the previous two examples we have used lumped capacitors at input and output of filters. We may as well use the approach involving OC input coupling line as follows.

Consider a filter with specifications

PBRipple=0.01 dB,  fp1=1499 MHz,   fp2=1501 MHz,  fq=1650 MHz, Nzero=1,  Ninf=3, Er=50

- The command set for the design is shown in Figure 12.75. Note that Type B option is selected.

- Standard Auto Extract command is used giving the circuit of Figure 12.75.a.

- The circuit is transformed to have equal shunt SC stub form after applying LRight-to-Sym Pi command to the LRight section of SC stubs. Simplify Circuit command eliminates transformers (Figure 12.75.b).

- Element values will be adjusted by inserting inverters at source and load ends by applying the command Inverters / Scale Cct By Inserting Inverters / Specify Inverter / K=55 ohms (Figure 12.75.c). Here the inverter constant is a trial value. It will be iterated after inspecting the resulting circuit at the end of all the following circuit transformations.

- The inverters are replaced by L-sections of OC stubs with negative shunt legs, by placing the cursor under the inverters and applying the command Inverters / Replace Ideal Inverter By / End Inverter to / OC Stub Pi-or-L / L-section: f0=1500 MHz, fq=1650 MHz (Figure 12.75.d).

- Simplify Circuit command combines the negative shunt OC stubs with the adjacent positive OC stubs (Figure 12.75.e).

- With cursor under the OC+SC stub resonator, applying the command Replace By Lumped / All SC Stub+Cap, the OC stub component of the resonator will be replaced by its lumped capacitor equivalent while the adjoining SC stub is also modified to keep the passband edges unaffected (Figure 12.75.f).

- The shunt SC Stub+Cap elements can be split into their SC stub and capacitor elements by applying the command Split / All Two Element Resonators (Figure 12.75.g).

- The section of the circuit at source and formed by the series OC stub followed by shunt SC stub is equivalent to a coupled line with transformer. The symmetric version is formed at load end by using Exchange command (Figure 12.75.h). In order to prepare the circuit for conversion into coupled line form without transformers, we do the following transformations:

- With cursor under the series OC stub at source end the command Replace By PCL / Combline / ocscCOMB is applied. The transformer turns ratio need be typed in. We accept the default turns ratio which gives symmetric coupled line. The circuit section formed by the series OC stub and the adjacent shunt SC stub will be replaced by a comb type coupled line section with left leg of the comb being OC and right leg SC (Figure 12.75.i). A transformer will be produced at source end. We note down this transformer turns ratio (n1=0.0303).

- Then, with  cursor under the series OC stub at load end, the command Replace By PCL / Combline / scocCOMB is applied. We again select the default turns ratio, giving the scocCOMB type coupled line with a transformer with a turns ratio equal to the inverse of the one at source end (n2=32.906). We note down this turns ratio also and apply Undo command twice to get the original circuit without coupled lines (Figure 12.75.k).

- We insert unity transformers at source and load ends (Figure 12.75.m).

- The transformer at source end is split to produce the transformers that we have noted down after conversion into coupled line forms by applying the command Split / Specify Left: n=0.0303. Similarly, the transformer at load end is also split by applying the command Split / Specify Left: n=32.906 to produce the transformer that was resulting from conversion into coupled line form at load end (Figure 12.75.n). Exchange command is used to move it to the load end (Figure 12.75.p).

- The transformer of turns ratio n1=0.0303 at source end is shifted to the load end and combined with the transformer at taht end of turns ratio n2=32.906 to form unity and deleted (Figure 12.75.q).

- Thus, the circuit is now ready for conversion into coupled line form. The circuit of Figure 12.75.q is saved as (for example) Block11.fil. It will be used to find the even and odd mode impedances of the coupled line sections. The circuit sections corresponding to the coupled lines are pointed in both Figure 12.75.p and Figure 12.75.q.

- Placing the cursor under the series arm OC stub on left and applying command Replace By PCL / Comblines / ocscCOMB / Sym  yields an ocscCOMB type coupled line as shown in Figure 12.75.r. Similarly, placing the cursor under the series OC stub at load end and applying the command Replace By PCL / Comblines / scocCOMB / Sym produce a scocCOMB type coupled line. The even and odd mode impedances of these coupled lines are shown in Figure 12.75.r.

- Load Block11.fil and use Exchange command to form a Pi-section of SC stubs in the middle part of the circuit (Figure 12.75.s-t). This Pi-section is equivalent to a comb type coupled line with SC ends. It can be converted into coupled line form by placing the cursor under the series arm SC stub of the Pi-section and applying the command Replace By PCL / Combline / scscCOMB / Symmetric (Figure 12.75.u). The even and odd mode impedances are shown underneath the coupled line.

- Note that since the shunt SC stubs are shared by the adjacent coupled line sections, we can’t convert all sections into coupled line forms in the same circuit. That is why we convert each section  into coupled line form one by one.

- The element values are scaled by both the inverters placed at ports and also the transformers resulting shown in Figure 12.75.p. Since we can’t predict these transformers, some iterations with various inverter constants are necessary to get realizable /desired impedances.

- Dielectric constants can be adjusted to Er=50 by using the command Set Dielectric Constant: Er=50 / Adjust Physical Lng.

- Figure 12.75.v shows pictorial descriptions of the filter.

- Figure 12.75. A trial design procedure for a block dielectric filter with OC end lines.

- Figure 12.75. A trial design procedure for a block dielectric filter with OC end lines (cont’d).

- Figure 12.75. A trial design procedure for a block dielectric filter with OC end lines (cont’d).

12. 22. INHOMOGENEOUS  BLOCK DIELECTRIC FILTERS

In all the previous dielectric block combline filters, one may introduce inhomogeneities between the neighboring coupled lines, for example by removing a piece of the dielectric, creating a hole (air dielectric) in the coupling medium [26]. This inhomogeneity will decrease the odd mode dielectric constant while the even mode dielectric constant will not be effected much. Thus, the even and odd mode electrical lengths, hence the even and odd mode quarter wave frequencies, fq,even , fq,odd will be different. Inhomogeneity reflects itself on the circuit model of scscCOMB as described in Figure 12.76.

Figure 12.76. a) All-stub model of inhomogeneous scscCOMB. b) All-stub model of homogeneous scscCOMB type  parallel coupled lines. c) Lumped element approximation for homogeneous or inhomogeneous scscCOMB type coupled lines having an effective finite transmission zero.

Figure 12.76.a shows the all-stub model of an inhomogeneous coupled line of scscCOMB type. Here, the shunt arm SC stub characteristic impedances are set completely by the even mode fields and their impedance is denoted as the even mode impedance Zoe. The series arm element is a parallel combination of two SC stubs whose characteristic impedances and electrical lengths are set by both even and odd mode fields, as follows: One SC stub has positive characteristic admittance equal to half of the odd mode characteristic impedance of the coupled line, Yoo/2 and the other one has  negative characteristic impedance equal to half of the even mode characteristic admittance of the coupled line, -Yoe/2. Electrical lengths of these SC stubs are also different, set by the even and odd mode effective dielectric constants:

o=(2)(f/fq,odd)=(wLko)/c

- e=(2)(f/fq,even)=(wLke)/c                                                                                                   where ke=SQRT(r, eff, even) and   ko=SQRT(r, eff, odd ). Since r, eff, odd <r, eff, even  ,  we have o<e.

- When the even and odd mode effective dielectric constants are equal, the stub model of inhomogeneous coupled line circuit of Figure 12.76.a degenerates naturally to the circuit model of homogeneous scscCOMB structure shown in Figure 12.76.b. Effects of introducing such an inhomogeneity shows itself as a shift in the finite transmission zero produced by the series arm stubs. In the homogeneous case, the series arm SC stub creates a finite transmission zero at exactly f=fq  where its SC termination is translated as open circuit to its input, stopping signal flow. In the inhomogeneous case, the total admittance of the series arm SC stubs becomes zero at a frequency where  Yoecote-Yoocoto=0  which occurs at a higher frequency than fq  depending on the difference between electrical lengths. Thus the series arm impedance of both homogeneous and inhomogeneous coupled lines can be approximated as a lumped element parallel LC resonator as shown in Figure 12.76.c where the resonance frequency is at fq for homogeneous case and greater than fq for inhomogeneous case.

Such filters usually require numerical electromagnetic simulations to characterize the element parameters. Different approaches can be used to design such filters [20 ], [21], [27], [28], [29]. In Filpro, most of such filters can be handled through analysis. In the following an analysis example is given on a simple filter with one scscCOMB section just to study the effect of changing odd mode effective dielectric constant.

EXAMPLE 12. 33. An inhomogeneous block dielectric filter

Let PBRipple=0.05 dB,  fp1=1490 MHz,   fp2=1510 MHz,  fq=1520 MHz, Nzero=3,  Ninf=1 and Er=36

Here, we have chosen fq =1520  MHz close to band edge to make electrical length for even mode close to 90 degrees. The odd mode electrical length will be set by trials, as will be described. The filter will be designed first as a homogeneous one with unity effective dielectric constant. Then effective dielectric constant will be changed to r=36. Finally, after converting the filter to coupled line form, the even and odd mode dielectric constants will be adjusted.

- Design command set is shown in Figure 12.77. Since fq=1520 MHz is too close to the main passband, the first spurious passband is also very close to the main passband, just on the right of fq.

- The synthesized circuit is shown in Figure 12.77.a.

- This circuit is put into symmetric form with equal shunt SC stubs after LRight-to-Sym Pi and Simplify Circuit commands (Figure 12.77.b).

- Then, the series  OC stubs are replaced by  lumped capacitors using command Replace By Lumped / f=1500” (Figure 12.77.c). This conversion lowers the spurious passband on the right of fq.

- Then the effective dielectric constant of each stub is changed to r=36 using Set Dielectric Constant / Er=36 / Adjust Physical Length (Figure 12.77.d).

- Then, with cursor under the middle series arm SC stub, the command Replace By PCL / Comblines / scscCOMB / Sym is applied converting the Pi-section of SC stubs into coupled line equivalent           (Figure 12.77.e). This is a homogeneous coupled line with equal common (even) and Pi (odd) mode dielectric constants, Eev=Eod =36.

- Homogeneous coupled line can be converted to inhomogeneous one by changing the even or odd mode effective dielectric constants. As an example, let’s decrease the odd mode effective dielectric constant Eod  using Tune Element command, from Eod=36 to Eod=34 with steps of 1. Variation of response is shown in Figure 12.77.f. The shift in the position of the transmission zero is clear.

- Both conversion of the series OC stubs and changing the odd mode dielectric constant will degrade response heavily. These degradations can be corrected by tuning other parameters of the filter. In Figure 12.77.g the circuit for Eod=35 is shown. In this circuit the odd mode impedance is tuned to recover the response. The tuned circuit and its response are shown in Figures 12.77.i and k. Return loss response passband is shifted. This shift can be taken into account by redesigning the filter with modified band edges.

Figure 12.77. Development of an inhomogeneous coupled line filter.

Figure 12.77. Development of an inhomogeneous coupled line filter (cont’d).

Figure 12.77. Development of an inhomogeneous coupled line filter (cont’d).

REFERENCES  AND FURTHER READING:

- M. Dishal, “Alignment and Adjustment of Synchronously Tuned Multiple Resonator Circuit Filters”, Proc. IRE, vol. 39, pp.1448-1455, Nov. 1951.

- G. L. Matthaei, “Comb-Line Bandpass Filters of Narrow or Moderate Bandwidth”, The Microwave Journal, Vol. 6, No. 9, pp. 82 - 91, Aug. 1963.

- G. L. Matthaei, L. Young, E. M. Jones, “Microwave Filters, Impedance Matching Networks and Coupling Structures”, Mc Graw Hill, NY, 1964.

- R. J. Wenzel, “Exact Design of TEM Microwave Networks Using Quarter-Wave Lines”, IEEE Trans. MTT,  Vol. MTT-12, pp. 94 - 111, Jan. 1964.

- M. C. Horton and R. J. Wenzel, “General Theory and Design of Optimum Quarter Wave TEM filters”, IEEE Trans. MTT,  vol. MTT-13, No. 3, pp. 316 - 327, May 1965.

- M. Dishal, “A simple design procedure for small percentage bandwidth round rod interdigital filters”, IEEE Trans. MTT,-13, pp. 696-698, Sept. 1965.

- R. M. Kurzrok, “Design of combline bandpass filters”, IEEE Trans. MTT-14, pp.351-353, July 1966.

- R. J. Wenzel, “Theoretical and practical applications of Capacitance matrix transformations to TEM Network Design”, IEEE Trans. MTT, vol. MTT-14, No. 12, pp. 635 - 647, Dec. 1966.

- G. I. Zysman, “Coupled Transmission Line networks in an Inhomogeneous Medium”, IEEE Trans. MTT, Vol. MTT- 17, No. 10,  pp. 753 - 759, Oct. 1969.

- R. Pregla, “Mikrowellenfilter aus verkoppelten Leitungen und konzentrierten Kapazitanen”, Archive der Elektrichen Ubertragung, vol. 23, No. 12, pp. 593-600, 1969.

- R. Levy, and J. D. Rhodes, “A Combline Elliptic Filter”, IEEE Trans. MTT, vol. MTT-19, pp. 26-29, Jan. 1971.

- R. J. Wenzel, “Synthesis of Comb-line and Capacitively Loaded Interdigital Bandpass Filters of Arbitrary Bandwidth”, IEEE Trans. MTT, vol. MTT-19, No. 8, pp. 678 - 686, Aug. 1971.

- E. G. Crystal, “Tapped-Line Coupled Transmission Lines with Applications to Interdigital and Combline Filters”, IEEE Trans. on  MTT,  vol. MTT-23, No. 12,  pp. 1007-1012, Dec. 1975.

- R. Briechle, “Microstrip Combline Filter”, 5. European Microwave Conference Proc., pp.436-440, 1975.

- A. Atia and A. Williams, “Measurement of Intercavity Coupling”, IEEE Trans. MTT-23, pp.519-522, June 1975.

- V. K. Tripathy, “Asymmetric Coupled Transmission Lines in an Inhomogeneous medium”, IEEE Trans. MTT, Vol. MTT- 23, No. 9,  pp. 734 -739, Sept. 1975.

- J. D. Rhodes, “Theory of Electrical Filters”, London, England, Wiley, 1976.

- G. R. Hoffman, “Resonated Combline Bandpass Filter”, 6. European Microwave Conference Proc., pp. 54-58, 1976.

- N. McDonald, “Measurements of Intercavity Couplings”, IEEE Trans. MTT, vol. MTT-24, p. 162, Mar. 1976.

- V. K. Tripathy,  “Equivalent Circuits and Characteristics of Inhomogeneous Nonsymmetrical Coupled Line Two-Port Circuits”, IEEE Trans. MTT,  pp. 140 -142,  Feb. 1977 .

- J. S. Wong, “Microstrip Tapped Line Filter Design”, IEEE Trans. MTT, vol. MTT-27, No. 1, pp. 44 - 50,  Jan. 1979.

- J. A. G. Malherbe, “Microwave Transmission Line Filters”, Artech House, 1980.

- C. Kuang, C. Tzuang and W. T. Lo, “Printed Circuit Realization of a Tapped Combline Bandpass Filter”, 1990 IEEE, MTT-S Conference Proc., pp.131 - 134, 1990.

- C. M. Tsai and K. C. Gupta, “A Generalized Model for Coupled Lines and its applications to Two-Layer Planar Circuits”, IEEE Trans. MTT, Vol. MTT- 40,  pp. 2190 - 2199, Dec. 1992.

- A. Fukasawa, “Analysis and composition of a new microwave filter configuration with inhomogeneous medium”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-30, No. 9, pp. 1367-1374, Sept. 1982.

- R. Levy, “Simplified analysis of inhomogeneous dielectric block combline filters”, 1990 IEEE MTT-S Digest,pp.135-138, 1990.

- S. Caspi, J. Adelman, “Design of  Combline and Interdigital Filters with Tapped-Line Input”, IEEE Trans MTT, vol. MTT-36, No. 4, pp. 759-763,  Jan. 1979.

- K. Wada, N. Otani, Y. Noguchi and J. Ishii,  “Tapped Line Interdigital Bandpass Filters with narrow bandwidth Using Asymmetric Broadside Coupled Coplanar Waveguides”, Proc. EuMC’ 95, pp. 487 - 492, 1995.

- C. Ernst, V. Postoyolka, “Tapped Line Interdigital Filter Equivalent Circuits”, 1997 IEEE, MTT-S Proceedings, pp. 801-804, Denver, 1997.

- K. Wada, I. Awai, Y. Yamashita, “Characteristics of /4 CPW resonators with tap excitation and their application to bandpass filters”, 1997 IEEE, MTT-S Proceedings, pp.717-720, Denver, 1997.

- M. Sugawara, M. Makimoto, S. Yamashita, “A design method of bandpass filters using dielectric filled coaxial resonators”, IEEE Trans. MTT-33, pp. 152-157, 1985.

- K. Hano, H. Kohriyama, K. I. Sawamoto, “A direct coupled /4 coaxial resonator bandpass filter for land mobile communication”, IEEE Trans. MTT-34, pp. 972-976, 1986.

- C. C. You, C. L. Huang, C. C. Wei, “Single block ceramic microwave bandpass filters”, Microwave Journal, pp. 24-35, No. 1994.

- R. Levy, H. W. Yao, K. A. Zaki, “Transitional combline/evanescent mode microwave filters”, IEEE  MTT-S, pp.461-464, 1996.

- H. W. Yao, K. A. Zaki, A. E. Atia, T. Dolan, “Improvements of spurious performance of combline filters”, IEEE, MTT-S, pp. 1099-1102, 1997.

- J. Ness, “A Unified Approach to the Design, Measurement and Tuning of Coupled Resonator Filters,” IEEE, MTT-46, No. 4, pp.343-351, April, 1998.

- I. Hunter, “Theory and Design of Microwave Filters,” IEE Press, 2001.

- C.Ernst, V. Rosloniec, “Comments on relationship between group delay and stored energy in microwave filters”,  IEEE Trans. MTT-49, No. 9, pp. 258-261, Sept. 2001.

REFERENCES FOR CALCULATION OF COUPLED LINE PARAMETERS:

- Wheeler, H. A., “The Transmission Line Properties of Round Wires Between Parallel Planes”, IRE, Trans. Antenna and Propagation, AP-3, No. 10, pp. 203-207, Oct. 1955.

- S. B. Cohn, “Shielded Coupled Microstrip Lines,” IRE Trans. MTT-3, No. 10, pp. 29-38, Oct. 1955.

- Wheeler, H. A., “Transmission Line Properties of Parallel Strips Separated by Dielectric Sheet,” IEEE MTT-13, pp. 172-185, 1965.

- Cristal, E.G., “Coupled Circular Cylindrical Rods Between Parallel Ground Planes”, IEEE Trans. MTT-12,  No. 7, pp. 428-438, July 1964.

- Getsinger, W. J., “Microstrip Dispersion Model,” IEEE MTT-21, pp. 34-39, 1973.

- Wheeler, H. A., “Transmission Line Properties of a Stripline Between Parallel Plates”, IEEE MTT-26, No.11, pp. 866-876, Nov. 1978.

- T Itoh, “Generalized Spectral Domain Method for Multiconductor Printed Lines and its Application to Tunable Suspended Microstrips,” IEEE MTT-26, No. 12, pp. 983-987, Dec. 1978.

- Gupta, K. C., R. Garg, I. J. Bahl, “Microstrip Lines and Slotlines”, Norwood, MA, Artech House, 1979.

- Hammerstad, E. and O. Jensen, “Accurate Models for Microstrip Computer Aided Design”, IEEE MTTS-1980, Washington D.C., pp. 407-409, May, 1980.

- Gupta, K. C., R. Garg, and R. Chadha, “Computer-Aided Design of Microwave Circuits,” Norwood, MA, Artech House, 1981.

- Kirsching, M. and R. H. Jansen, “Accurate Model for Effective Dielectric Constant of Microstrip with Validity up to Millimeter Wave Frequencies,” Elect. Letters, Vol.18, No. 6, pp. 272-273, March 1982.

- Kirsching, M., and R. Jansen, “Accurate Wide-Range Design Equations for the Frequency Dependent Characteristics of Parallel Coupled Microstrip Lines,” IEEE MTT-32, No. 1, pp. 83-90, Jan. 1984. Also in IEEE MTT-33, No. 3, p. 288, March 1985.

- Pramanick, P. and P. Bathia, “Computer Aided Design Models for Millimeter Wave Finlines and Suspended Substrate Lines”’ IEEE MTT-33, No. 12, pp. 1429-1435, Dec. 1985.

- Bedair, S. S., “Characteristics of Asymmetrical Coupled Shielded Microstrip Lines,” IEE Proc. Vol. 132, pp. 342-343, 1985.

- Abdallah, E. A., El-Deeb, N. A., “Fast Accurate Computational Method for Asymmetric Coupled Microstrip Parameters,” IEE Proc., Vol. 132, Pt. H, No. 1, pp. 69-70, 1985.

- Stracca, G. B., G. Macchiarella, and P. Politi, “Numerical Analysis of Various Configurations of Slab Lines,” IEEE MTT-34, No. 3, pp. 359-363, March 1986.

- E. Yamashita, M. Nakajima, K. Atsuki, “Analysis Method for Generalized Suspended Striplines,” IEEE MTT-34, No. 12, pp. 1457-1463, Dec. 1986.

- Hoffmann, R. K., “Handbook of Microwave Integrated Circuit”, Norwood, MA, Artech House, 1987.

- Tomar K. S., P. Bhartia, “New Quasi-Static Models for Computer Aided Design of Suspended and Inverted Microstrip Lines”, IEEE MTT-35, No. 4, pp. 453-457, April 1987.

- Rosloniec, S., “Design of Coupled Microstrip Lines by Optimization Methods,” IEEE MTT-35, No. 11, pp. 1072-1074, Nov. 1987.

- Shu, Y. Xiao-xia, Q. Wang Y., “Analysis Equations for Shielded Suspended Substrate Microstrip Line and Broadside Coupled Stripline,” IEEE MTTS Digest, pp.693-696, 1987.

- Wang Y., Gu, K., Shu, Y., “Synthesis Equations for Shielded Suspended Substrate Microstrip Line and Broadside Coupled Stripline,” IEEE MTTS Digest, pp.331-334, 1988.

- Rosloniec, S., “An Improved Algorithm for the Computer Aided Design of Coupled Slab Lines,” IEEE Trans. MTT-37, No. 1, pp. 258-261, Jan. 1989.