CHAPTER 14

CROSS-COUPLED RESONATOR FILTERS-1:

CASCADED TRIPLET AND CASCADED QUADRUPLET FILTERS

14. 1. INTRODUCTION

Direct Coupled and Cross-Coupled resonator filters are among the most popular microwave filters. Cross-Coupled resonator filters are mostly used when the finite transmission zeros are very close to passband or when linear phase response is needed to satisfy the extreme specifications on both amplitude and phase.

Figure 14.1 shows one classification of such filters. In this figure, the circles denote shunt LC resonators. Couplings between resonators are shown as heavy lines and may be inductive or capacitive.

- Figure 14.1.a shows cascadable coupled resonator N-tuplet sections.

- A coupled resonator pair is termed as a Doublet. It is a degree four element. If the coupled element is a capacitor then the doublet realizes three transmission zeros (TZ’s) at f=0 (Nzero=3) and one TZ at f= (Ninf=1). If the coupling element is an inductor then it realizes one TZ at f=0 (Nzero=1) and three TZ’s at f= (Ninf=3). That is, in either case Nzero+Ninf=4. Filters formed by cascading such doublets is termed as direct coupled resonator filters.

- A mutually coupled three resonator section is termed as Triplet. It is a degree N=6 section. It realizes a single finite transmission zero (FTZ). The total number of TZ’s at f=0 and f= is four, the same as in Doublet case: Nzero+Ninf=4, with either Nzero=3, Ninf=1 or Nzero=1, Ninf=3, depending on the coupling element types. The FTZ can be located in either lower or upper stopband by selecting the type of coupling elements properly. Filters formed by cascading such triplets are termed as Cascaded Triplet (CT ) filters and the triplets are referred to as CT sections.

- A mutually coupled four resonator section is named as Quadruplet. Numbering the resonators as 1-2-3-4, with 1 and 4 being the input and output respectively, depending on the cross-couplings a quadruplet can create the following transmission zero patterns:

- If the input signal follows the paths 1-2-4 and 1-3-4, then this structure can create a single FTZ which can be either in lower or upper stopband. The signals traveling along the two paths may cancel each other at one frequency after combining at the output leading to a transmission zero at that frequency. Such a quadruplet forms a degree 8 section, with Nzero+Ninf=6 and two degrees coming from the FTZ. In Filpro such quadruplet blocks are named as CQ1F sections.

- If the input signal follows the paths 1-2-3-4 and 1-4 then the signals at the output port 4 can be made to have equal amplitudes and opposite phases, thus cancelling each other, at two distinct frequencies, fz1 and fz2 which are symmetrically located on the two sides of the passband. The symmetry condition is fz1*fz2=fp1*fp2. The element 1-4 is named as the cross-coupling element while the elements on the path 1-2-3-4 are named as inline or mainline elements. The two FTZ’s can be created by selecting one of the coupling elements different from the other three coupling elements. In usual applications the cross-coupling element is selected as capacitive while the inline elements are inductive. The opposite case and other versions are also possible. This is also a degree 8 section with Nzero+Ninf=4 and four degrees coming from the two FTZ’s. In Filpro such quadruplets are named as CQBP sections (to differentiate them from CQLP elements which are used in LP filters). The filters formed by cascading CQ sections are referred to as CQ filters.

- If the 1-4 cross-coupling element is of the same type as the main line coupling elements, that is, if all the coupling elements are of the same type, then the quadruplet creates a complex conjugate quadruplet of transmission zeros: sk=k+jwk. The complex TZ’s are used for phase linearization (group delay flattening) purposes.

- The amplitude and phase of the signal on the cross-coupling path may also be adjusted by using more sophisticated circuit sections. The simplest of such cross-coupling circuits is a transmission line piece with inductive or capacitive couplings to resonators 1 and 4. On Filpro such quadruplets are simulated by the element named as CQTL. This element contains a transmission line and inverter combination as the 1-4 cross-coupling element.

- In addition to the 1-4 cross-coupling, if also a diagonal cross-coupling exist between either 2 and 4 or 1 and 3, then this structure can be used to create an asymmetric pair of FTZ’s. The signals in the paths 1-2-3-4 and 1-4 which create a symmetric pair of FTZ’s are now disturbed by the signal in the diagonal coupling path, displacing the two FTZ’s, hence leading to asymmetry. The FTZ’s may be in the same stopband or in the two stopband. This is also a degree 8 section with Nzero+Ninf=4 and four degrees coming from the two FTZ’s. In Filpro such elements are named as Generalized CQ (GCQ) sections as will be described ahead.

- A cross-coupled five resonator block is named as Quintuplet, six resonator block is named as SixTuplet, seven resonator block is named as SevenTuplet, etc. An N-resonator cross-coupled module can create N-2 FTZ’s. Such modules will be named as Canonical N-tuplets. The N-2 FTZ’s can be located arbitrarily in any one or both stopbands. However such arbitrarily located FTZ’s lead to diagonal cross-couplings which are usually troublesome for both realization and tuning. The diagonal cross-couplings can be avoided by sharing the FTZ’s equally and symmetrically to both stopbands. Such N-tuplet modules are named as Symmetric N-Tuplets. An N-resonator block may also be used to realize less than N-2 FTZ’s. Such N-Tuplets are named as non-canonical N-tuplets. In non-canonical N-tuplets some of the cross-couplings can be eliminated leading to simpler structures. On Filpro, Symmetric and canonical or non-canonical N-tuplet blocks up to degree 10 can be formed by either exact or approximate methods.

- The usual Chebyshev-Butterworth type Direct Coupled resonator filters are formed by cascading doublets (Figure 14.1.b).

- Figure 14.1.c shows a class of filters termed as Cascaded Triplet (CT) filters. It is formed by cascading Triplets of resonators (named as CT sections). Each CT section is used to create a single  s=jw axis finite transmission zero (FTZ). So, by cascading such triplets it is possible to realize general asymmetric responses having finite transmission zeros on either side of passband.

- Figure 14.1.d shows a filter formed by cascading CQ1F sections. Since each CQ1F section realizes a single FTZ, asymmetric responses can be realized like in CT filters, but with only vertical and horizontal type couplings (no tilted or diagonal coupling elements) which makes construction and tuning easier.

- Figure 14.1.e shows a structure termed as Cascaded Quadruplet  (CQ) filter made up by cascading CQBP elements. Since CQBP sections produce only symmetric FTZ’s, such filters can have only symmetric responses.

- Figure 14.1.f shows a more general form of Cascaded Quadruplet filter that will be termed as Generalized Cascaded Quadruplet (GCQ) filters. Such filters can be used to realize asymmetric responses.

- Figure 14.1.g shows a general filter formed by cascading CT and CQ sections.

- Figure 14.1.h shows the structure known as Canonical Cross Coupled filter. This structure is the most compact hence the most space efficient coupled resonator structure. With the given number of resonators, it can generate the maximum number of finite transmission zeros. They will be studied in the next chapter.

CT, CQ and GCQ filters may be considered as more complex forms of cascaded element filters or simpler special forms of Canonical Cross Coupled Resonator filters. Therefore they may be designed using both Cross-Coupled Synthesis approaches 1-11 or Cascade Synthesis approach 12, 13, 14. Filpro can design Direct Coupled, Cascaded Triplet, Cascaded Quadruplet and Generalized Cascaded Quadruplet filters and their cascaded combinations using the classical Cascade Synthesis approach. Canonical Cross Coupled filters can be designed using either LP prototype tables or by brute force approximation and tuning/optimisation approaches as will be described in the next chapter.

It is also possible to design CQ sections with complex transmission zeros for phase linearization purposes. This is done by brute force introduction of a cross-coupling and then tuning of the coupling elements. The cross-coupling elements may be combinations of transmission lines and inverters. The resulting CQ section is named as CQTL. The transmission lines can be used to adjust phase of cross-coupling signal to change the locations of finite transmission zeros or to adjust the phase response.

The main advantage of CT and CQ1F sections filters is that tuning of a finite transmission zero can be carried out without effecting the other finite transmission zeros. CQ and GCQ filters are more compact and efficient structures compared to CT and CQ1F sections, as they realize two FTZ’s. However tuning of quadruplets effect two finite transmission zeros simultaneously. Therefore more care is needed in construction and tuning. The price paid for the compactness and efficiency of Canonical Cross Coupled filters is that very high precision is required for construction, as all the cross coupling elements effect all the finite transmission zeros. Another disadvantage of Canonical Cross Coupled filters is that input and output ports of such filters are on the same side that may cause realization problems in some applications.

In general a coupled resonator filter may contain all CT, CQ1F, CQ, CQTL, GCQ elements or other N-tuplets, as well as direct coupled resonators. Some resonators may be shared by the adjacent triplets or quadruplets. Thus, many realizations are possible with a given number of resonators.

In this chapter the design of coupled resonator filters having CT, CQ1F, CQBP, CQTL and GCQ sections will be described.

Figure 14.1. N-tuplet coupled resonator sections and various Cascaded N-tuplet filters.

14. 2. ON CASCADED TRIPLET SECTIONS

Cascaded triplet (CT) filters are formed by cascading circuit sections consisting of three mutually coupled shunt LC resonators termed as triplets. Each triplet is responsible for the creation of a jw axis finite transmission zero which is either below or above the passband 12, 13, 14. In Filpro, CT filters will be designed using exact Synthesis approach together with some circuit transformation tools.

In Filpro, such three mutually coupled resonators are handled through components termed as Bridge Tee. In Figure 14.2.a, a general purpose Bridge Tee is described. This is a three node element. In this element, the arms are made up of parallel LC resonators. Node-1 and Node-3 are input and output ports respectivel. Node-2 is the middle node having a parallel LC resonator to ground. Simpler bridge circuits can be formed by setting some elements as zero or infinity, as required. Figure 14.2.b shows the All-Stub version where inductors are replaced by SC Stubs with impedance denoted as Zs, and capacitors are replaced by OC Stubs with impedance denoted by Zo. This element can be used for distributed element designs. Figure 14.2.c shows a mixed element type bridge where there is no OC stub, but instead we have lumped capacitors, a more realistic simulation of general practice. We can obtain Mixed CT element from both All-Stub CT using Replace By Lumped command or obtain from All-Lumped CT through the command Replace By Stub. If Replace By Lumped command is applied to a Mixed CT then an All-Lumped CT will be obtained. All these transformations are approximate in nature leading to some slight distortions in response that can be corrected by tuning some of the elements.

Figure 14.2.d shows a lumped element bridge that can be used for delay equalization in some cases.

Figure 14.2. General purpose Bridge Tee elements available in Filpro.

In Cascaded Triplet filters, simpler versions of the general purpose Bridge Tee are needed where the coupling elements are either single capacitor, inductor, SC or OC stub. That is one element of the second degree coupling branches will be set as zero or infinite. In Filpro, special and simpler cases of the lumped element Bridge Tee are defined as separate components for clarity in frequently used cases. In Figure 14.3, these simpler Bridge Tee’s and their use in coupled resonator triplets are shown. In these Bridge Tee’s couplings between resonators are either capacitive or inductive. The middle node is connected to ground through a simple parallel LC resonator. Thus, a coupled resonator triplet is formed by sticking two resonators on the two sides of a Bridge Tee, with the third resonator being the one at Node-2. The terminology used to differentiate various Bridge Tee types are also shown in the same figure, as LLL, LCL,.. etc. In this notation, the first letter indicates the type of bridge coupling between  input (Node-1) and output (Node-3). The second letter denotes coupling type between Node-1 and Node-2 and third letter denote coupling type between Node-2 and Node-3. In these circuits the feedback path between nodes 3 and 1 is used to stop signal flow between input and output at one frequency, thus forming a finite transmission zero at that frequency.

Finite Q of the shunt resonator inside the CT elements (L2C2) is taken into account by a resistor R2 placed in parallel with L2C2, as shown in Figure 14.4. Its default value is infinite (very large). The actual resistance R2 is calculated upon application of the command “Parasitics / Lossy Element / Q=..”.

Typical cascaded triplet filter structures may be formed by directly cascading the triplets as shown in Figure 14.5.a, in which case one resonator will be common to both of the adjacent triplets. The triplets may also be coupled capacitively or inductively as shown in Figure 14.5.b.

In Filpro, cascaded triplet filters can be synthesized using the Synthesis approach by specifying the desired transmission zeros directly. Then the sections containing the finite transmission zeros are put into standard forms for conversion into triplet form by circuit transformations prepared for this purpose.

Conversion of a synthesized filter sections into Cascaded Triplet form can be carried out using the circuit transformation command CQ/CT Oprs / Convert to CT / Left (Right) / CCC (LLL, CLL, ..). In the resulting circuit, the three shunt resonator inductors come out to be equal. This command is applicable to several circuit sections, as described in Figure 14.6-to-8. With the cursor under the series arm finite transmission zero (FTZ) element (series arm parallel LC resonator), application of this command places an inductive or capacitive feedback path between nodes 3 and 1 while eliminating either capacitance or inductance of the parallel LC resonator. The resulting bridge type (LLL, LCL, etc.) depends on the position of finite transmission zero with respect to the passband (i.e. whether it is in upper or lower stopband) and on the coupling elements of the circuits, as described below:

Figure 14.3. Bridge Tee elements and coupled resonator triplets.

Figure 14.4. Lossy model of CT elements.

Figure 14.5. Typical Cascaded Triplet filter topologies.

a) CT sections with three transmission zeros at infinity

Consider the circuit section shown in Figure 14.6.a. This section contains one transmission zero at f = 0 and three transmission zeros at f = . It has inductive coupling between nodes 1 and 2. It can be transformed into CT form using the command “CQ-CT Oprs / Convert to CT” as described in the figure.

Figure 14.6. CT sections with three transmission zeros at f=.

The resulting bridge types are shown on right for the cases when the finite transmission zero is in lower and upper stopband. It is seen that when the finite transmission zero is in upper stopband, the coupling between the nodes 2 and 3 also comes out to be inductive, leading to an all inductive (LLL type) Bridge Tee. When the finite transmission zero is in lower stopband, two solutions exists: The exact solution yields capacitive coupling between nodes 2 and 3. The other solution is also an LLL type, but has negative inductance bridge coupling. The negative  inductor is approximated as capacitance, leading to a CLL type Bridge Tee. This approximation leads to degradation in return loss which is negligible for narrowband filters and can be corrected using tuning-optimization for medium bandwidth filters. For wideband cases considerable distortions may arise in response. The transformations described in Figure 14.6.b are just the reversed forms of those in Figure 14.6.a, leading to right-left turned around Bridge Tee’s. If filter is designed as commensurate length distibuted element, then the same commands and same CT types are valid provided the terms L and C are interpreted as SC Stub and OC Stub respectively.

b) CT sections with three transmission zeros at f=0

The circuit section shown in Figure 14.7.a contains three transmission zeros at f=0 and one transmission zero at f=. This circuit section can be transformed into CT form using the command “CQ-CT Oprs / Convert to CT”, as described in the figure. The resulting bridge types are shown on the right for the cases when the finite transmission zero is in lower and upper stopband. It is seen that when the finite transmission zero is in lower stopband, the bridge will have three capacitive couplings (CCC type Bridge Tee). If the finite transmission zero is in upper stopband, the coupling between nodes 2 and 3 will be inductive, leading to CCL type Bridge Tee. For this case another solution exists in LCC form which is actually CCC form with negative bridging capacitor. The negative capacitor is approximated by a positive inductor giving the LCC type bridge tee. The approximation is good for narrow-medium bandwidth filters.

The circuits of Figure 14.7.b are just the reversed form of the circuit of Figure 14.7.a.

It should be noted that the couplings will be predominantly inductive if one transmission zero is placed at f=0 and three transmission zeros are placed at f=. The coupling elements will be predominantly capacitive if three transmission zeros are placed at f=0 and one transmission zero is placed at f=.

In these transformations inductors of all three shunt resonators come out to be equal automatically. Elements of CT section can be adjusted through the command “CQ-CT Oprs / Modify CT / Set L1, Set L2,..”. Since elements of CT are closely interrelated, changing one element value may effect other elements of the CT or even the whole circuit. Therefore application of Modify CT command requires some care.

Figure 14.7. CT sections with three transmission zeros at f=0.

Preparation of these circuit sections and design of higher order cascaded triplet filters will be described through the examples in the following sections. In these designs only the exact transformations that involve circuit sections with one transmission zero at f=0 and three transmission zeros at f= or three transmission zeros at f=0 and one transmission zero at f= will be used. Circuits having higher degrees are formed by selecting the number of transmission zeros at zero and infinity properly so as to form the necessary CT sections.

14. 3.  ON CQ SECTIONS WITH ONE FINITE TRANSMISSION ZERO (CQ1F sections)

The diagonal/tilted nature of cross-couplings in CT filters may be undesirable in some applications [18], [19]. In such cases a single finite transmission zero may also be realized as a cross-coupled quadruplet of resonators where the couplings are only vertical and horizontal. In Filpro this quadruplet is named as CQ1F section (CQ section with 1 FTZ). In this approach, a four direct coupled resonator circuit section having one FTZ is converted into a CQ1F section as described in Figure 14.8. The signal at input resonator-1 is split into two parts and signals travel to the output port-4 through the paths involving resonators 2 and 3. Proper phasing along the two paths can lead to cancellation of the signals at port 4, thus creating a finite transmission zero.

Figure 14.8.a shows the two circuit sections that can be converted into CQ1F sections. The circuit section at left top has one TZ at f=0 and 5 TZ’s at f= and termed as inductive prototype while the one on right has five TZ’s at f=0 and one TZ at f= and termed as capacitive prototype. The single FTZ represented by the series arm parallel LC section provides two degrees, thus the total degree is eight in both circuits (four resonators).

These circuit sections can be converted into CQ1F section by placing the cursor under the series arm parallel LC resonator and applying the command CQ-CT Oprs / Convert-to-CQ / Solution-1 (or Solution-2). The resulting CQ1F  element is shown in Figure 14.8.b.  Resonators 2 and 3 are hidden inside the CQ1F element while resonators 1 and 4 are left outside. The expanded version of the CQ1F element is shown in Figure 14.8.c. Although it is possible to use lumped inductors and capacitors as the coupling elements, it is found to be more convenient to use ideal inverters as the coupling elements between resonators. The following observations can be made by inspection of the CQ1F sections:

- For inductive prototype the inverters K12 and K34 come out to be positive always while for Solution-1, K13 is also positive and K24 is negative and for Solution-2, K24 is positive while K13 is negative.

- For capacitive prototype the inverters K12 and K34 come out to be negative always while for Solution-1, K13 is also negative and K24 is positive and for Solution-2, K24 is positive while K13 is positive. Approximation of coupling elements by inverters degrades return loss slightly which can be corrected by tuning K13 only. In some cases the other inverters may also be tuned slightly.

- It is seen that in any case one of the coupling elements has different sign than the other three coupling elements.

- Figure 14.8.d shows the coupling elements as inductors (for positive inverters) and capacitors (for negative inverters). The LC models of CQ1F element is not available on Filpro. If desired, it is possible to get the LC models of inverters together with the adjoining shunt LC resonators as follows: Using Insert command the shunt resonators and the relevant coupling inverter can be inserted into the circuit. Then with cursor under the inverter, the command Replace Inverter By / Pi-Tee Model / Inductive Pi (if K>0) or Capacitive Pi (if K<0) is applied (See the following example).

It is clear that in order to form a CQ1F section degree of the filter must be at least 8. Two CQ1F sections necessitates a minimum of degree 14 in which case the two CQ1F sections will share one resonators.

Figure 14.8. Conversion of four resonator section with a single FTZ into CQ1F section.

14. 4. CASCADED QUADRUPLET (CQ) SECTIONS

Four resonators can be used to create two finite transmission zeros (FTZ’s) by introducing cross-couplings between non-adjacent resonators thus forming cross-coupled quadruplets named as CQ sections. On Filpro, a variety of CQ elements are defined as follows:

a) Generalized CQ Sections (CQ sections with Diagonal Cross-Couplings):

In Figure 14.9 the transformations and equivalent circuits of CQ sections with diagonal cross-couplings are shown. The following points can be noted:

- CQ sections are obtained by circuit transformations from degree 8 (four resonators) direct coupled resonator circuit sections containing two FTZ elements.

- Figure 14.9.a shows two possible circuits leading to similar CQ sections. With the resonators being numbered as 1-2-3-4, the FTZ elements are located between nodes 2-3 and 3-4 while we have either an inductive or capacitive coupling between nodes 1 and 2. In either case the two adjacent FTZ elements are combined to form a single fourth order section and then the resulting circuit is converted into a CQ section by applying the command CQ-CT Oprs / Convert to CQ / … .

- The circuits of Figure 14.9.a yield a CQ element named as CQKR. Here the letter K implies that the coupling elements are ideal inverters while the letter R implies that the diagonal coupling element is between node-2 and node-4 (tilted Right).

- Figure 14.9.b shows the details of CQKR element. All the coupling elements are ideal inverters except the 2-4 diagonal cross-coupling which is shown as a parallel LC element with either L is made ineffective (L=) or C is made ineffective (C=0) to simulate capacitive or inductive diagonal cross-couplings respectively. The shunt resonators at nodes 2 and 3 have also parallel resistors to account for the finite Q of the resonators.

- Figure 14.9.c shows the CQ section after converting the ideal inverters of CQKR element into their Pi-section equivalents. This element is named as CQLR. In this circuit all the coupling elements are of parallel LC type with either L is made ineffective (L=) or C is made ineffective (C=0) to simulate capacitive or inductive couplings respectively.

- Figure 14.9.d shows two possible circuits leading to similar CQ sections. The FTZ elements are located between nodes 1-2 and 2-3 while we have either an inductive or capacitive coupling between nodes 3 and 4. In either case the two adjacent FTZ elements are combined to form a single fourth order section and then the resulting circuit is converted into a CQ section by applying the command CQ-CT Oprs / Convert to CQ / … .

- The circuits of Figure 14.9.d yields a CQ element named as CQKL. Here the letter K implies that the coupling elements are ideal inverters while the letter L implies that the diagonal coupling element is between node-1 and node-3 (tilted Left).

- Figure 14.9.b shows the details of CQKL element. All the coupling elements are ideal inverters except the 1-3 diagonal cross-coupling which is shown as a parallel LC element with either L is made ineffective (L=) or C is made ineffective (C=0) to simulate capacitive or inductive diagonal cross-couplings respectively. The shunt resonators at nodes 2 and 3 have also parallel resistors to account for the finite Q of the resonators.

- Figure 14.9.e shows the CQ section after converting the ideal inverters of CQKL element into their Pi-section equivalents. This element is named as CQLL. In this circuit all the coupling elements are of parallel LC type with either L is made ineffective (L=) or C is made ineffective (C=0) to simulate capacitive or inductive couplings respectively.

- In all cases the two FTZ elements provide four degrees. The other four degrees are provided by the transmission zeros at f=0 (Nzero) and f= (Ninf). That is Nzero+Ninf=4. The four degrees can be shared between zero and infinity either as Nzero=1, Ninf=3, leading to a series inductor between node-1 and node-2 of the degree 8 circuit sections shown in the figures (named as inductive prototype) or Nzero=3, Ninf=1 leading to a series capacitor between node-1 and node-2 (named as capacitive prototype).

- The two FTZ’s may be located in the same stopband or one in the upper stopband the other in the lower stopband. Such arbitrarily located FTZ pairs that lead to asymmetric responses can always be transformed into generalized CQ sections containing diagonal cross-couplings. On Filpro we have the following CQ elements that contain diagonal cross-couplings:

Figure 14.9. Generalized CQ sections and equivalent circuits.

- On Filpro a positive inverter is treated as inductive coupling and negative inverter as capacitive coupling.

- Coupling element signs and values depend on both the element type between node-1 and node-2 of the prototype and also on the locations of the two FTZ’s.

- For certain values of the FTZ’s, fz1 and fz2 the diagonal cross-coupling disappears simplifying the realization of the CQ section. Such a section has great practical importance. Therefore they are treated specially, as described in the next subsection.

CQKR (CQLR) and CQKL (CQLL) elements can be used to realize asymmetric responses.  However the existence of diagonal cross-couplings makes these elements unattractive because of practical realization problems. Therefore such elements are rarely used. It is much easier to realize asymmetric responses by using CT or CQ1F sections.

b)  Simple CQ Sections (CQ sections without diagonal cross-couplings):

CQ sections without diagonal cross-coupling have special practical importance because of simplicity of construction. On Filpro such CQ sections are named as CQBP element. The cross-coupling exists only between nodes 1 and 4 (K14) (Figure 14.10). Such a topology arises when the two FTZ’s, fz1 and fz2 are symmetrically located about the passband center. The symmetry condition depends on the relative numbers of transmission zeros of the filter above and below the passband. In the cases where we have equal numbers of transmission zeros at f=0 and f=, the conventional geometric symmetry condition fz1*fz2=fp1*fp2=f02 leads to elimination of the diagonal cross-couplings. However in most cases we have unequal numbers at f=0 and f=. In such cases we can use the symmetry condition fz1*fz2=fp1*fp2=f02 just to get a starting point for an iterative search by slightly adjusting fz1 or fz2 until the diagonal cross-coupling is eliminated.

CQBP section has the following properties:

- The main line coupling elements (1-2, 2-3, 3-4) are all positive (K12, K23, K34>0).

- The cross-coupling element (1-4) is negative (K14<0). In other words, the sign of K14 is opposite of the main line couplings.

- Such a CQ section arises only when inductance exists between nodes-1 and 2 of the original (direct coupled) version of the filter (inductive prototype).

- These properties make the implementation of CQBP sections much easier compared to the CQ sections with diagonal cross-coupling elements.

- The equivalent circuit of CQBP element is shown in Figure 14.10.a.

- Details of a CQBP element are shown in Figure 14.10.b.

- Figure 14.10.c shows the version of CQBP element after inverters are replaced by inductive and capacitive equivalents. It is named as CQLC.

- The resistors in parallel with the shunt LC resonators of node-2 and node-3 of all CQ elements are used to account for the finite Q of the resonators (Lossy resonators).

Figure 14.10. CQBP and CQLC elements.

c) Complex Transmission Zeros and Linear Phase CQ sections:

Complex conjugate TZ quadruplet’s (sk=k jwk ) are used for phase linearization (group delay flattening) purposes.  In reference [23] it was shown that a complex transmission zero can be extracted in the form of a fourth order section as shown in Figure 14.11.a. It was also shown that such a fourth order section, being part of an eight order section as shown in Figure 14.11.b, is equivalent to a CQ section in which K14 has the same sign as the main line couplings. So, CQ sections may also be used for phase linearization purposes by proper selection of the sign of the cross-coupling element K14. In the present version of Filpro complex TZ’s can’t be placed into the transfer function yet. One reason is that locations for the complex TZ’s, sk = k jwk  necessitates a search and optimization process (especially k ) while the same results can also be obtained in an easier way by placing the cross-coupling by brute force and then tuning the coupling elements, as will be described in the next section.

Figure 14.11. Extraction of a complex TZ as a fourth order section and formation of a linear phase CQ section.

d) CQ Sections with Transmission Line Cross-Coupling and Linear Phase CQ sections:

In order to CQ sections using transmission line piece for 1-4 cross coupling on Filpro, an element named as CQTL is defined. Its circuit is shown in Figure 14.12.a. The cross-coupling element is an Inverter+Transmission Line (TL) combination. Inverter sets the coupling level while the transmission line mainly sets the phase of coupling signal. Practical realization constraints may prevent close placement of resonators 1 and 4 to get the necessary coupling. In such cases the physical length of the transmission line (TL) piece can be adjusted to couple the resonators.

- Figure 14.12.b shows a Pi-model of an inverter in terms of constant reactance elements. It is split into two equal halves and a transmission line piece is inserted in between as shown in Figure 14.12.c. The constant reactance elements can then be replaced by capacitors or inductors to get more realistic circuit models as shown in the figure. Such a combination can be used to couple the resonators 1 and 4 to mimic the following circuits:

- A CQ section with two symmetric FTZ’s can be simulated by setting the length of transmission line as zero and selecting the sign of K14 as opposite of the main line inverters (or opposite of the sign of K23).

- A linear phase CQ section can be simulated by setting the length of transmission line as zero and selecting the sign of K14 to be the same as the sign of main line inverters (or the same sign as  K23).

- For both signs of K14, one can tune the length of the transmission line while inspecting the Insertion Loss, Return Loss or Delay of S21 to move the FTZ locations and/or to flatten the delay.

- For some limited ranges of length of transmission line it is possible to move the two FTZ’s into the same stopband to form asymmetric responses, as will be described later in this chapter.

Figure 14.12. CQTL element.

In the succeeding sections the use of CT, CQ1F, CQBP, CQKR, CQKL and CQTL elements in coupled resonator filters will be described through examples with the number of resonators varying between three and ten.

14. 5. THREE RESONATOR CT FILTERS

In Filpro, cascaded triplet filters can be designed using Synthesis approach together with the circuit transformation tools that was described above. In this section, examples will be presented for the design of single triplet (three mutually coupled resonator) filters in several possible forms to get more insight.

By properly selecting the numbers of transmission zeros at f=0 and f=, triplets can be synthesized basically in two forms:

i ) The option with single transmission zero at f=0 and three transmission zero at f=.

ii ) The option with three transmission zeros at f=0 and single transmission zero at f=.

CT filters may be synthesized in All-Lumped, All-Stub or Mixed Lumped+Distibuted forms. Different forms can be converted to each other using circuit transformations and approximations. The suggested usual approach is to start the design in lumped element form.

Example 14. 1: A three resonator CT filter with a finite transmission zero in lower stopband

Let’s design a lumped element filter with the following specifications:

PB Ripple=0.1 dB,  fp1=846 MHz, fp2=848 MHz,

A finite transmission zero at fz=845 MHz.

Let’s select the option with one transmission zero at f=0 and three transmission zeros at f=: Nzero=1, Ninf =3.

Degree of the filter will be six (minimum degree for CT filters- having three resonators). Two degrees will come from the finite transmission zero while the other four will come from the transmission zeros Nzero=1, Ninf=3.

- Filter is designed using Synthesis approach, as described in Figure 14.13.

- Transmission zero sections are extracted in the order shown in the figure, resulting in the circuit of Figure 14.13.a. This circuit needs modifications to be put into the standard form for conversion into a triplet.

- With the cursor under any inner element, applying the command Pi-Tee-L / Equal Shunt Inductors, we get the circuit shown in Figure 14.13.b where there are now three shunt arm LC resonators with equal inductors. Equal inductance is not a necessity, but it may ease adjustment of element values in the later stages of design. The resulting circuit is in one of the standard forms for conversion into CT form. The circuit is saved as CT01.FIL for use later.

Capacitive Input-Output Coupling:

- Impedance level of the whole circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends. This is possible using a single command Inverters / Scale Circuit By Inserting Inverter / Specify Adjacent Inductor (or Specify Adjacent Cap or Specify K) / L=10 nH ”. Here Specify Adjacent Inductor option is selected to make all shunt inductors equal to 10 nH. The resulting circuit is shown in Figure 14.13.c. We may as well specify inverter constant or specify capacitance adjacent to the inverter to adjust element values.

- Placing the cursor under the series arm resonator and applying command CQ-CT Oprs / Convert to CT /CLL we get CLL type bridge Figure 14.13.d). The option CLL (Approx) provides a capacitive cross-coupling between Nodes-1 and 3, a solution which is preferred in placing finite transmission zero into the lower stopband of combline filters. As the warning indicates, this is an approximate solution where the actual 1-3 coupling comes out as a negative inductor and it is converted into positive capacitor by brute force.

- The transformer is absorbed into the adjacent inverter  by applying the command Inverters / Absorb TRF By Inv (Figure 14.13.e). This circuit is saved as CT02.FIL to be used at later stages.

- The ideal inverters can be replaced by many practical equivalents inverters, like capacitive or inductive input coupling or coupling to the input-output resonator bars through Tapping, etc. For example let’s use capacitive input-output couplings. This is possible by placing the cursor under the inverters and applying the command Inverters/ Replace Inverter By / End Inverter to / Capacitive L (Figure 14.13.f). Combining the negative shunt capacitors with the positive neighboring capacitors (using Simplify Circuit command), we get the final circuit shown in Figure 14.13.g. It is saved as CT03.FIL.

- External Q and Coupling Coefficients of the circuit can be displayed by pressing the keys Ctrl+F6. The coupling coefficients and resonance frequencies of the three resonators can be displayed by placing the cursor under the CT element and pressing key combination ALT+F6. Note that the resonance frequencies of the three resonators are all different (Figure 14.13.g).

- With cursor under any resonator or CT element, applying the command Parasitics / All Lossy Resonators / Q=1000, 2000, .. we can set the Q of resonators to study the effect of losses (Figure 14.13.h-i). The loss resistors of the resonators 1 and 3 are shown as Rp while the loss resistor  R2 of the resonator inside CT element at node-2 is in parallel with the elements L2-C2. The circuit is saved as CT04.FIL. Responses for different Q’s are shown in Figure 14.13.k.

- Voltage, Current and Power at each node at a specified frequency can be displayed by pressing the keys ALT+F8, selecting one of the options Magnitude-Angle or Real-Imaginary, and typing in the source voltage and frequency. Figure 14.13.m shows examples at several frequencies. The maximum voltage should be searched at frequencies near the passband edge which is closer to the finite transmission zero (sharper edge).

- Figure 14.13.n shows pictorial descriptions of the filter. The capacitive coupling is obtained by extending a conducting probe with nailheads.

Figure 14.13. A three resonator CT filter with a finite transmission zero in lower stopband.

Figure 14.13. A three resonator CT filter with a finite transmission zero in lower stopband (continued).

Figure 14.13. A three resonator CT filter with a finite transmission zero in lower stopband (continued).

Input-Output Couplings by Tapping Resonators:

- Figure 14.14 describes development of the Tapped Input-Output version of the filter. For this purpose, first the circuit is converted into mixed Lumped+Distributed Element form.  This is carried out by placing the cursor under the shunt LC resonators and applying the command Replace By Stub / Par LC-to-Cap+SC Stub. This command displays the passband edges of the filter and a default length for the stubs (resonator rods). The default length corresponds to the quarter wavelength frequency fq=2f0. The user can change this length. After typing in the length, the parallel LC resonator is replaced by its Capacitor+SC Stub equivalent (Figure 14.14a-b). The SC stub is the circuit model of the resonator rods (posts) short circuited at one ends and terminated in capacitors at the other ends.

- Next the cursor is moved to point the CT element and Replace By Stub command is applied again.   The center frequency, passband edges of filter and a default length for the rods (stubs) are displayed. Typing in the desired length we get the mixed element version of the CT section. In this transformation all the inductors of the CT section are replaced by SC stubs while capacitors are not affected. Since filter is narrowband, no variation is observed in response.

- Finally, with cursor under the inverters the command Tapped PCL is applied. This command displays the resonance frequency of the end sections and quarter wavelength frequency of the stubs as parameters that can be adjusted by the designer. Usually the displayed default values give the best result. In this transformation Filpro calculates the tapping distance to the SC end of the end resonators and adjust also the capacitors loading these resonators (14.14.c).

- Figure 14.14.d shows a pictorial description of the CT filter with taps.

Figure 14.14.e shows the responses. The return loss response is degraded slightly which can be corrected by tuning a few elements.

Figure 14.14. Tapped Resonator version of the CT filter.

Input-Output Couplings by Impedance Transforming Resonators:

- Figure 14.15 describes development of the CT filter with impedance transforming resonators (rods).

- The circuit of CT01.FIL is loaded and it is converted into CT section without impedance scaling (Figure 14.15.a-b).

- All shunt LC resonators are converted into SC Stub+Cap type resonators by applying Replace By Stub command (Figure 14.15.b-c). This command converts the lumped element CT into mixed element CT.

- The command Inverters / Scale Cct By Inserting Inverters / Specify Inverter: K=55 is applied to insert inverters at both ends (Figure 14.15.d). The value K=55 is just a trial value that will be iterated later until all resonators have equal impedances. A value close to 50 ohms is a good starting point.

- The inverters are replaced by transmission lines which are quarter wavelength long at f=2f0=2*847=1694 MHz by applying the command Inverters / Replace Inverter By / Quarter Wave TLine: f0=1694 MHz (Figure 14.15.e).

- With cursor under the transmission lines the command Kuroda / Distribute Stub is applied to move part of the shunt SC stub on one sides of the transmission lines to the other side to have equal stubs. {If we had more shunt SC stubs in the circuit we would also apply the command Kuroda / All Equal Stubs }.

- Impedance of the shunt SC stub inside the CT element needs special treatment for adjustment. With cursor under the CT element the command CQ-CT Oprs / Modify CT / Set Zs2 / Default: Zs2=52.865 ohms. Here 52.865 ohms is the impedance of the SC stub on the left of CT. This command sets Zs2 to the specified value, but it may disturb the impedances of other shunt SC stubs on the two sides of the CT element. All the stubs inside and adjacent to CT element can be equated by applying the same command on CT iteratively by specifying impedance of Zs2 to be equal to the impedance of the shunt SC stub on the left of CT (Figure 14.15.f-g).

The extra shunt SC stubs adjacent to source and load are named as transformer resonators or transformer rods or redundant resonators. Figure 14.15.h shows pictorial descriptions of the resulting CT filters. The transformer rods are not increasing the order of filter, hence no contribution on response. Therefore it may not always be a good idea to use this approach. They can be used whenever the other approaches for impedance scaling are difficult to implement. It is usually preferable to use the extra resonators in a contributing manner.

Figure 14.15. CT filter with impedance transforming resonators.

Figure 14.15. CT filter with impedance transforming resonators (cont’d).

Example 14. 2: All-Stub approach for a CT filter with a finite transmission zero in lower stopband.

Mixed element CT filters may also be designed by using Distributed Element approach leading to the same filter circuit. Just as exercise let’s design a distributed element filter with following specifications:

PBRipple=0.05, fp1=775 MHz, fp2=825 MHz, fq=1000 MHz with finite transmission zero at fz=725 MHz.

Let’s again select the option with one transmission zero at f=0 and three transmission zeros at f=: Nzero=1, Ninf=3.

- All the design commands are the same as the previous example, except the selection of option Distributed. We get the All-Stub CT shown in Figure 14.16.a.

- The command Pi-Tee-L / Equal Shunt SC Stubs converts the circuit into coupled shunt resonator form with resonators formed by parallel OC and SC stubs (Figure 14.16.b).

- The circuit can be converted into CT form by applying the command CQ-CT Oprs / Convert to CT / CLL (14.16.c). The CLL choice produces a CT section with an OC stub bridging Nodes 1 and 3 (Zo13), and SC stubs between Nodes 1-2 and 2-3 (Zs12, Zs23). The approximate nature of CLL type solution shows itself in more pronounced way for the distributed element case, as is seen from the insertion loss plots of Figure 14.16.e before and after conversion to CT. The finite transmission zero of the original circuit is shifted appreciably. Depending on many other parameters of the filter, the FTZ may even be lost. This is because the negative SC stub between Node-1 and Node-3 is replaced by a positive OC stub, which may be a poor approximation. We can recover the response by tuning some of the parameters.

- All the shunt OC+SC stub elements are converted into Cap+SC Stub form by applying the command Replace By Lumped / All SC Stub+Cap  (Figure 14.16.d-e). This command converts also the Distributed Element CT into Mixed CT. This action will also effect response to some extent.

- It is usually desirable to have equal shunt SC stubs. In general this is achieved by applying the command Pi-Tee-L / All Equal SC Stubs. However since this is a low order filter this command is ineffective. We notice that the shunt SC stubs outside the CT are about 3.7058 ohms while the shunt SC stub inside the CT is Zs2=3.5849 ohms. Zs2 can be made equal to 3.7058 ohms by placing the cursor under the CT and applying the command CQ-CT Oprs / Modify CT / Set Zs2 / Default: Zs2=3.7058 ohms. This command will also change the SC stubs adjacent to the CT slightly. Therefore it may be necessary to repeat this command by specifying the impedance of the SC stub on the left of   the CT. All SC stubs can be equated in a few iteration (Figure 14.16.f-g). If necessary we may also use the options CQ-CT Oprs / Modify CT / Set Zs1 (on left of CT) or Set Zs3 (on right of CT), etc.

- Impedance scaling can be done in several ways as described in the previous examples. For example let’s scale the circuit by using capacitive input-output couplings. Applying the command Inverters / Scale Circuit By Inserting Inverters / Specify SC Stub: Z=80 ohms we can set the shunt SC stub impedances to 80 ohms (for example) (Figure 14.16.h-i).

- The inverter at load end can be used to absorb the transformer there by using the command Inverters / Absorb TRF (Figure 14.16.k).

- Then the inverters are replaced by their capacitive-L equivalents by using the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L (Figure 14.16.m).

- Simplify Circuit command combines the negative shunt capacitors with the adjacent positive capacitors (Figure 14.16.n).

- In Figure 14.16.p the External Q, Coupling Coefficients and Resonance Frequencies are displayed by using the keys CTRL+F6 and ALT+F6.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys.

Exercise: Use Replace By Lumped command to convert the mixed element circuit to lumped circuit and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach as described in the previous example.

Exercise: Scale impedance level by Transformer Resonator approach as described in the previous example.

Figure 14.16. Design of a mixed CLL type CT filter using Distributed Element approach.

Figure 14.16. Design of a mixed CLL type CT filter using Distributed Element approach.

Figure 14.16. Design of a mixed CLL type CT filter using Distributed Element approach.

# Example 14. 3: CCC Type CT filter with a low side transmission zero

When the finite transmission zero is in lower stopband, it is possible to get all couplings as capacitive by placing three transmission zeros at f =0 and a single transmission zero at f= , as described below.

Let’s design a lumped element filter with the specifications:

PB Ripple=0.1 dB,  fp1 =846 MHz, fp2=848 MHz,

A finite transmission zero at f=845 MHz.

- The element extraction order is shown in Figure 14.17.a, with the resulting circuit shown in Figure 14.17.b.

- With the cursor under the middle shunt element, applying the command Pi-Tee-L/All Equal Shunt/ Inductors, we get the circuit of Figure 14.17.c. This circuit is in standard form for conversion into bridge tee.

- Placing the cursor under the series arm parallel resonator, and applying the command CQ-CT Oprs / Convert to CT / Three Equal Shunt, we have the choice of  CCC type or LCC type bridge. Selecting CCC type we get we get the circuit shown in Figure 14.17.d (all capacitive couplings).

- Simplify Circuit command eliminates transformers, yielding a completely symmetric circuit (Figure 14.17.e).

- Element values may be adjusted by inserting inverters as is done in the previous example (Figure 14.17. f).

Figure 14.17. Development of a single triplet filter with all capacitive couplings having a finite transmission zero in lower stopband.

Example 14. 4: CT filters with a finite transmission zero in upper stopband

Let’s design a filter with the following specifications: PB Ripple=0.1 dB,  fp1=846 MHz, fp2=848 MHz, a finite transmission zero at f=849 MHz (in upper stopband).

This filter may also be designed either by placing a single transmission zero at zero and three transmission zeros at infinity, leading to all inductive couplings, or three transmission zeros at zero and single transmission zero at infinity, leading to mixed couplings. In cavity type filters it is easier to implement all inductive couplings. Therefore its design is described below. The other solutions can be obtained in the same way.

- Filter is designed using Synthesis approach, as was done in the previous examples giving the circuit of Figure 14.18.a.

- With cursor under any element application of the command Pi-Tee-L / All Equal Shunt / Inductors yields a circuit having three coupled shunt resonators with equal shunt inductors (Figure 14.18.b).

- This circuit has the standard form for conversion into CT section. Placing the cursor under the series arm parallel resonator and applying command CQ-CT Operations / Convert to CT produces the LLL type CT section (Figure 14.18.c). It should be noted that for this case there is no option to select.

- Element values of the circuit may be adjusted to desired level by the techniques described in Example 14.1. In Figure 14.18.d-e the approach for capacitive coupling is initiated where inverters are adjusted to make all shunt inductors equal at 10 nH. Then these inverters are replaced by their Capacitive-L section.  The coupling coefficients, External Q, Resonance Frequencies, lists of Node Voltage-Current-Power and effects of finite Q of resonators can be displayed as described in Example 14.1.

- All-Stub CT and Mixed CT filters can also be designed as in Example 14.2.

Exercise: Use Nzero=3 and Ninf=1 to design CT filters of LCC and LCL type.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.18. An LLL type CT filter with a finite transmission zero in upper stopband.

14. 4. FOUR RESONATOR CT AND CQ FILTERS

Four resonators can be used to design a filter which contains one CT section or one CQ1F section or one CQBP or one Generalized CQ section depending on the requirements. Let’s carry out one design for each case.

EXAMPLE 14. 5. A four resonator filter with a CT section

Let  PBRipple= 0.01 dB,  fp1=846 MHz, fp2=848 MHz,

A finite transmission zero is placed at f=845 MHz.

There should be a total of six transmission zeros at f=0 and f=. These transmission zeros may be shared in different ways to produce a variety of solutions. Let’s try the case Nzero=1 and Ninf=5 to get more inductive couplings which are easier to implement.

- Design command set is shown in Figure 14.19. One element extraction order and resulting circuit is shown in Figure 14.19.a.

- Application of the command Pi-Tee-L/Equal Shunt Inductors produces the circuit of Figure 14.19.b.

- Using the command CQ-CT Oprs / Convert to CT / Left / CLL converts the circuit section on left half into CLL type CT section (Figure 14.19.c). We could as well convert the right half into a CT section. The rightmost resonator is simply coupled to the CT section.

- It is seen that equality of the shunt inductors is lost. We can equate them again by applying the command Pi-Tee-L / Pi-to-Sym Pi command to the Pi-section on the right (Figure 14.19.d).

- Figures 14.17.e-f-g show the classical element value adjustment steps for capacitive input-output couplings. Tapped version or the version with impedance transforming resonators may also be used as described in Example 14.1.

- The External Q and Coupling Coefficients can be displayed by pressing the keys CTRL+F6 and selecting the option Direct Cpld / CQ-CT. The Coupling Coefficients can also be displayed together with resonance frequencies of the resonators by placing the cursor under the CT element and pressing the keys ALT+F6 (Figure 14.19.g).

- The node voltages, currents and powers can be displayed by pressing the keys ALT+F8.

- The effects of finite Q can be studied by placing the cursor under any resonator and applying the command Parasitics / All Lossy Resonators: f0=847 MHz, Q=…  (Figure 14.19.h-i).

- Figure 14.19.k shows a pictorial description of resonators.

- All-Stub CT and Mixed CT filters can also be designed as in Example 14.2.

Exercise: Find the frequency and the node at which maximum voltage and power appears by using ALT+F8 command.

Exercise: Repeat the design by placing a FTZ in upper stopband.

Exercise: Repeat the design by using Nzero=3 and Ninf=3.

Exercise: Repeat the design by using Nzero=5 and Ninf=1.

Exercise: Repeat the design by using tapped input-output couplings.

Exercise: Repeat the design by using impedance transforming resonators.

Exercise: Repeat the designs using all-stub approach.

Exercise: Repeat the design for wider bandwidth filters.

Figure 14.19. Development of a four resonator filter with a single triplet.

Figure 14.19. Development of a four resonator filter with a single triplet.

EXAMPLE 14. 6. A four resonator filter with a CQ1F section

A single finite transmission zero may also be created by using a special quadruplet section named as CQ1F in Filpro. Let’s design the previous filter in CQ1F form:

PBRipple= 0.01 dB,  fp1=846 MHz, fp2=848 MHz,

A finite transmission zero is placed at f=845 MHz.

Nzero=1, Ninf=5

- The design command set is the same as in the previous example giving the same circuit (Figure 14.20.a).

- Application of the command Pi-Tee-L/Equal Shunt Inductors produces the circuit of Figure 14.20.b.

- With cursor under the FTZ element the command CQ-CT Oprs / Convert to CQ / Solution-1 (Solution-2) is applied to give the circuits shown in Figure 15.18.c. Solutions are actually identical with top and bottom elements interchanged and the positions of negative inverters interchanged. Each solution has a negative inverter which means capacitive coupling. Let’s consider Solution-1 only.

- The circuit can be scaled by using the same techniques as described in Example-1. That is, they can be scaled by using capacitive input-output or by Tapping or by using Impedance Transforming resonators. Figures 14.18.d-e-f-g describe the approach using impedance scaling by inverters which are then replaced by capacitive L-sections.

- External Q and Coupling Coefficients can be displayed by pressing the keys CTRL+F6 and selecting the option Direct Cpld-CQ-CT. The Coupling Coefficients can also be displayed together with resonance frequencies of resonators by placing the cursor under the CQ1F element and pressing the keys  ALT+F6 (Figure 14.20.g).

- Effects of finite Q of resonators can be inspected by forming the lossy version of the filter by placing the cursor under any resonator and applying the command Parasitics / All Lossy Resonators (Figure 14.20.h-i).

- The node voltages, currents and powers can be displayed at selected frequencies by pressing the keys ALT+F8.  Figure 15.16.k shows that the maximum voltage point occurs at a frequency very close to the lower passband edge.

- Figure 14.20.m shows a pictorial description of the filter. The capacitive coupling is obtained by extending a conducting feedthru probe with nailheads. Note that the couplings are either in vertical or horizontal direction which may be easer to  implement compared to CT filters.

Exercise: Repeat the design for a filter with Nzero=3 and Ninf=3. Note the numbers of positive and negative (capacitive) couplings.

Exercise: Repeat the design for a filter with Nzero=5 and Ninf=1. Note the numbers of positive and negative (capacitive) couplings.

Exercise: Repeat the designs for a filter with a FTZ in upper stopband.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Figure 14.20. Design of a four resonator filter with a CQ1F section.

Figure 14.20. Design of a four resonator filter with a CQ1F section (cont’d).

Figure 14.20. Design of a four resonator filter with a CQ1F section (cont’d).

EXAMPLE 14. 7. A four resonator CQ filter with asymmetric finite transmission zero pair

Four resonators can be used more efficiently to produce a pair of finite transmission zeros (FTZ’s). If the two FTZ’s are arbitrarily located then we will get a diagonal cross-coupling between two of the resonators. If the FTZ’s have geometric symmetry (fz1*fz2=f02) then there will be no diagonal cross-coupling (to be studied in the next example).

Four resonators form up a degree 8 filter. Four of the degrees will be coming from the two FTZ’s. Therefore the remaining four degrees will be provided by the transmission zero at f=0 and f=. Let’s inspect two cases:

- PBRipple=0.01 dB, Nzero=1, Ninf=3, fz1=845 MHz, fz2=851 MHz.

- PBRipple=0.01 dB, Nzero=3, Ninf=1, fz1=845 MHz, fz2=851 MHz.

Design command set for both cases are shown in Figure 14.21. It is seen that no difference exists between responses. This is because the filter is a very narrowband one. Slight differences would show up for wider bandwidth cases.

Case-a:

  - The transmission zeros must be extracted in the order inf-inf-FTZ1-FTZ2--zero-inf  or inf-inf-FTZ1-FTZ2-inf-zero to form a proper circuit section for conversion into CQ form. We get the circuit of Figure 14.21.a.

  - Applying the command Pi-Tee-L / Equal Shunt Inds we get the circuit of Figure 14.21.b.

  - With cursor under the shunt resonator adjacent to the first FTZ section, the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections into adjacent position as shown in Figure 14.21.c.

  - Simplify Circuit (F6) command moves the transformer away and combines the adjacent shunt resonators (Figure 14.21.d).

  - The two adjacent FTZ elements are combined to form a single fourth order element (Figure 14.21.e).

  - With cursor under the fourth order section the command CQ-CT Oprs / Convert to CQ / Solution-1 (2, 3, 4) is applied to yield four possible solutions as shown in Figure 14.21.f. The resulting CQ element is named as CQKR. Here K means that it contains inverters as coupling elements. R means that there is a diagonal cross-coupling tilted Right (between nodes 2 and 4).

Case-b:

  - Exactly the same command sets are used to produce similar circuits are produced for this case also. The four possible solutions are shown in Figure 14.21.g.

  - So, we have overall eight possible solutions. All solutions have one or three negative (capacitive) coupling inverters. The location of negative inverter changes in each solution.

  - The diagonal cross-coupling is between nodes 2 and 4. In Solutions 1 and 2 it is capacitive  while in Solutions 3 and 4 it is inductive.

  - In Figure 14.21.h the External Q, Coupling Coefficients and Resonance Frequencies are displayed using the keys CTRL+F6 and ALT+F6.

  - The circuits can be scaled by using the same techniques as described in Example-1. That is, they can be scaled by using capacitive input-output or by Tapping or by using Impedance Transforming resonators. Figures 14.19.i-k-m describe the approach using impedance scaling by inverters for Case-a, Solution-1. The inverters are then replaced by capacitive L-sections.

  - Effects of finite Q of resonators can be inspected by forming the lossy version of the filter by placing the cursor under any resonator and applying the command Parasitics / All Lossy Resonators (Figure 14.21.m-n-p).

  - The node voltages, currents and powers can be displayed at selected frequencies by pressing the keys ALT+F8.  Figure 14.21.q shows one example.

  - Figure 14.21.r shows a pictorial description of the filter. The capacitive couplings are obtained by extending a conducting feedthru probe with nailheads.

Figure 14.21. A four resonator CQ filter with asymmetric FTZ’s.

Figure 14.21. A four resonator CQ filter with asymmetric FTZ’s (cont’d).

Figure 14.21. A four resonator CQ filter with asymmetric FTZ’s (cont’d).

Figure 14.21. A four resonator CQ filter with asymmetric FTZ’s (cont’d).

EXAMPLE 14. 8. A four resonator CQ filter with symmetric finite transmission zero pair

A CQ section without a diagonal cross-coupling can realize two symmetrically located finite transmission zeros. Here the symmetry condition can be taken as fz1*fz2=fp1*fp2 or fz1*fz2=f02.

Four resonators form up a degree 8 filter. Four of the degrees will be coming from the two FTZ’s. Therefore the remaining four degrees will be provided by the transmission zero at f=0 and f=. Let’s inspect a filter with following specs:

PBRipple=0.01 dB, Nzero=1, Ninf=3, fz1=845 MHz, fz2=849 MHz.

Design commands are the same as the previous case and are shown in Figure 14.22.

  - The transmission zeros are extracted in the order inf-inf-FTZ1-FTZ2--zero-inf giving the circuit of Figure 14.22.a.

  - Applying the command Pi-Tee-L / Equal Shunt Inds we get the circuit of Figure 14.22.b.

  - With cursor under the shunt resonator adjacent to the first FTZ section, the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections into adjacent position. Simplify Circuit (F6) command moves the transformer away and combines the adjacent shunt resonators (Figure 14.22.c).

  - The two adjacent FTZ elements are combined to form a single fourth order element (Figure 14.22.d).

  - With cursor under the fourth order section the command CQ-CT Oprs / Convert to CQ / Solution-1 (2, 3, 4) is applied to yield four possible solutions as shown in Figure 14.22.e. Solution-1 is of special practical importance because it is the easiest structure to implement. Therefore a special name is assigned to the element as CQBP. The other three solutions are obtained in the form of CQKR elements with ineffective diagonal cross-couplings. We will continue the discussion with Solution-1.

  - External Q, Coupling Coefficients and Resonance Frequencies are displayed using the keys CTRL+F6 and ALT+F6 (Figure 14.22.f).

  - Inverters of the CQBP section can be replaced by lumped inductors and capacitors by applying the command  CQ-CT Oprs / CQInv-to-CQLC (Figure 14.22.g-h).

  - The circuit can be scaled by using the same techniques as described in Example-1. That is, they can be scaled by using capacitive input-output or by Tapping or by using Impedance Transforming resonators. Figures 14.20.h-i-k describe the approach using impedance scaling by inverters and then replacing inverters by capacitive L-sections.

  - Effects of finite Q of resonators can be inspected by forming the lossy version of the filter by placing the cursor under any resonator and applying the command Parasitics / All Lossy Resonators (Figure 14.22.k-m-n).

  - The node voltages, currents and powers can be displayed at selected frequencies by pressing the keys ALT+F8.  Figure 14.22.p shows one example.

  - Figure 14.22.q shows a pictorial description of the filter.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Figure 14.22. Design of a CQ filter with symmetric FTZ’s.

Figure 14.22. Design of a CQ filter with symmetric FTZ’s (continued).

Figure 14.22. Design of a CQ filter with symmetric FTZ’s (continued).

14. 5. DESIGN OF FIVE RESONATOR CT-CQ FILTERS

Five resonators can be coupled in several ways to form different CT and CQ sections. Further, we may increase the number of alternatives by selecting different numbers of transmission zeros at zero and infinity. Some of the alternatives may be more advantageous for realization of element values. Therefore it may be necessary to survey all the possible alternatives. Let’s see some examples.

Example 14. 7: Five resonator CQ1F filters with a finite transmission zero in lower stopband

CQ1F quadruplets are used to create a single finite transmission zero. A five resonator filter can accommodate one CQ1F section and a simply coupled resonator.

Let’s design a filter with the following specifications:

PB Ripple=0.01 dB,  fp1 =846 MHz, fp2=848 MHz,

One finite transmission zero is placed at f=845 MHz in lower stopband. Five resonators make up a total degree of 10. Two degrees will be coming from the FTZ. The remaining eight degrees will be provided by Nzero and Ninf. Various Nzero-Ninf combinations are possible. Let’s study one case:

Nzero=1, Ninf=7.

- Design command set is shown in Figure 14.23. One possible extraction order gives the circuit shown in Figure 14.23.a.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.19.b.

- Placing the cursor under the FTZ element and applying command CQ-CT Operations/ Convert to CQ / Solution-1 we get CQ1F section. (Figure 14.23.c).

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends, as was done in the previous examples.

- Figure 14.23.d shows a pictorial description of a possible implementation.

Exercise: Repeat the design by extracting TZ’s in different orders and selecting the option Solution-2.

Exercise: Repeat the design by placing the FTZ in upper stopband.

## Exercise: Repeat the designs for Nzero=7 and Ninf=1.

Exercise: Repeat the designs for other Nzero-Ninf combinations.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.23. Design of a five resonator filter having a CQ1F section.

## Example 14. 8: Five resonator CT filters with two finite transmission zeros in lower stopband

Let’s design a filter with the following specifications:

PB Ripple= 0.01 dB,  fp1 = 846 MHz, fp2=848 MHz,

Two finite transmission zeros are placed at f=844.3 and f=845 MHz in lower stopband to get equal stopband ripples. The two finite transmission zeros will be realized by two triplets. Each triplet, when they are alone, has degree six, two of which comes from the finite transmission zero and four from the transmission zeros at f=0 and f=. However, when two triplets are cascaded directly, one resonator will be common to both triplets. Thus, the total filter degree will be 10, four of which will be due to the finite transmission zeros and six will be due to transmission zeros at f=0 and f=. The two triplets can be formed in different ways, by placing different numbers of transmission zeros at zero and infinity. Two cases are  described below:

a)  Solutions for Nzero=1, Ninf=5

Since the finite transmission zeros are in lower stopband, by placing more transmission zeros at infinity we will get CT sections with mixed inductive-capacitive couplings.

- Design steps are shown in Figure 14.24. One extraction order leads to the circuit of Figure 14.24.a.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.24.b.

- Placing the cursor under the first series arm parallel resonator and applying command CQ-CT Operations/ Convert to CT / Left /CLL we get CLL type CT section. Here, the options Left-Right exist because the triplets can be formed by the elements on the left or on the right of the FTZ element. The left option should be selected because the elements on the right will be used for the second triplet. Repeating the same command on the second FTZ element we get the CT filter of Figure 14.24.c.

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends, as was done in the previous examples.

- Figure 14.24.d shows a pictorial description of a possible implementation.

Exercise: Repeat the design by extracting transmission zeros in different orders and by selecting different CT sections

b)  Solutions for Nzero=5, Ninf=1

Since the finite transmission zeros are in lower stopband, bu placing more transmission zeros in lower stopband it is possible to get all capacitively coupled  CT sections.

- Design steps are shown in Figure 14.25. One extraction order leads to the circuit of Figure 14.25.a.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.25.b.

- Placing the cursor under the first series arm parallel resonator and applying command CQ-CT Operations/ Convert to CT / Left we get CCC type CT section. Here, the options Left-Right exist because the triplets can be formed by the elements on the left or on the right of the FTZ element. The left option should be selected because the elements on the right will be used for the second triplet. Repeating the same command on the second FTZ element we again get CCC type CT section (Figure 14.25.c).

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends, as was done in the previous examples.

- Figure 14.25.d shows a pictorial description of a possible implementation.

Exercise: Repeat the design by extracting transmission zeros in different orders.

Exercise: Repeat the design by using different Nzero-Ninf  combinations.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.24. A five resonator CT filter with Nzero=1, Ninf=5 and two finite transmission zeros in lower stopband.

Figure 14.25. A five resonator CT filter with Nzero=5, Ninf=1 and two finite transmission zeros in lower stopband.

Example 14. 9: Five resonator CT filters with two finite transmission zeros in upper stopband

Let’s design a filter with the following specifications:

PB Ripple= 0.01 dB,  fp1 = 846 MHz, fp2=848 MHz,

Two finite transmission zeros are placed at f=849.2 and f=850 MHz in upper stopband to get equal stopband ripples. Different Nzero-Ninf combinations can be used for the design. Two cases are described below:

a)  Solutions for Nzero=1, Ninf=5

Since the finite transmission zeros are in upper stopband, placing more transmission zeros at infinity it is possible to form CT sections with all inductive couplings.

- Design steps are shown in Figure 14.26. One extraction order leads to the circuit of Figure 14.26.a.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.26.b.

- Placing the cursor under the first series arm parallel resonator and applying command CQ-CT Operations/ Convert to CT / Left /LLL we get LLL type CT section. Here, the options Left-Right exist because the triplets can be formed by the elements on the left or on the right of the FTZ element. The left option should be selected because the elements on the right will be used for the second triplet. Repeating the same command on the second FTZ element we get the second CT section of LLL type (Figure 14.26.c).

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends, as was done in the previous examples.

- Figure 14.26.d shows a pictorial description of a possible implementation.

Exercise: Repeat the design by extracting transmission zeros in different orders.

b)  Solutions for Nzero=5, Ninf=1

Since the finite transmission zeros are in upper stopband, by placing more transmission zeros in lower stopband the resulting CT sections will have mixed inductive-capacitive couplings.

- Design steps are shown in Figure 14.27. One extraction order leads to the circuit of Figure 14.27a.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.27.b.

- Placing the cursor under the first series arm parallel resonator and applying command CQ-CT Operations/ Convert to CT / Left we get CCC type CT section. Here, the options Left-Right exist because the triplets can be formed by the elements on the left or on the right of the FTZ element. The left option should be selected because the elements on the right will be used for the second triplet. Repeating the same command on the second FTZ element we again get CCC type CT section (Figure 14.27.c).

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends, as was done in the previous examples.

- Figure 14.27.d shows a pictorial description of a possible implementation.

Exercise: Repeat the design by extracting transmission zeros in different orders.

Exercise: Repeat the design by using different Nzero-Ninf  combinations.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.26. A five resonator CT filter with Nzero=1, Ninf=5 and two FTZ’s in upper stopband.

Figure 14.27. A five resonator CT filter with Nzero=5, Ninf=1 and two FTZ’s in upper stopband.

Example 14. 10: Five resonator CQ filters with two finite transmission zeros in lower stopband

Let’s design a filter with the following specifications:

PB Ripple= 0.01 dB,  fp1 = 846 MHz, fp2=848 MHz,

Two finite transmission zeros are placed at f=844.3 and f=845 MHz in lower stopband. The total filter degree is 10, four of which will be due to the finite transmission zeros and six will be due to transmission zeros at f=0 and f=. The two triplets can be formed in different ways, by placing different numbers of transmission zeros at zero and infinity. One case is described below:

Solution for Nzero=1, Ninf=5

- Design steps are shown in Figure 14.26. One extraction order leads to the circuit of Figure 14.28.a.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.28.b.

- With cursor under the shunt resonator adjacent to the FTZ element the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements (Figure 14.28.c).

- The adjacent FTZ elements are combined to form a fourth order section (Figure 14.28.d).

- Placing the cursor under the fourth order section and applying command CQ-CT Operations/ Convert to CQ / Left / Solution-1 we get CQKR element (Figure 14.28.e). Here, the options Left-Right exist because the quadruplet can be formed by the elements on the left or on the right of the fourth order section. Equality of the shunt inductors is usually lost after conversion into CQ form. This property can be recovered by applying the command Pi-Tee-L / Pi-to-Sym Pi to the Pi-section on right.

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends, as was done in Example 14.7.

- Figure 14.28.f shows a pictorial description of a possible implementation. Note that in this solution only the diagonal cross-coupling came out to be capacitive.

Exercise: Repeat the design by extracting transmission zeros in different orders.

Exercise: Repeat the design for Nzero=5 and Ninf=1.

Exercise: Repeat the design for two transmission zeros in upper stopband.

Exercise: Repeat the design for two asymmetric finite transmission zeros, fz1=845 and fz2=851 MHz.

Exercise: Repeat the design for two symmetric finite transmission zeros, fz1=845 and fz2=849 MHz to get a CQBP section.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.28. A five resonator filter involving CQ element with two FTZ’s in lower stopband.

14. 6. DESIGN OF SIX RESONATOR CT AND CQ FILTERS

Six resonators can accommodate two CT sections or one CQ section. Let’s see some examples.

Example 14.11. Six resonator CQ1F  filter with a single FTZ in upper stopband

Let’s design a filter with the following specifications:

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

A FTZ is placed in upper stopband, at f=849 MHz

Six resonators form a degree 12 filter. One FTZ provides two degrees. The remaining ten degrees will be coming from the transmission zeros at f=0 and f=. These transmission zeros may be shared between f=0 and f= in different ways, as was done in the previous examples, yielding different equivalent circuits.

Let’s consider only the case with Nzero=1 and Ninf=9. The other cases may be obtained through similar steps (Exercise?).

- Design command set is shown in Figure 14.29 giving the circuit of Figure 14.29.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.29.b.  which is ready to conversion into CQ1F form.

- With cursor under the left FTZ element the command CQ-CT Oprs / Convert to CQ / Solution-1 is applied to convert the central part of the circuit into a CQ1F section. (Figure 14.29.c).

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.6).

- Figure 14.29.d shows a pictorial description.

Exercise: Repeat the design by extracting transmission zeros in different orders.

Exercise: Repeat the design for Nzero=9 and Ninf=1.

Exercise: Repeat the design for a transmission zeros in lower stopband.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.29. Six resonator CQ1F filter with a FTZ in upper stopband.

Example 14.12. Six resonator CT filters with two finite transmission zeros in lower stopband

Let’s design a filter with the following specifications:

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

Two finite transmission zeros in lower stopband, at f=844.9 and f=845.3 MHz.

Degree 12 filters need a total of eight transmission zeros at f=0 and f=. These transmission zeros may be shared between f=0 and f= in different ways, as was done in the previous examples, yielding a variety of equivalent circuits.

Let’s consider only the case with Nzero=1 and Ninf=7. The other cases may be obtained through similar steps (Exercise?). Several solutions are possible by extracting the transmission zeros in different orders. One case is described below.

- Design command set is shown in Figure 14.30 giving the circuit of Figure 14.30.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.30.b which is ready to conversion into CT form.

- With cursor under the left FTZ element the command CQ-CT Oprs / Convert to CT / Left / CLL is applied to convert the left half of the circuit into a CT section with capacitive cross coupling. Then with cursor under the right FTZ element the command CQ-CT Oprs / Convert to CT / CLL is applied completing the conversion (Figure 14.30.c).

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.1).

- Figure 14.30.d shows a pictorial description. Note the inductive coupling between the two trisections. Compared to the two-CT filter of Example 14.9, this new structure is simpler to handle because the middle CT section of Example 14.9 was loaded by three coupling holes to neighboring resonators.

Exercise: Repeat the design by extracting transmission zeros in different orders.

Exercise: Repeat the design for Nzero=7 and Ninf=1.

Exercise: Repeat the design for two transmission zeros in upper stopband.

Exercise: Repeat the design for finite transmission zeros, fz1=845 and fz2=851 MHz.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.30. A six resonator CT filter with two finite transmission zeros in lower stopband.

Example 14. 13: Six resonator CQ filters with a symmetric pair of finite transmission zeros

Let’s design a filter with the following specifications:

PB Ripple= 0.01 dB,  fp1 = 846 MHz, fp2=848 MHz,

Two finite transmission zeros are placed symmetrically about the passband f=845 and f=849 MHz to get a CQBP element. The total filter degree is 12, four of which will be due to the finite transmission zeros and the remaining eight degrees will be due to transmission zeros at f=0 and f=. The quadruplet can be formed in different ways, by placing different numbers of transmission zeros at zero and infinity and by extracting elements in different orders. One case is described below:

Solution for Nzero=1, Ninf=7

- Design steps are shown in Figure 14.31. The extraction order shown leads to the circuit of Figure 14.31.a.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.31.b.

- With cursor under the shunt resonator adjacent to the FTZ element the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements (Figure 14.31.c).

- The adjacent FTZ elements are combined to form a fourth order section (Figure 14.31.d).

- Placing the cursor under the fourth order section and applying command CQ-CT Operations/ Convert to CQ / Left / Solution-1 we get CQBP element (Figure 14.31.e). Here, the options Left-Right exist because the quadruplet can be formed by the elements on the left or on the right of the fourth order section.

- Equality of the shunt inductors is usually lost after conversion into CQ form. This property can be recovered by applying the command Pi-Tee-L / Equal Shunt Inds (Figure 14.31.f).

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends, as was done in Example 14.1. In Figure 14.31.f, the External Q, Coupling Coefficients and Resonance Frequencies are displayed.

- Figure 14.31.g shows a pictorial description of a possible implementation. Note the symmetry.

Exercise: Repeat the design by extracting transmission zeros in different orders.

Exercise: Repeat the design for Nzero=7 and Ninf=1.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.31. A six resonator CQ filter with symmetric transmission zeros.

Figure 14.31. A six resonator CQ filter with symmetric transmission zeros.

## Example 14. 14: Six resonator CT-CQ filters with three finite transmission zeros

The six resonators may be arranged to form a CQ and a CT section to create three FTZ’s as follows.

Let’s design a filter with the following specifications:

PB Ripple= 0.01 dB,  fp1 = 846 MHz, fp2=848 MHz,

Two finite transmission zeros are placed symmetrically about the passband at fz1=845 and fz2=848.95 MHz to get a CQ element without a diagonal cross-coupling (CQBP). A third FTZ is placed in upper stopband at fz3=849.5 MHz for higher selectivity on upper skirt. The total filter degree is 12, six of which will be due to the finite transmission zeros and the remaining six degrees will be due to transmission zeros at f=0 and f=. The quadruplet can be formed in different ways, by placing different numbers of transmission zeros at zero and infinity and by extracting elements in different orders. One case is described below:

Solution for Nzero=1, Ninf=5

- Design steps are shown in Figure 14.32. The extraction order is set to form proper circuit sections which can be converted into CQ and CT sections. The TZs Inf-Inf-FTZ1-FTZ2 will lead to a CQ section and the TZ’s  Inf-Inf-FTZ3 will lead to CT section.  Figure 14.32.a shows one extraction order.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.32.b.

- With cursor under the shunt resonator adjacent to the FTZ element the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements (Figure 14.32.c). Also with cursor under the rightmost FTZ element the command CQ-CT Oprs / Convert to CT is applied to form the CT section.

- The adjacent FTZ elements are combined to form a fourth order section (Figure 14.32.d).

- Placing the cursor under the fourth order section and applying command CQ-CT Operations/ Convert to CQ / Solution-1 we get CQBP element (Figure 14.32.e). In general we get a CQKR element. That is, we usually get a diagonal cross-coupling because in the presence of other FTZ’s, the symmetry conditions fz1*fz2=fp1*fp2 or fz1*fz2=f02 may not guarantee elimination of the diagonal cross-coupling. In such cases we eliminate diagonal cross-coupling by adjusting either fz1 or fz2 by trials. In this example fz2=848.95 MHz is found through such trials by keeping fz1=845 MHz fixed.

- Equality of the shunt inductors is usually lost after conversion into CQ-CT form. This property can be recovered by applying the command CQ-CT Oprs / Modify CQ or Modify CT whichever necessary. In this circuit the shunt inductor on right of CT is equated to the one on left (Figure 14.32.e-f).

- In Figure 14.32.f, the External Q, Coupling Coefficients and Resonance Frequencies are displayed.

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends. Then the inverters can be replaced by practical equivalents, as was done in Example 14.1. Figure 14.32.g-h-i describe capacitive input-output case.

- Figure 14.32.k shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting transmission zeros in different orders.

Exercise: Repeat the design for Nzero=5 and Ninf=1.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.32. A six resonator  CT-CQ filter.

Figure 14.32. A six resonator CT-CQ filter (cont’d).

14. 7. DESIGN OF SEVEN RESONATOR CT-CQ FILTERS

Seven resonators can be used to form two CQ sections to realize two, three or four FTZ’s or they can be used to form three CT sections to realize three FTZ’s as will be described in the examples.

Example 14. 15. Seven resonator CQ1F filter with two FTZ’s in upper stopband

Let’s design a degree 14 filter with the following specifications:

PB Ripple= 0.01 dB,  fp1 = 846 MHz, fp2 = 848 MHz,

Two finite transmission zeros in upper stopband, at f=849 MHz and f=849.5 MHz. Seven resonators make up a degree fourteen filter. Four degrees will be coming from the two FTZ’s and the remaining 10 degrees will be provided by TZ’s at f=0 and f=. Let’s consider the case with Nzero=1 and Ninf=9.

- Design command set is shown in Figure 14.33 giving the circuit of Figure 14.33.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.33.b.  which is ready to conversion into CQ1F form.

- With cursor under the FTZ elements the command CQ-CT Oprs / Convert to CQ / Solution-1 is applied to convert the circuit sections into CQ1F sections (Figure 14.33.c).

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.6).

- Figure 14.33.d shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting transmission zeros in different orders.

Exercise: Repeat the design for Nzero=9 and Ninf=1.

Exercise: Repeat the design for two transmission zeros in lower stopband.

Exercise: Repeat the design for one transmission zero in both lower and upper stopbands.

Exercise: Repeat the design for different Nzero-Ninf combinations.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.33. A seven resonator CQ1F filter.

Example 14.16. Seven resonator CT filters with three finite transmission zeros

Using seven resonators three CT sections can be formed each realizing one FTZ. Let’s design a filter with

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

One FTZ is place in lower stopband, at fz1=844.87 MHz. Two finite transmission zeros are placed in  upper stopband, at fz2=848.45 and fz3=848.74 MHz. They are adjusted to get 60 dB minimum stopband loss.

Seven resonators form a degree 14 filter. Six degrees will be coming form the three FTZ’s. The remaining eight degrees will be provided by the transmission zeros at f=0 and f=. These transmission zeros may be shared between f=0 and f= in different ways. Let’s consider only the case with Nzero=1 and Ninf=7. The other cases may be obtained through similar steps (Exercise?). Several solutions are possible by extracting the FTZ’s in different orders. One case is described below.

- Design command set is shown in Figure 14.34 giving the circuit of Figure 14.34.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.34.b which is ready to conversion into CT form.

- With cursor under the left FTZ element the command CQ-CT Oprs / Convert to CT / Left / CLL is applied to getting a CT section with capacitive cross coupling corresponding to the FTZ in lower stopband. Then with cursor under the second and third FTZ element the command CQ-CT Oprs / Convert to CT / Left is applied to form the second CT section with all inductive couplings corresponding to the FTZ’s in the upper stopband (Figure 14.34.c).

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.1).

- Figure 14.34.d shows a pictorial description. Note that cavities 3 and 5 have couplings to four neighbors which may make tuning difficult.

Exercise: Repeat the design by extracting the FTZ in lower stopband in the middle of the circuit.

Exercise: Repeat the design for Nzero=7 and Ninf=1.

Exercise: Repeat the design for two transmission zeros in lower stopband and one in upper stopband.

Exercise: Repeat the design for all three FTZ’s in upper (lower) stopband.

Exercise: Repeat the design for only two FTZ’s with the CT sections at the two ends of the filter.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.34. A seven resonator CT filter with three FTZ’s.

Example 14.17. Seven resonator CT-CQ filters with three finite transmission zeros

The seven resonators can be arranged to form one CQ section and one CT section. Let’s design a seven resonator filter with the same specs as the previous example:

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

One FTZ in lower stopband at fz1=844.87 MHz.

Two finite transmission zeros in upper stopband, at fz2=848.45 and fz3=848.74 MHz.

Nzero=1 and Ninf=7.

- Design steps are shown in Figure 14.35. The extraction order is set such that proper circuit sections will be formed for conversion into CQ and CT form. The TZs Inf-Inf-FTZ1-FTZ2 will lead to a CQ section and the TZ’s  Inf-Inf-FTZ3 will lead to CT section.  Figure 14.35.a shows one extraction order.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.35.b.

- With cursor under the shunt resonator adjacent to the FTZ element the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements (Figure 14.35.c). Also with cursor under the rightmost FTZ element the command CQ-CT Oprs / Convert to CT is applied to form the CT section.

- The adjacent FTZ elements are combined to form a fourth order section (Figure 14.35.d).

- Placing the cursor under the fourth order section and applying command CQ-CT Operations/ Convert to CQ / Solution-1 we get CQKR element (Figure 14.35.e). That is, this CQ will have a diagonal cross-coupling.

- Equality of the shunt inductors is usually lost after conversion into CQ-CT form. This property can be recovered by applying the command Pi-Tee-L / Pi-to-Sym Pi / Equal Inds to the Pi-section in the middle (Figure 14.35.e-f).

- In Figure 14.35.f, the External Q, Coupling Coefficients and Resonance Frequencies are displayed.

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends. Then the inverters can be replaced by practical equivalents, as was done in Example 14.1.

- Figure 14.35.g shows pictorial description of a possible implementation. Clearly the diagonal cross coupling between nodes 2 and 4 will be problematic. In general we don’t like diagonal cross-coupling because they are difficult to construct and tune. In the presence of other FTZ’s, diagonal cross-coupling may arise even if we satisfy the symmetry conditions fz1*fz2=fp1*fp2 or fz1*fz2=f02 between a pair of FTZ’s. In such case we can eliminate diagonal cross-coupling by adjusting either fz1 or fz2 by trials (Exercise?).

Exercise: Repeat the design by extracting transmission zeros in different orders.

Exercise: Repeat the design for Nzero=7 and Ninf=1.

Exercise: Repeat the design for two transmission zeros in lower stopband and one in upper stopband.

Exercise: Repeat the design for all three FTZ’s in upper (lower) stopband.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.35. A seven resonator CT-CQ filter.

Figure 14.35. A seven resonator CT-CQ filter (cont’d).

Example 14.18. Seven resonator CQ filters with four finite transmission zeros

The seven resonators can be arranged to form two CQ sections by sharing one resonator.

Let’s design a seven resonator filter with the following specs aiming at obtaining two CQBP sections (CQ sections without diagonal cross couplings):

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

Two FTZ’s in lower stopband, at fz1=844.4 MHz, fz2=845 MHz.

Two finite transmission zeros in upper stopband, at fz3=849 and fz4=849.56 MHz.

Seven resonators make up a degree of 14. Eight degrees will be due to the four FTZ’s. The remaining six degrees will be coming from Nzero and Ninf. Let’s take Nzero=1 and Ninf=5.

- Design steps are shown in Figure 14.36. The extraction order is set such that proper circuit sections will be formed for conversion into CQ sections. The TZs Inf-Inf-FTZ1-FTZ4 will lead to one CQ section and the TZ’s  Inf-Inf-FTZ2-FTZ3 will lead to the second CQ section. Figure 14.36.a shows such an extraction order.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.36.b.

- With cursor under the shunt resonator adjacent to the first FTZ element on left, the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements (Figure 14.36.c).

- The adjacent FTZ elements are combined to form a fourth order section (Figure 14.36.d).

- Placing the cursor under the fourth order section and applying command CQ-CT Operations/ Convert to CQ /Left / Solution-1 we get CQBP element (Figure 14.36.e). Fortunately this CQ came out to be without a diagonal element. The same procedure is applied on the next pair of FTZ elements to give the CQ circuit of Figure 14.36.g.

- In Figure 14.36.g, the External Q, Coupling Coefficients and Resonance Frequencies are displayed.

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends. Then the inverters can be replaced by practical equivalents, as was done in Example 14.1.

- Figure 14.36.h shows pictorial description of a possible implementation. It is seen that resonator 4 is heavily loaded by the four couplings windows to the adjacent resonators. Eight resonators case may ease this problem.

Exercise: Repeat the design by extracting FTZ pairs in different orders.

Exercise: Repeat the design for Nzero=5 and Ninf=1.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.36. A seven resonator CQ filter with symmetric response.

Figure 14.36. A seven resonator CQ filter with symmetric response.

14. 8. DESIGN OF EIGHT RESONATOR CT FILTERS

Eight resonator can be arranged either to form a two CQ1F element to realize two FTZ’s or to form three  CT elements to realize three FTZ’s or to form two CQ elements to realize four FTZ’s, as described in the following example.

Example 14. 19. Eight resonator CQ1F filters

Let’s design a degree 16 filter with the following specifications:

PB Ripple= 0.01 dB,  fp1 = 846 MHz, fp2 = 848 MHz,

Two finite transmission zeros are placed, one in each stopband, at f=845.4 MHz and f=848.6 MHz. Eight resonators make up a degree 16 filter. Four degrees will be coming from the two FTZ’s and the remaining 12 degrees will be provided by TZ’s at f=0 and f=. Let’s consider the case with Nzero=1 and Ninf=11.

- Design command set is shown in Figure 14.37 giving the circuit of Figure 14.37.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.37.b.  which is ready to conversion into CQ1F form.

- With cursor under the FTZ elements the command CQ-CT Oprs / Convert to CQ / Solution-1 is applied to convert the circuit sections into CQ1F sections (Figure 14.37.c).

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.6).

- Figure 14.37.d shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting FTZ’s in different orders.

Exercise: Repeat the design for Nzero=11 and Ninf=1.

Exercise: Repeat the design for two transmission zeros in lower stopband.

Exercise: Repeat the design for two transmission zero in upper stopband.

Exercise: Repeat the design for different Nzero-Ninf combinations.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.37. An eight resonator CQ1F filter.

Example 14.20. Eight resonator CT filters with three finite transmission zeros

Using eight resonators three CT sections can be formed each realizing one FTZ. Let’s design a filter with

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

One FTZ is placed in lower stopband, at fz1=845.2 MHz. Two finite transmission zeros are placed in upper stopband, both at at fz2=848.5 MHz.

Eight resonators form a degree 16 filter. Six degrees will be coming form the three FTZ’s. The remaining 10 degrees will be provided by the transmission zeros at f=0 and f=. These transmission zeros may be shared between f=0 and f= in different ways. Let’s consider only the case with Nzero=1 and Ninf=9. The other cases may be obtained through similar steps (Exercise?). Several solutions are possible by extracting the FTZ’s in different orders. One case is described below.

- Design command set is shown in Figure 14.38 giving the circuit of Figure 14.38.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.38.b which is ready to conversion into CT form.

- With cursor under the left FTZ element the command CQ-CT Oprs / Convert to CT / Left / CLL is applied to getting a CT section with capacitive cross coupling corresponding to the FTZ in lower stopband. Then with cursor under the second and third FTZ element the command CQ-CT Oprs / Convert to CT / Left is applied to form the second CT section with all inductive couplings corresponding to the FTZ’s in the upper stopband (Figure 14.38.c).

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.1).

- Figure 14.38.d shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting the FTZ in lower stopband in the middle of the circuit.

Exercise: Repeat the design for Nzero=9 and Ninf=1.

Exercise: Repeat the design for two transmission zeros in lower stopband and one in upper stopband.

Exercise: Repeat the design for all three FTZ’s in upper (lower) stopband.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.38. An eight resonator CT filter.

Example 14.21. Eight resonator CQ filters with four finite transmission zeros

The eight resonators can be arranged to form two CQ sections. Let’s design an eight resonator filter with the following specs aiming at obtaining two CQBP sections (CQ sections without diagonal cross couplings):

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

Two FTZ’s in lower stopband, at fz1=fz2=845.46 MHz.

Two FTZ’s in upper stopband, at fz3=fz4=848.54 MHz.

Eight resonators make up a degree of 16. Eight degrees will be due to the four FTZ’s. The remaining eight degrees will be coming from Nzero and Ninf. Let’s take Nzero=1 and Ninf=7.

- Design steps are shown in Figure 14.39. The extraction order is set such that proper circuit sections will be formed for conversion into CQ sections. The TZs Inf-Inf-FTZ1-FTZ4 will lead to one CQ section and the TZ’s  Inf-Inf-FTZ2-FTZ3 will lead to the second CQ section. Figure 14.39.a shows such an extraction order.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.39.b.

- With cursor under the shunt resonator adjacent to the first FTZ element on left, the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements (Figure 14.39.c).

- The adjacent FTZ elements are combined to form a fourth order section (Figure 14.39.d).

- Placing the cursor under the fourth order section and applying command CQ-CT Operations/ Convert to CQ /Left / Solution-1 we get CQBP element (Figure 14.39.e). Fortunately this CQ came out to be without a diagonal element. The same procedure is applied on the next pair of FTZ elements to give the CQ circuit of Figure 14.39.f.

- The conversions destroyed equality of shunt inductors. Equality can be recovered by applying the command Pi-Tee-L / Pi-to-Sym Pi / Equal Shunt Inds to the Pi-section in the middle (Figure 14.39.g).

- In Figure 14.39.g, the External Q, Coupling Coefficients and Resonance Frequencies are displayed.

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends. Then the inverters can be replaced by practical equivalents, as was done in Example 14.1.

- Figure 14.39.h shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting FTZ pairs in different orders.

Exercise: Repeat the design for Nzero=7 and Ninf=1.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.39. An eight resonator CQ filter with symmetric response.

Figure 14.39. An eight resonator CQ filter with symmetric response.

Example 14.22. Eight resonator CQ-CT filters with four finite transmission zeros

The eight resonators may also be arranged to form one CQ section and two CT sections. Let’s repeat the design of the previous filter:

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

Two FTZ’s in lower stopband, at fz1=fz2=845.46 MHz.

Two FTZ’s in upper stopband, at fz3=fz4=848.54 MHz.

Eight resonators make up a degree of 16. Eight degrees will be due to the four FTZ’s. The remaining eight degrees will be coming from Nzero and Ninf. Let’s take Nzero=1 and Ninf=7.

- Design steps are shown in Figure 14.40. The extraction order is set such that proper circuit sections will be formed for conversion into CQ sections. The TZs Inf-Inf-FTZ1-FTZ4 will lead to one CQ section while the TZ’s  Inf-Inf-FTZ2 and Inf-Inf-FTZ3 will lead to two CT sections. Figure 14.40.a shows such an extraction order.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.40.b.

- With cursor under the shunt resonator adjacent to the first FTZ element on left, the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements. Also with cursor under the two FTZ elements on right the command CQ-CT Oprs / Convert to CT is applied to convert them into CT sections (Figure 14.40.c).

- The adjacent FTZ elements are combined to form a fourth order section (Figure 14.40.d).

- Placing the cursor under the fourth order section and applying command CQ-CT Operations/ Convert to CQ /Left / Solution-1 we get CQBP element (Figure 14.40.e). The CQ came out to be without a diagonal element.

- The conversions destroyed equality of shunt inductors. Equality can be recovered by applying the command CQ-CT Oprs / Modify CT / Set L2 and Set L3  iteratively to two CT sections where in each application the inductance of the leftmost shunt inductor is typed in (Figure 14.40.f).

- In Figure 14.40.f, the External Q, Coupling Coefficients and Resonance Frequencies are displayed.

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends. Then the inverters can be replaced by practical equivalents, as was done in Example 14.1.

- Figure 14.40.g shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting FTZ pairs in different orders.

Exercise: Repeat the design for Nzero=7 and Ninf=1.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.40. An eight resonator CQ-CT filter.

Figure 14.40. An eight resonator CQ-CT filter (cont’d).

14. 9. NINE RESONATOR CT AND CQ FILTERS

Nine resonator filters can be arranged to form two CQ section to realize two or four FTZ’s, or four CT section to realize four FTZ’s.

Example 14. 23. Nine resonator CQ1F filters

Let’s design a degree 18 filter with the following specifications:

PB Ripple= 0.01 dB,  fp1= 846 MHz, fp2=848 MHz,

Two finite transmission zeros are placed, one in each stopband, at f=845.4 MHz and f=848.6 MHz. Nine resonators make up a degree 18 filter. Four degrees will be coming from the two FTZ’s and the remaining 14 degrees will be provided by TZ’s at f=0 and f=. Let’s consider the case with Nzero=1 and Ninf=13.

- Design command set is shown in Figure 14.41 giving the circuit of Figure 14.41.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.41.b.  which is ready to conversion into CQ1F form.

- With cursor under the FTZ elements the command CQ-CT Oprs / Convert to CQ / Solution-1 is applied to convert the circuit sections into CQ1F sections (Figure 14.41.c).

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.6).

- Figure 14.41.d shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting FTZ’s in different orders.

Exercise: Repeat the design for Nzero=13 and Ninf=1.

Exercise: Repeat the design for two transmission zeros in lower stopband.

Exercise: Repeat the design for two transmission zero in upper stopband.

Exercise: Repeat the design for different Nzero-Ninf combinations.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.41. A nine resonator CQ1F filter.

Example 14.24. Nine resonator CT filters with four finite transmission zeros

Using nine resonators four CT sections can be formed each realizing one FTZ. Let’s design a filter with

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

Two FTZ’s are placed in lower stopband, both at fz1=fz2=845.5 MHz. Two finite transmission zeros are placed in upper stopband, both at at fz3=fz4=848.5 MHz.

Nine resonators form a degree 18 filter. Eight degrees will be coming form the four FTZ’s. The remaining ten degrees will be provided by the transmission zeros at f=0 and f=. These transmission zeros may be shared between f=0 and f= in different ways. Let’s consider only the case with Nzero=1 and Ninf=9. The other cases may be obtained through similar steps (Exercise?). Several solutions are possible by extracting the FTZ’s in different orders. One case is described below.

- Design command set is shown in Figure 14.42 giving the circuit of Figure 14.42.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.42.b which is ready to conversion into CT form.

- With cursor under the left two FTZ elements, the command CQ-CT Oprs / Convert to CT / Left / CLL is applied to get a CT section with capacitive cross coupling corresponding to the FTZs in lower stopband. Then with cursor under the third and fourth FTZ elements the command CQ-CT Oprs / Convert to CT / Left is applied to form the third and fourth CT sections with all inductive couplings corresponding to the FTZ’s in the upper stopband (Figure 14.42.c).

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.1).

- Figure 14.42.d shows pictorial description of a possible implementation.

Exercise: Repeat the design for different Nzero and Ninf combinations. For example try to get the two lower stopband CT sections in CCC form.

Exercise: Repeat the design by extracting the FTZ’s in different orders.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.42. A nine resonator CT filter with four FTZ’s.

Example 14.25. Nine resonator CQ filters with four finite transmission zeros

Nine resonators can be arranged to form two CQ sections. Let’s design a nine resonator filter with two CQBP sections (CQ sections without diagonal cross couplings):

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

Two FTZ’s in lower stopband, at fz1=fz2=845.5 MHz.

Two FTZ’s in upper stopband, at fz3=fz4=848.5 MHz.

The FTZ’s are hoped to be symmetric enough to avoid diagonal cross couplings. If a diagonal cross coupling arises we have to adjust them and repeat the design procedure. Nine resonators make up a degree of 18. Eight degrees will be due to the four FTZ’s. The remaining ten degrees will be coming from Nzero and Ninf. Let’s take Nzero=1 and Ninf=9.

- Design steps are shown in Figure 14.43. The extraction order is set such that proper circuit sections will be formed for conversion into CQ sections. The TZs Inf-Inf-FTZ1-FTZ4 will lead to one CQ section and the TZ’s  Inf-Inf-FTZ2-FTZ3 will lead to the second CQ section. Figure 14.43.a shows such an extraction order.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.43.b.

- With cursor under the shunt resonator adjacent to the first FTZ element on left, the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements (Figure 14.43.c).

- The adjacent FTZ elements are combined to form a fourth order section (Figure 14.43.d).

- Placing the cursor under the fourth order section and applying command CQ-CT Operations/ Convert to CQ /Left / Solution-1 we get CQBP element (Figure 14.43.e). Fortunately this CQ came out to be without a diagonal element. The same procedure is applied on the next pair of FTZ elements to give the CQ circuit of Figure 14.43.f.

- The conversions destroyed equality of shunt inductors. Equality can be recovered by applying the command Pi-Tee-L / Pi-to-Sym Pi / Equal Shunt Inds to the Pi-section in the middle (Figure 14.43.g).

- In Figure 14.43.g, the External Q, Coupling Coefficients and Resonance Frequencies are displayed. Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends. Then the inverters can be replaced by practical equivalents, as was done in Example 14.1.

- Figure 14.43.h shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting FTZ pairs in different orders.

Exercise: Repeat the design for Nzero=9 and Ninf=1.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.43. A nine resonator CQ filter.

Figure 14.43. A nine resonator CQ filter (cont’d).

14. 10. DESIGN OF TEN RESONATOR CQ-CT FILTERS

Ten resonators can be arranged in variety of ways to form CQ and CT sections to realize two, three, four or five FTZ’s. Let’s see some of them.

Example 14. 26. Ten resonator CQ1F filters with three FTZ’s

Let’s design a degree 20 filter with the following specifications:

PB Ripple= 0.01 dB,  fp1= 846 MHz, fp2=848 MHz,

Two finite transmission zeros are placed, one in each stopband, at f=845.4 MHz and f=848.6 MHz. Ten resonators make up a degree 20 filter. Four degrees will be coming from the two FTZ’s and the remaining 16 degrees will be provided by TZ’s at f=0 and f=. Let’s consider the case with Nzero=1 and Ninf=13.

- Design command set is shown in Figure 14.44 giving the circuit of Figure 14.44.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.44.b.  which is ready to conversion into CQ1F form.

- With cursor under the FTZ elements the command CQ-CT Oprs / Convert to CQ / Solution-1 is applied to convert the circuit sections into CQ1F sections (Figure 14.44.c).

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.6).

- Figure 14.44.d shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting FTZ’s in different orders.

Exercise: Repeat the design for Nzero=13 and Ninf=1.

Exercise: Repeat the design for three transmission zeros in lower stopband.

Exercise: Repeat the design for three transmission zero in upper stopband.

Exercise: Repeat the design for different Nzero-Ninf combinations.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.44. A ten resonator CQ1F filter with three FTZ’s.

# Example 14.27. Ten resonator CT filters with four finite transmission zeros

Using ten resonators four CT sections can be formed each realizing one FTZ. Let’s design a filter with

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

Two FTZ’s are placed in lower stopband, both at fz1=fz2=845.5 MHz. Two finite transmission zeros are placed in upper stopband, both at at fz3=fz4=848.5 MHz.

Ten resonators form a degree 20 filter. Eight degrees will be coming form the four FTZ’s. The remaining twelve degrees will be provided by the transmission zeros at f=0 and f=. These transmission zeros may be shared between f=0 and f= in different ways. Let’s consider only the case with Nzero=1 and Ninf=11. The other cases may be obtained through similar steps (Exercise?). Several solutions are possible by extracting the FTZ’s in different orders. One case is described below.

- Design command set is shown in Figure 14.45 giving the circuit of Figure 14.45.a.

- After applying the command Pi-Tee-L / Equal Shunt Inductors we get the circuit of Figure 14.45.b which is ready to conversion into CT form.

- With cursor under the leftmost and rightmost FTZ elements, the command CQ-CT Oprs / Convert to CT / Left / CLL is applied to get CT sections with capacitive cross coupling corresponding to the FTZs in lower stopband. Then with cursor under the interior FTZ elements the command CQ-CT Oprs / Convert to CT / Left is applied to form the third and fourth CT sections with all inductive couplings corresponding to the FTZ’s in the upper stopband (Figure 14.45.c). The quality of shunt inductors can be recovered by applying the command Pi-Tee-L / Equal Shunt Inductors.

- Impedance scaling can be carried out as in the early examples (e.g. Example 14.1).

- Figure 14.45.d shows pictorial description of a possible implementation.

Exercise: Repeat the design for different Nzero and Ninf combinations. For example try to get the two lower stopband CT sections in CCC form.

Exercise: Repeat the design by extracting the FTZ’s in different orders.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.45. A ten resonator CT filter.

Example 14.28. Ten resonator CQ filters with six finite transmission zeros

Ten resonators can be arranged to form three CQ sections as shown below:

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

Three FTZ’s in lower stopband, at fz1=fz2=fz3=845.5 MHz.

Three FTZ’s in upper stopband, at fz4=fz5=fz6=848.5 MHz.

Ten resonators make up a degree of 20. Twelve degrees will be due to the six FTZ’s. The remaining eight degrees will be coming from Nzero and Ninf. Let’s take Nzero=1 and Ninf=7.

- Design steps are shown in Figure 14.46. The extraction order is set such that proper circuit sections will be formed for conversion into CQ sections. The TZs Inf-Inf-FTZ1-FTZ6 will lead to one CQ section and the TZ’s Inf-Inf-FTZ2-FTZ5 will lead to the second CQ section finally the TZ’s Inf-Inf-FTZ3-FTZ4. will lead to the third CQBP section. Figure 14.46.a shows such an extraction order.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.46.b.

- With cursor under the shunt resonator adjacent to the FTZ pairs, the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements (Figure 14.46.c).

- The adjacent FTZ elements are combined to form a fourth order section and then with cursor under the fourth order section applying command CQ-CT Operations/ Convert to CQ /Left / Solution-1 we get CQBP elements (Figure 14.46.d). All CQ sections came out to be without a diagonal element because the FTZ locations are adjusted to be symmetric.

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends. Then the inverters can be replaced by practical equivalents, as was done in Example 14.1.

- Figure 14.46.e shows pictorial description of a possible implementation.

Exercise: Repeat the design by extracting FTZ pairs in different orders.

Exercise: Repeat the design for Nzero=9 and Ninf=1.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Figure 14.46. A ten resonator CQ filter.

EXAMPLE 14. 29. Ten resonator CQ-CT filters with five finite transmission zeros

Ten resonators may also be arranged to form one CQ section and three CT sections to realize five FTZ’s. Let’s design a filter with following specs:

PB Ripple=0.01 dB,  fp1=846 MHz, fp2= 848 MHz,

Two FTZ’s in lower stopband, at fz1=fz2=845.5 MHz.

Three FTZ’s in upper stopband, at fz3=fz4=fz5=848.5 MHz.

Ten resonators make up a degree of 20. Ten degrees will be due to the five FTZ’s. The remaining ten degrees will be coming from Nzero and Ninf. Let’s take Nzero=1 and Ninf=9.

- Design steps are shown in Figure 14.47. The extraction order is set such that proper circuit sections will be formed for conversion into CQ sections. The TZs Inf-Inf-FTZ1-FTZ5 will lead to one CQ section while the TZ’s  Inf-Inf-FTZ2, Inf-Inf-FTZ3 and Inf-Inf-FTZ4 will lead to three CT sections. Figure 14.47.a shows such an extraction order.

- Shunt inductors are equated using Pi-Tee-L / Equal Shunt Inductors command Figure 14.47.b.

- With cursor under the shunt resonator adjacent to the first FTZ element on left, the command Double Norton / Inverse Geffe is applied to bring the two FTZ sections in adjacent position. Simplify Circuit command is used to move away the resulting transformer and combine the adjacent shunt elements. Then adjacent FTZ elements are combined to form a fourth order section. Also with cursor under the two FTZ elements on right the command CQ-CT Oprs / Convert to CT is applied to convert them into CT sections (Figure 14.47.c).

- Placing the cursor under the fourth order section and applying command CQ-CT Operations/ Convert to CQ /Left / Solution-1 we get CQBP element. The CQ section came out to be without a diagonal element (Figure 14.47.d).

- The conversions destroyed equality of shunt inductors. Equality can be recovered by applying the command CQ-CT Oprs / Modify CT / Set L2 and Set L3 iteratively to three CT sections where in each application the inductance of the leftmost shunt inductor will be typed in.

- Element values of the circuit may be adjusted to desired (realization) level by inserting ideal inverters at the source and load ends. Lossy form can be formed, coupling coefficients and node voltages can be displayed as was done in Example 14.1.

- Figure 14.47.e shows pictorial description of a possible implementation.

Exercise: Repeat the design by forming two CQ sections and one CT section.

Exercise: Repeat the design for different Nzero-Ninf combinations.

Exercise: Scale the impedance level by inverters and then use Parasitics / All Lossy Resonators command to inspect the effects of finite Q of resonators.

Exercise: Scale impedance level by Tapping approach.

Exercise: Scale impedance level by Transformer Resonator approach.

Exercise: Display Node Voltage, Current and Power using ALT+F8 keys (after impedance scaling).

Exercise: Design filters with 11, 12, 13, 14 resonators utilizing different CQ-CT combinations.

Figure 14.47. A ten resonator CQ-CT filter.