14. 11. LINEAR PHASE FILTERS USING CASCADED QUADRUPLET  (CQ)  SECTIONS

{Note: Linear phase filters are treated in a broader manner in Chapter-15. However for both emphasizing the link between CQ sections used for jw-axis TZ’s and CQ sections used for complex TZ’s, the Linear Phase CQ filters section of Chapter-15 is repeated below, hoping this will be more convenient  for the reader}

Linear phase filters require placement of complex conjugate quadruplets of TZ’s, si=ijwi in addition to the usual jw-axis TZ’s. The extraction of complex TZ’s in cascaded element form in the classical way leads to Darlington-D sections which are problematic for realization (Figure 14.48.a). Recently it was shown that a complex TZ quartet can be extracted as shown in Figure 14.8.b by zero shifting from both s=0 and s=. These circuit sections involve either series or shunt fourth order sections having some negative elements. The advantage of the new forms is that when they are extracted as part of circuit sections with either one TZ at s=0 and three TZ’s at s=  or  three TZ’s at s=0 and one TZ at s=, these circuit sections can be converted into CQ sections as described in Figure 14.49.

Figure 14.48. Extraction of a complex TZ quadruplet in different forms.

The negative element problems of the fourth order sections disappear after conversion into CQ form. It should be noted that the cross-coupling elements (1-4 coupling) are of the same type as the main line coupling elements. That is, all coupling elements are inductors or all capacitors. This is opposite of the CQ sections that create jw-axis FTZ’s where the cross-coupling elements were of opposite of the main line coupling elements. On the right of Figure 14.49, a more general form of CQ section is shown where the coupling elements are all inverters. The inverters can be replaced by their inductive or capacitive Pi-section equivalents to get the inductive or capacitive couplings.

In the present version of Filpro, placement and extraction of Complex TZ’s are not available. Instead, a brute force approach is developed and placed into Filpro leading to the usual CQ section with two symmetric FTZ’s or a CQ section with linear phase response or a more general and flexible CQ element named as CQTL. The CQTL element has transmission line and inverter combination as the 1-4 coupling element. The transmission line can be used in the cases where the resonators 1 and 4 can’t be made close enough for simple L or C type coupling. In some cases the transmission line may also be used to shift the FTZ’s to one side of the passband. The CQTL element is described in Figure 14.50:

- Figure 14.50.a shows the CQTL circuit in terms of ideal inverters. The cross-coupling element is shown as K14+TL.

- Figure 14.50.b shows the constant reactance Pi-model of an ideal inverter. In this model the series reactance is split into two equal parts to allow insertion of a transmission line piece.

- Figure 14.50.c shows the transmission line inserted version of the 1-4 cross-coupling element. The constant reactances can be replaced by their capacitive or inductive equivalents as shown in the figure.

- For zero length transmission line and K14 having the same sign as the main line inverters (same sign as K23), the CQTL element reduces to the Linear Phase CQ element described in Figure 14.49, realizing a complex TZ.

Figure 14.49. Conversion of circuit sections involving complex TZ’s into linear phase CQ sections.

Figure 14.50. CQ element with transmission line type cross-coupling (CQTL).

- For zero length Transmission Line and K14 having the sign opposite of the main line inverters (sign opposite of K23), the CQTL element reduces to the CQBP element realizing a pair of  symmetric jw-axis FTZ’s.

- The Inverter and Transmission Line combination sets both amplitude and phase of the cross-coupling signal. The signal amplitude is mainly set by the inverter while the phase is set mainly by the Transmission Line length. Characteristic impedance of the transmission line is also effective but usually it is fixed at a realizable value. The overall effect of insertion of the transmission line is that, as the length of the transmission line is changed both real and imaginary parts of the complex TZ leading to different responses.

- It should be noted that when the length of transmission line is about half wavelength at the passband center then the effect of transmission line will be just a 180 degree phase change. This is equivalent to changing the sign of the inverter K14. So, a linear phase CQ section can be converted into a CQBP section with two FTZ’s and vice versa by setting the length of transmission line.

- For line length about one half wavelength, the two FTZ’s can be moved to the lower stopband and for line length about a full wavelength the two FTZ’s can be moved into the upper stopband. Thus, the transmission line can be used to form asymmetric responses as well.

Let’s continue the descriptions on some examples involving the brute force technique for the formation of a CQ or CQTL element on Filpro.

#### EXAMPLE 14. 30.  Design of four  resonator CQ filter with FTZs  using brute force approach

Let’s design a CQ filter with four resonators with following specifications:

PBRipple=0.01 dB, fp1=980 MHz, fp2=1020 MHz.

The two FTZs will be located at about 960 MHz and 1030 MHz.

All inductors will be 10 nH.

The command set is shown in Figure 14.52.

- Design / By Prototype with Inverters approach is used to form the direct coupled version of the four resonator filter giving the circuit of Figure 14.52.a.

- The command Inverters / All Equal Left-Right is applied to equate inductors of all shunt resonators (Figure 14.52.b).

- Inductors of the resonators are scaled up to 10 nH by inserting inverters using the command Inverters / Scale Cct By Ins. Inverters / Specify Inductor: L=10 nH (Figure 14.52.c).

- Then the end inverters are replaced by their capacitive L-section equivalents using the command Inverters / Replace Inv By / End Inverter to / Capacitive-L (Figure 14.52.d).

- Simplify Circuit command combines the negative shunt capacitors of the L-sections with the positive neighbours (Figure 14.52.e). The circuit is now ready for brute force introduction of the 1-4 cross-coupling element. The circuit is saved as 401.fil. Its responses are shown in Figure 14.52.f.

- The circuit 401.fil is loaded (Figure 14.52.g) and with cursor under the middle inverter the command CQ-CT Oprs / Conv. to CQ-CN/ CQ / CQ with FTZs is applied. This command asks the impedance of cross-coupling inverter K14. This inverter will set the locations of FTZs. We accept the displayed default value to see the result. Later we will tune it to set the FTZs to the desired locations. We get the circuit shown in Figure 14.52.h. The middle part of the circuit is replaced by a CQBP element. This CQ section involves the two resonators on the two sides of the middle inverter, together with the inline coupling inverters K12, K23, K34 and cross-coupling inverter K14.

- The insertion loss response displays the two FTZs. They are moved to the desired locations 1030 MHz and 1060 MHz by applying Tune Element command to the cross-coupling element K14. This adjustement degrades the return loss. It is then recovered by tuning the mainline coupling inverter K23 (across K14). This adjustment will change locations of the FTZs slightly which will be recovered by tuning K14 again. Return loss is further improved by slightly tuning K12, K34,C2 and C3 also. Further improvements can be obtained by applying Monte Carlo-Best Case optimisation routine.

- The resulting insertion loss and return loss responses are shown in Figure 14.52.i. A slight widening in bandwidth and reduction in return loss level is seen. These changes can be taken into account by repeating the design with slightly adjusted specifications.

- Figures 14.52.k-m-n shows the effects of finite Q of the resonators.

- The Coupling Coefficients and Resonance Frequencies of the resonators can be displayed by placing the cursor under the CQBP element and pressing the keys AL+F6. The External Q and Coupling Coefficients can be displayed by pressing the keys CTRL+F6 (Figure 14.52.p.

- In Figure 14.52.q node voltage-current-power variables are displayed at several frequencies by pressing the keys ALT+F8. It is to be noted that voltages can assume extreme values just outside the passband.

Figure 14.52. Design of a four resonator CQ filter with FTZs.

Linear phase CQ filters can be synthesized in exact manner by specifying the TZ’s. However, specification of the real parts of the complex TZ’s needs experimentation and iteration until the targeted group delay requirement is obtained. Therefore the algorithms for placement and extraction of complex TZ’s are not integrated into Filpro. Instead, an easier version is developed for Filpro. As will be described in this section, it is also possible to get the same results by starting from a direct coupled resonator structure and introducing cross couplings by brute force to form the CQ sections for complex TZ’s and then tuning only the coupling elements of the CQ sections. This is a simpler approach especially for high degree filters which encounter numerical accuracy problems in element extraction stages of synthesis procedures. This approach can be applied either directly on bandpass filters or on lowpass prototypes.

Traditionally linear phase filters are designed by the canonical cross coupled (CCC) lowpass prototypes of Rhodes-Zabalawi [12] and Cloete [13]. In these prototypes the widest flat delay bandwidth can be obtained by structures with two transmission zeros (TZs) at s= (Ninf=2) and N-2 complex TZs where N is the degree of the prototype (number of resonators in the bandpass version). So, these filters can be used as reference to asses delay bandwidths. Since each lowpass CQ section have two TZ’s at infinity, the filters formed by cascading CQ sections have inherently high selectivity. Therefore delay bandwidths of CQ filters are narrower than the delay bandwidths of  CCC filters with the same number of resonators and with Ninf=2. When higher selectivity is desired in CCC filters the number of TZs at s= is increased at the expense of reduction in the number of complex TZ’s (if the total degree N is fixed) which in turn narrows the delay bandwidth. In such high selectivity cases linear phase CQ filters may form a viable alternative to the CCC filters of the same performance just because of ease of construction and tuning. In CQ filters all coupling coefficients are close to each other while in CCC filters especially cross coupling coefficients may have very wide dynamic ranges.

Figure 14.49 shows various circuit models of CQ sections used in Filpro. Circuit sections are converted first into CQ sections with inverters. The circuit model shown in Figure 14.49.a is used only for CQ sections which produce jw-axis finite transmission zeros. In general CQ models with diagonal cross couplings can be used for all purposes, like generation of asymmetric jw-axis finite transmission zeros or simulation of linear phase CQ sections. They are named as CQKL and CQKR where the letter K indicates that the element involves inverters and the letters L and R stand for the diagonal element tilted Left (between nodes 1 and 3) and tilted Right (between nodes 2 and 4). When needed, the ideal inverters are replaced by their inductive (if K>0) or capacitive (if K<0) Pi-section equivalents yielding LC type circuit models for the CQ sections, as described in the figure. Conversion from inverter-to-LC model is carried out by applying the command CQ-CT Oprs / CQ-Inv-to-CQ-LC.  In the LC models coupling elements appear as parallel LC sections. In these elements either L is made ineffective by setting L= (a large value) or C is made ineffective by setting C=0. If no diagonal element is needed, as in linear phase CQ sections or as in CQ sections with symmetric jw-axis transmission zeros, then both L and C of the diagonal coupling element are set to be ineffective. When the jw-axis transmission zero pair is symmetric then CQ section automatically assumes the simpler form shown in Figure 9.47.a.

# 14. 11.1. DIRECT DESIGN OF BANDPASS LINEAR PHASE CQ FILTERS

The procedure is applicable on direct coupled resonator bandpass filters. In Filpro, it is easiest to design such filters in inverter coupled forms, by using the approach Design / By Prototypes with Inverters. Built in LC Prototype or Synthesis approaches can also be used. Let’s describe the procedure on an example.

EXAMPLE 14.30. Design of a Chebyshev bandpass linear phase CQ filter

The filter has the following specifications: Passband edge frequencies: fp1=1940 MHz, fp2=1960 MHz. Passband Return Loss: 26 dB. Number of resonators: N=13

- As the first step, a direct coupled resonator filter will be designed to meet the amplitude and return loss specifications by using Design / By Prototype with Inverters approach. A prototype with shunt capacitors and degree 13 is selected giving the circuit of Figure 14.50.a.

- Then the command Filter Type is invoked and Lumped-BPF options are selected.

- Upon Find Circuit command Filpro asks to save the prototype. If it will be used in other designs, it can be saved. Otherwise there is no need to save it. The designed BPF circuit is displayed as shown in Figure 14.50.b.

- The command Inverters / All Equal Left-Right is applied to equate all the shunt LC resonators (Figure 14.50.c). The resulting unity transformer is deleted. Equal shunt resonators is usually a desirable feature, as the resonators will be identical.

- The next stage is conversion of into CQ form. Consider the circuit sections between nodes 1-4, 4-7, 7-10 and 10-13. Each section involves four resonators coupled by three inverters. These four node sections can be converted into cross coupled quadruplets (CQ sections) by enforcing cross couplings between nodes 1-4, 4-7, 7-10 and 10-13. In Filpro this is done by placing the cursor under the middle inverter of the circuit section between nodes 1 and 4, and applying the command CQ-CT Oprs / Convert to CQ / K14=200,000/... The elements between nodes 1 and 4 will be replaced by a CQ section. Either CQKL or CQKR can be selected, as the diagonal cross coupling elements of both are automatically set to be ineffective (Infinite inductor and zero capacitor). Let’s select CQKL option. Impedance of the cross coupling inverter K14 is selected very high to avoid any distortion in the original response. The same conversion is repeated on the next circuit section between nodes 4 and 7. Then Symmetrize command can be used to map the left half of the circuit into the right because the circuit is symmetric. Thus we get a circuit with four CQ sections as shown in Figure 14.50.d. Response of the circuit is the same as the original filter, because the cross coupling elements are yet ineffective.  Return Loss, Insertion Loss and Delay responses are plot for use as reference ( Figure 14.50.e).

- Now we start to decrease impedance of cross coupling inverter K14 of the first CQ section by using the command Tune Element / K14. Since the circuit is symmetric this operation will also tune the K14 of the fourth CQ section also simultaneously. Observe flattening of the delay response while K14 is decreased. Return loss degrades appreciably and skirt slopes of insertion loss decreases during this tuning operation. Set K14 to any reasonable level where return loss peaks start to disappear.

- Next, Tune the main line coupling inverter K23, right across K14 until all return loss peaks reappear.  Then tune K12 and K34 alternately, occasionally returning to K23 until equal return loss ripple condition is completely recovered.

- Repeat the procedure by further decreasing K14 to levels comparable to K12-K23-K34.

- Repeat the procedure on the second CQ element which tunes together with the third CQ (the symmetric counterpart).

- Best Case command of Filpro may also be used for fine tuning to get exact equal return loss ripple condition

- The frequency band in which the delay is constant will be exposed after recovering return loss.

- It is seen that insertion loss skirt slope lowers as K14 is decreased. This is expected because decreasing K14 is equivalent to moving a total of four transmission zeros at s=0 and s= to finite locations  on the complex s=+jw plane to form a complex conjugate quadruplet of transmission zeros.

- In order to get an idea on peculiarities, possible solutions and limitations of the linear phase CQ structures some trial designs are made as follows.

- K12, K23, K34 of all CQ elements are in the order of 100. As a first trial, K14 of all CQ elements are set to 1000 one by one. K23, K12 and K34 of a CQ element are adjusted to recover return loss response to equal ripple condition at 26 dB by using Tune Element command.  Resulting circuit element values, return loss and delay responses are shown  in Figure 14.50.f. Equal ripple return loss condition is recovered. It is seen that delay response started to flatten.

- Next, the K14 of the two inner CQ elements are set to 500 ohms and tuned to recover response. Delay is flattened more as seen in Figure 14.50.g. Then they are further reduced to K14=400 ohms, giving a ripple delay of about the desired 20 nS, as shown in Figure 14.50.h. Delay bandwidth became wider than the previous case.

- Figures 14.50.k and m show trials with different levels of K14, in the order of 250-to-700 ohms. It is seen that delay ripple increases as K14 level is decreased.

- One point to be noted from these experiments is the fact that one can always get equal ripple return loss response for any K14 level.

- One can set K14 of each CQ section to any desired value, as these experiments show. However it may be wise to choose them all equal. This may be an advantage compared to canonical cross coupled filters where large dynamic range may result for cross coupling elements.

- The following observations can be noted on group delay  response:

- Flattest group delay can be obtained for a critical value of the cross coupling inverters. In this case one can notice the small ripples on the delay response. The bandwidth for this flattest delay is about 50-60 % of the pass bandwidth of the filter.

- As the cross coupling inverter impedance is decreased below the critical level then a dip is observed in delay response, about the passband center.  Allowable delay ripple sets the delay bandwidth. Hence delay bandwidth can be increased by decreasing the cross coupling inverter impedances K14, accompanied by tuning of the relevant K12, K23 and K34.

- An optimizer can be used to spread the delay ripple to get a slightly wider delay bandwidth at the expense of unequal return loss ripples. However there is a basic limitation for maximum bandwidth for allowable delay ripple: Each CQ section needs a total of four TZ’s at s=0 and s= (the adjacent CQ sections may share common resonators). These TZ’s increase selectivity, but also limits the flat delay bandwidth. In this respect the maximum bandwidth obtainable from CQ filters is less than a canonical cross coupled filter with the same number of resonators which have a total of only four TZ’s at s=0 and s=.

Figure 14.50. Design of a linear phase CQ filter with 13 resonators.

Figure 14.50. Design of a linear phase CQ filter with 13 resonators (continued).

Figure 14.50. Design of a linear phase CQ filter with 13 resonators (continued).

Figure 14.50. Design of a linear phase CQ filter with 13 resonators (continued).

Figure 14.50. Design of a linear phase CQ filter with 13 resonators (continued).

## EXAMPLE 14.31. Synthesis of an elliptic linear phase CQ filter

It is also possible to cascade CQ sections some having FTZ’s and some with linear phase response. Design of such filters should start with the Synthesis approach where the FTZ’s are placed for the formation of CQ sections with FTZ’s and then the CQ sections with linear phase response should be formed by tuning as described in the previous example.

Let  fp1=975 MHz, fp2=1025 MHz and PBRipple=0.01 dB.

Setting the transmission zeros:

The numbers and locations of the TZ’s need be selected carefully by considering that we are aiming a CQ type filter. Let’s place two symmetrical FTZ’s, one at fz1=960 MHz and fz2=1041 MHz (roughly geometrically symmetric about f0=1000 MHz to avoid diagonal cross-coupling: fz2f02/fz1. We may readjust fz1 or fz2 if still a diagonal cross-coupling arises).

Let’s assume that we want mostly inductive couplings between adjacent resonators. Thus, place a single TZ at f=0 and the remaining ones at f=. The CQ section with FTZ’s needs eight degrees for realization (four resonators, each resonator providing two degrees), with four degrees coming from the two FTZ’s and the remaining four will be from the TZ’s at f=0 and f=. The group delay adjusting CQ section (creating a complex TZ at unknown location!) will also need four resonators, hence eight degrees. Thus, we may design a filter with total degree N=16 (eight resonator filter) with Nzero=1, NFTZ=4 and  Ninf=16-Nzero-NTFZ=11. The complex TZ’s of the second CQ section will be formed by moving some of the TZ’s at f= to complex locations by tuning, as will be described below. This set of TZ’s may form one solution.

Another solution would be to force the two CQ sections to share a common resonator, leading to a 7 resonator filter with total degree N=14. This would lead to the transmission zero pattern, Nzero=1, NFTZ=4 and Ninf=14-Nzero-NFTZ=14-1-4=9. This would lead to a filter with minimum number of resonators.

Naturally, we could place more transmission zeros at infinity (or zero), as dictated by the selectivity requirements, leading to higher number of resonators, eight of which are forming the two CQ sections and the remaining ones being simply (direct) coupled.

Let’s select the minimum number of resonator case (seven resonators, N=14) with Nzero=1, NFTZ=4 (two FTZ’s) and Ninf=9. Design steps are shown in Figure 14.51.

- Design / By Synthesis approach is selected. The options Lumped – Equiripple – Doubly Terminated – Type B are selected.

- Passband specifications and Transmission Zeros are typed in.

- The TZ’s are extracted in the order shown in the figure giving the circuit of Figure 14.51.a.

- The command Pi-Tee-L / Equal Shunt Inductors is applied to get the direct coupled resonator form of the filter (Figure 14.51.b).

- The command Double Norton / Inverse Geffe is applied to make the two FTZ sections adjacent. Simplify Circuit command is used to move the resulting transformer away (Figure 14.51.c).

- The two adjacent FTZ sections are Combined to form a single fourth order section (Figure 14.51.d).

- With cursor under the fourth order section the command CQ-CT Oprs / Convert to CQ / Left / Solution-1 is applied to form the CQ section corresponding to the two FTZ’s (Figure 14.51.e). Here we note that a CQBP section is formed. That is, no diagonal cross-coupling exists. This is made possible by adjusting the location of the second FTZ, fz2=1041 MHz.

- Thus, one of the CQ sections is formed. The resonators on the right of that CQ section numbered as 4, 5, 6 and 7) will be used to form the second CQ section that will create complex TZ quartet for linearizing the phase  (flattening the group delay- group delay equalization), as follows.

- With cursor under the series elements (here, series inductors) coupling  the resonators, the command Inverters / Pi-to-Inverter / f0=1000 MHz is applied to convert the  coupling elements into inverters (Figure 14.51.f). This is needed just because the algorithm for conversion into CQ section is based on inverter type couplings.

- It is usually preferred to have equal inductors in all shunt resonators. With cursor under the CQ section the command CQ-CT Oprs / Modify CQ / Equate Left-Right Inductors is applied to equate the inductors on the two sides of the CQ section (Figure 14.51.f-g). The resulting transformer is shifted away by Simplify Circuit command.

- In order to equate the shunt inductors in between the inverters the cursor is placed under any inverter and the command Inverters / All Equal Left-Right / Inductors is applied (Figure 14.51.h). Thus, all inductors are equated at the level L=0.4939 nH. This inductance level can be scaled up to the desired level by inserting inverters at source and load ends at this stage or at any later stage of the design procedure.

- Now we are ready to form the second CQ section which will equalize the group delay. This will be  done by brute force introduction of an inverter type cross-coupling between node-4 and node-7 by placing the cursor under the middle inverter (between node-5 and node-6) and applying the command CQ-CT Oprs / Convert to CQ: K14=2000000 /LfCQGenInv (Figure 14.51.k). This command replaces all the elements between node-4 and node-7 by a General CQ section (CQKL) with inverter type couplings and Left type diagonal cross-coupling. The diagonal cross-coupling is made ineffective automatically. Here, K14 is the inverter coupling node-1 and node-4 of the resulting CQ section. Actually these are node-4 and node-7 of the circuit respectively. Initially K14 is assigned a large value to simulate no cross-coupling case.

- The inverters K12, K23 and K34 of the CQ section are all taken as positive. They could as well be made negative without effecting the response). If K14 has the same sign as the other inverters   then the CQ section createsa  complex TZ quartet while if K14 is of opposite sign, then the CQ section will create jw-axis FTZ’s. Since our aim is to create complex TZ’s for group delay equalization we select positive sign for K14.

- The cross-coupling starts to show its effect on response when the value of K14 is decreased to the levels comparable to the other inverters. This is done by placing the cursor under the CQKL element and applying the command Tune Element: K14 (Figure 14.51.i).  When we start to tune (decrease) K14 all responses will start to degrade below a certain value of K14. Tuning procedure is done in a more controlled manner while observing the return loss.

- Figure 14.51.k shows the Insertion Loss, Return Loss and Group Delay before the start of the tuning. That is they are the responses of the original circuit.

- K14 is tuned down to K14=1000 where the return loss started to degrade (Figure 14.51.m). Then the coupling K23 is tuned to recover the return loss to some extent (Figure 14.51.n).

- Then K14 is further decreased again, down to K14=200 where the return loss degraded again but the group delay started to flatten (Figure 14.51.p).

- The return loss is then recovered by tuning K23 once more (Figure 14.51.q).

- Fine trimming on Return Loss is done by tuning all of K12, K23, K34 iteratively, giving the results shown in Figures 14.49.r and s.  These responses are obtained for K14=200. We could get other (possibly better) responses by setting K14 to different levels (Exercise?).

- Now let’s comment on these results. The new CQKL section has eight degrees (four resonators). Out of the 8 degrees, four degrees are due to the quarter of the complex TZ (at unknown location!) and four degrees from the TZ’s at f=0 and f=. In the original circuit we had nine TZ’s at f=. Four of these nine TZ’s are moved to complex conjugate locations to form the quartet of TZ’s of the CQKL section. This means that the number of TZ’s at f= is reduced to five. The effect of the complex TZ’s is seen as Group Delay flattening while the effect of the reduction in the number of the TZ’s at f= is seen in the Insertion Loss plot shown in Figure 14.51.s. The out of band attenuation is reduced from about 50 dB to about 40 dB. That is, the Group Delay equalization is done at the expense of less out of band attenuation.

- Figure 14.51.t shows scaling of the shunt inductors to 10 nH.

- Figures 14.49.u-v show replacement of end inverters by capacitive-L sections and also replacement of inverters of the CQ sections by their LC equivalents using the command CQ-CT Oprs / CQ-Invs-to-CQLC. These replacements will cause slight changes in responses.

Exercise: Use CTRL+F6 and ALT+F6 commands to display Coupling Coefficients, Resonance Frequencies and External Q.

Exercise: Set K14 to different levels and retune the circuit for the best return loss to see that flatter group delay can be obtained for higher K14 at the expense of less flat delay bandwidth.

Figure 14.51. Design of a linear phase CQ filter with Finite Transmission zeros.

Figure 14.51. Design of a linear phase CQ filter with Finite Transmission zeros.

Figure 14.51. Design of a linear phase CQ filter with Finite Transmission zeros.

Figure 14.51. Design of a linear phase CQ filter with Finite Transmission zeros.

Figure 14.51. Design of a linear phase CQ filter with Finite Transmission zeros.

## EXAMPLE 14.32. Synthesis of an elliptic linear phase CQ-CT filter

In the previous example the two FTZ’s were realized by a CQ section. This was the simplest realization because the FTZ’s were symmetric about the passband center. If the FTZ’s are asymmetrically located then their realization may be simpler by using two CT sections instead of a single CQ section.

Let’s consider the specifications,

fp1=975 MHz, fp2=1025 MHz and PBRipple=0.01 dB.

Two FTZ’s are located both at  fz1=fz2=965 MHz

Keeping Nzero=1, in order to accommodate two CT sections the number of TZ’s at infinity is increased to Ninf=11.

Design steps are shown in Figure 14.52:

- Design / By Synthesis approach is selected. The options Lumped – Equiripple – Doubly Terminated – Type B are selected.

- Passband specifications and Transmission Zeros are typed in.

- The TZ’s are extracted in the order shown in the figure giving the circuit of Figure 14.52.a.

- The command Pi-Tee-L / Equal Shunt Inductors is applied to get the direct coupled resonator form of the filter (Figure 14.52.b).

- With cursor under the FTZ element on left, the command CQ-CT Oprs / Convert to CT / Left / CLL is applied to form the first CT section in CLL form corresponding to the one of the FTZ’s. Then the same command is repeated by placing the cursor under the second FTZ element (Figure 14.52.c).

- The resonators in between the two CT sections will be used to form the CQ section that will create complex TZ quartet for group delay equalization, as follows.

- With cursor under the series inductors coupling  the resonators, the command Inverters / Pi-to-Inverter / f0=1000 MHz is applied to convert the  coupling elements into inverters (Figure 14.52.d).

- It is usually preferred to have equal inductors in all shunt resonators. These inductors can be equated as follows.

- With cursor under the CT section on left the command CQ-CT Oprs / Modify CT / Set L2 (Set L3) is applied a couple of times to equate L2 and L3 to the inductor of the first resonator at left (Figure 14.52.d). The resulting transformers are shifted away by Simplify Circuit command.

- In order to equate the shunt inductors in between the inverters the cursor is placed under any inverter and the command Inverters / All Equal Left-Right / Inductors is applied (Figure 14.52.d).

- Finally with cursor under the second CT section the command CQ-CT Oprs / Modify CT / Set L2 (Set L3) is applied a couple of times to equate L2 and L3 to the inductor of the first resonator at left (Figure 14.52.d). The resulting transformers are shifted away by Simplify Circuit command.

- Thus, all inductors are equated at the level L=0.4983 nH (Figure 14.52.e). This inductance level can be scaled up to the desired level by inserting inverters at source and load ends at this stage or at any later stage of the design procedure.

- Now we are ready to form the CQ section which will equalize the group delay. This will be  done by brute force introduction of an inverter type cross-coupling between node-3 and node-6 by placing the cursor under the middle inverter (between node-4 and node-5) and applying the command CQ-CT Oprs / Convert to CQ: K14=2000000 /LfCQGenInv (Figure 14.52.e-f). This command replaces all the elements between node-4 and node-7 by a General CQ section (CQKL) with inverter type couplings and Left type diagonal cross-coupling. The diagonal cross-coupling is made ineffective automatically. Here, K14 is the inverter coupling node-1 and node-4 of the resulting CQ section. Actually these are node-3 and node-6 of the circuit respectively. Initially K14 is assigned a large value to simulate no cross-coupling case.

- Then we start to tune (decrease) K14 in a more controlled manner while observing the return loss.

- Figure 14.52.g shows the Insertion Loss, Return Loss and Group Delay before the start of the tuning. That is they are the responses of the original circuit.

- K14 is tuned down until the return loss starts to degrade. Then the coupling K23 is tuned to recover the return loss to some extent. This cycle is repeated while occasionally tuning the other inverters and the capacitors of the resonators of the CQ section. Figures 14.50.h and i show the tuned version of the circuit and  its responses.

- The effect of the complex TZ’s created by the CQ section is seen as Group Delay flattening. The complex conjugate quartet of TZ’s are created by shifting four TZ’s from f=. The effect of the reduction in the number of the TZ’s at f= is seen in the Insertion Loss plot shown in Figure 14.51.s. The out of band attenuation is reduced from about 70 dB to about 60 dB. That is, the Group Delay equalization is done at the expense of less out of band attenuation.

- It should be noted that group delay has a tilt to one side. This is because of the sharp skirt slope on low side where the two FTZ’s are located.

Exercise: Set K14 to different levels and retune the circuit for the best return loss to see that flatter group delay can be obtained for higher K14 at the expense of less flat delay bandwidth.

Exercise: Scale all shunt inductors to 10 nH.

Exercise: Apply the command Parasitics / All Lossy Resonators: Q=1000 to the scaled circuit to see the effects of finite Q of the resonators.

Exercise: Use CTRL+F6 and ALT+F6 commands to display Coupling Coefficients, Resonance Frequencies and External Q.

Figure 14.52. Design of a linear phase CQ-CT filter.

Figure 14.52. Design of a linear phase CQ-CT filter.

Figure 14.52. Design of a linear phase CQ-CT filter.

14.11.2. LOWPASS PROTOTYPE TABLES FOR SOME HIGH ORDER LINEAR PHASE CQ FILTERS

The approach described in the previous examples can also be used on inverter coupled lowpass prototype filters to form lowpass prototype tables for linear phase CQ filters which can be used to design BP filters through the classical Prototype-to-BP mapping. The method is applied on the lowpass prototype filters with shunt capacitors coupled with inverters. Cross coupling inverters are inserted by brute force to form Lowpass CQ sections (CQLP elements), as was described in Figure 14.52.d. Tables are prepared  for prototypes with degrees 10-to-24 to get an idea on delay and insertion loss properties (Figure 14.53). It is seen that the same type of delay variation exists in all filters regardless of the number of resonators.  Another point which worth mentioning is the fact that as the number of CQ section increases the flat delay bandwidth decreases slightly because of the increase in the number of transmission zeros at s=, hence the increase in selectivity.

These LP Prototype CQ filters can be placed into Filpro through the command Design / By Inserting Elements and saved. Then Design / By User Defined Prototype approach they can be used to design the targeted linear phase bandpass filters, as described in the following example.

Figure 14.53. Lowpass prototype tables for linear phase CQ filters.

Figure 14.53. Lowpass prototype tables for linear phase CQ filters (continued).

Figure 14.53. Lowpass prototype tables for linear phase CQ filters (continued).

Figure 14.53. Lowpass prototype tables for linear phase CQ filters (continued).

Figure 14.53. Lowpass prototype tables for linear phase CQ filters (continued).

## EXAMPLE 14.32. Linear phase bandpass CQ-filter design  using LP Prototypes

Let’s design a bandpass filter with following specifications:

fp1=975 MHz,   fp2=1025 MHz,  PBRipple=0.01 dB

We will use the degree 10 LP Prototype with three CQ sections shown in Figure 14.53. This will lead to a 10 resonator filter. Let’s select the option with K14=6. Design command set is shown in Figure 14.54.

- First the LP Prototype is formed on Filpro using the command Design / By Inserting Element. First a shunt capacitor is inserted with value C=0.8102. Then Copy/Paste commands are used to insert the other shunt capacitors. Then a CQLP element is inserted and other CQLP elements are placed by using Copy/Paste command. The element values of the CQLP sections are typed in using Change command (Short Cut key: Space Bar) giving the circuit shown in Figure 14.54.b.  It is saved as CQPROT10. Its responses are shown in Figure 14.54.c.

- Next, this LP prototype is used to design the bandpass filter using the commands Design / By User Defined Prototype as described in Figure 14.54.d. At the end of the design Change Units command must be used to set the inductor and capacitor units as desired, because the units were automatically set to microHenry and microfarad during the design of the LP prototype.

- Figure 14.52.e shows the resulting linear phase CQ filter. Its responses are shown in Figures 14.54.f and g.

- If flatter group delay is desired we can use the prototypes with K14=8 or K14=12. However it should be noted that flatter group delay is accompanied by a slight narrowing in the flat delay bandwidth.

Figure 14.54. Design of a linear phase CQ filter with 10 resonators using LP Prototype.

Figure 14.54. Design of a linear phase CQ filter with 10 resonators using LP Prototype.

Figure 14.54. Design of a linear phase CQ filter with 10 resonators using LP Prototype.

REFERENCES AND SUGGESTED READINGS

- A. E. Atia and A. E. Williams, “Narrow Bandpass Waveguide Filters”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-20, pp. 258-265, April 1972.

- G. Pfitzenmaier, “Linear Phase Microwave Bandpass Filters”,  Pro. 3rd Eu. Mic Conf. B.2.1, 1973.

- A. E. Atia and A. E. Williams, “Nonminimum-phase, Optimum Amplitude Bandpass Waveguide Filters,” IEEE Trans. Microwave  Theory and Tech. Vol. MTT-22, pp. 425-432, April 1974.

- R. J. Wenzel, “Solving the Approximation Problem for Narrowband Bandpass Filters with Equiripple Passband Response and Arbitrary Phase Response”, 1975 IEEE MTT-S , Digest, p. 50, 1975.

- R. Levy, “Filters with Single Transmission Zeros at real or Imaginary Frequencies”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-14, No. 4, pp. 172-181, April 1976.

- J. D. Rhodes and I. H. Zabalawi, “Synthesis of symmetrical dual mode in line prototype networks”, Int. J. Circuit Theory and applications, vol. 8, pp. 145-160, 1980.

- J. D. Rhodes and R. J. Cameron, “General extracted pole synthesis technique with applications to low-loss TE011 mode filters”, IEEE Trans. MTT, Vol. 28, No. 9, pp. 1018-1026, Sept. 1980.

- R. J. Cameron and J. D. Rhodes, “Asymmetric Realizations for Dual-Mode Bandpass Filters”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-29, pp. 51-58, Jan. 1981.

- R. J. Cameron, “General Prototype Network Synthesis Methods for Microwave Filters”, ESA J., Vol. 6, no. 2, pp. 193-206, 1982.

[10] L. Field, I. C. Hunter, and J. G. Gardiner, “Asymmetric Bandpass Filter Using A Novel Microstrip 
        Circuit”, IEEE  Microwave and Guided Wave Letters, vol. 2, no. 6, pp.147-149, June 1992.

[11] H. C. Bell, Jr., “Canonical Asymmetric Coupled Resonator Filters”, IEEE Trans. Circuits and  Systems, Vol. CAS. 26, pp. 389-394, June 1982.

[12] R. Levy, “Direct Synthesis of cascade quadruplet (CQ) filters”, IEEE Trans. MTT, Vol. 43, No. 12,   
        pp. 2940-2945,  Dec. 1995.

[13] R. Herstig, R. Levy, K. Zaki, “Synthesis and design of cascaded trisection (CT) dielectric resonator   
       filters”, European Microwave Conference Proc., pp. 784-791, Israel, 1997.

[14] K. Zaki, R. Herstig,“Trisection Combline and Dielectric Resonator Filters”, IECS’97, December 15-18, 1997, Cairo, Egypt.

[15] H. L. Thal, “Design of microwave filters with arbitrary responses”, Microwave and Millimeter-Wave Computer aided Engineering, Vol. 7, No. 3, pp. 208-221, May 1997.

[16] W. A. Atia, K. A. Zaki and A. E. Atia, “Synthesis of general topology multiple coupled resonator filters by optimization”,  IEEE, MTT-S-1998 Proceedings, pp. 821-814, Maryland, 1998.

[17] G. Macchiarella, S. G. D’Oro, “Design of generalized comb filters with asymmetric transmission zeros using arbitrary cascaded Triplet and Quadruplet sections”, European Microwave Conference Proceedings, Vol.2, pp. 179-183, Amsterdam, 1998.

[18] R. J. Cameron, A. R. Harish, C. J. Radcliffe, “Synthesis of Advanced Microwave Filters Without Diagonal Cross-Couplings”, IEEE Trans. MTT, Vol. 50, No. 12, pp. 2862-2872, Dec. 2002.

[19] O. A. Sen, Y. Sen, N. Yildirim, “Synthesis of cascaded quadruplet filters involving complex transmission zeros”, IEEE MTT-S-2000 Int. Microwave Symp. Digest, Vol.3, pp 1177-1180, Boston, June 2000.

[20] N. Yildirim, O. A. Sen, Y. Sen, M. Karaaslan, D. Pelz, “A Review of Cascade Synthesis Theory Covering Cross-coupled Filters”, IEEE, Trans. MTT, Vol. 50, No: 6, pp. 1536-1543, June 2002.

[21] R. J. Cameron, “Advanced Coupling Matrix Synthesis Techniques for Microwave Filters”, IEEE Trans. MTT, Vol. 51, No. 1, pp. 1-10,  Jan. 2003.

[22] O. A. Sen, Y. Sen, N. Yildirim, “Synthesis of cascaded quadruplet filters involving complex transmission zeros”, IEEE MTT-S-2000 Int. Microwave Symp. Digest, Vol.3, pp 1177-1180, Boston, June 2000.

[23]  N. Yildirim, O. A. Sen, Y. Sen, M. Karaaslan, D. Pelz, “A Review of Cascade Synthesis Theory Covering Cross-coupled Filters”, IEEE, Trans. MTT, Vol. 50, No: 6, pp. 1536-1543, June 2002.

[24] J. S. Hong, M. J. Lancaster, “Microstrip Filters for RF/Microwave Applications”, John Wiley and Sons, 2001.