CHAPTER 15

SYMMETRIC CANONICAL AND NONCANONICAL CROSS-COUPLED FILTERS

15.1. INTRODUCTION

Stringent amplitude and phase requirements of most communication systems are difficult to satisfy by the cascaded element filters. Such requirements led to development of cross-coupled filters where nonadjacent elements of cascaded element filters coupled to each other to produce either a finite transmission zero (FTZ) close to the passband edges or to produce complex or s= axis finite transmission zeros to flatten the delay response leading to linear phase filters [1]-[20]. One classification of such filters is shown in Figure 15.1. The most general version of cross-coupled filters is named as asymmetric canonical cross-coupled filters and is described in Figure 15.1.a on a ten resonator filter. In this figure the circles denote resonators and heavy interconnecting lines are coupling elements. Such filters can produce maximum possible number of FTZ’s. The FTZ’s can be asymmetrically placed. (Complex TZ’s form complex conjugate quartets, hence symmetric always!). The diagonal cross-coupling elements are responsible for the asymmetry of the FTZ’s. The maximum number of FTZ’s that an N-resonator cross-coupled filter can realize is N-2. The second group of filters shown in Figure 15.1.b are named as symmetric canonical cross-coupled filters. They can also realize maximum possible number of FTZ’s, but the FTZ’s are symmetric with respect to the passband. The class of filters shown in Figure 15.1.c is named as symmetric noncanonical cross-coupled filters. Here the term noncanonical means that the number of FTZ’s that the filter can realize is less than  N-2. This is because of the absence of some of the cross-coupling elements.

Figure 15.1. A classification for cross-coupled filters.

The subclasses of cross-coupled filters termed as Cascaded Triplet and Cascaded Quadruplet filters which can realize a single s=jw axis transmission zero or two jw axis transmission zeros respectively are described in Chapter 14. This chapter is devoted to physically symmetric filters with cross-couplings between symmetric nodes.  That is, symmetric canonical or noncanonical filters will be designed. On Filpro, such filters are treated in two distinctly different approaches:

In the first approach approximate brute force technique is used to form Quadruplets (CQ filters), QuinTuplets, SixTuplets, SevenTuplets, EightTuplets, NineTuplets and TenTuplets where each N-Tuplet is treated as a single element. The transmission zeros can be either complex or on jw-axis or both. The resulting filters have to be optimized to reach the targeted response. This approach is faster and easier to use. However the effects of approximations on response may necessitate optimization. Although optimization can be carried out on Filpro for most cases using Tune Element command and/or Monte-Carlo-Best Case routines, some filters may necessitate the use of more general circuit simulators and optimizers.

In the second approach the lowpass prototype tables are given in terms of even and odd halves of the symmetric cross-coupled filters. The bandpass versions are designed by using these LP prototypes. The prototype element values are obtained either through a special cross-coupled synthesis algorithm or taken from the open publications. The following cross-coupled filter classes will be described using that approach:

- Design of filters with a single cross-coupling element creating either a single transmission zero on real axis of s=+jw plane producing a linear phase response or producing a symmetric pair of jw-axis finite transmission zeros on both sides of the passband. Since a single cross coupling forms a single coupled quadruplet of resonators, this filter is actually a special case of CQ (Cascaded Quadruplet) filters which can be designed in exact way using CQ approaches described in Chapter-25. Therefore this technique has just a historical value hence can be skipped.

- Design of filters with double cross-couplings having both s= and s=jw axis or complex finite transmission zeros leading to either linear phase filters with single loss pole on each side of passband or selective filters having double loss poles on each side of the passband. The approach starts with an approximate initial design and then optimized by tuning elements.

- Design of filters with three or more cross-couplings by starting from an approximate initial design and then optimizing by tuning elements.

- Linear phase filters with multiple cross-couplings designed from the classical (published) tabular lowpass prototypes.

- Linear phase filters with a single jw-axis finite transmission zero designed from tabular lowpass prototypes.

- Cross-coupled filters with different numbers of jw-axis and -axis finite transmission zeros, from the novel lowpass prototypes developed by the author for Filpro.

These filters may satisfy most of the practical requirements.

15. 2. BRUTE FORCE (APPROXIMATE) DESIGN OF N-TUPLET FILTERS

This approach can be used to design N-tuplets where the cross-couplings are inserted by brute force. The initial design starts with Design / By Prototype with Inverters approach to yield a direct coupled resonator filter with inverter couplings. Then carefully adjusted cross-coupling inverters are placed between symmetric resonators to yield a response which is close to the targeted response. Responses are then adjusted by using either Tune Element command or Monte Carlo-Best Case optimization routine of Filpro first on all cross-coupling elements and then on all the other elements. In the extreme cases one may always resort to a general simulator-optimizer software.

Let’s continue descriptions directly on examples.

15. 2. 1. CASCADED TRIPLET FILTERS

## Exact synthesis of Cascaded Triplet (CT) filters was described in the previous chapter. This section is prepared just for the sake of the completeness of the approach. In some cases it may be easier or faster to use this approach. Figure 15.2 shows a pictorial description of a Triplet element. The symbol used on Filpro is named as CTINV. Its equivalent circuit is shown in Figure 15.2.b. Note that in this symbol the resonators 1 and 3 are absent. CTINV will be cascaded by resonators 1 and 3 to form the actual triplet. It should also be noted that all the shunt inductors (Lp) are taken to be equal. The resistors Rp in parallel with LC resonators will account for the finite Q of the resonators. They are also taken to be equal. The inline coupling elements are the inverters K12 and K23.  K13 is the cross-coupling inverter.

Let’s see some examples involving CTINV elements on Filpro.

15. 2. 2. CASCADED QUADRUPLET FILTERS

## Exact synthesis of Cascaded Quadruplet (CQ) filters was described in the previous chapter. This section is prepared just for the sake of the completeness of the approach. In some cases it may be easier or faster to use this approach. Figure 15.2 shows a pictorial description of a quadruplet element. The symbol used on Filpro is named as CQBP. Its equivalent circuit is shown in Figure 15.2.b. Note that in this symbol the resonators 1 and 4 are absent. CQBP will be cascaded by resonators 1 and 4 to form a quadruplet. It should also be noted that all the shunt inductors (Lp) are taken to be equal. The resistors Rp in parallel with LC resonators will account for the finite Q of the resonators. They are also taken to be equal. The inline coupling elements are the inverters K12, K23 and K34.  K14 is the cross-coupling inverter.

Let’s see some examples involving CQBP elements on Filpro.

## Figure 15.2. A quadruplet (CQBP) and its equivalent circuit.

## EXAMPLE 15.1. Four resonator (quadruplet) filters

Let’s design four resonator filters with following specifications:

PBRipple=0.01 dB, fp1=990 MHz, fp2=1010 MHz.

It is possible to make the quadruplet filter to have either two jw-axis FTZ’s or a complex TZ quad to get linear phase response.

The design stages are shown in Figure 15.3.

- Design / By Prototype with Inverters technique is selected.

- Shunt Capacitors and Inverters option is selected.

- PBRipple and Prototype degree are typed in. After this command the LP Prototype circuit appears on the screen (Figure 15.3.a). This prototype will be used to design the desired bandpass filter.

- Filter Type command is invoked and Lumped / BPF options are selected.

- Passband and Terminations command is invoked and passband edge frequencies are typed in.

- Find Circuit command gives the direct coupled resonator filter with four resonators (Figure 15.3.b).

- All shunt inductors are equated by applying the command Inverters / All Equal Left-Right (Figure 15.3.c). The unity transformer at load end is deleted.

- Element values can be scaled to desired realization levels by inserting inverters at source and load ends. For example all shunt inductors can be scaled up to 10 nH by applying the command Inverters / Scale Cct By Ins. Invs / Specify Adjacent Ind: L=10 nH (Figure 15.3.d).

- Then these inverters can be replaced by their practical approximate equivalents. For example they can be replaced by capacitive-L sections by placing the cursor under them and applying the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L. Simplify Circuit command combines the negative shunt capacitors of the L-sections by their neighbors (Figure 15.3.e).

- This circuit is saved as FOUR.fil. It can be used to design two different cross-coupled filters, as follows.

Quadruplet filter with two FTZ’s:

- With cursor under the middle inverter the command CT-CQ-CN Oprs / Convert to CQ-CN / CQ / CQ with FTZs is applied giving the circuit of Figure 15.3.f where CQBP element is placed between node-1 and node-4. This command introduces a cross-coupling inverter K14 between resonators 1 and 4. A window appears displaying a default trial value for  K14. In order to get jw-axis FTZs the value of K14 must be negative. Its value sets the locations of the two geometrically symmetric FTZs. A pictorial description of CQBP element is shown on the right of the figure in the form of coaxial resonators. The negative cross-coupling is provided by a conducting bar with nail head ends for capacitive coupling to the inner conductors of coaxial resonators.

- The locations of FTZs are adjusted by tuning K14 using Tune Element command (Figure 15.3.g).

- Return loss appears to be degraded. It can be recovered to a great extent by tuning K23 (opposite to K14). Then all other elements are also tuned slightly for fine tuning of the responses (Figure 15.3.g).

- The finite Q of resonators can be taken into account by applying the command Parasitics / All Lossy Resonators: Q=..  (Figure 15.3.h-i-k). This command replaces the lossless LC resonators by their lossy counterparts with the specified Q.

- In Figure 15.3.m application of various commands are described to display coupling coefficients, nodal resonance frequencies and nodal Voltage-Current-Power variables as follows:

  - Coupling Coefficients and Resonance Frequencies of resonators can be displayed by placing the cursor under the CQBP element and pressing the key combination ALT+F6.

  - The keys CTRL+F6 displays External Q and Coupling Coefficients.

  - Node Voltage-Current-Power variables can be displayed by pressing the keys ALT+F8.

## Linear phase Quadruplet filter:

The direct coupled resonator filter FOUR.fil can also be converted into a linear phase quadruplet by introducing a cross-coupling inverter K14 with the same sign as K23 as follows:

- The circuit of Quad1.fil is loaded (Figure 15.3.n).

- With cursor under the middle inverter the command CT-CQ-CN Oprs / Convert to CQ-CN / CQ / Linear Phase CQ is applied to introduce a cross-coupling inverter K14 between resonator 1 and resonator 4. A window appears displaying a trial value for K14. In order to get a linear phase response the quadruplet must create a complex transmission zero. This is possible by selecting the sign of K14 as the same as K23, that is a positive value. K14 can be tuned to adjust the desired delay response. Figure 15.3.p shows the resulting circuit.

- Figure 15.3.q shows the insertion loss and return loss responses for the trial K14.

- Figure 15.3.r shows the variation of delay response as value of K14 is changed using Tune Element command.  After setting the delay response to the targeted level all the other parameters can be tuned to refine the return loss level.

Figure 15.3. Design of quadruplet filters.

Figure 15.3. Design of quadruplet filters.

Figure 15.3. Design of quadruplet filters.

Figure 15.3. Design of quadruplet filters.

## EXAMPLE 15. 2. Ten resonator Cascaded Quadruplet (CQ) filters

The approach described in the previous example can be applied to higher order direct coupled resonator filters to form cascaded quadruplets to design CQ filter. As an example let’s consider a ten resonator CQ filter with the same passband specifications as the previous examples:

PBRipple=0.01 dB, fp1=990 MHz, fp2=1010 MHz.

- As in the previous example, first a 10 resonator direct coupled resonator filter is designed using Design / By Prototype with Inverters approach (Figure 15.4.a-b).

- All shunt inductors are equated by applying the command Inverters / All Equal Left-Right. The transformer at load end is deleted (Figure 15.4.c)

- The shunt inductors are scaled to 10 nH by applying the command Inverters / Scale Cct by Inserting Inverters / Specify Ind: L=10 nH (Figure 15.4.c-d).

- Then the inverters at source and load ends are replaced by their capacitive-L equivalents by applying the command Inverters / Replace Inv by / End Inv to / Capacitive-L. Simplify Circuit command is used to absorb the negative shunt capacitors by their positive neighbors (Figure 15.4.d-e). The resulting circuit is ready for conversion into cross-coupled form. It is saved as TEN.fil. It can be used to design several different cross-coupled filters, as will be described.

- Let’s first design a CQ filter with maximum number of FTZ’s to get maximum selectivity. With cursor under the second, fifth and eighth inverters the command CT-CQ-CN Oprs / Convert to CQ-CN / CQ / CQ with FTZs is applied one by one to introduce cross-coupling inverters forming three quadruplets, hence a CQ filter (Figure 15.4.f). The displayed default K14 values are accepted.

- Figure 15.4.g shows a pictorial description of the structure in coaxial resonator realization. Resonators 4 and 7 seems to be loaded by too many couplings. This may lead to tuning difficulties. Uch cases can be avoided by using two more resonators (Exercise?).

- Figure 15.4.h shows the insertion loss and return loss responses. The FTZ locations can be adjusted by tuning K14 of each CQBP section. Then the return loss can be adjusted to the specified level by tuning all the other inverters. It is easiest to get equal ripple return loss by using Monte Carlo-Best Case routine. Figure 15.4.i show the circuit, return loss, insertion loss and coupling coefficients after such an optimization.

- Figures 15.4.k-to-n describes the design of a linear phase CQ filter where each of the three CQBP sections are selected as Linear Phase sections.

Exercise: It is also possible to select some of the CQBP sections as linear phase and the others as having FTZ’s to get a narrower flat delay bandwidth (Exercise?).

Figure 15.4. Design of ten resonator CQ filters.

Figure 15.4. Design of  ten resonator CQ filters.

Figure 15.4. Design of  ten resonator CQ filters.

Figure 15.4. Design of ten resonator CQ filters.

15. 2. 3. QUADRUPLETS WITH  TRANSMISSION LINE CROSS COUPLING

In some applications introduction of 1-to-4 cross-coupling may be troublesome because of possible physical distances between resonators. In such cases one may use transmission line for cross coupling purposes. The resulting quadruplet is named as CQTL. Figure 15.5 shows a pictorial description and development of the circuit model by starting from CQBP element. The cross-coupling inverter K14 of the CQBP element is first replaced by its constant reactance Pi-model. Then the series arm of the Pi-section is split into two equal parts and a transmission line piece is inserted in between the two pieces.

Figure 15.5. Development of the circuit model of CQTL element.

The delay associated with the transmission line may be used to move the two FTZ’s to the same stopband. Let’s see some examples on Filpro.

## EXAMPLE 15. 3. Quadruplets with transmission line cross-coupling

Let’s take the specifications of the filter to be the same as Example-15.1:

PBRipple=0.01 dB,  fp1=990 MHz, fp2=1010 MHz.

Four resonators will be used.

- First the direct coupled resonator version of the filter is designed in the same way as in Example-1, leading to the same filter FOUR.fil (Figure 15.6.a-e).

- In order to get two FTZs in lower stopband the cursor is placed under the middle inverter and the command CT-CQ-CN Oprs / Convert to CQ-CN / CQ / CQTL-LowSide FTZs: Lng14=…  is applied to give the circuit shown in Figure 15.6.f where a CQTL element exists. On the right side of the figure a pictorial description of CQTL element is given where the resonators and the coupling line are shown in coaxial (TEM) form.

- In this command a default value for the length of coupling transmission line, Lng14 is displayed. This length is slightly greater than half wavelength at passband center. This command introduces a transmission line inserted in between two halves of a coupling inverter K14. The characteristic impedance Z14 of the transmission line is set by the user at a realizable level. Its default value is 100 ohms. The parameters Lng14, Z14 and K14 set the locations of the FTZs. The default value of K14 is calculated by Filpro. For low side FTZ’s K14 is negative. That is the transmission line will couple to resonators 1 and 4 capacitively.

- For the default values two FTZ’s appear within the lower stopband as shown in Figure 15.6.g. Tune Element command and Monte Carlo-Best Case optimization routine can be used to adjust all parameters to shape both insertion loss and return loss.

- Figure 15.6.h show the circuit after optimization. Figure 15.6.i shows the optimized response. Return loss settled at less than the originally specified level. It is observed that bandwidth tends to widen towards the side where the FTZs exists. For this reason in general a more professional optimizer would be a better choice because Monte Carlo-Best Case routine works only on the return loss response, having no control on insertion loss.

- In order to get two FTZs in upper stopband the circuit of FOUR.fil is loaded (Figure 15.6.k), the cursor is placed under the middle inverter and the command CT-CQ-CN Oprs / Convert to CQ-CN / CQ / CQTL-HighSide FTZs: Lng14=…  is applied to give the circuit shown in Figure 15.6.m. In this command a default value for the length of coupling transmission line, Lng14 is slightly less  than full wavelength at passband center. K14 is calculated by Filpro and it is positive for high side FTZ’s. That is the transmission line will couple to resonators inductively (magnetic coupling).

- For the default values two FTZ’s appear within the upper stopband. In Figure 15.6.n the two FTZ’s coincided. Tune Element command and Monte Carlo-Best Case optimization routine can be used to adjust all parameters to shape both insertion loss and return loss.

- Figure 15.6.p show the circuit after optimization. Figure 15.6.q shows the optimized response. It is observed that bandwidth tends to widen towards the side where the FTZs exists.

Figure 15.6. Design of CQ filters with transmission line cross-couplings.

Figure 15.6. Design of CQ filters with transmission line cross-couplings.

Figure 15.6. Design of CQ filters with transmission line cross-couplings.

Figure 15.6. Design of CQ filters with transmission line cross-couplings.

15. 2. 4. FIVE RESONATOR CROSS-COUPLED FILTERS (QUINTUPLETS)

Cross-coupled filters with odd number of resonators are usually used for realization of asymmetric FTZs. Figure 15.7 shows symmetric canonical quintuplet. The cross-coupling elements are K15 and K24. On Filpro, the symbol used to denote such quintuplets is named as Quintuplet-1 to differentiate it from another quintuplet that will be described next. Asymmetric canonical quintuplets have also a diagonal cross-coupling K25 and can create 3 FTZ’s. Diagonal cross-couplings are usually troublesome to realize. In this section the design of some symmetric non-canonical quintuplets on Filpro will be described. By the term non-canonical we mean, in addition to the absence of the diagonal cross-coupling K25, there will be no cross coupling K24 either. Thus, the quintuplet that will be described here is a highly simplified version. The price paid for simplicity is that the number of FTZ’s will be less than three. When K24 is made ineffective the quintuplet can realize only one FTZ in either stopband. In this respect such a quintuplet may form an alternative to CT filters in the cases where it is difficult to adjust couplings of CT alternatives because of interaction of the fields in the coupling regions due to proximity of coupling apertures.

It is also possible to take the cross-coupling K24 into consideration on Filpro, but at the expense of some optimization trials.

Figure 17.7. Symmetric canonical quintuplet and its circuit model.

## EXAMPLE 15. 4. Non-canonical quintuplets with a single FTZ

Let’s design a five resonator filter in the form of non-canonical quintuplet realizing either a low side or a high side FTZ.  Specifications are as follows:

PBRipple=0.01 dB, fp1=990 MHz,  fp2=1010 MHz.

- The design approach is similar to the previous examples. That is, a direct coupled resonator version is designed and then converted into a quintuplet. The approach is described in Figure 15.8.

- Design / By Prototype with Inverters approach is used to design the filter (Figure 15.8.a-b).

- The command Inverters / All Equal Left-Right/ Inds is applied to equate all the shunt inductors (Figure 15.8.c).

- All the shunt inductors are scaled up to 10 nH (e.g.) by applying the command Inverters / Scale Cct by Ins. Inverters / Specify Inductor: L=10 nH (Figure 15.8.d).

- The end inverters are replaced by their capacitive-L section equivalents. The resulting circuit is saved as FIVE.fil (Figure 15.8.e).

- Quintuplet with Low Side FTZ:

- In order to convert the circuit into a quintuplet with a single FTZ in lower stopband, the cursor is placed under the LC resonator in the middle of the circuit and the command CT-CQ-CN Oprs / Conv. to CQ-CN / Quint-1 / LowSide FTZ: K15=… is applied. Here the cross-coupling inverter value K15 is a trial value setting the location of the FTZ. A negative K15 (capacitive cross-coupling) leads to a FTZ in lower stopband. Accepting the default value for K15, we get the circuit shown in Figure 15.8.f. In this circuit the quintuplet is named as Quint-1 (QN1) to differentiate it from another quintuplet that will be considered later. A pictorial description of Quint-1 is given on the right of Figure 15.8.f. In this circuit we have only one cross-coupling, K15. The second possible cross-coupling K24 is made infinite to make it ineffective. It is possible to adjust K24 (together with all other parameters of  Quint-1) to create three FTZs (Exercise?).

- Figure 15.8.g shows the insertion loss and return loss responses. Return loss response needs some adjustment. This is possible by tuning the parameters K12-K23-C2-C3. The easiest way is to use Monte Carlo-Best Case routine. Figures 15.8.h-i show the results after optimization using this routine.

- Figures 15.8.k-to-n describe the procedure for formation of a quintuplet with a high side FTZ. Note that K15 is positive (inductive cross-coupling).

Figure 15.8. Design of quintuplets with a single FTZ.

Figure 15.8. Design of quintuplets with a single FTZ.

Figure 15.8. Design of quintuplets with a single FTZ.

# EXAMPLE 15. 5. Non-canonical quintuplets with two FTZs in one stopband

Another quintuplet can be formed by brute force insertion of a fifth resonator coupled to resonators 1 and 4 of a quadruplet as described in Figure 15.9.

Figure 15.9. Quintuplet-2 and its circuit model.

Filter specifications are the same as the previous examples:

PBRipple=0.01 dB, fp1=990 MHz, fp2=1010 MHz.

- Design starts by formation of the same four resonator direct coupled filter as in the Example 15.1, saved as FOUR.fil (Figure 15.10.a-e).

- In order to get two Low side FTZs, with cursor under the middle inverter the command CT-CQ-CN Oprs / Conv. to CQ-CN /Quint-2 / 2 Lowside FTZs: K15=… is applied yielding a quintuplet named as Quint-2 (Figure 15.10.f).

- A pictorial description of the element Quint-2 is shown on the right of Figure 15.10.f. Note that there is no cross-coupling between resonators 1 and 4 (K14=infinity). The fifth resonator is inserted between resonators 1 and 4 by brute force. For two LowSide FTZ’s, all the couplings come out to be negative (capacitive).

- Insertion loss and return loss responses are shown in Figure 15.10.g. Return loss response needs some improvement and a bandwidth change is observed. These are normal because a fourth order filter is converted into a fifth order filter by brute force. Tuning/ optimization can be applied to improve the response.

- Figures 15.10.h-i-k describe the formation of the quadruplet with two FTZ’s in high side. Note that the couplings K12-K23 are positive while K15 is still negative.

- Figures 15.10.m-n show the results of Monte Carlo-Best Case optimization process. Bandwidth had widened and return loss level is decreased compared to the original specifications.

Figure 15.10. Design of quintuplets with two FTZ’s.

Figure 15.10. Design of quintuplets with two FTZ’s.

Figure 15.10. Design of quintuplets with two FTZ’s.

Figure 15.10. Design of quintuplets with two FTZ’s.

15. 2. 4. SIXTUPLETS

Filpro can be used to design symmetric canonical or non-canonical sixtuplets by brute force introduction of cross-couplings. Figure 15.11 describes symmetric canonical sixtuplet. A sixtuplet can realize at most four FTZ’s symmetrically located on the two sides of the passband through the cross-couplings K16 and K25. Asymmetrically located FTZ’s necessitates the use of diagonal cross-couplings K26 and K35. Filpro can design only symmetric canonical sixtuplets.

Let’s see some examples.

Figure 15.11. Symmetric sixtuplet and its circuit model.

EXAMPLE 15. 6. Sixtuplet filters

Let’s design some sixtuplet filters with the same passband specifications as the previous examples:

PBRipple=0.01 dB, fp1=990 MHz,  fp2=1010 MHz.

Design steps are shown in Figure 15.12.

- Design starts by formation of a six resonator direct coupled filter saved as SIX.fil (Figure 15.12.a-e).

- In order to get maximum selectivity, wth cursor under the middle inverter the command CT-CQ-CN Oprs / Conv. to CQ-CN / SIXtuplet / 4 FTZs is applied yielding a sixtuplet named as C6 (Figure 15.12.f). This sixtuplet have four FTZs, two in each stopband. Note that only K25 is negative (Capacitive). The Coupling Coefficients and External Q are displayed by pressing the keys CTRL+F6 and selecting the option DirCpld-CT-CQ-CN.

- A pictorial description of the sixtuplet is shown on the right of Figure 15.12.f.

- Insertion loss and return loss responses are shown in Figure 15.12.g. Return loss response needs some improvement which can be done using Tune Element command.

- Figures 15.12.h-i-k-m describe the formation of a linear phase sixtuplet with two FTZ’s, one in each stopband. Note that in this case only the coupling K16 is negative.

- Figures 15.12.n-p-q-r describe the formation of a linear phase sixtuplet. Note that in this case all the couplings are positive (same sign!).

- Responses can be adjusted by optimization.

Figure 15.12. Design of sixtuplet filters.

Figure 15.12. Design of sixtuplet filters.

Figure 15.12. Design of sixtuplet filters.

Figure 15.12. Design of sixtuplet filters.

EXAMPLE 15. 7. A ten resonator filter having a Sixtuplet element

Let’s design a ten resonator filter having a sixtuplet element. The passband specifications are the same as the previous examples:

PBRipple=0.01 dB, fp1=990 MHz,  fp2=1010 MHz.

Design steps are shown in Figure 15.13.

- Design starts by formation of a ten resonator direct coupled filter which is the same as in Example 15.2, hence saved as TEN.fil (Figure 15.13.a-e).

- In order to get some selectivity, with cursor under the middle inverter the command CT-CQ-CN Oprs / Conv. to CQ-CN / SIXtuplet / 4 FTZs is applied converting the middle part of the circuit into a sixtuplet (Figure 15.13.f). Note that only K25 is negative (Capacitive). The Coupling Coefficients and External Q are displayed by pressing the keys CTRL+F6 and selecting the option DirCpld-CT-CQ-CN.

- A pictorial description of the sixtuplet is shown on the right of Figure 15.13.g.

- Insertion loss and return loss responses are shown in Figure 15.13.h. This sixtuplet is intended to create four FTZs, two in each stopband. However insertion loss response indicates that the default values used for cross-coupling inverters are not close enough to the actual values. Therefore some tuning is needed to uncover the FTZ’s and also imrove the return loss.

- Using Tune Element command the cross-coupling inverters K16 and K25 are adjusted to form the four FTZ’s and then Monte Carlo-Best Case routine is applied to improve the return loss. The results obtained after a few optimization trials are shown in Figure 15.13.i.

Figure 15.13. Design of a ten resonator filter having a sixtuplet element.

Figure 15.13. Design of a ten resonator filter having a sixtuplet element.

Figure 15.13. Design of a ten resonator filter having a sixtuplet element.

15. 2. 5. SEVENTUPLETS

Filpro can be used to design symmetric non-canonical seventuplets by brute force introduction of cross-couplings (Figure 15.14). Canonical seventuplets can realize five FTZs through the cross-couplings K17, K26 and K35 and the diagonal cross-couplings K27 and K36. On Filpro, we will consider the cases where there is no diagonal cross-coupling. That is, the seventuplet will be physically symmetric. Further, default approximate values for cross-coupling inverters are formulated only for the cases where the cross-coupling K35 is made ineffective. Thus, we will be dealing with structures much simpler than the canonical counterparts. The price paid for the simplicity is that the number of FTZ’s will be less than five. Formulations on Filpro are made for obtaining either one or three FTZ’s as will be described in the following example.

Figure 15.14. Symmetric seventuplet and its circuit model.

EXAMPLE 15. 8. Seventuplet filters

Filter passband specifications are the same as the previous examples:

PBRipple=0.01 dB,  fp1=990 MHz,  fp2=1010 MHz.

- In order to design a seventuplet cross-coupled filter we start by first designing its direct coupled resonator version as shown in Figure 15.15.a-e. The resulting filter is saved as SEVEN.fil.

- Filpro can convert this circuit into a symmetric seventuplet that have three FTZ’s, one in one stopband and two in the other stopband. It is also possible to make one of the two cross-coupling elements ineffective, thus create only one FTZ in any stopband.

- In order to get two FTZ’s in lower stopband, the cursor is placed under the middle LC resonator and the command CT-CQ-CN Oprs / Conv. to CQ-CN / SEVENtuplet / Two LowSide FTZs is applied converting the whole circuit into a seventuplet (Figure 15.15.f).

- Note that for this case the cross-coupling inverter K17 is positive while K26 is negative (Capacitive). A pictorial description of the seventuplet is shown on the right of Figure 15.15.g.

- Insertion loss and return loss responses are shown in Figure 15.15.h. Return loss response indicates that the default values used for cross-coupling inverters and resonance frequencies of resonators are not close enough to the actual values. Therefore some tuning is needed to improve the return loss.

- Figures 15.15.i-to-n describes the design of the second option where two FTZ’s are created in upper stopband and one FTZ in lower stopband. Note that in this case K17 is negative and K26 is positive. Return loss is heavily degraded as in the previous case.

- The cross-coupling inverters K17 and K26 can be tuned to adjust the locations of FTZs. Return loss can be improved using Monte Carlo-Best Case routine (Figure 15.15.p).

Figure 15.15. Design of some non-canonical seventuplet filters.

Figure 15.15. Design of some non-canonical seventuplet filters.

Figure 15.15. Design of some non-canonical seventuplet filters.

Figure 15.15. Design of some non-canonical seventuplet filters.

15. 2. 6. EIGHTUPLETS

Filpro can be used to design symmetric canonical or non-canonical eighttuplets by brute force introduction of cross-couplings (Figure 15.16). Canonical eighttuplets can realize six symmetrically located FTZ’s  through the cross-couplings K18, K27 and K36. Asymmetric FTZ locations require the existence of diagonal cross-couplings K28 and K37. Filpro will be used to design only symmetric eighttuplets. Hence we will not have diagonal cross-couplings. Let’s see some such symmetric canonical examples.

Figure 15.16. Symmetric canonical eighttuplet and its circuit model.

## EXAMPLE 15. 9. Eightuplet filters

Filter passband specifications are the same as the previous examples:

PBRipple=0.01 dB,  fp1=990 MHz,  fp2=1010 MHz.

- In order to design an eighttuplet cross-coupled filter we start by first designing its direct coupled resonator version as shown in Figure 15.17.a-e. The resulting filter is saved as EIGHT.fil.

- Filpro can convert this circuit into a symmetric eighttuplets that can realize six FTZs or 4 FTZ’s plus one complex TZ quartet or two FTZs plus 2 complex TZ quartets or 3 complex TZ quartets through the cross-couplings K18, K27 and K36. On Filpro, default approximate values for cross-coupling inverters are formulated for all four cases.

- In order to get six FTZ’s (three in each stopband) the cursor is placed under the middle inverter and  the command CT-CQ-CN Oprs / Conv. to CQ-CN / EIGHTtuplet / Six FTZs is applied converting the whole circuit into an eighttuplet (Figure 15.17.f).

- Note that for this case the cross-coupling inverters K18 and K36 are negative (Capacitive) while K27 is positive. A pictorial description of the eighttuplet is shown in Figure 15.17.g.

- Insertion loss and return loss responses are shown in Figure 15.17.h. The six FTZ’s can be seen. Return loss response indicates that the default values used for cross-coupling inverters need some tuning.

- Figures 15.17.i-to-n describe the design of the second option where three complex TZ’s are realized to form a linear phase filter with no FTZ. Note that in this case all the cross-coupling inverters are positive.

- Figures 15.17.p-to-s describe the design of the third option where two complex TZ’s and two FTZ’s are realized to form a linear phase filter with two FTZs. Note that in this case only K18 is negative.

- Figures 15.17.t-to-w describe the design of the fourth option where one complex TZ and four FTZ’s are realized to form a linear phase filter four FTZs. Note that in this case the cross coupling inverters K27 and K36 are negative.

- Tune Element command can be used to adjust locations of FTZ’s or to adjust group delay flatness while Monte Carlo-Best Case routine can be used to improve the return loss.

Figure 15.17. Design of canonical eighttuplet filters.

Figure 15.17. Design of canonical eighttuplet filters.

Figure 15.17. Design of canonical eighttuplet filters.

Figure 15.17. Design of canonical eighttuplet filters.

Figure 15.17. Design of canonical eighttuplet filters.

15. 2. 7. NINETUPLETS

Filpro can be used to design symmetric canonical or non-canonical ninetuplets by brute force introduction of cross-couplings (Figure 15.18). Canonical ninetuplets can realize seven FTZs through the cross-couplings K19, K28, K37, K46 and the diagonal cross-couplings K29, K38 and K47. On Filpro, we will consider the cases where there is no diagonal cross-coupling. That is, the ninetuplet will be physically symmetric. Further, default approximate values for cross-coupling inverters are formulated only for the cases where the cross-couplings K37 and K46 are made ineffective. Thus, we will be dealing with structures much simpler than the canonical counterparts. The price paid for the simplicity is that the number of FTZ’s will be less than seven. Formulations on Filpro are made for obtaining either one or three FTZ’s as will be described in the following example.

Figure 15.18. Symmetric ninetuplet and its circuit model.

EXAMPLE 15. 10. Ninetuplet filters

Filter passband specifications are the same as the previous examples:

PBRipple=0.01 dB,  fp1=990 MHz,  fp2=1010 MHz.

- In order to design a ninetuplet cross-coupled filter we start by first designing its direct coupled resonator version as shown in Figure 15.19.a-e. The resulting filter is saved as NINE.fil.

- Filpro can convert this circuit into symmetric canonical ninetuplets that can realize the following special cases. In one case we place only one cross-coupling, namely K19 (or K28 or K37 or K46) to get one FTZ either in lower stopband or in upper stopband depending on the sign of the cross-coupling inverter. In the other two cases three FTZ’s are realized by using two cross-couplings. One of the FTZs will be in lower (upper) stopband and the other two FTZ’s will be in upper (lower) stopband. In order to get one FTZ in upper stopband, the cursor is placed under the middle inverter and  the command CT-CQ-CN Oprs / Conv. to CQ-CN / NINEtuplet / 1 FTZ is applied with value of  K19 being positive, converting the whole circuit into a ninetuplet (Figure 15.19.f). A pictorial description of the ninetuplet is shown in Figure 15.19.g.

- Insertion loss and return loss responses are shown in Figure 15.19.h. Return loss response indicates that the default values used for cross-coupling inverters and resonance frequencies of resonators are not close enough to the actual values. Therefore some tuning is needed to improve the return loss.

- Figures 15.19.i-to-n describe the design of where one FTZ is created in lower stopband.

- Figures 15.19.p-to-s describe the design where one FTZ is created in lower stopband and two in upper stopband.

- Figures 15.19.t-to-w describe the design where one FTZ is created in upper stopband and two in lower stopband.

- Using Tune Element command the cross-coupling inverters can be tuned to adjust the locations of FTZs. The return loss response may need more elaborate optimization involving all elements including the shunt capacitors using Monte Carlo-Best Case routine.

Figure 15.19. Design of ninetuplet filters.

Figure 15.19. Design of ninetuplet filters.

Figure 15.19. Design of ninetuplet filters.

Figure 15.19. Design of ninetuplet filters.

Figure 15.19. Design of ninetuplet filters.

15. 2. 8. TENTUPLETS

Filpro can be used to design symmetric canonical or non-canonical tentuplets by brute force introduction of cross-couplings (Figure 15.20). Symmetric canonical tentuplets can realize 8 symmetrically located FTZ’s through the cross-couplings K110, K29, K38 and K47. Asymmetrically placed FTZ’s necessitate the use of diagonal cross-couplings K210, K39, K48 and K57. Thus, symmetric tentuplets are much simpler to construct.  Filpro will be used to design only symmetric canonical tentuplets. Hence we will not have diagonal cross-couplings. Let’s see some tentuplet examples.

Figure 15.20. Symmetric canonical tentuplet and its circuit model.

## EXAMPLE 15.11. Tentuplet filters

Filter passband specifications are the same as the previous examples:

PBRipple=0.01 dB,  fp1=990 MHz,  fp2=1010 MHz.

- In order to design a tentuplet cross-coupled filter we start by first designing its direct coupled resonator version as shown in Figure 15.21.a-e. The resulting filter is saved as TEN.fil.

- Filpro can convert this circuit into symmetric canonical tentuplets that can realize either eight FTZs or 4 FTZ’s plus 2 complex TZ quartet or two FTZs plus 3 complex TZ quartet or 4 complex TZ quartets through the cross-couplings K1-10, K29, K38 and K46.

- In order to get eight FTZ’s (four in each stopband) the cursor is placed under the middle inverter and  the command CT-CQ-CN Oprs / Conv. to CQ-CN / TENtuplet / Eight FTZs is applied converting the whole circuit into an tentuplet (Figure 15.21.f).

- Note that for this case the cross-coupling inverters K29 and K47 are negative (Capacitive). A pictorial description of the tentuplet is shown in Figure 15.21.g.

- Insertion loss and return loss responses are shown in Figure 15.21.h. Return loss response indicates that the default values used for cross-coupling inverters need some slight tuning.

- Figures 15.21.i-to-n describe the design of the second option where four FTZ’s and two complex TZ’s are realized to form a linear phase filter with four FTZ. Note that in this case K38 and K47 are negative.

- Figures 15.21.p-to-s describe the design of the third option where three complex TZ’s and two FTZ’s are realized to form a linear phase filter with two FTZs. In this case only K47 is negative.

- Figures 15.21.t-to-w describe the design of the fourth option where four complex TZ’s are realized to form a linear phase filter with no FTZs. Note that in this case all the cross coupling inverters are positive.

- Tune Element command can be used to adjust locations of FTZ’s or to adjust group delay flatness while Monte Carlo-Best Case routine can be used to improve the return loss.

Figure 15.21. Design of tentuplet filters.

Figure 15.21. Design of tentuplet filters.

Figure 15.21. Design of tentuplet filters.

Figure 15.21. Design of tentuplet filters.

Figure 15.21. Design of tentuplet filters.