CHAPTER 17

MULTIPLEXER DESIGN

17. 1. INTRODUCTION

Multiplexers are used to divide frequency band into channels by interconnecting a number of specially designed channel filters. A diplexer is a two channel multiplexer, a triplexer is a three channel multiplexer and so on. In Figure 17.1 some typical parallel connected diplexers are shown while in Figure 17.2 typical parallel connected multiplexer structures are shown. The duals of these circuits are named as series connected multiplexers.

Figure 17.1.a) Lowpass-highpass diplexer. b) Bandpass-bandstop diplexer. c) Lowpass-bandpass diplexer.    d) Bandpass-bandpass diplexer

In applications where a wide frequency band should be covered without any gap between channels, the adjacent channels should be contiguous, that is, they should cross-over at 3 dB. In the usual communication applications, channels should be separated by some guard bands with a specified minimum cross-over loss. Such multiplexers are termed as noncontiguous. In some applications two adjacent channels may overlap. The signals with frequencies in the overlap region are divided equally into the two channels.

Since a multiplexer is a lossless multiport network, all the power available from the source at the input (Port-1) is supposed to be delivered to only one of the output ports depending on frequency, without any reflection. That is, input impedance of a multiplexer must be matched to the source at all frequencies within the passbands of all channels.

The theory for exact synthesis of multiplexers is still at infancy 21, 35.  Exact synthesis techniques for some lowpass-highpass diplexers are available in literature 31, 32, 37, 41, 42 and one such approach is placed into Filpro. A survey of literature shows that the practice of multiplexers is developed along the lines which are independent of exact filter synthesis techniques. Most of the successful design techniques depend on modifications of independently synthesized channel filters brought together in the topologies shown in Figures 17.1 or 17.2 to work as multiplexers. As complexity of multiplexing structures increase, circuit theory alone can’t cope with design challenges. Circuit theory should work together with electromagnetic field theory 33, 36, backed by powerful optimization techniques 27,28.

- Multiplexers may be designed by employing either singly terminated or doubly terminated channel filters. Design of multiplexers using singly terminated channel filters is both easy and well established.

Figure 17.2.a) Parallel bandpass filter multiplexer. b) Ladder type lowpass-highpass multiplexer.               c)  Bandpass-bandstop and lowpass-highpass multiplexer. d) Tree type multiplexer using lowpass-highpass diplexers. e) Manifold type multiplexer.

The theory of multiplexers employing singly terminated filters [1]-[8] are based on the concepts of complementary pair of filters. A parallel filter pair with nonoverlapping passbands must have input admittances Y1 and Y2 such that Y1 +Y2 =1 in the passbands of the channels. Similarly, a series connected filter pair input impedances must satisfy the relation Z1+Z2=1 in the passbands of the channels. That is, the reactive parts of channel input impedances are of opposite sign so they cancel to yield resistive input impedance matched to the source resistance over both channels. Cancellation is exact at the cross-over frequency for maximally flat channel filters [1]-[6] hence the name complementary pairs. For Chebyshev or elliptic channel filters, this effect is still observed but it is approximate, hence the name pseudo-complementary pairs.

On Filpro, different alternative approaches are available for multiplexer design:

- Complementary or pseudo-complementary LP-HP diplexers with controlled cross-over frequency and cross-over loss can be designed most easily using the menu item Design / LP-HP Diplexer. This command is prepared to design only LP-HP diplexers with any specified cross-over frequency and loss. However it can also be used to design normalized prototype LP-HP diplexers. These prototypes can then be used to design complementary pair LP-BP, BP-HP, BP-BP or BP-BS type diplexers and multiplexers. This approach is the main tool of Filpro.

- In general the desired channel filters can also be synthesized independently as singly terminated or doubly terminated filters and then combined to form of LP-HP, LP-BP-HP or BP-BP, BP-BS, etc type diplexers and multiplexers, followed by modifications and tuning-optimization to get the targeted response. These filters can be designed either by first forming a LP prototype and mapping into LP, HP, BP or BS form, or the filters can be synthesized directly as LP, HP, BP or BS filters. Usually Singly terminated adjacent channel filters form better complements. When doubly terminated filters are used as channel filters their first element on the diplexing inputs are deleted (foreshortening) because if they are without foreshortening they load each other heavily leading to gross degradation in response. Foreshortening may lead to degree reduction compared to singly terminated designs. Theoretically this should be possible because, if properly designed, the transfer characteristics of each channel may get contributions of other channels to increase their skirt slopes. That is, if there are M channels and if all channel filters are of degree N, then the transfer function of each path from the input of multiplexer to the output of any channel will have a degree of MxN. This fact may be exploited for improving responses of channels. However exact analytical handling and synthesis of such circuits is a formidable task. Therefore usually special approximate design procedures for special structures are needed [9],  [10],  [11], [17], [18], [20].

- All these comments are also applicable to all-stub commensurate length distributed element filters. For distributed element filters involving contributing Unit Elements we have the following extra property: A commensurate line highpass filter with N contributing unit elements and M transmission zeros at f=0 has a lowpass complements with N+M transmission zeros at f=. Similarly, a lowpass filter with N contributing unit elements and M transmission zeros at f= have highpass complements with N+M transmission zeros at f=0 [5], [6].

The present version of Filpro can be used to design, analyze and tune lumped and distributed element diplexers and multiplexers formed by singly or doubly terminated channel filters having maximally flat or equiripple passbands, with or without finite transmission zeros and with or without contributing unit elements.

All the configurations shown in Figures 17.1 and 2 can be handled except the tree-type structure shown in Figure 17.2.d because of its non-cascade structure. Also channel insertion losses of the manifold type multiplexer of Figure 17.2.e can’t be displayed for the same reason.

The realization problems due to extreme element values or due to resulting topology can be solved by using circuit transformation tools of Filpro. In general all multiplexers require some professional optimization. However, as will be described in the forthcoming examples, many useful multiplexers can be designed through manual optimization using Tune Element and Tune Multiplexer command or Monte Carlo optimization routine of Filpro. In fact, especially in multiplexers with high number of channels, direct application of general optimization techniques is usually not practical or feasible. It is the author’s experience that manual optimization while observing the response is a better way at the initial stages of design, giving hints on the directions to move and on the relative effects of various parameters on response and on possible solution patterns.

##### Lumped Element

##### LP-HP

DIPLEXERS

#### 17. 2.  CHEBYSHEV AND MAXIMALLY FLAT LP-HP DIPLEXERS

In Filpro, Chebyshev and maximally flat LP-HP diplexers can be designed in the shortest way (automatically) by using the command Design / LP-HP Diplexer. In this approach the diplexers are designed from singly terminated LP prototypes by adjusting the passband edge frequencies of the LP and HP sections automatically until the targeted cross-over loss is achieved at the specified cross-over frequency. The user specifies the cross-over loss, cross-over frequency of channels, degree of the filters, RS and RL. For Chebyshev filters the user also specifies a passband ripple (PBRipple) factor. Filpro synthesizes the filters using a singly terminated synthesis. Although Passband Ripple concept is not defined for singly terminated filters, the user must specify a passband ripple just as a parameter needed in the synthesis algorithm of the filters. The actual resulting passband ripple (hence return loss) after interconnection of the channel filters is unknown at the start of the design, but it will be substantially lower than the specified value. Therefore some trials may be needed for setting the targeted return loss level. Actually after some experimental trials one can get an idea on the relation between the PBRipple factor and Return Loss level of the diplexer.

The resulting LP and HP channel filters are obtainable from each other through the transformation s1/s. This relation leads to the geometric frequency symmetry property, S21(s)=S31(wc2/s) where wc is the cross-over frequency of insertion loss responses of the two channels [31], [41], [42]. The passband edge frequencies fpLP and fpHP of such pairs of lowpass and highpass channel filters are related to the crossover frequency  through the relation fpLPfpHP = fc2.

Design / LP-HP Diplexer technique of Filpro is important not only for the design of LP-HP diplexers but also for the design of all Chebyshev and Maximally Flat type LP-BP, BP-HP, BP-BP or BP-BS diplexers and multiplexers. This technique can be used to prepare Prototype LP-HP diplexers normalized to RS=RL=1 with fc=0.159 (corresponding to wc=1). Then either the LP or HP arm of this prototype can be used to design the LP, HP, BP or BS channel filters of all kinds of diplexers and multiplexers through Design / By User Defined Prototype approach of Filpro. This approach maps the cross-over frequency fc=0.159 to the passband edge frequencies of the resulting LP, HP, BP or BS filters with the passband edge loss being the same as the cross-over loss of the prototype LP-HP diplexer. Thus, the cross-over losses of the resulting multiplexers formed by combining these filters will have the same cross-over loss as the original prototype LP-HP diplexer.

In this section we will use Design / LP-HP Diplexer technique to design both an actual LP-HP diplexer and also some normalized prototype diplexers with user specified cross-over loss at the cross-over frequency fc=0.159.

# EXAMPLE 17. 1. Maximally Flat prototype LP-HP Diplexers

Let’s design maximally flat LP-HP diplexers with 3 dB and 10 dB cross-over losses, normalized to fc=0.159 (wc=1) and RS=RL=1 that can be used as the prototype for multiplexers to be designed in later sections. We will take degree of the filter as N=7.

- Figure 17.3.a shows the design command set for maximally flat LP-HP diplexer. Design / LP-HP Diplexer command with the options Lumped-Maximally Flat are selected. In the Diplexer Specs menu item, degree of the LP-Prototype filter, N=7, the cross-over loss 3.011 dB, cross-over frequency fc=0.159 and RS=RL=1 are typed in. In Diplexer Files menu, four filenames should be are typed in, with the first two being two different representations for the diplexer and the last two being the LP and HP filters alone. In the first one the LP channel filter appears as the main filter and the HP channel filter appears as an External 1-Port stick to the LP filter at its input to form the diplexer.  We named it as DLP1. In the second file the HP filter appears as the main filter and the LP filter appears as External 1-Port. We named it as DHP1. The third one is the name of the LP channel filter. It has a special importance for us because we will keep it to use as the normalized LP prototype for the design of diplexers later. We name it as PR1 (Prototype-1).  The fourth file is the name of the HP channel filter. It can also be used as a prototype for designing diplexers. Therefore we name it as PR1HP.

- Form Diplexer command designs and saves the filter files with the assigned names.

- Figure 17.3.b shows the circuit of PR1. It is a normalized singly terminated LP filter with  RS=RL=1 ohms.  Actually RS=0, but for convenience in analysis on Filpro, we use RS=1. Load / PR1.FIL command displays the circuit. From the Filter Info inset on the right bottom corner of the display we can read the passband edge frequency fp of this LP filter as fp(LP)=0.1596 (let’s note it as fpLP to differentiate it from the pasband edge frequency of the HP counterpart). It is slightly above  the cross-over frequency fc=0.159, adjusted to get the targeted cross-over loss.

- Figure 17.3.c shows the of PR1HP. It is a normalized singly terminated HP filter. Its passband edge frequency fp(HP)=0.1584  can be seen by loading this circuit and reading the Filter Info inset in the right bottom of the screen.  It is slightly smaller than the cross-over frequency fc=0.159.

- We can check that frequency symmetry property fpLP*fpHP=fc2 holds. These frequencies are determined by Design LP-HP Diplexer command in an iterative manner until the targeted cross-over loss at the cross-over frequency is achieved. Then the component filters are synthesized. This is a big advantage because it is a tedious process to set these frequencies correctly, as will be seen in the elliptic LP-HP diplexers where the command Design / LP-HP Diplexer command is not applicable.

- Figure 17.3.d shows one version of the complete diplexer circuit. In this version the LP channel PR1 appears explicitly and the HP channel (PR1HP) appears as External 1-Port. With cursor being under any element of this circuit we can apply Plot-Tune Multiplexer command to either plot responses of the diplexer or to tune the pointed element while observing the response.

- Figure 17.3.e shows the second version of the complete diplexer circuit. In this version the HP channel PR1HP appears explicitly and the LP channel (PR1) appears as External 1-Port. With cursor being under any element of this circuit we can apply Plot-Tune Multiplexer command to plot responses of the diplexer or to tune the pointed element.

- It should be noted that these two versions of the diplexer circuits can also be formed by loading one of the PR1 or PR1HP and inserting the other one as External 1-Port element. This is done by placing the cursor under the element adjacent to RS and applying the command External 1-Port / PR1HP (or PR1) / Shunt 1-Port.

- Figures 17.3.f and g show the diplexer circuits DPL2 and DHP2 obtained by repeating the design by setting the cross-over loss as 10 dB. In these figures only the diplexer circuits are shown from which the component LP and HP filters (PR2 and PR2HP) can be extracted by deleting the External 1-Port elements.

- Figures 17.3.h and i show the insertion loss and return loss responses of the two diplexers. It is seen that the cross-over losses are equal to the specified values. Note that the return loss is well below 30 dB (37.9 dB) for 3 dB cross-over loss case. That is, the LP-HP diplexer is well matched over the whole band. For 10 dB cross-over loss, the return loss makes a peak in the cross-over region.

- The LP sections PR1 and PR2 can be saved for later use in designing various LP-HP, BP-BP- BP-BS, etc. diplexers and multiplexers as will be seen in the succeeding sections of this chapter.

Figure 17.3. Design of a Maximally Flat prototype LP-HP diplexer.

Figure 17.3. Design of a Maximally Flat prototype LP-HP diplexer.

# EXAMPLE 17. 2. Prototype Chebyshev LP-HP Diplexers and Return Loss versus PBRipple

In order to inspect the relation between the specified PBRipple factor of the prototype and the return loss level of the resulting diplexers, let’s design degree N=7 Chebyshev LP-HP diplexers with 3 dB cross-over loss, normalized to fc=0.159 (wc=1) and RS=RL=1 and different PBRipple values. These diplexers can also be used as the prototypes for multiplexers to be designed in later sections.

- Figure 17.4.a shows the design command set for a Chebyshev LP-HP diplexer.

- Design / LP-HP Diplexer command with the options Lumped-Chebyshev are selected.

- In the Diplexer Specs menu item, degree of the LP Prototype filter, N=7, PBRipple=0.01 dB, the cross-over loss 3.011 dB, cross-over frequency fc=0.159 and RS=RL=1 are typed in.

- In Diplexer Files menu, four filenames should be are typed in, with the first two being two different representations for the diplexer and the last two being the LP and HP filters alone. In the first one the LP channel filter appears as the main filter and the HP channel filter appears as an External 1-Port stick to the LP filter at its input to form the diplexer. We named it as DLP3. In the second file the HP filter appears as the main filter and the LP filter appears as External one port. It is named as DHP3. The third one is the name of the LP channel filter. It has a special importance for us because we will keep it to use as the normalized LP prototype for the design of diplexers later. We name it as PR3. The fourth file is the name of the HP channel filter. It can also be used as a prototype for designing diplexers. Therefore we name it as PR3HP.

- Figure 17.4.b shows the circuits of PR3, PR3HP, DLP3 and DHP3. When these circuits are displayed on the screen, in the Filter Info inset on the right we can read the passband edge frequencies of the LP and HP filters as fp(LP)=0.1391 and fp(HP)=0.1818. It should be noted that these frequencies are appreciably far away from the cross-over frequency fc=0.159. These frequencies are determined by Design LP-HP Diplexer command in an iterative manner until the targeted cross-over loss at the cross-over frequency is achieved. This is a big advantage because it is a tedious process to set these frequencies correctly, as will be seen in the elliptic LP-HP diplexers where the command Design / LP-HP Diplexer command is not applicable.

- Insertion Losses of channels can be plot simultaneously by placing the cursor under any element of either DLP3 or DHP3 and applying the command Plot-Tune Multiplexer. This command can be used both to tune the elements while observing any response or just to plot the responses.

- Figure 17.4.c shows the diplexers circuits DLP4 and DHP4 obtained by repeating the design by setting PBRipple=0.1 dB.

- Insertion loss and return loss responses can be inspected in Figure 17.4.d and e. It is seen that for both PBRipple=0.01 dB and PBRipple=0.1 dB are well below 30 dB except in the cross-over region where they are about 30 dB. These responses are close to that of the maximally flat case. However insertion loss slopes are much steeper compared to Maximally flat case.

- In order to get steeper insertion loss slopes with more practical return loss levels we repeat the designs by changing PBRipple factor. Some of the typical results for different PBRipple values are shown in Figure 17.4.f-to-i with insertion loss and return loss responses shown in Figure 17.4.k and m. These circuits are selected by inspecting their return loss levels as follows.

- The circuits for PBRipple=0.3 dB (Figure 17.4.f) gave return loss which varies between 25 dB and 29 dB. The LP section of this diplexer is saved as PR5.

- The circuits for PBRipple=0.38 dB (Figure 17.4.g) gave almost equal ripple return loss at about 20 dB level. The LP section of this diplexer is saved as PR6.

- The circuits for PBRipple=1.39 dB (Figure 17.4.h) gave almost equal ripple return loss at about 16 dB level. The LP section of this diplexer is saved as PR7.

- The circuits for PBRipple=2.8 dB (Figure 17.4.i) gave almost equal ripple return loss at about 10 dB level. The LP section of this diplexer is saved as PR8.

- The passband edge frequencies of the LP and HP component filters are shown in the respective figures. It should be noted that we have the property fpLP*fpHP=fc2.

- The LP prototypes can be used in the design of diplexers and multiplexes in the late sections of this chapter.

Exercise: Repeat the designs for 10 dB cross-over loss and save them to design multiplexers with 10 dB cross-over loss in later sections.

Figure 17.4. Design of Chebyshev type normalized prototype LP-HP diplexers.

Figure 17.4. Design of Chebyshev type normalized prototype LP-HP diplexers.

Figure 17.4. Design of Chebyshev type normalized prototype LP-HP diplexers.

Figure 17.4. Design of Chebyshev type normalized prototype LP-HP diplexers.

# EXAMPLE 17. 3. Maximally Flat LP-HP Diplexer with 3 dB cross-over loss at fc=400 MHz

Just as exercise let’s design maximally flat LP-HP diplexers with 3 dB and 10 dB cross-over losses, with cross-over frequency fc=400 MHz, RS=RL=50 and degree N=7.

- Figure 17.5.a shows the design command set for maximally flat LP-HP diplexer.

- Design / LP-HP Diplexer command with the options Lumped-Maximally Flat are selected.

- In the Diplexer Specs menu item, degree of the LP-Prototype filter, N=7, the cross-over loss 3.011 dB, cross-over frequency fc=400 MHz and RS=RL=50 are typed in.

- In Diplexer Files menu, four filenames should be are typed in, with the first two being two different representations for the diplexer and the last two being the LP and HP filters alone. In the first one the LP channel filter appears as the main filter and the HP channel filter appears as an External 1-Port stick to the LP filter at its input to form the diplexer.  We named it as DMFLP1 (meaning Diplexer, Max. Flat, LP). In the second file the HP filter appears as the main filter and the LP filter appears as External 1-Port. It is named as DMFHP1. The third one is the name of the LP channel filter. It is named as MFLP1.  The fourth file is the name of the HP channel filter. We name it as MFHP1.

- Form Diplexer command is the main tool which designs the proper LP and HP filters by adjusting their passband edge frequencies so that when connected the cross-over loss will be the assigned value (3.011 dB) at the specified cross-over frequency. This command saves also the filter files with the assigned names.

- Figure 17.5.b shows one version of the diplexer circuit, DMFLP1. The explicitly displayed circuit is MFLP1 while the External 1-Port element is MFHP1. In the inset on the right bottom of the display we can read the passband edge frequency fp=401.481 MHz of the MFLP1 filter (let’s note it as fpLP to differentiate it from the pasband edge frequency of the HP counterpart). It is different from the cross-over frequency fc=400 MHz, adjusted to get the targeted cross-over loss. With cursor being under any element of this circuit we can apply Plot-Tune Multiplexer command to either plot responses of the diplexer or to tune the pointed element while observing the response.

- Figure 17.5.c shows the second version of the diplexer, DMFHP1. The explicitly displayed circuit is MFHP1 while the External 1-Port element is MFLP1. In the inset on the right bottom of the display we can read the passband edge frequency fp of the MFHP1 filter Its passband edge frequency fp (fpHP) can be seen in the inset on the right bottom of the screen.  It is also different from the cross-over frequency. We can check that frequency symmetry property fpLP*fpHP=fc2 holds. With cursor being under any element of this circuit we can apply Plot-Tune Multiplexer command to plot responses of the diplexer or to tune the pointed element.

- Figure 17.5.d shows the circuit of MFLP1. We can form the diplexer circuit DMFLP1 by placing the cursor under the element adjacent to RS and applying the command External 1-Port / MFHP1 / Shunt 1-Port as described in Figures 17.5.d-e.

- Figure 17.5.f shows the circuit of MFHP1. We can form the diplexer circuit DMFHP1 by placing the cursor under the element adjacent to RS and applying the command External 1-Port / MFLP1 / Shunt 1-Port as described in Figures 17.5.f-g.

- Figures 17.5.h-to-q describe the formation of series type diplexers. The component filters of parallel diplexers start with series element on their diplexing ports. Series diplexers are just duals of parallel diplexers. Therefore the filters of series diplexers start with shunt elements on their diplexing ports. So, series diplexers can be formed by finding the duals of the filters of parallel diplexers as shown in Figures 17.5.h-i and k-m. Then these filters are connected in series by using the command External 1-Port / .. / Series 1-Port as shown in Figures 17.4.m-q.

- Figure 17.5.r show the insertion loss and return loss responses of the diplexer (series or parallel). It is seen that the cross-over loss is 3.01 dB at fc=400 MHz. Return loss is greater than 37 dB.

- Figure 17.5.s shows group delays of the channels. It is seen that the channels have identical delays over the whole band.

Exercise: Design maximally flat diplexers with cross-over loss of 3 dB and 10 dB at fc=400 MHz, using higher degree filters. Inspect their passband edge frequencies.

Exercise: Design Chebyshev diplexers with cross-over loss of 3 dB at fc=400 MHz, using degree 7 filters and PBRipple=0.01 dB, 0.1 dB, 0.8 dB, 1.39 dB and 2.8 dB. Inspect their Return Loss levels.

# Figure 17.3. Design of a maximally flat LP-HP diplexer with fc=400 MHz.

Figure 17.5. Design of a maximally flat LP-HP diplexer with fc=400 MHz.

- Figure 17.5. Design of a maximally flat LP-HP diplexer with fc=400 MHz.

- EXAMPLE 17. 4. Design of LP-HP diplexers with partial overlapping channels using LP Prototypes

- Design / LP-HP Diplexer approach can’t design diplexers with overlapping passbands. Such diplexers can be designed by using the prototypes described in the previous two examples. Let’s design a LP-HP diplexers with partially overlapping passbands by using the Chebyshev prototype PR5 developed in the previous example. It is a degree N=7 LP prototype with PBRipple=0.3 dB that leads to about 25-29 dB return loss level. The cross-over frequency will be fc=400 MHz. The passband edge of the LP channel is fpLP=440 MHz and the passband edge of the HP channel is fpHP=360 MHz. That is, we will have overlap in the frequency range 360<f<440 MHz. In this band the insertion losses of the channels will be 3.01 dB.

- Design command set for the LP channel filter is shown in Figure 17.6.a. It is designed by using the technique Design / By  User Defined Prototype where PR5 is typed in as the LP prototype. The passband edge frequency is typed in as fp=440 MHz. The resulting circuit is saved as LP9.

- Design command set for the HP channel filter is shown in Figure 17.6.b. It is also designed by using the technique Design / By  User Defined Prototype where PR5 is typed in as the LP prototype. The passband edge frequency is typed in as fp=360 MHz. The resulting circuit is saved as HP9.

- In Figure 17.6.b-c the LP filter LP9 is inserted into HP9 as External 1-Port to form one of the two possible forms of the diplexers. We could as well insert  HP9 into LP9 to form the other version of the diplexer.

- Figures 17.6.d and e show the insertion loss and return loss responses. It is seen that insertion loss is varying at about 3.4 dB level while the return loss is about 10 dB level in the cross-over range. Cross-over loss can be made to approach 3 dB level by increasing the return loss in the cross-over range by tuning first few elements of both filters.

- Exercise: Tune the elements of both filters to improve return loss level while trying to keep the overlap frequency range the same.

- Figure 17.6. Design of a LP-HP diplexer with partially overlapping passbands.

- Figure 17.6. Design of a LP-HP diplexer with partially overlapping passbands.

17. 3. ELLIPTIC LP-HP DIPLEXER DESIGN USING SINGLY TERMINATED LP PROTOTYPES

LP-HP diplexers formed by filters having finite transmission zeros (FTZ) are named as Elliptic Diplexers. Design / LP-HP Diplexer command can not be used to design elliptic diplexers. There is no automated tool on Filpro to design elliptic diplexers. Therefore on Filpro such diplexers can be designed using the conventional methods, in several ways, as follows:

- Design from singly terminated or doubly terminated LP Prototypes: We can first design a singly terminated elliptic LP prototype and then use this prototype to design the channel filters of the targeted diplexer using Design / By User Defined Prototype approach. The tedious part of this method is that we don’t know the passband edge frequencies fpLP and fpHP of the channel filters. They need be found by trials until the targeted cross-over loss is obtained. Fortunately, we have to   make trials only on fpLP or fpHP. The other one can be found using the frequency symmetry property, fpLP*fpHP=fc2 where fc is the cross-over frequency. In such diplexers the FTZ’s will also have frequency symmetry. That is if the FTZ’s of the LP and HP channels are termed as fzLP and fzHP, then we will have fzLP*fzHP=fc2.

- Design from foreshortened doubly terminated LP Prototypes: It is also possible to design the LP prototype in Doubly Terminated form and then use foreshortening by deleting the first element of the LP prototype. However this is not easier or shorter than singly terminated cases. Therefore we will not use that approach.

- Direct synthesis of singly terminated or doubly terminated LP and HP filters: The LP prototype approach described above has some intrinsic limitations. This is because the transmission zeros of the LP prototype will map to frequency symmetric locations in the resulting LP and HP filters. In some applications it may be more advantageous to synthesize the component filters directly in singly terminated LP and HP forms with transmission zeros placed at the desired locations at the expense of some degradations in response. This approach will be treated in the next section.

###### EXAMPLE 17. 5: Design of a degree 7 elliptic diplexer with one FTZ in each channel

Let’s prepare a singly terminated elliptic LP prototype having a single FTZ and use it in the design of a LP-HP diplexer.

Normalized lowpass prototype filters are synthesized by taking the passband edge frequency as fp=0.159 corresponding to normalized radian passband edge frequency wp=1. Although the actual source side termination is RS=0 for singly terminated filters, we will specify RS=RL=1 just to be able to plot insertion loss to get an idea on the response shape.

Similarly, although the Passband Ripple is meaningless for singly terminated filters, we have to specify this parameter to run the synthesis algorithms of Filpro. Typical values of PBRipple factors that will give commonly used Return Loss levels like 10 dB, 20 dB, 26 dB, etc. can be determined by a few trial designs, as was done in Example 17.2.

Let’s design a set of diplexers with differing passband edges and all having the same crossover frequency fc=400 MHz, RS=RL=50 ohms using an elliptic LP prototype. The LP prototype will be of degree seven with Ninf=3 and one FTZ at fz=0.25, with PBRipple=0.01 dB.

Design steps are shown in Figures 17.7:

- First the prototype is synthesized. Design / By Synthesis technique is selected with Singly Terminated option. Type A option is needed to get series first elements because the resulting filters will be connected in parallel.

- PBRipple=0.01 dB is typed in.

- Three transmission zeros are placed at f= and one finite transmission zero is placed at fz=0.25.

- Element extraction is carried out using Standard Auto Extract command giving the circuit shown in Figure 17.7.b. After element extraction is completed Filpro asks whether to reverse the circuit. In usual multiplexer applications the resulting circuit must be reversed. This is related to the synthesis algorithm used in Filpro which carries out element extraction from the output port. The synthesized circuit is saved as PR15. The LP Prototype is ready.

- The LP and HP channel filters will be designed by using that LP prototype through Design / By User Defined Prototype approach. However we don’t know the passband edge frequencies fpLP and fpHP of the channel filters. They need be found by trial designs where we select an fpLP and use frequency symmetry property to find fpHP=fc2/fpLP. This seems a tedious procedure, but converges to the target in a few design trials.

- Though not necessary, just to get a rough idea on the possible limits of fpLP and fpHP, we may also design a Chebyshev diplexer of similar order (n=7) as the targeted elliptic channel filters and having fc=400 MHz with 3 dB cross-over loss using Design / LP-HP Diplexer approach as described in Figure 17.7.c. This diplexer may be termed as Reference Diplexer. The LP and HP channel filters of this diplexer are named as LP06 and HP06. Their passband edge frequencies are noted down as fpLPref =350 MHz and fpHPref=457 MHz by displaying them one by one (Figure 17.7.d-e). If we  adjust fpLP and fpHP of the targeted filters around these reference frequencies we may find their proper values with less trials (not for sure  ). If fpLP>fpLPref and fpHP<fpHPref then we will get overlapped channels. Otherwise, that is, if fpLP<fpLPref and fpHP>fpHPref  then the passbands will get away from each other, increasing the cross-over loss. Let’s make a survey by selecting differing fpLP-fpHP sets. It should be noted that in order to have the cross-over frequency unchanged for different cases we have to use the relation fpLP*fpHP=fc2.

- Figure 17.7.f gives the command set for the design of a LP filter through Design / By User Defined Prototype approach using PR15 as the prototype. We make a test with trial passband edge frequency fpLP=355 MHz, slightly higher than fpLPref=350 MHz. The resulting filter is saved as LP15A (Figure 17.7.g). Then a HP filter is designed using the same prototype, with passband edge frequency fpHP=fc2/fpLP=(400)2/355=451 MHz. It is saved as HP15A (Figure 17.7.h-i).

- Then diplexer circuits are formed as shown in Figure 17.7.k-m-n. Figure 17.7.p and q show zoomed and wideband responses of the diplexer. The cross-over loss is about 3.07 dB with return loss about 22 dB. It is seen that we are close to the target. The FTZ’s provide good isolation between channels about the FTZ frequencies.

- In order to see whether we can get better results the design is repeated by using fpLP=360 MHz and fpHP=444 MHz. Indeed we got about 26 dB return loss with cross-over loss being still about 3 dB (Figure 17.7.r).

- As the fpLP is increased further the corresponding fpHP decreases and the two bands start to overlap. In order to get an idea on the overlapping range, the design is repeated by using fpLP=380 MHz and fpHP=421 MHz. The evidence of overlap is seen from the change in the curvatures of the insertion loss plots and lowering of return loss in the cross-over frequency region (Figure 17.7.s).

- Then the design is repeated by taking fpLP=410 MHz and fpHP=390 MHz to broaden the overlap. The results are shown in Figure 17.7.t.

- Next we started to decrease fpLP which leads to increase in fpHP to adjust the cross-over loss to levels greater than 3 dB. In Figure 17.7.u, the results are shown for fpLP=310 MHz, fpHP=516 MHz. The cross-over loss is increased to 14 dB level.

- If very high cross-over loss is desired then by adjusting fpLP and fpHP we can force the FTZ’s coincide with the cross-over frequency.  In Figure 17.7.v the results are shown for fpLP=250 MHz  and fpHP=640 MHz. The FTZ’s moved to very close to fc=400 MHz (Exercise with fpLP and fpHP for finer adjustments).

- Figure 17.7.w shows a case where a guard band is formed between the two channels by adjusting the locations of the two FTZ’s. For fpLP=200 MHZ and fpHP=800 MHz, the signals in the guard band are attenuated by at least 40 dB.

Exercise: Repeat the above example by taking PBRipple=0.1 dB, 0.8 dB, 1.39 dB, 2.8 dB to get different return loss levels and compare these levels with those obtained in Chebyshev diplexer of Example-17.2.

Exercise: Design a degree eleven LP prototype with Ninf=3 and four FTZ’s, all at fz=0.18, with PBRipple=0.1 dB and form contiguous and other types of diplexers with cross-over frequency being fc=400 MHz.

Figure 17.7. Design of elliptic LP-HP diplexers by selecting different fpLP-fpHP pairs.

Figure 17.7. Design of elliptic LP-HP diplexers by selecting different fpLP-fpHP pairs.

Figure 17.7. Design of elliptic LP-HP diplexers by selecting different fpLP-fpHP pairs.

Figure 17.7. Design of elliptic LP-HP diplexers by selecting different fpLP-fpHP pairs.

Figure 17.7. Design of elliptic LP-HP diplexers by selecting different fpLP-fpHP pairs.

Figure 17.7. Design of elliptic LP-HP diplexers by selecting different fpLP-fpHP pairs.

Figure 17.7. Design of elliptic LP-HP diplexers by selecting different fpLP-fpHP pairs.

Figure 17.7. Design of elliptic LP-HP diplexers by selecting different fpLP-fpHP pairs.

EXAMPLE 17. 6: Design of a degree 7 elliptic diplexer with two FTZ’s in each channel

- In order to get further insight on the selection of passband edge frequencies of elliptic diplexers let’s continue exercising. Now we will place two FTZ’s in a degree 7 singly terminated LP prototype with following specifications:

PBRipple=0.1 dB, fp=0.159, RS=RL=1, Ninf=3, FTZ1=0.19, FTZ2=0.225.

The diplexer cross-over frequency is still fc=400 MHz.

- Figure 17.8.a describes the synthesis of the LP prototype. It is saved as PR17 (Figure 17.8.b).

- Figure 17.8.c describes the design of a trial LP channel by selecting fp=385 MHz (fpLP) using Design / By User Defined Prototype approach. It is saved as LP385 (Figure 17.8.d).

- The HP counterpart passband edge frequency is calculated as fpHP=fc2/fpLP=416 MHz. It is also designed using the same LP prototype and saved as HP416 (Figure 17.8.e).

- Diplexer is then formed by inserting LP385 into HP416 as External 1-Port (Figure 17.8.f).

- Responses of the diplexer is shown in Figure 17.8.g. Insertion loss looks good. However we are not sure about the return loss. It is making a spike at the cross-over frequency, down to about 20 dB. Let’s see if we can improve it by adjusting fpLP and fpHP.

- The design of LP and HP channel filters are repeated by selecting fpLP=384 MHz and fpHP=416.6 MHz, giving the responses shown in Figure 17.8.h. Return loss got worse, down to about 16 dB, indicating that the cross-over loss is starting to increase above the 3 dB level.

- Figure 17.8.i shows the results for fpLP=386 MHz and fpHP=414.5 MHz. It is seen that return loss is improved to about 26 dB. So, we are in the right direction.

- Figure 17.8.k shows the results for fpLP=387 MHz and fpHP=413.4 MHz. The return loss is infinite just at cross-over frequency and makes equal peaks on the two sides of that frequency (twin peaks). We think that this must be the best return loss response we can get.

- We continue by selecting fpLP=388 MHz and fpHP=412.3 MHz giving the responses shown in Figure 17.8.m. Indeed the return loss started to decrease again, indicating the start of overlapping passbands. This is also seen from the distortions in the insertion loss curvatures at the corners of the LP and HP filters.

- Figure 17.8.n shows the results for fpLP=395 MHz and fpHP=405 MHz. We are now in the overlap region as the further lowering of the return loss indicates.

- In conclusion, we may accept the twin peaks of the return loss response seen in Figure 17.8.k as the measure of the exact 3.011 dB cross-over loss leading to exact complementary behaviour. The frequency range of the twin peaks can be defined as the cross-over region. We note that the return loss has equal ripples outside the cross-over region.

- We also note that the return loss level is better than 30 dB. This is an unexpected result for the specification of PBRipple=0.1 dB. In Chebyshev diplexers we have got the same level for PBRipple=0.01 dB (See Example 17.2). It seems that the numbers and locations of FTZ’s and degree of the filters are also effecting the overall return loss level.

Figure 17.8. Design of elliptic LP-HP diplexers with two FTZ’s.

Figure 17.8. Design of elliptic LP-HP diplexers with two FTZ’s.

Figure 17.8. Design of elliptic LP-HP diplexers with two FTZ’s.

Figure 17.8. Design of elliptic LP-HP diplexers with two FTZ’s.

###### EXAMPLE 17. 7: Design of an elliptic diplexer using degree 11 singly terminated LP Prototypes

Let’s design a LP-HP diplexer with cross-over frequency fc=400 MHz, with 3 dB cross-over loss using a singly terminated elliptic LP prototype of degree N=11. The LP prototype will have Ninf=3 and four FTZ’s, all at fz=0.18. It will be designed with passband edge frequency fp=0.159 and by taking RS=RL=1. Let’s now try PBRipple=1 dB.

- Design steps of the LP prototype are shown in Figures 17.9.a. Design / By Synthesis technique is selected with Singly Terminated option. Type A option is needed to get series first elements because the resulting filters will be connected in parallel.

- Passband ripple is typed in as 1 dB.

- Three transmission zeros are placed at f= and four finite transmission zeros are placed at fz=0.18.

- Element extraction is carried out using Standard Auto Extract command giving the circuit shown in Figure 17.9.b. After element extraction is completed the resulting circuit is reversed. The synthesized circuit will be saved, with a name, for example, PR20. The LP Prototype is ready.

- Next the channel filters will be designed through User Defined Prototype approach by using PR20. Through the experience gained from the previous example we have question marks on the passband edge frequencies fpLP and fpHP to be used in the LP and HP filters to get the targeted cross-over frequency and loss. Just as a trial let’s use fpLP=fpHP=fc=400 MHz.

- The command set for the LP channel using PR20 is shown in Figure 17.9.c. Synthesized filter is saved as LP20 (Figure 17.9.d).

- The HP channel is also designed through the same commands, by selecting HPF option (Figure 17.9.e). Synthesized filter is saved as HP20 (Figure 17.9.f).

- Next two diplexer circuits will be formed. In one circuit the lowpass channel will be explicit and the highpass channel will appear as an External One Port circuit added in parallel at the input (diplexing) port and in the other the highpass filter will appear explicitly and the lowpass channel will appear as an external one port at the input port. Figure 17.9.g shows the second version.

- Figure 17.9.h shows the insertion loss and return loss responses. To our surprise, without any iterations on fpLP and fpHP, we hit all the targets in one trial. It is seen that cross-over loss is 3 dB right at fc=400 MHz and the return loss came out to be about 17 dB. Return loss is of equal ripple form outside the cross-over range at 18 dB level.

- The finite transmission zero of the LP prototype at f=0.18 is mapped to 350 MHz in the HP channel and to 450 MHz in the LP channel. If they need be located at different frequencies then the design have to be repeated by shifting the FTZ’s of the LP prototype.

Exercise: Repeat the design by changing fpLP and fpHP to inspect the variation of return loss.

Exercise: Repeat the design with lower passband ripple to improve the return loss level.

Exercise: Repeat the design by placing the FTZ’s of the prototype at different locations.

Exercise: Repeat the design to achieve 15 dB cross-over loss at fc=400 MHz.

Exercise: Repeat the design to have an overlap region between 390 MHz and 410 MHz.

Figure 17.9. Design of an elliptic LP-HP diplexer using LP prototype of degree 11.

Figure 17.9. Design of an elliptic LP-HP diplexer using LP prototype of degree 11.

17.  4. ELLIPTIC LP-HP DIPLEXER DESIGN BY DIRECT SYNTHESIS OF SINGLY TERMINATED FILTERS

In the previous sections the LP and HP filters of the diplexers were designed by using the same LP prototypes. This approach fixes the locations of the finite transmission zeros (FTZ’s) of the LP and HP sections leading to frequency symmetric diplexer characteristics. If it is desirable to set the locations of the FTZ’s of the LP and HP channels independently of each other than we can design the LP and HP filters directly (not by mapping from a prototype) and combine them. Let’s see if we encounter any problem on an example.

EXAMPLE 17. 8: Elliptic LP-HP Diplexers by direct synthesis of singly terminated LP and HP filters

Let’s design a diplexer with 3 dB crossover frequency at fc=400 MHz, RS=RL=50 ohms, each of degree 7. The LP channel will have the transmission zeros Ninf=3, FTZ1=475 MHz, FTZ2=550 MHz.

The HP channel will have transmission zeros, Nzero=3, FTZ1=FTZ2=300 MHz.

In the previous examples we were using Reference LP-HP Diplexers to get an idea on the possible locations and limits of the passband edge frequencies fpLP and fpHP of the LP and HP channels which are related through the frequency symmetry property fpLP*fpHP=fc2. Then we were iterating fpLP-fpHP pairs until the desired cross-over loss characteristics is obtained. We can still use that approach in this example. However we have a problem which makes the procedure for determination of fpLP and fpHP more tedious. Since the FTZ’s are not located symmetrically with respect to the cross-over frequency, the resulting filters will not have the frequency symmetry property. This is a disadvantage because we will adjust fpLP and fpHP independently of each other. However during the initial trials we may use the relation fpLP*fpHP=fc2 and then trim fpLP and fpHP by inspecting the resulting cross-over frequencies.

Just as an exercise and also to get an idea and insight on the possibility of the design of diplexers by brute force tuning, we will first make a design by taking fpLP=fpHP=fc=400 MHz and then tune elements of both filters get the targeted 3 dB cross-over loss. In the second approach we will make repeated designs by changing fpLP and fpHP until the cross-over frequency is settled at fc=400 MHz with 3 dB loss together with a good return loss.

- The command set for the design of the LP channel with fp=fc=400 MHz is shown in Figure 17.10. Synthesized filter is saved as LPE7 (Figure 17.10.a). The HP channel is also designed through the same commands, by taking fp=fc=400 MHz. It is saved as HPE7 (Figure 17.10.b).

- Then diplexer circuits are formed by first loading LPE7 and inserting HPE7 as External One Port  (Figure 17.10.c-d). Then HPE7 is loaded and LPE7 is inserted as External One Port (Figure 17.10.e-f). Both forms of the diplexer can be used for plotting responses or tuning elements.

- Figure 17.10.g and h show the resulting insertion loss and return loss responses before tuning. It is seen that we have overlapping channel conditions (as expected) and at cross-over frequency insertion loss of the channels are more than 3 dB. Return loss is about 10 dB around the cross-over frequency. These are due to the selection of  fpLP=fpHP=fc=400 MHz. We have forced the channels to overlap. We may either repeat the designs by trimming the edge frequencies until the cross-over loss is settled at 3.01 dB as will be done in the second part or we may use Tune Element command both to improve the return loss and also adjust the cross-over loss. Figures 17.7.i and k shows the tuned versions of the filters after a few rounds of rough tuning while observing the return loss only. The responses are shown in Figure 17.10.m and n. Note the shift in the cross-over frequency. Further tuning with consideration given also to the insertion loss and cross-over frequency may give better results.

- Figures 17.10.p shows a design with trial fpLP=380 MHz and fpHP=420 MHz. Results are not satisfactory. Return loss is about 10 dB, cross-over frequency is not at 400 MHz with loss different from 3 dB. Trials will be repeated by changing fpLP and fpHP. Since there are many possibilities this procedure may be tedious. Fortunately trials indicated that variation of return loss level can be used as a practical guide.  For example in Figure 17.10.q the designs with fpLP=370 MHz and fpHP=430 MHz gave a better return loss (22 dB) and hence concentrated our search about these frequencies.

- Figure 17.10.r shows the results for fpLP=372 MHz and fpHP=433.5 MHz. The worst return loss is about 24 dB and cross-over frequency settled at 400 MHz with 3 dB loss. fpLP and fpHP can be trimmed further to make the return loss equal ripple over the whole band.

- Comparison of element values of various solutions shows that the targeted circuit with desired response can be found easily by an optimizer by starting from a coarse estimation of fpLP-fpHP.

Figure 17.10. LP-HP diplexer design by direct synthesis of singly terminated LP and HP channel filters.

Figure 17.10. LP-HP diplexer design by direct synthesis of singly terminated LP and HP channel filters.

Figure 17.10. LP-HP diplexer design by direct synthesis of singly terminated LP and HP channel filters.

Figure 17.10. LP-HP diplexer design by direct synthesis of singly terminated LP and HP channel filters.

Figure 17.10. LP-HP diplexer design by direct synthesis of singly terminated LP and HP channel filters.

17. 5. LP-HP DIPLEXER DESIGN BY USING DOUBLY TERMINATED FILTERS AND FORESHORTENING

In the previous sections we have concentrated on the design of LP-HP diplexers by using singly terminated LP prototypes or direct synthesis of singly terminated LP and HP filters. It is also possible to get the same results by using doubly terminated LP prototypes or directly synthesized doubly terminated LP and HP filters.

When doubly terminated LP and HP filters are synthesized and paralleled to form a diplexer they load each other heavily, giving responses which don’t resemble diplexer characteristics. However elimination of the first elements of both filters (foreshortening) decreases the interaction of the two filters helping to form diplexer characteristics [4].

Let’s design the diplexer of the previous elliptic diplexer example now by using doubly terminated LP and HP filters.

EXAMPLE 17. 9: Elliptic LP-HP Diplexer design by direct synthesis of doubly terminated LP and HP filters

Let’s design a diplexer with 3 dB crossover frequency at fc=400 MHz, RS=RL=50 ohms, each of degree 7. The LP channel will have the transmission zeros Ninf=3, FTZ1=475 MHz, FTZ2=550 MHz.

The HP channel will have transmission zeros, Nzero=3, FTZ1=FTZ2=300 MHz.

- The command set for the design of the LP channel is shown in Figure 17.11. Note that we have selected the option Type B to get the first element on source side in shunt form. This shunt element will be deleted to make the first element a series one. The synthesized filter is saved as LPD7. Its first element is deleted and the resulting circuit is saved as LPD7A (Figure 17.11.a).

- The HP channel is also designed through the same commands, by selecting HPF option. Synthesized filter is saved as HPD7. Its first element is deleted and resulting circuit is saved as HPD7A (Figure 17.11.b).

- Next two diplexer circuits  are formed. First LPD7A is loaded and HPD7A is inserted as External One Port (Figure 17.11.c-d). Then HPD7A is loaded and LPD7 is inserted as External One Port (Figure 17.11.e-f). Both forms of the diplexer can be used for plotting responses or tuning elements using Plot / Tune Multiplexer command.

- Figure 17.11.g and h show the resulting insertion loss and return loss responses before tuning. It is seen that in cross-over region insertion loss of channels are degraded (overlapping bands) and return loss is less than 10 dB. We may use Tune Element command both to improve the return loss and also adjust the cross-over loss.

- Figures 17.11.i and k shows the tuned versions of the filters after a few rounds of rough tuning while observing the return loss only. The responses are shown in Figure 17.11.m and n. Note the shift in the cross-over frequency. Further tuning with consideration given also to the insertion loss and cross-over frequency may give better results.

Exercise: Synthesize a doubly terminated, Type B (starting with shunt element), Chebyshev LP prototype of degree N=8, fp=0.159 and PBRipple=0.01 dB. Delete the first (shunt) element on the source side and then use it to design the LP and HP channels of a LP-HP diplexer with cross-over frequency fc=400 MHz.

Exercise: Design a doubly terminated, Type B, elliptic LP prototype of degree N=8, with Ninf=4, and two FTZ’s, both at fz=0.25. Take fp=0.159 and PBRipple=0.01 dB. Delete the first (shunt) element on the source side and then use it to design the LP and HP channels of a LP-HP diplexer with cross-over frequency fc=400 MHz.

Figure 17.11. LP-HP diplexer design by direct synthesis of doubly terminated LP and HP channel filters.

Figure 17.11. LP-HP diplexer design by direct synthesis of doubly terminated LP and HP channel filters.

Figure 17.11. LP-HP diplexer design by direct synthesis of doubly terminated LP and HP channel filters.

Lumped Element

MULTIPLEXER

DESIGN

By Cascading

LP-HP Diplexers

17. 6.  MULTIPLEXER DESIGN BY CASCADING LP-HP DIPLEXERS

A variety of multiplexers can be formed by cascading LP-HP diplexers and LP and HP filters, like LP-BP-HP type or BP-BP-BP type. The BP channels make use of the back-to-back nature of LP and HP filters of the diplexers.

Let’s study the subject on triplexers. The same approach is valid for multiplexers of more channels.

EXAMPLE 17. 10: A Chebyshev LP-BP-HP triplexer by cascading LP-HP diplexers

Let’s split the complete frequency band into three regions: f<400 MHz, 400<f <450 MHz and f >450 MHz.  The proposed topology is shown in Figure 17.12.a. A LP-HP diplexer with fc=450 MHz is cascaded at its LP output port with another LP-HP diplexer with fc=400 MHz. The HP output of the first diplexer passes the frequencies above fc=450 MHz. The LP output of the first diplexer transfers the signals in the frequency range f<450 MHz to the second diplexer. The HP branch of this diplexer passes the frequency range 400-450 MHz, hence acts as a 400-450 MHz BPF. The LP branch of the diplexer passes the signals in the range 0<f<400 MHz.

- The command set for the designs of the diplexer with fc=450 MHz is shown in Figures 17.12.b. Filter degree is taken as N=7 with PBRipple=0.1 dB targeting to get the worst return loss level better than 26 dB. The individual filters of this diplexer are named as LP450 and HP450 while the their diplexer forms are named as DLP450 and DHP450.

- The command set for the designs of the diplexer with fc=400 MHz is shown in Figures 17.12.c. The individual filters of this diplexer are named as LP400 and HP400 while the their diplexer forms are named as DLP400 and DHP400.

- Figure 17.12.d and e shows the circuits of LP450-HP450 and LP400-HP400 pairs.

- The triplexer can be formed as shown in Figure 17.12.f. Here the first diplexer circuit DLP450 is loaded. In the circuit of DLP450 the LP section (LP450) is explicitly shown while HP450 is shown as External 1-Port. Then with cursor under the load end, the command Append Filter / DLP400 is applied to insert the second diplexer to the load end of the LP450 as shown in Figure 17.12.g. In this circuit LP400 appears in explicit form while HP400 appears as External 1-Port.

- Figure 17.12.h shows the insertion losses of the channels (outputs of HP450, HP400 and LP400) and the return loss of the whole triplexer. The cross-over frequencies are 400 MHz and 450 MHz with insertion losses of about 3.14 dB. The slight deviation of the cross-over losses from 3.01 dB and return loss level of about 23 dB instead of 27 dB is due to the interaction of the two diplexers and can be improved by tuning some elements of the diplexers.

Exercise: Design triplexers with closer cross-over frequencies, for example fc1=400 MHz, fc2=425 MHz and fc1=400 MHz, fc2=410 MHz and inspect the cross-over losses. Try to tune out the deviations.

Figure 17.12. Design of a LP-BP-HP triplexer by cascading two LP-HP diplexers.

Figure 17.12. Design of a LP-BP-HP triplexer by cascading two LP-HP diplexers.

EXAMPLE 17. 11: A Chebyshev BP-BP-BP triplexer by cascading LP-BP-HP triplexer with LP and HP filters

The triplexer of the previous example can be converted into a BP-BP-BP type triplexer by cascading it with proper LP and HP filters as shown in Figure 17.13.a.

Channel-1 is selected to pass the frequency range 300<f<400 MHz.

Channel-2 is required to pass the range 400<f<450 MHz, the same band as in the previous example.

Channel-3 is required to pass the range 450<f<500 MHz.

The upper frequency limit is set as 500 MHz by adding a 500 MHz LP filter at the input of the LP-BP-HP triplexer (TRIP1) of the previous example. Thus, the HP output of Diplexer-1 forms Channel-3, passing the range 450<f<500 MHz. The lower frequency of the triplexer can be set as 300 MHz by placing a 300 MHz HP filter at the LP output of Diplexer-2. So, channel-1 is 100 MHz wide while channels 2 and 3 are of 50 MHz wide.

- Figure 17.13.b shows the design of a 500 MHz LP filter. It can be selected as any type and degree. We have selected it as degree 7 Chebyshev filter with PBRipple=0.01 dB. It is saved as LP500.

- Figure 17.13.c shows the design of a 300 MHz HP filter. It can be selected as any type and degree. It also selected as a degree 7 Chebyshev filter with PBRipple=0.01 dB and saved as HP300.

- The formation of triplexer is described in Figures 17.13.f-to-t in step by step manner to see the effects of introducing the filters LP500 and HP300.

- First the LP filter LP500 is loaded. Then with cursor under the load of LP500, the command Append Filter: DLP450 is applied to append the Diplexer-1 of the previous example (Figure 17.13.g). Now this is a LP-BP diplexer with cross-over being at fc=450 MHz. LP500 limits the upper frequency at 500 MHz. 450-500 MHz range comes out of the HP arm of Diplexer-1, thus acting like a BPF with 450-500 MHz passband. The signals in the frequency range 0<f<450 MHz comes out of the LP branch of Diplexer-1. The responses are shown in Figure 17.13.h and i.  Cross-over loss at 450 MHz is 3.015 dB and return loss is close to 30 dB.

- Next, with the cursor at the load end of DLP450, the command Append Filter: DLP400 is applied to append the second diplexer (Figure 17.13.k-m-n-p). Now the signals in the frequency range 0<f<450 that are fed to Diplexer-2 are also split into two channels. 400-450 MHz  signals come out of the HP arm of Diplexer-2 and the signals with frequencies lower than 400 MHz come out of the LP arm of Diplexer-2. Thus, we have formed a LP-BP-BP triplexer with channels 0<f<400 MHz, 400<f<450 MHz and 450<f<500 MHz. Insertion loss plot shows that the cross-over losses are about 3.14 dB   at fc1=400 MHz and fc2=450 MHz. The overall return loss is reduced to about 25 dB.

- Finally the HP filter HP300 is appended to the LP arm of Diplexer-2 to convert that branch into a 300-400 MHz bandpass filter (Figure 17.13.q-r-s-t). So, a BP-BP-BP type triplexer is formed. Insertion loss plot shows that cross-over loss is 3.07 dB at fc1=400 MHz and fc2=450 MHz. Return loss is better than 20 dB in all three passbands. It can be improved by tuning some elements of all component filters.

Exercise: In the above example instead of LP500, use a LPF with fp=475 MHz and instead of HP300, use a HPF with fp=375 MHz to reduce the bandwidths of lower and upper channels to see the effects of interaction of the filters.

Exercise: Design BP-BP-BP triplexers with narrower bandwidths to see the limitations of multiplexer design by cascading LP-HP diplexers.

Exercise: Design multiplexers with more number of channels using the same approach.

Figure 17.13. Formation of a BP-BP-BP type triplexer using LP-HP diplexers.

Figure 17.13. Formation of a BP-BP-BP type triplexer using LP-HP diplexers.

Figure 17.13. Formation of a BP-BP-BP type triplexer using LP-HP diplexers.

Figure 17.13. Formation of a BP-BP-BP type triplexer using LP-HP diplexers.

EXAMPLE 17. 12: An elliptic LP-BP-HP triplexer by cascading elliptic LP-HP diplexers

Let’s now use elliptic filters to split the complete frequency band into three regions: f<400 MHz, 400<f<450 MHz and f>450 MHz.  The topology is the same as the previous example. A LP-HP diplexer with fc=450 MHz will be cascaded at its LP output port with a second LP-HP diplexer with fc=400 MHz. The HP output of the first diplexer will pass the frequencies above fc=450 MHz. The LP output of the first diplexer transfers the signals in the frequency range f<450 MHz to the second diplexer. The HP branch of this diplexer passes the frequency range 400-450 MHz, hence acts as a 400-450 MHz BPF. The LP branch of the diplexer passes the signals in the range 0<f<400 MHz.

All filters of the two LP-HP diplexers will be designed by using the same singly terminated elliptic LP prototype. The specifications of LP prototype are selected as follows (the same as Example 17.5):

PBRipple=0.1 dB, fp=0.159, RS=RL=1, Ninf=3, FTZ1=0.19, FZTZ2=0.225

- Design command set of the LP prototype is shown in Figure 17.12, giving the circuit of Figure 17.12.b. It is saved as PR17.

- Figures 17.12.c-to-h show the design steps of the diplexer with fc=400 MHz. The passband edge frequencies fpLP=387 MHz and fpHP=413.4 MHz of the LP and HP channel filters are found by trials until the return loss of the diplexer makes twin peaks in the cross-over region. Return loss came out to be about 30 dB in the cross-over region and close to 40 dB outside the cross-over region with almost equal ripples. The component filters are saved as LP387 and HP4134 respectively.

- One diplexer circuit is formed by loading LP387 and inserting HP4134 as External 1-Port as shown in Figure 17.12.f-g. It is saved as DLP400.

- The diplexer with fc=450 MHz is also formed in the same way, with fpLP=435 MHz and fpHP=465.5 MHz  found by trials until the twin peaks of return loss are observed in the crossover region (Figure 17.12.i-to-q). The component filters are saved as LP435 and HP466. The diplexer is formed by loading LP435 and inserting HP466 as External 1-Port as shown in Figure 17.12.p. It is saved as DLP450.  Its return loss is also about 33 dB in the cross-over region and close to 40 dB outside the cross-over region.

- Finally the LP-BP-HP triplexer is formed by loading the diplexer DLP450 and appending diplexer DLP400 as shown in Figure 17.12.r-s.

- Return loss of the triplexer is about 27 dB in the cross-over region and about 30 dB outside the cross-over region, as shown in Figure 17.12.t.

- The insertion loss characteristics observed in 400<f<450 MHz range looks like a prefect BPF with FTZ’s in both stopbands. The cross-over loss is about 3.01 dB at both cross-over frequencies, fc1=400 MHz and fc2=450 MHz.

Exercise: Design LP and HP filters and cascade them with the triplexer above to convert it to a BP-BP-BP triplexer.

Exercise: Design LP-BP-HP triplexers with very narrowband BP channels to see the limitations of the approach. As examples Figure 17.13.a shows responses of a triplexer with fc1=400 MHz and fc2=410 MHz. The insertion loss in the passband of the BP channel is close to 3 dB. Figure 17.13.b shows responses of a triplexer with fc1=400 MHz-fc2=405 MHz. Insertion loss of the BP channel approached to 4 dB. It is clear that as the two cross-over frequencies become closer, the rounded corners of LP and HP channels start to overlap with the passband of the BP region increasing the insertion loss in that range.

Figure 17.12. Design of an elliptic LP-BP-HP triplexer.

Figure 17.12. Design of an elliptic LP-BP-HP triplexer.

Figure 17.12. Design of an elliptic LP-BP-HP triplexer.

Figure 17.12. Design of an elliptic LP-BP-HP triplexer.

Figure 17.13. Responses of triplexers with narrowband BP channels.