Lumped Element

BP-BP

DIPLEXERS

and

MULTIPLEXERS

17. 7. CHEBYSHEV BP-BP DIPLEXER/ MULTIPLEXER DESIGN FROM LP PROTOTYPES

# In the previous section we have seen multiplexers formed by cascading LP-HP diplexers. That approach is limited to medium-wide bandwidth channels. In most applications involving narrow bandwidths multiplexers are formed by paralleling BP filters. Responses are easier to control if the BP filters are designed by mapping from the same LP prototype. However the quasi-complementary nature of adjacent Chebyshev filters lead to some degradation in responses. Such degradations can be controlled by using special circuit sections named as susceptance annulling or compensating sections. Modifications of the filters for adjustment of element values or for obtaining a specific topology will also degrade the responses. Such degradations can be corrected to a limited extent by tuning a few or all elements of all channel filters.

# The following four examples are intended to clarify the sources of degradations systematically. After locating these trouble points then we can pass to the actual designs that incorporate the necessary precautions to improve responses.

# EXAMPLE 17. 13. Analysis of interactions between channels of Chebyshev BP-BP multiplexers

- In order to study the interactions between the channel filters of Chebyshev multiplexers systematically, let’s develop a set of BP filters and connect them in parallel one by one and inspect the degradations on responses. Let’s first design an equal bandwidth  BP-BP diplexer with following channel edge frequencies:

Channel-A: fp1=300 MHz, fp2=fc=400 MHz

Channel-B: fp1=fc=400 MHz,  fp2=500 MHz.

The cross-over will be at 400 MHz. Let’s use the LP Prototype, PR5 developed in Example 7.2, with N=7, and PBRipple=0.3 dB which leads to return loss level of 25-29 dB. The design steps are repeated in Figure 17.14 for convenience. All the BP channel filters will be designed from this prototype, using Design / By User Defined Prototype approach. After interconnection of the filters the return loss level will degrade to some extent because of the interaction of the filters.

Figure 17.14. Design of the LP prototype using normalized LP-HP Diplexer approach.

- Figure 17.15.a shows the command set for the channel-A BPF. Design / By User Defined Prototype approach is used. This filter is saved as  BP5A.

- Figure 17.15.b shows the command set for the channel-B BPF. This filter is saved as  BP5B.

- One version of the diplexer is formed by inserting BP5A into BP5B as External 1-Port as shown in Figure 17.15.b-c and saved as DXBP5B. The second version of the diplexer is formed by inserting BP5B into BP5A as External 1-Port as shown in Figure 17.15.d-e and saved as DXBP5A.

- Figure 17.15.f and g shows the insertion loss and return loss responses of the diplexer. Cross-over loss and frequency are exact, as expected. However there are some degradations and deformations in the return loss compared to the prototype LP-HP diplexer. The sources of degradations are as follows:

  - The two filters tend complement each other at the cross-over frequency fc=400 MHz. Therefore return loss is good in the cross-over range, however NOT as good as in the prototype LP-HP diplexer. The reason is that, in the LP-HP prototype diplexer the LP and HP filters are forming geometrically symmetric pair. That is,

fpLPfpHP=fc2								(17.7.1)

This property is mapped to the two BP filters obtained from this LP Prototype as follows:

fp1a∗fp2b=fp2a∗fp1b=fc2					(17.7.2)

This is termed as geometric symmetry relation. It sets a certain ratio between the bandwidths of the two channels as follows:

Let’s define the bandwidths as

Ba=fp2a−fp1a							(17.7.3)

Bb=fp2b−fp1b							(17.7.4)

Using the relations given in Eq. 17.7.1 we can show that

BbBa=fc2fp1afp2a=fc2f0a2						(17.7.5)

where

f0a2=fp1afp2a							(17.7.6)

is the geometric passband center of Channel-A. Thus, in order to have complementary channels, the bandwidths of the channels should not be set arbitrarily. Otherwise we will get extra deformations in response.

  - In this example we have only fp2afp1b=fc2. The condition, fp1afp2b=fc2 is not satisfied. This is one factor creating part of the degradation in response.  In order to satisfy this condition we have to modify either fp1A or fp2B. That is the bandwidths of the two channels will be different. This case will be considered below.

  - The return losses near to the outer edges of the passbands of the two filters are degraded heavily. This is due to simply nonexistence of adjacent filters to complement the outer skirts of the channels. If we had BPF channels below Channel-A and above Channel-B, we would have better return losses within the passbands of Channel-A and Channel-B. However addition of redundant channels just for that purpose is a bit luxury. Instead we can place simpler elements (termed as susceptance annulling or compensating elements) to simulate the effects of redundant filters to improve the return losses of the channels. This case will also be studied later.

  - A third source for the degradation of passband return losses is the negative interference of the two filters in the cross-over region due to the existence of the outer skirts of the channels. In the LP-HP diplexers we had only the adjacent (inner) skirts of the LP and HP filters which were complementing each other. When the two channels are formed by BPF’s, the effects of the lower skirt of Channel-A and upper skirt of Channel-B come into picture. Since these skirts don’t complement each other return loss is degraded both in the cross-over region and in the passbands of the channels. Naturally this interaction gets more pronounced as the bandwidths get narrower (Exercise).

Frequency symmetric diplexer:

- Let’s check the return loss response of a frequency symmetric BPF pair by selecting upper edge frequency of Channel-B (renamed as Channel-B1) as fp2B=fc2/fp1A= (400)2 /300 = 533.3 MHz to satisfy the proper bandwidth ratio given in Eq. 17.7.4. This is described in Figure 17.16.a. The new filter is saved as BP5B1.

- BP5A is inserted into BP5B1 to form the diplexer as shown in Figure 17.16.b. It is seen that return loss in the cross-over range became symmetric about the cross-over frequency, at about 23.5 dB level. The return loss picture in the cross-over region resembles a squeezed version of the return loss of the prototype LP-HP diplexer which had 35 dB return loss about the cross-over frequency. The degradation from 35 dB to 23.5 dB is due to the effects of the outer skirts of the two BPF’s. We can tune elements of both channels to improve the response. However we will delay this stage to the forthcoming examples, after getting further insight on the factors leading to degradations.

Frequency symmetric diplexer with redundant channels (A QUDRUPLEXER)

In order to study the improvements in responses of interior channels, let’s add two more channels, Channel-C and Channel-D, with following band edges:

Channel-C: fp1C=533.3 MHz, 	fp2C=(533.3)2/ fp1B=(533.3)2/400=711 MHz.

Channel-D: fp1D=711 MHz,     	fp2D=(711)2 / fp1C= (711)2 /533.3 = 947.9 MHz.

Note that all four filters form geometrically symmetric pairs with their adjacent neighbors.

Connecting all four channels in parallel we can form a quadruplexer which may also be treated as a diplexer formed by Channel-B1 and Channel-C with the outer channels (A and D) being redundant, used just for improvement of the responses of B and C.

- Figure 17.17.a shows the command set for the design of Channel-C BPF. It is saved as  BP5C.

- In order to check return loss responses as more channels are inserted, a diplexer is formed by inserting BP5B1 into BP5C as External 1-Port as shown in Figure 17.17.b and saved as DXBP5C. Its return loss response is shown in Figure 17.17.c. It came out to be in the expected form and level within the cross-over region around fc1=533 MHz. It is close to 24 dB.

- Figure 17.17.d shows the command set for the design of Channel-D BPF. It is saved as  BP5D.

- In order to check the complementary property, a diplexer is formed by inserting Channel-C filter into Channel-D as shown in Figure 17.17.e. It is saved as DXCD. Return loss response is close to 24 dB level in the cross-over region around fc2=711 MHz, as expected (Figure 17.17.f).

- Next in order to inspect the effect of Channel-B1 on return loss, a triplexer is formed by inserting BP5B1 into DXCD as shown in Figure 17.17.g-h. It is saved as TRBCD. Return loss response shown in Figure 17.17.i shows that although the return loss about fc2=711 MHz is degraded slightly and the  return loss of Channel-C is improved considerably (better than 24 dB around band center). Insertion loss response shows that cross-over losses are unaffected by the new channels (Figure 17.17.k).

- Finally, Channel-A filter is also inserted into TRBCD to form a quadruplexer as shown in Figure 17.17.m. It is saved as QDBP5D (Figure 17.17.n). Return loss responses of all channels are effected appreciably by the addition of this channel. Only the middle channels (B1 and C) have acceptable return loss levels (Figure 17.17.p-q). This is because the adjacent channels (A and D) help in improving the responses of these channels.

OBSERVATIONS:

  - In all the above cases we have tried to inspect the interactions of adjacent filters in a multiplexer with geometrically symmetric adjacent channels, without any tuning or optimization procedure. The frequency symmetry property provides some improvement in return loss response in the cross-over ranges and also within the passbands of the interior channels. So, for arbitrary channel bandwidths we will get an extra degradation because of the deviations from frequency symmetry. Since in practice we will rarely be allowed to use such frequency symmetric channels, we should expect worse results for arbitrary bandwidth multiplexers  (without optimization!).

  - The outer channels help to improve the return loss levels of the interior channels. So, we may use either redundant outer channels to improve the return loss of the actual channels or use simpler circuit sections to simulate the actions of the redundant channels (usually termed as susceptance annulling elements or compensating elements), as will be described in the next example.

Figure 17.15. Design of a Chebyshev BP-BP diplexer without compensating elements.

Figure 17.15. Design of a Chebyshev BP-BP diplexer without compensating elements.

Figure 17.16. A frequency symmetric Chebyshev BP-BP diplexer without compensating elements.

Figure 17.17. A Chebyshev quadruplexer with frequency symmetric channels without compensating elements or a diplexer with redundant channels.

Figure 17.17. A Chebyshev quadruplexer with frequency symmetric channels without compensating elements or a diplexer with redundant channels.

Figure 17.17. A Chebyshev quadruplexer with frequency symmetric channels without compensating elements or a diplexer with redundant channels.

Figure 17.17. A Chebyshev quadruplexer with frequency symmetric channels without compensating elements or a diplexer with redundant channels.

EXAMPLE 17. 14. Chebyshev BP-BP diplexer with susceptance annulling elements

Let’s use the quadruplexer of the previous example to form a diplexer using Channel-B1 and Channel-C only by simplifying the filters of Channel-A and Channel-D, as follows:

- The quadruplexer of the previous example QDBP5D is shown in Figure 17.18.a. In this circuit the elements of BP5D are explicitly seen while the other channel elements are hidden in the External 1-Port circuits. By applying Tune Element command to the first few elements of BP5D we can see that it helps to improve the return loss of the adjacent Channel-C (Figure 17.18.b). We also notice that the elements far from the source end are almost ineffective. Therefore we Delete all the elements of BP5D except the first LC element in series (Figure 17.18.c). After deleting the others, this element can be treated either as a series LC with RL=0 or a shunt LC with RL=∞. We may also leave this circuit aside and insert the LC element in front of either BP5B1 or BP5C as a shunt element.

- The same quadruplexer can also be shown by first loading BP5A and inserting the other channels as External 1-Ports. Since only the first few elements of this circuit is effective in shaping the return loss of Channel-B1, we load BP5A and delete all elements except the first series LC resonator (Figure 17.18.d). This LC element can be inserted into BP5B1 as a shunt element on source side.

- In Figure 17.18.e-to-g, BP5B1 is loaded and the shunt LC elements (susceptance annulling or compensating elements) are inserted to the left of BP5B1 in parallel with the Channel-C filter BP5C. Thus, the diplexer circuit is simplified.

- Applying Tune Element command on the compensating elements we can improve the return loss of the diplexer as seen in Figure 17.18.h-i.

- Fine trimming of the elements of both BP5B1 and BP5C is possible by forming also the other version of the diplexer where BP5C appears explicitly and BP5B1 appears in External 1-Port form together with the compensating elements as described in Figures 17.18.k-to-p.  Fine tuning gives the circuit shown in Figure 17.18.q with responses shown in Figure 17.18.r and s. It is seen that the compensating elements act as FTZ sections which increase the selectivity of the channels also.  Cross-over loss is almost exact at fc=533.3 MHz. Return loss is better than 24 dB in both passbands.

CONCLUSIONS:

- If a diplexer is formed by paralleling two bandpass filters with arbitrary bandwidths the return loss is degraded slightly in the cross-over region because of the improper bandwidth ratio.

- The passband return losses of the channels are also effected by the presence of the outer skirts of the adjacent channels.

- The most effective factor that degrades the passband return losses are the absence of complementing (redundant) filters on lower side of the lower channel and upper side of the upper channel. The corrective effects of the redundant channels can be simulated by inserting two shunt arm series LC resonators. One of these resonators creates a FTZ in the lower stopband of the  lower channel filter and the other one creates a FTZ in  the upper stopband of the upper channel. Traditionally these FTZ elements are named as susceptance annulling sections.

- When multiplexers are formed by paralleling more than two filters, the susceptance annulling sections are used for correction of the return losses of only the outermost channels.

- The multiplexers formed by series connection of the BP filters are just the duals of the parallel connected versions. Hence the same comments apply to such multiplexers also. In such multiplexers the susceptance annulling sections (FTZ elements) are in the form of parallel LC sections, connected in series with the channel filters.

Figure 17.18. Design of a Chebyshev BP-BP diplexer with susceptance annuling elements.

Figure 17.18. Design of a Chebyshev BP-BP diplexer with susceptance annuling elements.

Figure 17.18. Design of a Chebyshev BP-BP diplexer with susceptance annuling elements.

Figure 17.18. Design of a Chebyshev BP-BP diplexer with susceptance annuling elements.

EXAMPLE 17. 15. A narrowband Chebyshev BP-BP diplexer with susceptance annulling sections

Just as another example for the application of the observations and conclusions reached in the previous example, let’s design a BP-BP diplexer in a shorter way. Channel specifications are as follows:

Channel-A: fp1a=400 MHz, fp2a=425 MHz.

Channel-B: fp1b=425 MHz, fp2b=450 MHz.

A passband return loss about 20 dB is targeted. We will use the LP prototype, PR5 that was used in the previous examples.

- Design of Channel-A is shown in Figure 17.19.a. The resulting filter is saved as CHA.

- Design of Channel-B is carried out using the same approach. The resulting filter is saved as CHB (Figure 17.19.b).

- The filter CHA is inserted into CHB as External 1-Port (Figure 17.19.c).

- The resulting response is shown in Figure 17.19.d. Return loss is heavily degraded.

- In order to improve the response the susceptance annulling sections of CHA and CHB are inserted as shown in Figure 17.19.e-f. Just as a trial we have selected the inductance of these shunt connected series LC resonators as 512 nH, the same as the first elements of CHA and CHB. The corresponding capacitor values are selected slightly higher for the CHA and slightly lower for CHB filter to locate the corresponding FTZ’s just below and just above the two passbands.  Then Plot/Tune Multiplexer command is applied to tune the capacitors to adjust the locations of the two FTZ’s properly to get the best return loss in both channels. The resulting circuit and responses are shown in Figures 17.19.g and h. The design is completed. If necessary the other elements of both filters may also be tuned.

Exercise: Design Triplexer and quadruplexers using the same approach.

Figure 17.19. Design of a narrowband BP-BP diplexer with susceptance annulling sections.

Figure 17.19. Design of a narrowband BP-BP diplexer with susceptance annulling sections.

# Effects of Scaling:

- In most cases the modifications made for adjustment of element values or for changing the topology of the circuits effect the complementary nature of the adjacent filters, thus degrading the response. As an example for the modifications that can be applied on the filters of the BP-BP diplexer designed in the above example, let’s make scale both channel filters by inserting inverters.

- The circuit of CHA is loaded and Find Dual command is applied to form the dual circuit where the first element will be in shunt form (Figure 17.20.a-b). This step is needed because the forthcoming modifications will add series elements at input which is necessary to form connection of the channel filters in parallel form. Otherwise we would have shunt first elements which is not proper for parallel connections of channels, as will be clear after the following stages.

- The command Inverters / Scale Cct By Inserting Inv / Specify Adjacent Ind: L=10 nH is applied to make the shunt inductor on left 10 nH (Figure 17.20.c).

- Then the inverter at source end only is replaced by its capacitive-L equivalent by applying the command Inverters / Replace Inv By / End Inv to / Capacitive-L (Figure 17.20.d). The command Split / All Two Element Resonators is applied to convert all LC elements their into L and C components (Figure 17.17.e).

- All the succeeding inductors can be adjusted to 10 nH by using the Pi-Tee-L transformations described in Chapter-6. However since they are exact transformations which do not effect input impedances we will not go into these transformations and leave the circuit as it is. It is saved as CHAD.

- Next the other channel filter CHB is loaded and the same transformations are applied to form the circuit scaled by inverters, but the inverter at source end is replaced by its inductive-L section equivalent (Figure 17.20.d-e-f-g-h). It is saved as CHBD.

- Then the filter CHAD is inserted into CHBD as External 1-Port  to form the diplexer (Figure 17.20.i).

- The insertion loss and return loss responses are shown in Figure 17.20.m. Return loss response is degraded to some extent. This degradation can’t be corrected by using susceptance annulling sections (Exercise).

- Return loss response can be improved slightly by tuning all elements of both filters as shown in Figure 17.20.n (Exercise).

- Replacement of inverter of one filter by Capacitive-L section and the other one by Inductive-L section helps improving the return loss. That is, the capacitive and inductive nature of the input impedances of the two filters tend to complement each other. As exercise replace the inverter of CHB also by Capacitive-L section and inspect the return loss.

- Figure 17.20. Effect of scaling the circuits by inverters.

- Figure 17.20. Effect of scaling the circuits by inverters.

- EXAMPLE 17. 16:  Coupled shunt resonator version of the BP-BP diplexer

- Direct coupled resonator versions of the bandpass filters are commonly preferred ones. The channel filters of the previous example, CHA and CHB can be converted into capacitively and inductively coupled series or shunt resonator forms. Couplings between resonators can be made all capacitive, all inductive or mixed ones, as desired. However these conversions also effect the input impedances and tend to destroy the complemantary nature of the filters, leading to degradations in return loss.

- Let’s make couplings all capacitive in one filter and all inductive in the other as an attempt to preserve the complementary nature.

- In Figure 17.21.a the circuit of CHA is loaded. With cursor under the series LC resonators, the command Inverters / Replace By Inverter Equivalent / K=50 is applied to convert them into shunt arm parallel resonators surrounded by two inverters of arbitrarily set 50 ohms impedances (Figure 17.21.b). Simplify Circuit command eliminates the unity phase changing transformers.

- With cursor under the first shunt LC resonator on left, the command Inverters / Scale Element By Inv / Inductor: L=10 nH is applied to set the shunt inductor of that resonator to 10 nH by adjusting the inverters on their two sides (Figure 17.21.c).

- Then the command Inverters / All Equal Left-Right / Inductors is applied to equate the inductors of all shunt resonators (Figure 17.21.d).

- With cursor under the rightmost inverter the command Inverters / Absorb TRF By Inverter is applied to absorb the transformer into inverter.

- Then with cursor under the inverters at source and load ends the command Inverters / Replace Inverters By / End Inverter to / Capacitive-L is applied to replace them with capacitive-L equivalents. Simplify Circuit command combines the negative shunt legs of L-sections with the adjacent positive elements (Figure 17.21.d-e).

- Then with cursor under the internal inverters the command Inverters / Replace Inv By / Pi-Tee Type / Capacitive Pi is applied to replace inverters by their capacitive Pi equivalents with negative shunt capacitors. Simplify Circuit command combines the negative shunt capacitors by their neighbors (Figure 17.21e-f).

- The resulting circuit is saved as CHAR.

- The same transformations are applied also on CHB, but with all inverters replace by their Inductive-L and Inductive-Pi equivalents (Figure 17.21.g-n). The resulting circuit is saved as CHBR (Figure 17.19.n).

- The filters CHAR is inserted into CHBR to form diplexer circuit as External 1-Port as shown in Figure 17.21.n-p.

- Figure 17.19.q and r show the insertion loss and return loss response without any tuning. Responses are degraded because of the interactions of the component filters. This degradation can’t be corrected by using susceptance annulling elements.

- Figure 17.21.s shows the return loss after tuning all elements of both filters. Return loss level is decreased considerably and bandwiths are narrowed.

- Exercise: Repeat the design by designing LP Prototypes with lower PBRipple values.

- Figure 17.21. Design of a diplexer with coupled shunt resonator type filters.

- Figure 17.21. Design of a diplexer with coupled shunt resonator type filters.

- Figure 17.21. Design of a diplexer with coupled shunt resonator type filters.

17. 8.  CHEBYSHEV BP-BP DIPLEXER DESIGN BY DIRECT SYNTHESIS OF SINGLY TERMINATED CHANNEL FILTERS

The previous BP-BP diplexers were designed by mapping from a LP prototype. The mapping function creates equal numbers of transmission zeros at f=0 and f=. The other versions with different numbers of transmission zeros at f=0 and f= can only be obtained by approximate circuit transformations involving inverters. Also the prototype-to-BP mapping function maps a single finite transmission zero (FTZ) into two FTZ’s, on the two sides of the passbands. This may not be a desirable feature when asymmetric responses are needed. Such problems can be avoided by synthesizing the channel filters directly as singly terminated bandpass filters. At first sight that approach appears to be attractive because of the ease of formation of the desired asymmetric responses by placing transmission zeros as necessary. However, as will be described in the following examples, it may be tedious to set the cross-over frequency and loss. Another tedious process is that the synthesized singly terminated filter pairs need unknown transformers at their input (diplexing) ports to form complementary pairs. These transformers need be determined by tuning trials. Due to such difficulties, this approach may not be the quickest one. However it may be useful especially when asymmetric finite transmission zeros are needed.

Let’s see some of such problems of direct design of singly terminated BP filters as diplexer channels on some examples, by starting form Chebyshev type filters and continuing with filters having FTZ’s.

EXAMPLE 17. 17: Direct synthesis of the filters of a BP-BP diplexer with Nzero=Ninf=5

Let’s design a diplexer formed by Chebyshev type channel filters of degree 10 with the following passband edges:

Channel-1: fp1=370 MHz,   fp2=430 MHz

Channel-2: fp1=430 MHz,  fp2=490 MHz

The  transmission zeros are shared between f=0 and f= as Nzero=5, Ninf=5, leading to degree 10  filters with topologies similar to those obtained by the LP-Prototype approach. Just to get an idea, different  PBRipple specifications will be studied, such as PBRipple=3.01 dB, 1 dB, 0.1 dB and 0.01 dB.

Design stages for PBRipple=3.01 dB are shown in Figure 17.22. The resulting Channel-1 filter is shown in Figure 17.21.a. A unity transformer is inserted at source end and it is saved as A3.FIL  (Figure 17.22.b). The filter of Channel-2 is also designed using the same steps and saved as B3.FIL (Figure 17.22.c). Then the two filters are combined to form the diplexer as shown in Figure 17.22.d. Figure 17.22.e shows the insertion loss and return loss responses. The passband return loss is unequal ripple. Using Monte Carlo and Tune Element commands return loss is made equal ripple at about 8 dB level as shown in Figure 17.22.f. The optimized filters are also shown in the same figure, saved as A3f and B3f respectively.

The filters of Channel-1 and Channel-2 for PBRipple=1 dB are also designed using the same steps and saved as A1.FIL and B1.FIL respectively and then the diplexer is formed (Figure 17.22g-h-i). Figure 17.22.k shows insertion and return loss responses before optimization while Figure 17.22.m shows the circuit and return loss after a rough optimization. Return loss is settled at about 11 dB level. Further optimization may improve it some more.

The results for PBRipple=0.1 dB are shown in Figure 17.22.n-p-q-r. The circuits are optimized giving a return loss level of about 14 dB as shown in Figure 17.22.s.

The results for PBRipple=0.01 dB are shown in Figure 17.22.t-u-v-w. The circuits are optimized giving a return loss level of about 20 dB as shown in Figure 17.22.x.

We notice that in all these cases the cross-over loss is slightly different from 3.01 dB. This is because the inner edges of the two passbands should be slightly different from fp=430 MHz. Since we don’t know these edge frequencies we should make some trials until the cross-over loss is settled at the desired level and frequency. Return loss levels that we have obtained by a coarse tuning and optimization are less than those we have obtained by using the LP prototype approach.

All the circuits obtained in this example need some modifications for adjustment of element values. These modifications are similar to those applied in the BP-BP diplexers of the previous examples, hence we will get further degradations in return loss levels (Exercise).

Figure 17.22. Design of a diplexer with singly terminated channel filters with Nzero=Ninf=5.

Figure 17.22. Design of a diplexer with singly terminated channel filters with Nzero=Ninf=5.

Figure 17.22. Design of a diplexer with singly terminated channel filters with Nzero=Ninf=5.

Figure 17.22. Design of a diplexer with singly terminated channel filters with Nzero=Ninf=5.

Figure 17.22. Design of a diplexer with singly terminated channel filters with Nzero=Ninf=5.

Figure 17.22. Design of a diplexer with singly terminated channel filters with Nzero=Ninf=5.

Figure 17.22. Design of a diplexer with singly terminated channel filters with Nzero=Ninf=5.

EXAMPLE 17. 18: Direct synthesis of the filters of BP-BP diplexers with different Nzero and Ninf

As exercise with different topologies that will result by placing different transmission zeros at f=0 and f=, let’s design singly terminated bandpass filters with following specifications:

Channel-1: fp1=370 MHz, fp2=430 MHz, Channel-2: fp1=430 MHz, fp2=490 MHz

For both channels two filters will be designed, one with Nzero=1, Ninf=9 and the other with Nzero=9, Ninf=1.

- Figures 17.23.a-b-c show the filters with Nzero=1 and Ninf=9. A unity transformer is inserted at their inputs. The transformer is unknown because there was no termination for its calculation in the synthesis stage (RS=0). It will be found by tuning after the formation of a diplexer by tuning the turns ratios of the transformers while observing the return loss. The circuits are saved as A19 and B19 respectively.

- Figures 17.23.d-e-f  show the filters with Nzero=9 and Ninf=1. A unity transformer is inserted at their inputs. They are saved as A91 and B91 respectively.

- Figure 17.23.g shows the formation of a diplexer by using the filters A19 and B91. The initial response is heavily degraded. This is because the actual transformers of the filters are not calculated yet. The transformer turns ratios are found by using Tune Element command while observing the return loss response of the diplexer. This is carried out by first loading A19 and inserting B91 as External 1-Port. Then the transformer of A19 is tuned slightly to improve the return loss in the passband of A19. Then B91 is loaded and A19 is inserted as External 1-Port. The transformer of B91 is tuned to improve the return loss in the passband of B91 until the return loss in both passbands become equal. This process is repeated iteratively until no more improvement is observed in both passbands. Beyond certain values of these transformers, no further improvement is observed. After that stage we continue improving the return loss by optimizing the element values of both circuits iteratively, using both Tune Element command and/or Monte Carlo-Best Case routine in an iterative manner by loading the two diplexer circuits alternatively. Figure 17.23.h shows the optimized circuits and responses. Return loss is improved to about 20 dB after a rough optimization process. Further improvement may be possible. The optimized filters are saved as A19f and B91f respectively.

- Figure 17.23.i shows the formation of a diplexer by using the filters A19 and B19. The transformer turns ratios are found by using Tune Element command while observing the return loss response of the diplexer, as described for A19-B91 case. Figure 17.23.k shows the optimized circuits and responses. Return loss is improved to about 16 dB after a rough optimization process. Further improvement is possible. The optimized filters are saved as A19g and B19g  respectively.

- Figure 17.23.m shows the formation of a diplexer by using the filters A91 and B19. The transformer turns ratios are found by using Tune Element command while observing the return loss response of the diplexer, as described for A19-B91 case. Figure 17.23.n shows the optimized circuits and responses. Return loss is improved to about 19 dB after a rough optimization process. Further improvement is possible. The optimized filters are saved as A91h and B19h respectively.

- Figures 17.23.p shows the formation of a diplexer by using the filters A91 and B91. The transformer turns ratios are found by using Tune Element command while observing the return loss response of the diplexer, as described for A19-B91 case. Figure 17.23.q shows the optimized circuits and responses. Return loss is improved to about 21 dB after a rough optimization process. Further improvement is possible. The optimized filters are saved as A91k and B91k respectively.

- Looking at the LP-HP complements in which case the LP branch has Nzero=0, Ninf>0 and the HP branch has Nzero>0 , Ninf=0, we expect that the pairs A19-B91 and A91-B19 form better complements than A19-B19 or A91-B91 pairs. However these examples show that none of these are more advantageous compared to the others. All pairs can form diplexers with reasonable return loss levels. Thus, we have shown that the BP-BP diplexer can be designed by direct synthesis of component filters as well as by mapping from the LP prototypes. However in this approach the cross-over frequency and cross-over loss adjustment needs iterative designs. This makes the direct design approach unattractive.

- Actually direct design approach shows its advantage in forming diplexers with filters having asymmetrically placed finite transmission zeros. This will be studied in the forthcoming examples.

- All of these circuits need modifications to adjust element values. The modifications are similar to the ones used in the previous BP-BP diplexer examples. One approach is described below.

Figure 17.23. Diplexers with singly terminated channel filters with Nzero=1, Ninf=9 and Nzero=9, Ninf=1.

Figure 17.23. Diplexers with singly terminated channel filters with Nzero=1, Ninf=9 and Nzero=9, Ninf=1.

Figure 17.23. Diplexers with singly terminated channel filters with Nzero=1, Ninf=9 and Nzero=9, Ninf=1.

Figure 17.23. Diplexers with singly terminated channel filters with Nzero=1, Ninf=9 and Nzero=9, Ninf=1.

Figure 17.23. Diplexers with singly terminated channel filters with Nzero=1, Ninf=9 and Nzero=9, Ninf=1.

Figure 17.23. Diplexers with singly terminated channel filters with Nzero=1, Ninf=9 and Nzero=9, Ninf=1.

Modifications and Element Value Adjustment:

## Let’s consider the pair A19f-B91f for topology changes and element value adjustment. If the filters are to be paralleled then the elements at the source end of the filters must be series elements. This constraint must be observed in the modifications.

- First, the circuit of A19f is loaded (Figure 17.24.a). We have too large and too small element values. Let’s search what can be done to adjust the element value ranges.

- This circuit has minimum number of inductors and all inductors are in series arms. If we would like to keep that property, and make all the inductors equal then, then the following approach works well.

- Find Dual command is applied to form the dual circuit. It is saved as A19fd (Figure 17.24.b).

- Then the command Pi-Tee-L / Equal Shunt Capacitors is applied to equate the shunt capacitors (Figure 17.24.c). Then Find Dual command is applied again giving the targeted equal series inductor version (Figure 17.24.d). This circuit is saved as A19f1. The series inductors are equated at 153 nH level.

- Any attempt to scale down the 153 nH inductance value will introduce shunt elements at input, hence not suitable for parallel connection. In such cases it may be preferable to use series connection of diplexer arms. For series connections we can use the following element value scaling  procedure.

- A19f1 is loaded and the command Inverters / Scale Circuit By Inserting Inverters / Specify Inductor: L=20 nH is applied. This command inserts inverters to scale all the series inductors to 20 nH (Figure17.24.e).

- Then the command Inverters / Absorb TRF By Inverter is applied to the inverter at load end to absorb the adjacent transformer.

- Finally, the end inverters are replaced by capacitive L-sections by applying the command Inverters / Replace Inv By / End Inverters to / Capacitive-L. Simplify Circuit command combines the negative capacitors of the L-sections with the neighboring positive capacitors giving the circuit of Figure 17.24.f. This circuit is saved as A19f1s.

- If series inductively coupled shunt resonator version is desired then the dual circuit A19fd is loaded and the command Pi-Tee-L / Equal Shunt Inductors is applied giving the shunt resonator filter with equal shunt inductors (Figure 17.24.g-h). The shunt inductors are equated at 3.11 nH.

- The shunt inductors can be scaled up for example to 50 nH by inserting inverters, as described in Figure 17.24.h-i. Then the inverters are replaced by capacitive L-sections giving the circuit of Figure 17.24.k. It is saved as A19f2.

- The same transformations are also applied on the filter B91f, resulting in the equal series inductor version of B91f1 (Figure 17.24.m-q), its scaled version, B91f1s with 20 nH series inductors (Figure 17.24.p-s), and the shunt resonator version B91f2 with 20 nH shunt inductors (Figure 17.24.t-w).

- Figure 17.24.x described the diplexer formed by paralleling A19f1 and B91f1. Since no element scaling is done, its response is the same as the original diplexer formed by the A19f-B91f  pair.

- Figure 17.24.y shows the diplexer formed by series connection of A19f1s and B91f1s. The transformations used in forming and scaling the circuit effected the input impedances of both filters and hence led to appreciable distortion on response. However the return loss response can be recovered to a certain extent by tuning all elements of both channels, as shown in the figure. The use of inverters and their replacement by capacitive L-sections led to some narrowing in the bandwidth.

- Figure 17.24.z shows the diplexer formed by paralleling the shunt resonator filters. Since this circuit also passed through some transformations effecting the input impedances the return loss is degraded and it can be recovered by tuning all elements of both channels, as shown in the figure.

Figure 17.24. Circuit transformations for modification of topology and element values.

Figure 17.24. Circuit transformations for modification of topology and element values.

Figure 17.24. Circuit transformations for modification of topology and element values.

Figure 17.24. Circuit transformations for modification of topology and element values.

Figure 17.24. Circuit transformations for modification of topology and element values.

###### Figure 17.24. Circuit transformations for modification of topology and element values.

17.  9. CHEBYSHEV BP-BP DIPLEXER DESIGN USING FORESHORTENED DOUBLY TERMINATED FILTERS

- It is possible to combine independently designed doubly terminated filters to form diplexers and multiplexers. This approach needs modifications mainly on the first few elements of the filters, though all elements need be tuned to get maximal improvements in response. The modifications may aim one of the two possibilities:

- The modifications must change the input impedances of the adjacent channels in such a way that they will load each other in aiding way to improve the overall response of the multiplexer (complementing each other).

- The filters must not load each other. That is, if a multiplexer is formed by paralleling the filters, then the input impedance of one filter must appear approximately as open circuit in the passbands of the other filters. In the series connected multiplexers the input impedances must appear as short circuit in the passbands of the other filters.

The first option may be used if the channel passbands are close to each other, like 3 dB cross-over loss cases. If the channels have relatively wide guard bands we may still follow the first option. However we may also prefer to use the second option if the resulting structure is more convenient. In any case an optimization stage is needed to adjust the response.

In both cases we will be interested in the input impedances of adjacent channels. The variation of input impedances with frequency can be checked most easily through Smith Chart. Inspections show that complementary properties of adjacent channels become more pronounced by deleting the first elements of the adjacent channels. This is termed as foreshortening the filters [4].

There are some useful rules of thumb in designing diplexer channel filters:

- If one channel has a transmission zero pattern Nzero=N1 and Ninf=N2, then the other channel will have Nzero=N2 and Ninf=N1. This may lead to the best return loss.

- Optimization will be easier if the adjacent filters has finite transmission zeros in the passband of  the other channel. The finite transmission zeros will decrease the loadings of the channel filters.

Let’s continue discussions along with some examples.

EXAMPLE 17. 19: A BP-BP diplexer formed by foreshortening doubly terminated coupled resonator filters

Let’s design the same BP-BP diplexer of the previous example, now by synthesizing doubly terminated filters and using foreshortening:

Channel-1: fp1=370 MHz, fp2=430 MHz, Nzero=1, Ninf=9

Channel-2: fp1=430 MHz, fp2=490 MHz, Nzero=9, Ninf=1

Design without scaling elements:

- Design stages of channel-1 filter are shown in Figures 17.25. Figure 17.25.a shows the synthesized filter. Element value spreads are too high. Let’s target to have a coupled shunt resonator filter with equal shunt  inductors. This is done by applying the command Pi-Tee-L / Equal Shunt Inductors (Figure 17.25.b). The filter is saved as DA1.FIL.

- The first element on source side is a shunt LC resonator. In forming a diplexer by paralleling filters the first elements need be series elements. In this approach we simply delete the first element (foreshortening) leaving a series inductor as the first element on source side. The foreshortened filter is saved as DA1R.FIL. Figure 17.25.c shows the Smith Chart responses in the passband of the Channel-2 (430-490 MHz) before and after deleting the first element. Clearly, the first shunt element presents a heavy loading to the second channel. After foreshortening we get an inductive input impedance which must be compensated (complemented or cancelled?) by tuning input impedance of the other channel (Channel-2) within its passband to avoid the degradation of response in that passband. Clearly, the cancellation of this impedance would be easier if the Channel-2 filter has a capacitive input impedance in its passband. In fact that is why we have selected Nzero=9 and Ninf=1 filter for channel-2 which leads to capacitive series coupling elements.

- Channel-2 filter is also designed in the same way, leading to the circuit shown in Figure 17.25.d. Its equal shunt inductor coupled resonator form is shown in Figure 17.25.e. All series coupling elements are capacitors. It is saved as DB1.FIL. It is foreshortened by deleting the shunt resonator at source end and saved as DB01R.FIL. Smith Chart responses in the passband of Channel-1 (370-430 MHz) before and after foreshortening are shown in Figure 17.25.f. After foreshortening we get a capacitive input impedance which will be compensated by tuning the input impedance of the other channel (Channel-1) within its passband (370-430 MHz) to avoid degradation of the response in that passband. This cancellation will be easier if the input impedance of Channel-1 is inductive. This is the main reason why we have selected the transmission zero pattern Nzero=1, Ninf=9 leading to all inductive series coupling elements.

- Figures 17.25.g and h show the foreshortened Channel-1 and Channel-2 filters.

- By inserting the filter DA1R into DB1R as External 1-Port we can configure diplexer form-1 as shown in Figure 17.25.i.

- Similarly by inserting the filter DB1R into DA1R as External 1-Port we can configure diplexer form-2 as shown in Figure 17.25.k-m. These two diplexer forms can be used to optimize the responses by using Tune Element and Monte Carlo commands in an alternating way.

- Figure 17.25.n shows the return loss response before tuning. By loading diplexer form-2 and diplexer form-2 in an alternating way the elements are tuned to improve the return loss response. Initially  Tune Element command is applied to all elements to get an idea on the effects of each element of both channels on the response. After getting some improvement, we resort to Monte Carlo method is applied for finer improvements. After a few rounds of iterative trials we get the circuits shown in Figure 17.25.p and q with the final response shown in Figure 17.25.r. A slight shift in cross-over frequency and some slight narrowing in the passband are observed.

- Figure 17.25.s shows the input impedances of both channels within their passbands and in the cross-over region. Note the symmetry with respect to the horizontal axis. The inductive reactance of channel-1 (370-430 MHz) in the 430-490 MHz region is compensated by the deformations in the passband response of channel-2. Similarly, the capacitive reactance of Channel-2 (430-490 MHz) in the 370-430 MHz region is compensated by the deformations in the passband response of Channel-1.

Exercise: Repeat the design by using Nzero=1 and Ninf=9 for Channel-2 filter to see the difficulties and limitations in improving the return loss response .

Exercise: Repeat the design by using Nzero=9 and Ninf=1 for Channel-1 filter and Nzero=1 and Ninf=9 for Channel-2. Tuning to improve the response will be easy.

Exercise: Repeat the design by using Nzero=9 and Ninf=1 for Channel-1 filter and Nzero=9 and Ninf=1 for Channel-2 to see the difficulties and limitations in improving the return loss response .

Figure 17.25. Diplexer design using foreshortened doubly terminated filters.

Figure 17.25. Diplexer design using foreshortened doubly terminated filters.

Figure 17.25. Diplexer design using foreshortened doubly terminated filters.

Figure 17.25. Diplexer design using foreshortened doubly terminated filters.

Figure 17.25. Diplexer design using foreshortened doubly terminated filters.

Design with scaled element values:

As it was seen in the previous examples, scaling element values usually changes the topology and hence input impedances of the filters leading to considerable degradations in response. Let’s consider the same example, now with element values scaled by inverters.

- Channel-1 filter DA1 is loaded (Figure 17.26.a) and all the shunt inductors are scaled to 10 nH by applying the command Inverters / Scale Cct By Inserting Inverters / Specify Inductor: L=10 nH (Figure 17.26.b). The circuit is saved as DA2.

- The first two elements are deleted and the resulting circuit is saved as DA2R (Figure 17.26.c).

- Figure 17.26.d shows the Smith Chart responses in the passband of Channel-2 (430-490 MHz) before scaling the circuit, after scaling by inverters and after foreshortening by deleting the first two elements. As expected the impedance plot is inverted after scaling by inverters. Foreshortening leaves behind an inductive input impedance far away from the center of Smith Chart.  This impedance need be compensated by tuning Channel-2 filter in its passband.

- Then Channel-2 filter DB1 is loaded (Figure 17.26.e) and all the shunt inductors are scaled to 10 nH by applying the command Inverters / Scale Cct By Inserting Inverters / Specify Inductor: L=10 nH (Figure 17.26.f). The circuit is saved as DB2.

- The first two elements are deleted and the resulting circuit is saved as DB2R (Figure 17.26.g).

- Figure 17.26.h shows the Smith Chart responses in the passband of Channel-1 (370-430 MHz) before scaling the circuit, after scaling by inverters and after foreshortening by deleting the first two elements. As expected the impedance plot is inverted after scaling by inverters. Foreshortening leaves behind an capacitive input impedance far away from the center of Smith Chart.  This impedance need be compensated by tuning Channel-1 filter in its passband.

- Two diplexer circuits are formed. In one case DA2R is loaded and DB2R is inserted as External 1-Port and in the other case DB2R is loaded and DA2R is inserted as External 1-Port (Figure 17.26.i-k-m-n).  Elements of these two diplexers will be tuned by loading them alternately and using Tune Element and Monte Carlo commands iteratively.

- Figure 17.26.p shows the return loss before tuning elements.

- Figures 17.26.q and r shows the resulting circuits after a few rounds of tuning. Figure 17.26.s shows the improved responses after tuning. Further improvement is possible.

- The inverters at load ends of the two filters can be replaced by their capacitive or inductive equivalents.

- Let’s compare responses and the number of elements in these circuits with those of the coupled resonator version solution of previous example, for example with the circuit of Figure 17.26.w. It is seen that foreshortening reduced the number of elements while the performances are roughly the same.

Figure 17.26. Diplexer formed by impedance scaled circuits.

Figure 17.26. Diplexer formed by impedance scaled circuits.

Figure 17.26. Diplexer formed by impedance scaled circuits.

Figure 17.26. Diplexer formed by impedance scaled circuits.

###### 17. 10. ELLIPTIC  BP-BP  DIPLEXER / MULTIPLEXER DESIGN

- Diplexer and multiplexers formed by elliptic channel filters can be designed in the same way as in the Chebyshev cases studied in the previous section. That is, the component filters can either be designed from Elliptic LP Prototpyes or they can be synthesized directly as singly terminated elliptic BP filters or they can be obtained by foreshortening doubly terminated elliptic filters. In this section some examples will be presented by starting from the LP Prototype approach.

###### EXAMPLE 17. 20. An elliptic BP-BP diplexer designed from LP Prototype

Let’s design an elliptic BP-BP diplexer with the following passband edge frequencies:

Channel-A: fp1a=400 MHz, fp2a=425 MHz.

Channel-B: fp1b=425 MHz, fp2b=450 MHz.

The 3 dB cross-over frequency is fc=fp2a=fp1b=425 MHz.

- We will use the elliptic LP prototype, PR15, used in the design of elliptic LP-HP diplexer of previous examples. Its design is repeated in Figure 17.27.a-b. It is a singly terminated, degree 7 filter with normalized passband edge frequency fp=0.159, Ninf=5 and a single FTZ at fz=0.25.

- Design steps of Channel-A are shown in Figures 17.27.c-d-e. In this design the passband edges are temporarily selected as fp1=400 MHz, fp2=425 MHz. The actual value of fp2 which will give 3 dB cross-over loss at fc=425 MHz is unknown. It will be found by trials. The resulting filter has a fourth order section in shunt arm. It can be left as it is, or Split command can be applied to split it into two LC resonators in shunt arm, if the element values come out to be better for realization. The filter is saved as CHAE.

- Design steps of Channel-B are shown in Figures 17.27.f-g-h. In this design the passband edges are temporarily selected as fp1=425 MHz, fp2=450 MHz. The actual value of fp1 which will give 3 dB cross-over loss at fc=425 MHz is unknown. It will be found by trials. The filter is saved as CHBE.

- In Figure 17.27.i the filter CHAE is connected to CHBE as External 1-Port to form a diplexer. Using the command Plot / Tune Multiplexer, the insertion loss response is inspected (Figure 17.27.k). The behavior of insertion loss curves of the channels in the cross-over region indicates that the channels are overlapping which means that we have to decrease fp2 of Channel-A and increase fp1 of Channel-B. After a few trials, it is found out that for Channel-A, fp2=424 MHz and for Channel-B, fp1=426 MHz gives the 3 dB cross-over loss. The resulting filters are saved as CHAE2 and CHBE2 respectively (Figure 17.27.m-n-p-q).

- After settling the cross-over region problems, we pass to improve the return loss within the channel passbands. These return loss levels can be improved by inserting susceptance annulling elements as in the previous problems. For this purpose, for  Channel-A we insert a shunt arm series LC element with L and C values selected to produce a FTZ in the lower stopband of this channel. In this stopband we already have a FTZ element which is in the interior parts of the filter. As an initial trial we assign the L and C values of that FTZ element to the susceptance annulling element (Figure 17.27.r). The actual L and C values will be found by tuning.

- Similarly, for the improvement of the return loss response of Channel-B we insert a shunt arm series LC element with L and C values selected to produce a FTZ in the upper stopband of this channel. In this stopband we already have a FTZ element which is in the interior parts of the filter. As an initial trial we assign the L and C values of that FTZ element to the susceptance annulling element (Figure 17.27.r). The actual L and C values will be found by tuning.

- Figure 17.27.s and t show the circuits before and after tuning the susceptance annulling elements. A slight tuning gave the targeted response, as shown in Figure 17.27.u.

Exercise: Scale the element values by inserting inverters as was done in the previous example and see the problems.

Exercise: Design Triplexer and quadruplexers using the same approach.

Figure 17.27. An elliptic BP-BP Diplexer designed from a LP prototype.

Figure 17.27. An elliptic BP-BP Diplexer designed from a LP prototype.

Figure 17.27. An elliptic BP-BP Diplexer designed from a LP prototype.

Figure 17.27. An elliptic BP-BP Diplexer designed from a LP prototype.

###### EXAMPLE 17. 21. An elliptic BP-BP diplexer designed by singly terminated synthesis

###### In the diplexers designed from LP prototypes a FTZ of the prototype maps to both sides of the passband of the resulting BP filters with geometric symmetry. In some applications the FTZ’s may be required on one side of the passbands or the FTZ’s may be needed as asymmetrically located. In such cases it is possible to synthesize the component filters directly as singly terminated BP filters.

Let’s design an elliptic BP-BP diplexer of the previous example using that approach:

Passband edges are as follows:

Channel-A: fp1a=400 MHz, fp2a=425 MHz.

Channel-B: fp1b=425 MHz, fp2b=450 MHz.

Cross-over loss will be 3 dB at fc=fp1b=fp2a=425 MHz.

Coupled resonator form is targeted.

- Both filters will be of degree 10 with following TZ’s:

Channel-A: Nzero=1, Ninf=7, and a FTZ at fz1=435 MHz.

The FTZ of this channel is placed at a frequency within the passband of Channel-B to increase the isolation between the channels.

Channel-B: Nzero=7, Ninf=1, and a FTZ at fz2=415 MHz.

The FTZ of this channel is placed at a frequency within the passband of Channel-A, to increase the isolation between the channels.

Just as a trial we will take PBRipple=0.01 dB for both channels.

- Figure 17.28.a-b describes the synthesis of the filter of Channel-A. The upper passband edge is taken slightly lower than fc=425 MHz to prevent channel overlapping.

- The command Pi-Tee-L / Equal Shunt Inductors is applied to form the coupled resonator form (Figure 17.28.c). Most of the coupling elements are inductive. This is because Ninf=7 while Nzero=1.

- We have a transformer at load end. Since the filter is singly terminated the displayed transformer  turns ratio is not correct because its calculation assumes double termination. Therefore we should treat that transformer as unknown. It will be found by trials after forming the diplexer. For that purpose we move the transformer to source end (Figure 17.28.d).  This circuit is saved as A21.

- Next the filter of Channel-B is synthesized in the same way (Figure 17.28.e-f-g-h). The resulting circuit is saved as B21. It is seen that most of the coupling elements are capacitive. This is due to Nzero=7 and Ninf=1.

- Next, turns ratios of the two transformer will be found. This is done by loading B21 and inserting A21 as External 1-Port in Series connected form and then tuning the transformer turns ratio using Plot/Tune Multiplexer command. Series connection is necessary because the first elements of the filters are shunt. Transformer is tuned until insertion loss and return loss responses of that channel get the expected levels (Figure 17.28.i-k-m). The resulting filter circuit is saved as B22.

- Next the same procedure is repeated by loading A21 and then inserting B22 as External 1-Port in Series form and tuning the transformer (Figure 17.28.n-p-q-r). Tuning of the transformers may be iterated until both channels get the best responses. Thus, synthesis of the channel filters is finished.

- In the next stage we will adjust the element values. For example we can set all shunt inductors to 10 nH by inserting inverters at source and load ends as shown in Figures 17.28.s-to-z2. The resulting filters are saved as A23 and B23.

- Finally, filters A23 and B23 are combined to form diplexer. Elements of both filters are tuned to improve the return loss responses. The resulting optimized circuits are saved as A24 and B24 (Figure 7.28.z5-z6).

- Optimized responses are shown in Figure 17.28.z7.

Figure 17.28. Elliptic diplexer by synthesizing singly terminated BP filters.

Figure 17.28. Elliptic diplexer by synthesizing singly terminated BP filters.

Figure 17.28. Elliptic diplexer by synthesizing singly terminated BP filters.

Figure 17.28. Elliptic diplexer by synthesizing singly terminated BP filters.

Figure 17.28. Elliptic diplexer by synthesizing singly terminated BP filters.

###### EXAMPLE 17. 22. An elliptic BP-BP diplexer designed by doubly terminated synthesis

###### It is also possible to design elliptic BP-BP diplexers and multiplexers by synthesizing the channels as doubly terminated filters. If the element values are realizable the we can apply foreshortening method. Otherwise, that is when element value scaling is done then we have to use brute force optimization to get the targeted response.

Let’s design the same elliptic BP-BP diplexer of the previous example using these approaches:

Passband edges are:

Channel-A: fp1a=400 MHz, fp2a=425 MHz.

Channel-B: fp1b=425 MHz, fp2b=450 MHz.

Cross-over loss will be 3 dB at fc=fp1b=fp2a=425 MHz.

Coupled resonator form is targeted.

- Both filters will be of degree 10 with following TZ’s:

Channel-A: Nzero=1, Ninf=7, and a FTZ at fz1=435 MHz.

Channel-B: Nzero=7, Ninf=1, and a FTZ at fz2=415 MHz.

PBRipple=0.01 dB for both channels.

- Figure 17.29.a describes the synthesis of Channel-A filter.

- Resulting filter is converted into coupled resonator form by applying the command Pi-Tee-L / Equal Shunt Inductors (Figure 17.29.b-c). It is saved as A31.

Foreshortening Approach:

- The first resonator at source end is deleted and resulting filter is saved as A32 (Figure 17.29.d).

- The same method is applied to synthesize the filter of Channel-B. Its coupled resonator form is saved as B31 (Figure 17.29.e-f-g-h). It is foreshortened and saved as B32.

- Then the filter  A32 is inserted into B32 to form a diplexer (Figure 17.29.i).

- Figure 17.29.k and m show the insertion loss and return loss plots. These responses can further be improved by tuning elements.

- This approach is good when the element values are realizable. It should be noted that in the initial design we have to select the degree of the filter two higher than the targeted degree because one resonator is eliminated in foreshortening.

Impedance Scaling and Tuning Approach:

- In most cases element values need be scaled. For these cases foreshortening doesn’t provide a solution. When two doubly terminated filters are connected to form diplexer, response degrades heavily and we have to use brute force optimization to get the targeted response.

- In Figure 17.29.n, the circuit of A31 is loaded.

- It is scaled by inserting inverters to make all shunt inductors 10 nH (Figure 17.29.p).

- Then the source end inverter is replace by its Inductive-L equivalent. The negative shunt leg of the L-section is absorbed into the adjacent positive inductor by applying Simplify Circuit command. Resulting circuit is saved as A34 (Figure 17.29.q).

- The same operation is also applied on the filter B31. Its source end inverter is replaced by Capacitive-L equivalent and it is saved as B34 (Figure 17.29.r-s-t).

- Then A34 is inserted into B34 to form diplexer. Elements of B34 are tuned slightly to improve return loss of that channel (Figure 17.29.u).

- Next, B34 is inserted into A34 to form diplexer and elements of A34 are tuned to improve response of that channel (Figure 17.29.v-w).

- These two stages are repeated alternately to improve responses slightly in each stage until the targeted return loss response is reached. Figure 17.29.x shows the responses after a few tuning stages while Figure 17.29.y shows the resulting circuits.

Figure 17.29. Diplexer design by using doubly terminated filters.

Figure 17.29. Diplexer design by using doubly terminated filters.

Figure 17.29. Diplexer design by using doubly terminated filters.

Figure 17.29. Diplexer design by using doubly terminated filters.

LP-BP

and

BP-HP

DIPLEXERS

###### 17. 11.  LP-BP DIPLEXERS

- It is possible to connect a LP and a BP filter to form a LP-BP diplexer. The LP filter will degrade the response of the BP filter heavily.  This degradation can be corrected to some extent by using compensating elements as described in the following example.

###### EXAMPLE 17. 23.  A Chebyshev LP-BP  Diplexer with susceptance annuling elements

Let’s design a LP-BP diplexer using the same prototype PR5 as in the previous Chebyshev BP-BP diplexer examples, with following channel edge frequencies:

LP Channel:    fp=fc=300 MHz.

BP Channel: fp1=fc=300 MHz,  fp2=400 MHz  (Channel-A of the previous examples).

- Figure 17.30.a shows the command set for the LP Channel filter. This filter is saved as  LP5A.

- The BP channel filter was already designed and saved as BP5A in Examples 17.13-14. In Figure 17.30.a-b one version of the diplexer is formed by inserting BP5A into LP5A as External 1-Port  and saved as DXBPLP. Figure 17.30.c shows the responses of diplexer. As expected the passband return loss of the BP channel is degraded heavily. Return loss response can be recovered to some extent by using compensating element as was done in the previous example.

- The second version of the diplexer is formed by inserting LP5A into BP5A as External 1-Port as shown in Figure 17.30.d-e and saved as DXLPBP. Then a shunt arm series LC element is inserted at the input of this diplexer as compensating element. The inductance and capacitance of the first series LC resonator of the BP5A are used as initial trial values for the compensating LC element. This element introduces a FTZ into the diplexer circuit. The frequency of FTZ can be moved out of the passband of the BP5A by tuning the inductance or capacitance or both of the FTZ element. The tuned version of the diplexer is shown in Figure 17.30.f. Its responses are shown in Figure 17.30.g. It is seen that some more tuning seems to necessary to improve return loss.

- Elements of both DXLPBP and DXBPLP are tuned by loading them alternately. Only a slight improvement is obtained as shown in Figures 17.30.h-i-k.

Exercise: Since the susceptance annulling element is in parallel with both channels, the FTZ produced by that element must be located above the passband of the BP channel. Check the compensating effect of susceptance annulling element for very wideband BP filter.

Exercise: In the above LP-BP diplexer, use higher order LP filters to inspect if further improvement on return loss is obtainable.

Exercise: Insert the LP filter LP5A into the quadruplexer of the previous examples and try to improve the overall return loss.

Exercise: Design LP-BP diplexers with very narrow and very wide BP channel bandwidths.

Exercise: Design LP-BP-BP-BP type multiplexers using the same approach.

Figure 17.30. Design of a Chebyshev LP-BP diplexer.

Figure 17.30. Design of a Chebyshev LP-BP diplexer.

Figure 17.30. Design of a Chebyshev LP-BP diplexer.

###### EXAMPLE 17. 24.  An elliptic LP-BP  Diplexer with susceptance annuling elements

Now let’s design an elliptic LP-BP diplexer using the elliptic LP Prototype, PR15 that was used in the previous elliptic BP-BP diplexer example. The channel edge frequencies are:

LP Channel:    fp=fc=300 MHz.

BP Channel: fp1=fc=300 MHz,  fp2=400 MHz.

###### Design of the elliptic LP prototype (PR15) is repeated in Figure 17.31.a-b, for convenience.

###### Design steps of the LP Channel are shown in Figure 17.31.c-d. The proper passband edge frequency for avoiding overlapping of the channels is set as fp=280 MHz after designing the diplexer several times with different passband edge frequencies for both channels. The resulting filter is saved as LPE15.

###### The BP Channel is also designed from the same LP prototype. It is saved as BPE15 (Figure 15.31.e). The proper lower passband edge frequency for avoiding overlapping of the channels is set as fp1=320 MHz after several design trials. The resulting filter is saved as BPE15.

###### LPE15 is inserted into BPE15 as External 1-Port to form a diplexer circuit (Figure 17.31.f).

###### The insertion and return loss responses shown in Figure 17.31.g-h displays the heavy loading of the LPF on the bandpass filter.

###### In Figure 17.31.i-k-m, insertion of a susceptance annulling section is described. First the fourth order section is split into two second order sections which create the two FTZ’s on the two sides of the passband. Then the FTZ section which creates the upper stopband FTZ is determined (using Tune Element command). It is copied and Paste into the source end of the diplexer as susceptance annulling element (Figure 17.31.m).

###### Then all elements are tuned to improve the return loss. The circuit shown in Figure 17.31.n and responses shown in Figures 17.31.p and q are obtained after a few rounds of tuning.

Exercise: Design LP-BP-BP-BP type multiplexers using the same approach.

Figure 17.31. Design of an elliptic LP-BP diplexer.

Figure 17.31. Design of an elliptic LP-BP diplexer.

Figure 17.31. Design of an elliptic LP-BP diplexer.

###### 17. 12. BP-HP DIPLEXERS AND MULTIPLEXERS

###### It is possible to connect a BP and a HP filter to form a BP-HP diplexer. The HP filter will degrade the response of the BP filter heavily.  This degradation can be corrected to some extent by using compensating elements as described in the following example.

###### EXAMPLE 17. 25.  A Chebyshev BP-HP  Diplexer with compensating elements

Let’s design a BP-HP diplexer using the same prototype PR5 as in the previous examples, with following channel edge frequencies:

BP Channel: fp1=711 MHz,  fp2=fc=947.9 MHz  (Channel-D of the previous examples).

HP Channel:    fp=fc=947.9 MHz.

The cross-over will be at 300 MHz.

- Figure 17.32.a shows the command set for the HP Channel filter. This filter is saved as  HP5D.

- The BP channel filter was already designed and saved as BP5D in the previous examples. In Figure 17.32.a-b one version of the diplexer is formed by inserting BP5D into HP5D as External 1-Port  and saved as DXBPHP. Figure 17.32.c shows the responses of diplexer. As expected the passband return loss of the BP channel is degraded heavily. Return loss response can be recovered to some extent by using compensating element as was done in the previous example.

- The second version of the diplexer is formed by inserting HP5D into BP5D as External 1-Port as shown in Figure 17.32.d-e and saved as DXHPBP. Then a shunt arm series LC element is inserted at the input of this diplexer as compensating element. The inductance and capacitance of the first series LC resonator of the BP5D are used as initial trial values for the compensating LC element. This element introduces a FTZ into the diplexer circuit. The frequency of FTZ can be moved out of the passband of the BP5D by tuning the inductance or capacitance or both of the FTZ element. The tuned version of the diplexer is shown in Figure 173.2.f. Its responses are shown in Figure 17.21.g. It is seen that some more tuning seems to necessary to improve return loss.

- Elements of both DXHPBP and DXBPHP are tuned by loading them alternately. Only a slight improvement is obtained as shown in Figures 17.32.h-i-k.

- Since the FTZ must be located below the passband of the BP channel, if very wideband BP filters are used then the compensation effect of the FTZ may not be sufficient (Exercise).

Exercise: In the above BP-HP diplexer, use higher order LP filters to inspect if further improvement on return loss is obtainable.

Exercise: Insert the HP filter HP5D into the quadruplexer of the previous examples and try to improve the overall return loss. The location of FTZ will be problematic.

Exercise: Design BP-HP diplexers with very narrow and very wide BP channel bandwidths.

Exercise: Design elliptic BP-HP diplexers using the LP prototype, PR15 as was done in the elliptic LP-BP diplexer of the previous example.

Exercise: Design BP-BP-BP-HP type multiplexers using the same approach.

Exercise: Design LP-BP-BP-BP-HP type multiplexers using the same approach.

Figure 17.32. Design of a Chebyshev BP-HP diplexer.

Figure 17.32. Design of a Chebyshev BP-HP diplexer.

Figure 17.32. Design of a Chebyshev BP-HP diplexer.

##### LUMPED ELEMENT

##### BP-BS

DIPLEXERS

17.  13.  BP-BS DIPLEXER DESIGN USING LP PROTOTYPES

Bandpass-Bandstop diplexers can be designed by using singly terminated LP prototypes. If the prototype is a maximally flat type then the resulting filters will be complementary while if the LP prototypes are of Chebyshev or elliptic type then the resulting channel filters will be quasi-complementary. The LP prototypes can always be formed by designing through singly terminated synthesis approach. However for maximally flat or Chebyshev cases it is easier and more convenient to use the LP branch of a Normalized LP-HP diplexer designed through Design / LP-HP Diplexer approach. This approach avoids iterative trials when the cross-over loss is greater than 3 dB.

The direct synthesis of BP channel filter through either singly terminated or doubly terminated approaches are possible. However BS filter can only be designed through mapping from a LP prototype. Therefore BP-BS diplexers are usually designed using mapping from LP prototypes only.

# EXAMPLE 17. 26: Normalized Prototype Chebyshev LP-HP diplexers

Let’s design a degree N=5 prototype LP-HP diplexer normalized to RS=RL=1 ohms and cross-over frequency fc=0.159. The cross-over loss is taken as 3.01 dB. PBRipple is taken as 1 dB. The resulting LP channel filter is saved as PR30. Also, as an example for high cross-over loss, design is repeated for N=5, PBRipple=0.01 dB and Cross-over loss=15 dB and resulting filter is saved as PR31. The results are shown in Figure 17.33.

# EXAMPLE 17. 27: Design of a Chebyshev BP-BS diplexer using a singly terminated LP prototype

Let’s first use the prototype PR30 to design a BP-BS diplexer with passband edges fp1=370 MHz and fp2=430 MHz. Design steps for both BP and BS channels are shown in Figure 17.34. The method Design / By User Defined Prototype is used for both filters. The BP filter comes out in the usual series LC in series arm and parallel LC in shunt arm. It is saved as BP30 (Figure 17.34.a). The BS filter comes out in the classical parallel LC in series arm and series LC in shunt arm, with all LC sections resonating at the passband center f0=400 MHz. It is saved as BS30 (Figure 17.34.b). Diplexer is formed by first loading BP30 and then inserting BS30 at its input as described in Figures 17.34.c-d, using External 1-Port command. Figure 17.34.e shows the responses. Perfect equiripple return loss at the level of 18 dB and 3 dB loss at the cross-over frequencies frequencies 370 MHz and 430 MHz.

It should be noted that any modifications on these filters, like impedance scaling by inverters or changes in topology (like series to shunt conversions, etc.) will change the input reactances of the channels filters. This would degrade all the characteristics of the diplexer, especially the return loss. Therefore tuning or optimization may be necessary in such cases to recover the response to some extent.

Exercise: Repeat the design using LP prototypes with PBRipple=0.1 dB and 0.01 dB.

Figure 17.33. Design of Chebyshev LP Prototypes of degree 5.

Figure 17.34. Design of a Chebyshev BP-BS diplexer.

Figure 17.34. Design of a Chebyshev BP-BS diplexer (cont’d).

EXAMPLE 17. 28. An elliptic BP-BS diplexer designed using singly terminated elliptic LP prototype

- Let’s design an elliptic bandpass-bandstop diplexer with cross-over frequencies fc1=370 MHz and fc2=430 MHz, using the elliptic LP prototype PR15 that was used in the previous elliptic diplexers.

- The design command set for the elliptic LP prototype is repeated in Figure 17.35.a, giving the circuit of Figure 17.35.b.

- Design steps of the BP channel is shown in Figure 17.35.c. The passband edge frequencies are set as fp1=372 MHz and fp2=428 MHz, to avoid channel overlapping (found after designing the complete diplexer several times).  The resulting circuit is saved as BP35 (Figure 17.35.d).

- Then the bandstop filter is designed with band edge frequencies fp1=368 MHz and fp2=432 MHz using the same prototype and saved as BS35 (Figure 17.35.e).

- The diplexer is formed by loading BP35  (BS35) and then inserting BS35 (BP35) as External 1-Port (Figure 17.35.f).

- Figures 17.35.g and h show the insertion loss and return loss responses respectively.  The peaks in return loss response imply that the band edge frequencies need some more trimming (Exercise?).

Exercise: Use Split command on the fourth order sections in both BP7A and BS7A to convert each one into two second  order sections and inspect the element values.

- Figure 17.35. Design of an elliptic BP-BS diplexer.

- Figure 17.35. Design of an elliptic BP-BS diplexer (cont’d).