##### DISTRIBUTED  ELEMENT

##### DIPLEXERS

##### and

##### MULTIPLEXERS

17. 14. ON DISTRIBUTED ELEMENT DIPLEXERS AND MULTIPLEXERS

Commensurate length distributed element multiplexers can be designed in similar ways as their lumped element counterparts with inductors replaced by SC stubs and capacitors replaced by OC stubs. However numerous realization difficulties limit the types of distributed element multiplexers, like realization of series SC stubs, severe limitation on realizable characteristic impedance levels of stubs and transmission lines, realization difficulties due to proximity of stubs, etc. In many structures unit elements (transmission line pieces with length commensurate with stubs) must exist in the circuit

- to provide physical separation between stubs,

- to convert unrealizable series SC stubs into shunt OC stubs

- to form equivalent circuits for conversion into coupled lines for easier realization.

- to adjust  impedance levels of some elements.

Some or all of these reasons may come into picture depending on the type of filter, bandwidth, production technology, etc. The use of Unit Elements have also limitations:

- Insertion of redundant unit elements from source side is usually not allowed because the complementary nature of channel filters would be destroyed.

- Complementary filters with contributing unit elements may not be possible to design.

Distributed element LP, HP and BP filters are realizable only in certain topologies. For these reasons only limited types of distributed element multiplexers can be realized.

Regarding the use of Filpro for the design, we can state the following remarks:

- Complementary or pseudo-complementary all-stub singly terminated diplexers and multiplexers can be designed in Filpro using either Design / LP-HP Diplexer technique in exactly the same ways as their lumped element counterparts. However practical realization difficulties makes that approach almost useless for most of the applications. Various circuit transformations and insertion of redundant unit elements may be necessary depending on the type of filters and construction technology. Even after such transformations one may not get a realizable solution. Depending on the targeted topology, one may get a realizable solution only by using brute force optimization.

- Complementary and pseudo complementary singly terminated LP-HP diplexers involving nonredundant unit elements had been covered in the works of Wenzel [5], [6]. These types of diplexers can also be handled in Filpro by using Design By Synthesis technique and selecting Singly Terminated option..

- Paralleled BP-BP diplexers and multiplexers employing interdigital, combline and other miniature stripline filters can be formed by designing the channel filters independently in Doubly Terminated form by using Design/ By Synthesis technique and then using optimization after paralleling them.

- Multiplexers can also be formed as cascades of LP-HP diplexers followed by optimization where the diplexers are design in Doubly Terminated form.

- In the following sections some typical examples will be presented.

17. 15. PARALLEL COUPLED LINES AS MULTIPLEXING THREE PORTS

- Parallel coupled lines are essential elements of distributed element highpass and bandpass filters, as described in CH-09,10, 11 and 12. Parallel coupled line pairs (PCL) are four port circuits. They can be converted into three port, two port or one port elements by terminating their unused ports in OC, SC, open or short circuited stubs, etc. as described in literature, for example by Sato and Crystal [7] and Malherbe [18]. In most of the distributed element diplexers and multiplexers parallel coupled lines are used as three port networks which can be used for splitting the input signal into two paths (diplexing PCL). Similarly three coupled lines with no coupling beyond the adjacent line can also be used for splitting input signal  into two paths.

- In this section a summary of three port parallel coupled lines will be presented with emphasis on two and three parallel coupled lines. Since derivation of the equivalent circuits of PCL sections are usually confusing and complex for many engineers with different backgrounds, detailed explanations are given below using several different approaches.

17. 15. 1. CIRCUIT MODELS OF PARALLEL COUPLED LINES

- In Figure 17.38.a a parallel coupled transmission line pair is shown in a homogeneous nonmagnetic medium in between two ground planes. For pure TEM mode operation the propagation constant is

β=ωv=ωμε=ωεrc

Such a coupled line pair can be described in several different ways as follows:

a) Three Line Model of a two-wire line above ground:

A coupled line pair can be characterized through their per unit length line-to-ground and line-to-line inductance and capacitance parameters L11, L12, L22 and C10, C20, C12, as shown in Figure 17.38.b. The pair L11-C10 can be assumed to describe a two-wire TEM mode transmission line between Line-1 and ground. Similarly, the pair L22-C20 can be assumed to describe a two-wire TEM mode line between Line-2 and ground. By the same reasoning the pair L12-C12 can be assumed to describe a two-wire TEM mode transmission line in between Line-1 and Line-2. So, we can model the coupled line pair as consisting of three two-wire TEM mode transmission lines as sown in Figure 17.38.b. At first sight the terminology of representing a two-line system with three lines may appear to be confusing. However, the two PCL system is actually a three wire system with the ground being the third wire.

Figure 17.38. Three Line model of a parallel coupled line pair.

For pure TEM mode operation the per unit length parameters are related to the propagation constant as follows:

β=ωL11C10=ωL22C20=ωL12C12=ωv=ωμε=ωεrc

Thus, L’s and C’s of a line are related to each other as follows:

L11=εrc2C10

L22εrc2C20

L12=εrc2C12

Thus, the three lines, hence the coupled line pair can be characterized through the per unit length capacitances C10, C20 and C12 only, as shown in Figure 17.38.c.

Characteristic impedances of each transmission line can be calculated through the definitions

Z10=L11C10=εrcC10

Z20=L22C20=εrcC20

Z12=L12C12=εrcC12

- In Figure 17.38.d the three uncoupled transmission lines are described now using their characteristic impedances. The ports of Line-1 are numbered as 1 and 4 while the ports of Line-2 are numbered as 2 and 3. For use in later descriptions, In Figure 17.38.e and f the network is repeated for the case of a single ground plane placed at bottom.  On the left we have Line-1 (or Line 1-4) and on the right we have Line-2 (or Line 2-3). This circuit model will be used to derive other equivalent circuits which may be easier to use on different applications.

b) Even-Odd Mode Description of a Symmetric Coupled Line Pair

If the two coupled lines are identical then we have L11=L22, C11=C22, thus simplifying the characterization through the impedances also:

Z11=Z22=L11C10=εrcC10,      Z12=L12C12=εrcC12

For such cases the excitations, voltage, current, power waves and coupled line pair can be decomposed even and odd mode halves as described in Figure 17.39. Representing the voltage waves on Line-1 and Line-2 as V1(x) and V2(x), we can always write

V1(x)=Ve(x)+Vod(x)

V2(x)=Ve(x)−Vod(x)

Under even mode excitation the two lines have identical voltages, currents and power. Thus, we can place  an OC wall on the symmetry plane between the two lines, splitting the mutual capacitor C12 into two equal capacitors, 2C12 as shown in Figure 17.39.c. Since the capacitors are open on the symmetry plane we can disregard them. The two halves form identical lines whose per unit length capacitance to ground is C10 only. It is named as even mode capacitance Cev:

Ce=C10

The characteristic impedance of the two identical lines can thus be calculated using the even mode capacitor as

Zev=εrcCev

Figure 17.39. Even-odd mode decomposition of a symmetric coupled line pair.

Under odd mode excitation the voltages on the two lines are equal in magnitude but opposite in phase. Thus, the symmetry plane has zero voltage. That is, it can be assumed as just extension of the ground plane. The mutual capacitor C12 is split by the symmetry plane into two pieces with capacitance 2C12. Since one ends of these capacitors are grounded the capacitors 2C12 become parallel with the line-to-ground capacitor C10. Thus, the total line-to-ground  per unit length capacitance is C10+2C12, and it is named as odd mode capacitance:

Cod=C10+2C12

The characteristic impedance of the odd mode half is then evaluated as

Zod=εrcCod

Thus, if the capacitances Cev and Cod can be calculated then the even and odd mode characteristic impedances describing the coupled line pair can be found. It is clear that only two parameters Zev and Zodd are sufficient to describe a symmetric PCL.

c) Norton and Pi-Tee-L Transformations on Three Line Network Model

Norton and Pi-Tee-L transformations can be applied on a three line network model of a coupled line pair circuit to form different equivalent four ports. Norton transformation is shown in Figure 17.40. Originally it was defined on lumped elements but can be generalized to cover distributed elements also. Norton transformation converts a series element (R, L, C, Z, jX, OC Stub, SC Stub, TL) into a Pi-section together with a transformer while a shunt element is converted into a Tee-section together with a transformer. The new elements are of the same type as the original element. One of the shunt elements of the Pi-section and one of the series elements of the Tee-section is negative always. So, Norton transformation can be used to find new equivalent circuits at the expense of increased redundancy where one element is replaced by four elements.

Figure 17.40. Norton transformations.

Pi-Tee-L transformations are more sophisticated versions of Norton transformations. A Pi, Tee or L section can be converted into another Pi, Tee or L section  as shown in Figure 17.41.

Figure 17.41. Pi-Tee-L transformations.

These transformations can also be generalized to parallel coupled lines as described in Figure 17.42. In Figure 17.42.a the per unit length capacitance Pi-section representing a parallel coupled line is shown. It can be converted into another capacitive Pi, Tee or L section using Pi-Tee-L transformations. In the figure a Pi-to-Pi transformation is shown. Since in the new Pi-section we have different capacitances the corresponding transmission lines will have different characteristic impedances as shown in Figures 17.42.b and c. In this transformation the interpretation of the transformer may need care. The transformer on the right of C’20 is actually running along the Line-2 (Line 2-2’), including the ports 2 and 2’. However, since we will use these circuits through their ports only, this transformer need be placed only at the two ends of the Line-2, at ports 2 and 2’ only, as shown in Figure 17.42.c. Note that in this figure the transmission line of impedance Z12 formed between the two wires is drawn wider than the transmission lines formed between wires and ground, just to ease visualization to differentiate the wire-to-wire and wire-to-ground types of transmission lines, from top view.

This transformation can be used to find different proper alternative equivalent parallel coupled line sections. The most common application is conversion of an asymmetric parallel coupled line (C20C10) into a symmetric one by imposing equal shunt capacitor condition (C’20=C’10, identical coupled lines). This transformation can be generalized to make all lines of an N-parallel coupled line system identical, as described in Figure 17.43 on a 5 coupled line system. By repeated application of Pi-to-Pi transformations it is possible to equate all the shunt capacitors (Figure 17.43.a). Figure 17.43.b shows pictorial descriptions of the initial and final 5 coupled line system. The resulting transformers may be collected at one end of the structure.

Figure 17.42. Pi-to-Pi transformation applied on a parallel coupled line.

Figure 17.43. Equating shunt impedances of an N-coupled line system for N=5.

- d) Two Line model of a PCL pair:

- Another important transformation is Pi-to-LLeft and Pi-to-LRight transformations applied on two parallel coupled lines. These transformations result in a two-line representation for a parallel coupled line system which is easier to handle for the formation of one, two, three and four port elements by placing various port conditions [18]. Figure 17.44.a describes this transformation for conversion of the Pi-section into an L-Right section. The figures in the top row shows the cross-sectional description while the bottom ones show the top view. In the LRight model the transformers appear on the right side, hence at ports 2 and 3 only. Figure 17.44.b describes Pi-to-LLeft transformation where the transformers appear on left side and hence at ports 1 and 4 only. In Figure 17.44.c the LRight and LLeft models are redrawn in an easy to use manner for imposition of the port conditions that will be described later. Note that in this circuit the four port is described by a transformer and two transmission lines. Impedances of the lines are conventionally referred to as Z1=Z’10 and Z2=Z’12 in the LRight model. In LLeft model we use the convention Z1=Z’20 and Z2=Z’12. The two models are easier to use for different port conditions.

- Equations relating the Pi-model and L-model of a parallel coupled line pair have special practical importance as will be seen later. Therefore let’s derive them.

- The ABCD  parameters of the Pi-section shown in Figure 17.44.b can be calculated as follows:

- ABCD=10Y1011Z120110Y201=1+Y20Z12Z12Y10+Y20(1+Y10Z12)1+Y10Z12	(17.15.1)

- The ABCD parameters of the new Pi-section formed after Pi-to-Pi transformation are

- A'B'C'D'=10Y'1011Z'120110Y'201k001k=k(1+Y'20Z'12)Z'12kk(Y'10+Y'20(1+Y'10Z'12))(1+Y'10Z'12)k										(17.17.2)

- Given Z10=1/Y10, Z12 and Z20=1/Y20, we can calculate the parameters Z’10, Z’12, Z’20 and k of the new Pi-section by equating the corresponding A, B, C and D parameters. Since we get three independent equations to solve for the four unknowns, we may set any one of the unknowns freely and then determine the other three. For example setting Z’20=0 we can form an L-section (LRight section) shown in Figure 17.44.b. The parameters Z’10, Z’12 and k of the LRight section can be solved as follows:

- Equating A parameters k is solved as

- k=1+Z12Z20								(17.15.3)

- Equating D parameters we get

- Z'12Z'10=k(1+Z12Z10)−1						(17.15.4)

- Inserting k from Eq. 17.15.3 into 17.15.4 we get

- Z'12Z'10=Z12(1Z10+1Z20+Z12Z10Z20)						(17.15.5)

- Equating B parameters, Z’12 is solved:

- Z'12=Z12(1+Z12Z20)							(17.15.6)

- Using Eq. 17.15.6 in 17.15.5  Z’10 can be solved:

- 1Z'10=1Z10+1Z20+1Z12+Z20							(17.15.7)

- Thus, given Z10, Z20 and Z12, the parameters Z’10, Z’12 and k can be solved.

- Figure 17.44. Pi-to-LRight and Pi-to-LLeft  transformations applied on a parallel coupled line pair.

- For a symmetric Pi-section, imposing the condition Z20=Z10, the equations are simplified as follows:

- Eq. 17.15.7 becomes

- Z'10=Z10Z12+Z10Z12+2Z10								(17.15.8)

- Eq. 17.15.6 becomes

- Z'12=Z12(1+Z12Z10)							(17.17.9)

- Eq. 17.15.3 gives the transformer turns ratio k as

- k=1+Z12Z10								(17.15.10)

- However this expression can be written in terms of Z’10 and Z’12 as follows:

- Equating D parameters we get

- 1+Z'12Z'10=k(1+Z12Z10)						(17.15.11)

- Using Eq.17.15.10 in 17.15.11 we get

- 1+Z'12Z'10=k2								(17.15.12)

- giving

- k=1+Z'12Z'10									(17.15.13)

- As will be seen in the following sections the L-section models for a parallel coupled line is useful for imposing the port conditions. Since it is frequently used we redefine the line impedances of L-sections as follows:

- Z'10=Z1	Z'12=Z2						(17.15.14)

- Thus, k can also be written in terms of Z1 and Z2 as

- k=1+Z2Z1									(17.15.15)

- Thus, a symmetric parallel coupled line can be represented by any pair of  Z1, Z2 or  Z1, k or  Z2, k. That is only two parameters need be specified for a complete description, with the third one being set inherently through the above interrelations.

- On the other hand realization of parallel coupled lines usually necessitates the knowledge of even and odd mode impedances. Therefore it is good to have the relations between the parameters Z1, Z2 and k of the   L-section model and Even-odd mode impedances:

- By definition, we have seen that under even mode excitation the two lines are separated by an OC symmetry plane, hence leading to

- Zev=Z10								(17.15.16)

- Under odd mode excitation the symmetry plane is a SC plane at ground potential. Therefore the series impedance Z12 is split into two equal pieces of impedance Z12/2 and the total line-to-ground impedance  from any line becomes parallel combination of Z10 and Z12/2, giving the odd mode impedance as

- Zod=Z10Z12Z12+2Z10							(17.15.17)

- Using Eq. 17.15.8 in 17.15.16 and 17.15.17, it can be shown that

- Zev=Z1(1+1k)	and	Zod=Z1(1−1k)			(17.15.18)

- Thus, knowing Z1 and k of the L-section model, we can find the even and odd mode impedances of symmetric parallel coupled lines easily. The reversed forms of the above equations are

- Z1=Zev+Zod2 ,   k=Zev+ZodZev−Zod	and  Z2=Z1(k2−1)	      	(17.15.19)

- 17. 15. 2. SERIES AND PARALLEL DIPLEXING USING TWO PARALLEL COUPLED LINE SECTIONS

- As examples for the applications of two PCL sections let’s see the formation of some three port elements that may be used in diplexing and multiplexing applications.

a) Type-1 series diplexing with two parallel coupled lines

Figure 17.45.a shows a two parallel coupled line section (PCL) with port-2 short circuited to ground. This PCL can be used for series diplexing. Development of its circuit model is described in the figure. The equivalent L-Left circuit model is shown in Figure 17.45.b. The SC at port-2 decouples the two lines giving the circuit of Figure 17.45.c. This SC also converts the line 2-2’ into a SC stub in parallel with the transformer of port-2’ as shown in Figure 17.45.d which can be redrawn as in Figure 17.45.e to display the series nature of the ports 1’ and 2’ in a clearer manner. The ground in between the ports 1’ and 2’ can be moved the left of the SC stub by introducing a phase changing 1:-1 transformer as in Figure 17.45.f. The transformer 1:k at port-1 can be moved to the right ports, canceling the transformer k:1 at port-1’ while introducing transformer 1:-k at port-2’ and the circuit can be redrawn as shown in Figure 17.45.g. With input applied from port-1, the outputs will be taken from port-1’ and port-2’. The shunt SC stub at port-2’ enforces that a HP filter or BP filter must be placed in that port with the available SC stub and the transformer being part of that filter. There is no restriction on the type of the filter that will be placed at port-1’. In LP-HP type diplexers and multiplexers this port is used for the LP channels. This structure is named as Type-1 series diplexing PCL. The transmission line at input is common to both channels. The interrelations between the even-odd mode impedances and k-Z1-Z2 parameters are also shown in the figure for convenience.  Figure 17.45.h shows the case when also Port-1’ is SC. In this case the circuit reduces to the classical interdigital type SC  PCL that is used in HP filters.

Miscellaneous series multiplexing circuits:

Just as extension of the type-1 series diplexing PCL section, the circuits shown in Figure 17.46 can be used for series multiplexing purposes.

Figure 17.45. Type-1 series diplexing PCL circuit.

Figure 17.46. Various possible series multiplexing circuits.

b) Type-2 series diplexing with two parallel coupled lines:

Figure 17.47.a shows an alternative series diplexing PCL with port-2’ short circuited to ground. With input applied from port-1, the outputs will be taken from port-1’ and port-2. Figure 17.47.b shows the equivalent L-Left circuit model. The SC at port-2’ decouples the two lines giving the circuit of Figure 17.47.c. This SC converts the line 2-2’ into a SC stub and the circuit can be redrawn as in Figure 17.47.d. The transformer 1:k at port-1 can be moved to the right ports, canceling the transformer k:1 at port-1’ while introducing transformer 1:k at port-2 and the circuit can be redrawn as shown in Figure 17.47.e. Clearly port-1’ and port-2 are in series. The shunt SC stub at port-2 enforces that a HP filter or BP filter must be placed in that port with the available SC stub and the transformer being part of that filter. There is no restriction on the type of the filter that will be placed at port-1’. For LP-HP diplexers we will name port-1’ as LP port. This structure is named as Type-2 series diplexing PCL. The interrelations between the parameter sets Z1-Z2-k and Zev-Zod of the PCL are also shown in the figure for convenience. In this circuit the transmission line (Z2) appears in port-1’ only. Therefore once k-Z1-Z2 parameters of the diplexing PCL are set, then the first TL element of the filter to be connected at that port  would also be set. This forms a constraint in the design of the channel filters. Therefore this series diplexing circuit seems to be less practical to use than Type-1 series diplexing.

Figure 17.47. Type-2 series diplexing with two parallel coupled lines.

c) Type-1 parallel diplexing with two parallel coupled lines:

Figure 17.48.a shows a two parallel coupled line system with port-2 open circuited. Figure 17.48.b shows the equivalent L-Right circuit model. The OC at port-2 makes the transformer ineffective and decouples the two lines giving the circuit of Figure 17.48.c. Line 2-2’ now becomes an OC stub and the circuit can be redrawn as shown in Figure 17.48.d. With input applied from port-1, the outputs will be taken from port-1’ and port-2’. It is seen that the circuits connected to port-1’ and port-2’ will be in parallel. The series OC stub at port-2’ enforces that a HP filter or BP filter must be placed in that port with the available OC stub and transformer being part of that filter. There is no restriction on the type of the filter that will be placed at port-1’. In LP-HP type diplexers and multiplexers that port is used as LP port. This structure is named as Type-1 parallel diplexing PCL. The interrelations between the even-odd mode impedances and k-Z1-Z2 parameters are also shown in the figure for convenience.  Figure 17.48.e shows the equivalent circuit when port-1’ is also left OC. In this case the PCL section becomes an interdigital type OC PCL two port that is used in the classical distributed element HP filters.

Figure 17.48. Type-1 paralel diplexing circuit with two parallel coupled lines.

d) Type-2 parallel diplexing with two parallel coupled lines:

Figure 17.49 shows an alternative parallel diplexing circuit with two PCL’s where now port-2’ of the PCL is left OC.  Figure 17.49.b shows the equivalent L-Right circuit model. The OC at port-2’ makes the transformer ineffective and decouples the two lines giving the circuit of Figure 17.49.c. Line 2-2’ now becomes an OC stub and the circuit can be redrawn as shown in Figure 17.49.d. With input applied from port-1, the outputs will be taken from port-1’ and port-2. The circuits to be connected to these ports will be in parallel. This three port is named as type-2 parallel diplexing PCL. The series OC stub at port-2 enforces that a HP filter or BP filter must be placed in that port with the available OC stub and the transformer being part of that filter. There is no restriction on the type of the filter that will be placed at port-1’ except that its first element must be a transmission line with impedance Z1. However since Z1, Z2 and k are interrelated, in designing and modifying the filter at port-2 we should also consider the impedance of the transmission line at port-1’. This fact actually forms a constraint. Therefore it seems that this parallel diplexing is less practical to use than Type-1 parallel diplexing PCL.

Figure 17.49. Type-2 parallel diplexing circuit with two parallel coupled lines.

17. 15. 3. PARALLEL DIPLEXING USING THREE PARALLEL COUPLED SECTIONS

Three parallel coupled line sections can be used to feed BP-BP diplexers and multiplexers through a common feed line. Therefore they have special importance for the class of multiplexer named as common junction (or common resonator or common transformer) multiplexers.

In order to understand the derivation of the circuit model for three PCL sections let’s review the derivation of the circuit model of a two PCL section with alternate ends short circuited to ground. Figure 17.50. shows the details of each transformation in this derivation.

- Figure 17.50.a-b shows the three line model of two parallel coupled lines described in the previous section. The characteristic admittances of the three lines are denoted as Y10, Y12 and Y20. From now on we will name the three lines with their admittances, like Line-Y10, Line-Y12, etc.

- In Figure 17.50.c, Port-1’ and Port-2 are short circuited to ground. Port-1 and Port-2’ form the input and output respectively.

- In Figure 17.50.d the SC conditions are imposed on the three line circuit model.

- The SC at port-1’ decouples the Line-Y10 from the Line-Y12. It is seen that the right terminals of the line-Y10 are both shorted to ground as shown in Figure 17.50.e. Therefore looking from port-1, Line-Y10 appears as a shunt SC stub between node-1 and ground, as shown in Figure 17.50.f.

- Similarly, the SC at port-2 decouples Line-20 from Line-Y12 as shown in Figure 17.50.e and Line-Y20 becomes a shunt SC stub between node-2’ and ground as shown in Figure 17.50.f.

- Note that the Line-Y12 has grounds at both ports, but at different nodes: On the left port, node-2 is grounded while on the right port node-1’ is grounded. Physically this means the wave traveling in between the wires 1-1’ and 2-2’ undergoes a 180 degrees of phase change when going from left port to right port. This in turn means that a virtual ground line (zero potential line) exists running along the symmetry axis between wires 1-1’ and 2-2’. From circuit theory point of view such a phase change can be modeled by a 1:-1 transformer action. Hence we can convert this two wire line into a more usual form with only one wire grounded by placing  a 1:-1 transformer as shown in Figure 17.50.g.

- Figure 17.50.h shows the final circuit model used for such a coupled line pair. It consists of two shunt SC stubs of admittances Y10 and Y20 on the two sides of a transmission line of admittance Y12, together with a phase changing transformer. The transformer is usually ineffective in most amplitude filtering applications. Therefore it is usually not shown in the circuit models.

- Now let’s try to model a three PCL sections in the same manner.

Figure 17.50. Circuit model of a two PCL system with SC at diagonal ports.

Parallel Diplexing with a three PCL section: Interdigital type parallel diplexing

- Figures 17.51.a and b show the five line model of the three PCL section. Three of the five lines are formed by the wires 1-1’, 2-2’, 3-3’ and ground. They are characterized by their admittances Y10, Y20 and Y30. The fourth line is formed by the pair of wires 1-1 and 2-2’ with characteristic admittance Y12 while the fifth line is formed by the pair 1-1’ and 3-3’ with characteristic admittance Y13.

- Such a three line section forms a six port circuit. With Port-1 being the input, the incoming signal can be transferred to the adjacent lines with the aim of forming a diplexer. In combline/CLIF applications one ends of the lines 2-2’ and 3-3’ must be short circuited to ground. For example in Figure 17.51.c, the node-2 of the line 2-2’ and node-3 of the line 3-3’ are short circuited to ground. Our aim is to take the diplexer outputs from ports 2’ and 3’.

- In Figure 17.51.d the effects of the short circuits at Port-2 and Port-3 are reflected into the circuit model. At port-2, Line-Y12 and Line-Y20 are decoupled at their left ends (Port-2 ends). This makes the two left end terminals of Line-Y20 short circuit to ground. Thus Line-Y20 is converted into a shunt SC stub of admittance Y20 between Node-2’ and ground as shown in Figure 17.51.e. Similarly Line-Y30 becomes a shunt SC stub of admittance Y30 between Node-3’ and ground as shown in Figure 17.51.e.

- The SC at port-1’ converts the Line-1-1’ also into a shunt SC stub of admittance Y10. So, we have shunt SC stubs at all three available ports, 1, 2’ and 3’.

- We notice that in the circuit of Figure 17.51.d the two branches formed between the nodes 1-2’ and 1-3’ are effectively in parallel. So, we can use this structure for parallel connection of two channel filters.

- We also note that the grounds on the two sides of Line-Y12 and Y13 are in opposite terminals. We can convert these two lines into a more usual form (with only one wire grounded) by placing 1:-1 phase changing transformers as shown in Figure 17.51.e.

- In usual applications the phase changing transformers are neglected. Therefore we can use the simpler model shown in Figure 17.51.f.

- The shunt SC stub at input may be split into two parts and associated the two halves with the two branches as shown in Figure 17.51.g. The transmission lines and stubs in the two branches may be used as parts of the channel filters that may be connected to ports 2’ and 3’, as will be described later in this chapter, in the subject of common junction interdigital and combline filters.

- This three line PCL section is named as interdigital type common resonator parallel diplexer because the SC ends of the adjacent resonators rods are alternating.

Figure 17.51. Circuit model of a three PCL system with SC at Ports 1’, 2 and 3: Interdigital type common resonator parallel diplexing.

Parallel Diplexing with a three PCL section: Comb type parallel diplexing

- In Figures 17.52.a and b the circuit model for three PCL section is repeated.

- In Figure 17.52.c, the ports 1’, 2’ and 3’ are short circuited to ground. For these port conditions the circuit model is simplified as described in Figure 17.52.d-e-f-g. The Line-Y10, Line-Y20 and Line-Y30 turn into shunt SC stubs at nodes 1’, 2’ and 3’ respectively. Line-Y12 and  Line-Y13 become series SC stubs connecting the nodes 1-2 and 1-3 respectively. So, we have Pi-sections of SC stubs as shown in Figure 17.52.g. Line-Y10 will be common to the channel filters and the two filters will be in parallel.

- Since the Pi-sections of SC stubs are circuit models of comb type PCL sections, this structure is named as comb type common resonator parallel diplexing PCL.

Figure 17.52. Circuit model of a three PCL system with SC at Ports 1’, 2’ and 3’: Comb type common resonator parallel diplexing.

Malherbe approach for modeling an interdigital type parallel diplexing PCL section:

The circuit model developed in Figure 17.51.g for the three PCL section is sufficient for us to use in parallel diplexing. However both as exercise with different derivation approaches and also for obtaining an alternative equivalent circuit model, let’s use the approach of Malherbe [17] to get another version of the same parallel diplexing three PCL section. This version may be easier to use when the coupling of the three PCL section is to be adjusted independently of the circuits connected at ports.

- Figure 17.53.a shows the three PCL section. Its per unit length capacitance model is shown  in Figure 17.53.b. The five capacitors are now representing the five transmission lines used in the previous section.

- In Figure 17.53.c the shunt capacitor in the middle (C10) is split into two parts and each part is associated with the capacitors on left and right to form two Pi-sections of capacitors.

- The Capacitive-Pi section on left is converted into an LRight section while the Pi-section on right is converted into an LLeft section as shown in Figure 17.53.d. This way, instead of five transmission lines, we now have four transmission lines and two transformers modeling the three PCL section.

- In Figure 17.53.e a pictorial description is shown together with the equations relating the parameters describing the adjacent pairs of lines, namely, triplet Z1-Z2-k or the pair Zev-Zod which were derived previously for two PCL cases.

- The transformers in the middle can be moved outward, to the left of Line 2-2’ and to the right of Line 3-3’ as shown in Figure 17.53.f. That is, the transformers are moved to the output ports as described from the top view of the circuit in Figure 17.53.g.

- We will use this circuit model to impose the SC port conditions, as follows:

- In Figure 17.54.a, the three PCL section is shown with the ports 1’, 2 and 3 short circuited to ground.

- Figure 17.54.b shows the SC ports on the circuit model. The SC at port-2 decouples Line-1-1’ and Line-2-2’ while the SC at port-3 decouples Line-1-1’ and 3-3’ as shown in Figure 17.54.c.  This decoupling expose the SC’s on the left of Line-2-2’ and 3-3’, converting them into shunt SC stubs as shown in Figure 17.54.d.

- The transmission lines are grounded at diagonal terminals. They are converted into usual form by inserting 1:-1 phase changing transformers as shown in Figure 17.54.e.

- The phase changing transformers are usually neglected in most applications. So, we end up with  circuit model shown in Figure 17.54.f which is equivalent to the circuit of Figure 17.66.g if the shunt SC stub at the input of Figure 17.66.g is moved to the output ports by Kuroda transformations. So, this is also an example for the close link between Kuroda transformations and Pi-Tee-L transformations.

Figure 17.53. Alternative circuit model of a three PCL system involving four transmission lines.

Figure 17.54. Malherbe model of a parallel diplexing three PCL section.

Sato-Cristal approach for modeling a parallel diplexing three PCL section:

A third approach for the derivation of the circuit model of three PCL section was provided by Sato and Cristal [] using graph representation for the multiple coupled lines. This approach is also included for those familiar with the graph theory.

Figure 17.55.a shows the graph representation of a three PCL section with no coupling between nonadjacent lines. The nodes of the graph correspond to the actual nodes of the PCL section. With Line-1-1’ being the middle line, the adjacent nodes of the graph are coupled to each other by SC stubs of characteristic admittances Y12 and Y13. Nodes of the same line are coupled by transmission lines of characteristic admittances Y11, Y22 and Y33.  Diagonal nodes are coupled to each other though transmission lines with negative characteristic admittances –Y12 and –Y13. The characteristic admittances are defined in terms of the static per unit length line-to-line and line-to-ground capacitances as defined in the figure. For example admittance Y11 of line 1-1’ has three components: Y10, Y12 and Y13 which are proportional to C10 (line-to-ground cap.), C12 and C13 respectively, though the definition Yik=vCik. The definition of the static capacitance matrix of the PCL section is also given in the figure in terms of charge per unit length (Qk) and line-to-ground voltage (Vk).

- Figure 17.55.b shows the three PCL section with nodes 1’, 2 and 3 grounded.

- Figure 17.55.c shows the corresponding graph.

- The grounds at these nodes eliminate the diagonal coupling elements 2-1’ and 3-1’ as shown in Figure 17.55.d.

- The SC stubs between nodes 1-2 and 1-2 can be combined to form a single shunt SC stub from node-1 to ground with total admittance of Y12+Y13 (Figure 17.55.d).

- The lines 2-2’ and 3-3’ become shunt SC stubs at nodes 2’ and 3’ respectively (Figure 17.55.d).

- Considering the ground at node-1’, it is clear that Line-1-1’ becomes a shunt SC stub at node-1 while the SC stubs between nodes 1’-2’ and 1’-3’ become shunt SC stubs at nodes 2’ and 3’ respectively (Figure 17.55.d).

- The lines connecting node-1 to node2 and node-3 have negative admittances –Y12 and –Y13 respectively. Using the identity shown in Figure 17.55.e, the equivalent circuit takes the form shown in Figure 17.55.f, now containing –1:1 phase changing transformer.

- Using also the equations given in Figure 17.55.a, the shunt SC stub at node-1, node-2’ and node-3’ become Y10, Y20 and Y30 respectively. Neglecting the phase changing transformers we get the final circuit  shown in Figure 17.55.g which is the same as the previous approaches.

Figure 17.55. Sato-Cristal approach for modeling of a three PCL section.

WIDEBAND COMPLEMENTARY LP-HP DIPLEXER DESIGN USING SINGLY TERMINATED FILTERS

17. 16. WIDEBAND COMPLEMENTARY LP-HP DIPLEXER DESIGN USING SINGLY TERMINATED FILTERS

As in the lumped element LP-HP diplexers, singly terminated distributed element filters can be used to design complementary LP-HP diplexers. On Filpro we can either use the approach Design / LP-HP Diplexer which uses the same singly terminated LP prototype to design both channels or we can synthesize the channel filters directly as singly terminated LP and HP filters.  Design / LP-HP Diplexer approach results in all-stub filters which necessitates introduction of redundant Unit Elements (TLs) at non-diplexed terminals while in direct synthesis approach we may use contributing Unit Elements.

Let’s see examples using both approaches.

Design / LP-HP Diplexer Approach:

As in the lumped element examples of the previous sections, complementary pair Maximally Flat and Quasi-complementary Chebyshev all-stub (commensurate length) LP-HP diplexers can be designed automatically using the Design / LP-HP Diplexer command.

The resulting LP and HP filters will be in all-stub form which is impossible to realize in most cases. The transmission line pieces (Unit Elements) are needed in LP filter for separation of stubs and conversion of the series SC stubs into shunt OC stubs. Unit Elements are also needed in the HP filter for the formation of parallel coupled line pieces. Such unit elements have to be inserted in a redundant manner from the load side only, because source side insertion would destroy the complementary nature of the channel filters.

Realizability of the resulting LP and HP filters depends on the impedances of stubs, Unit Elements and parallel coupled lines which in turn set by the bandwidth, the quarter wavelength frequency fq, degree of the filters and the selected PBRipple level. It is possible that some of the elements come out to be unrealizable for most cases. In order to overcome such realizability problems different exact or approximate circuit transformations need be applied. However even such transformations may not lead to a usable result. In some cases brute force optimization may be tried. If all such trials fail, then one have to resort to other diplexer design approaches like the design of the channel filters independently of each other in the desired topology with either Singly Terminated or doubly terminated synthesis approach and then apply optimization by including the interconnecting circuit also.

Distributed element LP filters can also be treated as BS filters having stop bands around fq and its odd multiples. Similarly, distributed element HP filters are actually BP filters having periodic passbands around fq and its odd multiples. Thus, a distributed element lowpass and highpass filter pair with the same fq and same passband edge frequency fq have periodic response and can work as a BP-BS diplexer with the band around fq being rejected in one channel.

EXAMPLE 17. 29: Wideband LP-HP diplexer design from LP prototypes

Let’s consider a wideband diplexer example with specifications

PBRipple=0.1 dB, N=5, with 3 dB cross-over loss at cross-over frequency fc= 4000 MHz. The quarter wavelength frequency for both LP and HP filters is fq=6000 MHz.

The frequency bands 4000<f<8000 MHz and all harmonic bands around odd multiples of fq=6000 MHz will pass through the HP channel, with the nearest harmonic band being 16000<f<2000 MHz. The bands 0<f<4000 MHz and harmonic bands around even multiplex of 2fq will pass through the LP channel, with the nearest harmonic being the band 8000<f<16000 MHz. If the harmonics are not desired then a band limiting LP filter can be placed at the input of the diplexer.

- The design command set is shown in Figure 17.56. The design is based on synthesis of singly terminated LP prototype from which the targeted LP and HP filters are designed by mapping. The passband edge frequencies of the LP and HP filters are automatically adjusted so that the desired cross-over loss is obtained at the specified cross-over frequency.

- The resulting diplexer circuits are shown in Figure 17.56.a and b. The two diplexer circuits are saved as LPCH01.fil and HPCH01.fil while the individual LP and HP channel filters are saved as LP01.fil and HP01.fil. HP01.fil appears as an External 1-Port in LPCH01.fil while LP01.fil appears as External 1-Port in HPCH01.fil.

- Response in Figure 17.56.c shows that the structure may also be treated as BP-BS diplexer where now the LP filter is acting as BS filter and HP filter acting as BP filter. Crossover loss is exactly 3.01 dB at fc=4000 MHz, 8000 MHz, 16000 MHz and so on. Response period is 2fq=12000 MHz.

- The component LP and HP filters are impossible to realize in their present form. Let’s try to put them into realizable forms.

Modifications on the LP filter:

- In Figure 17.56.d, the lowpass channel is shown. Five redundant Unit Elements (TL) are inserted from load side to be used for conversion of series SC stubs into shunt OC stubs. Redundant unit elements must not be inserted from source side because otherwise the complementary property would be lost.

- These unit elements are moved into the circuit by repeated application of Kuroda/Left/ Full Stub command, to convert the series arm SC stubs into shunt OC stubs, as shown in Figure 17.56.d-to-k.

- The rightmost shunt OC stub can be deleted as it has too high impedance. Degradation of insertion loss response due to this approximation would be negligible. However if this degradation is undesired then we may use parallel coupled line for the realization of the rightmost TL-Shunt OC Stub pair. This is done by placing the cursor under the rightmost TL and applying the command Replace by PCL / I-L-U Resonator Lines/ ocOS-L / Zev=90 ohms, Zstub=47 ohms… . The TL-OC Stub pair will be replaced by an L-resonator with a parallel coupled line (PCL) (Figure 17.56.m). The notation ocOS-L denotes that the left side of the coupled line is OC while its right side is terminated with an Open Stub (OS). The user can set the even mode impedance of the coupled line and the impedance of the open stub.

- All the impedances of the LP filter seem to be realizable in stripline-microstrip form. A pictorial description of a possible realization is shown in Figure 17.56.n.

Modifications on the HP filter:

- Next, let’s consider the highpass channel (Figure 17.56.p). One approach towards realization of this filter using parallel coupled lines may be as follows.

- Five redundant Unit Elements (TLs) are inserted from the load side (Figure 17.56.q). With cursor under the TL adjacent to the series OC stub, the command Kuroda / Left / Full Stub is applied to move the OC stub on left to the right of the TL (Figure 17.56.q-r). The resulting transformer is moved rightmost by pressing the keys CTRL+W.

- This command set is repeated on all TL-OC or SC stub pairs until a TL is placed on the right of each OC or SC stub as shown in Figure 17.56.u. This circuit is saved as HPCH02.fil.

- Next, with cursor under each TL, the command Kuroda / Left / Equal Stub – CTRL+W  is applied  to place equal impedance stubs on each side of the TLs (Figure 17.56.v). Thus we have formed Stub+TL+Stub triplets ready for conversion into parallel coupled line forms.

- First the OC Stub+TL+OC Stub triplets are converted into open circuited interdigital parallel  coupled lines by placing the cursor under the TLs and applying command Replace By PCL / Interdigital Line/ ococINTD / Symmetric (Figure 17.56.w).

- The SC Stub+TL+SC Stub triplets may also be converted into SC parallel coupled line form by placing the cursor under the TLs and applying command Replace By PCL / Interdigital Line/ scscINTD / Symmetric. However the impedances of the stubs and TLs seem to be realizable in their present form. Therefore we leave them as they are. The transformer at load side can be eliminated by making one of the PCL’s slightly asymmetric.

- The even and odd mode impedances of the OC interdigital parallel coupled lines seem to be unrealizable. Therefore we have to repeat the design by changing some of the design parameters (e.g. PBRipple). The rightmost PCL seems to be the most problematic because the series OC stub impedances are too low leading to too low odd mode impedance. One way to overcome this problem is as follows.

- First the circuit of HPCH02.fil is loaded and the OC Stub+TL+OC Stub triplets with equal stubs are formed only on the first four TLs on left (Figure 17.56.x-y).

- Then with cursor under the fifth TL the command Kuroda / Left / Specify TRF: n=.. is applied to create a transformer to absorb the transformer at the load end. The turns ratio n is selected to be the inverse of the transformer at load end. Simplify Circuit command moves the new transformer to the load end and combines with the transformer there (Figure 17.56.z1).

- Then the OC Stub+TL+OC Stub triplets with equal stubs are converted into open circuited interdigital PCL form as in the previous approach while the unequal series OC stubs on the two sides of the rightmost TL are converted into series lumped capacitors by applying the command Replace By Lumped / Single C: f0=4000 MHz (Figure 17.56.z2).

- Figure 17.56.z3 shows a pictorial description of this realization where the channel filters are directly connected.

Figure 17.56.  Development of a Chebyshev LP-HP (BP-BS) diplexer.

Figure 17.56.  Development of a Chebyshev LP-HP (BP-BS) diplexer.

Figure 17.56.  Development of a Chebyshev LP-HP (BP-BS) diplexer.

Figure 17.56.  Development of a Chebyshev LP-HP (BP-BS) diplexer.

Wenzel’s Direct Synthesis Approach:

In the previous example we have seen that realization of LP-HP diplexers using singly terminated LP prototypes lead to all-stub LP and HP filters with identical quarter wavelength frequencies. Further it is not possible to place contributing Unit Elements. In some cases we may like to assign different quarter wavelength frequencies to the LP and HP filters and we may like to place contributing Unit Elements. For such cases we have to synthesize the LP and HP filters separately. The difficulty of that separate synthesis approach is the formation of complementary or quasi-complementary pairs. Care and repeated trials are needed to adjust the number of transmission zeros and passband edge frequencies to get the targeted cross-over frequency with the desired cross-over loss. Fortunately, Wenzel [5], [6] had shown that complement of a maximally flat singly terminated distributed element HP filter with m transmission zeros at f=0 (Nzero=m) and n contributing unit elements (NUE=n) is a singly terminated LP filter with n+m transmission zeros at f= (Ninf=n+m). Conversely, a LP filter with n contributing unit elements (NUE=n) and m transmission zeros at infinity (Ninf=m) is a highpass filter with n+m transmission zeros at f=0 (Nzero=n+m). This fact simplifies the selection of the transmission zero patterns. For example we may select a single TZ at f=0 (Nzero=m=1) and n-1 UE’s (NUE=n-1) to form an interdigital or edge coupled line type HP filter of degree n. Then the corresponding LP filter will have n transmission zeros at f=. Such LP-HP diplexers can’t be designed using Design / LP-HP Diplexer approach of Filpro. The LP and HP channels have to be designed separately as singly terminated filters and then brought together to form the diplexer. These properties hold for quasi-complementary (Chebyshev) filters also. Further, in wideband HP channels we will see that given Nzero=1 and NUE=n-1 for the HP filter, its quasi-complementary counterpart LP filter can have either Ninf=n or Ninf=k, NUE=n-k. That is, we may use contributing UE’s in both LP and HP channels.

EXAMPLE 17. 30: Wideband LP-HP (LP-BP) diplexer design by direct synthesis of channel filters

Let’s consider a wideband diplexer with the 3 dB cutoff frequency fc=4000 MHz. The LP channel will pass 0<f<4000 MHz and the HP channel will pass 4000<f<8000 MHz.

Since the HP channel has a wide band is we may encounter difficulties with impedances values. Just as exercise to see the associated problems, let’s consider two options for the HP filter:

HP Option-1: Nzero=1, NUE=8.

In this option we will try to realize the HP filter in the form of OC edge or broadside coupled line form.

HP Option-2: Nzero=3, NUE=6.

In this option some sections of the HP filter will be in the form of OC edge coupled or broadside coupled line form while the rest will be TL’s and shunt SC stubs.

According to Wenzel’s rule, the LP filter that complements both of these HP filters will have Ninf=9. This necessitates the use of redundant UE’s to put the LP filter into a realizable form. However just as a test, instead of Ninf=9 let’s set Ninf=5 and NUE=4 so that we will be using contributing UE’s to improve selectivity.

Let’s select PBRipple=0.01 dB for both HP and LP filters.

Let’s discuss the question of quarter wavelength frequencies. As in the previous example, in order to make use of the BP nature of the HPF, the quarter wavelength frequency of the HPF can be set as fqHP=6 GHz.  This HPF will then act as a BPF with passband edges 4 and 8 GHz. The nearest spurious (harmonic) passband will then be the band 16<f<20 GHz. It is possible to stop this band by placing a band limiting LPF with fp<16 GHz, for example with fp=12 GHz, at the input of the diplexer.

The frequency band 0<f<4 GHz will pass through the LP channel, with the nearest harmonic being the band about 2fqLP. If we set the quarter wavelength frequency of the LPF to be the same as the HPF, that is, fqLP=6 GHz, then the spurious passsband will be in the range 8<f<16 GHz which is too close to the passband of the LPF. One more advantage of this approach (compared to Design / LP-HP Diplexer approach) is that we can set the quarter wavelength frequencies of the LP and HP filters different. If the spurious passbands are problematic then it is possible to push them away by selecting higher fq for both LP and HP channels. However in that case the upper band edge of the HPF will be moved away also. If this is not desirable then the band limiting LPF can be used to set the upper passband edge of the HP channel.

As an example for this case, let’s select it as fqLP=10 GHz so that the spurious passband will be pushed away to 16-20 GHz range which will be stopped by the band limiting LPF at input of the diplexer. Let’s also select the quarter wavelength frequency of the HP filter as fqHP=8 GHz to push its spurious passband above 15 GHz. Then its passband will be covering the range 4-12 GHz while the targeted HP channel should pass only 4000-8000 MHz band. The 8000 MHz limit can be provided by the band limiting LPF whose passband edge will be set at fp=8000 MHz.

Let’s start the designs:

Design of the band limiting LP filter:

Design steps of the band limiting LP filter are shown in Figure 17.57. Its passband edge frequency is selected as fp=8000 MHz. The quarter wavelength frequency is selected as fq=16000 MHz. Its degree can be set as desired, to provide the necessary selectivity at this end of the passband. We arbitrarily select a degree 13 LPF, with Ninf=7 and NUE=6. It is designed as a Doubly Terminated LPF, in Type B form. PBRipple=0.01 dB is selected. The transmission zeros are extracted in an alternating Inf-UE-Inf-UE-… . The resulting filter is saved as LP0.fil.

Figure 17.57. Design of the band limiting LPF.

Design of the LP channel filter:

- Design steps of the LP channel filter are shown in Figure 17.58. Its passband edge frequency is selected as fp=3700 MHz, after trials, in coordination with the passband edge frequency of the HP channel filter so that the cross-over frequency will be about 4000 MHz. The quarter wavelength frequency is selected as fq=10000 MHz. Degree is nine, with Ninf=4 and NUE=5. It is designed as a SinglyTerminated LPF, in Type A form with PBRipple=0.01 dB. The transmission zeros are extracted in an alternating Inf-UE-Inf-UE-… . The resulting filter is saved as LPX.fil.

Figure 17.58. Design of the LP channel filter.

Design of HP filter OPTION-1 and formation of the diplexer:

- Design steps of HP filter are shown in Figure 17.59. Its passband edge frequency is selected as fp=4400 MHz, after trials, in coordination with the passband edge frequency of the LPX so that the cross-over frequency will be about 4000 MHz. The quarter wavelength frequency is selected as fq=10000 MHz. Degree is nine, with Nzero=1 and NUE=8. It is designed as a SinglyTerminated HPF, in Type A form with PBRipple=0.01 dB. The transmission zeros are extracted in the order   UE-UE-UE-UE-Zero-UE-UE-UE-UE. The resulting filter is shown in Figure 17.59.a.

- A unity transformer is inserted at the source end to be used in the formation of the complement of the LP filter LPX (Figure 17.59.b).

- The LP channel filter LPX is inserted as Shunt External 1-Port to form the diplexer (Figure 17.59.c) and the transformer of the HP filter is tuned while observing the return loss. Tuning is stopped when return loss gets its best level (Figure 17.59.d). Now the HP filter can be assumed to be quasi complement of LPX. The circuit is saved as DPX.fil. Then the External 1-Port is deleted and the remaining HP filter is saved as HPX.fil (Figure 17.59.e). It will be modified later for realizability.

- Then the band limiting LP filter, LP0 is loaded. With cursor under the load, the command Append Filter: DPX is applied to cascade LP0 and DPX to complete the formation of the diplexer.

- Figure 17.59.h shows the insertion loss and return loss responses. The nearest spurious passbands are eliminated by the band limiting LPF. Return loss is about 20 dB and it can be further improved by tuning the elements of both channels. The cross-over frequency is about fc1=4000 MHz. It can be made to coincide with 4000 MHz exactly by redesigning the channel filters with slightly different passband edge frequencies or by tuning the elements of both channels slightly.

- In the next stage we will try to put the HP filter into a realizable form.

Figure 17.59. Design of the HP channel filter and formation of the diplexer.

Figure 17.59. Design of the HP channel filter and formation of the diplexer.

Modifications on the HP Channel filter for realization:

The HP filter, HPX is unrealizable in this form. It needs some modifications. Since this is a wideband filter, element values will come out to be problematic. Trials are necessary for both finding a proper topology and realizable element values. One example is described below:

Our target is to distribute the series OC stubs in proper ratios so that both the element values will be realizable and the transformer at load end will be eliminated.

- The circuit of HPX is loaded (Figure 17.60.a). With cursor under the OC stub in the middle the command Split / Specify Left-Right Ratio: Ratio=1.5 is applied to split the stub into two parts (Figure 17.60.b). Here the ratio 1.5 is just a random trail ratio.

- Then with the cursor under the TL’s adjacent to the two pieces of the stubs, the command Kuroda / Right (Left) / Full Stub is applied to move the stubs into the other sides of the TL’s. Simplify Circuit command moves the resulting transformers to load end (Figure 17.60.c).

- Then with cursor being under the TL’s adjacent to the stubs, the command Kuroda / Distribute Stub is applied to move parts of the stubs to the other sides of the TLs. This command is repeated by placing the cursor under all TL’s which is adjacent to a stub until an OC stub is placed on both sides of all TL’s, as shown in Figure 17.60.d.

- Our aim is now to form OC Stub+TL+OC Stub triplets around the TL’s numbered as 1, 3, 6 and 8 only. That is less number of PCL’s will be formed in expectation of better behaved impedances. For this purpose, with cursor under the third and sixth TL’s the command Kuroda / Equate Left-Right is applied to equate the stubs on both sides of these TL’s. The stubs of TLs 1 and 8 were already equal. We get the circuit of Figure 17.60.e. In this circuit we note that impedances of TL’s are in realizable ranges. This is provided by the split ratio 1.5 used at the first stage. If they come out to be too high or low we would repeat the previous steps by starting with a different split ratio.

- We also note that the transformer at load end is far from unity: n=0.64. At this stage we will try to make it unity. If an OC stub is moved from right to left of a TL then this transformation produces a transformer with turns ratio greater than one. So, we make a trial by placing the cursor under TL-2 and apply command Kuroda / Right / Specify TRF: n=1.2. Here n=1.2 is just a trial value. A piece of stub is moved to the left of TL-2 producing a transformer with turns ratio n=1.2 (Figure 17.60.f).

- Simplify Circuit command combines the adjacent stubs while also moving the transformer to the load end (Figure 17.60.g). The load end transformer is increased to 0.77.

- Translation of a piece of the stub destroys equality of the stubs of both TL-1 and TL-2. In order to equate their stubs the command Kuroda / Equate Left-Right is applied. Simplify Circuit command moves the resulting transformers to load end, increasing the transformer turns ratio to n=0.89, closer to unity (Figure 17.60.h). In order to make n=1, we repeat the stages e-to-h by assigning a split ratio slightly greater than 1.2. In Figure 17.60.i the circuit resulting after a few trials is shown with unity transformer.

- Next, we convert the stub+TL+stub triplets around TLs 1, 3, 6 and 8 into PCL sections by applying the command Replace by PCL / Interdigital Line / ococINTD / Symmetric. We get the circuit shown in Figure 17.60.k. In this case the even mode impedances are well behaved but the odd mode impedances of the right half are impossible to realize.

- We may continue playing with translating stubs between the TL’s to adjust the impedances. However an easier way is to use the Half Angle transformation described in Figure 17.60.k-m. With cursor under the PCL sections, the command Half Angle is applied. This command replaces the PCl sections with two half length TLs separated by a Pi-section of OC stubs (Figure 17.60.m).

- The OC stubs are then converted into lumped capacitors by applying the command Replace by Lumped / Single C: f=4400 MHz (Figure 17.60.n). The resulting circuit is saved as HPXA.fil. As will be seen this approximation doesn’t effect the responses much.

- Next, we insert the LP channel LPX as shunt External 1-Port to form the diplexer. It is saved as  DPXA.fil (Figure 17.60.p).Then we load the band limiting LP filter LP0 and append DPXA as shown in Figure 17.60.q-r. Figure 17.60.s shows the insertion loss and return loss responses without any tuning. No change is observed due to the approximations. Some of the very small shunt capacitors may be deleted and the adjacent TL impedances and lengths can be adjusted to compensate the effects of deleted capacitors. Response can be improved by tuning of some elements of all filters. Figure 17.60.t shows a possible realization schematics in stripline-like structure.

Figure 17.60. Modification of the HP filter for realization.

Figure 17.60. Modification of the HP filter for realization.

Figure 17.60. Modification of the HP filter for realization.

Design of HP filter OPTION-2 and formation of the diplexer:

In this option we place three TZ’s at f=0 (Nzero=3) in expectation of better behaved element values for the HP filter. The total degree of the filter will be kept at N=9. So we assign NUE=6. We will use the same LP filter, LPX of the previous example.

- Design steps of HP filter are shown in Figure 17.61. Its passband edge frequency is selected as fp=4400 MHz and the quarter wavelength frequency is selected as fq=10000 MHz, the same as the previous HP filter (HPX). It is designed as a SinglyTerminated HPF, in Type A form with PBRipple=0.01 dB. The transmission zeros are extracted in the order   Zero-UE-UE-UE-Zero-UE-UE-UE. The resulting filter is shown in Figure 17.61.a.

- A unity transformer is inserted at the source end to be used in the formation of the complement of the LP filter LPX (Figure 17.61.b).

- The LP channel filter LPX is inserted as Shunt External 1-Port to form the diplexer (Figure 17.61.c) and the transformer of the HP filter is tuned while observing the return loss. Tuning is stopped when return loss gets its best level (Figure 17.61.d). Now the HP filter can be assumed to be quasi complement of LPX. The circuit is saved as DPY.fil. Then the External 1-Port is deleted and the remaining HP filter is saved as HPY.fil (Figure 17.61.e). It will be modified later for realizability.

- Then the band limiting LP filter, LP0 is loaded. With cursor under the load, the command Append Filter: DPX is applied to cascade LP0 and DPY to complete the formation of the diplexer.

- Figure 17.61.h shows the insertion loss and return loss responses. The nearest spurious passbands are eliminated by the band limiting LPF. Return loss is about 20 dB and it can be further improved by tuning the elements of both channels. The cross-over frequency is about fc1=4000 MHz.

- In the next stage we will try to put the HP filter into a realizable form.

Figure 17.61. Design of the HP channel filter OPTION-2.

Figure 17.61. Design of the HP channel filter OPTION-2.

Modifications on the HP Channel filter HPY for realization:

The HP filter, HPY is unrealizable in this form. It needs some modifications. Since this is a wideband filter, element values will come out to be problematic. Our target is to distribute the series OC stubs and shunt SC stubs in proper ratios so that both the element values will be realizable and the transformer at load end will be eliminated.

- The circuit of HPY is loaded (Figure 17.62.a). With cursor under the TL’s adjacent to series OC stubs Kuroda / Left (Right) Equal Stub is applied to translate part of the stubs to the other side of the TL’s. Simplify Circuit command moves all the transformers to the load end (Figure 17.62.b).

- Then with the cursor under the shunt SC stub in the middle the command Split / Two Equal command is applied (Figure 17.62.c). We might as well split this stub unequally.

- With cursor being under the TL’s adjacent to the two pieces of the shunt SC stubs, the command Kuroda / Right (Left) / Full Stub is applied to move the stubs into the other sides of the TL’s. Simplify Circuit command moves the resulting transformers to load end (Figure 17.62.d-e).

- In the circuit of Figure 17.62.e we note that the transformer at load end is now closer to unity. However the impedances of the TL’s and shunt SC stubs in the middle part of the circuit are still too low. In order to increase them, with cursor being under the second TL from left, the command Kuroda / Right / Equal Stub is applied to move part of the SC stub on its right to its left. The impedances in the left half of the circuit are realizable. However the transformer at load end is decreased to n=0.68 (Figure 17.62.f). In order to make the transformer unity and also to adjust the level of impedances in the right half of the circuit the cursor is placed under the second TL from right and the command Kuroda Left / Specify TRF: n=1/0.68=1.4645 is applied giving the circuit of Figure 17.62.g where the element values are reasonable and the transformer is unity.

- Next, we convert the OC stub+TL+ OC stub triplets around the first and last TL’s into PCL sections by applying the command Replace by PCL / Interdigital Line / ococINTD / Symmetric. We get the circuit shown in Figure 17.62.h. We note that the even and odd mode impedances of the resulting PCL’s are problematic for realization. In order to reach at realizable impedances we first apply Half Angle transformation to the PCL sections as described in Figure 17.62.h-i. This command replaces the PCL sections with two half length TL’s separated by a Pi-section of OC stubs (Figure 17.62.i).

- Then the OC stubs are converted into lumped capacitors by applying the command Replace by Lumped / Single C: f=4400 MHz (Figure 17.62.k). The resulting circuit is saved as HPYA.fil.

- Next, we insert the LP channel LPX as shunt External 1-Port to form the diplexer. It is saved as  DPYA.fil (Figure 17.62.m).

- Then we load the band limiting LP filter LP0 and append DPYA as shown in Figure 17.62.n-p.

- Figure 17.62.q shows the insertion loss and return loss responses without any tuning. No change is observed due to the approximations. Some of the very small shunt capacitors may be deleted and the adjacent TL impedances and lengths can be adjusted to compensate the effects of deleted capacitors. Response can be improved by tuning of some elements of all filters.

- Figure 17.62.r shows a possible implementation schematics in stripline-like structure.

Figure 17.62. Modifications on the HP filter of Option-2.

Figure 17.62. Modifications on the HP filter of Option-2.

NARROWBAND LP-HP (BP-BS) DIPLEXER DESIGN USING DIPLEXING PARALLEL COUPLED LINE SECTIONS

17. 17. NARROWBAND LP-HP (BP-BS) DIPLEXER DESIGN USING DIPLEXING PARALLEL COUPLED LINE SECTIONS

In the previous examples we have seen design examples for LP-HP diplexers with wideband channels. In these examples we were able to eliminate the transformers of the HP channels by Kuroda transformations. Now let’s see some examples on LP-HP diplexers with narrow HP channels. As will be seen in the following example the transformer have to be absorbed into a diplexing PCL at input of the diplexer.

EXAMPLE 17. 31: Design of a LP-HP diplexer using parallel diplexing PCL

Let’s design a diplexer with HP channel covering the band f1=1450 MHz and f2=1550 MHz and the other channel will pass the frequency ranges f=0 to f=1450 MHz and also from f=1550 MHz up to first spurious passband, which is about 4500 MHz. The cross-over loss will be about 3 dB at fc1=1450 MHz and fc2=1550 MHz.

Such a diplexer can be designed by selecting fq=1500 MHz for both LP and HP filters. The bandwidth of HP filter will be between f1=1450 MHz and f2=1550 MHz, centered at fq=1500 MHz. So, the HP filter will actually be acting as a narrowband BP filter. During the synthesis of the HP filter we may select fp=1450 MHz. This value may need iteration after combining the HP filter with the LP filter. For the LP filter, with fq=1500 MHz the passband edge frequency may be selected as fp=1450 MHz which may also need iterative trimming after formation of the diplexer.

- Let the degree of both HP and LP filters be N=7. Let’s target to realize the HP filter in the form of OC parallel coupled line filter. Such a HP filter of degree N=7 must have six unit elements (NUE=6) and one transmission zero at zero (Nzero=1). Then, the complementing lowpass filter will have seven transmission zeros at infinity (Ninf=7).

The PBRipple is chosen as 0.1 dB which is another iteration parameter that will also set the cross-over loss.

Existence of a large transformer in the HP filter forces us to use a diplexing PCL. We will target a parallel diplexing PCL. That is, the channel filters will appear to be in parallel.

Synthesis of the LP filter:

- Design steps of the singly terminated Chebyshev LP filter are shown in Figure 17.63. Type A is chosen to get a series arm element on the source and load ends, because the channel filters will be connected in parallel. Seven TZ’s are placed at f=. Standard Auto Extract command is used to extract the elements (Figure 17.63.a). Reverse Circuit option is used. Seven TL pieces (redundant Unit Elements) are inserted from the load side (Figure 17.63.b).

- With cursor under the TL’s adjacent to stubs the command Kuroda / Left / Full Stub command is applied to move the TL’s between stubs until all the stubs become shunt OC stubs (Figure 17.63.c).

- Since the shunt SC stub impedances have unrealizable impedances we convert the TL-Shunt OC stub pairs into L-type PCL resonators by placing the cursor under each TL and applying the command Replace By PCL / I-L-U Lines / ocOS-L / Zev=75 ohms / Zst=…. In this command the designer has the option of setting the even mode impedance Zev of the resulting parallel coupled line. Filpro then displays the resulting impedance Zs of the Open Stub (OS). Zev can be iterated until all of the three parameters Zev, Zod and Zstub are realizable. This command is applied on all TL-Shunt OC stub pairs by selecting Zev=75 ohms except the last line on the right where Zev is selected as 64 ohms by inspecting the corresponding stub impedance Zs. The resulting circuit is shown in Figure 17.63.d. The name ocOS-L means that the left termination is an open circuit (oc), right termination is an open circuited stub (Open Stub-OS) and it is L-shaped. Element values seem to be realizable. The circuit is saved as LP37.fil.

- The resulting structure may be too long for some applications. In order to save space in longitudinal direction it is possible to fold the circuit. However folding may introduce undesired cross couplings between resonators. In order to avoid such couplings we can insert extra redundant TL’s from load end. For example in Figure 17.63.e-f, two TL’s are inserted. They are then moved to the right of the third shunt OC stub as shown in Figure 17.63.g. Converting the TL-OC Stub pairs into PCL equivalents we get the circuit shown in Figure 17.63.h. Figure 17.63.i shows one possible folding for the circuit.

Figure 17.63. Design steps of singly terminated lowpass filter.

Figure 17.63. Design steps of the LP channel filter of LP-HP diplexer.

Synthesis of singly terminated HP channel filter:

- Design steps of the highpass singly terminated filter are shown in Figure 17.64. Type A is selected to have a series first element because the channel filters will be connected in parallel. Transmission zeros are set as Nzero=1 and NUE=6.

- TZ’s are extracted in the order UE-UE-UE-zero-UE-UE-UE giving the circuit of Figure 17.64.a (after reversing the circuit).

- A unity ideal transformer is inserted at the input. This transformer will be tuned to form the complement of the LP filter LP37.fil (Figure 17.64.b).

- With cursor under the transformer, the command External 1-Port / LP37 / Shunt 1-Port is applied to insert LP37 to form the LP-HP diplexer (Figure 17.64.c).

- Figure 17.64.d shows the resulting response before tuning the transformer. The transformer is tuned while observing return loss and insertion loss of the diplexer by using Plot/Tune Multiplexer command. Tuning is stopped when the best return loss is achieved (Figure 17.64.e). The cross-over losses are about 3.5 dB which means that the passband edge frequencies of the LP and HP filters (fpLP and fpHP) need be adjusted slightly. They were both selected as fc1=1450 MHz. We should repeat the design by selecting fpLP<1450 MHz and fpHP>1450 MHz by trials until the cross-over losses settle at exactly 3.01 dB. Let’s continue with the present design, leaving the fun of adjustment of fpLP and fpHP to the user  .

- Figure 17.64.f shows the circuit after tuning the transformer. Now the LP and HP channels form a complementary (or quasi-complementary) pair.

- In the next stage we will try to put the circuit into a topology that is realizable in OC edge coupled line form. Since the HP filter is a narrowband one, element values will come out to be too high for realization in OC edge coupled line form. In doubly terminated filters redundant Unit Element were inserted at both ports to adjust element values. However in the present application input impedances of the channel filters must not be changed to preserve the complementary nature. Therefore we insert a unit element at the load end only by applying the command Insert / TL: Z=50 ohms (Figure 17.64.f-g).

- Next, with cursor under the TL’s adjacent to the series OC stub in the middle, the command Kuroda / Distribute Stub is applied to move part of the OC stub to the other side of the TL’s (Figure 17.64.g-h). This command is repeated until OC stubs are placed on both sides of all TL’s (Figure 17.64.i-k).

- Then the interior OC stubs are split into two by using Split / Two Equal command so that we get triplets of OC Stub+TL+OC stub which are the prototypes for conversion into edge coupled line sections (Figure 17.64.k-m).

- We usually like to have equal OC stub impedances and also to have symmetric parallel coupled lines. Both of these targets are reached by placing the cursor under any TL and applying the command Kuroda / All Equal Stubs / Include End Stubs. This command equates all the OC stubs  (Figure 17.64.n). The circuit is saved as HP37a.fil.

- Since the circuit is asymmetric we get a transformer at load end. It is possible to absorb this transformer into the interior PCL sections but this would make them asymmetric which is usually not desirable. We can absorb part or the whole transformer into the LP-HP diplexing PCL that will be placed at the inputs, as will be seen in the next stage.  So, let’s leave the transformer as it is.

Figure 17.64. Design steps of the HP channel filter of LP-HP diplexer.

Figure 17.64. Design steps of the HP channel filter of LP-HP diplexer.

Figure 17.64. Design steps of the HP channel filter of the LP-HP diplexer.

Formation of the diplexer using Parallel Diplexing PCL section:

- At this stage we will modify the input elements of the HP filter circuit so that a parallel diplexing PCL section will be formed.

- In Figure 17.65.a, the circuit of HP37a.fil is loaded. This circuit will be modified to form a parallel diplexing PCL input in such a way that its input part will resemble that of the circuit model of a Type-1 parallel diplexing PCL which is also shown in the same figure, taken from Figure 17.48. With input at Port-1, the LP and HP channels will be connected to Port-1’ and Port-2’ of the diplexing PCL respectively, with Port-2 being OC.

- The circuit model of the diplexing PCL has a TL on left whose impedance is defined as Z1. A series OC stub exists on the right of the TL with impedance defined as Z2. Also a transformer exists on the right of the OC stub with turns ratio as k:1. The parameters Z1, Z2 and k are related through Z2=Z1(k2-1)  to be realizable as a symmetric PCL. Since realizability conditions, analysis and design of symmetric PCL’s are easier to handle in terms of even and odd mode impedances Zev and Zod, the expressions of these impedances in terms of Z1, Z2 and k are also shown in the figure.

- One procedure towards the formation of such a diplexing PCL may be as follows:

- In order to preserve input match, a transmission line of impedance Z=RS=50 ohms is inserted from source side to act as the TL of impedance Z1 of the diplexing PCL section (Figure 17.65.b).

- The series OC stub on the right of this redundant TL is split into two pieces (Figure 17.65.c).

- Then with cursor under the redundant TL, the command Kuroda / Distribute Stub is applied to move part of the OC stub on its right to its left (Figure 17.65.d).

- Then the command Kuroda / All Equal Stubs / Include End Stubs is applied to equate all the series OC stubs (Figure 17.65.e). This circuit is saved as HP37b.fil.

- Thus, we have formed triplets of Series OC Stub+TL+Series OC stub. Each triplet can be converted into equivalent interdigital type OC PCL section by placing the cursor under the TL’s and applying the command Replace by PCL / Interdigital Line / ococINTD / Symmetric, as shown in Figure 17.65.f. The even and odd mode impedances of each PCL section are shown underneath each element. In the same figure a pictorial description of the filter is also shown. Thus, we have formed a filter which can be realized as an OC edge coupled or broadside coupled HP filter. The transformer left at the load end can be absorbed into the PCL sections.

- At this stage we claim that the first PCL section on source side is also acting as the targeted parallel diplexing PCL. We can prove this in two ways as follows:

- We note down its even and odd mode impedances, Zev=62.3 ohms and Zod=37.6 ohms of the first PCL section from Figure 17.65.f.

- In Figure 17.65.g the circuit of HP37b is loaded. With cursor under the first TL on left, the command Kuroda / Left / Full Stub is applied to move the stub on its left to its right (Figure 17.65.h).

- Then the translated stub is combined with the stub on its right (Figure 17.65.i).

- Then the transformer at the source end is moved to the right of the translated OC stub by applying the command Shift TRF / n-Right / n=2 (Figure 17.65.k).

- From the figure it is seen that we have formed the circuit model of the parallel diplexing PCL at the source end of the circuit. This circuit is saved as HP37c.fil. From the circuit we note that Z1=50 ohms, Z2=771 ohms and 1/k=0.2464 (k=4.0524). These values satisfy the constraint Z2=Z1(k2-1).

- Using these values we also calculate Zev=62.335 ohms and Zod=37.665 ohms which are the same as those we have seen in Figure 17.65.f. So, the first PCL section on left is also the parallel diplexing PCL section that we wanted to form. Port-1, Port-1’ and Port-2’ of the diplexing PCL are shown in the figure.

- As a second check, we form the diplexer and inspect its response to see if it is changed or not. In Figure 17.65.m the filter LP37.fil is inserted as shunt External 1-Port forming the parallel diplexer. Figure 17.65.n shows the corresponding pictorial description of the complete diplexer. Responses are shown in Figure 17.65.p. They are the same as the original response. Thus, the fist PCL on left is also the parallel diplexing PCL.

Figure 17.65. Formation of the parallel diplexing PCL and the diplexer.

Figure 17.65. Formation of the parallel diplexing PCL and the diplexer.

Figure 17.65. Formation of the parallel diplexing PCL and the diplexer.

# EXAMPLE 17. 32: Design of  a LP-HP (BP-BS) diplexer using series diplexing PCL

When the LP and HP filters of a diplexer are synthesized in such a way as their first elements are shunt (Type B in Synthesis approach), then they should be connected in series to form a contiguous diplexer. Realization of the HP filter will then be in either interdigital filter form or in SC edge coupled line form.

Let’s try to design the diplexer of the previous example now in series form just to see the problems of series diplexing PCL’s described in Figure 17.45 and 17.46.

The HP channel covers the band f1=1450 MHz and f2=1550 MHz and the other channel will pass the frequency ranges f=0 to f=1450 MHz and also from f=1550 MHz up to first spurious passband. The cross-over loss will be about 3 dB at fc1=1450 MHz and fc2=1550 MHz.

Let’s select fq=1500 MHz for both LP and HP filters. The bandwidth of HP filter will be between f1=1450 MHz and f2=1550 MHz, centered at fq=1500 MHz. So, the HP filter will actually be acting as a narrowband BP filter. During the synthesis of the HP filter we may select fp=1450 MHz. This value may need iteration after combining the HP filter with the LP filter. For the LP filter, with fq=1500 MHz the passband edge frequency may be selected as fp=1450 MHz which may also need iterative trimming after formation of the diplexer.

Let the degree of both HP and LP filters be N=7.

The PBRipple is chosen as 0.1 dB which is another iteration parameter that will also set the cross-over loss.

Synthesis of the LP filter:

- Design steps of the singly terminated Chebyshev LP filter are shown in Figure 17.66. Type B is chosen to get a shunt arm element on the source and load ends. Seven TZ’s are placed at f=.

- Standard Auto Extract command is used to extract the elements (Figure 17.66.a). Reverse Circuit option is used, as usual.

- Seven TL pieces (redundant Unit Elements) are inserted from the load side (Figure 17.66.b).

- With cursor under the TL’s adjacent to stubs the command Kuroda / Left / Full Stub command is applied to move the TL’s into between each stub until all the stubs become shunt OC stubs (Figure 17.66.c).

- Since the shunt SC stub impedances have unrealizable impedances we convert the TL-Shunt OC stub pairs into L-type PCL resonators by placing the cursor under each TL and applying the command Replace By PCL / I-L-U Lines / Osoc-L / Zev=75 ohms / Zst=… is applied. In this command the designer has the option of setting the even mode impedance Zev of the resulting parallel coupled line. Filpro then displays the resulting impedance Zst of the Open Stub (OS). Zev can be iterated until all of the three parameters Zev, Zod and Zstub are realizable. In this example this command is applied by selecting Zev=75 ohms on all TL-Shunt OC stub pairs except the last line on the right where Zev is selected as 64 ohms by inspecting the corresponding stub impedance Zst. The resulting circuit is shown in Figure 17.66.d. The name Osoc-L means that the left termination is an open stub (OS), right termination is an open circuit (oc) and it is L-shaped. Element values seem to be realizable. The circuit is saved as LP38.fil.

- Figure 17.66.e shows a schematic description for possible realization in stripline-like form. The stubs of the coupled lines are alternated in direction to avoid spurious couplings.

Figure 17.66. Design of the LP filter for the series LP-HP diplexer.

Synthesis of the HP filter and formation of the diplexer:

- Design steps of the highpass singly terminated filter are shown in Figure 17.67. Type B is selected to have a shunt first element because the channel filters will be connected in series. Transmission zeros are set as Nzero=1 and NUE=6.

- TZ’s are extracted in the order UE-UE-UE-zero-UE-UE-UE giving the circuit of Figure 17.67.a (after reversing the circuit).

- A unity ideal transformer is inserted at the input. This transformer will be tuned to form the complement of the LP filter LP37.fil (Figure 17.67.b).

- With cursor under the transformer, the command External 1-Port / LP38 / Series 1-Port is applied to insert LP38 to form the series LP-HP diplexer (Figure 17.67.c).

- Figure 17.67.d shows the resulting response before tuning the transformer. The transformer is tuned while observing return loss and insertion loss of the diplexer by using Plot/Tune Multiplexer command. Tuning is stopped when the best return loss is achieved. Then the cross-over losses are inspected (Figure 17.67.e). They are about 3.5 dB which means that the passband edge frequencies of the LP and HP filters need be adjusted slightly. They were both selected as fc1=1450 MHz. We should repeat the design by selecting fpLP<1450 MHz and fpHP>1450 MHz by trials until the cross-over losses settle at exactly 3.01 dB. Let’s continue the present design, leaving the fun of adjustment of fpLP and fpHP to the user  .

- Figure 17.67.f shows the circuit after tuning the transformer. Now the LP and HP channels form a complementary (or quasi-complementary) pair.

- In the next stage we will try to put the circuit into a realizable topology. Such a HPF with single TZ at f=0 and multiple UE’s can be realized either as SC edge coupled line form or as interdigital filter. Let’s target interdigital filter. Since the HP filter is a narrowband one, element values will come out to be  unrealizable. Therefore a redundant UE is inserted from the load end using the command  Insert / TL: Z=50 ohms (Figure 17.67.f-g).

- Next, with cursor under the TL’s adjacent to the shunt SC stubs, the command Kuroda / Distribute Stub is applied to move part of the OC stubs to the other side of the TL’s. This is repeated until SC stubs are placed on both sides of all TL’s (Figure 17.67.h).

- We usually like to have equal SC stub impedances to have a balanced distribution of impedance values throughout the circuit. This is done by placing the cursor under any TL and applying the command Kuroda / All Equal Stubs / Include End Stubs. This command equates all the SC stubs (Figure 17.67.i). This filter is saved as HP38a.fil.

- Since the circuit is asymmetric we get a transformer. It is possible to absorb this transformer into the interior PCL sections but this would make them asymmetric which is usually not desirable. We will follow the strategy of absorbing this transformer partially or as a whole into the diplexing PCL, as will be described in the next stage where a series diplexing PCL section will be formed.

Figure 17.67. Design of the HP filter of the series LP-HP diplexer.

Figure 17.67. Design of the HP filter of the series LP-HP diplexer.

Formation of the Series Diplexing PCL section:

We will use Type-1 series diplexing PCL of Figure 17.45 which is repeated in Figure 17.68.a. It will be formed by modifying the input section of the HP filter which is loaded in Figure 17.68.b.

The equivalent circuit of the series diplexing PCL has a TL at its input with impedance Z2/k2, a shunt SC stub of impedance Z1/k2 and a transformer of turns ratio 1:k in its HP port. For symmetric PCL’s the parameters Z1, Z2 and k are related through k2=1+Z2/Z1. We will modify the input part of the HP filter to simulate this diplexing PCL.  In most cases it is preferable to have impedances of all resonators (Shunt SC stubs) equal. For such cases the following approach can be used.

Solution as Interdigital HPF with Equal Impedance Resonators:

- We insert a redundant (50 ohms) TL at input of the filter as shown in Figure 17.68.b-c. Then with cursor under the inserted TL, the command Kuroda / Distribute Stub is applied to move part of the stub on its right to its left (Figure 17.68.d). This process introduces a transformer which scales up the impedances of the filter to realizable level. This is the main function of the redundant TLs. Thus, for diplexers, the redundant TL has two simultaneous uses: Formation of a diplexing PCL and impedance scaling of the HP filter.

- Next, we equate impedances of all resonators (SC stubs) by applying the command Kuroda / All Equal Stubs / Include End Stubs.  We get the filter of Figure 17.68.e. It is realizable as an interdigital filter with SC alternate ends. We save it as HP38B.

- The even and odd mode impedances of the adjacent pairs of coupled lines can be displayed on Filpro in two steps, as described in Figure 17.68.e and f. The TLs act as elements which couple the adjacent pairs of resonators (SC stubs). With cursor under the TLs, the command Replace by PCL / Interdigital Lines / scscINTD / Symmetric is applied. This command is first applied on the TL’s 1, 3, 5 and 7. The resulting even and odd mode impedances are shown in Figure 17.68.f. It is saved as HP38B1. Then Undo command is applied to recover the circuit of HP38B (Figure 17.68.g) and the same command is applied on the TL’s 2, 4, 6 and 8 to give the circuit of Figure 17.68.h which is saved as HP38B2. We note down the even and odd mode impedances of all adjacent pairs and summarize them in the schematic description shown in Figure 17.68.i.

- Note that the first PCL on the left of HP38B1 in Figure 17.68.f is treated as the diplexing PCL with terminals 1, 1’ and 2’ indicated explicitly. This needs verification. We note down its even and odd mode impedances, Zev=61.5 ohms and Zod=42 ohms to be used in verification procedure.

- Verification can be done in two ways. One way is described in Figures 17.68.k-to-p and the other way is checking the response by connecting the LP channel, as described in Figure 17.68.q-r-s.

- In Figure 17.68.k, the circuit of HP38C is loaded. We will convert the circuit of the leftmost PCL section into the standard form shown in Figure 17.68.a to expose the location of the Port-1’ where the LP channel filter will be connected. For this purpose, with cursor under the TL, the command Kuroda / Left / Full Stub is applied to move the stub on its left to its right (Figure 17.68.m). Then the translated stub is combined with the adjacent stub (Figure 17.68.n).

- Finally, the transformer resulting from the Kuroda transformation is moved to the right of the translated SC stub by applying the command Shift TRF / n-Right / n=2 (Figure 17.68.p). So, at the source end of the filter we have formed the standard circuit model of the diplexing PCL shown in Figure 17.68.a. In this circuit the location of Port-1’ at which the LP filter will be connected is exposed now. This circuit is saved as HP38C. We calculate and note down the values of Z1, Z2 and k. Using these values we also calculate the even and odd mode impedances Zev and Zod and find the same values as before (Zev=61.59 ohms and Zod=42.08 ohms). So, the leftmost sections (diplexing PCL sections) of both HP38B1 and HP38C are just two different equivalent versions. The circuit of HP38C is handier for connection of the LP channel.

- In Figure 17.68.q the circuit of LP38 is inserted at point 1’ in the HP38C, as Series External 1-Port to form the LP-HP diplexer (Figure 17.68.r). The transformer at load end can be handled using inverters or by absorbing it into the interior PCL sections. Responses are shown in Figure 17.68.s. It is seen that there is no change compared to the original diplexer response. Figure 17.68.t, a pictorial description of the diplexer is given for possible realization in stripline-like structures.

Figure 17.68. Formation of the Series Diplexing PCL line and the series LP-HP diplexer.

Figure 17.68. Formation of the Series Diplexing PCL line and the series LP-HP diplexer.

Figure 17.68. Formation of the Series Diplexing PCL line and the series LP-HP diplexer.

Solution as SC Edge Coupled Line HPF with equal resonators:

- Just as exercise for the formation of series diplexing PCL sections under different conditions, let’s  realize the HP channel filter in SC edge coupled line form.

- The circuit of HP38B is loaded and all the inner SC stubs are split into two parts so that each TL will have its own SC stubs on its two sides (Figure 17.69.a-b).

- Then with cursor under any inner TL the command Kuroda / All Equal Stubs / Include End Stubs is applied to make all shunt SC stubs equal (Figure 17.69.c). Thus, we have formed the triplets of SC stub+TL+SC Stub each of which is equivalent to a different SC edge coupled line section. The leftmost triplet will be the series diplexing PCL element.

- We have to convert the leftmost PCL section into the standard series diplexing PCL form shown in Figure 17.69.a. For this purpose we move the SC stub on the left of the leftmost TL to its right by using the command Kuroda / Left / Full Stub (Figure 17.69.d).

- Then the translated stub is combined with the stub on its right (Figure 17.69.e).

- Finally the transformer on the left side of the TL is moved to the right of the combined stub by using the command Shift TRF / n-Right: n=2 (Figure 17.69.f). This is the standard form for the diplexing PCL section in which the location of the Port-1’ is exposed. Also the parameters Z1, Z2 and k are recognized and noted down for calculation of the even and odd mode impedances of the diplexing PCL section. They come out as Zev=66.58 ohms and Zod=40 ohms.

- Next we convert the inner SC Stub+TL+SC Stub triplets into PCL form by placing the cursor under the inner TL’s and applying the command Replace By PCL / Interdigital Line/ scscINTD / Symmetric. We get the SC edge coupled line version of the filter (Figure 17.69.g). It is saved as  HP38D.

- Then we form the diplexer by inserting the filter LP38 as Series External 1-Port (Figure 17.69.h).

- Figure 17.69.i shows the responses which are the same as the original diplexer.

- Figure 17.69.k shows a pictorial description of the diplexer with SC edge coupled HPF.

Figure 17.69. SC edge coupled line realization of the HPF channel of the LP-HP diplexer.

Figure 17.69. SC edge coupled line realization of the HP channel of the LP-HP diplexer.

# Alternative Solutions for LP-HP Diplexers

- In some applications it may be desirable to have the impedances of the diplexing PCL to be different than the interior lines or, for example in the interdigital version of the previous diplexer we may like to decouple the diplexing PCL section from the remaining interdigital part of the filter, etc. In such cases we can use the equations relating the parameters Z1, Z2, k, Zev and Zod of the diplexing PCL to adjust impedances. We have to observe observing the following constraints:

  - Impedance of the TL at source end is termed as  Z2/k2. It must be equal to source resistance RS=50 ohms. Hence we have

Z2=k2RS=50k2 ohms. 						(a)

  - Z2 is also known  in terms of Z1 and k as

Z2=Z1(k2−1)							(b)

  - Equating (a) and (b),

Z2=Z1(k2−1)=k2RS						(c)

we get

Z1=k2RSk2−1								(d)

or

Z1k2=RSk2−1							(e)

which is the impedance of the SC stub on the right of the TL.

  - The transformer turns ratio k and the impedance Z1 are related to the even and odd mode impedances Zev and Zod of the diplexing PCL through

k=Zev+ZodZev−Zod 							(f)

and 	Z1=Zev+Zod2							(g)

  - Equations (d) and (e) gives the following constraints on Zev and Zod in terms of k and RS:			Zev+Zod=2k2RSk2−1							(h)

and	Zev−Zod=2kRSk2−1							(i)

  - Solving (h) and (i) for Zev and Zod, we get

Zev=Z2k(k−1)=kRSk−1   						(k)

and 	Zod=Z2k(k+1)=kRSk+1						(m)

- Thus, given the source resistance RS, the transformer turns ratio k sets all the other parameters (Z2, Z1, Zev and Zod). It seems that the transformer turns ratio k is a good iteration parameter. We can use the equations listed above in different manners for finding different solutions.

- Just as exercise with these equations let’s redesign the diplexer with interdigital HPF in a form where the impedances of the diplexing PCL set independently of the interdigital filter with the aim of absorbing the load end transformer. In the previous case the diplexing PCL were treated as an integral part of the interdigital filter and all the shunt SC stubs were equated, leaving a transformer at load end. Now we will form the diplexing PCL section separated form the interdigital filter and absorb the load end transformer into the diplexing PCL.  {The transformer could as well be absorbed into the interdigital filter, however in that case we would loose the equal resonator property}.

- In Figure 17.70.a the circuit model of the diplexing PCL is repeated for convenience.

- In Figure 17.70.b, the circuit of HP38A is loaded. We will modify its diplexing PCL part to absorb the transformer that remained at load end. This is possible if we can make the transformer turns ratio at load end equal to the transformer (k) of the diplexing PCL. The problem is that we don’t know the value of k. It will be found by iterative trials, as follows.

- We select a starting trial value of k=4 and calculate the corresponding Z1/k2 =3.33 ohms using Eqn.(e). Then the leftmost SC stub is split to form the SC stub with impedance 3.33 ohms by applying the command Split / Specify Left: Z=3.33 ohms (Figure 17.70.b-c). The left piece will be treated as the SC stub Z1/k2. It will be part of the diplexing PCL while the right piece will be part of the interdigital filter. The left piece is electromagnetically decoupled from the interdigital part of the circuit.

- Splitting action destroys the equal stub property of the interdigital part of the circuit. The command Kuroda / All Equal Stubs / Include End Stubs is applied recover the equality of the stubs of the interdigital part of the circuit. The leftmost stub is unaffected by this operation (Figure 17.70.d). Now we inspect the transformer turns ration at the load end. It is n=4.19, greater than the intended transformer value of k=4.

- We repeat the procedure now by specifying k=4.1. We get Z1/k2 =3.16 and load end transformer of turns ratio n=4.06, close to k=4.1 (Figure 17.70.e-f).

- Next, the procedure is repeated by selecting k=4.08. We get Z1/k2 =3.19 and n=4.09, very close to k=4.08 (Figure 17.70.g-h). Hence we stop the trials. The transformer at the load end will be used as the transformer of the diplexing PCL and no transformer will be left at load end.

- Now we start the formation of the circuit model of the diplexing PCL section. First a TL is inserted from the source end to act as Z2/k2 (Figure 17.70.h-i). Then the transformer at load end is shifted to the right of the SC stub which acts as Z1/k2. Thus, the diplexing PCL circuit model is formed as shown in Figure 17.70.k. The circuit is saved as HP38K.

- The even and odd mode impedances of the diplexing PCL section can be calculated using the formulae shown in the figure as Zev=66.23 ohms and Zod=40.16 ohms. Note that the diplexing PCL and the interdigital filter sections are now decoupled electromagnetically, connected through the node-2’ only.

- The diplexing PCL section can be put into an equivalent form involving a TL with shunt SC stubs on its two sides by applying the command Kuroda / Right / Equal Stubs. This command produces a transformer which cancels the transformer k on the right giving the circuit of Figure 17.70.m. This form is programmed on Filpro for conversion from the circuit model into PCL form to display its even and odd mode impedances. The PCL form can be obtained by placing the cursor under the TL and applying the command Replace by PCL / Interdigital Line / scscINTD / Symmetric as shown in Figure 17.70.n. The even and odd mode impedances are the same as the calculated ones, as expected. This circuit is saved as HP38M.

- The even and odd mode impedances of the adjacent coupled lines of the interdigital section can be displayed by converting the triplets of SC stub+TL+SC Stub into PCL form one by one, by applying the command Replace by PCL / Interdigital Line / scscINTD / Symmetric. In Figures 17.70.p, q and r the PCL equivalents of the triplets are shown together with their even and odd mode impedances. In Figure 17.70.s, the results are summarized.

- Then the diplexer is formed by inserting the LP filter (LP38) as Series External 1-Port at node-1’ of HP38K (Figures 17.70.t-u). Responses are the same as the original diplexer (Figures 17.70.v). Figures 17.70.w-x-y show various realization topologies.

- Figure 17.70. A diplexer with separate diplexing PCL and interdigital HPF.

- Figure 17.70. A diplexer with separate diplexing PCL and interdigital HPF.

- Figure 17.70. A diplexer with separate diplexing PCL and interdigital HPF.

- Figure 17.70. A diplexer with separate diplexing PCL and interdigital HPF.

EXAMPLE 17. 33: A digital elliptic LP-HP diplexer

Wenzel describes lowpass-highpass diplexers utilizing digital elliptic filters obtained from singly terminated elliptic filter lowpass prototypes 3. Such diplexers can be designed through Filpro by either using a lowpass prototype or by synthesizing the channel filters directly, with the latter approach being simpler.

As an example for the latter approach, let’s design a digital elliptic contiguous diplexer with cross-over frequency fc=8000 MHz by synthesizing the singly terminated elliptic LP and HP channel filters directly. The quarter wavelength frequency for both filters will be selected as fq=16000 MHz.

The LP and HP filters will be connected in parallel. Therefore we need series type elements on source side.

PBRipple=1 dB is selected for both channels, arbitrarily. This parameter may be iterated to get different overall diplexer return loss levels.

- Synthesis steps of the lowpass channel are shown in Figure 17.71.a. A series SC stub is needed as the first element on the source side for parallel connection to the HP filter. Also the finite transmission zero sections are desired to be in series arm as parallel resonators to be realized in the slabline technology described by Wenzel in [3]. These two constraints enforce us to synthesize an even degree. Degree 12 is selected with two transmission zeros at infinity and five finite transmission zeros all chosen at the same frequency fzLP=9000 MHz leading to a quasi-elliptic type filter. Type-B is selected during synthesis because the synthesized filter will be reversed, giving a series arm first element on source side. Transmission zero extraction order is shown in the figure. The synthesized circuit is saved as LP52.fil.

- Synthesis steps of highpass channel are shown in Figure 17.71.b. Again degree 12 is selected with two transmission zeros at zero and five finite transmission zeros all chosen at the same frequency fzHP=7111 MHz  calculated through the relation fzHP =fc2 /fzLP  which is needed in complementary filters. Type-B is selected during synthesis because the synthesized filter will be reversed, giving a series arm first element on source side. Transmission zero extraction order is shown in the figure. The synthesized circuit is saved as HP52.fil.

- In Figures 17.71.c, the diplexer is formed by inserting LP52 into HP52 as External 1-Port. Figure 17.71.d shows the responses.

- For realization see reference [3].

Figure 17.71. Synthesis of an elliptic LP-HP diplexer.

Figure 17.71. Synthesis of an elliptic LP-HP diplexer.

- DISTRIBUTED ELEMENT MULTIPLEXERS FORMED BY CASCADING

- LP-HP

- DIPLEXERS

- 17. 18. MULTIPLEXERS FORMED BY CASCADING LP-HP DIPLEXERS

- Multiplexers can be formed by cascading independently designed LP-HP diplexers. The general structure would be as shown in Figure 17.72, described on a triplexer. The LP filter at left end sets the upper frequency limit of the triplexer, denoted as fmx.  The next stage is the diplexer LP1-HP1 with cross-over frequency fc1. The signals in the frequency range fc1<f<fmx will pass through the filter HP1 while those in the range f<fc1 will pass through the filter LP1 and directed to the next diplexer LP2-HP2 with cross-over frequency fc2<fc1. The signals in the frequency range fc2<f<fc1 will pass through the filter HP2 while those in the range f<fc2 will pass through the filter LP2.

- Let’s continue discussions on an example.

- Figure 17.72. A triplexer formed by cascading LP-HP diplexers.

# EXAMPLE 17. 34. A triplexer formed by cascading two LP-HP diplexers

- The triplexer will split the frequency range  f<fmx=18 GHz into three channels:

- Channel-A will pass the frequency range  fc1 = 8 GHz < f < fmx =18 GHz.

- Channel-B will pass the frequency range  fc2 = 4 GHz < f < fc1 =8 GHz.

- Channel-C will pass the frequency range   f < fc2 = 4 GHz.

We will design two diplexers, one with cross-over frequency fc1=8 GHz, named as Diplexer-A with chnnale filters LPA-HPA and the other with cross-over frequency fc2=4 GHz, named as Diplexer-B with channel filters LPB-HPB. Also a LP filter with fp=fmx=18 GHz is needed to set the upper limit of the diplexer. It will be named as LPMX.

Let’s use degree n=9 singly terminated filters for both diplexers, with fp=0.01 dB.

The HP channels will be of Interdigital type. Therefore we will have Nzero=1 and NUE=8. In accordance with Wenzel’s rule, the complementing LP filters will have Ninf=9. However just as a trial we will use Ninf=5 and NUE=4. The LP and HP filters will be connected in series using Series Diplexing PCL sections.

Design of LPMX:

Design steps of LPMX is shown in Figure 17.73. Its passband edge frequency is selected as fp=18000 MHz. The quarter wavelength frequency is selected as fq=30000 MHz. It will set the upper frequency limit of the triplexer. Its degree can be set as desired, to provide the necessary selectivity at this end of the passband. We arbitrarily select a degree 13 LPF, with Ninf=7 and NUE=6. It is designed as a Doubly Terminated LPF, in Type B form. PBRipple=0.01 dB is selected. The transmission zeros are extracted in an alternating Inf-UE-Inf-UE-… . The resulting filter is saved as LPMX.fil.

Design of LPA:

Design steps of LPA is shown in Figure 17.74. Its passband edge frequency is selected as fp=7500 MHz, after trials, in coordination with the passband edge frequency of the HPA so that the cross-over frequency will be about 8000 MHz. The quarter wavelength frequency is selected as fq=20000 MHz. Degree is nine, with Ninf=5 and NUE=4. It is designed as a SinglyTerminated LPF, in Type B form. PBRipple=0.01 dB is selected. The transmission zeros are extracted in an alternating Inf-UE-Inf-UE-… . The resulting filter is saved as LPA.fil.

Figure 17.73. Design of the band limiting LPF.

Figure 17.74. Design of the filter LPA of Diplexer-A.

Design of HPA and formation of Diplexer-A:

- Design steps of HPA is shown in Figure 17.75. Its passband edge frequency is selected as fp=9200 MHz, after trials, in coordination with the passband edge frequency of the LPA so that the cross-over frequency will be about 8000 MHz. The quarter wavelength frequency is selected as fq=20000 MHz. Degree is nine, with Nzero=1 and NUE=8. It is designed as a SinglyTerminated HPF, in Type B form. PBRipple=0.01 dB is selected. The transmission zeros are extracted in the order   UE-UE-UE-UE-Inf-UE-UE-UE-UE. The resulting filter is shown in Figure 17.75.a.

- A unity transformer is inserted at the source end to form the complement of the LP filter LPA (Figure 17.75.b).

- The LP channel filter LPA is inserted as Series External 1-Port to form the diplexer (Figure 17.75.c).

- The transformer of the HP filter is tuned while observing the return loss. Tuning is stopped when return loss gets its best level (Figure 17.75.h).

- The External 1-Port is deleted and the remaining HP filter is saved as HPA.fil (Figure 17.75.d-e).

- The LP filter LPA is loaded and HPA is inserted as External 1-Port to form the diplexer which will be used in the formation of the triplexer. It is saved as DPXA.fil (Figure 17.75.f-g).

- Figure 17.75.h shows the insertion loss and return loss responses. The cross-over frequency is about fc1=8000 MHz. It can be made to coincide with 8000 MHz exactly by redesigning the channel filters with slightly different passband edge frequencies.

Design of LPB:

- Design steps of LPB is shown in Figure 17.76. Its passband edge frequency is selected as fp=3700 MHz, after trials, in coordination with the passband edge frequency of the HPB so that the cross-over frequency will be about 4000 MHz. The quarter wavelength frequency is selected as fq=10000 MHz. Degree is nine, with Ninf=5 and NUE=4. It is designed as a SinglyTerminated LPF, in Type B form. PBRipple=0.01 dB is selected. The transmission zeros are extracted in an alternating Inf-UE-Inf-UE-… . The resulting filter is saved as LPB.fil.

Design of HPB and formation of Diplexer B:

- Design steps of HPA is shown in Figure 17.77. Its passband edge frequency is selected as fp=4400 MHz, after trials, in coordination with the passband edge frequency of the LPB so that the cross-over frequency will be about 4000 MHz. The quarter wavelength frequency is selected as fq=10000 MHz. Degree is nine, with Nzero=1 and NUE=8. It is designed as a SinglyTerminated HPF, in Type B form. PBRipple=0.01 dB is selected. The transmission zeros are extracted in the order   UE-UE-UE-UE-Inf-UE-UE-UE-UE. The transmission zeros are extracted in the order   UE-UE-UE-UE-Inf-UE-UE-UE-UE. The resulting filter is shown in Figure 17.77.a.

- A unity transformer is inserted at the source end to form the complement of the LP filter LPA (Figure 17.77.b).

- The LP channel filter LPB is inserted as Series External 1-Port to form the diplexer (Figure 17.77.c).

- The transformer of the HP filter is tuned while observing the return loss. Tuning is stopped when return loss gets its best level (Figure 17.77.h).

- The External 1-Port is deleted and the remaining HP filter is saved as HPB.fil (Figure 17.77.d-e).

- The LP filter LPB is loaded and HPB is inserted as External 1-Port to form the diplexer which will be used in the formation of the triplexer. It is saved as DPXB.fil (Figure 17.77.f-g).

- Figure 17.77.h shows the insertion loss and return loss responses. The cross-over frequency is about fc1=4000 MHz. It can be made to coincide with 8000 MHz exactly by redesigning the channel filters with slightly different passband edge frequencies.

Figure 17.75. Design of the filter HPA and formation of Diplexer-A.

Figure 17.75. Design of the filter HPA and formation of Diplexer-A.

Figure 17.76. Design of the filter LPB.

Figure 17.77. Design of the filter HPB and formation of Diplexer-B.

Figure 17.77. Design of the filter HPB and formation of Diplexer-B.

Formation of the Triplexer:

- Formation of the triplexer is described in Figure 17.78.

- First, the band limiting LP filter, LPMX is loaded (Figure 17.78.a).

- Then with cursor under the load of LPMX, the command Append Filter / DPXA is applied to append the Diplexer-A (Figure 17.78.b).

- Figure 17.78.c shows the insertion losses of the 0-8000 MHz and 8000-18000 MHz channels, together with the return loss of the complete diplexer.

- Then with cursor under the load end of DPXA, the command Append Filter / DPXB is applied to append the Diplexer-B also, thus completing the formation of the triplexer (Figure 17.78.d).

- Figure 17.78.e shows the insertion losses of the channels and the return loss of the triplexer. The triplexer seems to work fine. The slight degradations about the cross-over region can be corrected by tuning.

Figure 17.78. Formation of the triplexer.

Figure 17.78. Formation of the triplexer.