## Distributed Element

BP-BP Diplexers and Multiplexers

17. 19.  ON DISTRIBUTED ELEMENT BP-BP DIPLEXERS AND MULTIPLEXERS

Alternative design approaches:

- On Filpro, distributed element BP-BP multiplexers can be designed in several ways.

- a) Multiplexer channel filters can be designed from the same singly terminated normalized LP prototype by using the classical LP Prototype-to-BP mapping. The normalized LP prototypes can be prepared in two ways:

  - The maximally flat or Chebyshev singly terminated LP prototypes can be obtained as the LP arm of a normalized LP-HP diplexer. The normalized LP-HP diplexer can be synthesized by using Design / LP-HP Diplexer command of Filpro with any desired cross-over loss at the normalized cross-over frequency wc=1 (fc=0.159) and normalized (unity) terminations. This technique yields complementary LP-HP pairs. We can use the LP arm of this diplexer as the LP prototype and design all the BP filters of channels using this prototype. The resulting adjacent filter pairs will also form complementary pairs with the desired cross-over loss.

  - Elliptic-like singly terminated LP prototypes can be directly synthesized and then used to design the BP versions by LP prototype-to-BP mapping.

- LP Prototype-to-BP mapping is especially easier for controlling the cross-over frequency and loss. However the resulting BP filters need circuit transformations for conversion into the desired topology, usually necessitating the use of inverters. When inverters are then replaced by their practical equivalents response may degrade appreciably especially in medium and wideband filters which means more efforts in optimization stage.

- b) It is always possible to design the singly terminated BP channel filters directly by using Design / By Synthesis technique and selecting Singly Terminated option. The advantage of this approach is that the targeted topology can be formed without approximate circuit transformations, hence less degradation in response. The disadvatage is that the adjacent passband edge frequencies of adjacent channels may need iterations to set the cross-over frequency and loss properly.

- c) It is also possible to use Doubly Terminated BP filters which may necessitate longer optimization or foreshortened Doubly Terminated BP filters which gives responses close to singly terminated cases.

- In any approach one has to use susceptance annulling elements at diplexing ends to adjust the passband return loss and also adjust the cross-over loss and cross-over frequency.

- For communication applications, channels need not be contiguous. On the contrary, a specified minimum level of attenuation is needed at cross-over frequencies. For such applications, singly terminated designs may not be optimum solutions. For such filters it may be better to use doubly terminated.

- In BP-BP multiplexers the channel filters can be connected in parallel or in series using diplexing PCL sections or multiplexing junctions or the filters can be connected to a common manifold.

In this section design of all-stub diplexers, triplexers and quadruplexers using singly terminated and  doubly terminated filters will be described just to get an idea by comparing the different approaches. The observations that will be made on these examples may be used as a guide for actual practical designs that will be given in later sections.

All-stub filters are rarely realizable without passing through some circuit transformations. Therefore this example is aimed at getting an idea on the conditions for the best possible return loss levels by inspecting the interactions between channels. We will also compare the approaches using singly terminated and doubly terminated filters.

Geometrical symmetry:

In studying the lumped element diplexers with singly terminated component filters we have seen that if the adjacent channels have geometric symmetry, then the interactions degrading the return loss level is minimized. As the channel bandwidths get far from geometric symmetry conditions return loss degradation gets higher. So, we may use such diplexers and multiplexers as reference to compare the actual designs with. All the other diplexers will have worse response than the reference diplexers. Geometrical symmetry is defined for lumped element LP-HP diplexers as follows:

fpLPfpHP=fc2

That is, the cross-over frequency is the geometric mean of the passband edge frequencies of the LP and HP filters, hence the name geometric symmetry. This property maps to a BP-BP type diplexer as

fp1a∗fp2b=fp2a∗fp1b=fc2

where fp1a and  fp2a are the passband edges of one channel (Channel-A) and  fp1b  and  fp2b are the passband edges of the other channel (Channel-B) respectively. fc  is the cross-over frequency. Geometric symmetry gives the best possible return loss level towards the inner sides of the passbands of both filters.

This property is derived for lumped element BP-BP diplexers. For distributed element BP-BP diplexers a new equation can be derived by using the LP prototype-to-distributed element BPF mapping functions. However simplicity of the above relation for lumped element filters tempts us to use it for distributed element filters as well, with the price being a little more degradation compared to the optimum geometrically symmetric case. Given the bandwidth of one channel, this relation gives the bandwidth of the adjacent channel. The channel bandwidths of such diplexers and multiplexers increase progressively as the channel center frequencies increase. This relation can be mapped to cross-over frequencies of multiplexer also as shown in Figure 17.79. Given the cross-over frequencies  fc,k-1 and fc,k, the next cross-over frequency fc,k+1 is calculated through the formula,

fc,k+1=fc,k2fc,k−1

- Multiplexers having bandwidths no satisfying these criteria will have extra distortions in responses (Exercise?).

- Let’s continue discussions on some classical microwave diplexers and multiplexers using interdigital and combline filters.

Figure 17.79. Geometrically symmetric channels.

17. 20. ON INTERDIGITAL AND COMBLINE MULTIPLEXERS

In microwave practice distributed element BP-BP multiplexers usually utilize combline, interdigital or capacitively loaded interdigital filters (CLIF’s). Their popularity comes from their compactness, ruggedness, simplicity of construction and tuning  [4], [13], [14], [15], [16], [20]. The transmission zero patterns of such filters are as follows:

- Combline filters have only one transmission zero at f=0 and the rest are all at infinity (Nzero=1, Ninf=N-1). This characteristics result in unequal slopes on lower and upper skirts. Therefore singly terminated combline filters of adjacent channels may not have exact complementary nature.

- Interdigital filters have Nzero=1 and NUE=N-1 and being a HPF, have symmetrical response about f=fq. Therefore we may expect better complementing property. However these points may depend on various factors like bandwidth, degree, targeted return loss level, etc.

- Capacitively Loaded Interdigital filters (CLIF’s) have Nzero=1, Ninf=N-1 and NUE=N. They have higher symmetry in insertion loss response compared to combline filters.

- Therefore we will make some trial designs to see the conditions under which singly terminated and doubly terminated designs are advantageous to use. However, even without experimentation, we have the following feeling:

- In a multiplexer with k-channels and with each channel having a degree N filter, there are kN elements that are used to shape the overall response of the multiplexer. That is, the response of each channel has contributions from kN elements, with some of these contributions being destructive and some constructive depending on frequency. Assuming noncomplementary adjacent channels, which is the case in doubly terminated filters, each channel has an excess immitance not canceled by its neighbor. If the sum of all these immitances can be made to add up to zero through an optimization process, yielding a good match at the input of the multiplexer then we may avoid the use of susceptance annulling networks while also improving the selectivities of the individual channels.

- This statement is valid for both singly terminated and doubly terminated designs. If the singly terminated designs yield complementary adjacent pairs, then convergence of the optimization process may be faster. However since the same number of elements exist in both singly and doubly terminated designs, we may expect to reach at the same responses using both approaches, but possibly with different element values because of difficulties in predicting the resulting return loss level of the diplexer that correspond to a specified passband ripple.

- In this section some examples will be made on singly and doubly terminated, contiguous and noncontiguous  interdigital and combline diplexers and triplexers to get some ideas on the validity of the above statements.

- Interconnection of interdigital and combline multiplexers

- Combline and interdigital filters present low impedances outside their passbands, tending to SC away from their passbands. Therefore if they are connected in parallel, the channel filters would load each other heavily. Fortunately, for medium / narrow channel bandwidths impedance scaling inverters that are used at terminations of the filters invert the input impedance of the filters. That is they present high impedance outside their passbands tending to OC away from the passband. Therefore for such filters parallel connection is possible which can be carried out by using  common junction parallel diplexing PCL sections or other methods that will be presented in this chapter. For wideband cases inverters can’t be used. Therefore for such filters special techniques have to be used so that their input impedances would not load each other.

A design method for combline filter multiplexers employing a common junction were first developed by Matthaei and Crystal [4] which were limited to narrow to octave band designs. Their approach was based on either singly terminated or doubly terminated prototypes. Both approaches require modifications at input elements of filters for both input impedance compensation and to solve interconnection problems when filters are connected at a common junction. Then several papers of LaTourette [13]- [15] present wider band multiplexers based on modifications of singly or doubly terminated filters and using extended junctions. Then, Rhodes and Levy [19] presented a complete theory for such multiplexers giving also the theory of common junction (resonator) transformers. Then Alseyab and Rhodes [20], [21] introduced multioctave common junction contiguous and noncontiguous multiplexer design procedures based on modification of the formulations for doubly terminated designs and then optimizing all elements of the resulting circuit.

- Figure 17.80 shows the two possible common resonator parallel diplexing-multiplexing structures that can be used for interconnection of medium and narrowband channel filters.

- The structure shown in Figure 17.80.a is known as interdigital type common junction parallel diplexing and multiplexing section. It is based on the three PCL sections which was defined in Figure 17.68. The name interdigital comes from the fact that the SC ends of the common resonator and the adjacent resonators (of the channel filters) are on opposite sides. Therefore we may also name this structure as opposite sided SC diplexing PCL section.

Figure 17.80.b shows comb type common junction parallel diplexing where the SC ends of common resonator and the adjacent resonators are on the same side. Therefore we may also name it as same sided SC diplexing PCL section.

It possible to feed more than two channels through the center input conductor which is common to all channels. These structures connect the filters in parallel. Both structures can be used to interconnect interdigital and combline filters.

Figure 17.80. a) Interdigital type common resonator parallel diplexing section, b)  Comb type common resonator parallel diplexing section.

# 17. 21. INTERDIGITAL BP-BP DIPLEXERS AND MULTIPLEXERS

Let’s make some examples to display the techniques available on Filpro for the design of multiplexers using interdigital filters.

EXAMPLE 17. 35: Interdigital diplexer and multiplexer design using Prototype LP-HP diplexers

Let’s use a singly terminated LP prototype filter to design channels of a contiguous diplexers, triplexers and quadruplexers. The LP prototype will be obtained as the LP arm of a LP-HP diplexer. Such diplexers are synthesized on Filpro in the most efficient way, with exact cross-over loss at the desired cross-over frequency.

Let’s consider multiplexer channels with following specifications:

Channel-A: fp1=10000 MHz, fp2=12000 MHz, fq=11000 MHz.

Channel-B: fp1=12000 MHz, fp2=14000 MHz, fq=13000 MHz.

Channel-C: fp1=14000 MHz, fp2=16000 MHz, fq=15000 MHz.

Channel-B: fp1=16000 MHz, fp2=18000 MHz, fq=1700 MHz.

Since the channel filters will be distributed element HP filters, we take the quarter wavelength frequencies of the two filters as the their passband centers. Let’s use five resonators.

We will use a LP prototype of degree N=5 with PBRipple 0.1 dB.

Preparation of the LP prototype is described in Figure 17.81.

- We first design a Chebyshev LP-HP diplexer with PBRipple=0.1 dB and degree N=5.

- The cross-over loss is taken as 3.01 dB with normalized cross-over frequency fc=0.159 MHz (corresponding to normalized radian frequency wp=1).

- Design LP-HP Diplexer algorithm synthesizes the singly terminated complementary pair LP and HP filters and prepares four files. One LP and one HP filters and two diplexer circuits. One of the diplexer circuits is formed by LP filter with the HP filter appearing as External 1-Port element and the other is formed by the HP filter with the LP filter appearing as External 1-Port element.

- In the figure the two diplexer circuits are shown together with insertion loss and return loss responses. Return loss is about 27 dB. We are interested in only the LP arm. We name is as LPP01 (LP Prototype with 0.1 dB ripple). We will use this normalized LP prototype filter to design the BP filters of the multiplexers.

The design of the channel filters are described in Figure 17.82.

Figure 17.81. Preparation of the singly terminated LP prototype.

Design of the channel filters:

- Design steps of Channel-A filter is described in Figures 17.82.a-to-h.

- Design / By User Defined Prototype option is selected.

- Load a Prototype command is invoked and the prototype LPP01 is typed in.

- Filter Type / Distributed-HPF options are selected.

- HP Filter edge frequency fp=10000 MHz and fq=11000 MHz (for Channel-A) are typed in.

- Find Circuit command gives the HPF shown in Figure 17.82.a. Topology of the circuit and element values need changes as follows.

- Find Dual command is applied to make the first element a shunt SC stub (Figure 17.82.b).

- With cursor under the series OC stubs, the command Inverters / Replace by Inverter Equivalent is applied to convert them into shunt SC stubs surrounded by ideal inverters of value K=50 ohms (Figure 17.82.c).

- Simplify Circuit command is applied to eliminate the phase changing transformers (Figure 17.82.d).

- The command Inverters / Equate All Elements is applied to equate the impedances of shunt SC stubs (Figure 17.82.e). The resulting transformer appears at load end.

- The transformer at load end is shifted to source end because it is needed at that end for proper operation of the singly terminated filter (Figure 17.82.f).

- Next, impedances of the shunt SC stubs will be adjusted to realization level, for example Z=70 ohms. This is done by applying the command Inverters / Scale Circuit by Inverters / Specify Adjacent SC Stub. In this command the term adjacent means “adjacent to source”. Therefore this command is applied after reversing the circuit so that there will be a shunt SC stub at source end (Figure 17.82.g-h).

- After scaling the circuit by inverters, the transformer at load end is absorbed into the load end inverter and the circuit is reversed again. (Figure 17.82.i). The filter is ready. It is saved as A101.

- The same approach is used to design the other channel filters which are saved as  B101, C101 and D101 respectively (Figure 17.82.k, m, n).

Figure 17.82. Design of singly terminated interdigital channel filters using the LP prototype.

Figure 17.82. Design of singly terminated interdigital channel filters using the LP prototype.

Formation of diplexer, triplexer and quadruplexer:

- A contiguous diplexer is formed in Figure 17.83.a-b by inserting B101 into A101 as External 1-Port element. Wide span insertion and return loss responses are shown in Figure 17.83.c. Return loss response is very good around the cross over region. This is because the two adjacent filters are complementary. However return loss response degrades heavily towards the outer edges. The main advantage of this approach appears as the tight control over the cross over frequency and loss, as shown in Figure 17.83.d. They are exact, at fc=11000 MHz, with 3.01 dB loss.

- A contiguous triplexer is formed in Figure 17.83.e-f by inserting C101 and B101 into A101 as External 1-Port elements. Wide span insertion and return loss responses are shown in Figure 17.83.g. Return loss response is very good in the passband of the middle channel, B101. This is because the two adjacent filters are complementing B101. Return loss responses of the outer channels degrade heavily towards the outer edges. The cross over frequencies and losses are shown in Figure 17.83.h. They are very close to the targeted values.

- A contiguous quadruplexer is formed in Figure 17.83.i-k by inserting D101, C101 and B101 into A101 as External 1-Port elements. Wide span insertion and return loss responses are shown in Figure 17.83.m. Return loss response is very good in the passbands of the middle channels, B101 and C101. This is because the they are complemented by the outer filters, A101 and D101. Return loss responses of the outer channels degrade heavily towards the outer edges. The cross over frequencies and losses are shown in Figure 17.83.n. They are very close to the targeted values.

In the actual realization of the interdigital diplexers and multiplexers, the ideal inverters will be replaced by their quarter wavelength transmission line equivalents. These transformations will effect both the cross-over loss and will lead to narrowing in banwidths of the channels. Therefore we will delay these transformations to inspect more details, as will be studied in the following subsections.

Let’s now see how to improve the return loss responses, first without any suseptance annulling elements and then using a common transformer as a susceptance annulling element.

Figure 17.82. Design of singly terminated interdigital channel filters using the LP prototype.

Figure 17.83. Formation of contiguous interdigital diplexer, triplexer and quadruplexer.

Figure 17.83. Formation of contiguous interdigital diplexer, triplexer and quadruplexer.

Optimization of contiguous diplexer without susceptance annulling elements:

Let’s consider only the contiguous diplexer formed by the filters A101 and B101, shown in Figure 17.84.

In order to get a flat passband return loss, we can optimize the inverter impedances. Since the impedances of the shunt SC stubs can be scaled to any level by using the inverters, it is not necessary to tune the SC stubs. Using Monte Carlo-Best Case routine together with occasional use of Tune Element command, we get about 10 dB return loss level shown in the figure. The optimized filters are saved as A102 and B102 respectively.

Optimization of contiguous diplexer with common resonator as susceptance annulling element:

- In Figure 17.84.a, the diplexer formed by A101 and B101 is repeated.

- We Copy one shunt SC stub and Paste it at the source end to use as common resonator (Figure 17.84.b). First the length of common resonator is tuned to get a return loss response which is roughly symmetric about the cross-over frequency (Figure 17.84.c). We get a slight improvement in return loss level.

- Next impedance of the common resonator is tuned. It is very effective on return loss level, as seen from Figure 17.84.d. As the impedance is decreased we get a higher return loss over wider bandwidth. That is, impedance of the common resonator should be selected as low as possible.

- Next, just to get an idea, we optimize the return loss level for various impedances of the common resonator. Figure 17.84.e and f  show the trials for Z=70 ohms. The widest bandwidth is obtained as f1=10100 MHz and f2=13950 MHz, at 11 dB return loss level. The optimized filters are saved as A111 and B111.

- Then we target a narrower bandwidth and get 14 dB return loss for f1=10250 MHz and f2=13750 MHz. The optimized filters are saved as A112 and B112.

- Then another trial is made by setting common resonator impedance as Z=30 ohms. For this impedance we get 17 dB return loss level for f1=10200 MHz and f2=13850 MHz (Figure 17.84.g-h-i). The optimized filters are saved as A113 and B113.

- In all of the optimizations, the cross-over frequency is shifted slightly. In Figure 17.84.k the shift in the last optimization is shown.

Figure 17.84. Optimization of the contiguous interdigital diplexer without susceptance annulling element.

Figure 17.84. Optimization of the contiguous interdigital diplexer with common resonator.

Figure 17.84. Optimization of the contiguous intedigital diplexer with common resonator.

Figure 17.84. Optimization of the contiguous interdigital diplexer with common resonator.

Replacing inverters with quarter wavelength transmission lines:

Circuit model of an interdigitally coupled line pair is two shunt SC stubs (resonators) separated (coupled) by a transmission line which has the same length as the SC stubs. Since this is a distributed element HP filter, SC stub lengths are quarter wavelength long at band center. Therefore in order to get the circuit model of the interdigital filter we will convert all the inverters into quarter wave transmission lines. In Figure 17.85, this operation is described on the filters A111 and B111, the pair for which we have got about 11 dB return loss. The new circuits are optimized to give about 14 dB return loss over a bandwidth from f1=10150 MHz to f2=13850 MHz. An appreciable shrinkage is observed in the total bandwidth. The reason for bandwidth shrinkage is that the conversion of ideal inverters into quarter wave lines which is not a good approximation for wideband filters. The cross-over frequency is also shifted slightly away from the targeted 12000 MHz. The optimized final circuits are saved as  A115 and B115 for comparison with the circuits of the approach that will be described in the next example.

Figure 17.85. Conversion of inverters into quarter wave transmission lines.

Figure 17.85. Conversion of inverters into quarter wave transmission lines.

# EXAMPLE 17. 36: Singly terminated BP synthesis approach for interdigital diplexer design

In the previous example which uses prototype LP-HP diplexer as starting point, we have seen that because of wide passbands, conversion of inverters into transmission line equivalents are causing appreciable band narrowing. Let’s now design the same contiguous diplexer of the previous example now by using exact singly terminated synthesis approach in which case we will design filters directly as BP filters instead of transformations from LP prototype. We expect less degradations in response because we will avoid inverter couplings. The main problematic point of this approach is determination of the inner passband edges of the channel filters.

The channel filters will be designed as distributed element HP filters which necessitates specification of only the lower PB edge, fp=fp1 and the quarter wavelength frequency fq which is also the passband center of the filter. The upper passband edge fp2 will then be automatically set as

fp2=fq+(fq-fp1)/2

We will take PBRipple=0.1 dB. Nzero=1 and NUE=4 to get a five resonator filter.

In the LP prototype approach of the previous example we have used the following band edge and quarter wave frequencies:

Channel-A: fpA=fp1A= 10000 MHz, fqA= 11000 MHz leading to fp2A= 12000 MHz.

Channel-B: fpB=fp1B= 12000 MHz, fqB= 13000 MHz leading to fp2B= 14000 MHz.

In direct synthesis of singly terminated HP filters the inner passband edge frequencies (on the two sides of the cross-over frequencies), that is fp2A and fp1B need be determined by iteration until the cross-over frequencies settle at the targeted values with the specified cross-over loss. The outer edges of the pass bands will be kept the same at fpA=fp1A=10000 MHz and fp2B=14000 MHz. Iterations will be made on the quarter wavelength frequencies fqA and fqB of the two filters which will in turn set the inner band edges, fp2A and fp1B. The formulation that relate the inner and outer band edge frequencies and the cross-over frequency is as follows.

- Given fp1A=10000 MHz, fp2B=14000 MHz and fc=12000 MHz, we select a trial fp2A=11880 MHz.

- Then fp1B is found as mirror image of fp2A around the cross over frequency fc as follows:

- fpB=fp1B=fc + (fc - fp2A) = 12000 + (12000 – 11880) = 12000 + 120 = 12120 MHz

- So, the quarter wave frequencies of the two channels are found as the arithmetic averages of the respective band edge frequencies as follows:

fqA=(fp1A+fp2A) /2 =  (10000+11880) / 2 = 10940 MHz

fqB=(fp1B+fp2B) /2 =  (12120+14000) / 2 = 13060 MHz

Then the frequency pairs (fp1A, fqA) and (fp1B, fqB) are used to design the two filters as described below and in Figure 17.118. Then the  filters are connected to form a diplexer. Responses are plot to check if the cross-over frequency  and loss specifications are satisfied or not. If not satisfied then a new trial fp2A is used to find a new set of fp1B, fqA and fqB. {Tedious procedure, however may ease the job of optimizer at the end of the design).

Procedure for quick design of the channel filters to set proper passband edge frequencies:

- Design steps of the filter of Channel-A are shown in Figure 17.86. Design / By Synthesis technique is selected with Singly Terminated, Type B options. PBRipple=0.1 dB, passband edge fp=10000 MHz and fq=10940 MHz are typed in. Transmission zeros are typed in as Nzero=1 and NUE=4.

- The transmission zeros are extracted in the order shown in the figure, giving the circuit shown in Figure 17.86.a.

- Using Insert command a unity transformer is inserted at source end which is necessary for the formation of complementary filter pair (Figure 17.86.b-c).

- Using Tune Element command the transformer turns ratio is found which will give the maximum return loss at band center as shown in Figure 17.86.d. The filter is saved as A121.

Figure 17.86. Iterative procedure for determination of the quarter wavelength frequencies and inner band edge frequencies of the interdigtal diplexer.

Figure 17.86. Iterative procedure for determination of the quarter wavelength frequencies and inner band edge frequencies of the interdigtal diplexer.

- The same approach is used to design channel-B filter, now by typing in the frequencies fpB=12120 MHz and fqB=13060 MHz. The resulting filter is saved as  B121 (Figure 17.86.e-f).

- Then a diplexer is formed by inserting B121 into A121 as a Series External 1-Port element (Figure 17.86.g-h). Series connection is needed because the filters present low impedance (SC) outside their passbands.

- Then we plot the close in view of the insertion loss and return loss to see if the cross-over loss and frequency are set correctly. If not, we repeat the whole design by starting with a new fqA. The frequencies fqA=10940 MHz, fpB=12120 MHz and fqB=13060 MHz were found using this approach giving the almost exact 3 dB cross-over loss at fc=12000 MHz.

Design of the channel filters:

After setting the cross-over frequency and loss correctly, we can continue the design as follows:

- The circuit of A121 is loaded (Figure 17.87.a).

- With cursor under the UE’s adjacent to the shunt SC stub the command Kuroda / Distribute Stub is applied  to move part of the shunt SC stub to the other side of the UE. This procedure is repeated until a shunt SC stub is place on both sides of each UE (Figure 17.87.b). Then the command Kuroda / All Equal Stubs is applied to equate the stub impedances (Figure 17.87.c).

- Since this is a narrowband filter impedance scaling is necessary. Also the input impedance must be inverted so that it will present high impedance to other channel filters when paralleled. Both aims can be reached as follows.

- The transformer at load end is shifted to source end by applying the command Shift TRF / LeftMost (Figure 17.87.d). This is necessary because the transformers at the inputs of the channel filters will be tuned to form the complements of the adjacent channel filters.

- Reverse Circuit command is applied to reverse the circuit (Figure 17.87.e).

- The command Inverters / Scale Cct by Inserting Inverters / Specify Stub: Z=70 ohms is applied to scale the shunt SC stubs to 70 ohms (e.g) (Figure 17.87.f).

- Then Reverse Circuit command is applied again to have the transformer at the input of the filter (Figure 17.87.g).

- With cursor under the inverter at input, the command Inverters / Absorb TRF is applied to absorb the transformer into the inverter (Figure 17.87.h). The circuit is saved as A124.

- Finally, the source end inverter is replaced by its quarter wavelength TL equivalent by applying the command Inverters / Replace Inverter by / Quarter Wave TL / fq=10940 MHz (Figure 17.87.i). So, the TL at input of the filter will be the main parameter to be tuned for the formation of complementary channel filters. The filter is saved as  A125.fil.

- The same approach is used to design the filter of Channel-B, B121, with fp=12120 MHz and fq=13060 MHz. The resulting filter with inverter input is saved as B124.fil (Figure 17.87.k). Then its input inverter is converted into quarter wave line equivalent and saved as B125 (Figure 17.87.m).

- First a diplexer is formed by using the filters with inverter inputs, A124 and B124 by connecting them in parallel, as shown in Figure 17.87.n to see if the cross-over frequency is set correctly. We get the expected insertion loss and return loss  responses shown in Figure 17.87.p and q. In this case the two filters have to be connected in parallel because the input inverters invert the input impedances of the filters so that they now present high impedance to each other.

- Then diplexer is formed now using A125 and B125 (Figure 17.87.r) to study the effect of replacing the source end inverters by quarter wave line equivalents.  We see the gross distortion in return loss around the cross-over frequency as shown in Figure 17.87.s. Ths distortion moves down the cross-over loss to about 4 dB (Figure 17.87.t-u). So, we need optimization which will be described next.

Figure 17.87. Formation of interdigital diplexer.

Figure 17.87. Formation of interdigital diplexer.

Figure 17.87. Formation of interdigital diplexer.

Optimization without common resonator:

In Figure 17.88 optimization of the diplexer formed by the filters A125 and B125 is described to improve return loss response. Only impedances of the transmission lines are tuned. We get about 13 dB return loss level with close to 3 dB cross-over loss. The optimized circuits are saved as A126 and B126.

- Optimization with common resonator:

- Next, we insert a common resonator to the input of the diplexer formed by A126 and B126, as shown in Figure 17.89.a-b. We simply Copy an inner shunt SC stub and Paste it to the right of source. Then tune its length while observing the return loss to get a balanced return loss level in both channel passbands. Next, we tune the impedance of the common resonator to study its effect. Figure 17.89.c shows the return loss responses without common resonator, with 70 ohms common resonator and with 30 ohms common resonator. It is seen that matching bandwidth gets wider with better return loss as the impedance of common resonator is decreased.

- We make a trial optimization with 70 ohms common resonator impedance. We get about 16 dB return loss, as shown in Figure 17.89.e, within a band from f1=10050 MHz to f2=13950 MHz. Remembering that in the previous example we have got about 14 dB return loss level within a band of f1=10150 MHz to f2=13850 MHz, from a similar circuit obtained by LP prototype approach, we note that the direct synthesis method of this example is advantageous. So, it may worth to use the tedious iterative approach for setting the band edge frequencies. The main difference between the final circuit of the previous example and this example seems to be the difference between the commensurate lengths because of different quarter wavelength frequencies.

- We note that there is a shift of about 15 MHz in the cross-over frequency (Figure 17.89.g).

Figure 17.88. Optimization without common resonator.

Figure 17.89. Optimization with common resonator.

Figure 17.89. Optimization with common resonator.

# EXAMPLE 17. 37: Diplexer design using doubly terminated interdigital filters

Let’s design the contiguous diplexer of the previous example now using doubly terminated interdigital filters just to compare the approaches. Observations may be generalized to triplexers and  noncontiguous diplexers. Determination of the band edges of the doubly terminated filters need some iterative trials. Let’s take them the same as in the previous example:

Channel-A: fpA= 10000 MHz, fqA=10940 MHz.

Channel-B: fpB= 12120 MHz, fqB=13060 MHz.

In order to make a fair comparison the return loss and insertion loss responses of the singly terminated and doubly terminated cases should be the same. Determination of the correct passband ripple of the doubly terminated case which corresponds to the 0.1 dB ripple of the singly terminated case necessitates trials. 0.1 dB ripple of the singly terminated case gives roughly 27 dB return loss. For doubly terminated case we will take the ripple as 0.01 dB which gives about 26 dB. So, we expect that this ripple will lead to a response which is close to that of the singly terminated case. Transmission zeros will be taken as Nzero=1, Ninf=4  to get five resonator interdigital filters.

- Design steps of the filter of Channel-A are shown in Figure 17.90. Doubly Terminated and Type B options are selected. The synthesized circuit is shown in Figure 17.90.a. With cursor under the UE’s adjacent to the shunt SC stub the command Kuroda / Distribute Stub is applied to move part of the shunt SC stub to the other side of the UE. This procedure is repeated until a shunt SC stub is place on both sides of each UE (Figure 17.90.b).

- Then the command Kuroda / All Equal Stubs is applied to equate the stub impedances (Figure 17.90.c).

- Resonators (shunt SC stubs) are scaled to 70 ohms by applying the command Inverters / Scale Cct by Inserting Invs / Specify SC stub: Z=70 ohms (Figure 17.90.d).

- Then the source end inverter is converted into TL by applying the command Inverters / Replace Inverter by / Quarter Wave TL: fq=10940 MHz (Figure 17.90.e). The circuit is saved as A131.

- The same approach is used to design the filter of Channel-B with fp=14000 MHz and fq=15000 MHz. The resulting filter is saved as B132 (Figure 17.90.f).

- Then two diplexer circuits are formed by inserting Channel-B filter into Channel-A and vice versa as External 1-Port, as shown in Figure 17.90.g.

- Only impedances of the TL’s of the two diplexer circuits are tuned while observing the return loss until the return loss gets the best possible value in each channel. Figure 17.90.h and I show return loss before and after optimization. We get about 11 dB return loss level. The optimized circuits are saved as A131 and B131 respectivel (Figure 17.90.k).

- Next, a common resonator is formed by copying a shunt SC stub and pasting it at the input of the diplexer (figure 17.90.m).  First the length of the common resonator is tuned to get balanced return loss in both passbands. For the time being its impedance will be left at 70 ohms.

- Impedance of the lines are optimized to give a 14 dB return loss level (Figure 17.90.n) which is the same as the return loss level we have got in the singly terminated case.  Return loss can be improved to more than 18 dB by setting the common resonator to about 30 ohms as shown in Figure 17.90.n.

- Zoomed insertion loss response shows that the cross-over frequency is shifted to about 11965 MHz, with about 3.18 dB loss (Figure 17.90.p).

- Figure 17.90.q shows widespan insertion loss responses of the singly terminated and doubly terminated diplxers. It is seen that they are close to each other. Hence we get almost the same circuits for the sinly terminated PBRiple=0.1 dB and doubly terminated PBRipple=0.01 dB. Figure 17.90.r and s show the element values of both singly terminated and doubly terminate cases for comparison. Element values are close to each other as expected.

In conclusion, we have seen that interdigital diplexers and multiplexers can be designed in several different ways, each one having slight advantages over the others.

Figure 17.90. Contiguous diplexers formed by doubly terminated interdigital filters.

Figure 17.90. Contiguous diplexers formed by doubly terminated interdigital filters.

Figure 17.90. Contiguous diplexers formed by doubly terminated interdigital filters.

Figure 17.90. Contiguous diplexers formed by doubly terminated interdigital filters.

Determination of the even and odd mode impedances:

The even and odd mode impedances of the adjacent coupled lines can be determined as described in Figure 17.91 on the Channel-A filter.

- Diplexer-1 (DXAB) is loaded and the external 1-Port (B03) is deleted (Figure 17.91.a-b).

- Length of the common resonator is equated to the commensurate length of this channel using Change command. This is necessary for the formulations in Filpro to work properly for finding even and odd mode impedances. This change introduces an error that can be corrected by slight tuning in the final circuits.

- Then with cursor under each TL, one by one, the command Replace by PCL / Interdigital PCL / scscINTD / Symmetric is applied to convert the triplets of SC stub+TL+SC stub  into PCL form (Figure 17.91.c). Since each shunt SC stub is shared by two adjacent TL’s, this conversion should be done one by one and Undo command should be applied to recover the original circuit on which the next pair of SC stub will be converted.  Figures 17.91.d and e shows the resulting even and odd mode impedances.

- The load end inverters can be converted into different practical equivalents. For example in Figure 17.91.e-f, conversion into Tapped PCL form is described (See also CH-12 on tapping).

- The Channel-B filter can be treated in the same way.

- Figure 17.91.g shows schematics for a possible realization.

Figure 17.91. Determination of even and odd mode impedances of the contiguous diplexer.