17. 23. COMBLINE BP-BP DIPLEXERS AND MULTIPLEXERS

Let’s repeat the same diplexer and multiplexer examples of the previous section now using combline filters instead of interdigital filters.

EXAMPLE 17. 38. Contiguous combline diplexer design using LP-HP diplexer prototype

Let’s use a singly terminated LP prototype filter to design channels of a contiguous diplexers, triplexers and quadruplexers. The LP prototype will be obtained as the LP arm of the LP-HP diplexer, LPP01 that was designed in the previous section. The multiplexer channels have the following specifications:

Channel-A: fp1=10000 MHz, fp2=12000 MHz.

Channel-B: fp1=12000 MHz, fp2=14000 MHz.

Channel-C: fp1=14000 MHz, fp2=16000 MHz.

Channel-B: fp1=16000 MHz, fp2=18000 MHz.

We will use a common quarter wave frequency for all channels: fq=20000 Mz. We will use the LP prototype LPP01 of degree N=5 with PBRipple 0.1 dB.

Design of the channel filters:

Design steps of Channel-A filter is described in Figures 17.92.a-to-h.

- Design / By User Defined Prototype option is selected. Load a Prototype command is invoked and the prototype LPP01 is typed in. Filter Type / Distributed-BPF options are selected.

- BP Filter edge frequency fp1=10000 MHz and fp2=12000 MHz (for Channel-A) are typed in.

- Find Circuit command gives the all stub filter shown in Figure 17.92.a. Topology of the circuit and element values need changes as follows. Find Dual command is applied to form the dual circuit which starts with shunt SC+OC stub resonators (Figure 17.92.b).

- The series arm SC+OC stub resonators are converted into shunt SC+OC stub resonators by applying the command Inverters / Replace by Inv Eqv with K=50 ohms (Figure 17.92.c). Simplify Circuit command eliminates the phase changing transformers (Figure 17.92.d).

- Then with cursor under any SC+OC stub resonator, the command Replace by Lumped / All SC Stub+Cap is applied to convert the OC stubs of all OC+SC Stub resonators into lumped capacitors (Figure 17.92.e).

- With cursor under the inverters the command Inverters / Replace Inv by / Pi-Tee Type / SC Stub Pi is applied to replace the inverters by their SC stub Pi sections with negative shunt SC stub arms (Figure 17.92.f). Simplify Circuit command combines the negative elements with adjacent positive elements (Figure 17.92.g).

- The command Pi-Tee-L/ Equal Shunt SC stubs is applied to equate the impedances of shunt SC stubs (Figure 17.92.h).

- The transformer at load end is shifted to source end because it is needed at that end for proper operation of the singly terminated filter (Figure 17.92.i).

- Reverse Circuit command is applied to reverse the circuit for adjustment  of impedances of the SC stubs (Figure 17.92.k). Reversal is needed because impedance scaling is done on the element adjacent to the source.

- Next, impedances of the shunt SC stubs will be adjusted to realization level, for example Z=70 ohms. This is done by applying the command Inverters / Scale Circuit by Inverters / Specify Adjacent SC Stub: K= 70 ohms (Figure 17.92.m).

- After scaling the circuit by inverters, the circuit is reversed again. (Figure 17.92.n).

- The transformer adjacent to the source end inverter is absorbed into the inverters by using the command Inverters / Absorb TRF (Figure 17.92.p). The Channel-A filter is ready. It is saved as A151.

- The same approach is used to design the other channel filters which are saved as  B151, C151 and D151 respectively (Figure 17.92.q,r,s). We are now ready to form multiplexers.

Figure 17.92. Design of combline multiplexer channel filters using LP prototype.

Figure 17.92. Design of combline multiplexer channel filters using LP prototype.

Formation of a contiguous combline diplexer:

Let’s form a contiguous diplexer by connecting the filters A151 and B151 in parallel.

- Two diplexer circuits are formed by inserting B151 into A151 and vice versa, as shunt External 1-Port element as shown in Figure 17.93.a-b.

- Then the impedances of the series SC stubs, the inverters and capacitors of the shunt resonators are tuned using Monte Carlo-Best Case optimization routine and occasionally Tune Element command, by loading the two circuits alternately to improve the return loss.

- Figure 17.93.c shows the responses before tuning. It is seen that matching is very good around the cross-over frequency, degrading towards the edges of the passband. The good match at cross-over frequency of fc=12000 MHz makes the cross-over loss almost exactly 3 dB.

- After optimization the return loss is improved to about 16 dB within a bandwidth from f1=10100 MHz to f2=13900 MHz, as shown in Figure 17.93.d.

- The band shrinkage is due to replacement of inverters with their approximate equivalents.

- The cross-over frequency is shifted by about 60 MHz which necessitates further adjustment.

- As will be shown later response can be improved by continuing optimization trials. The optimized diplexer circuits are shown in Figure 17.93.e.

- The filters are saved as A152 and B152 respectively.

Figure 17.93. Formation of a contiguous diplexer.

Formation of a contiguous combline triplexer:

Let’s form a contiguous triplexer by connecting the filters A151, B151 and C151 in parallel.

- Three triplexer circuits are formed by inserting two channels into the third one as shunt External 1-Port element alternately as shown in Figure 17.94.a-b.

- Then the impedances of the series SC stubs, the inverters and capacitors of the shunt resonators are tuned using Monte Carlo-Best Case optimization routine and occasionally Tune Element command by loading the three circuits alternately to improve the return loss.

- Figure 17.94.c shows the responses before tuning. It is seen that matching is very good in Channel-B because of the existence of complementing filters on the two sides. Return loss of the outer channels need improvement.

- After optimization the return loss is improved to 16.5 dB within a bandwidth from f1=10100 MHz to f2=15900 MHz, as shown in Figure 17.94.d. Further optimization trials will improve the response, as will be shown later, after studying the doubly terminated approach.

- The band shrinkage is due to replacement of the inner inverters by their approximate equivalents. The cross-over frequencies are shifted slightly.

- Optimized diplexer circuits are shown in Figure 17.94.e. They are saved as A153, B153 and C153 .

Figure 17.93. Formation of a contiguous diplexer.

Formation of a contiguous combline quadruplexer:

Let’s form a contiguous quadruplexer by connecting the filters A151, B151, C151 and D151 in parallel.

- Four quadruplexer circuits are formed by inserting three filters into the fourth one alternately as shunt External 1-Port element as shown in Figure 17.95.a-b.

- Then the impedances of the series SC stubs, the inverters and capacitors of the shunt resonators are tuned using Monte Carlo-Best Case optimization routine and occasionally Tune Element command by loading the three circuits alternately to improve the return loss.

- Figure 17.95.c shows the responses before tuning. It is seen that matching is very good in Channel-B and Channel-C because of the existence of complementing filters on the their two sides. Return loss of the outer channels need improvement.

After optimization the return loss is improved to 16 dB within a bandwidth from f1=10200 MHz to f2=17800 MHz, as shown in Figure 17.95.d. The band shrinkage is due to replacement of inverters with approximate equivalents. The cross-over frequencies are shifted slightly. Response can be improved much more as will be shown later, by using doubly terminated approach. The problem of the singly terminated filters is that the element values are too far from the global optimum, thus necessitating more powerful optimizers.  The optimized diplexer circuits are shown in Figure 17.95.e. They are saved as A154, B154,  C154 and D154 respectively.

Figure 17.94. Formation of a contiguous triplexer.

Figure 17.94. Formation of a contiguous triplexer.

Figure 17.95. Formation of a contiguous quadruplexer.

Figure 17.95. Formation of a contiguous quadruplexer.

EXAMPLE 17. 39. Contiguous combline diplexer design using singly terminated BP filters

- In order to compare different approaches for designing diplexers and multiplexers, let’s redesign the diplexer of the previous example by synthesizing the channel filters directly as singly terminated BP filters instead of using the LP prototype. We expect that this approach would give better results because of its exact nature avoiding the inner inverters of the previous approach. The disadvantage of this approach is that we don’t know the actual inner passband edge frequencies, fp2A and fp1B (Figure 17.96) of the two filters on the two sides of the cross-over frequency fc. We have to determine them by trials. Since fc is fixed we will assign trial fp2A and fp1B and make a design to check if the desired cross-over frequency and loss is satisfied or not. Since the inner side slopes of the two filters are different we don’t have the chance of selecting f2A and f1B symmetrically around fc. This fact makes the search for these frequencies slightly more tedious. Problem would of course be eased if a more professional optimizer is available.

- Figure 17.96. Asymmetry in the inner passband edges with respect to cross-over frequency.

- After a few complete design cycles, the passband edge frequencies are set as follows:

fp1A= 10000 MHz, fp2A= 11940 MHz for channel-A

and

fp1B= 12110 MHz, fp2B= 14000 MHz for channel-B.

The inner band edges fp2A=11940 MHz and fp1B=12110 MHz are found after a few trials to get the 3 dB cross-over frequency at about 12000 MHz. The common quarter wavelength frequency is taken as fq=20000 MHz.

Five resonator combline filters are targeted. Therefore the transmission zeros will be Nzero=1 and Ninf=9.

Let’s take PBRipple=0.1 dB, the same as in the previous approach.

Diplexer design using singly terminated filters:

#### Design steps of Channel-A filter are shown in Figure 17.97. Singly Terminated option is selected. After typing in Nzero=1 and Ninf=9, the transmission zeros are extracted using Standard Auto Extract command. The resulting circuit is reversed giving the circuit shown in Figure 17.97.a.

- Pi-Tee-L / Equal Shunt SC Stubs is applied to convert the circuit into coupled resonator form with SC+OC Stub type shunt resonators (Figure 17.97.b).

- Then with cursor under any shunt SC+OC stub resonator, the command Replace by Lumped / All SC Stub+Cap  is applied to convert the OC stubs of resonators into lumped capacitors (Figure 17.97.c). This operation destroys equality of the shunt SC stubs. This property is recovered by applying  Pi-Tee-L / Equal Shunt SC Stubs again (Figure 17.97.d).

- The transformer at load end will be used to form the complementary pairs after forming the diplexer. Therefore we shift it to source end (diplexing end) by applying the command Shift TRF / LeftMost (Figure 17.97.e).

- Next, we will scale the impedances of the shunt SC stubs. For this purpose we Reverse the circuit to make the load end SC Stub+Cap resonator adjacent to source (because scaling is done on the source side elements) (Figure 17.97.f).

- Then the command Inverters / Scale Cct by Inv / Specify SC Stub: Z=70 ohms is applied to scale the shunt SC stubs to 70 ohms (e.g.) (Figure 17.97.g).

- Then the circuit is reversed again to move the transformer to source side again (Figure 17.97.h).

- The transformer is absorbed into the source side inverter by applying the command Inverters / Absorb TRF by Inv giving the circuit of Figure 17.97.i. It is saved as  A160.

- The same procedure is applied to design the Channel-B filters. It is saved as  B160 (Figure 17.97.k).

- Next, the two filters are connected in parallel to form the diplexer circuits.  Since the source end inverters  of the two filters are not adjusted yet, the return loss of the diplexer will be heavily degraded. A diplexer circuit is formed by loading A160 and inserting B160 as External 1-Port (Figure 17.97.m-n). Using Tune Element command we tune the source end inverter of A160 while observing the return loss of the diplexer. Then the source end inverter of B160 is tuned using tuned Plot-Tune Multiplexer command. After a few iterations we get the circuits and responses shown in Figure 17.97.p. Thus, two singly terminated filters became complements, as the return loss at the cross-over frequency is high enough. The cross-over loss is 3 dB at fc=12000 MHz. The tuned filters are saved as A161 and B161 respectively.

- Next, we tune all elements of both filters by forming two diplexer circuits, one by inserting B161 into A161 and the other by inserting A161 into B161 as External 1-Port. (Figure 17.97.q).

- Both Tune Element and Monte Carlo-Best Case routine are used in the optimization. We get about 14.5 dB return loss over the complete band, from f1=10000 MHz to f2=14000 MHz. This level is close to what we have got in the LP Prototype approach of the previous example.

- We note that the cross-over frequency is shifted from 12 000 MHz to 11983 MHz (Figure 17.97.r).

- Figure 17.97.s shows the optimized circuits.

- The optimized diplexer circuits of the previous example which were designed using LP prototype approach are also shown for comparison with the new diplexer circuits (Figure 17.97.s). The element values are not identical, but not too far either. The differences are due to inverter approximations and also due to narrower bandwidth of the diplexer of the LP prototype approach.

Figure 17.97. Combline diplexer design by synthesizing channels as singly terminated BP filters.

Figure 17.97. Combline diplexer design by synthesizing channels as singly terminated BP filters.

Figure 17.97. Combline diplexer design by synthesizing channels as singly terminated BP filters.

Figure 17.97. Combline diplexer design by synthesizing channels as singly terminated BP filters.

Combline diplexer with common resonator:

We will investigate common resonator structures after studying the doubly terminated diplexer. As a preparation for comparison of different approaches, let’s form a diplexer with common resonator, using the singly terminated designs of this example.

- In Figure 17.98.a the diplexer circuit with filters A161 and B161 is shown, with A161 appearing explicitly and B161 as External 1-Port element. In order to form a common resonator, we Copy an inner resonator and Paste it to the diplexing end (Figure 17.98.b).

- The capacitor of the common resonator is tuned to get a balanced return loss response in both channels (Figure 17.98.c).

- Then all elements of  A161 are tuned except the 70 ohms SC stubs using both Tune Element and Monte Carlo-Best Case optimization routine while observing the return loss of Channel-A. The tuned A161 is saved as A168.

- Then B161 is loaded, the tuned common resonator and A168 are inserted. Now optimization is applied on this circuit. The optimized B161 is saved as B168.

- Then B168 is inserted into A168 and vice versa to optimize the return loss response further. This cycle is repeated until return loss gets its best level.

- Figure 17.98.d shows the return loss before optimization. We notice that insertion of the common resonator improved the return loss somewhat.

- Figure 17.98.e shows the optimized return loss. We got about 22 dB level. Further optimization may improve this level (Exercise?). The 3 dB cross-over frequency is almost exact. 12003 MHz.

- The optimized circuits are shown in Figure 17.98.f, for comparison with other designs.

# Figure 17.98. Combline diplexer with common resonator using singly terminated BP filters.

Figure 17.98. Combline diplexer with common resonator using singly terminated BP filters.

# EXAMPLE 17. 40. Contiguous combline diplexer design using DOUBLY TERMINATED BP filters

Let’s design the diplexer of the previous example now by using doubly terminated channel filters, for comparison of the approaches. In order to get the exact correspondence between singly terminated and doubly terminated approaches we have to search for the PBRipple of the doubly terminated filters that would match to the PBRipple=0.1 dB of the singly terminated approach. On Filpro, this is possible only by trials. In the case of interdigital filters we had tried PBRipple=0.01 dB for doubly terminated approach to get the same response as the singly terminated diplexer filters with PBRipple=0.1 dB. We couldn’t get exactly the same response but got rather close to the target. Just as a trial, let’s now take PBRipple=0.02 dB for the doubly terminated case to see if we can get closer to the response of the diplexer with singly terminated filters with PBRipple=0.1 dB.

We will take Nzero=1, Ninf=9 to get a five resonator combline filter.

Determination of the inner passband edge frequencies fp2A and fp1B needs trials, as in the case of singly terminated filters. They are selected to be the same as in the singly terminated approach of the previous example as fp2A=11940 MHz and fp1B=12110 MHz.

Design steps are described in Figure 17.99:

- Channel-A filter is designed first. Distributed Element, Equiripple PB, Doubly Terminated, Type B options are selected. The passband Transmission zeros are placed as Nzero=1 and Ninf=9 and Standard Auto Extract command is used to extract the transmission zeros giving the circuit of Figure 17.99.a.

- Pi-Tee-L / Equal Shunt SC Stubs command is applied to distribute the single shunt SC stub into the circuit with equal impedances forming five shunt SC Stub+OC Stub resonators (Figure 17.99.b).

- The shunt OC stubs are approximated by lumped capacitors by applying the command Replace by Lumped / All SC Stub+Cap (Figure 17.99.c). This conversion keeps the band edges of the filter unaffected.

- Conversion of the OC stubs into lumped capacitors will destroy the equal shunt SC stub property. This feature is recovered by applying the command Pi-Tee-L / Equal Shunt SC Stubs once more (Figure 17.99.d).

- Finally inverters are inserted at source and load ends to scale up the shunt SC stub resonator impedances to 70 ohms (e.g.) by applying the command Inverters / Scale Cct by Inserting Inverters / Specify SC stub: Z=70 ohms. It is saved as A170.fil  (Figure 17.99.e).

- The filter of Channel-B is also designed in the same way and saved as B170.fil (Figure 17.99.f).

- Next, the two filters are connected in parallel to form the diplexer circuits.  We form two diplexer circuits. In one, we load A170 and insert B170 as External 1-Port (Figure 17.99.g). Since the two filters are designed independently, the return loss of the diplexer will be heavily degraded, especially around the cross-over frequency (Figure 17.99.h). It will be improved by tuning elements of both channels. In this circuit we tune the elements of A170 while observing the return loss of the diplexer. The tuned A170 is saved as A171. Then, we load B170 and insert A171 as External 1-Port and tune the elements of B170. The tuned B170 is saved as B171. Return loss is optimized by loading the two diplexer circuits alternately until the best possible return loss level is obtained.

- We get the return loss response shown in Figure 17.99.i which is settled at about 25 dB, a level which is much larger than in the case of the singly terminated diplexers where we have got about 15-16 dB.

- Insertion loss response shows negligible shift in the cross-over frequency (Figure 17.99.k).

- Figure 17.99.m shows the wide span insertion loss reponses of both singly terminated diplexe with PBRipple=0.1 dB and the doubly terminated diplexer of this example, with PBRipple=0.02 dB. We got almost exact overlapping. So, we can say that the singly terminated diplexer with PBRipple=0.1 dB corresponds to the doubly terminated diplexer with PBRipple=0.02 dB for the given bandwidth and degree.

- Figure 17.99.n shows the optimized diplexer circuits, A171 and B171.

- Figure 17.114.p shows the singly terminated versions, A162 and B162 of the diplexer circuits found in the previous example. Comparison of these singly and doubly terminated filter element values shows that they are too far from each other, as the 10 dB difference between return loss levels also implies. Let’s speculate on this point.

Remarks: Singly Terminated or Doubly Terminated?

- We wonder the reasons why we have got such a large difference between return loss responses of the singly terminated and doubly terminated diplexers. In order to be sure on the results, in the previous two examples, the singly terminated diplexers were designed in two different ways, one being mapping from LP prototype and the other direct BP design. Both approaches yield (almost) the same circuits after optimization, with about 15-16 dB return loss level. One would expect that doubly terminated design would also converge to the same circuit and same response. However we have got 25 dB return loss level. The only reason we can think of is that the optimization trials on singly terminated cases stuck at some local optimum while optimization on doubly terminated diplexer reached to global optimum quickly.

- At this point we make a small cheating : We copy the source end inverter values of the optimized doubly terminated filters and assign these values to the source end inverters of the singly terminated filters. Then restart the optimization of the singly terminated diplexer. We got the same 25 dB return loss level as shown in Figure 17.99.q !!!. The resulting singly terminated filters are saved as A163 and B163 (Figure 17.99.r).

- In Figure 17.99.s the element values of doubly terminated diplexer are also shown for comparison. They are very close to each other.

Conclusion: On Filpro, the design of contiguous combline diplexers without common resonator is easier by using doubly terminated approach because the optimization catches the global optimum in a quicker way. The optimization trials on singly terminated approach may have difficulty to reach the global optimum because their starting values are too far from the final optimum values.

Figure 17.99. Contiguous combline diplexer design using doubly terminated filters.

Figure 17.99. Contiguous combline diplexer design using doubly terminated filters.

Figure 17.99. Contiguous combline diplexer design using doubly terminated filters.

Figure 17.99. Contiguous combline diplexer design using doubly terminated filters.

Formation of diplexer with common resonator:

We had already designed a diplexer with common resonator by inserting a common resonator into the singly terminated filters in the previous example and we have got about 22 dB return loss level (Figure 17.114). Let’s now design a diplexer with common resonator by using the optimized contiguous diplexer formed by doubly terminated approach, A171 and B171 by inserting a common resonator. The procedure is described in Figure 17.100.

- In Figure 17.100.a the diplexer circuit is shown formed by the filter A171 with B171 appearing as External 1-Port. We Copy any resonator and Paste at the input of the diplexer to use as common resonator (Figure 17.100.b). The second diplexer circuit is formed in similar way, by inserting A171 into B171 as External 1-Port (Figure 17.100.c).

- Then we tune the capacitor of the common resonator to get balanced return loss response in both channels (Figure 17.100.d). As should be expected, the return loss response is degraded by this brute force insertion of the common resonator.

- The return loss is recovered by tuning all capacitors, all series SC stubs and the inverters. We get the response shown in Figure 17.100.e where return loss is settled at about 25 dB level. Figure 17.100.f show the optimized circuits which are saved as  A180 and B180.

- For comparison, the common resonator diplexer circuits obtained by starting with singly terminated filters are shown in Figure 17.100.f  which gave 22.5 dB return loss level.  It is seen that by using the original singly terminated designs, we also got very close to the global optimum.

Conclusion: Both singly terminated and doubly terminated filters can be used to get the same common resonator diplexer. However singly terminate designs necessitates more tuning-optimization efforts on Filpro.

- In the next stage the diplexing end inverters of the two channels will be replaced by their targeted practical equivalents.

Figure 17.100. Formation of a common resonator diplexer using doubly terminated filters.

Figure 17.100. Formation of a common resonator diplexer using doubly terminated filters.

Interdigital or comb type common resonator feeds?

Up to now, we have left the source and load end inverters of the diplexers as they are. Let’s now replace them with some possible realizable equivalents.

The load end inverters can be replaced by different practical equivalents, like tapping, capacitive or inductive coupling, etc.

For the diplexers with common resonators, the source end (diplexing end) inverters represent couplings between the common resonator and the first resonators of each channel. That is, the common resonator and the first resonators of the channels will form parallel coupled lines (PCL). Two approaches are possible: Interdigital type common resonator feed (with adjacent resonators having SC ends on the opposite side with the common resonator) (Figure 17.101.a) or comb type common resonator feed (with adjacent resonators having SC ends on the same side with the common resonator) (Figure 17.101.b). Let’s inspect the two approaches separately:

Figure 17.101. Interdigital and comb type PCL diplexing elements.

Common resonator diplexer with opposite side SC ends (Interdigital type common resonator):

- We load the circuit A180 (Figure 17.102.a). With cursor under the source end inverter, the command Inverters / Replace Inverter by / SSsh+TL+SSsh is applied to convert the inverter into a TL piece with negative shunt SC stubs (SSsh) on its two sides (Figure 17.102.b).

- The negative shunt SC stubs could be paralleled with the shunt SC stubs of the neighboring resonators. However this would change the 70 ohms impedances of these resonators. Therefore we simply Delete the negative SC stubs. This will give extra distortion in return loss which can be compensated to some extent by tuning the capacitor of the adjacent resonator. The circuit is saved as A181 (Figure 17.102.c).

- The same procedure is repeated on B180 and the resulting circuit is saved as B181 (Figure 17.102.d). Then two diplexer circuits are formed and the common resonator is inserted (Figure 17.102.e). Figure 17.102.f shows a realization schematics describing the opposite side SC ends of the resonators on the two sides of the common resonator.

- Then the return loss response is optimized using both Tune Element and Monte Carlo-Best Case routine. We get greater than 26 dB return loss level as shown in Figure 17.102.g. The increase in return loss level may be attributed to better making use of the common resonator (?).

Figure 17.102. Formation of the combline diplexer with opposite side SC ends (interdigital type common resonator input).

Figure 17.102. Formation of the combline diplexer with opposite side SC ends (interdigital type common resonator input).

Comb type common resonator (common resonator diplexer with same side SC ends):

- We load the circuit A180 (Figure 17.103.a). With cursor under the source end inverter, the command Inverters / Replace Inverter by / End Inverter to / SC Stub L is applied to convert the inverter into an L-section of SC Stubs with negative shunt SC stub (Figure 17.103.b).

- The negative shunt SC stubs could be paralleled with the shunt SC stub of the adjacent resonator. However this would change the 70 ohms impedance of this resonators. Therefore we simply Delete the negative SC stubs. This will give extra distortion in return loss which can be compensated to some extent by tuning the capacitor of the adjacent resonator. The circuit is saved as A185 (Figure 17.103.c). The same procedure is repeated on B180 and the resulting circuit is saved as B185 (Figure 17.103.d). Then two diplexer circuits are formed and common resonator is inserted (Figure 17.103.e). Figure 17.103.f shows a realization schematics showing the SC ends of the resonators on the two sides of the common input resonator.

- Then the return loss response is optimized using both Tune Element and Monte Carlo-Best Case routine. We get about 19 dB return loss level as shown in Figure 17.103.g. This is about 7 dB less than the return loss level we have obtained by using interdigital type (opposite side SC ends) common resonator case.

Conclusion: Interdigital type common resonator feed structure leads to better match compared to comb type common resonator feed over the same bandwidth.

Figure 17.103. Formation of the combline diplexer with opposite side SC ends (interdigital type common resonator input).

Figure 17.103. Formation of the combline diplexer with same side SC ends (comb type common resonator input).

Tee-Junction realization of diplexers:

- The inverters at diplexing ends can be replaced by different practical equivalents for realization in different forms.  The common resonator input feed was one of the several solutions for parallel connection of the two filters. Another way to connect the two channel filters in parallel is to replace the source end inverters with TL’s which are quarter wavelength at the respective band centers. Let’s consider the doubly terminated filters A171 and B171 (without common resonators!).

- The circuit of A171 is loaded (Figure 17.104.a). With cursor being under the source end inverters the command Inverters / Replace Inverter by / Quarter Wave TL / f0=11000 MHz is applied. Here f0 is the passband center of channel filters. Impedance of the resulting TL will be the same as the inverter impedance. If it is too high or low it may also be adjusted after interconnection, through an optimization process. The resulting filter is saved as A177 (Figure 17.104.b).

- Then the circuit of B171 is loaded (Figure 17.104.c) and with cursor being under the source end inverters the command Inverters / Replace Inverter by / Quarter Wave TL / f0=13000 MHz is applied. The resulting filter is saved as B177 (Figure 17.104.d).

- Then two diplexer circuits are formed and elements of the filters are tuned to improve the return loss response (Figure 17.104.e).

- Return loss before optimization is shown in Figure 17.104.f.

- Return loss response is improved to about 23 dB with negligible shift in cross-over frequency (Figure 17.104.g).

- Figure 17.104.h shows the optimized circuits which are saved as  A178 and B178 respectively.

- Figure 17.104.i shows a realization schematics. Note that the two TL pieces have different lengths.

Figure 17.104. Parallel connection of combline channel filters through transmission lines.

Figure 17.104. Parallel connection of combline channel filters through transmission lines.

Parallel connection of diplexer filters through series capacitor (inductor) input/output couplings:

It is possible to use capacitive or inductive input couplings for the diplexers. In Figure 17.105 capacitive input couplings are described:

- The circuit of A171 is loaded (Figure 17.105.a). With cursor being under the source and load end inverters the command Inverters / Replace Inverter by / End Inverter to / Capacitive-L / f0=… is applied. The inverter will be replaced by its capacitive L-section equivalent with negative shunt capacitor.

- Simplify Circuit command combines the negative capacitor with the capacitor of the adjacent resonator. The resulting circuit is saved as  A220. The same procedure is applied on B171 and resulting circuit is saved as B220 (Figure 17.105.b).

- Then the two diplexer circuits are formed and elements of the filters are tuned to improve the return loss response (Figure 17.105.c-d). Only the capacitors and the series SC stubs are tuned to keep the shunt SC stub impedances unchanged at 70 ohms.

- Return loss response is recovered at about 16 dB after a rough optimization (Figure 17.105.e). Figure 17.105.f shows the optimized diplexer circuits. Figure 17.105.g shows a realization schematics.

Figure 17.105. Parallel connection of combline channel filters through series input capacitors.

Figure 17.105. Parallel connection of combline channel filters through series input capacitors.

Figure 17.105. Parallel connection of combline channel filters through series input capacitors.

#### EXAMPLE 17. 41. Contiguous combline quadruplexer design using singly terminated BP filters

#### In Example 17.38 we had designed a quadruplexer by using the filters which were designed from LP prototypes which necessitated the use of inverters to convert the series resonators into shunt resonators. The replacement of inverters led to band shrinkage. Also we had stopped tuning-optimization trials after reaching at about 16 dB of return loss level. In this example we will repeat the design of the quadruplexer now by using direct BP design of the singly terminated filters.

#### The advantage of singly terminated filters is that it is easier to form complementary adjacent pairs. When more than two contiguous channels are involved then the internal channels would have complementing neighbors on both sides, leading to good return loss. The singly terminated outside channels will be badly mismatched because of the absence of complementing channels. Therefore it may be easier to use doubly terminated filters for the outside channels, as will be described in the next example.

#### Specifications of the channels are as follows:

#### Channel-A: f1=10000 MHz, f2=12000 MHz. We will take fp1=10000 MHz, fp2=11750 MHz.

#### Channel-B: f1=12000 MHz, f2=14000 MHz. We will take fp1=12250 MHz, fp2=13750 MHz.

#### Channel-C: f1=14000 MHz, f2=16000 MHz. We will take fp1=14250 MHz, fp2=15750 MHz.

#### Channel-D: f1=16000 MHz, f2=18000 MHz. We will take fp1=16250 MHz, fp2=18000 MHz.

#### For the inner passband edge frequencies we have used some trial values. They may need iterative trials. The other specifications are as follows:

#### PBRipple= 0.1 dB,  Nzero=1, Ninf=9 so five resonators will be used in each filter.

#### Design steps of Channel-A filter are shown in Figure 17.106. Singly Terminated option is selected. After typing in Nzero=1 and Ninf=9, the transmission zeros are extracted using Standard Auto Extract command. The resulting circuit is reversed giving the circuit shown in Figure 17.106.a.

- Pi-Tee-L / Equal Shunt SC Stubs is applied to convert the circuit into coupled resonator form with SC+OC Stub type shunt resonators (Figure 17.106.b).

- Then with cursor under any shunt SC+OC stub resonator, the command Replace by Lumped / All SC Stub+Cap  is applied to convert the OC stubs of resonators into lumped capacitors (Figure 17.106.c). This operation destroys equality of the shunt SC stubs. This property is recovered by applying  Pi-Tee-L / Equal Shunt SC Stubs again (Figure 17.106.d).

- The transformer at load end will be used to form the complementary pairs after forming the diplexer (multiplexer). Therefore we shift it to the source end (diplexing end) by applying the command Shift TRF / LeftMost (Figure 17.106.e).

- Next, we will scale the impedances of the shunt SC stubs. For this purpose we Reverse the circuit to make the load end SC Stub+Cap resonator adjacent to source (because scaling is done on the source side elements) (Figure 17.106.f).

- Then the command Inverters / Scale Cct by Inv / Specify SC Stub: Z=70 ohms is applied to scale the shunt SC stubs to 70 ohms (e.g.) (Figure 17.106.g).

- Then the circuit is reversed again to move the transformer to source side again (Figure 17.106.h).

- The transformer is absorbed into the source side inverter by applying the command Inverters / Absorb TRF by Inv giving the circuit of Figure 17.106.i. It is saved as  A70.fil.

- The same procedure is applied to design the other channel filters. They are saved as  B70, C70 and D70 respectively (Figure 17.106.k-m-n).

Formation of a quadruplexer with inverter terminations:

- A quadruplexer circuit is formed by loading A70 and inserting the other channels as External 1-Port elements (Figure 17.106.p-q). In this quadruplexer, first only the source side inverters of all  filters are tuned to form complementary pairs while observing the return loss. The design of complementary filters is finished after this tuning procedure which is actually equivalent to determination of the transformer turns ratio of each filter which are absorbed into these inverters.

- Tuning of the inverter of the explicitly displayed channel (A70) is done by using Tune Element command. The source end inverters of other channels are tuned by using Plot-Tune Multiplexer command. This command displays the circuit of the pointed External 1-Port element. Then we move the cursor to point the element to be tuned (source end inverter) and tune it by up-down arrows. The tuned value of the elements of External 1-Port circuits CAN NOT be saved. Therefore we have to note down the tuned values and type them into the individual circuits  by loading them one by one.

- After formation of the complementary pairs by tuning source end inverters, we get the return loss response shown in Figure 17.106.r. The improved return loss levels at cross-over regions show that the channel filters are close to complementary state.

- Return loss levels towards the outer edges of the outer channels need improvement. This can be done by tuning the elements of all channel filters by using both Tune Element and Monte Carlo-Best Case optimization routine. Optimization of a channel filter need be done by loading that filter and then inserting the other channel filters as External 1-Port. Monte Carlo-Best Case routine can be applied only to the explicitly displayed channel. So, we load each channel filter one by one, insert the other channels, optimize the response, then delete the other filters and save the resulting channel filter. This procedure is repeated by loading each channel one by one (Figure 17.106.r-s-t). After long trials we get responses shown in Figure 17.106.x. Return loss is settled at about 20 dB level after a slight shrinkage in bandwidth. This level is about 4 dB higher than the return loss we have got by using LP prototype approach of Example 17.38.

- We have reached at a solution which we term as optimum. At this point we may ask the question “would we get a better solution if we had started with a better set of inner passband edge frequencies?”. The answer seems to lie in the element values of the two different quadruplexers we have designed. Figure 17.106.y shows the element values of the new quadruplexer. Comparison of these element values with those in Example 17.38 shows that they are not too far from each other. This means that if we had continued the optimization, we would get the same circuits in Example 17.38 also. This also means that we can get the same solution by starting with reasonable trial values for the inner passband edge frequencies.

- The inverters of filters may be realized in different way. Some examples will be given later, after the next example in which we will use both singly terminated and doubly terminated filters.

Figure 17.106. Design of a quadruplexer using singly terminated filters.

Figure 17.106. Design of a quadruplexer using singly terminated filters.

#### Figure 17.106. Design of a quadruplexer using singly terminated filters.

Figure 17.106. Design of a quadruplexer using singly terminated filters.

Figure 17.106. Design of a quadruplexer using singly terminated filters.

#### EXAMPLE 17. 40.  Contiguous combline quadruplexer design using mixed singly and doubly terminated filters

#### In the previous example we have used only singly terminated channel filters to design the quadruplexer just to make use of the complementary nature of the adjacent filters. However the outer channel filters were badly mismatched, forced us to use long iterative trials. Let’s now try the approach where the outer channels will be designed as doubly terminated to see if we can get a better or easier solution.

#### In this example we will try to form a contiguous quadruplexer by using singly terminated designs.

#### Specifications of the channels are repeated:

#### Channel-A: f1=10000 MHz, f2=12000 MHz. We will take fp1=10000 MHz, fp2=11750 MHz.

#### Channel-B: f1=12000 MHz, f2=14000 MHz. We will take fp1=12250 MHz, fp2=13750 MHz.

#### Channel-C: f1=14000 MHz, f2=16000 MHz. We will take fp1=14250 MHz, fp2=15750 MHz.

#### Channel-D: f1=16000 MHz, f2=18000 MHz. We will take fp1=16250 MHz, fp2=18000 MHz.

#### PBRipple= 0.02 dB,  Nzero=1, Ninf=9 so five resonators will be used in each filter.

#### Channel-B and Channel-C filters, B70 and C70 will be the same as in the previous examples. We rename them as B87 and C87 respectively, to be used in the new quadruplexer. So we have to design only Channel-A and Channel-D filters.

#### Design steps of Channel-A filter are shown in Figure 17.107.a. Doubly Terminated option is selected. After typing in Nzero=1 and Ninf=9, the transmission zeros are extracted using Standard Auto Extract command (Figure 17.107.a).

- Pi-Tee-L / Equal Shunt SC Stubs is applied to convert the circuit into coupled resonator form with SC+OC Stub type shunt resonators (Figure 17.107.b).

- Then with cursor under any shunt SC+OC stub resonator, the command Replace by Lumped / All SC Stub+Cap  is applied to convert the OC stubs of resonators into lumped capacitors (Figure 17.107.c). This operation destroys equality of the shunt SC stubs. This property is recovered by applying  Pi-Tee-L / Equal Shunt SC Stubs again (Figure 17.107.d).

- Then the command Inverters / Scale Cct by Inv / Specify SC Stub: Z=70 ohms is applied to scale the shunt SC stubs to 70 ohms (e.g.) (Figure 17.107.e). The circuit is saved as  A87.fil. The same procedure is applied to design Channel-D filter. It is saved as D87 respectively (Figure 17.107.f).

Formation of quadruplexer:

- First, the singy terminated channel filters, B87 and C87 are connected to each other end their source end inverters are tuned to form a complementary pair. They are saved as B88 and C88 (Figure 17.107.g-h).

- Next a quadruplexer is formed by inserting B88, C788 and D87 into A87 as External 1-Port and all elements of A87 are tuned to improve the response. The External 1–Port elements are deleted and the tuned A87 is saved as A88 (Figure 17.107.i-k).

- Then a quadruplexer is formed by inserting B88, C788 and A88 into D87 as External 1-Port and all elements of D87 are tuned to improve the response. The External 1–Port elements are deleted and the tuned D87 is saved as D88.

- This procedure is repeated by loading B88, C88, D88 and A88 one by one and inserting the other channels as External 1-Ports until the return loss gets its best level. Figure 17.107.m shows the return loss before optimization. We get the optimized responses shown in Figure 17.107.n. Return loss response is at about the same level (21 dB) as the quadruplexer with all singly terminated filters.

- Figure 17.107.p shows the optimized circuits.

- Figure 17.107.q shows insertion loss responses of the two quadruplexers overplotted for comparison. It is seen that the outer skirt slopes are higher in the quadruplexer with mixed singly and doubly terminated filters compared to the quadruplexer with all singly terminated filters. This shows that it is advantageous to use doubly terminated filters for the outer channels.

Figure 17.107. Quadruplexer design using both singly and doubly terminated filters.

Figure 17.107. Quadruplexer design using both singly and doubly terminated filters.

Figure 17.107. Quadruplexer design using both singly and doubly terminated filters.

Figure 17.107. Quadruplexer design using both singly and doubly terminated filters.

# EXAMPLE 17. 41. Realization of the junctions of the quadruplexers

In the previous two examples we have designed the quadruplexers using filters with inverter terminations. The load end inverters can be replaced by different practical equivalents, like capacitive or inductive L-sections or tappings. Realization of the source end inverters will need more care because they will be parts of the parallel connecting junctions. Let’s make some trials by using the quadruplexer of Example 17.39 (with all channels being singly terminated).

Common Resonator quadruplexer:

Although the common resonator junction seems difficult to form due to possible unaccounted couplings between the resonators of the channels, let’s start with that alternative just to get an idea.

- In Figure 17.108.a the quadruplexer of Example 17.39 is repeated. We insert a resonator at the input of the quadruplexer by brute force by copying any resonator from any channel filter and pasting it on the source side.

- After tuning the capacitor of the common resonator, we re-tune all elements of all channels slightly to improve the return loss. We get the responses shown in Figure 17.108.b. Return loss response settled at about 26 dB. A very good level.

- The tuned filters are shown in Figure 17.108.c-d-e-f.

- Figure 17.108.g shows the top view of a possible realization. It is clear that the crowding around the common resonator will possibly lead to unaccounted couplings between resonators. For this reason it may be better to use distributed junctions (extended junction or manifold type structures), as will be discussed next.

Figure 17.108. Formation of common resonator quadruplexer.

Figure 17.108. Formation of the common resonator quadruplexer.

Extended junction quadruplexer:

- The quadruplexer circuit described above may be difficult to construct because of the crowding at the common input junction. One way may be to use an extended junction approach where the channels are combined two-by-two at two subjunctions and then combined at the input junction as described in Figure 17.109. This approach CAN’T be simulated on Filpro because it necessitates to use External 1-Port elements inside External 1-Port elements which is not possible to implement on Filpro. If a general purpose simulator is available, then the diplexer subjunctions can be designed on Filpro and then transferred to the simulator for the formation and optimization on the input junction.

Figure 17.109. Extended junction quadruplexer.

Manifold type quadruplexer:

- The channel filters may also be combined through a manifold. An example is described in Figure 17.110.

- First a diplexer is formed by loading Channel-D filter, D70 and inserting Channel-C filter C70 as  shunt External 1-Port (Figure 17.110.a-b).

- Then a transmission line (or waveguide) is inserted to the left of the diplexer (Figure 17.123.c).

- Then Channel-B and Channel-A filters (B70 and A70) are inserted as shunt External 1-Port elements to the left of the transmission line. Finally, a common resonator is inserted between source and the A70-B70 pair (Figure 17.110.c-d).

- The elements of the explicitly displayed channel (D70), the parameters of the TL and the common resonator  is done are tuned by using Tune Element command.

- The elements of the External 1-Port filters are tuned by using Plot-Tune Multiplexer command. We note down the tuned values and type them into the individual circuits  later by loading them one by one.

- After tuning we get the return loss response shown in Figure 17.110.e. Return loss response settled at about 17 dB. However this level will decrease after the inverters are replaced by their practical equivalents (Exercise?).

- Figure 17.110.f shows the insertion loss responses of the channels while Figure 17.110.g shows the schematics of a possible implementation. The Y-junction on the right of the TL may be formed by replacing the inverters of D75 and C75 by quarter wavelength  TL.

#### Figure 17.110. A manifold type quadruplexer.

Figure 17.110. A manifold type quadruplexer.

#### 17. 24. SEPARABLE LP-HP-BP multiplexing

- The problems associated with common resonator junction mentioned in the previous example may be evaded by using a simple alternative where the physical separation and isolation between the channels is provided by a low order LP-HP diplexer with the bandwidth and selectivity of the channels provided by BP filters appended at the outputs of the LP-HP diplexers. The LP-HP diplexers provide physical separation between the channels, hence take the places of extended junctions.

#### EXAMPLE 17. 42. A separable diplexer

Let’s consider a diplexer with 3 dB cross-over frequency fc=12000 MHz, lower channel passband with fp1=1000 MHz, fp2=12000 MHz and upper channel with fp1=12000 MHz, fp2=14000 MHz.

The LP-HP diplexer can be designed by the techniques of section 17.16. Just for simplicity of the description, let’s use the quick design approach using the command Design / LP-HP Diplexer command of Filpro, without considering realizability:

- First we design a LP-HP diplexer with for example degree N=3, PBRipple=0.01 dB, as described in Figure 17.111. The resulting LP channel filter is saved as LP12 (Figure 17.111.a) and the HP filter is saved as HP12 (Figure 17.111.b).

- Next, we load LP12 and append the singly terminated BP filter A71 which was designed in Example 17.39 for the channel A with fp1=10000 MHz and fp2=12000 MHz. {We could as well append any BP filter covering the same band, for example the doubly terminated filter of Example 17.40}. The resulting filter is saved as LP12A (Figure 17.111.c).

- Then we load HP12 and append the singly terminated filter B71 of Example 12.39 and save the resulting circuit as HP12B (Figure 17.111.d).

- Next, we form the diplexer circuits by inserting HP12B into LP12A and vice versa (Figure 17.111.e) and tune (optimize) the elements for the best return loss. The optimized diplexer circuits are shown in Figure 17.111.g. We get the responses shown in Figure 17.111.h. Return loss is settled at about 25 dB.

- Figure 17.111.i and 17.111.k show the schematic topologies for a diplexer and quadruplexer respectively. {The quadruplexer can’t be analyzed on Filpro}.

Figure 17.111. Diplexer and multiplexer design using separable multiplexing approach.

Figure 17.111. Diplexer and multiplexer design using separable multiplexing approach.

Figure 17.111. Diplexer and multiplexer design using separable multiplexing approach.

#### 17. 25. ELLIPTIC COMBLINE DIPLEXERS

- The approaches used in Chebyshev diplexers and multiplexers that were described in the previous examples can be extended to cover elliptic channel filters also. The finite transmission zeros may be realized either by providing series arm parallel LC type couplings between resonators or by using cross-couplings that can result in Cascaded Triplets (CT) or cascaded quadruplets (CQ) type (or other more complex cross-coupled type) elements. Let’s see some examples.

#### EXAMPLE 17. 43. A noncontiguous elliptic combline diplexer design

- Let’s consider a noncontiguous diplexer that can be used as Transmit-Receive diplexer with following specifications:

Receiver channel: fp1=1710 MHz  and  fp2=1785 MHz.

Transmitter channel:  fp1=1805 MHz  and  fp2=1880 MHz.

Return loss will be greater than 16 dB. Insertion loss of each channel over the passband of the other channel (isolation) will be greater than 45 dB.

On Filpro it is possible to design combline type filters either as lumped element filters or distributed element filters. Since in the final stage the filters will be of mixed lumped+distributed element type, we can convert the resulting all lumped or all distributed element filter into mixed element type filter. In all the previous examples we have started the designs as distributed element filters. Just as a demonstration, in this example let’s use lumped element approach. In this approach the resonators will be of shunt arm parallel LC resonators and couplings between resonators will be of lumped LC or L or C type depending on the realization topology. In the final stage the lumped LC resonators can be converted into SC stub resonators in parallel with lumped capacitors and series coupling inductors can be converted into SC stubs. Lumped element approach is slightly advantageous when cross-coupled resonators are used.

Experimentation with Synthesis approach indicated that 4 resonators are needed and two finite transmission zeros should be created to satisfy the specifications. The procedure for the various possible receive and transmit filters are described below.

Design of receiver channel filters in lumped element form:

- Design / By Synthesis approach and Lumped-Equiripple-Doubly Terminated-Type B options are selected. After typing in fp1=1710 MHz and fp2=1785 MHz, the transmission zeros are entered.

- Degree of the filter is N=8 (4 resonators). We first try two FTZ’s which uses up four degrees. So, four degrees are left for the TZ’s at f=0 and f=. In order to get inductive couplings, we set  Nzero=1 and Ninf=3. Two FTZ’s are placed by trial using Tune FTZ command to get 45 dB loss within the passband of the other (transmit) channel. They come out to be fz1=1806 MHz and fz2=1832 MHz.

- The transmission zeros are extracted in the order FTZ1-Inf-Inf-FTZ2-Inf-Zero (Figure 17.112.a). This order of extraction gives close to symmetric structure.

- Then the command Pi-Tee-L / Equal Shunt Inductors is applied to convert the circuit into direct coupled resonator form with equal shunt inductors (Figure 17.112.b). This circuit is saved as A30.fil to be used later to design the direct  coupled combline filter.

- If cross-coupled realization is desired then we place the cursor under one of the series connected parallel LC elements and apply the command CT-CQ-CN Oprs / Convert to CT resulting in a triplet (CT sections) with inductive cross-couplings (Figure 17.112.c). The FTZ’s in upper stopband lead to LLL type CT sections where the first L shows the cross-coupling element (L13). The inset shows the details of the CT element. The nodes are shown as 1-2-3. The resonator at node-2 (L2-C2 ) is hidden in the CT element while the resonators on the left and right of the CT element are treated as L1-C1 and L3-C3 respectively. The parameter R2 symbolizes the loss resistor of the resonator C2-L2 and it is set as infinite.

- Note that the shunt inductors of the CT filter are not equal. They can be equated by first placing the cursor under the series connected parallel LC element and applying the command Pi-Tee-L / Equal Shunt Inductors to equate the first two shunt inductors. Then the cursor is moved under the CT element and the command CT-CQ-CN Oprs / Modify CT / Set L2 (Set L3) / Default: L2=.. is applied iteratively a few times. In applying this command we note down (Copy-CTRL+C) the value of L1 (the inductor of the resonator on the left of the CT element) and type in  (Paste-CTRL+V) this value as targeted value for L2 (L3). Changing any one of L1-L2-L3 changes all L1-L2-L3 because they are interrelated. Therefore a few iterations are needed to make all equal. We save the resulting circuit as A40.fil to be used later (Figure 17.112.d).

Figure 17.112. Design of lumped element receiver channel filters.

Direct coupled version of the receive channel filter with mixed elements and inverters:

- The design steps of direct coupled version of the receive filter are shown in Figure 17.113:

- The circuit A30 is loaded (Figure 17.113.a).

- With cursor under the shunt and series LC elements, the command Replace by Stub Eqv / Parallel LC-to-Cap+SC Stub / Specify fq / fq=3000 MHz, f1=1710 MHz, f2=1785 MHz is applied. This command converts the inductors of the LC resonators into SC stubs so that the LC elements become Cap+SC Stub elements (Figure 17.113.b). The physical length of SC stubs are set by the selected quarter wavelength frequency fq. It is arbitrarily set as fq=3000 MHz. f1 and f2 are set as the passband edges to calculate the new values of capacitors so that the pass bandwidth will not be effected by this transformation. The electrical length of stubs can be calculated through the formula

θ=π2f0fq=π217503000=52.50

- Next, with cursor being under the series inductors, the command Replace by Stub Eqv / SC Stub / f0=1750 MHz, fq=3000 MHz is applied to convert them into SC stubs with the same fq. This approximation causes little degradation in response. So, we get the SC Stub and Capacitor model of the filter (Figure 17.113.c).

- Next, we will scale the SC stub impedances to the desired realization level. This is done by applying the command Inverters / Scale Cct by Inserting Inverters / Specify SC Stub: Z=70 ohms. This command inserts ideal inverters at source and load ends to scale up the impedance level of the whole circuit. In this case we have selected the shunt SC stub impedances to be 70 ohms (Figure 17.113.d).

- The transformer at load end can be absorbed into the inverter of this port by applying the command Inverters / Absorb TRF (Figure 17.113.e). So, we get the circuit model of the filter with inverters at both ports. It is saved as A31.fil. It can be used in different implementations by replacing the inverters with different practical models, like replacing the source end inverters by capacitive-L sections or Quarter Wavelength TL and replacing the load end inverter by tapped line equivalent. These stages can be carried out after the formation of diplexer.

Figure 17.113. Conversion of direct coupled lumped element receiver filter into mixed element form with inverters.

Cross-coupled version of the receive channel filter with mixed elements and inverters:

In some cases it may be preferable to use cross-coupled topology. Design steps are as follows:

- The CT filter circuit of A40 is loaded (Figure 17.114.a). With cursor under the shunt LC resonators, the command Replace by Stub Eqv / Parallel LC-to-Cap+SC Stub / Specify fq / fq=3000 MHz, f1=1710 MHz, f2=1785 MHz is applied. This command converts the inductors of the LC resonators into SC stubs so that the LC elements become Cap+SC Stub elements (Figure 17.114.b). The physical length of SC stubs are set by the selected quarter wavelength frequency fq. It is arbitrarily set as fq=3000 MHz. f1 and f2 are set as the passband edges to calculate the new values of capacitors so that the pass bandwidth will not be effected by this transformation. The electrical length of stubs can be calculated through the formula

θ=π2f0fq=π217503000=52.50

- Next, the cursor is placed under the CT element and the command Replace by Stub / Specify fq: f0=1750 MHz, fq=3000 MHz is applied. This command converts the all-lumped CT section into a mixed element CT section. The shunt LC resonator of Node-2 is converted into a Cap+SC stub element while the coupling inductors L13-L12-L23 are converted into coupling SC stubs with impedances Zs13-Zs12-Zs23 respectively, as shown in the inset in Figure 17.114.c.

- Next, we scale the impedance level of the circuit so that all the shunt SC stubs will be, for example, 70 ohms. This is done by applying the command  Inverters / Scale Cct by Inserting Inverters / Specify SC Stub: Z=70 ohms (Figure 17.114.d). The transformer at load end can be absorbed into the adjacent inverter by applying the command Inverters / Absorb TRF. This circuit is saved as A41.fil.

- The inverters at source and load ends can be replaced by different practical equivalents. For example the source end inverter is replaced by its capacitive-L section equivalent to feed the input through a series capacitor while the load end inverter can be replaced by its Tapped equivalent.  These stages can be carried out after the formation of diplexer.

Figure 17.114. Conversion of cross-coupled lumped element receiver filter into mixed element form with inverters.

Design of transmitter channel filters in lumped element form:

The transmit channel filters are also designed by using the same steps. Figure 17.115 shows the design of the lumped element versions:

- Design / By Synthesis approach and. Lumped-Equiripple-Doubly Terminated-Type B options are selected. After typing in fp1=1805 MHz and fp2=1880 MHz, the transmission zeros are entered.

- Degree of the filter is N=8 (4 resonators). We first try two FTZ’s which uses up four degrees. So, four degrees are left for the TZ’s at f=0 and f=. In order to get inductive couplings, we set  Nzero=1 and Ninf=3. Two FTZ’s are placed. Their locations are adjusted by Tune FTZ command to get 45 dB loss within the passband of the other (receive) channel. They come out to be fz1=1759 MHz and fz2=1784 MHz.

- The transmission zeros are extracted in the order FTZ1-Inf-Inf-FTZ2-Inf-Zero (Figure 17.115.a). This order of extraction gives close to symmetric structure.

- Then the command Pi-Tee-L / Equal Shunt Inductors is applied to convert the circuit into direct coupled resonator form with equal shunt inductors (Figure 17.115.b). This circuit is saved as B30.fil to be used later to design the direct coupled combline filter.

- If cross-coupled realization is desired then we place the cursor under one of the series connected parallel LC elements and apply the command CT-CQ-CN Oprs / Convert to CT/ CLL resulting in a triplets (CT sections) with capacitive cross-couplings (Figure 17.115.c). The FTZ’s in lower stopband lead to one capacitive coupling element. The inset in Figure 17.115.c shows the details of the CLL element. The nodes are shown as 1-2-3. The resonator at node-2 (L2-C2) is hidden in the CT element while the resonators on the left and right of the CT element are treated as L1-C1 and L3-C3 respectively. We usually select the option CLL where the first letter C denotes the cross-coupling element (C13). The other option has an inline capacitive coupling which may be more problematic for realization. However the option CLL has also a small problem: It is obtained through an approximation. This approximation has negligible effects on response for narrowband filters. However our filter is not a narrowband one. Therefore the effects of approximation appears as shifts in the transmission zeros and some degradation in return loss. Therefore in using that option we have to check the response before and after this transformation. Indeed in our example the FTZ at fz2=1784 MHz is shifted to 1781 MHz.

- In order to get a satisfactory response, we repeat the design by assigning FTZ’s closer to the passband, at fz1=1759 MHz and fz2=1786 MHz, getting the circuit of Figure 17.115.e.

- Note that the shunt inductors of the CT filter are not equal. They can be equated by first placing the cursor under the series connected parallel LC element and applying the command Pi-Tee-L / Equal Shunt Inductors. Then the cursor is moved to the CLL elements and the command CT-CQ-CN Oprs / Modify CT / Set L2 (Set L3) / Default: L2=.. is applied iteratively a few times. In applying this command we note down (Copy-CTRL+C) the value of L1 (the inductor of the resonator on the left of the CT element) and type in  (Paste-CTRL+V) this value as targeted value for L2 (L3). Changing any one of L1-L2-L3 changes all L1-L2-L3 because they are interrelated. Therefore a few iterations are needed to make all equal.

- We save the resulting circuit as B40.fil to be used later (Figure 17.115.f). Figure 17.115.g shows the response which seems satisfactory.

Figure 17.115. Design of the lumped element version of the transmitter channel filters.

Figure 17.115. Design of the lumped element version of the transmitter channel filters.

Direct coupled version of the transmitter channel filter with mixed elements and inverters:

- The design steps of direct coupled version of the transmitter filter are shown in Figure 17.116:

- The circuit B30 is loaded (Figure 17.116.a).

- With cursor under the shunt and series LC elements, the command Replace by Stub Eqv / Parallel LC-to-Cap+SC Stub / Specify fq / fq=3000 MHz, f1=1805 MHz, f2=1880 MHz is applied. This command converts the inductors of the LC resonators into SC stubs so that the LC elements become Cap+SC Stub elements (Figure 17.116.b). The physical length of SC stubs are set by the selected quarter wavelength frequency fq. It is arbitrarily set as fq=3000 MHz. f1 and f2 are set as the passband edges to calculate the new values of capacitors so that the pass bandwidth will not be effected by this transformation. The electrical length of stubs can be calculated through the formula

θ=π2f0fq=π218403000=55.20

- Next, with cursor being under the series inductors, the command Replace by Stub Eqv / SC Stub / f0=1750 MHz, fq=3000 MHz is applied to convert them into SC stubs with the same fq. This approximation causes little degradation in response. So, we get the SC Stub and Capacitor model of the filter (Figure 17.116.c).

- Next, we will scale the SC stub impedances to the desired realization level. This is done by applying the command Inverters / Scale Cct by Inserting Inverters / Specify SC Stub: Z=70 ohms. This command inserts ideal inverters at source and load ends to scale up the impedance level of the whole circuit. In this case we have selected the shunt SC stub impedances to be 70 ohms (Figure 17.116.d).

- The transformer at load end can be absorbed into the inverter of this port by applying the command Inverters / Absorb TRF (Figure 17.116.e). So, we get the circuit model of the filter with inverters at both ports. It is saved as B31.fil. It can be used in different implementations by replacing the inverters with different practical models.

Figure 17.116. Conversion of direct coupled lumped element transmitter filter into mixed element form with inverters.

Cross-coupled version of the receive channel filter with mixed elements and inverters:

Design steps of the cross-coupled filter are as follows:

- The CT filter circuit of B40 is loaded (Figure 17.117.a). With cursor under the shunt LC resonators, the command Replace by Stub Eqv / Parallel LC-to-Cap+SC Stub / Specify fq / fq=3000 MHz, f1=1805 MHz, f2=1880 MHz is applied. This command converts the inductors of the LC resonators into SC stubs so that the LC elements become Cap+SC Stub elements (Figure 17.117.b). The physical length of SC stubs are set by the selected quarter wavelength frequency fq. It is arbitrarily set as fq=3000 MHz. f1 and f2 are set as the passband edges to calculate the new values of capacitors so that the pass bandwidth will not be effected by this transformation. The electrical length of stubs can be calculated through the formula

θ=π2f0fq=π218403000=55.20

- Next, the cursor is placed under the CT elements and the command Replace by Stub / Specify fq: f0=1750 MHz, fq=3000 MHz is applied. This command converts the all-lumped CT section into a mixed element CT section. The shunt LC resonator of Node-2 is converted into a Cap+SC stub element while the coupling inductors L12 and L23 are converted into coupling SC stubs with impedances Zs12 and Zs23 respectively. The cross-coupling capacitor is left as it is (Figure 17.117.c).

- Next, we scale the impedance level of the circuit so that all the shunt SC stubs will be, for example, 70 ohms. This is done by applying the command  Inverters / Scale Cct by Inserting Inverters / Specify SC Stub: Z=70 ohms (Figure 17.117.d).

- The transformer at load end can be absorbed into the adjacent inverter by applying the command Inverters / Absorb TRF. This circuit is saved as B41.fil (Figure 17.117.e). The inverters at source and load ends can be replaced by different practical equivalents. For example the source end inverter is replaced by its capacitive-L section equivalent to feed the input through a series capacitor while the load end inverter can be replaced by its Tapped equivalent.

Figure 17.117. Conversion of cross coupled lumped element transmitter filter into mixed element form with inverters.

Formation and optimization of the diplexer using direct coupled filters with inverters:

As usual, we form two diplexer circuits: In one of them the filter B31 is inserted into A31 as External 1-Port (Figure 17.118.a-b) and the elements of A31 are tuned. The optimized A31 is saved as A32. The other diplexer circuit is formed by inserting A32 into B31 as External 1-Port (Figure 17.118.c-d) and the elements of B31 are tuned and optimized B31 is saved as B32.

Return loss response is optimized by tuning all elements except the FTZ elements (because they would shift the FTZ’s) and shunt SC stubs (which we would like to keep at 70 ohms). Optimization is done using both Tune Element and Monte Carlo-Best Case routine alternately. The optimized circuits and responses are shown in Figure 17.118.f. We get about 17 dB return loss and about 46 dB rejection in the opposite passbands.

The coupling coefficients between adjacent resonators can be displayed by placing the cursor under the coupling elements and invoking the keys ALT+F6, as shown in Figure 17.118.g.

The effects of finite Q of the resonators can be displayed as follows:

- The circuit A32 is loaded (Figure 17.118.h) and with cursor under any shunt resonator, the command Replace by Lumped / All Lumped LC is applied to convert the Cap+SC Stub type resonators into lumped LC resonators (Figure 17.118.i).

- Then with cursor under any shunt LC resonator, the command Parasitics / All Lossy Resonators / f0=1750 MHz, Q=500 (e.g.) is applied converting the lossless LC resonators into lossy LC resonators as shown in Figure 17.118.k.

- The same procedure is applied to B32 giving the lossy circuit of Figure 17.118.p.

- Figure 17.118.q shows the responses for the lossy channel filters. The passband loss is now about 0.65 dB.

Formation and optimization of the diplexer using cross-coupled filters:

Exercise: Form two diplexer circuits: In one of them the filter B41 is inserted into A41 as External 1-Port and in the other A41 is inserted into B41 as External 1-Port.

Return loss response is optimized by tuning all elements except the cross-coupling elements (because they would shift the FTZ’s) and shunt SC stubs (which we would like to keep at 70 ohms). Optimization is done using both Tune Element and Monte Carlo-Best Case routine alternately.

Figure 17.118. Formation of the diplexer using direct coupled channel filters.

Figure 17.118. Formation of the diplexer using direct coupled channel filters.

Figure 17.118. Formation of the diplexer using direct coupled channel filters.

Figure 17.118. Formation of the diplexer using direct coupled channel filters.

Diplexer with series capacitor input and tapped output:

- If the input to the filter will be applied through a series capacitor then the inverter at source end can be replaced by its Capacitive-L equivalent by applying the command Inverters / Replace Inverter by / End Inverter to / Capacitive-L (Figure 17.119.a-b). The negative shunt capacitor can be absorbed into the capacitance of the adjacent resonator by applying Simplify Circuit command.

- If the output will be taken through a tap on the last resonator, then the position of the tap point can be determined by converting the inverter into its equivalent circuit model by applying the command Tapped PCL / Z0=70 ohms / fRes=1726.8 MHz, fq=3000 MHz. In this command Z0=70 ohms is the desired resonator (SC stub) impedance. fRes=1726.8 MHz is the resonance frequency of the resonator at load end. The load end inverter and its equivalent circuit are equated at that frequency, leading to the circuit piece shown at the load end of Figure 17.119.d. Here, we have a shunt SC stub of length LTap, less than the commensurate length of the filter. It shows the distance between the tap point and grounded end of the resonator rod (LTap). On the left of the shunt SC stub there is a transmission line of length Lng-LTap where Lng is the commensurate length of the filter. The impedances of both the shunt SC stub and TL are 70 ohms. Note that the capacitor of the load end resonator is also changed slightly. All these changes have negligible effects on response of the filter. This circuit will be used in the formation of the diplexer. It is saved as  A33.fil.

- The same procedure is applied on the transmit filter B32 and the resulting filter is saved as B33 (Figure 17.119.c-d). Figure 17.119.e shows a realization schematics.

- Figure 17.119.f shows the diplexer response. The approximate nature of the source end inverter equivalents cause some degradation in return loss which can be corrected by a slight optimization.

- Figure 17.119.g and h show the optimized circuits and responses. 17 dB return loss level is recovered.

Figure 17.119. Formation of the diplexer with series capacitor coupled input and tapped outputs.

Figure 17.119. Formation of the diplexer with series capacitor coupled input and tapped outputs.

Figure 17.119. Formation of the diplexer with series capacitor coupled input and tapped outputs.

- Diplexer with interdigital type common resonator input:

If an interdigital type common transformer resonator is to be used at input, then the following modifications should be made on the filters A32 and B32:

- The circuit A32 is loaded (Figure 17.120.a). With cursor under the source end inverter the command Inverters / Replace Inverter by / Quarter Wave TL: f0=fq=3000 MHz is applied. So, the source end inverter is replaced by a TL piece which has the same length as the stubs (Figure 17.120.b). This replacement degrades the response heavily. The resulting circuit is saved as A35.fil.

- Similarly, the circuit B32 is loaded (Figure 17.120.c). With cursor under the source end inverter the command Inverters / Replace Inverter by / Quarter Wave TL: f0=fq=3000 MHz. and the resulting circuit is saved as A35.fil (Figure 17.120.c-d).

- Then diplexer circuits are formed by using A35 and B35. Also a shunt SC stub of impedance Z=70 ohms is inserted at the input of the diplexer to work as a common transformer resonator (Figures 17.120.e-f-g). The return loss response of the diplexer is then optimized by tuning  i) the length of common resonator, ii) the impedance of the TL’s and series SC stubs in the middle of the circuits, iii) the capacitors of all shunt resonators and v) the inverters at load ends. Using both Tune Element and Monte Carlo-Best Case optimization routine alternately, we get the circuits and response shown in Figure 17.120.h. Return loss is settled at about 16 dB level.

- Figure 17.120.g shows a realization schematics, with the load end inverters also replaced by Tapped lines. Note that the common transformer is longer than the other resonators.

Figure 17.120. Formation of the diplexer with interdigital type common resonator input.

Figure 17.120. Formation of the diplexer with interdigital type common resonator input.

- Diplexer with comb type common resonator input:

If a comb type common transformer resonator is to be used at input, then the following modifications should be made on the filters A32 and B32:

- The circuit A32 is loaded (Figure 17.121.a). With cursor under the source end inverter the command Inverters / Replace Inverter by / Constant Reactance/ Pi section is applied. The Pi-section has negative shunt reactances and positive series reactance (Figure 17.121.b).

- Next, the negative shunt reactances are replaced by lumped capacitors by applying the command Constant Reactances / Replace Constant Reactance by / Capacitor.

- Then the series constant reactance is replaced by a series SC stub by applying the command Constant Reactances / Replace Constant Reactance by / SC Stub / Specify fq=3000 MHz. Simplify Circuit command is used to combine the shunt capacitor of second node with the SC Stub+Cap resonator at that node. The circuit is saved as  A38.fil (Figure 17.121.c).

- Similarly, the circuit B32 is loaded (Figure 17.121.d) and the same procedure is applied. The resulting circuit is saved as B38.fil (Figure 17.121.e-f-g).

- Then diplexer circuits are formed by using A38 and B38. Also a shunt SC stub of impedance Z=70 ohms is inserted at the input of the diplexer to work as a common transformer resonator (Figures 17.121.h-i-k-m-n-p). The return loss response of the diplexer is then optimized by tuning  i) the length of common resonator, ii) the impedance of the series SC stubs, iii) the capacitors of all shunt resonators and v) the inverters at load ends. Optimization is done by using both Tune Element and Monte Carlo-Best Case routine alternately. Responses before and after optimization are shown in Figure 17.121.q and r. Return loss is settled at about 17 dB level. Rejection is about 48 dB.

- The optimized circuits are shown in 17.121.s and t. The load end inverters of these circuits are converted into tapped equivalent. The dielectric constants of all elements can be set to any desired value, for example Er=11.5 by applying the command Set Dielectric Constant as shown in the figure. This operation changes the physical length of elements to keep the electrical length unchanged.

- Figure 17.121.u shows a realization schematics.

Figure 17.121. Formation of the diplexer with comb type common resonator input.

Figure 17.121. Formation of the diplexer with comb type common resonator input.

Figure 17.121. Formation of the diplexer with comb type common resonator input.

Figure 17.121. Formation of the diplexer with comb type common resonator input.

#### EXAMPLE 17. 44. An elliptic combline diplexer designed for block dielectric realization

- Let’s consider the diplexer of the previous example for realization in block dielectric form with the TZ’s at quarter wavelength frequency fq are used as FTZ’s by selecting fq close to the passband egde in the receive filter. The specifications are :

Receiver channel: fp1=1710 MHz  and  fp2=1785 MHz.

Transmitter channel:  fp1=1805 MHz  and  fp2=1880 MHz.

Return loss will be greater than 16 dB.

Insertion loss of each channel over the passband of the other channel will be greater than 45 dB.

Four resonators will be used in each channel.

The characteristic impedances of the resonators will be 40 ohms with dielectric constant being Er=11.5.

Let’s use distributed element approach for this example because we will use the TZ’s at quarter wave frequency  as the FTZ’s of the receiver filter.

Design of receiver filter:

Design steps of the receiver filter are shown in Figure 17.122.

- Design / By Synthesis approach and Distributed Element-Equiripple-Doubly Terminated-Type B options are selected. The passband edge frequencies fp1=1710 MHz and fp2=1785 MHz are typed in. The TZ’s at quarter wavelength frequency fq will be used as the FTZ’s. Therefore selection of fq sets the selectivity of the filter. After some design trials we set it close to the upper band edge, as fq=1820 MHz. In distributed element filter design Infinite frequency actually corresponds to fq.

- Degree of the filter is N=8 (4 resonators). In order to get inductive couplings, we set  Nzero=1 and the remaining 7 degrees will be at infinity (fq). So we set Ninf=7.

- The transmission zeros are extracted in the order Inf-Inf-Inf-Inf-Zero-Inf-Inf-Inf (Figure 17.122.a).

- Then the command Pi-Tee-L / Equal Shunt SC stubs is applied to convert the circuit into direct coupled resonator form with equal shunt SC stubs (Figure 17.122.b).

- Then with cursor under a shunt SC+OC Stub resonator the command Replace by Stub Eqv. / All SC Stub+Cap is applied to convert the OC stubs into lumped capacitors (Figure 17.122.c). This operation changes the SC stub impedances slightly. In order to recover the equality of the shunt SC stubs, the command Pi-Tee-L / All Equal Shunt / SC stubs is applied again (Figure 17.122.d).

- Next, the command Inverters / Scale Cct by Inserting Inverters / Specify SC stub: Z=40 ohms is applied to scale the shunt SC stub impedances to 40 ohms (Figure 17.122.e).

- The dielectric constants of all elements are adjusted to Er=11.5 by applying the command Set Dielectric Constant (Figure 17.122.f).

- Since the quarter wavelength frequency is close to the passband the conversion of OC stubs into lumped capacitors degraded the return loss response (Figure 17.122.f). Return loss is recovered by tuning the capacitors and series SC stubs (Figure 17.122.g). The resulting circuit is saved as A51.fil. Note that conversion of the shunt OC stubs into lumped capacitors eliminated the spurious passband in the upper stopband., as seen from the insertion loss plot in Figure 17.122.g. The stopband suppression is about 45 dB within the passband of the transmit filter.

Figure 17.122. Design of receiver filter with the TZ’s at quarter wavelength frequency acting as FTZ’s.

Figure 17.122. Design of receiver filter with the TZ’s at quarter wavelength frequency acting as FTZ’s.

Design of the transmitter channel filter:

The transmit channel filters are also designed by using the same steps as described in Figure 17.123.

- Design / By Synthesis approach and Distributed-Equiripple-Doubly Terminated-Type B options are selected. The passband edge frequencies fp1=1805 MHz, fp2=1880 MHz are typed in. The quarter wavelength frequency fq must be greater than fp2=1880 MHz of this filter. Therefore we can’t select the fq=1820 MHz of the receiver filter. We arbitrarily set fq=2200 MHz. So, the commensurate lengths of the receiver and transmitter filter will be different. This may be a disadvantage caused by our desire to use the fq of the receiver filter as the FTZ’s of that filter.

- Degree of the filter is N=8 (4 resonators). We first try two FTZ’s which uses up four degrees. So, four degrees are left for the TZ’s at f=0 and f=. In order to get inductive couplings, we set  Nzero=1 and Ninf=3. Two FTZ’s are placed. Their locations are adjusted by Tune FTZ command to get 45 dB loss within the passband of the other (receive) channel. They come out to be fz1=1756 MHz and fz2=1784 MHz. The transmission zeros are extracted in the order FTZ1-Inf-Inf-FTZ2-Inf-Zero (Figure 17.123.a). This order of extraction gives close to symmetric structure.

- Then the command Pi-Tee-L / Equal Shunt SC stubs is applied to convert the circuit into direct coupled resonator form with equal shunt inductors (Figure 17.123.b).

- Then with cursor under a shunt SC+OC Stub resonator the command Replace by Stub Eqv. / All SC Stub+Cap is applied to convert the OC stubs into lumped capacitors (Figure 17.123.c). This operation changes the SC stub impedances slightly. In order to recover the equality of the shunt SC stubs, the command Pi-Tee-L / All Equal Shunt / SC stubs is applied again (Figure 17.123.d).

- Next, the command Inverters / Scale Cct by Inserting Inverters / Specify SC stub: Z=40 ohms is applied to scale the shunt SC stub impedances to 40 ohms (Figure 17.123.e).

- The dielectric constants of all elements are adjusted to Er=11.5 by applying the command Set Dielectric Constant (Figure 17.123.f).

- The resulting circuit is saved as B51.fil.

Exercise: In the circuit of Figure 17.123.b, convert one of the FTZ sections into a CT section and continue the design through the same steps.

Formation of diplexer with inverter terminations:

- Two diplexer circuits are formed. One is formed by inserting B51 into A51 as External 1-Port and the other by inserting A51 into B51 as External 1-Port as described in Figure 17.124.a-d. Each diplexer circuit is optimized by tuning inverters, capacitors and series SC stubs only, using both Tune Element command and Monte Carlo-Best Case routine.

- Figure 17.124.e shows the response before optimization while Figure 17.124.f shows the optimized circuits and responses. The optimized channel filters are saved as  A52 and B52 respectively.

Figure 17.123. Design steps of the transmitter filter.

Figure 17.123. Design steps of the transmitter filter.

Figure 17.124. Formation of the diplexer with inverter terminations.

Figure 17.124. Formation of the diplexer with inverter terminations.

Formation of diplexer with comb type common resonator input:

The inverters at source ends can be converted into different forms for realization, as was described in the previous example. In this example only the comb type common resonator input will be described.

- The circuit of A52 is loaded (Figure 17.125.a). With cursor under the source end inverter the command Inverters / Replace Inv by / Constant Reactance / Pi-section is applied to convert the inverter to a constant reactance Pi-section with positive series reactance and negative shunt reactances (Figure 17.125.b).

- Then the negative shunt reactances are converted into shunt lumped capacitors by applying the command Constant Reactances / Replace Constant Reactance by / Capacitor (Figure 17.125.c).

- Next, the series reactance is converted into a series SC stub by applying the command Constant Reactances / Replace Constant Reactance by / SC Stub / Specify fq: fq=2200 MHz (Figure 17.125.d). Then the dielectric constant of the stub is changed to Er=11.5 (Figure 17.125.e). It  is then saved as  A55.

- The same procedure is applied to B52 and resulting circuit is saved as B55 (Figure 17.125.f-g).

- Next, diplexer circuits are formed and a shunt SC stub is inserted at their inputs to work as common resonator (Figure 17.125.h-p).

- Then the two circuits are optimized giving the circuits of Figure 17.125.q. Return loss converged to about 16 dB while rejections are about 44 dB within the passband of the receive filter.

- Note that the common resonator (SC Stub) at input together with the series and shunt SC stubs of the channel filters forms comb type feed network, as described in Figure 17.125.r. It is seen that lengths of the stubs of the two filters are different. If this forms an inconvenience then the same fq should be assigned to both filters in which case we should place FTZ’s to the receiver filter as well (Exercise).

Figure 17.125. Formation of diplexer with comb type common resonator input.

Figure 17.125. Formation of diplexer with comb type common resonator input.

Figure 17.125. Formation of diplexer with comb type common resonator input.

#### EXAMPLE 17. 45. An elliptic combline diplexer with brute force FTZ’s

- Let’s reconsider the diplexer of the previous example of block dielectric form, now aiming at higher stopband suppression together with higher skirt slope.  The specifications are :

Receiver channel: fp1=1710 MHz  and  fp2=1785 MHz.

Transmitter channel:  fp1=1805 MHz  and  fp2=1880 MHz.

Return loss will be greater than 16 dB.

Insertion loss of each channel over the passband of the other channel will be greater than 45 dB.

Four resonators will be used in each channel.

Let’s use distributed element approach for this example because we will use the TZ’s at quarter wave frequency  as the FTZ’s of the receiver filter.

Design of receiver filter:

Design steps of the receiver filter are shown in Figure 17.126.

- Design / By Synthesis approach and Distributed Element-Equiripple-Doubly Terminated-Type B options are selected. The passband edge frequencies fp1=1710 MHz, fp2=1785 MHz and fq=3000 MHz are typed in. For high selectivity and high suppression, three FTZ’s are placed, all at fz=1830 MHz.

- Degree of the filter is N=8 (4 resonators). Three FTZ’s use up 6 degrees. So, we set  Nzero=1 and the Ninf=1.

- The transmission zeros are extracted in the order FTZ1-FTZ2-FTZ3-Inf-Zero (Figure 17.126.a).

- Then the command Pi-Tee-L / Equal Shunt SC stubs is applied to convert the circuit into direct coupled resonator form with equal shunt SC stubs (Figure 17.126.b).

- Then with cursor under a shunt SC+OC Stub resonator the command Replace by Stub Eqv. / All SC Stub+Cap is applied to convert the OC stubs into lumped capacitors (Figure 17.126.c). This operation changes the SC stub impedances slightly. In order to recover the equality of the shunt SC stubs, the command Pi-Tee-L / All Equal Shunt / SC stubs is applied again (Figure 17.126.d).

- Next, the command Inverters / Scale Cct by Inserting Inverters / Specify SC stub: Z=40 ohms is applied to scale the shunt SC stub impedances to 40 ohms (Figure 17.126.e).

- We would like to create one more FTZ by brute force as follows:

- With cursor under the source end inverter, the command Inverters / Replace Inverter by / Constant Reactance / Pi-section (Figure 17.126.f).

- The negative shunt constant reactances are replaced by capacitors by applying the command Constant Reactances / Replace Const. Reactance by / Capacitor command (Figure 17.126.g). The second shunt capacitor is absorbed into the capacitor of the adjacent resonator.

- Then with cursor under the series constant reactance the command Constant Reactances / Replace Const. Reactance by / Parallel LC / Specify Inductor: L=4 nH is applied (Figure 17.126.h). Here, the value 4 nH is found by inspecting the inductor values of the other FTZ elements (series arm SC Stub+Cap elements) by converting them into lumped form. So, we get a parallel LC element in series arm, creating the fourth FTZ. Location of this FTZ can be adjusted by tuning L or C or both.

- Then the inductor of the lumped LC resonator is converted into SC stub to form a series arm SC Stub+Cap element by applying the command Replace by Stub / SC Stub+Cap (Figure 17.126.i). This circuit is saved as A62. Response is shown in Figure 17.126.k. The degradation in return loss is due to the brute force FTZ and can be corrected by optimization after the formation of diplexer.

Figure 17.126. Design of the receiver filter with a brute force FTZ.

Figure 17.126. Design of the receiver filter with a brute force FTZ in upper stopband.

Design of the transmitter channel filter:

The transmit channel filters are also designed by using the same steps as described in Figure 17.127.

- Design / By Synthesis approach and Distributed-Equiripple-Doubly Terminated-Type B options are selected. The passband edge frequencies fp1=1805 MHz, fp2=1880 MHz aand fq=3000 MHz are typed in.

- Degree of the filter is N=8 (4 resonators). We place three FTZ’s which uses up four degrees. So, we set  Nzero=1 and Ninf=1. The transmission zeros are extracted in the order FTZ1-FTZ2-FTZ3-Inf-Zero (Figure 17.127.a).

- Then the command Pi-Tee-L / Equal Shunt SC stubs is applied to convert the circuit into direct coupled resonator form with equal shunt inductors (Figure 17.127.b).

- Then with cursor under a shunt SC+OC Stub resonator the command Replace by Stub Eqv. / All SC Stub+Cap is applied to convert the OC stubs into lumped capacitors (Figure 17.127.c). This operation changes the SC stub impedances slightly. In order to recover the equality of the shunt SC stubs, the command Pi-Tee-L / All Equal Shunt / SC stubs is applied again (Figure 17.127.d).

- Next, the command Inverters / Scale Cct by Inserting Inverters / Specify SC stub: Z=40 ohms is applied to scale the shunt SC stub impedances to 40 ohms (Figure 17.127.e).

- One more FTZ is created by brute force as follows:

- Sign of the source end inverter is changed to get negative series constant reactance after the command Inverters / Replace Inverter by / Constant Reactance / Pi-section (Figure 17.127.f-g). Negative series constant reactance will be needed to get a FTZ in lower stopband.

- The positive shunt constant reactances are replaced by shunt SC stubs by applying the command Constant Reactances / Replace Const. Reactance by / SC Stub command (Figure 17.127.h). The second shunt SC stub is absorbed into the SC stub of the adjacent resonator. So, impedance of that SC stub is changed.

- Then with cursor under the negative series constant reactance the command Constant Reactances / Replace Const. Reactance by / Parallel LC / Specify Inductor: L=4 nH is applied (Figure 17.127.i). Here, the value 4 nH is found by inspecting the inductor values of the other FTZ elements (series arm SC Stub+Cap elements) by converting them into lumped form. So, we get a parallel LC element in series arm, creating the fourth FTZ in the lower stopband. Location of this FTZ can be adjusted by tuning L or C or both.

- Then the inductor of the lumped LC resonator is converted into SC stub to form a series arm SC Stub+Cap element by applying the command Replace by Stub / SC Stub+Cap (Figure 17.127.k). This circuit is saved as B62. Response is shown in Figure 17.127.m. The degradation in return loss is due to the brute force FTZ and can be corrected by optimization after the formation of diplexer.

Formation of diplexer with comb type common resonator input:

- Diplexer circuits are formed by inserting B62 into A62 and vice versa (Figure 17.128.a-d).

- The shunt SC stub at the source end of B62 will be used as the common resonator while its resonating capacitor is provided by the shunt capacitor of A62 at its source end.

- The load end inverters are replaced by their capacitive-L equivalents.

- Then the two circuits are optimized giving the circuits of Figure 17.128.e. Return loss converged to about 16 dB. Rejections at the edges of the opposite channel passbands are about 50 dB while suppression at far frequencies are greater than 60 dB (Figure 17.128.f). Note that the brute force FTZ’s are far from the passband edges. Therefore they have little contribution to the skirt slopes. They are basically effective in the out of band suppression level.

Figure 17.127. Design of the transmitter filter with a brute force FTZ in lower stopband.

Figure 17.127. Design of the transmitter filter with a brute force FTZ in lower stopband.

Figure 17.128. Formation of the diplexer with brute force FTZs.

Figure 17.128. Formation of the diplexer with brute force FTZs.

#### EXAMPLE 17. 45. A diplexer with CT-CQ channel filters

(with kind permission of Prof. Giuseppe Macchiarella)

In mobile communications base stations usually combline-like cavity filters are employed. Typical requirements for a diplexer in a GSM 1900 MHz combiners calls for very high selectivity necessitating several FTZ’s in both receiver and transmitter channels. As we have seen in the previous examples, in the classical approaches the common resonators are inserted by brute force and then optimization is used to shape up the response. Recently a novel method is developed by Macchiarella that can synthesize such diplexers [47]. In this approach the common resonator is assumed as an integral part of the diplexer during the synthesis of the prototype. Hence the results are exact, necessitating no optimization in the final circuit. Optimization is done during the formation of the prototype in locating the reflection zeros. In this example we will use Macchiarella’s diplexer results as reference to see if we can get the same results using the classical brute force insertion of the common resonator.

The specifications for the channels are as follows:

RX Filter:

fp1=1845.5 MHz, fp2=1915.5 MHz, PBRipple=0.02 dB.

10 resonators will be used leading to a total degree of 20. The transmission zeros are set as follows:

- Nzero=1, Ninf=11, FTZ1 = 1830 MHz, FTZ2= 1928.5 MHz, FTZ3=1932.1 MHz, FTZ4=1942.8 MHz.

TX Filter:

fp1=1925 MHz, fp2=1992 MHz, PBRipple=0.02 dB.

9 resonators will be used leading to a total degree of 18. The transmission zeros are set as follows:

Nzero=1, Ninf=11, FTZ1 = 1890 MHz, FTZ2= 1905 MHz, FTZ3=1910 MHz.

{PBRipple=0.02 dB leads to a Return Loss of about 23.5 dB while in the filters of Macchiarella, Return Loss was taken as 22 dB. Hence a slight deviation should be expected in the results. }

Design of the RX Filter:

Design steps of the receiver filter are shown in Figure 17.129.

- Design / By Synthesis / Lumped / BP / Equiripple / Doubly Terminated / Type B options are selected. PB and Terminations are typed in.

- Transmission Zeros are typed in. We observe the formation of upper stopband with about 100 dB suppression.

- Transmission zeros can be extracted in different orders so leading to different realizations using CT and CQ sections. In Figure 17.129.a the TZ’s are extracted in such a manner as to realize the FTZ’s using only CT sections.

- The command Pi-Tee-L / Equal Shunt Inductors is applied leading to coupled resonator topology with shunt LC resonators having equal inductors (Figure 17.129.b). The FTZ elements are in the form of series arm parallel LC elements.

- With cursor under the first FTZ element on left, the command CT-CQ-CN Oprs / Convert to CT / Left / CLL is applied to convert the indicated circuit section into CT form. This FTZ is in lower stopband. Hence we select the option CLL where the first letter C denotes the capacitive cross-coupling element (easier to realize than the other option).

- Then with cursor under the other FTZ elements the command CT-CQ-CN Oprs / Convert to CT / Left  is applied to convert the indicated circuit sections into CT form. These FTZ’s are in the upper stopband. Hence the cross-coupling element is of inductive type (LLL type CT sections).

- These conversions destroy the equality of shunt inductors. If desired their equality can be recovered using the commands Pi-Tee-L / Pi-to-Sym Pi and CT-CQ-CN Oprs / Modify CT / Set L2 and Set L3 commands iteratively (Figure 17.129.c-d).

- Thus, synthesis of the receiver filter is completed. However, since we intend to place a common resonator at input of the filter, we need a coupling element to the common resonator. We place an inverter of impedance K=50 ohms as the coupling element. The same inverter is also placed at the output port for coupling to the load impedance. The actual values of the coupling inverters will be found by optimization after the formation of the diplexer. The filter is saved as RX21 (Figure 17.129.e).

Figure 17.129. Design of the receiver filter.

Design of the TX Filter:

Design steps of the receiver filter are shown in Figure 17.130.

- Design / By Synthesis / Lumped / BP / Equiripple / Doubly Terminated / Type B options are selected. PB and Terminations are typed in.

- Transmission Zeros are typed in. We observe the formation of lower stopband with about 100 dB suppression.

- Transmission zeros can be extracted in different orders so leading to different realizations using CT and CQ sections. In Figure 17.130.a the TZ’s are extracted in such a manner as to realize one of the FTZ’s in CT form while the other two will be realized by a CQ section with diagonal cross-coupling.

- The command Pi-Tee-L / Equal Shunt Inductors is applied leading to coupled resonator topology with shunt LC resonators having equal inductors (Figure 17.130.b). The FTZ elements are in the form of series arm parallel LC elements.

- With cursor under the first FTZ element on left, the command CT-CQ-CN Oprs / Convert to CT / Left / CLL is applied to convert the indicated circuit section into CT form. This FTZ is in lower stopband. Hence we select the option CLL where the first letter C denotes the capacitive cross-coupling element (easier to realize than the other option) (Figure 17.130.c).

- The other two FTZ’s will be realized in CQ form. This is done by first bringing the two FTZ elements to be adjacent using the command Double Norton / Inverse Geffe and then combining them to form a fourth order section (Figure 17.130.d). Then with cursor under the fourth order section the command CT-CQ-CN Oprs / Convert to CQ/ Right / Solution-1  is applied to convert the indicated circuit section into CQ form. The resulting CQ section is of CQ13 type where the numbers 13 indicate that there is a diagonal cross-coupling between nodes 1 and 3 of the quadruplet. The convention for node numbering is shown in the inset Figure 17.130.e.

- These conversions destroy the equality of shunt inductors. If desired their equality can be recovered using the commands Pi-Tee-L / Pi-to-Sym Pi commands. We also insert K=50 ohms inverters at source and load ends to be used as coupling elements to the common resonator of the diplexer and to load respectively. The filter is saved as TX21 (Figure 17.130.e).

Figure 17.130. Design of the transmitter filter.

Formation of the diplexer and optimization:

On Filpro a diplexer is described using two different circuits. In one circuit the receiver filter appears explicitly and the transmitter filter appears as External 1-Port and in the other circuit transmitter filter appears explicitly while receiver appears as External 1-Port. Return loss of the diplexer can be plot using the command Plot / Return Loss  command (or using Shift-F2 keys). However since the insertion loss response  involves two channels a special Plot command is prepared: Plot/Tune Multiplexer.  Using this command we can plot the insertion loss or return loss of both channels simultaneously. This command is also used as Tune Element command for tuning the elements of both channels. However the tuned elements of the channel appearing as External 1-Port can not be saved. That is why we need to form two circuits to represent a single diplexer.

Figure 17.131 describes the formation of the two diplexer circuits.

Diplexer circuit-1:

- The circuit of RX21 is loaded (Figure 17.131.a).

- With cursor under the source end inverter the command External 1-Port / TX21 / Shunt 1-Port is applied to insert the transmit filter TX21 as shunt element (Figure 17.131.b).

- Then using Copy and Paste commands we place a resonator at the input to function as the common resonator. The trial value of the inductor of the common resonator should be selected as higher than the shunt inductors of the channel filters. The capacitor of the common resonator should be tuned so as the resonance frequency will be around the cross-over frequency of the channels.

- Also a K=50 ohms inverter is placed at source and to simulate coupling of the common resonator to source (Figure 17.131.c). Both of these two new elements will be tuned for shaping the response of the diplexer.

- Then the first five-six elements of the circuit are tuned roughly while observing the return loss response to both to improve the return loss of the receive channel and also balance the return loss levels of both channels. We tune only the capacitors of the shunt resonators (Figure 17.131.c).

- After a few tuning rounds we delete the external 1-port and save the resulting circuit as DRX22 (Figure 17.131.d). Then delete the input inverter and common resonator and save the circuit as RX22 (Figure 17.131.e).

Diplexer circuit-2:

- The circuit of TX21 is loaded (Figure 17.131.f).

- With cursor under the source end inverter the command External 1-Port / TX22 / Shunt 1-Port is applied to insert the tuned receive filter RX22 as shunt element.

- Then using Copy and Paste commands we copy the tuned input inverter and the common resonator at the input of DRX22 and paste it at the input of the new diplexer circuit (Figure 17.131.g).

- Then the first five-six elements of the circuit are tuned roughly while observing the return loss response to both to improve the return loss of the transmit channel and also balance the return loss levels of both channels. After a few tuning rounds we delete the external 1-port and save the resulting circuit as DTX22 (Figure 17.131.h). Then delete the input inverter and common resonator and save the circuit as TX22 (Figure 17.131.i).

- This cycle is repeated by alternating the two diplexer circuits, by tuning more and more elements at each round. The cross-coupling elements of the CT and CQ sections are not tuned to avoid shifts in FTZ’s. We tune only the capacitors of the shunt resonators. Manual tuning is continued until almost all return loss peaks are roughly uncovered. The peaks near the edges may be difficult to uncover. After uncovering as many return loss peaks as possible, iterations may be continued by using Monte Carlo-Best Case optimization routine.

- Experiments show that the initial trial values of the common resonator inductors may lead to optimized return loss levels which differ by about 1-2 dB. Therefore it may be advisable to repeat the optimization trials with different initial common resonator inductances.

- Figures 17.131.k-to-n show the results of one optimization trial. Return loss level in both channels are about 23 dB. Further continuation of optimization trials would yield better return loss levels. Insertion loss responses show extra FTZ’s due to interaction of the two filters.

- Figures 17.131.q-to-t show the Coupling Coefficients, Resonance Frequencies and External Q of the circuits.

- In Figure 17.131.u the coupling coefficients and resonance frequencies are compared with the exact synthesis results of Macchiarella. There are 0-to-5 % differences between results. Assuming the response of the approximate approach as satisfactory, 5% difference may be treated as percentage tolerance on these parameters. (??).

- In Figure 17.132, conversion of the lumped LC resonators into capacitively loaded SC stub  resonators (coaxial TEM resonators) with characteristic impedance of (eg) 70 ohms.

- The circuit of DRX22 is loaded (Figure 17.132.a).

- With cursor under the shunt LC resonators, the command Replace by Stub Eqv. / Par LC-to-Cap+SC Stub / Specify fq=3761 MHz is applied. Here, fq is the quarter wavelength frequency of the SC stubs. It is taken as twice the center frequency fo=1880.5 MHz of the receiver passband. Thus, the LC resonators are converted into capacitively loaded SC stub resonators (Figure 17.132.b).

- Next, characteristic impedances of the SC stubs are adjusted to (eg) 70 ohms level. This is done in two stages, as follows.

- With cursor under the source end inverter the command Inverters / Scale Inverter / Specify SC Stub: Zc=70 ohms is applied. This command scales up the inverter to produce a transformer on its right which  wll scale the SC stub to 70 ohms after shifting the transformer to load end using Simplify Circuit command. Then with cursor under the second inverter, the command Inverters / Equate All Elements / Equate SC Stubs is applied. This command equates the SC stub impedances on the two sides of that inverter by producing a proper transformer. Simplify Circuit command shifts this transformer to load end making all shunt SC stubs 70 ohms (Figure 17.132.c).

- The same commands are also applied to the circuit of DTX22 (Figure 17.132.d-e-f).

- Then diplexers are formed again and responses are plot as shown in Figures 17.132.g-h-i-k. Return loss response degrades slightly because of these conversions. However a slight tuning can recover return loss to a level which is close to 24 dB, higher than the specified 23.5 dB (Figure 17.132.m-n-p).

Figure 17.131. Formation and optimization of diplexer.

Figure 17.131. Formation and optimization of diplexer.

Figure 17.131. Formation and optimization of diplexer.

Figure 17.131. Formation and optimization of diplexer.

Figure 17.131. Formation and optimization of diplexer.

Figure 17.132. Conversion of LC resonators into capacitively loaded SC stub resonators.

Figure 17.132. Conversion of LC resonators into capacitively loaded SC stub resonators.

Figure 17.132. Conversion of LC resonators into capacitively loaded SC stub resonators.

Figure 17.132. Conversion of LC resonators into capacitively loaded SC stub resonators.