17. 26. MANIFOLD TYPE STRIPLINE MULTIPLEXERS

When number of channels to be paralleled increases it may become difficult to connect them at a single junction. In striplie-like structures channels can be placed along a manifold as shown in Figure 17.133. In this structure the channel filters are interconnected through Tee-junctions. The channels are thus separated physically by some length of lines (or waveguides) which may also function as parts of immitance compensation elements. Channel filters are shown as shunt connected External One-Port elements. In order to form the targeted channel responses optimization is needed on almost all elements of all channels together with the interconnecting lines. Optimization may be carried out by using Tune-Plot Multiplexer command to a limited extent. Filpro can display return loss of the whole multiplexer and insertion losses of all channels simultaneously using the command Tune/Plot Multiplexer. {When tune multiplexer option is selected on an external one port, the selected channel circuit is displayed and any element of this circuit can be tuned. However the final (tuned) value of this channel can’t be saved automatically. That is the automatic save key F2 doesn’t work for such circuits. Therefore the final value should be noted, the circuit of the channel should be loaded separately and the element value modification should be done on this circuit}.

Figure 17.133 A manifold (extended junction) multiplexer.

As a demonstration of capabilities and limitations of Filpro on this subject, let’s consider a simple example, starting from a diplexer and then adding a third channel.

EXAMPLE 17. 46. A manifold type stripline triplexer

Let’s target a triplexer with the channel filters in Minnis Class-D form of degree 14 and passband ripple of 0.03 dB (See Chapter 13). Channel filter band edges are:

Channel-1:   fp1=3860 MHz,  fp2=3940 MHz.

Channel-2    fp1=3960 MHz,  fp2=4040 MHz.

Channel-3:   fp1=4060 MHz,  fp2=4140 MHz.

Filters are designed using Synthesis approach with

Nzero=1,  Ninf=7,  NUE=6

Quarter wavelength frequency is taken as fq=10000 MHz

nd, redundant unit elements will be inserted from both source and load ends to get reasonable element values.

- Filters are synthesized and redundant unit elements are inserted from source sides. Then the OC stub Pi-sections are distributed between unit elements using the procedure of Minnis Class-D filters described in Chapter 13. We get the UE+OC stub Pi section topology shown in Figure 17.134a. The OC stub Pi sections are then converted into open circuited comb type parallel coupled lines. The circuits and element values for the three channels are shown in Figure 17.134.b.

- Figure 17.134. Synthesized channel filters.

- Let’s first form a diplexer using channel-1 and channel-2:

- An extra 50 ohms transmission line piece is added at inputs of both channels (Figure 17.135). Lengths of these lines are adjusted using Tune Element command while observing Smith Chart such that each filter will show highest input reactance in the passband of other channel. This way, interactions of channels will be minimized. These line pieces may also be used as extra parameters in optimizing the whole multiplexer. The diplexer circuits are formed by inserting channel-2 circuit into channel-1 and vice versa, as External One Port circuits, as described in Figure 17.134.a and b. In realization, the extra line pieces may form a Y or Tee junction whose junction discontinuities should also be taken into account in the circuit model.

- Figure 17.135. Tee junction diplexers formed by inserting one channel as External One Port into the other.

- Insertion and return loss responses of diplexer after tuning the lengths of extra lines only are shown in Figure 17.136. It is seen that simply by adjusting the line lengths we got about 20 dB of return loss in both channels.

- Figure 17.136. Insertion and return loss response after tuning the extra line lengths at inputs.

- Then the impedances of all elements of both filters are adjusted while observing the return loss of the complete diplexer, giving close to 30 dB return loss shown in Figure 17.137. Tuned element values are shown in Figure 17.137. It seems that design of such diplexers is straightforward using Filpro.

- Figure 17.137. Insertion and return loss response after tuning elements of both filters.

- Next, we will form the triplexer, using the following procedure:

- The third channel which occupies the 4060-4140 MHz band will be inserted at the load side of the diplexer.

- Using the same reasoning as in forming diplexer, we add a piece of 50 ohms line at the input of diplexer with length adjusted such that it will present a high reactance in the passband of the third channel. This line will be part of the manifold.

- Similarly, a piece of line is added at the input of third channel and its length is adjusted such that it will present a high input reactance in the bands of the first two channel (3860-4040 MHz). Since this band is wider compared to the single channel cases, compensation effect may not be as good as in the diplexer case.

- The channels are interconnected as described in Figure 17.138 where channels appear as External One Port circuits. Transmission line pieces are inserted between channel filters to act as the manifold. Ordering of channels along the manifold is selected randomly (not much reasoning- it may be changed to see if better alternatives can be produced or not-Exercise?). End of manifold is simulated by open circuit. In fact manifold ends in Channel-1. Therefore the last manifold line piece is effectively a piece of channel-1. Actually, it is the line piece added to the input of Channel-1 filter.

- Figure 17.138. Connection of channels to the manifold.

- Initial manifold line lengths are selected as half wavelength at 4000 MHz, the center frequency of multiplexer, so that filters will be effectively parallel connected.

- The manifold line lengths are tuned while observing the return loss of the whole multiplexer.

- Then, Channel-1 is deleted and reinserted using Append Filter command, giving the circuit shown in Figure 17.139.a.

- Figure 17.139.a) Triplexer with channel-1 circuit being explicitly displayed. b) Triplexer with channel-2 circuit being explicitly displayed.

- Elements of Channel-1 are tuned by observing return loss of the whole triplexer displaying all three channel responses simultaneously. It is saved by separating the channel circuit from the other channels.

- Then, Channel-2 circuit is appended while Channel-1 is reinserted as External One Port  (Figure 17.139.b). In this case the elements of Channel-2 circuit are available for tuning. They are also tuned while observing return loss of the triplexer and saved.

- Unfortunately, Channel-3 cannot be displayed explicitly for tuning in this version of Filpro. A general purpose analysis program suits better for analyzing such multiplexers.

- Even in this rough manual optimization, we got the response shown in Figure 17.140 in the first tour, which is not bad at all. If channel-3 were available explicitly for tuning, then we could optimize the whole response. It seems that an optimization software would give the targeted circuit quickly.

- Figure 17.140. Return loss of triplexer after one tour of manual optimization on Channel-1 and Channel-2.

17. 27. LP-HP  LADDER TYPE  MULTIPLEXERS IN STRIPLINE STRUCTURES

Doubly terminated filters with contributing unit elements and finite transmission zeros may also be used to form LP-HP multiplexers with very selective channel characteristics. Design requires trial-error type experimentation and tuning of element values. No attempt is made yet for a systematic design procedure. The following triplexer is a demonstration of “brute force manual tuning” example to study the nature of problem. It had been observed that tuning the first few elements of channels is very effective in shaping the responses of channels [19].

EXAMPLE 17. 47

Let’s design a triplexer having  the following channel characteristics:

Channel-1: fp1=0,  fp2=4250 MHz.

Channel-2: fp1=4250 MHz,  fp2=8600 MHz.

Channel-3: fp1=8700 MHz, fp2=18 GHz.

Return loss will be greater than 10 dB in the whole band up to 18 GHz.

Diplexer-1 will have cross-over at f =4250 MHz and diplexer-2 will have cross-over at 8500 MHz.

Channel filters will have high selectivity and degrees n13.

Structure of triplexer will be as shown in Figure 17.141.

Figure 17.141. Cascaded LP-HP triplexer with doubly terminated filters.

- Design steps of the lowpass channel of the first diplexer (low channel with f <4250 MHz) are shown in Figure 17.142.

- The passband edge is taken as fp=4200 MHz and passband ripple is taken as 0.05 dB to leave margin for manual optimization. Quarter wavelength frequency is taken as fq =10000 MHz.

- A doubly terminated distributed element lowpass filter is designed with one transmission zero at infinity, six contributing unit elements and three finite transmission zeros, all at f =5000 MHz.

- Transmission zeros are extracted in the order shown in the figure, in a way to have at least one UE adjacent to each series arm SC stub. Synthesized circuit is shown at the bottom of Figure 17.142.

- The series arm SC stubs are converted into shunt arm OC stubs using Kuroda / Right-Left / Full Stub commands (Figure 17.143.a).

- The shunt arm second order stub resonators are approximated by uniform single section OC stubs using command Replace By Shunt OC Stub / Single Stub giving the circuit of Figure 17.143.b. This circuit will be used as the initial trial filter for the lowpass channel of 4250 MHz. It is saved as LP0-4.

- Next, a highpass filter with fp=4300 MHz and fq=7500 MHz is designed. fq is set after a few synthesis trials by inspecting realizability of resulting filter.

- Degree of the filter is selected as N=11, as a trial. Command set for design is shown in Figure 17.144.

- One transmission zero is placed at zero, three finite transmission zeros are placed, all at 3750 MHz and four contributing unit elements are placed.

- Transmission zero extraction order is shown in the same figure. Elements are extracted in such a way as there will be a unit element adjacent to each series arm OC stub. Synthesized circuit is shown in Figure 17.145.a.

- With cursor under unit elements, the command “Kuroda / Right-Left / Equal Stub” is applied to form  Series OC Stub+Unit Element+Series OC Stub” triplets (Figure 17.145.b-c).

- The second order stub resonators are replaced by single open circuited stubs using command “Replace By Shunt OC Stub / Single Stub” (Figure 17.145.d).

- With cursor under unit elements, the command “Replace By PCL / Interdigital Type Lines / ococINTD / Symmetric” is applied to convert the OC stub+UE+OC Stub triplets into open circuited parallel coupled lines (Figure 17.145.e). This filter will be used as an initial trial for optimizations. It is saved as HP4-8.

Figure 17.142. Design steps of the 4200 MHz lowpass filter.

Figure 17.143. Circuit transformations on the 4200 MHz lowpass filter.

Figure 17.144. Design steps of highpass filter with fp=4300 MHz.

Figure 17.145. Circuit transformations for the highpass channel filter.

- Next, the lowpass and highpass filters are combined  to form the diplexer. In Figures 17.146.a-b, the lowpass channel is inserted into the highpass channel as an External One Port circuit using command External One Port / Shunt One Port / LPCH1. In this configuration, return loss of the diplexer and insertion loss of highpass channel can be observed and tuned by applying Tune Element command on elements of the highpass channel. We may as well use the command  Multiplexer / Tune and Plot Response to tune elements of both the displayed channel and the External One Port circuit while observing Return Loss or Insertion loss of all channels simultaneously.

Figure 17.146. Diplexer-1 with highpass channel being explicitly shown.

- In Figure 17.147.a-b, now the highpass filter is inserted into the lowpass filter. We may apply tuning procedures on this circuit as well.

Figure 17.147. Diplexer-1 with lowpass channel being explicit.

- Figure 17.148 shows the return loss of diplexer and insertion losses of channels when the two channels are combined without any tuning. Clearly, the result is not a success. Let’s try some cut and try approach with brute force tuning of element values using Tune Element command.

- All elements are tuned including length of lines while observing return loss and sometimes insertion loss. It was found that deleting the first shunt OC stub of lowpass filter eases tuning process a lot.

Figure 17.148. Initial return loss and insertion loss of diplexer-1 (without tuning).

- After a few rounds of tuning, the responses shown in Figure 17.149 are obtained which seems satisfactory.  The final element values are shown in Figure 17.150.

Figure 17.150. Optimized responses of diplexer-1.

Figure 17.150. Element values of optimized channel filters.

- The second diplexer is also designed and tuned using the same approach. The lowpass channel is designed with fp=8400 MHz while the highpass channel is designed with fp=8600 MHz. Quarter wavelength frequency for the lowpass filter is taken as fq=20000 MHz while for highpass filter it is taken as fq=15000 MHz. The optimized channel filters are shown in Figure 17.151. They are saved as LP0-8 and HP8-18 respectively.

- Now we are ready to form the triplexer circuits in different configurations to observe insertion losses of various channels.

Figure 17.151. Optimized channel filters of Diplexer-2.

- In Figure 17.152.a, the 8 GHz lowpass filter is displayed. The associated 8 GHz highpass filter is inserted into the left of this circuit as External One Port, giving the second diplexer (Figure 17.152.b).

- Then the 4 GHz highpass filter is inserted into the right of 8 GHz lowpass filter as an External One Port element (Figure 17.152.c).

- Finally, the 4 GHz lowpass filter is appended to the right, as shown in Figure 17.152.d.

- Return loss of this circuit is the required return loss of triplexer (Figure 17.153). It is seen that return loss is better than 10 dB everywhere up to 18 GHz.

- This configuration and other possible configurations for plotting insertion losses of one channel at a time are shown in Figure 17.154.

- Output of the circuit shown in Figure 17.154.a gives the insertion loss response of the appended 4 GHz lowpass channel.

- Insertion loss response of 4 GHz highpass filter (actually 4-8 GHz bandpass channel) can be observed in a similar way, by appending this channel to the right of all other channels, as described in Figure 17.154.b. Its insertion loss is overplot with that of Channel-1 (Figure 17.155).

- Insertion losses of external one ports at internal points of the multiplexer can’t be plot alone. Their insertion losses should be displayed together with all other channels using the command Multiplexers in tuning or plotting mode.

- The design may be accepted as successful as a starting point for reoptimization by applying a more professional optimizer. This structure may also be used to set the physical dimensions of the actual structure to be constructed and then an optimizer working in electromagnetic field domain may be applied to speed up the design.

Figure 17.152. Formation of triplexer by combining filters to observe output of 4 GHz lowpass filter.

Figure 17.153. Return loss of triplexer.

Figure 17.154. Circuit configurations for observing insertion losses of various channels.

Figure 17.155. Insertion loss responses of channels.

17. 27. HYBRID CHANNEL MULTIPLEXERS

Channel filters involving directional couplers (3 dB hybrids) are advantageous for ease of integration into multiplexer when number of channels is large 4, 43, 44. A hybrid coupled manifold multiplexer is shown in Figure 17.156. Each channel is made up of two 3 dB hybrids and two identical filters. Directional filters extract the signals within their passbands from the manifold and direct them to an output port, forming one channel of the multiplexer. Modularity of multiplexer is clear. New channels may be added easily without effecting the other channels, if all channel bandwidths are within the 3 dB coupling and isolation bandwidths of the couplers.

Figure 17.156. Hybrid channel multiplexer.

In order to clarify operation of channels, let’s first review basics of directional filters on a 3 dB branchline directional coupler with two branches. The number of branches may be increased depending on requirements and constraints.

a)  3 dB impedance transforming branchline couplers with ideal inverters.

A schematic drawing of a symmetric, impedance transforming two branch coupler is shown in Figure 17.157. The convention for port numbering and signals of these ports when input is applied to port-1 is as follows:

Port-1: Input signal port with termination resistance R1.

Port-2: Transmitted signal port with termination resistance R2.

Port-3: Coupled signal port with termination resistance R2.

Port-4: Isolated port with termination resistance R1.

Figure 17.157. Schematics of a  3 dB impedance transforming branchline coupler with different input-output terminations.

If the phase of transmitted port signal (Port-2) is taken as reference (0 degree), then the phase of coupled port signal is -90 degrees (Port-3). The arms are formed by ideal inverters. In order that this device work as a 3 dB coupler, the inverter impedances should satisfy the following relations:

Branch -1 impedance:  K1=R1

Branch -2 impedance:  K2=R2

Main arm impedances: K12=(R1*R2/2)1/2

If R1  = R2 , then the structure will have double symmetry.

In multiplexer applications, filters will appear as loads R2. If the transmitted and coupled port (Ports 2 and 3) terminations are different from the specified matched termination R2, then reflections will occur from these ports. If reflections are identical, then they will be directed to the isolated port (Port-4). No reflections will appear at input port. This way, the signals falling into the passband of filters connected to ports 2 and 3 will pass to the output of filters to be combined by the output hybrid. The signals outside the passband will see identical reactive mismatches at ports 2 and 3, thus will be directed to the isolated port (Port-4) and will be delivered to the next hybrid channel.

In usual directional coupler applications ideal inverters are replaced by quarter wavelength transmission lines and the device will work as a coupler within a band around the quarter wavelength frequency. In order to handle narrowband filters without a separate impedance matching section, the coupler itself may be designed as an impedance transforming one. If inverter impedances are too high or too low to be practical for realization as transmission line pieces, then ideal inverters can be replaced by other suitable equivalent practical inverter circuits. Then, the bandwidth of coupler will be set by these approximations.

Since the circuit is symmetric about the horizontal midplane, it can be analyzed using Even-Odd mode decompositions. As an example, let’s analyze a coupler with  R1=50 ohms and R2=500 ohms leading to         K1 =50 ohms, K2 =500 ohms  and K12=111.80339 ohms. In order to split coupling inverters into even and odd halves, we can use the constant reactance Pi or Tee models, as shown in Figure 17.143. In this figure, Pi model is used. It is seen that the even half is a constant reactance with impedance equal to negative of inverter constant while the odd half reactance is equal to the inverter constant itself.

Using this model, the even and odd halves of branch line coupler can be formed as shown in Figure 17.158.

Figure 17.158. Even and odd halves of an inverter using the constant reactance Pi model of inverter.

Figure 17.158. Even and odd halves of branch line coupler.

The even and odd mode halves of the coupler can be formed in Filpro using Design / By Inserting Elements approach  as shown in Figure 17.159. They are saved as EVEN1 and ODD1.

Figure 17.159. Even and odd halves of the coupler in Filpro.

- After forming the even and odd halves, the complete circuit can be analyzed using the command sets shown in Figure 17.160.

Figure 17.160. Command sets for analysis and tuning of directional coupler.

Figure 17.161. Amplitude display for S11  , S21  , S31 and S41  as R2 is changed.

- The coupler is analyzed as a symmetric cross-coupled circuit with command set Design / Symmetric Cross-coupler Ccts / Analyze Branch Line Couplers.

- At this step, file names of even and odd halves are asked by Filpro. After typing in the filenames (Even1 and Odd1), Filpro asks to select one of the two options: Plot Response Only and Tune Even-Odd Circuit Elements. Tune option is used for optimizing response by varying some elements of the circuit. In this experiment, only R2  is changed just as a demonstration (Figure 17.161).

- After choosing Tune option, first the even half circuit is displayed. The element to be tuned is pointed by the cursor and Enter key is used to record that element. Then the odd half circuit is displayed and the same element to be tuned is pointed in this circuit also. If the two pointed elements are the same, then Filpro asks the increment to be used in tuning.

- (Note: It is also possible to tune two different elements from the even and odd half circuits. If the two pointed elements in the two circuits are not the same elements, then tuning is done by using Esc key and typing in the new value of elements and pressing Enter.)

- For analysis of amplitudes, the Insertion Loss option is chosen. Amplitudes of all four signals, S11 , S21, S31 and S41 are displayed. Cursor moves only on S31. Note that return loss (S11) doesn’t appear in display because it is infinite for ideal impedance inverters.

- For analysis of phase and delay, the options Insertion Phase and Delay are chosen respectively. In phase response, besides the phases of S21 and S31 , their difference is also displayed.

b) Hybrid channel.

Channel modules are made up of two identical filters and two identical branch line (or other type) directional couplers, interconnected as described in Figure 17.162. If the directional couplers are of impedance transforming type, then they are placed in mirror image fashion so that the filters will operate into identical impedances. Inspection of phases of signals flowing to port-2’ and port-3’ shows that when the frequency of signal is in the passband of the filter, the whole power will come out of the output port-3. For signals in stopbands, filters will show reactive input impedance, thus they will be reflected and collected at the isolated port (Port-4 ). This way, the desired channel will be “dropped” at port-3 while all the rest will come out from the isolated port and transferred to the next directional filter.

Figure 17.162. A hybrid channel.

Let’s continue with an example.

EXAMPLE 17. 47. A channel dropping filter with directional couplers.

Let’s design a directional filter to drop the channel from 2000 MHz to 2250 MHz with a high selectivity. In Figure 17.163.a, a Minnis Class-B filter is shown, designed using the procedure described in Chapter 13 to operate into 50 ohms terminations.

Figure 17.163. a) A Minnis Type-B filter. b) Even and odd halves of 50 ohm branchline couplers. c) Even half of directional filter. d) Odd half of directional filter.

It has one transmission zero at both f=0 and  f=. Four finite transmission zeros are used, two at 1900 MHz and two at 2350 MHz. There are four contributing unit elements. Passband ripple is 0.01 dB (26 dB return loss). This filter will be placed in between two identical branchline couplers. The branchline coupler need not be an impedance transforming one. It is designed using the approach described above and saved as Even1 and Odd1. In Figure 17.163.b, the even and odd mode halves of that coupler are shown. In Figure 17.163.c, the filter is placed in between the even halves of coupler, forming the even half of directional filter, saved as Even2. The same filter is also placed in between the odd halves of the coupler to form the odd half of directional filter, saved as Odd2. Now we are ready to analyze this new four port in Filpro as if it is a single branch line coupler, using the same command set shown in Figure 17.160, with file names changed as Even2 and Odd2. Amplitudes of signals emerging from ports 1, 2, 3 and 4 are shown in Figure 17.164.

Figure 17.164. Amplitudes of signals emerging out of the four ports of the directional filter.

The device works as expected:

- The passband of filter appears between port-1 and Port-3. That is, the signals in frequency range 2000-2250 MHz are directed to port-3 ( signal S31 in the figure). At Port-3, signals outside this band are attenuated by more than 60 dB, the minimum stopband attenuation of the filters.

- The signals inside the pasband of the filters are attenuated by about 26 dB at the isolated port (Port-4, signal S14). This amount is exactly the return loss of filters. Signals outside the passband flow to Port-4 without any attenuation, back to manifold, to be delivered to other channel filters.

- The signals reaching to Port-2 (S21) and reflected signal  (S11) are attenuated heavily, about 90 dB, at all frequencies.

- However, if the ideal inverter is replaced by a transmission line which is quarter wavelength long at band center, f=2125 MHz and also the negative constant reactances by open circuited stubs and positive constant reactances by short circuited stubs of length half of the transmission line replacing the ideal inverter, heavy deformations in response will be observed at out of band frequencies, as shown in Figure 17.165. This is because the actual branchline coupler is a narrowband device. Wideband 3 dB couplers are required for this operation (e.g. parallel coupled line couplers) or the channels should be much narrower.

- Figure 17.165. Response of directional filter after the ideal inverters and constant reactances replaced by practical elements.

REFERENCES AND SUGGESTED READING:

- R.G.Veltrop and R.B.Wilds, “Modified Tables for the Design of Optimum Diplexers”, The Microwave Journal, 76, June 1964.

- R. J. Wenzel, “Applications of Exact Synthesis Methods to Multichannel Filter Design”, IEEE Trans. MTT-13, No. 1 , pp. 5 - 15, Jan. 1965.

- R. J. Wenzel, “Wideband High-Selectivity Diplexers Utilizing Digital Elliptic Filters”, IEEE Trans. MTT- 15, No. 12, pp. 669 - 680, Dec. 1967.

- G. L. Matthaei and E. G. Crystal: “ Theory and Design of Diplexers and Multiplexers”, Advances in Microwaves, V. 2, L. Young, Editor, Academic Press, 1967.

- R. J. Wenzel, “Some General Properties of Commensurate Linelength Complementary and pseudo- complementary Microwave Filters”, IEEE Trans. MTT-16, No. 3, pp. 191-192,  March 1968.

- R. J. Wenzel, “ Printed Circuit Complementary Filters for Narrow Bandwidth Multiplexers”, IEEE Trans. MTT- 16, No. 3, pp. 147-157, March 1968.

- E. G. Cristal and U. H. Gysel, “New Compact Multiplexer for Stripline and MIC”, 1973 European Microwave Conference, B3.1, 1973.

- C. K. Mok, “Design of Evanescent mode Waveguide Diplexers”, IEEE Trans. MTT, Vol. 21, pp.43-48, Jan. 1973.

- A. E. Atia, “Computer Aided Design of Waveguide Multiplexers”, IEEE Trans. MTT, Vol. 22, pp.332-338, March 1974.

- J. D. Rhodes, “Design Formulas for Bandpass Channel Diplexers”, IEEE Proc. PGMTT Int. Symp., pp. 112- 115, June 1976.

- J. D. Rhodes, “Direct Design of Symmetrical Interacting Bandpass Channel Diplexers” IEE Journal on Microwaves, Optics and Acoustics”, Vol. 1, pp. 33-40, Sept. 1976.

- J. L. Haines and J. D. Rhodes, “Direct Design Formulas for Asymmetric Bandpass Channel Diplexers”, IEEE Trans. MTT, Vol. 25, pp. 807-813, Oct. 1977.

- R. J. Wenzel and W. G. Erlinger, “Narrowband Contiguous Multiplexing Filters with Arbitrary Amplitude and Phase Response”, IEEE Proc. PGMTT Int. Symp., pp. 116- 118, June 1976.

- P. M. La Tourette, “Multi-octave Combline Filter Multiplexers”, IEEE MTT-S Symp. Digest, pp. 298-301, 1977.

- P. M. La Tourette, “Combline Filter Multiplexers”, Microwave Journal, pp. 55-59, Aug. 1977.

- P. M. La Tourette and J. L. Roberts, “Extended junction Multiplexers”, IEEE MTT-S Symp. Digest, pp. 214 - 216, 1978.

- J. A. G. Malherbe, “Microwave Transmission Line Filters”, Artech House, Dedham, 1980.

- J. D. Rhodes and R. Levy, “A Generalized Multiplexer Theory”, IEEE Trans. MTT, Vol. MTT-27, pp. 99-111, Feb. 1979.

- J. D. Rhodes and R. Levy, “Design of General Manifold Multiplexer”, IEEE Trans. MTT, Vol. MTT-27, pp. 111- 123, Feb. 1979.

- J. D. Rhodes and J. E. Dean, “MIC Broadband Filters and Multiplexers”, 1979 European Microwave Conference Proc., pp. 407-410, 1979.

- S. A. Alseyab, J. D. Rhodes, “Design Procedure for multioctave combline filter multiplexers”, IEE Proc., Vol. 127, Pt. H, No. 6, Dec. 1980.

- J. Neirynck, C. H. Carlin, “Synthesis of Lossless Reciprocal Three Port Based on a Canonic form of Its Scattering Matrix”, IEEE Trans. on Circuits and Systems, Vol. 28, 736-752, July 1981.

- H. L. Thal, “Transmit-receive multiplexer for the 12-14 GHz band”, IEEE Trans. MTT, Vol. 30, pp. 1324-1330, Sept. 1982.

- R. G. Egri, A. E. Williams and A. E. Atia, “A Contiguous band multiplexer design”, IEEE MTT-S Digest, pp. 86-88, 1983.

- M. H. Chen, “A 12 channel contiguous band multiplexer at Ku-band”, IEEE  MTT-S Digest, pp. 77-79, 1983.

- S. C. Holme, “A 12 GHz 12 channel contiguous multiplexer for satellite applications ”, IEEE  MTT-S Digest, pp. 295-296, 1984.

- R. Tong and D. Smith, “A 12 channel contiguous band multiplexer for satellite application”, IEEE   MTT-S Digest, pp. 297-298, 1984.

- J. W. Bandler, W. Kellerman and K. Madsen, “A superlinearly convergent minimax algorithm for microwave circuits design”, IEEE Trans. MTT, Vol. 33, pp. 1519- 1530, Dec. 1985.

- J. W. Bandler, S. H. Chen, S. Daijavad, W. Kellerman, M. Renault and Q. J. Zhang, “Large scale minimax optimization of microwave multiplexers ”, Proc. Eu. Mic. Conf. pp. 435-440, Dublin, Sept. 1986.

- R. Valdhieck and B. Varailhon, “Computer Aided Design of Parallel Connected Millimeter-wave Diplexers, Multiplexers”, IEEE MTT-S Dig., pp. 435- 438, 1988.

- J. Dittloff and F. Arndt, “Rigorous Design of Septate Multiplexers with Printed Circuit Elements”, IEEE MTT-S Dig., pp.431-434, 1988.

- Y. S. Zhu, W. K. Chen, “On the Design of Diplexer Having Butterworth Characteristics”, IEEE Trans. on Circuits and Systems, Vol. 35, 1455-1457, Nov. 1988.

- Z. M. Wang and W. K. Chen, “The Design of Symmetrical Diplexer Composed of Canonical Butterworth Two-Port Networks”, Proc. IEEE International Sym. On Circuits and Systems, 1179, 1988.

- J. Dittloff and F. Arndt, “Rigorous Field Theory Design of Millimeter-wave E-plane Integrated  Circuit Multiplexers”, IEEE Trans. MTT, vol. 37, pp. 340-350, Feb. 1989.

- Y. S. Zhu, W. K. Chen, “A Theorem on Complex normalized Reflection Coefficient and its applications”, IEEE Trans. on Circuits and Systems, Vol. 36, 1238-1244, Sept. 1989.

- D. C. Youla and F. Winter, “New Theorems in the Synthesis of Passive Multiplexers”, Int. Journal of Circuit Theory and Applications, Vol. 19, pp. 341- 364, 1991.

- X. P. Liang, K. A. Zaki and A. E. Atia, “A rigorous three plane mode matching technique for characterizing waveguide T-junctions and its applications in multiplexer design”, IEEE Trans. MTT, Vol. 39, pp. 2138-2147, Dec. 1991.

- D. C. Youla, S. U. Pillai, F. Winter, “Theory and Design of Maximally Flat Lowpass Highpass Reactance Ladder Diplexers”, IEEE Trans. on Circuits and Systems, Vol. 39, 337-350, May 1992.

- X. P. Liang, K. A. Zaki and A. E. Atia, “Channel expansion and tolerance analysis of waveguide manifold multiplexers”, IEEE Trans. MTT, Vol. 40, pp. 1591- 1594, July 1992.

- M. Guglielmi, “Optimum CAD procedure for manifold diplexers”, IEEE, MTT-S International Microwave Symposium Digest, pp. 1081-1084, June 1993.

- A. Morini and T. Rozzi, “Design of Optimum Three-port Symmetrical Junctions for Diplexer applications”, IEEE MTT-S Dig., pp. 739- 742, 1994.

- A. Morini and T. Rozzi, “Constraints to the Optimum Performance and Bandwidth Limitations of Diplexers Employing Symmetric Three-port Junctions”, IEEE Trans. MTT, vol. 44, no. 2, pp. 242- 248, Feb. 1996.

- O. Apaydin, “Novel Methods for Synthesis of Diplexers with Adjustable Crossover Attenuation”, M. Sc. Thesis, EEE Dept. Middle East Technical University, Ankara,Turkey, 1994.

- U. Koyaz, “Synthesis of Lumped and Distributed Filters and Diplexers”, Ph. D. Thesis, EEE Dept. Middle East Technical University, Ankara,Turkey, 1994.

- R. R. Mansour, S. Ye, V. Dokas, B. Dolley, G. Thomson, W. C. Tang, C. Kudsia, “Design Considerations of Superconductive Input Multiplexers for Satellite Applications”, IEEE Trans. MTT-44, No. 7, pp. 1213-1228, July 1996.

- S. H. Talisa, M. A. Janocko, D. L. Meier, J. Talvacchio, C. Moskowitz, D. C. Buck, R. S. Nye, S. J. Pieseski, G. R. Wagner, “High Temperature Superconducting Space Qualified Multiplexers and Delay Lines”, IEEE Trans. MTT-44, No. 7, pp. 1229-1239, July 1996.

- G. Macchiarella, S. Tamiazzo, “A novel approach to the design of microwave diplexers”, (to be published) (MTT-S, 2006).