CHAPTER 18

LUMPED ELEMENT IMPEDANCE MATCHING TECHNIQUES

18.1. INTRODUCTION

Impedance matching between a real or complex source and a real or complex load over a band of frequency is one of the basic historical problems of communication systems. Fundamental theories and practical approaches form broad subjects which can not be included in this software in its full generality [1]-[3]. However some classical elementary matching problems encountered in everyday practice of engineers [4]-[11] can be solved easily using Filpro. Matching structures can also be treated as special cases of lowpass, highpass or bandpass filters in which the reactive parts of source and load impedances can be absorbed in the first and last elements of the filter, if possible. Some examples were already presented in the previous chapters. However in most simple matching problems filter approach is an unnecessary luxury.

Consider a resistive load RL = 50  to be matched to a resistive source RS = 10 at and around f =100 MHz The simplest solution is, if allowed, of course to use an ideal transformer of turns ratio                     					n= ( RL / RS   )1/2  = 2.236

Figure 18.1. Frequency independent matching with ideal transformer.

giving frequency independent matching (Figure 18.1). However this is rarely allowed. Therefore we have to simulate the action of transformer by lumped L-C elements, resulting in impedance matching within some finite frequency ranges. For narrowband matching problems it is usually preferred (or enforced) to use minimum number of elements. Increasing the number of elements provides additional degree of freedom which can be used in various ways, like for example, a wider matching bandwidth, more convenient element values, less dissipation loss, etc. In Filpro, the command “Impedance Matching / Match Real RS-to-RL” can be used to generate a large number of two and three element matching structures automatically. The choice of matching structure depends on properties of application. The same circuits and some more alternatives may also be found using approaches like manual tuning while observing Insertion Loss, Return Loss or Smith Chart. These trial-error type techniques are very useful to get feeling of effects of elements on response. One may also find some solutions directly by applying the simple random optimization procedure termed as Best Case in Filpro. In this chapter several commonly used single frequency or narrow-medium bandwidth  matching techniques will be described, some one push button type, some using automatic and manual tuning-optimization approaches while some requires experimentation with Synthesis approach. When the number of elements increase, it may be preferable to use filter design approach in which case one sets terminations and desired passband ripple or passband edge loss (if maximally flat response is desired). The passband edge frequencies remain to be unknown because passband ripple, terminations and bandwidth are interrelated. Setting any two determines the third. Some examples will also be given on this approach.

18. 2. MATCHING  REAL RS  AND  RL  USING  IMPEDANCE MATCHING COMMAND

The circuit to be matched can be formed by using command Design/By Inserting Elements as described in Figure 18.2. This command asks terminations RS and RL and displays the RS-RL circuit automatically. Impedance matching elements may be inserted to the right of RS using Insert command (with cursor being under RL). If the command Impedance Matching is used, matching circuit elements are calculated and inserted automatically. Impedance Matching command displays the options shown in Figure 18.3. In matching resistive terminations, the option Match Real RS to RL will be selected. Then, the submenu will appear showing the available matching circuit types: L-Section, Pi-Section, Tee-Section, Inverter and Resistive L-Section. Each option leads to many different solutions. Let’s inspect these options one by one.

Figure 18.2. Formation of RS-RL circuit using Design / By Inserting Elements command.

Example 18. 1: Matching with L-Section:

If the option L-Section is selected then another submenu appears giving the alternatives Solution-1 and Solution-2 (Figure 18.3). Elements of the constant reactance L sections can be replaced by L, C and LC resonators to produce many alternative circuits using Replace Constant Reactance By / ... command as described in Figure 18.4.

Figure 18.3. Command set for finding impedance matching sections and L-Type matching circuits.

Figure 18.5. Replacement of constant reactances by L, C and LC elements.

Figure 18.5 shows the circuits and responses of simple lowpass and highpass type LC sections obtained by replacing positive constant reactances by inductors and negative constant reactances by capacitors.

Figure 18.5. Circuits and responses of simple L Type LC matching sections.

Further alternatives by replacing some constant reactances by series or parallel resonators are shown in Figure 18.6.

Figure 18.6. Impedance matching circuits obtained from Constant Reactance L sections.

Figure 18.6. Impedance matching circuits obtained from Constant Reactance L sections (continued).

Example 18. 2: Resistive L Matching section.

This command inserts a resistive L section between RS and RL (Figure 18.7). This type of matching is used when power loss is not important. LLeft section is obtained if RS>RL and LRight section is obtained for RS<RL. Insertion loss plot shows how many decibels will be lost in the resistances of matching section.

Figure 18.7. Resistive L matching section.

Example 18. 3: Pi Type Matching Circuits.

The command Impedance Matching / Match Real RS-to-RL / Pi Section  offers four alternatives with all elements in constant reactance form as described in Figure 18.8. The circuits may be left in constant reactance form or constant reactances may automatically be replaced by L or C, depending on the signs of constant reactances. The circuits left in constant reactance form may then be converted into more complicated matching sections containing   LC resonators using the command “Replace Constant Reactance By / ...” command. Some LC matching sections are shown in Figure 18.9.

Figure 18.8. Constant Reactance Pi sections.

Figure 18.9. Some Pi-type circuits obtained by replacing constant reactances by inductors, capacitors and LC resonators.

Example 18. 4: Tee Type Matching Circuits.

The command  “Impedance Matching / Match Real RS-to-RL / Tee Section” produces four constant reactance Tee sections as shown in Figure 18.10. Replacement of constant reactances with L, C and LC type elements will yield many alternative circuits some of which are shown in Figure 18.11.

Figure 18.10. Tee type constant reactance matching sections.

Figure 18.11. Some Tee type matching circuits obtained from Constant Reactance Tee circuits.

18. 3. MATCHING  COMPLEX  SOURCE  AND  LOAD

When both source and load impedances are complex, such problems are termed as “double matching” problems. In general, wideband double matching problems require special advanced techniques [1]-[3]. However some elementary complex loads can be matched using simple Pi-Tee-L type sections [4]-[8]. Some typical three element double-matching circuits suiting narrowband applications are shown in Figure 18.12.

Figure 18.12. Some three element double matching circuits.

In these circuits XS and XL are reactive parts of source and load impedances respectively. By increasing the number of elements in the matching circuit, further alternatives can be produced. However three element structures are the most commonly used ones, satisfying most of the narrowband matching requirements. It is seen that all these structures are in the form of Pi, Tee or L-sections. In the figure they are classified in accordance with transmission zeros at f = 0 and f = ¥. Since there are many alternative solutions for a given matching problem, in the initial stages of trying to choose the candidate circuits from those listed in Figure 18.12, one may follow a strategy to get a lowpass, bandpass or highpass type circuit, depending on the preferences arising from realization constraints or depending on the whether the reactances XS and XL are in series or in shunt arms. The automatic matching mode of Filpro through the command Match Complex ZS-to-ZL can then be used to generate the circuits. One may as well start with a desired circuit with arbitrary element values and use either manual optimization approach though Tune Element command or automatic optimization approach through Best Case command to find the element values.

In Filpro, complex source and load impedances must be formed by inserting the reactive parts of ZS and ZL using Insert command. The command  Match Complex ZS to ZL can be applied on limited circuit types, as described in Figure 18.13.a and b. In these circuits, the reactive parts of ZS and ZL are either in series or in parallel with RS and RL. The constant reactances in these circuits represent reactive elements like inductors and capacitors, LC resonators, OC or SC stubs.

Figure 18.13. Complex ZS-ZL type circuits that can be handled by Filpro.

The circuits of Figure 18.13.c and d can’t be handled automatically by Filpro. They have to be matched by the techniques used for matching real RS and RL. In the circuits of Figure 18.13.a and b, the elements on the left of the cursor form XS while those on the right of the cursor, including the one indicated by the cursor form XL. The cursor must be at the junction of reactive parts of ZS and ZL, on ZL side, as described in the example of Figure 18.14. When the command is applied, Filpro calculates the complete reactances in series or in parallel with RS and RL at the frequency specified by the user. The matching circuit type is selected by the user. Depending on values of RS, RL, XS, XL and signs of XS and XL, several solutions may exist or no solution may be found among the offered circuits. In the choice of  Pi-Tee types, if the reactive parts, XS and XL are both in series arms then a Tee circuit is offered automatically. If both XS and XL are in shunt arms then a Pi section is offered. If one of XS and XL is in series arm and the other is in shunt arm, then an LRight or LLeft section should be tried by the user by first eliminating the reactive parts, matching the resistive circuit by a proper L-Type matching section, and then splitting the elements of matching section such that one piece of the splitted sections will represent the reactive parts of load or source.

EXAMPLE 18. 5: Matching Complex Source and Load using L, Pi and Tee Sections.

Consider a complex source load pair with both having series capacitive reactances as shown in Figure 18.13. Application of the command Impedance Matching / Match Complex ZS-to-ZL / LLeft  or LRight offers two alternatives, as Solution-1 and Solution-2. Both solutions are in constant reactance forms. Replacement of constant reactances by L, C and LC sections produce various alternative circuits some of which are shown in Figure 18.14-to-17.

Figure 18.14. LLeft-Solution-1 matching section and some realizations by replacing constant reactances by L, C and LC elements.

Figure 18.15. LLeft-Solution-2 matching section and some realizations.

Figure 18.16. LRight-Solution-1 and some realizations.

Figure 18.17. LRight Solution-2 and some realizations.

The choice of Pi-Tee section yields a Tee section automatically, because both XS and XL are series elements (Figure 18.18). If XS and XL were both shunt elements, then Filpro would offer only a Pi type solution. The option Inverter+jX is a brute force solution where the reactive components of ZS and ZL are canceled by proper constant reactances converting the problem into a resistive matching problem which is then solved by inserting an inverter. The inverter should be replaced by its practical equivalents which usually involves three elements. Thus this matching section will typically have five elements which is not a desirable property. However this approach may also be useful in some trials.

It must be kept in mind that all the possible solutions are not covered by Filpro. Furthermore, the solutions offered automatically by Filpro may not be the best solutions either.

Figure 18.18. A Tee type solution.

Example 18. 6: Complex matching by tuning.

In many applications, trials by selecting a proper circuit from Figure 18.12 and optimizing it by manual iteration using Tune Element command of Filpro may prove to be giving more insight about the nature of matching problem and its solution. Tuning procedure automatically gives an idea on both bandwidth and sensitivity of response to variations in element values. More detailed sensitivity analysis of response can readily be carried out using Sensitivity Analysis, or Monte Carlo tools of Filpro. This feature is important in practice because many alternative structures exist and the designer has to choose the “best” solution in some respect. Sensitivity of response is among the primary criteria for finding that “best” solution in many applications [7]- [8].

Let’s see an example with a source of RS=10 , CS=5 pF to be matched to a load of RL = 50 , CL=20 pF at f=100 MHz (Figure 18.19). Since both capacitors are in series arm, they both form transmission zeros at f=0. Hence the candidate matching structures will be either in highpass or bandpass forms. The structures shown in Figure 18.19.a and b are tested by starting with unity initial values for tuning iterations. In tuning process insertion loss is watched initially over a wide spectrum (fmin=0 to fmax=1000 MHz) to discover formation of a passband. Then the passband is moved closer to f = 100 MHz by tuning elements iteratively. Tuning increments were chosen large in the initial iterations, getting finer as the solution is approached. In Figures 18.20.a and b, results of  Monte Carlo Analysis for the circuit of Figure 18.19.a are shown. All element values are allowed to vary 2 % and 5 % with Gaussian distribution. These experiments give an idea on the possible response deviations due to tolerances of element values. The same experiment is repeated for the circuit of Figure 18.19.b also, giving the results shown in Figure 18.21.a and b. In order to study the effects of individual elements on response, Monte Carlo Analysis can be applied by varying each element one by one. As an example, in the circuit of Figure 18.19.b only the capacitor C1 is allowed to vary randomly, with 5 % Gaussian distribution, giving the response shown in Figure 18.22.a. Then the experiment is repeated by varying the inductor L2 with 5 % deviation giving the response of Figure 18.22.b. Clearly, insertion loss is more sensitive to variations in inductance.

Figure 18.19. Two solutions for a double matching problem.

Figure 18.20. Monte Carlo Analysis results for the circuit of Figure 18.19.a by varying all elements.

Figure 18.21. Monte Carlo Analysis results for the circuit of Figure 18. 19.b by varying all elements.

Figure 18.22. Monte Carlo Analysis results of the circuit of Figure 18.19.b a) by varying only C1 and,      b) by varying only L2.

18. 4. IMPEDANCE  MATCHING USING  SMITH  CHART

In some cases  it may be more convenient to match the circuit by simple manual tuning while watching Smith Chart response of the circuit.

EXAMPLE 18. 7: Matching real RS-RL using Smith Chart.

As an example let’s match RS=50 Ohms to RL=10 Ohms by means of a lowpass two element LC circuit at f= 300 MHz using Smith Chart.

The steps in Filpro are as follows:

i ) Place the cursor under RL and use the command “Insert / Lossless Lumped Element / Csh=1” to insert a shunt capacitor of trial value C1 = 1 pF (Figure 18.23.a). Inspect the position of the impedance marker (+) on Smith Chart drawn with frequency scale fmin=fmax=300 MHz (point-1 in Figure 18. 23.c).

ii ) Use Tune Element command with 1 pF increments while watching the response on Smith Chart. Increase C1 until the impedance marker is on R = 1 circle (point-2 on Smith Chart). Save the final value of C1 ( 21.5 pF) using the key F2.

iii ) Insert a series inductor of trial value L2=1 nH on the left of C1 (Figure 18.23.b). Tune L2 with 1 nH increments at the beginning. Note that impedance marker moves on R=1 circle towards center of the chart. Stop when the marker is near the center. Save the value of L2 and retune it with finer steps until the marker(s) settle at center (point-3 on the chart). Save L2 ( 10.5nH).

iv ) In order to inspect the matching bandwidth, plot insertion loss with scales fmin=0, fmax=1000 MHz, ILmax=100 dB. Try other (finer) scales also. Note that bandwidth is limited on the low side by the degree of mismatch between RS and RL. In order to study that effect, let RS=1 , RL=50  and tune the circuit again resulting in C1=75 pF and L2=3.68 nH, with reduced bandwidth.

Exercise: Consider the case where RS=50  and RL=10 . Show that the reversed form of LC circuit (Figure 18. 24) solves this matching problem.

(Hint: Tune inductance to move admittance marker onto G = 1 circle.)

Figure 18.23. A two element lowpass matching circuit.

Figure 18.24.Two element matching circuit for RS >RL.

Example 18. 8: matching of a complex load to a real source using Smith Chart.

Some elementary complex loads like a reactive element (L or C or LC) in series or in shunt with RL can be matched to a real source if the reactive component can be treated as part of the matching circuit or as part of the “matching-filtering” circuit.

Consider a load of RL = 10 , L1 = 5 nH, to be matched to RS = 50  at f = 300 MHz (Figure 18.25.a). In the circuit of Figure 18.25.b, a lowpass type matching structure is shown.

Figure 18. 25. Matching a complex load to a real source with a lowpass circuit.

In this circuit first the inductor L2 is inserted on the left of L1 and Tune Element command is used to move the admittance marker on Smith Chart onto the G=1 circle (giving L2=5.5 nH). Then a shunt capacitor C3 is added and adjusted to move the admittance marker to the center of the chart (C3 = 21.5 pF). This approach is possible in this example because the load inductance L1 is less than the value L1+L2 = 10.5 nH needed to bring the marker onto G=1 circle. If it was greater than this value than other strategies would be needed. For example let L1=15 nH and RL=10  (Figure 18.26.a). In order to bring the admittance marker onto G=1 circle we have to reduce positive reactance created by the inductor L1. This is possible by adding a capacitor (C2) in series with L1. Tuning that capacitor and than adding a shunt inductor (L3) and tuning that one also we get the matching circuit shown in Figure 18.26.b. (Find L3). Note that this is a bandpass type structure, series capacitor and shunt inductor forming a pair of transmission zeros at f=0 while series inductor is forming a transmission zero at f=.

Figure 18.26. Matching a complex load to a real source with a bandpass type circuit.

Exercise: In Figure 18.27.a the load is parallel combination of an inductance and resistance with values RL = 10  and L1 = 15 nH. Show that it is possible to match that load to 50  source using the circuits of Figure 18. 27. b and c.

Figure 18.27. Matching a shunt inductive load.

Exercise: In Figure 18.28.a load is formed by series connection of C1 = 55 pF and RL = 10 . Show that the circuit of Figure 18.28.b solves the matching problem.

Figure 18.28. Matching a series capacitive load.

Exercise: Consider the series capacitive load of Figure 18.29.a with C1 = 10 pF and RL = 10 . Show that the method used in the previous exercise can’t solve the problem. One solution is shown in Figure 18.29.b. Get that solution using Smith Chart.

Figure 18.29. Another way for matching a series capacitive load.

Exercise: Consider the shunt capacitive load of Figure 18.30.a with RS = 50  and RL = 10 . Show that the circuit of Figure 18.30.b solves the matching problem.

Figure 18.30. Matching a shunt capacitive load with RL < RS.

18. 5. IMPEDANCE  MATCHING  WITH  BANDPASS FILTERS

Bandpass filters have many flexibilities for impedance matching within their passbands. This is clear in the Synthesis approach where an ideal transformer is sticked to the left of load resistance RL at the end of element extraction stages. Part of this transformer may be absorbed in RL to convert it into the desired level while the rest may be absorbed in the elements of filter through Norton, Pi-Tee-L or other possible transformations which produce transformers. There are some inherent limitations arising from interdependencies among RS/RL ratio, passband ripple (or return loss), degree of filter and bandwidth. That is, it is not possible to specify all four quantities independently of each other. Usually only three of these are specified in an iterative manner until the third one  converges to the desired value. Let’s inspect the problem on a simple example.

EXAMPLE 18. 9: An impedance matching bandpass filter.

Let’s match 100 ohms to 4 ohms in the band of 525 MHz-to-575 MHz with Chebyshev bandpass filter of degree 4 with two transmission zeros at both zero and infinity. Return loss should be greater than 20 dB in the band.

Solution:

One possible iterative approach for finding a proper solution is described in Figure 18.31. In this approach filter is designed using Synthesis approach with the specified passband edge frequencies, 0.0435 dB passband ripple (20 dB return loss) and with 4 ohms terminations at both ports. RL will be converted into 100 ohms by using the available Pi-Tee-L transformations. The synthesized circuit is shown in Figure 18.31.a. The ideal transformer is absorbed in the load using command Shift Trf / Absorb Trf in RL. The elements are interchanged using Exchange command to form a capacitive LLeft section (Figure 18.31.b). Then LLeft section is converted into LRight section using command Pi-Tee-L / LLeft-to-LRight producing the maximum turns ratio transformer as shown in Figure 18.31.c. Absorption of this transformer in the load increases RL to much higher level than the desired 100 ohms. This level gives us the upper limit of RL that can be matched to RS = 4 ohms within the specified band and with the specified passband return loss.

Figure 18.31. Determination of maximum RL that can be matched to RS = 4 ohms.

From this point on we may follow several different approaches depending on requirements. The easiest (shortest)  way is to convert the original LLeft section into Pi or Tee section with a specified transformer turns ratio such that RL will be 100 ohms after absorption of the transformer. This is described in Figure 18.32. First, the transformer turns ratio necessary to raise RL to 100 ohms is calculated (using Help / Calculators command) and then the Pi-Tee-L transformation is applied with that turns ratio.

Figure 18.32. Calculation of turns ratio of transformer and LLeft-to-Pi / Specify Transformer command to raise the load to 100 ohms.

However this approach increases the component count. If this is not desirable, then iterations should be made by varying passband edge frequencies or passband ripple or both until RL=100 ohms is obtained just by LLeft-to-LRight transformation. In Figure 18.33, the circuit obtained by iterating on passband edge frequencies is shown. The passband edges fp1 = 463 and fp2 = 637 MHz gave the transformer turns ratio necessary for RL=100 ohms.

If the resulting bandwidth comes out narrower than the minimum desired bandwidth, then degree of the filter should be increased.

Figure 18.33. Matching RS=4 ohms to RL=100 ohms using minimum number of components                            (by iterating on fp1 and fp2 ).

Exercise: Repeat the design with 30 dB return loss specification for minimum number of elements and compare the resulting bandwidth with the one in Figure 18.33.

Exercise: Repeat the design for maximally flat case with minimum number of elements and compare the bandwidths with the one in Figure 18.33. Band edges of maximally flat filters should be measured as the frequencies where return loss is 20 dB.

Exercise: Repeat the design with degree 4 Chebyshev bandpass filter having three transmission zeros at infinity and one transmission zero at zero.

Exercise: Repeat the design with degree 4  Chebyshev bandpass filter having three transmission zeros at zero and one transmission zero at infinity.

Exercise: Repeat the design with degree 6 Chebyshev bandpass filters having equal and different numbers of transmission zeros at zero and infinity. Compare bandwiths with those of degree 4.

Exercise: Repeat the design with degree 4 Chebyshev filter with ALL transmission zeros at infinity (quasi-lowpass filter!. See the next section.)

18. 6. IMPEDANCE  MATCHING WITH QUASI  LOWPASS  FILTERS

Impedance matching sections in the form of bandpass filters having no transmission zeros at zero are termed as quasi-lowpass filters. Since it is not possible to match the structure at DC and in a band close to DC, the resulting band is a quasi-lowpass type. In Filpro these filters are synthesized as bandpass filters. Matching bandwidths that can be obtained using quasi-lowpass filter approaches may be studied by actually designing them. Some examples were already presented in Chapter-3 on lowpass filters in the form of quasi lowpass maximally flat filters. This approach is repeated in the following examples.

Example 18. 10. Maximally flat impedance matching quasi lowpass filter.

Let’s design a maximally flat quasi-lowpass matching-filtering circuit with following  specifications:

fp1=500 MHz,  fp2=600 MHz, passband Return Loss>20 dB,   RS=4 ohms, RL=100 ohms. Delay should be minimum.

The steps of synthesis of a degree 4 filter in Filpro are shown in Figure 8.34.  The following points of synthesis procedure worth mentioning:

- Although it has a quasi-lowpass nature, it should be synthesized as a bandpass filter.

- The actual RS and RL values must be entered.

- Passband edge loss of the maximally flat filter should be chosen as exactly equal to the DC loss characterizing the mismatch between RS and RL. The DC loss can most easily be found by using first Design / By Inserting Elements mode and setting  RS=4 and RL=100 as required. Then plotting insertion loss will give the DC loss. In this example it is found as 8.2995 dB.

- The lower passband edge frequency of the filter should be set much smaller than the upper passband edge frequency. In this example it is set as fp1 = 1 MHz. As fp1 is increased the return loss at band center will start to degrade.

- The upper passband edge frequency is set as greater than the desired fp2. It should be found by trials. In the example with degree 4, it is found that if fp2 is set as 790 MHz then the actual passband will be positioned  in 500-600 MHz band.

- Degree of filter should always be even. Usually all transmission zeros are placed at infinity. If more selectivity is required then some finite transmission zeros may also be placed in the upper stopband.

- The resulting ideal transformers at the load end should be DELETED regardless of it’s turns ratio.

Figure 18.33. Command set for the synthesis of an impedance matching quasi lowpass (bandpass) filter.

In this example two filters are designed one with degree four and other degree 8 to study and compare their properties. Resulting circuits are shown in Figure 18.34.

Figure 18.34. Circuits for the degrees 4 and 8.

Their return loss and delays are shown in Figure 18.35. It is seen that pass bandwidth increases as the degree of filter is increased.

Figure 18.35. Return loss and delays of degree 4 and 8 filters.

Exercise: The above filter may also be obtained as follows:

- Use Synthesis approach again. However now specify the actually desired band edge loss to get the targeted minimum passband return loss. For example let the minimum return loss be 20 dB, corresponding to passband edge loss of  0.0434 dB.

- Set the RS and RL as desired.

- In this approach now both lower and upper passband edge frequencies are unknown. We have to make trials to set them properly to get the infinite return loss at band center, defined as f0 = (fp1*fp2)1/2 . Select trial fp1 and fp2 having this property. Synthesize the circuit. Inspect turns ratio of the transformer at load end. If it is different from unity, then try a new set of fp1 and fp2. Iterate until the transformer turns ratio comes out as unity. Design is completed.

Exercise: Impedance Matching Quasi Lowpass Chebyshev Filter.

The approach described in the previous exercise can also be used to synthesize a Chebyshev quasi lowpass filter. That is, specify the desired passband ripple, RS and RL. The unknowns are passband edge frequencies. Trial selections of these frequencies will yield nonunity transformers. Iterate design until the transformer becomes unity.

18. 7. DUAL  BAND  MATCHING

In some applications impedance matching may be required in two distinct bands. If a wideband filtering-matching structure is not convenient for some reason, then impedance matching may be achieved within the two frequency band by the same filtering circuit. That is the filter will have two passbands. Such a filter can be obtained by first designing two separate quasi-lowpass filters matching the source to the load at the desired bands and then forming a third filter, a double passband filter, by a simple frequency transformation from the two lowpass filters. The two quasi-lowpass filters must have identical degrees and topology. Let’s continue the discussion on an example.

Example 18.11. Dual Band Matching.

Let’s match 100 ohms to 4 ohms in the band of 525 MHz-to-575 MHz and to 8 ohms in the band of 1775 MHz-to-1825 MHz. Return loss should be greater than 20 dB in the respective bands. 4 ohms and 8 ohms forms an example of a load which varies with frequency.

As the first step, we synthesize two maximally flat filters using the approach described in the previous example, operating in the respective bands.

Quasi-lowpass filter-1:

This filter will match 4 ohms to 100 ohms in the band of  525-575 MHz. We can use the degree 4 filter of the previous example for this purpose. It is repeated in Figure 18.36.a and saved as LPF1.

Quasi-lowpass filter-2:

This filter will match 8 ohms to 100 ohms in the band 1775-1825 MHz. The DC loss comes out as 5.617 dB. Synthesizing a degree four maximally flat bandpass (quasi-lowpass) filter  with fp1 =1 MHz and fp2 = 2200 MHz using the approach of previous example yields the circuit shown in Figure 18.36.b. It is saved as LPF2.

Figure 18.36. Specifications, circuits and return loss responses of the two quasi-lowpass filters.

Dual Band Filter:

This filter is obtained from the filters LPF1 and LPF2 by the command set

“New / Design / Dual Passband Filter/ Filename of filter-1(lower frequency filter): LPF1 / Filename of filter-2 (higher frequency filter): LPF2 / Filename of dualband filter: Dualbnd1 / Center frequency of LPF-1: 550 MHz / Center frequency of filter-2: 1750 MHz”

Upon this command, Filpro will make the transformations described in Figure 18.37 and display the resulting filter under the specified filename, with higher frequency termination (Figure 18.38). This command replaces a series arm inductor by a series LC resonator and a shunt arm capacitor by a shunt arm parallel LC resonator. The L and C values of resulting resonators form the two unknowns which require two equations. Two equations are provided by equating reactance of the resonator to the reactance of the series inductor (shunt capacitor) of the lower and upper band quasi-lowpass filters at their passband center frequencies. Thus, the resulting LC resonator will have the same reactances as the corresponding series inductors or shunt capacitors at the center frequencies of respective passbands, leading to two passbands. These transformations are also included in the menu item L/C-to-Resonator /  sLCsr, PLCsh / Specify Reactances.

Figure 18.37. Transformation of series inductor into series LC resonator and shunt capacitor into parallel LC resonator.

The dualband filter can be inspected and analyzed by loading it. Responses of the new filter should be plotted twice, once for each termination. For example in Figure 18.39.a and b, return loss responses are shown for RS=4 ohms and RS=8 ohms respectively. Note that only one of the passbands is correct in each plot. Return losses in the bands corresponding to the respective terminations are almost the same as the original quasi-lowpass filters.

Figure 18.38. Command set for forming dual band matching filter using the two quasi-lowpass filters.

Figure 18.39. Return loss responses a) when termination is 4 ohms and b) when termination is 8 ohms.

18. 8. IMPEDANCE  SCALING TECHNIQUES FOR FILTERS

In some cases it may be necessary to scale up or down element values of a filter without changing terminations or it may be necessary to change terminations without changing element values. Several approaches can be used in Filpro for such purposes.

i) Ideal Inverters as matching sections:

In Figure 18.40, the ideal Inverter option is used for matching RS to RL. Inverter constant is calculated as the geometric mean of RS and RL ( K= (RS*RL)1/2 ). Resulting inverter may be replaced by a proper practical inverter using the command “Inverters / Replace Ideal Inverter By / ...” which offers a large number of alternative practical inverter circuits. Another approach for replacing ideal inverters may be to decompose it into constant reactance Pi or Tee models and then replace constant reactances by L, C or stubs.

Figure 18.40. Ideal inverter as matching section, its constant reactance Pi model and LC equivalent.

Figure 18.41. Insertion loss response of LC matching section replacing ideal inverters.

This technique may also be applied to a two port filter, as follows.

ii) Scaling Circuit By Inserting Inverters:

Assume that filter element values are better behaved for RS=RL=10 ohms, but we have them as 50 ohms. We can use Renormalize command making RS=RL=10 ohms to adjust element values. RS and RL can be changed back to 50 ohms by inserting the inverter of K=22.3 at input and output ports. This procedure is programmed in Filpro as the menu item “Inverters / Scale Circuit By Inserting Inverter / K=..”, as described in Figure 18.42.

Figure 18.42. Scaling element values without changing terminations.

iii) Scaling RS and RL without changing filter elements.

In some cases terminations may be desired to be changed without changing element values of the filter. This is possible using the command “Inverters / Scale RS/RL by Inserting Inverters /New RS=..”, as desribed in Figure 18.43 where 50 ohms terminations are converted into 10 ohms.

Figure 18.43. Scaling RS and RL without changing element values of the filter.

c ) Scaling Circuit by Inserting Constant Reactance L section:

The command  “Impedance Matching / Match Real RS to RL / L Section / Constant Reactance” options can be used to scale up or down the element values of a two port in between RS and RL to a desired impedance level without changing RS and RL. The procedure is as follows:

- The two port elements in between the source and load impedances RS and RL are scaled up or down to the desired impedance level by Renormalize command. The resulting RS1  (or RL1 ) is noted.

- The filter elements are deleted and the desired new source impedance RS1 (or RL1 ) is placed right across the original RS. Then they are matched by a constant reactance L section using the command Impedance Matching / Match Real RS-RL.

- The constant reactance L section is then inserted between the original RS and the scaled two port.

The constant reactances may then be replaced by proper L, C, LC or stubs, as required. This procedure is useful in finding matching circuits for some lumped and distributed element filters when the filter elements needs to be scaled up or down with respect to their terminations.

REFERENCES AND SUGGESTED READINGS:

- D. C. Youla, “A New Theory of Broadband Matching”, IEEE Trans. CT., pp. 30 - 50, March 1964.

- W.K. Chen, “Theory and Design of Broadband Matching Networks”, Pergamon Press, 1976.

- H. J. Carlin and B.S. Yarman, “Double Matching Problem: Analytic and Real Frequency Solutions”, IEEE, Trans. CAS, pp. 15 -28, Jan. 1983.

- R.E. Collin, “Foundations for Microwave Engineering”, Mc Graw Hill, NY 1966.

- A. J. Burwasser, “ TI- 59 program Computes Values For 14 Matching Networks”, RF Design, p.12, Nov/ Dec 1984.

- M. Albulet, “ A New CAD Method for Narrowband Matching Circuits”, RF Design, p.52, May 1994.

- R. Ursulean, “ Sensitivity Calculation for Narrowband Matching networks”, RF Design, p.7, Directory- 1996.

- A. N. Riddle and R. J. Trew, “Parameter Insensitive Matching Circuits for Low Cost Integrated Circuits”, 1983 IEEE MTT-S Symposium Digest, pp. 521- 523, 1983.

- American Radio Relay League, “ARRL Handbook”, 1985 Edition.

- F. Davies, “Application Note 267- Matching Network Design with Computer Solutions”, Application Note 267, Motorola RF Data Book, 1987.

- A. L. Eguizabal, “A Collection of Impedance Matching Networks Design Equations and Programs”, RF Design, pp. 88-92, Nov. 1995.