CHAPTER 19

STEPPED  IMPEDANCE TRANSFORMERS

AND
IMPEDANCE  TRANSFORMING  DISTRIBUTED ELEMENT FILTERS

19. 1. INTRODUCTION

Stepped or continuously nonuniform lines can be used for wideband matching purposes. Filpro can be used in a variety of ways for designing such transformers. Impedance transformers with small numbers of steps can be designed using either well known formulae for step impedances. When the numbers of steps are large, one may either use the readily available tables for finding impedances of steps or the Synthesis tool of Filpro can be used for this purpose. In the latter approach, one may synthesize the impedance matching circuit either as a distributed element All Unit Element highpass filter with quarter wavelength frequency fq chosen as the center of the matching band, leading to the classical multisection quarter wavelength stepped line transformers or one may design it as a distributed element All Unit Element  bandpass filter, with  quarter wavelength frequency chosen as higher than twice the center of matching band to get small physical size. One may as well add SC and OC stubs to provide additional features depending on requirements of applications.

Exponential lines are also available in Filpro for very wideband highpass filter type matching applications.

In the following sections some simple and sophisticated approaches will be described on examples.

19. 2. QUARTER WAVELENGTH  STEPPED TRANSFORMER DESIGN BY TUNING

Quarter wavelength stepped line transformers with small number of steps can be designed in simplest way using Tune Element command of Filpro. In order to get an idea on matching bandwidths of various transformers a single step transformer, a two step transformer and a three step transformer will be designed to match 50 ohms source to a 2 ohms load over a band of  f1 =1900 MHz and f2 =2000 MHz. Examples will be solved in every detail to demonstrate the possible uses of Filpro.

EXAMPLE  19. 1: Single step quarter wavelength transformer

A transmission line of impedance

Z=(RL*RS )1/2=( 2*50 )1/2 =10  ohms

provides matching between  50 and 2 ohms terminations at and around the frequency where the line is a quarter wavelength long.

- The circuit is formed using Insert command of Filpro as shown in Figure 19.1. Impedance is set as 10 ohms using Change command. Step length is chosen so that it become quarter wavelength at f=1950 MHz  (38.4 mm).

- One may as well insert a line with arbitrary length and use Tune Element command to find its length, thus avoid calculation of length.

- Figures 19.2 and 3 show insertion and return loss responses in narrow and wide spans providing perfect match at band center of 1950 MHz.

- Note that the passband response is similar to a maximally flat bandpass filter with 0.04 dB passband edge loss (20 dB return loss at band edges). This observation can be used to design the same circuit using Synthesis approach of Filpro as a maximally flat distributed element highpass or bandpass filter with 0.04 dB passband edge loss.

Figure 19. 1. Single step quarter wavelength transformer.

Figure 19. 2. Insertion and return loss bandwidths for a single section quarter wavelength transformer.

Figure 19. 3. Insertion and return loss for a single section quarter wavelength transformer in wide span.

EXAMPLE 19. 2: Two step quarter wavelength transformers

Two section quarter wavelength transformers provide matching at and around the band center at which section lengths are quarter wavelength long. Step impedances (Z1 , Z2 ) can be found using Synthesis approach of Filpro. However, for two sections, it may be simpler to find these impedances just by tuning impedance values. Select any value for Z1 and use Tune Element command on impedance Z2 (Figure 19. 4). A systematic search shows that for a certain combination of Z1 and Z2 , the passband edge loss is minimized in the matching band (1900-2000 MHz).

Figure 19. 4. Two step transformer impedances giving various passband edge losses.

- Starting with Z2 = 2 ohms, (equal to the load impedance), we find Z1 =10 ohms as the solution giving the match at band center. In fact this is expected, because this circuit is equivalent to the single section transformer.

- Similarly, choosing Z1 =50 ohms (same as high side termination), we get Z2 =10 ohms as the solution giving the match at band center, because this is also the same as the single section transformer.

- In between these two extremes, we can find solutions for Z1 corresponding to any selected Z2 (or vice versa). Let’s make a search to find a solution set that minimizes loss at band edges f = 1900 and f = 2000 MHz.

- In Figure 19. 5 insertion loss plots are shown for the impedance pairs (Z1 =10, Z2 =2 ) (Z1 =15, Z2 =3),       (Z1 = 20, Z2 = 4 ) and ohms.

Figure 19. 5. Insertion loss plots for increasing values of Z1 and Z2 (below the optimum values).

- In Figure 19. 6, the insertion loss plots for the pairs (Z1=30, Z2=6),  (Z1=35, Z2=7), (Z1=40, Z2=8), (Z1=45, Z2=9 ), (Z1=50, Z2=10 ) are shown. Tendency towards optimum solution (minimum passband edge loss) is clear from these graphs. It is somewhere between (Z1=20, Z2=4 ) and (Z1=25, Z2=5).

Figure 19. 6. Insertion loss plots for increasing values of  Z1 and Z2 (above the optimum values).

- Figure 19.7 shows more detailed plots in between these two limits. Optimum solution (lowest passband edge loss) is Z1 = 22.5 and Z2 = 4.5 ohms is clearly seen from this plot.

Figure 19. 7. Insertion loss plots for values of Z1 and Z2 about the optimum solution.

- For comparison with single section transformer, we note from Figure 19.7 that minimum passband edge loss is  0.00009 dB corresponding to 47 dB return loss. In single step case, we had 0.04 dB passband edge loss corresponding to about 20 dB return loss. Improvement is a lot. This is because the specified matching band 1900-2000 MHz is too narrow for a two section transformer. It occupies a narrow range centered in the passband of two section transformer. This is clearly seen in Figure 19.8 where the passbands of single and double step transformers are overplotted. It seems that two step transformer is not necessary to achieve the desired return loss in the specified matching band. An advantage of using two or more step transformers may be getting more degree of freedom for example to adjust impedance levels, as will be demonstrated in the following three step transformer case.

Figure 19. 8. Passbands of two and three section transformers.

EXAMPLE 19. 3: Three step quarter wavelength transformers

Three section quarter wavelength transformers may also be designed using Tune Element command in the same way as the two step transformers. The widest bandwidth optimum solution may be found by iterations. However, in practice, when the band of interest (matching band ) is not wide, a given matching constraint, for example a specified insertion loss may be satisfied by many solutions. Then we may have freedom of imposing additional constraints, like minimum or maximum impedance levels. In the following examples we will set impedances of one or two of the sections and show that very good matching can be obtained by  adjusting impedance of the third step.

In the examples, 50 ohms source will be matched to a 2 ohms load within the band f1 =1900 MHz,  f2 = 2000 MHz.

- The circuit is formed using Insert command of Filpro as shown in Figure 19.9. Initial step impedances are selected in a decreasing manner from source towards load end, arbitrarily. The wideband insertion loss response is shown in Figure 19.10. Step lengths are chosen so as they become quarter wavelength at f =1950 MHz  (38.4 mm).

Figure 19. 9. Three step quarter wavelength transformer.

Figure 19. 10. Insertion loss response for initial (trial) step impedances and of the three step transformer.

- Tune Element command can be applied applied to each element  so as to maximize the bandwidth. However, this procedure can’t be applied arbitrarily, as there are many solutions, hence many local optima. The following trials are made:

- Z3 is kept at 4 ohms Z1 is set at 45 ohms, 40 ohms and 36.2 ohms (SET-1 in Figure 19.11). For each value of  Z1 , impedance Z2 is tuned to get zero insertion loss at band center 1950 MHz. It is observed that as Z1 is decreased band edge loss increases showing that the triplet “Z1 = 45, Z2 =18 and Z3 =4” is optimum in this set (Figure 19.11).

Figure 19. 11. Variation of insertion loss with Z1 , Z2 and Z3.

- The wideband responses for these three cases are shown in Figure 19.12. Clearly, Z1 = 36.2 ohms case shows the widest bandwidth in this picture. However, a closer view is shown in Figure 19.13 where         Z1 = 45 ohm case has a flatter response for frequencies closer to passband center. Thus, it is seen that band edge loss definition  sets the optimum one.

Figure 19. 12. Wide span responses for some Z1 , Z2 and Z3 triplets of  SET-1.

Figure 19. 13. Close in views of responses for Z1 = 45 ohm and Z1 = 36.2 ohms cases.

- Then  Z1 is kept at 36.2 ohms arbitrarily, and Z3 is varied as 5 ohms, 6 ohms and 7 ohms (SET-2 in Figure 19.11). For each Z3 , Z2 is tuned for zero loss at 1950 MHz. It is observed that band edge loss increases as Z3 is increased (Figure 19.11). The wide span responses are shown in Figure 19.14. These graphs  confirms the narrow band observations. That is Z3 = 4 ohms case has the widest bandwidth in this set. As Z3 increases a narrower passband response starts to shape up.

Figure 19.14. Wide span responses for SET-2.

- The wide span responses for the two and three step optimum (Z1=36.2 ohms case) responses are shown in Figure 19.15, showing widening in bandwidth.

Figure 19.15. Insertion loss responses of two and three step transformers.

- As another experiment, Z3 is kept constant at 10 ohms (for easier realization). Z1 is set at values 40, 45 and 50 ohms (SET-3) while Z2 is tuned to get zero insertion loss at band center. The responses in 1900-2000 MHz range are shown in Figure 19.16. The wide span responses are shown in Figure 19. 17, overplotted with wideband responses of SET-2. Clearly, the case with Z1=50 ohms seems to be the best solution. However, SET-3 as a whole has much narrower bandwidth compared to SET-2. This is a result of constrained optimization. We have kept Z3 at 10 ohms which is much higher than the global optimum which must be somewhere in 3-5 ohms level, as Figures 19.12-to-14 indicate. That is, constraints may prevent reaching the global optimum which may be defined in different  ways.

Figure 19. 16. Responses when Z3 =10 ohms constraint is used.

Figure 19. 17. Wideband responses of SET-3 (with Z3 =10 ohms constraint) overplotted with                        responses of SET-2.

19. 3. EXPONENTIAL LINES FOR IMPEDANCE MATCHING

Continuously nonuniform lines may also be used in some wideband matching problems. Exponential lines are included in Filpro as a typical (and easy to analyze) representative of nonuniform lines. In order to get an idea on their matching capabilities, let’s see some examples.

EXAMPLE  19. 4: Exponential line for matching of 50 ohms to 2 ohms:

An exponential line piece can be inserted between RS and RL using command “Insert / Exponential Line Elements / ExpTLine”. Line parameters are shown in Figure 19.18. Here Z1 =50 ohms is the characteristic impedance at source end of line while Z2 is the load end characteristic impedance. The taper constant (exponent  ) is automatically calculated by Filpro. Length of line is found by Tune Element command while observing insertion loss. Lengths of exponential lines are about half wavelength at lower end of matching bandwidth. Close in views of insertion and return loss responses are shown in Figure 19.19. Exponential lines have highpass type matching-filtering characteristics. This is clearly seen in the wide span insertion loss response shown in Figure 19. 20.

Figure 19.18. Exponential line parameters.

Figure 19. 19. Inband insertion and return loss plots for the 50 ohms-to-2 ohms exponential line matching section.

Figure 19. 20. Wide span insertion loss plot for 50 ohms-to-2 ohms exponential line matching section.

EXAMPLE  19. 5: Exponential line matching for 10 ohms to 2 ohms transition

As a further information, exponential line matching between 10 ohms and 2 ohms terminations is also studied, giving the results shown in Figures 19.21 and 22. As expected, amplitude is flatter over wider bandwidth compared to 50 ohms to 2 ohms transition case.

Figure 19. 21. Insertion and return loss plots for exponential line matching between 10 ohms and  2 ohms terminations.

Figure 19. 22. Exponential line matching for 10 ohms to 2 ohms transition (wide span).

Comparison of insertion and return loss plots shows that the two and three section stepped transformers are much better than exponential line transformer. The reason is that the exponential line transformer is trying to match the 2 ohms load to 50 ohms source over a much wider (theoretically semi-infinite) bandwidth (with highpass characteristics), while the stepped transformers provide match only in a limited bandwidth. Therefore it is clear that stepped transformers would be preferred over exponential lines when finite matching bands are involved.

19. 4. DESIGN OF STEPPED LINE TRANSFORMERS  USING  SYNTHESIS  APPROACH

19. 4. 1. INTRODUCTION

Impedance matching circuits can be designed using Synthesis approach of Filpro in many ways. Basically the procedure is the same as synthesizing filters with equal terminations. Terminations may be specified in Synthesis stage or they may be left as 50 ohms and changed after synthesizing the circuit by making use of the transformers that results in element extraction stage or by generating transformers in the filter circuits through some circuit transformations. This process is easiest in lumped or distributed bandpass filters, as they have much more flexibility for circuit transformations because of existence of transmission zeros at both f=0 and f=. In such filters the basic mechanism for producing transformers is Pi-Tee-L transformations, Norton transformations and many other special transformations like Geffe, Inverse Geffe, RCsr-to-RCsh, etc. In distributed element filters with contributing or redundant unit elements, additional tools exist like Kuroda transformations. Generation of transformers is not easy in lowpass filters. Therefore limited tools exist for matching lowpass filters to arbitrary loads. One approach is explained in Chapter 3 where the lowpass filter is designed as a quasi bandpass filter. Lumped and distributed element highpass filters may also enjoy the flexibility of bandpass filters if they are handled as quasi bandpass filters.

In the previous sections, stepped impedance transformers were designed by brute force trial-error approach (Tune Element approach or using a more scientific name, Design By Iterated Analysis approach). All of these circuits can be designed using Synthesis approach also. Synthesis approach opens up more flexibilties in several respects as will be seen in the following subsections. In the foregoing sections mostly stepped impedance transformers will be treated. These circuits will be handled as distributed element highpass and bandpass filters. Bandpass filter approach will provide size reduction also.

19. 4. 2. SINGLE STEP  IMPEDANCE TRANSFORMING HIGHPASS FILTER

All unit element impedance transforming highpass filters are nothing but multisection quarter wavelength transformers. For small number of sections, it is straight forward to design them by Tune Element approach. However it may be time consuming to find the best solution. Synthesis approach is more systematic. One example is as follows:

EXAMPLE  19. 6: Synthesis of single step quarter wavelength transformer

A single step quarter wavelength transformer can be synthesized as a distributed element highpass filter with a single unit element.

Let’s match RS=50 ohms to RL=2 ohms at and around fq=1950 MHz with a single quarter wavelength transformer. Design steps are shown in Figure 19.23.

- In this approach, maximally flat response is chosen.

- RS=50 ohms and RL=2 ohms are assigned.

- The lower passband edge frequency fp1=1900 MHz as trial lower passband edge of the highpass filter.  The upper passband edge will automatically be fp2=2fq-fp1=2000 MHz.

- The center of matching band is assigned as the quarter wavelength frequency, fq =1950 MHz.

- A single unit element is assigned as transmission zero.

- The tricky part of the approach is the passband edge loss required in filter specification stage. The three quantities “passband edge loss, bandwidth and impedance ratio RL/RS” are closely linked to each other. Choosing any two will set the third one. In the above steps we have set impedance ratio and passband edge frequencies. Therefore the passband edge loss is inherently set, but it is unknown at this stage. Assignment of an arbitrary passband edge loss will result in a circuit with a transformer which is possibly non-unity and impossible to eliminate. The trick is here. We synthesize the filter by iterating the passband edge loss until this transformer comes out as unity.

- Iterations gave the passband edge loss as 0.037 dB which is close to the passband edge loss of single step transformer we have studied in the previous section (measured as close to 0.04 dB).

- The resulting circuit is shown at the end of Figure 19.23. Deleting the unity ideal transformer we get the single step quarter wavelength transformer.

Figure 19.23. Design steps of a single step quarter wavelength transformer using Synthesis approach.

Figure 19. 23. Design steps of a single step quarter wavelength transformer using Synthesis  approach (continued).

- The resulting impedance of unit element is close to the quarter wavelength transformer we have obtained by Tune Element approach. Difference is due incomplete iterations on passband edge loss.

- In Figure 19.24, a wider passband is enforced by choosing a lower passband edge frequency with the same quarter wavelength frequency. We get the same circuit, with higher passband edge loss, as expected.

Figure 19. 24. Single step quarter wavelength transformer synthesized as highpass filter with differing     passband edge frequencies.

19. 4. 3. TWO STEP IMPEDANCE TRANSFORMER AS BANDPASS FILTER

The single step highpass filter of previous section may also be treated as a two step (two unit element) bandpass filter with halved lengths (doubled quarter wavelength frequency) and equal step impedances. This can simply be demonstrated in Filpro by applying command “Half Angle Transform” to the unit element of the highpass filter. This command doubles the quarter wavelength frequency, thus splits the unit element into two equal impedance equal length unit elements defined in terms of the new (doubled) quarter wavelength frequency.

Denoting the quarter wavelength frequency of highpass case with fqHP and bandpass case with fqBP , we can also design the circuit of single step transformer of highpass approach as a bandpass filter by choosing          fqBP = 2fqHP.  No reduction in length or no change in impedances will occur. Therefore at first sight it may seem as superfluous attempt to handle the problem as a bandpass filter. However advantage of bandpass approach shows itself if we choose fqBP >2fqHP. This will cause size reduction at the expense of changes in step impedances in opposite directions. That is, as impedance of one step will decrease the other one will increase. Difference between the two impedances gets larger as fqBP  gets higher. Therefore too high fqBP  may lead to unrealizable element values. Let’s see some examples.

EXAMPLE  19. 7: Synthesis of two step impedance transformers as bandpass filters

Let’s design the bandpass filter in order to get the same circuit as the previous single unit element highpass filter. The command sets are shown in Figure 19. 25.

- Distributed Element, Maximally Flat, Bandpass Filter, Doubly Terminated, Type-A options are selected.

- RS=50 ohms and RL=2 ohms are assigned.

- Passband edge frequencies are selected in such a way as their arithmetic mean will be 1950 MHz (passband center). That is fp1=1900 MHz and fp2=2000 MHz are typed in.

- Quarter wavelength frequency is selected as twice the passband center frequency, fq=3900 MHz.

- Two unit elements are assigned as transmission zeros.

- Passband edge loss is left as trial parameter to be adjusted by iterations until the synthesized filter will have unity transformer at the element extraction stage. As expected, passband edge loss of 0.037 dB gave the same unit element impedances as the impedance of single step transformer of highpass filter approach.

- The resulting circuit is shown at the end of Figure 19.25. Deleting the transformer, we get the two step transformer.

- Note that for both unit elements the same impedance as the highpass case is obtained with halved lengths. Thus the total length is the same as the highpass case.

- In order to see the flexibilities offered  by bandpass filter approach, the quarter wavelength frequency is increased to first 5000 MHz and then to 7500 MHz. With the same passband edge frequencies, the circuits with unity transformers are obtained by iterating the passband edge loss. The results are shown in Figure 19. 26. It is seen that the first unit element (from left) impedance decreases while the second unit element impedance increases as fq  is increased. The total length decreases from 38.4 mm of  fq=3900 MHz case to 30 mm for fq=5000 MHz and to 20 mm for  fq=7500 MHz. An appreciable length reduction is obtained at the expense of changes in impedances from about 10 ohms to about 4 ohms level for low impedance line and to 20 ohms level for high impedance line.

Figure 19. 25. Design steps of two unit element impedance matching circuit as a bandpass filter.

Figure 19. 26. Two step impedance matching bandpass filter circuits with differing quarter wavelength frequencies.

- Figure 19. 27 shows the insertion loss responses for the three filters in the matching band. It is seen that passband edge loss is increasing slightly as fq is increased.

Figure 19. 27. Insertion loss responses of the three bandpass filters in the matching band.

- Figure 19.28 shows wide span insertion loss responses of the three filters. All filters has periodic responses with period of fq. As fq is increased, the first spurious passband moves away from the desired passband. In some applications this property may be taken as another advantage of bandpass filter approach, in addition to size reduction.

Figure 19. 28. Wide span insertion loss responses of the two step impedance transforming bandpass filters.

19. 4. 4. TWO UNIT ELEMENT HIGHPASS  AND FOUR UNIT ELEMENT BANDPASS FILTERS

From the discussions of the previous two sections, it is clear that quarter wavelength transformers designed as distributed element highpass filters are nothing but special cases of distributed element bandpass filters when the passband center is half of the quarter wavelength frequency. Therefore from now on we will concentrate on bandpass filters, and treat the highpass case as a special case of bandpass filters.

In this respect, a two step quarter wavelength transformer (highpass filter) is a special case of a four step (four unit element) bandpass filter. Therefore let’s synthesize four unit element bandpass filters with varying quarter wavelength frequencies.

EXAMPLE 19. 8: Four unit element bandpass filters

In this example we will design four unit element bandpass filters with equiripple passband response (as a variation to maximally flat cases) with a specified passband ripple. For example, let’s target to get a 30 dB return loss in the matching band from f1 =1900 MHz to f2 = 2000 MHz to match 50 ohms source to 2 ohms load. This corresponds to 0.00436 dB passband ripple. Since the triplet “Passband Ripple - Bandwidth - Impedance Ratio RL/RS”  are linked to each other, specification of ripple and RL/RS ratio fixes the bandwidth (passband edge frequencies) of the bandpass filter which is unknown to us. Therefore in this example we will iterate the passband edge frequencies until the circuit with unity transformer is obtained at the element extraction stage. The steps of synthesis are shown in Figure 19. 29.

- The options “Distributed Element, Bandpass, Equiripple, Doubly Terminated, Type A” will be chosen.

- Passband ripple will be set as 0.00436 dB and terminations will be chosen as RS=50 ohms and RL=2 ohms.

- Four unit elements will be specified as transmission zeros.

- In order to design two step quarter wavelength transformer (highpass filter with two unit elements), the quarter wavelength frequency should be chosen as twice the center of matching band:

- fq=2*1950=3900 MHz.

- In order to design four step transformer with shorter lengths we will choose fq higher than 3900 MHz.

- The passband edge frequencies of the bandpass filters should be chosen such that their arithmetic mean will be equal to the center frequency of matching band, i.e. 1950 MHz.

- Passband edge frequencies will be iterated until the extracted circuit will have a transformer of unity turns ratio.

- This procedure is applied to the three cases with fq=3900 MHz  (Filter-1), fq=5000 MHz (Filter-2) and fq=7500 MHz (Filter-3). Resulting element values are shown in Figure 19.30.

- For passband edges fp1=1745 MHz, fp2=2155 MHz  the first two and the last two unit elements of the resulting bandpass filter comes out to be equal. Therefore actually this is a two unit element highpass filter with doubled lengths (38.4 mm) and impedances 21.6 ohms and 4.44 ohms. The overall length is 76.8 mm.

- For fq=5000 MHz,  the passband edge frequencies are found to be fp1=1770 MHz and fp2=2130 MHz. Impedances of the first and third unit element decrease while those of the second and fourth increase. Step lengths are 15 mm each leading to 60 mm total length.

- For fq=7500 MHz, passband edge frequencies are found to be fp1=1782 MHz and fp2=2118 MHz. The respective step impedances further decrease and increase. Step lengths reduce to 10 mm, leading to 40 mm total length.

Figure 19. 29. Design steps of four unit element bandpass filters.

Figure 19. 30. Impedances and passband edge frequencies of the four step bandpass filters.

- Insertion loss responses of the three filters are shown in Figure 19.31. Note that 0.00436 dB ripple bandwidth is much wider than the required matching band, f=1900- 2000 MHz of the filter.

Figure 19. 31. Insertion loss responses of the four step bandpass filters.

- The wide span insertion loss responses are shown in Figure 19.32. The first spurious passband of filters are shifted away from the desired passband as fq is increased. Also the stopband loss of high fq filters are higher.

Figure 19. 32. Wide span insertion loss responses of four unit element bandpass filters.

19. 5. THREE STEP QUARTER WAVELENGTH TRANSFORMER

As a demonstration of some alternatives, a three step quarter wavelength transformer will be synthesized. The same transformer may also be synthesized as a six step bandpass filter.

EXAMPLE  19. 9: Three step transformer.

Let’s design a three section transformer with passband center at fq=1960 MHz as a distributed element highpass filter to match 2 ohms source to 50 ohms load, with a 30 dB return loss (0.00436 dB ripple) in band set as f1 =1930 MHz and f2 =1990 MHz.

Solution:

In quarter wavelength transformers the following quantities are closely interrelated:

Bandwidth - Passband Ripple - Impedance transformation ratio (RS/RL)

Passband ripple and impedance transformation ratio are fixed in the specifications. Passband center is also fixed. Therefore we are left with one parameters that can be adjusted to give the required impedance ratio:

Passband edge frequency fp.

Depending on requirements of applications, we may be forced to fix the passband center frequency as well. If this is the case, then the commensurate length of unit elements is also fixed and we are left with only one parameter to play with: Passband edge frequency fp. Thus, design will proceed by assigning some trial values for the passband edge frequency fp  of the highpass filter.

- The steps are described in the Figure 19.33.

- The options “Distributed, HPF, Equiripple, Doubly Terminated, Type A” are selected.

- The center of passband, fq=1960 MHz is assigned as the quarter wavelength frequency.

- Terminations are typed in as RS=50 ohms and RL=2 ohms.

- Passband ripple is set as 0.00436 dB (giving 30 dB return loss).

- The passband edge frequency that gives the required termination impedances will be found by trials. As an initial trial we set it as fp=1500 MHz, and then change it while observing the resulting transformer turns ratio. It is found as  fp =1490.165 MHz.

- Three unit elements are placed as transmission zeros.

- Elements are extracted using Standard Auto Extract command.

- The circuit shown in  Figure 19.34 will be obtained.

Figure 19.33. Command sets for synthesis of three unit element highpass filter.

Figure 19. 33. Command sets for synthesis of three unit element highpass filter (continued).

Figure 19. 34. Three step quarter wavelength transformer.

- As expected, the low end unit element impedance is rather low (3.25 ohms).

- Narrow and wide span response are shown in Figure 19.35. The desired matching band is a very small percentage of the passband of the filter. Obviously, this is an overkilling. Three step matching circuit is an unnecessary luxury for this application. If such a luxury is allowed than we would prefer to choose the quarter wavelength frequency high enough such that the matching band would remain in the lower edge of the filter passband so that physical length would be smaller (Exercise?).

- The lowest step impedance is 3.25 ohms. If a six element bandpass filter approach were used to get even smaller step lengths (Exercise!), then step impedances would deviate from the above highpass filter impedances in opposite directions. This would lead to minimum impedance less than 3.25 ohms which would make life (realization) more problematic.

Figure 19. 35. Narrow and wide span insertion loss of three step highpass filter.

19. 6. QUARTER WAVE TRANSFORMERS WITH A SHUNT SHORT CIRCUIT STUB

A simple highpass impedance transforming filter can be produced by synthesizing a filter with one transmission zero at zero and a number of unit elements. The single transmission zero at zero will be produced as a shunt SC stub which may also be used for bias path in active devices or the load itself may be of parallel combination of a resistor and a SC stub in which case the stub of filter will be combined with the stub of the load. If selectivity is not of much concern, then we have more flexibility to choose band edges and commensurate length (quarter wavelength frequency fq ) to get reasonable element values. Some examples are as follows:

EXAMPLE  19. 10: Single unit element single SC Stub matching circuit

50 ohms source will be matched to  2 ohms load over a band f1 =1900 MHz to f2 =2000 MHz with a single quarter wavelength transformer and a shunt SC stub. Steps of synthesis are shown in Figure 19.36. The following points need mentioning:

- Distributed Element, Maximally Flat, Highpass Filter, Doubly Terminated, Type B options are selected.

- Quarter wavelength frequency is chosen as the center of matching band: fq =1950 MHz.

- Highpass filter passband edge frequency fp and Passband Edge Loss are linked to each other (together with RL/RS ratio). Therefore if we set passband edge frequency, then we will iterate passband edge loss and vice versa until the transformer resulting in the element extraction stage will have unity turns ratio. In this example we set fp =1900 MHz and iterate passband edge loss. It came out to be 0.00165 dB after some trials.

Figure 19.36. Synthesis steps of a single unit element single shunt SC stub highpass impedance          transforming filter.

Figure 19. 36. Synthesis steps of a single unit element single shunt SC stub highpass impedance          transforming filter (continued).

- The circuit shown at the end of Figure 19.36 is obtained. The design is repeated for equiripple passband case using the same type of iteration on passband ripple. Resulting element values for both filters are shown in Figure 19.37. It is seen that the shunt SC stub has very small impedance in both cases.

Figure 19. 37. Element values for Maximally Flat  and Equiripple highpass filters together with a circuit obtained by increasing SC stub impedance using Tune Element command.

Figure 19. 38. Return loss responses for the synthesized and tuned SC stub impedances.

- The SC stub impedance may be raised to realizable values at the expense of reduction in passband return loss, as described in Figure 19.38.

EXAMPLE  19. 11: Bandpass filter version of single unit element single SC stub filter

The same problem may be treated as a bandpass filter as well. The resulting circuit will have an open circuited stub in parallel with the short circuited stub. This circuit may also be obtained by applying “Half Angle Transformation” to the elements of the highpass filter found in the previous example. Synthesis of this circuit is presented below (Figure 19.39), just as a demonstration.

- Distributed Element, Bandpass Filter, Maximally Flat, Doubly Terminated, Type B options are selected.

- Passband edge frequencies are set as the edges of matching band, leaving passband edge loss as floating quantity to be set by iteration: fp1 =1900 MHz, fp2 =2000 MHz.

- Quarter wavelength frequency is selected as double the center of passband: fq = 2*1950=3900 MHz to get the same circuit as the highpass filter. Smaller size will be obtained by selecting higher fq.

- Transmission zeros are: Two unit elements, one transmission zero at zero and one transmission zero at infinity. The two unit elements correspond to the splitted version of the single unit element of the highpass filter. The single transmission zero of highpass filter at zero maps as a SC and an OC stub in the bandpass version.

- Iterations on passband edge loss ended up in the same value as the highpass filter case: 0.00165 dB.

- Transmission zeros are extracted in the order UE-UE-Inf-Zero giving the circuit shown at the end of Figure 19.39.

- Design is repeated for fq =5000 MHz also. The element values are shown in the same figure. It is seen that as fq is increased impedance of  SC stub increases while that of the OC stub decreases. Similarly, impedance of the first unit element decrease while the second one increases. This is another  flexibility of bandpass filter approach over the highpass approach (in addition to size decrease). However, stub impedances are still too small for realization. Their impedances may be raised at the expense of reduction in return loss.

- Return losses of the two filters are shown in Figure 19.40. They are very good, leaving a margin for adjustment of stub impedances (Exercise!).

Figure 19.39. Design steps of impedance transforming bandpass filter with two unit elements,                                  a  SC stub and an OC stub.

Figure 19.40. Return loss responses of bandpass filters with fq=3900 MHz and fq=5000 MHz.

EXAMPLE  19. 12: Two unit element one SC stub highpass impedance transforming filter

Let’s see any advantage that may be offered in the same matching problem by increasing the number of unit elements in the highpass filter approach. Design steps are shown in Figure 19.41.

Figure 19.41. Design steps of highpass filter with two unit elements and a SC stub.

Figure 19.41. Design steps of highpass filter with two unit elements and a SC stub (continued).

- Distributed Element, Highpass Filter, Maximally Flat, Doubly Terminated, Type B options are selected.

- Center of matching band is chosen as quarter wavelength frequency: fq=1950 MHz

- In this example, passband edge loss is fixed at 0.001 dB while the passband edge frequency fp is left for iteration. Iterations ended up at fp=1747 MHz, a rather wide bandwidth compared with the required 1900-2000 MHz band.

- The resulting element values are shown in Figure 19.41. The SC stub impedance is still very small.

- The same problem is treated by setting  passband edge frequency at fp=1900 MHz and passband edge loss at 0.01 dB. In this case, the quarter wavelength frequency is iterated until unity transformer is obtained.  It came out to be fq=2462 MHz. The resulting circuit is shown in Figure 19.42. Element values are slightly decreased.

Figure 19.42. Design of two unit element highpass filter by tuning quarter wavelength frequency.

- In any case it is seen that the shunt SC stub impedance is too low for realization. Therefore let’s try brute force approach to change its impedance. Using Tune Element command on the impedance of SC stub, it is seen that bandwidth of filter is heavily effected (Figure 19.43.a). However, since the filter bandwidth is much too wide than the required matching bandwidth (1900-2000 MHz), return loss in matching band is not effected much by these changes. Impedance of the stub is increased from about one ohm to 50 ohms. The return loss in the matching band remains below 30 dB as shown in Figure 19.43.b.

Figure 19.43. a)Variation of wide span return loss as stub impedance is increased.                                                     b) Return loss in matching band for Zstub=50 ohms.

EXAMPLE  19.13: Bandpass filter approach with four unit elements, a SC stub and an OC stub

The bandpass filter version of the previous filter has four unit elements together with a SC and an OC stub.

- Design steps are shown in Figure 19. 44.

- The passband edge loss is fixed at 0.001 dB and passband edge frequencies are iterated such that the center of matching band (1950 MHz) is the arithmetic mean of fp1 and fp2.

- The case with fq = 3900 MHz would give the bandpass equivalent of the highpass filter of previous example. Therefore quarter wavelength frequency is chosen as fq=5000 MHz, to see if any improvement will be obtained over the highpass case.

- Passband edge frequencies are found to be fp1 =1767 MHz and fp2 = 2133 MHz.

- Resulting element values are shown in Figure 19. 45.a. Element values are still not good.

- In order to see if any improvement is obtained, Kuroda transformation is applied on the shunt OC stub and it is moved leftward (Figure 19.45.b). It’s impedance is increased slightly, but now impedance of one of the unit elements decreased too much.

Figure 19.44. Design steps of four unit element, a SC and an OC stub impedance transforming bandpass filter.

Figure 19. 45. a)Circuit of bandpass filter with four unit elements, a SC stub and OC stub.

b) Circuit obtained after Kuroda transformations.

EXAMPLE  19. 14: Highpass filter with three unit element and a SC stub.

Let’s consider matching of a 50 ohms source to 2 ohms load over a bandwidth from f1=1900 MHz  to  f2=2000 MHz using a highpass filter with one transmission zero at zero and three unit elements. Passband ripple will be 0.01 dB. Passband edge frequency and quarter wavelength frequency fq are left to designer.

The necessary command sets in Filpro are shown in Figure 19.46. fq is chosen as the center of passband, 1950 MHz. The design trials are repeated by adjusting the passband edge frequency fp until the transformer resulting in the element extraction stage is unity. This target is reached for the passband edge frequency of fp =1200 MHz. The resulting circuit is shown in Figure 19.46.

Figure 19.46. Design steps of a highpass filter with three unit elements and a SC stub.

Figure 19.46. Design steps of a highpass filter with three unit elements and a SC stub (continued).

- It is seen that the last unit element and the shunt SC stub impedances are too low. As a trial we may check increasing the SC stub impedance and see deformations in response. For this purpose, another filter is designed now with a higher return loss to allow for deformations in due to tuning of elements.

- The specifications are shown in Figure 19.47. Passband ripple is taken as 0.00436 dB (30 dB return loss). Resulting circuit is shown in Figure 19.48.a. The shunt SC stub impedance is increased while observing return loss using Tune Element command. The changes in response are shown in Figure 19.48.b. It is found that return loss in the matching band of 1900-2000 MHz is not effected much by the changes in stub impedance The stub impedance is increased up to 70 ohms. The resulting return loss for Zstub=70 ohms is shown in Figure 19.48.c. It remains about 30 dB in the matching band.

Figure 19.47. Specifications for the new three unit element and SC stub filter.

Figure 19.48. a) The highpass filter with three unit elements. b) Variation of wide span return loss response as SC stub impedance is tuned. c) Matching band return loss for 70 ohm SC stub impedance.

19. 7. QUARTERWAVE LINE MATCHING CIRCUITS WITH MULTIPLE SHUNT SC STUBS

In the previous examples with one, two and three unit elements you may have wondered on the position of the shunt SC stub. It was located at the small impedance termination in all of these examples. We may as well extract it in a different position. This example will illustrate the effect of the position of this stub in the circuit. The shunt SC stub can be moved in the circuit by applying Kuroda transformation which also produces transformers. These transformers can be used to eliminate the load end transformer which results during element extraction stage.  This feature adds one more free parameter in the search of the best matching circuit.

EXAMPLE 19. 15.  Five UE matching circuits with multiple stubs

Let’s consider a matching circuit  with five UE’s and a single TZ at f=0 to match RS=50 ohms to different loads, RL=2, 10, 25 ohms, with 20 dB or better return loss in an equiripple manner within a band centered at f0=1000 MHz. Lower band edge will be about 500 MHz or less, corresponding to a bandwidth of 500-to-1500 MHz or wider.

- Design command set is shown in Figure 19.49. Distributed Element / Highpass / Equiripple options are selected. Band edge frequency is set as fp=500 MHz with fq=1000 MHz and PBRipple=0.0436 dB (20 dB return loss).  Five UE’s and one TZ at f=0 are placed.

- Just as a curiosity, the transmission zeros are extracted in the order Zero-UE-UE-UE-UE-UE giving the circuit shown in  Figure 19.49.a. Note that UE impedances are getting very large towards the load end with a very small transformer turns ratio.  Our aim is to eliminate the transformer. In the previous examples we had eliminated transformers by adjusting the passband edge frequency fp and/or passband ripple. In this example we will do this by creating transformers using Kuroda transformation. With this feature we may be able to keep the passband edge and passband ripple unchanged.

- With cursor under the UE on the right of the SC stub apply command Kuroda / Left / Full Stub. This command will move the SC stub on the left of UE to its right while also producing a transformer with turns ratio larger than unity, as shown in Figure 19.49.b.

- Simplify Circuit (F6) command moves the transformer to rightmost position and combines it with the load end transformer there (Figure 19.49.c). The net transformer turns ratio is now larger, but still far from being unity. We need more transformers.

- We repeat the procedure by placing the cursor under the next UE (on the right of the translated SC stub) to produce another transformer. Simplify Circuit command gives the circuit of Figure 19.49.d. The net transformer is still small. Therefore we repeat the procedure once more getting the circuit of Figure 19.49.e where now turns ratio is  0.3856.

- At this stage we calculate the transformer turns that will cancel the transformer of 0.3856. That is the required turns ratio is n=1/0.3856=2.5931. Then the cursor is placed under the UE on the right of the SC stub and now the command Kuroda / Left / Specify Trf: n=2.5931 is applied. This command splits the SC stubs into two pieces and moves only one piece to the right of the UE while also producing the necessary transformer. Simplify Circuit command gives the circuit of Figure 19.49.f where there is no transformer.  Response is also shown in the same figure. Note the closeness of the next passband.

- Thus, we have managed to find a circuit without changing the passband edges and return loss, at the expense of using two shunt SC stubs. However inspection of the impedances of UE’s and stubs shows that it may be worthwhile to search other possible solutions by changing both passband edge and return loss.

- Figure 19.49.g shows the solution obtained by using the same approach but by specifying a lower passband edge frequency, fp=400 MHz. (wider bandwidth), leading to slightly higher impedances.

- Continuing  to increase the bandwidth by decreasing fp, we encounter a case where only a single SC stub exists as shown in Figure 19.49.h, for fp=372 MHz.

- Figure 19.49.i shows the case for 30 dB return loss with fp=500 MHz while Figure 19.49.j shows the single SC stub case for 30 dB return loss, encountered at fp=447 MHz.

- Figures 19.49.k, m and n show the circuits obtained for RL=10 ohms with different return loss and band edge frequencies.

- As a demonstration for the formation of different equivalent alternative circuits the circuit of Figure 19.49.m is modified in Figures 19.49.p-q-r-s by applying Kuroda transformations to re-distribute the SC stubs in between the UE’s. This may help achieving better impedance level distribution.

- The commensurate length is about 74 mm (with air dielectric). Therefore the quarter wavelength SC stubs increase the transversal dimensions of the filter . In order to decrease this dimension the quarter wavelength SC stubs can be replaced by shorter SC stubs in parallel with lumped capacitors. This procedure is described in Figures 19.49.t-to-v. First the SC stubs are converted into their lumped LC equivalents by applying the command Replace By Lumped / Lumped LC. Then the LC resonators are replaced by their Cap+SC Stub equivalents by applying the command Replace By Stub / Cap+SC Stub: fq=2000 MHz, f1=500, f2=1500 MHz.  fq=2000 MHz sets the length of  SC stubs at about 37 mm.

- As a variation, the solution for fq=4000 MHz is also shown to get an even shorter SC stubs (about 18 mm) (Figure 19.49.x-y). Note that amost no difference exists between the responses of the circuits with fq=2000 MHz and fq=4000 MHz.

- Figure 19.49.z shows a pictorial description.

Figure 19.49. Multistep quarterwave transformers with multiple SC stubs.

Figure 19.49. Multistep quarterwave transformers with multiple SC stubs (continued).

Figure 19.49. Multistep quarterwave transformers with multiple SC stubs (continued).

Figure 19.49. Multistep quarterwave transformers with multiple SC stubs (continued).

19. 8. NARROWBAND MATCHING ON SMITH CHART

There are variety of simple narrowband impedance matching techniques involving stubs and transmission line pieces, presented in most microwave textbooks, either as solved examples or as exercises. All such circuits can be configured, analyzed and optimized using Filpro.

Let’s see some techniques to match a complex load

ZL =10 + j10 ohms

to a real source of RS =50 ohms at frequency f =1000 MHz. Such a load can be simulated by inserting a series inductance of value L=1.6 nH on the left of the load RL=10 ohms as shown in Figure 19.50 (ZL=RL+jwL=10+j10). One may as well insert frequency invariant reactance of value X=10 ohms instead of inductance. However inductance is more realistic for inspecting wideband response of matching structures.

Figure 19.50. A complex load - real source circuit.

19. 8. 1. Single Stub Matching

In this approach, first a transmission line is inserted on the left of ZL (Figure 19.51.a) whose characteristic impedance Z1 and length l1 will be adjusted such that the impedance seen at the input of line looking towards the load will be Zin1 =50+ jX or the input admittance will be Yin1=0.02 + jB. Then either a series reactance of value -jX in the form of a series arm SC stub (Figure 19.51.b) or a shunt susceptance of -jB in the form of an (preferably) OC or SC stub (Figure 19.51.c) can be inserted to the left of the transmission line to cancel the reactive part. Impedances and lengths of stubs may be adjusted using Filpro until a match is seen by the source (Zin2=50 ohms).

Figure 19.51. Single stub matching circuits.

The procedure can be carried out in Filpro as follows:

a) Smith Chart approach.

- Use commands New / Design / By Inserting Elements to display circuit with RS=RL=50.

- Place the cursor under RL and use Change command (short cut key: Space bar) to make RL = 10 .

- With cursor under RL, use commands Insert / Lossless Lumped Elements / Lsr / L=1.6 nH to place the reactive part of the load.

- Plot Smith Chart response (short cut key: Shift+F7) by setting fmin= fmax= 1000 MHz.

- Inspect the positions of impedance marker (+) and admittance marker (X). Note also the load impedance Zin = 10 + j10.053 in the window on the top right of the screen, where S11, Yin and VSWR are also displayed

- With cursor under inductor, use commands Insert / TLine,stubs,res,Inv / TLine / Z0=50, Lng=l (mm), r=1 (relative permittivity)” to insert a transmission line of impedance 50  (Figure 19.52.a). Different choices for line impedance is possible leading to different solutions.

- With cursor under TLine, apply the command Tune Element. At this stage, we can choose either impedance or length of line for tuning. Choose length. Then tuning increment will be asked. Choose it as 1 mm for initial (coarse) trials. Then response display type will be asked. Choose Smith Chart, with fmin= fmax=1000 MHz ”. Inspect position of markers. In Figure 19.52.c, the position of Admittance Marker (X) is indicated as “Point -1”. Impedance marker (+) is symmetrically placed with respect to origin. Use up arrow to increase length of line (by 1 mm at each key stroke) to move Admittance Marker towards G=1 circle. Note that markers move on constant reflection coefficient circles (blue circles concentric with the center of the chart). Stop changing length when the marker is roughly on G=1 circle (Point-2 in Figure 19.52.c). Save the value of length l1 (10 mm) using the key “F2” and exit using “Esc”.

- Repeat previous step now with finer increment, e.g. 0.01 mm. by using up-down arrows while watching real part of Yin until it is exactly 0.02=1/50 mhos. Save (F2) and Esc. Note that l1= 9.87 mm and imaginary part of Yin is -1.844.

- Insert a shunt OC stub on the left of transmission line. Choose stub impedance as a trial, 50  and choose length l2=10 mm.

- With cursor under OC stub, use Tune Element command to adjust length l2 of the stub with increments that you choose (1 mm increment is too slow). Note that “up-arrow” moves the admittance marker towards the center of chart (matching point) on G= 1 circle (Figure 19.52.c). Save the value as the marker is near the center and repeat tuning with finer increments until the marker settles at origin (Point-3 in Figure 19.52.c). We get l2= 51.3 mm (Figure 19.52.b).

- Plot insertion loss from fmin=0 to fmax=2000 MHz. With ILmax=100 dB, to inspect the matching bandwidth (Figure 1.53).

Figure 19.52. Single Stub Matching using Smith Chart.

Figure 19.52. Single Stub Matching using Smith Chart (continued).

Figure 19.53. Insertion loss of matched circuit.

b) Insertion Loss Approach.

An alternative approach to solve the same problem is “trial-error” in nature, but giving more insight on effects of tuned elements on wideband response, as follows.

- Insert TLine and Shunt OC stub at the same time, each with impedances 50 ohms and lengths l1=l2=5 mm. as starting values.

- Plot insertion loss from fmin=0 to fmax=50,000 MHz. and ILmax=100 dB to see the wideband response (Figure 19.54.a).

- With cursor under TLine, use “Tune Element” command to increase length l1 with increment of 1 mm. while watching insertion loss response. See the formation of passbands as l1 is increased (Figure 19.54.b). One of these passbands will be shifted to be centered at f 1000 MHz. at the end of iterations with l1 and l2. Stop increasing l1 somewhere between 10 mm and 20 mm, and save (F2).

- Decrease upper frequency limit of insertion loss response to fmax=10000 MHz. Place cursor under OC stub and now tune length l2 with 1 mm increments. Decrease fmax to 2000 MHz. Stop increasing l2 when a passband is roughly settled near f 1000 MHz and save (Figure 19.54.c). Then pass to tune l1 again. Experiment by increasing and decreasing l1 and l2, each time with a narrower frequency band and smaller ILmax. We get the same solutions as in the previous approach (Figure 19.54.d).

Figure 19.54. Single stub matching by tuning elements while watching insertion loss.

Exercise: Repeat these experiments by choosing different impedances for transmission line and stubs.

Exercise: Repeat the same experiments by using a shunt SC stub in place of shunt OC stub.

Exercise: Repeat same type experiments by using a series arm SC stub.

Exercise: Repeat same type experiments by using a series arm OC stub.

19. 8. 2. Two-Step Line Matching

In this approach, a transmission line section is placed on the left of the load to cancel its reactive part, presenting a purely resistive input impedance Zin = Rin+ j0. Then another transmission line, working as a quarterwave transformer at the required frequency (f 1000 MHz in this example) is inserted on the left of the first line to match Rin to source resistance RS (Figure 19.55).

Figure 19.55. Two-step line matching.

The steps are as follows:

- Insert a TLine of impedance Z1=50  (or any other proper value) with trial length l1=5 mm. Inspect the position of Impedance marker on the Smith Chart (Point-1 in Figure 19.56)

- With cursor under TLine, use Tune Element command to increase length l1, with a suitable increment, until the impedance cursor is roughly on the real axis of Smith Chart (Point-2 in Figure 19.56). Repeat with finer increments until imaginary part of Zin is zero. Then save l1= 65.1 mm and read Rin=260.49 .

- Calculate the impedance Z2 of quarter wavelength transformer using

and insert a TLine with that impedance (You may use Calculator of FILPRO). The wavelength at f =1000 MHz is =c/f =300 mm. Hence set the length of transformer line as  l=/4 =75 mm. Inspect Smith Chart response (Point-3 in Figure 19.56). Matched

Exercise: Try matching with Z1=100  and Z2 =30 

Exercise: Insert two transmission lines with Z1=50  and Z2=100  Try to get matching network by changing l1 and l2 while watching insertion loss response only

Figure 19.56. Matching with Two-TLine approach using Smith Chart.

19. 8. 3. Single Line Matching

In stripline-like structures where it is easy to adjust both impedance and length of lines, it may be possible to match a complex load to a real source using a single transmission line piece only (Figure 19.57). In Filpro, this process can be carried out by varying impedance and length iteratively while watching either Smith Chart or by watching wideband insertion loss response as follows.

Figure 19.57. Single line matching.

a) Smith Chart - VSWR Approach:

i) Insert a line with initial trial impedance

and trial length  l<    e.g. l = 10 mm.

Inspect position of Impedance Marker on Smith Chart drawn with fmin= fmax=1000 MHz (Point-1 in Figure 19.58).

ii) Use Tune Element command to change length in 1 mm (or more) steps until input impedance becomes real ( 51.5 mm, Point-2 in Figure 19.58). Save this length (using the key F2).

iii) Now tune impedance of line with 0.5  steps while watching the VSWR. Stop when VSWR gets minimum value (VSWR  1.15 at Z=20.36, Point-3 in Figure 19. 58). Save that impedance (F2).

iv) Next tune length again, now with finer increments while watching VSWR, until it gets minimum value (VSWR 1.088 at l= 48.95 mm for 0.1 mm increment). Save and pass to tuning impedance. Repeat until the desired VSWR is reached (VSWR = 1.008 for  l= 47.7 and Z= 19.4) .

Figure 19.58. Single line matching with Smith Chart.

b) Insertion Loss Approach:

- Tune length  by watching insertion loss response scaled as fmin=0, fmax=5000 MHz, ILmax=10 dB. Set increment as 1 mm. Watch the formation of a passband approaching 1000 MHz as  is increased (Figure 19.59.a). Stop tuning when passband settles roughly around 1000 MHz and  Save.

- Next, tune impedance with 0.1  increments and insertion loss plot scaled as fmin=500 MHz, fmax=1500 MHz, ILmax=1 dB. Stop and save when insertion loss becomes zero at some frequency.

- Tune  with 0.1 mm increments to shift the passband center to f =1000 MHz.

Compare the results with those of previous approach.

Figure 19.59. Single line matching by tuning line parameters while watching insertion loss.

19. 8. 4. Double Stub Matching

This commonly used practical matching method uses two standard adjustable length shunt SC stubs separated by a transmission line of length /8 or 3/8, with 50  impedance each (Figure 19.60). In practice, engineers watch reflection or VSWR while they adjust stub lengths in a trial - error approach. The same approach can be followed in Filpro as well:

Figure 19.60. Double stub matching.

Insert two shunt SC stubs of impedances Z1 =Z3=50  with lengths  l1=l3=10 mm.

- Insert a transmission line between stubs, with impedance Z2=50  and length l1=3/4=112.5 mm.

- Tune l1 with 5 mm increments while watching insertion loss response with scales fmin= 0, fma=5000 MHz, and ILma= 100 dB. Note the formation of passbands (Figure 19.61). Stop somewhere around l1=75 mm.

- Tune l3 with again 5 mm increments to bring a passband nearer to f =1000 MHz.

- Tune l1 with finer increments, then tune l3, and so on, until the passband centers at f =1000 MHz.

Figure 19.61. Double stub matching by tuning lengths of stubs while watching insertion loss response.

Exercise: In order to get more flexibility in this approach, insert another transmission line with impedance Z4=50  and length l4=10 mm to the left of the load as shown in Figure 19.62. Try tuning l1, l2 and l3 to get a match around f = 1000 MHz. (We have found a match for l1=123 mm, l3= 41 mm and l4= 9mm.)

Figure 19.62. Double stub matching with two lines.

REFERENCES:

[1] G. Szentirmai, “Bandpass matching filter in the form of polynomial lowpass filter”, IEEE Trans. on Circuit  Theory, vol. CT-11, pp. 177178, March 1964.

[2] G. L. Matthaei, “Tables of Chebyshev Impedance Transforming Networks of Lowpass Filter Form”, Proc. IEEE, pp. 939-963, Aug. 1964.

[3] E. G. Crystal, “Tables of maximally flat impedance transforming networks of lowpass filter form”, IEEE Trans. on MTT, vol. MTT-13, pp. 693-695, Sept. 1965.

[4] R. Levy, “Synthesis of mixed lumped and distributed impedance transforming filters”, IEEE Trans. on MTT, vol. MTT-20, pp.223-233, March 1972.

[5] A. Podcameni and L. F. Conrado, “Broadbanding the quarter-wave transformer using two parallel stubs,” Electron. Lett. Vol. 17, no. 11, pp. 372-374, May 1981.

[6] J. M. Drozd, W. T. Joines, “Using parallel resonators to create improved maximally flat quarter wavelength transformer impedance matching networks,” IEEE Trans. on MTT, vol. MTT-47, pp.132-141, Feb. 1999.