CHAPTER 20

BRANCH LINE DIRECTIONAL COUPLERS

20. 1. DEFINITIONS

In this report a survey of 3 dB branchline couplers with impedance matching properties is presented. Weight is given on balance between output signal amplitudes and flatness of transmitted and coupled signals together with good isolation, input matching and flat, minimum delay. Only two and three branch couplers will be studied.

A schematic drawing of a symmetric two branch coupler is shown in Figure 20.1. The convention for port numbering is also indicated:

Port-1: Input signal port with termination resistance R1.

Port-2: Transmitted signal port with termination resistance R2.

Port-3: Coupled signal port with termination resistance R2.

Port-4: Isolated port with termination resistance R1.

Figure 20.1. Schematics of a 3 dB branchline coupler with different input-output terminations.

In order that this device work as a 3 dB coupler, the line impedances should satisfy the following relations:

Branch line-1 impedance: Z1 = R1

Branch line-2 impedance: Z2 = R2

Main line impedances:      Z12 = Sqrt (R1*R2/2)

The circuit is symmetric about the horizontal midplane. Analysis of such symmetric structures can be carried out in Filpro using Even-Odd mode decompositions.

In the first part of this survey, design and analysis experiments are performed on a 3 dB coupler with the frequency band “f1=1930 MHz,    f2 =1990 MHz”. In the second part, the performances are investigated in the bandwidth  “f1=1900 MHz,     f2=2000 MHz”.

Directional coupler will operate between terminations

R1 = 50 ohms,      R2 = 2 ohms

leading to  Z1= 50 ohms, Z2=2 Ohms  and Z12=7.071 ohms for a two branch coupler.

The schematic drawing for this coupler is shown in Figure 20.2.

Figure 20.2. Schematics of 3 dB coupler with R1 = 50 ohms and R2 = 2 ohms.

- The two ohms line will be approximated by lumped element equivalents.

- The best amplitude balance between coupled and transmitted signals will be targeted together with best flatness of signals with frequency.

- Isolation and input return loss should be kept below 20 dB.

- In the first part optimization of phase and delay responses will not be considered because these are exploratory experiments to study the efficiency of Filpro in analysis and optimization of such structures together with setting the most important parameters for adjustments to obtain the targeted responses.

20. 2. EXPERIMENTS WITH TWO BRANCH COUPLERS

Experiment-1: All transmission line coupler

The even and odd mode halves of the coupler is designed in Filpro using “Design / By Inserting Elements” approach (Figure 20.3):

Figure 20.3. Design steps of even and odd half circuits of coupler in Filpro.

- The passband center is taken as f0 = 1960 MHz, leading to 38.2653 mm for quarter wave length which is the lengths of both main arm and branch arm lines.

- The branch lines appear as shunt open circuited stubs of length 19.13265 mm (halved branch length) in the even mode half circuit, while they appear as shunt short circuited stubs of the same length in the odd half circuit.

- In this experiment the low impedance second branch line (2 ohms) is left as it is just to see the ideal response to be used as reference for comparison of the forthcoming lumped element approximations.

- After forming the even and odd halves, and saving them as Even1 and Odd1 in Filpro,  the complete circuit can be analyzed using the command sets shown in Figure 20.4.

Figure 20.4. Command sets for analysis and tuning of directional coupler.

- The coupler is analyzed as a symmetric cross-coupled circuit with command set “Design / Symmetric Cross-coupler Ccts / Analyze Directional Couplers”.

- At this step, filenames of even and odd halves are asked by Filpro. After typing in the filenames (Even1 and Odd1), Filpro asks to select one of the two options: “Plot Response Only” and “Tune Even-Odd Circuit Elements”. Tune option is used for optimizing response by varying some elements of the circuit. In this experiment, all elements of the circuit are tuned to study their effects. It is found that only variation of R2  is helpful to improve the balance between transmitted and coupled signals. Therefore experiments are continued by tuning  R2.

- After choosing Tune option, first the even half circuit is displayed. The element to be tuned is pointed by the cursor and Enter key is used to record that element. Then the odd half circuit is displayed and the same element to be tuned is pointed in this circuit also. If the two pointed elements are the same, then Filpro asks the increment to be used in tuning.

- (Note: It is also possible to tune two different elements from the even and odd half circuits. If the two pointed elements in the two circuits are not the same elements, then tuning is done by using Esc key and typing in the new value of elements and pressing Enter.)

- For analysis of amplitudes, the Insertion Loss option is chosen. For analysis of phase and delay, the options Insertion Phase and Delay are chosen respectively.

- In Figure 20.5, the amplitudes of the four port signals  S11, S21, S31 and S41 are displayed.

Figure 20.5. Wideband view of signal amplitudes.

- Figure 20.6 shows magnitudes, phases and delays of transmitted (S21 ) and coupled signals (S31 ) in a relatively wide band. It is seen that phase difference is almost flat at 90 degrees. However bandwidths of magnitudes of S21 and S31  are quite different causing amplitude imbalance between output ports.

Figure 20.6. Magnitudes, phases and delays of  S21 and S31.

- In Figure 20.7, close in views of transmitted (S21 ) and coupled (S31 ) signals are shown. It is seen that a perfect balance is achievable in the band by “detuning” the  R2 = 2 ohms port impedances to R2 = 1.60 ohms.

Figure 20.7. Close in views of S21 and S31 and effect of reducing R2 =2 ohm to R2 = 1.60 ohms.

- This improvement on the balance is obtained at the expense of reduction in isolation (S41 ). This is seen in Figure 20.8 where isolation is reduced to below 20 dB.

Figure 20.8. Port signals for R2=1.60 ohms.

- Figure 20.9 shows the delays of  S21 and S31   for R2 =1.60 ohms. Note that they are no more identical in the passband.

Figure 20.9. Wide and narrow span views of delays for R2=1.60 ohms.

EXPERIMENT-2: Single Pi-section approximation for low impedance line

In this experiment the low impedance branch is simulated by its lumped element approximation as described in Figure 20.10.

- In order to get an idea on the degree of approximation, first a transmission line is inserted between 50 ohms source-load circuit (Figure 20.11) and it is replaced by the lumped element model using command set “Replace TLine By / Tee Model / Lsr+Csh+Lsr”.

Figure 20.10. Simulation of Z2=2 ohms line by a single lumped element Tee model.

Figure 20.11. Tee model of  Z2 = 2 ohms line and its even-odd halves.

- Insertion losses of the circuit before and after replacement are shown in Figure 20.12. It is seen that single Tee model for this line is NOT a good approximation. In order to see the effect of this crude approximation, we proceed to form the even and odd half circuits by noting the resulting inductances and capacitance after even-odd decomposition of the lumped element branch.

Figure 20.12. Comparison of responses of circuit with actual 2 ohms line and circuit with

lumped element model of 2 ohms line (in wide and narrow spans).

- The even and odd half circuits are formed as shown in Figure 20.13 and saved with file names Even2 and Odd2.

Figure 20.13. Even and odd half circuits for single Tee lumped element approximation of  Z2 = 2 ohms line.

- The same analysis and tuning command lists of the previous experiment are used for analysis of the new coupler. The resulting port signal amplitudes are shown in Figure 20.14 for both R2 = 2 ohms and R2 = 1.60 ohms. Tuning didn’t helped to improve balance between S21 and S31..

Figure 20.14. Responses (S21 and S31) for the coupler of single Tee model for low impedance line.

EXPERIMENT-3: Two Pi-section approximation of low impedance line

Next, the low impedance line is approximated as two cascaded lumped element Tee models by first splitting the line into two equal pieces (Figure 20.15).

Figure 20.15. Two piece approximation for the low impedance line.

- Response of  two piece approximation is shown in Figure 20.16 together with single Tee and actual transmission line responses. Clearly, two piece approximation is much better.

Figure 20.16. Insertion loss responses of low impedance line, single piece and two piece

lumped element approximations.

- The coupler schematics with two piece approximation is shown in Figure 20.17 including both Pi and Tee models for the line pieces.

Figure 20.17. Circuit schematics for two piece lumped element approximations of low impedance line.

- The Pi-models of line pieces are found to yield better approximations. Therefore they are used to form the even and odd half circuits shown in Figure 20.18.

Figure 20.18. Even and odd half circuits for two piece approximation of low impedance lines.

- Amplitude responses are shown in Figure 20.19 and 20 for both R2=2 ohms and R2=1.60 ohms. Coupled and transmitted signal amplitude balance is better than the single piece approach.

Figure 20.19. Coupled and transmitted signal amplitudes for two piece approximation, R2=2 ohms and

R2 =1.60 ohms cases.

Figure 20.20. Isolation (S41) and return loss  (S11) signals for two piece approximation for R2=2 and      R2=1.60 ohms cases.

EXPERIMENT-4: Three Pi-section approximation of low impedance line

Finally, the low impedance line is split into three equal length pieces and each one is approximated by lumped elements (Figure 20.21).

Figure 20.21. Three piece approximation of the low impedance line.

- Approximation of the three pieces of the line by  lumped elements is shown in Figure 20.22.

Figure 20.22. Lumped element approximation of a three piece line.

- Comparison of responses of the actual line and lumped element approximations are shown in Figure 20.23. Three piece approximation tracks the actual line very well.

Figure 20.23. Three piece approximation of the low impedance line.

- The even and odd half circuits of the coupler formed by the three piece line approach are shown in Figure 20.24 together with responses for S21 and S31.

Figure 20.24. Even and odd half circuits for three piece approximation.

OBSERVATIONS:

- It is clear that a very good lumped element approximation is needed for simulation of the low impedance line.

- Amplitude balance between coupled and transmitted signals can be improved by a deliberate mismatch at ports 2 and 3. However this action degrades isolation and increases difference in delays of S21 and S31. Detuning may be left at an optimum level for the desired minimum isolation.

- Return loss is better than 20 dB in all cases.

20. 3. EXPERIMENT-8: COUPLERS WITH 2-ELEMENT IMPEDANCE MATCHING CIRCUITS

In this approach, the coupler is designed for impedance transformation from R1=50 ohms to  R2=10 ohms. Transition from 10 ohms ports to 2 ohms loads are accomplished through narrowband impedance matching circuits, as described in Figure 20.25.

Figure 20.25. Coupler with matching at output ports.

- The even and odd half circuits of the 50 ohms-to-10 ohms coupler are shown in Figure 20.26.

Figure 20.26. Basic coupler with 50 and 10 ohms port impedances.

- As the first trial, a two element lowpass matching circuit is placed at ports 2 and 3, giving the even and odd circuits shown in Figure 20.27. Matching circuits can be found either by trials using “Tune Element” command or  by designing a bandpass filter of degree 2 with no transmission zero at f = 0.

Figure 20.27. Even and odd half circuits with two element matching section.

- The coupled and transmitted signals are shown in Figure 20.28. It is seen that amplitude unbalance is about 0.04 dB. Coupled signal is flat while transmitted signal has 0.04 dB variation.

Figure 20.28. Coupled and transmitted signal amplitudes for coupler with two element matching section, in narrow and wide spans.

- Return loss and isolation are both well below 20 dB (Figure 20.29).

Figure 20.29. Return loss and isolation.

- Phase plots show the 90 phase difference between coupled and transmitted signals (Figure 20.30).

Figure 20.30. Phases of transmitted and coupled signals and their difference.

- Delays of coupled and transmitted signals are shown in Figure 20.31.

Figure 20.31.Delays of S21 and S31 in narrow and wide spans.

20. 4. EXPERIMENT-9: COUPLERS WITH 4-ELEMENT IMPEDANCE MATCHING CIRCUITS

In this experiment a four element matching circuit is used at the output ports giving the even and odd half circuits shown in Figure 20.32.

Figure 20.32. Even and odd half circuits of coupler with four element matching circuits.

- Coupled and transmitted signal amplitudes are shown in Figure 20.33. It is seen that both unbalance and flatness is below 0.02 dB.

Figure 20.33. Coupled and transmitted signal amplitudes in narrow and wide spans.

- Return loss and isolation are well below 20 dB as is seen from Figure 20.34.

Figure 20.34. Return loss and isolation of the coupler with four element matching sections..

- Signal phases are shown in Figure 20.35. S21 and S31  has 90 degree phase difference in the band.

Figure 20.35. Signal phases.

- Delays of  S21 and S31 are shown in Figure 20.36 and 37, in narrow and wide spans.

Figure 20.36. Delays of S21  and S31  in narrow span.

Figure 20.37. Delays of S21 and S31 in wide span.

OBSERVATIONS:

Impedance matching at ports may help to avoid or reduce the problems caused by approximation of low impedance lines as lumped elements. Higher order matching circuits help to balance the output signals. Return loss and isolation are well below 20 dB.

20. 5. THREE  BRANCH  COUPLERS

Ready tables for three branch couplers with asymmetric terminations are not available. All available formulations and tables are for doubly symmetric couplers with 50 ohms terminations at all ports. However, it is not a difficult problem to start with such a fully symmetric coupler designed for 50 ohms terminations and optimize the elements for other terminations. This approach is used in the following experiments.

EXPERIMENT 10:

- The initial coupler element values are taken from A. Aksen ([1], Table 4.2) and is shown in Figure 20.38.a. It is seen that the impedance of the first branch on left is high for realization in microstrip. Therefore in iterations towards finding an optimum, we will try to search for finding optima by assigning a lower value to this impedance to study its effect on overall response. In the first trial it is kept at 70 ohms. Iterations ended up in the circuit shown in Figure 20.38.b.

The following systematic iteration steps are followed:

- Decrease terminations R2 at ports 2 and 3  by a small step, for example to 45 ohms at the beginning. Return loss and isolation will degrade appreciably.

- Tune branch-1 (first from left-120 ohms) and the adjacent main arm line to improve return loss and isolation.

- Tune branch-2 and line-2 to improve return loss and isolation further.

- Tune branch-3 and line-2 to obtain balance between ports 2 and 3.

- Retune branch-1, 2 and line 1 to improve isolation and return loss further.

- Repeat the cycle until the best balance of S21 - S31 together with best return loss and isolation are obtained.

- Decrease R2 further and repeat this manual optimization scheme.

- In these experiments no consideration is given on phase and delay optimizations.

Figure 20.38. Initial and optimized three branch coupler.

- Even and odd half circuits of the resultant coupler are shown in Figure 20.39.

- Figure 20.39. Even and odd half circuits of three branch optimized coupler.

- The transmitted (S21) and coupled (S31) signals are shown in Figure 20.40. Unbalance is about 0.04 dB in the band.

Figure 20.40. Transmitted (S21 ) and coupled (S31 ) signals in narrow and wide spans.

- Isolation (S41) and return loss (S11) are shown in Figure 20.41. Return loss is below 50 dB while isolation is below 20 dB within the band.

Figure 20.41. Isolation (S41 ) and Return Loss (S11 ).

- Figure 20.42 shows the phases of S21 and S31 in narrow and wide spans.

Figure 20.42. Phases of S21 and S31 together with their difference.

- Figures 43 and 44 show delays of  S21 and S31   in narrow and wide spans respectively.

Figure 20.43. Delays of S21 and S31 in narrow span.

Figure 20.44. Delays of S21 and S31 in wide span.

OBSERVATIONS:

In the three branch coupler that is found by manual optimization, response flatness is not as good as expected. S21 and S31  has about 0.04 dB variation and unbalance which is about the same as two branch coupler. It is worse than the two branch coupler with 4 element matching section.

This is surprising, because three branch coupler has more parameters to adjust. Theoretically one expects better flatness. The reason may be that the first branch impedance is kept at lower value (70 ohms) than the initial 130 ohms value for 50 ohms-to-50 ohms coupler, so that manual optimization scheme converged to a solution which is not the best one in flatness. In order to gain more insight on the behaviour of circuit, in the succeeding experiments the first branch impedance will be set at different values and all other elements are optimized.

EXPERIMENT 11:

- The first branch is held constant at 130 ohms to study the effect of higher impedances. All other elements are tuned to get maximum flatness with better than 20 dB return loss. The circuit shown in Figure 20.45 is obtained.

Figure 20.45. Optimized circuit for 130 ohms first branch impedance.

- The transmitted and coupled port signals are shown in Figure 20.20.46. It is seen that a very slight improvement in flatness ( 0.03 dB ) is obtained at the expense of a higher branch line impedance. The slight improvement is partly due to the relatively low isolation and return loss levels (20-30 dB). If these parameters were also maximized, then flatness would be about 0.04 dB again.

Figure 20.46. Coupled and transmitted port signals for 130 ohm first branch impedance.

- Return loss and isolation are shown in Figure 20.47.

Figure 20.47. Isolation and return loss for 130 ohms first branch impedance.

- Phases of S21 and S31 are shown in Figure 20.48 in narrow and wide span forms.

Figure 20.48. Phases of S21 and S31  in narrow and wide spans.

- Delays are shown in Figure 20.49 and 50.

Figure 20.49. Delays of S21 and S31 in narrow span.

Figure 20.50. Delays of S21 and S31 in wide span.

EXPERIMENT 12:

- First branch line impedance is increased to 180 ohms. The optimized even and odd mode circuits are shown in Figure 20.51.

Figure 20.51. Even and odd half circuits for 180 ohms first branch impedance.

- The coupled and transmitted signals are shown in Figure 20.52 and 53. Roughly the same flatness as the previous case is obtained. The wide span pictures show a slight widening in coupled signal bandwidth compared to the 130 ohm branch case.

Figure 20.52. Coupled and transmitted signals for 180 ohms first branch impedance in narrow span.

Figure 20.53. Coupled and transmitted signals for 180 ohms first branch impedance in wide span.

- Isolation and return loss for this circuit are shown in Figure 20.54.

Figure 20.54. Isolation and return loss for 180 ohms first branch impedance.

- Phase graphs are given in Figure 20.55 and 56.

Figure 20.55. Narrow span phase plots of S21 and S31.

Figure 20.56. wide span phases of  S21 and S31.

- Delay plots are shown in Figure 20.57.

Figure 20.57. Narrow and wide span delays of S21 and S31.

- Further experiments are carried out by increasing the first branch impedance and it is seen that the element values are approaching to the case of two branch coupler, as expected.

CONCLUSIONS  ON  EXPERIMENTS FOR  1930 - 1990  MHz BAND

- Bandwidth of branchline couplers can be increased by using additional branches. However the improvement obtained in a very small percentage of the band is negligible. Since the bandwidth concerned in the above experiments are very narrow,  (60 MHz centered at 1960 MHz: 3 % ), very slight improvements are obtained in the flatness and amplitude balance between coupled and transmitted port signals.

- Coupled signal amplitude is flatter compared to the transmitted signal because of the difference between the input-output paths.

- Transmitted signal shows a narrower passband, partly due to the natural coupler operation and partly due to the large difference in impedances to be matched.

- Balance between coupled and transmitted signal amplitudes may be obtained by a separate equalizer circuit placed at one of these output ports.

- The following experiments will be carried out over a wider bandwidth to get more insight.

- However, before going into branchline couplers again, first simple impedance matching by stepped lines will be considered to review the basic limitations coming from differing terminations.

EXPERIMENT-13: Systematization of manual optimization for three baranch couplers

In the previous experiments on two and three branch couplers attention was focused on optimizing the amplitude balance between transmitted and coupled signals together with maximized return loss and isolation in a bandwidth of f1=1930 MHz, f2 =1990 MHz. In the experiments that will be described below, now the phase response is also taken into account together with the other parameters.

Experiments target optimization in the bandwidth from 1900 MHz to 2000 MHz  showing the effects of optimization better compared to the previous narrower band cases.

- Figure 20.58 shows S parameters of a two branch impedance transforming coupler from 50 ohms to 2 ohms, plotted in the band 1900-2000 MHz, for comparison with those of three branch couplers. Note the 0.12 dB variation in amplitude of S21 over the band and 0.8 dB amplitude imbalance between S21 and S31. S11  falls below 20 dB at band edges.

Figure 20.58. S-parameters of a two branch impedance transforming coupler.

Figure 20.58. S-parameters of a two branch impedance transforming coupler (continued).

- Figure 20.59 shows the even mode half circuit, S-parameters, phase difference between S21 and S31 , together with group delays of S21 and S31 of a three branch coupler, optimized for the best amplitude balance together with best return loss and isolation by taking the first branch as 115 ohms arbitrarily. This circuit is termed as Solution-1. It will be used as a starting point to find other better solutions by changing the first branch impedance. The steps in this search will also clarify effects of various elements on various responses of the circuit.

Figure 20.59. Responses of  Solution-1 (with 115 ohms first branch impedance).

Figure 20.59. Responses of  Solution-1 (with 115 ohms first branch impedance) (continued).

- In the search of a better solution, let’s change the first branch to 120 ohms (Figure 20.60). It will be seen that balance between amplitudes of S21 and S31 will be upset. Then, by tuning the impedance of line-1 (17.1 ohms) we recover the balance. Isolation and return loss is not effected much by this change, as is seen from responses in Figure 20.61. We call this circuit as Solution-2. There is no difference in the S-parameter magnitudes of the new and old circuit. The most notable difference appears in the phase responses. Phase peak-to-peak deviation of Solution-1 at band edges was 0.12 degrees, while that of Solution-2 is 0.24 degrees. There is a very slight increase in delay of  S21  (from 488 pS to 490 pS).

Figure 20.60. Finding Solution-2 by changing branch-1 impedance and then adjusting TLine-1 impedance.

Figure 20.61. Responses of Solution-2.

Figure 20.61. Responses of Solution-2 (continued).

- Then, in the circuit of Solution-2, we pass to branch-3. Tune it to get better isolation and return loss. This will degrade balance between amplitudes of S21  and S31. Recover balance by tuning Branch-2 only. Thus, we get Solution-3 shown in Figure 20.62, with responses shown in Figure 20.63. It is seen that

i )   Amplitude imbalance is reduced to 0.03 dB.

ii )  Return loss and isolation are degraded slightly.

iii ) Peak-to-peak phase deviation from 90 degrees is reduced to 0.06 degrees.

iv ) Delays are reduced to 482 pS.

Thus, the only improvement is obtained in Phase deviation from 90 degrees. This is obtained at the expense of a slight degradation in return loss and isolation.

Figure 20.62. Obtaining Solution-3 by tuning Branch-3 to improve isolation and return loss, then by tuning Branch-2 to recover balance.

Figure 20.63. Responses of Solution-3.

- OBSERVATION:  The procedure described above can be used to find new solutions. It seems to be systematic. Let’s check whether this observation holds on phase adjustments also.

- In Figure 20.64,  the first branch impedance is set to  125 ohms. Amplitude balance is lost. It is recovered by tuning Line-1 only, giving the circuit called Solution-4. Responses are shown in Figure 20.65. We note the following points:

i )  Amplitude balance is the same as previous cases.

ii ) Return loss and isolation are about 30 dB.

iii ) Peak-to-peak phase deviation is about 0.16 degrees.

iv ) Delay of S21  is about 482 pS, while that of S31  is about 486 pS.

Figure 20.64. Solution-4 even mode circuit.

Figure 20.65. Responses of Solution-4.

Figure 20.65. Responses of Solution-4.

- Next, in the circuit of Solution-4, we pass to Branch-3 again ( as we have done in the previous cases), but now tune Branch-3 impedance while observing the phase response (in previous cases we were observing isolation and return loss). It is observed that phase deviation can be adjusted nicely without degrading return loss and isolation. This adjustment degrades the amplitude balance which can be recovered by adjusting Branch-2 impedance. Resulting circuit (Solution-5) (even half) is shown in Figure 20.66 together with Solution-4. These two circuits form  the two steps of the systematic approach for optimization which takes phase response also into consideration. Responses of Solution-5 are shown in Figure 20.67.

Figure 20.66. Two steps of an optimization procedure which includes phase response also.

- OBSERVATIONS: The following observations give useful hints about the optimization procedure which includes phase response as well.

i ) Tuning Branch-3 decreases phase deviation while also reducing the delay and not effecting return loss and isolation much. Only amplitude balance is effected by this tuning.

ii )  Amplitude balance is corrected by tuning Branch-2. It was observed that tuning branch-2 doesn’t effect the adjusted phase. This is an important advantage for adjusting amplitude balance and phase balance, independently of each other.

iii ) Using this advantage, phase deviation is adjusted to be equal ripple about the 90 degrees, as shown in Figure 20.67.

iv ) As phase is adjusted, it was observed that delays of S21  and S31  are decreasing and approaching each other. Both delays are about 478 pS with small variation over the frequency band.

v ) Solution-5 seems to be the best one among the circuits studied in this set of experiments.

vi ) Better solutions may be searched using the same approach.

Figure 20.67. Responses of Solution-5.

Figure 20.67. Responses of Solution-5 (continued).

- Let’s check these observations on another experiment using 110 ohms for the first branch, to see also whether decreasing first branch impedance will yield a worse or better circuit.

- In Figure 20.68 degradation of S21 and S31 are shown after setting first branch to 110 ohms and tuning line-1 (to 16.7 ohms) to recover the amplitude balance. Balance is obtained at about 3.07 dB instead of 3.01 dB. This is because of degradation in return loss and isolation which are about 20-25 dB’s (Figure 20.69). Especially S11 requires improvement.

Figure 20.68. Amplitude balance for S21 and S31  for 110 ohm first branch and after tuning TLine-1

to 16.7 ohms.

- In order to improve S11, Branch-2 is tested first, giving no improvement on S11. TLine-2 is also tested giving very slight improvement on S11 but badly effecting S41. Finally, Branch-3 is tested, giving improvements in both S11  and S41 . Thus, S11  is maximized by tuning Branch-3. However, this time, amplitude balance is deeply effected.

- In order to recover amplitude balance, TLine-1, Branch-2 and TLine-2 are all tested, all being useless, because when balance is targeted, isolation and/or return loss were badly effected.

- Then, we have chosen arbitrarily, TLine-1, tuned it so that balance is improved slightly while isolation and return loss are degraded a lot.

- Then tune Branch-2, to improve isolation and return loss while improving balance slightly .

- Tune Branch-3 to equate amplitudes at the expense of degrading  return loss (which still remains below 20 dB).

- Tune Branch-2 to improve return loss and isolation while degrading balance.

- Recover the balance by retuning Branch-3 which also improves return loss.

- Retune TLine-2 to improve return loss more, degrading balance. Recover balance by tuning Branch-3.

- Thus far, a 26 dB return loss is obtained with equal amplitudes for S21 and S31  and good isolation.

- Now, it is the time for checking the phase response. Tune TLine-2 for equal ripple deviation from 90 degree phase difference. This action degrades amplitude balance. Amplitude balance is recovered by tuning branch-2 which doesn’t effect equal ripple property of the phase.

- Tune TLine-2 again to get a maximally flat phase difference. Correct amplitude imbalance by readjusting Branch-2.

- The circuit in Figure 20.70 and responses shown in Figure 20.71 are obtained.

Figure 20.69. Return loss and isolation after setting Branch-1 to 110 ohms and tuning Line-1 to 16.7 ohms.

OBSERVATIONS:

i ) Compared to the previous examples, this time convergence to a solution took more steps of iterations. This might be the indication of the LOCAL OPTIMUM  nature of the solution.

ii ) Solution is reached at the expense of tolerating NOT SO GOOD return loss and isolation (relative to the previous cases). However, they are still well below the 20 dB level (close to 30 dB).

iii ) We have again found an element whose adjustment is not effecting the phase response. Tuning Branch-2 is not effecting the phase response.

iv ) This time, a maximally flat phase difference is targeted and obtained. We could as well obtain an equal ripple deviation from 90 degrees.

v ) Delays of both S21 an S31 are about 480 pS and flat within the band.

Figure 20.70. The circuit obtained for 110 ohms first branch impedance.

Figure 20.71. Responses for the optimized circuit with 110 ohms first branch impedance.

20. 6. LUMPED ELEMENT BRANCHLINE COUPLERS

At low microwave frequencies and in MIMIC applications branchline coupler dimensions become too large to be practical. For such applications the main lines and branch lines may be approximated by their lumped element models. As an example let’s design a two branch 3 dB coupler operating on 50 ohms terminations with passband center at 2000 MHz. The element values of transmission line coupler are shown in Figure 20.72.a. Here the line lengths are quarter wavelength at f=2000 MHz. In Figure 20.72.b, the ideal inverter model of the branchline coupler is shown.  Response of that coupler is frequency independent and ideal. One may transform these circuits into lumped element form starting from either of these two circuits.

Figure 20.72. a) Two branch 3 dB directional coupler. B) Ideal branchline coupler made up of ideal inverters.

Even and odd half circuits needed in analysis of these couplers on Filpro are shown in Figure 20.73.a and b. The transmission lines forming the branches become open circuit stubs in even half and short circuit stubs in odd half circuit, as shown in Figure 20.73.a. The ideal inverters of the branches are first replaced by their constant reactance Pi (or Tee) models (Figure 20.73.b) and then the even and odd halves of the Pi models are placed into the circuits as shown in Figure 20.73.c. On Filpro, these circuits are formed using “Design / By Inserting Elements” approach and saved with proper names.

Figure 20.73. a) Even and odd halves of transmission line directional coupler. c) Constant reactance Pi model of ideal inverter and its even odd halves. c) Even-odd halves of coupler with ideal inverter.

Figure 20.74. Amplitudes and phases of signals at the ports of the transmission line coupler.

Figure 20.75. Amplitudes and phases of signals of coupler with ideal inverters.

Figure 20.74  shows the amplitude and phases of signals at the ports of transmission line coupler. It is seen that device operates properly in a narrow band around 2000 MHz. The coupler with ideal inverters have, as expected, ideal response. Coupling is 3.01 dB and phase difference between transmitted and coupled signals is 90 degrees at all frequencies with infinite return loss and isolation. Clearly, the branchline coupler with transmission lines is a crude approximation of the ideal inverter version where ideal inverters are simply replaced by quarter wavelength transmission lines. This link between transmission line and ideal inverter models forms a hint to get wider bandwidth couplers. The only thing we have to do is to find better models for approximating ideal inverters (good exercise?- consider the command “Replace Ideal Inverter By /...” to check the better candidates for approximating ideal inverters of a branchline coupler).

Transition from these couplers to lumped element couplers may be carried out by starting from either model. Some examples are given below.

- The transmission line pieces of the branchline coupler may be directly replaced by their lumped element equivalents using command “Replace TLine By / Pi Model / Csh+Lsr+Csh / f= 2000 MHz” while the shunt stubs are either directly replaced by their lumped model using the command “Replace By Lumped” or, better, their transmission line forms are replaced first by the lumped version and then its even and odd halves are formed. In Figure 20.76.a, the version where the main line is replaced by a single Pi LC model and shunt stubs are converted directly into lumped element form using command “Replace By Lumped”. The amplitude response shown in Figure 20.76.b indicates that this is not a good approximation. Amplitudes are equal only at 2000 MHz.

Figure 20.76. Replacement of quarter wavelength lines by single Pi sections to give a lumped element coupler.

- We may first split the main lines and branch lines into two or more sections and then replace each piece by lumped element Pi and /or Tee sections to get a better approximation for transmission lines (Exercise?).

- As we have noticed above, transmission line pieces are themselves not good approximations for the ideal inverters. Therefore better approximations may be obtained if we start conversion to lumped element form directly from the ideal inverters. Ideal inverters have many practical approximations. Some of these approximations might be useful for that purpose.

REFERENCES AND SUGGESTED READINGS:

[1] J. Reed, G. Wheeler, “A method of analysis of symmetrical four port networks”, IRE Trans. on Microwave Theory and Technique, Vol. MTT. 4, pp. 246-252, 1956.

[2]  J. Reed, “The multiple branch waveguide coupler”, IRE Trans. on Microwave Theory and Technique, Vol. MTT. 6, pp. 398-403, 1958.

[3]  K. G. Patterson, “A method for accurate design of a broadband multibranch waveguide coupler”, IRE Trans. on Microwave Theory and Technique, Vol. MTT. 7, pp. 466-473, 1959.

[4]  L. Young, “Synchronous branch guide directional couplers for low and high power applications”, IRE Trans. on Microwave Theory and Technique, Vol. MTT. 10, pp. 459-475, 1968.

[5]  R. Levy, L. E. Lind, “Synthesis of symmetrical branch guide directional couplers”, IEEE Trans. on Microwave Theory and Technique, Vol. MTT. 16, pp. 80-89, 1968.

[6]  G. B. Riblet, “A directional coupler with very flat coupling”, IEEE Trans. on Microwave Theory and Technique, Vol. MTT. 26, pp. 398-403, 1978.

[7]  M. Muraguchi, T. Yukitake, Y. Naito, “Optimum design of 3 dB branch line couplers using microstrip lines”, IEEE Trans. on Microwave Theory and Technique, Vol. MTT. 31, pp. 674-678, 1983.

[8]  G. Kemp, J. Hobdel, J. W. Biggin, “Ultra wideband quadrature coupler”, Electronics Letters, Vol. 19, pp. 197-199, 1983.

[9]  Y. C. Ho, “Transform Impedance with a branchline coupler”, Microwaves, 15, pp. 47-52, May 1976.