CHAPTER 21

PARALLEL  COUPLED  LINE  DIRECTIONAL  COUPLERS

21. 1. INTRODUCTION

Parallel coupled line directional couplers are made up of two coupled transmission lines operating in TEM or quasi TEM mode. The simplest of such couplers are made up of two identical, coupled uniform transmission lines, as described in Figure 21.1.a where Port-1 is defined as the input port, Port-2 is termed as transmitted signal port, Port-3 is isolated port and Port-4 is the coupled signal port. In general, couplers may be designed using symmetric or asymmetric, stepped or continuously nonuniform transmission lines. Assuming no reflection at input and perfect isolation from Port-3, the input power is shared between the transmitted port (Port-2) and coupled port (Port-4). Degree of coupling between lines sets the coupled signal level at Port-4. Directional coupling between the two lines occur through both electric and magnetic fields. The wave traveling on line-1 (from Port-1 to Port-2) creates a backward traveling wave on line-2, from Port-3 to Port-4. Thus no signal appears at Port-3 (hence the name isolated port) while part of the input power of Port-1 appears at the adjacent Port-4. Similarly, a wave launched from Port-2 towards Port-1 on line-1 induces a wave traveling from Port-4  to Port-3, transferring part of the input power to Port-3 while no signal appears at Port-4 (isolated from Port-2). Since the coupling occurs between oppositely directed waves, these couplers are also called backward couplers 1. This behaviour is opposite to that observed in branchline and waveguide directional couplers where a forward traveling wave on line-1 creates a forward wave on line-2.

Figure 21.1. A single section, uniform parallel coupled line (backward wave) directional coupler.

If all ports are terminated in the same resistance R= SQRT(Z0e*Z0o) where Z0e and Z0o  are even and odd mode characteristic impedances of the coupled lines, then this four port has the following properties:

- Input impedances at all ports are equal to R, hence appear as matched to the source of resistance R.

- Phase difference between transmitted signal (Port-2 signal) and coupled signal (Port-4 signal) is 90 degrees (with transmitted signal being lagged) independently of frequency (hence the name Quadrature couplers).

- Port-3 is isolated from Port-1 (no signal at Port-3 when Port-1 is driven) independently of frequency.

- Coupling coefficient k is defined as

k = 20 log(Z0e-Z0o)/(Z0e+Z0o)  dB

Thus, apparently, such a four port works as a directional coupler independently of frequency. Therefore these are inherently wideband elements. Bandwidth of such devices are set by the coupled signal level variation with frequency. The coupling coefficient k defined above gives the maximum level of coupled signal which occurs at frequencies where the line length is a quarter wavelength or its odd multiples (termed as center frequency). The coupled signal level decreases  as frequency goes away from center frequency, becoming zero at half wavelength frequency or at its multiples. Thus, coupled and transmitted signal levels vary periodically with frequency.

- Since the structure is symmetric, it can be analyzed as superposition of the even and odd mode halves shown in Figure 21.2.

Figure 21.2. Even and odd mode half circuits of the symmetric single section directional coupler.

Figure 21.3. a) Stepped symmetric directional coupler. b) Stepped asymmetric directional coupler.                     c) Nonuniform asymmetric directional coupler. d) Nonuniform symmetric directional coupler.

Other kinds of commonly used directional couplers are shown in Figure 21.3. The stepped and nonuniform couplers having symmetry along the lines have the quadrature phase relations between the transmitted and coupled signals. The asymmetric ones do not have this property. These structures are used to realize very wide band couplers. Impedances of stepped symmetric and asymmetric directional couplers are available in 2 and 3 respectively for some coupling ripples and some number of steps.

21. 2. PARALLEL COUPLED LINE COUPLER ANALYSIS ON FILPRO

Example 21.1. A 3 dB backward wave directional coupler analysis on Filpro.

Let’s design a 3 dB coupler that will operate at center frequency f0=2000 MHz with R=50 Ohms terminations.  Using the equations given above, the even and odd mode characteristic impedances come out as Z0e=120.71 ohms and Z0o=20.71 ohms. In order to have coupler center frequency at f0=2000 MHz, length of coupled lines will be l=37.5 mm (quarter wavelength at 2000 MHz). In Filpro the even and odd mode half circuits can be formed easily and saved separately using the following command set:

“Design / By Inserting Elements / Insert / TLine: Z=120.71 ( and 20.71) , Length=37.5 mm, Er=1 (dielectric constant)”

The resulting circuits are shown in Figure 21.4 and are saved as Even1 and Odd1.

Figure 21.4. Even and odd mode half circuits on Filpro.

Then the directional coupler formed by these halves are analyzed using the following command set (Figure 21.5):

Figure 21.5. Command set for analysis of parallel coupled line directional couplers.

“New / Design / Symmetric Cross-coupled Circuits / Analyze Parallel Coupled Line Couplers/ Even1, Odd1 / Plot Response Only”

Amplitudes of signals at each port as function of frequency are shown in Figure 21.6.a and b. Periodicity of responses is clear. Return loss (S11) and isolation (S31) are very high. Coupled and transmitted signals are equal (3.01 dB) at 2000 MHz and odd multiples. Phases of transmitted and coupled signals are shown in Figure 21.6.c together with their difference. It is seen that phase difference is flat 90 degrees at all frequencies.

Figure 21.6. a, b) Signal levels at the ports of directional coupler. c)Phases of transmitted and coupled signals

21.  3. LUMPED ELEMENT DIRECTIONAL COUPLERS

At low frequencies (UHF and below) and in MIMIC structures, coupled line lengths are too large to be practical. For such applications lumped element versions of the same couplers can be used. For example the previous directional coupler can be converted into lumped element form easily in Filpro as follows (Figure 21.7):

Example 21. 2: Lumped element directional couplers

- Load the circuit Even1.

- With cursor under the transmission line (with even mode characteristic impedance, Z0e =120.71 Ohms), apply command “Replace TLine By / Pi Model / Csh+Lsr+Csh / f = 2000 MHz”. The line will be replaced by its lumped element Pi equivalent made up of a series inductor surrounded by two equal shunt capacitors.

- Save the resulting circuit as Even2.

- Load the circuit Odd1.

- Repeat the same command by placing the cursor under the transmission line (having odd mode characteristic impedance, Z0o =20.71 Ohms).

- Save the resulting circuit as Odd2.

Figure 21.7. Conversion of parallel coupled line coupler into lowpass lumped element coupler.

The inductances of  the circuits Even2 and Odd2 can be treated as even and odd mode components of a coupled coil. Similarly, the shunt capacitors of the two circuits can be treated as even and odd mode decomposition of a capacitive Pi bank with one node on each line. That is the complete lumped element coupler can be described as shown in Figure 21.8. Element values of this complete four port circuit can be calculated in terms of the known even and odd mode element values as shown in the figure.

Figure 21.8. Lumped element directional coupler circuit and its even-odd mode decomposition.

Figure 21.9. Signal amplitudes and phase response of lumped element directional coupler with single Pi approximation of lines.

Figure 21.9. Signal amplitudes and phase response of lumped element directional coupler with single Pi approximation of lines (continued).

Wideband loss responses of this coupler are shown in Figure 21.9.a.  It is seen that transmitted signal path has a lowpass characteristics while coupling has a bandpass-like characteristics. Periodicity of responses is lost. Perfect return loss and isolation characteristics of coupled line coupler are now replaced by frequency dependent characteristics. Zoomed response curves are shown in Figure 21.8.b. It is seen that 3.01 dB coupling region is narrowed about f0 =2000 MHz. Phases of S21 and S41 are shown in Figure 21.9.c together with their difference. Phase difference is still about 90 degrees over a fairly wide band.

Example 21.3. Double Pi approximation for coupled lines.

Instead of replacing the transmission lines of the even and odd mode circuits by lumped elements as a whole, we may first split them into two or more pieces and then replace them by lumped element equivalents, as described below:

- Load the circuit Even1. Use the command “Split / Two Equal Pieces” to split into two lines with halved lengths. Save the circuit as Even21.

- Apply command “Replace TLine By / Pi Model / Csh+Lsr+Csh / f=2000” to replace the line by the 
       lumped element equivalent. Save the circuit as Even22.

- Repeat the same on Odd1 and save the resulting circuits as Odd21 and Odd22.

- The even and odd half circuits are shown in Figure 21.10. The complete four port coupler circuit is shown in Figure 21.11 where the element values should be calculated using the same formulae as described in the previous example. Amplitude and phase responses are shown in Figure 21.12. In Figure 21.12.b, signals S21 and S41 for transmission line coupler, single Pi coupler and double Pi coupler are overplotted in zoomed mode. Clearly, double Pi model is a much better approximation compared to the single line case.

Figure 21.10. Even and odd half circuits of the double Pi directional coupler and amplitude response for coupled and transmitted signals.

Figure 21.11. The complete four port circuit for the coupler.

Figure 21.12. Amplitude and phase responses of double Pi section coupler.

21. 4. DESIGN OF LUMPED ELEMENT COUPLERS USING IDEAL INVERTERS

More variations on lumped element directional couplers can be produced by starting from an ideal inverter model of the parallel coupled line directional coupler. Ideal inverter of the backward wave coupler can be obtained by replacing the transmission lines of even and odd halves by ideal inverters, as follows:

Example 21. 3. Design of lumped element couplers using inverters

- Load the circuits Even1 and Odd1 one by one  and apply command “Replace TLine By / Various Models / Ideal Inverter” (Figure 21.13.a). The lines will be replaced with ideal inverters of impedance equal to the even and odd mode characteristic impedances of the respective lines (Figure 21.13.b). Save the circuits as Even3 and Odd3 respectively. Amplitudes of S21 and S41 are exactly equal to 3.01 dB, at all frequencies. Return loss and isolation are infinite while phase difference between S21 and S41 are exactly 90 degrees at all frequencies. Thus, this model is an ideal directional coupler. Any practical coupler should be derivable from this model by approximating the ideal inverters with practical circuits.

Figure 21.13. Directional coupler with ideal inverters.

- For example quarter wavelength lines of transmission line coupler are just approximations of the inverters. Transmission lines act as inverter only at their quarter wavelength frequency. Therefore response of transmission line coupler deviates as frequency is moved away from the quarter wavelength frequency.

- The single Pi lumped element coupler can be derived from ideal inverter coupler by first converting the ideal inverters into their constant reactance Pi models using the command “Inverters / Replace Ideal Inverter By / Constant Reactance / Pi Cct” ) and then applying the command “Constant Reactances / Replace Constant Reactance By / +,- X -to- L or C / f = 2000”. The shunt constant reactances (negative) will be replaced by lumped capacitors while the series arm constant reactance (positive) will be replaced by an inductor, giving the usual single Pi model of directional coupler. Since inductors and capacitors are equal to the constant reactances only at a single frequency, this approximation is a narrowband one, which is in fact worse than the quarter wavelength line approximation, yielding the responses we have studied above.

- Just as an exercise, let’s try to approximate the inverter by some exotic elements. In Figure 21.14 the inverter of even and odd halves are replaced by constant reactance Pi sections, using the  command “Inverters / Replace Ideal Inverter By / Constant Reactance / Pi Cct”. The inverters will be replaced with Pi sections of constant reactances as shown in Figure 21.14.b. Save them as Even4 and Odd4.

- Load the circuits Even4 and Odd4, and with cursor under the middle element of the Pi sections, apply command  “Constant Reactances / Replace Constant Reactances By / +, - X by L or C / f = 2000 MHz”. Since these reactances are positive, they will be replaced by inductors (Figure 21.14.c). Save them as Even5 and Odd5.

- Load the circuit Even5. With cursor under the shunt elements of the Pi section, apply command “Constant Reactances / Replace Constant Reactances By / +, - X by L or C / f = 2000 MHz”. These elements will be replaced by shunt capacitors. Save the circuit as Even6 (Figure 21.14.d).

Figure 21.14. Development of a bandpass type directional coupler.

- Load the circuit Odd5. With the cursor under the shunt elements of the Pi section, apply command “Constant Reactances / Replace Constant Reactances By / Parallel LC / L=5 nH / f=2000 MHz”. The constant reactances will be replaced by parallel LC resonators (Figure 21.14.d). Thus, in the odd mode half circuit, instead of a shunt capacitor, we placed an LC resonator. The shunt inductors introduced into these arms represent lumped inductors connected between the nodes 1 and 4, and between the nodes 2 and 3., as described in the complete four port coupler shown in Figure 21.15. The relations between element values of even-odd halves and the complete circuit are also shown in the same figure. The extra inductors add a bandpass nature to both transmitted and coupled signal paths.

Figure 21.15. The complete four port lumped element directional coupler and its decomposition into even and odd halves.

Wide span and narrow span signal amplitudes are shown in Figure 21.16.a and b. The phase responses are shown in Figure 21.17. Clearly, this four port works also as a directional coupler over a fairly broad band however with worse deviation than the single Pi lumped coupler.

Figure 21.16. Signal amplitudes in wide and narrow span.

Figure 21.17. Phases of coupled and transmitted signals of bandpass type lumped coupler.

- All these examples show that wider bandwiths can be obtained only by using better practical inverter circuits to approximate the ideal inverters. Also the even and odd halves formed by these practical inverters should be in such a form as they will yield a realizable four port.

As another exercise, let’s inspect an unrealizable but better approximating inverter circuit. Using the command “Inverters / Replace Ideal Inverter By / Pi-Tee-L / Inductive Pi / f = 2000 MHz”, we get the even and odd mode circuits shown in Figure 21.18.a with negative shunt inductors. Amplitude responses in wide and zoomed forms are shown in Figure 21.18.b and c. Clearly, this response is much better than the previous ones. S21 has a highpass nature while S41 has a slight bandpass nature.

Figure 21.18. An unrealizable coupler circuit having negative elements.

Figure 21.18. An unrealizable coupler circuit having negative elements (continued).

21. 5. STEPPED PARALLEL COUPLED LINE DIRECTIONAL COUPLERS

For wider bandwidth applications, stepped directional couplers can be used. Crystal and Young gives impedance tables for symmetric (90 degree phase difference) multi section couplers.  As an example for the use of the readily available tables for multi-step directional couplers, let’s design a 3 dB coupler with 0.05 dB equal ripple coupling response.

Example 21. 4. A three section stepped line directional coupler.

Normalized even mode impedances are found from Table A-1 of  2  as Z0e1 = Z0e3 = 1.14888  and Z0e2 = 3.161. The denormalized impedances come out as Z0e1 = Z0e3 = 57.444 ohms, Z0o1 = Z0o3 = 43.52 ohms, Z0e2 = 158 ohms and Z0o2 =15.8 ohms. Using Insert command, the even and odd halves of the coupler are formed as shown in Figure 21.19.

Figure 21.19. Even and odd halves of a three section 3 dB directional coupler.

- Signal amplitudes and phases are shown in Figure 21.20. Clearly, bandwidth is widened a lot compared to the single section case. Phase difference is 90 degrees over a very wide band.

Figure 21.19. Signal amplitudes and phases of the three step directional coupler.

- If necessary, a narrow band response around 2000 MHz can be improved so that S21=S41 in this region, to approximate maximally flat case. This can be done by loading the individual circuits Even11 and Odd11 and then tuning the outside elements until the insertion loss becomes 3.01 dB for both circuits. Even and Odd circuit element impedances should be tuned in opposite directions to preserve their product  constant. One example for such a tuning gave the element values Z0e1 = Z0e3 = 57.204 ohms, Z0o1 = Z0o3 = 43.67 ohms, Z0e2 = 158 ohms and Z0o2 =15.8 ohms. The circuits and close in response are shown in Figure 21.20.a and b respectively.

- This circuit can be converted into lumped element form using the same approach as in the previous example. In Figure 21.20.c, the transmission lines are splitted into two pieces. Then each piece is replaced by its lumped Pi equivalent (Figure 21.20.d).

- Response of  resulting lumped element coupler is shown in Figure 21.21. Close in response shown in Figure 21.21.b compares favorably with those obtained from single section coupler. In fact it is almost identical to the response of the unrealizable coupler shown in Figure 21.18.

Figure 21.20. Three section coupler converted into lumped element form.

Figure 21.20. Three section coupler converted into lumped element form (continued).

Figure 21.21. Response of  lumped element coupler derived from a three section parallel coupled line coupler with double Pi approximation.

REFERENCES:

1  L. Young, “Parallel Coupled Lines and Directional Couplers”, Artech House, 1972.

2 E. G. Crystal and L. Young, “Theory and tables of optimum symmetrical TEM-mode coupled transmisson line directional Couplers”, IEEE, Trans. MTT,  pp. 544-558, Sept. 1965.

3 R. Levy, “General Synthesis of asymmetric multi-element coupled transmission line directional couplers”, IEEE Trans. MTT-11, No. 4, pp.226-279, July, 1963.