CHAPTER 22

RAT  RACE  HYBRIDS

22. 1. DISTRIBUTED  ELEMENT  RAT RACE  HYBRIDS

Rat race hybrids are 3 dB power dividing four ports with 0 degree or 180 degree phase difference between the output signals depending on input port (also called magic Tee or 0-180 degree directional coupler). The simplest form (which is also the most commonly used form) of rat race hybrid schematics is shown in Figure 22.1. The convention for port numbering and path lengths between the ports at the frequency of operation are also indicated. The full closed path is 5 quarter wavelengths at the frequency of operation and the transmission line has an impedance of R 2 where R is the termination resistance which is the same at all ports.

- When Port-1 is used as input, the signal power will be shared between Port-2 and Port-4 equally with zero degree phase difference, because both signals traverse a quarter wavelength path (-90 degree electrical length). No signal will reach to Port-3 (isolated port).

- When Port-3 is the input port, then the signal power will again be shared equally between Port-2 and Port-4, but now with 180 degrees of phase difference, because of the path length difference which is half wavelength at the frequency of operation. No signal will reach to Port-1 (isolated port).

- When signals are applied at Port-1 and Port-3 simultaneously, the splitted signals will combine at Port-4  (same phase lag with respect to the input ports) and will be subtracted at Port-2 ( 180 degree phase difference with respect to the input ports).  Therefore sometimes Port-2 is termed as Difference Port and Port-4 is called sum port.

Figure 1. Schematics of a 3 dB branchline coupler with different input-output terminations.

The circuit is symmetric about the horizontal midplane. Therefore analysis of this structures can be carried out in Filpro using Even-Odd mode decomposition approach. Let’s proceed through an example.

Example 22.1. A rat race hybrid at f=2000 MHz.

A rat race schematics for R=50 ohms to be used at f = 2000 MHz is shown in Figure 22.2.a when Port-1 is the input port. The branches 1-4 and 2-3 are open and short circuited in their middle in even and odd mode excitations respectively giving the schematics of Figure 22.2.b and c. Thus these halved arms become open and short circuited stubs respectively. In Filpro these half circuits can be configured using “Design / By Inserting Elements” approach, as shown in Figure 22.3.a and b. Quarter wavelength is 37.5 mm at f=2000 MHz. Thus, the length of stubs on left are 18.75 mm while the stubs on right are 56.25 mm. If Port-3 is the input port, then the even and odd half circuits should be reversed (right-left reversal, using Reverse Circuit command on Filpro).

Figure 22.2. a) Schematics of rat race coupler with R = 50 ohms to be used at f=2000 MHz when Port-1 is the input port. b) Even mode half circuit when Port-1 is the input port. c) Odd mode half circuit when Port-1 is the input port. d) Odd mode half circuit when Port-3 is the input port. e) Odd mode half circuit when Port-3 is the input port.

Figure 22.3. Even and odd mode half circuits on Filpro when Port-1 is the input.

Response of the circuit can be  plotted using the command set “Design / Symmetric Cross-coupled Cct / Analyze Rat Race Hybrid / Loss (or Phase or Delay)”. Loss response when Port-1 and Port-3 are used as  input are shown in Figure 22.4.a and b. Note that the transmitted signal amplitudes do not change when driving port is changed. Only self reflections at the input ports change. In Figure 22.4.c, the close in amplitude responses at ports 2 and 4 are shown. Signal amplitudes are equal only in a narrow band about the center frequency. In Figure 22.4.d, the phases of signals S12 and S14 (with Port-1 being the input port) are shown together with their phase difference. As expected, these signals are in phase in a narrow band around the center frequency (zero phase difference). In Figure 22.4.e, the phases of signals S23 and S43 and their difference are shown when input is applied to Port-3. Phase difference between the output ports are 180 degrees about the band center.

Figure 22.4. Various responses of rat race hybrid: a) Port signal amplitudes when Port-1 is the input. b) Port signal amplitudes when Port-3 is the input. c) Close in view of signal amplitudes reaching to Port-2 and Port-4 when Port-1 is the input. d) Phase difference between output signals when Port-1 is the input. e) Phase difference between output signals when Port-3 is the input.

Figure 22.4 (continued). e) Phase difference between output signals when Port-3 is the input.

Unequal power splitting between the output ports is possible by making characteristic impedance of adjacent line pieces unequal, as described in Figure 22.5. Ratio of powers delivered to the output ports are inversely proportional to the squares of characteristic impedances of the line segments connecting the input port to the respective output ports. Since the structure is symmetric with respect to the horizontal axis, such structures can also be analyzed using the even-odd mode approach described above. Limitations on realizable line impedances restrict the coupling to below -10 or -6 dB. In order to alleviate such difficulties, various practical structures are proposed by several authors 1  - 4. Among those, the symmetric ones can be analyzed using Filpro.

Figure 22.5. Rat race hybrid for unequal power division.

22. 2. IDEAL INVERTER  AND  LUMPED ELEMENT  MODELS FOR RAT RACE HYBRIDS

More variations on rat race hybrids can be obtained through their ideal inverter models. The ideal inverter model of a 3 dB rat race hybrid is shown in Figure 22.6. It is obtained from the transmission line model simply by replacing the quarter wavelength lines with ideal inverters. Inverter constants are equal to the characteristic impedance of line segments. The three quarter wavelength line segment is replaced by an inverter with inverter constant equal to the negative of the impedance of the line.

Figure 22.6. a) Ideal inverter model for rat race hybrid. b) Constant reactance Pi model of an ideal inverter and its even-odd halves. c) Even and odd half circuits of rat race hybrid when driven from Port-1.

Even-odd mode decomposition is carried out by first replacing the shunt arm inverters by Constant Reactance Pi or Tee models, as described in Figure 22.6.b. Figure 22.6.c shows the even and odd mode halves of the complete rat race hybrid when it is driven from Port-1. If the hybrid were driven from Port-3 reverse of the same circuits would be obtained. In Figure 22.7 the procedure is described for obtaining this ideal rat race  3 dB hybrid from the transmission line hybrid using the commands “Replace By Ideal inverter” and “Replace By Constant Reactance”. The resulting even and odd half circuits are shown in Figure 22.7.c. Amplitude response of this ideal rat race hybrid is shown in Figure 22.7.d. As expected, couplings to Port-2 and Port-4 are perfectly 3.01 dB, independent of frequency. This model may now be used to develop various practical hybrids by replacing the ideal elements by practical equivalents, using the commands “Inverters / Replace Ideal Inverter By / ...” and “Constant Reactances / Replace Constant Reactance By / ..”.  In Figure 22.8, conversion of positive and negative ideal inverters first into constant reactance Pi model and then into an LC model are shown. More complicated equivalents mat be developed using other practical inverters and/or other constant reactance models. The complete lumped element hybrid and its even and odd mode halves are shown in Figure 22.9. Amplitude and phase responses are shown in Figure 22.10. Better behaviour may be obtained by using double Pi lumped element models for the transmission line segments of the distributed element hybrid. It is clear that many lumped element alternatives can be generated. However, all come out to be frequency selective both in amplitude and phase.

Figure 22.7. a) Replacing shunt OC and SC stubs by constant reactances. b) Replacing transmission lines by ideal inverters. c) Ideal models of even and odd halves of  3.01 dB rat race hybrid.

Figure 22.8. Lumped element models of positive and negative inverters.

Figure 22.9. A simple lumped element model for rat race coupler and even odd half circuits.

Figure 22.10. Amplitude and phase responses of lumped element directional coupler.

22. 3. IMPEDANCE TRANSFORMING RAT RACE HYBRIDS

Ideal inverters on the upper and lower branches of hybrids (between ports 1-2 and 3-4) may also be used for impedance transformation purposes. The case where R1=R4 and R2=R3 preserve the symmetry with respect to the horizontal axis, as shown in Figure 22. 11. Therefore such structures can be analyzed using even-odd mode decomposition described above.

Figure 22.11. Impedance transforming rat race coupler.

REFERENCES AND SUGGESTED READINGS

1 J. A. Malherbe, “Microwave transmission line couplers”, Artech House, 1988.

2 A. K. Agrawal and G. F. Mikucki, “A printed circuit hybrid ring for arbitrary power divisions”, IEEE   
       Trans. MTT-34, pp. 1401-1407, Dec. 1986.

3 S. Rehnmark, “Wideband balanced line microwave hybrids”, IEEE Trans. MTT-25, pp. 825-830, Oct. 1977.

4 S. March, “A wideband stripline hybrid ring”, IEEE Trans. MTT-16, p. 361, June 1968.

5 D. I. Kim,  Y. Naito, “Broadband design of improved hybrid ring 3 dB directional couplers”, IEEE Trans. MTT-30, pp. 2040-2046, Nov. 1982.