CHAPTER 23

POWER DIVIDERS

23. 1. INTRODUCTION

A general one input N output power divider with a specified tapering in output power levels can be designed and analyzed on Filpro to a limited extent. Different topological structures are possible for N-way dividers. Some typical topologies are shown in Figure 23.1. The circuit of Figure 23.1.a is termed as radial divider. The number of branches are usually limited because of extreme impedances and difficulties of interconnection of many branches at a single junction. The structure in Figure 23.1.b is termed as series divider. In this structure the delays of output signals increase progressively. Therefore they are used for applications where phase differences are not important. The structures shown in Figure 23.1.c are named as corporate dividers. Such dividers are used for feeding antenna arrays having controlled tapered amplitudes and phases.

The topology and the number of nodes in corporate dividers are dictated by the applications and the desired operational bandwidth as will be described later in this chapter.

In these figures R1, R2, .., Rn are the load resistors. The input power Po is split and directed to N output ports in accordance with a specified set of power levels P1, P2, . . .,Pn. The elements denoted by Kxy are ideal inverters representing impedance matching and filtering networks between nodes x and y. In actual implementations the inverters would be replaced by lumped or distributed element matching and/or filtering networks.

In these circuits A power divider design problem can be posed as follows:

At any node, power is split into the branches in proportion to the input admittances of the branches. So, given the desired power distribution with tapering p1=P1/Po, p2=P2/Po, …, pn=Pn/Po and given the source impedance Ro and load impedances R1, R2, . . Rn, the problem is finding the proper matching inverters Kxy satisfying the following constraints:

- At each node the powers split into the arms must have the specified ratio.

- Impedances must be matched at all nodes. The input impedances at all ports are usually fixed as Ro, R1, R2, . . Rn. However the total impedance at each internal node x is a free parameter that can be set by the designer, leaving some flexibility for alternative solutions.

These two constraints are sufficient to find the unknown matching inverters in all branches by specifying the impedances at all intermediate nodes, as will be described in the following sections.

In most complex power dividers the junctions are usually formed by connecting two or three branches. Therefore let’s study these simpler versions. More complex dividers can be formed by interconnecting such simple two or three way dividers.

Figure 23.1. Some typical N-way dividers.

23. 2. TWO WAY DIVIDERS WITH UNEQUAL POWER SPLIT RATIOS AND ARBITRARY TERMINATIONS

Let’s consider a two way divider shown in Figure 23.2. Po is the input power, P1 and P2 are the powers that will flow from node-1 to node-2 and node-3 respectively.

Figure 23.2. An arbitrary two way divider.

Denoting the normalized powers as

p1=P1P0	 and  	p2=P2P0							(23.1)

we have

P1+P2=P0									(23.2)

or

p1+p2=1										(23.3)

Z12 and Z13 are the impedances seen looking from node-1 towards node-2 and node-3 respectively. The two impedances are paralleled at junction 1 giving the total impedance Z1:

1Z12+1Z13=1Z1								(23.4)

The node impedance Z1 is a free parameter set by the designer. Power will then be split in accordance with the impedance ratio

p2p3=Z13Z12									(23.5)

Then these two equations will yield the following solutions for the impedances Z12 and Z13:

Z12=Z1(1+p3p2)								(23.6)

Z13=Z1(1+p2p3)								(23.7)

So, if the power split ratio p3p2is given the designer should select the node impedance Z1 to find the impedances Z12 and Z13. Thus, the power divider problem reduces to determination of impedance matching circuits which are represented by the ideal inverters in Figure 23.1. The impedance matching circuits K12 and K13 must match the loads R2 and R3 to Z12 and Z13 respectively.

- The simplest practical dividers can be obtained by treating K12 and K13 as ideal inverters in which case they are calculated through the relations

K12=Z12R1								(23.8)

K13=Z13R2								(23.9)

The input inverter Ko1 will match Z1 to Ro. Hence,

Ko1=Z1Ro								(23.10)

The inverters are then replaced by their proper practical equivalent circuits involving lumped and/or distributed elements. Such dividers have usually narrow operational bandwidth.

- For wider bandwidths one can use more than one inverters at each branch to match R2 and R3 to Z12 and Z13 respectively. We will get higher degree matching circuits after replacing the inverters by their practical circuits. For such cases the inverter impedances are usually found by optimization.

- One may also design special impedance matching filters to match R2 and R3 to Z12 and Z13  respectively over a desired passband in a more controlled manner.

Obviously selection of the inner node impedance Z1 effects all the inverters impedances hence the parameters of the impedance matching circuits. Therefore many alternative divider circuits can be formed by selecting different Z1.

Let’s see some applications by starting from relatively simpler ones.

EXAMPLE 23. 1. A two way unequal power divider with different load impedances

Let’s consider a two way divider operating between a source of resistance Ro=50 ohms and load resistors are R2=20 ohms and R3=100 ohms. The input power Po will be split in a specified ratio of p2p3=3 to the loads R2 and R3 respectively. We place matching inverters Ko1, K12 and K13 as shown in the figure. The two output branches are combined at node-1. Let’s select a trial node impedance Z12=40 ohms. The input impedances of the branches which are shown as Z12 and Z13 can be evaluated in terms of Z1 and p2p3 ratio using Eqs. 23.6 and 23.7:

Z12=Z1(1+p3p2)=40(1+0.333)=53.2 ohms

Z13=Z1(1+p2p3)=40(1+3)=160 ohms

Thus, the load impedance R2=20 ohms should be raised to Z12=53.2 ohms through a matching circuit represented by the inverter K12. Similarly, the load R3=100 ohms should be raised to 160 ohms through a matching circuit represented by the inverter K13. Also the node impedance Z1=40 ohms should be raised to Ro=50 ohms through the inverter (or equivalent matching circuit) Ko1.

If a single inverter at each branch is sufficient for the application at hand then the inverter impedances can be evaluated using Eqs. 23.8, 9 and 10 as:

K12=20x53.2=32.62 ohms

K13=100x160=126.49 ohms

Ko1=50x40=44.72 ohms

It should be noted that realizability of the circuit and operational bandwidth can be inspected after converting the inverters into their practical forms that is set by the technology of the production (lumped, distributed, etc.).

a) Simulation of the divider with inverters on Filpro:

The divider circuit described above can be formed on Filpro as described in Figure 23.3:

- First we form the branch 1-2 using Design / By Inserting Elements command. We type in RS=50 ohms and RL=R2=20 ohms. The RS-RL circuit is formed (Figure 23.3.a).

- Then Insert/TLine-Stubs-Inverters/Inverter: K=K12=32.62 ohms command is applied to insert the inverter K12 (Figure 23.3.b).

- With cursor under the inverter K12, the command Insert / TLine, Stubs, Inverter / Inverter/  Ko1=44.72 ohms is applied to insert Ko1. Save the circuit as Ko12.fil  (Figure 23.3.c).

- Then we form the branch 1-3 using Design / By Inserting Elements command. We type in RS=40 ohms and RL=R2=100 ohms (Figure 23.3.d).

- Then Insert list appears. Inverter is selected and K=K13=126.49 ohms is typed in. The circuit is saved as K13.fil (Figure 23.3.e).

- The circuit Ko12.fil is loaded (Figure 23.3.f) and with cursor under the inverter K12, the command External 1-Port / K13.fil / Shunt 1-Port is applied to insert the branch 1-3 (K13.fil) into the circuit as External 1-Port element to complete the divider circuit. It is saved as Div01.fil (Figure 23.3.g).

- The divider output branches can be treated as if they are multiplexer output channels. Therefore both return loss and insertion loss responses of this circuit can be analyzed on Filpro using Tune/Plot Multiplexer command as shown in Figure 23.3.h. The two output powers are seen in the insertion loss plot as straight lines because of frequency independent inverter elements.

Figure 23.3. Two way unequal power divider with inverters.

Figure 23.3. Two way unequal power divider with inverters.

b) Lowpass type lumped element power divider:

The lumped element versions of the divider can be formed by replacing the inverters with LP type  practical equivalents as described below. Let’s design a divider working around fo=1000 MHz.

- Load Ko12.fil and with cursor under the inverters apply the command Inverters / Replace Inverter by / Constant Reactance / Pi-section is applied to convert the inverter into a Pi-section of constant reactances with positive series elements and negative shunt elements (Figure 23.4.a-b).

- Combine (or F6) command is applied to combine the adjacent shunt reactances (Figure 23.4.c).

- As a convention on Filpro, positive and negative constant reactances are treated as inductive and capacitive reactances respectively. They can be converted into actual lumped or distributed inductive or capacitive elements as follows.

- With cursor under the negative shunt reactances the command Constant Reactances / Replace Constant Reactance By / Capacitor: fo=1000 MHz to convert them into shunt  capacitors. Then with cursor under the positive series reactances the command Constant Reactances / Replace Constant Reactance By / Inductor: fo=1000 MHz to convert them into series  inductors. Thus we get the lumped element  version of the inverters Ko1 and K12 (Figure 23.4.d). It is saved as Ko12a.fil.

- Next, the branch K13.fil is loaded and the inverter is replaced by its constant reactance Pi-section equivalent (Figure 23.4.e-f). Then the negative and positive constant reactances are converted into lumped capacitors and inductors respectively using the same commands (Figure 23.4.g). The resulting circuit is saved as K13a.fil.

- Finally, K13a.fil is inserted into Ko12a.fil as External 1-Port as shown in Figures 23.4.h-i to form the LP type lumped element power divider. It is saved as Div02.fil.

- Responses in Figure 23.4.k shows that the divider works roughly in the frequency range of 800-1200 MHz. The bandwidth is set by the return loss level needed together with the tolerance on the power levels at output ports.

- Bandwidth limitation is due to the approximation of ideal inverters by their lumped element equivalents. If wider bandwidth is needed we have to use more inverters in each branch.

- It is also possible to replace the inverters by their other equivalents, for example Tee-sections of constant reactances to form alternative circuits (Exercise).

- The lowpass nature of the responses is due to negative shunt reactances and positive series reactances. Highpass alternative can be formed by changing the signs of all inverters and applying the same commands (Exercise).

Figure 23.4. LP type lumped element two way unequal power divider.

Figure 23.4. LP type lumped element two way unequal power divider.

c) Two way power divider using quarter wavelength transmission line transformers:

The distributed element versions of dividers can be formed by converting the inverters into transmission line pieces which are quarter wavelength long at the targeted passband center.

- The circuit of Ko12.fil is loaded (Figure 23.5.a).

- With cursor under the inverters the command Inverters / Replace Inverter By / Quarter Wave TLine: fo=1000 MHz is applied converting them into quarter wave TL’s at fo=1000 MHz (Figure 23.5.b). The circuit is saved as Ko12r.fil.

- Then K13.fil is loaded and the same command is applied (Figure 23.5.c-d). The circuit is saved as K13r.fil.

- Then K13r.fil is inserted into Ko12r.fil as External 1-Port thus forming the divider (Figure 23.5.e-f). Responses are shown in Figure 23.5.g. The divider has BP like response with  bandwidth being roughly the same as the lumped element version. Realizability depends on the characteristic impedances of the TL’s. The impedances obtained seem to be realizable.

Figure 23.5. Two way divider with quarter wavelength TL transformers.

EXAMPLE 23. 2. A wide band two way unequal power divider

Let’s consider the same two way divider as in the previous example, but operating in a much wider bandwidth. Let’s see what bandwidth we will get by using three inverters at each branch, as shown in Figure 23.6.

Figure 23.6. A wideband unequal two way divider.

The divider will be operating between a source of resistance Ro=50 ohms and load resistors are R2=20 ohms and R3=100 ohms. The output power split ratio is p2/p3=3. We place matching circuits shown as Ko1, K12 and K13 in the figure. The two output branches are combined at node-1. Let’s select the node impedance as Z1=40 ohms, the same as in the previous example. The input impedances of the branches Z12 and Z13 were found  using Eqs. 23.6 and 23.7 as:

Z12=Z1(1+p3p2)=40(1+0.333)=53.2 ohms

Z13=Z1(1+p2p3)=40(1+3)=160 ohms

Thus, the load impedance R2=20 ohms should be raised to Z12=53.2 ohms through a three inverter matching circuit K2a, K2b and K2c as shown in Figure 23.6. Similarly, the load R3=100 ohms should be raised to 160 ohms through a three inverter matching circuit K3a, K3b and K3c. Also the node impedance Z1=40 ohms should be raised to Ro=50 ohms through the three inverter matching circuit  K1a, K1b and K1c. The inverter impedances of the matching circuits can be found on Filpro using either through manual optimization (tuning) or through Monte Carlo optimization. When the number of elements get high it is easier and faster to use Monte Carlo-Best Case optimization routine. Let’s use Monte Carlo approach to describe the details of this routine.

Matching circuits of branch 1-2, 1-3 and o-1:

Formation of the matching circuits are described in Figure 23.7.

- The circuit of branch 1-2 will match R2=20 ohms to Z12=53.2 ohms.

- Design / By Inserting Elements command is invoked and three inverters are inserted with reasonably selected initial trial values for inverter impedances, K2a=45 ohms, K2b=35 ohms and K2c=25 ohms to match 20 ohms to 53.2 ohms. The circuit is saved as M12.fil.

- Impedances of the inverters will be determined by optimization through Monte Carlo-Best Case routine as follows.

  - First, a Monte Carlo List is prepared. This list consists of the parameters of the elements to be optimized. For the circuit the parameters are the three inverter impedances. We can either point each inverter one by one and apply the command Monte Carlo / Add a Parameter or simply apply Monte Carlo / Automatic Oprs / Add All First Parameters to place the inverter impedances into the list. One of the Uniform Distribution or Gaussian Distribution is selected. Parameter Tolerance (percentage variation to be applied on the element values) is typed in.

  - Next, Reference Line is placed on Return Loss plot. This can be done using the command Reference Lines / Auto Add A Reference Line / Return Loss Only: fmin=200 MHz, fmax=1800 MHz, Ref. Value= 20 dB (or any proper level). Reference Value can be checked by plotting Return Loss response. It must be placed close to the return loss level of the default response. The region between 0 dB and Ref. Value is shaded (forbidden region).

  - Finally, Best Case command is applied. This command varies the element parameters randomly within the specified percentage tolerance and plots the return loss.

  - If a solution is found which is NOT TOUCHING the shaded (forbidden) region then iteration stops. Filpro asks if the reference level will be increased or not to get a solution with a higher Return loss level. If iterations will be continued then a positive  increment  is typed in to increase Reference Value. Also the percentage Tolerance can be changed at this stage. The iterations are continued until satisfactory return loss response is obtained (usually equal ripple return loss response).

  - If a solution can’t be obtained during an iterative cycle (about 100 iterations in a cycle), then Filpro declares that no solution is found and asks to decrease the reference level and also type in a new percentage tolerance. In order to help find a solution we may also assign slightly narrower bandwidth (higher fmin and lower fmax).

- Since inverters are constant, no variation exists in frequency responses. So, we can assign any frequency range for the forbidden region (200-1800 MHz in this example.

- After a few rounds we get about 90 dB return loss level and stop optimization procedure. The branch 1-2 is ready as shown in Figure 23.7.c. It is saved as M12.fil.

- The same approach is applied to form branches 1-3 and o-1. They are saved as M13.fil and Mo1.fil (Figure 23.7.d, e).

- Next, the divider is formed as follows:

  - The circuit Mo1.fil is loaded (Figure 23.7.f).

  - With cursor under RL=40 ohms, the command Append Filter: M12.fil is applied to append the filter M12 to Mo1. At this stage we also make RL=20 ohms by using Change command because the load resistance of M12 is 20 ohms (Figure 23.7.g).

  - With cursor under the first inverter of M12.fil, the command External 1-Port / M13.fil / Shunt 1-Port is applied to insert the branch  M13 to complete the design of the divider (Figure 23.7.h).

- Insertion loss and return loss responses are plot using Plot-Tune Multiplexer command. We get the responses shown in Figure 23.7.i. Return loss is flat at about 40 dB. S21 and S31 are also flat at S21=1.2476 dB and S31=6.0286 dB. Their difference gives roughly  p2/p3=3  (4.77 dB), as expected.

Figure 23.6. Wideband unequal two way divider with inverters.

Figure 23.7. Wideband unequal two way divider with inverters.

Realization using Quarter Wavelength Transmission Line Transformers:

The inverters of the power divider designed above can be converted into quarter wavelength transmission lines at any desired frequency, as described in Figure 23.8 to get some wide band dividers with center frequency at fo=1000 MHz.

- The branch Mo1.fil is loaded. It matches RL=40 ohms  to RS=50 ohms (Figure 23.8.a).

- With cursor under the inverters the command Inverters / Replace Inverter by / Quart Wave TLine: fo=1000 MHz is applied giving the circuit shown in Figure 23.8.b.

- Since now we have frequency dependent elements it is wise to re-optimize the return loss responses of the branches for the best match in the desired band. Initially we intended to test the divider in the bandwidth of f1=200 MHz to f2=1800 MHz. We guess that this is a too wide band for a three section impedance transformer. The power divider output powers may have unacceptable variations (ripples). Therefore let’s also try narrower bandwidths to get better match.

- Using Monte-Carlo-Best Case routine with forbidden range being f1=200 MHz and f2=1800 MHz gave the circuit shown in Figure 23.8.c. Return loss is about 22 dB. It is saved as Mo1a.fil.

- Similarly the circuit shown in Figure 23.8.d is obtained for the narrower bandwidth f1=400 MHz, f2=1600 MHz, with return loss level of 30 dB.

- For the even narrower bandwidth of f1=600 MHz, f2=1400 MHz, the circuit shown in Figure 23.8.e is obtained with return loss level of 42 dB.

- As expected we get better return loss levels for narrower bandwidths.

- The quarter wavelength TLine versions of the branches 1-2 and 1-3 are also formed in the same way for the same frequency bands as described in Figures 23.8.f-k and Figures 23.8.m-r respectively.

- Inspection of return loss levels shows that the worst return loss comes out in the branch 1-2. This is because of the highest RS/RL ratio.  For the branch o-1, RS/RL=50/40=1.25, for branch 1-2, RS/RL=53.2/20=2.66 and for the branch 1-3, RS/RL=160/100=1.6.

- In Figure 23.8.s-v, a divider is formed to operate in the band of f1=400 MHz and f2=1600 MHz. We note that return loss is about 17 dB, set by the branch 1-2. The insertion loss S31 has too high ripple.

- In Figure 23.8.u-z, a divider is formed to operate in the narrower band of f1=600 MHz and f2=1400 MHz. We note that return loss is about 30 dB, set by the branch 1-2. The insertion loss S31 has less ripple compared to the previous case because of narrower bandwidth.

In summary, we note that the overall return loss level in the targeted band is set by the worst branch, the branch which has the highest RS/RL ratio. In this example it is the branch 1-2. In this respect if the best possible return loss is desired in the targeted band, then the intermediate node impedance (40 ohms at node-1 of this example) must be set carefully to provide roughly balanced RS/RL ratios in each branch.

If least ripple is desired in the output powers in the desired bands then higher number of elements must be used in each branch.

Exercise: Form the divider for f1=200 MHz and f2=1800 MHz. Compare the responses with the other dividers.

Exercise: Repeat the designs by assuming a lower Z1, for example Z1=30 ohms and compare the bandwidths versus best return loss with the above designs.

Exercise: Repeat the designs without o-1 branch. That is, connect the branches 1-2 and 1-3 directly at the source end.

Figure 23.8. Wideband unequal two way divider with quarter wavelength transmission lines.

Figure 23.8. Wideband unequal two way divider with quarter wavelength transmission lines.

Figure 23.8. Wideband unequal two way divider with quarter wavelength transmission lines.

Figure 23.8. Wideband unequal two way divider with quarter wavelength transmission lines.

Figure 23.8. Wideband unequal two way divider with quarter wavelength transmission lines.

23. 3.  SYMMETRIC TWO WAY POWER DIVIDERS AND COMBINERS

23. 3. 1. INTRODUCTION

Two way equal power dividers have many practical applications. Therefore we will treat them in more details in this section. Such dividers have equal terminations:

R1=R2=R3=R ohms							(23.11)

Also the two branches are combined right at the source end (without using branch o-1 of the previous approach) as shown in Figure 23.9.

Figure 23.9. Two way equal power divider with equal terminations.

Since  the input power is split equally to the two output ports we have:

p2p3=1									(23.12)

For such cases the analysis and design formulations described in the previous section for unequal power division with arbitrary terminations simplifies as follows:

The total impedance of the two branches must be equal to the source impedance:

Z1=2R									(23.13)

Therefore

Z12=Z13=Z1(1+p3p2)=2R						(23.14)

The elements shown as K represent impedance matching circuits between the load R and the input impedance 2R of the branches. Their simplest form are ideal inverters with impedance

K12=K13=K=Z12R2R2=2R					(23.15)

Such a circuit can be used as equal power divider if Port-1 is used as the input with Ports 2 and 3 being outputs. By reversing the circuit, it can be used as a power combiner if Ports 2 and 3 are inputs while Port-1 is the output.

The element Zm between node-2 and node-3 is named as isolation element, usually a resistor. It is ineffective when the device is used as a power divider because the node-2 and node-3 are at the same potential. It becomes effective when the device is used as a power combiner or when there are reflections from port-2 and port-3. It isolates port-2 and port-3, as will be described below.

Such symmetric three ports can be analyzed using the even-odd mode decomposition, as follows:

a) Power divider Return Loss analysis

When input signal is applied at Port-1, because of symmetry, no current passes through the mutual impedance Zm. Therefore the circuit can be simplified for return loss analysis purposes as shown in Figure 23.10. If K is a single ideal inverter then for K=2R, impedances at inputs of the two branches are 2R and when combined in parallel the source will see the impedance R, hence matched to the source. Power is divided into the two branches equally.

Figure 23.10. Equal power divider circuit simplified for return loss analysis.

b) Power combiner/divider isolation and return loss analysis

When reflections exists from port-2 and/or port-3 of a power divider because of possible mismatches at these ports or when we use the three port as a power combiner where the signals to be combined are applied to Port-2 and Port-3, analysis of power flow requires even-odd mode decomposition. This is described in Figure 23.11. Here, the port excitation voltages V2 and V3  are split into even and odd mode excitations, Vev and Vod.

Figure 23.11. Even-odd mode decomposition of a two way power divider-combiner.

The even mode excitation produces identical voltages on both arms. Therefore a virtual OC plane exists on the symmetry plane splitting the circuit into two identical and decoupled parts simplifying analysis. It should be noted that the OC symmetry plane splits the termination resistance R at Port-1 also split into two 2R ohms.

In the odd mode excitation, arms have equal but oppositely signed potentials. Therefore symmetry plane becomes is a virtual ground (SC) plane, splitting the circuit into two decoupled (odd half) circuits as shown in the figure. In the half circuits the termination resistance does not appear because it is shorted out by the ground plane.

When the three port is driven from ports 2 and 3, analysis of the complete circuit can be carried out in terms of the simpler even and odd half circuits, as follows.

The excitations at port-2 and port-3 are denoted by the voltages V2 and V3. They are split into even and odd mode components Vev and Vod:

V2=Vev+Vod

V3=Vev−Vod								(23.16)

Solving these two equations for Vev and Vod we get

Vev=V2+V32

Vod=V2−V32								(23.17)

The even mode half circuits are formed by applying the same voltage Vev to both port-2 and port-3 as shown in the figure. The odd mode half circuits are formed by applying +Vod to port-2 and −Vodto port-3 as shown in the figure. The isolation element is split into two parts by the OC and SC symmetry planes under the respective even and odd mode excitations. If it is a lumped element then it can be removed from the even half circuit because it is OC in its middle.

A similar decomposition can also be carried out in terms of incident and reflected scattering signals as follows:

Denoting the incident signals at ports 2 and 3 as a2 and a3, even and odd mode scattered signals aeand aocan be formed through the equations:

a2=ae+ao

a3=ae−ao									(23.18)

Solving these two equations we get

ae=a2+a32

ao=a2−a32									(23.19)

The reflected signals b2 and b3 at the ports can also be decomposed into even and odd half components be and bo as follows:

b2=be+bo

b3=be−ao									(23.20)

The reflected signals are related to the incident signals through the following equations:

be=Zev−RZev+Rae

bo=Zod−RZod+Rao								(23.21)

where Zev and Zod are the input impedances of the even and odd half one-port circuits.

The complete scattering parameters S22=S33 and S23=S32of the two port circuit between port-2 and port-3 can be determined by setting a2=1,  a3=0 and using the above definitions as follows:

S22=b2a2=be+boae+ao=be+bo=Zev−RZev+Rae+Zod−RZod+Rao=ZevZod−R2(Zev+R)(Zod+R)    (23.22)

S23=b3a2=be−boae+ao=be−bo=Zev−RZev+Rae−Zod−RZod+Rao=R(Zev−Zod)(Zev+R)(Zod+R)	 (23.23)

It is seen that the condition

Zev=Zod									(23.24)

provides infinite makes isolation between port-2 and port-3 (S23=S32=0=0). On the other hand the condition

ZevZod=R2									(23.25)

provides infinite return loss (S22=S33=0). The two conditions can be stated as a single equation:

Zev=Zod=R								(23.26)

Conclusion:

For a proper operation of the three port as a power divider-combiner, in the frequency range of interest,

- The network K should transform the load resistances R into 2R at the inputs of the branches.

- For a good isolation between port-2 and port-3, the even and odd mode half circuit input impedances must be equal: Zev=Zod=R

Some examples satisfying these requirements will be described below.

23. 3. 2. TWO WAY DIVIDERS WITH QUARTER WAVELENGTH TRANSMISSION LINES

The simplest microwave circuit that works as a power divider employs quarter wavelength transmission line pieces as the impedance transforming network. This is shown in Figure 23.12.a. Characteristic impedance of lines are selected as 2R. Isolation (or coupling) element Zm is selected as 2R.

Figure 23.12. Two way divider with single section quarter wavelength line transformers.

With these conditions, when the device is used as a power divider at the quarter wavelength frequency of the TL transformers, the isolation element is ineffective and the load resistancesR at ports 2 and 3 are  transformed into 2R, giving matched input at node-1 when combined in parallel as shown in Figure 23.12.b. When the device is used as a combiner or when reflections exists from ports 2 and 3, the analysis need be carried out on even and odd half circuits as described in Figure 23.12.c.

- Under even mode excitation the isolation resistance disappears due to the lumped nature of that element and being open circuited at its midpoint. The impedance R at port-1 is split into two 2Rohms by the OC symmetry plane. So, the even mode input impedance is set by the input impedance of the quarter wavelength transformer of impedance Z=2Rwhich transforms the load impedance 2Ras follows:

Zev=Z22R=R.

- Under odd mode excitation only half of the isolation resistance 2R appears in the odd half circuit. Since port-1 is SC, the quarter wavelength line transforms it as OC to node-2. Thus, at node-2 the only element is the resistance R, half of the isolation resistance. So, odd mode input impedance is Zod=R. This is an indication that the power taken from port-2 and port-3 under odd mode excitation is spent in the isolation resistance.

- We note that the conditions Zev=Zod=R are all satisfied. Thus, device works as power divider around a limited band about the quarter wavelength frequency. Bandwidth limitation comes from the bandwidth of the quarter wavelength TLine transformer. For operation in a wider band, we may use multisection quarter wavelength transformers as described in the previous section or nonuniform line transformers can be used.

On Filpro, analysis of dividers with isolation elements can only be carried out by forming the even and odd mode half circuits as described in the following examples.

EXAMPLE 23. 3. A power divider with single quarter wavelength transmission line transformers

Design procedure involves formation of the even and odd half circuits described above as shown in Figure 23.13.

- Design / By Inserting Elements command is invoked. RS=50 and RL=50 ohms are selected.

- Insert command appears. Select TLine-Stubs-.. / TLine option. Type in the parameters of TLine:  Zo=70.7 ohms, Lng=74.498 mm. Here length is set such that the TLine will operate as quarter wave transformer at fo=1000 MHz. Thus, the TLine is inserted.

- With cursor under RL, the command Insert / Constant Elements: Rsh =100 Ohms is applied to insert a 100 ohms resistor to the left of RL. This resistor will simulate the load resistor R1=50 ohms at port-1 which is split into two parts by the even mode excitation. After splitting the portion remaining in the even half circuits become 100 ohms.

- Thus, the even half circuit is ready. It is saved as Even01.fil (Figure 23.13.b). We will disregard the load resistor RL=50 ohms because it will NOT be taken into account during the forthcoming even-odd mode analysis on Filpro.

- The odd half circuit is also formed in a similar way. We first insert the same quarter wavelength TLine. Then insert a shunt resistor R=0 ohms to simulate R1=50 ohms at port-1 short circuited by the SC symmetry plane of the odd mode excitation. Then another resistor of 50 ohms is inserted on the left of the TLine to simulate the odd half of the 100 ohms isolation resistor. The circuit is saved as Odd01.fil (Figure 23.13.c). Filpro will disregard the load resistor RL=50 ohms at right.

- The even and odd half circuits will be used by Filpro to display both return loss at port-2 (S22) and isolation between port-2 and port-3 (S32), as described in Figure 23.13.d. The option Return Loss actually displays S22 while the option Insertion Loss displays S32 and actually it should be interpreted as the isolation between port-2 and port-3.

Figure 23.13. Analysis of power combiner-divider using even and odd half circuits.

Figure 23.13. Analysis of power combiner-divider using even and odd half circuits.

- The command set for displaying return loss and isolation are as follows: Design / Symmetric Cross-Coupled Ccts / Analyze Cross-Coupled Filter/ Plot Response Only / Insertion Loss (or Return Loss) as shown in Figure 23.13.e.

- Figure 23.13.f shows return loss and isolation which are perfect at fo=1000 MHz while degrading rapidly as frequency goes away from fo. It is seen that return loss and isolation bandwidths differ considerably.

- Power flow from port-1 to port-2 and port-3 can be inspected by removing the isolation resistor and then forming diplexer-like circuits where one branch is inserted into the other as External 1-Port, as described in the previous examples. Since isolation resistor doesn’t effect power flow from port-1 to port-2 and 3, we can safely remove it for this analysis.

# EXAMPLE 23. 4. A Power divider with three quarter wavelength transmission line sections

Wider band power divider-combiner circuits can be formed by using wide band impedance transforming circuits. In this example we will use three quarter wavelength TLine transformer sections to transform R=50 ohms to R=100 ohms, as described in Figure 23.14.

Figure 23.14. A two way divider-combiner with three section TLine transformers.

The isolation resistors Rm1, Rm2 and Rm3 are spread into the circuit as shown in the figure. In this design there are two stages:

- Design of three section transformers converting RL=50 ohms of port-2 to R=100 ohms at node-1 to be combined with the other branch. source end. This stage can be designed using Monte Carlo-Best Case optimization routine.

- Determination of isolation resistors. These resistors will be determined by manual to get the best isolation over the widest bandwidth.

Stage-1 is described in Figure 23.15. Using Design / By Inserting Elements command we first set RS=100 ohms and RL=50 ohms. Then using Insert command three TLine pieces with trial impedances of Z1=90 ohms, Z2=70 ohms and Z3=60 ohms are inserted. Then Monte Carlo-Best Case routine is applied with Reference line placed between frequencies f1=600 MHz and f2=1400 MHz until the best possible return loss level is obtained. After optimization the circuit is saved as M31.fil (Figure 23.15.c). Then the same circuit is inserted at source end of M31.fil to form the divider as shown in Figure 23.15.d where the responses are also shown. Insertion loss response shows perfect 3 dB power division while return loss at port-1 is about 32 dB over 600-1400 MHz band.

Stage-2 is described in Figure 23.16.

- The circuit M31.fil is loaded. The command Reverse Circuit is applied to bring port-2 to the source side where the even and odd mode excitations will be applied (Figure 23.16.a-b).

- The even mode half circuit is formed by inserting a resistor of R=100 ohms on the left of RL using the command Insert / Constant Elements: Rsh=100. This resistor simulates port-1 resistance of R1=50 ohms split into two equal and parallel parts by the even mode excitation (Figure 23.16.b-c). The load end resistor RL will be disregarded during analysis. The circuit is saved as Even31.fil. The resistor

- Then the odd half circuit is formed by inserting isolation resistors of trial value 50 ohms into Even31.fil and also changing R=100 ohms resistor to R=0 ohms because now it will be simulating R1 shorted out by the SC symmetry plane (Figure 23.16.d-e). The circuit is saved as  Odd31.fil.

- So, the even and odd half circuits are ready for analysis and tuning. The command set New / Design / Symmetric Cross Coupled Ccts / Analyze Cross-coupled filter / Even31-Odd31 / Tune Only Odd Circuit elements will be applied.

- This command displays the odd half circuit Odd31.fil for pointing the element to be tuned. We use Ctrl+Right / Left keys to move the cursor and point one of the isolation resistors to be tuned and press Enter key. Then Filpro asks Tuning Increment. We type in a proper value.

- Then select the option Insertion Loss which is actually the isolation function (S23). By using Up-Down arrows to tune the pointed resistor while watching the isolation (insertion loss) plot. After tuning of that element is finished we press Esc key. Then Filpro asks if another element to be tuned or not. We press YES option and select another resistor for tuning. After a few rounds of tuning we get some insight in which directions the resistors to be tuned.

- One result is shown in Figure 23.16.g where both Isolation (S23) and Return Loss (S22 and S33) are shown.

- The optimized circuit is saved as Odd32.fil (Figure 23.16.h).

- Figure 23.16.i shows a possible realization schematics.

Figure 23.15. Design of three section TLine impedance transformers.

Figure 23.16. Determination of isolation resistors of the divider.

Figure 23.16. Determination of isolation resistors of the three section divider/combiner.

EXAMPLE 23. 5. An ultra wideband power divider with seven quarter wavelength transmission line sections

Using the same approach as in the previous example, a 2-18 GHz power divider-combiner is designed. The design steps, resulting circuits and responses are shown in Figure 23.17. (Exercise  ).

Figure 23.17. Design of a wideband seven section divider-combiner circuit.

Figure 23.17. Design of a wideband seven section divider-combiner circuit.

Figure 23.17. Design of a wideband seven section divider-combiner circuit.

23. 5. TWO WAY DIVIDERS  WITH COUPLED QUARTER WAVELENGTH LINES

If the quarter wavelength lines of a transmission line type power divider are close to each other, then coupling will take place between the lines. Thus, the circuit will contain a parallel coupled line element, as shown in a three section divider shown in Figure 23.18 where the divider is now formed by cascading symmetric coupled line pairs. Symmetric coupled lines are characterized through even and odd mode characteristic impedances Zoe and Zoo with the condition Zoo<Zoe always.

- When the input is applied to port-1 the device will work as power divider (Figure 23.18.a). Since this input creates identical voltage waves on both pieces of symmetric coupled lines, the coupled line pairs will work under even mode excitations, hence present even mode characteristic impedances Zoe1, Zoe2 and Zoe3. When these impedances are calculated so as to provide match between R ohms and 2R ohms within the desired passband the device will work as a perfect power divider independently of the odd mode impedances and isolation resistors.

- When the excitations are applied to ports 2 and 3, then the effects of odd mode impedances and isolation resistors come into picture. The two excitations are split into even and odd mode giving the even and odd mode half circuits shown in Figure 23.18.b. Perfect return loss at ports 2 and 3 together with perfect isolation between port-2 and port-3 will be obtained only if the conditions

Zev=Zod=R

are satisfied. Satisfaction of the condition

Zod=R

is not possible without the help of the isolation resistors. So, the odd mode impedances can be determined together with the isolation resistors through trial-error and optimization.

Figure 23.7. Wilkinson divider with parallel coupled lines.

EXAMPLE 23. 6. Ultra wideband power divider coupled transmission line sections

Just as demonstration of the effects of the odd mode impedances and isolation resistors, let’s convert the isolated transmission lines of the power combiner of the previous example into parallel coupled lines. This is a simple procedure, as described in Figure 23.19:

- Load Even42.fil and save it as Even45.fil (Figure 23.19.a). Impedances of Tlines of this circuit will be treated as even mode impedances of coupled line pairs (Zoe).

- Load Odd42.fil. Impedances of the transmission lines of this circuit will be treated as  odd mode impedances Zoo. Since the TL impedances of this circuit are the same as those of the even half circuit (Zoo=Zoe) the pairs (Zoe, Zoo) represent uncoupled line pairs. In order to convert them into coupled line pairs the impedances of the Tlines of the odd half circuit will be decreased to have Zoo<Zoe. Just as example, let’s make all odd mode TLine impedances Zoo= 45 ohms. Coupling gets tighter as the difference Zoe-Zoo gets higher. The resulting circuit is saved as  Odd45.fil (Figure 23.19.b-c).

- In order to see the effects of these brute force changes on TLine impedances the return loss (S22=S33) of the output ports and insertion loss (isolation) between port-2 and port-3 are  displayed for the even-odd mode circuits Even45-Odd45 (Figure 23.19.d). The responses of the original circuits Even42-Odd42 are also shown in the same figure for comparison. We note that these brute force changes led to degradations in both return loss and isolation. Hence a new optimization is needed which have to be applied on both odd mode impedances and isolation resistors.

Figure 23.19. Effects of couplings between transmission lines of the two branches.

23. 6. POWER DIVIDERS WITH  IMPEDANCE TRANSFORMING FILTERS

Impedance transforming lumped or distribute element, lowpass, highpass or bandpass filters can also be used as impedance transforming circuits (Figure 23.20). The conditions for proper operation of the device as a power divider or combiner are the same as in the previous cases:

Zev=Zod=R

where R is the termination resistance. The condition Zev=R over a band of frequency is easy to satisfy. The filter will transform the even mode load, 2R into R. That is a 2:1 transformation is involved. There are several ready design techniques for impedance transforming filters, narrow or wide band, lowpass, highpass or bandpass form with Chebyshev or Maximally flat passband responses.

The following approach should be used in designing Chebyshev impedance matching filters:

In a lossless filter with given degree, the three parameters Passband Ripple-Pass Bandwidth-RS/RL Ratio are linked to each other. That is setting any two sets also the third one. Therefore some iterative trials are needed to achieve the targeted circuit with desired response. In the synthesis stage, after selecting the degree of the filter

- set a trial (acceptable) passband ripple,

- set trial passband edge frequencies and

- synthesize the filter.

In the resulting circuit, the transformer at the load end must be unity. If not, then design trial should be repeated by changing either passband ripple or band edge frequencies or both until unity transformer is obtained in which case the design is completed. If still no satisfactory result is achieved than we increase the degree of the filter.

The locations and values of isolation elements have to be found by tuning or optimization. The isolation elements may also be distributed into the filter as shown in the figure.

Let’s see some examples by starting with a low degree matching filter to get insight into the problems.

Figure 23.20. Two way divider/combiner with impedance transforming filters.

Example 23. 7: Power dividers with degree two filters and resistive and reactive isolation elements

Let’s consider a simple two way equal power divider with degree two impedance matching filters operating with 50 ohms terminations. The divider is intended for use in the frequency range of f1=900 MHz and f2=1100 MHz. The filters will transform 50 ohms terminations at port-2 and port-3 into 100 ohms at  port-1 which upon parallel connection will show a matched input at that port (Figure 23.21). The isolation elements shown as Zm  will be connected between Port-2 and Port-3.

Figure 23.21. Power divider with two element impedance matching filters.

Chebyshev filters will be used. Design steps of the filter are shown in Figure 23.22:

- New / Design / By Synthesis commands are invoked.

- Filter Type: Lumped / BP / Equiripple / Doubly Terminated / Type A options are selected. In selecting the options Type A or Type B, care should be observed. If RS<RL then we must select Type A and for RS>RL we must select Type B.  In this circuit we will set RS=50 ohms and RL=100 ohms. So Type A is selected which will give a circuit having series element on source side.

- Since the degree, passband edges and RS/RL ratio are fixed, the only parameter that we are free to adjust is the PBRipple. We type in the specifications: PBRipple= 0.0215 dB, f1=900 MHz, f2=1100 MHz, RS=50, RL=100. We make repeated designs by changing PBRipple until the transformer of the resulting circuit comes out to be unity.  It came out that PBRipple=0.0215 dB yields unity transformer.

- Transmission Zeros are typed in as Inf-Inf.

- Extract Transmission Zeros: Standard Auto Extract command is applied giving circuit shown in Figure 23.22.a.  The circuit is saved as M50.fil after deleting the unity transformer at load end. This filter will be used in the two arms of divider. The high impedance ends will be connected in parallel to form Port-1.

- Then even mode half circuit of the divider is formed by inserting a   shunt resistance of R=100 ohms which simulates the termination resistance of R1=50 ohms after it is split into two parallel halves by the OC symmetry plane (Figure 23.22.b). The load resistance should be disregarded because it is not taken into account during even-odd mode analysis.

- The input impedance of this circuit is termed as Zev. Inside the passband of the filter we have Zev=R=50ohms which is one of the requirements of the divider.

- The odd half circuit is formed by changing R=100 ohms resistance to R=0 ohms to simulate the SC symmetry plane. Since this SC is in parallel with the shunt capacitor of the filter, that capacitor is also eliminated as shown in Figure 23.22.c. So, we have a purely reactive (inductive) odd mode input impedance Zod while we need purely resistive input impedance of Zod=R. This requirement can be satisfied only by inserting an isolation resistance and a capacitor to cancel the inductive reactance. These stages are also shown in Figure 23.22.c. We first insert a shunt resistor of R=50 ohms into the odd half circuit to simulate halved isolation resistor of 100 ohms connected between node-2 and node-3. Then we also connect a shunt capacitor to simulate the mutual (isolation) capacitor between node-2 and node-3 (in parallel with the isolation resistor). The proper value of the mutual capacitor Cm will be found by tuning. So a trial value of 1 pF is typed in. The circuit is saved as Odd50.fil.

Figure 23.22. A divider with two element matching filters.

Figure 23.22. A divider with two element matching filters.

- In order to determine mutual capacitor, the command set shown in Figure 23.22.d is applied: New/ Design / Cross-Coupled Cct/ Analyze Cross-Coupled Filter: Even50, Odd50 / Tune Only Odd Circuit Elements.

- After this command, the odd half circuit appears on the screen. By using Left-Right arrows, we point the capacitor and press Enter key.  Tuning increment is typed in and using Up-Down arrows we tune the capacitor value while watching Insertion loss plot which represents the isolation between port-2 and port-3 or Return Loss plot which represent return loss of port-2 or port-2 (S22, S33) of the divider. We get the odd half circuit and responses shown in Figure 23.22.f. The effects of mutual capacitor Cm are clearly seen.

- Figure 23.22.g shows the complete picture of the divider circuit where the mutual coupling elements Rm and Cm are shown.

Series RC Isolation elements:

It is also possible to use different topologies for isolation elements. For example in Figure 23.23, a series RC type isolation element is used in the above design giving the slightly wider bandwidths for S23 and S22.

Figure 23.23. Divider with series RC type isolation element.

RLC isolation element with mutual inductance:

Possible inductive mutual coupling  between series inductors of the two arms may also be used to improve isolation. One possible form is shown in Figure 23.24.a. Here a parallel RC isolation element is placed which is supported also by the mutual inductance. The expanded version of the circuit is shown in Figure 23.25.b. The mutual inductance, contrary to the other isolation elements, effects both even and odd mode half circuits. Depending on the positions of dots on the inductors, the effective series inductance becomes L+M or L-M. Thus, to preserve the impedance transforming property of the filter, we have to keep the net inductance equal to the original inductance of the filter (7.9511 nH in this example). Thus, the actual self inductance L of the coupled coil will be higher or lower than the inductance of original filter. This property may provide flexibility in element values. In this circuit, tuning process will again be applied on only the odd half circuit, because we assume that the effective inductance, L+M or L-M is constant (equal to 7.9511 nH in this example). Thus the problem becomes identical to the case of parallel RC isolation element studied above.

Figure 23.24. Isolation elements including mutual inductance between inductors.

EXAMPLE 23. 8: High degree lumped element quasi-lowpass two way dividers

Let’s design a two way divider operating in the band  fp1 = 750 MHz and fp2 = 1250 MHz which will also match the RS=50 ohms source to RL= 6.25 ohms loads.

As a trial, we will design a degree six quasi-lowpass impedance matching filter which matches 100 ohms to 6.25 ohms and then combine two of such filters to form a divider. Then isolation elements will be set by tuning trials. Maximally flat or Chebyshev bandpass filters with no transmission zeros at f=0 can be used for this purpose.

Design procedure for maximally flat impedance transforming filters:

Figure 23.25 shows design steps of a maximally flat impedance transforming filter. As described in Chapter-3, the following points must be observed in this design:

- Passband Edge Loss of the maximally flat bandpass filter must be equal to DC loss of the filter. In order to find out this value, Design / By Inserting Elements command is applied and the terminations RS=6.25 ohms, RL=100 ohms  are typed in. We get the RS-RL circuit shown in Figure 23.25.a. Plot / Insertion Loss command displays the insertion loss between RS and RL which gives the DC loss as 6.54718 dB. We note down this value to be used as the PB Edge Loss of the maximally flat filter.

Then we start to design the quasi LP filter as a BP filter as follows.

- New / Design / By Synthesis commands are applied.

- Lumped / BP / Maximally Flat / Doubly Terminated / Type A options are selected. (Type-A should be selected if RL is larger than RS while Type-B is needed if RL is less than RS).

- The lower passband edge frequency is selected as an arbitrary but small value. In this example we set it as fp1=0.1 MHz. fp2 must be set greater than the actual desired value (1250 MHz). Let’s see what happens if we select fp2 =1250 MHz.

- Six transmission zeros are placed at infinity.

- Standard Auto Extract mode is used for element extraction giving the circuit of Figure 23.25.b.

- The resulting ideal transformer at the load end MUST BE DELETED regardless of it’s turns ratio, giving the circuit shown in Figure 23.25.c. {Why must we delete this transformer? This point is still not proven but it works!  ). The circuit is saved as MF01.fil.

- Insertion loss and return loss responses are shown in Figure 23.21.d. It is seen that matching bandwidth definition depends on the desired return loss level. For example if 10 dB return loss is good enough the matching band is 600-1100 MHz while for a 30 dB return loss  the band shrinks to 750-1000 MHz. If delay flatness is also important the band narrows further as seen from the Delay plot of Figure 23.25.d. Therefore if group delay is also important, fp2 must be moved away as much as possible.

Figure 23.25. Synthesis of a maximally flat impedance transforming bandpass (quasi-lowpass) filter.

Procedure for the design of a Chebyshev impedance transforming filter:

Since the three parameters Passband Ripple-Pass Bandwidth-RS/RL Ratio are linked to each other, setting any two sets also the third one. Therefore some iterative trials are needed to achieve the targeted circuit with desired response. In the synthesis stage,

- set a trial (acceptable) passband ripple,

- set trial passband edge frequencies and

- synthesize the filter.

In the resulting circuit, the transformer at the load end must be unity. If not, then design trial should be repeated by changing either passband ripple or band edge frequencies or both until unity transformer is obtained in which case the design is completed. If still no satisfactory result is achieved than we increase the degree of the filter.

Design steps of a trial are shown in Figure 23.26 for a quasi-lowpass impedance matching filter with band edges fp1 =1250 MHz and fp2 =1390 MHz and RS=6.25 ohms and RL=100 ohms. In this design trials are made by changing passband ripple. PBRipple= 0.0435 dB (20 dB return loss) gave the solution with unity transformer (Figure 23.26.c). The circuit is saved as CH01.fil. Group delay is also shown in the same figure. If this parameter is also important then the bandwidth gets narrower depending on the tolerance in delay variation in the band.

Figure 23.26. Design of a Chebyshev impedance transforming bandpass (quasi-lowpass) filter.

Formation of the divider with Chebyshev filters:

The two arms of the divider are formed by the two identical impedance transforming filter. Let’s use the filter CH01.fil for this purpose. The two filters are combined at their high impedance (100 ohms) ends and then terminated in 50 ohms at port-1 as shown in Figure 23.27. The other ends of filters are terminated in 6.25 ohms. The isolation elements can be connected between the symmetric nodes of two filters. Resistive elements are used. Their actual values are unknown at this stage and will be found through optimization by tuning. For the time being 500 ohms resistors are placed.

Figure 23.27. Formation of the divider.

Formation of even and odd half circuits:

- Analysis of isolation between Port-2 and Port-3 requires formation of the even and odd half circuits.

- Even half circuit is formed by loading the impedance transforming filter CH01.fil (Figure 23.28.a). Here, if the common port (Port-1) is at left side, than the filter circuit should be reversed using Reverse Circuit command. The filters designed above are already in the proper form (Port-1 on left side).

- Then, at the load end, a shunt resistor of value 100 ohms is inserted using Insert command (Figure 23.28.b). This resistor is parallel half of the 50 ohms termination of Port-1. It is split equally under even mode excitation. The load resistor seen in the schematics of this circuit should be disregarded, because in the Even-Odd mode analysis used in Filpro, it is automatically replaced by open circuit.

- The resulting circuit is saved with the filename EvenCH01.fil.

Figure 23.28. Formation of the even half circuit for analysis of isolation between Port-2 and Port-3.

- Formation of the odd half circuit is described in Figure 23.29. Under odd mode excitation, the symmetry plane is short circuited. Therefore the termination resistance at Port-1 is shorted out. On Filpro this fact is reflected into the odd half circuit by inserting a zero shunt resistor adjacent to RL of the schematics. The zero resistance also shorts out the capacitor on its left. Here, RL should be disregarded. It is not taken into consideration in calculations.

- Isolation (cross-coupling) elements are needed between the two arms of divider. These elements are halved through their mid points by the short circuit plane passing through the symmetry plane. Therefore halves of the isolation impedances will appear in the odd half circuit. These elements do not appear in the even half circuit. As a trial, we have used only resistive cross-coupling elements. Since their values are unknown, some high resistances are inserted at nodes on the left of each LC section, as shown in Figure 23.29.

- The resulting circuit is saved as OddCH01.fil.

Figure 23.29. Formation of odd half circuit.

- We are now ready to analyze the isolation (insertion loss) between Port-2 and Port-3. The two-port circuit between Port-2 and Port-3 can be treated as a cross-coupled filter. Thus, the command set for analysis of Cross-Coupled filters can also be used for analysis of isolation in such dividers.

- The command set is shown in Figure 23.30. Here, the option Tune only Odd Circuit Elements is selected because initially we will set the isolation resistances by tuning. In this option the odd half circuit is displayed and the element to be tuned is pointed by the cursor and Enter key is pressed.

- After selecting the elements to be tuned, Filpro asks tuning increment and we can display Insertion Loss which is actually the Isolation between port-2 and Port-3 or we can display Return Loss which is the return loss of Port-2 or Port-3.

- After finishing tuning an element, Esc key is pressed. Then Filpro asks if another element to be tuned or not.  Continuing this way, after a few rounds we get the circuit shown in Figure 23.30.b. Insertion Loss plot shown in Figure 23.30.c is actually the isolation between Port-2 and Port-3. Return Loss represents S22 or S33.

- Since isolation resistors are ineffective when the circuit is excited from Port-1, tuning of isolation resistors does not effect the return loss at Port-1 (S11). Therefore we can tune the isolation elements without worrying about return loss at Port-1.

- It is seen that the bandwidths of S11, S22 and S23 are all different. The bandwidths of S22 and S23 are narrower than that of S11.

Figure 23.30. Tuning isolation resistors.

Exercise: Since in the odd mode excitation the shunt capacitor adjacent to the SC, the odd mode input impedance may have nonzero reactive (inductive) part, so that Zod= R+jX within the pasband. The uncompensated inductive reactance jX may be cancelled by using  mutual capacitor(s).  Place a capacitor between node-2 and node-2 and tune it to see if any improvement will be obtained in S22 and S23.

23. 7.  N-WAY EQUAL POWER DIVIDERS

Development of N-way equal power dividers can be treated in the same way as two way equal power dividers. However isolation elements should be found by optimization.

As an example let’s inspect the design of a 3-way divider.

In Figure 23.31, a possible schematic for a 3-way divider is shown. The port impedances are given as

Ro=50 ohms, R1=20 ohms, R2=80 ohms, R3 = 100 ohms

Let’s define normalized output powers as

p1=P1Po, p2=P2Po  and  p3=P3Po

where Po is the input power. So we have

P1+P2+P3=Po								(23.27)

or

p1+p2+p3=1								(23.28)

The three branches are combined in parallel at node-0. The input impedances of the three branches are denoted as Zo1, Zo2 and Zo3. Hence the input impedances of the three branches will be combined in parallel to give a total impedance Zo:

1Zo1+1Zo2+1Zo3=1Zo							(23.29)

The inverter Koo will match the node impedance Zo to the source impedance Ro. Here Zo is a free parameter which can be used to design alternative equivalent divider circuits in the search for easy to realize inverter impedances. Now we have three unknowns: Zo1, Zo2 and Zo3. We need three equations to solve them. The input power will be split in accordance with the impedance ratios

p2p1=Zo1Zo2									(23.30)

p3p1=Zo1Zo3									(23.31)

p2p3=Zo3Zo1									(23.32)

The equations 23.4 and 23.5 together with Eq. 23.3 are sufficient to solve for the three unknowns. Eq. 23.6 is redundant and can be obtained from Eq. 23.3. These three equations will yield the following solutions for the impedances Zo1, Zo2 and Zo3:

Zo1=Zo(1+p2p1+p3p1)							(23.33)

Zo2=Zo(1+p1p2+p3p2)							(23.34)

Zo3=Zo(1+p1p3+p2p3)							(23.35)

So, the matching circuits (inverters) Ko1, Ko2 and Ko3. will match the load resistors R1, R2 and R3. to Zo1, Zo2 and Zo3. respectively. If the matching circuits are single inverters then the inverter constants can be found using the relations

Ko1=Zo1R1									(23.36)

Ko2=Zo2R2									(23.37)

Ko3=Zo3R3									(23.38)

The input inverter Koo  will match Zoto Roo. Hence,

Koo=ZooRo									(23.39)

In the next stage the inverters will be replaced by their proper practical equivalents giving the actual frequency dependent response. Since single inverters usually give narrow operational bandwidth, the matching circuits may be formed by cascading more than one inverter in each branch to get sufficient bandwidth.

Let’s see an example.

Figure 23.31. Three way divider.

# EXAMPLE-23. 3. Lumped element three way equal power divider circuits

Three way equal power divider circuits will be explored to inspect the relations between the parameters bandwidth-Return Loss-degree of matching circuit. We will design lumped element 3 way dividers operating around 1000 MHz.

In Figure 23.32 three possible alternative circuits are shown where three inverter stages are used for matching. Naturally wider bandwidths can be obtained by using higher degree matching circuits. If inverters are used as matching elements then the number of inverters set the bandwidth.

- The divider in Figure 23.32.a (Solution-1) has three inverters in each branch. Each branch transforms the load resistance of R ohms to 3R so that when paralleled gives R ohms to match the source resistance.

- The divider of Figure 23.32.b (Solution-2) has two inverters K2 and K3 in the three branches while the third inverter K1 is placed between source and common junction of the branches. The total impedance at the common junction is denoted by R´. So, each branch should transform the load impedance of R ohms to 3R´ohms. Then the inverter K1 transforms R´ to R to match it to the source. The intermediate impedance R´ can be selected as a free trial parameter for obtaining better behaved element values.

- The divider of Figure 23.32.c (Solution-3) has one inverter (K3) in the three branches while the second and third inverters K2 and K1 are placed between source and common junction of the branches. The total impedance at the common junction is denoted by R´´. So, each branch should transform the load impedance of R ohms to 3R´´ohms. Then the inverter K2 transforms R´´ to R´ and K1 transforms R´ to R to match it to the source. The intermediate impedances R´ and R´´ can be selected as free trial parameter for obtaining better behaved element values.

Figure 23.32. 3-way equal power divider alternatives.

- Each solution may have some advantages in some respects like element values, number of elements, wider bandwidth, etc.

- If isolation is needed among the output ports 2-3-4, then isolation resistors can be placed between the nodes of the branches, for example between the nodes numbered as 2-3-4 or 2´-3´-4´ or      between 2´´-3´´-4´´.

- Solution-1 can be converted into a special case of Solution-2 by connecting the nodes 2´´-3´´-4´´ to each other so that the three inverters K1 will be paralleled to yield a single inverter K1´.

- Similarly Solution-1 can be converted into a special case of Solution-3 by connecting the nodes 2´´-3´´-4´´ and 2´-3´-4´ to each other so that the three inverters K1 and K2 will be paralleled to yield single inverters K1´ and K2´.

Solution-1:

Let’s see the design of Solution-1 on Filpro. The other solutions can also be carried out in the same way.

Design steps are shown in Figure 23.33. We will form one branch of Solution-1. It will be a matching circuit between RL=50 ohms load and RS=150 ohms source.

- New / Design / By Inserting Elements command is invoked.

- RS=150 ohms and RL=50 ohms are typed in.

- Insert command appears. We insert three inverters with trial impedances set as 120 ohms, 90 ohms and 70 ohms, increasing towards source end (Figure 23.33.a).

- Return Loss plot shows that the trial impedances of the inverters are not proper. The best matching can be obtained by optimizing the inverter impedances. For this purpose we first set a reference line between 800 MHz and 1200 MHz, at about 15 dB return loss level just for optimization stage. The actual return loss level and bandwidth will be set after converting the inverters into lumped element equivalents.

- Then a Monte Carlo List is prepared by including Inverter impedances only and Best Case command is used iteratively until better than 100 dB return loss is obtained (Figure 23.33.b).

- We get the circuit shown in Figure 23.33.c. It is saved as PD31A.

- Then with cursor under the inverters the command Inverters / Replace Inverter by / Constant Reactance / Pi-Section is applied to convert inverters into Pi-sections of constant reactances with negative shunt legs(Figure 23.33.d-e).

- Simplify Circuit command combines the adjacent shunt reactances (Figure 23.33.f).

- Then with cursor under the shunt reactances the command Constant Reactances / Replace Constant Reactance by / Capacitor: fo=1000 MHz is applied to convert the negative constant reactances into lumped capacitors (Figure 23.33.g).

- Then with cursor under the series reactances the command Constant Reactances / Replace Constant Reactance by / Inductor: fo=1000 MHz is applied to convert the constant reactances into lumped inductors (Figure 23.33.h). The circuit is saved as PD31F. Its response shows a perfect match only at fo=1000 MHz.

- Matching bandwidth can be widened by optimizing the element values using Monte Carlo-Best Case routine.

- First a test is made to get best match in the band from f1=550 MHz to f2=1250 MHz. We got about 20 dB return loss within this band (Figure 23.33.i-k).

- Then optimization is repeated for the bandwidth f1=650 MHz and f2=1200 MHz. We got about 26 dB return loss level (Figure 23.33.m). This circuit is saved as PD31H. Clearly higher return loss would be obtained if the bandwidth is narrowed.

- Finally, a 3-way divider is formed by inserting PD31H into PD31H  twice as External 1-Port element using the command External 1-Port / PD31H / Shunt 1-Port (Figure 23.33.n-p). The source impedance is also changed to 50 ohms. As seen from the plots in Figure 23.33.m, the divider works well within the band f1=650 MHz and f2=1200 MHz. Power is divided into three branches equally at a level of –4.77 dB.

- Figure 23.33.q shows a possible placement for isolation resistors R12-R13-R23. They can also be placed between all nodes where the shunt capacitors are connected. Unfortunately this circuit can’t be simulated on Filpro. Isolation resistors can only be determined by optimization a more general circuit simulator.

Figure 23.33. Design of 3-way equal power dividers.

Figure 23.33. Design of 3-way equal power dividers.

Figure 23.33. Design of 3-way equal power dividers.

Figure 23.33. Design of 3-way equal power dividers.

Solution-2:

Solution-2 can be formed by first selecting an intermediate impedance level R´ and forming a two inverter matching section (K2, K3) to match the load R to 3R´ using the same approach as in Solution-1. Then R´is matched to the source R through the inverter K1´.

If Solution-1 is available and if also the resulting element values are acceptable then special cases of Solution-2 and Solution-3 can be formed from Solution-1 as described in Figure 23.34.

In Figure 23.34.a, the nodes 2´´-3´´-4´´ are at the same potential. Thus, they can be connected to each other making the three inverters of impedance K1 parallel, giving K1´= K1/3. Thus, Solution-2 is formed.

Similarly, the nodes 2´-3´-4´ are at the same potential. If they are also shorted to each other then we get K2´= K2/3, forming Solution-3 (exercise).

In Figure 23.34.b, this approach is applied on the lumped element version of  Solution-1. In this divider circuit we can short the nodes 2´´-3´´-4´´ which have the same potential to form Solution-2. Solution-3 can be formed by shorting the nodes 2´-3´-4´ (exercise). In all these solutions the response will be the same. Therefore one may select the solution with minimum number of element or the solution with preferable element values. However if isolations between the output ports is also important, one should check the isolations in all of these solutions after placing the isolation resistors (not available on Filpro!).

For wider bandwidth one may increase the number of inverters and repeat the design.

Figure 23.34. Formation of Solution-2 using Solution-1.

## EXAMPLE-23. 4. Distributed element three way equal power divider circuits

Let’s design the same three way equal power divider of the previous example using quarter wavelength transmission line transformers as impedance matching elements. That is, the inverters will be converted into quarter wavelength line at the center frequency of the relevant passband. Let’s take the center frequency as fo=450 MHz and see how much bandwidth we will get with structures similar to the previous example.

We can form different solutions.

Solution-1:

- In Figure 23.35.a the Solution-1 of the previous example, PD31A is loaded.

- With cursor under the inverters the command Inverters / Replace Inverter by / Quarter Wave TLine: fo=450 MHz is applied to convert the inverters into TL pieces which are quarter wavelength at fo=450 MHz (Figure 23.35.b).

- Return loss needs improvement. It is done by using Monte Carlo-Best Case optimization routine. For this purpose a reference line is placed arbitrarily covering the range f1=350 MHz and f2=550 MHz.

- After a slight optimization 20 dB return loss level is reached (Figure 23.35.c). Bandwidth is much wider than 350-550 MHz range. The resulting circuit is saved as PD32A. We note that during optimization the leftmost  TL impedance is increased from 115 ohms to 122 ohms.

- We continue optimization to get better return loss. We stopped at about 28 dB (Figure 23.35.d). The bandwidth is slightly narrowed but it is still much wider than the shaded region. The leftmost TL is increased to 125 ohms which may make realization problematic.

- Figure 23.35.e-f shows the power divider circuit formed by inserting PD32B into PD32B  twice as External 1-Port element using the command External 1-Port / PD32B / Shunt 1-Port. The source impedance is also changed to 50 ohms. The response is identical to the matching circuit PD32B. Power is divided into three branches equally at a level of –4.77 dB.

- Figure 23.35.g shows the exploded view of the divider. Isolation resistors can be placed  between nodes 2´´-3´´-4´´  and also between nodes 2´-3´-4´.

Solution-2:

Impedance levels of the quarter wavelength transformers can be controlled by using Solution-2 or Solution-3. As an example, let’s form Solution-2.

We can get a lower impedance level by selecting an intermediate impedance R´=35 ohms. The inverters K2 and K3 will match the loads RL=50 ohms to R´=35 ohms after interconnecting the three branches. So, K2 and K3 will raise impedance RL=50 ohms to 3R´=105 ohms. Then the leftmost inverter K1 will match R´=35 ohms to RS=50 ohms.

Design steps are given in Figure 23.36 (Exercise?).

Figure 23.35. Three way divider realized using quarter wavelength transformers (Solution-1).

Figure 23.35. Three way divider realized using quarter wavelength transformers (Solution-1).

Figure 23.36. Three way divider realized using quarter wavelength transformers (Solution-2).