CHAPTER 7

BASIC CONCEPTS ON DISTRIBUTED ELEMENT FILTERS

7. 1. INTRODUCTION

Distributed element concepts are needed whenever physical dimensions of electrical components are comparable to wavelength in the frequency range of operation. In the usual communication systems these concepts come into use above roughly 100-1000 MHz depending on the technology used (dictating sizes of components). Conversely, lumped element concepts may still be used even at millimeter wave frequencies (above 20-30 GHz) by using components with sufficiently small sizes. In some applications it may be advantageous to use both concepts in the same circuit (mixed or hybrid lumped-distributed element filters).

A class of distributed element filters termed as “commensurate line length filters” has well established synthesis techniques as an augmentation of lumped element filter synthesis  theory. This class of filters will be studied in this and the succeeding chapters. Hybrid (mixed lumped and distributed element) filters can be obtained by simple approximation techniques programmed as items of circuit transformation sections of Filpro.

The usual lossless distributed element filters are synthesized by proper interconnections of three basic elements shown in Figure 7. 1. a , b and c:

i) Short circuited stubs (termed as SS in Filpro).

ii) Open circuited stubs (termed as OS in Filpro).

iii) Transmission line pieces (termed as TLine or TL in Filpro)

Figure 7. 1. a) A transmission line (TL) , b) a short circuit  stub (SS) in series and shunt arm , c) an open circuit stub (OS) in series and shunt arm, and  d) a parallel coupled line (PCL).

In realization stages some stubs may be difficult or impossible to construct with the technology to be used. Many of such difficulties can be solved by replacing some stub-transmission line combinations with their “parallel coupled line (PCL)” equivalents. A parallel coupled line section is a four port network (Figure 7. 1.d). It can be used as one, two or three port network by terminating the unused ports in open circuit, short circuit, OC stub or SC stub to produce a large variety of components that can be used in realizing filters.

In most applications all these components are operated in TEM mode (Transverse Electro Magnetic fields). Under TEM mode of operation, stubs and transmission lines are characterized simply by two parameters:

i) Characteristic impedance ,

Zc=(L /C) 1/ 2                                                  				( 7.1)

ii) Electrical length ,

=wl(LC) 1/ 2 						( 7.2)

where l  is the physical length of line or stub, L and C are distributed inductance and capacitance per unit length , w = 2f is the frequency of operation. For lines operating in TEM mode

LC= =(0 r  )(0 r  )=r r / c2 						( 7.3)

(oo= 1/c2  , c = velocity of light),

where 0 and 0  are permeability and permittivity of free space, r and r are effective relative permeability and permittivity (dielectric constant) of medium of propagation respectively and  c . In the usual transmission lines having nonmagnetic dielectric medium of wave propagation, r=1. The expression for characteristic impedance can also be written as

Zc =/(cC)								( 7.4 )

where  (r)1/ 2

Hence knowledge of capacitance per unit length and relative dielectric constant are sufficient to determine characteristic impedance.

Electrical length  can be written in various forms in terms of different parameters :

 = l =(2l)/ =w =(wl)/c=(/2)(f /fq  ) 				( 7.5 )

where

=w(LC)1/2= w()1/2 = (w ) / c :Phase constant				( 7.6 )

=2/=c/(f )    	      :  	Guided wavelength		                             ( 7.7 )

 = l /c	      : 	One way time delay along length l 		( 7.8 )

fq= c / (4l) : 	Quarter wavelength frequency				( 7.9 )

(Frequency at which physical line length equals quarter wavelength where electrical length is =  / 2).

Hence a transmission line or stub may also be characterized in terms of the triplets

(Zc , r , l )    or   (Zc , r, fq )

together with frequency of operation.

Parallel coupled lines (PCL) operating in TEM or quasi-TEM mode are characterized by the following parameters:

Zc1  , Zc2 : 	Common mode characteristic impedances of line-1 and line-2.

Zp1  , Zp2 : 	Difference (pi ) mode characteristic impedances of line-1 and line-2.

c =wl(c )1/2/c=(/2)(f/fqc ): 	Common mode electrical length.		           (7.10 )

p = wl(p   )1/2/c=(/2)(f / fqp ) :  	Difference mode electrical length.                            (7.11 )

fqc =c/[4l(c )1/2 ] :          	Common mode quarter wavelength frequency.                      (7.12 )

fqp=c/[4l(p)1/2]:     	Difference mode quarter wavelength frequency.	           (7.13 )

where c and p are common and difference mode effective relative permittivities. The impedances are not independent of each other but related through [30], [37]

Zc2 = -Rc Rp Zc1  ,       Zp2 = -Rc Rp Zp1		 			                        (7.14 )

where Rc and Rp are called common and difference mode voltage ratios of coupled lines and are to be calculated in terms of line physical parameters (dimensions, c , p). If the two coupled lines are identical (symmetrical parallel coupled lines), then

Rc=-Rp=1								           (7.15 )

and the terminology “common” and “difference” modes are replaced by “even” and “odd” modes respectively. For such symmetrical lines we have

Zc1 = Zc2 = Zoe   : Even mode impedance.					( 7.16 )

Zp1 = Zp2 = Zoo   	: Odd mode impedance.					( 7.17 )

c = e                	: Even mode electrical length.				( 7.18 )

p = o                	: Odd mode electrical length.				( 7.19 )

fqc = fqe               	: Even mode quarter wavelength frequency.			( 7.20 )

fqp= fqo                	: Odd mode quarter wavelength frequency. 			( 7.21 )

For homogeneous medium of propagation, we have

c = p = r								( 7.22 )

and hence a single electrical length  and a single quarter wavelength frequency fq exists for both modes. In Filpro parallel coupled lines are described in terms of the parameter set

(Zc1  , Zp1  , Rc  , Rp , c  , p  , l )

Besides these elements, Filpro includes Exponential Lines and stubs in its library as examples of nonuniform transmission lines that may be useful in some filtering applications (Figure 7. 3). In these lines and stubs characteristic impedance changes from Z1 at its input end to Z2 at its output in an exponential manner with distance:

Zc (x) = Z1 exp(x)  with  Zc (l )=Z2   				(7.23 )

Tapering rate   is calculated by Filpro in terms of the specified Z1 and Z2.

Figure 7. 2. a)  Exponential  Line b)  Exponential  Shunt  OC  stub. c)  Exponential  Shunt SC stub.            d) Shunt resonator formed by Exponential Line-capacitor  combination.

7. 2. REACTANCE DESCRIPTION OF STUBS AND TRANSMISSION LINES

Lossless lines and stubs contribute to responses of filters through their reactances. Therefore a brief review of their reactive properties would be helpful in visualizing their functions in filters.

ABCD parameters of a uniform transmission line of characteristic impedance Zc are

A=D=cos   ,   B= jZc sin     ,   C = jYc sin   				( 7. 24 )

where Yc=1/Zc is characteristic admittance and  is the electrical length.

Input impedance of a line terminated by an impedance ZL can be written in terms of ABCD parameters as

Zin = (AZL + B)/(CZL + D)							 ( 7. 25 )

The input impedances of open and short circuited stubs can be found using 7. 24 and 7. 25 as

Zsc = B / D = j Zc tan  = j Xsc						( 7. 26 )

Zoc = A / C = -j Zc / tan  = j Xoc 						( 7. 27 )

Reactances Xsc and  Xoc can be written as functions of frequency as

Xsc = Zc tan (f / 2fq ) = Zc tan [wl (r  )1/2 /c]					( 7. 28 )

Xoc = - Zc / tan(f / 2fq  )= - Zc / tan[wl (r )1/2  /c]				( 7. 29 )

Variation of these reactances with frequency are drawn in Figure 7. 3.

Figure 7. 3. Frequency dependencies of reactances of  a) a SC stub and  b) an OC stub.

Following observations can be noted for later reference:

i) For the frequency range f<fq (quarter wavelength frequency),  SC stubs have inductive reactance while OC stubs have capacitive reactance. For low enough frequencies (e. g. for f <fq/4)  SC and OC stubs can be replaced by approximately equivalent lumped inductors (Le ) and lumped capacitors (Ce ) respectively using the approximation

tan    ,							( 7. 30 )

Xsc = Zc tan    Zc wl (r  )1/2 /c = wLe       				( 7. 31 )

Xoc= -Zc /tan   -c Zc / [wl (r  )1/2]= -1/ wCe            			( 7. 32 )

giving lumped element values

Le=Zcl(r )1/2/c    ,    Ce=l(r  )1/2/(cZc  )				( 7. 33 )

ii) At and around frequency  f=fq  , a SC stub behaves as a parallel resonator while an OC stub behaves as a series resonator.

iii) In the frequency range fq<f<2fq , a SC stub shows capacitive reactance while an OC stub shows inductive reactance.

iv) Around the frequency f=2fq ,  SC stubs behave as series resonator while OC stub behaves as parallel resonator.

v) These characteristics repeat along frequency axis with a period of 2fq . Therefore any circuit made up of SC and OC stubs of equal length  (same fq ) will have a periodic response with the same period of  2fq.

A transmission line piece of length l  has more complicated reactance models as shown in Figure 7.4. It can be represented by its Pi or Tee equivalent circuits in terms of SC and OC stubs of length   l / 2,  (Figure 7. 4.b and c). For low enough frequencies the stubs can be replaced by their lumped equivalents giving the circuits shown in Figure 7. 4. d and e. These models and their further variations are frequently used in filtering networks (see Replace Tline By command of Filpro).

Figure 7. 4. a) A transmission line. b) All-stub Pi model. c) All-stub Tee model. d) Lumped element Tee model. e) Lumped element Pi model.

7. 3. RICHARDS' TRANSFORMATION AND ALL-STUB FILTERS

Inspection of impedance, admittance, ABCD, scattering, etc. descriptions of SC stubs, OC stubs, transmission lines and circuits formed by combinations of these components show that all of them are rational functions of a common parameter “tan”  if all lines and stubs are of equal length or integer multiples of a common length l (hence the name commensurate length). The actual frequency variable w=2f doesn't appear explicitly in any function except in the electrical length  =f/2fq=w. Therefore it proved to be very useful to define a new frequency variable  as

 = tan  						 ( 7. 34 )

Analysis and synthesis of such networks become simpler in terms of this frequency. Corresponding to the complex frequency s=+jw, we define the new complex frequency (termed as Richards transformation     [ref]) as

S=+ j  = tanh s = tanh ( + jw)				( 7.35 )

For s=jw Equation 7.25  becomes identical to 7. 24. In this frequency domain input impedances of  SC and OC stubs can be written as

Zsc= jZctan =jZc 						( 7. 36 )

Zoc= 1/ jYc tan = 1/jYc  					              ( 7. 37 )

which may be generalized to the complete  S=+j  plane as

Zsc= SZc     ,      Zoc=1/SYc 					              ( 7. 38 )

Comparison of these expressions with impedance expressions of lumped inductors and capacitors,

Zind=sL ,       Zcap=1/sC  					              (7. 39 )

suggests that we can treat short circuit stubs as S-domain inductors with inductance L = Zc and open circuit stubs as S-domain capacitors with capacitance   C = Yc. These two similarities enable us to use almost all the analysis and synthesis techniques developed for lumped element circuits for analysis and synthesis of circuits made up of commensurate length stubs. So without the need for any new synthesis theory, one can design S-domain “all-stub” filters in exactly the same way as in lumped element filter design. All we have to do is to replace the quantities like s, w, wp , ws , etc.  of lumped element filters by S, , p, s ,... through transformations

 = tan (w/2wq ) 						(7. 40)

p= tan (wp/2wq ) 						(7. 41)

s= tan (ws /2wq ) 						(7. 42)

In the new frequency domain all inductors are to be treated as SC stubs with characteristic impedance equal to inductance and all capacitors are to be treated as OC stubs of admittance equal to capacitance. Thus:

- A series arm OC stub or a shunt arm SC stub forms a transmission zero at S=j=0  (Figure 7. 5. a , b).

- A series arm SC stub or a shunt arm OC stub forms a transmission zero at S=j=  (Figure 7.5.c,d).

- A series stub resonator (a SC stub in series with an OC stub) in shunt arm or a parallel stub resonator (a SC stub in parallel with an OC stub) in series arm (Figure 7.5.e and f) forms a finite transmission zero at frequency

o = 1/ (LC)1/2 = 1/ (ZscYoc)1/2						( 7. 43 )

Figure 7. 5. Transmission zero producing elements in  S = + j  domain: a , b) Transmission zeros at    = 0. c, d) Transmission zeros at  = . e, f ) S-domain finite transmission zeros.

Thus, S-domain all-stub filters can be designed by using the same kinds of transmission zeros as in lumped element filters. So, we can use either Design/By Prototype or Design/By Synthesis approaches to get filters having lowpass, highpass, bandpass or bandstop characteristics along  j axis. For example, in Figure 7. 6, some typical all-stub LP, HP, BP and BS filter circuits are shown.

Figure 7. 6. Some all-stub (S-domain) filter circuits. A) An S-domain lowpass filter,  b) an S-domain highpass filter, c)an S-domain bandpass filter and  c)an S-domain bandstop filter.

As explained previously, the s = jw domain response of all-stub filters are periodic with a period of 2fq. Let's inspect insertion loss characteristics of such filters along jw axis.

i) S-Domain lowpass filters.

In Figure 7. 7 insertion loss responses   () and  (f) of an  S-domain lowpass filter are shown. Mapping between the two graphs is obtained through the transformation

 = tan ( f / 2fq ) 						(7.44 )

which maps the frequencies

f = 0, 2fq , 4fq , 6fq  , . . .  to      = 0 					(7.45 )

and

f = fq , 3fq , 5fq , . . .        to       = 					(7.46 )

Figure 7.7. Periodic properties of insertion loss response of an S-domain lowpass filter.

The following observations can be noted for later reference:

- The lowpass characteristics seen in the region 0 <  <  is squeezed into the range 0 < f < fq along the actual frequency axis f.

-  (f) has infinitely many pass and stop bands with separation 2fq. It can be used as a quasi-lowpass filter if the first spurious passband around 2fq is chosen sufficiently far from the first passband  (in the range 0 < f < fq )  by selecting a high fq  (small stub length) or  if these passbands can be eliminated using other lowpass filters.

- Such structures can also be used as quasi-bandstop filters to reject the band around  fq  (together with the bands around 3fq , 5fq , . .).

- Some critical frequencies are mapped as follows:

p     maps to       f = fp ,   2fq - fp    ,   2fq + fp    , . . . 				( 7.47 )

s      maps to       f = fs  ,   2fq - fs   ,   2fq + fs     , . . . 				( 7.48 )

- In designing a lowpass filter, besides the usual specifications like p , fp , s , fs , fz  (finite TZs) etc., we  have to specify the quarter wavelength frequency fq which sets the common stub length l  through

l= c/[4fq (r)1/2]						                            ( 7.49 )

Then the critical frequencies fp , fs , fz , . . are also mapped to S-domain through

 = tan ( f / 2fq)

to give p , s , z . etc. after which the S-domain filter will be designed using either Prototype or   
     Synthesis approaches. The freedom of choosing fq  provides flexibility for designing alternative 
         structures having the same passband corner fp.

- In order to use such an all-stub filter as a bandstop filter, as it is seen from insertion loss response  (f), we have to specify the quarter wavelength frequency fq as the center of stopband. That is we have  no  freedom of choosing fq arbitrarily. The passband and stopband edge frequencies fp and fs set the stop bandwidth around fq , extending from fp1 = fp  to  fp2 = 2fq - fp . That is, fq  is related to fp1  and fp2  through

fq=(fp1+fp2) / 2								( 7.50 )

Such bandstop filters have unavoidable spurious stopbands around odd multiples of  fq.

- Since transmission zeros at = map to  f = fq , they become finite transmission zeros along jw axis.

Hence, for example a Chebyshev or Butterworth S-domain filter will act as a filter with finite   
     transmission zeros at f =fq , having sharper slopes compared to lumped element counterparts of the  
      same degree.

Exercise: Using Design/By Built-in LC Prototype approach, design a lumped element Chebyshev lowpass filter with specifications

p=0.1 dB ,     fp=4000 MHz ,   N=7

Plot insertion loss response up to f=30000 MHz. Next, design a distributed element Chebyshev filter with

p=0.1 dB ,   fp=4000 MHz , fq=8000 MHz ,   N=7

and overplot the response with the previous one and compare slopes of the two filter (See Figure 7.8).

Figure 7. 8. a) A Chebyshev lumped element lowpass filter with p=0.1 dB, fp=4000 Mhz and N=7.               b) A Chebyshev distributed element (all-stub) filter with p=0.1 dB, fp=4000 Mhz , N=7  and fq=8000 MHz. c) Overplotted Insertion losses of the lumped and distributed element filters.

Exercise:   Repeat design of  the previous exercise for fq=6000, 8000 and 12000 MHz. Inspect slopes of the skirts and stop bandwidths (See Figure 7.9).

Exercise: Using Design/By Built-in Prototype approach to design some Generalized Chebyshev, distributed element lowpass filter with specifications

p=0. 01 dB,    fp=4000 MHz , s=60 dB , N=7

fq=6000 MHz ,   8000 MHz ,   12000 MHz.

Inspect their wideband responses (multiple pass and stop bands). Inspect stub lengths.

Detailed design examples together with circuit transformation techniques to solve realization problems will be presented in Chapter 8.

Figure 7. 9. Insertion losses of lowpass filters with a) fq = 6000 MHz, b) fq = 8000 MHz and                     c) fq=12000 MHz.

ii) S-domain bandstop filters.

In Figure 7.10, insertion loss responses  () and  (f ) for an S-domain bandstop filter are shown.

The following points can be noted:

- The whole spectrum 0<<  is mapped into the range 0<f<fq and the mirror image (with respect to   fq ) of  response in this range appears in the range fq<f<2fq. This pattern repeats itself with period of  2fq.

- The stop band center is denoted by f0 and it is less than fq. Hence by choosing fq>>f0  it is theoretically possible to push the nearest spurious stop band (around 2fq-f0) away from f0. Therefore it seems that this approach is better than using a distributed lowpass filter as a bandstop filter. However realization difficulties makes S-domain bandstop filters a theoretical subject only. Therefore this subject will not be pursued any more.

Exercise: Design some distributed element bandstop filters and inspect their structures and responses. One example is shown in Figure 7.9 for  p=0.1 dB, fp1=3500 MHz,  fp2=5500 MHz and fq=12000 MHz.

Figure 7.10. Insertion loss characteristics of an S-domain bandstop filter.

Figure 7.11. Circuit and insertion loss response for an S-domain bandstop filter.

iii) S-domain highpass filters.

In Figure 7.12, insertion loss responses  () and  (f) for an S-domain highpass filter are shown.

We note the following observations:

- The whole range 0<<  is mapped into the range  0<f<fq  and the mirror image of   0<f<fq with respect to fq appears in the range   fq<f <2fq. That pattern repeats itself with a period of 2fq.

- (f) has infinitely many pass and stop bands. The first passband is centered at fq. Such filters are used as bandpass filters to pass the band around fq. The disadvantage of these bandpass filters is the presence of nearest spurious passband centered at 3fq. It has to be made ineffective by some means by for example, cascading a lowpass filter.

- In order to use such highpass filters as bandpass filter, the following points should be observed in the design stages:

Figure 7.12. Insertion loss characteristics of an S-domain highpass filter.

Figure 7.13. Circuit and insertion loss response of an S-domain highpass filter.

Let the passband edge frequencies of the desired filter be fp1 and fp2. The quarter wavelength frequency fq must be chosen as the passband center. It is the arithmetic mean of fp1 and fp2:

fq=(fp1+ fp2 )/2=f0

The lower passband edge frequency fp1 of bandpass filter must be chosen as passband edge frequency of highpass filter:

fp=fp1

- By choosing sufficiently high fq , the upper band edge at  2fq - fp  can be pushed away so that the filter can be used as a quasi-highpass filter.

Exercise: Using "Built-in Prototype " approach, design some Chebyshev and Generalized Chebyshev distributed element highpass filters. Inspect their structures and responses. An example is shown in Figure 7.13 for p=0.1 dB,   fp=4000 MHz,  fq=6000 MHz, giving an upper passband edge at f = 8000 MHz and nearest spurious passband  centered at f =3fq=18000 MHz.

Detailed examples together with circuit transformations for overcoming realizability problems will be presented in Chapter 9.

iv) S-domain bandpass filters.

In Figure 7.14 insertion loss curves () and (f) for an bandpass filter are shown.

Figure 7.12. Insertion loss characteristics of an S-domain bandpass filter.

The following observations are noted:

- Similar to all the previous S-domain filters, the whole range   0<< is squeezed into the range

- 0<f<fq with mirror image of that range with respect to fq appearing in the range fq<f<2fq.

- The desired passband center is denoted by f0 and it is less than fq. The nearest spurious passband is at 2fq-f0.

- Since fq and f0 are independently specifiable  (contrary to bandpass filters designed as S-domain highpass filters) this approach is more flexible to yield alternative structures having more desirable features. For example by choosing sufficiently high fq , the nearest spurious passband at 2fq - f0 can be pushed  away from the desired passband, producing a wide stopband centered at fq. in addition to that advantage, high fq leads also to smaller stub length l through

l=c/[4fq(r  )1/2 ]

making miniaturization possible.

- Comparison of insertion loss responses of  S-domain highpass and bandpass filters shows that  S-domain highpass filters are special cases of  S-domain bandpass filters. This point can be seen by setting fq=2f0 in bandpass filters. For that case the fist spurious passband will appear at 3f0, same as in highpass cases.

As an example, the circuit structure and insertion loss responses of S-domain bandpass filters with

p=0.1 dB, fp1=4000 MHz,  fp2=6000 MHz,  prototype degree N=7, with fq=8000, 10000 and 15000 MHz are shown in Figure 7.13. Note widening of the stopband as fq is increased. Note also that response is similar to that of  an S-domain highpass filter  when fq=2f0=10000 MHz.

Detailed examples will be given in Chapter 10.

Figure 7.13. a) Circuit of an S-domain bandpass filter. b , c , d) Insertion loss responses for fq=8000 MHz , 10000 MHz  and 15000 MHz respectively.

7. 4. DISTRIBUTED ELEMENT FILTERS WITH CONTRIBUTING UNIT ELEMENTS

The all-stub S-domain filters described so far has one common problem: They are usually unrealizable for several reasons:

- Some stub types may not be realizable with the technology of production used (e. g. series arm OC and SC stubs in stripline).

- Some stub types may be undesirable because of associated problems (e.g. shunt arm SC stubs in microstrip structures).

- Stubs require physical separation to avoid electromagnetic coupling. Furthermore, it is difficult (if not impossible) to connect more than two stubs at a node.

- Contrary to lumped elements, there are severe limitations on realizable element values. Typical dynamic ranges of realizable characteristic impedances are limited to roughly 1: 10.

Fortunately, most of these problems are solvable by equivalent circuit transformations, as will be described in the succeeding chapters. Some of these problems, like physical separation of adjacent stubs, conversion of series arm stubs to shunt form, etc. are solvable in a very simple manner by inserting transmission line pieces (of the same length as stubs) into circuits by circuit transformations in a redundant manner (not contributing to response). Such transmission line pieces are termed as unit elements ( UE ). Unit elements have no counterparts in lumped element circuit theory. Though unit elements are (assumed to be) lossless, in reality they have nonnegligible losses and their use makes filters longer. Therefore whenever possible, it is preferred to employ unit elements in a nonredundant manner (contributing to the response). This is possible by incorporating unit elements into the filter transfer function right at the beginning of synthesis procedure as a transmission zero section. Inspection of unit element transfer functions shows that they have a transmission zero at  S==1+ j0. Since this transmission zero is not on S= j axis, its contribution to response may not be expected to be as strong as those of stubs. However in many practically important  applications their contribution approach to those of stubs. This way, besides solving the problem of separating stubs physically, we also get the contributions of unit elements to improve the response, hence use the space allocated to filter more efficiently.

Since unit elements have no counterparts as those of SC and OC stubs, it is not possible design filters involving unit elements by the classical reactance transformations from prototype tables. Such filters are synthesized by augmenting the synthesis techniques used for all-stub filters to allow inclusion of unit elements. The Synthesis program of Filpro can handle lowpass, highpass and bandpass filters with contributing unit elements. Since the length of unit elements is the same as stubs, the periodicity of insertion loss response described in the previous section are valid for such filters also.

Element (transmission zero) extraction procedures in “stub + Unit Element” filters follow similar lines to those of the lumped element ones. Some examples to demonstrate synthesis procedures follows. Detailed design applications involve a variety of circuit transformations and will be presented in the succeeding chapters.

Example: Synthesis of a distributed element lowpass filter with contributing unit elements.

Let     p=0.1 dB, fp=4000 MHz, fq=12000 MHz,  Ninf =5,  NUE=6, RS=RL=50 

The steps of synthesis in Filpro are as follows:

“New / Design / By Synthesis /

Filter Type: Distributed, Lowpass, Equiripple, Doubly Terminated, Type A

Passband Specs and Terminations:  PBRipple=0.1, fp=4000 , fq=12000 , RS=50, RL=50

Specify TZs:  INF, INF, INF, INF, INF, UE, UE, UE, UE, UE, UE

As transmission zeros are entered, insertion loss is dynamically displayed on the screen for inspection of  contribution of each added transmission zero. For example, in Figure 7.14, insertion loss just after entering the five transmission zeros at =  (i.e. at f=fq) and insertion loss after entering all transmission zeros are shown. The contribution of unit elements is obvious.

After finishing the placement of transmission zeros, the next stage is extraction of transmission zero sections. They can be extracted in any order user prefers, leading to different circuits. One example is shown in Figure 7.15 where all stubs are of shunt open circuited type, separated by unit elements:

Extract TZs: UE, INF, UE, INF, UE , INF ,UE, INF, UE, INF, UE

As exercise, try other extraction orders.

Figure 7.14. Insertion loss response of the lowpass filter before and after placement  of unit elements.

Figure 7.15. One realization of the lowpass filter with contributing unit elements.

Example: Synthesis of a distributed element lowpass filter with contributing unit elements and finite transmission zeros.

Let p=0.1 dB, fp=4000 MHz, fq=12000 MHz,  Ninf=1,  NUE=4,   Four  FTZs at f=4500 MHz, RS=RL=50 

The steps of synthesis are as follows:

“New / Design / By Synthesis /

Filter Type: Distributed, Lowpass, Equiripple, Doubly Terminated, Type A

Passband Specs and Terminations:  PBRipple = 0.1, fp = 4000 , fq = 12000 , RS = 50, RL = 50

Specify TZs: INF, FTZ=4500, FTZ=4500, FTZ=4500, FTZ=4500, FTZ=4500, UE, UE, UE, UE

Insertion loss is shown in Figure 7.16. Transmission zeros have to be extracted by partial pole removal from infinity. Since there is only one transmission zero at infinity, it must not be extracted before all finite transmission zeros are extracted. Unit elements may be distributed between finite transmission zero sections. Hence, one possible order of element extraction may be as shown in Figure 7.16:

UE , FTZ , UE , FTZ , UE , FTZ , INF, UE

As exercise, try different orders of extraction and try synthesizing Type B circuits also. Realization of all such circuits require some circuit transformations. Further examples on distributed element lowpass filters together with circuit transformation techniques will be studied in the forthcoming chapters.

Figure 7.16. One realization of a lowpass filter with contributing unit elements and finite transmission zeros.

Exercise: Repeat the same example with 6 unit element to get the response and structure shown in Figure 7.17

Figure 7.17. A lowpass filter with six contributing unit elements.

Example: Synthesis of a distributed element highpass filter with contributing unit elements.

Let       p=0.1 dB, fp=4000 MHz, fq=6000 MHz,  Nzero =5,  NUE=6,  RS=RL=50 

This structure will act as a bandpass filter with passband edges at  fp1=4000 MHz and fp2=8000 MHz. The steps of synthesis are as follows:

“New / Design / By Synthesis /

Filter Type: Distributed, Highpass, Equiripple, Doubly Terminated, Type A

Passband Specs and Terminations:  PBRipple =0.1, fp=4000 , fq=6000 , RS=50, RL=50

Specify TZs: ZERO, ZERO, ZERO, ZERO, ZERO, UE, UE, UE, UE, UE, UE

Insertion loss response of filter after placing transmission zeros at zero and after placing all transmission zeros are shown in Figure 7.18.

Figure 7.18. Insertion loss of a highpass filter before and after placing six unit elements.

Transmission zero sections can be extracted in different orders, leading to different circuits. However the usual ones are physically symmetric realizations, because they evade transformers. All other realizations end up with an ideal transformer calculated at the source end. In Figure 7.19, a physically symmetric realization and an unsymmetric realization are shown. Further examples on distributed element highpass filters will be studied in Chapter 9.

Figure 7.19. A symmetric realization (a) and an asymmetric realization (b) for a highpass filter.

Example: Synthesis of a distributed element bandpass filter with contributing unit elements.

Let p=0.1 dB, fp1=4000 MHz, fp2=6000 MHz, fq=12000 MHz,  Nzero=5, Ninf=5, NUE=12, RS=RL=50

The steps are as follows:

“New / Design / By Synthesis /

Filter Type: Distributed, Bandpass, Equiripple, Doubly Terminated, Type A

Passband Specs and Terminations:  PBRipple=0.1, fp1 = 4000 , fp2 = 6000, fq=12000 , RS=50, RL=50

Specify TZs: ZERO, ZERO, ZERO, ZERO, ZERO, INF, INF, INF, INF, INF, UE, UE, UE, UE, UE, UE

Insertion loss response of filter after placing transmission zeros at zero and infinity, and after placing all transmission zeros are shown in Figure 7.20.

Figure 7. 22. Insertion loss response of the bandpass filter before and after placing the unit elements.

If  transmission zeros are extracted in a symmetric manner, then a physically symmetric structure without any transformer results as shown in Figure 7.21. All other extraction orders will result in an asymmetric structure with an ideal transformer at the load end. One such realization is shown in Figure 7.22.

Figure 7.21. A physically symmetric realization for a bandpass filter.

Figure 7.22. An asymmetric realization for a bandpass filter.

Bandpass filters with finite transmission zeros have properties similar to those of lumped element ones. Finite transmission zeros in lower stopband are extracted by partial pole removal from transmission zeros at zero, while finite transmission zeros in upper stopband are extracted by partial pole removal from infinity, as can be observed in the following example.

Example: Synthesis of a distributed element bandpass filter with contributing unit elements and finite transmission zeros.

Let       p=0.1 dB, fp1=4000 MHz, fp2=6000 MHz, fq=12000 MHz,  Nzero=1, Ninf=1, NUE=6,  two FTZs at f =3000 MHz,  two FTZs at f=7000 MHz,  RS=RL=50 

The steps of synthesis are as follows:

“New / Design / By Synthesis /

Filter Type: Distributed, Bandpass, Equiripple, Doubly Terminated, Type A

Passband Specs and Terminations:  PBRipple=0.1, fp1=4000 , fp2=6000, fq=12000, RS=50, RL=50

Specify TZs: ZERO,  INF, FTZ1=3000, FTZ2=3000 , FTZ3=7000, FTZ4=7000, UE, UE, UE, UE, UE, UE

Transmission zeros can be extracted in various orders. However very few of them will come out to be realizable because of severe limitations on realizable characteristic impedance values. One realization is shown in Figure 7.25 where improvement of insertion loss is also displayed as the unit elements and finite transmission zeros  are inserted.

As exercise, try extracting elements in different orders and also Type B circuits to see various possible forms for finite transmission zero sections. In almost all such realizations an ideal transformer will appear at the load end. Such transformers can be deleted after circuit transformations to put the structure into a physically symmetric form. These techniques will be presented in Chapter 10 together with some classical realizable topologies.

Figure 7.23. A bandpass filter with contributing unit elements and finite transmission zeros.

REFERENCES AND FURTHER READING

The literature listed below are by no means complete. They are typical of the sources through which the authors have learned the theory, became aware of the theoretical and technological problems and their possible solution techniques, some of which are then reformulated for adoption to Filpro.

Part-1) References for distributed element filter theory and applications:

- P. I. Richards, “Resistor Transmission Line Circuits”, Proc. IRE, Vol. 36, 217-220, February 1948.

- H. Ozaki, and  J. Ishii, “Synthesis of a Class of Stripline Filters”, IRE. Trans. Circuit Theory, Vol.CT-5, pp. 104-109, 1958.

- S. B. Cohn, “Parallel Coupled Transmission Line Resonator Filters”, IEEE Trans MTT, vol. MTT-6, No.2, pp. 223-231, Apr. 1958.

- G. L. Matthaei, “Interdigital Bandpass Filters”, IRE Trans. MTT, Vol. MTT-10, No. 6, pp. 479-491, Nov. 1962.

- G. L. Matthaei, “Comb-Line Bandpass Filters of Narrow or Moderate Bandwidth”, The Microwave Journal, Vol. 6, No. 9, pp. 82-91, Aug. 1963.

- G. L. Matthaei, “ Tables of impedance transforming networks of lowpass filter form”, Proc. IEEE, vol. 52, pp. 939-963, Aug. 1964.

- R. J. Wenzel, “Exact Design of TEM Microwave Networks Using Quarter-Wave Lines”, IEEE Trans. MTT,  Vol. MTT-12, pp. 94-111, Jan. 1964.

- Matthaei, G. L., Young L., E. M. T. Jones, “Microwave Filters, Impedance Matching Networks and  Coupling Structures”, Mc Graw Hill , NY, 1964.

- M. C. Horton and R. J. Wenzel, “General Theory and Design of Optimum Quarter Wave TEM filters”, IEEE Trans. MTT,  vol. MTT-13, No. 3, pp. 316-327, May 1965.

- B. M. Sciffmann and G. L. Matthaei, “Exact Design of Bandstop Microwave Filters”, IEEE Trans. MTT-12, pp.6-15,  January 1965.

- E. G. Crystal, “Tables of Maximally Flat Impedance Transforming  Networks of lowpass filter form”, IEEE Trans. On  MTT ,  vol. MTT-13, No.5,  pp. 693- 695,  Sept. 1965.

- R. J. Wenzel, “Theoretical and practical applications of Capacitance matrix transformations to TEM Network Design”, IEEE Trans MTT, vol. MTT-14, No.12, pp. 635-647, Dec. 1966.

- R. Levy, “A General Equivalent Circuit Transformation for Distributed Networks”, IEEE Trans. CT, Vol. CT-12, No 3,  pp.457-458, Sept. 1965.

- R. Levy and I. Whiteley, “Synthesis of Distributed Elliptic Function Filters from Lumped Constant Prototypes”, IEEE Trans. MTT, Vol. MTT-14, No. 11,  pp. 506-517, Nov. 1966.

- M. C. Horton and R. J. Wenzel, “The Digital Elliptic Filter- A Compact Sharp Cutoff Design for Wide Bandstop or Bandpass Requirements”, IEEE Trans. MTT, Vol. MTT-15, No 5,  pp. 307-314, May 1967.

- G. I. Zysman, “Coupled Transmission Line networks in an Inhomogeneous Medium”, IEEE Trans. MTT, Vol. MTT- 17, No. 10,  pp. 753 - 759, Oct. 1969.

- R. Levy, “ A New Class of Distributed Prototype Filters with applications to Mixed Lumped / Distributed Components”, IEEE Trans. Mtt, Vol. MTT-18, pp. 1064 - 1071, Dec. 1970.

- R. Levy, and J. D. Rhodes, “A Combline Elliptic Filter”, IEEE Trans MTT, vol. MTT-19, pp. 26-29, Jan. 1971.

- R. J. Wenzel, “Synthesis of Comb-line and Capacitively Loaded Interdigital Bandpass Filters of Arbitrary Bandwidth”, IEEE Trans MTT, vol. MTT-19, No. 8, pp. 678-686, Aug. 1971.

- E. G. Crystal, “New Design Equations for a Class of Microwave Filters”, IEEE Trans MTT, vol. MTT-19, No.5, pp. 486-490, May 1971.

- E. G. Crystal and S. Frankel, “Hairpin Line and Hybrid Hairpin Line Half Wave Parallel Coupled Line Filters”, IEEE Trans MTT, vol. MTT-20, No.11, pp. 719-728,  Nov.1972.

- R. Levy, “A Generalized Design Technique for Practical Distributed Reciprocal Ladder Networks”, IEEE Trans. MTT, vol. MTT-21, No.8 , pp. 519- 526, Aug. 1973.

- G. Hoffmann, “ The Direct Synthesis of Optimum Commensurate Distributed Filters by Digital Computer”, 1973 European Microwave Conference Proc., B.2.2, Brussels, 1973.

- G. Reiter and G. Hammer, “ Stripline Filters with Coupled /8 Line Sections”, 1973 European Microwave Conference, B.3.2, Brussels, 1973

- K. K. Pang, “Design of Microwave Filters by Sine-plane approach”, IEEE Trans MTT, vol. MTT-21, pp. 607-611, Oct. 1973.

- U. H. Gysel, “New Theory and Design for Hairpin Line Filters”, IEEE Trans MTT, vol. MTT-22, No. 5, pp. 523-531, May 1974.

- G. Reiter, “ Stripline Filters with Lumped Capacitors”, 5. European Microwave Conference Proc., pp. 426 - 430. 1975.

- R. Briechle, “Microstrip Combline Filter”, 5. European Microwave Conference Proc., pp.436-440, 1975.

- E. G. Crystal, “Tapped-Line Coupled Transmission Lines with Applications to Interdigital and Combline Filters”, IEEE Trans. On  MTT ,  vol. MTT-23, No. 12,  pp. 1007-1012, Dec. 1975.

- V. K. Tripathy, “Asymmetric Coupled Transmission Lines in an Inhomogeneous medium”, IEEE Trans. MTT, Vol. MTT- 23, No. 9,  pp. 734 -739, Sept. 1975.

- J. A. G. Malherbe, “TEM Pseudo Elliptic Bandstop Filters Using Noncommensurate Lines”, IEEE Trans. MTT, Vol. MTT-24, No 5,  pp. 242-248, May 1976.

- G. R. Hoffman, “Resonated Combline Bandpass Filter”, 6. European Microwave Conference Proc., pp. 54-58, 1976.

- J. D.  Rhodes, “Theory of Electrical Filters”, J. Wiley and Sons, NY, 1976.

- M. C. Horton, “ TEM Bandpass Filters Having Eight Wavelength Coupled Stubs”, 1977 IEEE, MTT- S Int. Microwave Symposium Digest, San Diego , 1977.

- J. A. G. Malherbe, “Realization of Elliptic Funtion Bandstop Filters  by means of Resonated Prototypes”,  IEEE Trans. MTT, Vol. MTT-25, No. 8, p. 717,  Aug.1977.

- R. N. Bates, “Design of Microstrip Bandstop Filters”, IEE Microwaves, Optics and Acoustics, pp.209-214,  Nov.1977.

- V. K. Tripathy,  “Equivalent Circuits and Characteristics of Inhomogeneous Nonsymmetrical Coupled Line Two-Port Circuits”, IEEE Trans. MTT,  pp. 140-142,  Feb. 1977 .

- L. Young, “Microwave Filters Using Parallel Coupled Lines”, Artech House,1980.

- J. A. G. Malherbe, “Microwave Transmission Line Filters”, Artech House, 1980.

- J. D. Rhodes and S. A. Alseyab, “The Generalized Chebyshev Low Pass Prototype Filters”, Int. J. Circuit Theory and Applications,Vol. 8, pp. 113-125, 1980.

- J. B. Minnis, “Printed Circuit Coupled Line Filters for Bandwidths up to and Greater than an Octave”, IEEE Trans MTT, vol. MTT-29, No. 3, pp. March 1981.

- B. Easter, K. Merza, “Parallel Coupled Line Filters for Inverted Microstrip and Suspended Substrate MICs”, 11. European Microwave Conference Proc., pp.164 - 168, A.2.4, 1981.

- J. B. Minnis, “Classes of Sub Miniature Microwave Printed Circuit Filters with Arbitrary Passband and Stopband Widths”, IEEE Trans. MTT, Vol. MTT-30,  pp. 1893-1900,  Nov 1982.

- S. A. Alseyab, “A Novel Class of Generalized Chebyshev Low Pass Prototype for Suspended Substrate Stripline Filters”, IEEETrans. Microwave  Theory and Tech. Vol. MTT-30, No. 9, pp. 1341-1347, Sept. 1982.

- P. La Tourette, “High Pass Filter Design”, 12. European Microwave Conference Proc., pp. 233-238, England, 1982.

- C. I. Mobbs and J. D. Rhodes, “A Generalized Chebyshev Suspended Stripline Bandpass Filter”, IEEE Trans. MTT, Vol. MTT-31,  pp. 397-402,  May 1983.

- C. Nguyen, C. Hsieh and D. W. Ball, “ Millimeter  Wave Printed Circuit Spurline Filters”, 1983 IEEE, MTT-S Proc., pp. 98 - 100, 1983.

- J. Ness  and S. Johnson, “Narrowband Elliptic Filters on Microstrip”, Microwaves and RF, Nov. 1984.

- C. Nguyen and K. Chang, “Analysis and Design of Spurline Bandstop Filters”, 1985 IEEE, MTT-S  Conference Proc., pp. 445 - 448, 1985.

- A. Riddle, “ High Performance Parallel Coupled Line Microstrip Filters”, 1988 IEEE, MTT-S Conference Proc., pp. 427 - 430, 1988.

- M. Gat, “ Commensurate Line Microstrip Bandpass Filters”, 1988 IEEE, MTT-S Conference Proc., pp. 423- 426, 1988.

- Y. Sen, “Direct Design of  Lumped and Distributed Filters Using Transformed Variable Techniques”, M.Sc. Thesis, EEE Dept. Middle East Technical University, Ankara,Turkey, 1989.

- C. Kuang, C. Tzuang and W. T. Lo, “ Printed Circuit Realization of a Tapped Combline Bandpass Filter”, 1990 IEEE, MTT-S Conference Proc., pp.131 - 134, 1990.

- H. C. Bell, “ Narrow Bandstop Filters”, 1991 IEEE, MTT- S Conference Proc., pp.539 - 542, 1991.

- S. Herbert, “Procedure Produces Miniature MIC bandpass filters”, Microwaves and RF, pp. 203-209, April 1990.

- C. M. Tsai and K. C. Gupta, “A Generalized Model for Coupled Lines and its applications to Two-Layer Planar Circuits”, IEEE Trans. MTT, Vol. MTT- 40,  pp. 2190 - 2199, Dec. 1992.

- J. C. Tippet and M.J. Shian, “Circuit Transformations for realization of a Class of Miniature  Printed Circuits”, IEEE,  MTT-S  Digest, pp.621-624, 1994.                                                                                                                             
                                                                                                                                                                         Part-2) References on characterization of transmission lines:

- S. B. Cohn, “ Shielded Coupled Strip Transmission Line”, IRE Trans. MTT., Vol. MTT-3, pp. 29 - 38, Oct. 1955.

- J. P. Shelton, Jr.,” Impedances of Offset Parallel Coupled Strip Transmission Lines”, IEEE Trans. MTT., Vol. MTT-14, pp. 7-15, Jan. 1966.

- W. J. Getsinger, “ Coupled Rectangular Bars Between Parallel Plates”, IRE Trans. MTT, Vol. MTT-10, pp. 65-72, Jan. 1962.

- S. M. Perlow, “ Analysis of Edge Coupled Shielded Strip and Slab Line Structures”, IEEE Trans. MTT, Vol. MTT-17, pp. 522-529, May 1969.

- S. Akhtarzad, et. al., “ The Design of Coupled Microstrip Lines”, IEEE Trans. MTT, Vol. MTT-23, pp.486-492, June 1975.

- H. A. Wheeler, “ Transmission Line Properties of a Strip on a Dielectric Sheet on a Plane”, IEEE Trans. MTT, Vol. MTT-25, pp. 631-647, Aug. 1977.

- J. I. Smith, “The Even and Odd Mode Capacitance Parameters for Coupled Lines in Suspended Substrate”, IEEE Trans. MTT- 19, pp. 424 - 431, May 1971.

- T. A. Milligan, “Dimensions of microstrip Coupled Lines and Interdigital Structures”, IEEE Trans. MTT, Vol. MTT- 25, May 1977.

- D. Pompei, O. Benevello, E. Rivier, “Parallel Coupled Line Microstrip Filters in an Inhomogeneous Medium”,  IEEE Trans. MTT, Vol. MTT- 26, No. 4, pp. 231- 238, May 1978.

- M. Kirshning and R. H. Jansen, “ Accurate Models for Effective Dielectric Constant of Microstrip with Validity up to millimeter Wave Frequencies”, Electronics Letters, 1982.

- E. Hammerstad and O. Jensen, “ Accurate Models for Microstrip Computer Aided Design”, 1980 IEEE, MTT- S Symposium Digest, pp. 407-409, 1980.

- Y. Shu, X. Qi and Y. Wang, “ Analysis Equations for Shielded Suspended Substrate Microstrip Line and Broadside Coupled Stripline”, 1987 IEEE, MTT-S Symposium Digest, pp.693-696, 1987.

- W. Yunyi, G. Kaijun and S. Yonghui, “ Synthesis Equations for Shielded Suspended Substrate Microstrip Line and Broadside Coupled Strip Line”, 1988 IEEE MTT-S Symposium Digest, pp. 331- 334, 1988.

- W. Yunyi, G. Ling Wang and Y. H. Shu, “ Analysis and Synthesis Equations for Edge Coupled Suspended Substrate Microstrip Line”, 1989 IEEE MTT-S Symposium Digest, pp. 1123-1126, 1989.

- T. Itoh (Editor ), “ Numerical Techniques for Microwave and Millimeter Wave Passive Structures”, J. Wiley and Sons, 1989.