CHAPTER 13

WAVEGUIDE  FILTERS

13. 1. RECTANGULAR WAVEGUIDES

Waveguide pieces operating at frequencies above or below their cutoff frequencies can be used as filter elements in various ways. In this chapter some typical examples will be described using only rectangular waveguides. Circular waveguides may also be treated in the same way.

Waveguides are characterized through the solution of Maxwell’s equations with perfectly conducting boundaries. Consider a rectangular waveguide with broad side (width) denoted by letter a and narrow side (height) denoted by letter b (in mm in Filpro) (Figure 13.1). Input-output couplings to the outside media can be made through probes (Figure 13.1.b) or loops.

A general solution involves infinitely many TE and TM wave modes. Let’s concentrate on only TE modes. Denoting the longitudinal direction as z-axis and transversal directions by x and y, the expressions for electrical and magnetic fields for TE modes are as follows:

Ez=0

Hz=H0zcosmπxacosnπybej(wt−βmnz)

Ex=E0xcosmπxasinnπybej(wt−βmnz)

Ey=E0ysinmπxacosnπybej(wt−βmnz)

Hx=H0xsinmπxacosnπybej(wt−βmnz)

Hy=H0ycosmπxasinnπybej(wt−βmnz)					(13.1.1)

where m=0, 1, 2, .., and n=0, 1, 2, ..   with the case n=m=0 being excluded. Each m-n pair describes one solution set named as the mode-mn. mn is the propagation constant given for the mode m-n as

βmn=ω2με−kcmn2=2πfcμrεr1−(fcmnf)2			(13.1.2)

where

kcmn=12(ma)2+(nb)2						(13.1.3)

and fcmn is the cutoff frequency of the relevant mode given as

fcmn=12με(ma)2+(nb)2=c2μrεr(ma)2+(nb)2	(13.1.4)

where c is the velocity of light, r and r are relative permittivity and relative permeability of the medium,  and  are the dielectric constant and permeability of the medium. It is clear that propagation constant mn is real for f>fc. That is, the signal propagates for f>fc while it attenuates exponentially for f<fc. The phase velocity for a TEmn mode in z-direction is

vpmn=ωβmn=1με1−(fcmnf)2=cμrεr1−(fcmnf)2	(13.1.5)

Figure 13.1. a) Rectangular waveguide. b) A feeding system. c) Ey(x) component of electric field in a rectangular waveguide carrying TE10 mode only. d) One definition for voltage difference between top and bottom walls of a waveguide. e) Currents on the top wall of the waveguide.

Electrical length of a piece of waveguide of length l in the mode mn is defined as

θmn=βmnl	(radians)								(13.1.6)

Electrical distance between two successive peaks is 2. The distance the wave must propagate in order to undergo a phase change of 2 radians is defined as as guide wavelength gmn and calculated as

λgmn=2πβmn								(13.1.7)

Denoting the wavelength at free space as 0, the relation among the triplet of frequency-wavelength-velocity of light is,

λ0=cf									(13.1.8)

In a dielectric / magnetic medium this relation becomes

λ0=1fμε=cfμrεr						(13.1.9)

The guided wavelength gmn may also be written in terms of 0 as

λgmn=λ01−(fcmnf)2							(13.1.10)

For TEmn modes a characteristic wave impedance is defined as follows:

Zwmn=ExHy=−EyHx=με1−(fcmnf)2=μrεr120π1−(fcmnf)2	(13.1.11)

A waveguide resonator can be formed by closing the two ends of a waveguide piece, thus forming a standing wave along the guide. If the length of waveguide resonator is denoted by l, then the expression for resonance frequencies will be

frmnp=12με(mπa)2+(nπb)2+(pπl)2=c2μrεr(mπa)2+(nπb)2+(pπl)2				(13.1.12)

where c is the velocity of light and  p is the number of half wavelengths along the distance l.

Considering a waveguide piece as shown in Figure 13.1.b, it can be fed from for example a coaxial line of 50 ohms impedance through a probe or a loop acting like an antenna. In order to get maximum power transfer from the coaxial line to waveguide or from waveguide to coaxial line, the transition from coaxial to waveguide must show a matched impedance, that is, 50 ohms. Special impedance matching techniques (adjustment of position and depth of probes, matching stubs, etc.) exist for such transitions. The power transferred to the waveguide can be extracted at another point along the waveguide by a similar transition. Depending on the frequency of the signal and the transversal dimensions of the waveguide, the power fed to the waveguide may be split into various modes described above to travel to the output port. Splitting of the power into different modes is usually undesirable because each mode may travel with different velocity leading to dispersion. Further, in order to have efficient power transfer to and from the waveguide, special transitions must be used optimized for a certain mode. For these reasons in practice a single mode of operation is desirable. This is possible if the frequency is selected to be above the lowest cutoff frequency and below the second cutoff frequency. This mode is named as dominant mode. For rectangular waveguides with b<a the dominant mode is TE10 (m=1, n=0) mode because it has the lowest cutoff frequency. For this mode the equations listed above simplify as follows:

For TE10 mode:

Ez=0,     Ex=0,    Hy=0

Hz=H0zcosπxaej(wt−βz)

Ey=Eyosinπxaej(wt−βz)

Hx=Hxosinπxaej(wt−βz)						(13.1.13)

The cutoff frequency of TE10 mode depends only on the width a of the waveguide:

fc=fc10=12aμ0μrε0εr=c2aμrεr					(13.1.14)

Propagation constant of TE10 mode simplifies also and can be put into different practical forms as follows:

β=2πfμrεrc1−(fcf)2						(13.1.15)

or

β=2πfcμrεrc(ffc)2−1=(πa)(ffc)2−1			(13.1.16)

Phase velocity and guided wavelength are

vp=ωβ=cμrεr1−(fcf)2						(13.1.17)

λg=λ01−(fcf)2								(13.1.18)

Wave impedance of TE10 mode is obtained as

- Zw=Ey(x,z)Hx(x,z)=EyoHxo=με1−(fcf)2=μrεr120π1−(fcf)2 (Ohms) 	(13.1.19)

- The direction of time average power flow and its instantaneous value per unit area is given by the Poynting vector:

- S=12EXH								(13.1.20)

- The component of Poynting vector in z-direction gives the power flow per unit area along the waveguide:

- Sz=12Ey(x,z)Hx(x,z)=12Eyosinπxae−jβzHxosinπxaejβz)12EyoHy0sin2(πxa)	(13.1.21)

- Power transfer in propagation direction can be calculated by integrating the time averaged pointing vector over the cross-sectional area of the waveguide:

- P=S(12)EXH.dS=S(12)EyoHy0sin2(πxa)dxdy=ab4Ey0Hx0	(13.1.22)

Using the wave impedance the expression for power may also be written as

P=ab4EyoHxo=abEyo24Zw=abZwHxo24					(13.1.23)

For frequencies f>fc, the wave impedance Zw is a real number. Therefore for these frequencies we have real power flowing along the waveguide.

Below the cutoff frequency, f<fc, the wave impedance becomes imaginary:

Zw=μrεr120π1−(fcf)2=jμrεr120π(fcf)2−1=jXw		(13.1.24)

So, at these frequencies both Poynting vector and the power P become imaginary which means no real power flow. The physical meaning of the imaginary power may be explained by inspecting the expressions for fields as follows:

If a TE10 mode wave is created at some point along the waveguide and if it can’t propagate, that is if its propagation constant  is imaginary, then the electric and magnetic fields decay exponentially, in accordance with the following  definitions:

Ey=Eyosinπxaejβz=Eyosinπxae−αz				(13.1.25)

Hx=Hxosinπxae−jβz=Hxosinπxae−αz				(13.1.26)

where

α=ωμrεrc(fcf)2−1							(13.1.27)

is now termed as attenuation constant. The reactive (imaginary) power flow associated with the decaying waves is used to store energy in the decay region, in much the same way as the reactive power in reactive elements like inductors and capacitors. Therefore the decay region of the waveguide can always be represented as a capacitive or inductive reactance. That is, the decaying fields see a reactive characteristic wave impedance Zw=jXw while propagating fields are assumed to see real characteristic impedance Zw.

The wave impedance defined above is the ratio of the fields Ex0 to Hy0 while in everyday life of engineer (and for analysis on Filpro), voltage-current-impedance concepts are more usual and easier to use. Therefore whenever possible we would like to define voltage, current and impedance concepts. However for waveguides we have some major definition problems as follows:

In transmission lines carrying only TEM mode, a unique voltage difference can be defined between any two points through the line integral of the electric field along any path. Tempted by this simplicity, we may attempt to define a voltage difference for example between the bottom wall (y=0) and top wall (y=b) of a rectangular waveguide. In TE10 mode Ey component has sinusoidal variation x, with maximum value Ey0 being along the central line, x=a/2 as shown in Figure 13.1.c. A voltage difference may be defined between top and bottom walls as the line integral of Ey(x):

V(x)=0bEy(x)dy							(13.1.28)

However contrary to the case of TEM systems, here we get a voltage which depends on the integration path. We get different voltages along different paths of integration. This is an important drawback of TE and TM waves compared to TEM waves. That is, we can’t talk about a unique voltage of a point or a unique voltage difference between two points.

- For currents the situation is also complicated, however not as severe as the voltage case. A rough description is shown in Figure 13.2.e for the currents on the walls of the waveguide. On the top wall of the guide part of the current flows in z-direction (propagation direction) while the rest flows in transverse directions. On the bottom wall, currents flow just in opposite direction to those in the top wall.

- However in spite of all these facts, we still want to make use of the simplicity of voltage-current-impedance concepts. This is possible only under carefully set definitions as follows:

# a) Maximum Voltage-Maximum Current approach:

We can treat the bottom wall as reference for voltage (V=0) and define the voltage of the top wall as the line integral of Ey(x=a/2)=Eyo along a straight path on the x=a/2 line (Figure 13.1.d). This would give the maximum possible voltage between upper and lower walls:

Vm=0bEy0dy=Ey0b							(13.1.29)

giving the result

Eyo=Vmb									(13.1.30)

- Naturally power flow in z-direction is associated with only z-component of the current. Therefore we may calculate the total z-directed current in top or bottom wall and treat it as the total current. The net z-directed current flowing on top (or bottom) wall can be calculated as the line integral of Hxo  over the range x=0 to x=a, giving

- Im=0aHx0dx=Hx0a							(13.1.31)

- or

Hxo=Ima									(13.1.32)

These two definitions can be used to define the characteristic impedance as follows:

Zc=VmIm=EyobHxoa=baZw						(13.1.33)

The simplicity of the above definitions is slightly shadowed by the fact that the product of voltage and current doesn’t give the actual power:

P=12VmIm=12abEy0Hx0						(13.1.34)

This is twice the power given in Eq. 13.1.23. The reason is that we have taken the maximum values for both electric and magnetic fields. In the definition of voltage we have assumed that voltage difference between the top and bottom walls is the same everywhere along x-axis which is not true. For current, we have assumed that Hx is constant over x, while it varies sinusoidally with x.  This discrepancy is of no problem in applications where the actual power flow calculations are not of interest.

# Maximum Voltage-Average current approach

Both electric field Ey(x) and magnetic field Hx(x) vary sinusoidally along x-axis. Instead of taking the peak values for both electric and magnetic fields, let’s now consider an averaged value for one of these two fields. Let’s keep the definition of voltage as the line integral of maximum electric field:

Vm=0bEy0dy=Ey0b							(13.1.35)

Let’s find the average value of Hx over the range x=0 to x=a:

Hav=1a0aHx0Sin(πxa)dx=2πHx0					(13.1.36)

Using this value for finding total z-directed effective current we get

Ie=2aHxoπ								(13.1.37)

The resulting V-I based characteristic impedance comes out as

ZcV,I=VmIe=πbEyo2aHxo=πb2aZw						(13.1.38)

However the product of voltage and current is still not giving the true power:

P=12VmIe=1πabEy0Hx0							(13.1.39)

This is also different from the power expression given in Eq. 13.1.23.

True Power-Average Current Approach:

The average effective current given in Eqs. 1.13.29-30 has a reasonable and logical definition. Therefore we may keep this definition and re-define the voltage in such a way that the total power will be correctly given by the product of voltage and current [Ragan]:

Using the expression

Ie=2aHxoπ								(13.1.40)

in the definition

P=12VeIe=ab4EyoHxo						(13.1.41)

we can solve the effective voltage as

Ve=πbEyo4								(13.1.42)

This approach gives a characteristic impedance

Zc=VeIe=π2b8aZw							(13.1.43)

- Since this definition is based on true power and logical total longitudinal current, power can be calculated by the formula

P=12VeIe=12ZcIe2						    	(13.1.44)

True Power-Maximum voltage Approach:

In some applications the definition of voltage based on maximum electric field may have an advantage. That is,

Vm=0bEy0dy=Ey0b							(13.1.45)

In such cases if also true power is desired than an effective current Ie can be found by using this expression in the definition

P=12VeIe=ab4EyoHxo						(13.1.46)

giving the effective current as

Ie=aHxo2								(13.1.47)

This approach gives the characteristic impedance as

Zc=VmIe=2EyobHxoa=2baZw						(13.1.48)

Then power can be calculated by the formula

P=12VmIe=12Vm2Zc							(13.1.49)

Note that all the expressions for the characteristic impedance described above can be put into the form

Zc=Zc∞1−(fcf)2							(13.1.50)

where Zc is the frequency independent part of the expressions for Zc when frequency goes to infinity. It is seen that all of these definitions involve the factor b/a and Zw. Therefore all have the same frequency dependent part.

When different waveguide pieces are connected to each other, then any one of these definitions can be used conveniently, giving the reflections at the junctions correctly (excepting the discontinuity reactances !). Only when true power calculations are needed then we have to use one of the formulations involving true power. Controversy starts when waveguide pieces are brought together with other lumped and TEM mode distributed elements or when ridges or fins of various sorts are placed into the guide. In such applications validity of the assumptions used have to be checked. In any case, because of using artificial voltage and/or current definitions and because of various discontinuities involved in the waveguides, the problem should be treated as an electromagnetic boundary value problem with the solutions found by using the above mentioned definitions and assumptions should be used just as a starting point for a search procedure for the actual solutions.

13. 2. RIDGED AND FINNED WAVEGUIDES

- Ridge waveguides are among useful elements for filters and for placement of various electronic components into a waveguide. They offer some advantages over the uniform waveguides in some applications due to their wide spurious free bandwidth and lower characteristic impedances compared to rectangular waveguides.

- Figure 13.2.a shows the conventions used for the dimensions of rectangular, single ridged, double ridged and finned waveguides. Here, a and b are the usual width and height of the waveguide, d is the separation (gap) between the ridges in double ridged case and it is the distance between the top of the ridge and top wall in the case of single ridged waveguide. s is the thickness of the ridge.

- Figure 13.2. Rectangular, single ridged, double ridged and finned waveguides.

- The basic assumption used in the treatment of such waveguides is that only TE10 mode propagates while the other modes excited because of the discontinuity created by the ridges are all treated as evanescent and taken into account by approximating their effect as a lumped capacitor. The basic effect of the ridges is to lower the cutoff frequency of the waveguide. The characteristic impedance is also reduced considerably compared to the nonridged version. For both single and double ridged waveguide the cutoff frequency is given by the following expression [W.J.R. Hoefer, M. N. Burton, “Closed form expressions for the parameters of Finned and Ridged waveguides,” IEEE MTT-30, No. 12, pp.2190-2194, Dec. 1982]:

- fc=cμrεr2(a−s)1+(4π)1+0.2b(a−s)ba−sLncsc(πd2b	(13.2.1)

- On Filpro this formula is used to calculate fc for rectangular, single ridged, double ridged and finned waveguides. For uniform rectangular waveguides, substituting d=b and s=0, this formula reduces to the classical expression:

- fc=c2aμrεr								(13.2.2)

- When the ridge gap d is small enough it is logical to assume that the wave in this region is a quasi-TEM wave. This assumption tempts us to use the definition of voltage in terms of the maximum electric field. That is,

- Vm=Ey0d								(13.2.3)

- So, the Maximum Voltage-Effective Current approach may be used. In that case effective total current need be calculated by integrating H-field along the contours of the ridges also [A.K. Sharma, W. J. R. Hoefer, “Empirical expressions for finline design”, IEEE, MTT-31, No. 4, pp. 350-356, April 1983].

- We may also use True Power-Maximum Voltage definition for the characteristic impedance given in Eq. 13.1.34. This approach necessitates finding the true power expression in the ridged cross-section.

In Filpro only the True Power-Maximum Voltage based characteristic impedance is used to characterize both rectangular waveguides and ridged waveguides. It is obtained by finding stored energy per unit length using transverse resonance method as follows:

Zc∞=μrεr(120π2bfcc)cos2α1Sin2α22α2+Sin2α2+bd2α1−Sin2α1		(13.2.4)

where

α1=πsfcc   and    α2=π(a−s)fcc					(13.2.5)

with c being the velocity of light. This is the expression for the impedance when frequency goes to infinity. The impedance at the frequency of interest is then calculated as follows:

Zc=μrεrZc∞1−(fcf)2						(13.2.6)

For nonridged rectangular waveguides using  b=d and s=0 we get

Zc∞=μrεr240πba	ohms						(13.2.7)

and hence

Zc=μrεr240πba1−(fcf)2	ohms					(13.2.8)

Finned waveguides are treated as the special cases by taking the thickness  s of the ridge as zero.

Remarks: From the descriptions given in this and the previous sections, the following points may be summarized on propagation characteristics of waveguides:

- Propagation constant  is zero at cutoff frequency f=fc, real for f>fc and imaginary for f<fc.

- ZC is infinite at cutoff frequency f=fc, real for f>fc  and imaginary for f<fc.

- Wave propagation takes place only when  and Zc are real, that is only for f>fc. Electromagnetic waves of a certain mode in a waveguide can propagate or decay depending on whether its frequency is above or below the cutoff frequency fc. Therefore cutoff frequency of a mode in a given waveguide is a critical parameter. fc is set by the physical cross-sectional dimensions and permittivity.

- For f<fc, the waves attenuate exponentially (evanescent waveguide modes). The purely reactive nature of this impedance indicates that the decaying waves store energy within the decay region and hence can be modeled as a purely reactive circuit section that can be used as part of the filters (classified as Evanescent Waveguide filters).

- Thus, a waveguide piece acts as a highpass filter with cutoff frequency around f=fc. In the passband (f>fc) the waveguide is similar to a TEM mode transmission line but with a geometry dependent wavelength and frequency dependent, artificially defined characteristic impedance. The artificial characteristic impedance definition necessitates the use of artificial source and load resistances as well for proper accounting of transfer of maximum power between source and load. Since the characteristic impedance is frequency dependent it is not possible to have a perfectly matched condition over wide frequency bands by placing real source and load impedances. Fortunately since waveguides are used in limited frequency ranges set by their cross-sectional dimensions, the terminations can be optimized for a good return loss by trimming the positions and geometry of the feed probes or loops.

- Care is needed when the filter involves mixed elements like inductors, capacitors, constant reactances and inverters defined within the waveguide. The definitions of impedances of such elements must be consistent with the definition used for the waveguide characteristic impedance.

- Distributed element nature of the waveguide pieces leads to some sort of periodicity for f>fc which leads to spurious passbands in filters.

13. 4.  WAVEGUIDES  AS  RESONATORS

- When operated above cutoff frequency, waveguide or TEM mode transmission line (TLine) pieces with proper terminations on both ports can be used to simulate series or parallel LC resonators. In order to clarify the link between lumped element LC resonators and waveguide and TLine resonators, let’s remember the basics of LC resonators.

Lumped element LC resonators:

Figures 13.3.a shows series LC resonator simulated on Filpro. Its resonance frequency is set at f0=12πLC=3000 MHz. In filtering applications we make use of the resonance characteristics of a resonator. That is, we are interested in its behavior in its passband around the resonance frequency and in its attenuation characteristics in the stopband. Since this behavior is set by the inductor L and capacitor C together with the source and load resistors, a resonator must be treated together with its terminations.  Indeed when we change RS and RL, the filtering characteristics change heavily, as seen in the figure. In order to have a narrow passband response, a series resonator likes to have small impedances as its terminations.  It is clear that the set of RS-L-C-RL parameters must have a certain relation to form a response with targeted pass bandwidth and stopband attenuation. This relation is provided by the concept of Loaded Quality factor (QL) or External Quality factor (Qe) of the resonator as follows.

- Any loss in a series LC resonator is represented as a small series resistor, Rsr. The unloaded quality factor Qu of such a resonator is defined as

Qu=woLsrRsr

- If Rsr=0 then the unloaded quality factor becomes infinite. Finite unloaded Q of the resonators are responsible for the passband insertion losses.

- On the other hand, the termination resistors RS and RL of filters together with the L and C elements set the pass bandwidth of the filter. Consider the first (or last) series resonator L1-C1 of a filter formed by resonators. This LC resonator appears to be in series with the source resistor RS. Thus, for this resonator, the source resistor may be treated as loss resistor in series with L1 and C1 and we may define the Loaded or External quality factor Qe as

Qe=w0L1RS

In the example of Figure 13.3.a, both RS and RL appear to be in series with the LC resonator. Therefore the external quality factor of this resonator is

Qe=w0L12RS=28.8

In Figure 13.3.b, a practical measure of the external quality factor is shown. The frequencies where insertion loss is 3.01 dB (half power points) are denoted as f1 and f2. For narrow/medium pass bandwidths the external quality factor can also be calculated as

Qe=f0f2−f1=3000104=28.8

This graphical approach is useful especially when the lumped inductor or capacitor values are difficult to measure or define.

Similar definitions apply for parallel resonators. An example is shown in Figure 13.3.c. A parallel LC resonator would like to see high terminations on both sides to display its resonance characteristics explicitly. Indeed for RS=RL=50 ohms, almost no resonance behavior is seen while for RS=RL=50000 a sharp resonance curve is displayed. The unloaded quality factor of a parallel LC resonator is defined as

Qu=w0CpRp

- where Cp is the shunt capacitor and Rp is a parallel resistor representing the losses in the resonator.

For the resonators adjacent to source (or load), the source resistor appears as a parallel resistor and it is used to define the external quality factor of the filter as

Qe=w0CpRS

The same practical measurement formula

Qe=f0f2−f1

is also valid for parallel resonators.

Figure 13.3. Lumped element series and parallel LC resonators.

Transmission Line Resonators:

A transmission lie piece of half wavelength long (or multiples) at a selected frequency fo (usually passband center) can act as a series resonator if it is terminated in SC at both ends. The SC terminations create a voltage standing wave with its peak in the middle of the line while the associated current standing wave will have peaks at the terminations (Figure 13.4.a). However in most filtering applications this resonator will be coupled to neighboring elements, hence will see nonzero impedances at its ends. Even in such cases, if the terminating impedances are smaller than its characteristic impedance Zc then there will still be a standing wave over the line with voltage minimums at terminations and voltage maximum at the midpoint of the line (Figure 13.4.b). Comparison of responses of a series LC resonator and a half wavelength (or multiples) TLine resonator with Zc>RS, RL shows that it is possible to  simulate a series LC resonator with such TLine resonators over limited frequency bands. Figure 13.4.c shows the commands on Filpro for conversion of a series LC resonator into a half wavelength and full wavelength TLine resonators.

Figure 13.4. Using half and full wavelength Tlines as series LC resonators.

Similarly, a transmission lie piece of half wavelength long (or multiples) at a selected frequency fo can act as a parallel resonator if it is terminated in OC at both ends. The OC terminations create a voltage standing wave with its minimum in the middle of the line and maxima at the ends of the line while the associated current standing wave will have minima the terminations and maximum at midpoint (Figure 13.5.a). However in most filtering applications this resonator will be coupled to neighboring elements, hence will see finite impedances at its ends. Even in such cases, if the terminating impedances are larger than its characteristic impedance Zc then there will still be a standing wave over the line with current minima at terminations and voltage maximum at the midpoint of the line (Figure 13.5.b). Comparison of responses of a parallel LC resonator and a half wavelength (or multiples) TLine resonator with Zc<RS, RL shows that it is possible to simulate a series LC resonator with such TLine resonators over limited frequency bands. Figure 13.5.c shows the commands on Filpro for conversion of a parallel LC resonator into a half wavelength and full wavelength TLine resonators.

Figure 13.5. Using half and full wavelength TLine resonators as parallel LC resonator.

In Figure 13.6.a, insertion loss of a transmission line piece with characteristic impedance Zc=50 ohms is shown with terminations RS=RL=5 ohms, thus simulating a series resonator. The first thing we notice is the periodic behavior of the response. Its length is selected as 50 mm, half wavelength at fo=3000 MHz, so that its first resonance is set at fo=3000 MHz. The higher order resonance frequencies form spurious pass bands at harmonic frequencies of 2fo, 3fo, etc. As Zc is increased the resonance curves around resonance frequencies become sharper. In Figure 13.4.b, insertion loss of a transmission line piece with characteristic impedance Zc=50 ohms is shown with terminations RS=RL=500 ohms, thus simulating a parallel resonator. The periodic behavior of the response is the same as the series resonator case. Its length is the same, 50 mm, half wavelength at fo=3000 MHz. In this case as Zc is decreased the resonance curves around resonance frequencies become sharper. Thus, it is clear that the same transmission line may be used both as a series or parallel resonator by using proper terminations. That is, in order to understand whether a transmission line is used as a series or parallel resonator, we have to inspect its terminations. In filtering applications the harmonic passbands form a limitation for the upper stopband.  Therefore special precautions are taken to make them ineffective.

Figure 13.6. Transmission line pieces simulating series and parallel resonators.

- Waveguide Resonators:

Waveguide pieces with half wavelength or multiples of half wavelength long can be used to simulate series and parallel resonators in the same way as the transmission line resonators. That is, in order that a half wavelength WG operate like a series LC resonator the impedances seen at its ports must be less than the characteristic impedance of the waveguide. If the opposite is true, that is, if the impedances terminating the WG are higher than the characteristic impedance of the WG then it behaves like a parallel resonator.

On Filpro, half and full wavelength WG resonators can be obtained by transformations from half and full wavelength TLine resonators as shown in Figure 13.7. This command, together with commands for conversion of LC resonators into TLine resonator described in the previous subsection, enables to form WG filters by starting  from lumped element LC filters.

Figure 13.7. Conversion of a TLine piece into approximately equivalent waveguide piece.

In Figure 13.8.a, a uniform (no ridge) waveguide  piece of width a=72.136 mm, height b=34.036 mm, r = r = 1 and length 69.368 mm (calculated to be half wavelength at fo=3000 MHz) is shown. Its characteristic impedance is Zc=494 ohms at f0=3000 MHz. Cutoff frequency is fc=2079.4 MHz. Insertion loss responses are shown for the terminations set lower than Zc.  The first resonance is around f0=3000 MHz. The higher order resonance frequencies are shifted away from the harmonics of f0. Figure 13.7.b shows the responses for terminations set above Zc. Note the appearance of a resonance frequency below f0=3000 MHz.

Although a similarity exists between transmission line and waveguide resonators, there are some major differences making waveguides more complex to handle:

- In transmission lines propagation constant  is a linear function of frequency leading to harmonic behavior of the spurious passbands. In waveguides  is a complex function of frequency. This leads to shifts in the higher order resonance frequencies of waveguide away from the multiples (harmonics) of the first resonance frequency.

- Characteristic impedance of waveguides is frequency dependent while it is constant in transmission lines.

- Transmission lines have no cutoff frequency while waveguides can propagate waves only above a cutoff frequency.

- A transmission line can be characterized only by its impedance, length and dielectric constant while a waveguide has many parameters for proper characterization. The width a, height b, dielectric constant, length l and for ridged WG’s, ridge number, ridge gap d and ridge width s. The cross-sectional parameters, a, b, d, s and dielectric constant are needed to set the characteristic impedance while the length l sets the resonance frequencies.

- Finally, another important difference is that besides the higher order resonance frequencies, the passbands of waveguides are also limited by the higher order TEmn and TMmn modes.

Figure 13.8. Waveguide pieces as series and parallel resonators.

13. 4. INTERRELATIONS AMONG THE PARAMETERS OF WAVEGUIDES

Since the spurious passbands and higher order modes form important limitations for waveguide filters, let’s inspect this problem in detail on a rectangular (no ridge) waveguide piece with r=1 operating on TE10 mode. Similar conditions apply for ridged waveguides, but some of the analytical derivations are tedious.

The resonance frequencies of a waveguide piece of width a, height b and length l can be found using Eq. 13.1.12. However it is more instructive to use the following approach:

Using Eq. 13.1.16, at resonance frequencies fk the electrical length l satisfies the following general relation

β(fk)l=lπa(fkfc)2−1=kπ	     k = 1, 2, . .				(13.4.1)

At the fundamental resonance  frequency f1  (k=1) we have

β(f1)l=πla(f1fc)2−1=π						(13.4.2)

So, given l and a we can find the cutoff frequency fc and hence the first resonance frequency f1 as

fc=c2a									(13.4.3)

f1=c2(1a2)+(1l2)							(13.4.4)

Or, given the desired first resonance frequency f1 and width a (hence fc), the length l can be found as

l=πβ(f1)=a(f1fc)2−1							(13.4.5)

In usual filtering applications the first few resonance frequencies nearest to the passband have to be taken into account. The usual problematic frequencies are the second resonance frequency f2, the third resonance frequency f3 and the cutoff frequencies of the nearest higher order modes like TE20 and TE30. These frequencies are related to each other through waveguide equations. Since they may be useful in setting the physical dimensions of the waveguide resonators the relations among these frequencies are of special interest in designing a waveguide filter [12].

The first, second and third resonance frequencies f1, f2 and f3 can be calculated using 13.4.1 as follows:

f1=c2(1a2)+(1l2)=fc1+(al)2					(13.4.6)

f2=c2(1a2)+(2l2)=fc1+4(al)2				(13.4.7)

f3=c2(1a2)+(3l2)=fc1+9(al)2					(13.4.8)

Alternatively, using 13.4.1, f2 can also be found in terms of f1 and fc as  follows:

β(f2)l=lπa(f2fc)2−1=2π						(13.4.9)

In this equation using  π=β(f1)l we get

β(f2)l=lπa(f2fc)2−1=2π=2β(f1)l

Using also 13.4.2

lπa(f2fc)2−1=2lπa(f1fc)2−1

we get the frequency f2 in terms of f1 and fc:

f2=4f12−3fc2								(13.4.10)

Using  fc=c2a ,   f2 can also be expressed in the form

f2=4f12−3(c2a)2							(13.4.11)

from which the width a can be solved in terms of f2 and f1 as

a=c234f12−f22								(13.4.12)

This result can be used in setting the width a to for a given f1 and desired location of f2.

Once f1 and f2 are set then the third resonance frequency f3 can be found in terms of f2 and f1 using 13.4.1 with k=3 as follows:

β(f3)l=lπa(f3fc)2−1=3π=3β(f1)l				(13.4.13)

f3=8f22−5f123								(13.4.14)

- The expressions for the other resonance frequencies can be found using the same approach.

- The cutoff frequencies of TE02 and TE03 modes are

- fc20=2fc=ca								(13.4.15)

- fc30=3fc=3c2a								(13.4.16)

- We wouldn’t like to see these modes traveling in our stopband of interest. The main upper stopband is limited to the range fp2<f<f2 where fp2 is the upper passband edge frequency and f2 is the second resonance frequency. The upper limit of the stopband may also be set by the cutoff frequencies of TE02 and TE03 modes, as well as f2. Therefore let’s inspect the location of f2 in a closer manner. It can be adjusted within a limited range as follows:

- In most waveguide filter applications the resonators are of half wavelength long at passband center f0. Thus the first resonance frequency f1 coincides with the passband center: f0=f1. In such filters if the resonators are excited and loaded symmetrically with respect to longitudinal symmetry axis of the waveguide (x=a/2 line) then TE02 mode wouldn’t appear, leaving us with the problem of dealing with the next higher order mode TE03. The cutoff frequency of TE30 mode is  3fc10 for uniform waveguides and less than 3fc10 for ridged waveguides. The second harmonic 2f0 of the main signal at passband center frequency f0 may fall into the passband of the TE30 mode, hence that mode may be excited. Therefore the cutoff frequency 3fc of TE30 mode must be selected to be greater than 2f0. This condition may be used to set an upper limit for the width a. For uniform waveguides we can do the following analysis:

fc30=3fc=3c2a>2f0=2f1						(13.4.17)

giving

amax=0.75cf1=0.75cf0							(13.4.18)

Using this condition in 13.4.11 we can define an upper limit for f2:

f2max=4f02−3fc2=1.633f0						(13.4.19)

The lower limit of f2 is set as follows: The third resonance frequency f3 must be greater than f2max:

f3min>f2max								(13.4.20)

Using 13.4.14 for f3,

f3min=8f2min2−5f023>f2max=1.633f0				(13.4.21)

we get

f2min=1.275f0								(13.4.22)

This yields the minimum value for the width a:

amin=0.56cf0								(13.4.23)

So, given the passband center f0, if half wavelength resonators are used then the first resonance frequency is f1=f0 and the second resonance frequency f2 can be set in the range

1.275f0<f2<1.633f0							(13.4.24)

corresponding to selection of the width a in the range

0.56cf0<a<0.75cf0							(13.4.25)

found in terms of f0(=f1) and f2 through

a=c234f02−f22								(13.4.26)

EXAMPLE 13.1 Simulation of a uniform rectangular waveguide piece on Filpro

Let’s analyze an air dielectric rectangular waveguide piece (r=1) on Filpro. The cross-sectional dimensions are width a=72.136 mm (2.84 inches), and height b=34.036 mm (1.34 inches) (WR-284 guide), and length l will be set by such that we will get a good match for example at about f0=3000 MHz. In this example we will make all calculations for fc, length l, resonance frequencies, etc. manually. In the forthcoming filter design examples all such calculations will be done by Filpro automatically.

The cutoff frequency for TE10 mode is calculated from Eq. 13.1.14 as

fc=fc10=c2aμrεr=c2a=2079.4 MHz

The limiting value of V-I based characteristic impedance for f>>fc is

Zc∞=VI=240πbaεr1−(fcf)2≈240πba≈355.76ohms

This impedance increases as the frequency decreases towards the cutoff. At f0=3000 MHz it becomes

Zc=VI=240πbaεr1−(fcf)2≈240πb0.72a≈494	ohms

- It is seen that the characteristic impedance varies with frequency appreciably. Therefore proper matching precautions must be used at ports which depends on the frequency of interest.

In order to have the fundamental (first) resonance at f1=3000 MHz, length of the guide must be

l=a(f1fc)2−1=72.1361.08145=69.368 mm

The command set to simulate this waveguide piece on Filpro is shown in Figure 13.9.

- Design / By Inserting Elements command is invoked. This command asks the source and load impedances. Since we are interested in the frequency range about f1=3000 MHz, we type in RS=RL=494 ohms.

- After setting RS=RL=494 ohms, Insert command will appear. Selecting the options Rectangular WG Elements and then the option rectWG, waveguide dimensions are asked. Setting width a=72.136 mm, height b=34.036 mm, RidgeHeight d=b=34.036 mm, ridge width s=0  Ridge#=0 (no ridge) and length=69.368 mm, the circuit shown in Figure 13.9.a will appear.

- Inspection of the insertion loss response clearly shows the propagation and cutoff regions together with the resonance frequencies. Passband starts from about 2400 MHz.

- Return loss response shows that matching is better about f1=f0=3000 MHz compared to other frequencies because we have set RS=RL=Zc(f0=3000 MHz)=494 ohms.

- We are also wondering about the response at higher frequencies, just to see its effect. Using Change command we can change RS=RL=355.76 ohms (Figure 13.9.b). Return loss plot shows that matching gets better towards higher frequencies. This is because RS=RL= 355.76 ohms holds for f>>fc. In practice the usual source-load impedances are 50 ohms. Therefore transitions are needed from waveguides to 50 ohms. This can be provided by trimming the position and dimensions of the input-output coupling probes or loops.

- As a conclusion, in dealing with waveguide elements on Filpro we have to keep in mind the frequency dependence of characteristic impedance presented by the waveguide.

- The series resonance frequencies are seen as the notches in return loss response. The dimensions a=72.136 mm and l=69.368 mm sets the first resonance frequency as

f1=c2(1a2)+(1l2)=3000 MHz

- The second and third resonance frequencies f2 and f3 are calculated as

f2=4f12−3fc2= 4800 MHz     and       f3=8f22−5f123= 6800 MHz

which are confirmed by the return loss plots.

- Let’s inspect the effects of changing the width a while keeping the first resonance frequency f1=3000 MHz and height b the same. This action will shift location of second resonance frequency f2.

- Let’s first try the maximum possible width amax. Given f1 we can calculate amax using Eq. 13.4.18 as

amax=0.75cf1=0.753∗10113∗109=75 mm

giving maximum f2 through 13.4.19 as

f2max=1.633f1=4899 MHz

The cutoff frequency is

fc=c2a=3∗1011150=2000  MHz

Length l of the waveguide is then calculated from 13.4.5 as

l=a(f1fc)2−1=751.25=67.084 mm

The characteristic impedance is

Zc=VI=240πba1−(fcf1)2≈459 ohms

- Response of this waveguide piece with RS=RL=459 ohms is shown in Figure 13.9.c. f2 came out to be 4890 MHz while the length of resonator is about 67 mm as calculated.

- For minimum width case, using 13.4.22 and 23  we get

amin=0.56cf1=0.563∗10113∗109=56mm,       f2min=1.275f1=3825 MHz.

The cutoff frequency, length and characteristic impedances are then calculated as

fc=c2a=3∗1011112=2678 MHz,          l=a(f1fc)2−1=560.2549=110.91 mm

Z=VI=240πba1−(fcf0)2≈789 ohms

Response of this waveguide is shown in Figure 13.6.d.  f2 came out to be about 3800 MHz. Length of the resonator is 110.9 mm. f2 (and also the others) is too close to f0.  Thus, if the second resonance frequency f2 must be kept far away from f1, then we should select the maximum possible width amax. This would also lead to shortest resonator length l. These facts must be taken into consideration in designing waveguide filters. So, the original choice of the standard waveguide WR284 for the first resonance of f1=3000 MHz is a good choice for keeping f2 far from f1, because its width a=72.136 mm is close to amax=75 mm.

Figure 13.9. Displaying a waveguide piece on Filpro.

Figure 13.9. Displaying a waveguide piece on Filpro.

EXAMPLE 13. 2. Waveguides as series resonators

- Just as a preparation for designing waveguide filters and also to inspect the effects of ridges, let’s design a lumped element bandpass filter of degree n=1 and transform its LC resonator  into TEM mode transmission line and waveguide resonators with and without ridges.

- Design command set is shown in Figure 13.10. The specifications are as follows:

- PBRipple=0.01 dB, fp1=2975 MHz, fp2=3025 MHz, Degree n=1.

- Design / By Built in LC Prototype approach is used to give the circuit shown in Figure 13.10.a. It has a series LC resonator as the filtering element. It is saved as F1a.fil.

- The series LC resonator is converted into a transmission line resonator which is half wavelength long at passband center f0=3000 MHz. This conversion is done by placing the cursor under the LC element and applying the command Replace by Tline / Half Wavelength Tline. The resulting circuit is saved as F1b.fil. The wideband insertion loss and close in return loss response of the resulting circuit is shown together with the original response. We make the following observations:

  - There are periodic harmonic passbands at multiples of f0=3000 MHz. These passbands are usually undesirable, therefore termed as spurious passbands.

  - Within the main passband, the response of the TLine resonator follows the response of LC resonator closely. Response of the TLine resonator start to deviate from the LC resonator outside the passband.

- In Figure 13.10.b, the LC resonator is converted into a full wavelength transmission line resonator. We note that characteristic impedance of the full wave resonator is halved compared to the half wave resonator while its length is doubled. Inspection of the return loss indicates that response of the full wavelength TLine resonator deviates slightly from that of the LC resonator. This is due to approximations involved. Although in most applications it is not necessary, return loss response can be corrected by tuning the impedance of the TLine, as described in Figure 13.10.c. Doubling of the length of TLine introduces a spurious passband below the main passband as shown in the figure, termed as subharmonic passband. Therefore in filters employing full wavelength resonators, in addition to the harmonic passbands, we have to deal with subharmonic passband also.

- In Figures 13.10.d and e, the half wavelength TLine resonator is converted into a half wavelength rectangular (no ridge) waveguide resonator. This is done in two stages, as described in the Figures.

  - In the first stage the TLine impedance is scaled properly so that it will be compatible with the waveguide selected for realization of the resonator. This is done placing the cursor under the TLine and applying the command Waveguide Oprs / Scale Circuit (TLine) for the selected WG: a=72.136 mm, b=34.036 mm, d=34.036 mm, s=0, Rdg#=0.  When no ridge is desired we have to type in d=b, s=0 and Rdg#=0 always. In applying this command Filpro displays some of the useful information about the waveguide to be used. These are the TE10 mode cutoff frequency fc10, TE30 mode cutoff frequency fc30, the first resonance frequency f1 which coincides with the passband center fo in this example, the nearest three higher order resonance frequencies f2, f3 and f4, the characteristic impedance Zc(fo) at passband center and the guided wavelength  at passband center. It may be useful to note down these parameters. Filpro asks whether to continue scaling. Invoking Yes option, this command inserts ideal inverters on both sides of the TLine to scale its impedance to the proper level (Figure 13.10.d). Also the source and load impedances are scaled to be equal to the characteristic impedance Zc(fo) of the waveguide.

  - In the second stage, with cursor under the scaled TLine, the command Waveguide Oprs / TLine-to-WG: a=72.136, b=34.016 mm, d=34.036, s=0,..  is applied to replace the half wavelength TLine with its waveguide equivalent (Figure 13.10.e). Length of the waveguide automatically comes out as half of the guided wavelength at f0.

- In Figure 13.10.f, the half wavelength TLine resonator is now converted into a half wavelength double ridged waveguide resonator. This is done by applying the command Waveguide Oprs / TL-to-WG: a=72.136, b=34.016 mm, d=20 mm, s=20 mm, Rdg#=2. Here, d (gap between ridges) and s (width of ridges) are selected to be 20 mm, arbitrarily. Conversion is done in two stages, as in the previous case. That is, first the circuit with TLine is scaled and then the TLine is replaced by its waveguide equivalent.

- In Figure 13.10.g, the TLine resonator is converted into a half wavelength single ridged waveguide resonator. This is done by applying the command Waveguide Oprs / TL-to-WG: a=72.136, b=34.016 mm, d=20 mm, s=20 mm, Rdg#=1.

- In Figure 13.10.h, the TLine resonator is converted into a double finned waveguide resonator. Fins are just special cases of ridges when the ridge width s tends to zero. So this conversion is done by applying the command Waveguide Oprs / TL-to-WG: a=72.136, b=34.016 mm, d=20 mm, s=0.1 mm, Rdg#=2. Here, s is taken arbitrarily as 0.1 mm. It may also be taken as s=0. That is, we treat a ridge with small width as a fin.

- Finally, in Figure 13.10.i the TLine resonator is converted into a single finned waveguide resonator. This is done by applying the command Waveguide Oprs / TL-to-WG: a=72.136, b=34.016 mm, d=20 mm, s=0.1 mm, Rdg#=1.

- In Figure 13.10.k and m insertion loss and return loss responses of all five WG resonators are shown in overplot form. We note down that different WG resonators are simulating the LC resonator well in the passband region. Differences between the ridged, finned and uniform waveguides appear in the shifts in the spurious passband locations.

- The most distinctive features of different WG resonators lie in their cutoff frequencies which are not clear in these plots. In order to expose them we can delete the inverters on the two sides of the WG pieces to as shown in Figures 13.10.n-to-s to form only WG circuits with matched terminations (at passband center). Figure 13.10.t shows the insertion loss and return loss variations for the different WG sections.

- In Figure 13.11, a more systematic search is made on the effects of varying the ridge gap d while keeping ridge width s constant. Variation of cutoff frequency with d is shown in Figure 13.11.b while variation of the higher order resonance frequencies is shown in Figure 13.11.c.

- In Figure 13.12 the ridge gap d is kept constant and ridge width s is varied in small steps.  For values between s=0 and s=30 mm, insertion loss in the cutoff region is little effected, showing that when s is less than half of waveguide width a, it has almost no effect on cutoff frequency, as seen in Figure 13.12.b. However when s becomes greater than 30 mm it starts to be highly effective. This is to be expected because when s approaches a the effective height b of the WG is heavily changed. Figure 13.12.c shows the variation of the higher order resonance frequencies with s. It is seen that s is not as effective as d on these parameters also.

- Finally, in Table-13.1 and Table-13.2, the effects of d and s on characteristic impedance are displayed. As the ridge gap d is decreased characteristic impedance is decreased uniformly. As the ridge width s is increased the impedance first decreases uniformly and when s is about half the guide width a, it reaches to a minimum and then starts to increase.

- Figure 13.10. Waveguide pieces as half wavelength series resonators.

- Figure 13.10. Waveguide pieces as half wavelength series resonators.

- Figure 13.10. Waveguide pieces as half wavelength series resonators.

- Figure 13.10. Waveguide pieces as half wavelength series resonators.

- Figure 13.10. Waveguide pieces as half wavelength series resonators.

- Figure 13.10. Waveguide pieces as half wavelength series resonators.

- Figure 13.10. Waveguide pieces as half wavelength series resonators.

- Figure 13.10. Waveguide pieces as half wavelength series resonators.

- Figure 13.11. Effects of ridge gap d on cutoff frequency and higher order resonance frequencies.

- Figure 13.11. Effects of ridge gap d on cutoff frequency and higher order resonance frequencies.

- Figure 13.12. Effects of ridge width  s on cutoff frequency.

- Figure 13.12. Effects of the ridge width  s on cutoff frequency and higher order resonance frequencies.

- Figure 13.13. Effects of the ridge width  s on cutoff frequency and higher order resonance frequencies.

13. 5. QUARTER WAVELENGTH IMPEDANCE MATCHING TRANSITIONS BETWEEN RIDGED AND NONRIDGED WAVEGUIDES

- In most cases transitions are needed between ridged and nonridged waveguides for impedance matching. Such transitions can be made by using quarter wavelength WG pieces with different ridge gaps.

Let’s describe the problem on an example.

# EXAMPLE 13. 3.  Transition between uniform WG and ridged WG

- Let’s consider a uniform waveguide designed to operate in the frequency range of 9000 MHz and 10000 MHz. Its width and height are a=20 mm, b=10 mm. It will be matched to a ridged WG with following dimensions:

- a=20 mm, b=10 mm, d=2.5 mm, s=4 mm.

- Matching will be done by inserting quarter wavelength WG transformers with different ridge gaps (d). We may insert one, two or more quarter wavelength matching sections depending on the matching bandwidth and passband return loss level. Ridge gaps will be used to adjust characteristic impedances of the quarter wavelength transformer WG sections.  Since analytical approach for finding proper ridge gaps is complex, we will use iterative trial-error, tuning or optimization approach. In order to start trials with reasonable ridge levels, let’s first design WG pieces with different ridge gaps.

- In Figure 13.14, a lumped element BPF of degree n=1 is designed with passband edges fp1=9000 MHz and fp2=10000 MHz, PBRipple=0.01 dB, using Design / By Built in LC Prototype approach. The resulting filter will be a single series LC resonator between RS=RL=50 ohms terminations. It is saved as WG01A.fil (Figure 13.14.a).

- With cursor under the LC resonator, the command Replace by TLine / Half WaveLength TLine / Er=1 is applied to convert the LC resonator into a half wavelength TLine resonator (Figure 13.14.b). It is saved as WG01B.fil. This TLine resonator will be used to design all ridged and nonridged WG pieces.

- Figure 13.14. Design of a TLine resonator.

- In Figure 13.15, the design steps for half wavelength ridged WG pieces with differing ridge gaps are described.

- In Figure 13.15.a, the circuit of WG01B.fil is loaded and with cursor under the TLine, the command WG Oprs / Scale CCt for WG: a=20, b=10, d=2.5, s=4 mm / Scale whole Cct  is applied to scale the TLine to a proper level for realization with the ridged WG with specified parameters. The TLine will be scaled by inserting inverters on both sides as shown in Figure 13.15.b. In applying this command the cutoff frequency, wavelength at passband center, resonance frequencies and characteristic impedances are displayed as an information that may be useful at later stages.

- Then with cursor under the scaled TLine, the command WG Oprs / TLine-to-WG is applied to convert the half wavelength TLine into the WG with specified cross-sectional dimensions and half guided wavelength long at passband center (Figure 13.15.c).

- Then the inverters are deleted to leave only the half wavelength WG with matched terminations which are equal to the characteristic impedance of the WG (Figure 13.15.d). It is saved as RWG1. Figure 13.15.e shows the return loss response. It is seen that the WG has perfect matching in the band of 9000-10000 MHz.

- In Figures 13.15.f-to-i, the same design is repeated for ridge gaps, d=4 mm, 6 mm, 8 mm and 10 mm (no ridge). These designs are needed as initial values for iterative design trials. The resulting circuits are saved as RWG2, RWG3, RWG4, RWG5  respectively.

- In Figure 13.16, the formation of quarter wave WG transformer sections is described.

- First, RWG1 is loaded. Then with cursor under RL, the command Append Filter: WG5 is applied to append RWG5 to the right of RWG1 (Figure 13.16.a-b).

- The load end termination is changed to those of RWG5 by using Change: RL=613.5 ohms (Figure 13.16.c). The circuit is saved as RWG15. Return loss response shows severe degradation. We have to insert quarter wavelength transformers to improve return loss.

- In Figures 13.16.d-to-h, design of a single quarter wavelength transformer is described. This is done by placing the cursor under RWG5 and applying the command Append Filter: RWG3. This command inserts RWG3 in between RWG1 and RWG5, to be used as matching section (Figure 13.16.d-e).  RWG3 has ridge gap of 6 mm, hence seems to be the best candidate between d=10 mm (no ridge) and d=2.5 mm. However it is a half wavelength WG. In order to form a quarter wavelength WG we apply the command Split / Two Equal and then Delete one of the two pieces (Figure 13.16.f).

- Return loss response is now better. However it can be improved further by tuning both ridge gap and length of the quarter wavelength transformer section. This can be done by applying Tune Element command giving the circuit and response shown in Figure 13.16.g. We got a return loss level of about 20 dB over the band of 9000-10000 MHz. The circuit is saved as RWG135.

- In order to get a better return loss level in that frequency band, let’s try placing three quarter wavelength WG pieces. First RWG15 is loaded (Figure 13.16.h).

- Then with cursor under RWG5, the command Append Filter: RWG2 is applied to insert RWG2 to the left of RWG5. Then with the cursor still being under RWG5, this command is repeated to insert RWG3 and RWG4 giving the circuit of Figure 13.16.i.

- Then the interior half wavelength WG pieces (RWG2, RWG3 and RWG4) are split into two equal parts and one piece is deleted to form three quarter wavelength transformer WG’s (Figure 13.16.k).

- Return loss response shows some improvement. Further improvement can be done by optimizing ridge gaps and lengths of the quarter wavelength transformers. This can be done by using Monte Carlo / Best Case optimization routine. The resulting circuit and return loss response are shown in Figure 13.16.m. More than 30 dB return loss is achieved. We may as well try only two quarter wavelength sections to get about 26 dB return loss level (Exercise?).

- Figure 13.15. Design of half wavelength WG resonators with different ridge gaps.

- Figure 13.15. Design of half wavelength WG resonators with different ridge gaps.

- Figure 13.15. Design of half wavelength WG resonators with different ridge gaps.

- Figure 13.16. Design of quarter wavelength impedance transforming WG sections.

- Figure 13.16. Design of quarter wavelength impedance transforming WG sections.

- Figure 13.16. Design of quarter wavelength impedance transforming WG sections.

13. 6. WAVEGUIDE FILTERS USING HALF WAVELENGTH RESONATORS

Waveguide resonators can be used in the same way as TEM mode transmission line resonators. A half wavelength waveguide piece can be treated as a series connected series resonator or a shunt connected parallel resonator depending on the couplings between adjacent resonators. The classical and easiest approach for designing waveguide filters is to use inverter coupled series resonator filter models as shown in Figure 13.17.a. On Filpro the series LC resonators can be converted into equivalent half wave TEM mode transmission line resonators (Figure 13.17.b). Then the Half Wavelength transmission line resonators are scaled up in impedance to proper levels so that they can be replaced by the equivalent half wavelength waveguide resonators with desired cross-sectional dimensions. The relations between waveguide impedance (Zg) and transmission line impedance (ZTL) are shown in Figure 13.17.b for half wavelength and quarter wavelength cases. The ideal inverters are used for adjustment of transmission line impedances, waveguide impedances and source-load impedances to proper levels so that the above mentioned transformations are possible, leading to the structure shown in Figure 13.17.c.  In this circuit the terminations (RS, RL) are also set to be equal to the guide impedance Zg. Then the transmission line resonators are replaced by their waveguide equivalents (Figure 13.17.d). Next the inverters are replaced by their proper practical equivalents some of which are shown in Figure 13.17.e. It is seen that the practical equivalent circuit sections involve shunt constant reactances. They can be implemented as discontinuities in waveguide like septums, irises or posts as shown in Figures 13.17.f-g and in Figure 13.18. The physical dimensions of these discontinuities can be found in the classical books on this subject [1]-[9]. Such filters can be designed on Filpro using a variety of approaches. Below some typical examples are given.

Figure 13.17. Main design stages of typical waveguide filters.

Figure 13.18. Some waveguide discontinuities that can be used as filter elements.

# EXAMPLE 13. 4. Waveguide filters  with half wave resonators using shunt inductive septums

- Let’s design some waveguide filters using half wavelength WG resonators coupled by shunt inductive septums. The passband information and degree of the filter  are as follows:

PBRipple=0.01 dB, fp1=2975 MHz, fp2=3025 MHz, NR=5 (five resonators- degree 10).

We will target to use the waveguide WR-284 (dimensions a=72.136 mm (2.84 inches), and b=34.036 mm (1.34 inches), operating in TE10 mode. However we will make modifications on this WG to form different resonator types as follows:

- In the first design we will use the same waveguide for all resonators and inspect the spurious passbands.

- In the second design we will use different widths (a) for each resonators to suppress the spurious passbands.

- Finally, in the third design we will use ridged WG resonators to suppress the spurious passbands.

In each case the WG resonators will be coupled to each other using shunt inductive septums.

All these WG filters will be designed by starting from a common transmission line resonator filter whose design steps are shown in Figure 13.19:

- Design / By Prototype with inverters approach is used and Prototype with shunt capacitors and inverters option is selected (Figure 13.19). Prototype degree and PBRipple are typed in. The lowpass prototype will appear on the screen. Then Filter Type command is invoked and Lumped Element and Bandpass options are selected. Then PB and Terminations command is invoked and passband edge frequencies are typed in. The terminations are left as RS=RL=50 ohms. Find Circuit command displays the bandpass filter with series LC resonators coupled by ideal inverters (Figure 13.19.a).

- The command Inverters / All Equal Left-Right / Inductors is applied to equate all the shunt inductors (Figure 13.19.b). The unity transformer at load end is deleted. It should be noted that resonance frequencies of all LC resonators are equal to the passband center f0=3000 MHz .

- All the series LC resonators will be replaced by waveguide pieces which are half (guided) wavelength long at the resonance frequencies of each LC resonator. On Filpro this replacement is carried out in three steps: First the LC resonators are converted into TEM mode half wave transmission lines, then impedances of the transmission lines are scaled up to a proper level for replacement by waveguides with desired dimensions and then the scaled up transmission lines are converted into waveguide pieces, as follows.

- Conversion of the LC resonators into TEM mode transmission line resonators is done by placing the cursor under each resonator one by one and applying the command Replace By TLine / Half Wavelength TLine (Figure 13.19.c). Lengths of the lines are half wavelength at the resonance frequency of the corresponding LC resonators. With these characteristic impedances, the half wavelength transmission lines are equivalent to the lumped series LC resonators over a fairly wide bandwidth. This circuit will be used as the prototype for designing waveguide filters employing waveguide resonators with different parameters, like equal width, different width, ridged, finned, etc. Therefore we save it as PROTWG1.fil.

WG filter with equal width resonators:

In Figure 13.20 design steps of WG filter with all resonators having identical cross-sectional dimensions of a=72.136 and b=34.036 mm.

- The circuit of ProtWG1 is loaded (Figure 13.20.a). The half wavelength transmission lines will be replaced by half wavelength waveguide pieces with desired cross-sectional dimensions. The parameters of the desired waveguide set the characteristic impedance of the corresponding TEM line. Therefore in order to be able to use the desired waveguide dimensions, we should first scale up the impedance level of the half wavelength transmission lines to proper level. This is done by placing the cursor under any transmission line and applying the command Waveguide Operations / Scale Circuit-TL for WG (Figure 13.20.a-b). This command displays a window to type in the width a=72.136 mm and the height b=34.036 mm. Since there is no ridge, we type in d=b=34.036 mm, s=0, Rdg#=0. Dielectric constant Er=1 and the passband center f0=3000 Mz  appear in default manner.

- After typing in these parameters another window appears asking if the whole circuit or only the transmission line pointed by the cursor will be scaled. If all the resonators are going to be realized by the same size waveguide then we prefer the option Scale the whole Circuit.

- Then another window will appear displaying the cutoff frequency (fc=2078 MHz, Guide Wavelength=138.55 mm (at passband center), the first four resonance frequencies f1=3000 MHz, f2=4801 MHz, f3=6816 MHz and f4=8901 MHz, the characteristic impedance of the waveguide at passband center, ZcWG=493 ohms and the characteristic impedance at infinite frequeny Zcinf=356 ohms. Also the question Scale circuit for Selected WG?  is asked. If Yes option is selected then the inverters are inserted to scale the whole circuit to RS=RL=ZcWG=493 ohms to scale the transmission line impedances to a level compatible with the selected WG cross-sectional dimensions (Figure 13.20.b). It is advisable to note down the displayed parameters fc and guide wavelength to be used in calculation of iris dimensions later. The higher order resonance frequencies gives also the locations of the spurious passbands. Here, f1=3000 MHz is the fundamental resonance frequency which is the center of the main passband.

- Next, with cursor under the transmission lines the command Waveguide Operations / Tline-to-WG is applied converting the transmission line pieces into waveguide pieces with the desired width and height (Figure 13.20.c).

- Next, the inverters will be replaced by their proper practical equivalents. This replacement is done by placing the cursor under the inverters and applying the command Waveguide Oprs / Inverter-to-WG. The equivalent circuit of inverters involves consists of a shunt constant reactance in between  two waveguide pieces of negative lengths as seen in Figure 13.20.d. The negative length waveguide pieces are combined with the adjacent positive length waveguides by applying Simplify Circuit command (Figure 13.20.e). The circuit is saved as WG11.fil.

- Conversion of ideal inverters into practical equivalents causes a slight narrowing in bandwidth while return loss level is improved somewhat as shown in Figure 13.20.f.

- The wideband insertion loss response in Figure 13.20.g shows the spurious passbands. The first spurious passband is centered at about f1=4800 MHz (second resonance frequency) set by the width a=72.136 mm and lengths of resonators which vary in the range 62mm-68 mm. The usable stopband is limited by the first spurious passband, necessitating the use of a LPF in cascade or the use of other techniques like using different width waveguide resonators as will be seen in the alternative approaches in this example. In the figure, the shaded region shows the frequency range where TE30 mode can be excited. The second spurious passband around f2=6816 MHz falls into this region.  Any intermodulation product falling into this passband may excite TE30 mode. Therefore this passband should also be taken into consideration.

- The constant reactances can be realized as irises or posts that will be described later.

Figure 13.19. Design of a coupled half wavelength transmission line resonator filter.

Figure 13.20. Design of a half wavelength resonator WG filter with equal width resonators.

Figure 13.20. Design of a half wavelength resonator WG filter with equal width resonators.

WG filter with unequal width resonators:

In the previous design using equal width resonators the spurious passbands are formed because the higher order resonance frequencies of all resonators were coinciding. It is possible to prevent the formation of spurious passbands by shifting the locations of the higher order resonance frequencies of the resonators by assigning different width to each resonator. This will move the higher order resonance frequencies away from each other thus at least suppress the spurious passbands. The approach is described in Figure 13.21.

- The circuit of ProtWG1 is loaded (Figure 13.21.a).

- With cursor under the first TLine resonator, the command WG Oprs / Scale TLine for WG / a=72.136, b=34.036, d=34.036, s=0, Rdg#=0, Er=1, f0=3000 MHz/ Scale TLine Only is applied. This command scales only the TLine pointed by the cursor. In applying this command we note down the displayed higher order resonance frequencies f2=4801 MHz, f3=6816 MHz,  and f4=8901 MHz to compare with the higher order resonance frequencies of the higher order resonators.

- Then with cursor being still under the same TLine (now scaled), the command WG Oprs / TLine-to-WG is applied to convert the scaled half wavelength TLine into half wavelength WG resonator (Figure 13.21.b-c).

- Next the cursor is moved to point the next TLine. Now we will use a different width WG for this resonator. Just as an example, let’s assign a=68 mm for this WG. The command WG Oprs / Scale TLine for WG / a=68, b=34.036, d=34.036,  s=0, .. / Scale TLine only is applied to scale the TLine (Figure 13.21.c-d). We again note down the higher order resonance frequencies, f2=4628 MHz, f3=6490 MHz, and f4=8433 MHz. We would like to spread all these higher order resonance frequencies evenly in the stopband of our interest. If any two of them comes out to be close to each other then a spurious passband will originate there.

- Next, this TLine is converted into WG by applying the command WG Oprs / TLine-to-WG (Figure 13.21.d-e).

- Then the third TLine is replaced with a WG with a=64 mm (Figures 13.21.e-f-g).

- Then the fourth TLine is replaced with a WG of width a=60 mm (Figure 13.21.g-h-i).

- Finally, the fifth Tline is replaced with a WG of width a=55 mm (Figure 13.21.i-k-m).

- Collecting and ordering the resonance frequencies, we have:

3704-4156-4421-4628-4645-5574-5708-6092-6490-6816-7098-7856-8433  MHz.

- We note that there is a crowding about 4600 MHz, hence expect appearance of a suppressed passband there. We will see.

- The last stage is the conversion of inverters into WG+Xsh+WG forms by applying the command WG Oprs / Inverter-to-WG (Figure 13.21.m). Simplify Circuit command combines the negative length WG pieces with their positive length neighbors (Figure 13.21.n). It is saved as WG12.fil.

- Figure 13.21.p shows that return loss level is better than expected at the expense of a slight narrowing in passband.

- Figure 13.21.q shows the wideband insertion loss. We note the disappearance of the spurious passbands we have seen in the design with equal width resonators. They are pushed down by about 40 dB. As we have suspected, the 40 dB stopband suppression level is set by the spikes about 4500-4600 MHz range where several higher order resonance frequencies were gathered (4421-4628-4645 MHz).

- The TE30 mode with lowest cutoff frequency may appear only in the first resonator, for f>fc30=6234 MHz. The other resonators have higher fc30 frequencies.

- Further improvement may be obtained by trimming some of these resonance frequencies. For example the resonance frequencies created by the third, fourth and fifth resonators may be shifted slightly by making their widths a=63 mm (instead of 64 mm), a=59 mm (instead of 60 mm) and a=57 m (instead of 55 mm), giving the circuit of Figure 13.21.r with insertion loss shown in Figure 13.21.s. The spurious passband about 4600 MHz is suppressed a few dB more.

Figure 13.21. Design of WG filter with unequal width resonators.

Figure 13.21. Design of WG filter with unequal width resonators.

Figure 13.21. Design of WG filter with unequal width resonators.

Figure 13.21. Design of WG filter with unequal width resonators.

WG filter with unequal ridge resonators:

- It is also possible to shift the locations of higher order resonance frequencies by using different ridge gaps in the waveguide resonators. Ridges can provide wider separations between resonance frequencies. We can use single or double ridges or fins. Let’s repeat the design using single ridged resonators with different ridge gaps d and same ridge width s:

- The circuit of ProtWG1 is loaded (Figure 13.22.a).

- Let’s take the first and last resonators as nonridged. Hence, with cursor under the first TLine resonator, the command WG Oprs / Scale TLine for WG / a=72.136, b=34.036, d=34.036, s=0, Rdg#=0, Er=1, f0=3000 MHz / Scale TLine only is applied. This command scales the pointed  TLine for the specified nonridged WG. In applying this command we note down the displayed higher order resonance frequencies. Then with cursor being still under the same TLine (now scaled), the command WG Oprs / TLine-to-WG is applied to convert the scaled half wavelength TLine into half wavelength WG resonator (Figure 13.22.b).

- Next the cursor is moved to point the next TLine. Now we will use a ridged WG for this resonator. Just as an example, let’s assign ridge gap of d=20 mm and ridge width s=10 mm for this WG. The command WG Oprs / Scale TLine for WG / a=72.136, b=34.036, d=20,  s=10, Rdg#=1,Er=1,f0=3000 MHz / Scale TLine only is applied to scale the TLine. We again note down the higher order resonance frequencies. Then this TLine is converted into WG by applying the command WG Oprs / TLine-to-WG (Figure 13.22c-d).

- Then the third TLine is replaced with a WG with d=10 mm and s=10 mm (Figures 13.22.e-f-g).

- Then the fourth TLine is replaced with a WG of width d=5 mm and s=10 mm (Figure 13.22.g-h-i).

- Finally, the fifth Tline is replaced with a nonridged WG (Figure 13.22.i-k-m).

- Collecting and ordering the resonance frequencies, we have:

4801-5246-5595-5760-6816-7642-8276-8573-8901-10081-10979-11398 MHz

- We note that there is a crowding about 5300-5800 MHz range, hence expect appearance of a suppressed passband in this region. We also note that compared to the unequal width resonators case where the first spurious passband was located about 4600 MHz, the first spurious passband is about 1000 MHz farther from the passband. Therefore we expect higher suppression of this spurious passband. Let’s see.

- The last stage of the design is the conversion of inverters into WG+Xsh+WG forms by applying the command WG Oprs / Inverter-to-WG (Figure 13.22.m). Each application of this command converts the inverter into a shunt constant reactance with negative length WG pieces on its two sides. Simplify Circuit command combines the negative length WG pieces with their positive length neighbors giving the circuit of Figure 13.22.n. It is saved as WG14.fil.

- Figure 13.22.p shows that return loss level has a slight degradation which can be corrected by tuning a few elements.

- Figure 13.22.q shows the wideband insertion loss. As we guessed correctly, the first spurious passband is now suppressed by about 60 dB. An approximately 20 dB improvement is obtained compared to unequal width resonator case. Further improvement may be obtained by adjusting the locations of some of the resonance frequencies in the 5200-5800 MHz range slightly by trimming the ridge gaps of the relevant resonators.

- Note however that the lowest cutoff frequency of possible TE30 modes is decreased to about fc30=5000 MHz.

Figure 13.22. Design of waveguide filters with unequal ridge resonators.

Figure 13.22. Design of waveguide filters with unequal ridge resonators.

Figure 13.22. Design of waveguide filters with unequal ridge resonators.

Realization of Constant Reactances as septums:

- In all of the above designs we have shunt constant reactances as coupling elements between WG resonators. These constant reactances will be realized as inductive septums. Therefore the final stage is to convert all the shunt reactances into septums. This is described in Figure 13.23 for equal width and nonridged filter, WG11.fil.

- The widths d1, d2 and d3 of the irises are calculated using the equations in Mathhaei-Young-Jones, p.453 together with the graphs of Marcuwitz for zero thickness symmetric irises.

- In order to check whether we can use circular irises or not, the formula shown in Figure 13.23.n is used to calculate the diameters of irises corresponding to the reactances. It is found out that the first and last reactances can’t be realized with circular irises. Wider window is needed for tighter coupling. Therefore they are left as shunt inductive septums. The inner reactances can be realized as circular irises. Their diameters are shown in the figure.

- In order to see the effects of narrowing the bandwidth, another filter is designed now with fp1=2990 MHz and fp2=3010 MHz. It is seen that for narrower bandwidths the gaps between septums (lengths of WG pieces) get longer while the septum openings (d’s) get narrower reducing the coupling between adjacent resonators.

- Clearly, other shapes can also be used as irises, if easier or more convenient for some reason. EM simulations can be used for fine trimming or for taking the thickness of irises into account.

- For unequal width and ridged resonators, the above mentioned calculations for septums can only be used as a starter for an optimization through EM simulation because of extra discontinuities due to unequal widths and ridges.

Figure 13.23. Calculation of septum dimensions for waveguide filter with equal width resonators.

# EXAMPLE 13. 3. A waveguide filter with shunt inductive posts

- As exercise with different tools of Filpro, let’s design the same bandpass filter of the Example 12.2 now by using shunt posts:

PBRipple=0.01 dB, fp1=2975 MHz, fp2=3025 MHz, NR=5 (five resonators- degree 10).

- In this case the shunt inductive reactances will be provided by vertical posts. Besides the shunt inductive reactance the posts have also capacitive series reactances which must be taken into consideration during the design stages.  One approach may be as follows.

- Design / By Prototype with inverters approach is used and Prototype with series inductors and inverters option is selected (Figure 13.17).

- Prototype degree and PBRipple are typed in. The lowpass prototype will appear on the screen.

- Then Filter Type command is invoked and Lumped Element and Bandpass options are selected.

- Then PB and Terminations command is invoked and passband edge frequencies are typed in. The terminations are left as RS=RL=50 ohms.

- Find Circuit command displays the bandpass filter with series LC resonators coupled by ideal inverters (Figure 13.17.a).

- The command Inverters / All Equal Left-Right / Inductors is applied to equate all the shunt inductors (Figure 13.17.b). The unity transformer at load end is deleted.

- The LC resonators are converted into half wave TEM mode transmission lines by applying the command Replace By Tline / Half WaveLength TLine (Figure 13.17.c).

- Next the half wavelength transmission lines will be scaled to the impedance level compatible with the selected waveguide size by applying the command Waveguide Operations / Scale Circuit for WG.  This command displays a window to type in the width and height of the waveguide to be used and effective relative dielectric constant and passband center. After typing in these parameters another window will appear asking to scale the whole circuit or the TLine only. Since the whole circuit will be realized using the same waveguide we select the option Scale the whole Circuit. Then the cutoff frequency (fc=2077.967 MHz), the first three resonance frequencies, Guide Wavelength, V-I based characteristic impedance of the waveguide (ZcWG=246.615 ohms) and the new characteristic impedance of the transmission lines (ZcTLine=474.05 ohms) are calculated and displayed. The original characteristic impedance of the transmission lines will be scaled up to this new level. Here we note down these parameters for use at later stages. Filpro asks the question Scale Circuit for the Selected WG?. Yes option is selected and the whole circuit is renormalized to the new RS=RL=ZcWG=246.615 ohms level to make the transmission line impedances 474.05 ohms. (Figure 13.17.d).

- Next, with cursor under the transmission lines, the command Waveguide Oprs / Tline-to-WG is applied. This command deletes the transmission lines and inserts half wavelength long waveguide pieces having the specified transversal dimensions (Figure 13.17.e).

- Thus, design of the filter with the desired waveguide resonators coupled by ideal inverters is finished. The next problem is the replacement of the ideal inverters by practical equivalents. In this example we will use practical inverters involving shunt cylindrical posts. This is achieved as follows.

- With cursor under the inverters, the command Inverters / Replace Ideal Inverter By / Constant Reactance / Tee Cct is applied to replace them by Tee sections with constant reactances. The series arms are negative reactances (Figure 13.17.f). Her we note down the shunt constant reactances as Xb1, Xb2 and Xb3 of the left half of the symmetric circuit. These parameters are used for calculation of the diameters d1, d2 and d3 of the posts by using the chart of Marcuwitz shown in Mathhaei-Young-Jones, p.454. The equivalent circuit of a post is a Tee section with positive (inductive) shunt constant reactance and two negative (capacitive) series constant reactances as shown in Figure 13.17.g. Then these diameters are used to calculate the corresponding series constant reactances Xa1, Xa2 and Xa3 of the Tee sections by using the same chart. Calculations are also shown in Figure 13.17.g.

- Next, we form the constant reactance Tee sections in the circuit corresponding to the respective posts by splitting the series reactances using Split / Specify Left command. Exchange command is used to position the reactances correctly so that the Tee sections corresponding to the posts are explicitly seen (Figure 13.17.h-n). After splitting the series reactances, there remains some unused reactances (not associated with a Tee section corresponding to a post). They will be absorbed into the adjacent waveguide pieces. This is done by brute force. We simply Delete the unused constant reactances on the two sides of a waveguide piece and then use Tune Element command to tune the length of waveguide until the response is recovered (Figure 13.17.p).

- This procedure is repeated on all three waveguide pieces and associated unused reactances. Each application of this procedure will add some degradation into response. This degradation can only be corrected by tuning all the shunt reactances slightly (Figure 13.17.q-r-s). However this tuning will effect the diameters of posts slightly. In order to take these slight changes into account we can repeat the design by using the new (modified-tuned) shunt reactances. However this stage may be skipped because there will be tuning process during implementation of the filter in any case.

- The negative series reactance that remained adjacent to RS and RL may be absorbed into the feed circuit of the waveguide. It is also possible to Delete them and then tune the adjacent shunt reactance and length of the nearest waveguide piece. Since this will change the diameter of the first and last posts, calculations and the last stages of the design may need iteration.

Figure 13.17. Design of waveguide filter with shunt inductive posts.

Figure 13.17. Design of waveguide filter with shunt inductive posts (continued).

Figure 13.17. Design of waveguide filter with shunt inductive posts (continued).

Figure 13.17. Design of waveguide filter with shunt inductive posts (continued).

EXAMPLE 13. 4. Waveguide filters with quarter wavelength couplings and shunt inductive irises

- Ideal inverters may also be approximated by quarter wavelength lines over small bandwidths. In this example the filters of previous examples will be repeated by using quarter wavelength waveguides as coupling elements between half wavelength resonators.  The main advantage of this approach is the ease of testing the individual resonators (See Mathhaei-Young-Jones, pp.479-480).

PBRipple=0.01 dB, fp1=2975 MHz, fp2=3025 MHz, NR=5 (five resonators- degree 10).

- Design / By Prototype with inverters approach is used and Prototype with shunt capacitors  and inverters option is selected (Figure 13.18).

- Prototype degree and PBRipple are typed in. The lowpass prototype will appear on the screen.

- Then Filter Type command is invoked and Lumped Element and Bandpass options are selected. Then PB and Terminations command is invoked and passband edge frequencies are typed in. The terminations are left as RS=RL=50 ohms. Find Circuit command displays the bandpass filter with series LC resonators coupled by ideal inverters (Figure 13.18.a).

- The command Inverters / All Equal Left-Right / Inductors is applied to equate all the shunt inductors (Figure 13.18.b). The unity transformer at load end is deleted.

- All the resonators are in shunt LC form. They can be converted into series arm series LC resonators by placing the cursor under the shunt LC resonators and applying the command Inverters / Replace By Inverter Eqv. / K=50.  This command converts the shunt resonators into series resonators in between two inverters with inverter constant K=50 together with a negative unity transformer as shown in Figure 13.18.c. Other values may also be used for K because they will be adjusted at later stages.

- The unity transformers are deleted (Figure 13.18.d). Note that we now have adjacent triplets of inverters.  The ones in the middle of the triplets are the coupling inverters while the ones adjacent to the series LC resonators are actually parts of these resonators, making the series resonators equivalent to the original shunt LC resonators. We should be careful to distinguish the two types of inverters, because they will be treated in different ways, as will be described below.

- Since the inverters at source and load ends have impedance K=RS=RL=50 ohms, we can simply Delete them without effecting the response. In general the inverters at source and load ends are absorbed into the terminating resistors by placing the cursor under RS and RL and applying the command Inverters / Scale RS-RL by Inv / Absorb Inverter into RS-RL as described in Figure 13.18.e.

- The series LC resonators are converted into Half Wavelength transmission line resonators by applying the command Replace By TLine / Half Wavelength TLine (Figure 13.18.f).

- Next, the coupling inverters are converted into quarter wavelength transmission lines by applying the command Inverters / Replace Inverter By / QuarterWave Tline / f0=3000 MHz (Figure 13.18.g).

- Now we will make preparations for conversion of the Half Wavelength transmission lines into Half Wavelength waveguides and the Quarter Wavelength transmission lines into Quarter Wavelength waveguides. The two types of transmission lines have to be scaled to different impedances to be equivalent to the waveguide counterparts. The procedure is as follows:

- With cursor under any Half Wave line resonator, the command Waveguide Oprs / Scale Cct for WG / Scale the whole Circuit is applied. The transversal dimensions of the desired waveguide are typed in. Filpro displays the corresponding cutoff frequency, the first three resonance frequencies, guide wavelength at f0=3000 MHz, the characteristic impedance ZcWG=246.615 ohms of the waveguide (V/I based) and the characteristic impedance of the correspoınding Half Wavelength TEM mode transmission line (ZcTLine=474.05 ohms) are displayed. Selecting Yes option will scale impedance level of the circuit so that all the half wave line impedances will be ZcTLine=474.05 ohms while the source and load resistances are scaled up to ZcWG=246.615 ohms (Figure 13.18.g-h).

- Next, the Quarter Wavelength transmission lines are to be scaled up so that when transformed into waveguide form they will act as Quarter Wavelength waveguides. This equivalence is possible only by scaling their impedances to the characteristic impedance of the guide, that is, ZcWG=246.615 ohms. This is done by placing the cursor under these lines and applying the command Inverters / Scale Element By Inverters / Zc=246.615 ohms. Impedance of the lines are scaled up to the desired value automatically by Filpro, by scaling the adjacent two inverters to proper values (Figure 13.18.h-i).

- Now we are ready for conversion of all the transmission lines and inverters to equivalent waveguides. First the transmission lines  must be converted into waveguides. This is done by applying  the command Waveguide Oprs / Tline-to-WG (Figures 13.18.i-k).

- Finally, the inverters are replace by their waveguide equivalents involving shunt reactances and negative lengths by applying the command  Inverters / Replace Inverter By / Constant Reactance / WG+Xsh+WG (Figure 13.18.k-m).

- The waveguide pieces with negative length are absorbed into the adjacent positive length waveguides by applying Simplify Circuit command. The final circuit is shown in Figure 13.18.n. The passband return loss level is improved slightly while almost no shrinkage exists in the bandwidth. The nearest spurious passband about  4800 MHz is widened appreciably compared to the previous approaches (Figure 13.18.p).

- The shunt reactances can be realized as irises as described in the previous examples.

- Figure 13.18.q shows a pictorial description. The main advantage of such a filter is the ease of tuning each resonator individually by physically separating the sections shown in the figure.

Figure 13.18. Design of a half wave resonator filter with quarter wavelength couplings.

Figure 13.18. Design of a half wave resonator filter with quarter wavelength couplings (cont’d).

Figure 13.18. Design of a half wave resonator filter with quarter wavelength couplings (cont’d).

Figure 13.18. Design of a half wave resonator filter with quarter wavelength couplings (cont’d).

# EXAMPLE 13. 5. Widebanding limitations of waveguide filters

# Since waveguides behave much like transmission lines their response presents kind of periodicity similar to those of commensurate length transmission line filters. Therefore bandwidth of propagating waveguide filters are limited by the nearest spurious passband, necessitating the use of LPF in cascade or other techniques. In this example we will design a wideband filter just to see the limitations.

# Filter specifications are as follows:

# PBRipple=0.01 dB, fp1=2750 MHz, fp=3250 MHz, Prototype degree=5

# The filter is designed by using the same procedure as in the previous examples (Figure 13.19).

# The effects of wide bandwidth shows itself at almost all steps, by starting from the conversion of LC resonators into transmission line resonators (Figure 13.19.c), continuing with conversion of transmission line resonators into waveguide resonators (Figure 13.19.d), then conversion of inverters into WG+Constant Reactance+WG pieces (Figure 13.19.e-f-g-h). Passband return loss response degrades appreciably. The original response can be recovered with even some extra improvements by tuning elements or, easier, by applying Best Case optimization routine.

# The final circuit and its passband return loss and wideband insertion loss responses are shown in Figures 13.19.i-k-m. Wideband response shows that the upper stopband is almost useless for most applications.  Different width resonator approach is also of no help (Exercise?). A LPF is needed to form a regular bandpass filter response.

# Figure 13.19. Design of a wideband waveguide filter.

Figure 13.19. Design of a wideband waveguide filter (cont’d).

Figure 13.19. Design of a wideband waveguide filter (cont’d).

13. 6. HALF WAVELENGTH FILTERS WITH DIFFERENT  WIDTH RESONATORS

When all the resonators are made up of the same waveguide the stopband performance of waveguide filters may not be satisfactory because of the spurious passbands around the second, third, fourth, etc.  resonance frequencies and other higher order modes. Often LP filters are needed to avoid the spurious passbands. However stopband performance may also be improved to a limited extent by using nonidentical waveguide sections as described by Riblet [10] and Morelli et. al. [11]. In the approach of reference [11] only the widths of waveguide resonators are made different so that their second resonance frequencies (f2’s) are spread in the stopband thus preventing the creation of the first spurious passband. The interrelations among the width a, length d, cutoff frequency fc and the resonance frequencies fk (k=1,2,..) were summarized in Section 13.4. For half wave resonator filters the first resonance frequency f1 must be selected as the passband center f0. It was shown that given the first resonance frequency f1=f0, the location of the second resonance frequency f2 can be set in the range

- f2min=1.275f0<f2<f2max=1.633f0				(13.6.1)

Location of the second resonance frequency f2 can be adjusted by adjusting the width a of the resonator through the relation

- a=c234f02−f22								(13.6.2)

- This relation gives the range for adjusting widths as

amin=0.56cf0<a<amax=0.75cf0					(13.6.3)

We can apply these relations to set different widths ak for each resonator in the range

0.56cf0<ak<0.75cf0							(13.6.4)

leading to different second resonance frequencies f2(k) in the range

1.275f0<f2k<1.633f0							(13.6.5)

This action prevents formation of the spurious passbands to a certain degree. That is, the passsband created by one  f2k    will be annihilated / supressed by the others to a limited extent. The corresponding lengths of the resonators are then

dk=ak(f0fc)2−1							(13.6.6)

The second resonance frequencies f2k can be spread in various ways which may worth trials to get relatively better stopband response. Some were studied in [11] as summarized below:

- Multiple Width Resonators with Equal Spacing for f2k:

- The second resonance frequencies can be spread in this interval as equally spaced

- f2min=f21<f22<f2k..<f2max					(13.6.7)

- with

- f2k=f2min+(k−1)δf							(13.6.8)

- and

- δf=f2max−f2minn							(13.6.9)

- where n is the number of resonators. Then the resulting widths are calculated as

- ak=c234f02−(f2k)2							(13.6.10)

- Two Width Resonators:

- We may collect the second resonance frequencies at only two frequencies, for example f2a and f2b. Thus, the waveguides will have only two widths, aa and ab, simplifying construction somewhat while also providing further improvement in a limited range in the stopband. For example we can select f2b  as the maximum f2:

- f2b=f2max=1.633f0							(13.6.11)

- Then f2a  can be determined so as f2a , f2b  and f3a  are equally spaced where f3a is related to f2a through 13.4.14 as

- f3a=8(f2a)2−5f023 							(13.6.12)

- This results in

- f2a=1.393f0								(13.6.13)

- Using 13.4.22 and 13.4.24 in 13.6.7 we can calculate aa and ab as follows:

- aa=0.6cf0,    	ab=0.75cf0						(13.6.14)

- Let’s see some examples using both two widths and multiple widths approaches.

# EXAMPLE 13. 6. Waveguide filters with different width resonators

- Let’s design the same narrowband filter as in the previous example with following parameters:

PBRipple=0.01 dB, fp1=2975 MHz, fp2=3025 MHz, NR=5 (five resonators- degree 10).

- Let’s make several designs by using waveguides with different width resonators.

- Case-1: Multiple Width Resonators with Equal Spacing for f2k:

- Given f0=3000 MHz, the minimum and maximum widths can be calculated as

- amin=0.56cf0=56	 mm  and   amax=0.75cf0=75 mm,

- These widths correspond to

- f2min=1.275f0=3825 MHz  and  f2max=83f0=1.633f0=4899  MHz

- The second resonance frequencies can be spread in this interval as equally spaced with spacing

- δf=f2max−f2minn=4899−38255=214.8  MHz

- The other second resonance frequencies can be calculated using the formula

- f2k=f2min+(k−1)δf

- as follows: f2(2)=3825+214.8=4039.8 MHz, f2(3)=3825+429.6=4254.6  MHz

- f2(4)=3825+644.4=4469.4  MHz

- Then the corresponding widths are calculated using 13.6.10, as follows:

a1=56 mm, a2=58.56 mm, a3=61.41 mm, a4=64.95 mm, a5=75 mm

- After setting these widths we can start the design. The design steps are shown in Figure 13.20.

- Design / By Prototype with inverters approach is used and Prototype with shunt capacitors and inverters option is selected. Prototype degree and PBRipple are typed in. Then Filter Type command is invoked and Lumped Element and Bandpass options are selected.

- Then PB and Terminations command is invoked and passband edge frequencies are typed in. The terminations are left as RS=RL=50 ohms. Find Circuit command displays the bandpass filter with shunt LC resonators coupled by ideal inverters (Figure 13.20.a).

- The command Inverters / All Equal Left-Right / Inductors is applied to equate all the shunt inductors (Figure 13.20.b). The unity transformer at load end is deleted.

- Next Replace By TLine / Half Wavelength TLine command is applied to convert all the LC resonators by TEM mode half wave transmission lines (Figure 13.20.c).

- Since in this case waveguides with all different widths will be used for resonators, the corresponding transmission lines need be scaled to different levels compatible with the selected waveguide. This is done by placing the cursor under each transmission line one by one and applying the command Waveguide Operations / Scale Circuit-TL for WG. This command displays a window to type in the width, height, dielectric constant of the selected waveguide and the passband center. After typing in these parameters another window appears asking if the whole circuit or only the transmission line pointed by the cursor will be scaled. This time we select the option Scale pointed TLine Only. Then another window will appear displaying the cutoff frequency fc=2676 MHz, the three resonance frequencies fres1=3000 MHz, f2=3808 MHz, f3res= 4866 MHz, Guide Wavelength, V/I based characteristic impedance of the waveguide and the new characteristic impedance of the corresponding half wave transmission line (ZTLine). {Note: The frequencies calculated by Filpro will be slightly different from the ones calculated using the above formulations because in the above  formulations velocity of light is taken as c=3*1011 mm/s while in Filpro, more accurate value is used for c}. Also the question Scale TLine for the selected WG?  is asked. If Yes option is selected then the transmission line is scaled to the displayed level (ZTLine) by adjusting the inverters on its two sides. If the TLine is at the source end then an inverter is also placed to the left of the TLine while also adjusting the source impedance to the characteristic impedance of the waveguide selected for the source end resonator. If the TLine is at the load end then an inverter is inserted at the load end and the load impedance is adjusted to be equal to the characteristic impedance of the waveguide selected for the load end resonator.

- For example at step-c, with cursor under the TLine at source end, the command Scale Cct-TL for EWG / Scale Pointed TLine Only is applied with a=56 mm, b=34 mm. The TLine impedance is scaled up from 1444 ohms to 2485 ohms by adjusting the inverter on the right of the TLine and also inserting an inverter on its left (source) side (Figure 13.20.d).

- Then with cursor being still under the same TLine, the command WG Oprs / TLine-to-WG/ a=60, b=34 mm is applied to convert the scaled TLine to WG (Figure 13.20.e). Length of the WG comes out to be 110 mm. The second resonance frequency fres2=4042 MHz, close to the calculated value of 4039.7 MHz.

- Next, the cursor is moved to point the next TLine and the same commands are applied now for a=58.56 mm, b=34 mm resulting in the WG with length 95.8 and fres2=4257 MHz, close to the calculated 4254.6 MHz (Figures 13.20.e-f-g).

- The same commands are repeated with a=61.41 mm, a=64.95 mm and a=75 mm giving the circuit of Figure 13.20.p. At load end another inverter is placed during scaling stage of the load end TLine.

- In the next stage the inverters are replaced by their practical equivalents involving shunt constant reactances surrounded by two negative length waveguides by applying the command Inverters / Replace Inverter By / Constant Reactance / WG+Xsh+WG (Figure 13.20.p-q). Passband return loss shows a slight narrowing of the bandwidth while return loss level is improved somewhat (Figure 13.20.r). Simplify Circuit command combines the adjacent waveguide pieces giving the circuit with WG resonators coupled by shunt constant reactances (Figure 13.20.s).

- Wideband insertion loss response is shown in Figure 13.20.t. It is seen that the spurious passband levels are decreased appreciably. This is because the second resonance frequencies of each waveguide are different, hence they are not aiding each other. That is, one spurious passband created by one waveguide is destroyed (made ineffective) by the other resonators. It is clear that the spreading of the first resonance frequencies helps also spreading of the third, fourth, etc. resonance frequencies leading to an improved overall stopband  response. (Compare with Figure 13.5.k of Example 13.2).

- Some spike type spurious passbands exists. If necessary they may be eliminated by further adjustment of the widths of resonators, like for example as will be done in the Two Widths case to be studied next.

Figure 13.20. Design of a waveguide filter with multiple width resonators.

Figure 13.20. Design of a waveguide filter with multiple width resonators (cont’d).

Figure 13.20. Design of a waveguide filter with multiple width resonators (cont’d).

Figure 13.20. Design of a waveguide filter with multiple width resonators (cont’d).

- Case-2: Two widths approach:

- A simpler structure can be obtained by using only two widths, aa and ab. The two widths can be selected as given in equations 13.6.14:

- aa=0.6cf0=0.63∗10113∗109=60  mm,            ab=0.75cf0=0.753∗10113∗109=75  mm

- leading to the second resonance frequencies

- f2a=f2min=1.393f0=4179 MHz,         f2b=f2max=1.633f0=4899 MHz.

Let take the height of all waveguides as b=34 mm. The design steps are the same as in the previous case and are shown in Figure 13.21. In this approach we have selected the widths of the first three resonators on source side as aa=60 mm and the two resonators on load sides as ab=75 mm.

We could as well select the widths of five resonators in mixed form for example, a(1)=60mm, a(2)=75 mm, a(3)=60 mm, etc. (Not much difference in stopband response. Exercise?). Figure 13.21.u shows the wideband insertion loss where the levels of the spurious passbands are formed around f1a and f1b with appreciably reduced levels. Note that compared to the equally spaced case the stopband response of this case is better because there are no spikes up to about 5000 MHz.

Figure 13.21. Design of a waveguide filter with two different widths.

Figure 13.21. Design of a waveguide filter with two different widths (cont’d).

Figure 13.21. Design of a waveguide filter with two different widths (cont’d).

Figure 13.21. Design of a waveguide filter with two different widths (cont’d).

13. 4. FULL WAVELENGTH AND MIXED RESONATOR FILTERS

Instead of Half Wavelength resonators used in the previous section, it is also possible to use Full Wavelength resonators for higher Q reasons. As a demonstration of the tools of Filpro on this subject  let’s repeat the narrowband filter of the previous section now by using Full Wavelength resonators.

EXAMPLE 13. 7.  Full wavelength resonator filters

- Specifications are:

PBRipple=0.01 dB, fp1=2975 MHz, fp2=3025 MHz, N=5

Let’s first design an equal width resonator filter by using the waveguide WR-284 with dimensions a=72.136 mm (2.84 inches), and b=34.036 mm (1.34 inches), operating in TE10 mode. Then we will try different width resonator cases.

Equal width resonators:

The design approach is shown in Figure 13.22.

- Design / By Prototype with inverters approach is used and Prototype with series inductors and inverters option is selected (Figure 13.22). Prototype degree and PBRipple are typed in. The lowpass prototype will appear on the screen. Then Filter Type command is invoked and Lumped Element and Bandpass options are selected. Then PB and Terminations command is invoked and passband edge frequencies are typed in. The terminations are left as RS=RL=50 ohms. Find Circuit command displays the bandpass filter with series LC resonators coupled by ideal inverters (Figure 13.22.a).

- The command Inverters / All Equal Left-Right / Inductors is applied to equate all the shunt inductors (Figure 13.22.b). The unity transformer at load end is deleted. The resulting circuit is saved as WG21.fil for use at other designs.

- All the series LC resonators are replaced by Full Wavelength TEM mode transmission line resonators by placing the cursor under each resonator one by one and applying the command Replace By TLine / Full Wavelength TLine (Figure 13.22.c). Let’s save this circuit as WG22.fil to use for the other designs also.

- The full wavelength transmission lines will be replaced by (guided) full wavelength waveguide pieces with desired cross-sectional dimensions. The parameters of the desired waveguide sets the characteristic impedance of the corresponding TEM line. Therefore in order to be able to use the desired waveguide dimensions, we should first scale up the impedances of transmission lines to proper level. This is done by placing the cursor under any transmission line and applying the command Waveguide Operations / Scale Circuit-TL for WG (Figure 13.5.c). This command displays a window to type in the width (a=72.136 mm), height (b=34.036 mm) and dielectric constant of the waveguide to be used and the passband center. After typing in these parameters another window appears asking if the whole circuit or only the transmission line pointed by the cursor will be scaled. If all resonators are going to be realized by the same size waveguide then we prefer the option Scale the whole Circuit. Then another window will appear displaying the cutoff frequency (fc=2077.9 MHz), the first three resonance frequencies, fres1=2342 MHz, fres2=3000 MHz and fres3=3853 MHz, Guide Wavelength=138.56 mm, the characteristic impedance of the waveguide (ZcWG=246.62 ohms) and the new characteristic impedance of the corresponding half wave transmission line (ZcTLine=474.05 ohms). Here the resonance frequency around which the actual passband frequency will be formed is f0=fres2=3000 MHz. Therefore the first resonance frequency fres1=2342 MHz <f0=3000 MHz  may be treated as subharmonic resonance frequency around which a spurious passband will be formed. The third resonance frequency fres3=3853 MHz above f0 will form the nearest spurious passband in the upper stopband. Selecting Yes option for the question Scale circuit for Selected WG?,  the whole circuit is rescaled to RS=RL=246.62  ohms which is the characteristic impedance of the selected waveguide to make the transmission line impedances 474.05 ohms by inserting inverters at source and load ends (Figure 13.22.d).

- Next, with cursor under the transmission lines the command Waveguide Operations / Tline-to-WG is applied converting the transmission line pieces into waveguide pieces with the desired width and height (Figure 13.22.e).

- Next, the inverters are replaced by their proper practical equivalents. This replacement is done by placing the cursor under the inverters and applying the command Inverters / Replace Inverter By / Constant Reactance / WG+Xsh+WG. The selected equivalent circuit involves a shunt constant reactance surrounded by two waveguide pieces of negative lengths as seen in Figure 13.22.f. The negative length waveguide pieces are combined with the adjacent positive length waveguides by applying Simplify Circuit command (Figure 13.22.g).

- Figure 13.22.h shows the passband return loss which shows little degradation.

- Figure 13.22.i shows the wideband insertion loss response. The subharmonic passband is formed around fres1=2342 MHz and the nearest spurious passband in upper stopband is formed around fres3=3853 MHz. These spurious passbands seem to be problematic. Several techniques can be used to make them ineffective or less effective. Some of them are studied next.

Unequal Width Resonators:

- One way to deal with the subharmonic and other spurious passband may be to use unequal width resonators. In full wavelength resonators f2 (fres2 on Filpro) is treated as the passband center f0 while f3 (fres3 on Filpro) forms the nearest spurious passband. In order to avoid the formation of the nearest spurious passband in upper stopband arising from f3, we can select different widths for the resonators so that f3 of each resonator will be at different location. Formulation for the optimum spread for best spurious passband suppression can be calculated in a way similar to the one developed for half wavelength resonators. The maximum width amax is usually limited by the second harmonic of the passband center to be less than the cutoff frequency of TE30 mode, giving the same constraint as in half wave resonator filters:

- amax=0.75cf0

- Using this constraint in the formula for f3 in terms of f2=f0, it can be shown that

- f3max=1.3f0

- Comparing with the corresponding relation,

- f2max=1.633f0

- of the half wave resonator filters we observe that the f3max is closer to the main passband at f0 (f2). That is in full wavelength filters the stopband range between main passband and the first spurious passband is narrower than in half wave resonator case. The minimum width can be set through the constraint

- f4min=f3max

- Using an analysis similar to that presented in section 13.1.1, it can be shown that

- f3min=1.135f0

- corresponding to the minimum width given roughly as

- amin=0.57cf0

- The third resonance frequencies f3k can be spread as equal spaced or they can be collected at certain selected frequencies, as was described in the haşf wave resonator cases.

- Just to get an idea let’s try the following cases.

- Two width resonators:

- Let’s first try the case with only two widths, a1=60 mm and a2=75 mm, as was done in the       previous example.

- The circuit of WG22.fil is loaded (Figure 13.22.k).

- The first three transmission line resonators are scaled for waveguide with dimensions a1=60 mm, b=34 mm and then converted into waveguides (Figure 13.22.k-m-n). The subharmonic passband corresponding to these waveguides is fres1=2632 MHz and the nearest spurious passband in upper stopband is around fres3=3528 MHz.

- Then the remaining two transmission lines are scaled and converted into waveguide resonators with dimensions a2=75 mm and b=34 mm (Figure 13.22.p-q). The subharmonic passband corresponding to these waveguides is fres1=2290 MHz and the nearest spurious passband in upper stopband is around fres3=3905 MHz.

- After converting the inverters into waveguides we get the circuit of Figure 13.22.s. Responses are shown in Figures 13.22.t and u. The resultant subharmonic spurious passband is about 2600 MHz and it is suppressed by about 35 dB. The upper stopband nearest spurious passband is about 3500 MHz and it is suppressed by 25 dB.

Multiple width resonators:

As another trial let a1=56 mm, a2=58 mm, a3=61 mm, a4=68 mm and a5=75 mm, similar to the case of multiple width half wave resonator of the previous example.

- The circuit of WG22.fil is loaded (Figure 13.22.v).

- Each transmission line resonators are scaled for the corresponding waveguides in the order a1, a2, a3, a4 and a5  and then converted into waveguides giving the circuit of Figure 13.22.x which is saved as WG25.fil.

- Then the order is changed as a1, a5, a2, a4, a3 giving the circuit of Figure 13.22.y which is saved as WG26.fil.

- Responses of both circuits are shown in Figure 13.22.z. Not much difference exists between responses of the two circuits. The existence of the spikes may force us to prefer the two widths case.

Mixed Half and Full Wavelength Resonators:

The level of subharmonic passband can be reduced by using mixed half wave and full wave resonators. Figure 13.13 describes such a design.

- The circuit of WG21.fil is loaded (Figure 13.12.a).

- The LC resonators at source and load ends are converted into Half Wavelength TLine Resonators while the inner LC resonators are converted into Full Wave TLine resonators (Figure 13.12.b).

- Then all TLine resonators are scaled for the same waveguide with a=72.136 mm, b=34.036 mm (Figure 13.12.c) and then converted into waveguide resonators (Figure 13.12.d).

- Then the inverters are converted into WG and constant reactance equivalents (Figure 13.12.e.

- Response is shown in Figure 13.12.f. It is seen that the levels of subharmonic passband and second resonance passband are reduced by about 30 dB.

Clearly, one can use both Half and Full wavelength resonators with different widths (Exercise?).

Figure 13.22. Design of full wavelength filters with equal and unequal width resonators.

Figure 13.22. Design of full wavelength filters with equal and unequal width resonators (cont’d).

Figure 13.22. Design of full wavelength filters with equal and unequal width resonators (cont’d).

Figure 13.22. Design of full wavelength filters with equal and unequal width resonators (cont’d).

Figure 13.22. Design of full wavelength filters with equal and unequal width resonators (cont’d).

Figure 13.22. Design of full wavelength filters with equal and unequal width resonators (cont’d).

Figure 13.23. A filter with mixed Half and Full wavelength resonators.

13. 5. EVANESCENT MODE WAVEGUIDE FILTERS

13. 5. 1. INTRODUCTION

The waveguide filters described in the previous section come out to be too bulky for some applications. By using waveguides far below the cutoff frequency fc of its dominant mode it is possible to realize miniaturized waveguide filters with similar or better performances [12]-[20].

At frequencies below cutoff the propagation constant  becomes imaginary, thus it becomes an attenuation constant:

β=ωεrc1−(fcf)2=jωεrc(fcf)2−1=jα

α=ωεrc(fcf)2−1=ωcεrc1−(ffc)2  Np/m

That is, the waves in all modes attenuate exponentially hence the name Evanescent Mode Waveguide (EWG). This attenuation should not be mixed up with attenuation due to resistive loss. It is due to reflection of the waves, much like attenuation in lossless (reactive) filter elements in their stopbands. Although in general the attenuation constant is frequency dependent, from the above equations it is seen that it becomes frequency independent for frequencies far below cutoff.

In the cutoff region the characteristic impedance becomes purely imaginary. For example the V/I based impedance becomes

ZcV,I=VI=E0xbH0ya=baZw=120πbaεr1−(fcf)2=−j120πbaεr(fcf)2−1=jXoThis is similar to the impedance of a lossless filter in its stopband. That is the waveguide acts as a reactive distributed element rather than a propagating transmission line. This reactive nature is exploited for realization of filters. Approximately equivalent circuit models of an EWG piece operating in TE10 mode are shown in Figure 13.13. Figure 13.14 shows the link between EWG and its circuit model involving inverters.

Figure 13.13. Pi, Tee and inverter models of an evanescent mode waveguide piece operating in  TE01 mode.

Figure 13.14. a) Pi and Tee models of an inverter in terms of lumped element inductors.                      b) Inverter equivalent of an EWG. c) EWG equivalent of an inverter.

Using the circuit models described in Figures 13.13 and 13.14, it is possible to convert the bandpass filter circuits shown in Figures 13.15.a or c into an EWG filter shown in Figure 13.15.b.as will be described in the examples below. That is, an evanescent waveguide piece can be converted into a filter by shunt capacitive loadings at proper positions along the waveguide. The circuit of Figure 13.15.a has a single transmission zero at f=0 and the rest at f=, the same as a combline filter. Therefore EWG filters has selectivities similar to combline filters or capacitively loaded interdigital filters (CLIFs).

Figure 13.15. Lumped element bandpass filter prototypes that can be converted into EWG filters.

Such filters can be designed in frequency ranges starting from VHF frequencies to millimeter wave frequencies, with bandwidths from 1-percent up to an octave or wider band and with wide stopbands. Actually the inductive reactances used in modeling EWG pieces are not lumped inductors. Compared to lumped inductors, they have higher rate of increase with frequency because the propagation factor tends to zero as frequency approaches to the cutoff frequency fc. The effect of this approximation shows itself as a shrink in bandwidth. However at frequencies well below the cutoff frequency the shrinkage in bandwidth reduces considerably.

- The lumped capacitors can be formed by capacitive posts such as shown in Figure 13.16. The posts have some inductive reactances which should be taken into consideration during the synthesis stage of the filter. That is, a good circuit model is necessary for the posts to be used during the synthesis. Since such inductances are geometry, frequency and mode dependent usually full EM modeling and simulation is needed for this purpose. However a rough estimate of these inductances can be made through the following procedure:

Figure 13.16. Capacitive posts in an EWG filter.

Figure 13.17.a shows the electric field lines of TE10 mode and the surface currents on the walls of the guide. A vertical post of height h<b can be placed inside the guide to get a lumped capacitor effect as shown in Figure 13.17.b. The electric field of TE10 mode will induce charges and hence current along the post. The charges accumulated at the top end of the post create strong electric field between the end of post and top wall, creating the capacitive effect. However existence of the post presents a discontinuity and complicates the problem basically by leading to local mode conversions.

1) TEM waves along the posts:

The current along the post completes its path through the top and side walls down to the bottom wall as shown in Figure 13.17.b. The electric field lines extending from the post to the side walls of the guide suggest existence of TEM wave propagation along the post, in much the same way as the TEM wave mode assumption along the fingers of combline or interdigital filters. In this respect the conducting post in between and parallel to the two side walls can be treated as a slab line with TEM mode characteristic impedance Z0 set by its radius and distances to side walls (s in the figure).  Since one end of the post is connected to ground plane (bottom or top wall) it can be simulated as a SC stub of length h. Thus, a standing wave exists on the SC stub with zero voltage (maximum current) at SC end and maximum voltage (zero current) at the capacitor end. Given the parameters h and Z0, the reactance XL of the post can be calculated as the input reactance of the SC stub as

XL=Z0 Tan

where  is the electrical length of the post. An equivalent lumped inductance can be derived as

L=Z0 Tan

where  is the passband center of the intended filter. Thus, we have an inductive reactance in series with the capacitor C, forming an LC type resonator. However, although the inductive and capacitive reactances seem to be in series, actually it is not a series LC resonator, but a parallel LC resonator simply because both ends of the LC element are connected to each other through the side walls of the guide. That, if the junction of SC stub and capacitor shown as node-A in the figure is taken as a node, the SC stub and capacitor are actually in parallel, extending from node-A to ground as shown in the circuit model of Figure 13.17.b. Thus, we actually have a parallel resonator similar to the resonators of combline and capacitively loaded interdigital filters.

The transversal TEM modes over the posts should decay in longitudinal direction of the guide.  If they don’t decay sufficiently at the position of the next post then the nature of coupling between adjacent posts will change. That is we may have both evanescent TE10 mode and TEM mode type couplings. If they decay sufficiently then we will have only Evanescent TE10 mode type coupling. This aspect of the problem depends on the geometry and dimensions of guide and posts and needs modal EM analysis.

2) TE20 mode:

The side surface of the post forms a conducting wall which forces the electric field solutions to be zero on its surface. Such a boundary condition can be satisfied by higher order modes like TE20, TE40, etc. As an example the electric field lines of TE0 mode are shown in Figure 13.17.c. Since these modes decay at a higher rate than the fundamental TE10 mode, they may lead to only local energy storage which means that the reactance of the LC resonator may need trimming.  However in cases where the adjacent resonator rods are close to each other (wideband filters), these higher order modes may also lead to extra (unaccounted) coupling between adjacent resonators.

In any case a good EM analysis will be helpful.

Links between EWG filters and Combline / Capacitively loaded interdigital filters:

- The similarity between combline filters, CLIF’s and EWG filters can further be detailed as follows:

- If a Combline filter or CLIF built in a housing with rectangular cross-section then the housing may also be treated as a waveguide. Therefore such combline and CLIF’s resemble EWG filters if the housing cross-sectional dimensions allow only evanescent waveguide modes to travel in the longitudinal directions within the housing.

- The main assumption used in the development of distributed element circuit models of combline and CLIFs is that only TEM mode waves travel in the transversal directions, along the rods. The assumption of TEM wave traveling in between a rod and ground plane enables us to define a characteristic impedance Zk0 for rod-k set by the distance between rod and ground and also by the radius (cross-sectional dimensions) of the rod together with the dielectric constant of the medium. The input excitation is coupled to the first rod to induce electric field/current along the rod which in turn create the TEM waves between rods and ground and in between adjacent rods.

- One end of each rod is connected to a capacitor (Ck) and the other end is connected to the housing wall which is assumed to be grounded. Thus, the rods can be modeled as SC stubs of impedance Zk0 between one terminal of the capacitor and ground. Since the other terminal of the capacitor is also grounded, the rod and capacitor forms a parallel resonator formed by a shunt SC stub and lumped element capacitor. The waves over each line form standing waves with maximum of current at the grounded end and maximum of the voltage at the other (capacitively loaded) end (Figure 13.18.a-b).

- Coupling between the adjacent rods of comblines and CLIFs is provided through the odd mode TEM waves traveling in between the rods. That is the adjacent rods, for example rods k and k+1 are assumed to form a two-wire-line of characteristic impedance Zk,k+1. In combline filters the medium between adjacent rods acts as a two-wire transmission line grounded at the same side ends, thus forming a SC stub. This SC stub is used as the coupling element between adjacent resonators. In Figure 13.18.a these are described on a two resonator combline structure where the capacitors are shown as C10, C20 and characteristic impedances of the shunt SC stubs are denoted by Z10 and Z20.  For frequencies f<fq where fq is the quarter wavelength frequency of the rods, the SC stubs can be approximated as lumped element inductors as shown in the figure.

- Figure 13.17. Field lines and TEM circuit model of a post in an EWG.

- Figure 13.18. Circuit models of Combline, Capacitively loaded interdigital and EWG filters.

- In CLIFs since the opposite ends of the adjacent rods are grounded the medium between adjacent rods acts as a two-wire transmission line. This transmission line is used in the circuit model as the element coupling the two adjacent resonators (Figure 13.18.b). For frequencies f<fq, the lumped element models of SC stubs and transmission lines can be used to get the lumped element form shown in the figure.

- In both Comb and CLIF circuits the housing is assumed to be at zero potential (ground) while the ungrounded ends of rods are assumed to be the nodes whose voltages with respect to ground are treated as the main unknown variables of the circuit models.

- Figure 13.18.c shows the circuit model of a piece of EWG filter. The coupling between adjacent resonators will be provided only by the evanescent waves supported by the housing cross-sectional dimensions. If the inductive Pi-model is used for the EWG, then the circuit model assumes the lumped element form shown in the figure where we have again shunt parallel resonators made up of parallel connection of shunt inductor of the Pi-model of EWG and the parallel resonator model of the capacitive posts. Since these shunt resonators are coupled to each other inductively (through the series inductor of the Pi-model of EWG), if the capacitors are placed at the same side ends of the rods then we have the same lumped element prototype as the Comb filters while if the capacitors are placed at the alternating ends we have a circuit model resembling that of the CLIF’s.

- It is seen that the differences between Comb-EWG and CLIF-EWG filters lies in the interpretation of the coupling mechanisms. In CLIF’s and Combs coupling is provided by the TEM waves traveling in between the adjacent rods, in the transversal plane of the housing while in EWG filters coupling is provided by the evanescent waveguide pieces modeling the waves traveling/attenuating  in longitudinal direction. In the present literature on this subject nothing much is said on the nature of the waves traveling in the longitudinal direction of Comb and CLIF’s.   As stated earlier, if the housing of a Combline or CLIF allows only evanescent waveguide modes to travel in the longitudinal directions within the housing, then we may have TEM waves in transversal direction and evanescent waves in longitudinal direction which means that such filters may also be treated as EWG filters.

- In the design of Combs and CLIFs, possible evanescent mode waves in longitudinal direction are usually assumed to be ineffective compared to the main TEM modes traveling along the coupled lines. However, as pointed out in [17], this may not hold under all conditions. That is the coupling between adjacent rods may get some contribution from the longitudinal traveling evanescent modes that are (unintentionally) supported by the housing. This means we may have mixed type couplings. For example in Combline filters, besides the series SC stub we should add a lumped inductor in parallel with the SC stub to simulate the EWG type coupling while in CLIFs the EWG type coupling inductor should be added in parallel with the transmission line. Experiments indicate that some of the evanescent modes may be suppressed because of grounding the alternate ends of adjacent rods. In combline filters the same side ends of rods are grounded. This may help generation of more evanescent modes because of asymmetry of the top and bottom halves of the structure. Effects of evanescent mode couplings show up as bandwidth widening.

- In conclusion it is clear that in the most general circuit model of Comb-CLIF-EWG structures couplings between adjacent resonators may be of combinations of TEM type and EWG type couplings which may also necessitates the incorporation of higher order EWG modes as well. Therefore a generally valid circuit model can only be developed after a thorough analysis of existing wave modes which necessitates the use of fullwave EM simulations.  In this respect the circuit models used on Filpro for synthesis of such filters can only be used just as a systematic way to get a good starting point for the real life designs which may need iterative trials, tuning and tweaking.

Returning to Filpro, EWG filters can be designed most easily using Synthesis approach. It is also possible to use the Prototype with Inverters approach but it is takes more time and for wideband filters too much tuning may be needed to correct the degradation in response because of the approximations involved. The basic idea common to both approaches will be to design the filter in one of the topologies shown in Figures 13.15.a or c and then convert the inductive Pi-sections or inverters into EWG pieces.  Let’s first see the circuit transformations that are needed in these designs.

- Terminations and End Inductance Issue:

The input/output feeds of the EWG filters can take different forms depending on various factors. For example we may have transitions form propagating waveguides to EWG filter or from 50 ohms coaxial line system to EWG filter. In any case we have a discontinuity and impedance transformation-matching problems. Some of these problems will be treated in the examples below.

- In the cases where the input/output feeds are coupled directly to the first and last posts, then we have the following problem:

- In an EWG filter the EM fields of resonators are strongest over the posts and decay exponentially as they travel away from the posts (Figure 13.19). Coupling between adjacent posts is provided by placing the adjacent post at a distance close enough so that the decaying evanescent fields can induce currents on them. On the other hand the fields of outermost posts towards ends of the waveguide have no such a job of coupling to another resonator. They either simply die out if the ends of the waveguide are sufficiently far from the outermost resonators or they are reflected back from the ends of the waveguide if the ends are closed by conducting walls or left open.

- It is clear that if the end pieces of the waveguide are left long enough, than we can leave them open or close them with a conducting or any kind of wall. The EM energy stored in the decaying waves towards the open or closed ends of the waveguide appear as extra shunt inductances in parallel with the inductances of outermost posts. We will call such inductances as End Inductance (Lend). This approach simplifies the solution for the problem of dealing with the waves that would be reflected from the closer placed walls. However this solution leads to longer structures.

- In order to decrease length of the filter we can close the ends of waveguides by conducting walls or leave them open at distances close to the outermost posts. The effects of waves reflected from these walls appear as modifications in the End Inductances.

- In order to isolate the filter from the effects of outside world, we prefer to use closed walls at the ends as shown in Figure 13.19.b. In such applications the effects of evanescent waves traveling back and forth between the outermost posts and the end walls must be taken into consideration during the design of the filter as will be demonstrated in the examples following.

Figure 13.19. End inductances of Open and Close Ended EWG filters.

13. 5. 2. CIRCUIT TRANSFORMATIONS  FOR  EVANESCENT WAVEGUIDE  FILTERS

In  this section, the circuit transformations of Filpro that involve Evanescent Waveguides (EWG ) will be described. Primary commands are shown in Figure 13.20 which can be displayed by invoking the menu item Waveguide Operations. Some commands are for Cross-Coupled evanescent waveguide filters (having letters CC in their names) and some for propagating waveguide filters. These commands will be described in the respective sections. Only the relevant commands will be described in this section

Figure 13.20. Commands involving evanescent waveguide elements.

- Scale Circuit/TL For EWG: This command is applied either to a selected Symmetric Inductive Pi Section with shunt legs formed by parallel LC elements (with cursor under the series arm inductor) or to a circuit section made up of Shunt Capacitor+Shunt inductor + Ideal Inverter+ Shunt inductor+Shunt Capacitor (with cursor under the inverter). The selected sections are termed as Reference Section. It scales the whole circuit such that either the Reference Section will be equivalent to an EWG with specified width (a), height (b) and dielectric constant r  at the specified frequency (for example passband center). Then, the complete circuit will be ready for conversion into EWG.

- Pi-to-EWG: A symmetric Pi section with shunt LC legs is replaced by an EWG with shunt capacitors on both sides (Figure 13.21.a). Frequency of approximation and desired EWG cross-sectional dimensions are set by the user. Length of EWG is calculated by Filpro. The capacitors of shunt legs will not be changed after the transformation if the Pi-section is the Reference Section. For all other Pi sections the capacitors will be trimmed to get a better approximation of the Pi section by the EWG piece.

Figure 13.21. Conversion of a bandpass sections with inductive Pi or inverters into EWG equivalent.

- Inv-to-EWG: An ideal inverter having shunt inductors and shunt capacitors on both sides (with inductors being identical) is equivalent to an EWG with capacitors on both sides (Figure 13.21.b). The shunt capacitors will not change after the transformation if the inverter belongs to the Reference Section. For all other sections the capacitors will change to some extent to get a better approximation of the circuit section with an EWG piece. Desired EWG cross-sectional dimensions and frequency of approximation are set by the user. Filpro calculates the length of EWG and trimmed capacitor values.

- Lsh-to-EWG: This command replaces a shunt inductor by a short or open circuited EWG with specified width (a), height (b) and dielectric constant r.  Length of EWG is calculated in terms of inverter constant and EWG specifications set by the user at the specified frequency f0 (for example passband center). Equivalence is exact only at the specified frequency.

- EWG-to-Pi, EWG-to-Tee, EWG-to-Inv: An EWG  may be replaced by its Pi, Tee equivalents, or by an ideal inverter surrounded by two identical shunt inductors (Figure 13.22 ). Frequency of approximation and EWG cross-sectional dimensions are set by the user. Length of EWG is calculated by Filpro.

Figure 13.22. Conversion of an EWG into inductive Pi, inductive Tee and inverter equivalents.

Let’s see some examples for application of some of these commands in the design of EWG bandpass filters.

13. 5. 3. NARROWBAND EWG FILTER  DESIGN

EWG filters can be designed by both Synthesis and Prototype with Inverters approaches. Inverters have some slight advantages in the ease of manipulation of the circuit topology and element values. However they best fit for narrow-medium bandwidth filters while Synthesis approach can be used for any bandwidth. Further, more detailed transformations are prepared for Synthesis approach. Therefore it will be the main tool for EWG filters on Filpro.

EXAMPLE 13. 8: A narrowband EWG filter using WR284 waveguide

- Let’s design a narrowband EWG filter using a waveguide with cross-sectional dimensions a=72.136 mm, (2.84 inhes) b=34.036 mm (1.34 inches) (WR284 waveguide). Specifications of the filter are:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, Nzero=1, Ninf=9 (degree 10, 5 resonators)

- Design command set is shown in Figure 13.23. Design / By Synthesis option is selected.

- Lumped / BPF / Equiripple / Doubly Terminated / Type B options are selected.

- In order to get the topology with series inductively coupled shunt LC resonators, the transmission zeros are selected as Nzero=1 and Ninf=9, leading to degree 10 filters which can be realized with 5 resonators.

- Invoking Standard Auto Extract command we get the circuit shown in Figure 13.23.a.

- With cursor under any series inductor the command Pi-Tee-L / Equal Shunt Inductors is applied to convert the circuit into the targeted inductively coupled shunt LC resonator form (Figure 13.23.b). This circuit is saved as EWG01.FIL for use at later stages.

- Now the circuit will be impedance scaled for realization with the desired waveguide cross-sectional dimensions. The Pi-section with series inductors plus shunt LC resonators will be converted into EWG pieces one by one. In order to carry out this process the inductors of Pi-sections must be at levels compatible with the selected EWG cross sectional dimension. All the Pi-sections have equal shunt inductors but different series inductors and it is not possible to equate them. Therefore we select one of the Pi-sections as Reference Section and scale the circuit such that the inductors of this Pi-section will be equivalent to the selected EWG piece. All the other Pi-sections will also be realized by the same EWG, but equivalence between the Pi-sections and EWG pieces will be approximate. Degradation caused by this approximation is minimized by trimming the shunt capacitors of the Pi sections. In this example we will select the Pi-section with highest series inductance as Reference Section (Figure 13.23.b).

- On Filpro impedance scaling is done by placing the cursor under the series inductor of the Reference Section and applying the command Waveguide Oprs / Scale Circuit For EWG. Filpro will then ask the parameters of the desired waveguide. We type in the width, a=72.136, the height, b=34.036, dielectric constant Er=1 and passband center, f0=1000 MHz.

- Then Filpro will ask the approximate characteristic impedance of the posts (Zpost) to be used and their lengths (LngPost). We type in, for example, ZPost=100 ohms. Length of posts must be less then the height b of the waveguide to allow for the capacitance gap. Let’s type in LngPost=33 mm. Then Filpro will display the equivalent inductance of the posts Lpost=13.197 nH. We note down this value for use at later stages and continue scaling.

- Then Filpro displays the information: fc=2077 MHz (cutoff frequency of the wavguide), Xo=97.65 ohms (characteristic impedance of the EWG), Lcr =15.54 nH (Critical End Inductance). Here we also note down the critical inductance Lcr=15.4 nH, as it will be effective in setting the terminations of the EWG filter.

- Filpro also asks to scale the circuit or not. If Yes option is selected the circuit is scaled by inserting inverters at source and load ends to a level such that the Reference Pi section of inductors will be equivalent to an EWG piece with desired cross-sectional dimensions (Figure 13.23.c).

- The impedances presented by the inverter-source resistance and inverter-load resistance combinations are what the EWG filter likes to see at its terminations. This should be provided by the input/output coupling circuits like probes, loops, tapping, matching circuits, etc. Some possible terminations will be described later in this example.

- Then the Pi-sections are converted into EWG pieces one by one by placing the cursor under the series inductors and applying the command Waveguide Operations / Pi-to-EWG. In each conversion parameters of the waveguide and posts are asked. Filpro calculates the lengths of each EWG (Figure 13.23.d). It should be noted that after conversion into EWG form, the inductances of the LC resonators in between EWG pieces will be equal to the specified Post Inductance (Lpost=13.197 nH). The circuit is saved as, for example, EWG02.FIL.

- When the first and last Pi-sections are converted into EWG we also get a shunt inductor on the source and load sides as shown in Figure 13.23.d. They are the excess inductors remaining after forming the LC resonator with the specified post inductance. They are termed as End Inductors, Lend. Their inductance will set whether the ends of the waveguide will be closed or open. If  Lend>Lcr then the ends of the waveguide will be open and if Lend<Lcr the ends of waveguide will be closed (SC ends). In this case we get Lend=17.3 nH>Lcr=15.54 nH. Therefore we will have an open ended filter if we leave the Lend as it is.  Let’s continue with this case and study the other (Closed Ended) cases later.

- With cursor under the End Inductors the command Waveguide Oprs / Lsh-to-EWG is applied to convert these inductors into OC EWG pieces (named as OE-Open EWG). Their lengths came out to be 37.8 mm, about half the length of the inner EWG pieces (Figure 13.23.e).

- Next the shunt LC resonators are converted into SC Stub+Capacitor form by applying the command Replace By Stub / Par LC-to-Cap+SC Stub: StubLng=33 mm, f1=975 MHz, f2=1025 MHz. In this conversion the resulting SC stubs represent the Posts. Therefore length of stubs should be the same as length of posts (LngPost) (Figure 13.23.f). In this circuit we note that instead of the specified Zpost=100 ohms, we got Zstub=120 ohms. This difference arises during the conversion of the lumped LC resonators to Cap+SC Stub type resonator. This is due to the fact that the passband center f0=1000 MHz of the filter is NOT far enough from the cutoff frequency fc=2077 MHz of the WR284 waveguide. This point should also be kept in mind in specifying the impedances of posts.

- Figure 13.23.g shows the return loss response. The shrink in the pass bandwidth is due to approximation of lumped element Pi-sections by EWG pieces. This shrinkage gets more pronounced as the passband center gets nearer to the cutoff frequency. It can be compensated only by designing a wider bandwidth filter.

- Figure 13.23.h shows the wideband response. The spurious passband levels seem not to be problematic.

- Treatment of inverters will be described later, after studying the versions with Closed Ends.

Figure 13.23. Design of a narrowband EWG filter using Synthesis approach.

Figure 13.23. Design of a narrowband EWG filter using Synthesis approach (cont’d).

Figure 13.23. Design of a narrowband EWG filter using Synthesis approach (cont’d).

Closed Ended Version with nonidentical posts:

Closed ends can be formed by adjusting the end inductors so that Lend<Lcr. One way to effect the end inductors is to change only the end posts by extracting some of its inductance and adding to the end inductance as described in Figure 13.24.

- The circuit of EWG02.FIL is loaded (Figure 13.24.a).

- The End Inductors are combined with the adjacent LC resonators using Combine command (Figure 13.24.b).

- Then these LC resonators are split into their L and C components by using the command Split / Two Element Resonator (Figure 13.24.c).

- Then the inductors are split into two parts to form the end inductors by using the command Split / Specify Left: L=13 nH (Figure 13.24.d). Here L=Lend=13 nH is a trial value for the end inductance selected to be less than Lcr=15.54 nH. Different values can be tried leading to different lengths and different post impedances.

- The remaining L and C are recombined to form new LC resonators representing the modified posts (Figure 13.24.e).

- The end inductors are converted into SC EWG form by applying the command Waveguide Operations / Lsh-to-EWG (Figure 13.24.f). The short circuited EWG is named as SE (SC EWG). It is seen that SC EWG length is about 31.6 mm, shorter than the Open Ended version.

- Finally the LC resonators are converted into Cap+SC Stub form by applying the command Replace By Stub / Par LC-to-Cap+SC Stub: Lng=33 mm. It is seen that impedances of the outermost posts are increased to 160 ohms while the inner posts are still at about 120 ohms. If we had extracted less inductance from the outermost LC resonators we would get smaller impedances for outermost posts at the expense of increased SC EWG lengths (Exercise?).

Figure 13.24. EWG filter with closed ends and nonidentical posts.

Closed ends with identical Posts:

- It is also possible to form identical posts as follows:

- The circuit of EWG01.FIL is loaded (Figure 13.25.a). The outermost LC resonators are split into their L and C components by applying the command Split / Two Element Resonator (Figure 13.25.b).

- Then the inductors are split into two parts by applying the command Split / Specify Left/Right Ratio=10. Here the ratio 10 is a trial value.  We get one piece of about 6 nH and other piece 0.6 nH (Figure 13.25.c). The larger inductance (6 nH) will be part of the end inductors. We Copy these inductors and then Delete them (Figure 13.25.d).

- In the remaining circuit we equate all shunt inductors by applying the command Pi-Tee-L / Equal Shunt Inductors (Figure 13.25.e).

- Then we Paste the deleted inductors into their original positions. So we have a circuit with all Pi-sections having equal shunt inductors and two single inductors at outermost positions to be used as part of the End Inductors (Figure 13.25.f).

- Next the circuit is scaled up in impedance for the EWG to be used by placing the cursor under the series inductor of the Reference Pi-section and applying the command Waveguide Operations / Scale Circuit For EWG: a=72.136, b=34.036, LngPost=33 mm, Zpost=100 ohms (Figure 13.25.g).

- Then the Pi-sections are converted into EWG pieces by applying the command Waveguide Operations / Pi-to-EWG (Figure 13.25.h).

- In converting the outermost Pi-sections into EWG, we get one more shunt inductance. Now at source and load ends we have two single inductors in parallel. We Combine them to form the End Inductors (Figure 13.25.i). Their value is about Lend=12.8 nH<Lcr=15.54 nH. So we will have Closed Ends.

- With cursor under the End Inductors the command Waveguide Operations / Lsh-to-EWG is applied to form SC EWG pieces (Figure 13.25.k). Their lengths came out to be 30.9 mm.

- Then the LC resonators are converted into Cap+SC Stub form by applying the command Replace By Stub / Par LC-to-Cap+SC Stub: Lng=33 mm (Figure 13.25.m). It is seen that all the SC stubs (representing Posts) have equal impedances of 124 ohms. Compared to the 120 ohms of the previous cases we have a slight increase in the impedances because the End Inductances are decreased. The circuit is saved as EWG04.FIL.

- Figure 13.25.n shows the passband return loss. Bandwidth shrinkage is due to conversion from LC resonators into EWG form. We should select a slightly wider bandwidth.

- These trials can be repeated by selecting different Split Ratio for the end inductors to adjust the length of Closed End section. In the limiting case when the Closed Ends get too close to the outermost posts then we may have TEM modes in the region between the posts and End walls in which case the circuit models of End sections will be similar to those of  Combline-Interdigital filters.

Figure 13.25. EWG filter with closed ends and identical posts.

Figure 13.25. EWG filter with closed ends and identical posts (cont’d).

Input-Output Couplings:

- In transitions from coaxial line systems to EWG various coupling mechanisms can be used like probes, loops together with impedance matching precautions.

- In EWG filters with coaxial terminations the input-output couplings should be made directly on the outermost resonators. Coupling can be made by loops to couple to the magnetic field of the resonators or a capacitive coupling can be made directly on the capacitively loaded ends of the outermost posts or direct tapping on the posts can be applied as in Combline and Interdigital filters.

- As an example let’s consider the circuit of EWG04.FIL which is loaded in Figure 13.26.a.

- Using Exchange command the Cap+SC Stub resonators are made adjacent to the inverters (Figure 13.26.b). With cursor under the inverters the command Inverters / Replace Inverter By / End Inverter to / Capacitive-L is applied to form a capacitive L-section with negative shunt capacitor (Figure 13.26.c). The negative shunt capacitors are absorbed into the positive capacitor of the adjacent resonators by applying Simplify Circuit or Combine commands (Figure 13.26.d).

- Figure 13.26.e shows a pictorial description of the filter. The coupling capacitors are shown in symbolic form. Actually they can be realized by extending a flat tapped probe from a side wall to the proximity of the post to create a capacitive effect. Figure 13.26.f shows the response. Degradations can be corrected by tuning a few elements.

- Figure 13.27 describes Tapping approach. The circuit of EWG04.FIL is loaded (Figure 13.27.a).

The Cap+SC Stub resonators are made adjacent to the inverters (Figure 13.27.b). With cursor under the inverters the command Tapped PCL is applied. This command displays two parameters which can be adjusted by the user slightly to get less degradation in response: fres is the resonance frequency of the section of the circuit between RS and Cap+SC Stub resonator as seen from the right of the Cap+SC Stub resonator when looking to left. The tapped circuit will be designed to have the same resonance. fq is the quarter wavelength frequency of the SC stub of the resonator and Zc is the impedance of the post to be tapped (after tapping). Usually the displayed default values give the best result. We get the tapped circuit shown in Figure 13.27.c. In this circuit the shunt SC stub adjacent to RS simulates the portion of post between tapping point and SC end. Its length is termed as LTap. The portion of the post between tapping point and capacitively loaded end is simulated as a transmission line. This approximation changes the capacitance of the resonator also slightly. Figures 13.27.d and e shows pictorial descriptions of side and top views. Figure 13.27.f shows the response.

Figure 13.26. EWG filter with capacitive input-output couplings.

Figure 13.27. EWG filter with tapped input-output couplings.

EXAMPLE 13. 9: Narrowband EWG filter using WR90 waveguide

Let’s repeat the previous example now using the waveguide WR90 with smaller cross-sectional dimensions, a=22.86 mm (0.9”) and b=10.16 mm (0.4”)

- Specifications of the filter are:

PBRipple=0.01 dB, fp1=975 MHz, fp2=1025 MHz, Nzero=1, Ninf=9 (degree 10, 5 resonators)

- Design command set is shown in Figure 13.28. Design / By Synthesis option is selected.

- Lumped / BPF / Equiripple / Doubly Terminated / Type B options are selected.

- In order to get the topology with series inductively coupled shunt LC resonators, the transmission zeros are selected as Nzero=1 and Ninf=9, leading to degree 10 filters which can be realized with 5 resonators.

- Invoking Standard Auto Extract command we get the circuit shown in Figure 13.28.a.

- With cursor under any series inductor the command Pi-Tee-L / Equal Shunt Inductors is applied to convert the circuit into the targeted inductively coupled shunt LC resonator form (Figure 13.28.b). This circuit is saved as EWG11.FIL for use at later stages, if necessary.

- The circuit is scaled for the selected waveguide by placing the cursor under the series inductor of the Reference Section (the Pi-section with highest series inductor) and applying the command Waveguide Oprs / Scale Circuit For EWG. Filpro will then ask the parameters of the desired waveguide. We type in the width, a=22.86 mm, the height, b=10.16 mm, dielectric constant Er=1 and passband center, f0=1000 MHz. Then Filpro will ask the approximate characteristic impedance of the posts (Zpost) to be used and their lengths (LngPost). We type in, for example, ZPost=100 ohms. Length of posts must be less then the height b of the waveguide to allow for the capacitance gap. Let’s type in LngPost=9 mm. Then Filpro will display the equivalent inductance of the posts Lpost=3.038 nH. We note down this value for use at later stages and continue scaling. Then Filpro displays the information: fc=6557 MHz (cutoff frequency of the wavguide), Xo=25.8 ohms (characteristic impedance of the EWG), Lcr =4.11 nH (Critical End Inductance). Here we also note down the critical inductance. Selecting Yes option, the circuit is scaled by inserting inverters at source and load ends to a level such that the Reference Pi section of inductors will be equivalent to an EWG piece with desired cross-sectional dimensions (Figure 13.28.c).

- Then the Pi-sections are converted into EWG pieces one by one by placing the cursor under the series inductors and applying the command Waveguide Operations / Pi-to-EWG. In each conversion parameters of the waveguide and posts are asked. Filpro calculates the lengths of each EWG which are about 21 and 18 mm (Figure 13.28.d). It should be noted that after conversion into EWG form the inductances of the LC resonators in between EWG pieces will be equal to the specified Post Inductance (Lpost=3.038 nH).

- When the first and last Pi-sections are converted into EWG we also get a shunt inductor on the source and load sides as shown in Figure 13.28.d. They are the End Inductors, Lend=4.27 nH. Since  Lend>Lcr the ends of the waveguide should be left open. If closed ends are desired then we can use the approaches described in the previous example. Let’s continue with the open ended case.

- With cursor under the End Inductors the command Waveguide Oprs / Lsh-to-EWG is applied to convert these inductors into OC EWG pieces (named as OE-Open EWG). Their lengths came out to be 10.4 mm, about half the length of the inner EWG pieces (Figure 13.28.e).

- Next the shunt LC resonators are converted into SC Stub+Capacitor form by applying the command Replace By Stub / Par LC-to-Cap+SC Stub: StubLng=9 mm, f1=975 MHz, f2=1025 MHz. In this conversion the resulting SC stubs represent the Posts. Therefore length of stubs should be the same as length of posts (LngPost) (Figure 13.28.f). Here we note that the stub impedances came out to be about 101 ohms, very close to the specified post impedances of 100 ohms, contrary to the prevous example where there were large deviations between the specified and obtained post impedances. This is due to the fact that the passband center f0=1000 MHz of the filter is far lower than  the cutoff frequency fc=6557 MHz of the WR90 waveguide.

- Figure 13.28.g shows the return loss response. There is almost no shrink in the pass bandwidth., owing to the fact f0<<fc.

- Conclusion: Approximations get better for f0<<fc. That is, if miniaturization is targeted then we can use waveguides with smaller cross-sectional dimensions (IF Q is not a problem!).

- Another advantage of WR90 waveguide (or the factor f0<<fc) compared to WR284 of the previous example appears in the wideband response shown in Figure 13.28.h. Spurious passbands disappeared.

Figure 13.28. EWG filter with WR90 waveguide.

Figure 13.28. EWG filter with WR90 waveguide (cont’d).

Figure 13.28. EWG filter with WR90 waveguide (cont’d).

13. 5. 4. ON WIDEBAND EWG FILTERS

The design approach presented in the previous section is also applicable to wideband filters. However in wideband filters the distances between adjacent posts come out to be too close and this causes higher order modes problem which can’t be modeled and solved on Filpro.  The discontinuities presented by the posts lead to higher order waveguide modes (TE20, TE40, etc.). In the narrowband EWG filters the distances between adjacent posts are usually large enough so that the higher order modes decay sufficiently when they reach to the adjacent posts. So these modes stay as local phenomena which effects the parameters of the relevant resonators only. They can be handled by tuning and trimming the posts and associated capacitors. However in wideband filters the higher order modes generated at a post may also travel to the adjacent posts, effecting the coupling between adjacent resonators. That is, the series inductors need be changed to account for these extra couplings which means the distance between the posts must be readjusted. Theoretically it is possible to form circuit models for an EWG piece to account for these waveguide modes, as shown in Figure 13.29. In this model the effects of higher order modes are shown as inductive Pi-sections in parallel with the inductive Pi-section of the fundamental TE10 mode. The transformers represent the strengths of fields of higher order modes relative to the fundamental mode. The difficulty of modeling lies in evaluation of these transformers because they are dependent on the geometry of posts and cross-sectional dimensions of the waveguide. For this reason a wideband EWG filter designed by the approach of Filpro can only serve as a starting point for an iterative cut and try procedure or for a full wave EM simulation approach.

If EM simulation is available it is clear that little difference exists between the treatments of EWG filters and Combline or Capacitively loaded interdigital filters. They can be treated more efficiently by taking all modes into account.

Figure 13.29. Circuit model of an EWG involving some higher order modes.

REFERENCES  AND  SUGGESTED  READINGS:

- G. L. Matthaei, L. Young, E. M. Jones, “Microwave Filters, Impedance Matching Networks and Coupling Structures”, Mc Graw Hill , NY, 1964 (Section 5.10).

- P. A. Rizzi, Microwave Engineering Passive Circuits, Prentice Hall, 1988 (pp.311-317).

- Slater, J. C., Microwave Transmission, Mc Graw Hill, NY, 1942,

- Marcuwitz, N., Waveguide Handbook, Radiation Lab. Series, vol. 10, Mc Graw Hill, NY, 1968.

- Southworth, G. C., Principles and Applications of Waveguide Transmission, D. Van Nostrand Co., Princeton, NJ 1950.

- Collin R. E., Foundations for Microwave Engineering, Mc Graw Hill Book Co., NY, 1992.

- Montgomery C. G., R. H. Dicke, and E. M. Purcell, Principles of Microwave Circuits, Rad. Lab. Series, vol. 8, Mc Graw Hill Book Co., NY.

- 1948, Schwinger, J. and D. Saxon, Discontinuities in Waveguides, Gordon and Breach Science Publishers, Inc., NY, 1968.

- L. Lewin, Theory of Waveguides, Newnes-Butterworths, 1975.

- H.J. Riblet, “Waveguide filter with nonidentical sections resonant at the same fundamental frequency and different harmonic frequencies”, U.S. Patent 3153208, Oct.1964.

- M. Morelli, I. Hunter, R. Parry, V. Postoyalko, “Stopband improvement of rectangular waveguide filters using different width resonators: Selection of Resonator widths”, 2001 MTTS Digest, pp. 1623-1626.

- G. Craven, “Waveguide bandpass filters using evanescent modes”, Electron. Lett., Vol. 2 No. 7, pp. 251-252, 1966.

- G. Craven, “Waveguide below cutoff: A new type of microwave integrated circuit”, Microwave Journal, Vol. 13, p.51, Aug. 1970.

- G. Craven and C. K. Mok, “The design of evanescent mode waveguide bandpass filters for a prescribed insertion loss characteristics”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-19, No. 3, pp. 295-308, 1971.

- C. K. Mok, “Design of Evanescent Mode Waveguide Diplexers”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-21, No. 1, pp. 43-48, Jan. 1973.

- R. V. Snyder, “New application of evanescent mode waveguide to filter design”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-25, No. 12, pp. 1013-1021, Dec. 1977.

- K. Schunemann, R. Knochel, and G. Begemann, “Components for microwave integrated circuits with evanescent mode resonators”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-25, No. 12, pp. 1026-1031, Sept. 1982.

- D. Willems, “Evanescent mode waveguide filters built in a day”, Microwaves and RF, pp. 117-124, July 1987.

- R. Levy, Hui-Wen Yao and Kawthar A. Zaki, “Transitional Combline/Evanescent Mode Microwave Filters”, 1996 MTTS Digest,

- A. Kirilenko, L. Rud, V. Tkachenko and D. Kulik, “Evanescent mode ridged waveguide bandpass filters with improved performance”, IEEE Trans. Microwave  Theory and Tech. Vol. MTT-50, No. 5, pp. 1324-1327, May 2002.