CHAPTER 16

# DUAL BAND FILTERS

16. 1. INTRODUCTION

Recent wireless applications created a demand on dual passband filters [1]-[13]. Naturally, it is always possible to form a dual band filter by cascading a bandstop filter with a bandpass filter and then use optimization to shape the response. However it is simpler to use direct synthesis techniques described in this chapter. In this chapter different approaches will be described for the design of dual band filters suitable for different applications:

- Simple lumped element BP filters without finite transmission zeros made up of second order LC resonators and inverter or LC type coupling elements can be converted into a dual passband filters with controllable bandwidths through transformations applied on the resonators. These transformations convert the second order resonators (single band resonators) into fourth order sections (dual band resonators) made up of a BP and a BS resonator. The BP resonators sets the outer edges of the two passbands, thus forming a BPF with a wide passband. The BS resonators form finite transmission zeros (FTZ) which create a stopband within the wide passband filter formed by the BP resonators, thus splitting the wide passband into two narrower passbands. Since all resonators of the original BPF are identical, the resulting FTZ’s will all be at the same frequency. The location of the FTZ’s sets the bandwidths (passband edges) of the two passbands. That is, for a given set of band edge frequencies for the two passbands, all the FTZ’s will come out to be at the same frequency. Hence it is not possible to spread the FTZ’s for example to get an equiripple stopband or to get high skirt slopes. The selectivity of the stopband response is set only by the degree of the original filter.

- If finite transmission zeros in the stopband between the two passband need be distributed to get high skirt slopes then another approach can be used. In this approach a magnitude and frequency normalized elliptic or quasi-elliptic BP filter is designed. Then the classical LP-to-BP mapping is applied on this prototype filter yielding a dual passband filter with FTZ’s. Locations of FTZ’s can be set as desired to shape the srtopband response. Since the band edge frequencies of the two passbands and the FTZ’s are all related to each other through the mapping function the two bandwidths can’t be set independently of each other. The band edge frequencies of the two passbands and the locations of the associated FTZ’s are geometrically symmetric with respect to the stopband center.

- For distributed element dual band filters with wide or medium stop bandwidths the easiest approach would be to use the natural periodic passbands of commensurate length filters. The first two pasbands can be used as the dual passbands. For such filters the two pass bandwidths come out to be equal. For filters without finite transmission zeros the two pass bandwidths can be made unequal by converting the commensurate line filter into noncommensurate form by adjusting the stub lengths. This approach is suitable for planar type filters.

- Narrowband, nonplanar, coupled resonator type filters can be treated either using the techniques described for lumped element filters or distributed element approach can be used. If lumped element approach will be used then a single band lumped element bandpass filter without finite transmission zeros can be designed to cover the first passband and then it will be converted into dual band filter by converting the single band resonators into dual band resonators using circuit transformations. In this approach the two pass bandwidths can be enforced to have equal or unequal bandwidths. If FTZ’s are needed to shape the stopband response then mapping from a normalized BP Prototype approach can be used. One can also use commensurate line distributed element Synthesis technique with or without finite transmission zeros, with the first two passbands targeted as the dual bands. If there are FTZ’s then the two passbands must have equal pass bandwidths.

- In some applications one may also use Stepped Impedance Resonator filters (SIR) to control the second bandwidth.

It will also be shown that the conversion of single band resonators into dual band resonators can be used to design bandstop filters with dual stopbands.

16. 2. LUMPED ELEMENT DUAL BAND FILTERS

On Filpro, lumped element dual band filters can be designed in two ways:

- A single passband BP filter can be converted into a dual band filter by converting the LC resonators of the single band filter into forth order resonators.

- A frequency and magnitude normalized BP prototype filter is designed and the classical LP Prototype-to-BP transformation is applied to form the desired dual band filter.

16. 2. 1. Conversion of a single band filter into a dual band filter:

A single passband BP filter made up of LC resonators can be converted into a dual band filter by converting the LC resonators into fourth order resonators. At passband center the shunt arm parallel LC resonators of a single band filter become OC while the series arm series LC resonators become SC and their element values set the pass bandwidth. These second order resonators can be converted into fourth order LC resonators which have two resonance frequencies which can be used to form the two pass bandwidths and an antiresonance (Finite Transmission Zero-FTZ) which forms the stopband between the two passbands. The transformations are shown in Figure 16.1.

Figure 16.1. Conversion of single band resonators into dual band resonators (fourth order sections).

This approach can be applied to the following classes of single PB filters:

- Single PB filters made up of series LC resonators in series arm and parallel LC resonators in shunt arm.

- Single PB filters made up of series arm series LC resonators coupled with ideal inverters.

- Single PB filters made up of shunt arm parallel LC resonators coupled with ideal inverters.

Replacement of ideal inverters with practical equivalents may lead to great degradations in responses of both passbands. Therefore optimization may be needed.

These transformations can also be applied to a single band BS filters to form dual band BS filters as well.

Let’s see some examples.

## EXAMPLE 16. 1.  Conversion of BP filter with series and parallel LC resonators into dual band

Let’s design a three resonator BP filter with following specifications:

PBRipple=0.01 dB, fp1=90 MHz, fp2=110 MHz

We will convert this filter into a dual band one later, after its design is finished. A BP filter made up of only series and shunt LC resonators can be designed using either Design / By Built-in LC Prototype approach or Design / By Synthesis approach. Let’s use Synthesis approach. A three resonator BP filter can be synthesized by assigning equal number of TZ’s at f=0 and f=:

Nzero=3 and Ninf=3.

Design steps are shown in Figure 16.2.

- Design / By Synthesis command is invoked and Lumped-BP-Equiripple-Doubly Terminated-Type B options are selected.

- Passband and Terminations are typed in. Transmission zeros Nzero=3 and Ninf=3 are entered.

- Transmission Zeros are extracted in the order Zero-Inf-Zero-Inf-Zero-Inf giving the circuit shown in Figure 16.2.a.

- Simplify Circuit command combines the adjacent element to form LC resonators (Figure 16.2.b). The circuit is saved as  DP101.

- With cursor under the resonators the command Resonator Oprs / Single PB-to-Dual PB / fp1B=150 MHz, fp2B=155 MHz is applied converting the second order LC resonators into  fourth order resonators creating the second passband with passband edge frequencies fp1B=150 MHz and fp2B=155 MHz. Figures 16.2.c and d show the dual band filter circuit and its response. As it is seen the dual band resonators consist of a BP and a BS resonator.

- As and example for a dual band filter with a wider second passband the circuit of DP101 is loaded (Figure 16.2.e) and with cursor under the resonators the command Resonator Oprs / Single PB-to-Dual PB / fp1B=150 MHz, fp2B=250 MHz is applied giving the circuit of Figure 16.2.f. It is seeen that the transformations has no approximation. The pasband edge frequencies and return loss levels are set as desired.

Figure 16.2. Conversion of a lumped element single band BPF into dual band BPF.

Figure 16.2. Conversion of a lumped element single band BPF into dual band BPF.

## EXAMPLE 16. 2.  Conversion of a BPF with inverters and LC resonators into dual band BPF

Let’s now design a three shunt LC resonator BP filter with inverters following specifications:

PBRipple=0.01 dB, fp1=90 MHz, fp2=110 MHz

Design steps are shown in Figure 16.3.

- Design / By Prototype with Inverters command is invoked. Prototype Degree, PBRipple and band edges are typed in. Find Circuit command gives the circuit shown in Figure 16.3.a.

- Inverters / Equate All Elements / Equal Inds command equates all shunt inductors (Figure 16.3.b).

- If desired the parameters of the resonators can be scaled to desired level by applying the command Inverters / Scale Cct by Invs / Specify Inductor: L=50 nH (e.g.) which inserts inverters at both source and load ends scaling  all shunt inductors to 50 nH (Figure 16.3.c).

- With cursor under the resonators the command Resonator Oprs / Single PB-to-Dual PB / fp1B=180 MHz, fp2B=220 MHz is applied converting the second order LC resonators into  fourth order resonators creating the second passband with passband edge frequencies fp1B=180 MHz and fp2B=220 MHz. Figure 16.3.d shows the dual band filter circuit and its response.

- For wide stopbands between the two passbands replacement of ideal inverters by practical ones lead to degradations in response. Let’s study these effects.

- With cursor under the inner inverters the command Inverters / Replace Inverter By / Constant Reactance / Pi-section is applied to replace the inverters by ideal constant reactance Pi-sections. This conversion doesn’t cause any degradation in response (Figure 16.3.e).

- Then with cursor under the shunt legs of the Pi-sections, the command Constant Reactances / Replace Constant Reactance By / Parallel LC / f1=90 MHz, f2=220 MHz (full band !) is applied. Thus, shunt constant reactances are replaced by parallel LC resonators (Figure 16.3.f).

- Simplify Circuit command combines the new LC resonators with the original LC resonator.

- Next, with cursor under the elements of the Pi-sections, the command Constant Reactances / Replace Constant Reactance By / Series LC / f1=90 MHz, f2=220 MHz (full band !) is applied. Thus, series constant reactances are replaced by series LC resonators (Figure 16.3.g). The effects of these conversions are apparent in the return loss plot. Since 90-220 MHz is a wide band, degradations are appreciable and unavoidable.

- The inverters at ends are replaced by constant reactance L-sections using the command Inverters/Replace Inverter By/End Inverter to/Const. Reactance-L(2). This L-section have a negative series constant reactance and a positive shunt constant reactance (Figure 16.3.h).

- Then the series constant reactances at source and load ends are replaced by series capacitors using the command Constant Reactances / Replace Constant Reactance By/ Capacitor/fo=100 MHz.

- Finally, the shunt constant reactances at source and load ends are replaced by shunt inductors using the command Constant Reactances / Replace Constant Reactance By / Inductor/ fo=100 MHz (Figure 16.3.i).

- Simplify Circuit command gives the circuit shown in Figure 16.3.k. We get appreciable degradation especially in the second passband. Optimization is needed to improve the response (Exercise?).

EXERCISE: Repeat the example by designing a filter with ınverters and series LC resonators.

OBSERVATION-1: It should be noted that the series resonators in shunt arm and parallel resonators in series arms shape up the stopband response. They act like Finite Transmission Zeros (FTZ’s), all at the same frequency. It is NOT possible to relocate these FTZ’s for example to get and equiripple stopband response.

OBSERVATION-2: Location of the FTZ’s is dependent on relative bandwidths of the two bands. It is closer to the narrower passband.

OBSERVATION-3: This approach is NOT applicable to filters with FTZ’s   (Exercise?).

OBSERVATION-4: This approach is NOT applicable to filters with Nzero  Ninf  (Exercise?).

Figure 16.3. Conversion of a BPF with shunt resonators and inverters into dual band filter.

Figure 16.3. Conversion of a BPF with shunt resonators and inverters into dual band filter.

16. 2. 2. Dual band filter design by mapping from a normalized BP prototype

We have seen that although the previous approach is very flexible in setting the bandwidths of the two passbands independently of each other, it is not possible to place the Finite Transmission Zero’s (FTZ’s) at different frequencies for shaping up of the stopband response. All FTZ’s come out to be at the same frequency, somewhere in the stopband whose location is dependent on relative bandwidths of the two bands. If the locations of the FTZ’s are important and if the relative bandwidths of the two passbands are not of primary importance then the approach that will be described in this section can be used.

The classical LP Prototype-to-BP transformation converts a frequency and magnitude normalized LP prototype filter into a BP filter through the mapping function

w= (wo/B)(w/wo-wo/w) 								(16.1)

where w is the normalized frequency. The normalized prototype LP filter have the band edge wp=1 and RS=RL=1.

If this transformation is applied to a normalized BP prototype then we get a dual passband BP filter [12]. The normalized BP prototype may be designed as having the upper band edge w2=1 and lower band edge w1<w2as described in Figure 16.4. In the figure the frequencies w1a, w2a and w1b, w2b are the desired passband edge frequencies of the two passbands which must be selected to satisfy the geometric symmetry constraint around the geometric center of the stopband wo:

w1a w2b =w2a w1b =wo2							(16.2)

or

fp1A fp2B = fp2A fp1B = fo2 							(16.3)

The lower band edge w1 of the prototype should be evaluated from the dual band filter specifications as

w1= Bs / B									(16.4)

where Bs is the stop bandwidth and B is the difference between outer edges of the two passbands:

Bs = w1b-w2a  									(16.5)

B = w2b-w1a  									(16.6)

Hence w1 may also be calculated as

w1 = Bs / B = (f1b-f2a) / (f2b-f1a) 						(16.7)

With the transformation given in Eq. 16.1, the lower edge w1  is mapped to the inner passband edges w2a and w1b  while the upper passband edge w2=1 of the prototype is mapped to the outer edge frequencies w1a and w2b.

In a typical design we select only three of the desired passband edge frequencies w1a, w2a, w1b and w2b. Then the last one is calculated using Eq. 16.2 (or 16.3). Then the bandwidths B and Bs are found using Eqs. 16.5 and 16.6. Finally Eq. 16.4 is used to find w1. Once the lower edge w1 of the normalized BP prototype is determined (together with the degree and passband ripple necessary for the targeted filter specifications) then using any one of the available design techniques, a normalized lumped element BP filter can be designed with passband edges

fp1= w1 /2   and   fp2=w2/2 =1/2 = 0.159 					(16.8)

and

RS=RL=1

Then this normalized BP filter will be treated as if it is a LP prototype and the classical LP-to-BP mapping is applied to yield a dual band lumped element BP filter.

The lower stopband of the prototype BP filter is mapped as the stopband of the dual band filter. Therefore the FTZ in the lower stopband of the prototype filter can be used to improve the stopband selectivity of the dual band filter. The FTZ’s in the upper stopband of the prototype filter are mapped to the outer stopbands of the dual band filter.

This approach yields two passbands whose bandwidths are geometrically symmetric about the stopband center fo. Therefore in this approach we don’t have the flexibility of setting the two pass bandwidths independently. The advantage of this approach appears in filters with finite transmission zeros (FTZ’s). If some FTZ’s are placed into the normalized prototype then they will also be mapped as FTZ’s which can be used to shape the stopband response.

Figure 16.4. Mapping between a normalized BPF and dual band filter.

EXAMPLE 16. 3. A dual band filter with equal ripple stopband

Let’s design a dual band filter with PBRipple = 0.01 dB, using a degree 6 BPF with finite transmission zeros whose locations are adjusted to get an equal ripple stopband wbeteen the two passbands. The passband edge frequencies of the first passband is desired to be

fp1A=90 MHz, fp2A=110 MHz

The lower edge of the second passband is fp1B=180 MHz. Then its upper edge fp2B is calculated using Eq. 16.3 as follows:

fp2B = (fp2A) (fp1B) / fp1A = (110) (180) / 90 = 220 MHz

Thus, the second passband will cover the range from fp1B=180 MHz to fp2B=220 MHz, leading to a bandwidth of  40 MHz while the first passband has a bandwidth of 20 MHz.  This difference gets larger when the passbands get away from each other. The geometric center of the stopband fo is calculated using Eq. 16.3 as

fo = 140.71 MHz

Now we are ready to calculate the parameters of the normalized BP prototype.

Using Eq. 16.5 and 16.6, we get the lower passband edge frequency w’1 of the normalized prototype:

w1= Bs / B = (f1b-f2a) / (f2b-f1a) =(180 - 110) / (220 - 90) = 70 / 130 = 0.538

So, we will design a normalized BP filter with lower band edge wp1= w1= 0.538 and upper band edge wp2 =wc = 1. Next we convert these parameters into the form that is needed in Filpro, as follows:

fp1 = w1 / 2 = 0.08574

fp2 = wc / 2 =  0.159

The FTZ’s can be placed during the design of the normalized BP prototype to get the desired selectivity or stopband loss level.

Design steps of the normalized prototype BPF:

Some of the possible normalized prototype BP filters  can be designed as described in Figure 16.5:

- Design / By Synthesis option is selected.

- Filter Type: Lumped / BP / Equiripple / Doubly Terminated / Type B options are selected.

- PB and Terminations are typed in as fp1=0.08574, fp2=0.159, RS=RL=1.

- Transmission Zeros are typed in as Zero, Inf, Inf, Inf, FTZ=0.025 to form a BPF with degree 6. Here we place the FTZ in the lower stopband of the prototype filter which will map into the stopband of the dual band filter. The frequency of the finite transmission zero can be tuned to adjust the lower stopband attenuation. In this case we tune it to get a lower stopband attenuation at about 40 dB. We have placed three TZ’s at infinity to get higher slope in the upper stopband. We couls as well place more TZ’s at f=0 to increase the stopband level of the dual band filter. At least one TZ is needed at f=0 because the FTZ in lower stopband must be extracted by partial pole removal from the TZ at f=0.

- Then the transmission zeros are extracted in the order Inf-FTZ-Zero-Inf-Inf. We get the circuit shown in Figure 16.5.a. It is saved as P103. We could as well make the extraction in different orders or manipulate the extracted circuit to get different toplogies. For example in this circuit we can apply the command Pi-Tee-L / Equal Shunt Inductors (or Capacitors) to form a coupled resonator version of the circuit (Figure 16.5.b). Let’s save it as P104.

- Another topology can be formed as follows. The circuit of P103 is loaded and, with cursor under the inductive Lright section, the command Pi-Tee-L / LRight-to-LLeft is applied converting it into an Lleft section.  Pressing the keys CTRL+W moves all transformers rightmost giving the circuit of Figure 16.5.c. It is saved as  P105. The main property of this circuit is that there are two  shunt inductors, symmetrically placed at input and output sides of the circuit.

Design steps of the dual band filters:

The normalized BP prototype filters designed above can be used to design the dual band filters as described in Figure 16.6:

- Design / By User Defined Prototype command is invoked.

- The prototype to be used is loaded using Load a Prototype option. Let’s load P103.

- Filter Type: Lumped –BPF options are selected.

- PB and Termination command is invoked and the outermost edge frequencies of the two passbands are typed in: fp1=fp1A=90 MHz, fp2=fp2B=220 MHz. {Care is needed at this stage !! }

- Find Circuit command gives the circuit shown in Figure 16.6.a. Change Units command may be needed at this stage to set the units of inductors and capacitors.

- Note that the capacitors of the prototype are mapped as parallel LC resonator while inductors are mapped as series LC resonators. Hence the parallel LC resonator of the prototype is mapped as fourth order section. Element values may come out to be better behaved if the fourth order section is split into two second order sections using Split command (Figure 16.6.b). Dual band filter is ready. Its response is shown in Figure 16.6.e. The resulting circuit is saved as DP103.

- Using the same set of commands we can design other alternative solutions, as described in Figures 16.6.c and d.

- The single TZ at f=0 of the prototype filter is mapped as a FTZ at the stopband center fo=140.7 MHz. Note that the FTZ of the prototype in its lower stopband is mapped to two frequencies on the two sides of the stopband center as seen in the insertion loss plot. Thus, considering also the FTZ at the stopband center, we get three FTZ’s to shape the stopband response. It is equiripple.

Exercise: Design prototype filters as duals of P103, P104 and P105 and then design the corresponding dual band filters to increase the number of alternative solutions.

Exercise:  Design dual band filters by setting Nzero=3 and Ninf=1 in the prototype filters.

Exercise: Design higher order filters with more FTZ’s.

Figure 16.5. Design of normalized BP prototype filters.

Figure 16.6. Design of dual band BP filters using BP prototypes.

16. 2. 3. DUAL BAND BANDSTOP FILTERS

The transformation of second order LC resonators into fourth order LC resonators can also be used to convert a single Band BSF into a dual band BSF. This is described in the following examples.

## EXAMPLE 16. 4. Conversion of a BSF with series and parallel LC resonators into dual band BSF

Let’s design a five resonator BS filter with following specifications:

PBRipple=0.01 dB, fp1=80 MHz, fp2=120 MHz

We will convert this filter into a dual band one later, after its design is finished. It should be noted that for BS filters, on Filpro the frequencies fp1 and fp2 are the 0.01 dB band edge frequencies of the lower and upper passbands. The desired stopband edge frequencies with the targeted stopband level have to be set through trials on fp1 and fp2.

On Filpro BS filters can be designed using either Design / By Built-in LC Prototype approach or Design / By Prototype with Inverters approach. Let’s first use Built-in LC Prototype approach.

In Figure 16.7, the design steps for a five resonator BSF with LC resonators is described with the following specifications:

PBRipple = 0.01 dB, fp1=80 MHz, fp2=120 MHz.

Design command set is shown in Figure 16.7.

- Design / By Built-in LC Prototype option is selected.

- Lumped / BSF / Chebyshev options are selected.

- Terminations are set as RS=RL = 50 ohms.

- PB Specs and Prototype Degree option is selected.

- After typing in PBRipple, fp1, fp2 and Prototype Degree N=5, Find Circuit command gives the circuit shown in Figure 16.7.a. Its response shows the lower and upper passbands and the resulting stopband. Passband edges are 80 and 120 MHz while a 25 dB stopband is observed between 88 MHz and 108 MHz.

- In order to form a second stopband, for example in the frequency range of fp1B=180 MHz, fp2B=250 MHz, with cursor under the resonators the command Resonator Oprs / Single Band-to-Dual Band / fp1B=180 MHz, fp2B=250 MHz is applied  converting all the second order LC resonators into fourth order ones (Figure 16.7.b).

- In order to see if better behaved element values will come out or not, Split command is applied to the fourth order sections to convert them into series or parallel combinations of second order resonators (Figure 16.7.c).

- Figure 16.7.d shows the response of the dual band BS filter. The specifications of fp1B=180 MHz and fp2B=250 MHz led to a 25 dB stop bandwidth for the second stopband between the frequencies 190 MHz and 221 MHz.

Exercise:  Repeat the design by specifying narrower second passband.

Exercise: Use Design / By Prototype with Inverters approach to design the same dual stopband filters.

Figure 16.7. Conversion of a single stopband BSF into a dual stopband BSF.

16. 3. PLANAR DUAL BAND MICROWAVE FILTERS

Distributed element dual passband filters can be designed in two ways:

- Dual band filters using periodic passbands:

Commensurate line BP filters have periodic passbands with a period of 2fq where fq is the quarter wavelength frequency. The nearest spurious passband of a BPF centered at fo is loacated at 2fq-fo. It can be used as the desired second passband by eliminating the other passbands using a LPF. These two passbands are mirror images of each other around fq and have equal bandwidths. That is, the two passbands are separated by a stopband centered around fq. The two pass bandwidths can be adjusted to a limited extent as will be described later. The width of the stopband effects the element values strongly. Therefore the planar dual band filters may be classified in accordance with the width of the stopband.

- Dual band filter design by transformations from single band filters:

In the second approach a commensurate line BPF designed to work as a single band filter is converted into a dual band filter by transforming their single band resonators into dual band resonators, in the same way as in the lumped element dual band filters discussed in the previous section. In this case we will not use the spurious passband at as the second passband, but the second passband will be created by brute force. In this approach locations of the two passbands and their bandwidths are independent of fq. The only constraint is that both passbands must be located below fq. This means fq can be selected as a free design parameter. It may be set as high as possible for size reduction.

- Stepped resonator filters as dual band filters

The usual edge coupled line filters with stepped impedance resonators aims at shifting the first spurious passband away from 2fo, or, if possible, eliminate it completely. In dual band applications we just want to shift the passband around 2fo to the desired location without degrading its return loss response. We will show that it is possible to relocate the first spurious passband to desired location on the two sides of 2fo, within limited ranges using stepped resonators.

Let’s inspect the approaches through examples.

16. 3. 1. PLANAR DUAL BAND FILTERS USING PERIODIC PASSBANDS

Wide stopband planar microwave filters can be designed using commensurate length distributed element filter synthesis approach. Such filters have periodic passbands. Denoting the passband center with fo and the quarter wavelength frequency with fq, the first two passbands will be centered at fo and 2fq-fo. Hence the two passbands will have arithmetic symmetry with respect to the stopband center fq. Therefore once the two passbands are selected, the quarter wavelength frequency fq is automatically set as the arithmetic mean of the two passband centers.

On Filpro we may use two approaches for the design: Design / By Synthesis and Design / By Prototype with Inverters. Each approach has slight advantages. Therefore both approach will be described. The resulting filters will have OC and SC stubs coupled with transmission lines or ideal inverters. Equivalent circuit transformations can be used to convert the inverters into transmission lines and stubs. If desired the SC stubs can be converted into OC stubs or vice versa. Initially the two bandwidths will come out to be equal. However the second bandwidth is controllable to a certain extent, as will be described in the following examples.

EXAMPLE 16. 5. Design of a wide stopband dual band filter using Synthesis approach

Let’s consider a three resonator dual band filter with PBRipple= 0.043 dB (20 dB return loss).

The two passbands separated by a wide stopband and will be named as A and B with the following specs:

fp1A= 900 MHz,  fp2A= 1100 MHz,  fp1B=1900 MHz,  fp2B=2100 MHz.

The quarter wavelength frequency must be the arithmetic mean of the two passband centers:

fq= 1500 MHz.

Design / By Synthesis approach yields exact circuits with or without contributing Unit Elements (UE: quarter wavelength TL pieces). For wide stopband dual band filters TL pieces are needed to avoid undesired couplings between adjacent resonators.

A three resonator filter will have a total degree of N=6. Coupling between the three resonators will be provided by two UE’s. So, in using Design / By Synthesis approach the transmission zeros can be selected as follows:  Nzero=1, Ninf=3, NUE=2.

The design command set is shown in Figure 16.8.

- Design / By Synthesis command is invoked.

- Distributed Element / BP / Equiripple / Doubly Terminated / Type B options are selected.

- Passband and Terminations are typed in for the first passband only: fp1=900 MHz, fp2=1100 MHz with fq=1500 MHz. Transmission zeros are type in. The second passband will also appear as shown in the inset plot between fp1B=1900 MHz and fp2B=2100 MHz.

- The transmission zeros are extracted in the order: Inf-UE-Inf-Zero-UE-Inf. We get the circuit shown in Fig.16.8.a.

- The single TZ at f=0 appears as a single shunt SC stub in the middle of the circuit. It will be distributed into the circuit using Kuroda Transformations to form OC+SC stub resonators. This is done by placing the cursor under the UE’s and applying the command Kuroda / Distribute Stub. Thus, we place a shunt SC stub in parallel with each shunt OC stub (Figure 16.8.b).

- It is preferable to have identical shunt SC stubs. This is possible by placing the cursor under the UE which has shunt SC stubs on both sides and applying the command Kuroda / All Equal Stubs / Include End Stubs. This command equates all SC stub impedances and combines them with the adjacent OC stubs thus forming OC and SC stub resonators (Figures 16.8.b-c-d). The circuit is saved as  DP01A.fil.

- Impedances of the OC and SC resonators and the UE’s seem to be realizable in microstrip form without any further adjustment (Figure 16.8.e).

- The response is periodic. The first two passbands are symmetrically located around the frequency fq=1500 MHz. The nearest higher order passband pair is located symmetrically around the 3fq=4500 MHz. The other pairs are located around odd multiples of fq. These spurious (higher order) passbands can be eliminated by cascading a LPF.

- The two pass bandwidths are equal. They can be made unequal as will be described later in this example.

- If advantageous, it is possible to make all stubs OC or all SC. This is possible as described below.

Alternative solution with all OC Stubs:

- The circuit of DP01A is loaded (Figure 16.9.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual PB is invoked. Then a new window will appear displaying the options Two OC Stubs, Two SC Stubs, OC and SC stubs, Set Lngs of SC and OC Stbs. Selecting Two OC Stubs option, a window will appear for typing in the band edge frequencies f1B and f2B of the second passband (Passband B). The default values are the usual second passband of the periodic response of the filter. If no change is desired in this passband then Enter key is pressed giving the circuit shown in Figure 16.9.b. All stubs are of OC type with length of one being twice the adjacent one. Impedances seem to be realizable.

- Figure 16.9.c shows a realization in microstrip form.

- Figure 16.9.d shows the return loss and insertion loss responses. We note the appearance of spurious passbands near the desired two passbands.

The higher order passbands around the odd multiples of fq=1500 MHz still exist and need be eliminated by a LPF.

Alternative solution with all SC Stubs:

- The circuit of DP01A is loaded (Figure 16.10.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual PB / Two SC Stubs is applied. In the screen a window will appear for typing in the band edge frequencies f1B and f2B of the second passband. The default values are the usual second passband of the periodic response of the filter. If no change is desired in this passband then Enter key is pressed  giving the circuit shown in Figure 16.10.b. All stubs are of OC type with length of one being twice the adjacent one. Impedances seem to be realizable. Figure 16.10.c shows a realization in microstrip form.

- Figure 16.10.d shows the return loss and insertion loss responses. We note that the spectrum is cleaner compared to the all OC stub alternative. There are no spurious passbands near the desired two passbands.

The higher order passbands around the odd multiples of fq=1500 MHz still exist and need be eliminated by a LPF.

Figure 16.8. Design of a wide stopband dual band filter having both  SC and OC stubs.

Figure 16.9. Realization of the wide stopband dual band filter using only OC stubs.

Figure 16.10. Realization of the wide stopband dual band filter using only SC stubs.

Narrower second passband alternative having only OC stubs:

The second passband can be made narrower by converting the commensurate line filter into a noncommensurate one. That is the length of stubs will not be equal or multiples of each other. This is done by converting the OC+SC stub resonator into two OC stubs with noncommensurate lengths, as described below.

- The circuit of DP01A is loaded (Figure 16.11.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual PB /  Two OC Stubs is applied. In the screen a window will appear for typing in the band edge frequencies f1B and f2B of the second passband. We type in the desired passband edges, for example fp1B=1950 MHz and fp2B=2050 MHz. The algorithm finds the solutions for the impedances and lengths of the OC stubs by incremental changes in lengths of the stubs. We get the solution shown in Figure 16.11.b. All stubs are of OC type with length of the longer ones being greater than twice the length of the shorter ones.

- Impedances seem to be realizable. Figure 16.11.c shows a realization in microstrip form.

- Figure 16.11.d shows the return loss and insertion loss responses. We note that the spectrum has spurious passbands near the desired two passbands.

Narrower second passband alternative having only SC stubs:

The second passband can also be made narrower by converting the OC+SC stub resonator into two SC stubs with noncommensurate lengths, as described below.

- The circuit of DP01A is loaded (Figure 16.12.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual PB / Two SC Stubs is applied. We type in the desired passband edges, for example fp1B=1950 MHz and fp2B=2050 MHz. We get the circuit shown in Figure 16.12.b. All stubs are of SC type with length of the longer ones being less than twice the length of the shorter ones.

- Impedances seem to be realizable. Figure 16.12.c shows a realization in microstrip form.

- Figure 16.12.d shows the return loss and insertion loss responses. We note that the spectrum is cleaner compared to the OC stub case. That is the spurious passbands near the desired two passbands are not too close to the passbands.

Narrower second passband alternative having both OC and SC stubs:

The second passband can also be made narrower by adjusting the lengths and impedances of both OC and SC stubs. However this approach needs adjustments in the transmission line impedances and lengths also. Therefore the resulting response needs be improved through optimization.

- The circuit of DP01A is loaded (Figure 16.13.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual PB / OC and SC Stubs is applied. The desired passband edges are typed in, for example fp1B=1950 MHz and fp2B=2050 MHz. We get the circuit shown in Figure 16.13.b. The SC stubs are longer than the OC stubs.

- Figure 16.13.c shows a realization in microstrip form.

- Figure 16.13.d shows the return loss and insertion loss responses. It is clear that the second passband is degraded heavily. We are far from the targeted response. The two reasons for the degradations are as follows: This transformation replaces the OC+SC stub resonators with their equivalent OC+SC stub resonators having proper second passband. However the transmission lines also need modifications.  The second reason is that this transformation yields different lengths for the outer and inner resonators which in turn leads to shifts in the second passband. These points will be clearer when we treat the same problem with Design / By Prototype with Inverters approach, later in the next example. With the present form, the desired solution can only be obtained through optimization or we may follow the trial-error approach for setting lengths of SC and OC stubs, as described next.

Figure 16.11. Design of the wide stopband dual band filter with narrower second passband

having only OC stubs.

Figure 16.12. Design of the wide stopband dual band filter with narrower second passband

having only SC stubs.

Figure 16.13. Design of the wide stopband dual band filter with narrower second passband having both OC and SC stubs.

Setting Lengths of both SC and OC stubs:

The command Resonator Oprs / Stubs for Dual Passband / Set SC and OC Stub Lngs: LngSC=…, LngOC=… is a manual version of the command Resonator Oprs / Stubs for Dual Passband / SC and OC Stubs. It is prepared to give more freedom to the designer for closer control on the parameters of the dual band filter. Using this command trial-error type designs can be made to study the effects of lengths of stubs. Some examples given below.

Narrower second passband by decreasing length of OC stubs only:

The second passband can be made narrower by decreasing the length of OC stubs while keeping the length of SC stubs the same. Decreasing the length of OC stub means decreasing the capacitive reactance of the OC+SC stub resonator. This operation narrows down the second passband while it is shifted upward in frequency. The degradation in the first passband is compensated by changing the impedances of both OC and SC stubs while keeping the length of SC stub the same. The operation is described in Figure 16.14:

- The filter DP01A is loaded (Fig.16.14.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual Passband / Set SC and OC Stub Lngs: LngSC=49.965 mm, LngOC=40 mm is applied. Thus we decrease the length of OC stub from the original 49.965 mm to 40 mm while keeping the length of the SC stub the same (Fig.16.14.b). Figure 16.14.c shows a pictorial description of the filter.

- This operation doesn’t effect first passband while the second passband gets narrower accompanied by an upward shift in frequency (Fig.16.14.d). Also the return loss level of the second passband is decreased.

- Figure 16.14.e shows the response when the length of OC stubs decreased to 35 mm. The second passband is narrowed down further.

- The limiting value for the length of OC stub is half the original length which is 25 mm for this example. For this case the second passband disappears as shown in Fig.16.14.f. The next passband now appears to be the second passband. This approach may be used to design dual band filters with very wide stopbands.

Narrower second passband by changing lengths of both OC and SC stubs:

In the previous case, in order to get a narrower second passband we had decreased the length of OC stub. This led to shift in the location of the second passband. In order to locate the second passband to the desired frequency range, we can compensate the decrease in LngOC by increasing the length of the SC stub by trials. The operation is described in Figure 16.15:

- The filter DP01A is loaded (Fig.16.15.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual Passband / Set SC and OC Stub Lngs: LngSC= 54.5 mm / LngSC = 47 mm is applied. Here, the lengths are found by trials targeting to locate the second passband in the frequency range of fp1B=1950 MHz and fp2B=2050 MHz. We have achived this target, but the return loss of the second passband needs improvement. It can be improved by a slight optimization using Monte Carlo-Best Case routine. One result is shown in Figure 16.15.c in narrow and wide span responses.  Equal ripple return loss is achieved quickly, with a slightly better return loss level than the original one.

Figure 16.14. Narrowing the second passband by decreasing length of the OC stubs.

Figure 16.15. Narrowing the second passband by decreasing lengths of the OC stubs and

increasing lengths of SC stubs.

Wider second passband alternative having only OC stubs:

The second passband can be made wider by converting the commensurate line filter into a noncommensurate one. That is the length of stubs will not be equal or multiples of each other. This is done by converting the OC+SC stub resonator into two OC stubs or two SC stubs with noncommensurate lengths, as described below.

- The circuit of DP01A is loaded (Figure 16.16.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual PB / Two OC Stubs is applied. In the screen a window will appear for typing in the band edge frequencies f1B and f2B of the second passband. We type in the desired passband edges, for example fp1B=1800 MHz and fp2B=2200 MHz. The algorithm finds the solutions for the impedances and lengths of the OC stubs by incremental changes in lengths of the stubs. The displayed default value gives the circuit shown in Figure 16.16.b. All stubs are of OC type with length of the longer ones being less than twice the length of the shorter ones.

- Some impedances are too large, making realization in microstrip impossible. One may try suspended stripline version. Figure 16.16.c shows a realization in microstrip form.

- Figure 16.16.d shows the return loss and insertion loss responses. We note that the spectrum has spurious passbands near the desired two passbands. The degradation in the second passband is due to the need of adjustments in the parameters of the transmission line also.

Wider second passband alternative having only SC stubs:

The second passband can also be made wider by converting the OC+SC stub resonator into two SC stubs as described below.

- The circuit of DP01A is loaded (Figure 16.17.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual PB / Two SC Stubs is applied. In the screen a window will appear for typing in the band edge frequencies f1B and f2B of the second passband. We type in the desired passband edges, for example fp1B=1800 MHz and fp2B=2200 MHz. All stubs are of SC type with length of the longer ones being greater than twice the length of the shorter ones.

- Impedances seem to be realizable. Figure 16.17.c shows a realization in microstrip form.

- Figure 16.17.d shows the return loss and insertion loss responses. We note that the spectrum is cleaner than the OC stub case. That is the spurious passbands near the desired two passbands are not too close to the passbands. The degradation in the second passband is due to the need of adjustments in the parameters of the transmission line also.

Wider second passband alternative having both OC and SC stubs:

The second passband can also be made wider by adjusting the lengths and impedances of both OC and SC stubs. However this approach needs adjustments in the transmission line impedances and lengths also. Therefore the resulting response needs be improved through optimization.

- The circuit of DP01A is loaded (Figure 16.18.a). With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual PB / OC and SC Stubs is applied. The second passband edges, fp1B=1800 MHz and fp2B=2200 MHz are typed in. We get the circuit shown in Figure 16.18.b. The OC stubs are longer than the SC stubs.  Figure 16.18.c shows a realization in microstrip form.

- Figure 16.18.d shows the return loss and insertion loss responses. The second passband seems be narrower than the specified bandwidth. This is due to different stub lengths for inner and outer resonators and due to the existence of the transmission line. The response can be corrected by tuning or optimization or we may follow the trial-error approach for setting lengths of SC and OC stubs, as described next.

Figure 16.16. Dual band filter with wider second passband having only OC stubs.

Figure 16.17. Dual band filter with wider second passband  having only SC stubs.

Figure 16.18. Dual band filter with wider second passband having both OC and SC stubs.

Wider second passband by decreasing length of SC stubs:

The second passband can be made wider by decreasing the length of SC stubs while keeping the length of OC stubs the same. Decreasing the length of SC stub means changing the inductive reactance of the OC+SC stub resonator. This change will effect the passband center and band edges of all passbands. The  degradation in the first passband can be compensated by changing the impedances of both OC and SC stubs while keeping the length of SC stub the same. This operation widens the second passband accompanied with an upward shift in frequency. The operation is described in Figure 16.19:

- The filter DP01A is loaded (Fig.16.19.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual Passband / Set SC and OC Stub Lngs: LngSC=40 mm, LngOC=49.965 mm is applied. Thus we decrease the length of SC stub to 40 mm while keeping the length of the SC stub the same (Fig.16.19.b).

- Fig.16.19.c shows a pictorial description of the filter.

- This operation doesn’t effect first passband while the second passband gets wider accompanied by an upward shift in frequency. Also several spurious passbands arise in nearby frequencies (Fig.16.19.d). Further, the return loss level of the second passband is decreased.

- Figure 16.19.e shows the response when the length of OC stubs decreased to 35 mm. The second passband is widened further with an appreciable degradation in return loss.

- Figure 16.19.f shows the response when the length of OC stubs decreased to half of the original length, 25 mm. The second passband is widened further with an almost equal ripple return loss. This value seems to give the widest second passband.

Wider second passband by changing lengths of both SC and OC stubs:

In the previous case, in order to get a narrower second passband we had decreased the length of SC stubs. This led to a shift in the second passband. In order to adjust the location of the second passband we can compensate the effect of the decrease in LngSC by increasing LngOC. The operation is described in Figure 16.20:

- The filter DP01A is loaded (Fig.16.20.a).

- With cursor under the OC+SC stub resonators, the command Resonator Oprs / Stubs for Dual Passband / Set SC and OC Stub Lngs: LngSC= 54.5 mm / LngSC = 47 mm is applied. Here, the lengths are found by trials targeting to locate the second passband in the frequency range of fp1B=1950 MHz and fp2B=2050 MHz. We have achived this target, but the return loss of the second passband needs improvement. It can be improved by a slight optimization using Monte Carlo-Best Case routine. One result is shown in Figure 16.20.c in narrow and wide span responses.  Equal ripple return loss is achieved quickly, with a slightly better return loss level than the original one.

Figure 16.19. Widening the second passband by decreasing length of the SC stubs.

Figure 16.20. Widening the second passband by decreasing length of  SC stubs and

increasing lengths of OC stubs.

## EXAMPLE 16. 6. Prototype with Inverters approach for wide stopband dual band filters

This example will be a repetition of the previous example using Design / By Prototype with Inverters approach. In the previous example we have used Design / By Synthesis approach and got Unit Elements (quarter wavelength TL pieces) as coupling elements between the stub resonators. In this approach coupling elements will be ideal inverters. The ideal inverters can be replaced by TL pieces to give (almost) the same circuits as in the previous example. So, in essence both approaches can give the same circuits. However in some cases inverters may be replaced by more sophisticated practical equivalents, like TL pieces surrounded by shunt SC stubs to yield better approximations. Let’s see the details on the example.

Let’s design a three resonator filter with the same specifications as the previous example:

PBRipple = 0.043 dB, fp1A=900 MHz, fp2A=1100 MHz, fq = 1500 MHz

The second passband will then be

fp1B=1900 MHz, fp2B=2100 MHz.

Design command set is shown in Figure 16.21.

- Design / By Prototype with Inverters option is selected.

- Shunt Caps and Inverters topology is selected.

- The parameters PBRipple=0.043 dB, Prototype Degree N=3 are typed in.

- Distributed element, BPF options are selected.

- The filter specs fp1=900 MHz, fp2=1100 MHz and fq=1500 MHz are typed in.

- Find Circuit command gives the circuit shown in Figure 16.21.a.

- The command Inverters / Equate All Elements / SC Stubs is applied to equate all shunt SC stub impedances (Figure 16.21.b). The response has the perfect dual bands. The circuit is saved as DP06.

- The ideal inverters can be replaced by their practical equivalents involving TL pieces and shunt SC stubs by placing the cursor under inverters and applying the command Inverters / Replace Inverter by / SSsh+TL+SSsh / Specify fq: fo=1000 MHz, fq=1500 MHz is applied. In this command fo is the first passband center and fq is the quarter wavelength frequency of the filter. We get the circuit shown in Figure 16.21.c.

- The shunt SC stubs on the two sides of TL pieces have negative impedances. Since they have the same length as the adjacent stub resonators they can be combines by applying Simplify Circuit (F6) command giving the circuit of Figure 16.21.d.

- This approximation leads to slight degradations in return loss responses of both passbands. The circuit is saved as DP06A.

- The circuit is similar to the exact circuit DP01A obtained using synthesis approach of the previous example. It is also shown in Figure 16.21.d for comparison of the element values. The stub impedances are almost the same while impedances of  the TL’s of DP06A are a bit higher than the exact case.

Alternative solutions with all OC stubs or all SC stubs and with narrower or wider second passband can be formed using the same commands as in the previous example. Some cases are presented below.

Figure 16.21. Design of dual band filter using Prototype with Inverters approach.

Alternative solutions with narrower second passband and with all OC or all SC Stubs:

- The circuit of DP06A is loaded (Figure 16.22.a).

- The version with all OC stubs and narrower second passband can be formed by placing the cursor under the OC+SC stub resonators and applying the command Resonator Oprs / Stubs for Dual PB/ Two OC Stubs; fp1B=1950 MHz, fp2B=2050 MHz. We get the all OC Stubs version of the circuit as shown in Figure 16.22.b.

- The version with all SC stubs can be obtained in similar way, by applying the command Resonator Oprs / Stubs for Dual PB / Two SC Stubs: fp1B=1950 MHz, fp2B=2050 MHz (Figures 16.22.c-d).

- We make an attempt to get the version with OC and SC Stubs by applying the command Resonator Oprs / Stubs for Dual PB / SC and OC Stubs: fp1B=1950 MHz, fp2B=2050 MHz, as described in Figure 16.22.e-f. However the second passband is unacceptably degraded. It can be corrected by tuning or optimization. However just as a demonstration, let’s follow a different approach.

- The circuit with inverters, DP06 is loaded (Figure 16.22.g).

- With cursor under the stub resonators the command Resonator Oprs / Stubs for Dual PB / SC and OC Stubs: fp1B=1950 MHz, fp2B=2050 MHz is applied giving the circuit of Figure 16.22.h, having distortionless dual band response.

- We Copy the length of SC stub, Lng=54.462 mm to use in inverter approximations.

- Then with cursor under the inverters the command Inverters / Replace Inverter By / SSsh+TL+SSsh / Specify Length: fo=1000 MHz, Lng=54.462 mm is applied. In this command we paste the copied length. We get the circuit shown in Figure 16.22.i.

- Simplify Circuit command combines the negative shunt SC stubs with adjacent positive SC stubs giving the circuit of Figure 16.22.k. Responses of both passbands are much better than the previous approach. Slight degradations can be corrected by tuning a few elements.

Figure 16.22. Alternative solutions with narrower second passband.

Figure 16.22. Alternative solutions with narrower second passband.

Alternative solutions with wider second passband and with all OC or all SC Stubs:

- The circuit of DP06A is loaded (Figure 16.23.a).

- The version with all OC stubs and wider second passband can be formed by placing the cursor under the OC+SC stub resonators and applying the command Resonator Oprs / Stubs for Dual PB/  Two OC Stubs; fp1B=1800 MHz, fp2B=2200 MHz. We get the all OC Stubs version of the circuit as shown in Figure 16.23.b. Second passband return loss response needs improvement. Degradation is mainly due to the effects of the transmission lines.

- The version with all SC stubs can be obtained in similar way, by applying the command Resonator Oprs / Stubs for Dual PB / Two SC Stubs: fp1B=1800 MHz, fp2B=2100 MHz (Figures 16.23.c-d). Return loss responses of both passbands need improvement.

- The version with OC and SC Stubs by applying the command Resonator Oprs / Stubs for Dual PB / SC and OC Stubs: fp1B=1800 MHz, fp2B=2100 MHz, as described in Figure 16.23.e-f. Return loss responses of both passbands need improvement.

Figure 16.23. Alternative solutions with wider second passband.

Figure 16.23. Alternative solutions with wider second passband.

16. 3. 2. PLANAR DUAL BAND FILTER DESIGN BY TRANSFORMATION FROM SINGLE BAND FILTERS

In the previous approach we have selected the quarter wavelength frequency fq as the center of the stopband between the two desired passbands. That is, once the two passbands are selected the quarter wavelength frequency fq, hence the length of stubs and the size of the filter are automatically set. We don’t have much flexibility for size reduction. Miniaturization is possible, at least theoretically, by selecting the quarter wavelength frequency fq higher than the second passband, just the same as in the cases of distributed element HP and BP filters. In distributed element HP filters the passband center is selected as fq while in distributed element BP filters fq was a free parameter that can be selected much higher than the passband center leading to miniature sizes.

In this approach we design a distributed element filter with a quarter wavelength frequency fq higher than the targeted second passband. That is, the first spurious passband will not be utilized as the second passband. Rather, the second passband will be created by transforming the OC+SC stub resonators into fourth order dual band resonators, just the same as in the formation of dual band filters from a single band filter in lumped element cases. Introduction of the second passband below fq introduces also new spurious passbands above fq. These spurious passbands have to be eliminated by using a LPF or they may also be utilized to form a multi passband filter.

Naturally applicability of the approach depends on the resulting topologies and impedances.

Let’s continue  discussions on examples.

EXAMPLE 16. 7. Wide stopband dual band filter in planar structures

Let’s design a three resonator distributed element filter with following specifications:

PBRipple=0.01 dB, fp1A=950 MHz, fp2A=1050 MHz and fq=2000 MHz.

The second passband will be between fp1B=1500 MHz and fp2B=1600 MHz.

The quarter wavelength frequency fq=2000 MHz will push the first spurious passband to about 3000 MHz. Hence it can’t be used as the second passband. We will create the second passband by converting the single band resonators into dual band resonators.

We can use either Design / By Synthesis or Design / By Prototype with Inverters approach. Both approach will involve some approximations and need a slight optimization. Let’s use Synthesis approach.

Design steps are shown in Figure 16.24.

- Design / By Synthesis option is selected.

- Distributed element, BPF, Equiripple, Singly Terminated, Type B  options are selected.

- The specs PBRipple=0.01 dB, fp1=950 MHz, fp2=1050 MHz and fq=2000 MHz are typed in.

- Transmission zeros are specified as Nzero=1, NUE=2 and Ninf=3 to get a topology with three resonators separated by two TL’s. Transmission zeros are extracted in the order inf-UE-inf-zero-UE-inf giving the circuit of Figure 16.24.a.

- With cursor under the UE’s the command Kuroda / Distribute Stub is applied to move part of the shunt SC Stub in the middle of the circuit to the other sides of the UE’s to have both OC and SC on two sides of each UE (Figure 16.24.b).

- Then with cursor under the UE having shunt SC stubs on both sides, the command Kuroda / All Equal Stub/ Include End Stubs is applied to equate SC stub impedances (Figure 16.24.c). Thus we have the OC+SC Stub resonators coupled by TL pieces. This is the classical BPF with periodic responses. The first spurious PB is around 2fq-fo=3000 MHz.

- Now we will create the desired second passband between f1B=1500 MHz and f2B=1600 MHz. This is done by placing the cursor under OC+SC Stub resonators and applying the command Resonator Oprs / Single Band-to-Dual Band / fp1B=1500 MHz, fp2B=1600 MHz. This operation replaces the parallel OC+SC Stub resonators with a fourth order section made up of parallel combination of a parallel OC+SC Stub and a series OC+SC Stub resonators (Figure 16.24.d).

- Figure 16.24.e shows a wide span insertion loss response. It is to be noted that the brute force second passband between 1500-1600 MHz is accompanied by its mirror image around fq=2000, in 2400-2500 MHz range. The periodicity of the whole response with a period of 2fq is clear.

- Figure 16.24.f shows return loss response in f=0-4000 MHz range. The second passband return loss response is both shifted in frequency and has heavy degradation. This is because the coupling TL’s need also some modification because they were synthesized for single passband only. Modifications can be carried out by either Tune Element or Monte Carlo-Best Case optimization routine. Results of a quick Monte Carlo routine are shown in Figure 16.24.g. Return loss level in both bands converged to about 21 dB. The circuit is saved as DP07.

Realization topologies in planar structures:

Realizability of the circuits in planar structures depend on the impedances of stubs and TL’s which are dependent on the parameters like PBRipple, fq, pass bandwidths and the stop bandwidth between the two passbands. In Figure 16.25 various alternative realization topologies are shown just to give an idea on possibilities. They are applicable only if the impedances come out to be realizable in strip, suspended strip or microstrip structures.

Realization using V-resonators and Stepped resonators:

In this approach the shunt arm series OC+SC Stub resonators are converted into stepped OC stub resonators while the parallel OC and SC stubs are left as they are. The conversion is shown in Figure 16.25.a-b.

- With cursor under the series OC+SC Stub resonator, the command Step Resonator is applied giving the stepped OC stubs shown in Figure 16.25.b.

- Figure 16.25.c shows a pictorial description for realization. Since the OC, SC and stepped stubs must be connected at the same point, the OC and SC stubs are bent to form a V-section, named as V-resonators. If the impedances Z1 and Z2 of the stepped resonator and impedances of OC and SC stubs are realizable then this would be a realizable topology.

Realization using both OC and SC stepped resonators:

In this topology we convert both the parallel and series OC+SC stub resonators into stepped stubs. If the impedances come out to be realizable then this topology seems to be simplest to use. The conversion steps are as follows:

- The circuit DP07 is loaded (Figure 16.25.d). With cursor under the parallel OC+SC Stub resonators, the command Step Resonators / SC Step Resonators is applied converting the parallel OC+SC stub resonators into stepped SC stub resonators.

- Then with cursor under the series OC+SC stub resonators, the command Step Resonators is applied converting them into stepped OC stub resonators. We get the circuit shown in Figure 16.25.e.

- Figure 16.25.f shows a pictorial description of the realization.

- We note that some step impedances are too low while some are too high. Low impedances may be approximated by lumped capacitors. This may lead to degradations in response necessitating optimization.

Realization using V-resonators and Tee-resonators:

The series OC+SC stub resonators may also be converted into Tee-resonators as follows:

- The circuit DP07 is loaded (Figure 16.25.g).

- With cursor under the series OC+SC Stub resonators, the command Resonator Oprs / Off-Line Resonators / Tee Resonator / fo=… / Same SC Stub as Adjacent Resonator is applied. We get the circuit shown in Figure 16.25.h. The element named as Tee-Resonator is described in Figure 16.25.i. The SC and OC stubs form a resonator. This resonator is coupled to the main filter line through a transmission line piece, forming a Tee-type geometry.

- Figure 16.25.k shows realization topology. Response is shown in Figure 16.25.m. Degradations can be corrected by optimization.

Exercise: Repeat the design by using Prototype with Inverters approach.

Figure 16.24. Dual band filter design by transformation form single band filter.

Figure 16.24. Dual band filter design by transformation form single band filter.

Figure 16.25. Realization topologies of the dual band filter in planar structures.

Figure 16.25. Realization topologies of the dual band filter in planar structures.

## 16. 3. 3. PLANAR DUAL BAND FILTERS WITH MEDIUM AND NARROW STOPBANDS

As the stop bandwidth between the two passbands gets narrower the shunt OC stub impedances get higher while the shunt SC stub impedances get lower leading to realization difficulties. High impedance OC stubs can be treated together with the adjacent transmission lines to form spur type or L-resonator type parallel coupled lines, as described in the following examples. The low impedance shunt SC stubs can be converted into stepped resonators to yield realizable impedances.

## EXAMPLE 16. 7. Medium stopband dual band filter design

Let’s now consider a three resonator dual band filter with following specifications:

PBRipple =0.043 dB, fp1A=900 MHz, fp2A=1100 MHz, fp1B=1400 MHz and fp2B=1600 MHz

The quarter wavelength frequency must be  selected as the center of stopband, fq=1250 MHz.

A three resonator filter will have a total degree of N=6. Coupling between the three resonators will be provided by two Unit Elements (UE). So, the transmission zeros can be selected as follows:  Nzero=1, Ninf=3, NUE=2

The design command set is shown in Figure 16.26.

- Design / By Synthesis commands are invoked.

- Distributed Element / BP / Equiripple / Doubly Terminated / Type B options are selected.

- Passband and Terminations are typed in for the first passband only: fp1=900 MHz, fp2=1100 MHz and fq=1250 MHz.

- Transmission zeros are type in. The second passband will also appear in the inset plot around fp1B=1400 MHz and fp2B=1600 MHz.

- The transmission zeros are extracted in the order: Inf-UE-Inf-Zero-UE-Inf. We get the circuit shown in Fig.16.26.a.

- The single TZ at f=0 appears as a shunt SC stub in the middle of the circuit. It will be distributed into the circuit using Kuroda Transformations to form OC+SC stub resonators. This is done by placing the cursor under the UE’s and applying the command Kuroda / Distribute Stub. Thus, we place a shunt SC stub in parallel with each shunt OC stub. However it is preferable to have identical shunt SC stubs. This is possible by placing the cursor under the UE which has shunt SC stubs on both sides and applying the command Kuroda / All Equal Stubs / Include End Stubs. This command equates all SC stub impedances and combines them with the adjacent OC stubs thus forming OC and SC stub resonators (Figs. 16.26.b-c-d). The circuit is saved as  DP02A.fil.

- Impedances of the OC stubs are too high for realization. They can be treated together with the adjacent TL elements to form spur type parallel coupled lines which will have realizable impedances. For this purpose we first apply command Split / All 2-Element Resonators to split the OC+SC stubs into their OC and SC stub components (Fig.16.26.e).

- Then with cursor under the load, using Insert command, a TL element is inserted at load end to form OC stub and TL pairs (Figure 16.26.f).

- Then with cursor under the TL elements the command Replace by PCL / Spur Lines / RgSpur/ n=.. is applied converting the OC stub+TL pairs into spur type parallel coupled lines (Fig.26.g). In this command the displayed default value for the parameter n leads to symmetric parallel coupled line. All the other values lead to asymmetric coupled lines.

- Figure 26.h shows a pictorial description of microstrip or suspended strip realization. The tightly coupled lines may be realized using both sides of a suspended dielectric layer.

- Figure 16.26.i shows the response. Since no approximation is involved the response remains unperturbed.

Figure 16.26. Design of a medium stop band  dual band filter.

Figure 16.26. Design of a dual band filter with medium stop bandwidth.

## EXAMPLE 16. 8. A narrow stopband dual band filter

Let’s now consider a three resonator dual band filter with following specifications:

PBRipple = 0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz, fp1B=1080 MHz and fp2B=1100 MHz

The quarter wavelength frequency must be  selected as the center of the stopband, fq=1060 MHz.

A three resonator filter will have a total degree of N=6. Coupling between the three resonators will be provided by two Unit Elements (UE). So, the transmission zeros can be selected as follows:  Nzero=1, Ninf=3, NUE=2

The design command set is shown in Figure 16.27.

- Design / By Synthesis commands are invoked. Distributed Element / BP / Equiripple / Doubly Terminated / Type B options are selected. Passband and Terminations are typed in for the first passband only: fp1=1020 MHz, fp2=1040 MHz and fq=1060 MHz.

- Transmission zeros are type in. The second passband will also appear in the inset plot around fp1B=1080 MHz and fp2B=1100 MHz.

- The transmission zeros are extracted in the order: Inf-UE-Inf-Zero-UE-Inf. We get the circuit shown in Fig.16.27.a.

- The single TZ at f=0 appears as a shunt SC stub in the middle of the circuit. It will be distributed into the circuit using Kuroda Transformations to form OC+SC stub resonators. This is done by placing the cursor under the UE’s and applying the command Kuroda / Distribute Stub. Thus, we place a shunt SC stub in parallel with each shunt OC stub. However it is preferable to have identical shunt SC stubs. This is possible by placing the cursor under the UE which has shunt SC stubs on both sides and applying the command Kuroda / All Equal Stubs / Include End Stubs. This command equates all SC stub impedances and combines them with the adjacent OC stubs thus forming OC and SC stub resonators (Figs. 16.27.b-c-d). The circuit is saved as  DP03A.fil.

- Impedances of the OC stubs are too high for realization even in Spur Line form. They can be treated together with the adjacent TL elements to form L-Resonator type parallel coupled lines to give realizable impedances. For this purpose we first apply command Split / All 2-Element Resonators to split the OC+SC stubs into their OC and SC stub components (Fig.16.27.e).

- Then with cursor under the load, using Insert command, a TL element is inserted at load end to form OC stub and TL pairs (Figure 16.27.f).

- Then with cursor under the TL elements the command Replace by PCL / I-L-U Resonators / OSoc-L / Zev=80 (?) ohms / n=3.207  is applied converting the OC stub+TL pairs into L-resonator type parallel coupled lines (Fig.16.27.g). In applying this command user can specify the even mode impedances of the coupled lines and inspect impedances of the resulting OC stubs connected to one port of the coupled line. The command should be repeated until a realizable stub impedance is obtained together with a realizable even-odd mode impedance pair. In this command the number n appearing in the dialog window denotes the default value for obtaining a symmetric parallel coupled line. Any other value will lead to asymmetric coupled lines. For the first two L-resonators on left Zev=80 ohms gave about 70 ohms for the stub impedance Zst. For the rightmost L-resonator Zev=65 ohms gave Zst=75 ohms.

- Next, we have to solve the problem of extremely low impedances of the SC stubs. This problem can be solved by converting them into stepped OC resonators by placing the cursor under these SC stubs and applying the command Step Resonator / Z1=18 (?) ohms (Figure 16.27.h). The user can specify the impedance of the first step and Filpro calculates impedance Z2 of the second step. Selection of Z1=18 ohms gave Z2=113 ohms. Higher Z1 leads to higher Z2.

- Figure 16.27.i shows a pictorial description of microstrip or suspended strip realization. The tightly coupled lines may be realized using both sides of a suspended dielectric layer.

- Figure 16.27.k shows the response. Since no approximation is involved the response remains undisturbed.

Figure 16.27. Design of a dual band filter with narrow stop bandwidth.

Figure 16.27. Design of a dual band filter with narrow stop bandwidth.

16. 3. 4. DUAL BAND FILTERS USING STEPPED IMPEDANCE RESONATORS

In the previous examples we had selected the quarter wavelength frequency fq of the dual band filter as the midpoint of the stopband in between the two passbands and obtained filters as cascades of shunt stubs and TL pieces (Unit Elements). For wide stopbands between the two passbands one may also use edge or broadside coupled line structures designed as commensurate line distributed element HP filters. Since the first two passbands of such filters occur at fo=fq and 2fq, at first sight this seems to be as a limitation. However this limitation can be overcome by using stepped impedance resonators. In the usual edge coupled line filters with stepped impedance resonators we aimed at shifting the first spurious passband away from 2fo, or, if possible, eliminate it completely. In dual band applications we just want to shift the passband around 2fo to the desired location without degrading its return loss response. In this example we will show that it is possible to relocate the first spurious passband to desired location on the two sides of 2fo, within limited ranges using stepped resonators and redundant Unit Elements (TL’s).

EXAMPLE 16. 10. Dual Band version of the Stepped Impedance Resonator filter

The filter specifications for the main passband are as follows:

PBRipple=0.01, fp1=fp1A=975 MHz,   fp2=fp2A=1025 MHz  and  N=5

If it is realized by using uniform resonator lines then it will have a second passband with passband edge frequencies fp1B=1950 MHz and fp2B=2050 MHz. We will try to shift the second passband to different locations by using stepped impedance resonators.

- The design steps of the filter are shown in Figure 16.28 giving the coupled resonator version of the filter (Figure 16.28.a). Using the commands CTRL+F6 and ALT+F6, we note down the External Q, Coupling Coefficients and Resonance Frequencies of the resonators to use later during realization and tuning stages.

- The resonators are equated by using the command Inverters / Equate All Elements (Figure 16.28.b-c).

- Using the command Replace by TLine / Half Wavelength TL the LC resonators are replaced by Half Wavelength TL pieces (Figure 16.28.c-d).

- The TL impedances are scaled to 50 ohms by applying the command Inverters / Scale Cct by Inverters / Specify Adjacent TL: Zo=50 ohms (Figure 16.28.d-e).

- The half wave TL pieces are replaced by 3-step resonators by applying the command Half Wave TL to Stepped TL / 3-Step TL / Z2=50 ohms, Z1 = 60 ohms, Lng2=37.474 mm. In applying this command we can select the parameters Z2, Z1 and Lng2 where Z2 is the impedance of the outer steps, Z1 is the impedance of the inner step and Lng2 is the length of outer steps. Lng1 of the inner step will be calculated by Filpro. We leave Z2 as 50 ohms and as an initial trial set Z1=60 ohms. Lng2 is a quarter of the length of the original half wavelength TL. With this choice Lng1 of the inner step will be twice Lng2. We get the circuit of Figure 16.28.f.

- The inner TL+Inverter+TL triplets are equivalent to ococINTD type parallel coupled lines. Conversion into coupled line form is carried out by applying the command Replace by PCL / Interdigital Line / ococINTD: fo=1000 MHz. Here we have to type in the passband center frequency fo=1000 MHz. So, we get the coupled line filter shown in Figure 16.28.g.

- In the previous examples the inverters at source and load ends were used to create Tapped input/output Lines. This action keeps the passband edges and return loss of the main passband least effected while destroying the responses of all other passbands. Since now we want to preserve the response of the second passband also, we should not use tapping approach or we should develop a new tapping technique which keeps the second passband also unaffected. A straightforward approach to solve this problem is to use redundant Unit Element at input and output. Let’s try that approach. For this purpose we insert 50 ohms (redundant) TL pieces at input and output ports (Figure 16.28.h). Length of these TL pieces should be larger than or equal to the length (Lng2) of the TL piece on the other side of the relevant inverter. We set their length to be the same as Lng2=37.485 mm.

- Thus, we form TL+Inverter+TL triplets at source and load ends also. They are also converted into coupled line form, thus completing the design (Figure 16.28.i).

- Figure 16.28.k shows the layout and wide span insertion loss responses for different Z1 values greater than Z2. We see that the first spurious passband (second passband of the dual band filter) is moved to the right of 2fo as Z1 is increased. For Z1=120 ohms, the second passband is moved to 3fo.

- Figure 16.28.m shows the layout and wide span insertion loss responses for different Z1 values less than Z2. We see that the first spurious passband (second passband of the dual band filter) is moved to the left of 2fo as Z1 is decreased. For Z1=20 ohms, the second and third passbands get too close, hence start to become useless as dual band filter.

- The 3-step resonators may be folded to reduce the longitudinal dimension.

Figure 16.28. Design of dual band stepped impedance resonator filters.

Figure 16.28. Design of dual band stepped impedance resonator filters.

Figure 16.28. Design of dual band stepped impedance resonator filters.