## 16. 4. NONPLANAR DUAL BAND COUPLED RESONATOR FILTERS

For high power applications usually coupled resonator type filters with high Q coaxial or cavity type resonators are needed [12], [13]. In this and the following sections the design methods for dual band filters involving coupled resonators will be described. As will be seen several different realizations are possible some involving offline resonators (Transverse Doublets), some involving cross-coupled triplet (CT) and/or quadruplet (CQ) type  resonators.

Three approaches will be described for the design of such filters:

- In the first approach a single band lumped element BP filter will be converted into dual band lumped element filter by single band-to-dual band transformations.  The two bands will be located on the two sides of a selected stopband center (fs) with arithmetic symmetry. The two bandwidths can be set as unequal. This approach is the simplest one for filters with no finite transmission zeros (FTZ’s) (Chebyshev-Butterworth type filters). For filters with FTZ’s this approach yields impractical structures.

- In the second approach, filters will be designed using the technique of direct synthesis of commensurate line distributed element filters. The first two passbands of the periodic responses will be used as the targeted dual bands. They are located on the two sides of the quarter wavelength frequency fq with arithmetic symmetry and equal bandwidths. This approach is applicable to filters with or without FTZ’s.

- In the third approach a frequency and magnitude normalized lumped element prototype BP filter will be designed. Then the classical LP-to-BP transformation function will be applied to this prototype to yield a dual band filter. The two passbands will have geometric symmetry with respect to the stopband center, leading to a wider second passband.

16. 4. 1. LUMPED ELEMENT APPROACH: CONVERSION OF A SINGLE PASSBAND FILTER INTO DUAL PASSBAND

It is possible to convert a single band lumped element BP filter into double band one using single band-to-dual band transformations. The two pass bandwidths can be unequal. For dual band filters with a narrow stopband and equal bandwidths the two passbands will be roughly mirror image of each other about the center of the stopband. For different bandwidths the center of stopband will be shifted to the wider band one. The approach is simple and applicable to filters without FTZ’s. For filters with FTZ’s we get impractical cross-coupled structures.

Let’s describe the approach on some examples.

EXAMPLE 16. 11:  A coaxial dual band Chebyshev filter using lumped element models

Let’s design a 5 resonator single band Chebyshev filter which will be converted into a 10 resonator dual band filter. The specs of the single band filter are

PBRipple=0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz

The center of stopband  is fs=1060 MHz and hence the second passband will have band edge frequencies, fp1B=1080 MHz, fp2B=1100 MHz.

Since no finite transmission zeros are required we can use any one of the Synthesis, Prototype with Inverters or Built-in LC Prototype approach. Let’s use Design / By Prototype with Inverters approach, as it seems slightly quicker. Design steps are described in Figure 16.29 :

- Design / By Prototype with Inverters option is selected. Choose a Standard Circuit / Shunt Caps+ Inverters options are selected.

- Since a 5 resonator BP filter is needed we type in Prototype Degree=5. Typing also PBRipple=0.01 dB we get the LP prototype shown in Figure 16.29a.

- Filter Type: Lumped / BPF options are selected.

- PB and Terminations command is invoked and the specification fp1=1020 MHz, fp2=1040 MHz are typed in.

- Find Circuit command gives the filter circuit shown in Figure 16.29.b. In this circuit the resonators are in the form of shunt arm parallel LC.

- With cursor under any inverter the command Inverters / Equate All Elements/ Equate Inductors is applied to make all resonators identical (Figure 16.29.c). The filter is saved as D60.

Dual band filter with equal bandwidths:

- With cursor under the resonators, the command Resonator Oprs / Single Band-to-Dual Band /Specify Second Band: fp1B=1080 MHz, fp2B=1100 MHz is applied. The parallel LC resonator is converted into a fourth order section made up of a series LC resonator in parallel with a parallel resonator (Figure 16.29.d). With this conversion the bandpass filter with fp1A=1020 MHz and fp2A=1040 MHz is now converted into a wider band bandpass filter with fp1A=1020 MHz and fp2B=1100 MHz, containing a stopband between fp2A=1040 MHz and fp1B=1080 MHz. Both BP and BS filters are centered at the stopband center fs=1060 MHz. The parallel LC resonators shape up the BP filter. They resonate (become OC) at band center of fs=1060 MHz. The series LC resonators shape up the BS filter. They also resonate at band center fs=1060 MHz (become SC). In this respect the series LC resonators form FTZ’s at that frequency. As will be seen in the following lines, the property of series and parallel resonances being at the same frequency splits the passband into two equal width passbands. For unequal pass bandwidths the resonance frequencies of series and parallel resonators will be different.

- With cursor under the series LC resonators, the command Resonator Oprs / Off-Line Resonators is applied to convert the shunt arm series LC resonators into parallel LC resonators coupled to the main line through inverters. These resonators, together with the inline parallel LC resonators at the relevant node form transverse Doublet elements. This element is shown in the inset of Figure 16.29.e. It contains an inverter followed by parallel combination of shunt lumped inductor Lp, a lumped capacitor Cp, a resistor Rp to account for finite Q of resonator, a SC stub of impedance Zsc and an OC stub of impedance Zoc. If the filter is designed as a Lumped Element one, then L and C become effective while the SC and OC stubs become ineffective, thus simulating a lumped LC resonator. If the filter were designed as a Distributed Element one then the off-line resonator would be formed by the SC and OC stubs while L and C would be ineffective. Note that in this filter the inductors and capacitors of off-line resonators of the doublets are identical to those of in-line resonators. This is because resonance frequencies of both  series and parallel resonators are the same: fs=1060 MHz.

- Thus we get a coupled resonator filter having 5 transverse Doublet sections (10 resonators). The filter is saved as D60a. Responses before and after conversion are shown in Figure 16.29.f. The two bandwidths are equal.

Dual band filter with wider second bandwidth:

The two passbands can be forced to have different bandwidths by shifting the resonance frequencies of series and parallel resonators. For example let’s make the passband edges of the second band fp1B=1080 MHz and fp2B=1140 MHz.

- The filter D60 is loaded (Figure 16.29.g).

- With cursor under the resonators, the command Resonator Oprs / Single Band-to-Dual Band /Specify Second Band: fp1B=1080 MHz, fp2B=1140 MHz is applied. The parallel LC resonator is replaced by a shunt arm series LC resonator in parallel with a shunt arm parallel resonator (Figure 16.29.h). Inspection shows that the resonance frequencies of the series and parallel resonators are different. The series resonators form FTZ’s which are all at the same frequency, close to the narrower passband (fosb=1050 MHz). The parallel resonators resonate at a frequency closer to the wider band (fopb=1087.5 MHz).

- With cursor under the series LC resonators, the command Resonator Oprs / Off-Line Resonators is applied to convert the shunt arm series LC resonators into OffLine Resonators, forming transverse Doublet elements (Figure 16.29.i). In this transformation Filpro forces the inductors of off-line resonators of doublet elements to be identical to the main-line resonator inductors. Since the resonance frequency of inline resonators are greater, their capacitors are smaller than the capacitors of the off-line resonators.

- Responses are shown in Figure 16.29.k.

Dual band filter with narrower second bandwidth:

Let’s now make the second passband narrower than the first passband: fp1B=1080 MHz, fp2B=1085 MHz.

- The filter D60 is loaded (Figure 16.29.m).

- With cursor under the resonators, the command Resonator Oprs / Single Band-to-Dual Band /Specify Second Band: fp1B=1080 MHz, fp2B=1085 MHz is applied. The parallel LC resonator is replaced by a shunt arm series LC resonator in parallel with a shunt arm parallel resonator (Figure 16.29.n). Inspection of the response shows that the series resonators form FTZ’s which are all at the same frequency, close to the narrower passband (fosb=1072 MHz). The parallel resonators resonate at the same frequency which is closer to the wider passband (fobp=1040 MHz).

- With cursor under the series LC resonators, the command Resonator Oprs / Off-Line Resonator is applied to convert the shunt arm series resonators into off-line parallel resonator coupled to the main line through an inverter. This resonator, together with the mainline resonator  at the relevant node forms a transverse Doublet (Figure 16.29.p). Note that Filpro forces the inductors of off-line resonators of doublet elements to be identical to the main-line resonator inductors. Since the resonance frequency of inline resonators are smaller their capacitors are greater than the capacitors of the off-line resonators.

- Responses are shown in Figure 16.29.q.

Impedance Scaling:

- In order to scale the circuit impedance level, for example to make all shunt inductors 10 nH the command Inverters / Scale Cct by Inverters / Specify Adjacent Inductor: L=10 nH. Thus all shunt inductors including those of the Doublets become 10 nH (Figure 16.29.r).

- Figure 16.29.s shows a schematics for realization with coaxial resonators. So, each resonator of the single band filter now became a doublet of resonators, increasing the total degree to 20.

Displaying Coupling Coefficients and Resonance Frequencies:

The present version of Filpro can’t display the coupling coefficients of the circuits having Transverse Doublets. Therefore an indirect way is used to display them, as follows:

- For example let’s load the filter D60a (Figure 16.29.t).

- All the off-line resonators are deleted.

- Then the key combination Ctrl+F6 is pressed and the option Cpld Res-CT-CQ is selected to display the External Q and Coupling Coefficients.

- The Coupling Coefficients and Resonance Frequencies of each resonator can be displayed by placing the cursor under the coupling inverters and pressing the keys ALT+F6.

- The Coupling Coefficients and Resonance Frequencies of off-line resonators can only be displayed by forming each doublet separately on Filpro. The doublets can be formed by noting down the element values and then forming doublets either by using Design / By Inserting Elements approach or (easier) by using Copy-Paste-Delete-Insert command on the displayed filter circuit.  After forming the doublet, with cursor under the inverter, ALT+F6 keys are pressed to display the Coupling Coefficients and Resonance frequencies.

Comment: Conversion from single band to dual band is an exact transformation when the single band filter consists of only LC resonators and ideal inverters. If the inverters of the original filter were replaced by their inductive or capacitive equivalents or by transmission line equivalents then the transformations would lead to great degradations in response which would be cumbersome to recover. Therefore for such applications it is wiser to use inverter coupled filters in the original BP filter.

Figure 16.29. Conversion of a single band Chebyshev filter into dual band filter.

Figure 16.29. Conversion of a single band Chebyshev filter into dual band filter.

Figure 16.29. Conversion of a single band Chebyshev filter into dual band filter.

Figure 16.29. Conversion of a single band Chebyshev filter into dual band filter.

EXAMPLE 16. 13:  A dual band filter with finite transmission zeros (Problematic!!)

Direct conversion of a single passband filter into a dual band one proved to be a simple procedure when the filter has no FTZ’s. However this approach leads to structures which are difficult to construct when there are FTZ’s in the single band filter. In order to see the problematic points let’s attempt to design a 5 resonator single band filter having FTZ’s in upper stopband and then convert it into a dual band one.

The specs of the single band filter are

PBRipple=0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz

The center of stopband  is fs=1060 MHz and hence the second passband will have band edge frequencies fp1B=1080 MHz, fp2B=1100 MHz.

Let’s design a degree 10 BP filter with Nzero=5, Ninf=1, FTZ1=1049 MHz, FTZ2=1054 MHz.

Design stages are shown in Figure 16.30:

- Design / By Synthesis approach is adopted. Lumped/ BP / Equiripple/ Doubly Terminated / Type B options are selected.

- PB and Terminations parameters and transmission zeros are typed in.

- The transmission zeros are extracted in such a way as the two FTZ’s will be at the two ends of the filter: FTZ1-Zero-Zero-Zero-Zero-FTZ2-Inf. We get the circuit shown in Figure 16.30.a.

- The command Pi-Tee-L / Equal Shunt Inductors is applied to convert the filter into coupled resonator form (Figure 16.30.b).

- The series capacitors are problematic to transform from single passband into dual passband form (needs iterative tuning trials!). Therefore they are converted into inverters by applying the command Inverters / Pi-to-Inverter: fo=1060 MHz (Figure 16.30.c). The inverters are formed by borrowing some negative shunt capacitors from the two adjacent LC resonators. The inverters are not effected from single band to dual band conversions.

- Thus the circuit is ready for conversion into dual band one (all elements are of LC resonators and inverters!).

- With cursor under the LC elements the command Resonator Oprs / Single Band to Dual Band is applied yielding the dual band filter shown in Figure 16.30.d. Responses are shown in Figure 16.30.e. The slight degradations are due to conversion of the series capacitors into inverters.

- Next, the shunt arm series LC elements (FTZ’s at fs=1060 MHz) are converted into trDoublet elements by applying the command Resonator Oprs / sLCsh-to-trDoublet (Figure 16.30.f).

- From the circuit we can recognize the circuit sections which are equivalent to cross-coupled CQ elements as indicated in the figure. However we have a problem for conversion of these sections into CQ form because of the existence of Off-Line Resonators (trDoublet elements). In the classical transformations we should have only single parallel LC resonators. Not Doublets. Therefore the present version of Filpro can’t convert them into CQ form. However just to get an idea on the possible resulting structure, let’s Delete the Off-Line Resonators and then convert the resulting sections into CQ form as described in Figures 16.30.g-h-i).

- In the circuit of Figure 16.30.i we can insert back the removed Off-Line Resonators to form the final circuit. The resonators of the trDoublet elements are numbered as primed.

- Figure 16.30.k shows a possible realization schematics in coaxial resonator form. As is seen, the existence of Off-Line Resonators complicates the realization a lot.

- The circuit may be simplified by eliminating some of the Off-Line Resonators elements, as will be described in the exact approaches of the following sections.

Figure 16.30. Conversion of a single band with FTZ’s into a dual band filter.

Figure 16.30. Conversion of a single band with FTZ’s into a dual band filter.

16. 4. 2. DISTRIBUTED ELEMENT APPROACH

In this approach we use the first two passbands of the periodic responses of commensurate line distributed element BPF’s. Therefore we get equal pass bandwidths. In this respect this approach is limited to dual band filters with equal bandwidths only. Its advantages side is the ability to place finite transmission zeros. The distributed element BPF is designed with the passband edges set as the edges of the first passband and the quarter wavelength frequency fq set as the arithmetic center of the stopband between the first two passbands. The first passband is shaped by the transmission zeros at =0,  = and at some finite frequencies in upper and lower stopbands. Here =Tan (f/2fq) is the transformed frequency. This transformation maps the TZ’s at =0 to f=0, 2fq, 4fq, .. . The TZ’s at  = are mapped to f=fq, 3fq, 5fq, ..  leading to periodic passbands. The frequency response in the range 0<f<fq is mapped to the range fq<f<2fq with mirror image symmetry around the quarter wavelength frequency fq thus forming the two desired passbands. In this mirror image mapping the FTZ’s in the upper stopband of the first passband are mapped as the FTZ’s in the lower stopband of the second passband. Therefore the stopband between the two passbands will be shaped by the FTZ’s in the upper stopband of the first passband together with their mirror images, the FTZ’s in the lower stopband of the second passband. The TZ’s at f=fq will also help shaping up of this stopband.

Thus, we have to shape only the first passband in the range 0<f<fq. The second passband will automatically come out as the mirror image.

Let’s continue discussions through examples.

## EXAMPLE 16. 14. A degree 6 Chebyshev dual band filter with off-line resonators (transverse doublets)

This example is just a demonstration that a distributed element BPF can be converted into a lumped element one through which the same techniques as the previous example can be used.

A six resonator Chebyshev filter will be designed. Desired specifications are:

PBRipple = 0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz, fp1B=1080 MHz and fp2B=1100 MHz

The quarter wavelength frequency will be selected as the center of the stopband, fq=1060 MHz.

Since no finite transmission zeros are required we can use any one of the Synthesis, Prototype with Inverters or Built-in LC Prototype approach. Let’s use Design / By Prototype with Inverters approach, as it seems slightly quicker. Design steps are described in Figure 16.31 :

- Design / By Prototype with Inverters option is selected. Choose a Standard Circuit / Shunt Caps+ Inverters options are selected.

- Since a degree 6 BP filter is needed we type in Prototype Degree=3. Typing also PBRipple=0.01 dB we get the LP prototype shown in Figure 16.31.a.

- Filter Type: Distributed / BPF options are selected.

- PB and Terminations command is invoked and the specification fp1=1020 MHz, fp2=1040 MHz and fq=1060 MHz are typed in.

- Find Circuit command gives the degree 6 filter circuit shown in Figure 16.31.b. In this circuit the resonators are in the form of parallel connection of commensurate length OC and SC stubs which are quarter wavelength long at fq=1060 MHz.

- With cursor under any inverter the command Inverters / Equate All Elements/ Equate Inductors is applied to make all resonators identical (Figure 16.31.c).

- With cursor under any resonator the command Split / All 2-Element Resonators is applied to split the OC+SC Stub resonators into their OC and SC stub components (Figure 16.31.d). In the figure the insertion loss responses are shown both in narrow and wide span. Note the desired dual passbands on the two sides of fq=1060 MHz. Due to periodicity of the responses of commensurate all stub networks we also have dual passbands around odd harmonics, 3fq, 5fq, .. .

- With cursor under the SC and OC stubs the command Replace By Lumped Eqv / LC Resonator is applied. This command converts the OC stubs into shunt parallel LC resonators while the SC stubs are converted into shunt arm series LC resonators giving the lumped element BPF topology that was studied in the previous example, but with equal pass bandwidths (Figure 16.31.e). These transformations preserve the first two passbands while the undesired spurious passbands around 3fq, 5fq,.. disappears as can be seen from the wide span response. The shunt arm parallel resonators are responsible for the BPF property while the shunt arm series LC resonators form the transmission zeros at fq=1060 MHz, thus responsible for the stopband around fq.

- Next we will convert the shunt arm series LC resonators into parallel resonators. This is done by placing the cursor under these resonators and applying the command Resonator Oprs / Off-Line Resonator. This command replaces the LC resonator by an Off-Line Resonator which forms a transverse Doublet together with the adjacent inline resonator. This element is a parallel LC resonator coupled to the mainline resonator by an inverter thus forming a doublet in transverse direction to the filter, as shown in the inset of Figure 16.31.f. The resonator is a general one which has both lumped L and C elements to form lumped LC resonators and also SC and OC stubs to form stub resonators. In this example only lumped L and C come out to be in effect because they are obtained from lumped LC resonators. Note that the off-set LC resonators are set to be identical to the inline resonators by setting the inverter constant K properly.

- Next, the resonator parameters are adjusted to realization levels. For example all inductors of the resonators are set as 10 nH by applying the command Inverters / Scale Cct by Inverters / Specify Adjacent Inductor: L=10 nH (Figure 16.31.g). The circuit is saved as Dual41.

- Figure 16.31.h shows an exploded view of the circuit. In this circuit we have two types of resonators: Inline resonators functioning as BP sections and off-line resonators functioning as BS elements. The off-line resonators are coupled to main-line resonators through inverters. The inverters may be realized in different forms like inductive-Pi, capacitive-Pi, transmission line, waveguide or evanescent waveguide depending on the realization technology. Figure 16.31.i shows a schematics for a possible realization using coaxial resonators.

- In order to inspect the effects of losses we apply the command Parasitics / All Lossy Resonators / Q=500 (e.g.). We get the circuit shown in Figure 16.31.k where all LC resonators have a parallel resistor set by the Q of the resonators. The stopband notch is reduced to about 90 dB while the passband insertion loss is close to 2 dB at band edges as seen in the plots of Figure 16.31.m. The lossy circuit is saved as Dual42.

- In order to check the sensitivity of the response to variations in coupling inverters, we form a Monte Carlo List by placing all inverter impedances into this list and allowing 5% variation in these impedances (Figure 16.31.n). Then using Monte Carlo / Start Analysis command the variations in response is plot as shown in Figure 16.31.p. It is seen that no serious sensitivity problem exists.

- External Q, Coupling Coefficients and Resonance Frequencies of the main line filter and  Off-Line resonators can be displayed as described in the previous example.

Figure 16.31. Coupled resonator dual band filter design using distributed element BPF approach.

Figure 16.31. Coupled resonator dual band filter design using distributed element BPF approach.

Figure 16.31 . Coupled resonator dual band filter design using distributed element BPF approach.

Figure 16.31. Coupled resonator dual band filter design using distributed element BPF approach.

## EXAMPLE 16. 15. Synthesis approach for the degree 6 Chebyshev dual band filter design leading to CT or CQ1F sections

Prototype with Inverters approach of the previous example yields three transmission zeros at both f=0 and at f=fq. The TZ’s at f=fq were realized by the three off-line resonators (Transverse Doublets). Using Synthesis approach it is possible to control the numbers of TZ’s at f=0 and f=fq that would lead to alternative structures that involve cross-coupled Triplets (CT sections).

Let’s describe the procedure on an example with the same specifications of the previous example:

PBRipple = 0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz, fp1B=1080 MHz and fp2B=1100 MHz

The quarter wavelength frequency will be selected as the center of the stopband, fq=1060 MHz.

Design steps are shown in Figure 16.32.

- Design / By Synthesis option is selected.

- Filter Type: Distributed / BP / Equiripple / Doubly Terminated / Type A options are selected.

- PB and Terminations: fp1=1020 MHz, fp2 = 1040 MHz, fq=1060 MHz are typed in. We will design the filter for the first passband. The second passband will come out automatically.

- Transmission Zeros: Nzero=5, Ninf=1. The number of TZ’s at =0 (f=0) are set to adjust the attenuation and slope outside the two passbands while the number of TZ’s at = (actually f=fq)  shapes the stopband in between the two passbands. For simplicity let’s place only one TZ at =.

- Extract TZ’s: Zero, Zero, Inf, Zero, Zero, Zero.

- We get the circuit shown in Figure 16.32.a. The command Pi-Tee-L / LRight-to-Sym Pi is applied to convert the LRight section of SC stubs into a symmetric Pi-section (Figure 16.32.b).

- Simplify Circuit (short Cut key: F6) command shifts the resulting transformer to load end (Figure 16.32.c).

- With cursor under the series arm OC stubs, the command Inverters / Replace by Inverter: K=50 is applied to convert them into shunt arm parallel SC stubs in between two inverters (Figure 16.32.d). Impedance of the inverter is not important at this stage. It will be adjusted later. So, we have only SC stub resonators and inverters.

- Simplify Circuit command shifts the resulting transformer to load end (Figure 16.32.e).

- All SC stubs are converted into parallel LC resonators by applying the command Replace by Lumped / Lumped LC (Figure 16.32.f). Note the single series arm parallel LC resonator forms the FTZ element. We can form different combinations of this FTZ element with adjacent resonators to create circuit sections which are equivalent to CT sections or CQ1F sections. Let’s see both alternatives.

Solution with a CT section:

- With cursor under the inverter on the right (or left) of the FTZ element, the command Inverters / Replace Inverter by / Pi-Tee Type / Inductive Pi (or Capacitive Pi) is applied to replace the inverter with its inductive Pi-section equivalent with negative shunt inductors (Figure 16.32.g).

- Simplify Circuit command combines the negative elements with neighboring positive element giving the circuit of Figure 16.32.h. We save this circuit as Dual44 to use later for finding other alternative solutions.

- In this circuit the section indicated in the figure is equivalent to a CT section. With cursor under the FTZ element (series arm parallel LC resonator), the command CT-CQ-CN Oprs / Convert to CT / CLL is applied giving the CT section shown in Figure 16.32.i. The cross-coupling is introduced between nodes 2 and 4 of the circuit. In the circuit model of the triplet element (CT section) named as CLL (shown in the inset), the left resonator is numbered as 1 and the right one is numbered as 3. The resonator 2 is hidden in the box. The first letter C in the name CLL indicates that the cross-coupling element is capacitive and it couples nodes 1 and 3. The couplings between nodes 1-2 and nodes 2-3 are inductive (hence LL). Thus, a solution involving one CT section is formed.

- If necessary, we can adjust the element values using inverters. For example all inductors of the resonators can be set to 10 nH as follows.

  - With cursor under any inner inverter the command Inverters / Equate All Elements / Equal Inductors is applied to equate all shunt inductors (Figure 16.32.k).

  - Then with cursor under the inverter at source end the command Inverter / Scale Inverter / Specify Adjacent inductor: L=10 nH is applied giving the circuit of Figure 16.32.m. After moving away the transformer to load end we get the circuit of Figure 16.32.n where all shunt inductors are 10 nH.

  - The transformer at load end is absorbed into the adjacent inverter by applying the command Inverters / Absorb TRF (Figure 16.32.p).

- It is amusing to see that the load end inverter came out to be equal to the source end inverter. The circuit is saved as Dual45.

- The parameters External Q and Coupling Coefficients of the circuit can be displayed as a list by pressing the keys CTRL+F6 and selecting the option Cpld Res-CT-CQ-CN. The nodes are numbered as 1, 2, .., 6. The coupling coefficients between resonators of the loads are listed as k12, k23, etc.

- In the circuits involving cross-coupled elements it may be necessary to know the resonance frequencies of the resonators because introduction of the cross-coupling effects them. This is done by placing the cursor under the coupling elements and pressing the keys ALT+F6. This command displays both coupling coefficient of the pointed section and also the resonance frequencies of the resonators on left and right of the coupling element as shown in the figure.

- Figure 16.32.q shows the response of the filter. Slight degradation in return loss is due to replacement of inverters and can be corrected by tuning a few elements.

- Figure 16.32.r shows a schematics for realization using coaxial resonators.

- In Figures 16.32.s-t-u the effects of losses are studied. With cursor under the resonators the command Parasitics / All Lossy Resonators / Q=500 is applied to convert them into their lossy models with Q=500. Responses are shown in Figure 16.32.u.

- If nodal Voltage-Current-Power parameters need be inspected then it would be useful to plot the  delay response. The maximum voltage occurs near the outer edges of the passbands, at frequencies where delay makes peaks. We note that frequency as f=1016 MHz. Then pressing the keys ALT+F8 and typing in the source voltage Vs=2000 Volts (e.g.) and frequency of interest, f=1017 MHz, we get the list shown in Figure 16.32.v.  The maximum voltage and its location is highlighted in the list. The highest voltage is about 8400 volts and it occurs on the right of the second inverter, on the left of the CT section.

Figure 16.32. Degree 6 Chebyshev dual band filter involving a CT section.

Figure 16.32. Degree 6 Chebyshev dual band filter involving a CT section.

Figure 16.32. Degree 6 Chebyshev dual band filter involving a CT section.

Figure 16.18. Degree 6 Chebyshev dual band filter involving CT section.

Solution with a CQ1F section:

If the tilted nature of the CT elements cause production problems then one may try CQ type solution to realize the single FTZ.

- The circuit of Dual44 is loaded (Figure 16.33.a). With cursor now under the inverter on the left of the FTZ element (series arm parallel LC), the command Inverters / Replace Inverter by / Pi-Tee Type / Inductive Pi (or Capacitive Pi) is applied to replace the inverter with its inductive Pi-section equivalent with negative shunt inductors (Figure 16.33.b).

- Simplify Circuit command combines the negative elements with neighboring positive element giving the circuit of Figure 16.33.c. The circuit section equivalent to a CQ1F element is indicated in the figure.

- With cursor under the FTZ element (series arm parallel LC resonator), the command CT-CQ-CN Oprs / Convert to CQ / Solution-1 is applied giving the CQ1F element shown in Figure 16.33.d. The input and output are taken from the diagonally opposite ports. Sign of the one of the couplings 1-2 or 1-3 or 2-4 or 3-4 can be negative.

- If necessary, we can adjust the element values using inverters. For example all inductors of the resonators can be set to 10 nH as follows.

  - With cursor under the inverter at source end the command Inverter / Scale Inverter / Specify Adjacent inductor: L=10 nH is applied giving the circuit of Figure 16.33.e. After moving away the transformer to load end we get the circuit of Figure 16.33.f where the shunt inductor on the right of the inverter is now 10 nH.

  - Then with cursor under any inner inverter the command Inverters / Equate All Elements / Equal Inductors is applied to equate all shunt inductors at 10 nH level (Figure 16.33.g).

  - The transformer at load end is absorbed into the adjacent inverter by applying the command Inverters / Absorb TRF (Figure 16.33.h). It is amusing to see that the load end inverter came out to be equal to the source end inverter. The circuit is saved as Dual47.

- The External Q and Coupling Coefficients can be displayed by pressing the keys CTRL+F6 and selecting the option Cpld Res-CT-CQ-CN.

- Coupling Coefficients and Resonance Frequencies can be displayed by placing the cursor under the coupling elements and pressing the keys ALT+F6.

- Figure 16.33.i shows the responses. Delay responses is also shown to determine the frequency of maximum delay which is also the frequency of maximum voltage in the circuit.

- Figure 16.33.k shows a schematics for realization with coaxial resonators.

- The effects of losses are studied in Figures 16.33.m. With cursor under the resonators the command Parasitics / All Lossy Resonators / Q=500 is applied to convert them into their lossy models with Q=500.

- The nodal Voltage-Current-Power parameters are displayed in Figure 16.33.n for f=1016 MHz, the frequency where maximum delay occurs. Pressing the keys ALT+F8 and typing in the source voltage Vs=2000 Volts and frequency of interest, f=1016 MHz, we get the list shown in Figure 16.33.n.  The maximum voltage and its location is highlighted in the list. The highest voltage is about 8770 volts and it occurs on the left resonator of the CQ1F section. This value is higher than we have got in the solution with CT section which was 8400 volts.

Figure 16.33. Degree 6 Chebyshev dual band filter involving a CQ1F section.

Figure 16.33. Degree 6 Chebyshev dual band filter involving a CQ1F section.

Figure 16.19. Degree 6 Chebyshev dual band filter involving a CQ1F section.

## EXAMPLE 16. 16. A degree 6 dual band filter with one finite transmission zero

Let’s now design a dual band filter with a single FTZ in upper stopband of the distributed element filter. The single FTZ fz will be in the range 0<f<fq and it will have a mirror image around fq, at 2fq-fz. Together with the TZ’s at f=fq, effectively we will have three FTZ’s to shape the stopband in between the two bands.

The specs of the targeted dual band filter are as follows:

PBRipple = 0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz

for the first passband and

fp1B=1080 MHz, fp2B=1100 MHz

for the second passband. The quarter wavelength frequency of the filter must be selected as the arithmetic center of the stopband:

fq=(fp2A+fp1B)/2 = 1060 MHz

The FTZ will be placed at fz=1054 MHz.

Design steps are shown in Figure 16.34.

- Design / By Synthesis option is selected.

- Filter Type: Distributed / BP / Equiripple / Doubly Terminated / Type B options are selected.

- PB and Terminations: fp1=1020 MHz, fp2 = 1040 MHz, fq=1060 MHz are typed in. We will design the filter for the first passband. The second passband will come out automatically.

- Transmission Zeros: Nzero=3, Ninf=1, FTZ=1054 MHz. The number of TZ’s at f=0 are set to adjust the attenuation and slope outside the two passbands while the FTZ is tuned to set the stopband level in between the two passbands, as seen in the figure. Although the TZ at infinity  appears as a FTZ at f=fq=1060 MHz, in the synthesis procedure for the commensurate line filters it is not treated as a FTZ. Therefore formally the only FTZ is the one at f=1054 MHz. However,  together with the TZ at f=fq and the mirror image of the FTZ of 1054 MHz about f=fq, we effectively have three FTZ’s shaping the stopband between the two passbands.

- Extract TZ’s: Zero, Zero, FTZ, Zero, Inf. As will be seen, the first three TZ’s, that is, zero-zero-FTZ is needed to form a quadruplet.

- We get the circuit shown in Figure 16.34.a. The command Pi-Tee-L / Equal Shunt SC Stubs is applied to convert the filter into coupled resonator form with all shunt resonators formed as parallel connection of OC and SC stubs (Figure 16.34.b).

- As will be seen later, different alternative solutions are possible by manipulating the circuit. As a preparation for this purpose the command Split / All 2-Element Resonators is applied to split the shunt OC+SC Stub resonators into their SC and OC components (Figure 16.34.c).

- Then using Exchange command a Pi-section of OC stubs is formed (Figure 16.34.d). This circuit is saved as D50a. It will be used as starting point for different realizations, as described below.

## SOLUTION-1: Coupled resonator filter with CQ section and transverse Doublets

- The circuit of D50a is loaded. With cursor under the series OC stub of the Pi-section, the command Pi-Tee-L / Pi-to-LRight is applied to convert the Pi-section into an LRight section (Figure 16.34.e-f). CTRL+W keys are used to move the transformer to load end.

- Exchange command is used to move the shunt OC stub in the left half of the circuit to be adjacent to the source (Figure 16.34.g). So, the two shunt OC stubs are located at the two ends of the circuit.

- With cursor under the series OC stub, the command Inverters / Replace by Inverter Equivalent: K=50 is applied to convert it into a shunt SC stub with 50 ohms inverters on both sides (Figure 16.34.g). Impedance of the inverters is not important at this stage. They can be adjusted later.

- Now we will convert all stubs into their lumped element equivalents while keeping the dual band response unchanged. This is done by applying the command Replace by Lumped / LC Resonator to all single OC and SC stubs. The OC stubs are converted into series LC resonators while the SC stubs are converted into parallel LC resonators. The series arm parallel OC+SC Stub element which is responsible for the two FTZ’s in the stopband is converted into a lumped element fourth order section by applying the command Replace by Lumped / Lumped Quad. This transformation keeps the two FTZ’s unaffected. Thus, we get the all-lumped element circuit of Figure 16.34.h.

- It is preferable to have all identical shunt LC resonators. This can be done by placing the cursor under  one of the inverters and applying the command Inverters / Equate all Elements / Equal Inds (Figure 16.34.i).

- In order to form a circuit section equivalent to a CQ section, we need to form an inductor or capacitor coupled shunt resonator adjacent to the fourth order section. This is possible as follows.

- With cursor under the inverter close to the fourth order section, the command Inverters / Replace Inverter by / Pi-Tee-Type / Inductive Pi is applied to replace it with an inductive Pi section with negative shunt elements. Simplify Circuit command is applied to absorb the negative shunt elements into the adjacent resonators (Figure 16.34.k).

- Thus, the equivalent circuit of CQ section is ready as indicated in Figure 16.34.k. With cursor under the fourth order section, the command CT-CQ-CN Oprs / Convert to CQ-CN/Solution-1 is applied to convert this circuit section into a CQ element (Figure 16.34.m).

- We note that the resulting CQ section is of CQBP type. It has no diagonal cross-coupling and all coupling elements are inverters. We also note that the shunt arm parallel resonators of the CQ element have equal inductors while the leftmost shunt arm parallel LC resonator has a different inductance. It can be equated to the others by applying the command Inverters / Equate all Elements / Equal Inds. The resulting transformer is moved away using CRTL+W keys or F6 (Simplify Circuit) command. Also, the shunt arm series LC resonators are converted into Transverse Doublets by applying the command Resonator Oprs / Off-Line Resonator. So, we get the coupled resonator version of the filter shown in Figure 16.34.p. The circuit is saved as D50b.

- Figure 16.34.p shows a possible realization schematics using coaxial resonators. It should be noted that we have seven resonators. One might expect six resonators because we have designed a degree six filter. The extra resonator is due to the FTZ. As will be seen in the following examples also, each FTZ leads to one extra resonator.

- Figure 16.34.q shows the responses. A slight degradation  exists in return loss.

- External Q, Coupling Coefficients and Resonance Frequencies of the main line resonators can be displayed by deleting the Doublets and then using CTRL+F6 and ALT+F6 keys as shown in Figure 16.34.r.

- Coupling coefficients of the doublets can be displayed by forming the circuits shown in Figure 16.34.s and then applying ALT+F6 command.

- The effects of finite Q of the resonators can be inspected by placing the cursor under any resonator and applying the command Parasitics / All Lossy Resonators: Q = 500 (e.g.) where the lossless resonators are converted into lossy resonators as shown in Figures 16.34.t-u.

- In the figure both insertion loss and delay responses are shown. The delay response can be used to find out the frequency of maximum delay. The maximum voltage in the circuit appears at that frequency. After finding that frequency (f=1045 MHz), the Voltage-Current-Power list at the nodes at this frequency can be displayed by applying the command ALT+F8. The source voltage is set (Vs=2000 volts in this example) and frequency is set as f=1045 MHz. We note that the maximum voltage is about 2170 volts and it occurs at the left node of the CQ section.

Figure 16.34. Design of degree 6 dual band elliptic filter using Distributed Element approach.

Figure 16.34. Design of degree 6 dual band elliptic filter using Distributed Element approach.

Figure 16.34. Design of degree 6 dual band elliptic filter using Distributed Element approach.

Figure 16.34. Design of degree 6 dual band elliptic filter using Distributed Element approach.

SOLUTION-2: Coupled resonator filter involving Doublets, CT and CQ sections

In the previous approach the off-line resonators (transverse Doublets) were used to realize the FTZ at f=fq. If these resonators are problematic to realize, an alternative solution can be developed as described in Figure 16.35.

- With cursor under the series OC stub of the Pi-section, the command Pi-Tee-L / Pi-to-Sym Pi is applied to equate the shunt leg impedances. CTRL+W keys are used to move the transformer to load end (Figure 16.35.a-b).

- Then the command Pi-Tee-L / Pi-to-Tee is applied to convert the Pi-section into a Tee-section (Figure 16.35.c)..

- Then the series OC stubs of the Tee-section are converted into shunt SC stubs in between inverters by applying the command Inverters / Replace by Inverter Equivalent: K=50. Using the same command the shunt OC stub of the Tee-section is converted into a series SC stub (Figure 16.35.d).

- The adjacent 50 ohms inverters are combined to yield unity negative transformers which are deleted (Figure 16.35.e).

- The shunt SC stub impedances are equated by placing the cursor under an inverter and applying the command Inverters / Equate All Elements / Equate SC stubs (Figure 16.35.f).

- Now we will convert all stubs into their lumped element equivalents while keeping the dual band response unchanged. This is done by applying the command Replace by Lumped / LC Resonator to all single OC and SC stubs. The OC stubs are converted into series LC resonators while the SC stubs are converted into parallel LC resonators (Figure 16.35.g).

- The series arm parallel OC+SC Stub element which is responsible for the two FTZ’s in the stopband is converted into a lumped element fourth order section by applying the command Replace by Lumped / Lumped Quad. This transformation keeps the two FTZ’s unaffected. In order to form a circuit section equivalent to a CQ section, we need to form an inductor or capacitor coupled shunt resonator adjacent to the fourth order section. For this purpose, with cursor under the inverter on source side, the command Inverters / Replace Inverter by / Pi-Tee-Type / Inductive Pi is applied to replace it with an inductive Pi section with negative shunt elements. The negative elements are absorbed into the adjacent resonators using Simplify Circuit command. Thus, a circuit section is formed which is equivalent to a CT section, as shown in Figure 16.35.h.

- With cursor under the parallel resonator in series arm, the command CT-CQ-CN Oprs / Convert to CT / CLL is applied to convert this circuit section into a CT element (Figure 16.35.i). CLL type CT section has capacitive cross-coupling between nodes 1 and 3 of the triplet.

- This conversion destroys the equality of the shunt resonator inductors. Equality can be recovered by applying the command Inverters / Equate All Elements / Equate SC stubs (Figure 16.35.k).

- Next we will form the equivalent circuit of a CQ section. This is done by replacing the inverter close to the fourth order section with its inductive Pi-section equivalent by applying the command Inverters / Replace Inverter by / Pi-Tee-Type / Inductive Pi. The negative elements are absorbed into the adjacent resonators using Simplify Circuit command. Thus, a circuit section is formed which is equivalent to a CT section, as shown in Figure 16.35.m.

- With cursor under the fourth order section, the command CT-CQ-CN Oprs / Convert to CQ-CN / Solution-1 is applied to convert this circuit section into a CQBP element.  Also, the shunt arm series LC resonator at load end is converted into Transverse Doublet by applying the command Resonator Oprs / Off-Line Resonators. So, we get the coupled resonator version of the filter shown in Figure 16.35.n. The circuit is saved as D50c. This circuit is composed of a Doublet, a Triplet and a Quadruplet. The responses are shown in the same figure. A slight degradation  exists in return loss

- Figure 16.35.p shows a possible realization schematics using coaxial resonators. We have seven resonators.

- The effects of finite Q of the resonators can be inspected by placing the cursor under any resonator and applying the command Parasitics / All Lossy Resonators: Q = 500 (e.g.) where the lossless resonators are converted into lossy resonators as shown in Figures 16.35.q-r. The delay response is used to find out the frequency of maximum delay. The maximum voltage in the circuit appears at that frequency. After finding that frequency (f=1075 MHz), the Voltage-Current-Power list at the nodes at this frequency can be displayed by applying the command ALT+F8. The source voltage is set (Vs=2000 volts in this example) and frequency is set as f=1075 MHz. We note that the maximum voltage is about 2400 volts, higher than that of the Solution-1  which was about 2170 volts. This voltage occurs at the inner node of the CT section.

- External Q, Coupling Coefficients and Resonance Frequencies of the main line resonators can be displayed by deleting the Doublets and then using CTRL+F6 and ALT+F6 keys as shown in Figure 16.35.s-t.

- Coupling coefficient of the doublet can be displayed by forming the circuit shown in Figure 16.35.u and then applying ALT+F6 command.

Figure 16.35. Solution with Doublet, Triplet and Quadruplets.

Figure 16.35. Solution with Doublet, Triplet and Quadruplets.

Figure 16.35. Solution with Doublet, Triplet and Quadruplets.

## EXAMPLE 16. 17. A degree 8 dual band filter with one finite transmission zero

Let’s now design a dual band filter of degree 8 and having a single FTZ in the upper stopband of the first passband. The specs of the targeted dual band filter are as follows:

PBRipple = 0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz

for the first passband and

fp1B=1080 MHz, fp2B=1100 MHz

for the second passband. The quarter wavelength frequency of the filter is selected as the arithmetic center of the stopband:

fq=(fp2A+fp1B)/2 = 1060 MHz

Design steps are shown in Figure 16.36.

- Design / By Synthesis option is selected.

- Filter Type: Distributed / BP / Equiripple / Doubly Terminated / Type B options are selected.

- PB and Terminations: fp1=1020 MHz, fp2 = 1040 MHz, fq=1060 MHz are typed in. We will design the filter for the first passband. The second passband will come out automatically.

- Transmission Zeros: Nzero=5, Ninf=1, FTZ=1054 MHz. The number of TZ’s at f=0 are set to adjust the attenuation and slope outside the two passbands while the FTZ is tuned to set the stopband level in between the two passbands, as seen in the figure. Although the TZ at infinity appears as a FTZ at f=fq=1060 MHz, in the synthesis procedure for the commensurate line filters it is not treated as a FTZ. Therefore formally the only FTZ is the one at f=1054 MHz. However, together with the TZ at f=fq and the mirror image of the FTZ of 1054 MHz about f=fq, we effectively have three FTZ’s shaping the stopband between the two passbands.

- Extract TZ’s: Zero, Zero, FTZ, Zero, Zero, Zero, Inf.

- We get the circuit shown in Figure 16.36.a. The command Pi-Tee-L / Equal Shunt SC Stubs is applied to convert the filter into coupled resonator form with all shunt resonators formed as parallel connection of OC and SC stubs (Figure 16.36.b).

- As will be seen later, different alternative solutions are possible by manipulating the circuit. As a preparation for this purpose the command Split / All 2-Element Resonators is applied to split the shunt OC+SC Stub resonators into their SC and OC components (Figure 16.36.c).

- Then using Exchange command two Pi-sections of OC stubs are formed (Figure 16.36.d). This circuit is saved as D52a. It can be used as starting point for different realizations, as described below.

## SOLUTION-1: Coupled resonator filter with CQ section and transverse Doublets

- The circuit of D52a is loaded. With cursor under the series OC stub of the Pi-section on left half, the command Pi-Tee-L / Pi-to-LRight is applied to convert the Pi-section into and LRight section. Then with cursor under the series OC stub of the Pi-section on the right half, the command Pi-Tee-L / Pi-to-LLeft is applied to convert it into an LLeft section (Figure 16.36.e-f). CTRL+W keys are used to move the transformer to load end.

- With cursor under the series OC stubs, the command Inverters / Replace by Inverter Equivalent: K=50 is applied to convert them into shunt SC stubs with 50 ohms inverters on both sides (Figure 16.46.g). Impedances of the inverters are not important at this stage. They can be adjusted later.

- Now we will convert all stubs into their lumped element equivalents while keeping the dual band response unchanged. This is done by applying the command Replace by Lumped / LC Resonator to all single OC and SC stubs. The OC stubs are converted into series LC resonators while the SC stubs are converted into parallel LC resonators. The series arm parallel OC+SC Stub element which is responsible for the two FTZ’s in the stopband is converted into a lumped element fourth order section by applying the command Replace by Lumped / Lumped Quad. This transformation keeps the two FTZ’s unaffected. Thus, we get the all-lumped element circuit of Figure 16.36.h.

- It is preferable to have all identical shunt LC resonators. This can be done by moving the leftmost and rightmost parallel LC resonators to be adjacent to the nearby inverters by using Exchange command and then placing the cursor under one of the inverters and applying the command Inverters / Equate all Elements / Equal Inds (Figure 16.46.h-i-k).

- In order to form a circuit section equivalent to a CQ section, we need to form an inductor or capacitor coupled shunt resonator adjacent to the fourth order section. This is possible as follows.

- With cursor under the inverter on the left (or right) of the fourth order section, the command Inverters / Replace Inverter by / Pi-Tee-Type / Inductive Pi is applied to replace it with an inductive Pi section with negative shunt elements. Simplify Circuit command is applied to absorb the negative shunt elements into the adjacent resonators (Figure 16.36.m).

- Thus, the equivalent circuit of CQ section is ready as indicated in Figure 16.22.m. With cursor under the fourth order section, the command CT-CQ-CN Oprs / Convert to CQ-CN/Solution-1 is applied to convert this circuit section into a CQBP element. CTRL+W command is applied to move the resulting transformer away (Figure 16.22.n).

- This conversion destroys equality of the shunt resonator inductances. They can be re-equated by applying the command Inverters / Equate all Elements / Equal Inds (Figure 16.36.p).

- Finally, the shunt arm series LC resonators are converted into Transverse Doublets by applying the command Resonator Oprs / Off-Line Resonator. So, we get the coupled resonator version of the filter shown in Figure 16.36.q. The circuit is saved as D52b. The responses are also shown in the same figure. A slight degradation  exists in return loss.

- The delay response is used to locate the frequency of maximum delay (f=1043 MHz) which can be used to find the magnitude and location of the maximum voltage in the circuit as described in Figure 16.36.r using ALT+F8 command. The Voltage-Current-Power list for Vs=2000 volts in this example and f=1043 MHz shows that the maximum voltage is about 2994 volts and it occurs at the left inner node of the CQ section.

- External Q, Coupling Coefficients and Resonance Frequencies of the main line resonators can be displayed by deleting the Doublets and then using CTRL+F6 and ALT+F6 keys as shown in Figure 16.36.s.

- Coupling coefficients of the doublets can be displayed by forming the circuits shown in Figure 16.36.t and then applying ALT+F6 command.

- Figure 16.36.u shows a possible realization schematics using coaxial resonators. It should be noted that we have nine resonators, one more than the degree 8 of the filter. The extra resonator is due to the single FTZ.

SOLUTION-2:  Solution with two CT and one CQ sections

Using the same approach as in the previous example, it is possible to form a solution having two CT and one CQ section (Exercise ) . The resulting circuit is shown in Figure 16.36.v.

Figure 16.36. Design of degree 8 dual band elliptic filter.

Figure 16.36. Design of degree 8 dual band elliptic filter.

Figure 16.36. Design of degree 8 dual band elliptic filter.

Figure 16.36. Design of degree 8 dual band elliptic filter.

Figure 16.36. Design of degree 8 dual band elliptic filter.

## EXAMPLE 16. 18. A degree 10 dual band filter with two finite transmission zeros

Let’s design a dual band filter of degree 10 and having two FTZ’s in the upper stopband of the first passband. The specs of the targeted dual band filter are as follows:

PBRipple = 0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz

for the first passband and

fp1B=1080 MHz, fp2B=1100 MHz

for the second passband. The quarter wavelength frequency of the filter is selected as the arithmetic center of the stopband:

fq=(fp2A+fp1B)/2 = 1060 MHz

Design steps are shown in Figure 16.37.

- Design / By Synthesis option is selected.

- Filter Type: Distributed / BP / Equiripple / Doubly Terminated / Type B options are selected.

- PB and Terminations: fp1=1020 MHz, fp2 = 1040 MHz, fq=1060 MHz are typed in. We will design the filter for the first passband. The second passband will come out automatically.

- Transmission Zeros: Nzero=5, Ninf=1, FTZ1=1050 MHz, FTZ2=1054 MHz. The number of TZ’s at f=0 are set to adjust the attenuation and slope outside the two passbands while the FTZ is tuned to set the stopband level in between the two passbands, as seen in the figure. Although according to the formal definition we have only two FTZ’s, considering the mirror images of these two FTZ’s and also the TZ at f=fq=1060 MHz (the TZ at infinity of the transformed frequency variable), we effectively have five FTZ’s as is seen from the insertion loss plot.

- Extract TZ’s: FTZ1, Zero, Zero, Zero, Zero, FTZ2, Zero, Inf.

- We get the circuit shown in Figure 16.37.a. The command Pi-Tee-L / Equal Shunt SC Stubs is applied to convert the filter into coupled resonator form with all shunt resonators formed as parallel connection of OC and SC stubs (Figure 16.37.b).

- The command Split / All 2-Element Resonators is applied to split the shunt OC+SC Stub resonators into their SC and OC components (Figure 16.37.c).

- Then using Exchange command a Pi-section of OC stubs is formed in the left half of the circuit (Figure 16.37.d).

- From that point on different alternative paths may be followed leading to different solutions. One possible solution is described below.

- With cursor under the series OC stub of the Pi-section on left half, the command Pi-Tee-L / Pi-to-LLeft is applied to convert the Pi-section into an LLeft section (Figure 16.37.e).

- Then Exchange command is applied to form a second Pi-section of OC stubs and with cursor under the series OC stub of the Pi-section on the right half, the command Pi-Tee-L / Pi-to-LRight is applied to convert it into an LRight section (Figure 16.37.f-g). CTRL+W keys are used to move the resulting transformers to the load end.

- With cursor under the series OC stubs, the command Inverters / Replace by Inverter Equivalent: K=50 is applied to convert them into shunt SC stubs with 50 ohms inverters on both sides (Figure 16.37.h). Impedances of the inverters are not important at this stage. They can be adjusted later.

- The command Inverters / Equate all Elements / Equal Inds is applied to equate the shunt SC stub impedance (Figure 16.37.i).

- Now we will convert all stubs into their lumped element equivalents while keeping the dual band response unchanged. This is done by applying the command Replace by Lumped / LC Resonator to all single OC and SC stubs. The OC stubs are converted into series LC resonators while the SC stubs are converted into parallel LC resonators (Figure 16.37.k).

- The series arm parallel OC+SC Stub elements which are responsible for the two FTZ’s in the stopband are converted into lumped element fourth order sections by applying the command Replace by Lumped / Lumped Quad. This transformation keeps the four FTZ’s unaffected. Thus, we get the all-lumped element circuit of Figure 16.37.m.

- With cursor under the inverter on the right of the fourth order section on left half, the command Inverters / Replace Inverter by / Pi-Tee-Type / Inductive Pi is applied to replace it with an inductive Pi section with negative shunt elements. Simplify Circuit command is applied to absorb the negative shunt elements into the adjacent resonators (Figure 16.37.n).

- Thus, the equivalent circuit of CQ section is ready as indicated in Figure 16.37.n. With cursor under the fourth order section, the command CT-CQ-CN Oprs / Convert to CQ-CN/Solution-1 is applied to convert this circuit section into a CQBP element. CTRL+W command is applied to move the resulting transformer away (Figure 16.37.p).

- This conversion destroys equality of the shunt resonator inductances. They can be re-equated by applying the command Inverters / Equate all Elements / Equal Inds (after moving the shunt resonator to be adjacent to the inverter using Exchange command) (Figure 16.37.q).

- Next, with cursor under the inverter on the left of the fourth order section on the right half, the command Inverters / Replace Inverter by / Pi-Tee-Type / Inductive Pi is applied to replace it with an inductive Pi section with negative shunt elements. Simplify Circuit command is applied to absorb the negative shunt elements into the adjacent resonators (Figure 16.37.r).

- Thus, the equivalent circuit of the second CQ section is ready as indicated in Figure 16.37.r. With cursor under the fourth order section, the command CT-CQ-CN Oprs / Convert to CQ-CN/ Solution-1 is applied to convert this circuit section into a CQBP element. CTRL+W command is applied to move the resulting transformer away (Figure 16.37.s).

- This conversion destroys equality of the shunt resonator inductances. They can be re-equated by placing the sursor under an inverter and applying the command Inverters / Equate all Elements / Equal Inds (Figure 16.37.t).

- Finally, the shunt arm series LC resonators are converted into Transverse Doublets by applying the command Resonator Oprs / Off-Line Resonators. So, we get the coupled resonator version of the filter shown in Figure 16.37.u. The responses are also shown in the same figure. A slight degradation  exists in return loss.

- The delay response is used to locate the frequency of maximum delay (f=1042 MHz) which can be used to find the magnitude and location of the maximum voltage in the circuit as described in Figure 16.37.v using ALT+F8 command. The Voltage-Current-Power list for Vs=2000 volts in this example and f=1042 MHz shows that the maximum voltage is 3872 volts and it occurs at the right inner node of the CQ section on left half. This voltage level is much higher than we have seen in the previous examples. We should expect this because of the very high delay at f=1042 MHz.

- Figure 16.37.x shows a possible realization schematics using coaxial resonators. It should be noted that we have 12 resonators, two more than the degree 10 of the filter. The two extra resonators are due to the two FTZ’s.

Figure 16.37. Design of degree 10 dual band elliptic filter.

Figure 16.37. Design of degree 10 dual band elliptic filter.

Figure 16.37. Design of degree 10 dual band elliptic filter.

16. 4. 3. DUAL BAND FILTER DESIGN USING NORMALIZED BP PROTOTYPES

The dual band filters studied in the previous subsections lead to equal bandwidths. The approach that will be described in this subsection will result in a slightly wider second bandwidth. For narrow stopbands between the two passbands the difference between the two bandwidths will be negligible. Therefore using the technique of this subsection we can get exactly the same solutions as in the technique of the previous approach.

The classical geometrically symmetric BP filters are designed by applying LP-to-BP transformation function

w= (wo/B)(w/wo-wo/w) 								(16.1)

to a normalized prototype LP filter with band edge wp=1 and RS=RL=1 where w is the normalized frequency.

If this transformation is applied to a normalized BP prototype then we get a dual passband BP filter [12]. The normalized BP prototype may be designed as having the upper band edge w2=1 and lower band edge w1<w2as described in Figure 16.38. In the figure the frequencies w1a, w2a and w1b, w2b are the desired passband edge frequencies of the two passbands which must be selected to satisfy the geometric symmetry constraint around the geometric center of the stopband wo,

w1a w2b =w2a w1b =wo2							(16.2)

or

fp1A fp2B = fp2A fp1B = fo2 							(16.3)

The lower band edge w1 of the prototype should be evaluated from the dual band filter specifications as

w1= Bs / B									(16.4)

where Bs is the stop bandwidth and B is the difference between outer edges of the two passbands:

Bs = w1b-w2a  									(16.5)

B = w2b-w1a  									(16.6)

Hence w1 may also be calculated as

w1 = Bs / B = (f1b-f2a) / (f2b-f1a) 						(16.7)

With the transformation given in Eq. 16.1, the lower edge w1  is mapped to the inner passband edges w2a and w1b  while the upper passband edge w2=1 of the prototype is mapped to the outer edge frequencies w1a and w2b.

In a typical design we select only three of the desired passband edge frequencies w1a, w2a, w1b and w2b. Then the last one is calculated using Eq. 16.2 (or 16.3). Then the bandwidths B and Bs are found using Eqs. 16.5 and 16.6. Finally Eq. 16.4 is used to find wx. Once the lower edge wx of the normalized BP prototype is determined (together with the degree and passband ripple necessary for the targeted filter specifications) then using any one of the available design techniques, a normalized lumped element BP filter can be designed with passband edges

fp1= w1 /2   and   fp2=w2/2 =1/2 = 0.159 					(16.8)

and

RS=RL=1

Then this normalized BP filter will be treated as if it is a LP prototype and the classical LP-to-BP mapping is applied to yield a dual band lumped element BP filter.

The lower stopband of the prototype BP filter is mapped as the stopband of the dual band filter. Therefore the FTZ in the lower stopband of the prototype filter can be used to improve the stopband selectivity of the dual band filter. The FTZ’s in the upper stopband of the prototype filter are mapped to the outer stopbands of the dual band filter.

Figure 16.38. Mapping between a normalized BPF and dual band filter.

The main property of this approach is that the pass bandwidths of the two bands will be different, related through the geometric symmetry property around wo. The lumped element filter may also be converted into distributed element form using approximate transformations which may lead to bandwidth changes.

Let’s see some examples on Filpro to see how these steps are carried out.

EXAMPLE 16. 19.  Dual band filters using degree 6 Chebyshev prototype

Let’s design a dual band filter with PBRipple = 0.01 dB, using a degree 6 Chebyshev BPF. The passband edge frequencies of the first passband is desired to be

fp1A=1020 MHz, fp2A=1040 MHz

The lower edge of the second passband is fp1B=1080 MHz. Then its upper edge fp2B is calculated using Eq. 16.3 as follows:

fp2B = (fp2A) (fp1B) / fp1A = (1040) (1000) / 1020 = 1101 MHz

Thus, the second passband will cover the range from fp1B=1080 MHz to fp2B=1101, leading to a bandwidth of  41 MHz while the first passband has a bandwidth of 40 MHz.  That is, a slight difference exists between the two bandwidths. This difference gets larger when the passbands get away from each other. The geometric center of the stopband fo is calculated using Eq. 16.3 as

fo = 1059.8 MHz

Now we are ready to calculate the parameters of the normalized BP prototype. Using Eq. 16.5 and 16.6, we get

w1= Bs / B = (f1b-f2a) / (f2b-f1a) =(1080 - 1040) / (1101 - 1020) = 40 / 81 = 0.4938

So, we will design a normalized BP filter with lower band edge wp1= w1= 0.4938 and upper band edge wp2 =wc = 1. Next we convert these parameters into the form that is needed in Filpro, as follows:

fp1 = w1 / 2 = 0.07863

fp2 = wc / 2 =  0.159

We can use different approaches leading to different final topologies, like filters having off-line resonators or filters having cross-coupled resonators. Some of the possible versions are described below.

A) Dual Band filter with off-line resonators (transverse Doublets):

Design of the normalized BP prototype:

We are now ready to design the normalized BP prototype. We can use any one of the design techniques on Filpro. Since we would like to have a coupled resonator type filter, the shortest approach seems to be Design / By Prototype with Inverters approach. Design steps are described in Figure 16.39.

- Design / By Prototype with Inverters option is selected.

- Choose a Standard Circuit / Shunt Capacitors and Inverters option is selected.

- The parameters Prototype Degree N=3 and Passband Ripple=0.01 dB are typed in. The circuit shown in Figure 16.39.a will be displayed.

- Filter Type: Lumped, BPF options are selected.

- Find Circuit command is invoked and the parameters fp1=0.07863, fp2=0.159 and RS=RL=1 are typed in. We get the circuit shown in Figure 16.39.b.

- The command Inverters / Equate All Elements / Equate Inductors is applied to get identical resonators (Figure 16.39.c). The resulting unity transformer is deleted (Figure 16.39.d). The command Change Units is used to set the units of inductors and capacitors as H and F (convention used in normalized prototypes!). This filter is the normalized prototype that will be used to design the dual band filter. Its response is shown in Figure 16.39.e. It is saved as Prot1.fil.

Design of the dual band filter:

Now we will design the dual band filter using the BP prototype Prot1. Design steps are shown in Figure 16.40.

- Design / By User Defined Prototype command is selected. Load a Prototype option is selected and the prototype Prot1 is loaded. Filter Type: Lumped, BPF options are selected.

- Passband and Terminations command is invoked and the lower edge of the first passband, fp1A=1020 MHz is typed in as fp1. The upper edge of the second passband, fp2B=1101 MHz is typed in as fp2.

- Find Circuit command gives the desired dual band filter shown in Figure 16.40.a. Change Units command is used to set the units of inductors and capacitors as nH and pF.

- Inverters / Scale Cct by Inverters / Specify Inductors: L=10 nH is applied to adjust the impedance level of the circuit to set all resonator inductors to 10 nH (e.g.) (Figure 16.40.b).

- Figure 16.40.c shows the dual band response. The circuit is saved as Dual1.

- This circuit may be converted into different forms for realization. For example the inverters may be replaced by quarter wavelength TL’s and the parallel and series LC resonators are converted into stubs. However that approach would give the circuits discussed in the previous sections which are easier to design by the approaches described in those sections.  One interesting approach is described in Figure 16.40.d-e-f where the shunt arm series LC resonators are converted into shunt arm parallel resonators, giving a coupled resonator version. This is done as follows:

- The circuit Dual51 is loaded (Figure 16.40.d).

- By placing the cursor under the series LC resonators and applying the command Resonator Oprs / Off-Line Resonator is applied converting the shunt arm series LC resonators into elements denoted by trDoublet (Figure 16.40.e). The trDoublet elements are formed by inverters followed by a shunt LC resonator. The exploded views of these are shown in Figure 16.40.f. That is, the shunt arm series LC resonators are converted into parallel LC resonators by inserting inverters in front of them. The inverters are adjusted so that the L and C values of these resonators are identical to the main line resonators. The resonators of these elements will be termed as Off-line Resonators (or off-line doublets or transverse doublets?) while the others will be termed as main-line resonators. It is seen that each node has a pair of coupled resonators, one mainline type and the other offline type.

- Thus, we get a circuit made up of all identical six shunt arm parallel LC resonators coupled by inverters. In this circuit the main line resonators form the BP response while the side resonators form the stopband response splitting the passband between fp1A=1020 MHz and fp2B=1101 MHz. This circuit is saved as Dual1a. Figure 16.40.g shows a schematics for a filter made up of coaxial resonators. The inverters may be replaced by proper practical equivalents like inductive-Pi, capacitive-Pi, TL, WG or Evanescent WG sections for realization using different technologies.

- By applying the command Parasitics / All Lossy Resonators: Q=500 we can form the lossy version of the filter for Q=500, as shown in Figure 16.40.h-i.

- Figure 16.40.k shows the responses after conversion into lossy form. The notch in stopband center is reduced to about 80 dB while passband loss is about 2 dB at band edges.

- In the present version of Filpro the External Q and Coupling Coefficient parameters can’t be displayed in the presence of the off-line resonators. For this reason in order to display these parameters we Delete the off-line resonators and use ALT+F6 and CTRL+F6 keys as described in Figure 16.40.m. The coupling coefficients of the offline resonators are displayed by forming the coupled resonator section on Filpro as shown in Figure 16.40.n. In this circuit we type in the inverter value of the trDoublet element as the coupling inverter impedance.

Observation: A dual band filter design from a degree N prototype BPF with no FTZ’s (Chebyshev, Butterworth, .. ) can be realized using N resonators.

Figure 16.39. Design of a degree 6 normalized Chebyshev BP prototype filter.

Figure 16.40. Coupled resonator type dual band filter designed from BP prototype.

Figure 16.40. Coupled resonator type dual band filter designed from BP prototype.

Figure 16.40. Coupled resonator type dual band filter designed from BP prototype.

## B) Dual band filter with a cross-coupled triplet (CT) or a quadruplet (CQ) section:

In this approach we will design the BP prototype using Design / By Synthesis approach to get easier control on the transmission zeros of the prototype filter.

Design of the prototype:

Design steps of the prototype are described in Figure 16.41.

- Design / By Synthesis option is selected.

- Filter Type: Lumped / BP / Equiripple / Doubly Terminated / Type A options are selected.

- PB and Terminations: fp1=0.07863, fp2=0.159 are typed in.

- Transmission Zeros: Nzero=1, Ninf=5. The number of TZ’s at f=0 are set to adjust the attenuation and slope outside the two passbands while the number of TZ’s at infinity will shape the stopband in between the two passbands. For simplicity let’s place only one TZ at f=0.

- Extract TZ’s: Inf, Inf, Inf, Zero, Inf, Inf.

- We get the circuit shown in Figure 16.41.a. The inductors will map into the dual band filter as series LC resonators while capacitors will map as parallel LC resonators. We aim at obtaining  series arm parallel resonator for forming the FTZ at stopband center. This will be provided by the series arm capacitor in the figure. In order to form a cross-coupled triplet (CT) section, the series arm parallel LC resonator must be surrounded by shunt LC resonators. This is possible by converting the capacitive-LLeft section into a capacitive-Pi section by applying the command Pi-Tee-L / LLeft-to-Sym Pi (Figure 16.41.b).

- Simplify Circuit command moves away the resulting transformer (Figure 16.41.c).

- We want only shunt arm parallel resonators. Therefore we convert the series arm inductors into shunt capacitors by applying the command Inverters / Replace by Inverter Equivalent: K=1 (Figure 16.41.d).

- The command Inverters / Equate All elements is applied to equate the shunt capacitors and the command Inverters / Absorb Trf is applied to absorb the load end transformer into the adjacent inverter (Figure 16.41.e). The resulting circuit is saved as Prot2. It will be used to design the dual band filter.

Design of the dual band filter:

Now we will design the dual band filter using the BP prototype Prot2. Design steps are shown in Figure 16.42.

- Design / By User Defined Prototype command is selected. Load a Prototype option is selected and the prototype Prot2 is loaded. Filter Type: Lumped, BPF options are selected.

- Passband and Terminations command is invoked and the lower edge of the first passband, fp1A=1020 MHz is typed in as fp1. The upper edge of the second passband, fp2B=1101 MHz is typed in as fp2.

- Find Circuit command gives the desired dual band filter shown in Figure 16.42.a. Change Units command is used to set the units of inductors and capacitors as nH and pF. The circuit is saved as Dual2.

The filter can be put into forms having either a cross-coupled triplet (CT) section or quadruplet CQ section. Let’s see both of them.

CT Version:

- The element that creates the FTZ at stopband center is the series arm parallel LC resonator. With cursor under the inverter on the left this FTZ element the command Inverters / Replace Inverter by / Pi-Tee-Type / Inductive Pi is applied to convert the inverter into an inductive Pi section with negative shunt arms (Figure 16.42.b). The negative elements are absorbed into the adjacent positive elements by applying Simplify Circuit command (Figure 16.42.c).

- Thus, we have formed a circuit section which is equivalent to a CT section as indicated in the figure. Now by placing the cursor under the FTZ element the command CT-CQ-CN Oprs / Convert to CT is applied and the option CLL is selected to get capacitive cross-coupling in the resulting CT section. We get the circuit of Figure 16.42.d.

- The resonator element values are effected by the transformation into CT form. We can re-equate them by placing the cursor under an inverter and applying the command Inverters / Equate All Elements / Equate Inds. The transformer at load end is absorbed into the adjacent inverter (Figure 16.42.e). Thus, the design is completed. It is saved as Dual2a.

- Figure 16.42.f shows a realization schematics using coaxial resonators. It is seen that we have six resonators.

- Figure 16.42.g shows the responses. The delay response is also plot to find the frequency of the maximum delay which is also the frequency of maximum voltage within the circuit.

- Figure 16.42.h shows the list of External Q , Coupling Coefficients and Resonance frequencies of the resonators.

- Figures 16.42.i-k-m show the effects of finite Q of resonators for Q=500. The nodal Voltage-Current-Power list shows the magnitude and location of the maximum voltage at frequency f=1016 MHz which is the frequency of maximum delay. It is 1906 volts and it occurs at node-2.

CQ Version:

- The circuit of Dual2 is loaded (Figure 16.42.n).

- A cross-coupled quadruplet that realizes a single FTZ is termed as CQ1F on Filpro. Equivalent circuit of a CQ1F section is formed by a series arm FTZ element (series arm parallel resonator) surrounded by simply coupled doublets on both sides. Such a circuit section is formed by converting the inverters on the two sides of the FTZ element into their inductive Pi-section equivalents as described in Figure 16.42.p.

- Then with cursor under the FTZ element the command CT-CQ-CN Oprs / Convert to CQ-CN / Solution-1 is applied. We get the CQ1F element shown in Figure 16.42.q. The circuit is saved as  Dual2b.

- The  External Q, Coupling Coefficients and Resonance frequencies of the resonators are also shown in the same figure.

- Figure 16.42.r shows a realization schematics using coaxial resonators. This structure has also six resonators.

- Figures 16.42.s-t show the effects of finite Q of resonators for Q=500.

- The nodal Voltage-Current-Power list shows the magnitude and location of the maximum voltage at frequency f=1016 MHz which is the frequency of maximum delay. It occurs at node-2, the same node as in the CT version, but the maximum voltage (1830 volts) is less than that of the CT case (1906 volts).

Figure 16.41. Synthesis of a degree 6 prototype BPF suitable for CT or CQ type realizations.

Figure 16.42. Design of the dual band degree 6 filter having a CT or CQ section.

Figure 16.42. Design of the dual band degree 6 filter having a CT or CQ section.

Figure 16.42. Design of the dual band degree 6 filter having a CT or CQ section.

Figure 16.42. Design of the dual band degree 6 filter having a CT or CQ section.

Figure 16.42. Design of the dual band degree 6 filter having a CT or CQ section.

EXAMPLE 16. 20:  A dual band filter design using an elliptic degree 6 BP Prototype

In this approach we will prepare a normalized BP prototype with finite transmission zeros (FTZ’s). Just as a trial let’s consider a small degree BPF, for example N=6. Let’s place FTZ’s only in lower stopband of the prototype filter which will map to the stopband between two passbands of the dual band filter.

Specs of the targeted dual band filter are as follows:

PBRipple = 0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz for the first passband and fp1B=1080 MHz, fp2B=1101 MHz.

Design of the normalized BP prototype:

Using these frequencies the lower passband edge of the normalized BP prototype filter is found as

fp1= BS / (2B)  = (1080 – 1040) / [2

The upper edge of the passband is

fp2 = 1 / 20.159

Design steps of the normalized prototype BPF are described in Figure 16.43:

- Design / By Synthesis option is selected.

- Filter Type: Lumped / BP / Equiripple / Doubly Terminated / Type B options are selected.

- PB and Terminations are typed in as fp1=0.07863, fp2=0.159, RS=RL=1.

- Transmission Zeros are typed in as Zero, Inf, Inf, Inf, FTZ=0.025 to form a BPF with degree 6. The finite transmission zero is tuned to adjust the lower stopband attenuation. The transmission zeros at f= are used to set the slope of the upper stopband. At least one TZ is needed at f=0 because the FTZ in lower stopband must be extracted by partial pole extraction form the TZ at f=0.

- Then the transmission zeros are extracted in the order Inf-Inf-FTZ-Inf-Zero. As will be seen, the first three TZ’s, that is, Inf-Inf-FTZ will be used to form CQ section in the dual band filter. That is, this order of extraction is necessery.

- We get the circuit shown in Figure 16.43.a. In this circuit we apply the command Pi-Tee-L / Equal Shunt Inductors (or Capacitors) to form a coupled resonator version of the circuit (Figure 16.43.b).

- Then with cursor under any shunt LC resonator, the command Split / All 2-Element Resonators is applied to split them into L and C components (Figure 16.43.c).

- Then we form a Pi-section of inductors by exchanging the locations of the first two elements on left end (Figure 16.43.d).

- Next, with cursor under the series inductor of the Pi-section, the command Pi-Tee-L / Pi-to-LRight is applied to convert the Pi-section into an LRight section (Figure 16.43.e). The keys Ctrl+W are pressed to move the resulting transformer to the load end.

- Using Exchange command the shunt inductor at left is moved to the source end (Figure 16.43.f).

- The aim of the last five stages of transformations is to reduce the number of shunt inductors. Later it will be seen that the shunt inductors of this normalized prototype will be mapped as shunt arm series LC resonator in the dual band filter. We would like to keep their numbers as small as possible and we prefer them to appear at the source and load ends, if possible. Now we have two shunt inductors, one at source end and the other is at load end.

- The normalized BP prototype is ready. We save it as Prot21. In the next stage, we will use this prototype to design the dual band filter.

Figure 16.43. Design of the elliptic degree 6 normalized prototype BP filter.

Design of the dual band filter:

The command set for the design of the dual band filter are shown in Figure 16.44.

- The command Design / By User Defined Prototype is invoked.

- Load a Prototype option is selected and Prot21 is typed in. The BP prototype will be displayed.

- Filter Type: Lumped / BPF options are selected.

- PB and Terminations command is invoked. The lower edge, f=1020 MHz of the first passband is typed in as fp1and the upper edge of the second passband 1101 MHz is typed in as fp2. RS=RL=50 ohms are typed in.

- Find Circuit command gives the circuit shown in Figure 16.44.a.

- Inductor and Capacitor units are set as nH, pF by using Change Units command.

- Figure 16.44.b shows the dual band response.

- We note that the inductors of the prototype are mapped as series LC resonators while the shunt capacitors are mapped as shunt parallel resonators. The series arm parallel LC resonator of the prototype (the FTZ producing element of the prototype) is mapped as fourth order section that leads to two FTZ’s which are geometrically symmetric with respect to the passband center.

- Our target is to form a circuit section which is equivalent to a CQ section for realization of the FTZ pair. Such a circuit section can be formed by producing an inductively or capacitively coupled resonator pair on the left or right of the fourth order section. For this purpose with cursor under the series arm series LC element, the command Inverters / Replace by Inverter Equivalent is applied to convert it into a shunt arm parallel LC resonator in between two inverters (Figure 16.44.c). This command asks the impedance of the inverter. We accept the displayed default value because element values will be adjusted later. We delete the unity phase changing transformer.

- With cursor under the inverter close to the fourth order section, the command Inverters / Replace Inverter by / Pi-Tee Type / Inductive Pi is applied to convert the inverter into inductive Pi-section with negative shunt elements (Figure 16.44.d).

- The negative shunt inductors are combined with adjacent positive inductors (Figure 16.44.e).

- Thus, we have formed the circuit section which is equivalent to a CQ section. It is shown in Figure 16.44.e. With cursor under the fourth order section the command CT-CQ-CN Oprs / Convert to CQ-CN / Solution-1 is applied to convert this circuit section into a CQ section (Figure 16.44.f).

- Ctrl+W keys are used to move away the resulting transformer (Figure 16.44.g).

- Inspection of the CQ section shows that, besides the usual 1-4 cross-coupling, there is a diagonal cross-coupling between resonators 2 and 4, leading to CQ24 type CQ section. We don’t expect this diagonal cross-coupling because the two FTZ’s are symmetrically located with respect to center frequency. It has arisen because of approximations involved in conversion to and from inverters. We can make the diagonal elements ineffective by setting C24=0 and L24= (a large number). Another way is to delete the CQ24 element and insert the classical CQBP element without diagonal cross-coupling.

- Inspection of the shunt inductors shows that the leftmost inductor is not equal to the others. If necessary, we may equate it to the other one by placing the cursor under the inverter and applying the command Inverters / Scale Inverter / Equal Inductors (Figure 16.44.h). Using Ctrl+W keys moves away the resulting transformer to the load end where it is combined with the transformer there. We notice that the total transformer is now unity. Hence it is deleted.

- Next, the shunt arm series resonators which form the TZ’s at stopband center are converted into off-line shunt arm parallel LC resonators by applying the command Resonator Oprs / Off-Line Resonator. These resonators are coupled to the mainline filter circuit through inverters. The combination of Inverter+Parallel LC resonator is named as Off-Line Resonator or transverse Doublet on Filpro.

- Thus, the coupled resonator version of the filter is formed. The circuit is saved as Dual21 (Figure 16.44.i).  Its response is shown in Figure 16.44.k. The degradations in return loss can be corrected by slight tuning.

- Next, we may scale impedance level of the filter (if necessary) to a desired level. For example we may scale all shunt inductors to 10 nH. For this purpose we move the leftmost shunt parallel LC resonator to be adjacent to source using Exchange command and then apply the command Inverters / Scale cct by Inserting Inverters / Specify Inductor: L=10 nH (Figure 16.44.m-n-p). This command inserts proper inverters at both ends of the filter.

- Resonator losses can be taken into account by placing the cursor under any resonator and applying the command Parasitics / All Lossy Resonators: Q=500 (e.g.). This circuit is saved as Dual22 (Figure 16.44.q). Lossy response is shown in Figure 16.44.r.

- Figure 16.44.s shows a realization schematics using coaxial resonators. We note that there are seven resonators. The off-line resonators are numbered as 2 and 7. They arise because of the shunt arm series LC resonators which were mapped from the shunt inductors of the BP prototype. In the prototype we have a single transmission zero at f=0 which appears as shunt inductors. This single TZ leads to two shunt inductors because of the FTZ extraction by zero shifting from f=0. Therefore the two off-line resonators are actually forming a single TZ at the stopband center. It is possible to increase the number of offline resonators by increasing the number of shunt inductors in the prototype (increased redundancy). For example we would leave the prototype BP filter with three shunt inductors. However this would lead to offline resonators within the CQ section, complicating the design and construction. That is why we have made Pi-to-LRight transformations in the prototype BPF to leave the shunt inductors only at the ports.

- External Q and Coupling Coefficients of the main line resonators can be displayed as shown in Figure 16.44.t. The present version of Filpro can’t display these parameters in the presence of the offline resonators. Therefore we delete them and then use CTRL+F6 and ALT+F6 keys for displaying them.

- The coupling coefficients of the offline resonators can be displayed by first forming the Resonator+Inverter+Resonator triplets on Filpro with inverter value taken from trDoublet element and then using ALT+F6 keys (Figure 16.44.u).

Figure 16.44. Design of the elliptic dual band filter using degree 6 prototype.

Figure 16.44. Design of the elliptic dual band filter using degree 6 prototype.

Figure 16.44. Design of the elliptic dual band filter using degree 6 prototype.

Figure 16.44. Design of the elliptic dual band filter using degree 6 prototype.

EXAMPLE 16. 21:  A dual band CQ filter designed from degree 10 elliptic prototype

In this example we will prepare a degree N=10 normalized BP prototype with two FTZ’s in lower stopband of the prototype filter which will map to the stopband of the dual band filter.

Specs of the targeted dual band filter are the same as the previous example:

PBRipple=0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz for the first passband and fp1B=1080 MHz, fp2B=1101 MHz.

Design of the normalized BP prototype:

Using these frequencies the lower passband edge of the normalized BP prototype filter is found as

fp1= BS / (2B)  = (1080 – 1040) / [2

The upper edge of the passband is

fp2 = 1 / 20.159

Design steps of the normalized prototype BPF are described in Figure 16.45:

- Design / By Synthesis option is selected.

- Filter Type: Lumped / BP / Equiripple / Doubly Terminated / Type B options are selected.

- PB and Terminations are typed in as fp1=0.07863, fp2=0.159, RS=RL=1.

- Transmission Zeros are typed in as Zero, Inf, Inf, Inf, Inf, Inf, FTZ1=0.0348, FTZ2=0.05 to form a BPF of degree 10. The finite transmission zeros are tuned to adjust the lower stopband attenuation to be equiripple. The transmission zeros at f= are used to set the slope and attenuation of the upper stopband. At least one TZ is needed at f=0 because the FTZ’s in lower stopband must be extracted by partial pole extraction form the TZ at f=0.

- Then the transmission zeros are extracted in the order FTZ1-Inf-Inf-Inf-Inf-FTZ2-Inf-Zero. As will be seen, the first three TZ’s, that is, FTZ1-Inf-Inf will be used to form one CQ section and the next triplet of TZ’s Inf-Inf-FTZ2 will be used to form the second CQ section to realize the four  FTZ’s of the dual band filter.

- We get the circuit shown in Figure 16.45.a. In this circuit we apply the command Pi-Tee-L / Equal Shunt Inductors (or Capacitors) to form a coupled resonator version of the circuit (Figure 16.45.b).

- Then with cursor under any shunt LC resonator, the command Split / All 2-Element Resonators is applied to split them into L and C components (Figure 16.45.c).

- With cursor under the series element of the Pi-section of inductors the command Pi-Tee-L / Pi-to-LLeft is applied to convert the Pi-section into an LLeft section (Figure 16.45.d). The keys Ctrl+W are pressed to move the resulting transformer to the load end (Figure 16.45.e).

- With cursor under the shunt leg of the LLeft section, Exchange command is applied to form a second inductive Pi-section as shown in Figure 16.45.f. Then this Pi-section is converted into LRight section using the command Pi-Tee-L / Pi-to-LRight (Figure 16.45.g).

- The resulting transformer is moved to the load end using CTRL+W command giving the circuit of Figure 16.45.h.

- The aim of the Pi-Tee-L transformations is to reduce the number of shunt inductors. We would like to keep their numbers as small as possible because they will lead to offline resonators, as was seen in the previous example. In this circuit we have reduced the number of shunt inductors three, one in the middle of the circuit and the other two at the two ports.

- The normalized BP prototype is ready. It is saved as Prot23. In the next stage, we will use this prototype to design the dual band filter.

Figure 16.45. Design of the elliptic, degree 10 prototype.

Figure 16.45. Design of the elliptic, degree 10 prototype BPF.

Design of the dual band filter using the elliptic prototype:

The command set for the design of the dual band filter are shown in Figure 16.46.

- The command Design / By User Defined Prototype is invoked.

- Load a Prototype option is selected and Prot23 is typed in. The BP prototype will be displayed.

- Filter Type: Lumped / BPF options are selected.

- PB and Terminations command is invoked. The lower edge, f=1020 MHz of the first passband is typed in as fp1and the upper edge of the second passband 1101 MHz is typed in as fp2. RS=RL=50 ohms are typed in.

- Find Circuit command gives the circuit shown in Figure 16.46.a.

- Inductor and Capacitor units are set as nH, pF by using Change Units command.

- Figure 16.46.b shows the dual band response.

- We note that the inductors of the prototype are mapped as series LC resonators while the shunt capacitors are mapped as shunt parallel resonators. The series arm parallel LC resonators of the prototype (the FTZ producing element of the prototype) are mapped as fourth order sections. Each fourth order section leads to two FTZ’s which are geometrically symmetric with respect to the passband center.

- Our target is to form circuit sections which are equivalent to CQ sections for realization of the pairs of FTZ’s. Such circuit sections can be formed by producing an inductively or capacitively coupled resonator pair on the left or right of the fourth order sections. For this purpose with cursor under the series arm series LC elements, the command Inverters / Replace by Inverter Equivalent is applied to convert them into shunt arm parallel LC resonator in between two inverters (Figure 16.46.c). This command asks the impedance of the inverter. We accept the displayed default value because element values will be adjusted later. We delete the unity phase changing transformers.

- Then with cursor under the inverters close to the fourth order sections, the command Inverters / Replace Inverter by / Pi-Tee Type / Inductive Pi is applied to convert the inverter into inductive Pi-section with negative shunt elements (Figure 16.46.d).

- The negative shunt inductors are combined with adjacent positive inductors. We also move the shunt LC resonator at load end to be adjacent to the fourth order section using Exchange command (Figure 16.46.e). Thus, we have formed two circuit sections which are equivalent to CQ sections. They are formed by three shunt LC resonators, with one pair of resonators being coupled through a fourth order section and the third one inductively coupled as shown in Figure 16.46.e.

- Next, with cursor under the fourth order sections the command CT-CQ-CN Oprs / Convert to CQ-CN / Solution-1 is applied to convert them into CQ sections and Ctrl+W keys are used to move away the resulting transformers giving the CQ version of the filter shown in Figure 16.46.f. Note that contrary to the previous example, now we have simple CQ sections (no diagonal cross-couplings).

- Inspection of the shunt inductors shows that the shunt inductors of the resonators in the left half are not equal to the shunt inductors on the right half. If necessary, we may equate them by placing the cursor under the inverter in the left half and applying the command Inverters / Scale Inverter / Equal Inductors (Figure 16.46.g). Using Ctrl+W keys moves away the resulting transformer to the load end where it is combined with the transformer there. Then the middle resonator is moved to be adjacent to the inverter in the right half and the same commands are repeated (Figure 16.46.h. We note that the total transformer is now unity. Hence it is deleted getting the circuit of Figure 16.46.i.

- Next, the shunt arm series resonators which form the TZ’s at stopband center are converted into off-line shunt arm parallel LC resonators by applying the command Resonator Oprs / Off-Line Resonator. These resonators are coupled to the mainline filter circuit through inverters. The combination of Inverter+Parallel LC resonator is named as Transverse Doublet on Filpro.

- Thus, the coupled resonator version of the filter is formed. The circuit is saved as Dual23 (Figure 16.46.k). Its response is shown in Figure 16.46.m. The degradations in return loss can be corrected by slight tuning.

- Figure 16.46.n shows a realization schematics using coaxial resonators. We note that there are 12 resonators. The offline resonators are numbered as 2, 7 and  12. They arise because of the shunt arm series LC resonators which were mapped from the shunt inductors of the BP prototype. In the prototype we have a single transmission zero at f=0 which appears as shunt inductors. This single TZ leads to three shunt inductors because the two FTZ’s are extracted by zero shifting from f=0. Thus, elliptic prototype with two FTZ’s leads to two extra resonators compared to a Chebyshev prototype of the same degree.

- External Q and Coupling Coefficients of the main line resonators can be displayed as shown in Figure 16.46.p-q. The present version of Filpro can’t display these parameters in the presence of the offline resonators. Therefore we delete them and then use CTRL+F6 and ALT+F6 keys for displaying them.

- The coupling coefficients of the offline resonators can be displayed by first forming the Resonator+Inverter+Resonator triplets on Filpro with inverter value taken from trDoublet element and then using ALT+F6 keys (Figure 16.46.r).

- Scaling of the element values can be done and the effects of finite Q of the resonators can be inspected using the same approach as the previous example.

Figure 16.46. Design of the elliptic dual band filter using degree 10 prototype.

Figure 16.46. Design of the dual band filter using degree 10 prototype.

Figure 16.46. Design of the elliptic dual band filter using degree 10 prototype.

Figure 16.46. Design of the elliptic dual band filter using degree 10 prototype.

EXAMPLE 16. 22:  A dual band CQ filter designed from degree 12 elliptic prototype

In this example we will prepare a degree N=12 normalized BP prototype with two FTZ’s in lower stopband of the prototype filter which will map to the stopband of the dual band filter.

Specs of the targeted dual band filter are the same as the previous example:

PBRipple=0.01 dB, fp1A=1020 MHz, fp2A=1040 MHz for the first passband and fp1B=1080 MHz, fp2B=1101 MHz.

Design of the normalized BP prototype:

Using these frequencies the lower passband edge of the normalized BP prototype filter is found as

fp1= BS / (2B)  = (1080 – 1040) / [2

The upper edge of the passband is

fp2 = 1 / 20.159

Design steps of the normalized prototype BPF are described in Figure 16.47:

- Design / By Synthesis option is selected.

- Filter Type: Lumped / BP / Equiripple / Doubly Terminated / Type B options are selected.

- PB and Terminations are typed in as fp1=0.07863, fp2=0.159, RS=RL=1.

- Transmission Zeros are typed in as Zero, Inf, Inf, Inf, Inf, Inf, Inf, Inf, FTZ1=0.0348, FTZ2=0.05 to form a BPF of degree 12. The finite transmission zeros are tuned to adjust the lower stopband attenuation to be equiripple. The transmission zeros at f= are used to set the slope and attenuation of the upper stopband. At least one TZ is needed at f=0 because the FTZ’s in lower stopband must be extracted by partial pole extraction form the TZ at f=0.

- Then the transmission zeros are extracted in the order Inf-Inf-FTZ1-Inf-Inf-FTZ2-Inf-Inf-Inf-Zero. As will be seen, the first three TZ’s, that is, FTZ1-Inf-Inf will be used to form one CQ section and the next triplet of TZ’s Inf-Inf-FTZ2 will be used to form the second CQ section to realize the four  FTZ’s of the dual band filter.

- We get the circuit shown in Figure 16.47.a. In this circuit we apply the command Pi-Tee-L / Equal Shunt Inductors (or Capacitors) to form a coupled resonator version of the circuit (Figure 16.47.b).

- Then with cursor under any shunt LC resonator, the command Split / All 2-Element Resonators is applied to split them into L and C components (Figure 16.47.c).

- Exchange command is used to form three Pi-sections of inductors as shown in Figure 16.47.d.

- With cursor under the series element of the Pi-sections of inductors the command Pi-Tee-L / Pi-to-Tee is applied to convert the Pi-sections into an Tee-sections (Figure 16.47.e). This transformation reduces the number of shunt inductors which will map as FTZ elements into the dual band filter.

- With cursor under the series inductors of the Tee sections, the command Inverters / Replace by Inverter Equivalent: K=1 is applied to convert them into shunt capacitors.

- Also with cursor under the shunt inductors of the Tee sections, the command Inverters / Replace by Inverter Equivalent: K=1 is applied to convert them into series capacitors. Thus, we get a circuit made up of inverters and capacitors as Figure 16.47.f.

- Simplify Circuit command combines the adjacent inverters and eliminates them (Figure 16.47.g).

- Inverters / Equate All Elements command equates the shunt capacitors (Figure 16.47.h). The normalized BP prototype is ready. It is saved as Prot24. In the next stage, we will use this prototype to design the dual band filter.

Figure 16.47. Design of an elliptic degree 12 prototype.

Figure 16.47. Design of an elliptic degree 12 prototype.

Design of the dual band filter using degree 12 prototype:

The command set for the design of the dual band filter are shown in Figure 16.48.

- The command Design / By User Defined Prototype is invoked.

- Load a Prototype option is selected and Prot24 is typed in. The BP prototype will be displayed.

- Filter Type: Lumped / BPF options are selected.

- PB and Terminations command is invoked. The lower edge, f=1020 MHz of the first passband is typed in as fp1and the upper edge of the second passband 1101 MHz is typed in as fp2. RS=RL=50 ohms are typed in.

- Find Circuit command gives the circuit shown in Figure 16.48.a.

- Inductor and Capacitor units are set as nH, pF by using Change Units command.

- Since the prototype was formed properly, the resulting dual band filter of Figure 16.48.a is ready for conversion into CT and CQ sections.

- Conversion into CT and CQ forms follows similar pattern for all sections.

- With cursor under the first inverter at left, the command Inverters / Replace Inverter by / Pi-Tee Type / Inductive Pi is applied to convert the inverter into inductive Pi-section with negative shunt elements. The negative shunt inductors are combined with adjacent positive inductors using Simplify Circuit command (Figure 16.48.b).

- Then, with cursor under the FTZ element (series arm parallel resonator) the command CT-CQ-CN Oprs / Convert to CT/ CLL is applied to convert the circuit section into a CT element with capacitive cross-coupling (Figure 16.48.c).

- This transformation destroys equality of the shunt inductors. This property can be recovered by applying the command Inverters / Equate All Elements / Equal Inds (Figure 16.48.d). Thus, transformation of the first circuit section on left into a CT section is completed.

- Next, with cursor under the second inverter the command Inverters / Replace Inverter by / Pi-Tee Type / Inductive Pi is applied to convert the inverter into inductive Pi-section with negative shunt elements. The negative shunt inductors are combined with adjacent positive inductors using Simplify Circuit command (Figure 16.48.e).

- Then, with cursor under the fourth order section the command CT-CQ-CN Oprs / Convert to CQ-CN/ Solution-1 is applied to convert the circuit section into a CQ24 element with diagonal cross-coupling (Figure 16.48.f). The diagonal cross-coupling elements results from numerical inaccuracies due to approximations. That is, they are ineffective as the diagonal element values indicate. Thus, it is actually the classical CQ sections with negative 1-4 cross-coupling.

- This transformation destroys equality of the shunt inductors. This property can be recovered by applying the command Inverters / Equate All Elements / Equal Inds (Figure 16.48.g). Thus, transformation of the first circuit section on left into a CT section is completed.

- The same operations are carried out to form the other CT and CQ sections, as described in Figures 16.48.h-to-r.

- Figure 16.48.s shows a realization schematics using coaxial resonators. We note that there are 14 resonators, two more than a Chebyshev counterpart of the same degree. The two extra resonators are due to the two FTZ elements.

- External Q and Coupling Coefficients of the main line resonators can be displayed as shown in Figure 16.48.t.

- The Coupling Coefficients and Resonator Frequencies are displayed in Figure 16.48.u.

- Scaling of the element values can be done and the effects of finite Q of the resonators can be inspected using the same approach as the previous examples.

Figure 16.48. Design of a dual band filter using degree 12 prototype.

Figure 16.48. Design of a dual band filter using degree 12 prototype.

Figure 16.48. Design of a dual band filter using degree 12 prototype.

Figure 16.48. Design of a dual band filter using degree 12 prototype.

### References

- C.-H. Chang, H.-A. Wu, H. J. Yang and C.-K.C. Tzuang, “Coalesced Single input single output dual band filter”, 2003 IEEE MTT-S Int. Digest, pp. 511-514, June 2001.

- C. Quendo, E. Rius and C. Person, “An original topology of dual band filter with transmission zeros,” 2003 IEEE MTTS International Microwave Symposium Digest, pp. 1093-1096.

- A. A. A. Apriyanu and Z. Y. Ping, “A dual band BPF for concurrent dual band wireless transceiver,” 2003 Electronics Packaging Technology Conference, pp. 145-149.

- L. C. Tsai and C. W. Hsue, “Dual band bandpass filters using equal length coupled-serial-shunted lines and Z-transform technique,”  IEEE, Microwave Theory and Techniques, Vol.52, No. 6, pp.1111-1117, April 2004.

- H.-M. Lee, C.-R. Chen, C.-C. Tsai and C.-M. Tsai, “Dual Band Coupling and Feed Structure for Microstrip Filter Design”, 2004 IEEE MTTS International Microwave Symposium Digest, pp. 1971-1974.

- H. M. Lee, C. M. Tsai and C. C. Tsai, “ Transmission Line Filter Design with Fully controllable Second Bandwidth” 2005 IEEE MTTS International Microwave Symposium Digest, pp. 2191-2194.

- H.-Y. Anita Yim, and K.-K. Michael Cheng “Novel Dual Band Planar Resonator and Admittance Inverter for Filter Design and Applications”, 2005 IEEE MTTS International Microwave Symposium Digest, pp. 2187-2190.

- S. Sun and L. Zhu, “Coupling Dispersion of Parallel Coupled Microstrip Lines for Dual Band Filters with Controllable Fractional Bandwidths”, 2005 IEEE MTTS International Microwave Symposium Digest, pp. 2195-2198.

- R. Bairawasubramanian, S. Pinel, J. Papapolymerou, J.Laskar, C. Quendo, E. Rius, A. Manchec, C. Person, “Dual Band Filters for WLAN Applications on Liquid Crystal Polymer Technology”, 2005 IEEE MTTS International Microwave Symposium Digest, pp. 533-536.

- Q. X. Chu and W. Ma, “A new dual band step impedance bandpass filter,” 2005 IEEE MTTS International Microwave Symposium Digest, pp. 610-613.

- J. T. Kuo, T. H. Yeh and C. C. Yeh, “Design of microstrip bandpass filters with a dual passband response,” IEEE, Microwave Theory and Techniques, Vol.53, No.46, pp.1331-1337, April 2005.

- G. Macchiarella, S. Tamiazzo “A design Technique for symmetric Dual Band filters,” 2005 IEEE MTTS International Microwave Symposium Digest, pp. 115-118.

- G. Macchiarella, S. Tamiazzo “Design Techniques for Dual Band filters,” IEEE, Microwave Theory and Techniques, Vol.53, No.11, pp.3265-3271, Nov. 2005.