# FILPRO

A FILTER-MULTIPLEXER DESIGN SOFTWARE

## NEVZAT YILDIRIM

Prof. Dr.

MIDDLE EAST TECHNICAL UNIVERSITY
ANKARA, TURKEY

PREFACE

The material for this book and the associated computer program Filpro had born from class notes for undergraduate and graduate microwave engineering and filter design courses and elaborated through M. Sc. and Ph. D. theses of a number of students at Middle East Technical University, Ankara, Turkey, verified and refined through industrial applications in a wide spectrum from audio to millimeterwave frequencies at Aselsan Military Industries Inc. However it’s main evolution speeded up on Internet, through guidance, suggestions, contributions and helpful criticisms of many colleages working at different companies and universities from diverse countries, from USA to Australia, Japan, Israel, England, Sweden, to name a few.

The program Filpro and this book (which may be treated as both a manual for Filpro and as a textbook on filter design) gather a large amount of information on lumped and distributed element filter synthesis and circuit transformation techniques which can be found only in a widely scattered form in various textbooks, handbooks, periodicals and conference proceedings. Some materials are unpublished anywhere.

Filpro is aimed to be a stand alone, integrated (analysis+synthesis+circuit transformation) filter design tool. However it would form a very handy companion for any filter textbook or handbook to speed up almost all stages of filter design process and to speed up search for alternative solutions compared to the classical approaches.

The motivation for collecting this much material on a single software arised from the (well-known) discouragingly tedious nature of all stages of classical filter design procedures as summarized below:

Typical filter design procedures can roughly be broken down into three distinct stages:

- Approximation and synthesis.

- Identifying topologies which correspond to realizable filters with desirable specifications.

- Circuit transformations to convert the synthesized structures into targeted topologies with convenient element values for realization with the intended manufacturing technology.

Filpro integrates these stages in a single compact software to give a clear overall picture of typical RF and microwave filter design processes and it introduces new features in all these stages to speed up iteration cycles needed in almost all design problems as follows:

i ) Approximation and Synthesis:

Approximation and synthesis techniques had matured long ago sufficiently to answer the needs for most of the amplitude filtering applications. Many academic and/or professional-commercial tools are available on this topic. However it is still a time consuming process to tailor response both to meet specifications and to get realizable results. A lot of iterations may be needed at these stages. In this area Filpro brings the following conveniences:

- In addition to reactance transformation techniques from built-in or external prototype tables, Filpro includes a powerful and accurate synthesis tool for lumped and distributed element filter design by specifying transmission zeros.

- User can watch the shaping up of filter response while transmission zeros are entered.

- User can tune transmission zeros while watching the response.

- It is possible to watch the changes in element values simultaneously as transmission zeros are tuned to get a desired response.

- Filpro learns the order of element extraction when it is carried out the first time, so that in forthcoming iterations Filpro can automatically execute element extraction process.

- The extracted elements are automatically displayed together with element values so that if desired, user can restart synthesis without going to the end of synthesis process.

- The synthesized circuit is displayed together with a cursor used to point elements. Parameters of pointed element are displayed in a box. User can move either the cursor or the circuit to handle very high degree filters.

- Various responses (insertion and return loss, phase, delay, Smith chart, VSWR, S11, Zin, Yin, etc. ) can be drawn instantly in single or overplot modes together with tools like marker, delta marker, rescale, replot and zoom, making life easier.

- Tools like element sensitivity, Monte Carlo, worst case, best case, yield analysis, together with brick wall type tolerance boundaries speed up decision process a lot.

All these and many other dynamic simulation-display tools speed up approximation and synthesis stages a lot compared to the classical approaches.

ii, iii ) Alternative Topologies and Circuit Transformations:

The second and third stages form usually the most significant part of any filter design process. However these are also the most cumbersome and usually least known stages: Discouraging for students, boring, time consuming and complicated for engineers. Having a synthesis tool or having a readily synthesized circuit doesn't mean much without the necessary tools to search for realizable alternative topologies and the tools to reach the targeted topologies. Usually special softwares are needed accepting the synthesized filters as input, applying all the necessary transformations and yielding the desired filter structure as output. This classical approach has two disadvantages:

- The special softwares can be used usually to get only one topology as output.

- The user will have little or no chance of observing or controlling the parameters in the intermediate steps of circuit transformations.

- Since there can be an indefinite number of candidate alternative structures which may compete in some respects, a designer must develop (or buy) as many tools as the number of targeted structures. Since this is usually impossible, design engineers are not sure about the optimality of their circuit. It is in these respects that Filpro makes its most important contributions:

Almost all circuit transformations found in literature necessary to reach almost any (practical, realizable, commonly used or special ) targeted topology for RF and microwave filters are programmed and placed into Filpro in an easy to use menu format. That is, the program contains a general purpose circuit transformation section from which the user can compose the set of transformations (prescription) necessary to reach a desired topology.

In a typical filter design process using Filpro circuit transformations are applied in the following way:

- Circuit cursor is used to point the portion of circuit to which the transformation will be applied.

- The necessary circuit transformation is chosen from the menu and the "Enter" key is pressed. That portion of the circuit will automatically be replaced by the new equivalent.

- Repeating that procedure for all transformations in the prescription, the desired circuit will be obtained.

- For some high degree filters on which a lot of iterations have to be carried out and a lot of circuit transformations have to be applied in each iteration, one may speed up the process using "macro" programming feature of Filpro in which case Filpro learns the necessary steps in the first iteration and can repeat them automatically in successive iterations.

Thus, Filpro has also the ability of carrying out all transformations with a single command

- Since Filpro is transparent to the user (element values are continuously observable ) and since user has all the control, he/she can intervene at all stages of circuit transformations to try new directions in the search of possible alternative solutions.

This approach turns the really tedious process of circuit transformations into an exciting video game, yielding desired circuits quickly.									      
In most applications necessary circuit transformations form a barrier, blocking the view of designer 
such that one may easily miss a very attractive, realizable solution. Besides, most of these transformations are really difficult to derive, discouraging students and engineers from even attempting to enter that area. Availability of various circuit transformations reduce such barriers a lot, making possible to discover and inspect alternative equivalent solutions quickly.

Calculation of physical production parameters (e.g. line thickness, length, etc.) and optimization are usually the final stages of any filter design process. Since many different production techniques exist, each requiring distinctly different theories and tools (most of which are readily available in academic or commercial forms) these stages are left outside the scope of Filpro.

Filpro had been used and tested in both microwave engineering and filter design courses very successfully. Only minimal theory is presented on blackboard. Students spend most of the class hours on computer, grasping the ideas and concepts quickly which would, otherwise, take many class hours on blackboard. Such acceleration is necessary in today’s curriculum conditions where little time is left for analog concepts. Students and engineers absorb the concepts needed in most of the applications easily and leave the course in satisfaction.

Since the software contains materials in a broad spectrum, from basic simple techniques to the most advanced research subjects, from the old, classical techniques to new tools that may be needed in modern, subminiature, superconducting or active microwave filters, it is also a very handy tool for those who will follow advanced academic or industrial research lines in any of the above mentioned stages of design process as a search tool for uncovering new solutions to new or old problems. It had been successfully tested and used in realizing many RF and microwave filters and multiplexers in industry from which considerable feedback is taken for improvements.

The software can also be used as a “quick-soft network analyzer” to simulate and analyze cascaded element type networks, other than filters, which can be configured easily by inserting elements chosen from the library of the program which contains over hundred different passive elements or element groups (including coupled lines, coupled coils, inverters, stubs, exponential lines, etc. ). In this respect, it had found application in laboratories of courses like microwaves, electronics and circuit theory or in industrial laboratories as a quick simulation and design tool. For example, optimum narrowband lumped and distributed element impedance matching structures can be quickly designed using Filpro through iterative analysis by tuning elements while observing Smith Chart or Insertion Loss response.

Active two ports can also be inserted after characterizing them through (measured or calculated) scattering parameters. Diplexers and most of the classical multiplexers can be configured and analyzed by connecting channels in parallel.

The software is a single diskette, PC type program, prepared “one per student/ engineer” in mind.

Chapter 1 is devoted to description of the use of Filpro as a design, analysis and circuit transformation tool. However, its power and capacity can best be evaluated through applications described in the succeeding chapters. Therefore, excepting Chapters 2 and 7, all the other chapters can be accepted as application manual of Filpro. In Chapters 2 and 7, the basic definitions related to lumped and distributed filters are summarized together with concepts thought as stepping stones in filter theory. With some additional details, this book may also be used as a textbook on RF and microwave filters.

Although almost all examples and applications found in open literature are included in this book as demonstration of capabilities of Filpro, there are too many circuit transformations left without proper examples for either lack of space or lack of experience with some types of filters. Therefore the prospective users of this software are expected to find novel applications. Authors would be very glad to hear from such colleagues and increase the spectrum of application examples.

This software is a culmination of about fifteen years of intensive work, during which many M. Sc. students had contributed in different ways through applications in their theses to whom we owe a lot of thanks:

- The formulation and programming of “synthesis” section was carried out by  Ünal Koyaz and Yakup Sen, as parts of their M. Sc. and Ph. D. Theses.

- Cross-coupled filter synthesis by pole placement is carried out by Yakup Sen as part of his Ph. D. Thesis.

- Diplexer/ multiplexer part is formulated and programmed by Ozlem Apaydin Sen as part of her M. Sc. Thesis.

- Remzi Uzun prepared the first version of Filpro as part of his M. Sc. thesis..

- Bülent Türkoglu prepared the second version of Filpro, as part of his M. Sc. Theses.

- Ü. Kenan Çaglar (M. Sc. ) had carried out research on miniature stripline lowpass filters.

- Mustafa Akkul (M. Sc. ) had carried out research and literature survey on wideband subminiature bandpass filters.

- Ibrahim Tekin carried out research and literature survey on narrowband bandpass filters.

- Ali Dengizek  carried out research and literature survey on stripline bandstop filters.

- Ali Aslan carried out research and literature survey on stripline highpass filters.

- Adnan Köksal (Ph. D. ) had caaried out research on impedance matching.

- Many of my former M. Sc. students at Middle East Technical University, Electrical and Electronics Engineering Department, Ankara, had contributed to development of Filpro in many respects.

- The results of all these academic efforts are brought together, structured and integrated in a very sophisticated software by Mehmet Karaaslan as part of his M. Sc. Thesis. He continued improving the software and widening it’s spectrum with a completely amateur enthusiasm even after graduation. Although our aim was purely academic since the beginning of these studies, mostly for easing the life of both teachers and students, the result came out to be a really professional one.

We owe special thanks to managers of Aselsan Electronic Industries Inc. for their continuous support throughout all these academic activities and for opening all facilities for manufacturing many kinds of filters.

Contents of the software is almost doubled after getting in touch with Dr. Peter Onno of  M/A-COM Corporate Research and Development Center, Lowell, Massachussets, for which the author is working as a consultant. Many suggestions based on his broad experience and lengthy discussions lead to widening the spectrum of Filpro a lot.

We also owe thanks to Prof. Dr. Richard Snyder of RS Microwave for many useful discussions and suggestions on improving many aspects of Filpro.

Also special thanks are due to Dieter Pelz of R.F.S. of Australia for many useful discussions and suggestions throughout the development of Cascaded Quadruplet and Cascaded Triplet filters.

Thanks are also due to Victor Sharir of Galor-MIC of Israel and John Parker, Consultant of Saab-Ericsson of Sweden, for encouragements and suggestions.

Prof. Dr. Nevzat YILDIRIM

Middle East Technical University

Electrical and Electronics Engineering Department

Ankara, Türkiye

(1. 11. 1997)